# Title:	Test Tolerances for inter-RAT SS-RSRP measurement accuracy tests in FR2

# Source:	Rohde & Schwarz

# 1. Introduction

The purpose of this document is to present the methodology and principles used for the development of Test Tolerances for SS-RSRP inter-RAT measurement accuracy. The serving cell is E-UTRA and the target cell is NR FR2.

The test cases in question, from TS 38.533, are:

- 8.5.2.1.2, E-UTRA – NR FR2 SS-RSRP measurement accuracy

The applicability of the analysis is:

- UE Power class 3

This document describes the process to derive the Test Tolerances. The calculations are provided in the accompanying spreadsheet.

# 2. Test case in Annex A of TS 38.133

The Test Requirements are defined in the following extracts from TS 38.133 v16.6.0 Annex A, with updates as proposed in TS 38.133 CRs at RAN4#97-e in revision marks.

A standalone test case is chosen, to show only the NR-related parameters.

<< Extract from TS 38.133 >>

##### A.8.5.2.1.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell

###### A.8.5.2.1.2.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clause 9.11.1 in TS 36.133 [15] for inter-RAT FR2 SS-RSRP measurements.

###### A.8.5.2.1.2.2	Test Parameters

Supported test configurations are shown in Table A.8.5.2.1.2.2-1. In this test case there are two cells on different carriers. Absolute accuracy requirements of SS-RSRP inter-RAT measurement are tested by using test setup in Table A.8.5.2.1.2.2-2 and Table A.8.5.2.1.2.2-3. In all test cases, Cell 2 is target cell. Cell 1 is the E-UTRA cell which specific test parameters for this test case are specified in Table A.3.7.2.1-1.

Table A.8.5.2.1.2.2-1: SS-RSRP Inter-RAT SS-RSRP supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.8.5.2.1.2.2-2: SS-RSRP Inter-RAT general test parameters

| Parameter | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
|  |  | Cell 2 | Cell 2 |
| SSB ARFCN |  | Freq1 | freq1 |
| Duplex mode |  | TDD | TDD |
| TDD configuration |  | TDDConf.3.1 | TDDConf.3.1 |
| BWchannel | MHz | 100: NRB,c = 66 | 100: NRB,c = 66 |
| Downlink initial BWP configuration |  | DLBWP.0.1 |  |
| Downlink dedicated BWP configuration |  | DLBWP.1.1 |  |
| Uplink initial BWP configuration |  | ULBWP.0.1 |  |
| Uplink dedicated BWP configuration |  | ULBWP.1.1 |  |
| DRX cycle configuration | ms | Not applicable |  |
| TRS configuration |  | TRS.2.1 TDD |  |
| TCI state |  | TCI.State.0 |  |
| PDSCH Reference measurement channel |  | - | - |
| RMSI CORESET Reference Channel |  | - | - |
| OCNG Patterns |  | OP.1 | OP.1 |
| SMTC configuration |  | SMTC.1 FR2 | SMTC.1 FR2 |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 | 120 |
| EPRE ratio of PSS to SSS | dB | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: Void Note 4: Void |  |  |  |

Table A.8.5.2.1.2.2-3: SS-RSRP Inter-RAT OTA related test parameters

| Parameter | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
|  |  | Cell 2 | Cell 2 |
| Angle of arrival configuration |  | Setup 1 according to A.3.15.1 | Setup 1 according to A.3.15.1 |
| Assumption for UE beamsNote 8 |  | Rough | Rough |
| Note 1 | dBm/15kHz Note 4 | -105 | N/A |
| Note 1 | dBm/SCS Note 4 | -96 | N/A |
|  | dB | 11 | N/A |
| Es | dBm/SCS Note 4 |  | (Table B.2.3-2 Rx Beam Peak +1dB) |
| SSB_RPNote 2 | dBm/SCS Note 4 | -85 | (Table B.2.3-2 Rx Beam Peak +1dB) |
| BB Note 7 | dB | 9.97 | -3.81 |
| IoNote 2 | dBm/95.04 MHz Note 4 | -55.65 | (Table B.2.3-2 Rx Beam Peak +30dB) |
| Note 1: Where used, interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SSB_RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Void Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: Void Note 6: Void. Note 7: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 36.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. Note 8: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |

###### A.8.5.2.1.2.3	Test Requirements

The SS-RSRP measurement accuracy for Cell 2 shall fulfil the requirement in clause 9.11.1 in TS 36.133 [15].

# 3. Discussion

The test consists of two subtests, each with one NR cell configured. Test 1 has Cell 2 with Es/Iot above the side condition and with high SSB_RP. In Test 2 Cell 2 has Es/Iot at the lower limit of the side condition, no applied noise, and SSB_RP at the lower limit of the side condition.

# 4. Calculation of Test Tolerances

General approach

The general approach is given in the steps below:

- Copy the originally specified key parameters from the core requirements

- Where relevant, calculate derived parameters from the core requirements

- Define uncertainties for a minimum set of parameters

- Define controlled parameters (critical to the test verdict), calculate sensitivity factors and uncertainty

- Determine which parameters to offset (apply Test Tolerances to) and by how much

- Recalculate original or derived parameters including Test Tolerances

- Check that the controlled parameters meet requirements to get the correct test verdict

Each step is explained below, and the calculations are given in the accompanying spreadsheet.

a) Original specified key parameters

The key parameters are copied from Tables A.8.5.2.1.2.2-1, A.8.5.2.1.2.2-2 and A.8.5.2.1.2.2-3 in TS 38.133. The key parameters are selected as the minimum set to define the cell power level, and which are subject to a test system uncertainty which may affect the verdict of the test. The angle of arrival of the radiated signals is also recorded.

A number of UE core requirement values and their associated parameters are also entered in this section, taking into account the angle of arrival of the radiated signals. The requirements are entered because they affect the radiated signal received by the UE, or affect how the UE sees the received signals at baseband.

The key parameters appear in section a) of the accompanying spreadsheet.

b) Derived parameters

A number of derived parameters are calculated, using the base information in a). The reason for deriving each additional parameter is given in the “Comment” column of section b) in the accompanying spreadsheet.

Some UE core requirements in section a) are converted into UE-related derived parameters, in form that allows them to be used in later sections of the spreadsheet. The rationale and method for UE gain G midpoint is given in TR 38.903 clause A.2.6, and for UE Spherical coverage gain midpoint in TR 38.903 clause A.2.1, although spherical coverage is not applicable in this test case and is greyed out.

c) Uncertainties

In Test 1, the Test system provides 1 NR cell on 1 frequency, with AWGN. We propose to control the following parameters:

- AWGN Absolute power, Noc averaged over BWConfig uncertainty: ±5.65 dB

- AWGN Absolute power, Noc for PRBs #RBJ-RBJ+19 uncertainty: ±5.65 dB

- Ratio of cell signal / AWGN, Ês / Noc averaged over BWConfig uncertainty: ±0.3 dB

- Ratio of cell signal / AWGN, Ês / Noc for PRBs #RBJ-RBJ+19 uncertainty: ±0.3 dB, assuming baseband combining of the signal and the AWGN

In Test 2, the Test system provides 1 NR cell on 1 frequency, without AWGN. We propose to control the following parameters:

- Cell 1 Ês averaged over BWConfig uncertainty: ±5.65dB

- Cell 1 Ês for PRBs #RBJ-RBJ+19 uncertainty: ±5.65dB

UE-related uncertainties are considered, together with selected UE core requirements that have an allowed range, and are therefore handled as a UE uncertainty:

- UE SS-RSRP absolute accuracy: Value taken from TS 38.133

- Spherical coverage gain variation: Value derived, using method defined in TR 38.903 A.2.1

- UE gain G variation: Value derived, using method defined in TR 38.903 A.2.6

In this test case Spherical coverage gain variation is not used, because the signals come from Rx beam peak direction, but is greyed out and left as a placeholder.

<< Extracts from TS 38.133 v16.6.0 >>

#### 10.1.5.1	Inter-frequency SS-RSRP accuracy requirements

##### 10.1.5.1.1	Absolute SS-RSRP Accuracy

Unless otherwise specified, the requirements for absolute accuracy of SS-RSRP in this clause apply to a cell on a frequency in FR2 that is on a different frequency than the serving cell.

The accuracy requirements in Table 10.1.5.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band for each relevant SSB.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.5.1.1-1: SS-RSRP Inter frequency absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 2 range |  |  |  |
|  |  |  | Minimum Io |  |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 1 |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |  |
| 6 | 9 | ≥-4 | Same value as SSB_RP in Table B.2.3-2, according to UE Power class, operating band and angle of arrival |  | N/A | -70 |
| 8 | 11 |  | N/A |  | -70 | -50 |
| Note 1: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. Note 2: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. Note 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |

<< clauses skipped >>

Table B.2.3-2: Conditions for inter-frequency measurements in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE power class |  |  |  | UE power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |
| Conditions | Rx Beam Peak | n257 | -126.3+Y1 | -111.8 | -110.1 | -125.8+Y4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-4 |
|  |  | n258 | -126.3+Y1 | -111.8 | -110.1 | -125.8+Y4 |  |  |
|  |  | n259 |  |  | -106.5 |  |  |  |
|  |  | n260 | -123.3+Y1 |  | -107.5 | -123.8+Y4 |  |  |
|  |  | n261 | -126.3+Y1 | -111.8 | -110.1 | -125.8+Y4 |  |  |
|  | Spherical coverage Note 1 | n257 | -118.3+Z1 | -100.8 | -99.2 | -116.8+Z4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-4 |
|  |  | n258 | -118.3+Z1 | -100.8 | -99.2 | -116.8+Z4 |  |  |
|  |  | n259 |  |  | -93.7 |  |  |  |
|  |  | n260 | -115.3+Z1 |  | -94.9 | -111.8+Z4 |  |  |
|  |  | n261 | -118.3+Z1 | -100.8 | -99.2 | -116.8+Z4 |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ΣMBP and Spherical coverage values are increased by ΣMBS, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |

Normal conditions with Io up to -50dBm are selected for Test 1 and Test 2. The nominal Io during Test 2 is close to <-70dBm, but after applying Test Tolerances and including uncertainties the value can be above -70dBm.

d) Controlled parameters critical to verdict

In many RRM test cases there is not a simple one-to-one relationship between the parameters that can be set by the test equipment, and their effect on parameters determining the test verdict.

It is therefore essential to identify those parameters determining the test verdict, and also the side conditions that should be met for the UE to be tested correctly. The 4 controlled parameters listed in the accompanying spreadsheet have been derived by study of the test case and by careful reading of the relevant clauses in TS 38.133. The reason for each parameter being critical to the test verdict is given briefly in the “Comment” column of section d) in the accompanying spreadsheet. Information about the value to be achieved is given in the “Requirement” column of section g) in the spreadsheet.

Having identified the parameters critical to the test verdict which need to be controlled, we now need to consider how they are affected by the parameters which can be set by the test equipment. This is done by working out “sensitivity factors”. A sensitivity factor is just the ratio (effect on a critical parameter y / a test equipment uncertainty x), and is usually in dB/dB.

Where there is a simple one-to-one relationship between the parameters that can be set by the test equipment, and their effect on parameters determining the test verdict, the sensitivity factors are derived by inspection as one, or if no effect, as zero.

Where there is not a simple one-to-one relationship between the parameters that can be set by the test equipment, and their effect on parameters determining the test verdict, the calculation of sensitivity factors is done using a method selected from TR 38.903. Cell 1 Es/Iot at UE baseband is calculated according to TR 38.903 A.2.4, and applied Io calculated according to TR 38.903 A.2.5.

Sensitivity factors for UE-reported SS-RSRP also follow methods defined in TR 38.903. The UE-related factors are from clause A.2.6, depending on the type of measurement and the the test setup. UE-reported SS-RSRP is affected by a combination of Test system uncertainties and UE-related uncertainties.

The method of combining uncertainties is different for Test 1 and Test 2 because Test 1 has applied AWGN, whereas Test 2 does not. In Test 1, the test system uncertainties are combined root-sum-square as they are uncorrelated. In Test 2, the test system uncertainties are combined root-sum-square where uncorrelated, and arithmetically where correlated. More detailed explanation is given in the accompanying spreadsheet, and in TR 38.903 A.2.7.

Having filled in the matrix of sensitivity factors, the accompanying spreadsheet calculates the overall uncertainty for each controlled parameter, taking into account the uncertainties and sensitivity factors for each parameter that can be set by the test equipment, and the UE-related uncertainties where relevant. This process follows the superposition principle. More details and explanation can be found in section 4 of TR 38.903.

e) Determine parameters to offset

In Test 1, the variation due to uncertainties would make Io greater than the -50dBm side condition, and the UE measurements may not be valid. The AWGN is therefore reduced by 0.1 dB to meet the side condition.

In Test 2, the variation due to uncertainties would make SSB_RP lower than the side condition (depending on the operating band group) and the UE measurements may not be valid. The SSB_RP is therefore increased by 5.65 dB to meet the side condition.

f) Parameters modified by Test Tolerances

Based on the decision in e), the set of parameters in a) and b) is reproduced in section f) of the accompanying spreadsheet, modified by the Test Tolerances (applied offsets).

Re-derived parameters are calculated using the same methods as were used in step b).

g) Check controlled parameters Min/Max

Using a format similar to that in step d), the nominal value of each controlled parameter is recalculated, as some will have changed from the original due to the application of the Test Tolerances in step f).

The minimum and maximum values, due to variability from uncertainties, of controlled parameters is then calculated and compared against the requirements (Es/IotBB, Applied SSB_RP and Applied Io). The critical requirement for each parameter is given in the “Requirement” column of section g) in the accompanying spreadsheet. The cases closest to limit are identified by turquoise cells in the spreadsheet. If all the requirements are met, then the chosen offsets are acceptable. It can be seen that with the uncertainty values and Test Tolerances proposed, the requirements are met.

For this test, the verdict is based on the SS-RSRP values reported by the UE. As stated earlier, the reported values are affected by:

- The uncertainties in the stimulus set by the Test system (Downlink signal)

- The uncertainties in the UE response (SS-RSRP reports)

Having ensured that the stimulus set by the Test system remains within the side conditions, and knowing its uncertainty, we now need to calculate the range of SS-RSRP values that a conformant UE could report. The mapping is defined in TS 38.133:

### 10.1.6 RSRP Measurement Report Mapping

The reporting range of SS-RSRP for L3 reporting is defined from -156 dBm to -31 dBm with 1 dB resolution. The reporting range of SS-RSRP and CSI-RSRP for L1 reporting is defined from -140 to -44 dBm with 1 dB resolution.

The mapping of measured quantity is defined in Table 10.1.6.1-1. The range in the signalling may be larger than the guaranteed accuracy range.

The reporting range of differential SS-RSRP and CSI-RSRP for L1 reporting is defined from 0 dBm to -30 dB with 2 dB resolution.

The mapping of measured quantity is defined in Table 10.1.6.1-2. The range in the signalling may be larger than the guaranteed accuracy range.

Table 10.1.6.1-1: SS-RSRP and CSI-RSRP measurement report mapping

| Reported value | Measured quantity value (L3 SS-RSRP) | Measured quantity value (L1 SS-RSRP and CSI-RSRP) | Unit |
| --- | --- | --- | --- |
| RSRP_0 | SS-RSRP<-156 | Not valid | dBm |
| RSRP_1 | -156≤ SS-RSRP<-155 | Not valid | dBm |
|  | << rows skipped >> |  |  |
| RSRP_15 | -142≤ SS-RSRP<-141 | Not valid | dBm |
| RSRP_16 | -141≤ SS-RSRP<-140 | RSRP<-140 | dBm |
| RSRP_17 | -140≤ SS-RSRP<-139 | -140≤RSRP<-139 | dBm |
| RSRP_18 | -139≤ SS-RSRP<-138 | -139≤ RSRP<-138 | dBm |
| … | … |  | … |
| RSRP_111 | -46≤ SS-RSRP<-45 | -46≤ RSRP<-45 | dBm |
| RSRP_112 | -45≤ SS-RSRP<-44 | -45≤ RSRP<-44 | dBm |
| RSRP_113 | -44≤ SS-RSRP<-43 | -44≤ RSRP | dBm |
| RSRP_114 | -43≤ SS-RSRP<-42 | Not valid | dBm |
|  | << rows skipped >> |  |  |
| RSRP_125 | -32≤ SS-RSRP<-31 | Not valid | dBm |
| RSRP_126 | -31≤ SS-RSRP | Not valid | dBm |
| RSRP_127 (Note) | Infinity | Infinity | dBm |
| Note: The value of RSRP_127 is applicable for RSRP threshold configured by the network as defined in TS 38.331 [2], but not for the purpose of measurement reporting. |  |  |  |

The acceptable range of UE reported SS-RSRP values is calculated taking into account the uncertainties in the stimulus set by the Test system, the variations and uncertainties in the UE response, and the UE measurement report mapping function. The calculation is done in section g) of the accompanying spreadsheet.

RSRP values calculated are for normal conditions. For extreme conditions, add 3dB extra at each end.

Absolute SS-RSRP reported values

The rationale and method is given in TR 38.903 clause A.2.6.2.

# 5. Treatment of other configurations within the same test case

Not applicable, as all the configurations defined have the same NR cell parameters.