DTS/INT 101 606-3 V0.0.6 (2012-08)

IMS Network Testing (INT);

Diameter Conformance testing for Gx interface;

Part 3: Abstract Test Suite (ATS) and partial Protocol

Implementation eXtra Information for Testing (PIXIT)

proforma specification

<

TECHNICAL SPECIFICATION

Reference

DTS/INT-00068-3

Keywords

ATS, PIXIT, DIAMETER

ETSI

650 Route des Lucioles

F-06921 Sophia Antipolis Cedex - FRANCE

Tel.: +33 4 92 94 42 00   Fax: +33 4 93 65 47 16

Siret N° 348 623 562 00017 - NAF 742 C

Association à but non lucratif enregistrée à la

Sous-Préfecture de Grasse (06) N° 7803/88

Important notice

Individual copies of the present document can be downloaded from:
http://www.etsi.org

The present document may be made available in more than one electronic version or in print. In any case of existing or perceived difference in contents between such versions, the reference version is the Portable Document Format (PDF). In case of dispute, the reference shall be the printing on ETSI printers of the PDF version kept on a specific network drive within ETSI Secretariat.

Users of the present document should be aware that the document may be subject to revision or change of status. Information on the current status of this and other ETSI documents is available at http://portal.etsi.org/tb/status/status.asp

If you find errors in the present document, please send your comment to one of the following services:
http://portal.etsi.org/chaircor/ETSI_support.asp

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© European Telecommunications Standards Institute yyyy.

All rights reserved.

DECTTM, PLUGTESTSTM, UMTSTM and the ETSI logo are Trade Marks of ETSI registered for the benefit of its Members.
3GPPTM and LTE™ are Trade Marks of ETSI registered for the benefit of its Members and
of the 3GPP Organizational Partners.
GSM® and the GSM logo are Trade Marks registered and owned by the GSM Association.

Contents

Intellectual Property Rights	5

Foreword	5

1	Scope	6

2	References	6

2.1	Normative references	6

2.2	Informative references	7

3	Definitions and abbreviations	7

3.1	Definitions	7

3.2	Abbreviations	7

4	Abstract Test Method (ATM)	7

4.1	Test architecture	7

4.1.1	Test method	7

4.1.2	Test machine configuration	7

4.1.2.1	Test configurations for PCEF testing	7

4.1.2.2	Test configurations for PCRF testing	8

4.1.3	Interconnection of TS and SUT	9

4.1.3.1	PCEF Role	9

4.1.3.2	PCRF Role	10

5	Untestable Test Purposes (TP)	11

6	ATS conventions	11

6.1	Testing conventions	11

6.1.1	Test cases Preamble and Postamble	11

6.2	Naming conventions	11

6.2.1	General guidelines	11

6.2.2	Test case grouping	12

6.2.3	Test case identifiers	12

Annex A (normative): DIAMETER Gx Abstract Test Suite (ATS)	14

A.1	The TTCN-3 Module	14

Annex B (normative): DIAMETER Gx Partial PIXIT proforma for IUT PCRF	15

B.1	Identification summary	15

B.2	ATS summary	15

B.3	Test laboratory	15

B.4	Client identification	15

B.5	SUT	16

B.6	Protocol layer information	16

B.6.1	Protocol identification	16

B.6.2	Configuration	16

Annex C (normative): DIAMETER Gx Partial PIXIT proforma for IUT PCEF	19

C.1	Identification summary	19

C.2	ATS summary	19

C.3	Test laboratory	19

C.4	Client identification	19

C.5	SUT	20

C.6	Protocol layer information	20

C.6.1	Protocol identification	20

C.6.2	Configuration	20

Annex D (normative): DIAMETER Gx PCTR Proforma for IUT PCRF	22

D.1	Identification summary	22

D.1.1	Protocol conformance test report	22

D.1.2	IUT identification	22

D.1.3	Testing environment	22

D.1.4	Limits and reservation	22

D.1.5	Comments	23

D.2	IUT Conformance status	23

D.3	Static conformance summary	23

D.4	Dynamic conformance summary	23

D.5	Static conformance review report	24

D.6	Test campaign report	24

D.7	Observations	25

Annex E (normative): DIAMETER Gx PCTR Proforma for IUT PCEF	26

E.1	Identification summary	26

E.1.1	Protocol conformance test report	26

E.1.2	IUT identification	26

E.1.3	Testing environment	26

E.1.4	Limits and reservation	26

E.1.5	Comments	27

E.2	IUT Conformance status	27

E.3	Static conformance summary	27

E.4	Dynamic conformance summary	27

E.5	Static conformance review report	28

E.6	Test campaign report	29

E.7	Observations	29

History		31

# Intellectual Property Rights

IPRs essential or potentially essential to the present document may have been declared to ETSI. The information pertaining to these essential IPRs, if any, is publicly available for ETSI members and non-members, and can be found in ETSI SR 000 314: "Intellectual Property Rights (IPRs); Essential, or potentially Essential, IPRs notified to ETSI in respect of ETSI standards", which is available from the ETSI Secretariat. Latest updates are available on the ETSI Web server (http://ipr.etsi.org).

Pursuant to the ETSI IPR Policy, no investigation, including IPR searches, has been carried out by ETSI. No guarantee can be given as to the existence of other IPRs not referenced in ETSI SR 000 314 (or the updates on the ETSI Web server) which are, or may be, or may become, essential to the present document.

# Foreword

This Technical Specification (TS) has been produced by IMS Network Testing (INT).

The present document is part 3 of a multi-part deliverable covering the test specifications for the Diameter protocol on the Gx interface, as identified below:

Part 1:	"Protocol Implementation Conformance Statement (PICS)";

Part 2:	"Test Suite Structure (TSS) and Test Purposes (TP)";

Part 3:	"Abstract Test Suite (ATS) and partial Protocol Implementation eXtra Information for Testing (PIXIT) proforma specification".

# 1	Scope

The present document specifies the Abstract Test Suite (ATS) and partial Protocol Implementation eXtra Information for Testing (PIXIT) proforma for the test specifications for Diameter protocol on the Gx interface as specified in TS 129 212 [1] in compliance with the relevant requirements and in accordance with the relevant guidance given in ISO/IEC 9646-7 [5] and ETS 300 406 [6].

The test notation used in the ATS is TTCN-3 (see ES 201 873-1 [7]).

The following test specification- and design considerations can be found in the body of the present document:

the overall test suite structure;

the testing architecture;

the test methods and port definitions;

the test configurations;

TTCN styles and conventions;

the partial PIXIT proforma;

the modules containing the TTCN-3 ATS.

Annex  A provides the Abstract TestSsuite (ATS) part of the ATS.

Annex B provides the Partial Implementation Extra Information for Testing (PIXIT) Proforma for the PCRF part of the ATS.

Annex C provides the Partial Implementation Extra Information for Testing (PIXIT) Proforma for the PCEF part of the ATS.

Annex D provides the PCTR Proforma for the PCRF part of the ATS.

Annex E provides the PCTR Proforma for the PCEF part of the ATS.

# 2	References

References are either specific (identified by date of publication and/or edition number or version number) or non-specific.

For a specific reference, subsequent revisions do not apply.

Non-specific reference may be made only to a complete document or a part thereof and only in the following cases:

if it is accepted that it will be possible to use all future changes of the referenced document for the purposes of the referring document;

for informative references.

Referenced documents which are not found to be publicly available in the expected location might be found at http://docbox.etsi.org/Reference.

NOTE:	While any hyperlinks included in this clause were valid at the time of publication ETSI cannot guarantee their long term validity.

## 2.1	Normative references

The following referenced documents are indispensable for the application of the present document. For dated references, only the edition cited applies. For non-specific references, the latest edition of the referenced document (including any amendments) applies.

[1]	ETSI TS 129 212: "Universal Mobile Telecommunications System (UMTS); LTE; Policy and charging control over Gx/Sd reference point (3GPP TS 29.212 version 10.5.0 Release 10)".

[2]	ETSI TS 101 606-2: "IMS Network Testing (INT); Diameter Conformance testing for Gx interface; Part 2: Test Suite Structure (TSS) and Test Purposes (TP)".

[3]	ISO/IEC 9646-1: "Information technology - Open Systems Interconnection - Conformance testing methodology and framework - Part 1: General concepts".

[4]	ISO/IEC 9646-6: "Information technology - Open Systems Interconnection - Conformance testing methodology and framework - Part 6: Protocol profile test specification".

[5]	ISO/IEC 9646-7: "Information technology - Open Systems Interconnection - Conformance testing methodology and framework - Part 7: Implementation Conformance Statements".

[6]	ETSI ETS 300 406: "Methods for testing and Specification (MTS); Protocol and profile conformance testing specifications; Standardization methodology".

[7]	ETSI ES 201 873-1: "Methods for Testing and Specification (MTS); The Testing and Test Control Notation version 3; Part 1: TTCN-3 Core Language".

## 2.2	Informative references

The following referenced documents are not essential to the use of the present document but they assist the user with regard to a particular subject area. For non-specific references, the latest version of the referenced document (including any amendments) applies.

Not applicable.

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in ISO/IEC 9646-7 [5], and TS 129 212 [1] apply.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in ISO/IEC 9646-1 [3], ISO/IEC 9646-6 [4], ISO/IEC 9646-7 [5], and TS 129 212 [1] apply.

# 4	Abstract Test Method (ATM)

This clause describes the ATM used to test the Diameter protocol on the Gx interface at the PCEF side and at the PCRF side.

## 4.1	Test architecture

### 4.1.1	Test method

The test method chosen is the remote test method. Remote test method means that the test tool (the test machine  + the executable test suite) shall behave as a PCRF when the IUT is a PCEF and shall behave as a PCEF when the IUT is a PCRF. As the exchange between the test system and the IUT is at the diameter message level, the lower layers of the test machine shall be totally conformant with the corresponding lower layers specifications to use the remote test method.

### 4.1.2	Test machine configuration

#### 4.1.2.1	Test configurations for PCEF testing

The Gx interface is located between PCRF and the SUT. The GTP interfaces are used to trigger the SUT.

Figure 1: Test architecture with PCEF as SUT

#### 4.1.2.2	Test configurations for PCRF testing

The Gx interface is located between PCEF and PCRF. The Rx interface is located between PCEF and the SUT.

Figure 2: Test architecture with PCRF as SUT

### 4.1.3	Interconnection of TS and SUT

#### 4.1.3.1	PCEF Role

Figure 3 and figure 4 show the interconnection of TS and SUT in terms of signalling message flows. Diameter messages are transferred over the DIAM port. Lower Layer Primitives are transferred over the LLPP port. Some PCEF tests may require additional triggering via the GTP ports. This configuration can be disabled using PIXIT parameter “GTPsupport”. GTPv1-U messages are exchanged at ports GTP1 and GTP2 (see figure 4).

Figure 3: Interconnection for PCEF role without IP interfaces

Figure 4: Interconnection for PCEF role with IP interfaces (GTP)

#### 4.1.3.2	PCRF Role

Figure 5 and 6 show the interconnection of TS and SUT in terms of signalling message flows. Diameter messages are transferred over the DIAM port. Lower Layer Primitives are transferred over the LLPP port. Some PCRF tests may require additional triggering via the Rx interface (see figure 6).

Figure 5: Interconnection for PCRF role

Figure 6: Interconnection for PCRF role with AF interface (Rx)

#### 4.1.3.3	Test Adapter

For execution of the tests the Test Adapter (TA) shall be developed. There are two possibilities to communicate over TA:

ATS povides only Diameter messages or

ATS provides Diameter messages and LL primitives

# 5	ATS conventions

The ATS conventions are intended to give a better understanding of the ATS but they also describe the conventions made for the development of the ATS. These conventions shall be considered during any later maintenance or further development of the ATS.

The ATS conventions contain two clauses, the naming conventions and the implementation conventions. The naming conventions describe the structure of the naming of all ATS elements. The implementation conventions describe the functional structure of the ATS.

To define the ATS, the guidelines of the document ETS 300 406 [6] were considered.

## 5.1	Testing conventions

### 5.1.1	Test cases Preamble and Postamble

As described in the test method clause the test tool shall behave as a PCRF when the IUT is a PCEF and shall behave as a PCEF when the IUT is a PCRF. For that reason the test case preambles and postambles are named as follows:

IUT is a PCRF (example  TC_PCRF_IPS_01)

f_preamble_PCEF

f_postamble_PCEF

Note:	The tester also behaves as a Diameter Client.

IUT is a PCEF (example  TC_PCEF_IPS_01)

f_preamble_PCRF

f_postamble_PCRF

Note:	The tester also behaves as a Diameter Server.

## 5.2	Naming conventions

### 5.2.1	General guidelines

The naming conventions are based on the following underlying principles:

In most cases, identifiers should be prefixed with a short alphabetic string (specified in table 2) indicating the type of TTCN-3 element it represents.

Suffixes should not be used except in those specific cases identified in table 2.

Prefixes and suffixes should be separated from the body of the identifier with an underscore ("_"):

EXAMPLE:	c_sixteen, t_wait_max.

Only module names, data type names and module parameters should begin with an upper-case letter. All other names (i.e. the part of the identifier following the prefix) should begin with a lower-case letter.

The start of second and subsequent words in an identifier should be indicated by capitalizing the first character. Underscores should not be used for this purpose.

EXAMPLE:	f_authenticateUser.

Table 2 specifies the naming guidelines for each element of the TTCN-3 language indicating the recommended prefix, suffixes (if any) and capitalization.

Table 2:	TTCN-3 naming convention

| Language element | Naming convention | Prefix | Suffix | Example | Notes |
| --- | --- | --- | --- | --- | --- |
| Module | Use upper-case initial letter | DiameterGx_ | none | DiameterGx_Steps |  |
| TSS grouping | Use all upper-case letters | none | none | TP_PCEFRole_IPS |  |
| Message template | Use lower-case initial letter | m_ | none | m_authApplicationId |  |
| Message template with wildcard or matching expression | Use lower-case initial letters | mw_ | none | mw_subscriptionId |  |
| Port instance | Use upper-case initial letter | none | none | DiameterPort |  |
| Constant | Use lower-case initial letter | c_ | none | c_maxRetransmission |  |
| Function | Use lower-case initial letter | f_ | none | f_authentication() |  |
| Altstep | Use lower-case initial letter | a_ | none | a_receive() |  |
| Variable | Use lower-case initial letter | v_ | none | v_basicId |  |
| PICS values | Use all upper case letters | PC_ _ | none | PC_PCRF_DATA_CON | Note |
| PIXIT values | Use all upper case letters | PX_ | none | PX_DIAMETER_IP_ADDR | Note |
| Parameterization | Use lower-case initial letter | p_ | none | p_macId |  |
| Enumerated Value | Use lower-case initial letter | e_ | none | e_synCpk |  |
| NOTE: In this case it is acceptable to use underscore as a word delimiter. |  |  |  |  |  |

### 5.2.2	Test case grouping

The ATS structure is based on theTest Purposes for the Diameter protocol on the Gx interface as defined in ETSI TS 101 606-2 [2].

### 5.2.3	Test case identifiers

The test cases have been divided according to the functionalities into several groups.

The test case names are built up according to the following scheme:

Table 3: TC identifier naming convention scheme

Identifier: "<tc>"_"<iut >"_"<scope >"_"<number>" <tc> = Test Case: fixed to "TC" <iut> = type of IUT: PCRF or PCEF <scope> = group IPS Initial Provisioning Session MSI Modification of Session Information INV Invalid Behaviour ST Session Termination PCC PCC rules EMS Emergency services UMC Usage monitoring control IRS IMS Restoration Support MPS Multimedia Priority Support SDC Sponsored Data Connectivity RAU Reporting Accumulated Usage <number> = sequential number (01-99)

NOTE:	This naming scheme results into an one-to-one correspondence between the test purpose identifiers as defined in ETSI TS 101 606-2 [2] and the test case identifiers.
The TP identifier of the test case TC_xxx_01 is TP_xxx_01.

Annex A (normative):
DIAMETER Gx Abstract Test Suite (ATS)

This ATS has been produced using the Testing and Test Control Notation (TTCN-3) according to ES 201 873-1 [7].

# A.1	The TTCN-3 Module

The TTCN-3 library modules corresponding to the ATS are contained in archive ts101606v010101p0.zip which accompanies the present document.

Annex B (normative):
DIAMETER Gx Partial PIXIT proforma for IUT PCRF

Notwithstanding the provisions of the copyright clause related to the text of the present document, ETSI grants that users of the present document may freely reproduce the Partial PIXIT proforma in this annex so that it can be used for its intended purposes and may further publish the completed Partial PIXIT.

The PIXIT Proforma is based on ISO/IEC 9646-6 [4]. Any needed additional information can be found in this international standard document.

# B.1	Identification summary

Table B.1

| PIXIT Number: |  |
| --- | --- |
| Test Laboratory Name: |  |
| Date of Issue: |  |
| Issued to: |  |

# B.2	ATS summary

Table B.2

| Protocol Specification: | TS 129 212 [1] |
| --- | --- |
| Protocol to be tested: |  |
| ATS Specification: | TS 101 606-3 |
| Abstract Test Method: | TS 101 606-3 clause 4 |

# B.3	Test laboratory

Table B.3

| Test Laboratory Identification: |  |
| --- | --- |
| Test Laboratory Manager: |  |
| Means of Testing: |  |
| SAP Address: |  |

# B.4	Client identification

Table B.4

| Client Identification: |  |
| --- | --- |
| Client Test manager: |  |
| Test Facilities required: |  |

# B.5	SUT

Table B.5

| Name: |  |
| --- | --- |
| Version: |  |
| SCS Number: |  |
| Machine configuration: |  |
| Operating System Identification: |  |
| IUT Identification: |  |
| PICS Reference for IUT: |  |
| Limitations of the SUT: |  |
| Environmental Conditions: |  |

# B.6	Protocol layer information

## B.6.1	Protocol identification

Table B.6

| Name: | TS 129 212 |
| --- | --- |
| Version: |  |
| PICS References: |  |

## B.6.2	Configuration

Tables in this clause need to be filled by the IUT Manufacturer to specify how the IUT needs to be configured with IUT specific values or describe IUT specific procedures required for complete testing of the IUT.

Table B.7: PIXIT for the Gx interface

|  | Gx PIXIT Summary |  |
| --- | --- | --- |
| 1 | PX_DIAMETER_GX_ETS_IPADDR | charstring, IP address of the test system |
| 2 | PX_DIAMETER_GX_SUT_IPADDR | charstring, IP address of the system under test |
| 3 | PX_DIAMETER_GX_ETS_PORT | charstring, Port number of the test system |
| 4 | PX_DIAMETER_GX_SUT_PORT | charstring, Port number of the system under test |
| 5 | PX_SessionID | UTF8String, Session-Id AVP (AVP Code 263) |
| 6 | PX_Logical_Access_ID | octetstring, the logical Access ID in case of xDSL IP-CAN type |
| 7 | PX_Physical_Access_ID | UTF8String, the physical Access ID in case of xDSL IP-CAN type |
| 8 | PX_OriginHost | charstring, the Origin-Host AVP (AVP Code 264) |
| 9 | PX_OriginRealm | charstring, the Origin-Realm AVP (AVP Code 296) |
| 10 | PX_DestinationHost | charstring,the Destination-Host AVP (AVP Code 293) |
| 11 | PX_DestinationRealm | charstring, the Destination-Realm AVP (AVP Code 283) |
| 12 | PX_UE_framedIpAddress | octetstring the Framed-IP-Address AVP (AVP Code 8) |
| 13 | PX_UE_framedIp6Address | octetstring, the Framed-IPv6-Prefix AVP (AVP Code 97) |
| 14 | PX_SUBSCRIPTION_ID_TYPE | enumerated type, Subscription_Id_Ty_Type, avp_Data within Subscription_Id_Type_AVP |
| 15 | PX_SUBSCRIPTION_ID_DATA | UTF8String, avp_Data within Subscription_Id_AVP |
| 16 | PX_IP_CAN_TYPE | enumerated type, IP_CAN_Type, avp_Data within IP_CAN_Type_AVP |
| 17 | PX_RAT_TYPE | enumerated type, RAT_Type, avp_Data within RAT_Type_AVP |
| 18 | PX_APN_AGGREGATE_MAX_BITRATE_UL_AVP_DATA | UInt32, avp_Data within APN_Aggregate_Max_Bitrate_UL_AVP |
| 19 | PX_3GPP_MS_TIMEZONE_DATA | octetstring, avp_Data within ThreeGPP_MS_TimeZone_DATA |
| 20 | PX_CALLED_STATION_ID_DATA | UTF8String, avp_Data within Called_Station_Id_AVP |
| 21 | PX_PDN_CONNECTION_ID_DATA | octetstring, avp_Data within PDN_Connection_ID_AVP |
| 22 | PX_BEARER_USAGE_DATA | enumerated type, Bearer_Usage, avp_Data within Bearer_Usage_AVP |
| 23 | PX_ANCA_ipv4 | The ANCA (Access-Network-Charging-Address) in IPv4Addr format |
| 24 | PX_ANCA_ipv6 | The ANCA (Access-Network-Charging-Address) in type IPv6Addr format |
| 25 | PX_SGW_AGW_ipv4 | The SGW_AGW address (3GPP-EPS and 3GPP2 accesses) in IPv4Addr format |
| 26 | PX_SGW_AGW_ipv6 | The SGW_AGW address (3GPP-EPS and 3GPP2 accesses) in IPv6Addr format |
| 27 | PX_ACCESS_NETWORK_CHARGING_IDENTIFIER_VALUE | octetstring, avp_Data within Access_Network_Charging_Identifier_Value_AVP |
| 28 | PX_ROUTING_RULE_IDENTIFIER_VALUE | octetstring, avp_Data within Routing_Rule_Identifier_AVP |
| 29 | PX_NETWORK_REQUEST_SUPPORT | enumerated type Network_Request_Support, avp_Data within Network_Request_Support_AVP is of enumerated type Network_Request_Support |
| 30 | PX_CHARGING_RULE_NAME_DATA | octetstring, avp_Data within Charging_Rule_Name_AVP |
| 31 | PX_PCC_RULE_STATUS | enumerated type, PCC_Rule_Status, avp_Data within PCC_Rule_Status_AVP |
| 32 | PX_PRECEDENCE_DATA | UInt32, avp_Data within Precedence_AVP |
| 33 | PX_PACKET_FILTER_IDENTIFIER_DATA | octetstring, avp_Data within Packet_Filter_Identifier_AVP |
| 34 | PX_RULE_FAILURE_CODE | enumerated type, PCC_Rule_Status, avp_Data within Rule_Failure_Code_AVP |
| 35 | PX_OFFLINE_DATA | enumerated type, PCC_Rule_Status, avp_Data within Offline_AVP |
| 36 | PX_MPS_IDENTIFIER | octectstring, avp_Data within MPS_Identifier_AVP (parameter is part of AAR message) |
| 37 | PX_ONLINE_DATA | enumerated type, PCC_Rule_Status, avp_Data within Online_AVP |

Table B.8: PIXIT for the Rx interface

|  | Rx PIXIT Summary |  |
| --- | --- | --- |
| 1 | PX_DIAMETER_RX_ETS_IPADDR | charstring, IP address of the test system |
| 2 | PX_DIAMETER_RX_SUT_IPADDR | charstring, IP address of the system under test |
| 3 | PX_DIAMETER_RX_ETS_PORT | charstring, Port number of the test system |
| 4 | PX_DIAMETER_RX_SUT_PORT | charstring, Port number of the system under test |
| 5 | PX_ANCA_ipv4 | charstring, The ANCA (Access-Network-Charging-Address) in IPv4Addr format |
| 6 | PX_ANCA_ipv6 | charstring, The ANCA (Access-Network-Charging-Address) in type IPv6Addr format |

Annex C (normative):
DIAMETER Gx Partial PIXIT proforma for IUT PCEF

Notwithstanding the provisions of the copyright clause related to the text of the present document, ETSI grants that users of the present document may freely reproduce the Partial PIXIT proforma in this annex so that it can be used for its intended purposes and may further publish the completed Partial PIXIT.

The PIXIT Proforma is based on ISO/IEC 9646-6 [4]. Any needed additional information can be found in this international standard document.

# C.1	Identification summary

Table C.1

| PIXIT Number: |  |
| --- | --- |
| Test Laboratory Name: |  |
| Date of Issue: |  |
| Issued to: |  |

# C.2	ATS summary

Table C.2

| Protocol Specification: | TS 129 212 (3GPP TS 29.212 version 10.5.0 Release 10) [1] |
| --- | --- |
| Protocol to be tested: |  |
| ATS Specification: | TS 101 606-2 |
| Abstract Test Method: | TS 101 606-3 clause 4 |

# C.3	Test laboratory

Table C.3

| Test Laboratory Identification: |  |
| --- | --- |
| Test Laboratory Manager: |  |
| Means of Testing: |  |
| SAP Address: |  |

# C.4	Client identification

Table C.4

| Client Identification: |  |
| --- | --- |
| Client Test manager: |  |
| Test Facilities required: |  |

# C.5	SUT

Table C.5

| Name: |  |
| --- | --- |
| Version: |  |
| SCS Number: |  |
| Machine configuration: |  |
| Operating System Identification: |  |
| IUT Identification: |  |
| PICS Reference for IUT: |  |
| Limitations of the SUT: |  |
| Environmental Conditions: |  |

# C.6	Protocol layer information

## C.6.1	Protocol identification

Table C.6

| Name: | TS 129 212 (3GPP TS 29.212 version 10.5.0 Release 10) |
| --- | --- |
| Version: |  |
| PICS References: |  |

## C.6.2	Configuration

Tables in this clause need to be filled by the IUT Manufacturer to specify how the IUT needs to be configured with IUT specific values or describe IUT specific procedures required for complete testing of the IUT.

Table C.7: PIXIT for the Gx interface

|  | Gx PIXIT Summary |  |
| --- | --- | --- |
| 1 | PX_DIAMETER_GX_ETS_IPADDR | charstring, IP address of the test system |
| 2 | PX_DIAMETER_GX_SUT_IPADDR | charstring, IP address of the system under test |
| 3 | PX_DIAMETER_GX_ETS_PORT | charstring, Port number of the test system |
| 4 | PX_DIAMETER_GX_SUT_PORT | charstring, Port number of the system under test |
| 5 | PX_SessionID | UTF8String, the Session-Id AVP (AVP Code 263) |
| 6 | PX_OriginHost | charstring, the Origin-Host AVP (AVP Code 264) |
| 7 | PX_OriginRealm | charstring, the Origin-Realm AVP (AVP Code 296) |
| 8 | PX_DestinationHost | charstring, the Destination-Host AVP (AVP Code 293) |
| 9 | PX_DestinationRealm | charstring, the Destination-Realm AVP (AVP Code 283) |
| 10 | PX_UE_framedIpAddress | octetstring the Framed-IP-Address AVP (AVP Code 8) |
| 11 | PX_UE_framedIp6Address | octetstring, the Framed-IPv6-Prefix AVP (AVP Code 97) |
| 12 | PX_BEARER_IDENTIFIER | octetstring, avp_Data within Bearer_Identifier_AVP |
| 13 | PX_UNKNOWN_RATING_GROUP | UInt32, avp_Data within Rating_Group_AVP |
| 14 | PX_CHARGING_RULE_NAME_DATA | octetstring, avp_Data within Charging_Rule_Name_AVP |
| 15 | PX_CHARGING_RULE_NAME_DATA2 | octetstring, avp_Data (2nd rule) within Charging_Rule_Name_AVP |
| 16 | PX_CHARGING_RULE_NAME_DATA3 | octetstring, avp_Data (3rd rule) within Charging_Rule_Name_AVP |
| 17 | PX_CHARGING_RULE_NAME_DATA_NONEXIST | octetstring, avp_Data within Charging_Rule_Name_AVP |
| 18 | PX_CHARGING_RULE_BASE_NAME_DATA | octetstring, avp_Data within PX_CHARGING_RULE_BASE_NAME_DATA |
| 19 | PX_PACKET_FILTER_IDENTIFIER_DATA | octetstring, avp_Data within Packet_Filter_Identifier_AVP |
| 20 | PX_MONITORING_KEY | octectstring, avp_Data within MONITORING_KEY_AVP (parameter is part of RAR/CCR message) |
| 21 | PX_MONITORING_KEY2 | octectstring, avp_Data within MONITORING_KEY_AVP (parameter is part of RAR/CCR message) |
| 22 | PX_REVALIDATION_TIME | octectstring, avp_Data within Revalidation_Time_AVP (parameter is part of RAR message) |
| 23 | PX_ACTIVATION_TIME | octectstring, avp_Data within Rule_Activation_Time_AVP (parameter is part of Charging_Rule_Install_AVP) |
| 24 | PX_DEACTIVATION_TIME | octectstring, avp_Data within Rule_DeActivation_Time_AVP (parameter is part of Charging_Rule_Install_AVP) |
| 25 | PX_EMERGENCY_APN | UTF8String, avp_Data within Called_Station_Id_AVP (parameter is part of Called_Station_Id_AVP) |
| 26 | PX_GTPsupport | Boolean indicator, TRUE if GTP interface is accessible to triggrt Diameter events |

Annex D (normative):
DIAMETER Gx PCTR Proforma for IUT PCRF

Notwithstanding the provisions of the copyright clause related to the text of the present document, ETSI grants that users of the present document may freely reproduce the PCTR proforma in this annex so that it can be used for its intended purposes and may further publish the completed PCTR.

The PCTR proforma is based on ISO/IEC 9646-6 [4]. Any needed additional information can be found in this International standard document.

# D.1	Identification summary

## D.1.1	Protocol conformance test report

Table D.1

| PCTR Number: |  |
| --- | --- |
| PCTR Date: |  |
| Corresponding SCTR Number: |  |
| Corresponding SCTR Date: |  |
| Test Laboratory Identification: |  |
| Test Laboratory Manager: |  |
| Signature: |  |

## D.1.2	IUT identification

Table D.2

| Name: |  |
| --- | --- |
| Version: |  |
| Protocol specification: |  |
| PICS: |  |
| Previous PCTR if any: |  |

## D.1.3	Testing environment

Table D.3

| PIXIT Number: |  |
| --- | --- |
| ATS Specification: | TS 101 606-3 |
| Abstract Test Method: | TS 101 606-3 clause 4 |
| Means of Testing identification: |  |
| Date of testing: |  |
| Conformance Log reference(s): |  |
| Retention Date for Log reference(s): |  |

## D.1.4	Limits and reservation

Additional information relevant to the technical contents or further use of the test report, or the rights and obligations of the test laboratory and the client, may be given here. Such information may include restriction on the publication of the report.

## D.1.5	Comments

Additional comments may be given by either the client or the test laboratory on any of the contents of the PCTR, for example, to note disagreement between the two parties.

# D.2	IUT Conformance status

This IUT has or has not been shown by conformance assessment to be non-conforming to the specified protocol specification.

Strike the appropriate words in this sentence. If the PICS for this IUT is consistent with the static conformance requirements (as specified in clause C.3 in the present document) and there are no "FAIL" verdicts to be recorded (in clause C.6 in the present document) strike the words "has or", otherwise strike the words "or has not".

# D.3	Static conformance summary

The PICS for this IUT is or is not consistent with the static conformance requirements in the specified protocol.

Strike the appropriate words in this sentence.

# D.4	Dynamic conformance summary

The test campaign did or did not reveal errors in the IUT.

Strike the appropriate words in this sentence. If there are no "FAIL" verdicts to be recorded (in clause C.6 of the present document) strike the words "did or" otherwise strike the words "or did not".

Summary of the results of groups of test:

# D.5	Static conformance review report

If clause C.3 indicates non-conformance, this clause itemises the mismatches between the PICS and the static conformance requirements of the specified protocol specification.

# D.6	Test campaign report

Table D.4: PCRF test cases

| ATS Reference | Selected? | Run? | Verdict | Observations (Reference to any observations made in clause D.7) |
| --- | --- | --- | --- | --- |
| TC_PCRF_IPS_01 | Yes/No | Yes/No |  |  |
| TC_PCRF_IPS_02 | Yes/No | Yes/No |  |  |
| TC_PCRF_IPS_03 | Yes/No | Yes/No |  |  |
| TC_PCRF_IPS_04 | Yes/No | Yes/No |  |  |
| TC_PCRF_IPS_05 | Yes/No | Yes/No |  |  |
| TC_PCRF_IPS_06 | Yes/No | Yes/No |  |  |
| TC_PCRF_MSI_01 | Yes/No | Yes/No |  |  |
| TC_PCRF_MSI_02 | Yes/No | Yes/No |  |  |
| TC_PCRF_MSI_03 | Yes/No | Yes/No |  |  |
| TC_PCRF_MSI_04 | Yes/No | Yes/No |  |  |
| TC_PCRF_MSI_05 | Yes/No | Yes/No |  |  |
| TC_PCRF_MSI_06 | Yes/No | Yes/No |  |  |
| TC_PCRF_INV_01 | Yes/No | Yes/No |  |  |
| TC_PCRF_INV_02 | Yes/No | Yes/No |  |  |
| TC_PCRF_ST_01 | Yes/No | Yes/No |  |  |
| TC_PCRF_ST_02 | Yes/No | Yes/No |  |  |
| TC_PCRF_ST_03 | Yes/No | Yes/No |  |  |
| TC_PCRF_PCC_01 | Yes/No | Yes/No |  |  |
| TC_PCRF_PCC_02 | Yes/No | Yes/No |  |  |
| TC_PCRF_PCC_03 | Yes/No | Yes/No |  |  |
| TC_PCRF_PCC_04 | Yes/No | Yes/No |  |  |
| TC_PCRF_PCC_05 | Yes/No | Yes/No |  |  |
| TC_PCRF_PCC_06 | Yes/No | Yes/No |  |  |
| TC_PCRF_PCC_07 | Yes/No | Yes/No |  |  |
| TC_PCRF_PCC_08 | Yes/No | Yes/No |  |  |
| TC_PCRF_PCC_09 | Yes/No | Yes/No |  |  |
| TC_PCRF_EMS_01 | Yes/No | Yes/No |  |  |
| TC_PCRF_UMC_01 | Yes/No | Yes/No |  |  |
| TC_PCRF_IRS_01 | Yes/No | Yes/No |  |  |
| TC_PCRF_IRS_02 | Yes/No | Yes/No |  |  |
| TC_PCRF_MPS_01 | Yes/No | Yes/No |  |  |
| TC_PCRF_MPS_02 | Yes/No | Yes/No |  |  |
| TC_PCRF_SDC_01 | Yes/No | Yes/No |  |  |
| TC_PCRF_SDC_02 | Yes/No | Yes/No |  |  |

# D.7	Observations

Additional information relevant to the technical content of the PCTR is given here.

Annex E (normative):
DIAMETER Gx PCTR Proforma for IUT PCEF

Notwithstanding the provisions of the copyright clause related to the text of the present document, ETSI grants that users of the present document may freely reproduce the PCTR proforma in this annex so that it can be used for its intended purposes and may further publish the completed PCTR.

The PCTR proforma is based on ISO/IEC 9646-6 [4]. Any needed additional information can be found in this International standard document.

# E.1	Identification summary

## E.1.1	Protocol conformance test report

Table E.1

| PCTR Number: |  |
| --- | --- |
| PCTR Date: |  |
| Corresponding SCTR Number: |  |
| Corresponding SCTR Date: |  |
| Test Laboratory Identification: |  |
| Test Laboratory Manager: |  |
| Signature: |  |

## E.1.2	IUT identification

Table E.2

| Name: |  |
| --- | --- |
| Version: |  |
| Protocol specification: |  |
| PICS: |  |
| Previous PCTR if any: |  |

## E.1.3	Testing environment

Table E.3

| PIXIT Number: |  |
| --- | --- |
| ATS Specification: | TS 101 606-3 |
| Abstract Test Method: | TS 101 606-3 clause 4 |
| Means of Testing identification: |  |
| Date of testing: |  |
| Conformance Log reference(s): |  |
| Retention Date for Log reference(s): |  |

## E.1.4	Limits and reservation

Additional information relevant to the technical contents or further use of the test report, or the rights and obligations of the test laboratory and the client, may be given here. Such information may include restriction on the publication of the report.

## E.1.5	Comments

Additional comments may be given by either the client or the test laboratory on any of the contents of the PCTR, for example, to note disagreement between the two parties.

# E.2	IUT Conformance status

This IUT has or has not been shown by conformance assessment to be non-conforming to the specified protocol specification.

Strike the appropriate words in this sentence. If the PICS for this IUT is consistent with the static conformance requirements (as specified in clause C.3 in the present document) and there are no "FAIL" verdicts to be recorded (in clause C.6 in the present document) strike the words "has or", otherwise strike the words "or has not".

# E.3	Static conformance summary

The PICS for this IUT is or is not consistent with the static conformance requirements in the specified protocol.

Strike the appropriate words in this sentence.

# E.4	Dynamic conformance summary

The test campaign did or did not reveal errors in the IUT.

Strike the appropriate words in this sentence. If there are no "FAIL" verdicts to be recorded (in clause C.6 of the present document) strike the words "did or" otherwise strike the words "or did not".

Summary of the results of groups of test:

# E.5	Static conformance review report

If clause C.3 indicates non-conformance, this clause itemises the mismatches between the PICS and the static conformance requirements of the specified protocol specification.

# E.6	Test campaign report

Table E.4: PCEF test cases

| ATS Reference | Selected? | Run? | Verdict | Observations (Reference to any observations made in clause D.7) |
| --- | --- | --- | --- | --- |
| TC_PCEF_IPS_01 | Yes/No | Yes/No |  |  |
| TC_PCEF_IPS_02 | Yes/No | Yes/No |  |  |
| TC_PCEF_IPS_03 | Yes/No | Yes/No |  |  |
| TC_PCEF_IPS_04 | Yes/No | Yes/No |  |  |
| TC_PCEF_IPS_05 | Yes/No | Yes/No |  |  |
| TC_PCEF_IPS_06 | Yes/No | Yes/No |  |  |
| TC_PCEF_MSI_01 | Yes/No | Yes/No |  |  |
| TC_PCEF_MSI_02 | Yes/No | Yes/No |  |  |
| TC_PCEF_MSI_03 | Yes/No | Yes/No |  |  |
| TC_PCEF_MSI_04 | Yes/No | Yes/No |  |  |
| TC_PCEF_MSI_05 | Yes/No | Yes/No |  |  |
| TC_PCEF_ST_01 | Yes/No | Yes/No |  |  |
| TC_PCEF_ST_02 | Yes/No | Yes/No |  |  |
| TC_PCEF_PCC_01 | Yes/No | Yes/No |  |  |
| TC_PCEF_PCC_02 | Yes/No | Yes/No |  |  |
| TC_PCEF_PCC_03 | Yes/No | Yes/No |  |  |
| TC_PCEF_PCC_04 | Yes/No | Yes/No |  |  |
| TC_PCEF_PCC_05 | Yes/No | Yes/No |  |  |
| TC_PCEF_PCC_06 | Yes/No | Yes/No |  |  |
| TC_PCEF_PCC_07 | Yes/No | Yes/No |  |  |
| TC_PCEF_PCC_08 | Yes/No | Yes/No |  |  |
| TC_PCEF_PCC_09 | Yes/No | Yes/No |  |  |
| TC_PCEF_PCC_10 | Yes/No | Yes/No |  |  |
| TC_PCEF_PCC_11 | Yes/No | Yes/No |  |  |
| TC_PCEF_PCC_12 | Yes/No | Yes/No |  |  |
| TC_PCEF_PCC_13 | Yes/No | Yes/No |  |  |
| TC_PCEF_PCC_14 | Yes/No | Yes/No |  |  |
| TC_PCEF_PCC_15 | Yes/No | Yes/No |  |  |
| TC_PCEF_PCC_16 | Yes/No | Yes/No |  |  |
| TC_PCEF_PCC_17 | Yes/No | Yes/No |  |  |
| TC_PCEF_PCC_18 | Yes/No | Yes/No |  |  |
| TC_PCEF_PCC_19 | Yes/No | Yes/No |  |  |
| TC_PCEF_PCC_20 | Yes/No | Yes/No |  |  |
| TC_PCEF_EMS_01 | Yes/No | Yes/No |  |  |
| TC_PCEF_EMS_02 | Yes/No | Yes/No |  |  |
| TC_PCEF_UMC_01 | Yes/No | Yes/No |  |  |
| TC_PCEF_RAU_01 | Yes/No | Yes/No |  |  |
| TC_PCEF_RAU_02 | Yes/No | Yes/No |  |  |
| TC_PCEF_RAU_03 | Yes/No | Yes/No |  |  |
| TC_PCEF_RAU_04 | Yes/No | Yes/No |  |  |
| TC_PCEF_RAU_05 | Yes/No | Yes/No |  |  |
| TC_PCEF_RAU_06 | Yes/No | Yes/No |  |  |
| TC_PCEF_RAU_07 | Yes/No | Yes/No |  |  |
| TC_PCEF_IRS_01 | Yes/No | Yes/No |  |  |
| TC_PCEF_IRS_02 | Yes/No | Yes/No |  |  |

# E.7	Observations

Additional information relevant to the technical content of the PCTR is given here.

History

| Document history |  |  |
| --- | --- | --- |
| V0.0.1 | February 2012 | STF443: Early draft - skeleton |
| V0.0.2 | May 2012 | STF443: Early draft – test architecture added |
| V0.0.3 | June 2012 | STF443: additional test configuration illustration |
| V0.0.4 | July 2012 | STF 443 additional clauses |
| V0.0.5 | August 2012 | STF443:Listing of ATS parameters, additional tests, corrections |
| V0.0.6 | August 2012 | STF443: Final corrections – Final draft for approval |