ETSI TS 124 379 V13.2.0 (2016-10)

LTE;

Mission Critical Push To Talk (MCPTT) call control;

Protocol specification

(3GPP TS 24.379 version 13.2.0 Release 13)

Technical Specification

Reference

RTS/TSGC-0124379vd20

Keywords

LTE

ETSI

650 Route des Lucioles

F-06921 Sophia Antipolis Cedex - FRANCE

Tel.: +33 4 92 94 42 00   Fax: +33 4 93 65 47 16

Siret N° 348 623 562 00017 - NAF 742 C

Association à but non lucratif enregistrée à la

Sous-Préfecture de Grasse (06) N° 7803/88

Important notice

The present document can be downloaded from:
http://www.etsi.org/standards-search

The present document may be made available in electronic versions and/or in print. The content of any electronic and/or print versions of the present document shall not be modified without the prior written authorization of ETSI. In case of any existing or perceived difference in contents between such versions and/or in print, the only prevailing document is the print of the Portable Document Format (PDF) version kept on a specific network drive within ETSI Secretariat.

Users of the present document should be aware that the document may be subject to revision or change of status. Information on the current status of this and other ETSI documents is available at https://portal.etsi.org/TB/ETSIDeliverableStatus.aspx

If you find errors in the present document, please send your comment to one of the following services:
https://portal.etsi.org/People/CommiteeSupportStaff.aspx

Copyright Notification

No part may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm except as authorized by written permission of ETSI.
The content of the PDF version shall not be modified without the written authorization of ETSI.
The copyright and the foregoing restriction extend to reproduction in all media.

© European Telecommunications Standards Institute 2016.

All rights reserved.

DECTTM, PLUGTESTSTM, UMTSTM and the ETSI logo are Trade Marks of ETSI registered for the benefit of its Members.
3GPPTM and LTE™ are Trade Marks of ETSI registered for the benefit of its Members and
of the 3GPP Organizational Partners.
GSM® and the GSM logo are Trade Marks registered and owned by the GSM Association.

# Contents

Contents	2

Foreword	19

Modal verbs terminology	19

1	Scope	20

2	References	20

3	Definitions, symbols and abbreviations	23

3.1	Definitions	23

3.2	Abbreviations	24

4	General	25

4.1	MCPTT overview	25

4.2	URI and address assignments	26

4.3	MCPTT speech	26

4.4	Warning Header Field	26

4.4.1	General	26

4.4.2	Warning texts	27

4.5	MCPTT session identity	29

4.6	MCPTT priority calls and alerts	30

4.6.1	MCPTT emergency group calls	30

4.6.2	MCPTT emergency private calls	31

4.6.3	MCPTT emergency alerts	32

4.6.4	MCPTT imminent peril group call	33

4.7	End-to-end communication security	34

4.8	Protection of sensitive application data.	35

4.9	Pre-established session	38

4.10	MCPTT client ID	39

4.11	Off-network MCPTT	39

5	Functional entities	39

5.1	Introduction	39

5.2	MCPTT client	39

5.3	MCPTT server	40

5.3.1	General	40

5.3.2	Functional connectivity models	41

6	Common procedures	44

6.1	Introduction	44

6.2	MCPTT client procedures	44

6.2.1	SDP offer generation	44

6.2.2	SDP answer generation	44

6.2.3	Commencement modes	45

6.2.3.1	Automatic commencement mode	45

6.2.3.1.1	Automatic commencement mode for private calls	45

6.2.3.1.2	Automatic commencement mode for group calls	46

6.2.3.2	Manual commencement mode	46

6.2.3.2.1	Manual commencement mode for private calls	46

6.2.3.2.2	Manual commencement mode for group calls	46

6.2.4	Leaving an MCPTT session initiated by MCPTT client	47

6.2.4.1	On-demand session case	47

6.2.4.2	Pre-established session case	47

6.2.5	Releasing an MCPTT session initiated by MCPTT client	48

6.2.5.1	On-demand session case	48

6.2.5.2	Pre-established session case	48

6.2.6	Receiving an MCPTT session release request	48

6.2.7	Receiving a SIP MESSAGE request	48

6.2.8	Priority call conditions	49

6.2.8.1	MCPTT emergency group call conditions	49

6.2.8.1.1	SIP INVITE request or SIP REFER request for originating MCPTT emergency group calls	49

6.2.8.1.2	Resource-Priority header field for MCPTT emergency group calls	50

6.2.8.1.3	SIP re-INVITE request for cancelling MCPTT in-progress emergency group state	50

6.2.8.1.4	Receiving a SIP 2xx response to a SIP request for a priority call	51

6.2.8.1.5	Receiving a SIP 4xx response, SIP 5xx response or SIP 6xx response to a SIP request for a priority group call	52

6.2.8.1.6	Determining authorisation for initiating or cancelling an MCPTT emergency alert	52

6.2.8.1.7	Determining authorisation for cancelling the in-progress emergency state of an MCPTT group	53

6.2.8.1.8	Determining authorisation for originating a priority group call	53

6.2.8.1.9	SIP request for originating MCPTT imminent peril group calls	54

6.2.8.1.10	Determining authorisation for cancelling an imminent peril group call	54

6.2.8.1.11	SIP re-INVITE request for cancelling MCPTT in-progress imminent peril group state	54

6.2.8.1.12	Resource-Priority header field for MCPTT imminent peril group calls	55

6.2.8.1.13	Receiving a SIP INFO request in the dialog of a SIP request for a priority group call	55

6.2.8.1.14	SIP re-INVITE request for cancelling the in-progress emergency group state of a group by a third-party	56

6.2.8.1.15	Retrieving Resource-Priority header field values	56

6.2.8.2	Request for an originating broadcast group call	57

6.2.8.3	MCPTT emergency private call conditions	57

6.2.8.3.1	Authorisations	57

6.2.8.3.1.1	Determining authorisation for initiating an MCPTT emergency private call	57

6.2.8.3.1.2	Determining authorisation for cancelling an MCPTT emergency private call	58

6.2.8.3.1.3	Determining authorisation for initiating or cancelling an MCPTT emergency alert to a MCPTT user	58

6.2.8.3.2	SIP request for originating MCPTT emergency private calls	58

6.2.8.3.3	Resource-Priority header field for MCPTT emergency private calls	59

6.2.8.3.4	Receiving a SIP 2xx response to a SIP request for an MCPTT emergency private call	59

6.2.8.3.5	Receiving a SIP 4xx response, SIP 5xx response or SIP 6xx response to a SIP request for an MCPTT emergency private call	60

6.2.8.3.6	SIP re-INVITE request for cancelling MCPTT emergency private call state	60

6.2.8.3.7	Receiving a SIP INFO request in the dialog of a SIP request for a priority private call	61

6.2.8.3.8	SIP re-INVITE request for cancelling the MCPTT emergency private call state by a third-party	62

6.2.9	Location information	62

6.2.9.1	Location information for location reporting	62

6.3	MCPTT server procedures	62

6.3.1	Distinction of requests sent to the MCPTT server	62

6.3.1.1	SIP INVITE request	62

6.3.1.2	SIP REFER request	63

6.3.1.3	SIP MESSAGE request	63

6.3.1.4	SIP SUBSCRIBE request	64

6.3.2	Participating MCPTT Function	64

6.3.2.1	Requests initiated by the served MCPTT user	64

6.3.2.1.1	SDP offer generation	64

6.3.2.1.1.1	On-demand session	64

6.3.2.1.1.2	Pre-established session	65

6.3.2.1.2	SDP answer generation	65

6.3.2.1.2.1	On-demand session	65

6.3.2.1.2.2	Pre-established session establishment	66

6.3.2.1.3	Sending an INVITE request on receipt of an INVITE request	66

6.3.2.1.4	Sending an INVITE request on receipt of a REFER request	67

6.3.2.1.5	Response to an INVITE request	67

6.3.2.1.5.1	Provisional responses	67

6.3.2.1.5.2	Final response	68

6.3.2.1.6	Sending a SIP BYE request on receipt of a SIP BYE request	68

6.3.2.1.7	Sending a SIP BYE request on receipt of a SIP REFER request	69

6.3.2.1.8	Priority call conditions	69

6.3.2.1.8.1		Determining authorisation for originating a priority group call	69

6.3.2.1.9	Generating a SIP re-INVITE request on receipt of a SIP re-INVITE request	70

6.3.2.1.10	Sending a SIP INVITE request on receipt of SIP 3xx response	70

6.3.2.2	Requests terminated to the served MCPTT user	71

6.3.2.2.1	SDP offer generation	71

6.3.2.2.2	SDP answer generation	72

6.3.2.2.2.1	On-demand session	72

6.3.2.2.2.2	Pre-established session	72

6.3.2.2.3	SIP INVITE request towards the terminating MCPTT client	72

6.3.2.2.4	Response to a SIP INVITE request	73

6.3.2.2.4.1	Provisional response	73

6.3.2.2.4.2	Final response	73

6.3.2.2.5	Automatic Commencement Mode	73

6.3.2.2.5.1	General	73

6.3.2.2.5.2	Automatic commencement for On-Demand session	74

6.3.2.2.5.3	Automatic commencement for pre-established session	75

6.3.2.2.6	Manual Commencement Mode	76

6.3.2.2.6.1	General	76

6.3.2.2.6.2	Manual commencement for On-Demand session	76

6.3.2.2.6.3	Manual commencement for Pre-established session	77

6.3.2.2.7	SIP MESSAGE request towards the terminating MCPTT client	79

6.3.2.2.8	SIP BYE request towards the terminating MCPTT client	79

6.3.2.2.8.1	On-demand	79

6.3.2.2.8.2	Using pre-established session	80

6.3.2.2.9	Populate MIME bodies	80

6.3.2.2.10	Generating a SIP re-INVITE request towards the terminating MCPTT client	80

6.3.2.2.11	Generating a SIP MESSAGE request towards the terminating MCPTT client	80

6.3.3	Controlling MCPTT function	81

6.3.3.1	Request initiated by the controlling MCPTT function	81

6.3.3.1.1	SDP offer generation	81

6.3.3.1.2	Sending an INVITE request	81

6.3.3.1.3	Receipt of a SIP response to a SIP INVITE request	82

6.3.3.1.3.1	Final response	82

6.3.3.1.4	Sending a SIP MESSAGE request	82

6.3.3.1.5	Sending a SIP BYE request	82

6.3.3.1.6	Sending a SIP re-INVITE request for MCPTT emergency group call	83

6.3.3.1.7	Sending a SIP INVITE request for MCPTT emergency group call	84

6.3.3.1.8	Sending a SIP UPDATE request for Resource-Priority header field correction	85

6.3.3.1.9	Generating a SIP re-INVITE request	85

6.3.3.1.10	Generating a SIP re-INVITE request to cancel an in-progress emergency	85

6.3.3.1.11	Generating a SIP MESSAGE request for notification of in-progress emergency or imminent peril status change	86

6.3.3.1.12	Populate mcptt-info and location-info MIME bodies for emergency alert	86

6.3.3.1.13	Authorisations	87

6.3.3.1.13.1	Determining authorisation for initiating an MCPTT emergency alert	87

6.3.3.1.13.2	Determining authorisation for initiating an MCPTT emergency group or private call	88

6.3.3.1.13.3	Determining authorisation for cancelling an MCPTT emergency alert	89

6.3.3.1.13.4	Determining authorisation for cancelling an MCPTT emergency call	89

6.3.3.1.13.5	Determining authorisation for initiating an MCPTT imminent peril call	89

6.3.3.1.13.6	Determining authorisation for cancelling an MCPTT imminent peril call	90

6.3.3.1.13.7	Sending a SIP OPTIONS request to authorise an MCPTT user at a non-controlling MCPTT function of a MCPTT group	90

6.3.3.1.14	Generating a SIP 403 response for priority call request rejection	92

6.3.3.1.15	Sending a SIP re-INVITE request for MCPTT imminent peril group call	92

6.3.3.1.16	Handling the expiry of timer TNG2 (in-progress emergency group call timer)	93

6.3.3.1.17	Validate priority request parameters	93

6.3.3.1.18	Sending a SIP INFO request in the dialog of a SIP request for a priority call	94

6.3.3.1.19	Retrieving Resource-Priority header field values	94

6.3.3.2	Requests terminated by the controlling MCPTT function	95

6.3.3.2.1	SDP answer generation	95

6.3.3.2.2	Receipt of a SIP INVITE request	95

6.3.3.2.3	Sending a SIP response to a SIP INVITE request	95

6.3.3.2.3.1	Provisional response	95

6.3.3.2.3.2	Final response	96

6.3.3.2.4	Receiving a SIP BYE request	96

6.3.3.3	Handling of the acknowledged call setup timer (TNG1)	97

6.3.3.4	Generating a SIP NOTIFY request	99

6.3.3.5	Handling of the group call timer (TNG3)	99

6.3.3.5.1	General	99

6.3.3.5.2	Interaction with the in-progress emergency group call timer (TNG2)	100

6.3.4	Non-controlling MCPTT function of an MCPTT group	100

6.3.4.1	Request initiated by the non-controlling MCPTT function of an MCPTT group	100

6.3.4.1.1	SDP offer generation	100

6.3.4.1.2	Sending an INVITE request towards the MCPTT client	101

6.3.4.1.3	Sending a SIP INFO request	102

6.3.4.1.4	Sending an INVITE request towards the controlling MCPTT function	102

6.3.4.2	Requests terminated by the non-controlling MCPTT function of an MCPTT group	103

6.3.4.2.1	SDP answer generation	103

6.3.4.2.2	Sending a SIP response to the SIP INVITE request	103

6.3.4.2.2.1	Sending a SIP 183 (Session Progress) response	103

6.3.4.2.2.2	Sending a SIP 200 (OK) response	104

6.3.4.3	Generating a SIP NOTIFY request	104

6.3.5	Retrieving and processing a group document	105

6.3.5.1	General	105

6.3.5.2	Rules for retrieving Group Document(s)	105

6.3.5.3	Rules for joining a group session	109

6.3.5.4	Rules for initiating a prearranged group session	109

6.3.5.5	Determining the group members to invite	109

6.3.6	Affiliation check	110

6.3.7	Error handling	110

6.3.7.1	Public service identity does not exist	110

6.3.8	Session release policy	110

6.3.8.1	Session release policy for group call	110

6.3.8.2	Session release policy for private call	111

6.4	Implicit floor request	111

6.5	Handling of MIME bodies in a SIP message	111

6.6	Confidentiality and Integrity Protection	112

6.6.1	General	112

6.6.1.1	Applicability and exclusions	112

6.6.1.2	Performing XML content encryption	112

6.6.1.3	Performing integrity protection on an XML body	112

6.6.1.4	Verifying integrity of an XML body and decrypting XML elements	113

6.6.2	Confidentiality Protection.	113

6.6.2.1	General	113

6.6.2.2	Keys used in confidentiality protection procedures	113

6.6.2.3	Procedures for sending confidentiality protected content	114

6.6.2.3.1	MCPTT client	114

6.6.2.3.2	MCPTT server	114

6.6.2.3.3	Content Encryption in XML elements	114

6.6.2.3.4	Attribute URI Encryption	115

6.6.2.4	Procedures for receiving confidentiality protected content	115

6.6.2.4.1	Determination of confidentiality protected content	115

6.6.2.4.2	Decrypting confidentiality protected content in XML elements	115

6.6.2.4.3	Decrypting confidentiality protected URIs in XML attributes	116

6.6.2.5	MCPTT server copying received XML content	116

6.6.3	Integrity Protection of XML documents.	117

6.6.3.1	General	117

6.6.3.2	Keys used in integrity protection procedures	118

6.6.3.3	Sending integrity protected content	119

6.6.3.3.1	MCPTT client	119

6.6.3.3.2	MCPTT server	119

6.6.3.3.3	Integrity protection procedure	119

6.6.3.4	Receiving integrity protected content	120

6.6.3.4.1	Determination of integrity protected content	120

6.6.3.4.2	Verification of integrity protected content	120

6.7	Priority sharing	120

7	Registration and service authorisation	120

7.1	General	120

7.2	MCPTT client procedures	121

7.2.1	SIP REGISTER request for service authorisation	121

7.2.1A	Common SIP PUBLISH procedure	122

7.2.2	SIP PUBLISH request for service authorisation and MCPTT service settings	122

7.2.3	Sending SIP PUBLISH for MCPTT service settings only	123

7.3	MCPTT server procedures	123

7.3.1	General	123

7.3.1A	Confidentiality and Integrity Protection	123

7.3.2	SIP REGISTER request for service authorisation	125

7.3.3	SIP PUBLISH request for service authorisation and service settings	125

7.3.4	Receiving SIP PUBLISH request for MCPTT service settings only	126

7.3.5	Receiving SIP PUBLISH request with "Expires=0"	126

8	Pre-established session	127

8.1	General	127

8.2	Session establishment	127

8.2.1	MCPTT client procedures	127

8.2.2	Participating MCPTT function procedures	128

8.3	Session modification	129

8.3.1	MCPTT client procedures	129

8.3.1.1	MCPTT client initiated	129

8.3.1.2	Participating MCPTT function initiated	129

8.3.2	Participating MCPTT function procedures	129

8.3.2.1	MCPTT client initiated	129

8.3.2.2	Participating MCPTT function initiated	130

8.4	Session release	130

8.4.1	MCPTT client procedures	130

8.4.1.1	MCPTT client initiated	130

8.4.1.2	Participating MCPTT function initiated	130

8.4.2	Participating MCPTT function procedures	131

8.4.2.1	MCPTT client initiated	131

8.4.2.2	Participating MCPTT function initiated	131

9	Affiliation	132

9.1	General	132

9.2	Procedures	132

9.2.1	MCPTT client procedures	132

9.2.1.1	General	132

9.2.1.2	Affiliation status change procedure	132

9.2.1.3	Affiliation status determination procedure	133

9.2.1.4	Procedure for sending affiliation status change request in negotiated mode to target MCPTT user	134

9.2.1.5	Procedure for receiving affiliation status change request in negotiated mode from authorized MCPTT user	134

9.2.2	MCPTT server procedures	135

9.2.2.1	General	135

9.2.2.2	Procedures of MCPTT server serving the MCPTT user	135

9.2.2.2.1	General	135

9.2.2.2.2	Stored information	135

9.2.2.2.3	Receiving affiliation status change from MCPTT client procedure	136

9.2.2.2.4	Receiving subscription to affiliation status procedure	139

9.2.2.2.5	Sending notification of change of affiliation status procedure	140

9.2.2.2.6	Sending affiliation status change towards MCPTT server owning MCPTT group procedure	140

9.2.2.2.7	Affiliation status determination from MCPTT server owning MCPTT group procedure	142

9.2.2.2.8	Procedure for authorizing affiliation status change request in negotiated mode sent to served MCPTT user	144

9.2.2.2.9	Forwarding affiliation status change towards another MCPTT user procedure	144

9.2.2.2.10	Forwarding subscription to affiliation status towards another MCPTT user procedure	145

9.2.2.3	Procedures of MCPTT server owning the MCPTT group	146

9.2.2.3.1	General	146

9.2.2.3.2	Stored information	146

9.2.2.3.3	Receiving group affiliation status change procedure	147

9.2.2.3.4	Receiving subscription to affiliation status procedure	148

9.2.2.3.5	Sending notification of change of affiliation status procedure	149

9.3	Coding	149

9.3.1	Extension of application/pidf+xml MIME type	149

9.3.1.1	Introduction	149

9.3.1.2	Syntax	149

9.3.2	Extension of application/simple-filter+xml MIME type	151

9.3.2.1	Introduction	151

9.3.2.2	Syntax	151

10	Group call	152

10.0	General	152

10.1	On-network group call	153

10.1.1	Prearranged group call	153

10.1.1.1	General	153

10.1.1.2	MCPTT client procedures	153

10.1.1.2.1	On-demand prearranged group call	153

10.1.1.2.1.1	Client originating procedures	153

10.1.1.2.1.2	Client terminating procedures	154

10.1.1.2.1.3	MCPTT upgrade to in-progress emergency or imminent peril	156

10.1.1.2.1.4	MCPTT in-progress emergency cancel	157

10.1.1.2.1.5	MCPTT in-progress imminent peril cancel	158

10.1.1.2.1.6	MCPTT client receives SIP re-INVITE request	160

10.1.1.2.2	Prearranged group call using pre-established session	161

10.1.1.2.2.1	Client originating procedures	161

10.1.1.2.2.2	Client terminating procedures	161

10.1.1.2.3	End group call	162

10.1.1.2.3.1	Client originating procedures on-demand	162

10.1.1.2.3.2	Client originating procedures using pre-established session	162

10.1.1.2.3.3	Client terminating procedures	162

10.1.1.2.4	Re-join procedure	162

10.1.1.2.4.1	On demand session establishment	162

10.1.1.2.4.2	Pre-established session	162

10.1.1.3	Participating MCPTT function procedures	162

10.1.1.3.1	Originating procedures	162

10.1.1.3.1.1	On demand prearranged group call	162

10.1.1.3.1.2	Prearranged group call using pre-established session	165

10.1.1.3.1.3	Reception of a SIP re-INVITE request from served MCPTT client	166

10.1.1.3.2	Terminating Procedures	167

10.1.1.3.3	End group call at the originating participating MCPTT function	168

10.1.1.3.3.1	Receipt of SIP BYE request for ending group call on-demand	168

10.1.1.3.3.2	Receipt of SIP REFER "BYE" request for ending group call using pre-established session	168

10.1.1.3.4	End group call at the terminating participating MCPTT function	168

10.1.1.3.4.1	Receipt of SIP BYE request for private call on-demand	168

10.1.1.3.4.2	Receipt of SIP BYE request when ongoing pre-established session	169

10.1.1.3.5	Re-join procedures	169

10.1.1.3.5.1	Originating procedures - on demand prearranged group call	169

10.1.1.3.5.2	Originating procedures - prearranged group call using pre-established session	169

10.1.1.3.6	Reception of a SIP re-INVITE request for terminating MCPTT client for priority call	169

10.1.1.4	Controlling MCPTT function procedures	170

10.1.1.4.1	Originating Procedures	170

10.1.1.4.1.1	INVITE targeted to an MCPTT client	170

10.1.1.4.1.2	INVITE targeted to the non-controlling MCPTT function of an MCPTT group	171

10.1.1.4.2	Terminating Procedures	172

10.1.1.4.3	End group call at the terminating controlling MCPTT function	180

10.1.1.4.4	End group call initiated by the controlling MCPTT function	180

10.1.1.4.4.1	General	180

10.1.1.4.4.2	SIP BYE request for releasing MCPTT session for a group call	180

10.1.1.4.4.3	SIP BYE request toward a MCPTT client	181

10.1.1.4.5	Re-join procedures	181

10.1.1.4.5.1	Terminating procedures	181

10.1.1.4.6	Late call entry initiated by controlling MCPTT function	182

10.1.1.4.7	Receipt of a SIP re-INVITE request	182

10.1.1.4.8	Handling of a SIP re-INVITE request for imminent peril session	186

10.1.1.5	Non-controlling function of an MCPTT group procedures	188

10.1.1.5.1	Originating procedures	188

10.1.1.5.2	Terminating procedures	189

10.1.1.5.2.1	General	189

10.1.1.5.2.2	Initiating a prearranged group call	189

10.1.1.5.2.3	Joining an ongoing prearranged group call	191

10.1.1.5.2.4	Splitting an ongoing prearranged group call	192

10.1.1.5.3	Rejoin procedures	192

10.1.1.5.3.1	Terminating procedures	192

10.1.1.5.3.2	Late call entry initiated by non-controlling MCPTT function	193

10.1.1.5.4	SIP OPTIONS request authorization procedure	193

10.1.1.5.5	Initiating a temporary group session	194

10.1.2	Chat group (restricted) call	195

10.1.2.1	General	195

10.1.2.2	MCPTT client procedures	195

10.1.2.2.1	On-demand chat group call	195

10.1.2.2.1.1	MCPTT client joins a chat MCPTT group session	195

10.1.2.2.1.2	MCPTT client receives SIP re-INVITE request	197

10.1.2.2.1.3	MCPTT in-progress emergency cancel	198

10.1.2.2.1.4	MCPTT upgrade to in-progress emergency or imminent peril	199

10.1.2.2.1.5	MCPTT in-progress imminent peril cancel	201

10.1.2.2.1.6	MCPTT client receives a SIP INVITE request for an MCPTT group call	202

10.1.2.2.2	Chat group call within a pre-established session	203

10.1.2.2.2.1	MCPTT client joins a chat MCPTT group session	203

10.1.2.3	Participating MCPTT function procedures	205

10.1.2.3.1	On-demand chat group call	205

10.1.2.3.1.1	MCPTT chat session establishment	205

10.1.2.3.1.2	Reception of a SIP re-INVITE request from served MCPTT client	207

10.1.2.3.1.3	Reception of a SIP INVITE request for terminating MCPTT client	208

10.1.2.3.1.4	Reception of a SIP re-INVITE request for terminating MCPTT client	209

10.1.2.3.2	Chat group call within a pre-established session	209

10.1.2.3.2.1	MCPTT chat session establishment	209

10.1.2.3.2.2	MCPTT chat session establishment for terminating user within a pre-established session	212

10.1.2.4	Controlling MCPTT function procedures	212

10.1.2.4.1	On-demand chat group call	212

10.1.2.4.1.1	MCPTT chat session establishment	212

10.1.2.4.1.2	Receipt of a SIP re-INVITE request	217

10.1.2.4.1.3	Handling of a SIP re-INVITE request for imminent peril session	221

10.1.2.5	Non-controlling function of an MCPTT group procedures	224

10.1.2.5.1	Terminating procedures	224

10.1.2.5.1.1	General	224

10.1.2.5.1.2	Initiating a chat group session	224

10.1.2.5.1.3	Joining an ongoing chat group call	225

10.1.2.5.1.4	Splitting an ongoing chat group call	226

10.1.2.5.1.5	MCPTT client joining the temporary group chat session	226

10.1.2.5.1.6	Receipt of a SIP re-INVITE request from an MCPTT client	226

10.1.2.5.1.7	SIP OPTIONS request authorization procedure	226

10.1.2.5.1.8	Initiating a temporary group session	226

10.1.3	Subscription to the conference event package	228

10.1.3.1	General	228

10.1.3.2	MCPTT client	228

10.1.3.3	Participating MCPTT function	229

10.1.3.4	Controlling MCPTT function	230

10.1.3.4.1	Receiving a subscription to the conference event package	230

10.1.3.4.2	Sending notifications to the conference event package	231

10.1.3.4.3	Sending subscriptions to the conference event package	231

10.1.3.4.4	Terminating a subscription	232

10.1.3.5	Non-controlling MCPTT function	232

10.1.3.5.1	Receiving subscriptions to the conference event package	232

10.1.3.5.2	Sending notifications to the conference event package	233

10.1.3.5.3	Sending a subscription to the conference event package	233

10.1.3.5.4	Terminating a subscription	234

10.2	Off-network group call	235

10.2.1	General	235

10.2.1.1	Common Procedures	235

10.2.1.1.1	MONP message transport	235

10.2.1.1.2	Session description	235

10.2.2	Basic call control	236

10.2.2.1	General	236

10.2.2.2	Basic call control state machine	236

10.2.2.3	Call Control states	237

10.2.2.3.1	S1: start-stop	237

10.2.2.3.2	S2: waiting for call announcement	237

10.2.2.3.3	S3: part of ongoing call	237

10.2.2.3.4	S4: pending user action without confirm indication	237

10.2.2.3.5	S5: pending user action with confirm indication	237

10.2.2.3.6	S6: ignoring incoming call announcements	237

10.2.2.3.7	S7: waiting for call announcement after call release	237

10.2.2.4	Procedures	238

10.2.2.4.1	General	238

10.2.2.4.1.1	Call announcement timer calculation	238

10.2.2.4.1.1.1	Periodic call announcement timer calculation	238

10.2.2.4.1.1.2	Call announcement timer calculation after CALL PROBE	238

10.2.2.4.1.2	Max duration timer calculation	238

10.2.2.4.2	Call Probe	238

10.2.2.4.2.1	Originating UE call probe initiation	238

10.2.2.4.2.2	Originating UE call probe retransmission	239

10.2.2.4.2.3	Terminating UE receiving GROUP CALL PROBE message when participating in the ongoing call	239

10.2.2.4.3	Call setup	239

10.2.2.4.3.1	UE not receiving any response to GROUP CALL PROBE message	239

10.2.2.4.3.2	Originating UE receiving a GROUP CALL ANNOUNCEMENT message	240

10.2.2.4.3.3	Terminating UE receiving a GROUP CALL ANNOUNCEMENT message when not participating in the ongoing call	240

10.2.2.4.3.4	MCPTT user accepts the terminating call with confirm indication	241

10.2.2.4.3.5	MCPTT user accepts the terminating call without confirm indication	242

10.2.2.4.3.6	Originating UE receiving GROUP CALL ACCEPT message	242

10.2.2.4.3.7	MCPTT user rejects the terminating call	242

10.2.2.4.3.8	MCPTT user does not act on terminating call	242

10.2.2.4.4	Periodic group call announcement	243

10.2.2.4.4.1	Originating UE sending periodic call announcement	243

10.2.2.4.4.2	Terminating UE receiving periodic call announcement	243

10.2.2.4.5	Call release	244

10.2.2.4.5.1	MCPTT user leaves the call when GROUP CALL ANNOUNCEMENT was sent or received	244

10.2.2.4.5.2	Receiving GROUP CALL ANNOUNCEMENT message for rejected or released call	244

10.2.2.4.5.3	MCPTT user initiates originating call for rejected or released call	244

10.2.2.4.5.4	No GROUP CALL ANNOUNCEMENT messages for rejected or released call	245

10.2.2.4.5.5	MCPTT user leaves the call when GROUP CALL PROBE was sent	245

10.2.2.4.5.6	MCPTT user initiates originating call for released call	245

10.2.2.4.5.7	Receiving GROUP CALL ANNOUNCEMENT message for released call	245

10.2.2.4.5.8	No GROUP CALL ANNOUNCEMENT messages for released call	246

10.2.2.4.5.9	Max duration reached	246

10.2.2.4.6	Merge of calls	246

10.2.2.4.6.1	Merge of two calls	246

10.2.2.4.7	Error handling	247

10.2.2.4.7.1	Unexpected MONP message received	247

10.2.2.4.7.2	Unexpected indication from MCPTT user	247

10.2.2.4.7.3	Unexpected expiration of a timer	247

10.2.3.	Call type control	247

10.2.3.1	General	247

10.2.3.2	Call type control state machine	247

10.2.3.3	Call Control states	249

10.2.3.3.1	T0: waiting for call to establish	249

10.2.3.3.2	T1: in-progress emergency group call	249

10.2.3.3.3	T2: in-progress basic group call	249

10.2.3.3.4	T3: in-progress imminent peril group call	249

10.2.3.4	Procedures	249

10.2.3.4.1	General	249

10.2.3.4.2	User initiated the call probe	249

10.2.3.4.3	Received GROUP CALL ANNOUNCEMENT message as a response to GROUP CALL PROBE message	250

10.2.3.4.4	Received GROUP CALL ANNOUNCEMENT with MCPTT user acknowledgement required	250

10.2.3.4.5	Received GROUP CALL ANNOUNCEMENT without MCPTT user acknowledgement required	251

10.2.3.4.6	Call started	251

10.2.3.4.7	Upgrade call	252

10.2.3.4.7.1	Originating user upgrading the call	252

10.2.3.4.7.2	Terminating UE receiving a GROUP CALL ANNOUNCEMENT message when participating in the ongoing call	253

10.2.3.4.8	Downgrade call	254

10.2.3.4.8.1	Originating user downgrading emergency group call	254

10.2.3.4.8.2	Retransmitting GROUP CALL EMERGENCY END	255

10.2.3.4.8.3	Terminating user downgrading emergency group call	255

10.2.3.4.8.4	Originating user downgrading imminent peril group call	256

10.2.3.4.8.5	Retransmitting GROUP CALL IMMINENT PERIL END	256

10.2.3.4.8.6	Terminating user downgrading imminent peril group call	257

10.2.3.4.8.7	Reset TFG13 (implicit downgrade) timer	257

10.2.3.4.8.8	Implicit priority end	257

10.2.3.4.9	Merge of two calls	258

10.2.3.4.10	Call release after call establishment	258

10.2.3.4.11	Call release or reject before call establishment	259

10.2.3.4.12	Error handling	259

10.2.3.4.12.1	Unexpected MONP message received	259

10.2.3.4.12.2	Unexpected indication from MCPTT user	259

10.2.3.4.12.3	Unexpected expiration of a timer	259

10.3	Off-network Broadcast group call	259

10.3.1	General	259

10.3.2	Basic call control	259

10.3.2.1	General	259

10.3.2.2	Call control state machine	259

10.3.2.3	Broadcast group call Control states	260

10.3.2.3.1	B1: start-stop	260

10.3.2.3.2	B2: in-progress broadcast group call	260

10.3.2.3.3	B3: pending user action	260

10.3.2.3.4	B4: ignoring same call ID	260

10.3.2.4	Procedures	261

10.3.2.4.1	User initiating a broadcast group call	261

10.3.2.4.2	Terminating UE receiving a GROUP CALL BROADCAST message when not participating in the in-progress broadcast group call	261

10.3.2.4.3	MCPTT user accepts the terminating call	262

10.3.2.4.4	MCPTT user rejects the terminating call	262

10.3.2.4.5	MCPTT user does not act on terminating call	262

10.3.2.4.6	Terminating user releasing the call	262

10.3.2.4.7	Originating user releasing the call	262

10.3.2.4.8	Receiving GROUP CALL BROADCAST END message	263

10.3.2.4.9	Originating UE retransmitting GROUP CALL BROADCAST message	263

10.3.2.4.10	Ignoring same call ID	263

10.3.2.4.11	Releasing the call	263

11	Private call	264

11.0	General	264

11.1	On-network private call	264

11.1.1	Private call with floor control	264

11.1.1.1	General	264

11.1.1.2	MCPTT client procedures	264

11.1.1.2.1	On-demand private call	264

11.1.1.2.1.1	Client originating procedures	264

11.1.1.2.1.2	Client terminating procedures	266

11.1.1.2.1.3	Client terminating procedures for reception of SIP re-INVITE request	267

11.1.1.2.1.4	MCPTT in-progress emergency cancel	269

11.1.1.2.1.5	Upgrade to MCPTT emergency private call	270

11.1.1.2.2	Private call using pre-established session	271

11.1.1.2.2.1	Client originating procedures	271

11.1.1.2.2.2	Client terminating procedures	272

11.1.1.3	Participating MCPTT function procedures	272

11.1.1.3.1	Originating procedures	272

11.1.1.3.1.1	On-demand private call	272

11.1.1.3.1.2	Private call initiation using pre-established session	275

11.1.1.3.1.3	Receipt of SIP re-INVITE for MCPTT private call from the served user	278

11.1.1.3.2	Terminating procedures	279

11.1.1.3.3	Receipt of SIP re-INVITE request by terminating participating function	280

11.1.1.4	Controlling MCPTT function procedures	280

11.1.1.4.1	Originating procedures	280

11.1.1.4.2	Terminating procedures	281

11.1.1.4.3	Receiving a SIP re-INVITE for upgrade to emergency private call	283

11.1.1.4.4	Receiving a SIP re-INVITE for cancellation of emergency private call	285

11.1.1.4.5	Sending a SIP re-INVITE for upgrade to emergency private call	287

11.1.1.4.6	Sending a SIP re-INVITE for cancellation of emergency private call	287

11.1.2	Private call without floor control	288

11.1.2.1	General	288

11.1.2.2	MCPTT client procedures	288

11.1.2.3	Participating MCPTT function procedures	289

11.1.2.3.1	Originating procedures	289

11.1.2.3.2	Terminating procedures	289

11.1.2.4	Controlling MCPTT function procedures	289

11.1.2.4.1	Originating procedures	289

11.1.2.4.2	Terminating procedures	289

11.1.3	Ending the private call initiated by MCPTT client	289

11.1.3.1	MCPTT client procedures	289

11.1.3.1.1	On-demand private call	289

11.1.3.1.1.1	Client originating procedures	289

11.1.3.1.1.2	Client terminating procedures	289

11.1.3.1.2	Private call using pre-established session	289

11.1.3.1.2.1	Client originating procedures	289

11.1.3.1.2.2	Client terminating procedures	289

11.1.3.2	Participating MCPTT function procedures	290

11.1.3.2.1	Originating procedures	290

11.1.3.2.1.1	Receipt of SIP BYE request for on-demand private call	290

11.1.3.2.1.2	Receipt of REFER "BYE" request for private call using pre-established session	290

11.1.3.2.2	Terminating procedures	290

11.1.3.2.2.1	Receipt of SIP BYE request for private call on-demand	290

11.1.3.2.2.2	Receipt of SIP BYE request when ongoing pre-established session	290

11.1.3.3	Controlling MCPTT function procedures	290

11.1.3.3.1	Terminating procedures	290

11.1.4	Ending the private call initiated by the MCPTT server	290

11.1.4.1	General	290

11.1.4.2	MCPTT client procedures	291

11.1.4.3	Participating MCPTT function procedures	291

11.1.4.3.1	Originating procedures	291

11.1.4.3.2	Terminating procedures	291

11.1.4.3.2.1	Receipt of SIP BYE request for private call on-demand	291

11.1.4.3.2.2	Receipt of SIP BYE request when ongoing pre-established session	291

11.1.4.4	Controlling MCPTT function procedures	291

11.2	Off-network private call	291

11.2.1	General	291

11.2.1.1	Common procedures	291

11.2.1.1.1	Sending/Receiving a message	291

11.2.1.1.2	Session description	291

11.2.2	Basic call control	292

11.2.2.1	General	292

11.2.2.2	Private call control state machine	292

11.2.2.3	Call Control states	293

11.2.2.3.1	P0: start-stop	293

11.2.2.3.2	P1: ignoring same call id	293

11.2.2.3.3	P2: waiting for call response	293

11.2.2.3.4	P3: waiting for release response	293

11.2.2.3.5	P4: part of ongoing call	293

11.2.2.3.6	P5: pending	294

11.2.2.4	Procedures	294

11.2.2.4.1	General	294

11.2.2.4.2	Private call setup	294

11.2.2.4.2.1	Initiating a private call	294

11.2.2.4.2.2	Private call setup request retransmission	295

11.2.2.4.2.3	Ringing notification to the user	295

11.2.2.4.2.4	No response to private call setup request with automatic commencement mode	296

11.2.2.4.2.5	No response to private call setup request with manual commencement mode	296

11.2.2.4.2.6	No response to private call setup request after waiting for user acknowledgement	296

11.2.2.4.2.7	Private call setup request rejected	296

11.2.2.4.2.8	Private call setup request accepted	296

11.2.2.4.2.9	User cancels the private call setup request	297

11.2.2.4.3	Private call setup in automatic commencement mode	297

11.2.2.4.3.1	Unable to establish media	297

11.2.2.4.3.2	Responding to private call setup request when not participating in the ongoing call	298

11.2.2.4.3.3	Private call accept retransmission	299

11.2.2.4.3.4	Establishing the call	300

11.2.2.4.3.5	Call failure	300

11.2.2.4.4	Private call setup in manual commencement mode	300

11.2.2.4.4.1	Incoming private call	300

11.2.2.4.4.2	No response from the user	300

11.2.2.4.4.3	User accepts the private call setup request	301

11.2.2.4.4.4	Private call accept retransmission	302

11.2.2.4.4.5	Establishing the call	303

11.2.2.4.4.6	Call failure	303

11.2.2.4.4.7	User rejects the private call setup request	303

11.2.2.4.4.8	Caller cancels the private call setup request before call establishment	303

11.2.2.4.5	Private call release	304

11.2.2.4.5.1	Releasing a private call	304

11.2.2.4.5.2	Private call release retransmission	304

11.2.2.4.5.3	No response to private call release	304

11.2.2.4.5.4	Acknowledging private call release after call establishment	305

11.2.2.4.5.5	Private call release acknowledged	305

11.2.2.4.5.6	Max duration reached	305

11.2.2.4.5.7	Stop ignoring same call id	305

11.2.2.4.5.8	No response to emergency private call setup request	306

11.2.2.4.5.9	No response to emergency private call cancel	306

11.2.2.4.6	Error handling	306

11.2.2.4.6.1	Unexpected MONP message received	306

11.2.2.4.6.2	Unexpected indication from MCPTT user	306

11.2.2.4.6.3	Unexpected expiration of a timer	306

11.2.3	Call type control	306

11.2.3.1	General	306

11.2.3.2	Call type control state machine	306

11.2.3.3	Call Control states	307

11.2.3.3.1	Q0: waiting for the call to be established	307

11.2.3.3.2	Q1: in-progress private call	307

11.2.3.3.3	Q2: in-progress emergency private call	307

11.2.3.4	Procedures	308

11.2.3.4.1	General	308

11.2.3.4.2	Outgoing call initiated	308

11.2.3.4.3	Received incoming call	308

11.2.3.4.4	Establishing the private call	308

11.2.3.4.5	Upgrade call	309

11.2.3.4.5.1	User upgrades private call to emergency private call	309

11.2.3.4.5.2	Emergency private call setup request retransmission	309

11.2.3.4.5.3	Emergency private call setup request accepted	310

11.2.3.4.5.4	Emergency private call setup request rejected	310

11.2.3.4.5.5	No response to emergency private call setup request	310

11.2.3.4.5.6	Responding to emergency private call setup request when participating in the ongoing call	311

11.2.3.4.6	Downgrade call	311

11.2.3.4.6.1	User cancels the emergency private call	311

11.2.3.4.6.2	Emergency private call cancel retransmission	312

11.2.3.4.6.3	Emergency private call cancel accepted	312

11.2.3.4.6.4	No response to emergency private call cancel	313

11.2.3.4.6.5	Responding to emergency private call cancel	313

11.2.3.4.7	Call Release	313

11.2.3.4.8	Error handling	313

11.2.3.4.8.1	Unexpected MONP message received	313

11.2.3.4.8.2	Unexpected indication from MCPTT user	313

11.2.3.4.8.3	Unexpected expiration of a timer	313

12	Emergency alert	314

12.0	General	314

12.1	On-network emergency alert	314

12.1.1	Client procedures	314

12.1.1.1	Emergency alert origination	314

12.1.1.2	Emergency alert cancellation	315

12.1.1.3	MCPTT client receives an MCPTT emergency alert or call notification	316

12.1.2	Participating MCPTT function procedures	318

12.1.2.1	Receipt of a SIP MESSAGE request for emergency notification from the served MCPTT client	318

12.1.2.2	Receipt of a SIP MESSAGE request for emergency notification for terminating MCPTT client	319

12.1.3	Controlling MCPTT function procedures	320

12.1.3.1	Handling of a SIP MESSAGE request for emergency notification	320

12.1.3.2	Handling of a SIP MESSAGE request for emergency alert cancellation	321

12.2	Off-network emergency alert	324

12.2.1	General	324

12.2.2	Basic state machine	324

12.2.2.1	General	324

12.2.2.2	Emergency alert state machine	324

12.2.2.3	Emergency alert states	324

12.2.2.3.1	E1: Not in emergency state	324

12.2.2.3.2	E2: Emergency state	324

12.2.3	Procedures	325

12.2.3.1	Originating user sending emergency alert	325

12.2.3.2	Emergency alert retransmission	325

12.2.3.3	Terminating user receiving emergency alert	325

12.2.3.4	Terminating user receiving retransmitted emergency alert	326

12.2.3.5	Originating user cancels emergency alert	326

12.2.3.6	Terminating user receives GROUP EMERGENCY ALERT CANCEL message	326

12.2.3.7	Implicit emergency alert cancel	327

13	Location procedures	327

13.1	General	327

13.2	Participating MCPTT function location procedures	327

13.2.1	General	327

13.2.2	Location reporting configuration	327

13.2.3	Location information request	328

13.2.4	Location information report	328

13.2.5	Abnormal cases	328

13.3	MCPTT client location procedures	329

13.3.1	General	329

13.3.2	Location reporting configuration	329

13.3.3	Location information request	329

13.3.4	Location information report	330

13.3.4.1	Report triggering	330

13.3.4.2	Sending location information report	330

14	MBMS transmission usage procedure	331

14.1	General	331

14.2	Participating MCPTT function MBMS usage procedures	331

14.2.1	General	331

14.2.2	Sending MBMS bearer announcement procedures	331

14.2.2.1	General	331

14.2.2.2	Sending an initial MBMS bearer announcement procedure	331

14.2.2.3	Updating an announcement	333

14.2.2.4	Cancelling an MBMS bearer announcement	333

14.2.3	Receiving an MBMS bearer listening status from an MCPTT client	334

14.2.4	Abnormal cases	334

14.3	MCPTT client MBMS usage procedures	334

14.3.1	General	334

14.3.2	Receiving an MBMS bearer announcement	335

14.3.3	The MBMS bearer listening status report procedure	335

14.3.3.1	Conditions for sending an MBMS listening status report	335

14.3.3.2	Sending the MBMS bearer listening status report	336

14A	MCPTT Service Continuity	337

14A.1	General	337

14A.2	Service continuity from on-network MCPTT service to UE-to-network relay MCPTT service	338

14A.2.1	Remote UE	338

14A.2.2	SCC AS	338

14A.3	Service continuity from UE-to-network relay MCPTT service to on-network MCPTT service	339

14A.3.1	Remote UE	339

14A.3.2	SCC AS	339

15 Off-network message formats	339

15.1	MONP message functional definitions and contents	339

15.1.1	General	339

15.1.2	GROUP CALL PROBE message	339

15.1.2.1	Message definition	339

15.1.3	GROUP CALL ANNOUNCEMENT message	340

15.1.3.1	Message definition	340

15.1.4	GROUP CALL ACCEPT message	340

15.1.4.1	Message definition	340

15.1.5	PRIVATE CALL SETUP REQUEST message	341

15.1.5.1	Message definition	341

15.1.6	PRIVATE CALL RINGING message	341

15.1.6.1	Message definition	341

15.1.7	PRIVATE CALL ACCEPT message	342

15.1.7.1	Message definition	342

15.1.8	PRIVATE CALL REJECT message	342

15.1.8.1	Message definition	342

15.1.9	PRIVATE CALL RELEASE message	343

15.1.9.1	Message definition	343

15.1.10	PRIVATE CALL RELEASE ACK message	343

15.1.10.1	Message definition	343

15.1.11	PRIVATE CALL ACCEPT ACK message	344

15.1.11.1	Message definition	344

15.1.12	PRIVATE CALL EMERGENCY CANCEL message	344

15.1.12.1	Message definition	344

15.1.13	PRIVATE CALL EMERGENCY CANCEL ACK message	344

15.1.13.1	Message definition	344

15.1.14	GROUP CALL IMMINENT PERIL END message	345

15.1.14.1	Message definition	345

15.1.15	GROUP CALL EMERGENCY END message	345

15.1.15.1	Message definition	345

15.1.16	GROUP EMERGENCY ALERT message	346

15.1.16.1	Message definition	346

15.1.17	GROUP EMERGENCY ALERT ACK message	346

15.1.17.1	Message definition	346

15.1.18	GROUP EMERGENCY ALERT CANCEL message	347

15.1.18.1	Message definition	347

15.1.19	GROUP EMERGENCY ALERT CANCEL ACK message	347

15.1.19.1	Message definition	347

15.1.20	GROUP CALL BROADCAST message	348

15.1.20.1	Message definition	348

15.1.21	GROUP CALL BROADCAST END message	348

15.1.21.1	Message definition	348

15.2	General message format and information elements coding	348

15.2.1	General	348

15.2.2	Message type	349

15.2.3	Call identifier	350

15.2.4	Refresh interval	350

15.2.5	MCPTT group ID	351

15.2.6	SDP	351

15.2.7	Commencement mode	352

15.2.8	Reason	352

15.2.9	Confirm mode indication	352

15.2.10	MCPTT user ID	353

15.2.11	Call type	353

15.2.12	User location	353

15.2.13	Organization name	354

15.2.14	Call start time	354

15.2.15	Last call type change time	355

15.2.16	Probe response	355

Annex A (informative): Signalling flows	356

A.1	Group regrouping flow	356

A.1.1	General	356

A.1.2	Use case description	356

A.1.3	Signalling flow	357

Annex B (normative): Timers	376

B.1	General	376

B.2	On-network timers	376

B.2.1	Timers in the controlling MCPTT function	376

B.3	Off-network timers	377

B.3.1	Timers in off-network group call	377

B.3.1.1	Basic call control	377

B.3.1.2	Call type control	378

B.3.2	Timers in off-network private call	379

B.3.3	Timers in off-network broadcast call	381

B.3.4	Timers in off-network emergency alert	382

Annex C (normative): Counters	384

C.1	General	384

C.2	Off-network counters	384

C.2.1	Counters in off-network group call	384

C.2.2	Counters in off-network private call	384

Annex D (normative): Media feature tags within the current document	386

D.1	General	386

D.2	Definition of media feature tag g.3gpp.mcptt	386

Annex E (normative): ICSI values defined within the current document	387

E.1		General	387

E.2		Definition of ICSI value for MCPTT service	387

E.2.1	URN	387

E.2.2	Description	387

E.2.3	Reference	387

E.2.3	Contact	387

E.2.4	Registration of subtype	387

E.2.5	Remarks	387

Annex F (normative): XML schemas	388

F.1	XML schema for MCPTT Information	388

F.1.1	General	388

F.1.2	XML schema	388

F.1.3	Semantic	389

F.1.4	IANA registration template	391

F.2	XML schema for MBMS usage information	393

F.2.1	General	393

F.2.2	XML schema	393

F.2.3	Semantic	393

F.2.4	IANA registration template	394

F.3	XML schema for MCPTT location information	396

F.3.1	General	396

F.3.2	XML schema	396

F.3.3	Semantic	401

F.3.4	IANA registration template	405

F.4	XML schema for MCPTT (de)-affiliation requests	406

F.4.1	General	406

F.4.2	XML schema	407

F.4.3	Semantic	407

F.4.4	IANA registration template	407

F.5	XML schema for the floor request	409

F.5.1	General	409

F.5.2	XML schema	409

F.5.3	Semantic	410

F.5.4	IANA registration template	410

F.6	XML schema for integrity protection of MIME bodies	412

F.6.1	General	412

F.6.2	XML schema	412

Annex G (informative): On-network emergency and imminent peril related states	416

G.1	MCPTT emergency state	416

G.2	In-progress emergency group state	416

G.3	MCPTT emergency group state	417

G.4	MCPTT emergency group call state	417

G.5	MCPTT emergency alert state	418

G.6	In-progress imminent peril group state	420

G.7	MCPTT imminent peril group state	420

G.8	MCPTT imminent peril group call state	421

G.9	In-progress emergency private call state	422

G.10	MCPTT emergency private priority state	422

G.11	MCPTT emergency private call state	423

G.12	MCPTT private emergency alert state	424

Annex H (informative): On-network routing considerations	425

H.1	General	425

H.2	Group Call	425

H.3	Private Call	427

Annex I (normative): MCPTT Off-Network Protocol (MONP) message coding rules	427

I.1	General	427

I.2	MONP messages	428

I.2.1	Components of a MONP message	428

I.2.2	Format of standard information elements	428

I.2.2.1	Information element type and value part	428

I.2.2.2	Length indicator	428

I.2.2.3	Information element identifier	429

I.2.2.4	Categories of IEs; order of occurrence of IEI, LI, and value part	429

I.2.2.5	Method for IE structure	431

I.2.2.6	Imperative part of a standard MONP message	432

I.2.2.6.1	Standard information elements of the imperative part	432

I.2.2.7	Non-imperative part of a standard MONP message	432

I.2.2.8	Presence requirements of information elements	433

I.2.2.9	Description of standard MONP messages	434

Annex J (informative): INFO packages defined in the present document	434

J.1	Info package for transfer of floor requests	434

J.1.1	Scope	434

J.1.2	g.3gpp.mcptt-floor-request info package	434

J.1.2.1	Overall description	434

J.1.2.2	Applicability	434

J.1.2.4	Info package name	435

J.1.2.5	Info package parameters	435

J.1.2.6	SIP options tags	435

J.1.2.7	INFO message body parts	435

J.1.2.8	Info package usage restrictions	435

J.1.2.9	Rate of INFO Requests	435

J.1.2.10	Info package security considerations	435

J.1.2.11	Implementation details and examples	435

J.2	Info package for transfer of MCPTT information	436

J.2.1	Scope	436

J.2.2	g.3gpp.mcptt-info info package	436

J.2.2.1	Overall description	436

J.2.2.2	Applicability	436

J.2.2.4	Info package name	436

J.2.2.5	Info package parameters	437

J.2.2.6	SIP options tags	437

J.2.2.7	INFO message body parts	437

J.2.2.8	Info package usage restrictions	437

J.2.2.9	Rate of INFO Requests	437

J.2.2.10	Info package security considerations	437

J.2.2.11	Implementation details and examples	437

Annex K (informative): IANA UDP port registration form	437

Annex L (informative): Change history	440

History	446

# Foreword

This Technical Specification (TS) has been produced by ETSI 3rd Generation Partnership Project (3GPP).

The present document may refer to technical specifications or reports using their 3GPP identities, UMTS identities or GSM identities. These should be interpreted as being references to the corresponding ETSI deliverables.

The cross reference between GSM, UMTS, 3GPP and ETSI identities can be found under http://webapp.etsi.org/key/queryform.asp.

# Modal verbs terminology

In the present document "shall", "shall not", "should", "should not", "may", "need not", "will", "will not", "can" and "cannot" are to be interpreted as described in clause 3.2 of the ETSI Drafting Rules (Verbal forms for the expression of provisions).

"must" and "must not" are NOT allowed in ETSI deliverables except when used in direct citation.

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document specifies the session control protocols needed to support Mission Critical Push To Talk (MCPTT). The present document specifies both on-network and off-network protocols.

Mission critical communication services are services that require preferential handling compared to normal telecommunication services, e.g. in support of police or fire brigade.

The MCPTT service can be used for public safety applications and also for general commercial applications (e.g., utility companies and railways).

The present document is applicable to User Equipment (UE) supporting the MCPTT client functionality, and to application servers supporting the MCPTT server functionality.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.179: "Mission Critical Push To Talk (MCPTT) over LTE; Stage 1".

[3]	3GPP TS 23.179: "Functional architecture and information flows to support mission critical communication services; Stage 2".

[4]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[5]	3GPP TS 24.380: "Mission Critical Push To Talk (MCPTT) floor control Protocol specification".

[6]	IETF RFC 3841 (August 2004): "Caller Preferences for the Session Initiation Protocol (SIP)".

[7]	IETF RFC 4028 (April 2005): "Session Timers in the Session Initiation Protocol (SIP)".

[8]	Void .

[9]	IETF RFC 6050 (November 2010): "A Session Initiation Protocol (SIP) Extension for the Identification of Services".

[10]	IETF RFC 3550 (July 2003): "RTP: A Transport Protocol for Real-Time Applications".

[11]	Void.

[12]	IETF RFC 4566 (July 2006): "Session Description Protocol".

[13]	IETF RFC 3605 (October 2003): "Real Time Control Protocol (RTCP) attribute in Session Description Protocol (SDP)".

[14]	IETF RFC 3325 (November 2002): "Private Extensions to the Session Initiation Protocol (SIP) for Asserted Identity within Trusted Networks".

[15]	IETF RFC 5626 (October 2009): "Managing Client-Initiated Connections in the Session Initiation Protocol (SIP)".

[16]	IETF RFC 3840 (August 2004): "Indicating User Agent Capabilities in the Session Initiation Protocol (SIP)".

[17]	IETF RFC 5245 (April 2010): "Interactive Connectivity Establishment (ICE): A Protocol for Network Address Translator (NAT) Traversal for Offer Answer Protocols".

[18]	IETF RFC 5373 (November 2008): "Requesting Answering Modes for the Session Initiation Protocol (SIP)".

[19]	Void.

[20]	IETF RFC 5366 (October 2008): "Conference Establishment Using Request-Contained Lists in the Session Initiation Protocol (SIP)".

[21]	IETF RFC 2046 (November 1996): "Multipurpose Internet Mail Extensions (MIME) Part Two: Media Types".

[22]	IETF RFC 4488 (May 2006): "Suppression of Session Initiation Protocol (SIP) REFER Method Implicit Subscription".

[23]	IETF RFC 4538 (June 2006): "Request Authorization through Dialog Identification in the Session Initiation Protocol (SIP)".

[24]	IETF RFC 3261 (June 2002): "SIP: Session Initiation Protocol".

[25]	IETF RFC 3515 (April 2003): "The Session Initiation Protocol (SIP) Refer Method".

[26]	IETF RFC 6665 (July 2012): "SIP-Specific Event Notification".

[27]	IETF RFC 7647 (September 2015): "Clarifications for the use of REFER with RFC6665".

[28]	3GPP TS 24.334: "Proximity-services (ProSe) User Equipment (UE) to Proximity-services (ProSe) Function Protocol aspects; Stage 3".

[29]	IETF RFC 4412 (February 2006): "Communications Resource Priority for the Session Initiation Protocol (SIP)".

[30]	IETF RFC 4575 (August 2006): "A Session Initiation Protocol (SIP) Event Package for Conference State".

[31]	3GPP TS 24.381: "Mission Critical Push To Talk (MCPTT) group management Protocol specification".

[32]	IETF RFC 4483 (May 2006): "A Mechanism for Content Indirection in Session Initiation Protocol (SIP) Messages.

[33]	IETF RFC 3428 (December 2002): "Session Initiation Protocol (SIP) Extension for Instant Messaging".

[34]	IETF RFC 4964 (October 2007): "The P-Answer-State Header Extension to the Session Initiation Protocol for the Open Mobile Alliance Push-to-talk over Cellular".

[35]	IETF RFC 7614 (August 2015): "Explicit Subscriptions for the REFER Method".

[36]	IETF RFC 5318 (December 2008): "The Session Initiation Protocol (SIP) P-Refused-URI-List Private-Header (P-Header)".

[37]	IETF RFC 3903 (October 2004): "Session Initiation Protocol (SIP) Extension for Event State Publication".

[38]	IETF RFC 5368 (October 2008): "Referring to Multiple Resources in the Session Initiation Protocol (SIP)".

[39]	IETF RFC 5761 (April 2010): "Multiplexing RTP Data and Control Packets on a Single Port".

[40]	3GPP TS 23.003: "Numbering, addressing and identification".

[41]	3GPP TS 23.203: "Policy and charging control architecture".

[42]	3GPP TS 29.468: "Group Communication System Enablers for LTE (GCSE_LTE); MB2 Reference Point; Stage 3".

[43]	3GPP TS 24.008: "Mobile Radio Interface Layer 3 specification; Core Network Protocols; Stage 3".

[44]	IETF RFC 3264 (June 2002): "An Offer/Answer Model with the Session Description Protocol (SDP)".

[45]	3GPP TS 24.383: "Mission Critical Push To Talk (MCPTT) Management Object (MO)".

[46]	3GPP TS 33.179: "Security of Mission Critical Push-To-Talk (MCPTT)".

[47]	IETF RFC 4567 (July 2006): "Key Management Extensions for Session Description Protocol (SDP) and Real Time Streaming Protocol (RTSP)".

[48]	draft-holmberg-dispatch-mcptt-rp-namespace-01.txt (June 2016) "Resource-Priority Namespace for Mission Critical Push To Talk service".

Editor's note [CT1#96, C1-161109]: The above document cannot be formally referenced until it is published as an RFC.

[49]	3GPP TS 24.382: "Mission Critical Push To Talk (MCPTT) identity management Protocol specification.

[50]	3GPP TS 24.384: "Mission Critical Push To Talk (MCPTT) configuration management Protocol specification".

[51]	IETF RFC 3856 (August 2004): "A Presence Event Package for the Session Initiation Protocol (SIP)".

[52]	IETF RFC 3863 (August 2004): "Presence Information Data Format (PIDF)".

[53]	IETF RFC 7519 (May 2015): "JSON Web Token (JWT)".

[54]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[55]	IETF RFC 4354 (January 2006): "A Session Initiation Protocol (SIP) Event Package and Data Format for Various Settings in Support for the Push-to-Talk over Cellular (PoC) Service".

[56]	3GPP TS 24.007: "Mobile radio interface signalling layer 3; General aspects".

[57]	3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE); Stage 2".

[58]	3GPP TS 24.237: "IP Multimedia Subsystem (IMS) Service Continuity; Stage 3".

[59]	3GPP TS 29.199-9: "Open Service Access (OSA); Parlay X Web Services; Part 9: Terminal location".

[60]	W3C: "XML Encryption Syntax and Processing Version 1.1", https://www.w3.org/TR/xmlenc-core1/.

[61]	W3C: "XML Signature Syntax and Processing (Second Edition)", http://www.w3.org/TR/xmldsig-core/.

[62]	IETF RFC 2392 (August 1998): "Content-ID and Message-ID Uniform Resource Locators".

[63]	IETF RFC 4661 (September 2006): "An Extensible Markup Language (XML)-Based Format for Event Notification Filtering".

[64]	IETF RFC 6086 (January 2011): "Session Initiation Protocol (SIP) INFO Method and Package Framework".

[65]	IETF RFC 3891 (September 2004): "The Session Initiation Protocol (SIP) Replaces Header".

[66]	3GPP TS 24.216: "Communication continuity managed object".

[67]	IETF RFC 4122 (July 2005): "A Universally Unique IDentifier (UUID) URN Namespace".

[68]	IETF RFC 2045 (November 1996): "Multipurpose Internet Mail Extensions (MIME) Part One: Format of Internet Message Bodies".

[69]	3GPP TS 26.179: "Mission Critical Push To Talk (MCPTT) Codecs and media handling".

[70]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[71]	IETF RFC 4648 (October 2006): "The Base16, Base32, and Base64 Data Encodings".

[72]	IETF RFC 5627 (October 2009): "Obtaining and Using Globally Routable User Agent URIs (GRUUs) in the Session Initiation Protocol (SIP)".

[73]	3GPP TS 29.283: "Diameter Data Management Applications".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

An MCPTT user is affiliated to an MCPTT group: The MCPTT user is listed as a member of the MCPTT group in the MCPTT group document, the MCPTT server owning the MCPTT group has authorised the MCPTT user's interest in the MCPTT group and the MCPTT server serving the MCPTT user has authorised the MCPTT user's interest in the MCPTT group.

An MCPTT user is affiliated to an MCPTT group at an MCPTT client: The MCPTT user is affiliated to the MCPTT group, the MCPTT client has a registered IP address for an IMPU related to the MCPTT ID, and the MCPTT server serving the MCPTT user has authorised the MCPTT user's interest in the MCPTT group at the MCPTT client.

Affiliation status: Applies for an MCPTT user to an MCPTT group and has one of the following states:

a)	the "not-affiliated" state indicating that the MCPTT user is not interested in the MCPTT group and the MCPTT user is not affiliated to the MCPTT group;

b)	the "affiliating" state indicating that the MCPTT user is interested in the MCPTT group but the MCPTT user is not affiliated to the MCPTT group yet;

c)	the "affiliated" state indicating that the MCPTT user is affiliated to the MCPTT group and there was no indication that MCPTT user is no longer interested in the MCPTT group; and

d)	the "deaffiliating" state indicating that the MCPTT user is no longer interested in the MCPTT group but the MCPTT user is still affiliated to the MCPTT group.

Group identity: An MCPTT group identity or a temporary MCPTT group identity.

In-progress emergency private call state: the state of two participants when an MCPTT emergency private call is in progress.

In-progress imminent peril group state: the state of a group when an MCPTT imminent peril group call is in progress.

MCPTT client ID: is a globally unique identification of a specific MCPTT client instance. MCPTT client ID is a UUID URN as specified in IETF RFC 4122 [67].

MCPTT emergency alert state: MCPTT client internal perspective of the state of an MCPTT emergency alert.

MCPTT emergency group state: MCPTT client internal perspective of the in-progress emergency state of an MCPTT group maintained by the controlling MCPTT function.

MCPTT emergency group call state: MCPTT client internal perspective of the state of an MCPTT emergency group call.

MCPTT emergency private call state: MCPTT client internal perspective of the state of an MCPTT emergency private call.

MCPTT emergency private priority state: MCPTT client internal perspective of the in-progress emergency private call state of the two participants of an MCPTT emergency private call maintained by the controlling MCPTT function.

MCPTT imminent peril group call state: MCPTT client internal perspective of the state of an MCPTT imminent peril group call.

MCPTT imminent peril group state: MCPTT client internal perspective of the state of an MCPTT imminent peril group.

MCPTT private emergency alert state: MCPTT client internal perspective of the state of an MCPTT private emergency alert targeted to an MCPTT user.

MCPTT speech: Conversational audio media used in mission critical push to talk systems as defined by 3GPP TS 22.179 [2] and 3GPP TS 23.179 [3].

Media-floor control entity: A media control resource shared by participants in an MCPTT session, controlled by a state machine to ensure that only one participant can access the media resource at the same time.

Temporary MCPTT group identity: A group identity representing a temporary grouping of MCPTT group identities formed by the group regrouping operation as specified in 3GPP TS 24.381 [31].

Trusted mutual aid: A business relationship whereby the Partner MCPTT system is willing to share the details of the members of an MCPTT group that it owns with the Primary MCPTT system.

Untrusted mutual aid: A business relationship whereby the Partner MCPTT system is not willing to share the details of the members of an MCPTT group that it owns with the Primary MCPTT system.

For the purposes of the present document, the following terms and definitions given in 3GPP TS 22.179 [2] apply:

In-progress emergency

MCPTT emergency alert

MCPTT emergency group call

MCPTT emergency state

Partner MCPTT system

Primary MCPTT system

For the purpose of the present document, the following terms and definitions given in 3GPP TS 24.380 [5] apply:

MBMS subchannel

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ECGI			E-UTRAN Cell Global Identification

IPEG			In-Progress Emergency Group

IPEPC			In-Progress Emergency Private Call

IPIG			In-Progress Imminent peril Group

MBMS			Multimedia Broadcast and Multicast Service

MBSFN			Multimedia Broadcast multicast service Single Frequency Network

MCPTT			Mission Critical Push To Talk

MCPTT group ID	MCPTT group IDentity

MEA			MCPTT Emergency Alert

MEG			MCPTT Emergency Group

MEGC			MCPTT Emergency Group Call

MEPC			MCPTT Emergency Private Call

MEPP			MCPTT Emergency Private Priority

MES			MCPTT Emergency State

MIME			Multipurpose Internet Mail Extensions

MIG			MCPTT Imminent peril Group

MIGC			MCPTT Imminent peril Group Call

MONP			MCPTT Off-Network Protocol

MPEA			MCPTT Private Emergency Alert

NAT			Network Address Translation

PLMN			Public Land Mobile Network

QCI			QoS Class Identifier

RTP			Real-time Transport Protocol

SAI			Service Area Identifier

SDP			Session Description Protocol

SIP			Session Initiation Protocol

SSRC			Synchronization SouRCe

TGI			Temporary MCPTT Group Identity

TMGI			Temporary Mobile Group Identity

UE			User Equipment

URI			Uniform Resource Identifier

# 4	General

## 4.1	MCPTT overview

The MCPTT service supports communication between several users (i.e. group call), where each user has the ability to gain access to the permission to talk in an arbitrated manner. The MCPTT service also supports private calls between two users. Group calls and private calls can be provided on-network and off-network. In this release of the present document, support is only allowed for MCPTT speech communications.

The present document provides the call control protocol enhancements to support the MCPTT architectural procedures specified in 3GPP TS 23.179 [3].

For on-network calls, the present document makes use of the existing IMS procedures specified in 3GPP TS 24.229 [4], and provides new IMS application procedures specific for MCPTT. For on-network group calls, the procedures in the present document allow the use of unicast or multicast bearers.

For off-network calls, the present document utilises the procedures for ProSe direct discovery for public safety and the procedures for one-to-one ProSe direct communication for Public Safety, as specified in 3GPP TS 24.334 [28]. The present document specifies the MCPTT Off-Network Protocol (MONP) and the MONP application procedures.

For on-network and off-network calls, the present document provides support for MCPTT emergency calls, MCPTT imminent-peril calls and MCPTT emergency alerts.

NOTE:	MCPTT emergency calls do not utilise emergency bearers. Instead the EPS bearer priority of a normal bearer is adjusted.

The MCPTT procedures provided by the present document refer to:

-	the floor-control procedures defined in 3GPP TS 24.380[5];

-	the group management procedures defined in 3GPP TS 24.381 [31];

-	the identity management procedures defined in 3GPP TS 24.382 [49];

-	the security procedures defined in 3GPP TS 33.179 [46]; and

-	the PS-PS access transfer procedures procedures defined in 3GPP TS 24.237 [58].

The MCPTT procedures provided by the present document access the configuration parameters provided by 3GPP TS 24.383 [45] and 3GPP TS 24.384 [50].

Codecs and media handling for MCPTT are specified in 3GPP TS 26.179 [69];

The following procedures are provided within this document:

-	common procedures are specified in clause 6;

-	procedures for registration in the IM CN subsystem and service authorisation are specified in clause 7;

-	procedures for pre-established session establishment, modification and release are specified in clause 8;

-	procedures for affiliation are specified in clause 9;

-	procedures for on-network and off-network group call are specified in clause 10;

-	procedures for on-network and off-network private call are specified in clause 11;

-	procedures for on-network and off-network emergency alert are specified in clause 12;

-	location procedures are specified in clause 13;

-	MBMS transmission usage procedures are specified in clause 14; and

-	MCPTT service continuity procedures are specified in clause 14A.

The MCPTT UE primarily obtains access to the MCPTT service via E-UTRAN, using the procedures defined in 3GPP TS 24.301 [70].

## 4.2	URI and address assignments

In order to support MCPTT, the following URI and address assignments are assumed:

1)	the participating MCPTT function is configured to be reachable using:

a)	the public service identity identifying the pre-established session on the MCPTT server serving the MCPTT user;

b)	the MBMS public service identity of the participating MCPTT function; and

c)	the public service identity of the participating MCPTT function serving the MCPTT user.

NOTE:	The above PSI values are configured with the same URI. However, in the present document the above names on the URIs are used for the purpose of readability.

## 4.3	MCPTT speech

A session that contains MCPTT speech is either a full-duplex session or a half-duplex session with an SDP media component containing an audio media type with a codec suitable for conversational speech that exists between an MCPTT client and an MCPTT server.

If the MCPTT speech session is a half-duplex session, it additionally contains a media component that describes the characteristics of the media-floor control entity.

## 4.4	Warning Header Field

### 4.4.1	General

The MCPTT server can include a free text string in a SIP response to a SIP request. When the MCPTT server includes a text string in a response to a SIP INVITE request the text string is included in a Warning header field as specified in IETF RFC 3261 [24]. The MCPTT server includes the Warning code set to 399 (miscellaneous warning) and includes the host name set to the host name of the MCPTT server.

EXAMPLE: Warning: 399 "100 User not authorised to make group calls"

### 4.4.2	Warning texts

The text string included in a Warning header field consists of an explanatory text preceded by a 3-digit text code, according to the following format in Table 4.4.2-1.

Table 4.4.2-1 ABNF for the Warning text

warn-text      =/  DQUOTE mcptt-warn-code SP warn-mcptt-text DQUOTE

mcptt-warn-code = DIGIT DIGIT DIGIT

mcptt-warn-text = *( qdtext | quoted-pair )

Table 4.4.2-2 defines the warning texts that are defined for the Warning header field when a Warning header field is included in a response to a SIP INVITE request as specified in subclause 4.4.1.

Table 4.4.2-2: Warning texts defined for the Warning header field

| Code | Explanatory text | Description |
| --- | --- | --- |
| 100 | function not allowed due to <detailed reason> | The function is not allowed to this user. The <detailed reason> will be either "group definition", "access policy", "local policy", or "user authorisation", or can be a free text string. |
| 101 | service authorisation failed | The service authorisation of the MCPTT ID against the IMPU failed at the MCPTT server. |
| 102 | too many simultaneous affiliations | The MCPTT user already has N2 maximum number of simultaneous affiliations. |
| 103 | maximum simultaneous MCPTT group calls reached | The number of maximum simultaneous MCPTT group calls supported for the MCPTT user has been exceeded. |
| 104 | isfocus not assigned | A controlling MCPTT function has not been assigned to the MCPTT session. |
| 105 | subscription not allowed in a broadcast group call | Subscription to the conference event package rejected during a group call initiated as a broadcast group call. |
| 106 | user not authorised to join chat group | The MCPTT user is not authorised to join this chat group. |
| 107 | user not authorised to make private calls | The MCPTT user is not authorised to make private calls. |
| 108 | user not authorised to make chat group calls | The MCPTT user is not authorised to make chat group calls. |
| 109 | user not authorised to make prearranged group calls | The MCPTT user is not authorised to make group calls to a prearranged group. |
| 110 | user declined the call invitation | The MCPTT user declined to accept the call. |
| 111 | group call proceeded without all required group members | The required members of the group did not responded within the acknowledged call time, but the call still went ahead. |
| 112 | group call abandoned due to required group members not part of the group session | The group call was abandoned, as the required members of the group did not respond within the acknowledged call time. |
| 113 | group document does not exist | The group document requested from the group management server does not exist. |
| 114 | unable to retrieve group document | The group document exists on the group management server but the MCPTT server was unable to retrieve it. |
| 115 | group is disabled | The group has the <disabled> element set to "true" in the group management server. |
| 116 | user is not part of the MCPTT group | The group exists on the group management server but the requesting user is not part of this group. |
| 117 | the group identity indicated in the request is a prearranged group | The group id that is indicated in the request is for a prearranged group, but did not match the request from the MCPTT user. |
| 118 | the group identity indicated in the request is a chat group | The group id that is indicated in the request is for a chat group, but did not match the request from the MCPTT user, |
| 119 | user is not authorised to initiate the group call | The MCPTT user identified by the MCPTT ID is not authorised to initiate the group call. |
| 120 | user is not affiliated to this group | The MCPTT user is not affiliated to the group. |
| 121 | user is not authorised to join the group call | The MCPTT user identified by the MCPTT ID is not authorised to join the group call. |
| 122 | too many participants | The group call has reached its maximum number of participants. |
| 123 | MCPTT session already exists | Inform the MCPTT user that the group call is currently ongoing. |
| 124 | maximum number of private calls reached | The maximum number of private calls allowed at the MCPTT server for the MCPTT user has been reached. |
| 125 | user not authorised to make private call with automatic commencement | The MCPTT user is not authorised to make a private call with automatic commencement. |
| 126 | user not authorised to make private call with manual commencement | The MCPTT user is not authorised to make a private call with manual commencement. |
| 127 | user not authorised to be called in private call | The called MCPTT user is not allowed to be part of a private call. |
| 128 | isfocus already assigned | The MCPTT server owning an MCPTT group received a SIP INVITE request destined to the MCPTT group from another MCPTT server already assigned as the controlling MCPTT function and the MCPTT server owning the MCPTT group does not support mutual aid or supports trusted mutual aid but does not authorise trusted mutual aid. |
| 137 | the indicated group call does not exists | The participating MCPTT function cannot find an ongoing group session associated with the received MCPTT session identity. |
| 138 | subscription of conference events not allowed | The controlling MCPTT function could not allow the MCPTT user to subscribe to the conference event package. |
| 139 | integrity protection check failed | The integrity protection of an XML MIME body failed. |
| 140 | unable to decrypt XML content | The XML content cannot be decrypted. |
| 141 | user unknown to the participating function | The participating function is unable to associate the public user identity with an MCPTT ID. |
| 142 | unable to determine the controlling function | The participating function is unable to determine the controlling function for the group call or private call. |
| 143 | not authorised to force auto answer | The calling user is not authorised to force auto answer on the called user. |
| 144 | user not authorised to call this particular user | The calling user is not authorised to call this particular called user. |
| 145 | unable to determine called party | The participating function was unable to determine the called party from the information received in the SIP request. |
| 146 | T-PF unable to determine the service settings for the called user | The service settings have not been uploaded by the terminating client to the terminating participating server. |
| 147 | user is authorized to initiate a temporary group call | The non-controlling MCPTT function has authorized a request from the controlling MCPTT function to authorize a user to initiate an temporary group session. |
| 148 | MCPTT group is regrouped | The MCPTT group hosted by a non-controlling MCPTT function is part of a temporary group session as the result of the group regroup function. |
| 149 | SIP-INFO request pending | The MCPTT client needs to wait for a SIP-INFO request with specific content, before taking further action. |
| 150 | invalid combinations of data received in MIME body | The MCPTT client included invalid combinations of data in the SIP request. |

## 4.5	MCPTT session identity

The MCPTT session identity is a SIP URI, which identifies the MCPTT session between:

-	the MCPTT client and the participating MCPTT function;

-	the participating MCPTT function and the controlling MCPTT function

- the controlling MCPTT function and the non-controlling MCPTT function; and

-	the non-controlling MCPTT function and the participating MCPTT function.

The MCPTT session identity shall be a GRUU as defined in IETF RFC 5627 [72] assigned by the MCPTT server as per 3GPP TS 24.229 [4].

The MCPTT session identity identifies the MCPTT session in such a way that e.g.:

-	the MCPTT user is able to subscribe to the participant information of the ongoing MCPTT session;

-	the MCPTT user is able to re-join an ongoing MCPTT session; and

-	the IM CN subsystem is able to route an initial SIP request to the controlling MCPTT function.

The controlling MCPTT function allocates a unique MCPTT session identity hosted at the controlling MCPTT function for the MCPTT session at the time of session establishment.

The non-controlling MCPTT function allocates a unique MCPTT session identity hosted at the non-controlling MCPTT function for the MCPTT session at the time of session establishment.

When protection of sensitive application data is required by the MCPTT operator, the MCPTT session identity cannot contain identity information that is classed as sensitive such as the MCPTT ID or the MCPTT Group ID, as specified in subclause 4.8.

The controlling MCPTT function and non-controlling MCPTT function send the MCPTT session identity towards the MCPTT client during MCPTT session establishment by including it in the Contact header field of the final SIP response to a session initiation request.

The participating MCPTT function allocates a unique MCPTT session identity hosted at the participating MCPTT function for the MCPTT session when it receives a MCPTT session identity in the Contact header field of a SIP request or a SIP response from the controlling MCPTT function or non-controlling MCPTT function and includes it in the Contact header field of the SIP request or SIP response sent towards the MCPTT client. The participating MCPTT function maintains a mapping of the MCPTT session identities it sends to the MCPTT client to the corresponding MCPTT session identities received from the controlling MCPTT function.

The MCPTT client can cache the MCPTT session identity until a time when it is no longer needed.

The MCPTT session identity is also used in floor control requests and responses as specified in 3GPP TS 24.380 [5].

## 4.6	MCPTT priority calls and alerts

### 4.6.1	MCPTT emergency group calls

MCPTT emergency group calls as defined by 3GPP TS 23.179 [3] are supported by the procedures in this specification. The following MCPTT emergency group call functionalities are described:

-	MCPTT emergency group call origination;

-	upgrade of an MCPTT group call to an MCPTT emergency group call; and

-	in-progress group emergency cancel.

NOTE 1:	In-progress group emergency cancel means the cancellation of the in-progress emergency state of the group, which is managed by the controlling MCPTT function.

The above functionalities are supported using both MCPTT prearranged group calls and MCPTT chat group calls.

Key aspects of MCPTT emergency group calls include:

-	adjusted EPS bearer priority for all participants whether or not they themselves are in an emergency condition (i.e. have their MCPTT emergency state set). This is achieved by using the Resource-Priority header field as specified in IETF RFC 4412 [29] with namespaces defined for use by MCPTT specified in draft-holmberg-dispatch-mcptt-rp-namespace [48];

-	pre-emptive floor control priority over MCPTT users in MCPTT emergency group calls who themselves do not have their MCPTT emergency state set;

-	restoration of normal EPS bearer priority to the call participants when the in-progress emergency group state is cancelled;

-	restoration of normal floor control priority participants when the in-progress emergency group state is cancelled;

-	requires the MCPTT user to be authorised to either originate or cancel an MCPTT emergency group call;

-	requests to originate MCPTT emergency group calls may also include an indication of an MCPTT emergency alert; and

-	requests to cancel MCPTT emergency group calls may also include an indication of cancelling a previously issued MCPTT emergency alert.

There are a number of states that are key in managing these aspects of MCPTT emergency group calls, which include:

-	MCPTT emergency state: as defined in 3GPP TS 22.179 [2] and 3GPP TS 23.179 [3], indicates that the MCPTT user is in a life-threatening situation. Managed by the MCPTT user of the device or an authorised MCPTT user. While the MCPTT emergency state is set on the client, all calls originated by the client will be MCPTT emergency calls, assuming the MCPTT user is authorised for MCPTT emergency calls on them.

-	in-progress emergency group state: as defined in 3GPP TS 22.179 [2] and 3GPP TS 23.179 [3], indicates whether or not there is an MCPTT emergency group call ongoing on the specified group. This state is managed by the controlling MCPTT function. All group calls originated on this MCPTT group when in an in-progress emergency state are MCPTT emergency group calls until this state is cancelled, whether or not the originator is themselves in an MCPTT emergency state.

-	MCPTT emergency group (MEG) state: this is an internal state managed by the MCPTT client which tracks the in-progress emergency state of the group as defined in 3GPP TS 22.179 [2] and 3GPP TS 23.179 [3] and managed by the controlling MCPTT function. Ideally, the MCPTT client would not need to track the in-progress emergency group state, but doing so enables the MCPTT client to request MCPTT emergency-level priority earlier than otherwise possible. For example, if the MCPTT user wishes to join an MCPTT emergency group call and is not in MCPTT emergency state itself, the MCPTT client should have emergency level priority. If it has knowledge of the in-progress emergency state of the group, it can request priority by including a Resource-Priority header field set to the MCPTT namespace specified in draft-holmberg-dispatch-mcptt-rp-namespace [48], and appropriate priority level in the SIP INVITE request (or SIP re-INVITE request).

-	MCPTT emergency group call (MEGC) state: this is an internal state managed by the MCPTT client which in conjunction with the MCPTT emergency alert state aids in managing the MCPTT emergency state and related actions.

-	MCPTT emergency alert (MEA) state: this is also an internal state of the MCPTT client which in conjunction with the MCPTT emergency group call state aids in managing the MCPTT emergency state and related actions.

NOTE 2:	The above states and their transitions are described in Annex G.

### 4.6.2	MCPTT emergency private calls

MCPTT emergency private calls as defined by 3GPP TS 23.179 [3] are supported by the procedures in this specification. The following MCPTT emergency private call functionalities are specified in the present document:

-	MCPTT emergency private call origination with optional MCPTT emergency alert initiation;

-	upgrade of an MCPTT private call to an MCPTT emergency private; and

-	cancellation of the MCPTT emergency private call priority.

Key aspects of MCPTT emergency private calls include:

-	adjusted EPS bearer priority for both participants whether or not they are both in an emergency condition (i.e. both have their MCPTT emergency state set). This is achieved by using the Resource-Priority header field as specified in IETF RFC 4412 [29] with namespaces defined for use by MCPTT specified in draft-holmberg-dispatch-mcptt-rp-namespace [48];

-	the initiator of the MCPTT emergency private call can override the other MCPTT user in the MCPTT emergency private call unless that user also has their MCPTT emergency state set;

-	restoration of normal EPS bearer priority to the call according to system policy (e.g., configured time limit for the emergency priority of an MCPTT emergency private call or cancellation of the emergency condition of the private call);

-	restoration of normal floor control priority participants when the emergency elevated priority is cancelled;

-	requires the MCPTT user to be authorised to either originate or cancel an MCPTT emergency private call;

-	requires the targeted MCPTT user to be authorised to receive an MCPTT emergency private call;

-	requests to originate MCPTT emergency private calls may also include an indication of an MCPTT emergency alert; and

-	the originator of the MCPTT emergency private call can request that the call use either manaual or automatic commencement mode.

There are a number of states that are key in managing these aspects of MCPTT emergency private calls, which include:

-	MCPTT emergency state (MES): as defined in 3GPP TS 22.179 [2] and 3GPP TS 23.179 [3], indicates that the MCPTT user is in a life-threatening situation. Managed by the MCPTT user of the device or an authorised MCPTT user. While the MCPTT emergency state is set on the client, all MCPTT group and private calls originated by the client will be MCPTT emergency calls, assuming the MCPTT user is authorised for MCPTT emergency calls on them.

-	MCPTT private emergency alert (MPEA) state: this is an internal state of the MCPTT client which in conjunction with the MCPTT emergency private call state aids in managing the MCPTT emergency state and related actions.

-	MCPTT emergency private call (MEPC) state: this is an internal state managed by the MCPTT client which in conjunction with the MCPTT emergency alert state aids in managing the MCPTT emergency state and related actions.

-	In-progress emergency private call (IPEPC) state: indicates whether or not there is an MCPTT emergency private call in-progress for the two participants. This state is managed by the controlling MCPTT function. All private calls originated between these two participants when in an in-progress emergency private call state are MCPTT emergency private calls until this state is cancelled, whether or not the originator is in an MCPTT emergency state.

-	MCPTT emergency private priority (MEPP) state: this is an internal state managed by the MCPTT client which tracks the in-progress emergency private call state of the private call managed by the controlling MCPTT function. Ideally, the MCPTT client would not need to track the in-progress emergency private priority state, but doing so enables the MCPTT client to request MCPTT emergency-level priority earlier than otherwise possible. For example, if the MCPTT user wishes to join an MCPTT emergency private call and is not in the MCPTT emergency state, the MCPTT client should have emergency level priority. If it has knowledge of the in-progress emergency private priority state of the private call (i.e., the two participants), it can request priority by including a Resource-Priority header field set to the MCPTT namespace specified in draft-holmberg-dispatch-mcptt-rp-namespace [48], and appropriate priority level in the SIP INVITE request (or SIP re-INVITE request).

NOTE:	The above states and their transitions are described in Annex G.

### 4.6.3	MCPTT emergency alerts

MCPTT emergency alerts as defined by 3GPP TS 23.179 [3] are supported by the procedures in this specification. The following MCPTT emergency group call functionalities are specified in the present document:

-	MCPTT emergency alert origination; and

-	MCPTT emergency alert cancellation.

MCPTT emergency alerts are supported procedurally by two general mechanisms. One mechanism is embedded within the MCPTT emergency call (both emergency private call and emergency group call using both prearranged and chat session models) signalling procedures documented in clause 10 and clause 11 of this specification. The other mechanism utilizes SIP MESSAGE requests and is documented in clause 12.

MCPTT emergency alerts can be initiated or cancelled as options in the following signalling procedures documented in clause 10 and clause 11:

-	MCPTT emergency group call initiation;

-	MCPTT group call upgraded to MCPTT emergency call;

-	MCPTT emergency group call cancellation (i.e., in-progress emergency state of the group set to false);

-	MCPTT emergency private call initiation; and

-	MCPTT private call upgrade to MCPTT emergency private call.

MCPTT emergency alerts can also be initiated or cancelled as described in the procedures of clause 12 which include:

-	MCPTT emergency alert initiation; and

-	MCPTT emergency alert cancellation (with optional cancelling of the in-progress emergency state of a group).

When MCPTT emergency alerts are initiated as an option in initiating or upgrading to an MCPTT emergency group call or are initiated using SIP MESSAGE requests, they are targeted to an MCPTT group, and, if not already affiliated, will result in the initiator being implicitly affiliated to the MCPTT group. When initiated as an option in initiating or upgrading to an MCPTT emergency private call, an MCPTT emergency alert is targeted to an individual MCPTT user, not to an MCPTT group.

Key aspects of MCPTT emergency alerts include:

-	MCPTT emergency (MES) state: the MCPTT client's MCPTT emergency state as described in clause G.1 is set upon initiation of an MCPTT emergency alert. While the MCPTT emergency state is set, assuming the MCPTT user has the needed authorisations, if the user initiates a private call and is authorised to do so, the MCPTT private call will be an MCPTT emergency private call. Similarly, assuming the needed authorisations, any subsequent MCPTT group call initiated by an MCPTT user with the MCPTT emergency state set will be an MCPTT emergency group call.

-	MCPTT emergency alert (MEA) state: the MCPTT client maintains the internal MCPTT emergency alert state (MEA) which aids in the management of the MCPTT emergency state as described in clause G.5.

-	MCPTT private emergency alert (MPEA) state: the MCPTT client maintains the MCPTT private emergency alert state of an MCPTT emergency alert targeted to an MCPTT user which aids in the management of the MCPTT emergency state.

-	In-progress emergency group (IPEG) state : MCPTT emergency alert initiation or cancellation in and of itself does not impact the in-progress emergency state of the targeted group, which is maintained by the controlling MCPTT function, nor does it impact the priority of the EPS bearers. However, in setting the MCPTT emergency state, assuming an MCPTT user is authorised to make MCPTT emergency calls on the targeted group, any subsequent MCPTT group call the MCPTT user initiates on the group will cause the in-progress emergency state of the group to be set as described in clause G.2 and will result in upgraded priority of the EPS bearers used in the MCPTT emergency call.

-	Authorisations for emergency alerts: MCPTT users need to be authorised to initiate MCPTT emergency alerts and additionally need to be authorised to cancel MCPTT emergency alerts. The parameters related to these authorisations are specified in 3GPP TS 24.383 [45] and 3GPP TS 24.384 [50].

### 4.6.4	MCPTT imminent peril group call

MCPTT imminent peril group calls as defined by 3GPP TS 23.179 [3] are supported by the procedures in this specification. The following MCPTT imminent peril group calls functionalities are specified in the present document:

-	MCPTT imminent peril group calls origination;

-	upgrade of an MCPTT group call to an MCPTT imminent peril group call;

-	upgrade from an MCPTT imminent peril group call to an MCPTT emergency group call; and

-	cancellation of the in-progress imminent peril state of the group.

Key aspects of MCPTT imminent peril include:

-	adjusted EPS bearer priority for all participants when the in-progress imminent peril state of the group is set whether or not they themselves initiated an imminent peril group call. This is achieved by using the Resource-Priority header field as specified in IETF RFC 4412 [29] with namespaces defined for use by MCPTT specified in draft-holmberg-dispatch-mcptt-rp-namespace [48];

-	restoration of normal EPS bearer priority to the call when the in-progress imminent peril group state is cancelled; and

-	requires the MCPTT user to be authorised to either originate or cancel an MCPTT imminent peril group call.

Relationship to other MCPTT priority group call types:

-	A normal MCPTT group call can be upgraded to an MCPTT imminent peril group call;

-	An MCPTT imminent peril group call can be upgraded to an MCPTT emergency group call;

-	When either an MCPTT imminent peril group call or an MCPTT emergency group call (i.e., their respective "in-progress" states) the group call returns to the priority designated for normal group calls, i.e., their is no direct transition from an MCPTT emergency group call to an MCPTT imminent peril group call;

-	MCPTT imminent peril functionality is only applicable to MCPTT group calls, not MCPTT private calls; and

-	MCPTT imminent peril group calls have no associated alert capabilities such as the MCPTT emergency alert capability which is associated with MCPTT emergency group calls.

There are a number of states that are key in managing these aspects of MCPTT imminent peril group calls, which include:

-	MCPTT imminent peril group (MIG) state: this is an internal state of the MCPTT client which in conjunction with the MCPTT imminent peril group call state aids the client in managing the use of the Resource-Priority header field and related actions.

-	MCPTT imminent peril group call (MIGC) state: this is an internal state managed by the MCPTT client which in conjunction with the MCPTT imminent peril group state aids the client in managing the use of the Resource-Priority header field and related actions.

-	In-progress imminent peril group (IPIG) state: this a state of the MCPTT group which is managed by the controlling MCPTT function. While an MCPTT group is in an in-progress imminent peril group state, all participants in group calls using this group will receive elevated priority.

The above states and their transitions are described in Annex G.

## 4.7	End-to-end communication security

If a mission critical organisation requires MCPTT users to communicate using end-to-end security, a security context needs to be established between the initiator of the call and the recipient(s) of the call, prior to the establishment of media, or floor control signalling. This provides assurance to MCPTT users that no unauthorised access to communications is taking place within the MCPTT network. An MCPTT key management server (KMS) manages the security domain. For any end-point to use or access end-to-end secure communications, it needs to be provisioned with keying material associated to its identity by the KMS as specified in 3GPP TS 33.179 [46].

For group calls, the security context is set up at the time of creation of the group or temporary group. The group management server creates group call keying material associated with the group and distributes it to all members of the group or temporary group, in advance of the initiation of a group call as specified in 3GPP TS 24.381 [31] and 3GPP TS 33.179 [46]. The establishment of a security context for group calls has no impact on this specification.

For private calls, the security context is initiated at call setup. An end-to-end security context is established that is unique to the pair of users involved in the call. The procedure involves transferral of an encapsulated private call key (PCK) and private call key id (PCK-ID) from the initiator to the terminator. The PCK is encrypted using the terminator's MCPTT ID and domain-specific material provided from the KMS. The PCK and PCK-ID are distributed within a MIKEY payload within the SDP offer of the private call request. This payload is called a MIKEY-SAKKE I_MESSAGE, as defined in IETF RFC 6509 [47], which ensures the confidentiality, integrity and authenticity of the payload. The encoding of the MIKEY payload in the SDP offer is described in IETF RFC 4567 [47] using an "a=keymgmt" attribute. The payload is signed using a key associated to the identity of the initiating user. At the terminating side, the signature is validated. If valid, the UE extracts and decrypts the encapsulated PCK. The MCPTT UE also extracts the PCK-ID. This process is described in 3GPP TS 33.179 [46]. With the PCK successfully shared between the two MCPTT UEs, the UEs are able to use SRTP/SRTCP to create an end-to-end secure session.

End-to-end security is independent of the transmission path and hence is applicable to both on and off-network communications. With a security context established, the group call key and private call key can be used to encrypt media and, if required, floor control traffic between the end-points as described in 3GPP TS 24.380 [5] clause 13.

## 4.8	Protection of sensitive application data.

In certain deployments, for example, in the case that the MCPTT operator uses the underlying SIP core infrastructure from the carrier operator, the MCPTT operator can prevent certain sensitive application data from being visible in the clear to the SIP layer. The following data are classed as sensitive application data:

-	MCPTT ID;

-	MCPTT group ID;

-	user location information;

-	emergency, alert and imminent-peril indicators; and

-	access token (containing the MCPTT ID).

The above data is transported as XML content in SIP messages. in XML elements or XML attributes.

Data is transported in attributes in the following circumstances in the procedures in the present document:

-	an MCPTT ID and an MCPTT Group ID in an XML document published in SIP PUBLISH request for affiliation according to IETF RFC 3856 [51];

-	an MCPTT ID or an MCPTT Group ID in XML document notified in a SIP NOTIFY request for affiliation according to IETF RFC 3856 [51];

-	an MCPTT ID in application/resource-lists+xml document included in an SIP INVITE request setting up a private call according to IETF RFC 5366 [20];

-	an MCPTT ID in application/resource-lists+xml document included in an SIP INVITE request setting up a group call to a temporary group involving a non-controlling function that works in "Trusted Mode" according to IETF RFC 5366 [20], whereby the participants are returned to the controlling function in a MIME body of a SIP 403 (Forbidden) with the P-Refused-URI-List header field according to IETF RFC 5318 [36];

-	an MCPTT ID in XML document provided in SIP NOTIFY request of a conference event package according to IETF RFC 4575 [30]; and

-	an MCPTT ID or MCPTT Group ID in application/resource-lists+xml document according to IETF RFC 5366 [20], included in a SIP REFER request when using a pre-established session (the application/resource-lists+xml MIME body is pointed to by a Cid-URL as specified in IETF RFC 2392 [62] contained in the Refer-To header field of the SIP REFER request);

3GPP TS 33.179 [46] describes a method to provide confidentiality protection of sensitive application data in elements by using XML encryption (i.e. xmlenc) and in attributes by using an attribute confidentiality protection scheme described in subclause 6.6.2.3 of the present document. Integrity protection can also be provided by using XML signatures (i.e. xmlsig).

Protection of the data relies on a shared XML protection key (XPK) used to encrypt and sign data:

-	between the MCPTT client and the MCPTT server, the XPK is a client-server key (CSK); and

-	between MCPTT servers and between MCPTT domains, the XPK is a SIP protection key (SPK).

The CSK (XPK) and a key-id CSK-ID (XPK-ID) are generated from keying material provided by the key management server. Identity based public key encryption based on MIKEY-SAKKE is used to transport the CSK between SIP end-points. The encrypted CSK is transported from the MCPTT client to the MCPTT server when the MCPTT client performs service authorisation as described in clause 7 and is also used during service authorisation to protect the access token.

The SPK (XPK) and a key-id SPK-ID (XPK-ID) are directly provisioned in the MCPTT servers.

Configuration in the MCPTT client and MCPTT server is used to determine whether one or both of confidentiality protection and integrity protection are required.

The following four examples give a brief overview of the how confidentiality and integrity protection is applied to application data in this specification.

EXAMPLE 1:	Pseudo code showing how confidentiality protection is represented in the procedures in the document for sensitive data sent by the originating client.

IF configuration is set for confidentiality protection of sensitive data

THEN

Encrypt data element using the CSK (XPK) by following TS 33.179;

Include in an <EncryptedData> element of the XML MIME body according to TS 33.179:

(1) the encryption method;

(2) the key-id (XPK-ID);

(3) the cipher data;

Encrypt URIs in attribute using the CSK (XPK) by following subclause 6.6.2.3;

ELSE

include application data into XML MIME body in clear text;

ENDIF;

EXAMPLE 2:	Pseudo code showing how integrity protection is represented in the procedures in the present document for data sent by the originating client.

IF configuration is set for integrity protection of application data

THEN

Use a method to hash the content as specified in TS 33.179;

Generate a signature for the hashed content using the CSK (XPK) as specified in TS 33.179;

Include within a <Signature> XML element of the XML MIME body according to TS 33.179:

(1) a cannonicalisation method to be applied to the signed information;

(2) the signature method used for generating the signature;

(3) a reference to the content to be signed;

(4) the hashing method used;

(5) the hashed content;

(6) the key-id (XPK-ID);

(7) the signature value;

ENDIF;

EXAMPLE 3:	Pseudo code showing how confidentiality protection is represented in the procedures in the present document at the server side when receiving encrypted content.

IF configuration is set for confidentiality protection of sensitive data

THEN

Check that the XML content contains the <EncryptedData> element;

Check that the XML document contains a URI with the domain name for MCPTT confidentiality protection;

Return an error if the <EncryptedData> element or domain name for MCPTT confidentiality protection are not found;

Otherwise:

(1) obtain the CSK (XPK) using the CSK-ID (XPK-ID) in the received XML body;

(2) for encrypted data in elements, decrypt the data elements using the CSK as specified in TS 33.179 as required;

(3) for encrypted URIs in attributes, decrypt the URIs using the CSK as specified in subclause 6.6.2.3;

ENDIF;

EXAMPLE 4:	Pseudo code showing how integrity protection is represented in the procedures in the present document at the server side when receiving signed content.

IF configuration is set for integrity protection of application data

THEN

Check that the XML content contains the <Signature> element;

Return an error if the <Signature> element is not found;

Otherwise:

(1) obtain the CSK (XPK) using the CSK-ID (XPK-ID) in the received XML body;

(2) verify the signature of the content using the CSK;

Return an error if the validation of the signature fails;

IF validation of the signature passes

THEN

decrypt any data found in <EncryptedData> elements;

decrypt any encrypted URIs found in attributes;

ENDIF;

ENDIF;

The content can be re-encrypted and signed again using the SPK between MCPTT servers.

The following examples show the difference between normal and encrypted data content. In this example consider the MCPTT client initiating a prearranged group session.

EXAMPLE 5:	<mcptt-info> MIME body represented with data elements in the clear:

Content-Type: application/vnd.3gpp.mcptt-info+xml

<?xml version="1.0"?>

<mcptt-info>

<mcptt-Params>

<session-type>prearranged</session-type>

<mcptt-request-uri type="Normal">

<mcpttURI>sip:group123@mcpttoperator1.com></mcpttURI>

</mcptt-request-uri>

</mcptt-Params>

</mcptt-info>

EXAMPLE 6:	<mcptt-info> MIME body represented with the <mcptt-request-uri> encrypted:

Content-Type: application/vnd.3gpp.mcptt-info+xml

<?xml version="1.0"?>

<mcptt-info>

<mcptt-Params>

<session-type>prearranged</session-type>

<mcptt-request-uri type="Encrypted">

<EncryptedData xmlns='http://www.w3.org/2001/04/xmlenc#'

Type='http://www.w3.org/2001/04/xmlenc#Content'>

<EncryptionMethod Algorithm="http://www.w3.org/2009/xmlenc11#aes128-gcm"/>

<ds:KeyInfo>

<ds:KeyName>base64XpkId</KeyName>

</ds:KeyInfo>

<CipherData>

<CipherValue>A23B45C5657689090</CipherValue>

</CipherData>

</EncryptedData>

</mcptt-request-uri>

</mcptt-Params>

</mcptt-info>

EXAMPLE 7:	pidf+xml MIME body represented with clear URIs in attributes:

Content-Type: application/pidf+xml

<?xml version="1.0" encoding="UTF-8"?>

<presence entity="sip:somebody@mcptt.org">

<tuple id="acD4rhU87bK">

<status>

<affiliation group="sip:thegroup@mcptt.org"/>

</status>

</tuple>

</presence>

EXAMPLE 8:	pidf+xml MIME body represented with encrypted URIs in attributes:

Content-Type: application/pidf+xml

<?xml version="1.0" encoding="UTF-8"?>

<presence entity="sip:c4Hrt45XG8IohRFT67vfdr3V;iv=45RtfVgHY23k8Ihy;xpk-id=b7UJv9;alg=128-aes-gcm@mc1-encryption.3gppnetwork.org">

<tuple id="acD4rhU87bK">

<status>

<affiliation group="sip:98yudFG45tx_89TYGedb4ujF ;iv=FGD567kjhfH7d4-D;key-id=eV9kl7;alg=128-aes-gcm@mc1-encryption.3gppnetwork.org"/>

</status>

</tuple>

</presence>

## 4.9	Pre-established session

When establishing a pre-established session, the MCPTT client negotiates the media parameters, including establishing IP addresses and ports using interactive connectivity establishment (ICE) as specified in IETF RFC 5245 [17] with the participating MCPTT function, prior to using the pre-established session for establishing MCPTT sessions with other MCPTT users. The procedures for establishing, modifying and releasing a pre-established session are defined in clause 8.

The pre-established session can later be used in MCPTT calls. This avoids the need to negotiate media parameters (including evaluating ICE candidates) and reserving bearer resources during the MCPTT call establishment that results in delayed MCPTT call establishment.

The use of pre-established session on the origination side is compatible with the use of on demand session on the termination side. The use of pre-established session on the termination side is compatible with the use of on demand session on the origination side.

The MCPTT client procedures for:

-	leaving a MCPTT session using a pre-established session that was initiated by the MCPTT client are defined in subclause 6.2.4.2;

-	releasing a MCPTT session using a pre-established session that was initiated by the MCPTT client are defined in subclause 6.2.5.2;

-	establishing a pre-arranged group call using a pre-established session are defined in subclause 10.1.1.2.2;

-	rejoining a pre-arranged group call using a pre-established session are defined in subclause 10.1.1.2.4.2;

-	joining a chat MCPTT group session using a pre-established session are defined in subclause 10.1.2.2.2;

-	establishing a private call using a pre-established session are defined in subclause 11.1.1.2.2; and

-	releasing a private call using a pre-established session are defined in subclause 11.1.3.1.2.

The participating MCPTT function procedures for:

-	establishing a MCPTT session using automatic commencement mode are defined in subclause 6.3.2.2.5.3;

-	establishing a MCPTT session using manual commencement mode are defined in subclause 6.3.2.2.6.3;

-	releasing a MCPTT session using a pre-established session are defined in subclause 6.3.2.2.8.2;

-	establishing a pre-arranged group call using a pre-established session are defined in subclause 10.1.1.3.1.2;

-	releasing a pre-arranged group call using a pre-established session are defined in subclause 10.1.1.3.3.2;

-	rejoining a pre-arranged group call using a pre-established session are defined in subclause 10.1.1.3.5.2;

-	establishing a MCPTT group session using a pre-established session are defined in subclause 10.1.2.3.2;

-	originating a private call from a MCPTT client using a pre-established session are defined in subclause 11.1.1.3.1.2;

-	establishing a private call to a MCPTT client using a pre-established session are defined in subclause 11.1.1.3.2;

-	releasing a private call initiated by the served MCPTT client using a pre-established session are defined in subclause 11.1.3.2.1.2; and

-	releasing a private call initiated by the remote MCPTT client using a pre-established session are defined in subclause 11.1.3.2.2.2.

## 4.10	MCPTT client ID

The MCPTT client assigns the MCPTT client ID when the MCPTT client is used for the first time. The MCPTT client generates the MCPTT client ID as specified in subclause 4.2 of IETF RFC 4122 [67].

The MCPTT client preserves the MCPTT client ID:

-	while the MCPTT client is SIP registered as specified in 3GPP TS 24.229 [4];

-	while the MCPTT client is not SIP registered as specified in 3GPP TS 24.229 [4] and the UE serving the MCPTT client is switched on;

-	while the UE serving the MCPTT client is switched off; and

-	while the UE serving the MCPTT client is power-cycled.

NOTE:	MCPTT client ID is not preserved when the UE is reset to factory settings.

## 4.11	Off-network MCPTT

Off-network services are available for the user if the value of "/<x>/<x>/OffNetwork/Authorised" leaf node present in user profile as specified in 3GPP TS 24.383 [45] is set to "true".

# 5	Functional entities

## 5.1	Introduction

This clause associates the functional entities with the MCPTT roles described in the stage 2 architecture document (see 3GPP TS 23.179 [3]).

## 5.2	MCPTT client

To be compliant with the procedures in the present document, an MCPTT client shall:

-	act as the user agent for all MCPTT application transactions (e.g. initiation of a group call); and

-	support handling of the MCPTT client ID as described in subclause 4.10.

To be compliant with the on-network procedures in the present document, an MCPTT client shall:

-	support the MCPTT client on-network procedures defined in 3GPP TS 23.179 [3];

-	support the GCS UE procedures defined in 3GPP TS 23.468 [57] for unicast delivery, MBMS delivery and service continuity;

-	act as a SIP UA as defined in 3GPP TS 24.229 [4];

-	generate SDP offer and SDP answer in accordance with 3GPP TS 24.229 [4] and subclause 6.2;

-	act as a floor participant responsible for floor requests and implement the on-network procedures for floor requests as specified in 3GPP TS 24.380 [5];

-	for registration and service authorisation, implement the procedures specified in subclause 7.2;

-	for pre-established sessions, implement the procedures specified in subclause 8.2.1, subclause 8.3.1, subclause 8.4.1, and the procedures specified in 3GPP TS 24.380 [5];

-	for affiliation, implement the procedures specified in subclause 9.2;

-	for group call functionality (including broadcast, emergency and imminent peril), implement the MCPTT client procedures specified in subclause 10.1; and

-	for private call functionality (including emergency), implement the MCPTT client procedures specified in subclause 11.1;

-	for emergency alert, implement the procedures specified in subclause 12.1;

-	for location reporting, implement the procedures specified in subclause 13.3; and

-	for MBMS transmission usage, implement the procedures in subclause 14.3.

To be compliant with the off-network procedures in the present document, an MCPTT client shall:

-	support the off-network procedures defined in 3GPP TS 23.179 [3];

-	support the MCPTT off-network protocol (MONP) defined in clause 15;

-	act as a floor participant for floor requests and implement the off-network procedures for floor requests as specified in 3GPP TS 24.380 [5];

-	act as a floor control server providing distributed floor control and implement the off-network procedures for floor control as specified in 3GPP TS 24.380 [5];

-	implement the procedures for ProSe direct discovery for public safety use as specified in 3GPP TS 24.334 [28];

-	implement the procedures for one-to-one ProSe direct communication for Public Safety use as specified in 3GPP TS 24.334 [28];

-	for group call functionality (including emergency and imminent peril), implement the MCPTT client procedures specified in subclause 10.2;

-	for broadcast group call functionality implement the procedures specified in subclause 10.3; and

-	for private call functionality (including emergency), implement the MCPTT client procedures specified in subclause 11.2.

To be compliant with the service continuity procedures in the present document, an MCPTT client shall:

-	implement the registration requirements for service continuity as specified in subclause 7.2.1; and

-	implement the procedures specified in clause 14A.

To be compliant with the on-network and off-network procedures in the present document requiring end-to-end private call security key distribution, an MCPTT client shall support the procedures specified in 3GPP TS 33.179 [46].

To be compliant with the procedures for confidentiality protection of XML elements in the present document, the MCPTT client shall implement the procedures specified in subclause 6.6.2.

To be compliant with the procedures for integrity protection of XML MIME bodies in the present document, the MCPTT client shall implement the procedures specified in subclause 6.6.3.

## 5.3	MCPTT server

### 5.3.1	General

An MCPTT server can perform the controlling role for group calls and private calls as defined in 3GPP TS 23.179 [3].

An MCPTT server can perform the participating role for group calls and private calls as defined in 3GPP TS 23.179 [3].

An MCPTT server can perform a non-controlling role for temporary group calls involving groups from multiple MCPTT systems as specified in 3GPP TS 23.179 [3].

An MCPTT server can perform a non-controlling role for temporary group calls involving groups only from the primary MCPTT system.

An MCPTT server performing the participating role can serve an originating MCPTT user.

An MCPTT server performing the participating role can serve a terminating MCPTT user.

The same MCPTT server can perform the participating role and controlling role for the same group session.

The same MCPTT server can perform the participating role and non-controlling role for the same group session.

When referring to the procedures in the present document for the MCPTT server acting in a participating role for the served user, the term, "participating MCPTT function" is used.

When referring to the procedures in the present document for the MCPTT server acting in a controlling role for the served user, the term "controlling MCPTT function" is used.

When referring to the procedures in the present document for the MCPTT server acting in a non-controlling role for a group call, the term "non-controlling MCPTT function of an MCPTT group" is used.

To be compliant with the procedures in the present document, an MCPTT server shall:

-	support the MCPTT server procedures defined in 3GPP TS 23.179 [3];

-	implement the role of an AS performing 3rd party call control acting as a routing B2BUA as defined in 3GPP TS 24.229 [4];

-	support the GCS AS procedures defined in 3GPP TS 23.468 [57] for unicast delivery, MBMS delivery and service continuity;

-	generate SDP offer and SDP answer in accordance with 3GPP TS 24.229 [4] and subclause 6.3;

-	implement the role of a centralised floor control server and implement the on-network procedures for floor control as specified in 3GPP TS 24.380 [5];

-	for registration and service authorisation, implement the procedures specified in subclause 7.3;

-	for pre-established sessions, implement the procedures specified in subclause 8.2.2, subclause 8.3.2, subclause 8.4.2 and the procedures specified in 3GPP TS 24.380 [5];

-	for affiliation, implement the procedures specified in subclause 9.2.2;

-	for group call functionality (including broadcast, emergency and imminent peril), implement the MCPTT server procedures specified in subclause 10.1;

-	for private call functionality (including emergency), implement the MCPTT server procedures specified in subclause 11.1; and

-	for priority sharing, implement the MCPTT server procedures in subclause 6.7.

To be compliant with the procedures in the present document requiring the distribution of private call keying material between MCPTT clients as specified in 3GPP TS 33.179 [46], an MCPTT server shall ensure that the keying material is copied from incoming SIP messages into the outgoing SIP messages.

To be compliant with the procedures for confidentiality protection of XML elements in the present document, the MCPTT server shall implement the procedures specified in subclause 6.6.2.

To be compliant with the procedures for integrity protection of XML MIME bodies in the present document, the MCPTT server shall implement the procedures specified in subclause 6.6.3.

### 5.3.2	Functional connectivity models

The following figures give an overview of the connectivity between the different functions of the MCPTT server as described in subclause 5.3.1.

NOTE:	Separate boxes are shown for each of the functions of the MCPTT server. In each MCPTT system, these functions can be physically combined into one MCPTT server or can be implemented on more than one MCPTT server. For example, there could be an instantiation of an MCPTT server that only serves as a controlling MCPTT function, but not as a participating MCPTT function for any MCPTT clients. When an MCPTT server supports more than one function, then sending requests from one function to another does not incur a traversal of the underlying IMS SIP core network.

Figure 5.3.2-1 shows the basic functions of the MCPTT server when operating within the primary MCPTT system.

Figure 5.3.2-1: Functions of the MCPTT server in the primary MCPTT system

Figure 5.3.2-2 shows the use of the non-controlling MCPTT function of an MCPTT group within the primary MCPTT system. This can occur due to group re-grouping of groups within the same MCPTT system, where the MCPTT server(s) of one or more of the constituent groups are not controlled by the same controlling MCPTT function as that of the temporary group. The non-controlling MCPTT function of an MCPTT group either provide the identities of the users of the group to the controlling MCPTT function, or the non-controlling MCPTT function of an MCPTT group can invite the users of the group on behalf of the controlling MCPTT function.

Figure 5.3.2-2: The non-controlling function operating in the primary MCPTT system

Figure 5.3.2-3 shows the roles of the MCPTT server in a mutual aid relationship between a primary MCPTT system and a partner MCPTT system. Here, the controlling MCPTT function is in the primary MCPTT system and the called user is homed in a partner MCPTT system.

Figure 5.3.2-3: Mutual aid relationship between the primary MCPTT system and a partner MCPTT system with the controlling MCPTT function in the primary MCPTT system

Figure 5.3.2-4 shows the roles of the MCPTT server in a mutual aid relationship between a primary MCPTT system and a partner MCPTT system. Here, the controlling MCPTT function is in the partner MCPTT system.

Figure 5.3.2-4: Mutual aid relationship between the primary MCPTT system and a partner MCPTT system with the controlling MCPTT function in the partner MCPTT system

Figure 5.3.2-5 shows the roles of the MCPTT server in a mutual aid relationship between a primary MCPTT system and a partner MCPTT with the use of a non-controlling MCPTT function of an MCPTT group within the partner MCPTT system. This can occur due to group re-grouping where the MCPTT server(s) of one or more of the constituent groups are homed on the partner system. If the primary MCPTT system and partner MCPTT system operate in a trusted mutual aid relationship, then the non-controlling MCPTT function of an MCPTT group can provide the identities of the users of the group to the controlling MCPTT function. If the primary MCPTT system and partner MCPTT system operate in an untrusted mutual aid relationship, then the non-controlling MCPTT function of an MCPTT group invites the users of the group on behalf of the controlling MCPTT function.

Figure 5.3.2-5: Mutual aid relationship between the primary MCPTT system and a partner MCPTT system involving the use of a non-controlling MCPTT function of an MCPTT group in the partner MCPTT system

Figure 5.3.2-6 illustrates a functional connectivity model involving multiple partner systems where the partner system that owns the group does not home any of the group members.

Figure 5.3.2-6: : Mutual aid relationship between the primary MCPTT system and more than one partner MCPTT system

Other functional connectivity models can exist.

5.4	MCPTT UE-to-network relay

To be compliant with the procedures in the present document for service continuity, an MCPTT UE- to-network relay shall support the UE-to-network relay procedures as specified in 3GPP TS 24.334 [28] and 3GPP TS 23.179 [3].

# 6	Common procedures

## 6.1	Introduction

This clause describes the common procedures for each functional entity as specified.

## 6.2	MCPTT client procedures

### 6.2.1	SDP offer generation

The SDP offer shall contain only one SDP media-level section for MCPTT speech according to 3GPP TS 24.229 [4] and, if floor control shall be used during the session, shall contain one SDP media-level section for a media-floor control entity according to 3GPP TS 24.380 [5].

When composing an SDP offer according to 3GPP TS 24.229 [4] the MCPTT client:

1)	shall set the IP address of the MCPTT client for the offered MCPTT speech media stream and, if floor control shall be used, for the offered media-floor control entity;

NOTE:	If the MCPTT client is behind a NAT the IP address and port included in the SDP offer can be a different IP address and port than the actual IP address and port of the MCPTT client depending on the NAT traversal method used by the SIP/IP Core.

2)	shall include an "m=audio" media-level section for the MCPTT media stream consisting of:

a)	the port number for the media stream selected; and

b)	the codec(s) and media parameters and attributes with the following clarification:

i)	if the MCPTT client is initiating a call to a group identity;

ii)	if the <preferred-voice-encodings> element is present in the group document retrieved by the group management client as specified in 3GPP TS 24.381 [31] containing an <encoding> element with a "name" attribute; and

iii)	if the MCPTT client supports the encoding name indicated in the value of the "name" attribute;

then the MCPTT client:

i)	shall insert the value of the "name" attribute in the <encoding name> field of the "a=rtpmap" attribute as defined in IETF RFC 4566 [12]; and

c)	"i=" field set to "speech" according to 3GPP TS 24.229 [4];

3)	if floor control shall be used during the session, shall include an "m=application" media-level section as specified in 3GPP TS 24.380 [5] clause 12 for a media-floor control entity, consisting of:

a)	the port number for the media-floor control entity selected as specified in 3GPP TS 24.380 [5]; and

b)	the 'fmtp' attributes as specified in 3GPP TS 24.380 [5] clause 14; and

4)	if end-to-end security is required for a private call and the SDP offer is not for establishing a pre-established session, shall include the MIKEY-SAKKE I_MESSAGE in an "a=key-mgmt" attribute as a "mikey" attribute value in the SDP offer as specified in IETF RFC 4567 [47].

### 6.2.2	SDP answer generation

When the MCPTT client receives an initial SDP offer for an MCPTT session, the MCPTT client shall process the SDP offer and shall compose an SDP answer according to 3GPP TS 24.229 [4].

When composing an SDP answer, the MCPTT client:

1)	shall accept the MCPTT speech media stream in the SDP offer;

2)	shall set the IP address of the MCPTT client for the accepted MCPTT speech media stream and, if included in the SDP offer, for the accepted media-floor control entity;

NOTE:	If the MCPTT client is behind a NAT the IP address and port included in the SDP answer can be a different IP address and port than the actual IP address and port of the MCPTT client depending on the NAT traversal method used by the SIP/IP Core.

3)	shall include an "m=audio" media-level section for the accepted MCPTT speech media stream consisting of:

a)	the port number for the media stream;

b)	media-level attributes as specified in 3GPP TS 24.229 [4]; and

c)	"i=" field set to "speech" according to 3GPP TS 24.229 [4]; and

4)	if included in the SDP offer, shall include the media-level section of the offered media-floor control entity consisting of:

a)	an "m=application" media-level section as specified in 3GPP TS 24.380 [5] clause 12; and

b)	'fmtp' attributes as specified in 3GPP TS 24.380 [5] clause 14.

### 6.2.3	Commencement modes

#### 6.2.3.1	Automatic commencement mode

##### 6.2.3.1.1	Automatic commencement mode for private calls

When performing the automatic commencement mode procedures, the MCPTT client:

1)	shall accept the SIP INVITE request and generate a SIP 200 (OK) response according to rules and procedures of 3GPP TS 24.229 [4];

2)	shall include the option tag "timer" in a Require header field of the SIP 200 (OK) response;

3)	shall include the g.3gpp.mcptt media feature tag in the Contact header field of the SIP 200 (OK) response;

4)	shall include the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" in the Contact header field of the SIP 200 (OK) response;

5)	shall include the Session-Expires header field in the SIP 200 (OK) response and start the SIP session timer according to IETF RFC 4028 [7]. The "refresher" parameter in the Session-Expires header field shall be set to "uas";

6)	shall, if the incoming SIP INVITE request contains a Replaces header field, include in the SDP answer in the SIP 200 (OK) response to the SDP offer the parameters used for the pre-established session identified by the contents of the Replaces header field;

7)	shall, if the incoming SIP INVITE request does not contain a Replaces header field, include an SDP answer in the SIP 200 (OK) response to the SDP offer in the incoming SIP INVITE request according to 3GPP TS 24.229 [4] with the clarifications given in subclause 6.2.2;

NOTE:	In the case of a new emergency call where the terminating client is using a pre-established session, the SIP INVITE request containing a Replaces header is used to replace the pre-established session.

8)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcptt-called-party-id> element set to the MCPTT ID of the called MCPTT user, in the SIP 200 (OK) response;

9)	shall send the SIP 200 (OK) response towards the MCPTT server according to rules and procedures of 3GPP TS 24.229 [4];

10)	shall, if the incoming SIP INVITE request contains a Replaces header field, release the pre-established session identified by the contents of the Replaces header field; and

11)	shall interact with the media plane as specified in 3GPP TS 24.380 [5] subclause 6.2.

When NAT traversal is supported by the MCPTT client and when the MCPTT client is behind a NAT, generation of SIP responses is done as specified in this subclause and as specified in IETF RFC 5626 [15].

##### 6.2.3.1.2	Automatic commencement mode for group calls

When performing the automatic commencement mode procedures, the MCPTT client shall follow the procedures in subclause 6.2.3.1.1 with the following clarification:

-	The MCPTT client may include a P-Answer-State header field with the value "Confirmed" as specified in IETF RFC 4964 [34] in the SIP 200 (OK) response.

#### 6.2.3.2	Manual commencement mode

##### 6.2.3.2.1	Manual commencement mode for private calls

When performing the manual commencement mode procedures:

1)	if the MCPTT user declines the MCPTT session invitation the MCPTT client shall send a SIP 480 (Temporarily Unavailable) response towards the MCPTT server with the warning text set to: "110 user declined the call invitation" in a Warning header field as specified in subclause 4.4, and not continue with the rest of the steps in this subclause.

The MCPTT client:

1)	shall accept the SIP INVITE request and generate a SIP 180 (Ringing) response according to rules and procedures of 3GPP TS 24.229 [4];

2)	shall include the option tag "timer" in a Require header field of the SIP 180 (Ringing) response;

3)	shall include the g.3gpp.mcptt media feature tag in the Contact header field of the SIP 180 (Ringing) response;

4)	shall include the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" in the Contact header field of the SIP 180 (Ringing) response;

5)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcptt-called-party-id> element set to the MCPTT ID of the called MCPTT user, in the SIP 180 (Ringing) response; and

6)	shall send the SIP 180 (Ringing) response to the MCPTT server.

When sending the SIP 200 (OK) response to the incoming SIP INVITE request, the MCPTT client shall follow the procedures in subclause 6.2.3.1.1.

When NAT traversal is supported by the MCPTT client and when the MCPTT client is behind a NAT, generation of SIP responses is done as specified in this subclause and as specified in IETF RFC 5626 [15].

##### 6.2.3.2.2	Manual commencement mode for group calls

When performing the manual commencement mode procedures:

1)	the terminating MCPTT client may automatically generate a SIP 183 (Session Progress) in accordance with 3GPP TS 24.229 [4], prior to the MCPTT user's acknowledgement; and

2)	if the MCPTT user declines the MCPTT session invitation the MCPTT client shall send a SIP 480 (Temporarily Unavailable) response towards the MCPTT server with the warning text set to: "110 user declined the call invitation" in a Warning header field as specified in subclause 4.4, and not continue with the rest of the steps in this subclause.

When generating a SIP 183 (Session Progress) response, the MCPTT client:

1)	shall include the following in the Contact header field:

a)	the g.3gpp.mcptt media feature tag; and

b)	the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt"; and

2)	may include a P-Answer-State header field with the value "Unconfirmed" as specified in IETF RFC 4964 [34];

When sending the SIP 200 (OK) response to the incoming SIP INVITE request, the MCPTT client shall follow the procedures in subclause 6.2.3.1.2.

When NAT traversal is supported by the MCPTT client and when the MCPTT client is behind a NAT, generation of SIP responses is done as specified in this subclause and as specified in IETF RFC 5626 [15].

### 6.2.4	Leaving an MCPTT session initiated by MCPTT client

#### 6.2.4.1	On-demand session case

Upon receiving a request from an MCPTT user to leave an MCPTT session established using on-demand session signalling, the MCPTT client:

1)	shall interact with the media plane as specified in 3GPP TS 24.380 [5];

2)	shall generate a SIP BYE request according to 3GPP TS 24.229 [4];

3)	shall set the Request-URI to the MCPTT session identity to leave; and

4)	shall send a SIP BYE request towards MCPTT server according to 3GPP TS 24.229 [4].

Upon receiving a SIP 200 (OK) response to the SIP BYE request, the MCPTT client shall interact with the media plane as specified in 3GPP TS 24.380 [5].

#### 6.2.4.2	Pre-established session case

Upon receiving a request from an MCPTT user to leave an MCPTT session within a pre-established session, the MCPTT client:

1)	shall interact with the media plane as specified in 3GPP TS 24.380 [5];

2)	shall generate an initial SIP REFER request outside a dialog in accordance with the procedures specified in 3GPP TS 24.229 [4], IETF RFC 4488 [22] and IETF RFC 3515 [25] as updated by IETF RFC 6665 [26] and IETF RFC 7647 [27];

3)	shall set the Request-URI of the SIP REFER request to the public service identity identifying the pre-established session on the MCPTT server serving the MCPTT user;

4)	shall include the Refer-Sub header field with value "false" according to rules and procedures of IETF RFC 4488 [22];

5)	shall include the Supported header field with value "norefersub" according to rules and procedures of IETF RFC 4488 [22];

6)	shall set the Refer-To header field of the SIP REFER request to the MCPTT session identity to leave;

7)	shall include the "method" SIP URI parameter with the value "BYE" in the URI in the Refer-To header field;

8)	shall include a Target-Dialog header field as specified in IETF RFC 4538 [23] identifying the pre-established session; and

9)	shall send the SIP REFER request according to 3GPP TS 24.229 [4].

Upon receiving a SIP 2xx response to the SIP REFER request, the MCPTT client shall interact with media plane as specified in 3GPP TS 24.380 [5].

### 6.2.5	Releasing an MCPTT session initiated by MCPTT client

#### 6.2.5.1	On-demand session case

When the MCPTT client wants to release an MCPTT session established using on-demand session signalling, the MCPTT client:

1)	shall interact with the media plane as specified in 3GPP TS 24.380 [5];

2)	shall generate a SIP BYE request according to 3GPP TS 24.229 [4];

3)	shall set the Request-URI to the MCPTT session identity to release; and

4)	shall send a SIP BYE request towards MCPTT server according to 3GPP TS 24.229 [4].

Upon receiving a SIP 200 (OK) response to the SIP BYE request, the MCPTT client shall interact with the media plane as specified in 3GPP TS 24.380 [5].

#### 6.2.5.2	Pre-established session case

When the MCPTT client wants to release an MCPTT session using a pre-established session, the MCPTT client:

1)	shall interact with the media plane as specified in 3GPP TS 24.380 [5];

2)	shall generate an initial SIP REFER request outside a dialog in accordance with the procedures specified in 3GPP TS 24.229 [4], IETF RFC 4488 [22] and IETF RFC 3515 [25] as updated by IETF RFC 6665 [26] and IETF RFC 7647 [27];

3)	shall set the Request-URI of the SIP REFER request to the public service identity identifying the pre-established session on the MCPTT server serving the MCPTT user;

4)	shall include the Refer-Sub header field with value "false" according to rules and procedures of IETF RFC 4488 [22];

5)	shall include the Supported header field with value "norefersub" according to rules and procedures of IETF RFC 4488 [22];

6)	shall set the Refer-To header field of the SIP REFER request to the MCPTT session identity to release;

7)	shall include the "method" SIP URI parameter with the value "BYE" in the URI in the Refer-To header field;

8)	shall include a Target-Dialog header field as specified in IETF RFC 4538 [23] identifying the pre-established session; and

9)	shall send the SIP REFER request according to 3GPP TS 24.229 [4].

Upon receiving a SIP 2xx response to the SIP REFER request, the MCPTT client shall interact with media plane as specified in 3GPP TS 24.380 [5].

### 6.2.6	Receiving an MCPTT session release request

Upon receiving a SIP BYE request, the MCPTT client:

1)	shall interact with the media plane as specified in 3GPP TS 24.380 [5]; and

2)	shall send SIP 200 (OK) response towards MCPTT server according to 3GPP TS 24.229 [4].

### 6.2.7	Receiving a SIP MESSAGE request

Upon receipt of a SIP MESSAGE request the MCPTT client shall handle the request in accordance with 3GPP TS 24.229 [4] and IETF RFC 3428 [33].

If the SIP MESSAGE request contains:

-	an application/ vnd.3gpp.mcptt-info+xml MIME body; and

-	a "text/plain" MIME body;

then the MCPTT client:

1)	shall validate the MCPTT ID in the <mcptt-request-uri> element of the application/vnd.3gpp.mcptt-info+xml MIME body; and

2)	if the validation of the MCPTT ID succeeds, shall render the message content in the text/plain MIME body to the MCPTT user identified by the MCPTT ID.

### 6.2.8	Priority call conditions

This subclause contains common procedures to be used for MCPTT emergency group calls and MCPTT imminent peril group calls.

#### 6.2.8.1	MCPTT emergency group call conditions

##### 6.2.8.1.1	SIP INVITE request or SIP REFER request for originating MCPTT emergency group calls

This subclause is referenced from other procedures.

When the MCPTT emergency state is set and this MCPTT user and MCPTT group are authorised to initiate MCPTT emergency group calls as determined by the procedures of subclause 6.2.8.1.8, the MCPTT client:

1)	shall include in the application/vnd.3gpp.mcptt-info+xml MIME body in the SIP INVITE request or SIP REFER request, an <emergency-ind> element set to "true" and if the MCPTT emergency group call state is set to "MEGC 1: emergency-gc-capable", shall set the MCPTT emergency group call state to "MEGC 2: emergency-call-requested";

2)	if the MCPTT user has also requested an MCPTT emergency alert to be sent and this is an authorised request for MCPTT emergency alert as determined by the procedures of subclause 6.2.8.1.6, and the MCPTT emergency alert state is set to "MEA 1: no-alert", shall:

a)	set the <alert-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body to "true" and set the MCPTT emergency alert state to "MEA 2: emergency-alert-confirm-pending"; and

b)	perform the procedures specified in subclause 6.2.9.1 for the MCPTT emergency alert trigger;

3)	if the MCPTT user has not requested an MCPTT emergency alert to be sent and the MCPTT emergency alert state is set to "MEA 1: no-alert", shall set the <alert-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body to "false"; and

4)	if the MCPTT client emergency group state of the group is set to a value other than "MEG 2: in-progress" set the MCPTT client emergency group state of the MCPTT group to "MEG 3: confirm-pending".

NOTE 1:	This is the case of an MCPTT user already being in the MCPTT emergency state it initiated previously while originating an MCPTT emergency group call or MCPTT emergency alert. All group calls the MCPTT user originates while in MCPTT emergency state will be MCPTT emergency group calls.

When the MCPTT emergency state is clear and the MCPTT emergency group call state is set to "MEGC 1: emergency-gc-capable" and the received SIP request contains an authorised request for MCPTT emergency group call as determined by the procedures of subclause 6.2.8.1.8, the MCPTT client shall set the MCPTT emergency state and perform the following actions:

1)	shall include in the application/vnd.3gpp.mcptt-info+xml MIME body in the SIP INVITE request or SIP REFER request an <emergency-ind> element set to "true" and set the MCPTT emergency group call state to "MEGC 2: emergency-call-requested" state;

2)	if the MCPTT user has also requested an MCPTT emergency alert to be sent and this is an authorised request for MCPTT emergency alert as determined by the procedures of subclause 6.2.8.1.6, shall:

a)	include in the application/vnd.3gpp.mcptt-info+xml MIME body the <alert-ind> element set to "true" and set the MCPTT emergency alert state to "MEA 2: emergency-alert-confirm-pending"; and

b)	perform the procedures specified in subclause 6.2.9.1 for the MCPTT emergency alert trigger;

3)	if the MCPTT user has not requested an MCPTT emergency alert to be sent, shall set the <alert-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body to "false"; and

4)	if the MCPTT client emergency group state of the group is set to a value other than "MEG 2: in-progress" shall set the MCPTT client emergency group state of the MCPTT group to "MEG 3: confirm-pending".

NOTE 2:	This is the case of an initial MCPTT emergency group call and optionally an MCPTT emergency alert being sent. As the MCPTT emergency state is not sent, there is no MCPTT emergency alert outstanding.

NOTE 3:	An MCPTT group call originated by an affiliated member of an MCPTT group which is in an in-progress emergency state (as tracked on the MCPTT client by the MCPTT client emergency group state) but is not in an MCPTT emergency state of their own will also be an MCPTT emergency group call. The <emergency-ind> and <alert-ind> elements of the application/vnd.3gpp.mcptt-info+xml MIME body do not need to be included in this case and hence no action needs to be taken in this subclause.

##### 6.2.8.1.2	Resource-Priority header field for MCPTT emergency group calls

This subclause is referenced from other procedures.

If the MCPTT emergency group call state is set to either "MEGC 2: emergency-call-requested" or "MEGC 3: emergency-call-granted" and this is an authorised request for an MCPTT emergency group call as determined by the procedures of subclause 6.2.8.1.8, or the MCPTT client emergency group state of the group is set to "MEG 2: in-progress", the MCPTT client shall include in the SIP INVITE request or SIP REFER request a Resource-Priority header field populated with the values for an MCPTT emergency group call as specified in subclause 6.2.8.1.15.

NOTE:	The MCPTT client ideally would not need to maintain knowledge of the in-progress emergency state of the group (as tracked on the MCPTT client by the MCPTT client emergency group state) but can use this knowledge to provide a Resource-Priority header field set to emergency level priority, which starts the infrastructure priority adjustment process sooner than otherwise would be the case.

If this is an authorised request to cancel the MCPTT emergency group call as determined by the procedures of subclause 6.2.8.1.7, and the MCPTT client emergency group state of the group is "no-emergency" or "cancel-pending", the MCPTT client shall include in the SIP INVITE request or SIP REFER request a Resource-Priority header field populated with the values for a normal MCPTT group call as specified in subclause 6.2.8.1.15.

##### 6.2.8.1.3	SIP re-INVITE request for cancelling MCPTT in-progress emergency group state

This subclause is referenced from other procedures.

If the MCPTT emergency group call state is set to "MEGC 3: emergency-call-granted" and the MCPTT emergency alert state is set to "MEA 1: no-alert", the MCPTT client shall generate a SIP re-INVITE request according to 3GPP TS 24.229 [4] with the clarifications given below.

NOTE 1:	This procedure assumes that the calling procedure has verified that the MCPTT user has made an authorised request for cancelling MCPTT in-progress emergency group state of the group.

The MCPTT client:

1)	shall include in the SIP re-INVITE request an application/vnd.3gpp.mcptt-info+xml MIME body as defined in clause F.1 with the <emergency-ind> element set to "false";

2)	shall clear the MCPTT emergency state; and

3)	shall set MCPTT emergency group state of the MCPTT group to "MEG 3: cancel-pending"

NOTE 2:	This is the case of an MCPTT user who has initiated an MCPTT emergency group call and wants to cancel it.

If the MCPTT emergency group call state is set to "MEGC 3: emergency-call-granted" and the MCPTT emergency alert state is set to a value other than "MEA 1: no-alert" and the MCPTT user has indicated only the MCPTT emergency group call should be cancelled, the MCPTT client:

1)	shall include in the SIP re-INVITE request an application/vnd.3gpp.mcptt-info+xml MIME body as defined in clause F.1 with the <emergency-ind> element set to "false"; and

2)	shall set the MCPTT emergency group state of the MCPTT group to "MEG 3: cancel-pending".

NOTE 3:	This is the case of an MCPTT user has initiated both an MCPTT emergency group call and an MCPTT emergency alert and wishes to only cancel the MCPTT emergency group call. This leaves the MCPTT emergency state set.

If the MCPTT emergency group call state is set to "MEGC 3: emergency-call-granted" and the MCPTT emergency alert state is set to a value other than "MEA 1: no-alert" and the MCPTT user has indicated that the MCPTT emergency alert on the MCPTT group should be cancelled in addition to the MCPTT emergency group call, the MCPTT client:

1)	shall include in the SIP re-INVITE request an application/vnd.3gpp.mcptt-info+xml MIME body as defined in clause F.1 with the <emergency-ind> element set to "false";

2)	shall if this is an authorised request to cancel an MCPTT emergency alert as determined by the procedures of subclause 6.2.8.1.6:

a)	include in the application/vnd.3gpp.mcptt-info+xml MIME body an <alert-ind> element set to "false";

b)	 set the MCPTT emergency alert state to "MEA 4: Emergency-alert-cancel-pending"; and

c)	clear the MCPTT emergency state;

3)	should, if this is not an authorised request to cancel an MCPTT emergency alert as determined by the procedures of subclause 6.2.8.1.6, indicate to the MCPTT user that they are not authorised to cancel the MCPTT emergency alert; and

4)	shall set the MCPTT emergency group state of the MCPTT group to "MEG 3: cancel-pending".

NOTE 4:	This is the case of an MCPTT user that has initiated both an MCPTT emergency group call and an MCPTT emergency alert and wishes to cancel both.

##### 6.2.8.1.4	Receiving a SIP 2xx response to a SIP request for a priority call

In the procedures in this subclause, a priority group call refers to an MCPTT emergency group call or an MCPTT imminent peril group call.

On receiving a SIP 2xx response to a SIP request for a priority group call, the MCPTT client:

1)	if the MCPTT emergency group call state is set to "MEGC 2: emergency-call-requested" or "MEGC 3: emergency-call-granted":

a)	shall set the MCPTT client emergency group state of the group to "MEG 2: in-progress" if it was not already set;

b)	if the MCPTT emergency alert state is set to "MEA 2: emergency-alert-confirm-pending" and the SIP 2xx response to the SIP request for a priority group call does not contain a Warning header field as specified in subclause 4.4 with the warning text containing the mcptt-warn-code set to "149", shall set the MCPTT emergency alert state to "MEA 3: emergency-alert-initiated;

c)	shall set the MCPTT emergency group call state to "MEGC 3: emergency-call-granted"; and

d)	shall set the MCPTT imminent peril group call state to "MIGC 1: imminent-peril-capable" and the MCPTT imminent peril group state to "MIG 1: no-imminent-peril";

##### 6.2.8.1.5	Receiving a SIP 4xx response, SIP 5xx response or SIP 6xx response to a SIP request for a priority group call

In the procedures in this subclause, a priority group call refers to an MCPTT emergency group call or an MCPTT imminent peril group call.

Upon receiving a SIP 4xx response, SIP 5xx response or a SIP 6xx response to a SIP request for a priority group call the MCPTT client:

1)	if the MCPTT emergency group call state is set to "MEGC 2: emergency-call-requested" or "MEGC 3: emergency-call-granted":

a)	shall set the MCPTT emergency group call state to "MEGC 1: emergency-gc-capable";

b)	if the MCPTT client emergency group state of the group is "MEG 3: confirm-pending" shall set the MCPTT client emergency group state of the group to "MEG 1: no-emergency";

c)	if the SIP 4xx response, SIP 5xx response or SIP 6xx response contains an application/vnd.3gpp.mcptt-info+xml MIME body with an <alert-ind> element set to a value of "true", shall set the MCPTT emergency alert state to "MEA 3: emergency-alert-initiated";

NOTE 1:	This is the case where the MCPTT user has requested an MCPTT emergency alert in addition to the MCPTT emergency and the MCPTT emergency alert is granted.

d)	if the SIP 4xx response, SIP 5xx response or SIP 6xx response contains an application/vnd.3gpp.mcptt-info+xml MIME body with an <alert-ind> element set to a value of "false", shall set the MCPTT emergency alert state to "MEA 1: no-alert"; and

NOTE 2:	this is the case where the MCPTT user has requested an MCPTT emergency alert in addition to an MCPTT emergency call and the MCPTT emergency alert is not granted.

e)	otherwise if the SIP 4xx response, SIP 5xx response or SIP 6xx response does not contain an application/vnd.3gpp.mcptt-info+xml MIME body or if the MIME body does not include an <alert-ind> element, if the MCPTT emergency alert state is set to "MEA 2: emergency-alert-confirm-pending" shall set the MCPTT emergency alert state to "MEA 1: no-alert"; and

2)	if the MCPTT imminent peril group call state is set to "MIGC 2: imminent-peril-call-requested" or "MIGC 3: imminent-peril-call-granted":

a)	shall set the MCPTT imminent peril group state to "MIG 1: no-imminent-peril"; and

b)	shall set the MCPTT imminent peril group call state to "MIGC 1: imminent-peril-capable".

##### 6.2.8.1.6	Determining authorisation for initiating or cancelling an MCPTT emergency alert

If the MCPTT client receives a request from the MCPTT user to send an MCPTT emergency alert and:

1)	if the <allow-activate-emergency-alert> element of the <ruleset> element of the MCPTT user profile document identified by the MCPTT ID of the calling MCPTT user (see the MCPTT user profile document in 3GPP TS 24.384 [50]) is set to a value of "true"; and

2)	if the "entry-info" attribute of the <entry> element of the <Recipient> element contained within the <EmergencyAlert> element of the MCPTT user profile document (see the MCPTT user profile document in 3GPP TS 24.384 [50]) is set to a value of:

a)	"DedicatedGroup", and if the <uri-entry> element of the <entry> element of the <Recipient> element of the <EmergencyAlert> element of the <MCPTT-group-call> element of the MCPTT user profile document (see the MCPTT user profile document in 3GPP TS 24.384 [50]) contains the MCPTT group identity of the MCPTT group targeted by the calling MCPTT user; or

b)	"UseCurrentlySelectedGroup" and the <allow-MCPTT-emergency-alert> element of the <list-element> of the group document identified by the MCPTT group identity targeted for the emergency alert is set to a value of "true" as specified in 3GPP TS 24.381 [31];

then the MCPTT emergency alert request shall be considered to be an authorised request for an MCPTT emergency alert. In all other cases, it shall be considered to be an unauthorised request for an MCPTT emergency alert.

If the MCPTT client receives a request from the MCPTT user to cancel an MCPTT emergency alert to an MCPTT group, and if the <allow-cancel-emergency-alert> element of the <ruleset> element of the MCPTT user profile document identified by the MCPTT ID of the calling MCPTT user (see the MCPTT user profile document in 3GPP TS 24.384 [50]) is set to a value of "true", then the MCPTT emergency alert cancellation request shall be considered to be an authorised request to cancel an MCPTT emergency alert. In all other cases, it shall be considered to be an unauthorised request to cancel an MCPTT emergency alert.

##### 6.2.8.1.7	Determining authorisation for cancelling the in-progress emergency state of an MCPTT group

When the MCPTT client receives a request from the MCPTT user to cancel the in-progress emergency state of a group the MCPTT client and:

1)	if the <allow-cancel-group-emergency> element of the <ruleset> element of the MCPTT user profile document identified by the MCPTT ID of the calling MCPTT user (see the MCPTT user profile document in 3GPP TS 24.384 [50]) is set to a value of "true", then the in-progress emergency group state cancel request shall be considered to be an authorised request for in-progress emergency group state cancellation; or

2)	if the <allow-cancel-group-emergency> element of the <ruleset> element of the MCPTT user profile document identified by the MCPTT ID of the calling MCPTT user (see the MCPTT user profile document in 3GPP TS 24.384 [50]) is set to a value of "false", then the in-progress emergency group state cancel request shall be considered to be an unauthorised request for in-progress emergency group state cancellation.

##### 6.2.8.1.8	Determining authorisation for originating a priority group call

When the MCPTT client receives a request from the MCPTT user to originate an MCPTT emergency group call the MCPTT client shall check the following:

1)	if the <allow-emergency-group-call> element of the <ruleset> element of the MCPTT user profile document identified by the MCPTT ID of the calling user (see the MCPTT user profile document in 3GPP TS 24.384 [50]) is set to a value of "true" and

a)	if the "entry-info" attribute of the <entry> element of the <MCPTTGroupInitiation> element contained within the <EmergencyCall> element of the MCPTT user profile document (see the MCPTT user profile document in 3GPP TS 24.384 [50]) is set to a value of "DedicatedGroup" and if the <uri-entry> element of the <entry> element of the <MCPTTGroupInitiation> element contains the identity of the MCPTT group targeted by the calling MCPTT user; or

b)	if the "entry-info" attribute of the <entry> element of the <MCPTTGroupInitiation> element is set to a value of "UseCurrentlySelectedGroup";

then the MCPTT emergency group call request shall be considered to be an authorised request for an MCPTT emergency group call;

In all other cases, the request to originate an MCPTT emergency group call shall be considered to be an unauthorised request to originate an MCPTT emergency group call.

When the MCPTT client receives a request from the MCPTT user to originate an MCPTT imminent peril group call the MCPTT client shall check the following:

1	if the <allow-imminent-peril-call> element of <ruleset> element of the MCPTT user profile document identified by the MCPTT ID of the calling user (see the MCPTT user profile document in 3GPP TS 24.384 [50]) is set to a value of "true" and:

a)	if the "entry-info" attribute of the <entry> element of the <MCPTTGroupInitiation> element contained within the <ImminentPerilCall> element of the MCPTT user profile document (see the MCPTT user profile document in 3GPP TS 24.384 [50]) is set to a value of "DedicatedGroup" and if the <MCPTTGroupInitiation> element contains the identity of the MCPTT group targeted by the calling MCPTT user; or

b)	if the "entry-info" attribute of the <entry> element of the <MCPTTGroupInitiation> element is set to a value of "UseCurrentlySelectedGroup";

then the MCPTT imminent peril group call request shall be considered to be an authorised request for an MCPTT imminent peril group call;

In all other cases, the request to originate an MCPTT imminent peril group call shall be considered to be an unauthorised request to originate an MCPTT imminent peril group call.

##### 6.2.8.1.9	SIP request for originating MCPTT imminent peril group calls

This subclause is referenced from other procedures.

When the MCPTT client receives a request from the MCPTT user to originate an MCPTT imminent peril group call, and this is an authorised request for an MCPTT imminent peril group call as determined by the procedures of subclause 6.2.8.1.8, the MCPTT client:

1)	if the MCPTT client imminent peril group state is set to "MIGC 1: imminent-peril-capable" and the in-progress emergency state of the group is set to a value of "false":

a)	shall include in the SIP request a MIME mcpttinfo body as defined in Annex F.1 with the <imminentperil-ind> element set to "true" and set the MCPTT emergency group call state to "MIGC 2: imminent-peril-call-requested" state; and

b)	if the MCPTT client imminent peril group state of the group is set to a value other than "MIG 2: in-progress" shall set the MCPTT client emergency group state of the MCPTT group to "MIG 3: confirm-pending".

NOTE:	An MCPTT group call originated by an affiliated member of an MCPTT group which is in an in-progress imminent peril state (as tracked on the MCPTT client by the MCPTT client imminent peril group state) will also have the priority associated with MCPTT imminent peril group calls. The <imminentperil-ind> element of the MIME mcpttinfo body does not need to be included in this case, nor do any state changes result and hence no action needs to be taken in this subclause.

##### 6.2.8.1.10	Determining authorisation for cancelling an imminent peril group call

When the MCPTT client receives a request from the MCPTT user to cancel an MCPTT imminent peril group call the MCPTT client shall:

1)	if the <allow-cancel-imminent-peril> element of the <ruleset> element of the MCPTT user profile document identified by the MCPTT ID of the calling user (see the MCPTT user profile document in 3GPP TS 24.384 [50]) is set to a value of "true" the MCPTT imminent peril call cancellation request shall be considered to be an authorised request to cancel the MCPTT imminent peril group call; or

2)	if the <allow-cancel-imminent-peril> element of the <ruleset> element of the MCPTT user profile document identified by the MCPTT ID of the calling user (see the MCPTT user profile document in 3GPP TS 24.384 [50]) is set to a value of "false" the MCPTT imminent peril call cancellation request shall be considered to be an unauthorised request to cancel the MCPTT imminent peril group call.

##### 6.2.8.1.11	SIP re-INVITE request for cancelling MCPTT in-progress imminent peril group state

This subclause is referenced from other procedures.

If the MCPTT imminent peril group call state is set to "MIGC 3: imminent-peril-call-granted" or the MCPTT imminent peril group state of the MCPTT group is set to "MIG 2: in-progress", the MCPTT client shall generate a SIP re-INVITE request according to 3GPP TS 24.229 [4] with the clarifications given below.

NOTE 1:	This procedure assumes that the calling procedure has verified that the MCPTT user has made an authorised request for cancelling the in-progress imminent peril group state of the group.

The MCPTT client:

1)	shall include in the SIP re-INVITE request an application/vnd.3gpp.mcptt-info+xml MIME body as defined in clause F.1 with the <imminentperil-ind> element set to "false"; and

2)	shall set MCPTT imminent peril group state of the MCPTT group to "MIG 4: cancel-pending".

NOTE 2:	This is the case of an MCPTT user who has initiated an MCPTT imminent peril group call and wants to cancel it, or another authorised member of the group who wishes to cancel the in-progress imminent peril state of the group.

##### 6.2.8.1.12	Resource-Priority header field for MCPTT imminent peril group calls

This subclause is referenced from other procedures.

When the MCPTT imminent peril group call state is set "MIGC 2: imminent-peril-call-requested" or "MIGC 3: imminent-peril-call-granted" and this MCPTT user and group is authorised to originate MCPTT imminent peril group calls as determined by the procedures of subclause 6.2.8.1.8, or the MCPTT client imminent peril state of the group is set to "MIG 2: in-progress", the MCPTT client:

1)	shall include in the SIP INVITE request or SIP REFER request a Resource-Priority header field populated with the values for an MCPTT imminent peril group call as specified in subclause 6.2.8.1.15.

NOTE:	The MCPTT client ideally would not need to maintain knowledge of the in-progress imminent peril state of the group (as tracked on the MCPTT client by the MCPTT client imminent peril group state) but can use this knowledge to provide a Resource-Priority header field set to imminent peril level priority, which starts the infrastructure priority adjustment process sooner than otherwise would be the case.

When the MCPTT imminent peril group call state is set to "MIGC 1: imminent-peril-gc-capable" and the MCPTT user is authorised to cancel MCPTT imminent peril group calls as determined by the procedures of subclause 6.2.8.1.10, or the MCPTT client imminent peril group state of the group is "MIG 1: no-imminent-peril" or "MIG 4: cancel-pending", the MCPTT client:

1)	shall include in the SIP INVITE request or SIP REFER request a Resource-Priority header field populated with the values for a normal MCPTT group call as specified in subclause 6.2.8.1.15.

##### 6.2.8.1.13	Receiving a SIP INFO request in the dialog of a SIP request for a priority group call

This subclause is referenced from other procedures.

Upon receiving a SIP INFO request within the dialog of the SIP request for a priority group call:

-	with the Info-Package header field containing the g.3gpp.mcptt-info package name;

-	with the application/vnd.3gpp.mcptt-info+xml MIME body associated with the info package according to IETF RFC 6086 [54]; and

-	with one or more of the <alert-ind>, <imminentperil> ind and <emergency-ind> elements set in the application/vnd.3gpp.mcptt-info+xml MIME body;

the MCPTT client:

1)	if the MCPTT emergency group call state is set to "MEGC 3: emergency-call-granted":

a)	if the MCPTT emergency alert state is set to "MEA 2: emergency-alert-confirm-pending":

i)	if the <alert-ind> element is set to a value of "false", shall set the MCPTT emergency alert state to "MEA 1: no-alert"; and

ii)	if the <alert-ind> element is set to a value of "true", shall set the MCPTT emergency alert state to "MEA 3: emergency-alert-initiated";

2)	if the MCPTT imminent peril group call state is set to "MIGC 2: imminent-peril-call-requested" or "MIGC 3: imminent-peril-call-granted":

a)	if the <imminentperil-ind> element set to a value of "true" or if there is no <imminentperil-ind> element included, shall:

i)	set the MCPTT imminent peril group call state to "MIGC 3: imminent-peril-call-granted"; and

ii)	set the MCPTT imminent peril group state to "MIG 2: in-progress"; and

b)	if the <imminentperil-ind> element set to a value of "false" and an <emergency-ind> element set to a value of "true", shall:

i)	set the MCPTT imminent peril group state to "MIG 1: no-imminent-peril";

ii)	set the MCPTT imminent peril group call state to "MIGC 1: imminent-peril-capable"; and

iii)	set the MCPTT client emergency group state of the group to "MEG 2: in-progress"; and

NOTE 1:	This is the case of an MCPTT client attempting to make an imminent peril group call when the group is in an in-progress emergency group state. The MCPTT client will then receive a notification that the imminent peril call request was denied, however they will be participating at the emergency level priority of the group. This could occur for example when an MCPTT client requests an imminent peril call to a group that they are not currently affiliated with.

NOTE 2:	the MCPTT client emergency group state above is the MCPTT client's view of the in-progress emergency state of the group.

3)	if the SIP request for a priority group call sent by the MCPTT client did not contain an <originated-by> element and if the MCPTT emergency alert state is set to "MEA 4: Emergency-alert-cancel-pending":

a)	if the <alert-ind> element contained in the SIP INFO request is set to a value of "true", shall set the MCPTT emergency alert state to "MEA 3: emergency-alert-initiated"; and

b)	if the <alert-ind> element contained in the SIP INFO request is set to a value of "false", shall set the MCPTT emergency alert state to "MEA 1: no-alert".

##### 6.2.8.1.14	SIP re-INVITE request for cancelling the in-progress emergency group state of a group by a third-party

This subclause is referenced from other procedures.

Upon receiving an authorised request to cancel an in-progress emergency group state of a group as determined by the procedures of subclause 6.2.8.1.7 from an MCPTT user, the MCPTT client shall generate a SIP re-INVITE request according to 3GPP TS 24.229 [4] with the clarifications given below.

The MCPTT client:

1)	shall include in the SIP re-INVITE request an application/vnd.3gpp.mcptt-info+xml MIME body as defined in clause F.1 with the <emergency-ind> element set to "false";

2)	shall set MCPTT emergency group state of the MCPTT group to "MEG 3: cancel-pending"; and

3)	if the MCPTT user has indicated that an MCPTT emergency alert on the MCPTT group originated by another MCPTT user should be cancelled and this is an authorised request for an MCPTT emergency alert cancellation as determined by the procedures of subclause 6.2.8.1.6:

a)	shall include in the application/vnd.3gpp.mcptt-info+xml MIME body an <alert-ind> element set a value of "false"; and

b)	shall include in the application/vnd.3gpp.mcptt-info+xml MIME body an <originated-by> element set to the MCPTT ID of the MCPTT user who originated the MCPTT emergency alert.

NOTE:	When an MCPTT emergency alert is cancelled by a MCPTT user other than its originator, the <originated-by> element is needed to identify which MCPTT emergency alert is being cancelled, as more than one MCPTT user could have originated emergency alerts to the same group.

##### 6.2.8.1.15	Retrieving Resource-Priority header field values

This subclause is referenced from other procedures.

When determining the Resource-Priority header field namespace and priority values as specified in draft-holmberg-dispatch-mcptt-rp-namespace [48] for an MCPTT emergency group call or MCPTT emergency private call the MCPTT client:

1)	shall retrieve the value of the <resource-priority-namespace> element contained in the <emergency-resource-priority> element contained in the <OnNetwork> element of the MCPTT service configuration document (see the service configuration document in 3GPP TS 24.384 [50]); and

2)	shall retrieve the value of the <resource-priority-priority> element contained in the <emergency-resource-priority> element contained in the <OnNetwork> element of the MCPTT service configuration document (see the service configuration document in 3GPP TS 24.384 [50]).

When determining the Resource-Priority header field namespace and priority values as specified in draft-holmberg-dispatch-mcptt-rp-namespace [48] for an MCPTT imminent peril group call the MCPTT client:

1)	shall retrieve the value of the <resource-priority-namespace> element contained in the <imminent-peril-resource-priority> element contained in the <OnNetwork> element of the MCPTT service configuration document (see the service configuration document in 3GPP TS 24.384 [50]); and

2)	shall retrieve the value of the <resource-priority-priority> element contained in the <imminent-peril-resource-priority> element contained in the <OnNetwork> element of the MCPTT service configuration document (see the service configuration document in 3GPP TS 24.384 [50]).

When determining the Resource-Priority header field namespace and priority values as specified in draft-holmberg-dispatch-mcptt-rp-namespace [48] for a normal MCPTT group or private call the MCPTT client:

1)	shall retrieve the value of the <resource-priority-namespace> element contained in the <normal-resource-priority> element contained in the <OnNetwork> element of the MCPTT service configuration document (see the service configuration document in 3GPP TS 24.384 [50]); and

2)	shall retrieve the value of the <resource-priority-priority> element contained in the <normal-resource-priority> element contained in the <OnNetwork> element of the MCPTT service configuration document (see the service configuration document in 3GPP TS 24.384 [50]).

NOTE:	The "normal" Resource-Priority header field value is needed to return to a normal priority value from a priority value adjusted for an MCPTT emergency group or private call or an MCPTT imminent peril group call. The "normal" priority received from the EPS by use of the "normal" Resource-Priority header field value is expected to be the same as the "normal" priority received from the EPS when initiating a call with no Resource-Priority header field included.

#### 6.2.8.2	Request for an originating broadcast group call

NOTE:	This subclause is referenced from other procedures.

When the MCPTT user initiates a broadcast group call, the MCPTT client:

1)	in the case of the prearranged group call is initiated on-demand, shall include in the application/vnd.3gpp.mcptt-info+xml MIME body the <broadcast-ind> element set to "true" as defined in clause F.1; and

2)	in the case the prearranged group call is initiated using a pre-established session, shall include in the application/vnd.3gpp.mcptt-info+xml MIME body in the "body" URI header field in the Refer-To header field the <broadcast-ind> element set to "true" as defined in clause F.1.

#### 6.2.8.3	MCPTT emergency private call conditions

##### 6.2.8.3.1	Authorisations

###### 6.2.8.3.1.1	Determining authorisation for initiating an MCPTT emergency private call

If the MCPTT client receives a request from the MCPTT user to originate an MCPTT emergency private call and:

1)	if the <allow-emergency-private-call> element of the <ruleset> element of the MCPTT user profile document identified by the MCPTT ID of the calling user (see the MCPTT user profile document in 3GPP TS 24.384 [50]) is set to a value of "true"; and

a)	if the "entry-info" attribute of the <entry> element of the <Recipient> element contained within the <EmergencyCall> element of the MCPTT user profile document (see the MCPTT user profile document in 3GPP TS 24.384 [50]) is set to a value of "UsePreConfigured" and if the <uri-entry> element of the <entry> element of the <Recipient> element contains the MCPTT ID of the MCPTT user targeted by the calling MCPTT user; or

b)	if the "entry-info" attribute of the <entry> element of the <Recipient> element contained within the <EmergencyCall> element of the MCPTT user profile document (see the MCPTT user profile document in 3GPP TS 24.384 [50]) is set to a value of "LocallyDetermined";

then the MCPTT client shall consider the MCPTT emergency private call request to be an authorised request for an MCPTT emergency private call. In all other cases the MCPTT client shall consider the MCPTT emergency private call request to be an unauthorised request for an MCPTT emergency private call.

###### 6.2.8.3.1.2	Determining authorisation for cancelling an MCPTT emergency private call

If the MCPTT client receives a request from the MCPTT user to cancel an MCPTT emergency private call and if the <allow-cancel-private-emergency-call> element of the <ruleset> element of the MCPTT user profile document identified by the MCPTT ID of the calling user (see the MCPTT user profile document in 3GPP TS 24.384 [50]) is set to a value of "true", then the MCPTT emergency private call cancellation request shall be considered to be an authorised request for an MCPTT emergency private call cancellation.

In all other cases, the MCPTT emergency private call cancellation request shall be considered to be an unauthorised request for an MCPTT emergency private call cancellation.

###### 6.2.8.3.1.3	Determining authorisation for initiating or cancelling an MCPTT emergency alert to a MCPTT user

If the MCPTT client receives a request from the MCPTT user to send an MCPTT emergency alert to an MCPTT user and:

1)	if the <allow-activate-emergency-alert> element of the <ruleset> element of the MCPTT user profile document identified by the MCPTT ID of the calling MCPTT user as specified in 3GPP TS 24.384 [50] is set to a value of "true"; and

2)	if the "entry-info" attribute of the <entry> element of the <Recipient> element contained within the <EmergencyAlert> element of the MCPTT user profile document (see the MCPTT user profile document in 3GPP TS 24.384 [50]) is set to a value of:

a)	"UsePreConfigured", and if the <uri-entry> element of the <entry> element of the <Recipient> element of the <EmergencyAlert> element of the <MCPTT-group-call> element of the MCPTT user profile document (see the MCPTT user profile document in 3GPP TS 24.384 [50]) contains the MCPTT ID of the targeted MCPTT user; or

b)	"LocallyDetermined";

then the MCPTT emergency alert request shall be considered to be an authorised request for an MCPTT emergency alert. In all other cases, it shall be considered to be an unauthorised request for an MCPTT emergency alert.

If the MCPTT client receives a request from the MCPTT user to cancel an MCPTT emergency alert to an MCPTT user, and if the <allow-cancel-emergency-alert> element of the <ruleset> element of the MCPTT user profile document identified by the MCPTT ID of the calling MCPTT user as specified in 3GPP TS 24.384 [50] is set to a value of "true", then the MCPTT emergency alert cancellation request shall be considered to be an authorised request to cancel an MCPTT emergency alert. In all other cases, it shall be considered to be an unauthorised request to cancel an MCPTT emergency alert.

##### 6.2.8.3.2	SIP request for originating MCPTT emergency private calls

This subclause is referenced from other procedures.

When the MCPTT emergency private call state is set to "MEPC 1: emergency-pc-capable" and this is an authorised request for an MCPTT emergency private call as determined by the procedures of subclause 6.2.8.3.1.1, the MCPTT client:

1)	shall set the MCPTT emergency state if not already set;

2)	shall include in the application/vnd.3gpp.mcptt-info+xml MIME body in the SIP request an <emergency-ind> element set to "true" and set the MCPTT emergency private call state to "MEPC 2: emergency-pc-requested";

3)	if the MCPTT user has also requested an MCPTT emergency alert to be sent and this is an authorised request for MCPTT emergency alert as determined by the procedures of subclause 6.2.8.3.1.3, shall:

a)	include in the application/vnd.3gpp.mcptt-info+xml MIME body the <alert-ind> element set to "true" and set the MCPTT private emergency alert state to "MPEA 2: emergency-alert-confirm-pending"; and

b)	perform the procedures specified in subclause 6.2.9.1 for the MCPTT emergency alert trigger;

4)	if the MCPTT user has not requested an MCPTT emergency alert to be sent, shall set the <alert-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body to "false"; and

5)	if the MCPTT emergency private priority state of this private call is set to a value other than "MEPP 2: in-progress" shall set the MCPTT emergency private priority state to "MEPP 3: confirm-pending".

##### 6.2.8.3.3	Resource-Priority header field for MCPTT emergency private calls

This subclause is referenced from other procedures.

If the MCPTT emergency private call state is set to either "MEPC 2: emergency-pc-requested" or "MEPC 3: emergency-pc-granted" and this is an authorised request for an MCPTT emergency private call as determined by the procedures of subclause 6.2.8.3.1.1, or the MCPTT emergency private priority state of the call is set to "MEPP 2: in-progress", the MCPTT client shall include in the SIP request a Resource-Priority header field populated with the values for an MCPTT emergency private call as specified in subclause 6.2.8.1.15.

NOTE:	The MCPTT client ideally would not need to maintain knowledge of the in-progress emergency state of the call (as tracked on the MCPTT client by the MCPTT client emergency private state) but can use this knowledge to provide a Resource-Priority header field set to emergency level priority, which starts the infrastructure priority adjustment process sooner than otherwise would be the case.

If this is an authorised request to cancel the MCPTT emergency private call as determined by the procedures of subclause 6.2.8.3.1.2, or the MCPTT emergency private priority state of the private call is "MEPP 1: no-emergency" or "MEPP 3: cancel-pending", the MCPTT client shall include in the SIP request a Resource-Priority header field populated with the values for a normal MCPTT private call as specified in subclause 6.2.8.1.15.

##### 6.2.8.3.4	Receiving a SIP 2xx response to a SIP request for an MCPTT emergency private call

This subclause is referenced from other procedures.

On receiving a SIP 2xx response to a SIP request for an MCPTT emergency private call and if the MCPTT emergency private call state is set to "MEPC 2: emergency-pc-requested" or "MEPC 3: emergency-pc-granted", the MCPTT client:

1)	shall set the MCPTT emergency private priority state of the call to "MEPP 2: in-progress" if it was not already set;

2)	shall set the MCPTT emergency private call state to "MEPC 3: emergency-pc-granted"; and

3)	if the MCPTT private emergency alert state is set to "MPEA 2: emergency-alert-confirm-pending" and the SIP 2xx response to the SIP request for a priority private call does not contain a Warning header field as specified in subclause 4.4 with the warning text containing the mcptt-warn-code set to "149", shall set the MCPTT private emergency alert state to "MPEA 3: emergency-alert-initiated".

##### 6.2.8.3.5	Receiving a SIP 4xx response, SIP 5xx response or SIP 6xx response to a SIP request for an MCPTT emergency private call

Upon receiving a SIP 4xx response, SIP 5xx response or a SIP 6xx response to a SIP request for an MCPTT emergency private call and if the MCPTT emergency private call state is set to "MEPC 2: emergency-pc-requested" or "MEPC 3: emergency-pc-granted", the MCPTT client:

1)	shall set the MCPTT emergency private call state to "MEPC 1: emergency-pc-capable";

2)	if the MCPTT emergency private priority state of the private call is "MEPP 3: confirm-pending" shall set the MCPTT emergency private priority state of the private call to "MEPP 1: no-emergency";

3)	if the SIP 4xx response, SIP 5xx response or SIP 6xx response contains an application/vnd.3gpp.mcptt-info+xml MIME body with an <alert-ind> element set to a value of "true", shall set the MCPTT private emergency alert state to "MPEA 3: emergency-alert-initiated";

NOTE 1:	this is the case where the MCPTT user has requested an MCPTT emergency alert in addition to the MCPTT emergency private call and the MCPTT emergency alert is granted.

4)	if the SIP 4xx response, SIP 5xx response or SIP 6xx response contains an application/vnd.3gpp.mcptt-info+xml MIME body with an <alert-ind> element set to a value of "false", shall set the MCPTT private emergency alert state to "MPEA 1: no-alert"; and

NOTE 2:	this is the case where the MCPTT user has requested an MCPTT emergency alert in addition to an MCPTT emergency call and the MCPTT emergency alert is not granted.

5)	 if the SIP 4xx response, SIP 5xx response or SIP 6xx response does not contain an application/vnd.3gpp.mcptt-info+xml MIME body or if the MIME body does not include an <alert-ind> element, if the MCPTT private emergency alert state is set to "MPEA 2: emergency-alert-confirm-pending" shall set the MCPTT emergency alert state to "MPEA 1: no-alert".

##### 6.2.8.3.6	SIP re-INVITE request for cancelling MCPTT emergency private call state

This subclause is referenced from other procedures.

When the MCPTT emergency private call state is set to "MEPC 3: emergency-pc-granted" and the MCPTT emergency alert state is set to "MPEA 1: no-alert", the MCPTT client shall generate a SIP re-INVITE request according to 3GPP TS 24.229 [4] with the clarifications given below.

NOTE 1:	This procedure assumes that the MCPTT client in the calling procedure has verified that the MCPTT user has made an authorised request for cancelling MCPTT the in-progress emergency private call state of the call.

The MCPTT client:

1)	shall include in the SIP re-INVITE request an application/vnd.3gpp.mcptt-info+xml MIME body as defined in clause F.1 with the <emergency-ind> element set to "false";

2)	shall clear the MCPTT emergency state; and

3)	shall set MCPTT emergency private priority state of the MCPTT emergency private call to "MEPP 3: cancel-pending".

NOTE 2:	This is the case of an MCPTT user who has initiated an MCPTT emergency private call and wants to cancel it.

When the MCPTT emergency private call state is set to "MEPPC 3: emergency-pc-granted" and the MCPTT emergency alert state is set to a value other than "MPEA 1: no-alert" and the MCPTT user has indicated only the MCPTT emergency private call should be cancelled, the MCPTT client:

1)	shall include in the SIP re-INVITE request an application/vnd.3gpp.mcptt-info+xml MIME body as defined in clause F.1 with the <emergency-ind> element set to "false"; and

2)	shall set the MCPTT emergency private priority state of the MCPTT emergency private call to "MEPP 3: cancel-pending";

NOTE 3:	This is the case of an MCPTT user has initiated both an MCPTT emergency private call and an MCPTT emergency alert and wishes to only cancel the MCPTT emergency private call. This leaves the MCPTT emergency state set.

When the MCPTT emergency private call state is set to "MEPC 3: emergency-pc-granted" and the MCPTT emergency alert state is set to a value other than "MPEA 1: no-alert" and the MCPTT user has indicated that the MCPTT emergency alert on the MCPTT private call should be cancelled in addition to the MCPTT emergency private call, the MCPTT client:

1)	shall include in the SIP re-INVITE request an application/vnd.3gpp.mcptt-info+xml MIME body as defined in annex F.1 with the <emergency-ind> element set to "false";

2)	shall, if this is an authorised request to cancel an MCPTT emergency alert as determined by the procedures of subclause 6.2.8.3.1.3:

a)	include in the application/vnd.3gpp.mcptt-info+xml MIME body an <alert-ind> element set to "false"; and

b)	set the MCPTT private emergency alert state to "MPEA 4: emergency-alert-cancel-pending";

3)	if this is not an authorised request to cancel an MCPTT emergency alert as determined by the procedures of subclause 6.2.8.3.1.3, should indicate to the MCPTT user they are not authorised to cancel this MCPTT emergency alert;

4)	shall set the MCPTT emergency private priority state of the MCPTT to "MEPP 3: cancel-pending"; and

5)	shall clear the MCPTT emergency state.

NOTE 4:	This is the case of an MCPTT user that has initiated both an MCPTT emergency private call and an MCPTT emergency alert and wishes to cancel both.

##### 6.2.8.3.7	Receiving a SIP INFO request in the dialog of a SIP request for a priority private call

This subclause is referenced from other procedures.

Upon receiving a SIP INFO request within the dialog of the SIP request for a priority private call:

-	with the Info-Package header field containing the g.3gpp.mcptt-info package name;

-	with the application/vnd.3gpp.mcptt-info+xml MIME body associated with the info package according to IETF RFC 6086 [54]; and

-	with one or more of the <alert-ind>, <imminentperil> ind and <emergency-ind> elements set in the application/vnd.3gpp.mcptt-info+xml MIME body;

the MCPTT client:

1)	if the MCPTT private emergency alert state is set to "MPEA 2: emergency-alert-confirm-pending":

a)	if the <alert-ind> element is set to a value of "false", shall set the MCPTT private emergency alert state to "MPEA 1: no-alert"; and

b)	if the <alert-ind> element set to a value of "true", shall set the MCPTT private emergency alert state to "MPEA 3: emergency-alert-initiated"; and

2)	if the MCPTT private emergency alert state is set to "MPEA 4: Emergency-alert-cancel-pending":

a)	if the <alert-ind> element is set to a value of "true", shall set the MCPTT private emergency alert state to "MPEA 3: emergency-alert-initiated"; and

b)	if the <alert-ind> element is set to a value of "false", shall set the MCPTT private emergency alert state to "MPEA 1: no-alert".

##### 6.2.8.3.8	SIP re-INVITE request for cancelling the MCPTT emergency private call state by a third-party

This subclause is referenced from other procedures.

Upon receiving a request to cancel the MCPTT emergency private call state from an MCPTT user other than the originator of the MCPTT emergency private call, the MCPTT client shall generate a SIP re-INVITE request according to 3GPP TS 24.229 [4] with the clarifications given below.

The MCPTT client:

NOTE 1: This procedure assumes that the calling procedure has verified that the MCPTT user has made an authorised request for cancelling the MCPTT emergency private call state of the call.

1)	shall include in the SIP re-INVITE request an application/vnd.3gpp.mcptt-info+xml MIME body as defined in clause F.1 with the <emergency-ind> element set to "false";

2)	shall set the MCPTT emergency private priority state of the MCPTT emergency private call to "MEPP 3: cancel-pending"; and

3)	if the MCPTT user has indicated that an MCPTT emergency alert associated with the MCPTT emergency private call originated by another MCPTT user should be cancelled and this is an authorised request for an MCPTT emergency alert cancellation as determined by the procedures of subclause 6.2.8.3.1.3:

a)	shall include in the application/vnd.3gpp.mcptt-info+xml MIME body an <alert-ind> element set to a value of "false"; and

b)	shall include in the application/vnd.3gpp.mcptt-info+xml MIME body an <originated-by> element set to the MCPTT ID of the MCPTT user who originated the MCPTT emergency alert.

NOTE 2:	When an MCPTT emergency alert is cancelled by a MCPTT user other than its originator, the <originated-by> element is needed to identify which MCPTT emergency alert is being cancelled, as conceivably each participant in the MCPTT emergency private call could have originated an MCPTT emergency alert.

### 6.2.9	Location information

#### 6.2.9.1	Location information for location reporting

This procedure is initiated by the MCPTT client when it is including location report information as part of a SIP request for a specified location trigger.

The MCPTT client:

1)	shall include an application/vnd.3gpp.location-info+xml MIME body as specified in Annex F.3 with a <Report> element included in the <location-info> root element; and

2)	shall include in the <Report> element the specific location information configured for the specified location trigger.

## 6.3	MCPTT server procedures

### 6.3.1	Distinction of requests sent to the MCPTT server

#### 6.3.1.1	SIP INVITE request

The MCPTT server needs to distinguish between the following initial SIP INVITE requests for originations and terminations:

-	SIP INVITE requests routed to the participating MCPTT function as a result of processing initial filter criteria at the S-CSCF in accordance with the origination procedures as specified in 3GPP TS 24.229 [4] with the Request-URI set to a public service identity of the participating MCPTT function that identifies the pre-established session set-up. Such requests are known as "SIP INVITE request for establishing a pre-established session" in the procedures in the present document;

-	SIP INVITE requests routed to the participating MCPTT function as a result of processing initial filter criteria at the S-CSCF in accordance with the origination procedures as specified in 3GPP TS 24.229 [4] and the Request-URI is set to a public service identity of the participating MCPTT function that does not identify the pre-established session set-up. Such requests are known as "SIP INVITE request for originating participating MCPTT function" in the procedures in the present document;

-	SIP INVITE requests routed to the participating MCPTT function as a result of processing initial filter criteria at the S-CSCF in accordance with the termination procedures as specified in 3GPP TS 24.229 [4] and the Request-URI contains a PSI of the terminating participating MCPTT function. Such requests are known as "SIP INVITE request for terminating participating MCPTT function" in the procedures in the present document;

-	SIP INVITE requests routed to the controlling MCPTT function as a result of PSI routing on the originating side in accordance with the originating procedures as specified in 3GPP TS 24.229 [4], or as a result of direct PSI routing, in accordance with the termination procedures as specified in 3GPP TS 24.229 [4], the Request-URI is set to a public service identity for MCPTT private call and the Contact header field does not contain the isfocus media feature tag specified in IETF RFC 3840 [16]. Such requests are known as "SIP INVITE request for controlling MCPTT function of a private call" in the procedures in the present document;

-	SIP INVITE requests routed to the controlling MCPTT function as a result of PSI routing on the originating side in accordance with the originating procedures as specified in 3GPP TS 24.229 [4], or as a result of direct PSI routing, in accordance with the termination procedures as specified in 3GPP TS 24.229 [4], the Request-URI is set to a public service identity serving an MCPTT group and the Contact header field does not contain the isfocus media feature tag specified in IETF RFC 3840 [16]. Such requests are known as "SIP INVITE request for controlling MCPTT function of an MCPTT group" in the procedures in the present document; and

-	SIP INVITE requests routed to the non-controlling MCPTT function of an MCPTT group as a result of direct PSI routing, in accordance with the termination procedures as specified in 3GPP TS 24.229 [4], the Request-URI is set to a public service identity serving an MCPTT group and the Contact header field contains the isfocus media feature tag specified in IETF RFC 3840 [16]; Such requests are known as "SIP INVITE request for non-controlling MCPTT function of an MCPTT group" in the procedures in the present document.

#### 6.3.1.2	SIP REFER request

The MCPTT server needs to distinguish between the following initial SIP REFER request for originations and terminations:

-	SIP REFER requests routed to the participating MCPTT function as a result of processing initial filter criteria at the S-CSCF in accordance with the origination procedures as specified in 3GPP TS 24.229 [4] with the Request-URI set to a public service identity identifying the pre-established session on the participating MCPTT function. Such requests are known as "SIP REFER request for a pre-established session" in the procedures in the present document.

#### 6.3.1.3	SIP MESSAGE request

The MCPTT server needs to distinguish between the following SIP MESSAGE request for originations and terminations:

-	SIP MESSAGE requests routed to the participating MCPTT function as a result of processing initial filter criteria at the S-CSCF in accordance with the origination procedures as specified in 3GPP TS 24.229 [4] with the Request-URI set to the MBMS public service identity of the participating MCPTT function. Such requests are known as "SIP MESSAGE request for an MBMS listening status update" in the procedures in the present document;

-	SIP MESSAGE request routed to the participating MCPTT function as a result of initial filter criteria containing a Content-Type header field set to "application/vnd.3gpp.mcptt-location-info+xml" and includes an XML body containing a Location root element containing a Report element. Such requests are known as "SIP MESSAGE request for location reporting" in the present document;

-	SIP MESSAGE requests routed to the originating participating MCPTT function as a result of initial filter criteria with the Request-URI set to the public service identity of the participating MCPTT function and containing a Content-Type header field set to "application/vnd.3gpp.mcptt-info+xml" and includes an XML body containing a <mcpttinfo> root element containing a <mcptt-Params> element containing an <emergency-ind> element or an <alert-ind> element. Such requests are known as "SIP MESSAGE requests for emergency notification for originating participating MCPTT function" in the procedures in the present document;

-	SIP MESSAGE requests routed to the terminating participating MCPTT function as a result of initial filter criteria with the Request-URI set to the public service identity of the terminating participating MCPTT function and containing a Content-Type header field set to "application/vnd.3gpp.mcptt-info+xml" and includes an XML body containing a <mcpttinfo> root element containing a <mcptt-Params> element containing an <emergency-ind> element or an <alert-ind> element. Such requests are known as "SIP MESSAGE requests for emergency notification for terminating participating MCPTT function" in the procedures in the present document;

-	SIP MESSAGE requests routed to the controlling MCPTT function as a result of initial filter criteria with the Request-URI set to the public service identity of the controlling MCPTT function and containing a Content-Type header field set to "application/vnd.3gpp.mcptt-info+xml" and includes an XML body containing a <mcpttinfo> root element containing a <mcptt-Params> element containing an <emergency-ind> element or an <alert-ind> element. Such requests are known as "SIP MESSAGE requests for emergency notification for controlling MCPTT function" in the procedures in the present document;

-	SIP MESSAGE request routed to the MCPTT client as a result of initial filter criteria containing a Content-Type header field set to "application/vnd.3gpp.mcptt-location-info+xml" and includes an XML body containing a Location root element containing a Configuration element. Such requests are known as "SIP MESSAGE request for location report configuration" in the present document; and

-	SIP MESSAGE request routed to the MCPTT client as a result of initial filter criteria containing a Content-Type header field set to "application/vnd.3gpp.mcptt-location-info+xml" and includes an XML body containing a Location root element containing a Request element. Such requests are known as "SIP MESSAGE request for location report request" in the present document.

#### 6.3.1.4	SIP SUBSCRIBE request

The MCPTT server needs to distinguish between the following SIP SUBSCRIBE request for originations and terminations:

-	SIP SUBSCRIBE requests routed to the participating MCPTT function with the Request-URI set to the MCPTT session identity identifying the participating MCPTT function and the Event header field set to "conference". Such requests are known as "SIP SUBSCRIBE request for conference event status subscription" in the procedures in the present document;

-	SIP SUBSCRIBE requests routed to the controlling MCPTT function with the Request-URI set to the MCPTT session identity identifying the controlling MCPTT function and containing an Event header field set to "conference". Such requests are known as "SIP SUBSCRIBE request for event status subscription in the controlling MCPTT function" in the procedures in the present document; and

-	SIP SUBSCRIBE requests routed to the non-controlling MCPTT function with the Request-URI set to the MCPTT session identity identifying the non-controlling MCPTT function and containing an Event header field set to "conference". Such requests are known as "SIP SUBSCRIBE request for event status subscription in the non-controlling MCPTT function" in the procedures in the present document.

### 6.3.2	Participating MCPTT Function

#### 6.3.2.1	Requests initiated by the served MCPTT user

##### 6.3.2.1.1	SDP offer generation

###### 6.3.2.1.1.1	On-demand session

This subclause is referenced from other subclauses.

The SDP offer is generated based on the received SDP offer. The SDP offer generated by the participating MCPTT function:

1)	shall contain only one SDP media-level section for MCPTT speech as contained in the received SDP offer; and

2)	shall contain an SDP media-level section for one media-floor control entity, if present in the received SDP offer.

When composing the SDP offer according to 3GPP TS 24.229 [4], the participating MCPTT function:

1)	shall replace the IP address and port number for the offered media stream in the received SDP offer with the IP address and port number of the participating MCPTT function, if required;

NOTE 1:	Requirements can exist for the participating MCPTT function to be always included in the path of the offered media stream, for example: for the support of features such as MBMS, lawful interception and recording. Other examples can exist.

2)	shall replace the IP address and port number for the offered media floor control entity, if any, in the received SDP offer with the IP address and port number of the participating MCPTT function; and

NOTE 2:	If the participating MCPTT function and the controlling MCPTT function or the participating MCPTT function and the non-controlling MCPTT function are in the same MCPTT server, and the participating MCPTT function does not have a dedicated IP address or a dedicated port number for media floor control or media stream, the replacement of the IP address or the port number is omitted.

3)	shall contain an "a=key-mgmt" attribute field with a "mikey" attribute value, if present in the received SDP offer.

###### 6.3.2.1.1.2	Pre-established session

This subclause is referenced from other subclauses.

When composing an SDP offer according to 3GPP TS 24.229 [4], the participating MCPTT function:

1)	shall set the IP address of the participating MCPTT function for MCPTT speech from the SDP negotiated during the pre-established session establishment;

2)	shall set the IP address of the participating MCPTT function for the offered media-floor control entity from the SDP negotiated during the pre-established session establishment, if present in the received SDP offer;

3)	shall contain only one SDP media-level section for MCPTT speech obtained from the SDP negotiated during the pre-established session establishment consisting of:

a)	the port number for the MCPTT speech; and

b)	the codec(s), media parameters and attributes as in the SDP negotiated during the pre-established session establishment;

4)	shall include the media-level section of the offered media-floor control entity from the SDP negotiated during the pre-established session establishment, if any media-floor control entity is offered consisting of:

a)	the media-floor control entity parameters as in the SDP negotiated during the pre-established session establishment; and

b)	the port number for the selected media-floor control entity selected as specified in 3GPP TS 24.229 [4]; and

5)	shall contain an "a=key-mgmt" attribute field with a "mikey" attribute value if present in the received SDP offer.

##### 6.3.2.1.2	SDP answer generation

###### 6.3.2.1.2.1	On-demand session

When composing the SDP answer according to 3GPP TS 24.229 [4], the participating MCPTT function:

1)	shall replace the IP address and port number in the received SDP answer with the IP address and port number of the participating MCPTT function, for the accepted media stream in the received SDP offer, if required; and

NOTE 1:	Requirements can exist for the participating MCPTT function to be always included in the path of the offered media stream, for example: for the support of features such as MBMS, lawful interception and recording. Other examples can exist.

2)	shall replace the IP address and port number in the received SDP answer with the IP address and port number of the participating MCPTT function, for the accepted media-floor control entity, if present in the received SDP offer.

NOTE 2:	If the participating MCPTT function and the controlling MCPTT function or the participating MCPTT function and the non-controlling MCPTT function are in the same MCPTT server, and the participating MCPTT function does not have a dedicated IP address or a dedicated port number for media floor control or media stream, the replacement of the IP address or the port number is omitted.

###### 6.3.2.1.2.2	Pre-established session establishment

When composing the SDP answer according to 3GPP TS 24.229 [4], the participating MCPTT function:

1.	shall set the IP address and port number to those of the participating MCPTT function for each accepted media stream from the list contained in the received SDP offer and for each accepted media stream in the received SDP offer; and

2.	shall set the IP address and port number to those of the participating MCPTT function, for the accepted media-floor control entity, if present in the received SDP offer.

##### 6.3.2.1.3	Sending an INVITE request on receipt of an INVITE request

This subclause is referenced from other procedures.

When generating an initial SIP INVITE request according to 3GPP TS 24.229 [4], on receipt of an incoming SIP INVITE request, the participating MCPTT function:

1)	shall include in the SIP INVITE request all Accept-Contact header fields and all Reject-Contact header fields, with their feature tags and their corresponding values along with parameters according to rules and procedures of IETF RFC 3841 [6] if included in the incoming SIP INVITE request;

2)	should include the Session-Expires header field according to IETF RFC 4028 [7]. It is recommended that the "refresher" header field parameter is omitted. If included, the "refresher" header field parameter shall be set to "uac";

3)	shall include the option tag "timer" in the Supported header field;

4)	shall copy the contents of the P-Asserted-Identity header field of the incoming SIP INVITE request to the P-Asserted-Identity header field of the outgoing SIP INVITE request;

5)	shall include the g.3gpp.mcptt media feature tag into the Contact header field of the outgoing SIP INVITE request;

6)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), into the P-Asserted-Service header field of the outgoing SIP INVITE request;

7)	if the incoming SIP INVITE request contained a MIME resource-lists body with the MCPTT ID of the invited MCPTT user, shall copy the MIME resource-lists body, according to rules and procedures of IETF RFC 5366 [20];

8)	if the incoming SIP INVITE request contained an application/vnd.3gpp.mcptt-info+xml MIME body, shall copy the contents of the application/vnd.3gpp.mcptt-info+xml MIME body of the incoming SIP INVITE request to the outgoing SIP INVITE request; and

9)	if the incoming SIP INVITE request contained an application/vnd.3gpp.location-info+xml MIME body, shall copy the contents of the application/vnd.3gpp.location-info+xml MIME body of the incoming SIP INVITE request to the outgoing SIP INVITE request.

##### 6.3.2.1.4	Sending an INVITE request on receipt of a REFER request

This subclause is referenced from other procedures.

When generating an initial SIP INVITE request according to 3GPP TS 24.229 [4], on receipt of an incoming SIP REFER request, the participating MCPTT function:

1)	shall include in the SIP INVITE request all header fields included in the headers portion of the SIP URI contained in the <entry> element of the application/resource-lists MIME body, referenced by the "cid" URL in the Refer-To header field in the incoming SIP REFER request;

2)	should include the Session-Expires header field according to IETF RFC 4028 [7]. It is recommended that the "refresher" header field parameter is omitted. If included, the "refresher" header field parameter shall be set to "uac";

3)	shall include the option tag "timer" in the Supported header field;

4)	shall copy the contents of the P-Asserted-Identity header field of the incoming SIP REFER request to the P-Asserted-Identity header field of the outgoing SIP INVITE request;

5)	shall include the g.3gpp.mcptt media feature tag and the g.3gpp.icsi-ref media feature tag with the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" into the Contact header field of the outgoing SIP INVITE request;

6)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), into the P-Asserted-Service header field of the outgoing SIP INVITE request; and

7)	shall include in the SIP INVITE request the option tag "tdialog" in a Supported header field according to the rules and procedures of IETF RFC 4538 [23];

8)	shall include in the SIP INVITE request an SDP offer as specified in subclause 6.3.2.1.1.2 based upon:

a)	the SDP negotiated during the pre-established session establishment and any subsequent pre-established session modification; and

b)	the SDP offer (if any) included in the"body" URI parameter of the SIP-URI contained in the <entry> element of the application/resource-lists MIME body, referenced by the "cid" URL in the Refer-To header field in the incoming SIP REFER request for a pre-established session;

9)	shall determine if the SIP REFER request is regarded as being received with an implicit floor request;

a)	if according to subclause 6.4, the SIP REFER request is regarded as being received with an implicit floor request, the participating MCPTT function shall include the "mc_implicit_request" media level attribute in the associated UDP stream for the floor control in the SDP offer of the SIP INVITE request; and

b)	if, according to subclause 6.4, the SIP REFER request is regarded as being not received with an implicit floor request, the participating MCPTT function shall not include the "mc_implicit_request" media level attribute in the associated UDP stream for the floor control in the SDP offer of the SIP INVITE request;

10)	shall copy the application/vnd.3gpp.mcptt-info+xml MIME body from the "body" URI header field of the SIP-URI in the application/resource-lists MIME body, referenced by the "cid" URL in the Refer-To header field of the SIP REFER request, to the outgoing SIP INVITE request;

11)	shall include the <mcptt-calling-user-identity> element set to the MCPTT ID of the calling user in the application/vnd.3gpp.mcptt-info+xml MIME body of the outgoing SIP INVITE request; and

12)	if the incoming SIP REFER request contained an application/resource-lists MIME body in the "body" URI header field of the SIP-URI contained in the <entry> element of an application/resource-lists MIME body, referenced by the "cid" URL in the Refer-To header field, shall copy the application/resources-lists MIME body in the "body" URI header field to the SIP INVITE request;

##### 6.3.2.1.5	Response to an INVITE request

###### 6.3.2.1.5.1	Provisional responses

NOTE:	This subclause is referenced from other procedures

When sending SIP provisional responses other than the SIP 100 (Trying) response, the participating MCPTT function shall generate a SIP provisional response according to 3GPP TS 24.229 [4] and:

1)	shall include the following in the Contact header field:

a)	the g.3gpp.mcptt media feature tag;

b)	the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt";

c)	the isfocus media feature tag; and

d)	an MCPTT session identity mapped to the MCPTT session identity if provided in the Contact header field of the incoming provisional response;

2)	shall include the "norefersub" option tag in a Supported header field in accordance with 3GPP TS 24.229 [4];

3)	may include a Resource-Share header field in accordance with subclause 5.7.1.20.2 in 3GPP TS 24.229 [4]; and

4)	if the incoming SIP provisional response contained an application/vnd.3gpp.mcptt-info+xml MIME body, shall copy the application/vnd.3gpp.mcptt-info+xml MIME body to the outgoing SIP provisional response.

###### 6.3.2.1.5.2	Final response

This subclause is referenced from other procedures.

When sending SIP 200 (OK) responses, the participating MCPTT function shall generate a SIP 200 (OK) response according to 3GPP TS 24.229 [4] and:

1)	shall include the option tag "timer" in a Require header field;

2)	shall include the Session-Expires header field according to rules and procedures of IETF RFC 4028 [7], "UAS Behavior". If the "refresher" parameter is not included in the received request, the "refresher" parameter in the Session-Expires header field shall be set to "uac";

3)	shall include the following in the Contact header field:

a)	the g.3gpp.mcptt media feature tag;

b)	the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt"; and

c)	the isfocus media feature tag;

4)	shall include the option tag "tdialog" in a Supported header field according to rules and procedures of IETF RFC 4538 [23];

5)	shall include the option tag "norefersub" in a Supported header field according to rules and procedures of IETF RFC 4488 [22];

6)	may include a Resource-Share header field in accordance with subclause 5.7.1.20.2 in 3GPP TS 24.229 [4]; and

7)	if the incoming SIP 200 (OK) response contained an application/vnd.3gpp.mcptt-info+xml MIME body, shall copy the application/vnd.3gpp.mcptt-info+xml MIME body to the outgoing SIP 200 (OK) response.

##### 6.3.2.1.6	Sending a SIP BYE request on receipt of a SIP BYE request

Upon receiving a SIP BYE request from the MCPTT client, the participating MCPTT function:

1)	shall interact with the media plane as specified in subclause 6.4 in 3GPP TS 24.380 [5];

2)	shall generate a SIP BYE request as specified in 3GPP TS 24.229 [4];

3)	shall set the Request-URI to the MCPTT session identity as included in the received SIP BYE request;

4)	shall copy the contents of the P-Asserted-Identity header field of the incoming SIP BYE request to the P-Asserted-Identity header field of the outgoing SIP BYE request; and

5)	shall send the SIP BYE request toward the controlling MCPTT function, according to 3GPP TS 24.229 [4].

Upon receiving a SIP 200 (OK) response to the SIP BYE request the terminating MCPTT function shall forward a SIP 200 (OK) response to the MCPTT client and shall interact with the media plane as specified in subclause 6.4 in 3GPP TS 24.380 [5] for releasing media plane resources associated with the SIP session with the controlling MCPTT function.

##### 6.3.2.1.7	Sending a SIP BYE request on receipt of a SIP REFER request

Upon receiving a SIP REFER request with the "method" SIP URI parameter set to value "BYE" in the URI in the Refer-To header field from the MCPTT client, the participating MCPTT function:

1)	if the user identified by the MCPTT ID is not authorised, shall reject the "SIP REFER request for pre-established session" with a SIP 403 (Forbidden) response to the SIP BYE request, with warning text set to "100 function not allowed due to <detailed reason>" as specified in subclause 4.4, and shall not continue with the rest of the steps;

2)	if the SIP REFER request contained a Refer-Sub header field containing "false" value and a Supported header field containing "norefersub" value, shall handle the SIP REFER request as specified in 3GPP TS 24.229 [4], IETF RFC 3515 [25] as updated by IETF RFC 6665 [26], and IETF RFC 4488 [22] without establishing an implicit subscription;

3)	shall generate a SIP 200 (OK) response to the SIP REFER request, and in the SIP 200 (OK) response:

a)	shall include the Supported header field with value "norefersub" according to rules and procedures of IETF RFC 4488 [22]; and

b)	shall check the presence of the Refer-Sub header field of the SIP REFER request and if it is present and set to the value "false" shall include the Refer-Sub header field with value "false" according to rules and procedures of IETF RFC 4488 [22];

NOTE:	In accordance with IETF RFC 4488 [22], the participating MCPTT function inserts the Refer-Sub header field containing the value "false" in the SIP 200 (OK) response to the SIP REFER request to indicate that it has not created an implicit subscription.

4)	shall send the SIP 200 (OK) response to the SIP REFER request towards MCPTT client according to 3GPP TS 24.229 [4];

5)	shall generate a SIP BYE request, and in the SIP BYE request:

a)	shall set the Request-URI to the MCPTT session identity which was included at the Refer-To header field of the received REFER request; and

b)	shall copy the contents of the P-Asserted-Identity header field of the received REFER request to the P-Asserted-Identity header field of the outgoing SIP BYE request; and

6)	shall send the SIP BYE request toward the controlling MCPTT function according to 3GPP TS 24.229 [4].

Upon receiving a SIP 200 (OK) response to the SIP BYE request the participating MCPTT function shall interact with the media plane as specified in subclause 6.4 in 3GPP TS 24.380 [5] for releasing media plane resources associated with the SIP session with the controlling MCPTT function.

##### 6.3.2.1.8	Priority call conditions

This subclause contains common procedures to be used for MCPTT emergency group calls and MCPTT imminent peril group calls.

###### 6.3.2.1.8.1		Determining authorisation for originating a priority group call

When the participating MCPTT function receives a request from the MCPTT client to originate an MCPTT emergency group call and needs to determine if the request is an authorised request for an MCPTT emergency call, the participating MCPTT function shall check the following:

1)	if the <allow-emergency-group-call> element of the <ruleset> element of the MCPTT user profile document identified by the MCPTT ID of the calling user (see the MCPTT user profile document in 3GPP TS 24.384 [50]) is set to a value of "true" and:

a)	if the "entry-info" attribute of the <entry> element of the <MCPTTGroupInitiation> element contained within the <EmergencyCall> element of the MCPTT user profile document (see the MCPTT user profile document in 3GPP TS 24.384 [50]) is set to a value of "DedicatedGroup" and if the <uri-entry> element of the <entry> element of the <MCPTTGroupInitiation> element contains the identity of the MCPTT group targeted by the calling MCPTT user; or

b)	if the "entry-info" attribute of the <entry> element of the <MCPTTGroupInitiation> element is set to a value of "UseCurrentlySelectedGroup";

then the participating MCPTT function shall consider the MCPTT emergency group call request to be an authorised request for an MCPTT emergency group call;

In all other cases, the participating MCPTT function shall consider the request to originate an MCPTT emergency group call to be an unauthorised request to originate an MCPTT emergency group call.

When the participating MCPTT function receives a request from the MCPTT client to originate an MCPTT imminent peril group call and needs to determine if the request is an authorised request for an MCPTT imminent peril group call the participating MCPTT function shall check the following:

1)	if the <allow-imminent-peril-call> element of <ruleset> element of the MCPTT user profile document identified by the MCPTT ID of the calling user (see the MCPTT user profile document in 3GPP TS 24.384 [50]) is set to a value of "true"; and

a)	if the "entry-info" attribute of the <MCPTTGroupInitiation> element contained within the <ImminentPerilCall> element of the MCPTT user profile document (see the MCPTT user profile document in 3GPP TS 24.384 [50]) is set to a value of "DedicatedGroup" and if the <uri-entry> element of the <entry> element of the <MCPTTGroupInitiation> element contains the identity of the MCPTT group targeted by the calling MCPTT user; or

b)	if the "entry-info" attribute of the <entry> element of the <MCPTTGroupInitiation> element is set to a value of "UseCurrentlySelectedGroup";

then the participating MCPTT function shall consider the MCPTT imminent peril group call request to be an authorised request for an MCPTT emergency group call;

In all other cases, the participating MCPTT function shall consider the request to originate an MCPTT imminent peril group call to be an unauthorised request to originate an MCPTT imminent peril call.

##### 6.3.2.1.9	Generating a SIP re-INVITE request on receipt of a SIP re-INVITE request

This subclause is referenced from other procedures.

When generating a SIP re-INVITE request according to 3GPP TS 24.229 [4] on receipt of an incoming SIP re-INVITE request, the participating MCPTT function:

1)	if the incoming SIP re-INVITE request contained a MIME resource-lists body with the MCPTT ID of the invited MCPTT user, shall copy the MIME resource-lists body, according to rules and procedures of IETF RFC 5366 [20];

2)	if the incoming SIP re-INVITE request contained an application/vnd.3gpp.mcptt-info+xml MIME body, shall copy the application/vnd.3gpp.mcptt-info+xml MIME body; and

3)	if the incoming SIP re-INVITE request contained an application/vnd.3gpp.location-info+xml MIME body, shall copy the application/vnd.3gpp.location-info+xml MIME body.

##### 6.3.2.1.10	Sending a SIP INVITE request on receipt of SIP 3xx response

This subclause is referenced from other procedures.

Upon:

1)	receipt of a SIP INVITE request or SIP REFER request from the MCPTT client;

2)	having sent a SIP INVITE request to the controlling MCPTT function; and

3)	having received a SIP 302 (Moved Temporarily) response from the controlling MCPTT function with:

a)	a Contact header field containing a SIP-URI; and

b)	an application/vnd.3gpp.mcptt-info+xml MIME body with an <mcptt-request-uri> element;

the participating MCPTT function:

1)	shall generate a SIP INVITE request with the Request-URI set to the contents of the Contact header field of the SIP 302 (Moved Temporarily) response;

2)	shall include in the SIP INVITE request all Accept-Contact header fields and all Reject-Contact header fields, with their feature tags and their corresponding values along with parameters according to rules and procedures of IETF RFC 3841 [6] if included in the original incoming SIP INVITE or SIP REFER request from the MCPTT client;

3)	should include the Session-Expires header field according to IETF RFC 4028 [7]. It is recommended that the "refresher" header field parameter is omitted. If included, the "refresher" header field parameter shall be set to "uac";

4)	shall include the option tag "timer" in the Supported header field;

5)	shall copy the contents of the P-Asserted-Identity header field of the incoming SIP INVITE or SIP REFER request from the client to the P-Asserted-Identity header field of the outgoing SIP INVITE request;

6)	shall include the g.3gpp.mcptt media feature tag into the Contact header field of the outgoing SIP INVITE request;

7)	shall include the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" in the Contact header field of the outgoing SIP INVITE request;

8)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), into the P-Asserted-Service header field of the outgoing SIP INVITE request;

9)	if an SIP INVITE request was received from the client containing an application/vnd.3gpp.mcptt-info+xml MIME body, shall copy the contents of the application/vnd.3gpp.mcptt-info+xml MIME body of the original incoming SIP INVITE request to the outgoing SIP INVITE request;

10)	if a SIP REFER request was received from the client with a "cid" URL pointing to an application/resource-lists MIME body as specified in IETF RFC 5366 [20] containing SIP-URI with a "body" URI header field containing an application/vnd.3gpp.mcptt-info MIME body, shall copy the contents of the application/vnd.3gpp.mcptt-info+xml MIME body un the INVITE request to the outgoing SIP INVITE request;

11)	shall copy the contents of the <mcptt-request-uri> element of the application/vnd.3gpp.mcptt-info+xml MIME body received in the SIP 302 (Moved Temporarily) response, to the <mcptt-request-uri> element of the application/vnd.3gpp.mcptt-info+xml MIME body of the outgoing SIP INVITE request;

12)	shall set the <mcptt-calling-user-identity> element of the application/vnd.3gpp.mcptt-info+xml MIME body of the SIP INVITE request to the MCPTT ID of the calling user that was determined when the participating MCPTT function received the SIP INVITE request or SIP REFER request from the client ; and

13)	if the <session-type> element is received in the application/vnd.3gpp.mcptt-info+xml MIME body of the SIP 3xx response, shall set the <session-type> element of the application/vnd.3gpp.mcptt-info+xml MIME body of the SIP INVITE request to the value of the <session-type> element received in the SIP 3xx response.

#### 6.3.2.2	Requests terminated to the served MCPTT user

##### 6.3.2.2.1	SDP offer generation

The participating MCPTT function shall follow the procedures in subclause 6.3.2.1.1.

##### 6.3.2.2.2	SDP answer generation

###### 6.3.2.2.2.1	On-demand session

The participating MCPTT function shall follow the procedures in subclause 6.3.2.1.2.1.

###### 6.3.2.2.2.2	Pre-established session

When composing an SDP answer according to 3GPP TS 24.229 [4], the MCPTT server:

1)	shall set the IP address of the MCPTT server for the accepted MCPTT speech media stream from the received SDP offer, which was also negotiated during the pre-established session establishment;

2)	shall set the IP address of the MCPTT server for the accepted media-floor control entity from the received SDP offer, which was also negotiated during the pre-established session establishment, if present in the received SDP offer;

3)	shall include the media-level section for the accepted MCPTT speech media stream from the received SDP offer, which was also negotiated in pre-established session establishment, consisting of:

a)	the port number for MCPTT speech; and

b)	the codec(s) and media parameters selected by the MCPTT server from the received SDP offer; and

4)	shall include for the media-floor control entity, that is offered in the SDP offer from the MCPTT server and accepted in the SDP answer by MCPTT client, the media-level section of each offered media-floor control entity consisting of:

a)	the media-floor control entity parameters contained in the received SDP offer, restricted to media-floor control entity parameters negotiated during the pre-established session establishment; and

b)	the port number for selected media-floor control entity selected as specified in 3GPP TS 24.229 [4].

##### 6.3.2.2.3	SIP INVITE request towards the terminating MCPTT client

The participating MCPTT function shall generate an initial SIP INVITE request according to 3GPP TS 24.229 [4] and:

1)	shall include in the SIP INVITE request all Accept-Contact header fields and all Reject-Contact header fields, with their feature tags and their corresponding values along with parameters according to rules and procedures of IETF RFC 3841 [6] if included in the incoming SIP INVITE request;

2)	should include the Session-Expires header field according to IETF RFC 4028 [7]. It is recommended that the "refresher" header field parameter is omitted. If included, the "refresher" header field parameter shall be set to "uac";

3)	shall include the option tag "timer" in the Supported header field;

4)	shall include the following in the Contact header field:

a)	the g.3gpp.mcptt media feature tag;

b)	the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt";

c)	the isfocus media feature tag;

d)	an MCPTT session identity mapped to the MCPTT session identity provided in the Contact header field of the incoming SIP INVITE request; and

e)	any other uri-parameter provided in the Contact header field of the incoming SIP INVITE request;

5)	shall include the option tag "tdialog" in a Supported header field according to rules and procedures of IETF RFC 4538 [23];

6)	shall include the option tag "norefersub" in a Supported header field according to rules and procedures of IETF RFC 4488 [22];

7)	may include a Resource-Share header field in accordance with subclause 5.7.1.20.3 in 3GPP TS 24.229 [4]; and

8)	if the incoming SIP INVITE request contained an application/vnd.3gpp.mcptt-info+xml MIME body, shall copy the application/vnd.3gpp.mcptt-info+xml MIME body to the outgoing SIP INVITE request.

##### 6.3.2.2.4	Response to a SIP INVITE request

###### 6.3.2.2.4.1	Provisional response

This subclause is referenced from other procedures.

When sending a SIP provisional response other than the SIP 100 (Trying) response to the SIP INVITE request, the participating MCPTT function shall generate a SIP provisional response according to 3GPP TS 24.229 [4] and:

1)	shall include the following in the Contact header field:

a)	the g.3gpp.mcptt media feature tag; and

b)	the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt";

2)	if the outgoing SIP provisional response is to be sent in response to the receipt of a SIP provisional response and the response contains an application/vnd.3gpp.mcptt-info+xml MIME body, shall copy the application/vnd.3gpp.mcptt-info+xml MIME body to the outgoing SIP provisional response; and

3)	if the incoming SIP INVITE request included the Supported header field with the value "100rel" and according to local policy, may include the Require header field with the value "100rel".

###### 6.3.2.2.4.2	Final response

This subclause is referenced from other procedures.

When sending SIP 200 (OK) responses, the participating MCPTT function shall generate a SIP 200 (OK) response according to 3GPP TS 24.229 [4] and:

1)	shall include the option tag "timer" in a Require header field;

2)	shall include the Session-Expires header field according to rules and procedures of IETF RFC 4028 [7], "UAS Behavior". If no "refresher" parameter was included in the SIP INVITE request, the "refresher" parameter in the Session-Expires header field shall be set to "uas";

3)	shall include the following in the Contact header field:

a)	the g.3gpp.mcptt media feature tag;

b)	the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt"; and

c)	an MCPTT session identity mapped to the MCPTT session identity provided in the Contact header field of the received SIP INVITE request from the controlling MCPTT function;

4)	shall include the option tag "tdialog" in a Supported header field according to rules and procedures of IETF RFC 4538 [23]; and

5)	if the incoming SIP response contained an application/vnd.3gpp.mcptt-info+xml MIME body, shall copy the application/vnd.3gpp.mcptt-info+xml MIME body to the outgoing SIP 200 (OK) response.

##### 6.3.2.2.5	Automatic Commencement Mode

###### 6.3.2.2.5.1	General

When receiving a "SIP INVITE request for terminating participating MCPTT function" that requires automatic commencement mode:

1)	if:

a)	the invited MCPTT client has one or more pre-established sessions without an associated MCPTT session;

b)	the media-level section for the offered MCPTT speech media stream is the same as the media-level section for MCPTT speech media stream in an existing pre-established session; and

c)	the media-level section of the offered media-floor control entity is the same as the media-level section for media-floor control entity in an existing pre-established session;

then the participating MCPTT function shall perform the actions specified in subclause 6.3.2.2.5.3; or

2)	otherwise the participating MCPTT function shall perform the actions specified in subclause 6.3.2.2.5.2.

###### 6.3.2.2.5.2	Automatic commencement for On-Demand session

When receiving a "SIP INVITE request for terminating participating MCPTT function" for an on-demand session that requires automatic commencement mode the participating MCPTT function:

1)	if:

a)	the incoming SIP INVITE request contained a Priv-Answer-Mode header field set to the value of "Auto";

b)	no Answer-Mode header field or Priv-Answer-Mode header field were received in the incoming SIP INVITE request and the Answer-Mode Indication received in the application/poc-settings+xml MIME body received from the invited MCPTT client as defined in subclause 7.3.3 or subclause 7.3.4 is set to "auto-answer"; or

c)	the incoming SIP INVITE request contained an Answer-Mode header field set to "Auto" and the Answer-Mode Indication received in the application/poc-settings+xml MIME body received from the invited MCPTT client as defined in subclause 7.3.3 or subclause 7.3.4 is set to "auto-answer";

then:

a)	shall generate a SIP 183 (Session Progress) response to the "SIP INVITE request for terminating participating MCPTT function" as specified in subclause 6.3.2.2.4.1; and

NOTE:	The SIP 183 (session Progress) response can be sent reliably or unreliably depending on the content of the received SIP INVITE request. Regardless of if the SIP 183 (Session Progress) response is sent reliably or unreliably, SDP is not included in the SIP 183 (Session Progress) response.

b)	shall set the P-Answer-State header field to "Unconfirmed" in the SIP 183 (Session Progress) response;

2)	shall copy the public user identity contained in the Request-URI of the incoming "SIP INVITE request for terminating participating MCPTT function" to the P-Asserted-Identity header field of the SIP 183 (Session Progress) response;

3)	shall generate a SIP INVITE request as specified in subclause 6.3.2.2.3;

4)	shall set the Request-URI to the public user identity associated to the MCPTT ID of the MCPTT user to be invited;

5)	shall perform the procedures specified in subclause 6.3.2.2.9 to include any MIME bodies in the received SIP INVITE request, into the outgoing SIP INVITE request;

6)	shall copy the contents of the P-Asserted-Identity header field of the incoming "SIP INVITE request for terminating participating MCPTT function" to the P-Asserted-Identity header field of the outgoing SIP INVITE request;

7)	if the Priv-Answer-Mode header field is present in the incoming SIP INVITE request with a value of "Auto", shall include a Priv-Answer-Mode header field with the value "Auto" in the outgoing SIP INVITE request. Otherwise, if the Answer-Mode header field is present in the incoming SIP INVITE request, the participating MCPTT function shall include an Answer-Mode header field with the value "Auto" in the outgoing SIP INVITE request;

8)	if no Answer-Mode header field or Priv-Answer-Mode header field were received in the incoming SIP INVITE request and the Answer-Mode Indication received in the application/poc-settings+xml MIME body received from the invited MCPTT client as defined in subclause 7.3.3 or subclause 7.3.4 is set to "auto-answer", shall set the Answer-Mode header field to "Auto" in the outgoing SIP INVITE request;

9)	shall include in the SIP INVITE request an SDP offer based on the SDP offer in the received "SIP INVITE request for terminating participating MCPTT function" as specified in subclause 6.3.2.2.1;

10)	if the received SIP INVITE request contains a Resource-Priority header field, shall include a Resource-Priority header field with the contents set as in the received Resource-Priority header field; and

11)	shall send the SIP INVITE request towards the MCPTT client according to 3GPP TS 24.229 [4].

If the SIP 183 (Session Progress) response was sent reliably, then upon receiving a SIP PRACK request, the participating MCPTT function shall generate a SIP 200 (OK) response to the SIP PRACK request and forward the SIP 200 (OK) response, according to 3GPP TS 24.229 [4].

Upon receiving a SIP 200 (OK) response to the above SIP INVITE request sent to the MCPTT client, the participating MCPTT function:

1)	if the SIP 183 (Session Progress) was sent unreliably, shall send the SIP 200 (OK) response immediately; and

2)	if the SIP 183 (Session Progress) was sent reliably and,

a)	if the SIP PRACK request to the SIP 183 (Session Progress) response has been received by the participating MCPTT function and the SIP 200 (OK) response to the SIP PRACK request has been sent, shall send the SIP 200 (OK) response immediately;

b)	if the SIP PRACK request to the SIP 183 (Session Progress) response has not yet been received, then upon receipt of the SIP PRACK request, the participating MCPTT function shall generate a SIP 200 (OK) response to the SIP PRACK request and forward the SIP 200 (OK) response, according to 3GPP TS 24.229 [4], before sending the SIP 200 (OK) response to the "SIP INVITE request for terminating participating MCPTT function".

When the participating MCPTT function sends the SIP 200 (OK) response to the "SIP INVITE request for terminating participating MCPTT function", the participating MCPTT function:

1)	shall generate a SIP 200 (OK) response as described in the subclause 6.3.2.2.4.2;

2)	shall include in the SIP 200 (OK) response an SDP answer based on the SDP answer in the received SIP 200 (OK) response as specified in subclause 6.3.2.2.2.1;

3)	shall copy the P-Asserted-Identity header field from the incoming SIP 200 (OK) response to the outgoing SIP 200 (OK) response;

4)	shall interact with the media plane as specified in 3GPP TS 24.380 [5]; and

5)	shall forward the SIP 200 (OK) response according to 3GPP TS 24.229 [4].

The participating MCPTT function shall forward any other SIP response that does not contain SDP along the signalling path to the originating network according to 3GPP TS 24.229 [4].

###### 6.3.2.2.5.3	Automatic commencement for pre-established session

NOTE:	This subclause is referenced from other procedures.

When receiving a "SIP INVITE request for terminating participating MCPTT function" for a pre-established session that requires automatic commencement mode the participating MCPTT function:

1)	if the received SIP INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcpttinfo> element containing the <mcptt-Params> element with the <emergency-ind> element set to a value of "true":

a)	shall perform the procedures of subclause 6.3.2.2.5.2 with the following clarifications:

i)	include in the outgoing SIP INVITE request a SIP Replaces header field populated with the call-id, to-tag and from-tag of the targeted pre-established session as specified in IETF RFC 3891 [65]; and

ii)	include in the SDP offer the current media parameters used by the targeted pre-established session identified by the Replaces header field; and

b)	shall skip the remaining steps;

2)	shall generate a SIP 200 (OK) response to the "SIP INVITE request for terminating participating MCPTT function" as described in the subclause 6.3.2.2.4.2;

3)	shall include in the SIP 200 (OK) response an SDP answer as specified in the subclause 6.3.2.2.2.2 based on the SDP negotiated during the pre-established session establishment and SDP offer received in the "SIP INVITE request for terminating participating MCPTT function";

4)	shall set the P-Answer-State header field to "Unconfirmed" in the SIP 200 (OK) response;

5)	shall set the P-Asserted-Identity header field to the same value as included in the SIP INVITE request that was sent by the MCPTT client when the pre-established session was established, in the SIP 200 (OK) response;

6)	shall send the SIP 200 (OK) response to the SIP INVITE request according to 3GPP TS 24.229 [4]; and

7)	shall interact with the media plane as specified in 3GPP TS 24.380 [5] clause 9.

##### 6.3.2.2.6	Manual Commencement Mode

###### 6.3.2.2.6.1	General

When receiving a "SIP INVITE request for terminating participating MCPTT function" that requires manual commencement mode:

1)	if:

a)	the invited MCPTT client has one or more pre-established sessions without an associated MCPTT session;

b)	the media-level section for the offered MCPTT speech media stream is the same as the media-level section for MCPTT speech media stream in the existing pre-established session; and

c)	the media-level section of the offered media-floor control entity is the same as the media-level section for media-floor control entity in the existing pre-established session;

then the participating MCPTT function shall perform the actions specified in subclause 6.3.2.2.6.3; or

2)	otherwise the participating MCPTT function shall perform the actions specified in subclause 6.3.2.2.6.2.

###### 6.3.2.2.6.2	Manual commencement for On-Demand session

When receiving a "SIP INVITE request for terminating participating MCPTT function" for an on-demand session that requires manual commencement mode the participating MCPTT function:

1)	shall generate a SIP INVITE request as specified in subclause 6.3.2.2.3;

2)	shall set the Request-URI to the public user identity associated to the MCPTT ID of the MCPTT user to be invited;

3)	shall perform the procedures specified in subclause 6.3.2.2.9 to include any MIME bodies in the received SIP INVITE request;

4)	if the Answer-Mode header field is present in the incoming SIP INVITE request, participating MCPTT function, shall include an Answer-Mode header field with the value "Manual";

5)	if no Answer-Mode header field was received in the incoming SIP INVITE request and the Answer-Mode Indication received in the application/poc-settings+xml MIME body received from the invited MCPTT client as defined in subclause 7.3.3 or subclause 7.3.4 is set to "manual-answer", shall set the Answer-Mode header field to "Manual" in the outgoing SIP INVITE request;

6)	shall copy the contents of the P-Asserted-Identity header field of the incoming "SIP INVITE request for terminating participating MCPTT function" to the P-Asserted-Identity header field of the outgoing SIP INVITE request;

7)	shall include in the SIP INVITE request an SDP offer based on the SDP offer in the received "SIP INVITE request for terminating participating MCPTT function" as specified in subclause 6.3.2.2.1;

8)	if the received SIP INVITE request contains a Resource-Priority header field, shall include a Resource-Priority header field with the contents set as in the received Resource-Priority header field; and

9)	shall send the SIP INVITE request towards the MCPTT client according to 3GPP TS 24.229 [4].

Upon receiving a SIP 180 (Ringing) response to the above SIP INVITE request, the participating MCPTT function:

NOTE 1:	A SIP 180 (Ringing) response is received from a terminating MCPTT client in the case of a private call.

1)	shall generate a SIP 180 (Ringing) response as specified in subclause 6.3.2.2.4.1;

2)	shall include the P-Asserted-Identity header field as received in the incoming SIP 180 (Ringing) response; and

3)	shall forward the SIP 180 (Ringing) response according to 3GPP TS 24.229 [4].

Upon receiving a SIP 183 (Session Progress) response to the above SIP INVITE request, the participating MCPTT function:

NOTE 2:	A SIP 183 (Session Progress) response can be received from a terminating MCPTT client in the case of a group call.

1)	shall generate a SIP 183 (Session Progress) response as specified in subclause 6.3.2.2.4.1;

2)	shall include the P-Asserted-Identity header field as received in the incoming SIP 183 (Session Progress) response;

3)	shall include the P-Answer-State header field if received in the incoming SIP 183 (Session Progress) request; and

4)	shall forward the SIP 183 (Session Progress) response according to 3GPP TS 24.229 [4].

Upon receiving a SIP 200 (OK) response to the SIP INVITE request sent to the MCPTT client, the participating MCPTT function:

When the participating MCPTT function sends the SIP 200 (OK) response the participating MCPTT function:

1)	shall generate a SIP 200 (OK) response as described in the subclause 6.3.2.2.4.2;

2)	shall include in the SIP 200 (OK) response an SDP answer based on the SDP answer in the received SIP 200 (OK) response as specified in subclause 6.3.2.2.2.1;

3)	shall copy the P-Asserted-Identity header field from the incoming SIP 200 (OK) response to the outgoing SIP 200 (OK) response;

4)	shall interact with the media plane as specified in 3GPP TS 24.380 [5] subclause 6.4; and

5)	shall forward the SIP 200 (OK) response according to 3GPP TS 24.229 [4].

The participating MCPTT function shall forward any other SIP response that does not contain SDP along the signalling path to the originating network according to 3GPP TS 24.229 [4].

###### 6.3.2.2.6.3	Manual commencement for Pre-established session

When receiving a "SIP INVITE request for terminating participating MCPTT function" for a pre-established session that requires manual commencement mode the participating MCPTT function:

1)	if the received SIP INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcpttinfo> element containing the <mcptt-Params> element with the <emergency-ind> element set to a value of "true":

a)	shall perform the procedures of subclause 6.3.2.2.6.2 with the following clarifications:

i)	include in the outgoing SIP INVITE request a SIP Replaces header field populated with the call-id, to-tag and from-tag of the targeted pre-established session as specified in IETF RFC 3891 [65]; and

ii)	include in the SDP offer the current media parameters used by the targeted pre-established session identified by the Replaces header field; and

b)	shall skip the remaining steps;

2)	shall generate a SIP re-INVITE request as described in subclause 6.3.2.2.3;

NOTE 1:	A SIP re-INVITE request cannot include an Answer-Mode header field as specified in IETF RFC 5373 [18] so Manual Answer is implied with a SIP re-INVITE request within the existing SIP dialog of the pre-established session.

3)	shall copy the contents of the application/vnd.3gpp.mcptt-info+xml MIME body of the incoming "SIP INVITE request for terminating participating MCPTT function" to the outgoing SIP re-INVITE request;

4)	shall include in the SIP re-INVITE request an SDP offer based on the SDP offer in the received SIP INVITE request as specified in the subclause 6.3.2.2.1; and

5)	shall send the SIP re-INVITE request towards the MCPTT client according to 3GPP TS 24.229 [4];

Upon receiving a SIP 180 (Ringing) response to the above SIP re-INVITE request, the participating MCPTT function:

NOTE 2:	A SIP 180 (Ringing) response is received from a terminating MCPTT client in the case of a private call.

1)	shall generate a SIP 180 (Ringing) response as specified in subclause 6.3.2.2.4.1;

2)	shall include the P-Asserted-Identity header field as received in the incoming SIP 180 (Ringing) response; and

3)	shall forward the SIP 180 (Ringing) response according to 3GPP TS 24.229 [4].

Upon receiving a SIP 183 (Session Progress) response to the above SIP re-INVITE request, the participating MCPTT function:

NOTE 3:	A SIP 183 (Session Progress) response can be received from a terminating MCPTT client in the case of a group call.

1)	shall generate a SIP 183 (Session Progress) response as specified in subclause 6.3.2.2.4.1;

2)	shall include the P-Asserted-Identity header field as received in the incoming SIP 183 (Session Progress) response;

3)	shall include the P-Answer-State header field as received in the incoming SIP 183 (Session Progress) request; and

4)	shall forward the SIP 183 (Session Progress) response according to 3GPP TS 24.229 [4].

Upon receiving a SIP 200 (OK) response to the SIP re-INVITE request, the participating MCPTT function:

1)	if the received SDP answer includes changes in codecs or media formats, shall interact with the media plane as specified in 3GPP TS 24.380 [5] for updating the media plane with the newly negotiated codecs and media parameters from the received SDP answer;

2)	shall generate a SIP 200 (OK) response as described in the subclause 6.3.2.2.4.2;

3)	shall copy the P-Asserted-Identity header field from the incoming SIP 200 (OK) response to the outgoing SIP 200 (OK) response;

4)	shall include in the SIP 200 (OK) response, an SDP answer based on the SDP answer in the received SIP 200 (OK) response, as specified in subclause 6.3.2.2.2.1;

5)	shall interact with the media plane as specified in 3GPP TS 24.380 [5]; and

NOTE 4:	The participating MCPTT function sends a MCCP Connect message, in order to give MCPTT session identity to the terminating MCPTT client.

6)	shall send the SIP 200 (OK) response to the SIP INVITE request according to 3GPP TS 24.229 [4].

##### 6.3.2.2.7	SIP MESSAGE request towards the terminating MCPTT client

This subclause is referenced from other procedures.

When receiving a SIP MESSAGE request containing:

-	an application/vnd.3gpp.mcptt-info+xml MIME body; and

-	a text/plain MIME body;

the participating MCPTT function:

1)	shall verify if a binding exists between the public user identity in the Request-URI of the SIP MESSAGE request and an MCPTT ID;

2)	if a binding does not exist between the public user identity and a MCPTT ID or a binding exists, but the validity period of the binding has expired, shall reject the SIP MESSAGE request with a SIP 404 (Not Found) response and not continue with any of the remaining steps;

3)	shall verify that the MCPTT ID in the <mcptt-request-uri> matches the binding in the participating MCPTT function, and if the match fails, shall reject the SIP MESSAGE request with a SIP 404 (Not Found) response and not continue with any of the remaining steps;

4)	shall generate a SIP MESSAGE request accordance with 3GPP TS 24.229 [4] and IETF RFC 3428 [33];

5)	shall include in the SIP MESSAGE request all Accept-Contact header fields and all Reject-Contact header fields, with their feature tags and their corresponding values along with parameters according to rules and procedures of IETF RFC 3841 [6] that were received (if any) in the incoming SIP MESSAGE request;

6)	shall set the Request-URI of the outgoing SIP MESSAGE request to the public user identity that was in the Request-URI of the incoming SIP MESSAGE request;

7)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body with a <mcptt-request-uri> element set to the MCPTT ID of the MCPTT user that was in the <mcptt-request-uri> element of the application/vnd.3gpp.mcptt-info+xml MIME body of the incoming SIP MESSAGE request;

8)	shall copy the "text/plain" MIME body from the incoming SIP MESSAGE request to the outgoing SIP MESSAGE request;

9)	shall copy the contents of the P-Asserted-Identity header field of the incoming SIP MESSAGE request to the P-Asserted-Identity header field of the outgoing SIP INVITE request; and

10)	shall send the SIP MESSAGE request towards the MCPTT client according to rules and procedures of 3GPP TS 24.229 [4].

##### 6.3.2.2.8	SIP BYE request towards the terminating MCPTT client

###### 6.3.2.2.8.1	On-demand

Upon receiving a SIP BYE request from the controlling MCPTT function, the participating MCPTT function:

1)	shall interact with the media plane as specified in subclause 6.4 in 3GPP TS 24.380 [5] for releasing media plane resource associated with the SIP session with the controlling MCPTT function;

2)	shall generate a SIP BYE request according to 3GPP TS 24.229 [4];

3)	shall copy the contents of the P-Asserted-Identity header field of the incoming SIP BYE request to the P-Asserted-Identity header field of the outgoing SIP BYE request; and

4)	shall send the SIP BYE request to the MCPTT client according to 3GPP TS 24.229 [4].

Upon receiving a SIP 200 (OK) response to the SIP BYE request the participating MCPTT function:

1)	shall send a SIP 200 (OK) response to the SIP BYE request received from the controlling MCPTT function according to 3GPP TS 24.229 [4]; and

2)	shall interact with the media plane as specified in 3GPP TS 24.380 [5] for releasing media plane resources associated with the SIP session with the MCPTT client.

###### 6.3.2.2.8.2	Using pre-established session

Upon receiving a SIP BYE request from the controlling MCPTT function, the participating MCPTT function:

1)	shall interact with the media plane as specified in subclause 9.3 in 3GPP TS 24.380 [5] for disconnecting the media plane resources towards the controlling MCPTT function;

2)	shall send a SIP 200 (OK) response to the controlling MCPTT function;

3)	shall interact with the media plane as specified in 3GPP TS 24.380 [5] for disconnecting media plane resources towards the MCPTT client from the media plane resources towards the controlling MCPTT function; and

4)	shall maintain the pre-established session towards the MCPTT client.

###### 6.3.2.2.9	Populate MIME bodies

This subclause is referenced from other procedures.

If the incoming SIP request contains an application/vnd.3gpp.mcptt-info+xml MIME body, the participating MCPTT function:

1)	if not already copied, shall copy the contents of the application/vnd.3gpp.mcptt-info+xml MIME body received in the SIP request into an application/vnd.3gpp.mcptt-info+xml MIME body included in the outgoing SIP request.

If the received SIP request contains an application/vnd.3gpp.location-info+xml MIME body as specified in Annex F.3:

1)	if not already copied, shall copy the contents of the application/vnd.3gpp.location-info+xml MIME body received in the SIP request into an application/vnd.3gpp.location-info+xml MIME body included in the outgoing SIP request.

If the received SIP request contains an application/resource-lists+xml MIME body:

1)	if not already copied, shall copy the contents of the application/resource-lists+xml MIME body received in the SIP request into an application/resource-lists+xml MIME body included in the outgoing SIP request.

##### 6.3.2.2.10	Generating a SIP re-INVITE request towards the terminating MCPTT client

This subclause is referenced from other procedures.

The participating MCPTT function shall generate a SIP re-INVITE request according to 3GPP TS 24.229 [4] and:

1)	shall include the option tag "tdialog" in a Supported header field according to rules and procedures of IETF RFC 4538 [23];

2)	may include a Resource-Share header field in accordance with subclause 5.7.1.20.3 in 3GPP TS 24.229 [4];

3)	shall perform the procedures specified in subclause 6.3.2.2.9 to copy any MIME bodies in the received SIP re-INVITE request to the outgoing SIP re-INVITE request; and

4)	if the received SIP re-INVITE request contains a Resource-Priority header field, shall include a Resource-Priority header field with the contents set as in the received Resource-Priority header field.

##### 6.3.2.2.11	Generating a SIP MESSAGE request towards the terminating MCPTT client

This subclause is referenced from other procedures.

The participating MCPTT function shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [4] and IETF RFC 3428 [33] and:

1)	shall include in the SIP MESSAGE request all Accept-Contact header fields and all Reject-Contact header fields, with their feature tags and their corresponding values along with parameters according to rules and procedures of IETF RFC 3841 [6] that were received (if any) in the incoming SIP MESSAGE request;

2)	shall set the Request-URI of the outgoing SIP MESSAGE request to the public user identity associated to the MCPTT ID of the MCPTT user that was in the Request-URI of the incoming SIP MESSAGE request;

3)	shall populate the outgoing SIP MESSAGE request MIME bodies as specified in subclause 6.3.2.2.9; and

4)	shall copy the contents of the P-Asserted-Identity header field of the incoming SIP MESSAGE request to the P-Asserted-Identity header field of the outgoing SIP MESSAGE request.

### 6.3.3	Controlling MCPTT function

#### 6.3.3.1	Request initiated by the controlling MCPTT function

##### 6.3.3.1.1	SDP offer generation

The SDP offer is generated based on the received SDP offer. The SDP offer generated by the controlling MCPTT function:

1)	when initiating a new MCPTT session the SDP offer;

a)	shall contain only one SDP media-level section for MCPTT speech media stream as contained in the received SDP offer; and

b)	shall contain an SDP media-level section for one media-floor control entity, if present in the received SDP offer; and

2)	when adding a new MCPTT user to an existing MCPTT Session, the SDP offer shall contain the media stream currently used in the MCPTT session.

When composing the SDP offer according to 3GPP TS 24.229 [4], the controlling MCPTT function:

1)	shall replace the IP address and port number for the offered media stream in the received SDP offer with the IP address and port number of the controlling MCPTT function;

2)	for the MCPTT speech media stream, shall include all media-level attributes from the received SDP offer;

3)	shall replace the IP address and port number for the offered media floor control entity, if any, in the received SDP offer with the IP address and port number of the controlling MCPTT function; and

4)	for the offered media floor control entity, shall include the offered media floor control entity 'fmtp' attributes as specified in 3GPP TS 24.380 [5] clause 14.

##### 6.3.3.1.2	Sending an INVITE request

This subclause is referenced from other procedures.

The controlling MCPTT function shall generate an initial SIP INVITE request according to 3GPP TS 24.229 [4].

The controlling MCPTT function:

1)	shall include in the Contact header field an MCPTT session identity for the MCPTT session with the g.3gpp.mcptt media feature tag, the isfocus media feature tag and the g.3gpp.icsi-ref media feature tag with the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" according to IETF RFC 3840 [16];

2)	shall include an Accept-Contact header field containing the g.3gpp.mcptt media feature tag along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [6];

3)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Asserted-Service-Id header field according to IETF RFC 6050 [9] in the SIP INVITE request;

4)	shall include an Accept-Contact header field with the media feature tag g.3gpp.icsi-ref with the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" along with parameters "require" and "explicit" according to IETF RFC 3841 [6];

5)	shall include a Referred-By header field with the public user identity of the inviting MCPTT client;

6)	should include the Session-Expires header field according to rules and procedures of IETF RFC 4028 [7]. The refresher parameter shall be omitted;

7)	shall include the Supported header field set to "timer";

8)	shall include an unmodified Answer-Mode header field if present in the incoming SIP INVITE request; and

9)	if the incoming SIP INVITE request contained an application/vnd.3gpp.mcptt-info+xml MIME body, shall copy the application/vnd.3gpp.mcptt-info+xml MIME body to the outgoing INVITE request.

##### 6.3.3.1.3	Receipt of a SIP response to a SIP INVITE request

###### 6.3.3.1.3.1	Final response

On receipt of the SIP 200 (OK) response to the initial outgoing SIP INVITE request the controlling MCPTT function:

1)	shall start the SIP session timer according to rules and procedures of IETF RFC 4028 [7]; and

2)	shall cache SIP feature tags, if received in the Contact header field, and if the specific feature tags are supported.

##### 6.3.3.1.4	Sending a SIP MESSAGE request

This subclause is referenced from other procedures.

The controlling MCPTT function:

1)	shall generate an SIP MESSAGE request in accordance with 3GPP TS 24.229 [4] and IETF RFC 3428 [33];

2)	shall include an Accept-Contact header field containing the g.3gpp.mcptt media feature tag along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [6];

3)	shall include an Accept-Contact header field with the media feature tag g.3gpp.icsi-ref with the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" along with parameters "require" and "explicit" according to IETF RFC 3841 [6];

4)	shall set the Request-URI to a public service identity identifying the terminating participating MCPTT function associated with the MCPTT ID of the MCPTT user;

5)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body with a <mcptt-request-uri> element set to the MCPTT ID of the MCPTT user;

6)	shall include a "text/plain" MIME body with the required text to render to the MCPTT user;

7)	shall include in the P-Asserted-Identity header field, the public service identity of the controlling MCPTT function; and

8)	shall send the SIP MESSAGE request towards the MCPTT client according to rules and procedures of 3GPP TS 24.229 [4].

##### 6.3.3.1.5	Sending a SIP BYE request

When a participant needs to be removed from the MCPTT session, the controlling MCPTT function:

1)	shall interact with the media plane as specified in 3GPP TS 24.380 [5] for the MCPTT session release;

2)	shall generate a SIP BYE request according to 3GPP TS 24.229 [4]; and

3)	shall send the SIP BYE request to the MCPTT clients according to3GPP TS 24.229 [4].

If timer TNG3 (group call timer) has not expired, then when the last MCPTT client is removed from the MCPTT session, the controlling MCPTT function shall stop timer TNG3 (group call timer).

When the MCPTT group session needs to be released, the controlling MCPTT function shall send SIP BYE requests as described in this subclause, to all participants of the group session.

Upon receiving a SIP 200 (OK) response to a SIP BYE request the controlling MCPTT function shall interact with the media plane as specified in subclause 6.3 in 3GPP TS 24.380 [5] for releasing media plane resources associated with the SIP session with the MCPTT clients.

##### 6.3.3.1.6	Sending a SIP re-INVITE request for MCPTT emergency group call

This subclause is referenced from other procedures.

The controlling MCPTT function shall generate a SIP re-INVITE request according to 3GPP TS 24.229 [4].

The controlling MCPTT function:

1)	shall include an SDP offer with the media parameters as currently established with the terminating MCPTT client according to 3GPP TS 24.229 [4];

2)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcptt-calling-user-id> element set to the MCPTT ID of the initiating MCPTT user;

3)	if the in-progress emergency group state of the group is set to a value of "true" the controlling MCPTT function:

a)	shall include a Resource-Priority header field with the namespace populated with the values for an MCPTT emergency group call as specified in subclause 6.3.3.1.19;

b)	shall include in the application/vnd.3gpp.mcptt-info+xml MIME body the <emergency-ind> element set to a value of "true";

c)	if the <alert-ind> element is set to "true" in the received SIP re-INVITE request and MCPTT emergency alerts are authorised for this group and MCPTT user as determined by the procedures of subclause 6.3.3.1.13.1, shall populate the application/vnd.3gpp.mcptt-info+xml MIME body and application/vnd.3gpp.location-info+xml MIME body as specified in subclause 6.3.3.1.12. Otherwise, shall set the <alert-ind> element to a value of "false" in the application/vnd.3gpp.mcptt-info+xml MIME body; and

d)	if the in-progress imminent peril state of the group is set to a value of "true" shall include in the application/vnd.3gpp.mcptt-info+xml MIME body an <imminentperil-ind> element set to a value of "false"; and

NOTE:	If the imminent peril state of the group is true at this point, the controlling function will be setting it to false as part of the calling procedure. This is in effect an upgrade of an MCPTT imminent peril group call to an MCPTT emergency group call.

4)	if the in-progress emergency group state of the group is set to a value of "false":

a)	shall include a Resource-Priority header field populated with the values for a normal MCPTT group call as specified in subclause 6.3.3.1.19; and

b)	if the received SIP re-INVITE request contained an application/vnd.3gpp.mcptt-info+xml MIME body with the <emergency-ind> element set to a value of "false" and this is an authorised request to cancel an MCPTT emergency group call as determined by the procedures of subclause 6.3.3.1.13.4:

i)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body with the <emergency-ind> element set to a value of "false"; and

ii)	if the received SIP re-INVITE request contained an application/vnd.3gpp.mcptt-info+xml MIME body with the <alert-ind> element set to a value of "false" and this is an authorised request to cancel an MCPTT emergency alert as determined by the procedures of subclause 6.3.3.1.15, shall:

A)	include in the application/vnd.3gpp.mcptt-info+xml MIME body an <alert-ind> element set to a value of "false"; and

B)	if the received SIP request contains an <originated-by> element in the application/vnd.3gpp.mcptt-info+xml MIME body, copy the contents of the received <originated-by> element to an <originated-by> element in the application/vnd.3gpp.mcptt-info+xml MIME body in the outgoing SIP re-INVITE request.

##### 6.3.3.1.7	Sending a SIP INVITE request for MCPTT emergency group call

This subclause is referenced from other procedures.

This subclause describes the procedures for inviting an MCPTT user to an MCPTT session associated with an MCPTT emergency group call or MCPTT imminent peril group call. The procedure is initiated by the controlling MCPTT function as the result of an action in subclause 10.1.2.4.1.1.

The controlling MCPTT function:

1)	shall generate a SIP INVITE request as specified in subclause 6.3.3.1.2;

2)	shall set the Request-URI to the address of the terminating participating function associated with the MCPTT ID of the targeted MCPTT user;

3)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcpttinfo> element containing the <mcptt-Params> element populated as follows:

a)	the <mcptt-request-uri> element set to the value of the MCPTT ID of the targeted MCPTT user;

b)	the <mcptt-calling-user-id> element set to the value of the MCPTT ID of the calling MCPTT user; and

c)	the <mcptt-calling-group-id> element set to the value of the MCPTT group ID of the emergency group call.

4)	shall include in the P-Asserted-Identity header field the public service identity of the controlling MCPTT function;

5)	shall include in the SIP INVITE request an SDP offer based on the SDP offer in the received SIP INVITE request from the originating network according to the procedures specified in subclause 6.3.3.1.1; and

6)	if the in-progress emergency group state of the group is set to a value of "true" the controlling MCPTT function:

a)	shall include a Resource-Priority header field populated with the values for an MCPTT emergency group call as specified in subclause 6.3.3.1.19;

b)	shall include in the application/vnd.3gpp.mcptt-info+xml MIME body an <emergency-ind> element set to a value of "true";

c)	if the <alert-ind> element is set to "true" in the received SIP INVITE request and the requesting MCPTT user and MCPTT group are authorised for the initiation of MCPTT emergency alerts as determined by the procedures of subclause 6.3.3.1.13.1, shall populate the application/vnd.3gpp.mcptt-info+xml MIME body and the application/vnd.3gpp.location-info+xml MIME body as specified in subclause 6.3.3.1.12. Otherwise, shall set the <alert-ind> element to a value of "false" in the application/vnd.3gpp.mcptt-info+xml MIME body; and

d)	if the in-progress imminent peril state of the group is set to a value of "true" shall include in the application/vnd.3gpp.mcptt-info+xml MIME body an <imminentperil-ind> element set to a value of "false"; and

NOTE:	If the imminent peril state of the group is true at this point, the controlling function will set it to false as part of the calling procedure.

7)	if the in-progress emergency state of the group is set to a value of "false" and the in-progress imminent peril state of the group is set to a value of "true", the controlling MCPTT function:

a)	shall include a Resource-Priority header field populated with the values for an MCPTT imminent peril group call as specified in subclause 6.3.3.1.19; and

b)	shall include in the application/vnd.3gpp.mcptt-info+xml MIME body with the <imminentperil-ind> element set to a value of "true".

##### 6.3.3.1.8	Sending a SIP UPDATE request for Resource-Priority header field correction

This subclause is referenced from other procedures.

This subclause describes the procedures for updating an MCPTT session associated with an MCPTT emergency group call or MCPTT imminent peril group call when the received SIP INVITE request did not include a correctly populated Resource-Priority header field. The procedure is initiated by the controlling MCPTT function for the purpose of providing the correct Resource-Priority header field.

1)	shall generate a SIP 183 (Session Progress) response according to 3GPP TS 24.229 [4] with the clarifications provided specified in subclause 6.3.3.2.3.1;

2)	shall include the option tag "100rel" in a Require header field in the SIP 183 (Session Progress) response;

3)	shall include in the SIP 183 (Session Progress) response an SDP answer to the SDP offer in the incoming SIP INVITE request as specified in the subclause 6.3.3.2.1; and

4)	shall send the SIP 183 (Session Progress) response towards the MCPTT client according to 3GPP TS 24.229 [4].

Upon receiving a SIP PRACK request to the SIP 183 (Session Progress) response the controlling MCPTT function:

1)	shall send the SIP 200 (OK) response to the SIP PRACK request according to 3GPP TS 24.229 [4].

2)	shall generate a SIP UPDATE request according to 3GPP TS 24.229 [4] with the following clarifications:

3)	shall include in the SIP UPDATE request an SDP offer based on the SDP offer in the received SIP INVITE request from the originating network according to the procedures specified in subclause 6.3.3.1.1;

4)	if the in-progress emergency group state of the group is set to a value of "true" the controlling MCPTT function shall include a Resource-Priority header field populated for an MCPTT emergency group call as specified in subclause 6.3.3.1.19; and

NOTE 1:	This is the case when the sending MCPTT client did not send a Resource-Priority header field populated appropriately to receive emergency-level priority. In this case, the Resource-Priority header field is populated appropriately to provide emergency-level priority.

5)	if the in-progress emergency group state of the group is set to a value of "false" the controlling MCPTT function:

a)	if the in-progress imminent peril state of the group is set to a value of "false", shall include a Resource-Priority header field populated for a normal priority MCPTT group call as specified in subclause 6.3.3.1.19; and

b)	if the in-progress imminent peril state of the group is set to a value of "true", shall include a Resource-Priority header field populated for an MCPTT imminent peril group call as specified in subclause 6.3.3.1.19.

NOTE 2:	This is the case when the sending MCPTT client incorrectly populated a Resource-Priority header field for emergency-level or imminent peril-level priority and the controlling MCPTT function re-populates it as appropriate to an imminent peril level priority or normal priority level.

##### 6.3.3.1.9	Generating a SIP re-INVITE request

This subclause is referenced from other procedures.

This subclause describes the procedures for generating a SIP re-INVITE request to be sent by the controlling MCPTT function.

The controlling MCPTT function:

1)	shall generate an SIP re-INVITE request according to 3GPP TS 24.229 [4]; and

2)	shall include an SDP offer with the media parameters as currently established with the terminating MCPTT client according to 3GPP TS 24.229 [4] with the clarifications specified in subclause 6.3.3.1.1.

##### 6.3.3.1.10	Generating a SIP re-INVITE request to cancel an in-progress emergency

This subclause is referenced from other procedures.

This subclause describes the procedures for generating a SIP re-INVITE request to cancel the in-progress emergency state of an MCPTT group. The procedure is initiated by the controlling MCPTT function when it determines the cancellation of the in-progress emergency state of an MCPTT group is required.

The controlling MCPTT function:

1)	shall generate a SIP re-INVITE request as specified in 3GPP TS 24.229 [4] with the clarifications specified in subclause 6.3.3.1.9;

2)	shall include a Resource-Priority header field populated with the values for a normal MCPTT group call as specified in subclause 6.3.3.1.19; and

3)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body with the <emergency-ind> element set to a value of "false".

##### 6.3.3.1.11	Generating a SIP MESSAGE request for notification of in-progress emergency or imminent peril status change

This subclause is referenced from other procedures.

This subclause describes the procedures for generating a SIP MESSAGE request to notify affiliated but not participating members of an MCPTT group of the change of status of the in-progress emergency state, imminent peril state or emergency alert status of an MCPTT group. The procedure is initiated by the controlling MCPTT function when there has been a change of in-progress imminent peril, in-progress emergency or the emergency alert status of an MCPTT group.

The controlling MCPTT function:

1)	shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [4] and IETF RFC 3428 [33];

2)	shall include an Accept-Contact header field containing the g.3gpp.mcptt media feature tag along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [6];

3)	shall include an Accept-Contact header field with the media feature tag g.3gpp.icsi-ref with the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" along with parameters "require" and "explicit" according to IETF RFC 3841 [6];

4)	shall set the Request-URI to the address of the terminating participating function associated with the MCPTT ID of the targeted MCPTT user;

5)	shall include a P-Asserted-Identity header field set to the public service identity of controlling MCPTT function;

6)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcpttinfo> element containing the <mcptt-Params> element with the <mcptt-request-uri> element set to the value of the MCPTT ID of the targeted MCPTT user; and

7)	shall include in the application/vnd.3gpp.mcptt-info+xml MIME body an <mcptt-calling-group-id> element set to the MCPTT group ID of the MCPTT group on which the MCPTT emergency call, imminent peril call or the emergency alert state has changed.

##### 6.3.3.1.12	Populate mcptt-info and location-info MIME bodies for emergency alert

This subclause is referenced from other procedures.

This subclause describes the procedures for populating the application/vnd.3gpp.mcptt-info+xml and application/vnd.3gpp.location-info+xml MIME bodies for an MCPTT emergency alert. The procedure is initiated by the controlling MCPTT function when it has received a SIP request initiating an MCPTT emergency alert and generates a message containing the MCPTT emergency alert information required by 3GPP TS 23.179 [3].

The controlling MCPTT function:

1)	shall include, if not already present, an application/vnd.3gpp.mcptt-info+xml MIME body as specified in Annex F.1, and set the <alert-ind> element to a value of "true";

2)	shall determine the value of the MCPTT user's Mission Critical Organization from the <MissionCriticalOrganization> element, of the MCPTT user profile document (see the MCPTT user profile document in 3GPP TS 24.384 [50]);

3)	shall include in the <mcpttinfo> element containing the <mcptt-Params> element containing an <mc-org> element set to the value of the MCPTT user's Mission Critical Organization; and

4)	shall copy the contents of the application/vnd.3gpp.location-info+xml MIME body in the received SIP request into an application/vnd.3gpp.location-info+xml MIME body included in the outgoing SIP request.

##### 6.3.3.1.13	Authorisations

###### 6.3.3.1.13.1	Determining authorisation for initiating an MCPTT emergency alert

If the controlling MCPTT function has received a SIP request targeted to an MCPTT group with the <alert-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body set to a value of "true", the controlling MCPTT function shall check the following conditions:

1)	if the <allow-activate-emergency-alert> element of the <ruleset> element of the MCPTT user profile document identified by the MCPTT ID of the calling user (see the MCPTT user profile document in 3GPP TS 24.384 [50]) is set to a value of "true";

a)	if the "entry-info" attribute of the <entry> element of the <Recipient> element contained within the <EmergencyAlert> element of the MCPTT user profile document (see the MCPTT user profile document in 3GPP TS 24.384 [50]) is set to a value of "DedicatedGroup" and:

i)	if the MCPTT group identity targeted for the emergency alert is contained in the <uri-entry> element of the <entry> element of the <Recipient> element contained within the <EmergencyAlert> element of the MCPTT user profile document (see the MCPTT user profile document in 3GPP TS 24.384 [50]); and

ii)	if the <allow-MCPTT-emergency-alert> element of the <list-element> of the group document identified by the MCPTT group identity is set to a value of "true" as specified in 3GPP TS 24.381 [31]; or

b)	if the "entry-info" attribute of the <entry> element of the <Recipient> element contained within the <EmergencyAlert> element of the MCPTT user profile (see the MCPTT user profile document in 3GPP TS 24.384 [50]) is set to a value of "UseCurrentlySelectedGroup" and the <allow-MCPTT-emergency-alert> element of the <list-element> of the group document identified by the MCPTT group identity targeted for the emergency alert is set to a value of "true" as specified in 3GPP TS 24.381 [31].

then the MCPTT emergency alert request shall be considered to be an authorised request for an MCPTT emergency alert targeted to a MCPTT group. In all other cases, the MCPTT emergency alert request shall be considered to be an unauthorised request for an MCPTT emergency alert targeted to an MCPTT group.

If the controlling MCPTT function has received a SIP request targeted to an MCPTT user with the <alert-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body set to a value of "true", the controlling MCPTT function shall check the following conditions:

1)	if the <allow-activate-emergency-alert> element of the <ruleset> element of the MCPTT user profile document identified by the MCPTT ID of the calling user (see the MCPTT user profile document in 3GPP TS 24.384 [50]) is set to a value of "true"; and

a)	if the "entry-info" attribute of the <entry> element of the <Recipient> element contained within the <EmergencyAlert> element of the MCPTT user profile document (see the MCPTT user profile document in 3GPP TS 24.384 [50]) is set to a value of "UsePreConfigured" and the MCPTT ID of the MCPTT user targeted for the call is contained in the <uri-entry> element of the <entry> element of the <Recipient> element contained within the <EmergencyAlert> element (see the MCPTT user profile document in 3GPP TS 24.384 [50]); or

b)	if the "entry-info" attribute of the <entry> element of the <Recipient> element contained within the <EmergencyAlert> element of the MCPTT user profile document (see the MCPTT user profile document in 3GPP TS 24.384 [50]) is set to a value of "LocallyDetermined";

then the MCPTT emergency alert request shall be considered to be an authorised request for an MCPTT emergency alert targeted to an MCPTT user. In all other cases, it shall be considered to be an unauthorised request for an MCPTT emergency alert targeted to an MCPTT user.

###### 6.3.3.1.13.2	Determining authorisation for initiating an MCPTT emergency group or private call

If the controlling MCPTT function has received a SIP request for an MCPTT group call with the <emergency-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body set to a value of "true" and:

1)	if the <allow-emergency-group-call> element of the <ruleset> element of the MCPTT user profile document identified by the MCPTT ID of the calling user (see the MCPTT user profile document in 3GPP TS 24.384 [50]) is set to a value of "true" and:

a)	if the "entry-info" attribute of the <entry> element of the <MCPTTGroupInitiation> element contained within the <EmergencyCall> element of the MCPTT user profile document (see the MCPTT user profile document in 3GPP TS 24.384 [50]) is set to a value of "DedicatedGroup" and:

i)	if the <uri-entry> element of the <entry> element of the <MCPTTGroupInitiation> element contains the identity of the MCPTT group targeted by the calling MCPTT user; and

ii)	if the <allow-MCPTT-emergency-call> element of the <list-service> element of the group document identified by the targeted MCPTT group identity is set to a value of "true" as specified in 3GPP TS 24.381 [31];

then the controlling MCPTT function shall consider the MCPTT emergency group call request to be an authorised request for an MCPTT emergency group call and skip the remaining steps; or;

b)	if the "entry-info" attribute of the <entry> element of the <MCPTTGroupInitiation> element contained within the <EmergencyCall> element of the MCPTT user profile document (see the MCPTT user profile document in 3GPP TS 24.384 [50]) is set to a value of "UseCurrentlySelectedGroup" and if the <allow-MCPTT-emergency-call> element of the <list-service> element of the group document identified by the targeted MCPTT group identity is set to a value of "true" as specified in 3GPP TS 24.381 [31];

then the controlling MCPTT function shall consider the MCPTT emergency group call request to be an authorised request for an MCPTT emergency group call and skip the remaining steps; or

2)	if the controlling MCPTT function does not consider the MCPTT emergency group call request to be an authorised request for an MCPTT emergency group call by step 1) above, then the controlling MCPTT function shall consider the MCPTT emergency group call request to be an unauthorised request for an MCPTT emergency group call.

If the controlling MCPTT function has received a SIP request for an MCPTT private call with the <emergency-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body set to a value of "true" and:

1)	if the <allow-emergency-private-call> element of the <ruleset> element of the MCPTT user profile document identified by the MCPTT ID of the calling user (see the MCPTT user profile document in 3GPP TS 24.384 [50]) is set to a value of "true"; and

a)	if the "entry-info" attribute of the <entry> element of the <Recipient> element contained within the <EmergencyCall> element of the MCPTT user profile document (see the MCPTT user profile document in 3GPP TS 24.384 [50]) is set to a value of "UsePreConfigured" and if the MCPTT ID targeted for the call is contained in the <uri-entry> element of the <entry> element of the <Recipient> element contained within the <EmergencyCall> element (see the MCPTT user profile document in 3GPP TS 24.384 [50]); or

b)	if the "entry-info" attribute of the <entry> element of the <Recipient> element contained within the <EmergencyCall> element of the MCPTT user profile document (see the MCPTT user profile document in 3GPP TS 24.384 [50]) is set to a value of "LocallyDetermined";

then the controlling MCPTT function shall consider the MCPTT emergency private call request to be an authorised request for an MCPTT emergency private call and skip step 2) below; or

2)	if the controlling MCPTT function does not consider the MCPTT emergency private call request to be an authorised request for an MCPTT emergency private call by step 1) above, then the controlling MCPTT function shall consider the MCPTT emergency private call request to be an unauthorised request for an MCPTT emergency private call.

###### 6.3.3.1.13.3	Determining authorisation for cancelling an MCPTT emergency alert

If the controlling MCPTT function has received a SIP request with the <alert-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body set to a value of "false" and:

1)	if the <allow-cancel-emergency-alert> element of the <ruleset> element of the MCPTT user profile document identified by the MCPTT ID of the calling user (see the MCPTT user profile document in 3GPP TS 24.384 [50]) is set to a value of "true", then the MCPTT emergency alert cancellation request shall be considered to be an authorised request for an MCPTT emergency alert cancellation; and

2)	if the <allow-cancel-emergency-alert> element of the <ruleset> element of the MCPTT user profile document identified by the MCPTT ID of the calling user (see the MCPTT user profile document in 3GPP TS 24.384 [50]) is set to a value of "false", then the MCPTT emergency alert cancellation request shall be considered to be an unauthorised request for an MCPTT emergency alert cancellation.

###### 6.3.3.1.13.4	Determining authorisation for cancelling an MCPTT emergency call

If the controlling MCPTT function has received a SIP request for an MCPTT group call with the <emergency-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body set to a value of "false" and:

1)	if the <allow-cancel-group-emergency> element of the <ruleset> element of the MCPTT user profile document identified by the MCPTT ID of the calling user (see the MCPTT user profile document in 3GPP TS 24.384 [50]) is set to a value of "true", then the MCPTT emergency call cancellation request shall be considered to be an authorised request for an MCPTT emergency group call cancellation; and

2)	If the <allow-cancel-group-emergency> element of the <ruleset> element of the MCPTT user profile document identified by the MCPTT ID of the calling user (see the MCPTT user profile document in 3GPP TS 24.384 [50]) is set to a value of "false", then the MCPTT emergency group call cancellation request shall be considered to be an unauthorised request for an MCPTT emergency group call cancellation.

If the controlling MCPTT function has received a SIP request for an MCPTT private call with the <emergency-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body set to a value of "false" and:

1)	if the <allow-cancel-private-emergency-call> element of the <ruleset> element of the MCPTT user profile document identified by the MCPTT ID of the calling user (see the MCPTT user profile document in 3GPP TS 24.384 [50]) is set to a value of "true", then the MCPTT emergency private call cancellation request shall be considered to be an authorised request for an MCPTT emergency private call cancellation; and

2)	if the <allow-cancel-private-emergency-call> element of the <ruleset> element of the MCPTT user profile document identified by the MCPTT ID of the calling user (see the MCPTT user profile document in 3GPP TS 24.384 [50]) is set to a value of "false" or not present, then the MCPTT emergency private call cancellation request shall be considered to be an unauthorised request for an MCPTT emergency private call cancellation.

###### 6.3.3.1.13.5	Determining authorisation for initiating an MCPTT imminent peril call

If the controlling MCPTT function has received a SIP request with the <imminentperil-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body set to a value of "true" and:

1)	if the <allow-imminent-peril-call> element of the <ruleset> element of the MCPTT user profile document identified by the MCPTT ID of the calling user (see the MCPTT user profile document in 3GPP TS 24.384 [50]) is set to a value other than  "true" the request for initiating an MCPTT imminent peril call shall be considered to be an unauthorised request for an MCPTT imminent peril call and skip the remaining steps;

2)	if the <allow-imminent-peril-call> element of the <list-service> element of the group document identified by the targeted MCPTT group identity is set to a value other than "true" as specified in 3GPP TS 24.381 [31], the request for initiating an MCPTT imminent peril call shall be considered to be an unauthorised request for an MCPTT imminent peril call and skip the remaining steps;

3)	if the "entry-info" attribute of the <entry> element of the <MCPTTGroupInitiation> element contained within the <ImminentPerilCall> element of the MCPTT user profile document (see the MCPTT user profile document in 3GPP TS 24.384 [50]) is set to a value of "DedicatedGroup" and if the MCPTT group identity targeted for the call is contained in the <uri-entry> element of the <entry> element of the <MCPTTGroupInitiation> element contained within the <ImminentPerilCall> element (see the MCPTT user profile document in 3GPP TS 24.384 [50]); or

4)	if the "entry-info" attribute of the <entry> element of the <MCPTTGroupInitiation> element contained within the <ImminentPerilCall> element of the MCPTT user profile document (see the MCPTT user profile document in 3GPP TS 24.384 [50]) is set to a value of "UseCurrentlySelectedGroup".

then the MCPTT imminent peril call request shall be considered to be an authorised request for an MCPTT imminent peril call. In all other cases, it shall be considered to be an unauthorised request for an MCPTT imminent peril call.

###### 6.3.3.1.13.6	Determining authorisation for cancelling an MCPTT imminent peril call

If the controlling MCPTT function has received a SIP request with the <imminentperil-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body set to a value of "false" and:

1)	if the <allow-cancel-imminent-peril> element of the <ruleset> element of the MCPTT user profile document identified by the MCPTT ID of the calling user (see the MCPTT user profile document in 3GPP TS 24.384 [50]) is set to a value of "true", then the MCPTT emergency call cancellation request shall be considered to be an authorised request for an MCPTT imminent peril call cancellation; and

2)	if the <allow-cancel-imminent-peril> element of the <ruleset> element of the MCPTT user profile document identified by the MCPTT ID of the calling user (see the MCPTT user profile document in 3GPP TS 24.384 [50]) is set to a value of "false" or not present, then the MCPTT emergency call cancellation request shall be considered to be an unauthorised request for an MCPTT imminent peril call cancellation.

###### 6.3.3.1.13.7	Sending a SIP OPTIONS request to authorise an MCPTT user at a non-controlling MCPTT function of a MCPTT group

This subclause is referenced from other procedures.

The controlling MCPTT function:

1)	if the <associated-group-id> element is included in the application/vnd.3gpp.mcptt-info+xml MIME body of the incoming SIP INVITE request, shall generate a SIP OPTIONS request according to 3GPP TS 24.229 [4] and the IETF RFC 3261 [24] populated as follows:

a)	shall set the Request-URI to the public service identity of the non-controlling MCPTT function associated with the MCPTT Group ID which was present in the <associated-group-id> element in the application/vnd.3gpp.mcptt-info+xml MIME body of the incoming SIP INVITE request;

NOTE 1:	How the controlling MCPTT function finds the address of the non-controlling MCPTT function is out of the scope of the current release.

b)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Asserted-Service-Id header field according to IETF RFC 6050 [9];

c)	shall include in the P-Asserted-Identity header field, the public service identity of the controlling MCPTT function;

d)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body where:

i)	the <mcptt-request-uri> element shall be set to the value of the <associated-group-id> element in the application/vnd.3gpp.mcptt-info+xml MIME body of the incoming SIP INVITE request; and

ii)	the <mcptt-calling-user-id> element is set to the same value as in the <mcptt-calling-user-id> element in the application/vnd.3gpp.mcptt-info+xml MIME body of the incoming SIP INVITE request;

e)	shall include the following in the Contact header field:

i)	the g.3gpp.mcptt media feature tag; and

ii)	the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt"; and

f)	send the SIP OPTIONS request as specified in 3GPP TS 24.229 [4]; and

2)	if the <associated-group-id> element is not included in the application/vnd.3gpp.mcptt-info+xml MIME body of the incoming SIP INVITE request, shall for each constituent MCPTT group ID in the <constituent-MCPTT-group-ID> element not homed at the controlling MCPTT function generate a SIP OPTIONS request according to 3GPP TS 24.229 [4] and IETF RFC 3261 [24] populated as follows:

a)	shall set the Request-URI to the public service identity of the non-controlling MCPTT function associated with the MCPTT group ID in the <constituent-MCPTT-group-ID> element;

NOTE 2:	How the controlling MCPTT function finds the address of the non-controlling MCPTT function is out of the scope of the current release.

b)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Asserted-Service-Id header field according to IETF RFC 6050 [9];

c)	shall include in the P-Asserted-Identity header field, the public service identity of the controlling MCPTT function;

d)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body where:

i)	the <mcptt-request-uri> element shall be set to the MCPTT group ID in the <constituent-MCPTT-group-ID> element; and

ii)	the <mcptt-calling-user-id> element is set to the same value as in the <mcptt-calling-user-id> element in the application/vnd.3gpp.mcptt-info+xml MIME body of the incoming SIP INVITE request;

e)	shall include the following in the Contact header field:

i)	the g.3gpp.mcptt media feature tag; and

ii)	the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt"; and

f)	send the SIP OPTIONS request as specified in 3GPP TS 24.229 [4].

Upon receipt of the first SIP 200 (OK) response to the SIP OPTIONS request with the mcptt-warn-code set to "147" in a Warning header field as specified in subclause 4.4, the controlling MCPTT function shall return a SIP 302 (Moved Temporarily) response to the "SIP INVITE request for controlling MCPTT function of an MCPTT group" populated as follows:

1)	the URI in the Contact header field set to the P-Asserted-Identity received in the SIP 200 (OK) response;

2)	an application/vnd.3gpp.mcptt-info MIME body with:

a)	the <mcptt-request-uri> element set to the same value as received in the <mcptt-request-uri> in the SIP 2xx response to the SIP OPTIONS request; and

b)	the <session-type> element set to the value received in the <session-type> element in the appication/vnd.3gpp.mcptt.info+xml MIME body of the received SIP 2xx response to the SIP OPTIONS request; and

3)	if more than one OPTIONS request were sent, shall remove any cached SIP response and ignore any other responses to any other OPTIONS request.

Upon receipt of a SIP 404 (Not Found) response to the SIP OPTIONS request such that the mcptt-warn-code set to "113" in a Warning header field as specified in subclause 4.4, the controlling MCPTT function:

1)	if more than one SIP OPTIONS request were sent and if no other responses to SIP OPTIONS request are expected; shall send a SIP 404 (Not Found) response to "SIP INVITE request for controlling MCPTT function of an MCPTT group" and include the Warning header field received in the SIP 404 (Not Found) response; and

2)	if more than one OPTIONS request were sent and other responses to SIP OPTIONS request are expected, shall cache the received SIP 404 (Not Found) response.

Upon receipt of a SIP 403 (Forbidden) response to the SIP OPTIONS request, the mcptt-warn-code set to "106" or "109" in a Warning header field as specified in subclause 4.4 and if more than one OPTIONS request were sent and if no other responses to the SIP OPTIONS request are expected, the controlling MCPTT function:

1)	if a SIP 404 (Not Found) response is cached, send a SIP 404 (Not Found) response to "SIP INVITE request for controlling MCPTT function of an MCPTT group" and include the Warning header field received in the SIP 404 (Not Found) response; and

2)	if a SIP 404 (Not Found) response is not cached, shall return a SIP 403 (Forbidden) response to "SIP INVITE request for controlling MCPTT function of an MCPTT group" and include the Warning header field received in the SIP 403 (Forbidden) response.

Upon receipt of any other response to the SIP OPTIONS response than specified above and if more than one OPTIONS request were sent and if no other responses to the SIP OPTIONS request are expected, the controlling MCPTT function:

1)	if a SIP 404 (Not Found) response is cached, send a SIP 404 (Not Found) response to "SIP INVITE request for controlling MCPTT function of an MCPTT group" and include the Warning header field received in the SIP 404 (Not Found) response; and

2)	if a SIP 404 (Not Found) response is not cached, shall return a SIP 403 (Forbidden) response to "SIP INVITE request for controlling MCPTT function of an MCPTT group".

NOTE 3:	The reason for selecting the SIP 404 (Not Found) response when a SIP 404 (Not Founf) response is cached is to indicate that it was a valid request but the MCPTT user identified in the <mcptt-calling-user-id> is not a member of any of the constituent MCPTT groups in the temporary group document.

##### 6.3.3.1.14	Generating a SIP 403 response for priority call request rejection

If the controlling MCPTT function has received a SIP request with the <emergency-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body is set to "true" and this is an unauthorised request for an MCPTT emergency call as determined by the procedures of subclause 6.3.3.1.13.2, the controlling MCPTT function shall:

1)	include in the SIP 403 (Forbidden) response an application/vnd.3gpp.mcptt-info+xml MIME body as specified in Annex F.1 with the <mcpttinfo> element containing the <mcptt-Params> element with the <emergency-ind> element set to a value of "false";

2)	if the received SIP request included an <alert-ind> element in the application/vnd.3gpp.mcptt-info+xml set to a value of "true" and this is an authorised request for an emergency alert as determined by the procedures of subclause 6.3.3.1.13.1, include in the SIP 403 (Forbidden) response an <alert-ind> element in the application/vnd.3gpp.mcptt-info+xml MIME body set to a value of "true";

3)	if the received SIP request included an <alert-ind> element in the application/vnd.3gpp.mcptt-info+xml set to a value of "true" and this is an unauthorised request for an emergency alert as determined by the procedures of subclause 6.3.3.1.13.1, include in the SIP 403 (Forbidden) response an <alert-ind> element in the application/vnd.3gpp.mcptt-info+xml MIME body set to a value of "false"; or

4)	if the received SIP request included an <alert-ind> element in the application/vnd.3gpp.mcptt-info+xml set to a value of "false", include in the SIP 403 (Forbidden) response an <alert-ind> element in the application/vnd.3gpp.mcptt-info+xml MIME body set to a value of "false".

##### 6.3.3.1.15	Sending a SIP re-INVITE request for MCPTT imminent peril group call

This subclause is referenced from other procedures.

The controlling MCPTT function shall generate a SIP re-INVITE request according to 3GPP TS 24.229 [4].

The controlling MCPTT function:

1)	shall include in the Contact header field an MCPTT session identity for the MCPTT session with the g.3gpp.mcptt media feature tag and the isfocus media feature tag according to IETF RFC 3840 [16];

2)	shall include an SDP offer with the media parameters as currently established with the terminating MCPTT client according to 3GPP TS 24.229 [4];

3)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcptt-calling-user-id> element set to the MCPTT ID of the initiating MCPTT user;

4)	if the in-progress imminent peril state of the group is set to a value of "true" the controlling MCPTT function:

a)	shall include a Resource-Priority header field populated with the values for an MCPTT imminent peril group call as specified in subclause 6.3.3.1.19; and

b)	shall include in the application/vnd.3gpp.mcptt-info+xml MIME body an <imminentperil-ind> element set to a value of "true"; and

5)	if the in-progress imminent peril state of the group is set to a value of "false":

a)	shall include a Resource-Priority header field populated with the values for a normal MCPTT group call as specified in subclause 6.3.3.1.19; and

b)	shall include in the application/vnd.3gpp.mcptt-info+xml MIME body an <emergency-ind> element set to a value of "false" and the <imminentperil-ind> element set to a value of "false".

##### 6.3.3.1.16	Handling the expiry of timer TNG2 (in-progress emergency group call timer)

Upon expiry of timer TNG2 (in-progress emergency group call timer) for an MCPTT group, the controlling MCPTT function:

1)	shall set the in-progress emergency state of the group to a value of "false";

2)	shall, if an MCPTT group call or MCPTT group session is in progress on the indicated group, for each of the participating members:

a)	generate a SIP re-INVITE request as specified in subclause 6.3.3.1.10; and

b)	send the SIP re-INVITE request towards the MCPTT client according to 3GPP TS 24.229 [4]; and

3)	shall for each affiliated but non-participating members member of the group:

a)	generate a SIP MESSAGE request according to subclause 6.3.3.1.11 and include in the application/vnd.3gpp.mcptt-info+xml MIME body an <emergency-ind> element set to a value of "false";

b)	shall include in the P-Asserted-Identity header field the public service identity of the controlling MCPTT function; and

c)	send the SIP MESSAGE request towards the MCPTT client according to rules and procedures of 3GPP TS 24.229 [4].

Upon receiving a SIP 200 (OK) response to a re-SIP INVITE request the controlling MCPTT function shall interact with the media plane as specified in 3GPP TS 24.380 [5].

##### 6.3.3.1.17	Validate priority request parameters

This subclause is referenced from other procedures. This procedure validates the combinations of <emergency-ind>, <imminentperil-ind> and <alert-ind> in the application/vnd.3gpp.mcptt-info+xml MIME body included in:

1)	a SIP INVITE request or SIP re-INVITE request; or

2)	the body "URI" header field of the SIP URI included in the application/resource-lists MIME body which is pointed to by a "cid" URL located in the Refer-To header of a SIP REFER request;

Upon receiving a SIP request as specified above with the <emergency-ind> element set to a value of "true", the controlling MCPTT function shall only consider the following as valid combinations:

1)	<imminentperil-ind> not included and <alert-ind> not included; or

2)	<imminentperil-ind> not included and <alert-ind> set to "true".

Upon receiving a SIP request as specified above with the <emergency-ind> element set to a value of "false", the controlling MCPTT function shall only consider the following as valid combinations:

1)	<imminentperil-ind> not included and <alert-ind> not included; or

2)	<imminentperil-ind> not included and <alert-ind> included.

Upon receiving a SIP request as specified above with the <imminentperil-ind> element included the controlling MCPTT function shall only consider the request as valid if both the <emergency-ind> and <alert-ind> are not included.

If the combination of the <emergency-ind>, <imminentperil-ind> or <alert-ind> indicators is invalid, the controlling MCPTT function shall send a SIP 403 (Forbidden) response with the warning text set to "150 invalid combinations of data received in MIME body" in a Warning header field as specified in subclause 4.4.

##### 6.3.3.1.18	Sending a SIP INFO request in the dialog of a SIP request for a priority call

This subclause is referenced from other procedures and describes how the controlling MCPTT function generates a SIP INFO request due to the receipt of a SIP request for a priority call.

The controlling MCPTT function:

1)	shall generate a SIP INFO request according to rules and procedures of 3GPP TS 24.229 [4] and IETF RFC 6086 [64];

2)	shall include the Info-Package header field set to g.3gpp.mcptt-info in the SIP INFO request;

3)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body in the SIP INFO request and:

a)	if the received SIP request contained application/vnd.3gpp.mcptt-info+xml MIME body with the <alert-ind> element set to a value of "true" and this is an unauthorised request for an MCPTT emergency alert as specified in subclause 6.3.3.1.13.1, shall set the <emergency-ind> element to a value of "true" and the <alert-ind> element to a value of "false";

b)	if the received SIP request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <alert-ind> element set to a value of "false" and if this is an unauthorised request for an MCPTT emergency alert cancellation, shall set <alert-ind> element to a value of "true"; and

c)	if the received SIP request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <imminentperil-ind> element set to a value of "true", this is an authorised request for an MCPTT imminent peril group call and the in-progress emergency state of the group is set to a value of "true", shall set the <imminentperil-ind> element to a value of "false" and the <emergency-ind> element set to a value of "true"; and

4)	shall send the SIP INFO request towards the inviting MCPTT client in the dialog created by the SIP request from the inviting MCPTT client, as specified in 3GPP TS 24.229 [4].

##### 6.3.3.1.19	Retrieving Resource-Priority header field values

This subclause is referenced from other procedures.

When determining the Resource-Priority header field namespace and priority values as specified in draft-holmberg-dispatch-mcptt-rp-namespace [48] for an MCPTT emergency group call or MCPTT emergency private call the controlling MCPTT function:

1)	shall retrieve the value of the <resource-priority-namespace> element contained in the <emergency-resource-priority> element contained in the <OnNetwork> element of the MCPTT service configuration document (see the service configuration document in 3GPP TS 24.384 [50]); and

2)	shall retrieve the value of the <resource-priority-priority> element contained in the <emergency-resource-priority> element contained in the <OnNetwork> element of the MCPTT service configuration document (see the service configuration document in 3GPP TS 24.384 [50]).

When determining the Resource-Priority header field namespace and priority values as specified in draft-holmberg-dispatch-mcptt-rp-namespace [48] for an MCPTT imminent peril group call the controlling MCPTT function:

1)	shall retrieve the value of the <resource-priority-namespace> element contained in the <imminent-peril-resource-priority> element contained in the <OnNetwork> element of the MCPTT service configuration document (see the service configuration document in 3GPP TS 24.384 [50]); and

2)	shall retrieve the value of the <resource-priority-priority> element contained in the <imminent-peril-resource-priority> element contained in the <OnNetwork> element of the MCPTT service configuration document (see the service configuration document in 3GPP TS 24.384 [50])

When determining the Resource-Priority header field namespace and priority values as specified in draft-holmberg-dispatch-mcptt-rp-namespace [48] for a normal MCPTT group or private call the controlling MCPTT function:

1)	shall retrieve the value of the <resource-priority-namespace> element contained in the <normal-resource-priority> element contained in the <OnNetwork> element of the MCPTT service configuration document (see the service configuration document in 3GPP TS 24.384 [50]); and

2)	shall retrieve the value of the <resource-priority-priority> element contained in the <normal-resource-priority> element contained in the <OnNetwork> element of the MCPTT service configuration document (see the service configuration document in 3GPP TS 24.384 [50]).

NOTE:	The "normal" Resource-Priority header field value is needed to return to a normal priority value from a priority value adjusted for an MCPTT emergency group or private call or an MCPTT imminent peril group call. The "normal" priority received from the EPS by use of the "normal" Resource-Priority header field value is expected to be the same as the "normal" priority received from the EPS when initiating a call with no Resource-Priority header field included.

#### 6.3.3.2	Requests terminated by the controlling MCPTT function

##### 6.3.3.2.1	SDP answer generation

When composing the SDP answer according to 3GPP TS 24.229 [4], the controlling MCPTT function:

1)	for the accepted media stream in the received SDP offer:

a)	shall replace the IP address and port number in the received SDP offer with the IP address and port number of the controlling MCPTT function; and

2)	for the accepted media-floor control entity, if present in the received SDP offer:

a)	shall replace the IP address and port number in the received SDP offer with the IP address and port number of the controlling MCPTT function, for the accepted media-floor control entity, if present in the received SDP offer; and

b)	shall include 'fmtp' attributes as specified in 3GPP TS 24.380 clause 14.

##### 6.3.3.2.2	Receipt of a SIP INVITE request

On receipt of an initial SIP INVITE request the controlling MCPTT function shall cache SIP feature tags, if received in the Contact header field and if the specific feature tags are supported.

##### 6.3.3.2.3	Sending a SIP response to a SIP INVITE request

###### 6.3.3.2.3.1	Provisional response

When sending SIP provisional responses with the exception of the SIP 100 (Trying) response to the SIP INVITE request the controlling MCPTT function:

1)	shall generate the SIP provisional response;

2)	shall include a P-Asserted-Identity header field with the public service identity of the controlling MCPTT function;

3)	shall include an MCPTT session identity in the Contact header field; and

4)	shall include the following in the Contact header field:

a)	the g.3gpp.mcptt media feature tag;

b)	the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt"; and

c)	the isfocus media feature tag.

###### 6.3.3.2.3.2	Final response

When sending a SIP 200 (OK) response to the initial SIP INVITE request, the controlling MCPTT function:

1)	shall generate the SIP 200 (OK) response according to rules and procedures of 3GPP TS 24.229 [4];

2)	shall include the Session-Expires header field and start supervising the SIP session according to rules and procedures of IETF RFC 4028 [7], "UAS Behavior". The "refresher" parameter in the Session-Expires header field shall be set to "uac";

3)	shall include the option tag "timer" in a Require header field;

4)	shall include a P-Asserted-Identity header field with the public service identity of the controlling MCPTT function;

5)	shall include a SIP URI for the MCPTT session identity in the Contact header field identifying the MCPTT session at the controlling MCPTT function;

6) shall include the following in the Contact header field:

a)	the g.3gpp.mcptt media feature tag;

b)	the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt"; and

c)	the isfocus media feature tag;

7)	shall include Warning header field(s) received in incoming responses to the SIP INVITE request;

8)	shall include the option tag "tdialog" in a Supported header field according to rules and procedures of IETF RFC 4538 [23];

9)	shall include the "norefersub" option tag in a Supported header field according to IETF RFC 4488 [22];

10) shall include the "explicitsub" and "nosub" option tags in a Supported header field according to IETF RFC 7614 [35]; and

11)	shall interact with the media plane as specified in 3GPP TS 24.380 [5].

##### 6.3.3.2.4	Receiving a SIP BYE request

Upon receiving a SIP BYE request the controlling MCPTT function:

1)	shall interact with the media plane as specified in subclause 6.3 in 3GPP TS 24.380 [5] for releasing the media plane resource associated with the SIP session towards the MCPTT client;

NOTE:	The non-controlling MCPTT function is also regarded as a MCPTT client in a temporary MCPTT group session.

2)	shall generate a SIP 200 (OK) response and send the SIP response towards the MCPTT client according to 3GPP TS 24.229 [4];

3)	shall check the MCPTT session release policy as specified in subclause 6.3.8.1 and subclause 6.3.8.2 whether the MCPTT session needs to be released for each participant of the MCPTT session;

4)	if release of the MCPTT session is required, perform the procedures as specified in the subclause 6.3.3.1.5; and

5)	if a release of the MCPTT session is not required, shall send a SIP NOTIFY request to all remaining MCPTT clients in the MCPTT session with a subscription to the conference event package as specified in subclause 10.1.3.4.2.

Upon receiving a SIP 200 (OK) response to the SIP BYE request the controlling MCPTT function shall interact with the media plane as specified in subclause 6.3 in 3GPP TS 24.380 [5] for releasing media plane resources associated with the SIP session with the MCPTT participant.

#### 6.3.3.3	Handling of the acknowledged call setup timer (TNG1)

When the controlling MCPTT function receives a SIP INVITE request to initiate a group session and there are members of the group document retrieved from the group management server that are affiliated and are marked as <on-network-required> as specified in 3GPP TS 24.381 [31], then the controlling MCPTT function shall start timer TNG1 (acknowledged call setup timer) with a timer value as described in Annex B.2.1, prior to sending out SIP INVITE requests inviting group members to the group session.

When the controlling MCPTT function receives all SIP 200 (OK) responses to the SIP INVITE requests, from all affiliated and <on-network-required> members then the controlling MCPTT function shall stop timer TNG1 (acknowledged call setup timer) and if the local counter of the number of SIP 200 (OK) responses received from invited members is greater than or equal to the value of the <on-network-minimum-number-to-start> element of the group document, the controlling MCPTT function shall send a SIP 200 (OK) response to the initiating MCPTT client.

NOTE 1:	MCPTT clients that are affiliated but are not <on-network-required> members that have not yet responded will be considered as joining an ongoing session when the controlling MCPTT function receives SIP 200 (OK) responses from these MCPTT clients.

After expiry of timer TNG1 (acknowledged call setup timer) and the local counter of the number of SIP 200 (OK) responses received from invited members is less than the value of the <on-network-minimum-number-to-start> element of the group document, then the controlling MCPTT function shall wait until further responses have been received from invited clients and the value of the local counter of the number of SIP 200 (OK) responses received from invited members is equal to the <on-network-minimum-number-to-start>, before continuing with the timer TNG1 expiry procedures in this subclause.

After expiry of timer TNG1 (acknowledged call setup timer) and the local counter of the number of SIP 200 (OK) responses received from invited members is greater or equal to the value of the <on-network-minimum-number-to-start> element of the group document, the controlling MCPTT function shall execute the steps described below:

1)	if the <on-network-action-upon-expiration-of-timeout-for-acknowledgement-of-required-members> element configured in the group document for the action on expiry of the timer is set to "proceed" indicating that the controlling MCPTT function should proceed with the setup of the group call, then the controlling MCPTT function:

a)	shall perform the following actions:

i)	generate a SIP 200 (OK) response to the SIP INVITE request as specified in the subclause 6.3.3.2.2 before continuing with the rest of the steps;

ii)	include in the SIP 200 (OK) response the warning text set to "111 group call proceeded without all required group members" in a Warning header field as specified in subclause 4.4;

iii)	include in the SIP 200 (OK) response an SDP answer to the SDP offer in the incoming SIP INVITE request as specified in the subclause 6.3.3.2.1;

iv)	interact with the media plane as specified in 3GPP TS 24.380 [5]; and

NOTE 2:	Resulting media plane processing is completed before the next step is performed.

v)	send a SIP 200 (OK) response to the inviting MCPTT client according to 3GPP TS 24.229 [4];

b)	when a SIP 200 (OK) response to a SIP INVITE request is received from an invited MCPTT client the controlling MCPTT function may send an in-dialog SIP MESSAGE request to the MCPTT client that originated the group session with the text "group call proceeded without all required group members";

c)	when the controlling MCPTT function receives a SIP BYE request from an invited MCPTT client, shall take the actions specified in subclause 6.3.3.2.4 and may send an in-dialog SIP MESSAGE request to the MCPTT client that originated the group session with the text "group call proceeded without all required group members"; and

d)	shall generate a notification package as specified in subclause 6.3.3.4 and send a SIP NOTIFY request according to 3GPP TS 24.229 [4] to the MCPTT clients which have subscribed to the conference state event; and

2)	if the <on-network-action-upon-expiration-of-timeout-for-acknowledgement-of-required-members> element configured in the group document for the action on expiry of the timer is set to "abandon" indicating that the controlling MCPTT function should abandon the setup of the group call, then the controlling MCPTT function shall:

a)	send a SIP 480 (Temporarily Unavailable) response to the MCPTT client that originated the group session with the warning text set to "112 group call abandoned due to required group members not part of the group session" in a Warning header field as specified in subclause 4.4;

b)	for each confirmed dialog at the controlling MCPTT function, send a SIP BYE request towards the MCPTT clients invited to the group session in accordance with 3GPP TS 24.229 [4] and interact with the media plane as specified in 3GPP TS 24.380 [5]; and

c)	for each non-confirmed dialog at the controlling MCPTT function, send a SIP CANCEL request towards the MCPTT clients invited to the group session in accordance with 3GPP TS 24.229 [4].

If the controlling MCPTT function receives a final SIP 4xx, 5xx or 6xx response from an affiliated and <on-network-required> group member prior to expiry of timer TNG1 (acknowledged call setup timer) and based on policy, the controlling MCPTT function decides not to continue with the establishment of the group call without the affiliated and <on-network-required> group member, then the controlling MCPTT function:

NOTE 3:	It is expected that this action is taken if the policy is to abandon the call on expiry of timer TNG1 (acknowledged call setup timer).

1)	shall stop timer TNG1 (acknowledged call setup timer); and

2)	shall forward the final SIP 4xx, 5xx or 6xx response towards the inviting MCPTT client with the warning text set to "112 group call abandoned due to required group member not part of the group session" in a Warning header field as specified in subclause 4.4.

If:

1)	the controlling MCPTT function receives a final SIP 4xx, 5xx or 6xx response from an affiliated and <on-network-required> group member prior to expiry of timer TNG1 (acknowledged call setup timer);

2)	the local counter of the number of SIP 200 (OK) responses received from invited members is greater than or equal to the value of the <on-network-minimum-number-to-start> element of the group document; and

3)	based on policy, the controlling MCPTT function decides to continue with the establishment of the group call without the affiliated and <on-network-required> group member;

then the controlling MCPTT function:

NOTE 4:	It is expected that this action is taken if the policy is to proceed with the call on expiry of timer TNG1 (acknowledged call setup timer).

1)	if all other invited clients have not yet responded, shall continue running timer TNG1 (acknowledged call setup timer); and

2)	if all other invited clients have responded with SIP 200 (OK) responses, shall

a)	stop timer TNG1 (acknowledged call setup timer);

b)	generate SIP 200 (OK) response to the SIP INVITE request as specified in the subclause 6.3.3.2.2 before continuing with the rest of the steps;

c)	include in the SIP 200 (OK) response the warning text set to "111 group call proceeded without all required group members" in a Warning header field as specified in subclause 4.4;

d)	include in the SIP 200 (OK) response an SDP answer to the SDP offer in the incoming SIP INVITE request as specified in the subclause 6.3.3.2.1;

e)	interact with the media plane as specified in 3GPP TS 24.380 [5]; and

NOTE 5:	Resulting media plane processing is completed before the next step is performed.

f)	send a SIP 200 (OK) response to the inviting MCPTT client according to 3GPP TS 24.229 [4].

#### 6.3.3.4	Generating a SIP NOTIFY request

The controlling MCPTT function shall generate a SIP NOTIFY request according to 3GPP TS 24.229 [4] with the clarification in this subclause.

In the SIP NOTIFY request, the controlling MCPTT function:

1)	shall set the P-Asserted-Identity header field to the public service identity of the controlling MCPTT function;

2)	shall include an Event header field set to the 'conference' event package;

3)	shall include an Expires header field set to 3600 seconds according to IETF RFC 4575 [30], as default value;

4)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Preferred-Service header field according to IETF RFC 6050 [9]; and

5)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcpttinfo> element containing the <mcptt-Params> element with:

a)	the <mcptt-calling-group-id> set to the value of the MCPTT group ID;

b)	if the target is a MCPTT user, the value of <mcptt-request-uri> element set to the value of MCPTT ID of the targeted MCPTT user; and

c)	if the target is the non-controlling MCPTT function, the value of <mcptt-request-uri> element set to the constituent MCPTT group ID.In the SIP NOTIFY request, the controlling MCPTT function shall include an application/conference-info+xml MIME body according to IETF RFC 4575 [30] with the following limitations:

1)	the controlling MCPTT function shall include the MCPTT group ID of the MCPTT group in the "entity" attribute of the <conference-info> element;

2)	for each participant in the MCPTT session with the exception of non-controlling MCPTT functions, the controlling MCPTT function shall include a <user> element. The <user> element shall:

NOTE:	Non-controlling MCPTT functions will appear as a participant in temporary group sessions.

a)	include the "entity" attribute. The "entity" attribute:

i)	shall for the MCPTT client, which initiated, joined or re-joined an MCPTT session, include the MCPTT ID of the MCPTT user which originates SIP INVITE request; and

ii)	shall for an invited MCPTT client include the MCPTT ID of the invited MCPTT user in case of a prearranged group call or chat group call;

b)	shall include a single <endpoint> element. The <endpoint> element:

i)	shall include the "entity" attribute; and

ii)	shall include the <status> element indicating the status of the MCPTT session according to RFC 4575; and

c)	may include <role> element.

NOTE:	The usage of <role> is only applicable for human consumption.

#### 6.3.3.5	Handling of the group call timer (TNG3)

##### 6.3.3.5.1	General

When the controlling MCPTT function receives a SIP INVITE request to initiate a group session, then after an MCPTT session identity has been allocated for the group session and if the <on-network-maximum-duration> element is present in the group document as specified in 3GPP TS 24.381 [31], the controlling MCPTT function: shall start timer TNG3 (group call timer) with the value obtained from the <on-network-maximum-duration> element of the group document as specified in 3GPP TS 24.381 [31].

If the <on-network-maximum-duration> element is not present in the group document as specified in 3GPP TS 24.381 [31], then the controlling MCPTT function shall not start timer TNG3 (group call timer).

NOTE 1:	The configuration of <on-network-maximum-duration> element in 3GPP TS 24.381 [31] is mandated for a pre-arranged group and is optional for a chat group.

When merging two or more active group calls into a temporary group call, the controlling MCPTT function(s) hosting the active group calls shall stop timer TNG3 (group call timer) for each group call, and the controlling MCPTT function hosting the temporary group call shall start timer TNG3 (group call timer) for the temporary group call.

NOTE 2:	If the MCPTT server(s) hosting the independent active group calls are different to the MCPTT server that will host the temporary group call, then the MCPTT server(s) hosting the independent active group calls become non-controlling MCPTT function(s) of an MCPTT group, for the temporary group call.

When splitting a temporary group call into independent group calls, the controlling MCPTT function hosting the temporary group call shall stop timer TNG3 (group call timer) and the controlling MCPTT function(s) hosting the independent group calls shall start TNG3 (group call timer) for each group call.

When the last MCPTT client leaves the MCPTT session, the controlling MCPTT function shall stop timer TNG3 (group call timer).

On expiry of timer (group call timer), the controlling MCPTT function shall release the MCPTT session by following the procedures in subclause 6.3.3.1.5;

##### 6.3.3.5.2	Interaction with the in-progress emergency group call timer (TNG2)

If the controlling MCPTT function starts timer TNG2 (in-progress emergency group call timer), it shall not start timer TNG3 (group call timer).

If timer TNG3 (group call timer) is running and the MCPTT group call is upgraded to an MCPTT emergency group call, then the controlling MCPTT function shall stop timer TNG3 (group call timer) and shall start timer TNG2 (in-progress emergency group call timer) with the value obtained from the <group-time-limit> element of the <emergency-call> element of the <on-network> element of the service configuration document as specified in 3GPP TS 24.384 [50].If timer TNG2 (in-progress emergency group call timer) is running and the MCPTT emergency group call is cancelled, then the controlling MCPTT function shall stop timer TNG2 (in-progress emergency group call timer) and shall start timer TNG3 (group call timer) with the value obtained from the <on-network-maximum-duration> element of the group document as specified in 3GPP TS 24.381 [31].

If timer TNG2 (in-progress emergency group call timer) is running and subsequently expires, then the controlling MCPTT function shall start timer TNG3 (group call timer) with the value obtained from the <on-network-maximum-duration> element of the group document as specified in 3GPP TS 24.381 [31].

NOTE:	The above conditions for starting timer TNG2 (in-progress emergency group call timer) and timer TNG3 (group call timer) also apply in the case that these timers are re-started. For example: the case where the timer TNG3 was initially running, the MCPTT group call is upgraded to an MCPTT emergency group call and then the MCPTT emergency group call is cancelled.

### 6.3.4	Non-controlling MCPTT function of an MCPTT group

#### 6.3.4.1	Request initiated by the non-controlling MCPTT function of an MCPTT group

##### 6.3.4.1.1	SDP offer generation

The SDP offer is generated based on the received SDP offer. The SDP offer generated by the non-controlling MCPTT function of an MCPTT group:

1)	shall include only one SDP media-level section for MCPTT speech as contained in the received SDP offer; and

2)	shall include an SDP media-level section for one media-floor control entity, if present in the received SDP offer.

When composing the SDP offer according to 3GPP TS 24.229 [4], the non-controlling MCPTT function of an MCPTT group:

1)	shall replace the IP address and port number for the offered media stream in the received SDP offer with the IP address and port number of the non-controlling MCPTT function;

2)	shall include all media-level attributes from the received SDP offer;

3)	shall replace the IP address and port number for the offered media floor control entity, if any, in the received SDP offer with the IP address and port number of the non-controlling MCPTT function; and

4)	shall include the offered media floor control entity 'fmtp' attributes as specified in 3GPP TS 24.380 [5] clause 14.

##### 6.3.4.1.2	Sending an INVITE request towards the MCPTT client

This subclause is referenced from other procedures.

The non-controlling MCPTT function of an MCPTT group shall generate initial SIP INVITE requests according to 3GPP TS 24.229 [4].

For each SIP INVITE request, the non-controlling MCPTT function of an MCPTT group:

1)	shall generate a new MCPTT session identity for the MCPTT session with the invited MCPTT client and include it in the Contact header field together with the g.3gpp.mcptt media feature tag, the g.3gpp.icsi-ref media feature tag with the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt", and the isfocus media feature tag according to IETF RFC 3840 [16];

2)	shall include an Accept-Contact header field containing the g.3gpp.mcptt media feature tag along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [6];

3)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Asserted-Service-Id header field according to IETF RFC 6050 [9] in the SIP INVITE request;

4)	shall include an Accept-Contact header field with the media feature tag g.3gpp.icsi-ref with the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" along with parameters "require" and "explicit" according to IETF RFC 3841 [6];

5)	shall set the Request-URI to the public service identity of the terminating participating function associated to the MCPTT ID of the MCPTT user to be invited;

NOTE 1:	How the controlling MCPTT function finds the address of the terminating MCPTT participating function is out of the scope of the current release.

NOTE 2:	If the terminating MCPTT user is part of a partner MCPTT system, then the public service identity can identify an entry point in the partner network that is able to identify the terminating participating MCPTT function.

6)	shall copy the application/vnd.3gpp.mcptt-info+xml MIME body in the received SIP INVITE request to the outgoing SIP INVITE request;

7)	shall update the application/vnd.3gpp.mcptt-info+xml MIME body with: a <mcptt-request-uri> element set to the MCPTT ID of the invited MCPTT user;

8)	shall include the public service identity of the non-controlling MCPTT function in the P-Asserted-Identity header field;

9)	shall include the received Referred-By header field with the public user identity of the inviting MCPTT client;

10)	should include the Session-Expires header field according to rules and procedures of IETF RFC 4028 [7]. The refresher parameter shall be omitted;

11)	shall include the Supported header field set to "timer";

12)	shall include an unmodified Answer-Mode header field, if present in the incoming SIP INVITE request;and

13)	shall include the warning text set to "yyy MCPTT group is regrouped" in a Warning header field as specified in subclause 4.4;

NOTE 3:	As long as the MCPTT group is regrouped the floor control messages in the media plane includes a grouped regrouped indication as specified in 3GPP TS 24.380 [5].

##### 6.3.4.1.3	Sending a SIP INFO request

This subclause is referenced from other procedures.

The non-controlling MCPTT function shall generate a SIP INFO request according to rules and procedures of 3GPP TS 24.229 [4] and IETF RFC 6086 [64].

The non-controlling MCPTT function:

1)	shall include the Info-Package header field set to g.3gpp.mcptt-floor-request;

2)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcptt-request-uri> set to the temporary MCPTT group ID and the <mcptt-calling-group-id> element with the constituent MCPTT group ID; and

3)	shall include an application/vnd.3gpp.mcptt-floor-request+xml MIME body with the Content-Disposition header field set to "Info-Package". For each current speaker the application/vnd.3gpp.mcptt-floor-request+xml MIME body shall be populated as follows:

a)	the <floor-type> element set to "general" or "dual" as described in subclause F.5.3;

b)	the SSRC of the MCPTT client with the permission to send media in the <ssrc> element;

c)	the actual floor priority in the <floor-priority> element;

d)	the MCPTT ID of the MCPTT user with the permission to send media in the <user-id> element;

e)	the queueing capability in the <queueing-capability> element of the <track-info> element;

f)	the participant type in the <participant-type> in the <track-info> element;

g)	one or more <floor-participant-reference> elements in the <track-info> element in the same order as the would appear in the Track Info field as specified in 3GPP TS 24.380 [5] subclause 8.2.3.13; and

h)	if available, additional information in the <floor-indicator> element.

##### 6.3.4.1.4	Sending an INVITE request towards the controlling MCPTT function

This subclause is referenced from other procedures.

The non-controlling MCPTT function shall generate a SIP INVITE request according to rules and procedures of 3GPP TS 24.229 [4].

The non-controlling MCPTT function:

1)	shall include in the Contact header field the g.3gpp.mcptt media feature tag, the g.3gpp.icsi-ref media feature tag with the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt", and the isfocus media feature tag according to IETF RFC 3840 [16];

2)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Asserted-Service-Id header field according to IETF RFC 6050 [9] in the SIP INVITE request;

3)	shall set the Request-URI to the public service identity of the controlling MCPTT function based on the <mcptt-request-uri> element received in the "SIP INVITE request for controlling MCPTT function of an MCPTT group";

NOTE 1:	How the non-controlling MCPTT function finds the address of the controlling MCPTT function is out of the scope of the current release.

NOTE 2:	If the terminating MCPTT user is part of a partner MCPTT system, then the public service identity can identify an entry point in the partner network that is able to identify the terminating participating MCPTT function.

4)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body with:

a)	the <session-type> element set to "prearranged";

NOTE 3:	The <session-type> element set to "prearranged" regardless of which type of group the constituent MCPTT group is.

b)	the <mcptt-request-uri> element set to the TGI retrieved from the <on-network-regrouped> element in the group document;

c)	the <mcptt-calling-user-id> element set to the constituent MCPTT group ID; and

d	the <required> element set to "true", if the group document retrieved from the group management server contains <on-network-required> group members as specified in 3GPP TS 24.381 [31];

5)	shall include the public service identity of the non-controlling MCPTT function in the P-Asserted-Identity header field;

6)	should include the Session-Expires header field according to rules and procedures of IETF RFC 4028 [7]. The refresher parameter shall be omitted; and

7)	shall include the Supported header field set to "timer".

#### 6.3.4.2	Requests terminated by the non-controlling MCPTT function of an MCPTT group

##### 6.3.4.2.1	SDP answer generation

When composing the SDP answer according to 3GPP TS 24.229 [4], the non-controlling MCPTT function of an MCPTT group:

1)	for the accepted media stream in the received SDP offer:

a)	shall replace the IP address and port number in the received SDP offer with the IP address and port number of the non-controlling MCPTT function; and

2)	for the accepted media-floor control entity, if present in the received SDP offer:

a)	shall replace the IP address and port number in the received SDP offer with the IP address and port number of the non-controlling MCPTT function; and

b)	shall include 'fmtp' attributes as specified in 3GPP TS 24.380 [5] clause 14.

##### 6.3.4.2.2	Sending a SIP response to the SIP INVITE request

###### 6.3.4.2.2.1	Sending a SIP 183 (Session Progress) response

When sending a SIP 183 (Session Progress) the non-controlling MCPTT function of an MCPTT group:

1)	shall generate a SIP 183 (Session Progress) response according to 3GPP TS 24.229 [4];

2)	shall include the following in the Contact header field:

a)	the g.3gpp.mcptt media feature tag; and

b)	the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt";

3)	shall include the public service identity of the non-controlling MCPTT function in the P-Asserted-Identity header field; and

4)	shall include the option tag "tdialog" in a Supported header field according to rules and procedures of IETF RFC 4538 [23];

###### 6.3.4.2.2.2	Sending a SIP 200 (OK) response

When sending a SIP 200 (OK) response, the non-controlling MCPTT function of an MCPTT group:

1)	shall generate the SIP 200 (OK) response according to rules and procedures of 3GPP TS 24.229 [4];

2)	shall include the Session-Expires header field and start supervising the SIP session according to rules and procedures of IETF RFC 4028 [7], "UAS Behavior". The "refresher" parameter in the Session-Expires header field shall be set to "uac";

3)	shall include the option tag "timer" in a Require header field;

4)	shall include the public service identity of the non-controlling MCPTT function in the P-Asserted-Identity header field;

5) shall include the following in the Contact header field:

a)	the g.3gpp.mcptt media feature tag; and

b)	the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt";

6)	shall include Warning header field(s) received in incoming responses to the SIP INVITE request;

7)	shall include the option tag "tdialog" in a Supported header field according to rules and procedures of IETF RFC 4538 [23]; and

8)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcptt-called-party-id> element set to the constituent MCPTT group ID and the <floor-state> element set to the state of the floor.

#### 6.3.4.3	Generating a SIP NOTIFY request

The non-controlling MCPTT function shall generate a SIP NOTIFY request according to 3GPP TS 24.229 [4] with the clarification in this subclause.

In the SIP NOTIFY request, the non-controlling MCPTT function:

1)	shall set the P-Asserted-Identity header field to the public service identity of the non-controlling MCPTT function;

2)	shall include an Event header field set to the "conference" event package;

3)	shall include an Expires header field set to 3600 seconds according to IETF RFC 4575 [30], as default value;

4)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Preferred-Service header field according to IETF RFC 6050 [9]; and

5)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcpttinfo> element containing the <mcptt-Params> element with:

a)	the <mcptt-calling-group-id> set to the value of the constituent MCPTT group ID;

b)	if the target is a MCPTT user, the value of <mcptt-request-uri> element set to the MCPTT ID of the targeted MCPTT user; and

c)	if the target is the controlling MCPTT function the value of <mcptt-request-uri> element set to the temporary MCPTT group ID.

In the SIP NOTIFY request, the non-controlling MCPTT function shall include application/conference-info+xml MIME body according to IETF RFC 4575 [30] as specified in subclause 6.3.3.4 with the following exceptions:

1)	the non-controlling MCPTT function shall not regard the controlling MCPTT function as a participant and not include the controlling MCPTT function in a <user> element; and

NOTE:	The controlling MCPTT function initiated the temporary group call and will appear as a participant in the group session.

2)	the non-controlling MCPTT function shall include stored conference status information received in SIP NOTIFY requests from the non-controlling MCPTT function in subclause 10.1.3.5.3 and status information about own participants.

### 6.3.5	Retrieving and processing a group document

#### 6.3.5.1	General

This subclause describes how an MCPTT server accesses a group document from a group management server. The MCPTT server which accesses a group document performs the role of a controlling MCPTT function or performs the role of a non-controlling MCPTT function of an MCPTT group when accessing a group document. In such cases, for a group call:

-	the controlling MCPTT function and group management server are both located in the primary MCPTT system;

-	the controlling MCPTT function and group management server are both located in a partner MCPTT system;

-	the controlling MCPTT function is located in the primary MCPTT system and accesses a group management server in the primary MCPTT system and a non-controlling MCPTT function of an MCPTT group is located in a partner MCPTT system and accesses a group management server in the partner MCPTT system; or

-	the controlling MCPTT function and non-controlling MCPTT function(s) of an MCPTT group are located in the primary MCPTT system and access group management servers in the primary MCPTT system.

When the MCPTT server receives a SIP INVITE request that requires it to access a group document, it uses an MCPTT group ID or a temporary MCPTT group identity (TGI) which was created by the group regrouping operation as specified in 3GPP TS 24.381 [31].

The MCPTT server can cache the group document associated with an MCPTT group or temporary group, and can subscribe to be notified of changes to the group document associated with an MCPTT group or temporary group as specified in 3GPP TS 24.381 [31].

NOTE 1:	During the group regrouping operation as specified in 3GPP TS 24.381 [31], the controlling MCPTT function is notified of the constituent MCPTT group identities associated with the TGI.

If the group data associated with an MCPTT group ID or TGI cached in the MCPTT server is removed, the MCPTT server re-subscribes for changes in the group information associated with the MCPTT group ID or TGI.

NOTE 2:	Re-subscription can occur prior to the receipt of an SIP INVITE request containing an MCPTT group ID or TGI of a group document which is no longer cached on the MCPTT server.

#### 6.3.5.2	Rules for retrieving Group Document(s)

NOTE 1:	In this subclause, "MCPTT server" can refer to either the controlling MCPTT function of an MCPTT group or the non-controlling MCPTT function of an MCPTT group.

Upon receipt of a SIP INVITE request:

1)	if the MCPTT server is not yet subscribed to the group document for the group identity in the <mcptt-request-uri> element of the application/vnd.3gpp.mcptt-info+xml MIME body of the SIP INVITE request, the MCPTT server shall subscribe to the "xcap-diff" event-package for the group document of this group identity as specified in 3GPP TS 24.381 [31];

NOTE 2:	The group identity in the <mcptt-request-uri> element is either an MCPTT group ID or a temporary MCPTT group identity (TGI).

NOTE 3:	As a group document can potentially have a large content, the controlling MCPTT function of an MCPTT group can subscribe to the group document indicating support of content-indirection as defined in IETF RFC 4483 [32], by following the procedures in 3GPP TS 24.381 [31].

2)	upon receipt of a SIP 404 (Not Found) response as a result of attempting to subscribe to the "xcap-diff" event-package for the group document of the group identity in the <mcptt-request-uri> element of the application/vnd.3gpp.mcptt-info+xml MIME body of the SIP INVITE request as specified in 3GPP TS 24.381 [31], the MCPTT server shall send the SIP 404 (Not Found) response with the warning text set to "113 group document does not exist" in a Warning header field as specified in subclause 4.4. Otherwise, continue with the rest of the steps;

3)	upon receipt of any other SIP 4xx, SIP 5xx or SIP 6xx response as a result of attempting to subscribe to the "xcap-diff" event-package for the group document of the group identity in the <mcptt-request-uri> element of the application/vnd.3gpp.mcptt-info+xml MIME body of the SIP INVITE request as specified in 3GPP TS 24.381 [31], the MCPTT server shall send the SIP final response with the warning text set to "114 unable to retrieve group document" in a Warning header field as specified in subclause 4.4 and shall not continue with the rest of the steps;

4)	upon receipt of a notification from the group management server containing the group document for the group identity in the <mcptt-request-uri> element of the application/vnd.3gpp.mcptt-info MIME body of the SIP INVITE request, or if the group document is already cached:

a)	if the MCPTT server is a non-controlling function of an MCPTT group, then the MCPTT server shall exit this subclause; and

b)	if the MCPTT server is a controlling function of an MCPTT group, then the MCPTT server shall determine if the group document is for a TGI or an MCPTT group ID as follows:

i)	if the group document includes an <on-network-temporary> element, then the group document is associated with a TGI;

ii)	if the group document does not include an <on-network-temporary> element or an <on-network-regrouped> element, then the group document is associated with an MCPTT ID that has not been regrouped; and

iii)	if the group document does not include an <on-network-temporary> element but includes an <on-network-regrouped> element, then the group document is associated with an MCPTT ID that has been regrouped;

5)	if the SIP INVITE request is a "SIP INVITE request for controlling function of an MCPTT group" and the group identity in the <mcptt-request-uri> element of the application/vnd.3gpp.mcptt-info+xml MIME body of the SIP-INVITE request is an MCPTT group ID that has not been re-grouped, the MCPTT server shall access the following elements from the group document as specified in 3GPP TS 24.381 [31]:

a)	if the <on-network-disabled> element is present in the group document, shall send a SIP 403 (Forbidden) response with the warning text set to "115 group is disabled" in a Warning header field as specified in subclause 4.4 and shall not continue with the rest of the steps;

b)	if the <list> element of the <list-service> element does not contain an entry matching the MCPTT ID of the user in the SIP INVITE request, shall send a SIP 403 (Forbidden) response with the warning text set to "116 user is not part of the MCPTT group" in a Warning header field as specified in subclause 4.4 and shall not continue with the rest of the steps;

c)	if the <on-network-invite-members> element is set to "true" and if the SIP INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <session-type> element containing a value not set to "prearranged", shall return a SIP 404 (Not Found) response with the warning text set to "117 the group identity indicated in the request is a prearranged group" as specified in subclause 4.4 "Warning header field" and shall not continue with the rest of the steps; and

d)	if the <on-network-invite-members> element is set to "false" and if the SIP INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <session-type> element containing a value not set to "chat" shall return a SIP 404 (Not Found) response with the warning text set to "118 the group identity indicated in the request is a chat group" as specified in subclause 4.4 "Warning header field" and shall not continue with the rest of the steps;

6)	if the SIP INVITE request is a "SIP INVITE request for controlling function of an MCPTT group" and the group document for the group identity in the <mcptt-request-uri> element of the application/vnd.3gpp.mcptt-info MIME body of the SIP INVITE request is associated with an MCPTT group ID that has been regrouped, the MCPTT server:

a)	shall obtain the TGI associated with the regrouped group, by accessing the "temporary-MCPTT-group-ID" attribute of the <regrouped> element of the group document associated with the MCPTT ID in the <mcptt-request-uri> element of the application/vnd.3gpp.mcptt-info MIME body of the SIP INVITE request;

b)	if not hosting the TGI, shall:

i)	stop processing the SIP INVITE request; and

ii)	return a SIP 302 (Moved Temporarily) response with:

A)	a Contact header field set to the PSI of the MCPTT server hosting the TGI; and

B)	an application/vnd.3gpp.mcptt-info MIME body with a <mcptt-request-uri> element set to the TGI; and

c)	if hosting the TGI, and the call to the temporary group is in progress, shall:

i)	associate the MCPTT ID of the calling user with the temporary group call;

ii)	interact with the media plane as specified in 3GPP TS 24.380 [5]; and

iii)	exit this subclause;

d)	if hosting the TGI and the call to the temporary group is not in progress, shall subscribe to the "xcap-diff" event-package for the group document of the TGI as specified in 3GPP TS 24.381 [31], if not already subscribed;

e)	upon receipt of a SIP 404 (Not Found) response as a result of attempting to subscribing to the "xcap-diff" event-package for the group document(s) for the MCPTT group ID(s) associated to the TGI as specified in 3GPP TS 24.381 [31], shall send the SIP 404 (Not Found) response with the warning text set to "113 group document does not exist" in a Warning header field as specified in subclause 4.4 and shall not continue with the rest of the steps;

f)	upon receipt of any other SIP 4xx, SIP 5xx or SIP 6xx response as a result of attempting to subscribe to the "xcap-diff" event-package for the group document(s) for the MCPTT group ID(s) associated to the TGI as specified in 3GPP TS 24.381 [31], shall send the SIP final response with the warning text set to "114 unable to retrieve group document" in a Warning header field as specified in subclause 4.4 and shall not continue with the rest of the steps;

g)	upon receipt of a notification containing the group document for the TGI, or if the group document is already cached, shall obtain the MCPTT IDs of the constituent groups by accessing the <constituent-MCPTT-group-ID> element(s) of the group document for the TGI; and

h)	if:

i)	the <associated-group-id> element with an MCPTT ID is included in the application/vnd.3gpp.mcptt-info MIME body;

ii)	the MCPTT ID is present in one of the instances of the <constituent-MCPTT-group-ID> element in the group document; and

iii)	the group is not homed by the MCPTT server;

shall exit this procedure and authorize the MCPTT user at a non-controlling MCPTT function of a MCPTT group;

NOTE 4:	The non-controlling function of an MCPTT group can be located in the primary MCPTT system or a partner MCPTT system.

7)	for the MCPTT ID of each constituent group, shall follow the actions below in step 8); and

8)	if the SIP INVITE request is a "SIP INVITE request for controlling function of an MCPTT group" and the group document for the group identity in the <mcptt-request-uri> element of the application/vnd.3gpp.mcptt-info MIME body of the SIP INVITE request retrieved is associated with a TGI and if:

a)	the application/vnd.3gpp.mcptt-info+xml includes an <associated-group-id> element, determine if the constituent MCPTT group identified by the <associated-group-id> element is homed at the MCPTT server and if that is the case:

i)	shall subscribe to the "xcap-diff" event-package for the group document for the constituent MCPTT group ID as specified in 3GPP TS 24.381 [31], if not already subscribed;

ii)	upon receipt of a SIP 404 (Not Found) response as a result of attempting to subscribing to the "xcap-diff" event-package for the group document for the MCPTT group ID associated to the TGI as specified in 3GPP TS 24.381 [31], shall send the SIP 404 (Not Found) response with the warning text set to "113 group document does not exist" in a Warning header field as specified in subclause 4.4 and shall not continue with the rest of the steps; and

iii)	upon receipt of any other SIP 4xx, SIP 5xx or SIP 6xx response as a result of attempting to subscribe to the "xcap-diff" event-package for the group document for the MCPTT group ID associated to the TGI as specified in 3GPP TS 24.381 [31], shall send the SIP final response with the warning text set to "114 unable to retrieve group document" in a Warning header field as specified in subclause 4.4 and shall not continue with the rest of the steps; and

b)	the application/vnd.3gpp.mcptt-info+xml does not include an <associated-group-id> element, for each MCPTT ID contained in each instance of the <constituent-MCPTT-group-ID> element of the group document for the TGI:

i)	shall determine if the group identity is homed on the controlling MCPTT function of an MCPTT group or homed on a non-controlling MCPTT function of an MCPTT group;

NOTE 5:	The non-controlling function of an MCPTT group can be located in the primary MCPTT system or a partner MCPTT system.

ii)	for each constituent MCPTT group ID that is homed on the controlling MCPTT function of an MCPTT group shall subscribe to the "xcap-diff" event-package for the group document for the constituent MCPTT group ID as specified in 3GPP TS 24.381 [31], if not already subscribed;

NOTE 6:	As soon as an error occurs when subscribing for a group document of a constituent MCPTT group ID, the controlling MCPTT function of an MCPTT group stops subscribing to any further group documents of constituent MCPTT group IDs.

iii)	upon receipt of a SIP 404 (Not Found) response as a result of attempting to subscribing to the "xcap-diff" event-package for the group document(s) for the MCPTT group ID(s) associated to the TGI as specified in 3GPP TS 24.381 [31], shall send the SIP 404 (Not Found) response with the warning text set to "113 group document does not exist" in a Warning header field as specified in subclause 4.4 and shall not continue with the rest of the steps;

iv)	upon receipt of any other SIP 4xx, SIP 5xx or SIP 6xx response as a result of attempting to subscribe to the "xcap-diff" event-package for the group document(s) for the MCPTT group ID(s) associated to the TGI as specified in 3GPP TS 24.381 [31], shall send the SIP final response with the warning text set to "114 unable to retrieve group document" in a Warning header field as specified in subclause 4.4 and shall not continue with the rest of the steps; and

c)	when all group document(s) for all constituent groups homed at the MCPTT server have been retrieved and if the MCPTT ID of the user identified in the <mcptt-calling-user-id> element of the application/vnd.3gpp.mcptt-info+xml MIME body:

i)	is a member of one the retrieved constituent MCPTT groups, received, shall exit this procedure; and

ii)	is not a member of any of the retrieved constituent group documents, shall determine that the MCPTT ID of the MCPTT user needs to be authorised by a non-controlling MCPTT function of an MCPTT group and exit this procedure.

#### 6.3.5.3	Rules for joining a group session

The following conditions shall be met for the controlling MCPTT function to allow an MCPTT user to join an existing group session:

1)	an <entry> element exists in the <list> element of the group document for the MCPTT user;

2)	a <rule> exists in the group document with:

a)	the <is-list-member> element of the <conditions> element present and with the <join-handling> element of the corresponding <actions> element set to "true"; or

b)	the <identity> element of the <conditions> element containing an entry matching the MCPTT ID in the SIP INVITE request, with the <join-handling> element of the <actions> element set to "true"; and

3)	if the <supported-services> element is present, it contains:

a)	a <service> element containing an "enabler" attribute which is set to the MCPTT ICSI; and

b)	if a <group-media> element is present, an entry set to "MCPTT speech".

If all of the above conditions are not met, then the MCPTT user shall not be authorised to join the group session.

#### 6.3.5.4	Rules for initiating a prearranged group session

The following conditions shall be met for a controlling MCPTT function or non-controlling MCPTT function of an MCPTT group to initiate a group session for the requesting MCPTT user:

1)	if the <on-network-regrouped> element in the <list-service> element is present in the group document and if the MCPTT ID indicated in the incoming INVITE request is the same as the MCPTT group ID in the "temporary-MCPTT-group-ID" attribute of the <on-network-regrouped> element; or

2)	if the <on-network-regrouped> element in the <list-service> element of the MCPTT group document is not present in the group document:and:

and:

1)	an <entry> element exists in the <list> element of the group document for the MCPTT user;

2)	a <rule> exists in the group document with:

a)	the <is-list-member> element of the <conditions> element present and with the <allow-initiate-conference> element of the corresponding <actions> element set to "true"; or

b)	the <identity> element of the <conditions> element containing an entry matching the MCPTT ID in the SIP INVITE request, with the <allow-initiate-conference> element of the <actions> element is set to "true"; and

3)	if the <supported-services> element is present, it contains:

a)	a <service> element containing an "enabler" attribute which is set to the MCPTT ICSI; and

b)	if a <group-media> element is present, an entry set to "MCPTT speech".

then the MCPTT user shall not be authorised to initiate the group session.

#### 6.3.5.5	Determining the group members to invite

The MCPTT server shall only invite affiliated group members to a group session. The MCPTT server determines the affiliated members from the entries contained in the <list> element of the group document by following the procedures specified in subclause 6.3.6.

NOTE 1:	The term "affiliated group members" used above also includes those members that are implicitly affiliated by the controlling MCPTT function.

If the number of members of the MCPTT group exceeds the value contained in the <on-network-max-participant-count> element the MCPTT server shall invite only <on-network-max-participant-count> members from the list, but shall prioritise inviting those group members to the group session that have an <entry> element in the <list> element with a <on-network-required> element present.

NOTE 2:	The <on-network-max-participant-count> element indicates the maximum number of participants allowed in the group session. The <on-network-required> element is used to determine which group members need to acknowledge the group call before audio transmission can proceed.

NOTE 3:	Other requirements for how the controlling MCPTT function selects which of the <on-network-max-participant-count> members to invite is outside the scope of this specification.

NOTE 4:	It is assumed that validation checks are performed at the group management server to ensure that the <on-network-max-participant-count> cannot be less than the number of <on-network-required> users.

### 6.3.6	Affiliation check

The MCPTT server checks if an MCPTT user is affiliated to an MCPTT group at an MCPTT client by following the procedures specified below:

1.	the MCPTT server shall find the applicable MCPTT group information entry as an MCPTT group information entry of the list of MCPTT group information entries described in subclause 9.2.2.3.2, such that the MCPTT group ID of the MCPTT group information entry is equal to the MCPTT group identity of the MCPTT group. If the applicable MCPTT group information entry cannot be found, then the MCPTT server shall determine that the MCPTT user is not affiliated to the MCPTT group at the MCPTT client and the MCPTT server shall not continue with rest of the steps;

2.	the MCPTT server shall find the applicable MCPTT user information entry as an MCPTT user information entry of the list of MCPTT user information entries of the applicable MCPTT group information entry, such that the MCPTT ID of the MCPTT user information entry is equal to the MCPTT ID of the MCPTT user. If the applicable MCPTT user information entry cannot be found, then the MCPTT server shall determine that the MCPTT user is not affiliated to the MCPTT group at the MCPTT client and the MCPTT server shall not continue with rest of the steps;

3.	if the MCPTT client ID of the MCPTT client cannot be found in the list of MCPTT client information entries of the applicable MCPTT user information entry, then the MCPTT server shall determine that the MCPTT user is not affiliated to the MCPTT group at the MCPTT client and the MCPTT server shall not continue with rest of the steps;

4.	if the expiration time of the applicable MCPTT user information entry has been reached, then the MCPTT server shall determine that the MCPTT user is not affiliated to the MCPTT group at the MCPTT client and the MCPTT server shall not continue with rest of the steps; and

5.	the MCPTT server shall determine that the MCPTT user is affiliated to the MCPTT group at the MCPTT client.

### 6.3.7	Error handling

#### 6.3.7.1	Public service identity does not exist

Upon receiving a request that includes the Request-URI set to a public service identity that is not allocated in the participating or the controlling MCPTT function, the participating or the controlling MCPTT function shall return a SIP 404 (Not Found) response.

### 6.3.8	Session release policy

#### 6.3.8.1	Session release policy for group call

If:

1)	the controlling MCPTT function receives an indication from the media plane that the T4 (Inactivity) timer specified in 3GPP TS 24.380 [5] expired;

2)	there are only one or no participants in the MCPTT session;

3)	if the call is a pre-arranged group call and if it is according to local policy, the initiator of the group call leaves the MCPTT session;

4)	the minimum number of affiliated MCPTT group members is not present; or

5)	timer TNG3 (group call timer) expires;

the controlling MCPTT function shall release the MCPTT session for the group call.

#### 6.3.8.2	Session release policy for private call

If:

1)	the controlling MCPTT function receives an indication from the media plane that the T4 (Inactivity) timer specified in 3GPP TS 24.380 [5] expired;

2)	the MCPTT session has lasted longer than the maximum of duration of private call; or

3)	there are only one or no participants in the MCPTT session;

the controlling MCPTT function shall release the MCPTT session for a private call.

## 6.4	Implicit floor request

An initial SIP INVITE request fulfilling one of the following criteria shall be regarded by the MCPTT server as an implicit floor request when the MCPTT client:

1)	initiates an MCPTT speech session or initiates a pre-established session; and

2)	includes the "mc_implicit_request" 'fmtp' attribute in the associated UDP stream for the floor control in the SDP offer/answer as specified in 3GPP TS 24.380 [5] clause 12.

In all other cases the SIP INVITE request shall be regarded as received without an implicit floor request.

When using a pre-established session the MCPTT server shall regard the SIP REFER request as an implicit floor request:

1)	if the pre-established session was established with an implicit floor request and if an SDP offer is not included in a "body" parameter in the headers portion of the SIP URI in the <entry> element of the application/resource-lists MIME body, referenced by the "cid" URL in the Refer-To header field;

2)	if the pre-established session was established with an implicit floor request, an SDP offer is included in a "body" parameter in the headers portion of the SIP URI in the <entry> element of the application/resource-lists MIME body, referenced by the "cid" URL in the Refer-To header field, and this SDP includes the "mc_implicit_request" media level attribute in the associated UDP stream for the floor control in the SDP offer; or

3)	if the pre-established session was established without an implicit floor request and the SDP offer in a "body" parameter in the headers portion of the SIP URI in the <entry> element of the application/resource-lists MIME body, referenced by the "cid" URL in the Refer-To header field includes the "mc_implicit_request" media level attribute in the associated UDP stream for the floor control in the SDP offer.

In all other cases the SIP REFER request shall be regarded as received without an implicit floor request.

## 6.5	Handling of MIME bodies in a SIP message

The MCPTT client and the MCPTT server shall support several MIME bodies in SIP request and SIP responses.

When the MCPTT client or the MCPTT server sends a SIP message and the SIP message contains more than one MIME body, the MCPTT client or the MCPTT server:

1)	shall, as specified in IETF RFC 2046 [21], include one Content-Type header field with the value set to multipart/mixed and with a boundary delimiter parameter set to any chosen value;

2)	for each MIME body:

a)	shall insert the boundary delimiter;

b)	shall insert the Content-Type header field with the MIME type of the MIME body; and

c)	shall insert the content of the MIME body;

3)	shall insert a final boundary delimiter; and

4)	if an SDP offer or an SDP answer is one of the MIME bodies, shall insert the application/sdp MIME body as the first MIME body.

NOTE:	The reason for inserting the application/sdp MIME body as the first body is that if a functional entity in the underlying SIP core does not understand multiple MIME bodies, the functional entity will ignore all MIME bodies with the exception of the first MIME body. The order of multiple MCPTT application MIME bodies in a SIP message is irrelevant.

When the MCPTT client or the MCPTT server sends a SIP message and the SIP message contains only one MIME body, the MCPTT client or the MCPTT server:

1)	shall include a Content-Type header field set to the MIME type of the MIME body; and

2)	shall insert the content of the MIME body.

## 6.6	Confidentiality and Integrity Protection

### 6.6.1	General

#### 6.6.1.1	Applicability and exclusions

The procedures in subclauses 6.6 apply in general to all procedures described in clause 9, clause 10, clause 11 and clause 12 with the exception that the confidentiality and integrity protection procedures for the registration and service authorisation procedures are described in clause 7.

#### 6.6.1.2	Performing XML content encryption

Whenever the MCPTT UE includes XML elements or attributes pertaining to the data specified in subclause 4.8 in SIP requests or SIP responses, the MCPTT UE shall perform the procedures in subclause 6.6.2.3.1.

Whenever the MCPTT server includes XML elements or attributes pertaining to the data specified in subclause 4.8 in SIP requests or SIP responses, the MCPTT server shall perform the procedures in subclause 6.6.2.3.2, with the exception that when the MCPTT server receives a SIP request with XML elements or attributes in an MIME body that need to be copied from the incoming SIP request to an outgoing SIP request without modification, the MCPTT server shall perform the procedures specified in subclause 6.6.2.5.

NOTE:	The procedures in subclause 6.6.2.3.1 and subclause 6.6.2.3.2 first determine (by referring to configuration) if confidentiality protection is enabled and then call the necessary procedures to encrypt the contents of the XML elements if confidentiality protection is enabled.

#### 6.6.1.3	Performing integrity protection on an XML body

The functional entity shall perform the procedures in the subclause just prior to sending a SIP request or SIP response.

1)	The MCPTT UE shall perform the procedures in subclause 6.6.3.3.1; and

2)	The MCPTT server shall perform the procedures in subclause 6.6.3.3.2.

NOTE:	The procedures in subclause 6.6.3.3.1 and subclause 6.6.3.3.2 first determine if integrity protection of XML MIME bodies is required and then calls the necessary procedures to integrity protect each XML MIME body if integrity protection is required. Each XML MIME body has its own signature.

#### 6.6.1.4	Verifying integrity of an XML body and decrypting XML elements

Whenever the functional entity (i.e. MCPTT UE or MCPTT server) receives a SIP request or a SIP response, the functional entity shall perform the following procedures before performing any other procedures.

1)	The functional entity shall determine if integrity protection has been applied to an XML MIME body by following the procedures in subclause 6.6.3.4.1 and if integrity protection has been applied:

a)	shall use the keying information described in subclause 6.6.3.2 and the procedures described in subclause 6.6.3.4.2 to verify the integrity of the XML MIME body; and

b)	if the integrity protection checks fail shall not perform any further procedures in this clause;

2)	The functional entity shall determine whether confidentiality protection has been applied to XML elements in XML MIME bodies in a SIP request or SIP response, pertaining to the data specified in subclause 4.8, by following the procedures in subclause 6.6.2.4.1, and if confidentiality protection has been applied:

a)	shall use the keying information described in subclause 6.6.2.2 along with the procedures described in subclause 6.6.2.4.2 to decrypt the received values; and

b)	if any decryption procedures fail, shall not perform any further procedures in this clause.

### 6.6.2	Confidentiality Protection.

#### 6.6.2.1	General

In general, confidentiality protection is applied to specific XML elements and attributes in XML MIME bodies in SIP requests and responses as specified in subclause 4.8. However in the case of SIP REFER requests used for pre-established sessions, confidentiality protection is required for:

the targeted MCPTT ID or MCPTT Group ID, placed in a "uri" attribute of an <entry> element of an application/resource-lists+xml MIME body that is pointed to by a "cid" URL located in the Refer-To header field of the SIP REFER request; and

-	sensitive XML data included in MIME bodies which are placed in the hname "body" URI header field of the URI included in the "uri" attribute of the <entry> element of the application/resource-lists+xml MIME body.

Configuration for applying confidentiality protection is not selective to a specific XML element or attribute of the data described in subclause 4.8. If configuration for confidentiality protection is turned on, then all XML elements and attributes described in subclause 4.8 are confidentiality protected. If configuration for confidentiality protection is turned off, then no XML content in SIP requests and SIP responses are confidentiality protected.

#### 6.6.2.2	Keys used in confidentiality protection procedures

Confidentiality protection uses an XPK to encrypt the data which (depending on who is the sender and who is the receiver of the encrypted information) can be a Client-Server Key (CSK) or Server Protection Key (SPK) as specified in subclause 4.8. An XPK-ID (CSK-ID/SPK-ID) is used to key the XPK (CSK/SPK). It is assumed that before the procedures in this subclause are called, the CSK/CSK-ID and/or SPK/SPK-ID are available on the sender and recipient of the encrypted content as described in subclause 4.8.

The procedures in subclause 6.6.2.3 and subclause 6.6.2.4 are used with a XPK equal to the CSK and a XPK-ID equal to the CSK-ID in the following circumstances as described in 3GPP TS 33.179 [46]:

1)	MCPTT client sends confidentiality protected content to an MCPTT server; and

2)	MCPTT server sends confidentiality protected content to an MCPTT client.

The procedure in subclause 6.6.2.3 and subclause 6.6.2.4 are used with a XPK equal to the SPK and a XPK-ID equal to the SPK-ID in the following circumstances as described in 3GPP TS 33.179 [46]:

1)	MCPTT server sends confidentiality protected content to an MCPTT server in the same domain; and

2)	MCPTT server sends confidentiality protected content to an MCPTT server in another domain.

#### 6.6.2.3	Procedures for sending confidentiality protected content

##### 6.6.2.3.1	MCPTT client

If the <confidentiality-protection> element in the Service Configuration document as specified in 3GPP TS 24.384 [50] is set to "true" or no <confidentiality-protection> element is present in the Service Configuration document, then sending confidentiality protected content from the MCPTT client to the MCPTT server is enabled, and the MCPTT client:

1)	shall use the appropriate keying information specified in subclause 6.6.2.2;

2)	shall perform the procedures in subclause 6.6.2.3.3 to confidentiality protect XML elements containing the content described in subclause 4.8; and

3)	shall perform the procedures in subclause 6.6.2.3.4 to confidentiality protect URIs in XML attributes for URIs described in subclause 4.8.

If the <confidentiality-protection> element in the Service Configuration document as specified in 3GPP TS 24.384 [50] is set to "false", then sending confidentiality protected content from the MCPTT client to the MCPTT server is disabled, and content is included in XML elements and attributes without encryption.

##### 6.6.2.3.2	MCPTT server

If the <confidentiality-protection> element in the Service Configuration document as specified in 3GPP TS 24.384 [50] is set to "true" or no <confidentiality-protection> element is present in the Service Configuration document, then sending confidentiality protected content from the MCPTT server to the MCPTT client is enabled. If the <allow-signalling-protection> element of the <protection-between-mcptt-servers> element is set to "true" in the Service Configuration document as specified in 3GPP TS 24.384 [50] or no <allow-signalling-protection> element is present in the Service Configuration document, then sending confidentiality protected content between MCPTT servers is enabled.

When sending confidentiality protected content, the MCPTT server:

1)	shall use the appropriate keying information specified in subclause 6.6.2.2;

2)	shall perform the procedures in subclause 6.6.2.3.3 to confidentiality protect XML elements containing the content described in subclause 4.8, and

3)	shall perform the procedures in subclause 6.6.2.3.4 to confidentiality protect URIs in XML attributes for URIs described in subclause 4.8.

If the <confidentiality-protection> element in the Service Configuration document as specified in 3GPP TS 24.384 [50] is set to "false", then sending confidentiality protected content from the MCPTT server to the MCPTT client is disabled, and then content is included in XML elements and attributes without encryption.

If the <allow-signalling-protection> element of the <protection-between-mcptt-servers> element in the Service Configuration document as specified in 3GPP TS 24.384 [50] is set to "false", then sending confidentiality protected content between MCPTT servers is disabled, and content is included in XML elements and attributes without encryption.

##### 6.6.2.3.3	Content Encryption in XML elements

The following procedures shall be performed by an MCPTT client or an MCPTT server:

1)	perform encryption as specified in W3C: "XML Encryption Syntax and Processing Version 1.1", https://www.w3.org/TR/xmlenc-core1/ [60] subclause 4.3, using the "AES-128-GCM algorithm HMAC" as the encryption algorithm and the XPK as the key; and

2)	follow the semantic for the element of the MIME body as described in Annex F of the present document, to include the encrypted content in the MIME body ensuring that the necessary XML elements required for confidentiality protection are included as specified in 3GPP TS 33.179 [46].

##### 6.6.2.3.4	Attribute URI Encryption

The following procedures shall be performed by an MCPTT client or an MCPTT server:

1)	perform encryption as specified in [aes-gcm], using the "AES-128-GCM algorithm HMAC" as the encryption algorithm and the XPK as the key, with a 96 bit randomly selected IV; and

2)	replace the URI to be protected in the attribute by a URI constructed as follows:

a)	the URI schema is "sip: ";

b) the first part of the userinfo part is the base64 encoded result of the encryption of the original attribute value;

c)	the string ";iv=" is appended to the result of step b);

d)	the base64 encoding of the IV (section 5 of IETF RFC 4648 [rfc4648]) is appended to the result of step c);

e)	the string ";key-id=" is appended to the result of step d);

f)	the base64 encoding of the XPK-ID according to 3GPP 33.179 [46] is appended to the result of step e);

g)	the string ";alg=128-aes-gcm" is appended to the result of step f); and

h)	the string "@" followed by the domain name for MCPTT confidentiality protection as specified in 3GPP TS 23.203is appended to the result of step g).

#### 6.6.2.4	Procedures for receiving confidentiality protected content

##### 6.6.2.4.1	Determination of confidentiality protected content

The following procedure is used by the MCPTT client or MCPTT server to determine if an XML element is confidentiality protected.

1)	if an XML element contains the <EncryptedData> XML element, then the content of the XML element is confidentiality protected; and

2)	if an XML element does not contain the <EncryptedData> XML element, then the content of the XML element is.not confidentiality protected.

The following procedure is used by the MCPTT client or MCPTT server to determine if a URI in the XML attribute is confidentiality protected.

1)	if an XML attribute is a URI with the domain name for MCPTT confidentiality protection as specified in the 3GPP TS 23.003 [40], then the URI is confidentiality protected; and

2)	if an XML attribute is a URI without the domain name for MCPTT confidentiality protection as specified in the 3GPP TS 23.003 [40], then the URI is not confidentiality protected.

##### 6.6.2.4.2	Decrypting confidentiality protected content in XML elements

The following procedure shall be performed by an MCPTT client or an MCPTT server to decrypt an individual XML element that has a type of "encrypted" within an XML MIME body:

1)	if the <EncryptedData> XML element or any of its sub-elements as described in 3GPP TS 33.179 [46] are not present in the MIME body then send a SIP 403 (Forbidden) response with the warning text set to "140 unable to decrypt XML content" in a Warning header field as specified in subclause 4.4, and exit this procedure. Otherwise continue with the rest of the steps;

2)	perform decryption on the <EncryptedData> element as specified in W3C: "XML Encryption Syntax and Processing Version 1.1", https://www.w3.org/TR/xmlenc-core1/ [60] subclause 4.4 to decrypt the contents of the <CipherValue> element contained within the <CipherData> element;

3)	if the decryption procedure fails, then send a SIP 403 (Forbidden) response with the warning text set to "140 unable to decrypt XML content" in a Warning header field as specified in subclause 4.4. Otherwise continue with the rest of the steps; and

4)	return success of this procedure together with the decrypted XML element.

##### 6.6.2.4.3	Decrypting confidentiality protected URIs in XML attributes

The following procedure shall be performed by an MCPTT client or an MCPTT server to decrypt a URI in an attribute in a XML document:

1)	the value between ";iv=" and the next ";" provides the base64 encoded value of the 96 bit IV and the value between ";=key-id" and the next ";" defines the key which has been used for encryption, i.e. "CSK" or "SPK"; and

2)	the original URI is obtained by decrypting the base64 encoded string between the "sip: " URI prefix and the next ";" using the "AES-128-GCM algorithm HMAC" as the decryption algorithm with IV and key as determined in step 1). This value replaces the encrypted URI as the value of the XML attribute.

#### 6.6.2.5	MCPTT server copying received XML content

The following procedure is executed when an MCPTT server receives a SIP request containing XML MIME bodies, where the content needs to be copied from the incoming SIP request to the outgoing SIP request.

The MCPTT server:

1)	shall copy the XML elements from the XML MIME body of the incoming SIP request that do not contain a <EncryptedData> XML element, to the same XML body in the outgoing SIP request;

2)	for each encrypted XML element in the XML MIME body of the incoming SIP request as determined by subclause 6.6.2.4.1:

a)	shall use the keying information described in subclause 6.6.2.2 to decrypt the content within the XML element by following the procedures specified in subclause 6.6.2.4.2, and shall continue with the steps below if the encrypted XML element was successfully decrypted;

b)	if confidentiality protection is enabled as specified in subclause 6.6.2.3.2, then for each decrypted XML element:

i)	shall re-encrypt the content within the XML element using the keying information described in subclause 6.6.2.2 and by following the procedures specified in subclause 6.6.2.3.3; and

ii)	shall include the re-encrypted content into the same XML MIME body of the outgoing SIP request; and

c)	if confidentiality protection is disabled as specified in subclause 6.6.2.3.2, shall include the decrypted content in the same XML MIME body of the outgoing SIP request.

3)	for each encrypted XML URI attribute in the XML MIME body of the incoming SIP request as determined by subclause 6.6.2.4.1:

a)	shall use the keying information described in subclause 6.6.2.2 to decrypt the URI value of the XML attribute by following the procedures specified in subclause 6.6.2.4.3, and shall continue with the steps below if the encrypted XML attribute value was successfully decrypted;

b)	if confidentiality protection is enabled as specified in subclause 6.6.2.3.2, then for each decrypted XML element:

i)	shall re-encrypt the URI value of the XML attribute using the keying information described in subclause 6.6.2.2 and by following the procedures specified in subclause 6.6.2.3.4; and

ii)	shall include the re-encrypted attribute value into the same XML MIME body of the outgoing SIP request; and

c)	if confidentiality protection is disabled as specified in subclause 6.6.2.3.2, shall include the decrypted value in the same XML MIME body of the outgoing SIP request.

### 6.6.3	Integrity Protection of XML documents.

#### 6.6.3.1	General

Integrity protection can be applied to a whole XML MIME body. When integrity protection is enabled, all XML MIME bodies transported in SIP requests and responses are integrity protected. The following XML MIME bodies used in the present specification in SIP signalling can be integrity protected:

-	application/vnd.3gpp.mcptt-info+xml;

-	application/poc-settings+xml;

-	application/resources-list+xml;

-	application/vnd.3gpp.mcptt-mbms-usage-info+xml;

-	application/vnd.3gpp.mcptt-location-info+xml;

-	application/vnd.3gpp.mcptt-affiliation-command+xml;

-	application/vnd.3gpp.mcptt-floor-request+xml; and

-	application/conference-info+xml.

If integrity protection is enabled, and one or more of the XML MIME bodies complying to the types listed above are included in a SIP request or SIP response, then a MIME body of type application/vnd.3gpp.mcptt-signed+xml is included in the SIP request or SIP response containing one or more signatures pointing to those XML MIME bodies as illustrated in Figure 6.6.3.3-1.

In order to integrity protect the XML MIME bodies listed above in this subclause in SIP requests and SIP responses, the MCPTT client and MCPTT server shall for each MIME body, include the Content-ID header field as specified in IETF RFC 2045 [68] containing a Content-ID ("cid") Uniform Resource Locator (URL) as specified in IETF RFC 2392 [62].

Figure 6.6.3.1-1: Integrity Protection of XML MIME bodies in SIP requests and SIP responses

Each MIME body that is integrity protected is assigned a unique signature.

When integrity protecting the XML content in SIP REFER request used for pre-established sessions, the application/vnd.3gpp.mcptt-signed+xml MIME type can appear twice in the SIP REFER request as illustrated in Figure 6.6.3.1-2.

Figure 6.6.3.1-2: Integrity Protection of XML MIME bodies in SIP REFER requests

-	an application/vnd.3gpp.mcptt-signed+xml MIME body is included in the SIP REFER request with a signature pointing to the application/resource-lists MIME body; and

NOTE 1:	Sensitive XML content placed in the application/resource-lists MIME body can be encrypted.

-	an application/vnd.3gpp.mcptt-signed+xml MIME is included in the hname "body" URI header field of the URI in the "uri" attribute of the <entry> element of the application/resource-lists MIME body in the SIP REFER request, containing signatures pointing to the XML MIME bodies included in the "body" URI header field.

NOTE 2:	Sensitive XML content placed in the hname "body" URI header field can be encrypted.

Configuration for applying integrity protection is not selective to a specific MIME body. If configuration for integrity protection is turned on, then all XML MIME bodies in SIP requests and responses are integrity protected. If configuration for integrity protection is turned off, then no XML MIME bodies in SIP requests and SIP responses are integrity protected.

#### 6.6.3.2	Keys used in integrity protection procedures

Integrity protection uses an XPK to sign the data which (depending on who is the sender and who is the receiver of the signed information) can be a Client-Server Key (CSK) or Server Protection Key (SPK) as specified in subclause 4.8. An XPK-ID (CSK-ID/SPK-ID) is used to key the XPK (CSK/SPK). It is assumed that before the procedures in subclause 6.6.3.3 and subclause 6.6.3.4 are called, the CSK/CSK-ID and/or SPK/SPK-ID are available on the sender and recipient of the integrity protected content, as described in subclause 4.8.

The procedures in subclause 6.6.3.3 and subclause 6.6.3.4 shall be used with a XPK equal to the CSK and a XPK-ID equal to the CSK-ID in the following circumstances as described in 3GPP TS 33.179 [46]:

1)	MCPTT client sends integrity protected content to an MCPTT server; and

2)	MCPTT server sends integrity protected content to an MCPTT client.

The procedure in subclause 6.6.3.3 and subclause 6.6.3.4 shall be used with a XPK equal to the SPK and a XPK-ID equal to the SPK-ID in the following circumstances as described in 3GPP TS 33.179 [46]:

1)	MCPTT server sends integrity protected content to an MCPTT server in the same domain; and

2)	MCPTT server sends integrity protected content to an MCPTT server in another domain.

#### 6.6.3.3	Sending integrity protected content

##### 6.6.3.3.1	MCPTT client

If the <integrity-protection> element in the Service Configuration document as specified in 3GPP TS 24.384 [50] is set to "true" or no <integrity-protection> element is present in the Service Configuration document, then sending integrity protected content from the MCPTT client to the MCPTT server is enabled, and the MCPTT client shall use the appropriate keying information specified in subclause 6.6.3.2 and shall perform the procedures in subclause 6.6.3.3.3 to integrity protect XML MIME bodies.

NOTE:	Each XML MIME body is integrity protected separately.

If the <integrity-protection> element in the Service Configuration document as specified in 3GPP TS 24.384 [50] is set to "false", then sending integrity protected content from the MCPTT client to the MCPTT server is disabled, and all XML MIME bodies are sent without integrity protection.

##### 6.6.3.3.2	MCPTT server

If the <integrity-protection> element in the Service Configuration document as specified in 3GPP TS 24.384 [50] is set to "true", or no <integrity-protection> element is present in the Service Configuration document, then sending integrity protected content from the MCPTT server to the MCPTT client is enabled. If the <allow-signalling-protection> element of the <protection-between-mcptt-servers> element is set to "true" in the Service Configuration document as specified in 3GPP TS 24.384 [50] or no <allow-signalling-protection> element is present in the Service Configuration document, then sending integrity protected content between MCPTT servers is enabled.

When sending integrity protected content, the MCPTT server shall use the appropriate keying information specified in subclause 6.6.3.2 and shall perform the procedures in subclause 6.6.3.3.3 to integrity protect XML MIME bodies.

NOTE:	Each XML MIME body is integrity protected separately.

If the <integrity-protection> element in the Service Configuration document as specified in 3GPP TS 24.384 [50] is set to "false", then sending integrity protected content from the MCPTT server to the MCPTT client is disabled, and all XML MIME bodies are sent without integrity protection.

If the <allow-signalling-protection> element of the <protection-between-mcptt-servers> element in the Service Configuration document as specified in 3GPP TS 24.384 [50] is set to "false", then sending integrity protected content between MCPTT servers is disabled, and content is included in XML elements without encryption.

##### 6.6.3.3.3	Integrity protection procedure

The following procedure shall be performed by the MCPTT client and MCPTT server to integrity protect the XML bodies defined by the MIME types listed in subclause 6.6.3.1:

1)	include a Content-Type header field set to "application/vnd.3gpp.mcptt-signed+xml";

2)	for each of the MIME types defined in subclause 6.6.3.1 where the content defined by these MIME types is to be integrity protected:

a)	perform reference generation as specified in W3C: "XML Signature Syntax and Processing (Second Edition)", http://www.w3.org/TR/xmldsig-core [61] subclause 3.1.1 using the SHA256 algorithm to produce a hash of the MIME body and continue with the procedures below if reference generation is successful;

b)	perform signature generation as specified in W3C: "XML Signature Syntax and Processing (Second Edition)", http://www.w3.org/TR/xmldsig-core [61] subclause 3.1.2 using the HMAC-SHA256 signature method and the XPK as the key and continue with the procedures below if signature generation is successful; and

3)	follow the schema defined in Annex F.6.2 and the semantic described in Annex F.6.3 to create the application/vnd.3gpp.mcptt-signed+xml MIME body containing signatures referring to the XML MIME bodies included in the SIP request or SIP response.

#### 6.6.3.4	Receiving integrity protected content

##### 6.6.3.4.1	Determination of integrity protected content

The following procedure is used by the MCPTT client or MCPTT server to determine if an XML MIME body is integrity protected.

1)	if the <Signature> XML element is not present in the XML MIME body, then the content is not integrity protected; and

2)	if the <Signature> XML element is present in the XML MIME body, then the content is integrity protected.

##### 6.6.3.4.2	Verification of integrity protected content

The following procedure is used by the MCPTT client or MCPTT server to verify the integrity of an XML MIME body:

1)	if the required sub-elements of the <Signature> as described in 3GPP TS 33.179 [46] are not present in the MIME body and if not present, are not known to the sender and recipient by other means, then the integrity protection procedure fails and exit this procedure. Otherwise continue with the rest of the steps;

2)	perform reference validation on the <Reference> element as specified in W3C: "XML Signature Syntax and Processing (Second Edition)", http://www.w3.org/TR/xmldsig-core [61] subclause 3.2.1;

3)	if reference validation fails, then send a SIP 403 (Forbidden) response towards the functional entity with the warning text set to: "139 integrity protection check failed" in a Warning header field as specified in subclause 4.4, and do not continue with the rest of the steps in this subclause;

4)	obtain the XPK using the XPK-ID in the received XML body and use it to perform signature validation of the value of the <SignatureValue> element as specified in W3C: "XML Signature Syntax and Processing (Second Edition)", http://www.w3.org/TR/xmldsig-core [61] subclause 3.2.2;

5)	if signature validation fails, then send a SIP 403 (Forbidden) response towards the functional entity with the warning text set to: "139 integrity protection check failed" in a Warning header field as specified in subclause 4.4, and do not continue with the rest of the steps in this subclause; and

6)	return success of the integrity protection of the XML document passes the integrity protection procedure.

## 6.7	Priority sharing

The participating MCPTT function shall enable or disable priority sharing as specified in 3GPP TS 24.229 [4].

# 7	Registration and service authorisation

## 7.1	General

This clause describes the procedures for SIP registration and MCPTT service authorization for the MCPTT client and the MCPTT service. The MCPTT UE can use SIP REGISTER or SIP PUBLISH for MCPTT server settings to perform service authorization for MCPTT. The decision which method to use is based on implementation and on availability of an access-token received as outcome of the user authentication procedure as described in 3GPP TS 24.382 [49].

## 7.2	MCPTT client procedures

### 7.2.1	SIP REGISTER request for service authorisation

When the MCPTT client performs SIP registration the MCPTT client shall perform the registration procedures as specified in 3GPP TS 24.229 [4].

The MCPTT client shall include the following media feature tags in the Contact header field of the SIP REGISTER request:

1)	the g.3gpp.mcptt media feature tag; and

2)	the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt".

NOTE 1:	If the MCPTT client logs off from the MCPTT service but the MCPTT UE remains registered the MCPTT UE performs a re-registration as specified in 3GPP TS 24.229 [4] without both the g.3gpp.mcptt media feature tag and the g.3gpp.icsi-ref media feature tag in the Contact header field of the SIP REGISTER request.

If the MCPTT client supports MCPTT service continuity, then the MCPTT client shall follow the IMS registraton procedures for PS to PS service continuity as specified in subclause 6.2.2 of 3GPP TS 24.237 [58].

If the MCPTT client, upon performing SIP registration:

1)	has successfully finished the user authentication procedure as described in 3GPP TS 24.382 [49];

2)	has available an access-token;

3)	based on implementation decides to use SIP REGISTER for service authorization;

4)	confidentiality protection is disabled as specified in subclause 6.6.2.3.1; and

5)	integrity protection is disabled as specified in subclause 6.6.3.3.1;

then the MCPTT client shall include an application/vnd.3gpp.mcptt-info+xml MIME body as defined in Annex F.1 with the <mcptt-access-token> element set to the value of the access token received during the user authentication procedures, in the SIP REGISTER request.

NOTE 2:	the access-token contains the MCPTT ID of the user.

If the MCPTT client, upon performing SIP registration:

1)	has successfully finished the user authentication procedure as described in 3GPP TS 24.382 [49];

2)	has an available access-token;

3)	based on implementation decides to use SIP REGISTER for service authorization; and

4)	either confidentiality protection is enabled as specified in subclause 6.6.2.3.1 or integrity protection is enabled as specified in subclause 6.6.3.3.1;

then the MCPTT client:

1)	shall include an application/mikey MIME body with the CSK as MIKEY-SAKKE I_MESSAGE as specified in 3GPP TS 33.179 [46] in the body of the SIP REGISTER request;

2)	if confidentiality protection is enabled as specified in subclause 6.6.2.3.1, shall encrypt the received access-token using the client server key (CSK) and shall include in the body of the SIP REGISTER request, an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcptt-access-token> element set to the encrypted access-token, as specified in subclause 6.6.2.3.3;

3)	if confidentiality protection is disabled as specified in subclause 6.6.2.3.1, shall include an application/vnd.3gpp.mcptt-info+xml MIME body as defined in Annex F.1 with the <mcptt-access-token> element set to the value of the access token received during the user authentication procedures; and

4)	if integrity protection is enabled as specified in subclause 6.6.3.3.1, shall use the CSK to integrity protect the application/vnd.3gpp.mcptt-info+xml MIME body by following the procedures in subclause 6.6.3.3.3.

### 7.2.1A	Common SIP PUBLISH procedure

This procedure is only referenced from other procedures.

When populating the SIP PUBLISH request, the MCPTT client shall:

1)	shall set the Request-URI to the public service identity identifying the participating MCPTT function serving the MCPTT user;

2)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Preferred-Service header field according to IETF RFC 6050 [9];

3)	shall set the Event header field to the "poc-settings" value; and

4)	shall set the Expires header field according to IETF RFC 3903 [37], to 4294967295, if the MCPTT user is not removing the MCPTT service settings, otherwise to remove the MCPTT service settings the MCPTT client shall set the Expires header field to zero.

NOTE 1:	4294967295, which is equal to 232-1, is the highest value defined for Expires header field in IETF RFC 3261 [24].

NOTE 2:	The expiration timer of the MCPTT client service settings is only applicable for the MCPTT client service settings from this MCPTT client that matches the Instance Identifier URN. The expiration timer of MCPTT user service settings is also updated in the MCPTT server if expiration timer of MCPTT client service settings is updated in the MCPTT server.

NOTE 3:	Removing the MCPTT service settings by setting the Expires header field to zero, logs off the MCPTT client from the MCPTT service.

### 7.2.2	SIP PUBLISH request for service authorisation and MCPTT service settings

If based on implementation the MCPTT client decides to use SIP PUBLISH for MCPTT server settings to also perform service authorization and

1)	has successfully finished the user authentication procedure as described in 3GPP TS 24.382 [49]; and

2)	has available an access-token;

then the MCPTT client:

1)	shall perform the procedures in subclause 7.2.1A;

2)	if confidentiality protection is disabled as specified in subclause 6.6.2.3.1 and integrity protection is disabled, shall include in the body of the SIP PUBLISH request, an application/vnd.3gpp.mcptt-info+xml MIME body as specified in Annex F.1 with the <mcptt-access-token> element set to the value of the access token received during the user authentication procedures;

3)	if either confidentiality protection is enabled as specified in subclause 6.6.2.3.1 or integrity protection is enabled as specified in subclause 6.6.3.3.1 shall include an application/mikey MIME body with the CSK as MIKEY-SAKKE I_MESSAGE as specified in 3GPP TS 33.179 [46] in the body of the SIP PUBLISH request;

4)	if confidentiality protection is enabled as specified in subclause 6.6.2.3.1, shall encrypt the received access-token using the client server key (CSK) and shall include in the body of the SIP PUBLISH request, an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcptt-access-token> element set to the encrypted access-token, as specified in subclause 6.6.2.3.3;

5)	if confidentiality protection is disabled as specified in subclause 6.6.2.3.1, shall include in the body of the SIP PUBLISH request, an application/vnd.3gpp.mcptt-info+xml MIME body as specified in Annex F.1 with the <mcptt-access-token> element set to the value of the access token received during the user authentication procedures in the body of the SIP PUBLISH request;

6)	shall include an application/poc-settings+xml MIME body containing the Answer-Mode Indication setting in the <am-settings> element of the poc-settings event package set to the current answer mode setting ("auto-answer" or "manual-answer") of the MCPTT client according to IETF RFC 4354 [55]; and

7)	if integrity protection is enabled as specified in subclause 6.6.3.3.1, shall use the CSK to integrity protect the application/vnd.3gpp.mcptt-info+xml MIME body and application/poc-settings+xml MIME body by following the procedures in subclause 6.6.3.3.3.

The MCPTT client shall send the SIP PUBLISH request according to 3GPP TS 24.229 [4].

### 7.2.3	Sending SIP PUBLISH for MCPTT service settings only

To set, update, remove or refresh the MCPTT service settings, the MCPTT client shall generate a SIP PUBLISH request according 3GPP TS 24.229 [4], IETF RFC 3903 [37] and IETF RFC 4354 [55]. In the SIP PUBLISH request, the MCPTT client:

1)	shall perform the procedures in subclause 7.2.1A;

2)	if confidentiality protection is enabled as specified in subclause 6.6.2.3.1, shall encrypt the targeted MCPTT ID using the client server key (CSK) and shall include in the body of the SIP PUBLISH request, an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcptt-request-uri> element set to the encrypted targeted MCPTT ID, as specified in subclause 6.6.2.3.3;

3)	if confidentiality protection is disabled as specified in subclause 6.6.2.3.1, shall include an application/vnd.3gpp.mcptt-info+xml MIME body as specified in Annex F.1 with the <mcptt-request-uri> set to the cleartext targeted MCPTT ID;

4)	shall include an application/poc-settings+xml MIME body containing the Answer-Mode Indication setting in the <am-settings> element of the poc-settings event package set to the current answer mode setting ("auto-answer" or "manual-answer") of the MCPTT client according to IETF RFC 4354 [55]; and

5)	if integrity protection is enabled as specified in subclause 6.6.3.3.1, shall use the CSK to integrity protect the application/vnd.3gpp.mcptt-info+xml MIME body and application/poc-settings+xml MIME body by following the procedures in subclause 6.6.3.3.3.

The MCPTT client shall send the SIP PUBLISH request according to 3GPP TS 24.229 [4].

On receiving the SIP 200 (OK) response to the SIP PUBLISH request the MCPTT client may indicate to the MCPTT User the successful communication of the MCPTT service settings to the MCPTT server.

## 7.3	MCPTT server procedures

### 7.3.1	General

The MCPTT server obtains information that it needs to implement service authorization specific requirements from:

a)	any received third-party SIP REGISTER request (e.g. including information contained in the body of the third-party SIP REGISTER request) as specified in 3GPP TS 24.229 [4]. The body will carry the SIP REGISTER request as sent by the MCPTT client, containing information needed for service authorization; or

b)	any received SIP PUBLISH request for MCPTT server settings containing the g.3gpp.mcptt media feature tag along with the "require" and "explicit" header field parameters. The body of the SIP PUBLISH request will contain information needed for service authorization.

### 7.3.1A	Confidentiality and Integrity Protection

When the MCPTT server receives a SIP REGISTER request sent from the MCPTT client contained within a message/sip MIME body of a received third-party SIP REGISTER request or a SIP PUBLISH request, it first determines whether XML MIME bodies included in the request are integrity protected. If XML MIME bodies are integrity protected the MCPTT server validates the signature of each of the XML MIME bodies. If the integrity protection check(s) pass or the XML MIME bodies are not integrity protected, the MCPTT server then determines whether the content in specific XML elements is confidentiality protected. If XML content is confidentiality protected, the MCPTT server decrypts the protected content.

Upon receiving:

-	a SIP REGISTER request containing an application/vnd.3gpp.mcptt-info+xml MIME body within a message/sip MIME body of the SIP REGISTER request sent from the MCPTT client; or

-	a SIP PUBLISH request containing an application/vnd.3gpp.mcptt-info+xml MIME body and an application/poc-settings+xml MIME body;

the MCPTT server:

1)	shall determine if integrity protection has been applied to XML MIME bodies in the SIP request by following the procedures in subclause 6.6.3.4.1 for each XML MIME body;

2)	if integrity protection has been applied, shall use the keying data described in subclause 6.6.3.2 and the procedures described in subclause 6.6.3.4.2 to verify the integrity of each of the XML MIME bodies; and

3)	if all integrity protection checks succeed, shall continue with the remaining steps of this subclause.

Upon receiving:

-	a SIP REGISTER request containing an application/vnd.3gpp.mcptt-info+xml MIME body with an <mcptt-access-token> element within a message/sip MIME body of the SIP REGISTER request sent from the MCPTT client; or

-	a SIP PUBLISH request containing an application/vnd.3gpp.mcptt-info+xml MIME body with an <mcptt-access-token> element, and an application/poc-settings+xml MIME body;

the MCPTT server:

1)	shall determine if confidentiality protection has been applied to the <mcptt-access-token> element in the application/vnd.3gpp.mcptt-info+xml MIME body, by following the procedures in subclause 6.6.2.4.1;

2)	if confidentiality protection has been applied:

a)	shall use the keying information received in the MIKEY-SAKKE I_MESSAGE as specified in 3GPP TS 33.179 [46], along with the procedures described in subclause 6.6.2.4.2 to decrypt the received access token in the <mcptt-access-token> element in the application/vnd.3gpp.mcptt-info+xml MIME body; and

b)	if the decryption procedure succeeds, shall identify the MCPTT ID from the decrypted value and continue with the remaining steps of this subclause; and

3)	if confidentiality protection has not been applied, shall identify the MCPTT ID from <mcptt-access-token> element received in the application/vnd.3gpp.mcptt-info+xml MIME body.

Upon receiving a SIP PUBLISH request containing an application/vnd.3gpp.mcptt-info+xml MIME body with an <mcptt-request-uri> element, and an application/poc-settings+xml MIME body, the MCPTT server:

1)	shall determine if confidentiality protection has been applied to the <mcptt-request-uri> element in the application/vnd.3gpp.mcptt-info+xml MIME body by following the procedures in subclause 6.6.2.4.1;

2)	if confidentiality protection has been applied:

a)	shall use the keying information described in subclause 6.6.2.2 along with the procedures described in subclause 6.6.2.4.2 to decrypt the received encrypted MCPTT ID in the <mcptt-request-uri> element; and

b)	if all decryption procedures succeed, shall identify the data from the decrypted value and continue with the remaining steps of this subclause; and

3)	if confidentiality protection has not been applied, shall identify the MCPTT ID from the contents of the <mcptt-request-uri> element in the application/vnd.3gpp.mcptt-info+xml MIME body.

### 7.3.2	SIP REGISTER request for service authorisation

The MCPTT server shall support obtaining service authorization specific information from the SIP REGISTER request sent from the MCPTT client and included in the body of a third-party SIP REGISTER request.

NOTE 1:	3GPP TS 24.229 [4] defines how based on initial filter criteria the SIP REGISTER request sent from the UE is included in the body of the third-party SIP REGISTER request.

Upon receiving a third party SIP REGISTER request with a message/sip MIME body containing the SIP REGISTER request sent from the MCPTT client, the MCPTT server:

1)	shall identify the IMS public user identity from the third-party SIP REGISTER request;

2)	shall identify the MCPTT ID from the SIP REGISTER request sent from the MCPTT client and included in the message/sip MIME body of the third-party SIP REGISTER request by following the procedures in subclause 7.3.1A;

3)	shall perform service authorization for the identified MCPTT ID as described in 3GPP TS 33.179 [46];

4)	if service authorization was successful, shall bind the MCPTT ID to the IMS public user identity; and

NOTE 2:	The MCPTT server will store the binding MCPTT ID, IMS public user identity and an identifier addressing the MCPTT server in an external database.

5)	if service authorization was not successful, shall send a SIP 403 (Forbidden) response towards the MCPTT server with the warning text set to: "101 service authorisation failed" in a Warning header field as specified in subclause 4.4.

### 7.3.3	SIP PUBLISH request for service authorisation and service settings

The MCPTT server shall support obtaining service authorization specific information from a SIP PUBLISH request for MCPTT server settings.

Upon receiving a SIP PUBLISH request containing:

1)	an Event header field set to the "poc-settings" value;

2)	an application/poc-settings+xml MIME body; and

3)	an application/vnd.3gpp.mcptt-info+xml MIME body containing an <mcptt-access-token> element;

the MCPTT server:

1)	shall identify the IMS public user identity from the P-Asserted-Identity header field;

2)	shall perform the procedures in subclause 7.3.1A;

3)	shall perform service authorization for the identified MCPTT ID as described in 3GPP TS 33.179 [46];

4)	if service authorization was successful, shall bind the MCPTT ID to the IMS public user identity;

NOTE:	The MCPTT server will store the binding MCPTT ID, IMS public user identity and an identifier addressing the MCPTT server in an external database.

5)	if service authorization was not successful, shall send a SIP 403 (Forbidden) response towards the MCPTT server with the warning text set to: "101 service authorisation failed" in a Warning header field as specified in subclause 4.4, and not continue with the rest of the steps in this subclause;

6)	shall process the SIP PUBLISH request according to rules and procedures of IETF RFC 3903 [37] and if processing of the SIP request was not successful, do not continue with the rest of the steps;

7)	shall cache the received MCPTT service settings until the MCPTT service settings expiration timer expires; and

8)	shall send a SIP 200 (OK) response according 3GPP TS 24.229 [4];

9)	shall use the Answer-Mode Indication setting in the <am-settings> element of the poc-settings event package as the current Answer-Mode Indication of the MCPTT client; and

10)	shall download the MCPTT user profile from the MCPTT user database as defined in 3GPP TS 29.283 [73] if not already stored at the MCPTT server.

### 7.3.4	Receiving SIP PUBLISH request for MCPTT service settings only

Upon receiving a SIP PUBLISH request containing:

1)	an Event header field set to the "poc-settings" value;

2)	an application/poc-settings+xml MIME body; and

3)	an application/vnd.3gpp.mcptt-info+xml MIME body containing an <mcptt-request-uri> element;

The MCPTT server:

1)	shall identify the IMS public user identity from the P-Asserted-Identity header field;

2)	shall perform the procedures in subclause 7.3.1A;

3)	shall verify that a binding between the IMS public user identity in the Request-URI and the MCPTT ID in the <mcptt-request-uri> element of the application/vnd.3gpp.mcptt-info+xml exists at the MCPTT server;

4)	if a binding does not exist between the IMS public user identity and the MCPTT ID in the request or the binding exists, but the validity period of the binding has expired, shall reject the SIP PUBLISH request with a SIP 404 (Not Found) response and not continue with any of the remaining steps;

5)	shall process the SIP PUBLISH request according to rules and procedures of IETF RFC 3903 [37] and if processing of the SIP request was not successful, do not continue with the rest of the steps;

6)	shall cache the received MCPTT service settings until the MCPTT service settings expiration timer expires;

7)	shall send a SIP 200 (OK) response according 3GPP TS 24.229 [4];

8)	shall use the Answer-Mode Indication setting in the <am-settings> element of the poc-settings event package as the current Answer-Mode Indication of the MCPTT client; and

9)	shall download the MCPTT user profile from the MCPTT user database as defined in 3GPP TS 29.283 [73] if not already stored at the MCPTT server.

### 7.3.5	Receiving SIP PUBLISH request with "Expires=0"

Upon receiving a SIP PUBLISH request containing:

1)	an Event header field set to the "poc-settings" value; and

2)	an Expires header field set to 0;

the MCPTT server:

1)	shall identify the IMS public user identity from the P-Asserted-Identity header field;

2)	shall process the SIP PUBLISH request according to rules and procedures of IETF RFC 3903 [37] and if processing of the SIP request was successful, continue with the rest of the steps;

3)	shall remove the MCPTT service settings;

4)	shall remove the binding between the MCPTT ID and public user identity; and

5)	shall send a SIP 200 (OK) response according to 3GPP TS 24.229 [4].

# 8	Pre-established session

## 8.1	General

The MCPTT client may establish one or more pre-established sessions to the participating MCPTT function at any time after SIP registration and setting the service settings as defined in subclause 7.2.2 or subclause 7.2.3.

The MCPTT client may use the pre-established session for originating MCPTT calls after pre-established session establishment.

The participating MCPTT function may use the pre-established session for terminating MCPTT calls after pre-established session establishment.

The MCPTT client may initiate the modification of the media parameters of a pre-established session as defined in subclause 8.3.1.1.

The participating MCPTT function may initiate the modification of the media parameters of a pre-established session as defined in subclause 8.3.2.2.

The MCPTT client may initiate the release of a pre-established session as defined in subclause 8.4.1.1.

The participating MCPTT function may initiate the release of a pre-established session as defined in subclause 8.4.2.2.

## 8.2	Session establishment

### 8.2.1	MCPTT client procedures

When the MCPTT client initiates a pre-established session the MCPTT client shall:

1)	gather ICE candidates according to IETF RFC 5245 [17]; and

NOTE 1:	ICE candidates are only gathered on interfaces that the MCPTT UE uses to obtain MCPTT service.

2)	generate an initial SIP INVITE request by following the UE originating session procedures specified in 3GPP TS 24.229 [4], with the clarifications given below.

The MCPTT client:

1)	shall set the Request-URI of the SIP INVITE request to the public service identity of the participating MCPTT function serving the MCPTT user;

2)	may include a P-Preferred-Identity header field in the SIP INVITE request containing a public user identity as specified in 3GPP TS 24.229 [4];

3)	shall include the g.3gpp.mcptt media feature tag in the Contact header field of the SIP INVITE request according to IETF RFC 3840 [16];

4)	shall include an Accept-Contact header field with the media feature tag g.3gpp.mcptt along with parameters "require" and "explicit" according to IETF RFC 3841 [6];

5)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Preferred-Service header field according to IETF RFC 6050 [9] in the SIP INVITE request;

6)	shall include an Accept-Contact header field with the media feature tag g.3gpp.icsi-ref set to the value "urn:urn-7:3gpp-service.ims.icsi.mcptt" along with parameters "require" and "explicit" according to IETF RFC 3841 [6];

7)	shall include the "timer" option tag in the Supported header field;

8)	should include the Session-Expires header field according to IETF RFC 4028 [7] and should not include the "refresher" header field. The "refresher" header field parameter shall be set to "uac" if included;

9)	shall include an SDP offer according to 3GPP TS 24.229 [4] with the clarifications given in subclause 6.2.1, and include ICE candidates in the SDP offer as per IETF RFC 5245 [17]; and

10)	shall send the SIP INVITE request according to 3GPP TS 24.229 [4].

Upon receiving a SIP 2xx response to the SIP INVITE request the MCPTT client:

1)	shall interact with the media plane as specified in 3GPP TS 24.380 [5].

NOTE 2:	If ICE candidate evaluation results in candidate pairs other than the default candidate pair being selected a further offer answer exchange using the procedures in subclause 8.3 will be needed.

### 8.2.2	Participating MCPTT function procedures

Upon receipt of a "SIP INVITE request for establishing a pre-established session" the participating MCPTT function:

1)	shall check whether the public service identity is allocated and perform the actions specified in subclause 6.3.7.1 if it is not allocated. Otherwise, continue with the rest of the steps;

2)	shall determine the MCPTT ID of the MCPTT user establishing the pre-established session and perform actions to verify the MCPTT ID of the MCPTT client and authorise the request according to local policy, and if not authorised, the participating MCPTT function shall return a SIP 403 (Forbidden) response with the warning text set to "100 function not allowed due to <detailed reason>" as specified in subclause 4.4. Otherwise, continue with the rest of the steps;

3)	shall validate the media parameters and if the MCPTT speech codec is not offered in the SIP INVITE request shall reject the request with a SIP 488 (Not Acceptable Here) response. Otherwise, continue with the rest of the steps;

4)	shall verify that the media resources are available to support the media parameters and if not shall reject the request with a SIP 500 (Server Internal Error) response, and shall not continue with the rest of the steps;

5)	shall allocate a URI to be used to identify the pre-established session;

6)	shall generate a SIP 200 (OK) response to the SIP INVITE request according to subclause 6.3.2.1.5.2; and

a)	shall include a Contact header field containing the URI that identifies the pre-established session;

b)	shall include the public service identity in the P-Asserted-Identity header field;

c)	shall include a Supported header field containing the "norefersub" option tag; and

d)	shall include an SDP answer as specified in 3GPP TS 24.229 [4] with the clarifications in subclause 6.3.2.1.2.2 and include ICE candidates in the SDP answer as per IETF RFC 5245 [17];

7)	shall interact with the media plane as specified in 3GPP TS 24.380 [5];

NOTE 1:	Resulting media plane processing is completed before the next step is performed.

8)	shall send the SIP 200 (OK) response towards the MCPTT client according to the rules and procedures of the 3GPP TS 24.229 [4]; and

9)	shall evaluate the ICE candidates according to IETF RFC 5245 [17].

NOTE 2:	If ICE candidate evaluation results in candidate pairs other then the default candidate pair being selected a further offer answer exchange using the procedures in subclause 8.3 will be needed.

## 8.3	Session modification

### 8.3.1	MCPTT client procedures

#### 8.3.1.1	MCPTT client initiated

When the MCPTT client needs to modify the pre-established session outside of an MCPTT session, the MCPTT client:

1)	shall generate a SIP UPDATE request or a SIP re-INVITE request according to 3GPP TS 24.229 [4];

2)	shall include an SDP offer according to 3GPP TS 24.229 [4] with the clarifications given in subclause 6.2.1, and include ICE candidates in the SDP offer as per IETF RFC 5245 [17], if required; and

3)	shall send the SIP request towards the MCPTT server according to rules and procedures of 3GPP TS 24.229[4].

On receipt of the SIP 200 (OK) response the MCPTT client:

1)	shall interact with media plane as specified in 3GPP TS 24.380 [5], if there is change in media parameters or codecs in the received SDP answer, compared to those in the previously agreed SDP; and

2)	shall interact with media plane as specified in 3GPP TS 24.380 [5], if there is a media stream, that is currently used in the pre-established session, marked as rejected in the received SDP answer.

NOTE:	The MCPTT client keeps resources for previously agreed media stream, media parameters and codecs until it receives a SIP 200 (OK) response.

#### 8.3.1.2	Participating MCPTT function initiated

Upon receiving a SIP UPDATE request or a SIP re-INVITE request to modify an existing pre-established session without associated MCPTT session, the MCPTT client:

1)	shall validate that the received SDP offer includes at least one media stream for which the media parameters and at least one codec is acceptable by the MCPTT client and if not reject the request with a SIP 488 (Not Acceptable Here) response. Otherwise, continue with the rest of the steps; and

2)	shall generate a SIP 200 (OK) response as follows:

a)	shall include an SDP answer according to 3GPP TS 24.229 [4] with the clarifications given in subclause 6.2.2, and include ICE candidates in the SDP answer as per IETF RFC 5245 [17]. if required.

### 8.3.2	Participating MCPTT function procedures

#### 8.3.2.1	MCPTT client initiated

Upon receiving a SIP UPDATE request or a SIP re-INVITE request to modify an existing pre-established session without associated MCPTT session, the participating MCPTT function:

1)	shall validate that the received SDP offer includes at least one media stream for which the media parameters and at least one codec is acceptable by the participating MCPTT function and if not reject the request with a SIP 488 (Not Acceptable Here) response. Otherwise, continue with the rest of the steps; and

2)	shall generate a SIP 200 (OK) response as follows:

a)	include an SDP answer according to 3GPP TS 24.229 [4] based on the received SDP offer with the clarifications given in the 6.3.2.1.2.2, and include ICE candidates in the SDP answer as per IETF RFC 5245 [17], if required; and

b)	include a Contact header field containing the URI that identifies the pre-established session and send a SIP 200 (OK) response according to rules and procedures of 3GPP TS 24.229[4].

#### 8.3.2.2	Participating MCPTT function initiated

When the participating MCPTT function needs to modify the pre-established session outside of an MCPTT session, the participating MCPTT function:

1)	shall generate a SIP UPDATE request or a SIP re-INVITE request according to 3GPP TS 24.229 [4];

2)	shall include an SDP offer according to 3GPP TS 24.229 [4], and include ICE candidates in the SDP offer as per IETF RFC 5245 [17], if required;

3)	shall interact with the media plane as specified in 3GPP TS 24.380 [5], if removing a media-floor control entity; and

4)	shall send the SIP request towards the MCPTT client according to rules and procedures of 3GPP TS 24.229[4].

On receipt of the SIP 200 (OK) response the participating MCPTT function:

1)	shall interact with media plane as specified in 3GPP TS 24.380 [5], if there is change in media parameters or codecs in the received SDP answer, compared to those in the previously agreed SDP;

2)	shall interact with media plane as specified in 3GPP TS 24.380 [5], if there is a media stream, that is currently used in the pre-established session, marked as rejected in the received SDP answer; and

3)	shall interact with media plane as specified in 3GPP TS 24.380 [5], if there is a media stream accepted in the received SDP answer, that is not currently used by the participant in the pre-established session.

NOTE:	The participating MCPTT function keeps resources for previously agreed media stream, media-floor control entities, media parameters and codecs until it receives a SIP 200 (OK) response.

## 8.4	Session release

### 8.4.1	MCPTT client procedures

#### 8.4.1.1	MCPTT client initiated

NOTE:	The MCPTT client needs to be prepared to release the pre-established session when receiving a SIP BYE request generated by the SIP core (e.g. due to network release of media plane resources).

When a MCPTT client needs to release a pre-established session as created in subclause 8.2.1, the MCPTT client:

1)	shall generate a SIP BYE request according to rules and procedures of 3GPP TS 24.229 [4];

2)	shall set the Request-URI of the SIP BYE request to the URI that identifies the pre-established session;

3)	shall send the SIP BYE request towards the participating MCPTT function within the SIP dialog of the pre-established session according to rules and procedures of the 3GPP TS 24.229 [4]; and

4)	shall, upon receiving a SIP 200 (OK) response to the SIP BYE request interact with the media plane as specified in 3GPP TS 24.380 [5].

#### 8.4.1.2	Participating MCPTT function initiated

Upon receiving a SIP BYE request from the participating MCPTT function within a pre-established session the MCPTT client shall check whether there are any MCPTT sessions using the pre-established session, and:

1)	if there is an established MCPTT session then the MCPTT client shall remove the MCPTT client from the MCPTT session by performing the procedures for session release for each MCPTT session as specified in 3GPP TS 24.380 [5]; and

2)	if there is no MCPTT session using the pre-established session, then the MCPTT client shall:

a)	interact with the media plane as specified in 3GPP TS 24.380 [5] for disconnecting the media plane resources towards the participating MCPTT function; and

b)	shall generate and send a SIP 200 (OK) response to the SIP BYE request according to rules and procedures of 3GPP TS 24.229 [4].

### 8.4.2	Participating MCPTT function procedures

#### 8.4.2.1	MCPTT client initiated

Upon receiving a SIP BYE request from the MCPTT client within a pre-established session the participating MCPTT function:

1)	shall check whether there is a MCPTT session using the pre-established session, and:

a)	if there is an established MCPTT session then the participating MCPTT function shall remove the MCPTT client from the MCPTT session by performing the procedures as specified in subclause 6.3.2.1.6; and

b)	if there is a MCPTT session in the process of being established, then the participating MCPTT function:

i)	shall send a SIP CANCEL request to cancel the MCPTT session in the process of being established as specified in 3GPP TS 24.229 [4]; and

ii)	shall release the MCPTT session as specified in the subclause subclause 6.3.2.1.6, if a SIP 200 (OK) response for the SIP INVITE request is received from the remote side; and

c)	if there is no MCPTT session using the pre-established session, then the participating MCPTT function shall:

i)_	interact with the media plane as specified in 3GPP TS 24.380 [5] for disconnecting the media plane resources towards the MCPTT client; and

ii)	shall generate and send a SIP 200 (OK) response to the SIP BYE request according to rules and procedures of 3GPP TS 24.229 [4].

Upon receiving a SIP 200 (OK) response to the SIP BYE request from the remote side, the participating MCPTT function:

1)	shall interact with the media plane as specified in 3GPP TS 24.380 [5] for releasing media plane resources towards the remote side;

2)	shall interact with the media plane as specified in 3GPP TS 24.380 [5] for releasing media plane resources towards the MCPTT client; and

3)	shall send a SIP 200 (OK) response to the SIP BYE request to the MCPTT client.

#### 8.4.2.2	Participating MCPTT function initiated

When a participating MCPTT function needs to release a pre-established session as created in subclause 8.2.2, the participating MCPTT function:

1)	shall first release any participants of all MCPTT calls that are using the pre-established session using the procedures as follows. The participating MCPTT function:

a)	shall interact with the media plane as specified in subclause 6.4 in 3GPP TS 24.380 [5];

b)	shall generate a SIP BYE request as specified in 3GPP TS 24.229 [4];

c)	shall set the Request-URI to the MCPTT session identity;

d)	shall set the contents of the P-Asserted-Identity header field to the P-Asserted-Identity header field of the MCPTT client that"s pre-established session is being release;

e)	shall send the SIP BYE request toward the controlling MCPTT function, according to 3GPP TS 24.229 [4]; and

f)	shall, upon receiving a SIP 200 (OK) response to the SIP BYE request the terminating MCPTT function shall interact with the media plane as specified in subclause 6.4 in 3GPP TS 24.380 [5];

2)	shall generate a SIP BYE request according to rules and procedures of 3GPP TS 24.229 [4];

3)	shall set the Request-URI of the SIP BYE request to the URI that identifies the pre-established session;

4)	shall send the SIP BYE request towards the MCPTT client within the SIP dialog of the pre-established session according to rules and procedures of the 3GPP TS 24.229 [4]; and

5)	shall, upon receiving a SIP 200 (OK) response to the SIP BYE request interact with the media plane as specified in 3GPP TS 24.380 [5].

# 9	Affiliation

## 9.1	General

The affiliation description consists of procedures and coding.

## 9.2	Procedures

### 9.2.1	MCPTT client procedures

#### 9.2.1.1	General

The MCPTT client procedures consist of:

-	an affiliation status change procedure;

-	an affiliation status determination procedure;

-	a procedure for sending affiliation status change request in negotiated mode to target MCPTT user; and

-	a procedure for receiving affiliation status change request in negotiated mode from authorized MCPTT user.

In order to obtain information about success or rejection of changes triggered by the affiliation status change procedure for an MCPTT user, the MCPTT client needs to initiate the affiliation status determination procedure for the MCPTT user before starting the affiliation status change procedure for the MCPTT user.

#### 9.2.1.2	Affiliation status change procedure

In order:

-	to indicate that an MCPTT user is interested in one or more MCPTT group(s) at an MCPTT client;

-	to indicate that the MCPTT user is no longer interested in one or more MCPTT group(s) at the MCPTT client;

-	to refresh indication of an MCPTT user interest in one or more MCPTT group(s) at an MCPTT client due to near expiration of the expiration time of an MCPTT group with the affiliation status set to the "affiliated" state received in a SIP NOTIFY request in subclause 9.2.1.3;

-	to send an affiliation status change request in mandatory mode to another MCPTT user; or

-	any combination of the above;

the MCPTT client shall generate a SIP PUBLISH request according to 3GPP TS 24.229 [4], IETF RFC 3903 [37], and IETF RFC 3856 [51].

In the SIP PUBLISH request, the MCPTT client:

1)	shall set the Request-URI to the public service identity identifying the originating participating MCPTT function serving the MCPTT user;

2)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body. In the application/vnd.3gpp.mcptt-info+xml MIME body, the MCPTT client shall include the <mcptt-request-uri> element set to the MCPTT ID of the MCPTT user;

3)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Preferred-Service header field according to IETF RFC 6050 [9];

4)	if the targeted MCPTT user is interested in at least one MCPTT group at the targeted MCPTT client, shall set the Expires header field according to IETF RFC 3903 [37], to 4294967295;

NOTE 1:	4294967295, which is equal to 232-1, is the highest value defined for Expires header field in IETF RFC 3261 [24].

5)	if the targeted MCPTT user is no longer interested in any MCPTT group at the targeted MCPTT client, shall set the Expires header field according to IETF RFC 3903 [37], to zero; and

6)	shall include an application/pidf+xml MIME body indicating per-user affiliation information according to subclause 9.3.1. In the MIME body, the MCPTT client:

a)	shall include all MCPTT groups where the targeted MCPTT user indicates its interest at the targeted MCPTT client;

b)	shall include the MCPTT client ID of the targeted MCPTT client;

c)	shall not include the "status" attribute and the "expires" attribute in the <affiliation> element; and

d)	shall set the <p-id> child element of the <presence> root element to a globally unique value.

The MCPTT client shall send the SIP PUBLISH request according to 3GPP TS 24.229 [4].

#### 9.2.1.3	Affiliation status determination procedure

NOTE 1:	The MCPTT UE also uses this procedure to determine which MCPTT groups the MCPTT user successfully affiliated to.

In order to discover MCPTT groups:

1)	which the MCPTT user at an MCPTT client is affiliated to; or

2)	which another MCPTT user is affiliated to;

the MCPTT client shall generate an initial SIP SUBSCRIBE request according to 3GPP TS 24.229 [4], IETF RFC 3856 [51], and IETF RFC 6665 [26].

In the SIP SUBSCRIBE request, the MCPTT client:

1)	shall set the Request-URI to the public service identity identifying the originating participating MCPTT function serving the MCPTT user;

2)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body. In the application/vnd.3gpp.mcptt-info+xml MIME body, the MCPTT client shall include the <mcptt-request-uri> element set to the MCPTT ID of the targeted MCPTT user;

3)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Preferred-Service header field according to IETF RFC 6050 [9];

4)	if the MCPTT client wants to receive the current status and later notification, shall set the Expires header field according to IETF RFC 6665 [26], to 4294967295;

NOTE 2:	4294967295, which is equal to 232-1, is the highest value defined for Expires header field in IETF RFC 3261 [24].

5)	if the MCPTT client wants to fetch the current state only, shall set the Expires header field according to IETF RFC 6665 [26], to zero; and

6)	shall include an Accept header field containing the application/pidf+xml MIME type; and

7)	if requesting MCPTT groups where the MCPTT user is affiliated to at the MCPTT client, shall include an application/simple-filter+xml MIME body indicating per client restrictions of presence event package notification information according to subclause 9.3.2.

In order to re-subscribe or de-subscribe, the MCPTT client shall generate an in-dialog SIP SUBSCRIBE request according to 3GPP TS 24.229 [4], IETF RFC 3856 [51], and IETF RFC 6665 [26]. In the SIP SUBSCRIBE request, the MCPTT client:

1)	if the MCPTT client wants to receive the current status and later notification, shall set the Expires header field according to IETF RFC 6665 [26], to 4294967295;

NOTE 3:	4294967295, which is equal to 232-1, is the highest value defined for Expires header field in IETF RFC 3261 [24].

2)	if the MCPTT client wants to de-subscribe, shall set the Expires header field according to IETF RFC 6665 [26], to zero; and

3)	shall include an Accept header field containing the application/pidf+xml MIME type.

Upon receiving a SIP NOTIFY request according to 3GPP TS 24.229 [4], IETF RFC 3856 [51], and IETF RFC 6665 [26], if SIP NOTIFY request contains an application/pidf+xml MIME body indicating per-user affiliation information constructed according to subclause 9.3.1, then the MCPTT client shall determine affiliation status of the MCPTT user for each MCPTT group at the MCPTT client(s) in the MIME body. If the <p-id> child element of the <presence> root element of the application/pidf+xml MIME body of the SIP NOTIFY request is included, the <p-id> element value indicates the SIP PUBLISH request which triggered sending of the SIP NOTIFY request.

#### 9.2.1.4	Procedure for sending affiliation status change request in negotiated mode to target MCPTT user

NOTE:	Procedure for sending affiliation status change request in negotiated mode to several target MCPTT users is not supported in this version of the specification.

Upon receiving a request from the MCPTT user to send an affiliation status change request in negotiated mode to a target MCPTT user, the MCPTT client shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [4] and IETF RFC 3428 [33]. In the SIP MESSAGE request, the MCPTT client:

1)	shall set the Request-URI to the public service identity identifying the originating participating MCPTT function serving the MCPTT user;

2)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body. In the application/vnd.3gpp.mcptt-info+xml MIME body, the MCPTT client shall include the <mcptt-request-uri> element set to the MCPTT ID of the target MCPTT user;

3)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Preferred-Service header field according to IETF RFC 6050 [9] in the SIP MESSAGE request;

4)	shall include an application/vnd.3gpp.mcptt-affiliation-command+xml MIME body as specified in Annex F.4; and

5)	shall send the SIP MESSAGE request according to rules and procedures of 3GPP TS 24.229 [4].

On receiving a SIP 2xx response to the SIP MESSAGE request, the MCPTT client shall indicate to the user that the request has been delivered to an MCPTT client of the target MCPTT user.

#### 9.2.1.5	Procedure for receiving affiliation status change request in negotiated mode from authorized MCPTT user

Upon receiving a SIP MESSAGE request containing:

1)	the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Asserted-Service header field according to IETF RFC 6050 [9]; and

2)	an application/vnd.3gpp.mcptt-affiliation-command+xml MIME body with a list of MCPTT groups for affiliation under the <affiliate> element and a list of MCPTT groups for de-affiliation under the <de-affiliate> element;

then the MCPTT client:

1)	shall send a 200 (OK) response to the SIP MESSAGE request;

2)	shall seek confirmation of the list of MCPTT groups for affiliation and the list of MCPTT groups for de-affiliation, resulting in an accepted list of MCPTT groups for affiliation and an accepted list of MCPTT groups for de-affiliation; and

3)	if the user accepts the request:

a)	shall perform affiliation for each entry in the accepted list of MCPTT groups for affiliation for which the MCPTT client is not affiliated, as specified in subclause 9.2.1.2; and

b)	shall perform de-affiliation for each entry in the accepted list of MCPTT groups for de-affiliation for which the MCPTT client is affiliated, as specified in subclause 9.2.1.2.

### 9.2.2	MCPTT server procedures

#### 9.2.2.1	General

The MCPTT server procedures consist of:

-	procedures of MCPTT server serving the MCPTT user; and

-	procedures of MCPTT server owning the MCPTT group.

#### 9.2.2.2	Procedures of MCPTT server serving the MCPTT user

##### 9.2.2.2.1	General

The procedures of MCPTT server serving the MCPTT user consist of:

-	a receiving affiliation status change from MCPTT client procedure;

-	a receiving subscription to affiliation status procedure;

-	a sending notification of change of affiliation status procedure;

-	a sending affiliation status change towards MCPTT server owning MCPTT group procedure;

-	an affiliation status determination from MCPTT server owning MCPTT group procedure;

-	a procedure for authorizing affiliation status change request in negotiated mode sent to served MCPTT user;

-	a forwarding affiliation status change towards another MCPTT user procedure; and

-	a forwarding subscription to affiliation status towards another MCPTT user procedure.

##### 9.2.2.2.2	Stored information

The MCPTT server shall maintain a list of MCPTT user information entries. The list of the MCPTT user information entries contains one MCPTT user information entry for each served MCPTT ID.

In each MCPTT user information entry, the MCPTT server shall maintain:

1)	an MCPTT ID. This field uniquely identifies the MCPTT user information entry in the list of the MCPTT user information entries; and

2)	a list of MCPTT client information entries.

In each MCPTT client information entry, the MCPTT server shall maintain:

1)	an MCPTT client ID. This field uniquely identifies the MCPTT client information entry in the list of the MCPTT client information entries; and

2)	a list of MCPTT group information entries.

In each MCPTT group information, the MCPTT server shall maintain:

1)	an MCPTT group ID. This field uniquely identifies the MCPTT group information entry in the list of the MCPTT group information entries;

2)	an affiliation status;

3)	an expiration time;

4)	an affiliating p-id; and

5)	a next publishing time.

##### 9.2.2.2.3	Receiving affiliation status change from MCPTT client procedure

Upon receiving a SIP PUBLISH request such that:

1)	Request-URI of the SIP PUBLISH request contains either the public service identity identifying the originating participating MCPTT function serving the MCPTT user, or the public service identity identifying the terminating participating MCPTT function serving the MCPTT user;

2)	the SIP PUBLISH request contains an application/vnd.3gpp.mcptt-info+xml MIME body containing the<mcptt-request-uri> element which identifies an MCPTT ID served by the MCPTT server;

3)	the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Asserted-Service header field according to IETF RFC 6050 [9];

4)	the Event header field of the SIP PUBLISH request contains the "presence" event type; and

5)	SIP PUBLISH request contains an application/pidf+xml MIME body indicating per-user affiliation information according to subclause 9.3.1;

then the MCPTT server:

1)	shall identify the served MCPTT ID in the <mcptt-request-uri> element of the application/vnd.3gpp.mcptt-info+xml MIME body of the SIP PUBLISH request;

2)	if the Request-URI of the SIP PUBLISH request contains the public service identity identifying the originating participating MCPTT function serving the MCPTT user, shall identify the originating MCPTT ID from public user identity in the P-Asserted-Identity header field of the SIP PUBLISH request;

3)	if the Request-URI of the SIP PUBLISH request contains the public service identity identifying the terminating participating MCPTT function serving the MCPTT user, shall identify the originating MCPTT ID in the <mcptt-calling-user-identity> element of the application/vnd.3gpp.mcptt-info+xml MIME body of the SIP PUBLISH request;

4)	if the originating MCPTT ID is different than the served MCPTT ID and the originating MCPTT ID is not authorized to modify affiliation status of the served MCPTT ID, shall send a 403 (Forbidden) response and shall not continue with the rest of the steps;

5)	if the Expires header field of the SIP PUBLISH request is not included or has nonzero value lower than 4294967295, shall send a SIP 423 (Interval Too Brief) response to the SIP PUBLISH request, where the SIP 423 (Interval Too Brief) response contains a Min-Expires header field set to 4294967295, and shall not continue with the rest of the steps;

6)	if the Expires header field of the SIP PUBLISH request has nonzero value, shall determine the candidate expiration interval to according to IETF RFC 3903 [37];

7)	if the Expires header field of the SIP PUBLISH request has zero value, shall set the candidate expiration interval to zero;

8)	shall respond with SIP 200 (OK) response to the SIP PUBLISH request according to 3GPP TS 24.229 [4], IETF RFC 3903 [37]. In the SIP 200 (OK) response, the MCPTT server:

a)	shall set the Expires header field according to IETF RFC 3903 [37], to the candidate expiration time;

9)	if the "entity" attribute of the <presence> element of the application/pidf+xml MIME body of the SIP PUBLISH request is different than the served MCPTT ID, shall not continue with the rest of the steps;

10)	shall identify the served MCPTT client ID in the "id" attribute of the <tuple> element of the <presence> element of the application/pidf+xml MIME body of the SIP PUBLISH request;

11)	shall consider an MCPTT user information entry such that:

a)	the MCPTT user information entry is in the list of MCPTT user information entries described in subclause 9.2.2.2.2; and

b)	the MCPTT ID of the MCPTT user information entry is equal to the served MCPTT ID;

as the served MCPTT user information entry;

12)	shall consider an MCPTT client information entry such that:

a)	the MCPTT client information entry is in the list of MCPTT client information entries of the served MCPTT user information entry; and

b)	the MCPTT client ID of the MCPTT client information entry is equal to the served MCPTT client ID;

as the served MCPTT client information entry;

13)	shall consider a copy of the list of the MCPTT group information entries of the served MCPTT client information entry as the served list of the MCPTT group information entries;

14)	if the candidate expiration interval is nonzero:

a)	shall construct the candidate list of the MCPTT group information entries as follows:

i)	for each MCPTT group ID which has an MCPTT group information entry in the served list of the MCPTT group information entries, such that the expiration time of the MCPTT group information entry has not expired yet, and which is indicated in a "group" attribute of an <affiliation> element of the <status> element of the <tuple> element of the <presence> root element of the application/pidf+xml MIME body of the SIP PUBLISH request:

A)	shall copy the MCPTT group information entry into a new MCPTT group information entry of the candidate list of the MCPTT group information entries;

B)	if the affiliation status of the MCPTT group information entry is "deaffiliating" or "deaffiliated", shall set the affiliation status of the new MCPTT group information entry to the "affiliating" state and shall reset the affiliating p-id of the new MCPTT group information entry; and

C)	shall set the expiration time of the new MCPTT group information entry to the current time increased with the candidate expiration interval;

ii)	for each MCPTT group ID which has an MCPTT group information entry in the served list of the MCPTT group information entries, such that the expiration time of the MCPTT group information entry has not expired yet, and which is not indicated in any "group" attribute of the <affiliation> element of the <status> element of the <tuple> element of the <presence> root element of the application/pidf+xml MIME body of the SIP PUBLISH request:

A)	shall copy the MCPTT group information entry into a new MCPTT group information entry of the candidate list of the MCPTT group information entries; and

B)	if the affiliation status of the MCPTT group information entry is "affiliated" or "affiliating":

-	shall set the affiliation status of the new MCPTT group information entry to the "de-affiliating" state; and

-	shall set the expiration time of the new MCPTT group information entry to the current time increased with twice the value of timer F; and

iii)	for each MCPTT group ID:

A)	which does not have an MCPTT group information entry in the served list of the MCPTT group information entries; or

B)	which has an MCPTT group information entry in the served list of the MCPTT group information entries, such that the expiration time of the MCPTT group information entry has already expired;

and which is indicated in a "group" element of the <affiliation> element of the <status> element of the <tuple> element of the <presence> root element of the application/pidf+xml MIME body of the SIP PUBLISH request:

A)	shall add a new MCPTT group information entry in the candidate list of the MCPTT group information list for the MCPTT group ID;

B)	shall set the affiliation status of the new MCPTT group information entry to the "affiliating" state;

C)	shall set the expiration time of the new MCPTT group information entry to the current time increased with the candidate expiration interval; and

D)	shall reset the affiliating p-id of the new MCPTT group information entry;

b)	determine the candidate number of MCPTT group IDs as number of different MCPTT group IDs which have an MCPTT group information entry:

i)	in the candidate list of the MCPTT group information entries; or

ii)	in the list of the MCPTT group information entries of an MCPTT client information entry such that:

A)	the MCPTT client information entry is in the list of the MCPTT client information entries of the served MCPTT user information entry; and

B)	the MCPTT client ID of the MCPTT client information entry is not equal to the served MCPTT client ID;

with the affiliation status set to the "affiliating" state or the "affiliated" state and with the expiration time which has not expired yet; and

c)	if the candidate number of MCPTT group IDs is bigger than N2 value of the served MCPTT ID, shall based on MCPTT service provider policy reduce the candidate MCPTT group IDs to that equal to N2;

NOTE:	The MCPTT service provider policy can determine to remove an MCPTT group ID based on the order it appeared in the PUBLISH request or based on the importance or priority of the MCPTT group or some other policy to determine which MCPTT groups are preferred.

15)	if the candidate expiration interval is zero, constructs the candidate list of the MCPTT group information entries as follows:

a)	for each MCPTT group ID which has an entry in the served list of the MCPTT group information entries:

i)	shall copy the MCPTT group entry of the served list of the MCPTT group information into a new MCPTT group information entry of the candidate list of the MCPTT group information entries;

ii)	shall set the affiliation status of the new MCPTT group information entry to the "de-affiliating" state; and

iii)	shall set the expiration time of the new MCPTT group information entry to the current time increased with twice the value of timer F;

16)	shall replace the list of the MCPTT group information entries stored in the served MCPTT client information entry with the candidate list of the MCPTT group information entries;

17)	shall perform the procedures specified in subclause 9.2.2.2.6 for the served MCPTT ID and each MCPTT group ID:

a)	which does not have an MCPTT group information entry in the served list of the MCPTT group information entries and which has an MCPTT group information entry in the candidate list of the MCPTT group information entries with the affiliation status set to the "affiliating" state;

b)	which has an MCPTT group information entry in the served list of the MCPTT group information entries with the expiration time already expired, and which has an MCPTT group information entry in the candidate list of the MCPTT group information entries with the affiliation status set to the "affiliating" state;

c)	which has an MCPTT group information entry in the served list of the MCPTT group information entries with the affiliation status set to the "deaffiliating" state or the "deaffiliated" state and with the expiration time not expired yet, and which has an MCPTT group information entry in the candidate list of the MCPTT group information entries with the affiliation status set to the "affiliating" state; or

d)	which has an MCPTT group information entry in the served list of the MCPTT group information entries with the affiliation status set to the "affiliated" state and with the expiration time not expired yet, and which has an MCPTT group information entry in the candidate list of the MCPTT group information entries with the affiliation status set to the "de-affiliating" state;

18)	shall identify the handled p-id in the <p-id> child element of the <presence> root element of the application/pidf+xml MIME body of the SIP PUBLISH request; and

19)	shall perform the procedures specified in subclause 9.2.2.2.5 for the served MCPTT ID.

##### 9.2.2.2.4	Receiving subscription to affiliation status procedure

Upon receiving a SIP SUBSCRIBE request such that:

1)	Request-URI of the SIP SUBSCRIBE request contains either the public service identity identifying the originating participating MCPTT function serving the MCPTT user, or the public service identity identifying the terminating participating MCPTT function serving the MCPTT user;

2)	the SIP SUBSCRIBE request contains an application/vnd.3gpp.mcptt-info+xml MIME body containing the<mcptt-request-uri> element which identifies an MCPTT ID served by the MCPTT server;

3)	the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Asserted-Service header field according to IETF RFC 6050 [9]; and

4)	the Event header field of the SIP SUBSCRIBE request contains the "presence" event type;

the MCPTT server:

1)	shall identify the served MCPTT ID in the <mcptt-request-uri> element of the application/vnd.3gpp.mcptt-info+xml MIME body of the SIP PUBLISH request;

2)	if the Request-URI of the SIP SUBSCRIBE request contains the public service identity identifying the originating participating MCPTT function serving the MCPTT user, shall identify the originating MCPTT ID from public user identity in the P-Asserted-Identity header field of the SIP SUBSCRIBE request;

3)	if the Request-URI of the SIP SUBSCRIBE request contains the public service identity identifying the terminating participating MCPTT function serving the MCPTT user, shall identify the originating MCPTT ID in the <mcptt-calling-user-identity> element of the application/vnd.3gpp.mcptt-info+xml MIME body of the SIP SUBSCRIBE request;

4)	if the originating MCPTT ID is different than the served MCPTT ID and the originating MCPTT ID is not authorized to modify affiliation status of the served MCPTT ID, shall send a 403 (Forbidden) response and shall not continue with the rest of the steps; and

5)	shall generate a 200 (OK) response to the SIP SUBSCRIBE request according to 3GPP TS 24.229 [4], IETF RFC 6665 [26].

For the duration of the subscription, the MCPTT server shall notify the subscriber about changes of the information of the served MCPTT ID, as described in subclause 9.2.2.2.5.

##### 9.2.2.2.5	Sending notification of change of affiliation status procedure

In order to notify the subscriber about changes of the served MCPTT ID, the MCPTT server:

1)	shall consider an MCPTT user information entry such that:

a)	the MCPTT user information entry is in the list of MCPTT user information entries described in subclause 9.2.2.2.2; and

b)	the MCPTT ID of the MCPTT user information entry is equal to the served MCPTT ID;

as the served MCPTT user information entry;

2)	shall consider the list of the MCPTT client information entries of the served MCPTT user information entry as the served list of the MCPTT client information entries;

3)	shall generate an application/pidf+xml MIME body indicating per-user affiliation information according to subclause 9.3.1 and the served list of the MCPTT client information entries with the following clarifications:

a)	the MCPTT server shall not include information from an MCPTT group information entry with the expiration time already expired;

b)	the MCPTT server shall not include information from an MCPTT group information entry with the affiliation status set to the "deaffiliated" state;

c)	if the SIP SUBSCRIBE request creating the subscription of this notification contains an application/simple-filter+xml MIME body according to subclause 9.3.2, the MCPTT server shall restrict the application/pidf+xml MIME body according to the application/simple-filter+xml MIME body; and

d)	if this procedures is invoked by procedure in subclause 9.2.2.2.3 where the handled p-id value was identified, the MCPTT server shall set the <p-id> child element of the <presentity> root element of the application/pidf+xml MIME body of the SIP NOTIFY request to the handled p-id value; and

4)	send a SIP NOTIFY request according to 3GPP TS 24.229 [4], and IETF RFC 6665 [26] with the MIME body. In the SIP NOTIFY request, the MCPTT server shall include the generated application/pidf+xml MIME body indicating per-user affiliation information.

##### 9.2.2.2.6	Sending affiliation status change towards MCPTT server owning MCPTT group procedure

NOTE 1:	Usage of one SIP PUBLISH request to carry information about change of affiliation state of several MCPTT users served by the same MCPTT server is not supported in this version of the specification.

In order:

-	to send an affiliation request of a served MCPTT ID to a handled MCPTT group ID;

-	to send an de-affiliation request of a served MCPTT ID from a handled MCPTT group ID; or

-	to send an affiliation request of a served MCPTT ID to a handled MCPTT group ID due to near expiration of the previously published information;

the MCPTT server shall generate a SIP PUBLISH request according to 3GPP TS 24.229 [4], IETF RFC 3903 [37] and IETF RFC 3856 [51]. In the SIP PUBLISH request, the MCPTT server:

1)	shall set the Request-URI to the public service identity of the controlling MCPTT function associated with the handled MCPTT group ID;

2)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body. In the application/vnd.3gpp.mcptt-info+xml MIME body, the MCPTT server:

a)	shall include the <mcptt-request-uri> element set to the handled MCPTT group ID; and

b)	shall include the <mcptt-calling-user-id> element set to the served MCPTT ID;

3)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Asserted-Service header field according to IETF RFC 6050 [9];

4)	if sending an affiliation request, shall set the Expires header field according to IETF RFC 3903 [37], to 4294967295;

NOTE 1:	4294967295, which is equal to 232-1, is the highest value defined for Expires header field in IETF RFC 3261 [24].

5)	if sending an de-affiliation request, shall set the Expires header field according to IETF RFC 3903 [37], to zero;

6)	shall include an P-Asserted-Identity header field set to the public service identity of the MCPTT server according to 3GPP TS 24.229 [4];

7)	 shall set the current p-id to a globally unique value;

8)	shall consider an MCPTT user information entry such that:

a)	the MCPTT user information entry is in the list of MCPTT user information entries described in subclause 9.2.2.2.2; and

b)	the MCPTT ID of the MCPTT user information entry is equal to the served MCPTT ID;

as the served MCPTT user information entry;

9)	for each MCPTT group information entry such that:

a)	the MCPTT group information entry has the "affiliating" affiliation status, the MCPTT group ID set to the handled MCPTT group ID, the expiration time has not expired yet and the affiliating p-id is not set;

b)	the MCPTT group information entry is in the list of the MCPTT group information entries of an MCPTT client information entry; and

c)	 the MCPTT client information entry is in the list of the MCPTT client information entries of the served MCPTT user information entry;

shall set the affiliating p-id of the MCPTT group information entry to the current p-id; and

10)	shall include an application/pidf+xml MIME body indicating per-group affiliation information constructed according to subclause 9.2.3.2. The MCPTT server shall indicate all served MCPTT client IDs, such that:

a)	the affiliation status is set to "affiliating" or "affiliated", and the expiration time has not expired yet in an MCPTT group information entry with the MCPTT group ID set to the handled MCPTT group;

b)	the MCPTT group information entry is in the list of the MCPTT group information entries of an MCPTT client information entry;

c)	the MCPTT client information entry has the MCPTT client ID set to the served MCPTT client ID; and

d)	the MCPTT client information entry is in the list of the MCPTT client information entries of the served MCPTT user information entry.

The MCPTT server shall set the <p-id> child element of the <presence> root element to the current p-id.

The MCPTT server shall not include the "expires" attribute in the <affiliation> element.

The MCPTT server shall send the SIP PUBLISH request according to 3GPP TS 24.229 [4].

If timer F expires for the SIP PUBLISH request sent for a (de)affiliation request of served MCPTT ID to the MCPTT group ID or upon receiving a SIP 3xx, 4xx, 5xx or 6xx response to the SIP PUBLISH request, the MCPTT server:

1)	shall remove each MCPTT group ID entry such that:

a)	the MCPTT group information entry has the MCPTT group ID set to the handled MCPTT group ID;

b)	the MCPTT group information entry is in the list of the MCPTT group information entries of an MCPTT client information entry; and

c)	 the MCPTT client information entry is in the list of the MCPTT client information entries of the served MCPTT user information entry.

##### 9.2.2.2.7	Affiliation status determination from MCPTT server owning MCPTT group procedure

NOTE 1:	Usage of one SIP SUBSCRIBE request to subscribe for notification about change of affiliation state of several MCPTT users served by the same MCPTT server is not supported in this version of the specification.

In order to discover whether a served MCPTT user was successfully affiliated to a handled MCPTT group in the MCPTT server owning the handled MCPTT group, the MCPTT server shall generate an initial SIP SUBSCRIBE request according to 3GPP TS 24.229 [4], IETF RFC 3856 [51], and IETF RFC 6665 [26].

In the SIP SUBSCRIBE request, the MCPTT server:

1)	shall set the Request-URI to the public service identity of the controlling MCPTT function associated with the handled MCPTT group ID;

2)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body. In the application/vnd.3gpp.mcptt-info+xml MIME body, the MCPTT server:

a)	shall include the <mcptt-request-uri> element set to the handled MCPTT group ID; and

b)	shall include the <mcptt-calling-user-id> element set to the served MCPTT ID;

3)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Asserted-Service header field according to IETF RFC 6050 [9];

4)	if the MCPTT server wants to receive the current status and later notification, shall set the Expires header field according to IETF RFC 6665 [26], to 4294967295;

NOTE 2:	4294967295, which is equal to 232-1, is the highest value defined for Expires header field in IETF RFC 3261 [24].

5)	if the MCPTT server wants to fetch the current state only, shall set the Expires header field according to IETF RFC 6665 [26], to zero; and

6)	shall include an Accept header field containing the application/pidf+xml MIME type.

In order to re-subscribe or de-subscribe, the MCPTT server shall generate an in-dialog SIP SUBSCRIBE request according to 3GPP TS 24.229 [4], IETF RFC 3856 [51], and IETF RFC 6665 [26]. In the SIP SUBSCRIBE request, the MCPTT server:

1)	if the MCPTT server wants to receive the current status and later notification, shall set the Expires header field according to IETF RFC 6665 [26], to 4294967295;

NOTE 3:	4294967295, which is equal to 232-1, is the highest value defined for Expires header field in IETF RFC 3261 [24].

2)	if the MCPTT server wants to de-subscribe, shall set the Expires header field according to IETF RFC 6665 [26], to zero; and

3)	shall include an Accept header field containing the application/pidf+xml MIME type.

Upon receiving a SIP NOTIFY request according to 3GPP TS 24.229 [4], IETF RFC 3856 [51], and IETF RFC 6665 [26], if SIP NOTIFY request contains an application/pidf+xml MIME body indicating per-group affiliation information constructed according to subclause 9.3.1, then the MCPTT server:

1)	for each served MCPTT ID and served MCPTT client ID such that the application/pidf+xml MIME body of SIP NOTIFY request contains:

a)	a <tuple> element of the root <presence> element;

b)	the "id" attribute of the <tuple> element indicating the served MCPTT ID;

c)	an <affiliation> child element of the <status> element of the <tuple> element;

d)	the "client" attribute of the <affiliation> element indicating the served MCPTT client ID; and

d)	the "expires" attribute of the <affiliation> element indicating expiration of affiliation;

perform the following:

a)	if an MCPTT group information entry exists such that:

i)	the MCPTT group information entry has the "affiliating" affiliation status, the MCPTT group ID set to the handled MCPTT group ID, and the expiration time has not expired yet;

ii)	the MCPTT group information entry is in the list of the MCPTT group information entries of an MCPTT client information entry with the MCPTT client ID set to the served MCPTT client ID;

iii)	the MCPTT client information entry is in the list of the MCPTT client information entries of a served MCPTT user information entry with the MCPTT ID set to the served MCPTT ID; and

iv)	the MCPTT user information entry is in the list of MCPTT user information entries described in subclause 9.2.2.2.2; and

shall set the affiliation status of the MCPTT group information entry to "affiliated"; and

shall set the next publishing time of the MCPTT group information entry to the current time and half of the time between the current time and the expiration of affiliation; and

2) for each MCPTT group information entry such that:

a)	the MCPTT group information entry has the "affiliated" affiliation status or the "deaffiliating" affiliation status, the MCPTT group ID set to the handled MCPTT group ID, and the expiration time has not expired yet;

b)	the MCPTT group information entry is in the list of the MCPTT group information entries of an MCPTT client information entry with the MCPTT client ID set to a served MCPTT client ID;

c)	 the MCPTT client information entry is in the list of the MCPTT client information entries of the served MCPTT user information entry with the MCPTT ID set to a served MCPTT ID; and

d)	the MCPTT user information entry is in the list of MCPTT user information entries described in subclause 9.2.2.2.2; and

for which the application/pidf+xml MIME body of SIP NOTIFY request does not contain:

a)	a <tuple> element of the root <presence> element;

b)	the "id" attribute of the <tuple> element indicating the served MCPTT ID;

c)	an <affiliation> child element of the <status> child element of the <tuple> element; and

d)	the "client" attribute of the <affiliation> element indicating the served MCPTT client ID.

perform the following:

a)	shall set the affiliation status of the MCPTT group information entry to "deaffiliated"; and

b)	shall set the expiration time of the MCPTT group information entry to the current time; and

3)	if a <p-id> element is included in the <presence> root element of the application/pidf+xml MIME body of the SIP NOTIFY request, then for each MCPTT group information entry such that:

a)	the MCPTT group information entry has the "affiliating" affiliation status, the MCPTT group ID set to the handled MCPTT group ID, the expiration time has not expired yet and with the affiliating p-id set to the value of the <p-id> element;

b)	the MCPTT group information entry is in the list of the MCPTT group information entries of an MCPTT client information entry with the MCPTT client ID set to a served MCPTT client ID;

c)	 the MCPTT client information entry is in the list of the MCPTT client information entries of the served MCPTT user information entry with the MCPTT ID set to a served MCPTT ID; and

d)	the MCPTT user information entry is in the list of MCPTT user information entries described in subclause 9.2.2.2.2; and

for which the application/pidf+xml MIME body of SIP NOTIFY request does not contain:

a)	a <tuple> element of the root <presence> element;

b)	the "id" attribute of the <tuple> element indicating the served MCPTT ID;

c)	an <affiliation> child element of the <status> child element of the <tuple> element; and

d)	the "client" attribute of the <affiliation> element indicating the served MCPTT client ID;

perform the following:

a)	shall set the affiliation status of the MCPTT group information entry to "deaffiliated"; and

b)	shall set the expiration time of the MCPTT group information entry to the current time.

##### 9.2.2.2.8	Procedure for authorizing affiliation status change request in negotiated mode sent to served MCPTT user

Upon receiving a SIP MESSAGE request such that:

1)	Request-URI of the SIP MESSAGE request contains the public service identity identifying the terminating participating MCPTT function serving the MCPTT user;

2)	the SIP MESSAGE request contains an application/vnd.3gpp.mcptt-info+xml MIME body containing the<mcptt-request-uri> element and the <mcptt-calling-user-identity> element;

3)	the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Asserted-Service header field according to IETF RFC 6050 [9]; and

4)	the SIP MESSAGE request contains an application/vnd.3gpp.mcptt-affiliation-command+xml MIME body;

then the MCPTT server:

1)	shall identify the served MCPTT ID in the <mcptt-request-uri> element of the application/vnd.3gpp.mcptt-info+xml MIME body of the SIP MESSAGE request;

2)	shall identify the originating MCPTT ID in the <mcptt-calling-user-identity> element of the application/vnd.3gpp.mcptt-info+xml MIME body of the SIP MESSAGE request;

3)	if the originating MCPTT ID is not authorized to send an affiliation status change request in negotiated mode to the served MCPTT ID, shall send a 403 (Forbidden) response and shall not continue with the rest of the steps;

4)	shall set the Request-URI of the SIP MESSAGE request to the public user identity bound to the served MCPTT ID in the MCPTT server; and

5)	shall include an Accept-Contact header field with the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [6];

before forwarding the SIP MESSAGE request further.

##### 9.2.2.2.9	Forwarding affiliation status change towards another MCPTT user procedure

Upon receiving a SIP PUBLISH request such that:

1)	Request-URI of the SIP PUBLISH request contains the public service identity identifying the originating participating MCPTT function serving the MCPTT user;

2)	the SIP PUBLISH request contains an application/vnd.3gpp.mcptt-info MIME body containing the<mcptt-request-uri> element which identifies an MCPTT ID not served by the MCPTT server;

3)	the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Asserted-Service header field according to IETF RFC 6050 [9];

4)	the Event header field of the SIP PUBLISH request contains the "presence" event type; and

5)	SIP PUBLISH request contains an application/pidf+xml MIME body indicating per-user affiliation information according to subclause 9.3.1;

then the MCPTT server:

1)	shall identify the target MCPTT ID in the <mcptt-request-uri> element of the application/vnd.3gpp.mcptt-info MIME body of the SIP PUBLISH request;

2)	shall identify the originating MCPTT ID from public user identity in the P-Asserted-Identity header field of the SIP PUBLISH request;

3)	shall generate a SIP PUBLISH request from the received SIP PUBLISH request. In the generated SIP PUBLISH request, the MCPTT server:

a)	shall set the Request-URI to the public service identity identifying the terminating participating MCPTT function serving the target MCPTT ID;

b)	shall include a P-Asserted-Identity header field containing the public service identity identifying the originating participating MCPTT function serving the MCPTT user;

c)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body. In the application/vnd.3gpp.mcptt-info+xml MIME body, the MCPTT server:

A)	shall include the <mcptt-request-uri> element set to the target MCPTT ID; and

B)	shall include the <mcptt-calling-user-id> element set to the originating MCPTT ID; and

d)	shall include other signalling elements from the received SIP PUBLISH request; and

4)	shall send the generated SIP PUBLISH request according to 3GPP TS 24.229 [4].

The MCPTT server shall forward received SIP responses to the SIP PUBLISH request.

##### 9.2.2.2.10	Forwarding subscription to affiliation status towards another MCPTT user procedure

Upon receiving a SIP SUBSCRIBE request such that:

1)	Request-URI of the SIP PUBLISH request contains the public service identity identifying the originating participating MCPTT function serving the MCPTT user;

2)	the SIP PUBLISH request contains an application/vnd.3gpp.mcptt-info MIME body containing the<mcptt-request-uri> element which identifies an MCPTT ID not served by MCPTT server;

3)	the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Asserted-Service header field according to IETF RFC 6050 [9]; and

4)	the Event header field of the SIP SUBSCRIBE request contains the "presence" event type;

then the MCPTT server:

1)	shall identify the target MCPTT ID in the <mcptt-request-uri> element of the application/vnd.3gpp.mcptt-info MIME body of the SIP SUBSCRIBE request;

2)	shall identify the originating MCPTT ID from public user identity in the P-Asserted-Identity header field of the SIP SUBSCRIBE request;

3)	shall generate a SIP SUBSCRIBE request from the received SIP SUBSCRIBE request. In the generated SIP SUBSCRIBE request, the MCPTT server:

a)	shall set the Request-URI to the public service identity identifying the terminating participating MCPTT function serving the target MCPTT ID;

b)	shall include a P-Asserted-Identity header field containing the public service identity identifying the originating participating MCPTT function serving the MCPTT user;

c)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body. In the application/vnd.3gpp.mcptt-info+xml MIME body, the MCPTT server:

A)	shall include the <mcptt-request-uri> element set to the target MCPTT ID; and

B)	shall include the <mcptt-calling-user-id> element set to the originating MCPTT ID; and

d)	shall include other signalling elements from the received SIP SUBSCRIBE request; and

4)	shall send the generated SIP SUBSCRIBE request according to 3GPP TS 24.229 [4].

The MCPTT server shall forward any received SIP responses to the SIP SUBSCRIBE request, any received SIP NOTIFY request and any received SIP responses to the SIP NOTIFY request.

#### 9.2.2.3	Procedures of MCPTT server owning the MCPTT group

##### 9.2.2.3.1	General

The procedures of MCPTT server owning the MCPTT group consist of:

-	receiving group affiliation status change procedure;

-	receiving subscription to affiliation status procedure; and

-	sending notification of change of affiliation status procedure.

NOTE:	Usage of CSC-3 part of MCPTT group affiliation procedure and of CSC-3 part of MCPTT group de-affiliation procedure is not specified in this version of the specification.

##### 9.2.2.3.2	Stored information

The MCPTT server shall maintain a list of MCPTT group information entries.

In each MCPTT group information entry, the MCPTT server shall maintain:

1)	an MCPTT group ID. This field uniquely identifies the MCPTT group information entry in the list of the MCPTT group information entries; and

2)	a list of MCPTT user information entries.

In each MCPTT user information entry, the MCPTT server shall maintain:

1)	an MCPTT ID. This field uniquely identifies the MCPTT user information entry in the list of the MCPTT user information entries;

2)	a list of MCPTT client information entries; and

3)	an expiration time.

In each MCPTT client information entry, the MCPTT server shall maintain:

1)	an MCPTT client ID. This field uniquely identifies the MCPTT client information entry in the list of the MCPTT client information entries.

##### 9.2.2.3.3	Receiving group affiliation status change procedure

Upon receiving a SIP PUBLISH request such that:

1)	Request-URI of the SIP PUBLISH request contains the public service identity of the controlling MCPTT function associated with the served MCPTT group;

2)	the SIP PUBLISH request contains an application/vnd.3gpp.mcptt-info+xml MIME body containing the <mcptt-request-uri> element and the <mcptt-calling-user-identity> element;

3)	the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Asserted-Service header field according to IETF RFC 6050 [9];

4)	the Event header field of the SIP PUBLISH request contains the "presence" event type; and

5)	SIP PUBLISH request contains an application/pidf+xml MIME body indicating per-group affiliation information constructed according to subclause 9.2.3.2;

then the MCPTT server:

1)	shall identify the served MCPTT group ID in the <mcptt-request-uri> element of the application/vnd.3gpp.mcptt-info+xml MIME body of the SIP PUBLISH request;

2)	shall identify the handled MCPTT ID in the <mcptt-calling-user-identity> element of the application/vnd.3gpp.mcptt-info+xml MIME body of the SIP PUBLISH request;

3)	if the Expires header field of the SIP PUBLISH request is not included or has nonzero value lower than 4294967295, shall send a SIP 423 (Interval Too Brief) response to the SIP PUBLISH request, where the SIP 423 (Interval Too Brief) response contains a Min-Expires header field set to 4294967295, and shall not continue with the rest of the steps;

4)	if an MCPTT group for the served MCPTT group ID does not exist in the group management server according to 3GPP TS 24.381 [31], shall reject the SIP PUBLISH request with SIP 403 (Forbidden) response to the SIP PUBLISH request according to 3GPP TS 24.229 [4], IETF RFC 3903 [37] and IETF RFC 3856 [51] and skip the rest of the steps;

5)	if the handled MCPTT ID is not a member of the MCPTT group identified by the served MCPTT group ID, shall reject the SIP PUBLISH request with SIP 403 (Forbidden) response to the SIP PUBLISH request according to 3GPP TS 24.229 [4], IETF RFC 3903 [37] and IETF RFC 3856 [51] and skip the rest of the steps;

6)	shall respond with SIP 200 (OK) response to the SIP PUBLISH request according to 3GPP TS 24.229 [4], IETF RFC 3903 [37]. In the SIP 200 (OK) response, the MCPTT server:

a)	shall set the Expires header field according to IETF RFC 3903 [37], to the selected expiration time;

7)	if the "entity" attribute of the <presence> element of the application/pidf+xml MIME body of the SIP PUBLISH request is different than the served MCPTT group ID, shall not continue with the rest of the steps;

8)	if the handled MCPTT ID is different from the MCPTT ID in the "id" attribute of the <tuple> element of the <presence> root element of the application/pidf+xml MIME body of the SIP PUBLISH request, shall not continue with the rest of the steps;

9)	shall consider an MCPTT group information entry such that:

a)	the MCPTT group information entry is in the list of MCPTT group information entries described in subclause 9.2.2.3.2; and

b)	the MCPTT group ID of the MCPTT group information entry is equal to the served MCPTT group ID;

as the served MCPTT group information entry;

10)	if the selected expiration time is zero:

a)	shall remove the MCPTT user information entry such that:

i)	the MCPTT user information entry is in the list of the MCPTT user information entries of the served MCPTT group information entry; and

ii)	the MCPTT user information entry has the MCPTT ID set to the served MCPTT ID;

11)	if the selected expiration time is not zero:

a)	shall consider an MCPTT user information entry such that:

i)	the MCPTT user information entry is in the list of the MCPTT user information entries of the served MCPTT group information entry; and

ii)	the MCPTT ID of the MCPTT user information entry is equal to the handled MCPTT ID;

as the served MCPTT user information entry; and

b)	shall set the following information in the served MCPTT user information entry:

i)	set the MCPTT client ID list according to the "client" attributes of the <affiliation> elements of the <status> element of the <tuple> element of the <presence> root element of the application/pidf+xml MIME body of the SIP PUBLISH request; and

ii)	set the expiration time according to the selected expiration time;

12)	shall identify the handled p-id in the <p-id> child element of the <presence> root element of the application/pidf+xml MIME body of the SIP PUBLISH request; and

13)	shall perform the procedures specified in subclause 9.2.2.3.5 for the served MCPTT group ID.

##### 9.2.2.3.4	Receiving subscription to affiliation status procedure

NOTE:	Usage of one SIP SUBSCRIBE request to subscribe for notification about change of affiliation state of several MCPTT users served by the same MCPTT server is not supported in this version of the specification.

Upon receiving a SIP SUBSCRIBE request such that:

1)	Request-URI of the SIP SUBSCRIBE request contains the public service identity of the controlling MCPTT function associated with the served MCPTT group;

2)	the SIP SUBSCRIBE request contains an application/vnd.3gpp.mcptt-info+xml MIME body containing the<mcptt-request-uri> element and the <mcptt-calling-user-identity> element;

3)	the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Asserted-Service header field according to IETF RFC 6050 [9]; and

4)	the Event header field of the SIP SUBSCRIBE request contains the "presence" event type;

then the MCPTT server:

1)	shall identify the served MCPTT group ID in the <mcptt-request-uri> element of the application/vnd.3gpp.mcptt-info+xml MIME body of the SIP SUBSCRIBE request;

2)	shall identify the handled MCPTT ID in the <mcptt-calling-user-identity> element of the application/vnd.3gpp.mcptt-info+xml MIME body of the SIP SUBSCRIBE request;

3)	if the Expires header field of the SIP SUBSCRIBE request is not included or has nonzero value lower than 4294967295, shall send a SIP 423 (Interval Too Brief) response to the SIP SUBSCRIBE request, where the SIP 423 (Interval Too Brief) response contains a Min-Expires header field set to 4294967295, and shall not continue with the rest of the steps;

4)	if an MCPTT group for the served MCPTT group ID does not exist in the group management server according to 3GPP TS 24.381 [31], shall reject the SIP SUBSCRIBE request with SIP 403 (Forbidden) response to the SIP SUBSCRIBE request according to 3GPP TS 24.229 [4], IETF RFC 3903 [37] and IETF RFC 3856 [51] and skip the rest of the steps;

5)	if the handled MCPTT ID is not a member of the MCPTT group identified by the served MCPTT group ID, shall reject the SIP SUBSCRIBE request with SIP 403 (Forbidden) response to the SIP SUBSCRIBE request according to 3GPP TS 24.229 [4], IETF RFC 3903 [37] and IETF RFC 3856 [51] and skip the rest of the steps; and

6)	shall generate a SIP 200 (OK) response to the SIP SUBSCRIBE request according to 3GPP TS 24.229 [4], IETF RFC 6665 [26].

For the duration of the subscription, the MCPTT server shall notify subscriber about changes of the information of the served MCPTT ID, as described in subclause 9.2.2.2.5.

##### 9.2.2.3.5	Sending notification of change of affiliation status procedure

In order to notify the subscriber identified by the handled MCPTT ID about changes of the affiliation status of the served MCPTT group ID, the MCPTT server:

1)	shall consider an MCPTT group information entry such that:

a)	the MCPTT group information entry is in the list of MCPTT group information entries described in subclause 9.2.2.3.2; and

b)	the MCPTT group ID of the MCPTT group information entry is equal to the served MCPTT group ID;

2)	shall consider an MCPTT user information entry such:

a)	the MCPTT user information entry is in the list of the MCPTT user information entries of the served MCPTT group information entry; and

b)	the MCPTT ID of the MCPTT user information entry is equal to the handled MCPTT ID;

as the served MCPTT user information entry;

3)	shall generate an application/pidf+xml MIME body indicating per-group affiliation information according to subclause 9.3.1 and the served list of the served MCPTT user information entry of the MCPTT group information entry with following clarifications:

a)	the MCPTT server shall include the "expires" attribute in the <affiliation> element; and

b)	if this procedures is invoked by procedure in subclause 9.2.2.3.3 where the handled p-id was identified, the MCPTT server shall set the <p-id> child element of the <presentity> root element of the application/pidf+xml MIME body of the SIP NOTIFY request to the handled p-id value; and

4)	send a SIP NOTIFY request according to 3GPP TS 24.229 [4], and IETF RFC 6665 [26]. In the SIP NOTIFY request, the MCPTT server shall include the generated application/pidf+xml MIME body indicating per-group affiliation information.

## 9.3	Coding

### 9.3.1	Extension of application/pidf+xml MIME type

#### 9.3.1.1	Introduction

The parent subclause of this subclause describes an extension of the application/pidf+xml MIME body specified in IETF RFC 3863 [52]. The extension is used to indicate:

-	per-user affiliation information; and

-	per-group affiliation information.

#### 9.3.1.2	Syntax

The application/pidf+xml MIME body indicating per-user affiliation information is constructed according to IETF RFC 3863 [52] and:

1)	contains a <presence> root element according to IETF RFC 3863 [52];

2)	contains an "entity" attribute of the <presence> element set to the MCPTT ID of the MCPTT user;

3)	contains one <tuple> child element according to IETF RFC 3863 [52] per each MCPTT client of the <presence> element;

4)	can contain a <p-id> child element defined in the XML schema defined in table 9.3.1.2-1, of the <presence> element set to an identifier of a SIP PUBLISH request;

5)	contains an "id" attribute of the <tuple> element set to the MCPTT client ID;

6)	contains one <status> child element of each <tuple> element;

7)	contains one <affiliation> child element defined in the XML schema defined in table 9.3.1.2-1, of the <status> element, for each MCPTT group in which the MCPTT user is interested at the MCPTT client;

8)	contains a "group" attribute of each <affiliation> element set to the MCPTT group ID of the MCPTT group in which the MCPTT user is interested at the MCPTT client;

9)	can contain a "status" attribute of each <affiliation> element indicating the affiliation status of the MCPTT user to MCPTT group at the MCPTT client; and

10)	can contain an "expires" attribute of each <affiliation> element indicating expiration of affiliation of the MCPTT user to MCPTT group at the MCPTT client.

The application/pidf+xml MIME body indicating per-group affiliation information is constructed according to IETF RFC 3856 [51] and:

1)	contains the <presence> root element according to IETF RFC 3863 [52];

2)	contains an "entity" attribute of the <presence> element set to the MCPTT group ID of the MCPTT group;

3)	contains one <tuple> child element according to IETF RFC 3863 [52] of the <presence> element;

4)	can contain a <p-id> child element defined in the XML schema defined in table 9.3.1.2-1, of the <presence> element set to an identifier of a SIP PUBLISH request;

5)	contains an "id" attribute of the <tuple> element set to the MCPTT ID of the MCPTT user;

6)	contains one <status> child element of each <tuple> element;

7)	contains one <affiliation> child element defined in the XML schema defined in table 9.3.1.2-1, of the <status> element, for each MCPTT client at which the MCPTT user is interested in the MCPTT group;

8)	contains one "client" attribute defined in the XML schema defined in table 9.3.1.2-2, of the <affiliation> element set to the MCPTT client ID; and

9)	can contain an "expires" attribute defined in the XML schema defined in table 9.3.1.2-2, of the <affiliation> element indicating expiration of affiliation of the MCPTT user to MCPTT group at the MCPTT client.

Table 9.3.1.2-1: XML schema with elements and attributes extending the application/pidf+xml MIME body

<?xml version="1.0" encoding="UTF-8"?>

<xs:schema

targetNamespace="urn:3gpp:ns:mcpttPresInfo:1.0"

xmlns:xs="http://www.w3.org/2001/XMLSchema"

xmlns:mcpttPI10="urn:3gpp:ns:mcpttPresInfo:1.0"

elementFormDefault="qualified" attributeFormDefault="unqualified">

<!-- MCPTT specific child elements of tuple element -->

<xs:element name="affiliation" type="mcpttPI10:affiliationType"/>

<xs:complexType name="affiliationType">

<xs:sequence>

<xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:attribute name="group" type="xs:anyURI" use="optional"/>

<xs:attribute name="client" type="xs:anyURI" use="optional"/>

<xs:attribute name="status" type="mcpttPI10:statusType" use="optional"/>

<xs:attribute name="expires" type="xs:dateTime" use="optional"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:simpleType name="statusType">

<xs:restriction base="xs:string">

<xs:enumeration value="affiliating"/>

<xs:enumeration value="affiliated"/>

<xs:enumeration value="deaffiliating"/>

</xs:restriction>

</xs:simpleType>

<xs:element name="p-id" type="xs:string"/>

</xs:schema>

The application/pidf+xml MIME body refers to namespaces using prefixes specified in table 9.3.1.2-2.

Table 9.3.1.2-2: Assignment of prefixes to namespace names in the application/pidf+xml MIME body

| Prefix | Namespace |
| --- | --- |
| mcpttPI10 | urn:3gpp:ns:mcpttPresInfo:1.0 |
| NOTE: The "urn:ietf:params:xml:ns:pidf" namespace is the default namespace so no prefix is used for it in the application/pidf+xml MIME body. |  |

### 9.3.2	Extension of application/simple-filter+xml MIME type

#### 9.3.2.1	Introduction

The parent subclause of this subclause describes extension of the application/simple-filter+xml MIME body specified in IETF RFC 4661 [63].

The extension is used to indicate per-client restrictions of presence event package notification information and per-user restrictions of presence event package notification information.

#### 9.3.2.2	Syntax

The application/simple-filter+xml MIME body indicating per client restrictions of presence event package notification information is constructed according to IETF RFC 4661 [63] and:

1)	contains a <filter-set> root element according to IETF RFC 4661 [63];

2)	contains a <ns-bindings> child element according to IETF RFC 4661 [63], of the <filter-set> element;

3)	contains a <ns-binding> child element according to IETF RFC 4661 [63], of the <ns-bindings> element where the <ns-binding> element:

A)	does not contain a "prefix" attribute according to IETF RFC 4661 [63]; and

B) contains an "urn" attribute set to the "urn:ietf:params:xml:ns:pidf" value;

4)	contains a <ns-binding> child element according to IETF RFC 4661 [63], of the <ns-bindings> element where the <ns-binding> element:

A)	contains a "prefix" attribute according to IETF RFC 4661 [63], set to "mcpttPI10"; and

B)	contains an "urn" attribute according to IETF RFC 4661 [63], set to the "urn:3gpp:ns:mcpttPresInfo:1.0" value;

5)	contains a <filter> child element according to IETF RFC 4661 [63], of the <filter-set> element where the <filter> element;

A)	contains an "id" attribute set to a value constructed according to IETF RFC 4661 [63];

B)	does not contain an "uri" attribute of the <filter> child element according to IETF RFC 4661 [63]; and

C)	does not contain an "domain" attribute according to IETF RFC 4661 [63];

6)	contains a <what> child element according to IETF RFC 4661 [63], of the <filter> element; and

7)	contains an <include> child element according to IETF RFC 4661 [63], of the <what> element where the <include> element;

A)	does not contain a "type" attribute according to IETF RFC 4661 [63]; and

B)	contains the value, according to IETF RFC 4661 [63], set to concatenation of the '//presence/tuple[@id="' string, the MCPTT client ID, and the '"]' string.

The application/simple-filter+xml MIME body indicating per user restrictions of presence event package notification information is constructed according to IETF RFC 4661 [63] and:

1)	contains a <filter-set> root element according to IETF RFC 4661 [63];

2)	contains a <ns-bindings> child element according to IETF RFC 4661 [63], of the <filter-set> element;

3)	contains a <ns-binding> child element according to IETF RFC 4661 [63], of the <ns-bindings> element where the <ns-binding> element:

A)	does not contain a "prefix" attribute according to IETF RFC 4661 [63]; and

B) contains an "urn" attribute set to the "urn:ietf:params:xml:ns:pidf" value;

4)	contains a <ns-binding> child element according to IETF RFC 4661 [63], of the <ns-bindings> element where the <ns-binding> element:

A)	contains a "prefix" attribute according to IETF RFC 4661 [63], set to "mcpttPI10"; and

B)	contains an "urn" attribute according to IETF RFC 4661 [63], set to the "urn:3gpp:ns:mcpttPresInfo:1.0" value;

5)	contains a <filter> child element according to IETF RFC 4661 [63], of the <filter-set> element where the <filter> element;

A)	contains an "id" attribute set to a value constructed according to IETF RFC 4661 [63];

B)	does not contain an "uri" attribute of the <filter> child element according to IETF RFC 4661 [63]; and

C)	does not contain an "domain" attribute according to IETF RFC 4661 [63];

6)	contains a <what> child element according to IETF RFC 4661 [63], of the <filter> element; and

7)	contains an <include> child element according to IETF RFC 4661 [63], of the <what> element where the <include> element;

A)	does not contain a "type" attribute according to IETF RFC 4661 [63]; and

B)	contains the value, according to IETF RFC 4661 [63], set to concatenation of the '//presence/tuple[@id="' string, the MCPTT ID, and the '"]' string.

# 10	Group call

## 10.0	General

This subclause describes the group call procedures for on-network and off-network.

For on-network, prearranged group call including emergency group call for each functional entity are specified in subclause 10.1.1 and chat group (restricted) call including emergency group call for each functional entity are specified in subclause 10.1.2.

Off-network group call and off-network broadcast group call are specified in subclause 10.2 and subclause 10.3.

## 10.1	On-network group call

### 10.1.1	Prearranged group call

#### 10.1.1.1	General

#### 10.1.1.2	MCPTT client procedures

##### 10.1.1.2.1	On-demand prearranged group call

###### 10.1.1.2.1.1	Client originating procedures

Upon receiving a request from an MCPTT user to establish an MCPTT prearranged group session the MCPTT client shall generate an initial SIP INVITE request by following the UE originating session procedures specified in 3GPP TS 24.229 [4], with the clarifications given below.

The MCPTT client:

1)	if the MCPTT user has requested the origination of an MCPTT emergency group call or is originating an MCPTT prearranged group call and the MCPTT emergency state is already set, the MCPTT client shall comply with the procedures in subclause 6.2.8.1.1;

2)	if the MCPTT user has requested the origination of an MCPTT imminent peril group call, the MCPTT client shall comply with the procedures in subclause 6.2.8.1.9;

3)	if the MCPTT user has requested the origination of a broadcast group call, the MCPTT client shall comply with the procedures in subclause 6.2.8.2;

4)	shall include the g.3gpp.mcptt media feature tag and the g.3gpp.icsi-ref media feature tag with the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" in the Contact header field of the SIP INVITE request according to IETF RFC 3840 [16];

5)	shall include an Accept-Contact header field containing the g.3gpp.mcptt media feature tag along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [6];

6)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Preferred-Service header field according to IETF RFC 6050 [9] in the SIP INVITE request;

7)	shall include an Accept-Contact header field with the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [6];

8)	should include the "timer" option tag in the Supported header field;

9)	should include the Session-Expires header field according to IETF RFC 4028 [7]. It is recommended that the "refresher" header field parameter is omitted. If included, the "refresher" header field parameter shall be set to "uac";

10)	shall set the Request-URI of the SIP INVITE request to the public service identity identifying the participating MCPTT function serving the MCPTT user;

NOTE 1:	The MCPTT client is configured with public service identity identifying the participating MCPTT function serving the MCPTT user.

11)	may include a P-Preferred-Identity header field in the SIP INVITE request containing a public user identity as specified in 3GPP TS 24.229 [4];

12)	if the MCPTT emergency state is already set or the MCPTT client emergency group state for this group is set to "MEG 2: in-progress", the MCPTT client shall include the Resource-Priority header field and comply with the procedures in subclause 6.2.8.1.2;

13)	if the MCPTT client imminent peril group state for this group is set to "MIG 2: in-progress" or "MIG 3: confirm-pending" shall include the Resource-Priority header field and comply with the procedures in subclause 6.2.8.1.12;

14)	shall contain in an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcpttinfo> element containing the <mcptt-Params> element with:

a)	the <session-type> element set to a value of "prearranged";

b)	the <mcptt-request-uri> element set to the group identity; and

NOTE 2:	The MCPTT client does not include the MCPTT ID of the originating MCPTT user in the body, as this will be inserted into the body of the SIP INVITE request that is sent from the originating participating MCPTT function.

c)	if the group identity can be determined to be a TGI and if the MCPTT client can associate the TGI with a MCPTT group ID, the <associated-group-id> element set to the MCPTT group ID;

NOTE 3:	The text "can associate the TGI with a MCPTT group ID" means that the MCPTT client is able to determine that there is a constituent group of the temporary group that it is a member of.

NOTE 4:	The MCPTT client is informed about temporary groups and regouping of MCPTT groups that the user is a member of as specified in 3GPP TS 24.381 [31].

NOTE 5:	If the MCPTT user selected a TGI where there are several MCPTT groups where the MCPTT user is a member, the MCPTT client selects one of those MCPTT groups.

15)	shall include an SDP offer according to 3GPP TS 24.229 [4] with the clarifications given in subclause 6.2.1;

16)	if an implicit floor request is required, shall indicate this as specified in subclause 6.4; and

17)	shall send the SIP INVITE request towards the MCPTT server according to 3GPP TS 24.229 [4].

On receiving a SIP 2xx response to the SIP INVITE request, the MCPTT client:

1)	shall interact with the user plane as specified in 3GPP TS 24.380 [5] ;

2)	if the MCPTT emergency group call state is set to "MEGC 2: emergency-call-requested" or "MEGC 3: emergency-call-granted" or the MCPTT imminent peril group call state is set to "MIGC 2: imminent-peril-call-requested" or "MIGC 3: imminent-peril-call-granted", the MCPTT client shall perform the actions specified in subclause 6.2.8.1.4; and

3)	may subscribe to the conference event package as specified in subclause 10.1.3.1.

On receiving a SIP 4xx response, a SIP 5xx response or a SIP 6xx response to the SIP INVITE request:

1)	if the MCPTT emergency group call state is set to "MEGC 2: emergency-call-requested" or "MEGC 3: emergency-call-granted"; or

2)	if the MCPTT imminent peril group call state is set to "MIGC 2: imminent-peril-call-requested" or "MIGC 3: imminent-peril-call-granted";

the MCPTT client shall perform the actions specified in subclause 6.2.8.1.5.

On receiving a SIP INFO request where the Request-URI contains an MCPTT session ID identifying an ongoing group session, the MCPTT client shall follow the actions specified in subclause 6.2.8.1.13.

###### 10.1.1.2.1.2	Client terminating procedures

In the procedures in this subclause:

1)	emergency indication in an incoming SIP INVITE request refers to the <emergency-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body; and

2)	imminent peril indication in an incoming SIP INVITE request refers to the <imminentperil-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body.

Upon receipt of an initial SIP INVITE request, the MCPTT client shall follow the procedures for termination of multimedia sessions in the IM CN subsystem as specified in 3GPP TS 24.229 [4] with the clarifications below.

The MCPTT client:

1)	may reject the SIP INVITE request if either of the following conditions are met:

a)	MCPTT client does not have enough resources to handle the call; or

b)	any other reason outside the scope of this specification;

and skip the rest of the steps;

2)	if the SIP INVITE request is rejected in step 1), shall respond toward participating MCPTT function either with appropriate reject code as specified in 3GPP TS 24.229 [4] and warning texts as specified in subclause 4.4.2 or with SIP 480 (Temporarily unavailable) response not including warning texts if the user is authorised to restrict the reason for failure and skip the rest of the steps of this subclause;

NOTE:	If the SIP INVITE request contains an emergency indication or imminent peril indication, the MCPTT client can by means beyond the scope of this specification choose to accept the request.

3)	shall check if a Resource-Priority header field is included in the incoming SIP INVITE request and may perform further actions outside the scope of this specification to act upon an included Resource-Priority header field as specified in 3GPP TS 24.229 [4];

4)	if the SIP INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcpttinfo> element containing the <mcptt-Params> element with the <emergency-ind> element set to a value of "true":

a)	should display to the MCPTT user an indication that this is a SIP INVITE request for an MCPTT emergency group call and:

i)	should display the MCPTT ID of the originator of the MCPTT emergency group call contained in the <mcptt-calling-user-id> element of the application/vnd.3gpp.mcptt-info+xml MIME body;

ii)	should display the MCPTT group identity of the group with the emergency condition contained in the <mcptt-calling-group-id> element; and

iii)	if the <alert-ind> element is set to "true", should display to the MCPTT user an indication of the MCPTT emergency alert and associated information;

b)	shall set the MCPTT emergency group state to "MEG 2: in-progress";

c)	shall set the MCPTT imminent peril group state to "MIG 1: no-imminent-peril"; and

d)	shall set the MCPTT imminent peril group call state to "MIGC 1: imminent-peril-gc-capable"; otherwise

5)	if the SIP INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcpttinfo> element containing the <mcptt-Params> element with the <imminentperil-ind> element set to a value of "true":

a)	should display to the MCPTT user an indication that this is a SIP INVITE request for an MCPTT imminent peril group call and:

i)	should display the MCPTT ID of the originator of the MCPTT imminent peril group call contained in the <mcptt-calling-user-id> element of the application/vnd.3gpp.mcptt-info+xml MIME body; and

ii)	should display the MCPTT group identity of the group with the imminent peril condition contained in the <mcptt-calling-group-id> element; and

b)	shall set the MCPTT imminent peril group state to "MIG 3: in-progress";

6)	may display to the MCPTT user the MCPTT ID of the inviting MCPTT user;

7)	shall perform the automatic commencement procedures specified in subclause 6.2.3.1.2 if one of the following conditions are met:

a)	SIP INVITE request contains an Answer-Mode header field with the value "Auto" and the MCPTT service setting at the invited MCPTT client for answering the call is set to automatic commencement mode; or

b)	SIP INVITE request contains an Answer-Mode header field with the value "Auto" and the MCPTT service setting at the invited MCPTT client for answering the call is set to manual commencement mode, yet the invited MCPTT client allows the call to be answered with automatic commencement mode;

8)	shall perform the manual commencement procedures specified in subclause 6.2.3.2.2 if one of the following conditions are met:

a)	SIP INVITE request contains an Answer-Mode header field with the value "Manual" and the MCPTT service setting at the invited MCPTT client for answering the call is to use manual commencement mode; or

b)	SIP INVITE request contains an Answer-Mode header field with the value "Manual" and the MCPTT service setting at the invited MCPTT client for answering the call is set to automatic commencement mode, yet the invited MCPTT client allows the call to be answered with manual commencement mode; and

9)	when the SIP 200 (OK) response to the SIP INVITE request is sent, may subscribe to the conference event package as specified in subclause 10.1.3.1.

###### 10.1.1.2.1.3	MCPTT upgrade to in-progress emergency or imminent peril

This subclause covers both on-demand session and pre-established sessions.

Upon receiving a request from an MCPTT user to upgrade the MCPTT group session to an emergency condition or an imminent peril condition on an MCPTT prearranged group, the MCPTT client shall generate a SIP re-INVITE request as specified in 3GPP TS 24.229 [4], with the clarifications given below.

1)	if the MCPTT user is requesting to upgrade the MCPTT group session to an in-progress emergency group state and this is an unauthorised request for an MCPTT emergency call as determined by the procedures of subclause 6.2.8.1.8, the MCPTT client:

a)	should indicate to the MCPTT user that they are not authorised to upgrade the MCPTT group session to an in-progress emergency group state; and

b)	shall skip the remaining steps of the current subclause;

2)	if the MCPTT user is requesting to upgrade the MCPTT group session to an in-progress imminent peril state and this is an unauthorised request for an MCPTT imminent peril group call as determined by the procedures of subclause 6.2.8.1.8, the MCPTT client:

a)	should indicate to the MCPTT user that they are not authorised to upgrade the MCPTT group session to an in-progress imminent peril group state; and

b)	shall skip the remaining steps of the current subclause;

3)	if the MCPTT user has requested to upgrade the MCPTT group session to an MCPTT emergency call, the MCPTT client:

a)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body populated as specified in subclause 6.2.8.1.1;

b)	if an indication of an MCPTT emergency alert is to be included, shall perform the procedures specified in subclause 6.2.9.1 for the MCPTT emergency alert trigger; and

c)	shall include a Resource-Priority header field and comply with the procedures in subclause 6.2.8.1.2.

4)	if the MCPTT user has requested to upgrade the MCPTT group session to an MCPTT imminent peril call, the MCPTT client:

a)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body populated as specified in subclause 6.2.8.1.9; and

b)	shall include a Resource-Priority header field and comply with the procedures in subclause 6.2.8.1.12;

5)	if the SIP re-INVITE request is to be sent within an on-demand session, shall include in the SIP re-INVITE request an SDP offer according to 3GPP TS 24.229 [4] with the clarifications specified in subclause 6.2.1;

6)	if the SIP re-INVITE request is to be sent within a pre-established session, shall include an SDP offer in the SIP re-INVITE request according to 3GPP TS 24.229 [4], based upon the parameters already negotiated for the pre-established session;

NOTE:	The SIP re-INVITE request can be sent within an on-demand session or a pre-established session. If the SIP re-INVITE request is sent within a pre-established session, the media-level section for the offered MCPTT speech media stream and the media-level section of the offered media-floor control entity are expected to be the same as was negotiated in the existing pre-established session.

7)	if an implicit floor request is required, shall indicate this as specified in subclause 6.4; and

8)	shall send the SIP re-INVITE request according to 3GPP TS 24.229 [4].

On receiving a SIP 2xx response to the SIP re-INVITE request the MCPTT client:

1)	shall interact with the user plane as specified in 3GPP TS 24.380 [5]; and

2)	shall perform the actions specified in subclause 6.2.8.1.4.

On receiving a SIP INFO request where the Request-URI contains an MCPTT session ID identifying an ongoing group session, the MCPTT client shall follow the actions specified in subclause 6.2.8.1.13.

On receiving a SIP 4xx response, SIP 5xx response or a SIP 6xx response to the SIP re-INVITE request the MCPTT client shall perform the actions specified in subclause 6.2.8.1.5.

###### 10.1.1.2.1.4	MCPTT in-progress emergency cancel

This subclause covers both on-demand session and pre-established sessions.

Upon receiving a request from an MCPTT user to cancel the in-progress emergency condition on a prearranged MCPTT group, the MCPTT client shall generate a SIP re-INVITE request by following the UE originating session procedures specified in 3GPP TS 24.229 [4], with the clarifications given below.

The MCPTT client:

1)	if the MCPTT user is not authorised to cancel the in-progress emergency group state of the MCPTT group as determined by the procedures of subclause 6.2.8.1.7, the MCPTT client:

a)	should indicate to the MCPTT user that they are not authorised to cancel the in-progress emergency group state of the MCPTT group; and

b)	shall skip the remaining steps of the current subclause;

2)	shall, if the MCPTT user is cancelling an in-progress emergency condition and optionally an MCPTT emergency alert originated by the MCPTT user, include an application/vnd.3gpp.mcptt-info+xml MIME body populated as specified in subclause 6.2.8.1.3;

3)	shall, if the MCPTT user is cancelling an in-progress emergency condition and an MCPTT emergency alert originated by another MCPTT user, include an application/vnd.3gpp.mcptt-info+xml MIME body populated as specified in subclause 6.2.8.1.14;

4)	shall include in the application/vnd.3gpp.mcptt-info+xml MIME body with the <mcpttinfo> element containing the <mcptt-Params> element with:

a)	the <session-type> element set to a value of "prearranged"; and

b)	the <mcptt-request-uri> element set to the group identity;

NOTE 1:	The MCPTT ID of the originating MCPTT user is not included in the body, as this will be inserted into the body of the SIP INVITE request that is sent by the originating participating MCPTT function.

5)	shall include the g.3gpp.mcptt media feature tag in the Contact header field of the SIP re-INVITE request according to IETF RFC 3840 [16];

6)	if the SIP re-INVITE request is to be sent within an on-demand session, shall include in the SIP re-INVITE request an SDP offer according to 3GPP TS 24.229 [4] with the clarifications specified in subclause 6.2.1;

7)	if the SIP re-INVITE request is to be sent within a pre-established session, shall include an SDP offer in the SIP re-INVITE request according to 3GPP TS 24.229 [4], based upon the parameters already negotiated for the pre-established session;

NOTE 2:	The SIP re-INVITE request can be sent within an on-demand session or a pre-established session. If the SIP re-INVITE request is sent within a pre-established session, the media-level section for the offered MCPTT speech media stream and the media-level section of the offered media-floor control entity are expected to be the same as was negotiated in the existing pre-established session.

8)	shall include a Resource-Priority header field and comply with the procedures in subclause 6.2.8.1.2; and

9)	shall send the SIP re-INVITE request according to 3GPP TS 24.229 [4].

On receiving a SIP 2xx response to the SIP re-INVITE request, the MCPTT client:

1)	shall interact with the user plane as specified in 3GPP TS 24.380 [5];

2)	shall set the MCPTT emergency group state of the group to "MEG 1: no-emergency";

3)	shall set the MCPTT emergency group call state of the group to "MEGC 1: emergency-gc-capable"; and

4)	if the MCPTT emergency alert state is set to "MEA 4: Emergency-alert-cancel-pending", the sent SIP re-INVITE request did not contain an <originated-by> element in the application/vnd.3gpp.mcptt-info+xml MIME body and the SIP 2xx response to the SIP request for a priority group call does not contain a Warning header field as specified in subclause 4.4 with the warning text containing the mcptt-warn-code set to "149", shall set the MCPTT emergency alert state to "MEA 1: no-alert".

On receiving a SIP INFO request where the Request-URI contains an MCPTT session ID identifying an ongoing group session, the MCPTT client shall follow the actions specified in subclause 6.2.8.1.13.

On receiving a SIP 4xx response, SIP 5xx response or SIP 6xx response to the SIP re-INVITE request:

1)	if the SIP 4xx response, SIP 5xx response or SIP 6xx response contains an application/vnd.3gpp.mcptt-info+xml MIME body with an <emergency-ind> element set to a value of "true", the MCPTT client shall set the MCPTT emergency group state as "MEG 2: in-progress";

2)	if the SIP 4xx response, SIP 5xx response or SIP 6xx response contains an application/vnd.3gpp.mcptt-info+xml MIME body with an <alert-ind> element set to a value of "false" and the sent SIP re-INVITE request did not contain an <originated-by> element in the application/vnd.3gpp.mcptt-info+xml MIME body, the MCPTT client shall set the MCPTT emergency alert state to "MEA 1: no-alert";

NOTE 3:	This is the case where the MCPTT client requested the cancellation of the MCPTT emergency alert in addition to the MCPTT emergency alert and the cancellation of the MCPTT emergency alert was successful.

3)	if the SIP 4xx response, SIP 5xx response or SIP 6xx response contains an application/vnd.3gpp.mcptt-info+xml MIME body with an <alert-ind> element set to a value of "true" and the sent SIP re-INVITE request did not contain an <originated-by> element in the application/vnd.3gpp.mcptt-info+xml MIME body, the MCPTT client shall set the MCPTT emergency alert state to "MEA 3: emergency-alert-initiated"; and

4)	if the SIP 4xx response, SIP 5xx response or SIP 6xx response did not contain an application/vnd.3gpp.mcptt-info+xml MIME body with an <alert-ind> element and did not contain an <originated-by> element, the MCPTT emergency alert (MEA) state shall revert to its value prior to entering the current procedure.

NOTE 4:	This is the case where the MCPTT client requested the cancellation of the MCPTT emergency alert in addition to the MCPTT emergency alert and the cancellation of the MCPTT emergency alert was not successful.

NOTE 5:	If the in-progress emergency group state cancel request is rejected, the state of the session does not change, i.e. continues with MCPTT emergency group call level priority.

###### 10.1.1.2.1.5	MCPTT in-progress imminent peril cancel

This subclause covers both on-demand session and pre-established sessions.

Upon receiving a request from an MCPTT user to cancel the in-progress imminent peril condition on a prearranged MCPTT group, the MCPTT client shall generate a SIP re-INVITE request by following the procedures specified in 3GPP TS 24.229 [4], with the clarifications given below.

The MCPTT client:

1)	if the MCPTT user is not authorised to cancel the in-progress imminent peril group state of the MCPTT group as determined by the procedures of subclause 6.2.8.1.10, the MCPTT client:

a)	should indicate to the MCPTT user that they are not authorised to cancel the in-progress imminent peril group state of the MCPTT group; and

b)	shall skip the remaining steps of the current subclause;

2)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body populated as specified in subclause 6.2.8.1.11; and

3)	shall include a Resource-Priority header field and comply with the procedures in subclause 6.2.8.1.12;

4)	shall include in the application/vnd.3gpp.mcptt-info+xml MIME body with the <mcpttinfo> element containing the <mcptt-Params> element with:

a)	the <session-type> element set to a value of "prearranged"; and

b)	the <mcptt-request-uri> element set to the group identity;

NOTE 1:	The MCPTT ID of the originating MCPTT user is not included in the body, as this will be inserted into the body of the SIP re-INVITE request that is sent by the originating participating MCPTT function.

5)	shall include the g.3gpp.mcptt media feature tag in the Contact header field of the SIP re-INVITE request according to IETF RFC 3840 [16];

6)	if the SIP re-INVITE request is to be sent within an on-demand session, shall include in the SIP re-INVITE request an SDP offer according to 3GPP TS 24.229 [4] with the clarifications specified in subclause 6.2.1;

7)	if the SIP re-INVITE request is to be sent within a pre-established session, shall include an SDP offer in the SIP re-INVITE request according to 3GPP TS 24.229 [4], based upon the parameters already negotiated for the pre-established session; and

NOTE 2:	The SIP re-INVITE request can be sent within an on-demand session or a pre-established session. If the SIP re-INVITE request is sent within a pre-established session, the media-level section for the offered MCPTT speech media stream and the media-level section of the offered media-floor control entity are expected to be the same as was negotiated in the existing pre-established session.

8)	shall send the SIP re-INVITE request according to 3GPP TS 24.229 [4].

On receiving a SIP 2xx response to the SIP re-INVITE request, the MCPTT client:

1)	shall interact with the user plane as specified in 3GPP TS 24.380 [5];

2)	shall set the MCPTT imminent peril group state of the group to "MIG 1: no-imminent-peril"; and

3)	shall set the MCPTT imminent peril group call state of the group to "MIGC 1: imminent-peril-gc-capable".

On receiving a SIP 4xx, SIP 5xx response or SIP 6xx response to the SIP re-INVITE request:

1)	if the SIP 4xx response, SIP 5xx response or SIP 6xx response:

a)	contains an application/vnd.3gpp.mcptt-info+xml MIME body with an <imminentperil-ind> element set to a value of "true"; or

b)	does not contain an application/vnd.3gpp.mcptt-info+xml MIME body with an <imminentperil-ind> element;

then the MCPTT client shall set the MCPTT imminent peril group state as "MIG 2: in-progress".

NOTE 3:	This is the case where the MCPTT client requested the cancellation of the MCPTT imminent peril in-progress state and was rejected.

###### 10.1.1.2.1.6	MCPTT client receives SIP re-INVITE request

This subclause covers both on-demand session and pre-established sessions.

Upon receipt of a SIP re-INVITE request the MCPTT client:

1)	if the SIP re-INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcpttinfo> element containing the <mcptt-Params> element with the <emergency-ind> element set to a value of "true":

a)	should display to the MCPTT user the MCPTT ID of the originator of the MCPTT emergency group call and an indication that this is an MCPTT emergency group call;

b)	if the <mcpttinfo> element containing the <mcptt-Params> element contains an <alert-ind> element set to "true", should display to the MCPTT user an indication of the MCPTT emergency alert and associated information;

c)	shall set the MCPTT emergency group state to "MEG 2: in-progress"; and

d)	shall set the MCPTT imminent peril group state to "MIG 1: no-imminent-peril";

2)	if the SIP re-INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcpttinfo> element containing the <mcptt-Params> element with the <imminentperil-ind> element set to a value of "true":

a)	should display to the MCPTT user the MCPTT ID of the originator of the MCPTT imminent peril group call and an indication that this is an MCPTT imminent peril group call; and

b)	shall set the MCPTT imminent peril group state to "MIG 2: in-progress";

3)	if the SIP re-INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcpttinfo> element containing the <mcptt-Params> element with the <emergency-ind> element set to a value of "false":

a)	should display to the MCPTT user the MCPTT ID of the MCPTT user cancelling the MCPTT emergency group call;

b)	if the <mcpttinfo> element containing the <mcptt-Params> element contains an <alert-ind> element set to "false":

i)	should display to the MCPTT user an indication of the MCPTT emergency alert cancellation and the MCPTT ID of the MCPTT user cancelling the MCPTT emergency alert; and

ii)	if the SIP re-INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body including an <originated-by> element:

A)	should display to the MCPTT user the MCPTT ID contained in the <originated-by> element of the MCPTT user that originated the MCPTT emergency alert; and

B)	if the MCPTT ID contained in the <originated-by> element is the MCPTT ID of the receiving MCPTT user shall set the MCPTT emergency alert state to "MEA 1: no-alert";

c)	shall set the MCPTT emergency group state to "MEG 1: no-emergency"; and

d)	if the MCPTT emergency group call state of the group is set to "MEGC 3: emergency-call-granted", shall set the MCPTT emergency group call state of the group to "MEGC 1: emergency-gc-capable";

4)	if the SIP re-INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcpttinfo> element containing the <mcptt-Params> element with the <imminentperil-ind> element set to a value of "false":

a)	should display to the MCPTT user the MCPTT ID of the MCPTT user cancelling the MCPTT imminent peril group call and an indication that this is an MCPTT imminent peril group call; and

b)	shall set the MCPTT imminent peril group state to "MIG 1: no-imminent-peril";

5)	shall check if a Resource-Priority header field is included in the incoming SIP re-INVITE request and may perform further actions outside the scope of this specification to act upon an included Resource-Priority header field as specified in 3GPP TS 24.229 [4];

6)	shall accept the SIP re-INVITE request and generate a SIP 200 (OK) response according to rules and procedures of 3GPP TS 24.229 [4];

7)	shall include the g.3gpp.mcptt media feature tag in the Contact header field of the SIP 200 (OK) response;

8)	shall include the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" in the Contact header field of the SIP 200 (OK) response;

9)	if the SIP re-INVITE request was received within an on-demand session, shall include an SDP answer in the SIP 200 (OK) response to the SDP offer in the incoming SIP re-INVITE request according to 3GPP TS 24.229 [4] with the clarifications given in subclause 6.2.2;

10)	if the SIP re-INVITE request was received within a pre-established session, shall include an SDP answer in the SIP 200 (OK) response to the SDP offer in the incoming SIP re-INVITE request according to 3GPP TS 24.229 [4], based upon the parameters already negotiated for the pre-established session;

NOTE:	The SIP re-INVITE request can be received within an on-demand session or a pre-established session. If the SIP re-INVITE request is received within a pre-established session, the media-level section for the MCPTT speech media stream and the media-level section of the media-floor control entity are expected to be the same as was negotiated in the existing pre-established session.

11)	shall send the SIP 200 (OK) response towards the MCPTT server according to rules and procedures of 3GPP TS 24.229 [4]; and

12)	shall interact with the media plane as specified in 3GPP TS 24.380 [5].

##### 10.1.1.2.2	Prearranged group call using pre-established session

###### 10.1.1.2.2.1	Client originating procedures

Upon receiving a request from an MCPTT user to establish an MCPTT group session using an MCPTT group identity identifying a prearranged MCPTT group within the pre-established session, the MCPTT client shall generate a SIP REFER request as specified in IETF RFC 3515 [25] as updated by IETF RFC 6665 [26] and IETF RFC 7647 [27], and in accordance with the UE procedures specified in 3GPP TS 24.229 [4], with the clarifications given below.

The MCPTT client shall follow the procedures specified in subclause 10.1.2.2.2.1 with the clarification in step 2) of subclause 10.1.2.2.2.1 that:

1)	the <entry> element in the application/resource-lists MIME body shall contain a "uri" attribute set to the prearranged MCPTT group identity;

2)	the <session-type> element of the application/vnd.3gpp.mcptt-info MIME body in the hname "body" URI header field shall be set to a value of "prearranged"; and

3)	if the MCPTT user has requested the origination of a broadcast group call, the MCPTT client shall comply with the procedures in subclause 6.2.8.2.

###### 10.1.1.2.2.2	Client terminating procedures

Upon receiving a SIP re-INVITE request within a pre-established Session without an associated MCPTT session or when generating SIP responses to the SIP re-INVITE request, the MCPTT client shall follow the procedures in subclause 10.1.1.2.1.2.

NOTE:	In subclause 10.1.1.2.1.2, the reader is assumed to replace occurrences of SIP INVITE request with SIP re-INVITE request.

##### 10.1.1.2.3	End group call

###### 10.1.1.2.3.1	Client originating procedures on-demand

When an MCPTT client wants to leave the MCPTT session that has been established using on-demand session, the MCPTT client shall follow the procedures as specified in subclause 6.2.4.1.

###### 10.1.1.2.3.2	Client originating procedures using pre-established session

When an MCPTT client wants to leave the MCPTT session within a pre-established session, the MCPTT client shall follow the procedures as specified in subclause 6.2.4.2.

###### 10.1.1.2.3.3	Client terminating procedures

Upon receiving a SIP BYE request for releasing the prearranged MCPTT group call, the MCPTT client shall follow the procedures as specified in subclause 6.2.6.

##### 10.1.1.2.4	Re-join procedure

###### 10.1.1.2.4.1	On demand session establishment

Upon receiving a request from an MCPTT user to re-join an ongoing MCPTT session or triggered by coming back from out of coverage, the MCPTT client shall generate an initial SIP INVITE request by following the UE originating session procedures specified in 3GPP TS 24.229 [4], with the clarifications given below.

NOTE:	How an MCPTT client is informed whether it comes back from out of coverage is out of scope of present document.

The MCPTT client shall follow the procedures specified in subclause 10.1.1.2.1.1 with the clarification in step 9) of subclause 10.1.1.2.1.1 that the Request-URI of the SIP INVITE request shall contain a URI of the MCPTT session identity to re-join.

###### 10.1.1.2.4.2	Pre-established session

Upon receiving a request from an MCPTT user to re-join an ongoing MCPTT session within the pre-established session or triggered by coming back from out of coverage, the MCPTT client shall generate a SIP REFER request as specified in IETF RFC 3515 [25] as updated by IETF RFC 6665 [26] and IETF RFC 7647 [27], and in accordance with the UE procedures specified in 3GPP TS 24.229 [4], with the clarifications given below.

The MCPTT client shall follow the procedures specified in subclause 10.1.1.2.2.1 with the clarification in step 1) of subclause 10.1.1.2.2.1 that the Refer-To header field of the SIP REFER request:

1)	shall contain a URI of the MCPTT session identity to re-join; and

2)	shall contain a Content-Type URI header field containing an application/vnd.3gpp.mcptt-info+xml MIME type of the "body" URI header field and the body URI header field containing the <mcptt-info> element with the <mcptt-Params> element and with the <session-type> element set to a value of "prearranged".

#### 10.1.1.3	Participating MCPTT function procedures

##### 10.1.1.3.1	Originating procedures

###### 10.1.1.3.1.1	On demand prearranged group call

In the procedures in this subclause:

1)	group identity in an incoming SIP INVITE request refers to the group identity from the <mcptt-request-uri> element of the application/vnd.3gpp.mcptt-info+xml MIME body of the incoming SIP INVITE request;

2)	emergency indication in an incoming SIP INVITE request refers to the <emergency-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body; and

3)	imminent peril indication in an incoming SIP INVITE request refers to the <imminentperil-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body.

Upon receipt of a "SIP INVITE request for originating participating MCPTT function" containing an application/vnd.3gpp.mcptt-info+xml MIME body with the <session-type> element set to a value of "prearranged", the participating MCPTT function:

1)	if unable to process the request due to a lack of resources or a risk of congestion exists, may reject the SIP INVITE request with a SIP 500 (Server Internal Error) response. The participating MCPTT function may include a Retry-After header field to the SIP 500 (Server Internal Error) response as specified in IETF RFC 3261 [24]. Otherwise, continue with the rest of the steps;

NOTE 1:	if the SIP INVITE request contains an emergency indication or an imminent peril indication set to a value of "true" and this is an authorised request for originating a priority call as determined by subclause 6.3.2.1.8.1, the participating MCPTT function can according to local policy choose to accept the request.

2)	shall determine the MCPTT ID of the calling user from public user identity in the P-Asserted-Identity header field of the SIP INVITE request, and shall authorise the calling user;

NOTE 2:	The MCPTT ID of the calling user is bound to the public user identity at the time of service authorisation, as documented in subclause 7.3.

3)	if through local policy in the participating MCPTT function, the user identified by the MCPTT ID is not authorised to initiate prearranged group calls, shall reject the "SIP INVITE request for originating participating MCPTT function" with a SIP 403 (Forbidden) response to the SIP INVITE request, with warning text set to "109 user not authorised to make prearranged group calls" in a Warning header field as specified in subclause 4.4;

4)	if the user identified by the MCPTT ID already has N2 simultaneous affiliations and the user is not affiliated to the group identified in the "SIP INVITE request for originating participating MCPTT function", shall reject the "SIP INVITE request for originating participating MCPTT function" with a SIP 486 (Busy Here) response with warning text set to "102 too many simultaneous affiliations" in a Warning header field as specified in subclause 4.4;

NOTE 3:	N2 is the total number of MCPTT groups that an MCPTT User can be affiliated to simultaneously.

NOTE 4:	if the SIP INVITE request contains an emergency indication or an imminent peril indication, the participating MCPTT function can by means beyond the scope of this specification choose to allow an exception to the N2 limit. Alternatively, a lower priority affiliation of the MCPTT user could be cancelled to allow for the new affiliation.

5).	shall validate the media parameters and if the MCPTT speech codec is not offered in the SIP INVITE request shall reject the request with a SIP 488 (Not Acceptable Here) response. Otherwise, continue with the rest of the steps;

6)	shall check if the number of maximum simultaneous MCPTT group calls supported for the MCPTT user as specified in the <MaxSimultaneousCallsN6> element of the <MCPTT-group-call> element of the MCPTT user profile document (see the MCPTT user profile document in 3GPP TS 24.384 [50]) has been exceeded. If exceeded, the participating MCPTT function shall respond with a SIP 486 (Busy Here) response with the warning text set to "103 maximum simultaneous MCPTT group calls reached" in a Warning header field as specified in subclause 4.4. Otherwise, continue with the rest of the steps;

NOTE 5:	If the SIP INVITE request contains an emergency indication or an imminent peril indication, the participating MCPTT function can by means beyond the scope of this specification choose to allow for an exception to the limit for the maximum simultaneous MCPTT sessions supported for the MCPTT user. Alternatively, a lower priority session of the MCPTT user could be terminated to allow for the new session.

7)	shall determine the public service identity of the controlling MCPTT function associated with the group identity in the SIP INVITE request;

NOTE 6:	The public service identity can identify the controlling function in the primary MCPTT system or a partner MCPTT system.

NOTE 7:	How the participating MCPTT server discovers the public service identity of the controlling MCPTT function associated with the group identity is out of scope of the current release.

8)	shall generate a SIP INVITE request as specified in subclause 6.3.2.1.3;

9)	shall set the Request-URI to the public service identity of the controlling MCPTT function associated with the group identity which was present in the incoming SIP INVITE request;

10)	shall not copy the following header fields from the incoming SIP INVITE request to the outgoing SIP INVITE request, if they were present in the incoming SIP INVITE request:

a)	Answer-Mode header field as specified in IETF RFC 5373 [18]; and

b)	Priv-Answer-Mode header field as specified in IETF RFC 5373 [18];

11)	shall set the <mcptt-calling-user-identity> element of the application/vnd.3gpp.mcptt-info+xml MIME body of the SIP INVITE request to the MCPTT ID of the calling user;

12)	shall include in the SIP INVITE request an SDP offer based on the SDP offer in the received SIP INVITE request from the MCPTT client as specified in subclause 6.3.2.1.1.1;

13)	if the received SIP INVITE request contains an application/vnd.3gpp.location-info+xml MIME body as specified in clause F.3 and if not already copied, shall copy the contents of the application/vnd.3gpp.location-info+xml MIME body received in the SIP INVITE request into an application/vnd.3gpp.location-info+xml MIME body included in the outgoing SIP request;

14)	if a Resource-Priority header field was included in the received SIP INVITE request, shall include a Resource-Priority header field according to rules and procedures of 3GPP TS 24.229 [4] set to the value indicated in the Resource-Priority header field of the SIP INVITE request from the MCPTT client; and

NOTE 8:	The participating MCPTT function will leave verification of the Resource-Priority header field to the controlling MCPTT function.

15)	shall forward the SIP INVITE request, according to 3GPP TS 24.229 [4].

Upon receipt of a SIP 302 (Moved Temporarily) response to the above SIP INVITE request in step 15), the participating MCPTT function:

1)	shall generate a SIP INVITE request as specified in subclause 6.3.2.1.10;

2)	shall include an SDP offer based upon the SDP offer in the received SIP INVITE request from the MCPTT client as specified in subclause 6.3.2.1.1.1; and

3)	shall forward the SIP INVITE request according to 3GPP TS 24.229 [4].

Upon receipt of a SIP 2xx response in response to the above SIP INVITE request in step 15), the participating MCPTT function:

1)	shall generate a SIP 200 (OK) response as in subclause 6.3.2.1.5.2;

2)	shall include in the SIP 200 (OK) response an SDP answer as specified in the subclause 6.3.2.1.2.1;

3)	shall include Warning header field(s) that were received in the incoming SIP 200 (OK) response;

4)	shall include the public service identity received in the P-Asserted-Identity header field of the incoming SIP 200 (OK) response into the P-Asserted-Identity header field of the outgoing SIP 200 (OK) response;

5)	shall include an MCPTT session identity mapped to the MCPTT session identity provided in the Contact header field of the received SIP 200 (OK) response;

6)	shall send the SIP 200 (OK) response to the MCPTT client according to 3GPP TS 24.229 [4];

7)	shall interact with Media Plane as specified in 3GPP TS 24.380 [5]; and

8)	shall start the SIP Session timer according to rules and procedures of IETF RFC 4028 [7].

###### 10.1.1.3.1.2	Prearranged group call using pre-established session

Upon receipt of a "SIP REFER request for a pre-established session", with:

1)	the Refer-To header field containing a Content-ID ("cid") Uniform Resource Locator (URL) as specified in IETF RFC 2392 [62] that points to an application/resource-lists MIME body as specified in IETF RFC 5366 [20] containing an <entry> element with a "uri" attribute containing a SIP-URI set to a pre-arranged group identity;

2)	a body" URI header field of the SIP-URI specified above containing an application/vnd.3gpp.mcptt-info MIME body with the <session-type> element set to "prearranged"; and

3)	a Content-ID header field set to the "cid" URL;

the participating MCPTT function:

1)	if unable to process the request due to a lack of resources or a risk of congestion exists, may reject the SIP INVITE request with a SIP 500 (Server Internal Error) response. The participating MCPTT function may include a Retry-After header field to the SIP 500 (Server Internal Error) response as specified in IETF RFC 3261 [24] and shall not continue with the rest of the steps;

Editor's Note [CT1#95, C1-154715]: The priority mechanisms are still to be specified for MCPTT. The priority mechanisms may be a factor in determining whether to reject or allow the request.

2)	shall check if the number of maximum simultaneous MCPTT group calls supported for the MCPTT user as specified in the <MaxSimultaneousCallsN6> element of the <MCPTT-group-call> element of the MCPTT user profile document (see the MCPTT user profile document in 3GPP TS 24.384 [50]) has been exceeded. If exceeded, the participating MCPTT function shall respond with a SIP 486 (Busy Here) response with the warning text set to "103 maximum simultaneous MCPTT group calls reached" in a Warning header field as specified in subclause 4.4 and shall not continue with the rest of the steps;

3)	shall determine the MCPTT ID of the calling user from public user identity in the P-Asserted-Identity header field of the SIP REFER request;

NOTE 1:	The MCPTT ID of the calling user is bound to the public user identity at the time of service authorisation, as documented in subclause 7.3.

4)	if the participating MCPTT function cannot find a binding between the public user identity and an MCPTT ID or if the validity period of an existing binding has expired, then the participating MCPTT function shall reject the SIP REFER request with a SIP 404 (Not Found) response with the warning text set to "141 user unknown to the participating function" in a Warning header field as specified in subclause 4.4, and shall not continue with any of the remaining steps;

5)	if through local policy in the participating MCPTT function, the user identified by the MCPTT ID is not authorised to initiate prearranged group calls, shall reject the "SIP REFER request for pre-established session" with a SIP 403 (Forbidden) response to the SIP INVITE request, with warning text set to "109 user not authorised to make prearranged group calls" in a Warning header field as specified in subclause 4.4;

6)	if the "SIP REFER request for a pre-established session" contained a Refer-Sub header field containing "false" value and a Supported header field containing "norefersub" value, shall handle the SIP REFER request as specified in 3GPP TS 24.229 [4], IETF RFC 3515 [25] as updated by IETF RFC 6665 [26], and IETF RFC 4488 [22] without establishing an implicit subscription;

7)	shall generate a final SIP 200 (OK) response to the "SIP REFER request for a pre-established session" according to 3GPP TS 24.229 [4];

NOTE 2:	In accordance with IETF RFC 4488 [22], the participating MCPTT function inserts the Refer-Sub header field containing the value "false" in the SIP 200 (OK) response to the SIP REFER request to indicate that it has not created an implicit subscription.

8)	shall send the response to the "SIP REFER request for a pre-established session" towards the MCPTT client according to 3GPP TS 24.229 [4];

9)	shall retrieve the group identity within the <entry> element of the application/resource-lists MIME body, referenced by the "cid" URL contained in the Refer-To header field of the SIP REFER request;

10)	shall determine the public service identity of the controlling MCPTT function associated with the group identity in the application/resource-lists MIME body referenced by the Refer-To header of the SIP REFER request. If the participating MCPTT function is unable to identify the controlling MCPTT function associated with the group identity, it shall reject the REFER request with a SIP 404 (Not Found) response with the warning text "142 unable to determine the controlling function" in a Warning header field as specified in subclause 4.4, and shall not continue with any of the remaining steps;

NOTE 3:	The public service identity can identify the controlling function in the primary MCPTT system or a partner MCPTT system.

NOTE 4:	How the participating MCPTT server discovers the public service identity of the controlling MCPTT function associated with the group identity is out of scope of the current document.

11)	shall generate a SIP INVITE request as specified in subclause 6.3.2.1.4;

12)	shall set the Request-URI of the SIP INVITE request to the public service identity of the controlling MCPTT function associated with the group identity;

13)	shall copy the group identity from the "uri" attribute of the <entry> element of the application/resource-lists MIME body pointed to by the "cid" URL in the Refer-to header field of the SIP REFER request, to the <mcptt-request-uri> element of the application/vnd.3gpp.mcptt-info+xml MIME body in the SIP INVITE request; and

14)	shall forward the SIP INVITE request according to 3GPP TS 24.229 [4].

Upon receiving SIP provisional responses for the SIP INVITE request the participating MCPTT function:

1)	shall discard the received SIP responses without forwarding them.

Upon receipt of a SIP 302 (Moved Temporarily) response to the SIP INVITE request the participating MCPTT function:

1)	shall generate a SIP INVITE request as specified in subclause 6.3.2.1.10;

2)	shall include in the SIP INVITE request an SDP offer based upon the SDP offer negotiated during the pre-established session establishment, any subsequent pre-established session modification and the SDP offer (if any) included in the"body" URI parameter of the SIP-URI contained in the <entry> element of the application/resource-lists MIME body, referenced by the "cid" URL in the Refer-To header field in the incoming SIP REFER request from the MCPTT client; and

3)	shall forward the SIP INVITE request according to 3GPP TS 24.229 [4].

Upon receiving a SIP 200 (OK) response for the SIP INVITE request the participating MCPTT function:

1)	shall interact with the media plane as specified in 3GPP TS 24.380 [5].

###### 10.1.1.3.1.3	Reception of a SIP re-INVITE request from served MCPTT client

This subclause covers both on-demand session and pre-established sessions.

Upon receipt of a SIP re-INVITE request for an MCPTT session identifying an on-demand prearranged MCPTT group session, the participating MCPTT function:

1)	if unable to process the request due to a lack of resources or a risk of congestion exists, may reject the SIP re-INVITE request with a SIP 500 (Server Internal Error) response. The participating MCPTT function may include a Retry-After header field to the SIP 500 (Server Internal Error) response as specified in IETF RFC 3261 [24] and skip the rest of the steps;

NOTE 1:	If the SIP re-INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <emergency-ind> element set to a value of "true", the participating MCPTT function can choose to accept the request.

2)	shall determine if the media parameters are acceptable and the MCPTT speech codec is offered in the SDP offer and if not, reject the request with a SIP 488 (Not Acceptable Here) response. Otherwise, continue with the rest of the steps;

NOTE 2:	If the received SIP re-INVITE request is received within a pre-established session associated with an MCPTT group session, the media-level section for the offered MCPTT speech media stream and the media-level section of the offered media-floor control entity are expected to be the same as was negotiated in the existing pre-established session.

3)	shall generate an outgoing SIP re-INVITE request as specified in subclause 6.3.2.1.9;

4)	shall, if the SIP re-INVITE request was received within an on-demand session, include in the SIP re-INVITE request an SDP offer based on the SDP offer in the received SIP re-INVITE request as specified in subclause 6.3.2.1.1;

5)	shall, if the SIP re-INVITE request was received within a pre-established session, include in the SIP re-INVITE request an SDP offer based upon the previously negotiated SDP for the pre-established session as specified in subclause 6.3.2.1.1.2;

6)	if the received SIP re-INVITE request contains a Resource-Priority header field, shall include a Resource-Priority header field with the contents set as in the received Resource-Priority header field; and

NOTE 3: The controlling MCPTT function will determine the validity of the Resource-Priority header field.

7)	shall forward the SIP re-INVITE request according to 3GPP TS 24.229 [4].

Upon receipt of a SIP 2xx response to the above SIP re-INVITE request in step 7) the participating MCPTT function:

1)	shall generate a SIP 200 (OK) response as specified in the subclause 6.3.2.1.5.2;

2)	shall include in the SIP 200 (OK) response an SDP answer as specified in the subclause 6.3.2.1.2.1;

3)	shall include Warning header field(s) that were received in the incoming SIP 200 (OK) response;

4)	shall copy the contents received in the P-Asserted-Identity header field of the incoming SIP 200 (OK) response into the P-Asserted-Identity header field of the outgoing SIP 200 (OK) response;

5)	shall send the SIP 200 (OK) response towards the MCPTT client according to 3GPP TS 24.229 [4]; and

6)	shall interact with the media plane as specified in 3GPP TS 24.380 [5].

Upon receipt of a SIP 403 (Forbidden) response to the above SIP INVITE request in step 7) the participating MCPTT function:

1)	shall generate a SIP 403 (Forbidden) response according to 3GPP TS 24.229 [4];

2)	shall copy, if included in the received SIP 403 (Forbidden) response, the application/vnd.3gpp.mcptt-info+xml MIME body MIME body to the outgoing SIP (Forbidden) response;

3)	shall include Warning header field(s) that were received in the incoming SIP 403 (Forbidden) response;

4)	shall forward the SIP 403 (Forbidden) response to the MCPTT client according to 3GPP TS 24.229 [4]; and

5)	shall interact with the media plane as specified in 3GPP TS 24.380 [5].

##### 10.1.1.3.2	Terminating Procedures

In the procedures in this subclause:

1)	emergency indication in an incoming SIP INVITE request refers to the <emergency-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body; and

2)	imminent peril indication in an incoming SIP INVITE request refers to the <imminentperil-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body.

NOTE 1:	This subclause covers both on-demand session and pre-established sessions.

Upon receipt of a "SIP INVITE request for terminating participating MCPTT function", the participating MCPTT function:

1)	if unable to process the request due to a lack of resources or a risk of congestion exists, may reject the SIP INVITE request with a SIP 500 (Server Internal Error) response. The participating MCPTT function may include a Retry-After header field to the SIP 500 (Server Internal Error) response as specified in IETF RFC 3261 [24], and shall not continue with the rest of the steps;

NOTE 2:	if the SIP INVITE request contains an emergency indication or an imminent peril indication set to a value of "true" and this is an authorised request for originating a priority call as determined by subclause 6.3.2.1.8.1, the participating MCPTT function can according to local policy choose to accept the request.

2)	shall check the presence of the isfocus media feature tag in the URI of the Contact header field and if it is not present then the participating MCPTT function shall reject the request with a SIP 403 (Forbidden) response with the warning text set to "104 isfocus not assigned" in a Warning header field as specified in subclause 4.4, and shall not continue with the rest of the steps;

3)	if the Answer-Mode Indication in the application/poc-settings+xml MIME body has not yet been received from the invited MCPTT client as defined in subclause 7.3.3 or subclause 7.3.4, shall reject the request with a SIP 480 (Temporarily Unavailable) response with the warning text set to "146 T-PF unable to determine the service settings for the called user" in a Warning header field as specified in subclause 4.4, and shall not continue with the rest of the steps;

4)	shall use the MCPTT ID present in the <mcptt-request-uri> element of the application/vnd.3gpp.mcptt-info+xml MIME body of the incoming SIP INVITE request to retrieve the binding between the MCPTT ID and public user identity;

5)	if the binding between the MCPTT ID and public user identity does not exist, then the participating MCPTT function shall reject the SIP INVITE request with a SIP 404 (Not Found) response. Otherwise, continue with the rest of the steps;

6)	shall perform the automatic commencement procedures specified in subclause 6.3.2.2.5.1 and according to IETF RFC 5373 [18] if the "SIP INVITE request for terminating participating MCPTT function" does not contain an Answer-Mode header field and the Answer-Mode Indication received in the application/poc-settings+xml MIME body received from the invited MCPTT client as per subclause 7.3.3 or subclause 7.3.4 is set to "auto-answer"; and

7)	shall perform the manual commencement procedures specified in subclause 6.3.2.2.6.1 and according to IETF RFC 5373 [18] if the "SIP INVITE request for terminating participating MCPTT function" does not contain an Answer-Mode header field and the Answer-Mode Indication received in the application/poc-settings+xml MIME body received from the invited MCPTT client as per subclause 7.3.3 or subclause 7.3.4 is set to "manual-answer".

##### 10.1.1.3.3	End group call at the originating participating MCPTT function

###### 10.1.1.3.3.1	Receipt of SIP BYE request for ending group call on-demand

Upon receiving from the MCPTT client a SIP BYE request the participating MCPTT function shall follow the procedures as specified in subclause 6.3.2.1.6.

###### 10.1.1.3.3.2	Receipt of SIP REFER "BYE" request for ending group call using pre-established session

Upon receiving from the MCPTT client a SIP REFER request when using a pre-established session with the method SIP-URI parameter set to value "BYE" in the URI in the Refer-To header field the participating MCPTT function shall follow the procedures as specified in subclause 6.3.2.1.7.

##### 10.1.1.3.4	End group call at the terminating participating MCPTT function

###### 10.1.1.3.4.1	Receipt of SIP BYE request for private call on-demand

Upon receiving a SIP BYE request from the controlling MCPTT function, the participating MCPTT function shall follow the procedures as specified in subclause 6.3.2.2.8.1.

###### 10.1.1.3.4.2	Receipt of SIP BYE request when ongoing pre-established session

Upon receiving a SIP BYE request from the controlling MCPTT function and if the MCPTT session id refers to an MCPTT user that has a pre-established session with the participating MCPTT function, the participating MCPTT function shall follow the procedures as specified in subclause 6.3.2.2.8.2.

##### 10.1.1.3.5	Re-join procedures

###### 10.1.1.3.5.1	Originating procedures - on demand prearranged group call

Upon receipt of a "SIP INVITE request for originating participating MCPTT function" containing an application/vnd.3gpp.mcptt-info+xml MIME body with the <session-type> element set to a value of "prearranged", the participating MCPTT function shall follow the procedures specified in subclause 10.1.1.3.1.1 with the clarification in step 9) of subclause 10.1.1.3.1.1 that the Request-URI of the SIP INVITE request shall contain a URI of the MCPTT session identity which mapped to the MCPTT session identity provided in Request-URI header field of the "SIP INVITE request for originating participating MCPTT function".

###### 10.1.1.3.5.2	Originating procedures - prearranged group call using pre-established session

Upon receipt of a "SIP REFER request for a pre-established session", with the Refer-To header containing an application/vnd.3gpp.mcptt-info+xml MIME type content in a "body" URI header field and with the <session-type> element set to "prearranged" the participating MCPTT function shall follow the procedures specified in subclause 10.1.1.3.1.2 with the clarification in step 9) of subclause 10.1.1.3.1.2 that the Request-URI of the SIP INVITE request shall contain a URI of the MCPTT session identity which mapped to the MCPTT session identity provided in the Refer-to header field of the "SIP REFER request for a pre-established session".

##### 10.1.1.3.6	Reception of a SIP re-INVITE request for terminating MCPTT client for priority call

In the procedures in this subclause:

1)	emergency indication in an incoming SIP INVITE request refers to the <emergency-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body; and

2)	imminent peril indication in an incoming SIP INVITE request refers to the <imminentperil-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body.

Upon receipt of a SIP re-INVITE request for a terminating MCPTT client of a MCPTT group containing an emergency indication or imminent peril indication, the participating MCPTT function:

1)	shall check if a Resource-Priority header field is included in the incoming SIP INVITE request and may perform further actions outside the scope of this specification to act upon an included Resource-Priority header field as specified in 3GPP TS 24.229 [4];

2)	shall generate an outgoing SIP re-INVITE request as specified in subclause 6.3.2.2.10;

3)	shall include in the SIP re-INVITE request an SDP offer based on the SDP offer in the received SIP re-INVITE request as specified in subclause 6.3.2.2.1; and

4)	shall send the SIP re-INVITE request towards the MCPTT client according to 3GPP TS 24.229 [4].

Upon receiving a SIP 200 (OK) response to the above SIP re-INVITE request sent to the MCPTT client, the participating MCPTT function:

1)	shall generate a SIP 200 (OK) response as described in the subclause 6.3.2.2.4.2;

2)	shall include in the SIP 200 (OK) response an SDP answer based on the SDP answer in the received SIP 200 (OK) response as specified in subclause 6.3.2.2.2.1;

3)	shall interact with the media plane as specified in 3GPP TS 24.380 [5]; and

4)	shall forward the SIP 200 (OK) response according to 3GPP TS 24.229 [4].

#### 10.1.1.4	Controlling MCPTT function procedures

##### 10.1.1.4.1	Originating Procedures

###### 10.1.1.4.1.1	INVITE targeted to an MCPTT client

This subclause describes the procedures for inviting an MCPTT user to an MCPTT session. The procedure is initiated by the controlling MCPTT function as the result of an action in subclause 10.1.1.4.2 or as the result of receiving a SIP 403 (Forbidden) response as described in this subclause.

The controlling MCPTT function:

1)	shall generate a SIP INVITE request as specified in subclause 6.3.3.1.2;

2)	shall set the Request-URI to the public service identity of the terminating participating MCPTT function associated to the MCPTT user to be invited.;

NOTE 1:	How the controlling MCPTT function finds the address of the terminating MCPTT participating function is out of the scope of the current release.

NOTE 2:	If the terminating MCPTT user is part of a partner MCPTT system, then the public service identity can identify an entry point in the partner network that is able to identify the terminating participating MCPTT function.

3)	shall set the P-Asserted-Identity header field to the public service identity of the controlling MCPTT function;

4)	shall include in the application/vnd.3gpp.mcptt-info+xml MIME body in the outgoing SIP INVITE request:

a)	the <mcptt-request-uri> element set to the MCPTT ID of the terminating user; and

b)	the <mcptt-calling-group-id> element set to the group identity;

NOTE 3:	The <mcptt-calling-user-id> is already included in the MIME body as a result of calling subclause 6.3.3.1.2 in step 1).

5)	shall include in the SIP INVITE request an SDP offer based on the SDP offer in the received SIP INVITE request from the originating network according to the procedures specified in subclause 6.3.3.1.1;

6)	if the in-progress emergency state of the group is set to a value of "true" the controlling MCPTT function:

a)	shall include a Resource-Priority header field populated with the values for an MCPTT emergency group call as specified in subclause 6.3.3.1.19;

b)	if the received SIP INVITE request contained an application/vnd.3gpp.mcptt-info+xml MIME body with the <emergency-ind> element set to a value of "true":

i)	shall include in the outgoing SIP INVITE request in the application/vnd.3gpp.mcptt-info+xml MIME body an <emergency-ind> element set to a value of "true"; and

ii)	if the <alert-ind> element is set to "true" in the received SIP INVITE request and the requesting MCPTT user and MCPTT group are authorised for the initiation of MCPTT emergency alerts as determined by the procedures of subclause 6.3.3.1.13.1, shall populate the application/vnd.3gpp.mcptt-info+xml MIME body and the application/vnd.3gpp.location-info+xml MIME body as specified in subclause 6.3.3.1.12. Otherwise, shall set the <alert-ind> element to a value of "false"; and

c)	if the in-progress imminent peril state of the group is set to a value of "true" shall include in the application/vnd.3gpp.mcptt-info+xml MIME body an <imminentperil-ind> element set to a value of "false";

7)	if the in-progress emergency state of the group is set to a value of "false" and the in-progress imminent peril state of the group is set to a value of "true", the controlling MCPTT function:

a)	shall include a Resource-Priority header field populated with the values for an MCPTT imminent peril group call as specified in subclause 6.3.3.1.19; and

b)	shall include in the application/vnd.3gpp.mcptt-info+xml MIME body with the <imminentperil-ind> element set to a value of "true"; and

8)	shall send the SIP INVITE request towards the terminating network in accordance with 3GPP TS 24.229 [4].

Upon receiving a SIP 183 (Session Progress) response containing a Require header field with the option tag "100rel" and containing a P-Answer-State header field with the value "Unconfirmed" in response to the SIP INVITE request the controlling MCPTT function:

1)	shall send a SIP PRACK request towards the MCPTT client according to 3GPP TS 24.229 [4].

Upon receiving a SIP 200 (OK) response for the SIP INVITE request the controlling MCPTT function:

1)	shall interact with the media plane as specified in 3GPP TS 24.380 [5] subclause 6.3;

2)	shall send a SIP NOTIFY request to all participants with a subscription to the conference event package as specified in subclause 10.1.3.4; and

3)	shall increment the local counter of the number of SIP 200 (OK) responses received from invited members, by 1.

NOTE 4:	The notifications above could be sent prior to the SIP 200 (OK) response being sent to the inviting MCPTT client. These notifications received by MCPTT clients that are group members do not mean that the group session will be successfully established.

NOTE 5:	The procedures executed by the controlling MCPTT function prior to sending a response to the inviting MCPTT client are specified in subclause 10.1.1.4.2.

###### 10.1.1.4.1.2	INVITE targeted to the non-controlling MCPTT function of an MCPTT group

The controlling MCPTT function:

1)	shall generate a SIP INVITE request as specified in subclause 6.3.3.1.2;

2)	shall set the Request-URI to the public service identity of the non-controlling MCPTT function serving the group identity of the MCPTT group owned by the partner MCPTT system;

3)	shall set the P-Asserted-Identity to the public service identity of the controlling MCPTT function;

4)	shall include in the application/vnd.3gpp.mcptt-info+xml MIME body in the outgoing SIP INVITE request:

a)	the <mcptt-request-uri> element set to the group identity of the MCPTT group hosted by the non-controlling MCPTT function in the partner MCPTT system; and

b)	the <mcptt-calling-group-id> element set to the group identity of the group served by the controlling MCPTT function;

5)	shall include the Recv-Info header field set to g.3gpp.mcptt-floor-request;

6)	shall include in the SIP INVITE request an SDP offer based on the SDP offer in the received SIP INVITE request from the originating network according to the procedures specified in subclause 6.3.3.1.1; and

7)	shall send the SIP INVITE request towards the partner MCPTT system in accordance with 3GPP TS 24.229 [4].

Upon receiving SIP 403 (Forbidden) response for the SIP INVITE request, if according to local policy and if:

1)	the response contains a Warning header field with the MCPTT warning code "128"; and

2)	the response contains a P-Refused-URI-List header field and an application/resource-lists+xml MIME body as specified in IETF RFC 5318 [36];

NOTE 1:	The application/resource-lists+xml MIME body contains MCPTT IDs identifying MCPTT users in a partner MCPTT system that needs to be invited to the prearranged group call in case of group regrouping using interrogating method as specified in 3GPP TS 23.179 [3] subclause 10.6.2.4.2.

then the controlling MCPTT function:

1)	shall check if the number of members of the MCPTT group exceeds the value contained in the <on-network-max-participant-count> element of the group document as specified in 3GPP TS 24.381 [31]. If exceeded, the controlling MCPTT function shall invite only <on-network-max-participant-count> members from the application/resource-lists+xml MIME body; and

NOTE 2:	The <on-network-max-participant-count> element indicates the maximum number of participants allowed in the prearranged group session It is operator policy that determines which participants in the application/resource-lists+xml MIME body are invited to the group call.

2)	shall invite MCPTT users as specified in this subclause using the list of MCPTT IDs in URI-List.

Upon receiving a SIP 200 (OK) response for the SIP INVITE request the controlling MCPTT function:

1)	shall interact with the media plane as specified in 3GPP TS 24.380 [5] subclause 6.3;

NOTE 3:	The procedures executed by the controlling MCPTT function prior to sending a response to the inviting MCPTT client are specified in subclause 10.1.1.4.2.

2)	if at least one of the invited MCPTT clients has subscribed to the conference package, shall subscribe to the conference event package in the non-controlling MCPTT function as specified in subclause 10.1.3.4.3; and

3)	if the 200 (OK) response includes the <floor-state> element set to "floor-taken", shall wait for a SIP INFO request containing a floor request from the non-controlling MCPTT function.

Upon receiving a SIP INFO request containing a floor request where:

1)	the Request-URI contains an MCPTT session ID identifying an ongoing temporary group session; and

2)	the application/vnd.3gpp.mcptt-info+xml MIME body contains the <mcptt-calling-group-id> element with the MCPTT group ID of a MCPTT group invited to the temporary group session;

then the controlling MCPTT function:

1)	shall send a SIP 200 (OK) response to the SIP INFO request to the non-controlling MCPTT function as specified in 3GPP TS 24.229 [4]; and

2)	shall interact with the media plane as specified in 3GPP TS 24.380 [5] subclause 6.3.

##### 10.1.1.4.2	Terminating Procedures

In the procedures in this subclause:

1)	MCPTT ID in an incoming SIP INVITE request refers to the MCPTT ID of the originating user from the <mcptt-calling-user-id> element of the application/vnd.3gpp.mcptt-info+xml MIME body of the incoming SIP INVITE request;

2)	group identity in an incoming SIP INVITE request refers to the group identity from the <mcptt-request-uri> element of the application/vnd.3gpp.mcptt-info+xml MIME body of the incoming SIP INVITE request;

3)	MCPTT ID in an outgoing SIP INVITE request refers to the MCPTT ID of the called user in the <mcptt-request-uri> element of the application/vnd.3gpp.mcptt-info+xml MIME body of the outgoing SIP INVITE request;

4)	MCPTT ID in an incoming SIP 200 (OK) response refers to the MCPTT ID of the contacted user in the <mcptt-called-party-id> element of the application/vnd.3gpp.mcptt-info+xml MIME body of the incoming SIP 200 (OK) response;

5)	indication of required group members in a SIP 183 (Session Progress) response refers to the <required> element of the application/vnd.3gpp.mcptt-info+xml MIME body set to "true" in a SIP 183 (Session Progress) sent by the non-controlling MCPTT function of an MCPTT group;

6)	emergency indication in an incoming SIP INVITE request refers to the <emergency-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body; and

7)	imminent peril indication in an incoming SIP INVITE request refers to the <imminentperil-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body.

Upon receipt of a "SIP INVITE request for controlling MCPTT function of an MCPTT group", the controlling MCPTT function:

1)	if unable to process the request due to a lack of resources or a risk of congestion exists, may reject the SIP INVITE request with a SIP 500 (Server Internal Error) response. The controlling MCPTT function may include a Retry-After header field to the SIP 500 (Server Internal Error) response as specified in IETF RFC 3261 [24] and skip the rest of the steps;

NOTE 1:	if the SIP INVITE request contains an emergency indication or an imminent peril indication set to a value of "true" and this is an authorised request for originating an MCPTT emergency group call as determined by subclause 6.3.3.1.13.2, or for originating an MCPTT imminent peril group call as determined by subclause 6.3.3.1.13.5, the controlling MCPTT function can according to local policy choose to accept the request.

2)	shall determine if the media parameters are acceptable and the MCPTT speech codec is offered in the SDP offer and if not reject the request with a SIP 488 (Not Acceptable Here) response and skip the rest of the steps;

3)	shall reject the SIP request with a SIP 403 (Forbidden) response and not process the remaining steps if:

a)	an Accept-Contact header field does not include the g.3gpp.mcptt media feature tag; or

b)	an Accept-Contact header field does not include the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt";

4)	if received SIP INVITE request includes an application/vnd.3gpp.mcptt-info+xml MIME body with an <emergency-ind> element included or an <imminent-peril> element included, shall validate the request as described in subclause 6.3.3.1.17;

5)	shall retrieve the necessary group document(s) from the group management server for the group identity contained in the SIP INVITE request and carry out initial processing as specified in subclause 6.3.5.2;

6)	if the result of the initial processing in subclause 6.3.5.2 was:

a)	that authorization of the MCPTT ID is required at a non-controlling MCPTT function of a MCPTT group is required, perform the actions in subclause 6.3.3.1.13.7 and do not continue with the rest of the steps in this subclause; and

b)	that a SIP 3xx, 4xx, 5xx or 6xx response to the "SIP INVITE request for controlling MCPTT function of an MCPTT group" has been sent, do not continue with the rest of the steps in this subclause;

7)	shall perform the actions as described in subclause 6.3.3.2.2;

8)	shall maintain a local counter of the number of SIP 200 (OK) responses received from invited members and shall initialise this local counter to zero;

9)	shall determine if an MCPTT group call for the group identity is already ongoing by determining if an MCPTT session identity has already been allocated for the group call and the MCPTT session is active;

10)	if the SIP INVITE request contains an unauthorised request for an MCPTT emergency group call as determined by subclause 6.3.3.1.13.2 and:

a)	if the SIP INVITE request contains an alert indication set to "true" and this is an authorised request for an MCPTT emergency alert meeting the conditions specified in subclause 6.3.3.1.13.1, shall:

i)	cache the information that this MCPTT user has initiated an MCPTT emergency alert;

ii)	for each of the other affiliated member of the group:

A)	generate a SIP MESSAGE request notification of the MCPTT user's emergency alert indication as specified in subclause 6.3.3.1.11 with the clarifications of subclause 6.3.3.1.12; and

B)	send the SIP MESSAGE request according to 3GPP TS 24.229 [4]; and

iii)	reject the "SIP INVITE request for controlling MCPTT function of an MCPTT group" with a SIP 403 (Forbidden) response to the SIP INVITE request with the following clarifications:

A)	shall generate the SIP 403 (Forbidden) response as specified in subclause 6.3.3.1.14; and

B)	shall send the SIP 403 (Forbidden) response as specified in 3GPP TS 24.229 [4] and skip the rest of the steps;

b)	if the SIP INVITE request contains an alert indication set to "true" and this is an unauthorised request for an MCPTT emergency alert as specified in subclause 6.3.3.1.13.1, shall reject the "SIP INVITE request for controlling MCPTT function of an MCPTT group" with a SIP 403 (Forbidden) response to the SIP INVITE request with the following clarifications:

i)	shall generate the SIP 403 (Forbidden) response as specified in subclause 6.3.3.1.14; and

ii)	shall send the SIP 403 (Forbidden) response as specified in 3GPP TS 24.229 [4] and skip the rest of the steps;

c)	if the SIP INVITE request contains an alert indication set to "false" shall reject the SIP INVITE request with a SIP 403 (Forbidden) response with the following clarifications:

i)	shall generate the SIP 403 (Forbidden) response as specified in subclause 6.3.3.1.14; and

ii)	shall send the SIP 403 (Forbidden) response as specified in 3GPP TS 24.229 [4] and skip the rest of the steps;

11)	if the SIP INVITE request contains an unauthorised request for an MCPTT imminent peril group call as determined by subclause 6.3.3.1.13.5, shall reject the SIP INVITE request with a SIP 403 (Forbidden) response with the following clarifications:

a)	shall include in the SIP 403 (Forbidden) response an application/vnd.3gpp.mcptt-info+xml MIME body as specified in clause F.1 with the <mcpttinfo> element containing the <mcptt-Params> element with the <imminentperil-ind> element set to a value of "false"; and

b)	shall send the SIP 403 (Forbidden) response as specified in 3GPP TS 24.229 [4] and skip the rest of the steps;

12)	if a Resource-Priority header field is included in the SIP INVITE request:

a)	if the Resource-Priority header field is set to the value indicated for emergency calls and the SIP INVITE request does not contain an authorised request for an MCPTT emergency call as determined in step 8) above and the in-progress emergency state of the group is set to a value of "false", shall reject the SIP INVITE request with a SIP 403 (Forbidden) response and skip the rest of the steps; or

b)	if the Resource-Priority header field is set to the value indicated for imminent peril calls and the SIP INVITE request does not contain an authorised request for an MCPTT imminent peril call as determined in step 9) above and the in-progress imminent peril state of the group is set to a value of "false", shall reject the SIP INVITE request with a SIP 403 (Forbidden) response and skip the rest of the steps;

13)	if the MCPTT group call is not ongoing then:

a)	if the user identified by the MCPTT ID is not affiliated to the group identity contained in the SIP INVITE request as specified in subclause 6.3.6, is not a constituent MCPTT group ID and this is an authorised request for an MCPTT emergency group call or MCPTT imminent peril group call as determined by steps 8) and 9) above, shall affiliate the MCPTT user with the MCPTT prearranged group and store the affiliation information locally. Otherwise shall return a SIP 403 (Forbidden) response with the warning text set to "120 user is not affiliated to this group" in a Warning header field as specified in subclause 4.4 and skip the rest of the steps below;

b)	if the user identified by the MCPTT ID is not authorised to initiate the prearranged group session as specified in subclause 6.3.5.4, shall send a SIP 403 (Forbidden) response with the warning text set to: "119 user is not authorised to initiate the group call" in a Warning header field as specified in subclause 4.4 and skip the rest of the steps below; and

c)	shall check if a Resource-Priority header field is included in the incoming SIP INVITE request and may apply any preferential treatment to the SIP request as specified in 3GPP TS 24.229 [4];

d)	shall create a prearranged group session and allocate an MCPTT session identity for the prearranged group call, and shall handle timer TNG3 (group call timer) as specified in subclause 6.3.3.5;

e)	if the group identity in the "SIP INVITE request for controlling MCPTT function of an MCPTT group" is a TGI:

i)	shall for each of the constituent MCPTT groups homed on the primary MCPTT system:

A)	if the controlling MCPTT function does not own the MCPTT group identified by the MCPTT group ID, then generate a SIP INVITE request towards the MCPTT server that owns the MCPTT group identity by following the procedures in subclause 10.1.1.4.1.2; and

NOTE 2:	The MCPTT server that the SIP INVITE request is sent to acts as a non-controlling MCPTT function;

B)	if the controlling MCPTT function owns the MCPTT group identified by the MCPTT group ID then:

I)	determine the members to invite to the prearranged MCPTT group call as specified in subclause 6.3.5.5;

II)	invite each group member determined in step A) above, to the group session, as specified in subclause 10.1.1.4.1.1; and

III)	interact with the media plane as specified in 3GPP TS 24.380 [5] subclause 6.3; and

ii)	shall for each of the constituent MCPTT groups homed on the partner MCPTT system generate a SIP INVITE request for the MCPTT group identity homed on the partner MCPTT system as specified in subclause 10.1.1.4.1.2; and

f)	if the group identity in the SIP INVITE request for controlling MCPTT function of an MCPTT group is an MCPTT group ID:

i)	shall determine the members to invite to the prearranged MCPTT group call as specified in subclause 6.3.5.5;

ii)	if necessary, shall start timer TNG1 (acknowledged call setup timer) according to the conditions stated in subclause 6.3.3.3;

iii)	if the received SIP INVITE request includes an application/vnd.3gpp.mcptt-info+xml MIME body with an <emergency-ind> element set to a value of "true":

A)	shall cache the information that this MCPTT user has initiated an MCPTT emergency call;

B)	if the received SIP INVITE contains an alert indication set to a value of "true" and this is an authorised request for an MCPTT emergency alert meeting the conditions specified in subclause 6.3.3.1.13.1, shall cache the information that this MCPTT user has initiated an MCPTT emergency alert; and

C)	if the in-progress emergency state of the group is set to a value of "false":

I)	shall set the value of the in-progress emergency state of the group to "true"; and

II)	shall start timer TNG2 (in-progress emergency group call timer) and handle its expiry as specified in subclause 6.3.3.1.16;

iv)	if the in-progress emergency state of the group is set to a value of "false" and if the received SIP INVITE request contains an imminent peril indication set to a value of "true", the controlling MCPTT function shall:

A)	shall cache the information that this MCPTT user has initiated an MCPTT imminent peril call; and

B)	if the in-progress imminent peril state of the group is set to a value of "false", shall set the in-progress imminent peril state of the group to a value of "true";

v)	shall invite each group member determined in step i) above, to the group session, as specified in subclause 10.1.1.4.1.1; and

vi)	shall interact with the media plane as specified in 3GPP TS 24.380 [5] subclause 6.3; and

14) if the MCPTT group call is ongoing then:

a)	if the user identified by the MCPTT ID in the SIP INVITE request is not affiliated to the group identity contained in the SIP INVITE request as specified in subclause 6.3.6, is not a constituent MCPTT group ID and this is an authorised request for an MCPTT emergency group call or MCPTT imminent peril group call as determined by steps 8) and 9) above, shall affiliate the MCPTT user with the MCPTT prearranged group and store the affiliation information locally. Otherwise shall return a SIP 403 (Forbidden) response with the warning text set to "120 user is not affiliated to this group" in a Warning header field as specified in subclause 4.4, and skip the rest of the steps below;

b)	if the user identified by the MCPTT ID in the SIP INVITE request is not authorised to join the prearranged group session as specified in subclause 6.3.5.3, shall send a SIP 403 (Forbidden) response with the warning text set to "121 user is not allowed to join the group call" in a Warning header field as specified in subclause 4.4 and skip the rest of the steps below;

c)	shall check if a Resource-Priority header field is included in the incoming SIP INVITE request and may apply any preferential treatment to the SIP request as specified in 3GPP TS 24.229 [4];

d)	if <on-network-max-participant-count> as specified in 3GPP TS 24.381 [31] is already reached:

i)	if, according to local policy, the user identified by the MCPTT ID in the SIP INVITE request is deemed to have a higher priority than an existing user in the group session, may remove a participant from the session by following subclause 10.1.1.4.4.3, and skip the next step; and

NOTE 3:	The local policy for deciding whether to admit a user to a call that has reached its maximum amount of participants can include the <user-priority> and the <participant-type> of the user as well as other information of the user from the group document as specified in 3GPP TS 24.381 [31]. The local policy decisions can also include taking into account whether the imminent-peril indicator or emergency indicator was received in the SIP INVITE request.

ii)	shall return a SIP 486 (Busy Here) response with the warning text set to "122 too many participants" to the originating network as specified in subclause 4.4 and skip the rest of the steps;

e)	if the received SIP INVITE request includes an application/vnd.3gpp.mcptt-info+xml MIME body with an <emergency-ind> element set to a value of "true":

i)	shall cache the information that this MCPTT user has initiated an MCPTT emergency call;

ii)	if the received SIP INVITE contains an alert indication set to a value of "true" and this is an authorised request for an MCPTT emergency alert meeting the conditions specified in subclause 6.3.3.1.13.1, shall cache the information that this MCPTT user has initiated an MCPTT emergency alert;

iii)	if the in-progress emergency state of the group is set to a value of "false":

A)	shall set the value of the in-progress emergency state of the group to "true";

B)	shall start timer TNG2 (in-progress emergency group call timer) and handle its expiry as specified in subclause 6.3.3.1.16; and

C)	shall generate SIP re-INVITE requests for the MCPTT emergency group call to the other call participants of the MCPTT group as specified in subclause 6.3.3.1.6;

iv)	if the in-progress imminent peril state of the group is set to a value of "true":

A)	for each of the other affiliated member of the group generate a SIP MESSAGE request notification of the MCPTT user's imminent peril indication as specified in subclause 6.3.3.1.11, setting the <imminentperil-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body to a value of "true"; and

B)	send the SIP MESSAGE request as specified in 3GPP TS 24.229 [4]; and

v)	upon receiving a SIP 200 (OK) response to the SIP re-INVITE request the controlling MCPTT function shall interact with the media plane as specified in 3GPP TS 24.380 [5];

f)	if the in-progress emergency state of the group is set to a value of "false" and if the SIP INVITE request contains an imminent peril indication set to a value of "true", the controlling MCPTT function:

i)	shall cache the information that this MCPTT user has initiated an MCPTT imminent peril call; and

ii)	if the in-progress imminent peril state of the group is set to a value of "false":

A)	shall set the in-progress imminent peril state of the group to a value of "true";

B)	shall generate SIP re-INVITE requests for the MCPTT imminent peril group call to the other call participants of the MCPTT group as specified in subclause 6.3.3.1.15; and

C)	upon receiving a SIP 200 (OK) response to the SIP re-INVITE request the controlling MCPTT function shall interact with the media plane as specified in 3GPP TS 24.380 [5]; and

iii)	if the in-progress imminent peril state of the group is set to a value of "true":

A)	for each of the other affiliated member of the group generate a SIP MESSAGE request notification of the MCPTT user's imminent peril indication as specified in subclause 6.3.3.1.11, setting the <imminentperil-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body to a value of "true"; and

B)	send the SIP MESSAGE request as specified in 3GPP TS 24.229 [4];

g)	shall generate a SIP 200 (OK) response as specified in the subclause 6.3.3.2.4.2;

h)	shall include in the SIP 200 (OK) response an SDP answer to the SDP offer in the incoming SIP INVITE request as specified in the subclause 6.3.3.2.1;

i)	shall include in the SIP 200 (OK) response with the warning text set to "123 MCPTT session already exists" as specified in subclause 4.4;

j)	if the received SIP re-INVITE request contains an alert indication set to a value of "true" and this is an unauthorised request for an MCPTT emergency alert as specified in subclause 6.3.3.1.13.1, shall include in the SIP 200 (OK) response the warning text set to "149 SIP INFO request pending" in a Warning header field as specified in subclause 4.4;

k)	if the received SIP re-INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <imminentperil-ind> element set to a value of "true" and if the in-progress emergency state of the group is set to a value of "true", shall include in the SIP 200 (OK) response the warning text set to "149 SIP INFO request pending" in a Warning header field as specified in subclause 4.4;

NOTE 4:	In this case, the request was for an imminent peril call but a higher priority MCPTT emergency call was already in progress on the group. Hence, the imminent peril call request aspect of the request is denied but the request is granted with emergency level priority.

l)	shall interact with media plane as specified in 3GPP TS 24.380 [5] subclause 6.3;

NOTE 5:	Resulting media plane processing is completed before the next step is performed.

m)	shall send the SIP 200 (OK) response towards the inviting MCPTT client or inviting non-controlling MCPTT function according to 3GPP TS 24.229 [4];

n)	shall generate a notification to the MCPTT clients, which have subscribed to the conference state event package that the inviting MCPTT User has joined in the MCPTT group session, as specified in subclause 6.3.3.4;

NOTE 6:	As a group document can potentially have a large content, the controlling MCPTT function can notify using content-indirection as defined in IETF RFC 4483 [32].

o)	shall send a SIP NOTIFY request to each MCPTT client according to 3GPP TS 24.229 [4];

p)	Upon receiving a SIP ACK to the above SIP 200 (OK) response and the SIP 200 (OK) response contained the warning text set to "149 SIP INFO request pending" in a Warning header field as specified in subclause 4.4, shall follow the procedures in subclause 6.3.3.1.18; and

q)	shall not continue with the rest of the subclause.

Upon receiving a SIP 183 (Session Progress) response to the SIP INVITE request specified in subclause 10.1.1.4.1 containing a P-Answer-State header field with the value "Unconfirmed" as specified in IETF RFC 4964 [34], the timer TNG1 (acknowledged call setup timer) is not running, the controlling MCPTT function supports media buffering and the SIP final response is not yet sent to the inviting MCPTT client:

1)	shall generate a SIP 200 (OK) response to SIP INVITE request as specified in the subclause 6.3.3.2.3.2;

2)	shall include the warning text set to "122 too many participants" as specified in subclause 4,4 in the SIP 200 (OK) response, if the prearranged MCPTT group has more than <on-network-max-participant-count> members as specified in 3GPP TS 24.381 [31];

3)	shall include in the SIP 200 (OK) response an SDP answer to the SDP offer in the incoming SIP INVITE request as specified in the subclause 6.3.3.2.1;

4)	shall include a P-Answer-State header field with the value "Unconfirmed";

5)	if the SIP INVITE request contains an alert indication set to a value of "true" and this is an unauthorised request for an MCPTT emergency alert as specified in subclause 6.3.3.1.13.1, shall include in the SIP 200 (OK) response the warning text set to "149 SIP INFO request pending" in a Warning header field as specified in subclause 4.4;

6)	if the received SIP INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <imminentperil-ind> element set to a value of "true" and if the in-progress emergency state of the group is set to a value of "true", shall include in the SIP 200 (OK) response the warning text set to "149 SIP INFO request pending" in a Warning header field as specified in subclause 4.4;

7)	shall interact with the media plane as specified in 3GPP TS 24.380 [5] subclause 6.3;

NOTE 7:	Resulting user plane processing is completed before the next step is performed.

8)	shall send the SIP 200 (OK) response towards the inviting MCPTT client according to 3GPP TS 24.229 [4];

9)	shall generate a notification to the MCPTT clients, which have subscribed to the conference state event package that the inviting MCPTT User has joined in the MCPTT group session, as specified in subclause 6.3.3.4; and

NOTE 8:	As a group document can potentially have a large content, the controlling MCPTT function can notify using content-indirection as defined in IETF RFC 4483 [32].

10)	shall send a SIP NOTIFY request to each MCPTT client according to 3GPP TS 24.229 [4].

Upon receiving a SIP 183 (Session Progress) response for a SIP INVITE request as specified in subclause 10.1.1.4.1.2 containing an indication of required group members, the timer TNG1 (acknowledged call setup timer) is running and all SIP 200 (OK) responses have been received to all SIP INVITE requests sent to MCPTT clients specified in subclause 10.1.1.4.1.1, then the controlling MCPTT function shall wait until the SIP 200 (OK) response has been received to the SIP INVITE request specified in subclause 10.1.1.4.1.2 before generating a SIP 200 (OK) response to the "SIP INVITE request for controlling MCPTT function of an MCPTT group".

Upon receiving a SIP 200 (OK) response for a SIP INVITE request as specified in subclause 10.1.1.4.1 that was sent to an affiliated and <on-network-required> group member as specified in 3GPP TS 24.381 [31]; and

1)	if the MCPTT ID in the SIP 200 (OK) response matches to the MCPTT ID in the corresponding SIP INVITE request;

2)	there are no outstanding SIP 200 (OK) responses to SIP INVITE requests which were sent to affiliated and <on-network-required> group members as specified in 3GPP TS 24.381 [31]; and

3)	there is no outstanding SIP 200 (OK) response to a SIP INVITE request sent in subclause 10.1.1.4.1.2 where the SIP 183 (Session Progress) response contained an indication of required group members;

the controlling MCPTT function:

1)	shall stop timer TNG1 (acknowledged call setup timer) as described in subclause 6.3.3.3;

2)	shall generate SIP 200 (OK) response to the SIP INVITE request as specified in the subclause 6.3.3.2.3.2 before continuing with the rest of the steps;

3)	shall include the warning text set to "122 too many participants" as specified in subclause 4.4 in the SIP 200 (OK) response, if all members were not invited because the prearranged MCPTT group has been exceeded the <on-network-max-participant-count> members as specified in 3GPP TS 24.381 [31];

4)	shall include in the SIP 200 (OK) response an SDP answer to the SDP offer in the incoming SIP INVITE request as specified in the subclause 6.3.3.2.1;

5)	shall interact with the media plane as specified in 3GPP TS 24.380 [5] subclause 6.3;

NOTE 9:	Resulting media plane processing is completed before the next step is performed.

6)	shall send a SIP 200 (OK) response to the inviting MCPTT client according to 3GPP TS 24.229 [4];

7)	shall generate a notification to the MCPTT clients, which have subscribed to the conference state event package that the inviting MCPTT user has joined in the MCPTT group session, as specified in subclause 6.3.3.4; and

NOTE 10:	As a group document can potentially have a large content, the controlling MCPTT function can notify using content-indirection as defined in IETF RFC 4483 [32].

8)	shall send the SIP NOTIFY request to the MCPTT clients according to 3GPP TS 24.229 [4].

Upon:

1)	receiving a SIP 200 (OK) response for a SIP INVITE request as specified in subclause 10.1.1.4.1;

2)	the timer TNG1 (acknowledged call setup timer) is not running;

3)	the local counter of the number of SIP 200 (OK) responses received from invited members is equal to the value of the <on-network-minimum-number-to-start> element of the group document;

4)	the controlling MCPTT function supports media buffering; and

5)	the SIP final response has not yet been sent to the inviting MCPTT client;

the controlling MCPTT function according to local policy:

1)	shall generate SIP 200 (OK) response to the SIP INVITE request as specified in the subclause 6.3.3.2.2;

2)	shall include the warning text set to "122 too many participants" as specified in subclause 4.4 in the SIP 200 (OK) response, if all members were not invited because the prearranged MCPTT group has exceeded the <max-participant-count> members as specified in 3GPP TS 24.381 [31];

3)	shall include in the SIP 200 (OK) response an SDP answer to the SDP offer in the incoming SIP INVITE request as specified in the subclause 6.3.3.2.1;

4)	if the SIP INVITE request contains an alert indication set to a value of "true" and this is an unauthorised request for an MCPTT emergency alert as specified in subclause 6.3.3.1.13.1, shall include in the SIP 200 (OK) response the warning text set to "149 SIP INFO request pending" in a Warning header field as specified in subclause 4.4;

5)	if the received SIP INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <imminentperil-ind> element set to a value of "true" and if the in-progress emergency state of the group is set to a value of "true", shall include in the SIP 200 (OK) response the warning text set to "149 SIP INFO request pending" in a Warning header field as specified in subclause 4.4;

6)	shall interact with the media plane as specified in 3GPP TS 24.380 [5] subclause 6.3;

NOTE 11:	Resulting media plane processing is completed before the next step is performed.

7)	shall send a SIP 200 (OK) response to the inviting MCPTT client according to 3GPP TS 24.229 [4];

8)	shall generate a notification to the MCPTT clients, which have subscribed to the conference state event package that the inviting MCPTT user has joined in the MCPTT group session, as specified in subclause 6.3.3.4; and

NOTE 12:	As a group document can potentially have a large content, the controlling MCPTT function can notify using content-indirection as defined in IETF RFC 4483 [32].

9)	shall send the SIP NOTIFY request to the MCPTT clients according to 3GPP TS 24.229 [4].

Upon expiry of timer TNG1 (acknowledged call setup timer), if there are outstanding SIP 200 (OK) responses to SIP INVITE requests sent to affiliated and <on-network-required> group members as specified in 3GPP TS 24.381 [31], the controlling MCPTT function shall follow the procedures specified in subclause 6.3.3.3.

If timer TNG1 (acknowledged call setup timer) is running and a final SIP 4xx, 5xx or 6xx response is received from an affiliated and <on-network-required> group member as specified in 3GPP TS 24.381 [31], the controlling MCPTT function shall follow the relevant procedures specified in subclause 6.3.3.3.

If:

1)	timer TNG1 (acknowledged call setup timer) is not running;

2)	the local counter of the number of SIP 200 (OK) responses received from invited members is equal to the value of the <on-network-minimum-number-to-start> element of the group document; and

3)	a final SIP 4xx, 5xx or 6xx response is received from an invited MCPTT client;

then the controlling MCPTT function shall perform one of the following based on policy:

1)	send the SIP final response towards the inviting MCPTT client, according to 3GPP TS 24.229 [4], if a SIP final response was received from all the other invited MCPTT clients and the SIP 200 (OK) response is not yet sent; or

2)	remove the invited MCPTT client from the MCPTT Session as specified in subclause 6.3.3.1.5, if a SIP final response other than 2xx or 3xx was received from all the invited MCPTT clients and the SIP 200 (OK) response is already sent. the controlling MCPTT function may invite an additional member of the prearranged MCPTT group as specified in subclause 10.1.1.4.1 that has not already been invited, if the prearranged MCPTT group has more than <on-network-max-participant-count> members as specified in 3GPP TS 24.381 [31], and all members have not yet been invited.

If the group identity in the "SIP INVITE request for controlling MCPTT function of an MCPTT group" is a TGI and constituent MCPTT groups were invited as specified in subclause 10.1.1.4.1.2 and,

1)	if all non-controlling MCPTT functions hosting the constituent MCPTT groups have responded with a SIP 2xx, SIP 3xx, SIP 4xx, SIP 5xx or SIP 6xx responses to the "SIP INVITE request for non-controlling MCPTT function of an MCPTT group"; and

2)	if all expected SIP INFO requests containing a floor request are received;

then the controlling MCPTT function shall indicate to the media plane that all final responses are received.

NOTE 13:	If the SIP 200 (OK) response to the SIP INVITE request for non-controlling MCPTT function of an MCPTT group included the application/vnd.3gpp.mcptt-info+xml MIME body with the <floor-state> element set to "floor-taken", the controlling MCPTT function expects that the non-controlling MCPTT functions sends a SIP INFO request containing a floor request.

Upon receiving a SIP ACK to the SIP 200 (OK) response sent towards the inviting MCPTT client, and the SIP 200 (OK) response was sent with the warning text set to "149 SIP INFO request pending" in a Warning header field as specified in subclause 4.4, the controlling MCPTT function shall follow the procedures in subclause 6.3.3.1.18.

##### 10.1.1.4.3	End group call at the terminating controlling MCPTT function

Upon receiving a SIP BYE request the controlling MCPTT function shall follow the procedures as specified in subclause 6.3.3.2.4.

##### 10.1.1.4.4	End group call initiated by the controlling MCPTT function

###### 10.1.1.4.4.1	General

This subclause describes the procedures of each functional entity for ending the group call initiated by the controlling MCPTT function.

###### 10.1.1.4.4.2	SIP BYE request for releasing MCPTT session for a group call

When the MCPTT session for group call needs to be released as specified in subclause 6.3.8.1, the controlling MCPTT function shall follow the procedures in subclause 6.3.3.1.5.

###### 10.1.1.4.4.3	SIP BYE request toward a MCPTT client

When an MCPTT client needs to be removed from the MCPTT session (e.g. due to de-affiliation or admitting a higher priority user), the controlling MCPTT function shall follow the procedures in subclause 6.3.3.1.5.

After successful removing the MCPTT client from the MCPTT session, the controlling MCPTT function may generate a notification to the MCPTT clients, which have subscribed to the conference state event package that an MCPTT user has been removed from the MCPTT session, as specified in subclause 6.3.3.4 and send the SIP NOTIFY request to the MCPTT client according to 3GPP TS 24.229 [4].

##### 10.1.1.4.5	Re-join procedures

###### 10.1.1.4.5.1	Terminating procedures

Upon receipt of a SIP INVITE request that includes an MCPTT session identity of an ongoing MCPTT session in the Request-URI the controlling MCPTT function:

1)	if unable to process the request due to a lack of resources or a risk of congestion exists, may reject the SIP INVITE request with a SIP 500 (Server Internal Error) response. The controlling MCPTT function may include a Retry-After header field to the SIP 500 (Server Internal Error) response as specified in IETF RFC 3261 [24]. Otherwise, continue with the rest of the steps;

NOTE 1:	if the SIP INVITE request contains an emergency indication or an imminent peril indication set to a value of "true" and this is an authorised request for originating an MCPTT emergency group call as determined by subclause 6.3.3.1.13.2, or for originating an MCPTT imminent peril group call as determined by subclause 6.3.3.1.13.5, the controlling MCPTT function can according to local policy choose to accept the request.

2)	shall reject the SIP request with a SIP 404 (Not Found) response if the MCPTT group call represented by the MCPTT session identity in Request-URI header is not present;

3)	shall determine if the media parameters are acceptable and the MCPTT speech codec is offered in the SDP offer and if not, reject the request with a SIP 488 (Not Acceptable Here) response. Otherwise, continue with the rest of the steps;

4)	shall reject the SIP request with a SIP 403 (Forbidden) response and not process the remaining steps if:

a)	an Accept-Contact header field does not include the g.3gpp.mcptt media feature tag; or

b)	an Accept-Contact header field does not include the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt";

5)	shall determine the MCPTT ID of the calling user;

6)	if the user identified by the MCPTT ID is not authorised to join the prearranged group session as specified in subclause 6.3.5.3, shall send a SIP 403 (Forbidden) response with the warning text set to "121 user is not authorised to join the group call" in a Warning header field as specified in subclause 4.4. Otherwise continue with the rest of the steps below;

7)	shall perform the actions on receipt of an initial SIP INVITE request as described in subclause 6.3.3.2.2;

8)	if the user identified by the MCPTT ID is not affiliated to the MCPTT group ID associated with the MCPTT session identity as specified in subclause 6.3.3.5, shall return a SIP 403 (Forbidden) response with the warning text set to "120 user is not affiliated to this group" in a Warning header field as specified in subclause 4.4;

9)	shall check if a Resource-Priority header field is included in the incoming SIP INVITE request and may apply any preferential treatment to the SIP request as specified in 3GPP TS 24.229 [4];

10)	if <on-network-max-participant-count> as specified in 3GPP TS 24.381 [31] is already reached:

a)	if, according to local policy, the user identified by the MCPTT ID in the SIP INVITE request is deemed to have a higher priority than an existing user in the group session, may remove a participant from the session by following subclause 10.1.1.4.4.3, and skip the next step; and

NOTE 2:	The local policy for deciding whether to admit a user to a call that has reached its maximum amount of participants can include the <user-priority> and the <participant-type> of the user as well as other information of the user from the group document as specified in 3GPP TS 24.381 [31]. The local policy decisions can also include taking into account whether the imminent-peril indicator or emergency indicator was received in the SIP INVITE request.

b)	shall return a SIP 486 (Busy Here) response with the warning text set to "122 too many participants" to the originating network as specified in subclause 4.4 Otherwise, continue with the rest of the steps;

11)	shall generate a SIP 200 (OK) response as specified in the subclause 6.3.3.2.4.2;

12)	shall include in the SIP 200 (OK) response an SDP answer to the SDP offer in the incoming SIP INVITE request as specified in the subclause 6.3.3.2.1;

13)	shall interact with media plane as specified in 3GPP TS 24.380 [5] subclause 6.3;

NOTE 3:	Resulting media plane processing is completed before the next step is performed.

14)	shall send the SIP 200 (OK) response towards the inviting MCPTT client according to 3GPP TS 24.229 [4];

15)	shall generate a notification to the MCPTT clients, which have subscribed to the conference state event package that the inviting MCPTT User has joined in the MCPTT group session, as specified in subclause 6.3.3.4; and

NOTE 4:	As a group document can potentially have a large content, the controlling MCPTT function can notify using content-indirection as defined in IETF RFC 4483 [32].

16)	shall send a SIP NOTIFY request to each MCPTT client according to 3GPP TS 24.229 [4].

##### 10.1.1.4.6	Late call entry initiated by controlling MCPTT function

When controlling MCPTT function is notified that an MCPTT client is newly affiliated or comes back from out of coverage, the controlling MCPTT function shall invite the MCPTT client to join an ongoing MCPTT group call by following the procedures specified in subclause 10.1.1.4.1.

NOTE:	How the MCPTT function is informed when an MCPTT client is coming back from out of coverage is out of scope of present document.

##### 10.1.1.4.7	Receipt of a SIP re-INVITE request

In the procedures in this subclause:

1)	emergency indication in an incoming SIP INVITE request refers to the <emergency-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body; and

2)	imminent peril indication in an incoming SIP INVITE request refers to the <imminentperil-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body.

Upon receipt of a SIP re-INVITE request for an MCPTT session identity identifying an on-demand prearranged MCPTT group session, the controlling MCPTT function:

1)	if unable to process the request due to a lack of resources or a risk of congestion exists, may reject the SIP re-INVITE request with a SIP 500 (Server Internal Error) response. The controlling MCPTT function may include a Retry-After header field to the SIP 500 (Server Internal Error) response as specified in IETF RFC 3261 [24] and skip the rest of the steps;

NOTE 1:	If the SIP re-INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <emergency-ind> element set to a value of "true", the controlling MCPTT function can choose to accept the request.

2)	if received SIP re-INVITE request includes an application/vnd.3gpp.mcptt-info+xml MIME body with an <emergency-ind> element included or an <imminent-peril> element included, shall validate the request as described in subclause 6.3.3.1.17;

3)	if the received SIP re-INVITE request contains an unauthorised request for an MCPTT emergency call as determined by subclause 6.3.3.1.13.2:

a)	if the SIP re-INVITE request contains an alert indication set to "true" and this is an authorised request for an MCPTT emergency alert meeting the conditions specified in subclause 6.3.3.1.13.1, shall:

i)	for each of the other affiliated members of the group send an MCPTT emergency alert:

A)	generate a SIP MESSAGE request notification of the MCPTT user's emergency alert indication as specified in subclause 6.3.3.1.11 with the clarifications of subclause 6.3.3.1.12; and

B)	send the SIP MESSAGE request according to 3GPP TS 24.229 [4];

ii)	cache the MCPTT ID of the MCPTT user that has initiated an MCPTT emergency alert; and

iii)	reject the SIP re-INVITE request with a SIP 403 (Forbidden) response to the SIP re-INVITE request with the following clarifications:

A)	shall generate the SIP 403 (Forbidden) response as specified in subclause 6.3.3.1.14; and

B)	shall send the SIP 403 (Forbidden) response as specified in 3GPP TS 24.229 [4] and skip the rest of the steps;

b)	if the SIP re-INVITE request contains an alert indication set to "true" and this is an unauthorised request for an MCPTT emergency alert as determined by subclause 6.3.3.1.13.1, shall reject the SIP re-INVITE request with a SIP 403 (Forbidden) response to the SIP re-INVITE request with the following clarifications:

i)	shall generate the SIP 403 (Forbidden) response as specified in subclause 6.3.3.1.14; and

ii)	shall send the SIP 403 (Forbidden) response as specified in 3GPP TS 24.229 [4 and skip the rest of the steps;

c)	if the SIP re-INVITE request contains an alert indication is set to "false" shall reject the SIP re-INVITE request with a SIP 403 (Forbidden) response to the SIP re-INVITE request, with the following clarifications:

i)	shall generate a SIP 403 (Forbidden) response as specified in subclause 6.3.3.1.14; and

ii)	shall send the SIP 403 (Forbidden) response as specified in 3GPP TS 24.229 [4] and skip the rest of the steps;

4)	if the received SIP re-INVITE request contains an imminent peril indication set to "true" for an MCPTT imminent peril group call and this is an unauthorised request for an MCPTT imminent peril group call as determined by subclause 6.3.3.1.13.6, shall reject the SIP re-INVITE request with a SIP 403 (Forbidden) response with the following clarifications:

a)	shall include in the SIP 403 (Forbidden) response an application/vnd.3gpp.mcptt-info+xml MIME body as specified in clause F.1 with the <mcpttinfo> element containing the <mcptt-Params> element with the <imminentperil-ind> element set to a value of "false"; and

b)	shall send the SIP 403 (Forbidden) response as specified in 3GPP TS 24.229 [4] and skip the rest of the steps;

5)	if a Resource-Priority header field is included in the received SIP re-INVITE request:

a)	if the Resource-Priority header field is set to the value indicated for emergency calls and the SIP re-INVITE request does not contain an authorised request for an MCPTT emergency call as determined in step 2) above and the in-progress emergency state of the group is set to a value of "false", shall reject the SIP re-INVITE request with a SIP 403 (Forbidden) response and skip the rest of the steps; and

b)	if the Resource-Priority header field is set to the value indicated for imminent peril calls and the SIP re-INVITE request does not contain an authorised request for an MCPTT imminent peril call as determined in step 3) above and the in-progress imminent peril state of the group is set to a value of "false", shall reject the SIP INVITE request with a SIP 403 (Forbidden) response and skip the rest of the steps;

6)	if the received SIP re-INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <emergency-ind> element set to a value of "true" and is an authorised request to initiate an MCPTT emergency group call as determined by subclause 6.3.3.1.13.2, the controlling MCPTT function shall:

i)	shall cache the MCPTT ID of the MCPTT user that has initiated an MCPTT emergency call;

ii)	if the received SIP INVITE contains an alert indication set to a value of "true" and this is an authorised request for an MCPTT emergency alert meeting the conditions specified in subclause 6.3.3.1.13.1, shall cache the MCPTT ID of the MCPTT user that has initiated an MCPTT emergency alert;

iii)	if the in-progress emergency state of the group is set to a value of "true":

A)	for each of the other affiliated member of the group generate a SIP MESSAGE request notification of the MCPTT user's emergency indication as specified in subclause 6.3.3.1.11, setting the <emergency-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body to a value of "true";

B)	send the SIP MESSAGE request as specified in 3GPP TS 24.229 [4]; and

C)	if the in-progress imminent peril state of the group is set to a value of "true", shall set it to a value of "false"; and

iv)	if the in-progress emergency state of the group is set to a value of "false":

A)	shall set the value of the in-progress emergency state of the group to "true";

B)	shall start timer TNG2 (in-progress emergency group call timer) and handle its expiry as specified in subclause 6.3.3.1.16;

NOTE 2:	The interactions of TNG2 with the TNG3 (group call timer) are explained in subclause 6.3.3.5.2.

C)	shall generate SIP re-INVITE requests for the MCPTT emergency group call to the other participants of the MCPTT group call as specified in subclause 6.3.3.1.6;

D)	shall send the SIP re-INVITEs towards the other participants of the MCPTT group call; and

E)	upon receiving a SIP 200 (OK) response to the SIP re-INVITE request the controlling MCPTT function shall interact with the media plane as specified in 3GPP TS 24.380 [5];

7)	if the received SIP re-INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <emergency-ind> element set to a value of "false" and is an unauthorised request for an MCPTT emergency group call cancellation as determined by subclause 6.3.3.1.13.4:

a)	if an <alert-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body is included and set to "false" and is an authorised request for an MCPTT emergency alert cancellation as determined by subclause 6.3.3.1.13.3 and there is an outstanding MCPTT emergency alert for this MCPTT user:

i)	for each of the other affiliated members of the group shall:

A)	generate a SIP MESSAGE request notification of the cancellation of the MCPTT user's emergency alert as specified in subclause 6.3.3.1.11;

B)	if the received SIP re-INVITE request contains an <originated-by> element in the application/vnd.3gpp.mcptt-info+xml MIME body, copy the contents of the received <originated-by> element to an <originated-by> element in the application/vnd.3gpp.mcptt-info+xml MIME body in the outgoing SIP MESSAGE request;

C) set the <alert-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body to a value of "false"; and

D)	send the SIP MESSAGE request according to 3GPP TS 24.229 [4];

ii)	if the received SIP re-INVITE request contains an <originated-by> element in the application/vnd.3gpp.mcptt-info+xml MIME body, shall clear the cache of the MCPTT ID of the MCPTT user identified by the <originated-by> element as having an outstanding MCPTT emergency alert;;

iii)	if the received SIP re-INVITE request does not contain an <originated-by> element in the application/vnd.3gpp.mcptt-info+xml MIME body, shall clear the cache of the MCPTT ID of the sender of the SIP re-INVITE as having an outstanding MCPTT emergency alert; and

iv)	shall reject the SIP re-INVITE request with a SIP 403 (Forbidden) response to the SIP re-INVITE request with the following clarifications:

A)	include in the SIP 403 (Forbidden) response an application/vnd.3gpp.mcptt-info+xml MIME body as specified in clause F.1 with the <mcpttinfo> element containing the <mcptt-Params> element with the <emergency-ind> element set to a value of "true";

B) include in the application/vnd.3gpp.mcptt-info+xml MIME body and <alert-ind> element set to a value of "false"; and

C)	shall send the SIP 403 (Forbidden) response as specified in 3GPP TS 24.229 [4] and skip the rest of the steps; and

b)	if an <alert-ind> element of the mcpttinfo MIME body is included and set to "false" and is an unauthorised request for an MCPTT emergency alert cancellation as determined by subclause 6.3.3.1.13.3 and there is an outstanding MCPTT emergency alert for this MCPTT user:

i)	shall reject the SIP re-INVITE request with a SIP 403 (Forbidden) response to the SIP re-INVITE request with the following clarifications:

A)	include in the SIP 403 (Forbidden) response an application/vnd.3gpp.mcptt-info+xml MIME body as specified in annex F.1 with an <emergency-ind> element set to a value of "true"; and

B) include in the application/vnd.3gpp.mcptt-info+xml MIME body and <alert-ind> element set to a value of "true"; and

ii)	shall send the SIP 403 (Forbidden) response as specified in 3GPP TS 24.229 [4] and skip the rest of the steps;

8)	if the received SIP re-INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <emergency-ind> element set to a value of "false" and is determined to be an authorised request for an MCPTT emergency call cancellation as specified in subclause 6.3.3.1.16 and the in-progress emergency state of the group to is set to a value of "true" the controlling MCPTT function:

a)	shall set the in-progress emergency group state of the group to a value of "false";

b)	shall clear the cache of theMCPTT ID of the MCPTT user as having an outstanding MCPTT emergency group call;

c)	if an <alert-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body is included and set to "false" and is determined to be an authorised request for an MCPTT emergency alert cancellation as specified in subclause 6.3.3.1.13.3 and there is an outstanding MCPTT emergency alert for this MCPTT user shall:

i)	if the received SIP re-INVITE request contains an <originated-by> element in the application/vnd.3gpp.mcptt-info+xml MIME body, clear the cache of the MCPTT ID of the MCPTT user identified by the <originated-by> element as having an outstanding MCPTT emergency alert; or

ii)	if the received SIP re-INVITE request does not contain an <originated-by> element in the application/vnd.3gpp.mcptt-info+xml MIME body, clear the cache of the MCPTT ID of the sender of the SIP re-INVITE request as having an outstanding MCPTT emergency alert;

d)	shall generate SIP re-INVITE requests to the participants in the group call as specified in subclause 6.3.3.1.6. The MCPTT controlling function:

i)	for each of the other participants in the group call shall send the SIP re-INVITE request towards the MCPTT client as specified in 3GPP TS 24.229 [4]; and

ii)	Upon receiving a SIP 200 (OK) response to the SIP re-INVITE request the controlling MCPTT function shall interact with the media plane as specified in 3GPP TS 24.380 [5];

NOTE 3:	Subclause 6.3.3.1.6 will inform the group call participants of the cancellation of the MCPTT group's in-progress emergency state and the cancellation of the MCPTT emergency alert if applicable.

e)	shall stop timer TNG2 (in-progress emergency group call timer); and

NOTE 4:	The interactions of TNG2 with the TNG3 (group call timer) are explained in subclause 6.3.3.5.2;

f)	for each of the affiliated members of the group that are not participating in the call:

i)	generate a SIP MESSAGE request notification of the cancellation of the MCPTT user's emergency call as specified in subclause 6.3.3.1.11;

ii)	set the <emergency-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body to a value of "false";

iii)	if indicated above in step 7) c), set the <alert-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body to a value of "false"; and

iv)	send the SIP MESSAGE request according to 3GPP TS 24.229 [4];

9)	if the received SIP re-INVITE request contains an imminent peril indication and the in-progress emergency group state of the group is set to a value of "false", shall perform the procedures specified in subclause 10.1.1.4.8 and skip the rest of the steps.

Upon receiving a SIP 200 (OK) response to a SIP re-INVITE request the controlling MCPTT function shall interact with the media plane as specified in 3GPP TS 24.380 [5];

1)	shall generate a SIP 200 (OK) response according to rules and procedures of 3GPP TS 24.229 [4];

2)	shall include in the SIP 200 (OK) response an SDP answer according to 3GPP TS 24.229 [4] with the clarifications specified in subclause 6.3.3.2.1;

3)	shall include the "norefersub" option tag in a Supported header field according to IETF RFC 4488 [22];

4)	shall include the "tdialog" option tag in a Supported header field according to IETF RFC 4538 [23];

5)	if the received SIP re-INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <alert-ind> element set to a value of "true" and if this is an unauthorised request for an MCPTT emergency alert as determined by subclause 6.3.3.1.13.1, shall include in the SIP 200 (OK) response the warning text set to "149 SIP INFO request pending" in a Warning header field as specified in subclause 4.4;

6)	if the received SIP re-INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <alert-ind> element set to a value of "false" and if this is an unauthorised request for an MCPTT emergency alert cancellation as determined by subclause 6.3.3.1.13.3, shall include in the SIP 200 (OK) response the warning text set to "149 SIP INFO request pending" in a Warning header field as specified in subclause 4.4;

7)	if the received SIP re-INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <imminentperil-ind> element set to a value of "true", this is an authorised request for an MCPTT imminent peril group call and the in-progress emergency state of the group is set to a value of "true", shall include in the SIP 200 (OK) response the warning text set to "149 SIP INFO request pending" in a Warning header field as specified in subclause 4.4;

NOTE 5:	In this case, the request was for an imminent peril call but a higher priority MCPTT emergency call was already in progress on the group. Hence, the imminent peril call request aspect of the request is denied but the request is granted with emergency level priority.

8)	shall interact with media plane as specified in 3GPP TS 24.380 [5]; and

9)	shall send the SIP 200 (OK) response towards the MCPTT client according to 3GPP TS 24.229 [4].

Upon receiving a SIP ACK to the SIP 200 (OK) response sent towards the inviting MCPTT client, and the SIP 200 (OK) response was sent with the warning text set to "149 SIP INFO request pending" in a Warning header field as specified in subclause 4.4, the controlling MCPTT function shall follow the procedures in subclause 6.3.3.1.18.

Upon receipt of a SIP 2xx response for an outgoing SIP MESSAGE request, shall handle according to 3GPP TS 24.229 [4].

##### 10.1.1.4.8	Handling of a SIP re-INVITE request for imminent peril session

This procedure is initiated by the controlling MCPTT function as the result of an action in subclause 10.1.1.4.7.

In the procedures in this subclause:

1)	imminent peril indication in an incoming SIP INVITE request refers to the <imminentperil-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body.

When the controlling function receives a SIP re-INVITE request with an imminent peril indication set to "true", the controlling function:

1)	if the in-progress emergency state of the group is set to a value of "false" and if the SIP INVITE request contains an imminent peril indication set to a value of "true" or the in-progress imminent peril state of the group to "true", the controlling MCPTT function shall:

NOTE: 1	The calling procedure has already determined that this is not an unauthorised request for an MCPTT imminent peril call, therefore that check does not need to be repeated in the current procedure.

a)	if the in-progress imminent peril state of the group is set to a value of "true" and this MCPTT user is indicating a new imminent peril indication:

i)	for each of the other affiliated member of the group generate a SIP MESSAGE request notification of the MCPTT user's imminent peril indication as specified in subclause 6.3.3.1.11 with the following clarifications;

A)	set the <imminentperil-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body to a value of "true"; and

B)	send the SIP MESSAGE request as specified in 3GPP TS 24.229 [4];

b)	if the in-progress imminent peril state of the group is set to a value of "false";

i)	set the value of the in-progress imminent peril state of the group to "true";

ii)	generate SIP re-INVITE requests for the MCPTT imminent peril group call to participants in the MCPTT group call as specified in subclause 6.3.3.1.15;

iii)	send the SIP re-INVITES to all of the other participants in the MCPTT group call;

iv)	for each of the affiliated members of the group not participating in the group call, generate a SIP MESSAGE request notification of the MCPTT user's imminent peril indication as specified in subclause 6.3.3.1.11 with the following clarifications;

A)	set the <imminentperil-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body to a value of "true"; and

B)	send the SIP MESSAGE request as specified in 3GPP TS 24.229 [4]; and

c)	cache the information that this MCPTT user has initiated an MCPTT imminent peril call;

2)	if the SIP re-INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <imminentperil-ind> element set to a value of "false" and is an unauthorised request for an MCPTT imminent peril group call cancellation as determined by subclause 6.3.3.1.13.6 shall:

a)	reject the SIP re-INVITE request with a SIP 403 (Forbidden) response to the SIP re-INVITE request; and

b)	include in the SIP 403 (Forbidden) response an application/vnd.3gpp.mcptt-info+xml MIME body as specified in Annex F.1 with the <mcpttinfo> element containing the <mcptt-Params> element with the <imminentperil-ind> element set to a value of "false";

c) send the SIP 403 (Forbidden) response as specified in 3GPP TS 24.229 [4]; and

d) skip the rest of the steps;

3)	if the SIP re-INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <imminentperil-ind> element set to a value of "false" and is determined to be an authorised request for an MCPTT imminent peril call cancellation as specified in subclause 6.3.3.1.13.6 and the in-progress imminent peril state of the group to is set to a value of "true" the controlling MCPTT function shall:

a)	set the in-progress imminent peril state of the group to a value of "false";

b)	cache the information that this MCPTT user no longer has an outstanding MCPTT imminent peril group call;

c)	generate SIP re-INVITES requests to the other participants in the MCPTT group call as specified in subclause 6.3.3.1.15. The MCPTT controlling function:

i)	for each participant shall send the SIP re-INVITE request towards the MCPTT client as specified in 3GPP TS 24.229 [4]; and

ii)	Upon receiving a SIP 200 (OK) response to the SIP re-INVITE request the controlling MCPTT function interact with the media plane as specified in 3GPP TS 24.380 [5]; and

NOTE 2:	Subclause 6.3.3.1.15 will inform the affiliated and joined members of the cancellation of the MCPTT group's in-progress emergency state and the cancellation of the MCPTT emergency alert if applicable.

d)	for each of the affiliated members of the group not participating in the call shall:

i)	generate a SIP MESSAGE request notification of the cancellation of the MCPTT user's imminent peril call as specified in subclause 6.3.3.1.11;

ii)	set the <imminentperil-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body to a value of "false"; and

iii)	send the SIP MESSAGE request according to 3GPP TS 24.229 [4];

4)	shall include in the SIP 200 (OK) response an SDP answer according to 3GPP TS 24.229 [4] with the clarifications specified in subclause 6.3.3.2.1;

5)	shall include the "norefersub" option tag in a Supported header field according to IETF RFC 4488 [22];

6)	shall include the "tdialog" option tag in a Supported header field according to IETF RFC 4538 [23];

7)	shall interact with media plane as specified in 3GPP TS 24.380 [5]; and

8)	shall send the SIP 200 (OK) response towards the MCPTT client according to 3GPP TS 24.229 [4].

Upon receipt of a SIP 2xx response for an outgoing SIP MESSAGE request, shall handle according to 3GPP TS 24.229 [4].

#### 10.1.1.5	Non-controlling function of an MCPTT group procedures

##### 10.1.1.5.1	Originating procedures

This subclause describes the procedures for inviting an MCPTT user to an MCPTT session. The procedure is initiated by the non-controlling MCPTT function of an MCPTT group as the result of an action in subclause 10.1.1.5.2 or subclause 10.1.1.5.5.

The non-controlling MCPTT function:

1)	shall invite the MCPTT clients as specified in subclause 6.3.4.1.2;

2)	shall include in each SIP INVITE request an SDP offer based on the SDP offer in the received SIP INVITE request from the controlling MCPTT function according to the procedures specified in subclause 6.3.4.1.1; and

3)	shall send each SIP INVITE request towards the terminating network in accordance with 3GPP TS 24.229 [4].

For each SIP 183 (Session Progress) response received to each SIP INVITE request sent to an MCPTT client, the non-controlling MCPTT function of an MCPTT group:

1)	For each SIP 183 (Session Progress) response containing the option tag "100rel", shall send a SIP PRACK request towards the MCPTT client according to 3GPP TS 24.229 [4]; and

2)	shall cache the received response;

For each SIP 200 (OK) response received to each SIP INVITE request sent to an MCPTT client, the non-controlling MCPTT function of an MCPTT group:

1)	shall cache the SIP 200 (OK) response;

2)	shall start the SIP session timer according to rules and procedures of IETF RFC 4028 [7]; and

3)	if at least one of the participants has subscribed to the conference event package, shall send a SIP NOTIFY request to all participants with a subscription to the conference event package as specified in subclause 10.1.3.5.2.

On receipt of a SIP 3xx, 4xx, 5xx or 6xx response from an invited MCPTT client, the non-controlling MCPTT function of an MCPTT group:

1)	shall send an SIP ACK request towards the MCPTT client as specified in 3GPP TS 24.229 [4];

2)	shall remove the cached provisional responses received from the MCPTT client, if any cached provisional responses exists; and

3)	if the procedures are inititated by the receipt of the "SIP INVITE request for non-controlling MCPTT function of an MCPTT group" as specified in subclause 10.1.1.5.2, shall cache the SIP 3xx, 4xx, 5xx or 6xx response.

##### 10.1.1.5.2	Terminating procedures

###### 10.1.1.5.2.1	General

When receiving the "SIP INVITE request for non-controlling MCPTT function of an MCPTT group" the MCPTT server can be acting as a controller MCPTT function in an ongoing prearranged group call or, if an prearranged group call is not ongoing, be initiated as an non-controlling MCPTT function and invite MCPTT users.

If a prearranged group call is not ongoing the MCPTT server shall perform the actions specified in subclause 10.1.1.5.2.2.

If the "SIP INVITE request for non-controlling MCPTT function of an MCPTT group" is received when a prearranged group call is ongoing, the controlling MCPTT function may switch from operating in a controlling MCPTT function mode to operate in a non-controlling MCPTT function mode as specified in subclause 10.1.1.5.2.3.

When operating in the non-controlling mode and a SIP BYE request is received from the controlling MCPTT function, the non-controlling MCPTT function shall change from operating in the non-controlling mode to operating in the controlling mode as specified in subclause 10.1.1.5.2.4.

###### 10.1.1.5.2.2	Initiating a prearranged group call

Upon receipt of a "SIP INVITE request for non-controlling MCPTT function of an MCPTT group" and if a prearranged group call is not ongoing, the non-controlling MCPTT function of an MCPTT group:

NOTE 1:	The Contact header field of the SIP INVITE request contains the "isfocus" feature media tag.

1)	if unable to process the request due to a lack of resources or a risk of congestion exists, may reject the SIP INVITE request with a SIP 500 (Server Internal Error) response. The controlling MCPTT function may include a Retry-After header field to the SIP 500 (Server Internal Error) response as specified in IETF RFC 3261 [24]. Otherwise, continue with the rest of the steps;

2)	shall determine if the media parameters are acceptable and the MCPTT speech codec is offered in the SDP offer and if not reject the request with a SIP 488 (Not Acceptable Here) response. Otherwise, continue with the rest of the steps;

3)	shall reject the SIP request with a SIP 403 (Forbidden) response and not process the remaining steps if:

a)	an Accept-Contact header field does not include the g.3gpp.mcptt media feature tag; or

b)	an Accept-Contact header field does not include the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt";

4)	if the partner MCPTT system does not have a mutual aid relationship with the primary MCPTT system identified by the contents of the P-Asserted-Identity, shall reject the "SIP INVITE request for non-controlling MCPTT function of an MCPTT group" with a SIP 403 (Forbidden) response, with warning text set to "128 isfocus already assigned" in a Warning header field as specified in subclause 4.4, and shall not process the remaining steps;

5)	if a trusted mutual aid relationship exists between the partner MCPTT system and the primary MCPTT system and the procedure in 3GPP TS 23.179 [3] subclause 10.6.2.4.2 is supported:

a)	shall generate a SIP 403 (Forbidden) response as specified in 3GPP TS 24.229 [4];

b)	shall retrieve the group members of the prearranged group identified by the <mcptt-request-uri> element of the application/vnd.3gpp.mcptt-info+xml MIME body of the SIP INVITE request, as specified in subclause 6.3.5.2;

c)	if the retrieval of group members was successful shall include a P-Refused-URI-List header field populated with affiliated members of the prearranged group in accordance with the IETF RFC 5318 [36]:

d)	if the retrieval of group members was not successful, shall include the warning text set to "128 isfocus already assigned" in a Warning header field as specified in subclause 4.4; and

e)	shall send the SIP 403 (Forbidden) response towards the controlling MCPTT function as specified in 3GPP TS 24.229 [4]; and

f)	shall not process the remaining steps;

6)	shall retrieve the group document from the group management server for the MCPTT group ID contained in the <mcptt-request-uri> element of the application/vnd.3gpp.mcptt-info+xml MIME body of the SIP INVITE request and carry out initial processing as specified in subclause 6.3.5.2 and continue with the rest of the steps if the checks in subclause 6.3.5.2 succeed;

7)	shall cache the content of the SIP INVITE request, if received in the Contact header field and if the specific feature tags are supported;

8)	shall check if a Resource-Priority header field is included in the incoming SIP INVITE request and may apply any preferential treatment to the SIP request as specified in 3GPP TS 24.229 [4];

9)	determine the members to invite to the prearranged MCPTT group call as specified in subclause 6.3.5.5;

10)	if the group document retrieved from the group management server contains <on-network-required> group members as specified in 3GPP TS 24.381 [31], shall send a SIP 183 (Session Progress) response to the SIP INVITE request for non-controlling MCPTT function of an MCPTT group as specified in subclause 6.3.4.2.2.1 and shall populate the response with an application/vnd.3gpp.mcptt-info+xml MIME body containing the <required> element set to "true".

11)	if the group document retrieved from the group management server does not contain any <on-network-required> group members as specified in 3GPP TS 24.381 [31], may, according to local policy, send a SIP 183 (Session Progress) response to the SIP INVITE request for non-controlling MCPTT function of an MCPTT group as specified in subclause 6.3.4.2.2.1;

12)	shall invite each group member determined in step 9) above, to the group session, as specified in subclause 10.1.1.5.1; and

13)	shall interact with the media plane as specified in 3GPP TS 24.380 [5] subclause 6.3;

Unless a SIP response has been sent to the controlling MCPTT function as specified in step 10 or 11 above, the non-controlling MCPTT function of an MCPTT group shall wait for the first SIP provisional response or first SIP 200 (OK) response from one of the invited MCPTT clients, before sending a response to the SIP INVITE request for non-controlling MCPTT function of an MCPTT group.

Upon receiving the first 18x response to a SIP INVITE request sent to an invited MCPTT client as specified in subclause 10.1.1.5.1, not containing a P-Answer-State header field, and if a SIP 183 (Session Progress) response has not already been sent in response to the SIP INVITE request for non-controlling MCPTT function of an MCPTT group, the non-controlling MCPTT function of an MCPTT group:

1)	shall generate a SIP 183 (Session Progress) response as described in subclause 6.3.4.2.2.1; and

2)	shall forward the SIP 183 (Session Progress) response to the controlling MCPTT function according to 3GPP TS 24.229 [4].

Upon receiving the first 18x response to a SIP INVITE request sent to an invited MCPTT client as specified in subclause 10.1.1.5.1, containing a P-Answer-State header field with the value "Unconfirmed" as specified in IETF RFC 4964 [34], a SIP 183 (Session Progress) response has not already been sent in response to the SIP INVITE request for non-controlling MCPTT function of an MCPTT group and the non-controlling MCPTT function of an MCPTT group supports media buffering, the non-controlling MCPTT function of an MCPTT group:

1)	shall generate SIP 200 (OK) response to the SIP INVITE request as specified in the subclause 6.3.4.2.2.2 before continuing with the rest of the steps;

2)	shall include in the SIP 200 (OK) response an SDP answer to the SDP offer in the incoming SIP INVITE request as specified in the subclause 6.3.4.2.1;

3)	shall interact with the media plane as specified in 3GPP TS 24.380 [5] subclause 6.3.5; and

NOTE 2:	Resulting media plane processing is completed before the next step is performed.

4)	shall send a SIP 200 (OK) response to the controlling MCPTT function according to 3GPP TS 24.229 [4].

If the group document does not contain any <on-network-required> group members as specified in 3GPP TS 24.381 [31], then upon receiving the first SIP 200 (OK) response to a SIP INVITE request sent to an invited MCPTT client as specified in subclause 10.1.1.5.1, the non-controlling MCPTT function of an MCPTT group:

1)	shall generate SIP 200 (OK) response to the SIP INVITE request as specified in the subclause 6.3.4.2.2.2 before continuing with the rest of the steps;

2)	shall include in the SIP 200 (OK) response an SDP answer to the SDP offer in the incoming SIP INVITE request as specified in the subclause 6.3.4.2.1;

3)	shall interact with the media plane as specified in 3GPP TS 24.380 [5] subclause 6.3.5; and

NOTE 3:	Resulting media plane processing is completed before the next step is performed.

4)	shall send a SIP 200 (OK) response to the controlling MCPTT function according to 3GPP TS 24.229 [4];

If the group document contains <on-network-required> group member(s) as specified in 3GPP TS 24.381 [31], then the non-controlling MCPTT function of an MCPTT group shall wait until all SIP 200 (OK) responses to SIP INVITE requests have been received from the <on-network-required> MCPTT clients before sending a SIP 200 (OK) response back to the controlling MCPTT function, as specified above.

If all invited MCPTT clients have rejected SIP INVITE requests with a SIP 3xx, 4xx, 5xx or 6xx response, the non-controlling MCPTT function of an MCPTT group:

1)	shall generate a SIP reject response as specified in 3GPP TS 24.229 [4];

2)	shall, from the list of reject response codes cached by the non-controlling MCPTT function of an MCPTT group, select the highest prioritized cached reject response code as specified in IETF RFC 3261 [24]; and

3)	shall send the reject response towards the controlling MCPTT function as specified in 3GPP TS 24.229 [4].

###### 10.1.1.5.2.3	Joining an ongoing prearranged group call

Upon receipt of a "SIP INVITE request for non-controlling MCPTT function of an MCPTT group" and if a prearranged group call is already ongoing, the non-controlling MCPTT function of an MCPTT group:

NOTE 1:	The Contact header field of the SIP INVITE request contains the "isfocus" feature media tag.

1)	shall determine if the media parameters are acceptable and the MCPTT speech codec is offered in the SDP offer and if not, reject the request with a SIP 488 (Not Acceptable Here) response. Otherwise, continue with the rest of the steps;

2)	shall reject the SIP request with a SIP 403 (Forbidden) response and not process the remaining steps if:

a)	an Accept-Contact header field does not include the g.3gpp.mcptt media feature tag; or

b)	an Accept-Contact header field does not include the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt";

3)	if the partner MCPTT system does not have a mutual aid relationship to merged an ongoing prearranged call with the primary MCPTT system identified by the contents of the P-Asserted-Identity, shall reject the "SIP INVITE request for non-controlling MCPTT function of an MCPTT group" with a SIP 403 (Forbidden) response, with warning text set to "128 isfocus already assigned" in a Warning header field as specified in subclause 4.4, and shall not process the remaining steps;

4)	shall cache the content of the SIP INVITE request, if received in the Contact header field and if the specific feature tags are supported;

5)	shall check if a Resource-Priority header field is included in the incoming SIP INVITE request and may apply any preferential treatment to the SIP request as specified in 3GPP TS 24.229 [4];

6)	shall generate SIP 200 (OK) response to the SIP INVITE request as specified in the subclause 6.3.4.2.2.2 before continuing with the rest of the steps;

7)	shall include in the SIP 200 (OK) response an SDP answer to the SDP offer in the incoming SIP INVITE request as specified in the subclause 6.3.4.2.1;

8)	shall instruct the media plane to initialise the switch to non-controlling mode as specified in 3GPP TS 24.380 [5] subclause 6.5.2.3;

NOTE 2:	Resulting media plane processing is completed before the next step is performed. The media plane indicates the state of the floor and if the state is "floor-taken", information about the current speaker.

9)	if the media plane provided information about the current speaker, cache the information about the current speaker(s);

10)	shall send a SIP 200 (OK) response to the controlling MCPTT function according to 3GPP TS 24.229 [4]; and

11)	if at least one of the MCPTT clients in the pre-arranged group session has a subscription to the conference event package, shall subscribe to the conference event package from the controlling MCPTT function as specified in subclause 10.1.3.5.3.

###### 10.1.1.5.2.4	Splitting an ongoing prearranged group call

Upon receipt of a SIP BYE request or a final SIP reject response from the controlling MCPTT function, the non-controlling MCPTT function of an MCPTT group:

1)	if keeping the prearranged group call active is according to the release policy in subclause 6.3.8.1, shall request media plane to switch to controlling mode as specified in 3GPP TS 24.380 [5] subclause 6.3.5;

NOTE 1:	Resulting media plane processing is completed before the next step is performed.

2)	if a SIP BYE request was received, shall send a SIP 200 (OK) response to the SIP BYE request; and

3)	if keeping the prearranged group call active is according to the release policy in subclause 6.3.8.1 and if at least one of the remaining MCPTT clients has subscribed to the conference package, shall send a NOTIFY request to all participants with a subscription to the conference event package as specified in subclause 10.1.3.5.2.

NOTE 2:	The SIP NOTIFY request will indicate that all participants, with the exception of the MCPTT users belonging to the constituent MCPTT group hosted by the non-controlling MCPTT function, have left the group session.

##### 10.1.1.5.3	Rejoin procedures

###### 10.1.1.5.3.1	Terminating procedures

Upon receipt of a SIP INVITE request that includes an MCPTT session identity of an ongoing MCPTT session in the Request-URI the non-controlling MCPTT function act as a controlling MCPTT function towards the MCPTT client and shall perform the actions in the subclause 10.1.1.4.5.1 with the following clarifications:

1)	the MCPTT session identity in the Contact header field of the SIP 200 (OK) response shall be the MCPTT session identity generated by the non-controlling MCPTT function; and

2)	the subclause 10.1.3.5.2 shall be used when sending the SIP NOTIFY request for subscriptions to the conference event package.

###### 10.1.1.5.3.2	Late call entry initiated by non-controlling MCPTT function

When non-controlling MCPTT function is notified that an MCPTT client is newly affiliated or comes back from out of coverage, the non-controlling MCPTT function shall invite the MCPTT client to join an ongoing MCPTT group call by following the procedures specified in subclause 10.1.1.5.1.

NOTE:	How the MCPTT function is informed when an MCPTT client is coming back from out of coverage is out of scope of present document.

##### 10.1.1.5.4	SIP OPTIONS request authorization procedure

Upon receipt of an SIP OPTIONS request containing a P-Asserted-Identity header field containing the public service identity of a MCPTT server not authorized to send the SIP OPTIONS request, the non-controlling MCPTT function of an MCPTT group shall send a SIP 403 (Forbidden) response as specified in 3GPP TS 24.229 and exit this subclause.

Upon receipt of an SIP OPTIONS request containing a P-Asserted-Identity header field containing the public service identity of a MCPTT server authorized to send the SIP OPTIONS request, the non-controlling MCPTT function of an MCPTT group shall perform the actions in this subclause.

The non-controlling MCPTT function shall retrieve the group document from the group management server for the MCPTT group ID contained in the <mcptt-request-uri> element of the application/vnd.3gpp.mcptt-info+xml MIME body of the SIP OPTIONS request with the following clarifications:

NOTE:	The action of the non-controlling MCPTT function of an MCPTT group on receipt of the SIP OPTIONS request mimics the actions of the non-controlling MCPTT function of an MCPTT group on receipt of the SIP INVITE request.

The non-controlling MCPTT function shall:

1)	if the non-controlling MCPTT function fails to retrieve the group document from the group management server, send a shall send the SIP 404 (Not Found) response to the SIP OPTIONS request with the warning text set to "113 group document does not exist" in a Warning header field as specified in subclause 4.4;

2)	if the non-controlling MCPTT function successfully retrieves the group document from the group management server or if the group document was already cached and if one of the following conditions are fulfilled:

a)	if the constituent MCPTT group is a chat group and the rules for joining a group conference as specified in subclause 6.3.5.3 are fulfilled; or

b)	if the constituent MCPTT group is a prearranged group and the rules for initiating a prearranged group session as specified in subclause 6.3.5.4;

then the non-controlling MCPTT function:

a)	shall send the SIP 200 (OK) response to the SIP OTIONS response as specified in 3GPP TS 24.229 [4] and the IETF RFC 3261 [24] populated as follows:

i)	shall include a warning text set to "147 user is authorized to initiate a temporary group call" in a Warning header field as specified in subclause 4.4;

ii)	shall include an application/vnd.3gpp.mcptt-info MIME body with:

A)	the <session-type> element set to "chat", if the constituent MCPTT group is a chat group; and

B)	the <session-type> element set to "prearranged", if the constituent MCPTT group is a prearranged group; and

iii)	shall include the P-Asserted-Identity of the non-controlling MCPTT function of an MCPTT group; and

3)	if none of the conditions in step 2 above) are fulfilled, shall send a SIP 403 (Forbidden) response with the warning text set to "119 user is not authorised to initiate the group call" in a Warning header field as specified in subclause 4.4.

##### 10.1.1.5.5	Initiating a temporary group session

Upon receiving a "SIP INVITE request "SIP INVITE request for controlling MCPTT function of an MCPTT group" when a prearranged group session is not ongoing, the non-controlling MCPTT-function shall:

NOTE 1:	The difference between a "SIP INVITE request "SIP INVITE request for controlling MCPTT function of an MCPTT group" and a "SIP INVITE request for non-controlling MCPTT function of an MCPTT group" is that the latter SIP INVITE request contains the isfocus media feature tag in the Contact header field.

1)	if unable to process the request due to a lack of resources or a risk of congestion exists, may reject the SIP INVITE request with a SIP 500 (Server Internal Error) response. The non-controlling MCPTT function may include a Retry-After header field to the SIP 500 (Server Internal Error) response as specified in IETF RFC 3261 [24]. Otherwise, continue with the rest of the steps;

2)	shall determine if the media parameters are acceptable and the MCPTT speech codec is offered in the SDP offer and if not reject the request with a SIP 488 (Not Acceptable Here) response. Otherwise, continue with the rest of the steps;

3)	shall reject the SIP request with a SIP 403 (Forbidden) response and not process the remaining steps if:

a)	an Accept-Contact header field does not include the g.3gpp.mcptt media feature tag; or

b)	an Accept-Contact header field does not include the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt";

4)	shall retrieve the group document from the group management server for the MCPTT group ID contained in the <mcptt-request-uri> element of the application/vnd.3gpp.mcptt-info+xml MIME body of the SIP INVITE request and carry out initial processing as specified in subclause 6.3.5.2 and continue with the rest of the steps if the checks in subclause 6.3.5.2 succeed;

NOTE 2:	If the checks are not succesful, the SIP response to the "SIP INVITE request "SIP INVITE request for controlling MCPTT function of an MCPTT group" is already sent in the subclause 6.3.5.2.

5)	shall cache the content of the SIP INVITE request;

6)	shall check if a Resource-Priority header field is included in the incoming SIP INVITE request and may apply any preferential treatment to the SIP request as specified in 3GPP TS 24.229 [4];

7)	shall authorize the MCPTT user in the <mcptt-calling-user-identity> element in the application/vnd.3gpp.mcptt-info+xml MIME body of the "SIP INVITE request for controlling MCPTT function of an MCPTT group" as specified in subclause 6.3.5.4, if the MCPTT user is unauthorized to initiated a pre-arranged group session the non-controlling MCPTT function shall send a SIP 403 (Forbidden) response with the warning text set to "119 user is not authorised to initiate the group call" in a Warning header field as specified in subclause 4.4.

8)	shall generate a SIP INVITE request to the controlling MCPTT function as specified in subclause 6.3.4.1.4; and

9)	shall send the SIP INVITE request to the controlling MCPTT function as specified in 3GPP TS 24.229 [4].

Upon receipt of a SIP 2xx response to the SIP INVITE request sent to the controlling MCPTT function as specified above, the non-controlling MCPTT function:

1)	shall send the SIP ACK request to the controlling MCPTT function as specified in 3GPP TS 24.229 [4];

2)	shall generate a SIP 200 (OK) to the "SIP INVITE request for controlling MCPTT function of an MCPTT group" as specified in 3GPP TS 24.229 populated as follows:

a)	shall include an SDP answer as specified in subclause 6.3.4.2.1 based on the SDP answer in the SIP 200 (OK) response;

b)	shall include the public service identifier of the non-controlling MCPTT function in the P-Asserted-Identity header field; and

c)	shall include the warning text set to "148 MCPTT group is regrouped" in a Warning header field as specified in subclause 4.4;

NOTE 3:	As long as the MCPTT group is regrouped the floor control messages in the media plane includes a grouped regrouped indication as specified in 3GPP TS 24.380 [5].

3)	shall start acting as a non-controlling MCPTT function and interact with the media plane as specified in 3GPP TS 24.380 [5] subclause 6.5;

4)	shall determine the members to invite to the prearranged MCPTT group call as specified in subclause 6.3.5.5; and

5)	shall invite each group member determined in step 2) above, to the group session, as specified in subclause 10.1.1.5.1.

Upon receipt of other final SIP responses with the exception of the SIP 2xx response to the INVITE request sent to the controlling MCPTT function as specified above, the non-controlling MCPTT function:

1)	shall send the SIP ACK response to the controlling MCPTT function as specified in 3GPP TS 24.229 [4]; and

2)	shall start acting as a controlling MCPTT function as specified in subclause 10.1.1.4 and invite members as specified in subclause 6.3.4.1.2.

NOTE 4:	Regardless if the controlling MCPTT function accepts or rejects the SIP INVITE request sent above the prearranged group session continues to be initiated with only the members of the group homed on the non-controlling MCPTT function of the group being invited to the group call.

The non-controlling MCPTT function shall handle SIP responses (other than the SIP 2xx response) to the SIP INVITE requests sent to invited members as specified in 3GPP TS 24.229.

Upon receipt of a SIP 2xx response to SIP INVITE requests sent to invited members, the non-controlling MCPTT function:

1)	shall send the SIP ACK request as specified in 3GPP TS 24.229 [4]; and

2)	shall interact with the media plane as specified in 3GPP TS 24.380 [5].

### 10.1.2	Chat group (restricted) call

#### 10.1.2.1	General

#### 10.1.2.2	MCPTT client procedures

##### 10.1.2.2.1	On-demand chat group call

###### 10.1.2.2.1.1	MCPTT client joins a chat MCPTT group session

Upon receiving a request from an MCPTT user to establish an MCPTT group session using an MCPTT group identity, identifying a chat MCPTT group, the MCPTT client shall generate an initial SIP INVITE request by following the UE originating session procedures specified in 3GPP TS 24.229 [4], with the clarifications given below.

The MCPTT client:

1)	if the MCPTT user has requested the origination of an MCPTT emergency group call or is originating an MCPTT chat group call and the MCPTT emergency state is already set, the MCPTT client shall comply with the procedures in subclause 6.2.8.1.1;

2)	if the MCPTT user has requested the origination of an MCPTT imminent peril group call, the MCPTT client shall comply with the procedures in subclause 6.2.8.1.9;

3)	shall include the g.3gpp.mcptt media feature tag and the g.3gpp.icsi-ref media feature tag with the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" in the Contact header field of the SIP INVITE request according to IETF RFC 3840 [16];

4)	shall include an Accept-Contact header field containing the g.3gpp.mcptt media feature tag along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [6];

5)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Preferred-Service header field according to IETF RFC 6050 [9] in the SIP INVITE request;

6)	shall include an Accept-Contact header field with the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [6];

7)	should include the "timer" option tag in the Supported header field;

8)	should include the Session-Expires header field according to IETF RFC 4028 [7]. It is recommended that the refresher parameter is omitted. If included, the refresher parameter shall be set to "uac";

9)	shall set the Request-URI of the SIP INVITE request to the public service identity identifying the participating MCPTT function serving the MCPTT user;

NOTE 1:	The MCPTT client is configured with public service identity identifying the participating MCPTT function serving the MCPTT user.

10)	may include a P-Preferred-Identity header field in the SIP INVITE request containing a public user identity as specified in 3GPP TS 24.229 [4];

11)	if the MCPTT emergency state is already set or the MCPTT client emergency group state for this group is set to "MEG 2: in-progress", the MCPTT client shall comply with the procedures in subclause 6.2.8.1.2;

12)	if the MCPTT client imminent peril group state for this group is set to "MIG 2: in-progress" or "MIG 3: confirm-pending" shall include the Resource-Priority header field and comply with the procedures in subclause 6.2.8.1.12;

13)	shall contain an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcpttinfo> element containing the <mcptt-Params> element with:

a)	the <session-type> element set to a value of "chat"; and

b)	the <mcptt-request-uri> element set to the group identity;

NOTE 2:	The MCPTT ID of the originating MCPTT user is not included in the body, as this will be inserted into the body of the SIP INVITE request that is sent by the originating participating MCPTT function.

14)	shall include in the SIP INVITE request an SDP offer according to 3GPP TS 24.229 [4] with the clarifications specified in subclause 6.2.1;

15)	if an implicit floor request is required, shall indicate this as specified in subclause 6.4; and

16)	shall send the SIP INVITE request according to 3GPP TS 24.229 [4].

On receiving a SIP 2xx response to the SIP INVITE request, the MCPTT client:

1)	shall interact with the user plane as specified in 3GPP TS 24.380 [5]; and

2)	if the MCPTT emergency group call state is set to "MEGC 2: emergency-call-requested" or "MEGC 3: emergency-call-granted" or the MCPTT imminent peril group call state is set to "MIGC 2: imminent-peril-call-requested" or "MIGC 3: imminent-peril-call-granted", the MCPTT client shall perform the actions specified in subclause 6.2.8.1.4.

On receiving a SIP 4xx response, a SIP 5xx response or a SIP 6xx response to the SIP INVITE request:

1)	if the MCPTT emergency group call state is set to "MEGC 2: emergency-call-requested" or "MEGC 3: emergency-call-granted"; or

2)	if the MCPTT imminent peril group call state is set to "MIGC 2: imminent-peril-call-requested" or "MIGC 3: imminent-peril-call-granted";

the MCPTT client shall perform the actions specified in subclause 6.2.8.1.5.

On receiving a SIP INFO request where the Request-URI contains an MCPTT session ID identifying an ongoing group session, the MCPTT client shall follow the actions specified in subclause 6.2.8.1.13.

###### 10.1.2.2.1.2	MCPTT client receives SIP re-INVITE request

This subclause covers both on-demand session and pre-established sessions.

Upon receipt of a SIP re-INVITE request the MCPTT client:

1)	if the SIP re-INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcpttinfo> element containing the <mcptt-Params> element with the <emergency-ind> element set to a value of "true":

a)	should display to the MCPTT user the MCPTT ID of the originator of the MCPTT emergency group call and an indication that this is an MCPTT emergency group call;

b)	if the <mcpttinfo> element containing the <mcptt-Params> element contains an <alert-ind> element set to "true", should display to the MCPTT user an indication of the MCPTT emergency alert and associated information;

c)	shall set the MCPTT emergency group state to "MEG 2: in-progress"; and

d)	shall set the MCPTT imminent peril group state to "MIG 1: no-imminent-peril"; otherwise

2)	if the SIP re-INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcpttinfo> element containing the <mcptt-Params> element with the <imminentperil-ind> element set to a value of "true":

a)	should display to the MCPTT user the MCPTT ID of the originator of the MCPTT imminent peril group call and an indication that this is an MCPTT imminent peril group call; and

b)	shall set the MCPTT imminent peril group state to "MIG 2: in-progress";

3)	if the SIP re-INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcpttinfo> element containing the <mcptt-Params> element with the <emergency-ind> element set to a value of "false":

a)	should display to the MCPTT user the MCPTT ID of the MCPTT user cancelling the MCPTT emergency group call;

b)	if the <mcpttinfo> element containing the <mcptt-Params> element contains an <alert-ind> element set to "false":

i)	should display to the MCPTT user an indication of the MCPTT emergency alert cancellation and the MCPTT ID of the MCPTT user cancelling the MCPTT emergency alert; and

ii)	if the SIP re-INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body including an <originated-by> element:

A)	should display to the MCPTT user the MCPTT ID contained in the <originated-by> element of the MCPTT user that originated the MCPTT emergency alert; and

B)	if the MCPTT ID contained in the <originated-by> element is the MCPTT ID of the receiving MCPTT user, shall set the MCPTT emergency alert state to "MEA 1: no-alert";

c)	shall set the MCPTT emergency group state to "MEG 1: no-emergency"; and

d)	if the MCPTT emergency group call state of the group is set to "MEGC 3: emergency-call-granted", shall set the MCPTT emergency group call state of the group to "MEGC 1: emergency-gc-capable";

4)	if the SIP re-INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcpttinfo> element containing the <mcptt-Params> element with the <imminentperil-ind> element set to a value of "false":

a)	should display to the MCPTT user the MCPTT ID of the MCPTT user cancelling the MCPTT imminent peril group call and an indication that this is an MCPTT imminent peril group call; and

b)	shall set the MCPTT imminent peril group state to "MIG 1: no-imminent-peril";

5)	may check if a Resource-Priority header field is included in the incoming SIP re-INVITE request and may perform further actions outside the scope of this specification to act upon an included Resource-Priority header field as specified in 3GPP TS 24.229 [4];

6)	shall accept the SIP re-INVITE request and generate a SIP 200 (OK) response according to rules and procedures of 3GPP TS 24.229 [4];

7)	shall include the g.3gpp.mcptt media feature tag in the Contact header field of the SIP 200 (OK) response;

8)	shall include the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" in the Contact header field of the SIP 200 (OK) response;

9)	if the SIP re-INVITE request was received within an on-demand session, shall include an SDP answer in the SIP 200 (OK) response to the SDP offer in the incoming SIP re-INVITE request according to 3GPP TS 24.229 [4] with the clarifications given in subclause 6.2.2;

10)	if the SIP re-INVITE request was received within a pre-established session, shall include an SDP answer in the SIP 200 (OK) response to the SDP offer in the incoming SIP re-INVITE request according to 3GPP TS 24.229 [4], based upon the parameters already negotiated for the pre-established session; and

NOTE:	The SIP re-INVITE request can be received within an on-demand session or a pre-established session associated with an MCPTT group session. If the SIP re-INVITE request is received within a pre-established session, the media-level section for the MCPTT speech media stream and the media-level section of the media-floor control entity are expected to be the same as was negotiated in the existing pre-established session.

11)	shall send the SIP 200 (OK) response towards the MCPTT server according to rules and procedures of 3GPP TS 24.229 [4].

###### 10.1.2.2.1.3	MCPTT in-progress emergency cancel

This subclause covers both on-demand session and pre-established sessions.

Upon receiving a request from an MCPTT user to cancel the in-progress emergency condition on a chat MCPTT group, the MCPTT client shall generate a SIP re-INVITE request as specified in 3GPP TS 24.229 [4], with the clarifications given below.

The MCPTT client:

1)	if the MCPTT user is not authorised to cancel the in-progress emergency group state of the MCPTT group as determined by the procedures of subclause 6.2.8.1.7, the MCPTT client:

a)	should indicate to the MCPTT user that they are not authorised to cancel the in-progress emergency group state of the MCPTT group; and

b)	shall skip the remaining steps of the current subclause;

2)	shall, if the MCPTT user is cancelling an in-progress emergency condition and optionally an MCPTT emergency alert originated by the MCPTT user,  include an application/vnd.3gpp.mcptt-info+xml MIME body populated as specified in subclause 6.2.8.1.3;

3)	shall, if the MCPTT user is cancelling an in-progress emergency condition and optionally an MCPTT emergency alert originated by another MCPTT user, include an application/vnd.3gpp.mcptt-info+xml MIME body populated as specified in subclause 6.2.8.1.14;

4)	shall, if the SIP re-INVITE request is to be sent within an on-demand session, include in the SIP re-INVITE request an SDP offer according to 3GPP TS 24.229 [4] with the clarifications specified in subclause 6.2.1;

5)	if the SIP re-INVITE request is to be sent within a pre-established session, shall include an SDP offer in the SIP re-INVITE request according to 3GPP TS 24.229 [4], based upon the parameters already negotiated for the pre-established session;

NOTE 1:	The SIP re-INVITE request can be sent within an on-demand session or a pre-established session associated with an MCPTT group session. If the SIP re-INVITE request is sent within a pre-established session, the media-level section for the MCPTT speech media stream and the media-level section of the media-floor control entity are expected to be the same as was negotiated in the existing pre-established session.

6)	shall include a Resource-Priority header field and comply with the procedures in subclause 6.2.8.1.2; and

7)	shall send the SIP re-INVITE request according to 3GPP TS 24.229 [4].

On receiving a SIP 2xx response to the SIP re-INVITE request, the MCPTT client:

1)	shall set the MCPTT emergency group state of the group to "MEG 1: no-emergency";

2)	shall set the MCPTT emergency group call state of the group to "MEGC 1: emergency-gc-capable"; and

3)	if the MCPTT emergency alert state is set to "MEA 4: Emergency-alert-cancel-pending", the sent SIP re-INVITE request did not contain an <originated-by> element in the application/vnd.3gpp.mcptt-info+xml MIME body and the SIP 2xx response to the SIP request for a priority group call does not contain a Warning header field as specified in subclause 4.4 with the warning text containing the mcptt-warn-code set to "149", shall set the MCPTT emergency alert state to "MEA 1: no-alert".

On receiving a SIP 4xx response, SIP 5xx response or SIP 6xx response to the SIP re-INVITE request:

1)	if the SIP 4xx response, SIP 5xx response or SIP 6xx response contains an application/vnd.3gpp.mcptt-info+xml MIME body with an <emergency-ind> element set to a value of "true", the MCPTT client shall set the MCPTT emergency group state as "MEG 2: in-progress";

2)	if the SIP 4xx response, SIP 5xx response or SIP 6xx response contains an application/vnd.3gpp.mcptt-info+xml MIME body with an <alert-ind> element set to a value of "false" and the sent SIP re-INVITE request did not contain an <originated-by> element in the application/vnd.3gpp.mcptt-info+xml MIME body, the MCPTT client shall set the MCPTT emergency alert state to "MEA 1: no-alert"; and

NOTE 2:	This is the case where the MCPTT client requested the cancellation of the MCPTT emergency alert in addition to the MCPTT emergency alert and the cancellation of the MCPTT emergency alert was successful.

3)	if the SIP 4xx response, SIP 5xx response or SIP 6xx response contains an application/vnd.3gpp.mcptt-info+xml MIME body with an <alert-ind> element set to a value of "true" and the sent SIP re-INVITE request did not contain an <originated-by> element in the application/vnd.3gpp.mcptt-info+xml MIME body, the MCPTT client shall set the MCPTT emergency alert state to "MEA 3: emergency-alert-initiated"; and

4)	if the SIP 4xx response, SIP 5xx response or SIP 6xx response did not contain an application/vnd.3gpp.mcptt-info+xml MIME body with an <alert-ind> element and did not contain an <originated-by> element, the MCPTT emergency alert (MEA) state shall revert to its value prior to entering the current procedure.

NOTE 3:	This is the case where the MCPTT client requested the cancellation of the MCPTT emergency alert in addition to the MCPTT emergency alert and the cancellation of the MCPTT emergency alert was not successful.

NOTE 4:	If the in-progress emergency group state cancel request is rejected, the state of the session does not change, i.e. continues with MCPTT emergency group call level priority.

On receiving a SIP INFO request where the Request-URI contains an MCPTT session ID identifying an ongoing group session, the MCPTT client shall follow the actions specified in subclause 6.2.8.1.13.

###### 10.1.2.2.1.4	MCPTT upgrade to in-progress emergency or imminent peril

This subclause covers both on-demand session and pre-established sessions.

Upon receiving a request from an MCPTT user to upgrade the MCPTT group session to an emergency condition or an imminent peril condition on a chat MCPTT group, the MCPTT client shall generate a SIP re-INVITE request as specified in 3GPP TS 24.229 [4], with the clarifications given below.

1)	if the MCPTT user is requesting to upgrade the MCPTT group session to an in-progress emergency group state and is not authorised to do so as determined by the procedures of subclause 6.2.8.1.8, the MCPTT client:

a)	should indicate to the MCPTT user that they are not authorised to upgrade the MCPTT group session to an in-progress emergency group state; and

b)	shall skip the remaining steps of the current subclause;

2)	if the MCPTT user is requesting to upgrade the MCPTT group session to an in-progress imminent peril state and is not authorised to do so as determined by the procedures of subclause 6.2.8.1.8, the MCPTT client:

a)	should indicate to the MCPTT user that they are not authorised to upgrade the MCPTT group session to an in-progress imminent peril group state; and

b)	shall skip the remaining steps of the current subclause;

3)	if the MCPTT user has requested to upgrade the MCPTT group session to an MCPTT emergency call, the MCPTT client:

a)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body populated as specified in subclause 6.2.8.1.1;

b)	if an indication of an MCPTT emergency alert is to be included, shall perform the procedures specified in subclause 6.2.9.1 for the MCPTT emergency alert trigger; and

c)	shall include a Resource-Priority header field and comply with the procedures in subclause 6.2.8.1.2.

4)	if the MCPTT user has requested to upgrade the MCPTT group session to an MCPTT imminent peril call, the MCPTT client:

a)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body populated as specified in subclause 6.2.8.1.9; and

b)	shall include a Resource-Priority header field and comply with the procedures in subclause 6.2.8.1.12;

5)	if the SIP re-INVITE request is to be sent within an on-demand session, shall include in the SIP re-INVITE request an SDP offer according to 3GPP TS 24.229 [4] with the clarifications specified in subclause 6.2.1;

6)	if the SIP re-INVITE request is to be sent within a pre-established session, shall include an SDP offer in the SIP re-INVITE request according to 3GPP TS 24.229 [4], based upon the parameters already negotiated for the pre-established session;

NOTE:	The SIP re-INVITE request can be sent within an on-demand session or a pre-established session associated with an MCPTT group session. If the SIP re-INVITE request is sent within a pre-established session, the media-level section for the offered MCPTT speech media stream and the media-level section of the offered media-floor control entity are expected to be the same as was negotiated in the existing pre-established session.

7)	if an implicit floor request is required, shall indicate this as specified in subclause 6.4;

8)	shall include a Resource-Priority header field and comply with the procedures in subclause 6.2.8.1.2; and

9)	shall send the SIP re-INVITE request according to 3GPP TS 24.229 [4].

On receiving a SIP 2xx response to the SIP re-INVITE request the MCPTT client:

1)	shall interact with the user plane as specified in 3GPP TS 24.380 [5]; and

2)	shall perform the actions specified in subclause 6.2.8.1.4.

On receiving a SIP 4xx response, SIP 5xx response or SIP 6xx response to the SIP re-INVITE request the MCPTT client shall perform the actions specified in subclause 6.2.8.1.5.

On receiving a SIP INFO request where the Request-URI contains an MCPTT session ID identifying an ongoing group session, the MCPTT client shall follow the actions specified in subclause 6.2.8.1.13.

###### 10.1.2.2.1.5	MCPTT in-progress imminent peril cancel

This subclause covers both on-demand session and pre-established sessions.

Upon receiving a request from an MCPTT user to cancel the in-progress imminent peril condition on a chat MCPTT group, the MCPTT client shall generate a SIP re-INVITE request by following the procedures specified in 3GPP TS 24.229 [4], with the clarifications given below.

The MCPTT client:

1)	if the MCPTT user is not authorised to cancel the in-progress imminent peril group state of the MCPTT group as determined by the procedures of subclause 6.2.8.1.10, the MCPTT client:

a)	should indicate to the MCPTT user that they are not authorised to cancel the in-progress imminent peril group state of the MCPTT group; and

b)	shall skip the remaining steps of the current subclause;

2)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body populated as specified in subclause 6.2.8.1.11;

3)	shall include a Resource-Priority header field and comply with the procedures in subclause 6.2.8.1.12;

4)	shall include in the application/vnd.3gpp.mcptt-info+xml MIME body with the <mcpttinfo> element containing the <mcptt-Params> element with:

a)	the <session-type> element set to a value of "chat"; and

b)	the <mcptt-request-uri> element set to the group identity;

NOTE 1:	The MCPTT ID of the originating MCPTT user is not included in the body, as this will be inserted into the body of the SIP re-INVITE request that is sent by the originating participating MCPTT function.

5)	shall include the g.3gpp.mcptt media feature tag in the Contact header field of the SIP re-INVITE request according to IETF RFC 3840 [16];

6)	if the SIP re-INVITE request is to be sent within an on-demand session, shall include in the SIP re-INVITE request an SDP offer according to 3GPP TS 24.229 [4] with the clarifications specified in subclause 6.2.1;

7)	if the SIP re-INVITE request is to be sent within a pre-established session, shall include an SDP offer in the SIP re-INVITE request according to 3GPP TS 24.229 [4], based upon the parameters already negotiated for the pre-established session; and

NOTE 2:	The SIP re-INVITE request can be sent within an on-demand session or a pre-established session associated with an MCPTT group session. If the SIP re-INVITE request is sent within a pre-established session, the media-level section for the offered MCPTT speech media stream and the media-level section of the offered media-floor control entity are expected to be the same as was negotiated in the existing pre-established session.

8)	shall send the SIP re-INVITE request according to 3GPP TS 24.229 [4].

On receiving a SIP 2xx response to the SIP re-INVITE request, the MCPTT client:

1)	shall interact with the user plane as specified in 3GPP TS 24.380 [5];

2)	shall set the MCPTT imminent peril group state of the group to "MIG 1: no-imminent-peril"; and

3)	shall set the MCPTT imminent peril group call state of the group to "MIGC 1: imminent-peril-gc-capable".

On receiving a SIP 4xx response, SIP 5xx response or SIP 6xx response to the SIP re-INVITE request:

1)	if the SIP 4xx response, SIP 5xx response or SIP 6xx response:

a)	contains an application/vnd.3gpp.mcptt-info+xml MIME body with an <imminentperil-ind> element set to a value of "true"; or

b)	does not contain an application/vnd.3gpp.mcptt-info+xml MIME body with an <imminentperil-ind> element;

then the MCPTT client shall set the MCPTT imminent peril group state as "MIG 2: in-progress".

NOTE 2:	This is the case where the MCPTT client requested the cancellation of the MCPTT imminent peril in-progress state and was rejected.

###### 10.1.2.2.1.6	MCPTT client receives a SIP INVITE request for an MCPTT group call

This procedure is used for MCPTT emergency and MCPTT imminent peril calls when the MCPTT client is affiliated but not joined to the chat group.

In the procedures in this subclause:

1)	emergency indication in an incoming SIP INVITE request refers to the <emergency-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body; and

2)	imminent peril indication in an incoming SIP INVITE request refers to the <imminentperil-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body.

Upon receipt of an initial SIP INVITE request, the MCPTT client:

1)	may reject the SIP INVITE request if either of the following conditions is met:

a)	MCPTT client does not have enough resources to handle the call; or

b)	any other reason outside the scope of this specification;

and skip the rest of the steps;

2)	if the SIP INVITE request is rejected in step 1), shall respond toward participating MCPTT function either with appropriate reject code as specified in 3GPP TS 24.229 [4] and warning texts as specified in subclause 4.4.2 or with SIP 480 (Temporarily unavailable) response not including warning texts if the user is authorised to restrict the reason for failure and skip the rest of the steps of this subclause;

NOTE 1:	if the SIP INVITE request contains an emergency indication or imminent peril indication, the MCPTT client can by means beyond the scope of this specification choose to accept the request.

3)	if the SIP INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcpttinfo> element containing the <mcptt-Params> element with the <emergency-ind> element set to a value of "true":

a)	should display to the MCPTT user an indication that this is a SIP INVITE request for an MCPTT emergency group call and:

i)	should display the MCPTT ID of the originator of the MCPTT emergency group call contained in the <mcptt-calling-user-id> element of the application/vnd.3gpp.mcptt-info+xml MIME body;

ii)	should display the MCPTT group identity of the group with the emergency condition contained in the <mcptt-calling-group-id> element; and

iii)	if the <alert-ind> element is set to "true", should display to the MCPTT user an indication of the MCPTT emergency alert and associated information;

b)	shall set the MCPTT emergency group state to "MEG 2: in-progress";

c)	shall set the MCPTT imminent peril group state to "MIG 1: no-imminent-peril"; and

d)	shall set the MCPTT imminent peril group call state to "MIGC 1: imminent-peril-gc-capable"; otherwise

4)	if the SIP INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcpttinfo> element containing the <mcptt-Params> element with the <imminentperil-ind> element set to a value of "true":

a)	should display to the MCPTT user an indication that this is a SIP INVITE request for an MCPTT imminent peril group call and:

i)	should display the MCPTT ID of the originator of the MCPTT imminent peril group call contained in the <mcptt-calling-user-id> element of the application/vnd.3gpp.mcptt-info+xml MIME body; and

ii)	should display the MCPTT group identity of the group with the imminent peril condition contained in the <mcptt-calling-group-id> element; and

b)	shall set the MCPTT imminent peril group state to "MIG 3: in-progress";

5)	shall check if a Resource-Priority header field is included in the incoming SIP INVITE request and may perform further actions outside the scope of this specification to act upon an included Resource-Priority header field as specified in 3GPP TS 24.229 [4];

6)	shall accept the SIP INVITE request and generate a SIP 200 (OK) response according to rules and procedures of 3GPP TS 24.229 [4];

7)	shall include the option tag "timer" in a Require header field of the SIP 200 (OK) response;

8)	shall include the g.3gpp.mcptt media feature tag in the Contact header field of the SIP 200 (OK) response;

9)	shall include the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" in the Contact header field of the SIP 200 (OK) response;

10)	shall include the Session-Expires header field in the SIP 200 (OK) response and start the SIP session timer according to IETF RFC 4028 [7]. If no "refresher" parameter was included in the received SIP INVITE request the "refresher" parameter in the Session-Expires header field shall be set to "uas", otherwise shall include a "refresher" parameter set to the value received in the Session-Expires header field the received SIP INVITE request;

11)	shall include an SDP answer in the SIP 200 (OK) response to the SDP offer in the incoming SIP INVITE request according to 3GPP TS 24.229 [4] with the clarifications given in subclause 6.2.2;

12)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcptt-called-party-id> element set to the MCPTT ID of the called MCPTT user, in the SIP 200 (OK) response;

13)	 shall send the SIP 200 (OK) response towards the MCPTT server according to rules and procedures of 3GPP TS 24.229 [4]; and

14)	shall interact with the media plane as specified in 3GPP TS 24.380 [5].

##### 10.1.2.2.2	Chat group call within a pre-established session

###### 10.1.2.2.2.1	MCPTT client joins a chat MCPTT group session

Upon receiving a request from an MCPTT user to establish an MCPTT group session using an MCPTT group identity identifying a chat MCPTT group within the pre-established session, the MCPTT client shall generate a SIP REFER request as specified in IETF RFC 3515 [25] as updated by IETF RFC 6665 [26] and IETF RFC 7647 [27], and in accordance with the UE procedures specified in 3GPP TS 24.229 [4], with the clarifications given below.

The MCPTT client:

1)	shall set the Request URI of the SIP REFER request to the session identity of the pre-established session;

2)	shall set the Refer-To header field of the SIP REFER request as specified in IETF RFC 3515 [25] with a Content-ID ("cid") Uniform Resource Locator (URL) as specified in IETF RFC 2392 [62] that points to an application/resource-lists MIME body as specified in IETF RFC 5366 [20], and with the Content-ID header field set to this "cid" URL;

3)	shall include in the application/resource-lists MIME body a single <entry> element containing a "uri" attribute set to the chat group identity, extended with the following URI header fields:

NOTE:	Characters that are not formatted as ASCII characters are escaped in the following URI header fields;

a)	the Accept-Contact header field containing the g.3gpp.mcptt media feature tag along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [6];

b)	an Accept-Contact header field with the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [6]; and

c)	an hname "body" URI header field populated with:

i)	an application/sdp MIME body containing an SDP offer, if the session parameters of the pre-established session require modification or if implicit floor control is required, according to the conditions specified in subclause 6.4; and

ii)	an application/vnd.3gpp.mcptt-info MIME body with the <session-type> element set to a value of "chat";

4)	if the MCPTT user has requested the origination of an MCPTT emergency group call or is originating an MCPTT group call and the MCPTT emergency state is already set comply with the procedures in subclause 6.2.8.1.1;

5)	if the MCPTT emergency state is already set or the MCPTT client emergency group state for this group is set to "MEG 2: in-progress" shall include the Resource-Priority header field and comply with the procedures in subclause 6.2.8.1.2;

6)	if the MCPTT user has requested the origination of an MCPTT imminent peril group call, the MCPTT client shall comply with the procedures in subclause 6.2.8.1.9;

7)	if the MCPTT client imminent peril group state for this group is set to "MIG 2: in-progress" or "MIG 3: confirm-pending" shall include the Resource-Priority header field and comply with the procedures in subclause 6.2.8.1.12;

8)	shall include a P-Preferred-Service header field set to the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), according to IETF RFC 6050 [9];

9)	shall include the following according to IETF RFC 4488 [22]:

a)	the option tag "norefersub" in the Supported header field; and

b)	the value "false" in the Refer-Sub header field.

10)	shall include a Target-Dialog header field as specified in IETF RFC 4538 [23] identifying the pre-established session;

11)	shall include the g.3gpp.mcptt media feature tag in the Contact header field of the SIP REFER request according to IETF RFC 3840 [16]; and

12)	shall send the SIP REFER request according to 3GPP TS 24.229 [4].

On receiving a final SIP 2xx response to the SIP REFER request, the MCPTT client:

1)	shall interact with the media plane as specified in 3GPP TS 24.380 [5]; and

2)	if the MCPTT emergency group call state is set to "MEGC 2: emergency-call-requested" or "MEGC 3: emergency-call-granted" or the MCPTT imminent peril group call state is set to "MIGC 2: imminent-peril-call-requested" or "MIGC 3: imminent-peril-call-granted", shall perform the actions specified in subclause 6.2.8.1.4.

On receiving a SIP 4xx response, SIP 5xx response or a SIP 6xx response to the SIP REFER request:

1)	if the MCPTT emergency group call state is set to "MEGC 2: emergency-call-requested" or "MEGC 3: emergency-call-granted"; or

2)	if the MCPTT imminent peril group call state is set to "MIGC 2: imminent-peril-call-requested" or "MIGC 3: imminent-peril-call-granted";

the MCPTT client shall perform the actions specified in subclause 6.2.8.1.5.

On receiving a SIP INFO request where the Request-URI contains an MCPTT session ID identifying an ongoing group session, the MCPTT client shall follow the actions specified in subclause 6.2.8.1.13.

#### 10.1.2.3	Participating MCPTT function procedures

##### 10.1.2.3.1	On-demand chat group call

###### 10.1.2.3.1.1	MCPTT chat session establishment

In the procedures in this subclause:

1)	group identity in an incoming SIP INVITE request refers to the group identity from the <mcptt-request-uri> element of the application/vnd.3gpp.mcptt-info+xml MIME body of the incoming SIP INVITE request;

2)	emergency indication in an incoming SIP INVITE request refers to the <emergency-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body;

3)	imminent peril indication in an incoming SIP INVITE request refers to the <imminentperil-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body.

Upon receipt of a "SIP INVITE request for originating participating MCPTT function" for a group identity identifying a chat MCPTT group containing an application/vnd.3gpp.mcptt-info+xml MIME body with the <session-type> element set to a value of "chat", the participating MCPTT function:

1)	if unable to process the request due to a lack of resources or a risk of congestion exists, may reject the SIP INVITE request with a SIP 500 (Server Internal Error) response. The participating MCPTT function may include a Retry-After header field to the SIP 500 (Server Internal Error) response as specified in IETF RFC 3261 [24]. Otherwise, continue with the rest of the steps;

NOTE 1:	if the SIP INVITE request contains an emergency indication set to a value of "true" or an imminent peril indication set to a value of "true" and this is an authorised request for originating a priority call as determined by subclause 6.3.2.1.8.1, the participating MCPTT function can according to local policy choose to accept the request.

2)	shall determine the MCPTT ID of the calling user from public user identity in the P-Asserted-Identity header field of the SIP INVITE request, and authorise the calling user;

NOTE 2:	The MCPTT ID of the calling user is bound to the public user identity at the time of service authorisation, as documented in subclause 7.3.

3)	if through local policy in the originating participating MCPTT function, the user identified by the MCPTT ID is not authorised to make chat group calls, shall reject the "SIP INVITE request for originating participating MCPTT function" with a SIP 403 (Forbidden) response to the SIP INVITE request, with warning text set to "108 user not authorised to make chat group calls" in a Warning header field as specified in subclause 4.4;

4)	if the MCPTT user is not affiliated with the chat MCPTT group identified by the group identity in the SIP INVITE request, and already has N2 simultaneous affiliations, shall reject the "SIP INVITE request for originating participating MCPTT function" with a SIP 486 (Busy Here) response with the warning text set to "102 too many simultaneous affiliations" in a Warning header field as specified in subclause 4.4. Otherwise, continue with the remaining steps;

NOTE 3:	N2 is the total number of MCPTT groups that an MCPTT user can be affiliated to simultaneously as specified in 3GPP TS 23.179 [3].

NOTE 4:	if the SIP INVITE request contains an emergency indication set to a value of "true" or an imminent peril indication set to a value of "true" and this is an authorised request for originating a priority call as determined by subclause 6.3.2.1.8.1, the participating MCPTT function can according to local policy choose to allow an exception to the N2 limit. Alternatively, a lower priority affiliation of the MCPTT user could be cancelled to allow for the new affiliation.

5)	shall determine if the media parameters are acceptable and the MCPTT speech codec is offered in the SDP offer and if not, reject the request with a SIP 488 (Not Acceptable Here) response. Otherwise, continue with the rest of the steps;

6)	shall check if the number of maximum simultaneous MCPTT group calls supported for the MCPTT user as specified in the <MaxSimultaneousCallsN6> element of the <MCPTT-group-call> element of the MCPTT user profile document (see the MCPTT user profile document in 3GPP TS 24.384 [50]) has been exceeded. If exceeded, the MCPTT function shall respond with a SIP 486 (Busy Here) response with the warning text set to "103 maximum simultaneous MCPTT group calls reached" in a Warning header field as specified in subclause 4.4. Otherwise, continue with the rest of the steps;

NOTE 5:	If the SIP INVITE request contains an emergency indication set to a value of "true" or an imminent peril indication set to a value of "true" and this is an authorised request for originating a priority call as determined by subclause 6.3.2.1.8.1, the participating MCPTT function can according to local policy choose to allow for an exception to the limit for the maximum simultaneous MCPTT sessions supported for the MCPTT user.

7)	shall determine the public service identity of the controlling MCPTT function associated with the group identity in the SIP INVITE request;

NOTE 6:	The public service identity can identify the controlling function in the primary MCPTT system or a partner MCPTT system.

NOTE 7:	How the participating MCPTT server discovers the public service identity of the controlling MCPTT function associated with the group identity is out of scope of the current document.

8)	shall generate a SIP INVITE request as specified in subclause 6.3.2.1.3;

9)	shall set the Request-URI to the public service identity of the controlling function associated with the group identity present in the incoming SIP INVITE request;

10)	shall include the MCPTT ID of the calling user in <mcptt-calling-user-id> element of the application/vnd.3gpp.mcptt-info+xml MIME body of the SIP INVITE request;

11)	shall include in the SIP INVITE request an SDP offer based on the SDP offer in the received SIP INVITE request as specified in subclause 6.3.2.1.1.1;

12)	if the received SIP INVITE request contains an application/vnd.3gpp.location-info+xml MIME body as specified in Annex F.3; and

a)	if not already included, shall include a Content-Type header field set to "application/vnd.3gpp.location-info+xml"; and

b)	if not already copied, shall copy the contents of the application/vnd.3gpp.location-info+xml MIME body received in the SIP INVITE request into an application/vnd.3gpp.location-info+xml MIME body included in the outgoing SIP request;

NOTE 8:	Note that the application/vnd.3gpp.mcptt-info+xml MIME body will already have been copied into the outgoing SIP INVITE request by subclause 6.3.2.1.3.

13) if a Resource-Priority header field was included in the received SIP INVITE request, shall include a Resource-Priority header field according to rules and procedures of IETF RFC 4412 [29] set to the value indicated in the Resource-Priority header field of the SIP INVITE request from the MCPTT client; and

NOTE 9:	The participating MCPTT function will leave verification of the Resource-Priority header field to the controlling MCPTT function.

14) shall forward the SIP INVITE request according to 3GPP TS 24.229 [4].

Upon receipt of a SIP 302 (Moved Temporarily) response to the above SIP INVITE request in step 14), the participating MCPTT function:

1)	shall generate a SIP INVITE request as specified in subclause 6.3.2.1.10;

2)	shall include an SDP offer based upon the SDP offer in the received SIP INVITE request from the MCPTT client as specified in subclause 6.3.2.1.1.1; and

3)	shall forward the SIP INVITE request according to 3GPP TS 24.229 [4];

Upon receipt of a SIP 2xx response to the above SIP INVITE request in step 14) the participating MCPTT function:

1)	shall generate a SIP 200 (OK) response as specified in the subclause 6.3.2.1.5.2;

2)	shall include in the SIP 200 (OK) response an SDP answer as specified in the subclause 6.3.2.1.2.1;

3)	shall include Warning header field(s) that were received in the incoming SIP 200 (OK) response;

4)	shall include the public service identity received in the P-Asserted-Identity header field of the incoming SIP 200 (OK) response into the P-Asserted-Identity header field of the outgoing SIP 200 (OK) response;

5)	shall send the SIP 200 (OK) response to the MCPTT client according to 3GPP TS 24.229 [4]; and

6)	shall interact with the media plane as specified in 3GPP TS 24.380 [5].

Upon receipt of a SIP 403 (Forbidden) response to the above SIP INVITE request in step 14) the participating MCPTT function:

1)	shall generate a SIP 403 (Forbidden) response according to 3GPP TS 24.229 [4];

2)	shall include Warning header field(s) that were received in the incoming SIP 403 (Forbidden) response;

3)	shall forward the SIP 403 (Forbidden) response to the MCPTT client according to 3GPP TS 24.229 [4]; and

4)	shall interact with the media plane as specified in 3GPP TS 24.380 [5].

###### 10.1.2.3.1.2	Reception of a SIP re-INVITE request from served MCPTT client

This subclause covers both on-demand session and pre-established sessions.

Upon receipt of a SIP re-INVITE request for a served MCPTT client of a chat MCPTT group, the participating MCPTT function:

1)	if unable to process the request due to a lack of resources or a risk of congestion exists, may reject the "SIP INVITE request for terminating participating MCPTT function" with a SIP 500 (Server Internal Error) response. The participating MCPTT function may include a Retry-After header field to the SIP 500 (Server Internal Error) response as specified in IETF RFC 3261 [24]. Otherwise, continue with the rest of the steps;

NOTE 1:	If the SIP re-INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <emergency-ind> element set to a value of "true", the participating MCPTT function may by means beyond the scope of this specification choose to accept the request.

2)	shall determine if the media parameters are acceptable and the MCPTT speech codec is offered in the SDP offer and if not, reject the request with a SIP 488 (Not Acceptable Here) response. Otherwise, continue with the rest of the steps;

NOTE 2:	If the received SIP re-INVITE request is received within a pre-established session associated with an MCPTT group session, the media-level section for the offered MCPTT speech media stream and the media-level section of the offered media-floor control entity are expected to be the same as was negotiated in the existing pre-established session.

3)	shall generate an outgoing SIP re-INVITE request as specified in subclause 6.3.2.1.9;

4)	shall, if the SIP re-INVITE request was received within an on-demand session, include in the SIP re-INVITE request an SDP offer based on the SDP offer in the received SIP re-INVITE request as specified in subclause 6.3.2.1.1.1;

5)	shall, if the SIP re-INVITE request was received within a pre-established session, include in the SIP re-INVITE request an SDP offer based upon the previously negotiated SDP for the pre-established session as specified in subclause 6.3.2.1.1.2;

6)	if the received SIP re-INVITE request contains a Resource-Priority header field, shall include a Resource-Priority header field with the contents set as in the received Resource-Priority header field; and

NOTE 3: The controlling MCPTT function will determine the validity of the Resource-Priority header field.

7)	shall forward the SIP re-INVITE request according to 3GPP TS 24.229 [4].

Upon receipt of a SIP 2xx response to the above SIP re-INVITE request in step 6) the participating MCPTT function:

1)	shall generate a SIP 200 (OK) response as specified in the subclause 6.3.2.1.5.2;

2)	if the SIP 200 (OK) response is to be sent within an on-demand session, shall include in the SIP 200 (OK) response an SDP answer as specified in the subclause 6.3.2.1.2.1;

3)	if the SIP 200 (OK) response is to be sent within a pre-established session shall include in the SIP 200 (OK) response an SDP answer based upon the previously negotiated SDP for the pre-established session;

4)	shall include Warning header field(s) that were received in the incoming SIP 200 (OK) response; and

5)	shall send the SIP 200 (OK) response to the MCPTT client according to 3GPP TS 24.229 [4];

Upon receipt of a SIP 403 (Forbidden) response to the sent SIP re-INVITE request the participating MCPTT function:

1)	shall generate a SIP 403 (Forbidden) response according to 3GPP TS 24.229 [4];

2)	shall copy, if included in the received SIP 403 (Forbidden) response, the application/vnd.3gpp.mcptt-info+xml MIME body MIME body to the outgoing SIP (Forbidden) response;

3)	shall include Warning header field(s) that were received in the incoming SIP 403 (Forbidden) response; and

4)	shall forward the SIP 403 (Forbidden) response to the MCPTT client according to 3GPP TS 24.229 [4];

###### 10.1.2.3.1.3	Reception of a SIP INVITE request for terminating MCPTT client

This subclause covers both on-demand session and pre-established sessions.

Upon receipt of a "SIP INVITE request for terminating participating MCPTT function", for a terminating MCPTT client of a chat MCPTT group, the participating MCPTT function:

1)	if unable to process the request due to a lack of resources or a risk of congestion exists, may reject the "SIP INVITE request for terminating participating MCPTT function" with a SIP 500 (Server Internal Error) response. The participating MCPTT function may include a Retry-After header field to the SIP 500 (Server Internal Error) response as specified in IETF RFC 3261 [24]. Otherwise, continue with the rest of the steps;

NOTE:	If the SIP INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <emergency-ind> element set to a value of "true", the participating MCPTT function can by means beyond the scope of this specification choose to accept the request.

2)	shall check the presence of the isfocus media feature tag in the URI of the Contact header field and if it is not present then the participating MCPTT function shall reject the request with a SIP 403 (Forbidden) response with the warning text set to "104 isfocus not assigned" in a Warning header field as specified in subclause 4.4. Otherwise, continue with the rest of the steps;

3)	if:

a)	the invited MCPTT client has a pre-established session without an associated MCPTT session such that:

i)	the media-level section for the offered MCPTT speech media stream is the same as the media-level section for MCPTT speech media stream in the existing pre-established session; and

ii)	the media-level section of the offered media-floor control entity is the same as the media-level section for media-floor control entity in the existing pre-established session;

then the participating MCPTT function may according to local policy perform the actions specified in subclause 10.1.2.3.2.2 and skip the remaining steps of the current procedure;

4)	shall generate a SIP INVITE request as specified in subclause 6.3.2.2.3;

5)	shall set the Request-URI to the public user identity associated with the MCPTT ID of the MCPTT user to be invited based on the contents of the Request-URI of the received "SIP INVITE request for terminating participating MCPTT function";

6)	shall copy the contents of the P-Asserted-Identity header field of the incoming "SIP INVITE request for terminating participating MCPTT function" to the P-Asserted-Identity header field of the outgoing SIP INVITE request;

7)	shall include in the SIP INVITE request an SDP offer based on the SDP offer in the received "SIP INVITE request for terminating participating MCPTT function" as specified in subclause 6.3.2.2.1;

8)	if the received SIP INVITE request contains a Resource-Priority header field, shall include a Resource-Priority header field with the contents set as in the received Resource-Priority header field;

9)	shall perform the procedures specified in subclause 6.3.2.2.9 to include any MIME bodies in the received SIP INVITE request; and

10)	shall send the SIP INVITE request towards the MCPTT client according to 3GPP TS 24.229 [4].

Upon receiving a SIP 200 (OK) response to the above SIP INVITE request sent to the MCPTT client, the participating MCPTT function:

1)	shall generate a SIP 200 (OK) response as described in the subclause 6.3.2.2.4.2;

2)	shall include in the SIP 200 (OK) response an SDP answer based on the SDP answer in the received SIP 200 (OK) response as specified in subclause 6.3.2.2.2.1;

3)	shall interact with the media plane as specified in 3GPP TS 24.380 [5]; and

4)	shall forward the SIP 200 (OK) response according to 3GPP TS 24.229 [4].

###### 10.1.2.3.1.4	Reception of a SIP re-INVITE request for terminating MCPTT client

This subclause covers both on-demand session and pre-established sessions.

Upon receipt of a SIP re-INVITE request for a terminating MCPTT client of a chat MCPTT group, the participating MCPTT function:

1)	shall check if a Resource-Priority header field is included in the incoming SIP re-INVITE request and may perform further actions outside the scope of this specification to act upon an included Resource-Priority header field as specified in 3GPP TS 24.229 [4];

2)	if the outgoing SIP re-INVITE request will be sent in the dialog of a pre-established session, perform the actions in subclause 10.1.2.3.2.2 and skip the remaining steps of the current procedure;

3)	shall generate an outgoing SIP re-INVITE request as specified in subclause 6.3.2.2.10;

4)	shall include in the SIP re-INVITE request an SDP offer based on the SDP offer in the received SIP re-INVITE request as specified in subclause 6.3.2.2.1; and

5)	shall send the SIP re-INVITE request towards the MCPTT client according to 3GPP TS 24.229 [4].

Upon receiving a SIP 200 (OK) response to the above SIP re-INVITE request sent to the MCPTT client, the participating MCPTT function:

1)	shall generate a SIP 200 (OK) response as described in the subclause 6.3.2.2.4.2;

2)	shall include in the SIP 200 (OK) response an SDP answer based on the SDP answer in the received SIP 200 (OK) response as specified in subclause 6.3.2.2.2.1; and

3)	shall forward the SIP 200 (OK) response according to 3GPP TS 24.229 [4].

##### 10.1.2.3.2	Chat group call within a pre-established session

###### 10.1.2.3.2.1	MCPTT chat session establishment

In the procedures in this subclause:

1)	emergency indication in an incoming SIP INVITE request refers to the <emergency-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body; and

2	imminent peril indication in an incoming SIP INVITE request refers to the <imminentperil-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body.

Upon receipt of a "SIP REFER request for a pre-established session", with:

1)	the Refer-To header containing a Content-ID ("cid") Uniform Resource Locator (URL) as specified in IETF RFC 2392 [62] that points to an application/resource-lists MIME body as specified in IETF RFC 5366 [20] containing an <entry> element with a "uri" attribute containing a SIP-URI set to a chat group identity;

2)	a body" URI header field of the SIP-URI specified above containing an application/vnd.3gpp.mcptt-info MIME body with the <session-type> element set to "chat"; and

3)	a Content-ID header field set to the "cid" URL;

the participating MCPTT function:

1)	if unable to process the request due to a lack of resources or a risk of congestion exists, may reject the SIP REFER request with a SIP 500 (Server Internal Error) response. The participating MCPTT function may include a Retry-After header field to the SIP 500 (Server Internal Error) response as specified in IETF RFC 3261 [24] and shall not continue with the rest of the steps;

NOTE 1:	if the SIP REFER request contains an emergency indication or an imminent peril indication set to a value of "true" and this is an authorised request for originating a priority call as determined by subclause 6.3.2.1.8.1, the participating MCPTT function can according to local policy choose to accept the request.

2)	shall determine the MCPTT ID of the calling user from public user identity in the P-Asserted-Identity header field of the SIP REFER request;

NOTE 2:	The MCPTT ID of the calling user is bound to the public user identity at the time of service authorisation, as documented in subclause 7.3.

3)	if the participating MCPTT function cannot find a binding between the public user identity and an MCPTT ID or if the validity period of an existing binding has expired, then the participating MCPTT function shall reject the SIP REFER request with a SIP 404 (Not Found) response with the warning text set to "141 user unknown to the participating function" in a Warning header field as specified in subclause 4.4and shall not continue with any of the remaining steps.

4)	if through local policy in the participating MCPTT function, the user identified by the MCPTT ID is not authorised to make chat group calls, shall reject the SIP REFER request with a SIP 403 (Forbidden) response to the SIP INVITE request, with warning text set to "108 user not authorised to make group calls" in a Warning header field as specified in subclause 4.4.2;

5)	shall retrieve the group identity within the <entry> element of the application/resource-lists MIME body, referenced by the "cid" URL contained in the Refer-To header field of the SIP REFER request;

6)	shall check if the MCPTT user is affiliated to the group identity in the <entry> element of the application/resource-lists MIME body, referenced by the "cid" URL contained in the Refer-To header field of the SIP REFER request. If the MCPTT user is not affiliated with the chat MCPTT group and already has N2 simultaneous affiliations, shall reject the SIP REFER request with a SIP 486 (Busy Here) response with the warning text set to "102 too many simultaneous affiliations" in a Warning header field as specified in subclause 4.4, and shall not continue with the remaining steps;

NOTE 3:	N2 is the total number of MCPTT groups that an MCPTT user can be affiliated to simultaneously as specified in 3GPP TS 23.179 [3].

7)	shall check if the number of maximum simultaneous MCPTT group calls supported for the MCPTT user as specified in the <MaxSimultaneousCallsN6> element of the <MCPTT-group-call> element of the MCPTT user profile document (see the MCPTT user profile document in 3GPP TS 24.384 [50]) has been exceeded. If exceeded, the participating MCPTT function shall respond with a SIP 486 (Busy Here) response with the warning text set to "103 maximum simultaneous MCPTT group calls reached" in a Warning header field as specified in subclause 4.4, and shall not continue with the rest of the steps;

8)	shall check if a Resource-Priority header field is included in the SIP REFER request according to rules and procedures of 3GPP TS 24.229 [4];

Editor's Note [CT1#95, C1-154552]: some action may be needed if a Resource-Priority header field is included here if it has a different value than was present when the pre-established session was established.

9)	shall generate a final SIP 200 (OK) response to the "SIP REFER request for a pre-established session" according to 3GPP TS 24.229 [4];

10)	if the "SIP REFER request for a pre-established session" contained a Refer-Sub header field containing the value "false" and a Supported header field containing "norefersub" value, shall handle the SIP REFER request as specified in 3GPP TS 24.229 [4], IETF RFC 3515 [25] as updated by IETF RFC 6665 [26], and IETF RFC 4488 [22] without establishing an implicit subscription;

11)	shall send the SIP 200 (OK) response to the SIP REFER request towards the MCPTT client according to 3GPP TS 24.229 [4];

NOTE 4:	In accordance with IETF RFC 4488 [22], the participating MCPTT function inserts the Refer-Sub header field containing the value "false" in the SIP 200 (OK) response to the SIP REFER request to indicate that it has not created an implicit subscription.

12)	shall determine the public service identity of the controlling MCPTT function associated with the group identity in the application/resource-lists MIME body pointed to by the "cid" URL in the Refer-To header field of the SIP REFER request. If the participating MCPTT function is unable to identify the controlling MCPTT function associated with the group identity, it shall reject the REFER request with a SIP 404 (Not Found) response with the warning text "142 unable to determine the controlling function" in a Warning header field as specified in subclause 4.4, and shall not continue with the remaining steps;

NOTE 5:	The public service identity can identify the controlling function in the primary MCPTT system or a partner MCPTT system.

NOTE 6:	How the participating MCPTT server discovers the public service identity of the controlling MCPTT function associated with the group identity is out of scope of the current document.

13)	shall generate a SIP INVITE request as specified in subclause 6.3.2.1.4;

14)	shall set the Request-URI of the SIP INVITE request to the public service identity of the controlling MCPTT function associated with the group identity;

15)	shall copy the group identity from the "uri" attribute of the <entry> element of the application/resource-lists MIME body pointed to by the "cid" URL in the Refer-to header field of the SIP REFER request, to the <mcptt-request-uri> element of the application/vnd.3gpp.mcptt-info+xml MIME body in the SIP INVITE request;

16)	shall include a Resource-Priority header field according to rules and procedures of 3GPP TS 24.229 [4] set to the value indicated in the pre-established session establishment request from the MCPTT client; and

17)	shall forward the SIP INVITE request according to 3GPP TS 24.229 [4].

Upon receipt of a SIP 2xx response to the above SIP INVITE request in step 16) the participating MCPTT function shall follow procedures specified in 3GPP TS 24.229 [4], with the clarifications given below:

1)	shall cache the contact received in the Contact header field; and

2)	shall interact with the media plane as specified in 3GPP TS 24.380 [5].

Upon receipt of a SIP 302 (Moved Temporarily) response to the SIP INVITE request in step 16), the participating MCPTT function:

1)	shall generate a SIP INVITE request as specified in subclause 6.3.2.1.10;

2)	shall include in the SIP INVITE request an SDP offer based upon the SDP offer negotiated during the pre-established session establishment, any subsequent pre-established session modification and the SDP offer (if any) included in the"body" URI parameter of the SIP-URI contained in the <entry> element of the application/resource-lists MIME body, referenced by the "cid" URL in the Refer-To header field in the incoming SIP REFER request from the MCPTT client; and

3)	shall forward the SIP INVITE request according to 3GPP TS 24.229 [4];

###### 10.1.2.3.2.2	MCPTT chat session establishment for terminating user within a pre-established session

Upon receipt of a SIP INVITE request or a SIP re-INVITE request for a terminating MCPTT client of a chat MCPTT group using a pre-established session, the participating MCPTT function:

1)	shall generate an outgoing SIP re-INVITE request as specified in subclause 6.3.2.2.10 to be sent within the dialog of the pre-established session;

2)	if the received SIP INVITE request or SIP re-INVITE request contains a Resource-Priority header field, shall include a Resource-Priority header field with the contents set as in the received Resource-Priority header field;

3)	shall include in the SIP re-INVITE request an SDP offer based on the SDP offer in the received SIP INVITE request or SIP re-INVITE request as specified in subclause 6.3.2.1.1.2; and

NOTE:	The media-level section for the offered MCPTT speech media stream and the media-level section of the offered media-floor control entity are expected to be the same as in the existing pre-established session as per the conditions for pre-established session usage specified in subclause 10.1.2.3.1.3.

4)	shall send the SIP re-INVITE request towards the MCPTT client according to 3GPP TS 24.229 [4].

Upon receiving a SIP 200 (OK) response to the above SIP re-INVITE request sent to the MCPTT client, the participating MCPTT function:

1)	shall generate a SIP 200 (OK) response as described in the subclause 6.3.2.2.4.2;

2)	shall include in the SIP 200 (OK) response an SDP answer as specified in subclause 6.3.2.2.2.2; and

3)	shall send the SIP 200 (OK) response according to 3GPP TS 24.229 [4].

#### 10.1.2.4	Controlling MCPTT function procedures

##### 10.1.2.4.1	On-demand chat group call

###### 10.1.2.4.1.1	MCPTT chat session establishment

In the procedures in this subclause:

1)	MCPTT ID in an incoming SIP INVITE request refers to the MCPTT ID of the originating user from the <mcptt-calling-user-id> element of the application/vnd.3gpp.mcptt-info+xml MIME body of the incoming SIP INVITE request;

2)	group identity in an incoming SIP INVITE request refers to the group identity from the <mcptt-request-uri> element of the application/vnd.3gpp.mcptt-info+xml MIME body of the incoming SIP INVITE request;

3)	MCPTT ID in an outgoing SIP INVITE request refers to the MCPTT ID of the called user in the <mcptt-request-uri> element of the application/vnd.3gpp.mcptt-info+xml MIME body of the outgoing SIP INVITE request;

4)	emergency indication in an incoming SIP INVITE request refers to the <emergency-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body; and

5)	alert indication in an incoming SIP INVITE request refers to the <alert-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body.

Upon receipt of a "SIP INVITE request for controlling MCPTT function of an MCPTT group" containing a group identity identifying a chat MCPTT group, the controlling MCPTT function:

1)	if unable to process the request due to a lack of resources or a risk of congestion exists, may reject the SIP INVITE request with a SIP 500 (Server Internal Error) response. The controlling MCPTT function may include a Retry-After header field to the SIP 500 (Server Internal Error) response as specified in IETF RFC 3261 [24] and skip the rest of the steps;

NOTE 1:	If the SIP INVITE request contains an emergency indication set to a value of "true", the controlling MCPTT function can by means beyond the scope of this specification choose to accept the request.

2)	shall reject the SIP request with a SIP 403 (Forbidden) response and not process the remaining steps if:

a)	an Accept-Contact header field does not include the g.3gpp.mcptt media feature tag;

b)	an Accept-Contact header field does not include the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt"; or

c)	the isfocus media feature tag is present in the Contact header field;

3)	if received SIP INVITE request includes an application/vnd.3gpp.mcptt-info+xml MIME body with an <emergency-ind> element included or an <imminent-peril> element included, shall validate the request as described in subclause 6.3.3.1.17;

4)	shall retrieve the necessary group document(s) from the group management server for the group identity contained in the SIP INVITE request and carry out initial processing as specified in subclause 6.3.5.2 and continue with the rest of the steps if the checks in subclause 6.3.5.2 succeed;

5)	if the SIP INVITE request contains unauthorised request for an MCPTT emergency group call as determined by subclause 6.3.3.1.13.2; and

a)	if the SIP INVITE request contains an alert indication set to "true" and this is an authorised request for an MCPTT emergency alert meeting the conditions specified in subclause 6.3.3.1.13.1, shall:

i)	cache the information that this MCPTT user has initiated an MCPTT emergency alert;

ii)	for each of the other affiliated member of the group:

A)	generate a SIP MESSAGE request notification of the MCPTT user's emergency alert indication as specified in subclause 6.3.3.1.11 with the clarifications of subclause 6.3.3.1.12; and

B)	send the SIP MESSAGE request according to 3GPP TS 24.229 [4]; and

iii)	reject the "SIP INVITE request for controlling MCPTT function of an MCPTT group" with a SIP 403 (Forbidden) response to the SIP INVITE request, with the following clarifications:

A)	shall generate the SIP 403 (Forbidden) response as specified in subclause 6.3.3.1.14; and

B)	shall send the SIP 403 (Forbidden) response as specified in 3GPP TS 24.229 [4] and skip the rest of the steps;

b)	if the SIP INVITE request contains an alert indication set to "true" and this is an unauthorised request for an MCPTT emergency alert as specified in subclause 6.3.3.1.13.1, shall reject the "SIP INVITE request for controlling MCPTT function of an MCPTT group" with the following clarifications:

i)	shall generate the SIP 403 (Forbidden) response as specified in subclause 6.3.3.1.14; and

ii)	shall send the SIP 403 (Forbidden) response as specified in 3GPP TS 24.229 [4] and skip the rest of the steps;

c)	if the SIP INVITE request contains an alert indication set to "false" shall reject the SIP INVITE request with a SIP 403 (Forbidden) response with the following clarifications:

i)	shall generate the SIP 403 (Forbidden) response as specified in subclause 6.3.3.1.14; and

ii)	shall send the SIP 403 (Forbidden) response as specified in 3GPP TS 24.229 [4] and skip the rest of the steps;

6)	if the SIP INVITE request contains an unauthorised request for an MCPTT imminent peril group call as determined by subclause 6.3.3.1.13.6, shall reject the SIP INVITE request with a SIP 403 (Forbidden) response with the following clarifications:

a)	shall include in the SIP 403 (Forbidden) response an application/vnd.3gpp.mcptt-info+xml MIME body as specified in clause F.1 with the <mcpttinfo> element containing the <mcptt-Params> element with the <imminentperil-ind> element set to a value of "false"; and

b)	shall send the SIP 403 (Forbidden) response as specified in 3GPP TS 24.229 [4] and skip the rest of the steps;

7)	if a Resource-Priority header field is included in the SIP INVITE request:

a)	if the Resource-Priority header field is set to the value indicated for emergency calls and the SIP INVITE request does not contain an authorised request for an MCPTT emergency call as determined in step 4 above and the in-progress emergency state of the group is set to a value of "false", shall reject the SIP INVITE request with a SIP 403 (Forbidden) response and skip the remaining steps; and

b)	if the Resource-Priority header field is set to the value indicated for imminent peril calls and the SIP INVITE request does not contain an authorised request for an MCPTT imminent peril call as determined in step 5) above and the in-progress imminent peril state of the group is set to a value of "false", shall reject the SIP INVITE request with a SIP 403 (Forbidden) response; and skip the remaining steps;

8)	if the MCPTT user identified by the MCPTT ID in the SIP INVITE request is not affiliated with the chat MCPTT group identified by the group identity in the SIP INVITE request, the controlling MCPTT function shall affiliate the MCPTT user with the MCPTT chat group and store the affiliation information locally;

Editor's Note [CT1#94, C1-153737]: the mechanism for implicit affiliation is to be determined, including how this information is made available to the participating server to enable its check for maximum simultaneous affiliations (N2).

9)	if the MCPTT user was newly affiliated in step 7) above, update the MCPTT user's affiliation status in the controlling MCPTT function;

10)	shall determine if the media parameters are acceptable and the MCPTT speech codec is offered in the SDP offer and if not reject the request with a SIP 488 (Not Acceptable Here) response and skip the rest of the steps;

11)	shall create a chat group session and allocate an MCPTT session identity for the chat group session if the MCPTT chat group session identity does not already exist, and may handle timer TNG3 (group call timer) as specified in subclause 6.3.3.5;

12)	if the chat group session is ongoing and the <on-network-max-participant-count> as specified in 3GPP TS 24.381 [31] is already reached:

a)	if, according to local policy, the user identified by the MCPTT ID in the SIP INVITE request is deemed to have a higher priority than an existing user in the chat group session, may remove a participant from the session by following subclause 10.1.1.4.4.3, and skip the next step; and

NOTE 2:	The local policy for deciding whether to admit a user to a call that has reached its maximum amount of participants can include the <user-priority> and the <participant-type> of the user as well as other information of the user from the group document as specified in 3GPP TS 24.381 [31]. The local policy decisions can also include taking into account whether the imminent-peril indicator or emergency indicator was received in the SIP INVITE request.

b)	shall return a SIP 486 (Busy Here) response with the warning text set to "122 too many participants" to the originating network as specified in subclause 4.4 Otherwise, continue with the rest of the steps;

13)	if the SIP INVITE request contains an emergency indication set to a value of "true" or the in-progress emergency state of the group to "true" the controlling MCPTT function shall:

a)	validate that the SIP INVITE request includes a Resource-Priority header field populated with the values for an MCPTT emergency group call as specified in subclause 6.3.3.1.19, and if not:

i)	perform the actions specified in subclause 6.3.3.1.8;

ii)	send the SIP UPDATE request generated in subclause 6.3.3.1.8 towards the initiator of the SIP INVITE request according to 3GPP TS 24.229 [4]; and

iii)	upon receiving a SIP 200 (OK) response to the SIP UPDATE request sent in subclause 6.3.3.1.8, proceed with the rest of the steps.

NOTE 3:	Verify that the Resource-Priority header is included and properly populated for both ongoing and newly- entered in-progress emergency states of the specified group.

b)	if the in-progress emergency state of the group is set to a value of "true" and this MCPTT user is indicating a new emergency indication:

i)	for each of the other affiliated members of the group generate a SIP MESSAGE request notification of the MCPTT user's emergency indication as specified in subclause 6.3.3.1.11 with the following clarifications:

A)	set the <emergency-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body to a value of "true";

B)	if the received SIP INVITE contains an alert indication set to a value of "true" and this is an authorised request for an MCPTT emergency alert meeting the conditions specified in subclause 6.3.3.1.13.1, perform the procedures specified in subclause 6.3.3.1.12; and

C)	send the SIP MESSAGE request as specified in 3GPP TS 24.229 [4];

ii)	cache the information that this MCPTT user has initiated an MCPTT emergency call; and

iii)	if the SIP INVITE request contains an authorised request for an MCPTT emergency alert as determined in step i) B) above, cache the information that this MCPTT user has initiated an MCPTT emergency alert; and

c)	if the in-progress emergency state of the group is set to a value of "false":

i)	shall set the value of the in-progress emergency state of the group to "true";

ii)	shall start timer TNG2 (in-progress emergency group call timer) and handle its expiry as specified in subclause 6.3.3.1.16;

iii)	shall generate SIP re-INVITE requests for the MCPTT emergency group call to the other affiliated and joined participants of the chat MCPTT group as specified in subclause 6.3.3.1.6;

iv)	shall generate SIP INVITE requests for the MCPTT emergency group call to the affiliated but not joined members of the chat MCPTT group as specified in subclause 6.3.3.1.7;

A)	for each affiliated but not joined member shall send the SIP INVITE request towards the MCPTT client as specified in 3GPP TS 24.229 [4]; and

B)	upon receiving a SIP 200 (OK) response to the SIP INVITE request the controlling MCPTT function shall interact with the media plane as specified in 3GPP TS 24.380 [5];

v)	shall cache the information that this MCPTT user has initiated an MCPTT emergency call; and

vi)	if the <alert-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body is set to "true" and is an authorised request for an MCPTT emergency alert as specified in subclause 6.3.3.1.13.1, shall cache the information that this MCPTT user has initiated an MCPTT emergency alert; and

vii)	if the in-progress imminent peril state of the group is set to a value of "true", shall set it to a value of "false";

14)	if the in-progress emergency state of the group is set to a value of "false" and if the SIP INVITE request contains an imminent peril indication set to a value of "true" or the in-progress imminent peril state of the group to "true", the controlling MCPTT function shall:

a)	validate that the SIP INVITE request includes a Resource-Priority header field populated with the values for an MCPTT imminent peril group call as specified in subclause 6.3.3.1.19, and if not:

i)	perform the actions specified in subclause 6.3.3.1.8;

ii)	send the SIP UPDATE request generated in subclause 6.3.3.1.8 towards the initiator of the SIP INVITE request according to 3GPP TS 24.229 [4]; and

iii)	upon receiving a SIP 200 (OK) response to the SIP UPDATE request sent in subclause 6.3.3.1.8 proceed with the rest of the steps.

NOTE 4:	Verify that the Resource-Priority header is included and properly populated for both ongoing and newly- entered in-progress imminent peril states of the specified group.

b)	if the in-progress imminent peril state of the group is set to a value of "true" and this MCPTT user is indicating a new imminent peril indication:

i)	for each of the other affiliated member of the group generate a SIP MESSAGE request notification of the MCPTT user's imminent peril indication as specified in subclause 6.3.3.1.11 with the following clarifications;

A)	set the <imminentperil-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body to a value of "true"; and

B)	send the SIP MESSAGE request as specified in 3GPP TS 24.229 [4]; and

ii)	cache the information that this MCPTT user has initiated an MCPTT imminent peril call; and

c)	if the in-progress imminent peril state of the group is set to a value of "false":

i)	shall set the value of the in-progress imminent peril state of the group to "true";

ii)	shall generate SIP re-INVITE requests for the MCPTT imminent peril group call to the other affiliated and joined participants of the chat MCPTT group as specified in subclause 6.3.3.1.15;

iv)	shall generate SIP INVITE requests for the MCPTT imminent peril call to the affiliated but not joined members of the chat MCPTT group as specified in subclause 6.3.3.1.7;

A)	for each affiliated but not joined member shall send the SIP INVITE request towards the MCPTT client as specified in 3GPP TS 24.229 [4]; and

B)	Upon receiving a SIP 200 (OK) response to the SIP INVITE request the controlling MCPTT function shall interact with the media plane as specified in 3GPP TS 24.380 [5]; and

v)	shall cache the information that this MCPTT user has initiated an MCPTT imminent peril call;

15)	shall accept the SIP request and generate a SIP 200 (OK) response to the SIP INVITE request according to 3GPP TS 24.229 [4];

16)	shall include in the SIP 200 (OK) response an SDP answer according to 3GPP TS 24.229 [4] with the clarifications specified in subclause 6.3.3.2.1 unless the procedures of subclause 6.3.3.1.8 were performed in step 9) or step 10 above;

17)	should include the Session-Expires header field and start supervising the SIP session according to IETF RFC 4028 [7]. It is recommended that the "refresher" header field parameter is omitted. If included, the "refresher" header field parameter shall be set to "uac";

18)	shall include the "timer" option tag in a Require header field;

19)	shall include the following in a Contact header field:

a)	the g.3gpp.mcptt media feature tag;

b)	the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt";

c)	the MCPTT session identity; and

d)	the media feature tag isfocus;

20)	shall include the "tdialog" option tag in a Supported header field according to IETF RFC 4538 [23];

21)	if the SIP INVITE request contains an alert indication set to a value of "true" and this is an unauthorised request for an MCPTT emergency alert as specified in subclause 6.3.3.1.13.1, shall include in the SIP 200 (OK) response the warning text set to "149 SIP INFO request pending" in a Warning header field as specified in subclause 4.4;

22)	if the received SIP INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <imminentperil-ind> element set to a value of "true" and if the in-progress emergency state of the group is set to a value of "true", shall include in the SIP 200 (OK) response the warning text set to "149 SIP INFO request pending" in a Warning header field as specified in subclause 4.4;

NOTE 5:	In this case, the request was for an imminent peril call but a higher priority MCPTT emergency call was already in progress on the group. Hence, the imminent peril call request aspect of the request is denied but the request is granted with emergency level priority.

23)	shall interact with media plane as specified in 3GPP TS 24.380 [5];

24)	shall send the SIP 200 (OK) response to the MCPTT client according to 3GPP TS 24.229 [4]; and

25)	if the chat group session was already ongoing and if at least one of the participants has subscribed to the conference event package, shall send a SIP NOTIFY request to all participants with a subscription to the conference event package as specified in subclause 10.1.3.4.2.

Upon receiving a SIP ACK to the SIP 200 (OK) response sent towards the inviting MCPTT client, and the SIP 200 (OK) response was sent with the warning text set to "149 SIP INFO request pending" in a Warning header field as specified in subclause 4.4, the controlling MCPTT function shall follow the procedures in subclause 6.3.3.1.18.

###### 10.1.2.4.1.2	Receipt of a SIP re-INVITE request

In the procedures in this subclause:

1)	emergency indication in an incoming SIP re-INVITE request refers to the <emergency-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body; and

2)	imminent peril indication in an incoming SIP re-INVITE request refers to the <imminentperil-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body.

Upon receipt of a SIP re-INVITE request for an MCPTT session identity identifying a chat MCPTT group session, the controlling MCPTT function:

1)	if unable to process the request due to a lack of resources or a risk of congestion exists, may reject the SIP re-INVITE request with a SIP 500 (Server Internal Error) response. The controlling MCPTT function may include a Retry-After header field to the SIP 500 (Server Internal Error) response as specified in IETF RFC 3261 [24] and skip the rest of the steps;

NOTE 1:	if the SIP re-INVITE request contains an emergency indication or an imminent peril indication set to a value of "true" and this is an authorised request for originating an MCPTT emergency group call as determined by subclause 6.3.3.1.13.2, or for originating an MCPTT imminent peril group call as determined by subclause 6.3.3.1.13.5, the controlling MCPTT function can according to local policy choose to accept the request.

2)	if received SIP re-INVITE request includes an application/vnd.3gpp.mcptt-info+xml MIME body with an <emergency-ind> element included or an <imminent-peril> element included, shall validate the request as described in subclause 6.3.3.1.17;

3)	if the SIP re-INVITE request contains an unauthorised request for an MCPTT emergency call as determined by subclause 6.3.3.1.13.2:

a)	if the <alert-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body is set to "true" and this is an authorised request for an MCPTT emergency alert meeting the conditions specified in subclause 6.3.3.1.13.1, shall:

i)	for each of the other affiliated members of the group send an MCPTT emergency alert:

A)	generate a SIP MESSAGE request notification of the MCPTT user's emergency alert indication as specified in subclause 6.3.3.1.11 with the clarifications of subclause 6.3.3.1.12; and

B)	send the SIP MESSAGE request according to 3GPP TS 24.229 [4];

ii)	cache the MCPTT ID of the MCPTT user that has initiated an MCPTT emergency alert; and

iii)	reject the "SIP INVITE request for controlling MCPTT function of an MCPTT group" with a SIP 403 (Forbidden) response to the SIP INVITE request with the following clarifications:

A)	shall generate the SIP 403 (Forbidden) response as specified in subclause 6.3.3.1.14; and

B)	shall send the SIP 403 (Forbidden) response as specified in 3GPP TS 24.229 [4] and skip the rest of the steps;

b)	if the <alert-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body is set to "true" and is an unauthorised request for an MCPTT emergency alert as determined by subclause 6.3.3.1.13.1, shall reject the SIP re-INVITE request with a SIP 403 (Forbidden) response to the SIP INVITE request with the following clarifications:

i)	shall generate the SIP 403 (Forbidden) response as specified in subclause 6.3.3.1.14; and

ii)	shall send the SIP 403 (Forbidden) response as specified in 3GPP TS 24.229 [4]. Otherwise, continue with the rest of the steps; and

c)	if the <alert-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body is set to "false" shall reject the SIP re-INVITE request with a SIP 403 (Forbidden) response to the SIP re-INVITE request, with the following clarifications:

i)	shall generate a SIP 403 (Forbidden) response as specified in subclause 6.3.3.1.14; and

ii)	shall send the SIP 403 (Forbidden) response as specified in 3GPP TS 24.229 [4] and skip the rest of the steps;

4)	if the SIP re-INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <emergency-ind> element set to a value of "true" and is an authorised request to initiate an MCPTT emergency group call as determined by subclause 6.3.3.1.13.2, the controlling MCPTT function shall:

a)	validate that the SIP re-INVITE request includes a Resource-Priority header field is populated correctly for an MCPTT emergency group call as specified in subclause 6.3.3.1.19, and if not:

i)	shall perform the actions specified in subclause 6.3.3.1.8; and

ii)	upon receiving a SIP 200 (OK) response to the SIP UPDATE request sent in subclause 6.3.3.1.8 shall proceed with the rest of the steps.

NOTE 2:	Verify that the Resource-Priority header is included and properly populated for both ongoing and newly-entered in-progress emergency states of the specified group.

b)	if the in-progress emergency state of the group is set to a value of "true" and this MCPTT user is indicating a new emergency indication:

i)	shall cache the MCPTT ID of the MCPTT user that has initiated an MCPTT emergency call;

ii)	if the SIP re-INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <alert-ind> element set to a value of "true" and is an authorised request for an MCPTT emergency alert as determined by subclause 6.3.3.1.13.1, shall cache the MCPTT ID of the MCPTT user that has initiated an MCPTT emergency alert; and

iii)	for each of the other affiliated members of the group, generate a SIP MESSAGE request notification of the MCPTT user's emergency indication as specified in subclause 6.3.3.1.11 with the following clarifications:

A)	set the <emergency-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body to a value of "true";

B)	if the received SIP INVITE contains an alert indication set to a value of "true" and this is an authorised request for an MCPTT emergency alert meeting the conditions specified in subclause 6.3.3.1.13.1, perform the procedures specified in subclause 6.3.3.1.12; and

C)	send the SIP MESSAGE request as specified in 3GPP TS 24.229 [4]; and

c)	if the in-progress emergency state of the group is set to a value of "false":

i)	shall set the value of the in-progress emergency state of the group to "true";

ii)	shall cache the MCPTT ID of the MCPTT user that has initiated an MCPTT emergency call;

iii)	if the SIP re-INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <alert-ind> element set to a value of "true" and this is an authorised request for an MCPTT emergency alert as specified in subclause 6.3.3.1.13.1, shall cache the MCPTT ID of the MCPTT user that has initiated an MCPTT emergency alert;

iv)	shall start timer TNG2 (in-progress emergency group call timer) and handle its expiry as specified in subclause 6.3.3.1.16;

v)	shall generate SIP re-INVITE requests for the MCPTT emergency group call to the other affiliated and joined participants of the chat MCPTT group as specified in subclause 6.3.3.1.6. The MCPTT controlling function:

A)	for each affiliated and joined member shall send the SIP re-INVITE request towards the MCPTT client as specified in 3GPP TS 24.229 [4]; and

B)	Upon receiving a SIP 200 (OK) response to the SIP re-INVITE request the controlling MCPTT function shall interact with the media plane as specified in 3GPP TS 24.380 [5]; and

vi)	shall generate SIP INVITE requests for the MCPTT emergency group call to the affiliated but not joined members of the chat MCPTT group as specified in subclause 6.3.3.1.7. The controlling MCPTT function:

A)	for each affiliated but not joined member shall send the SIP INVITE request towards the MCPTT client as specified in 3GPP TS 24.229 [4]; and

B)	Upon receiving a SIP 200 (OK) response to the SIP INVITE request the controlling MCPTT function shall interact with the media plane as specified in 3GPP TS 24.380 [5]; and

vii)	if the in-progress imminent peril state of the group is set to a value of "true", shall set it to a value of "false";

5)	if the SIP re-INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <emergency-ind> element set to a value of "false" and is an unauthorised request for an MCPTT emergency group call cancellation as determined by subclause 6.3.3.1.13.4:

a)	if an <alert-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body is included and set to "false" and is an authorised request for an MCPTT emergency alert cancellation as determined by subclause 6.3.3.1.13.3 and there is an outstanding MCPTT emergency alert for this MCPTT user:

i)	for each of the other affiliated members of the group shall:

A)	generate a SIP MESSAGE request notification of the cancellation of the MCPTT user's emergency alert as specified in subclause 6.3.3.1.11;

B)	if the received SIP re-INVITE request contains an <originated-by> element in the application/vnd.3gpp.mcptt-info+xml MIME body, copy the contents of the received <originated-by> element to an <originated-by> element in the application/vnd.3gpp.mcptt-info+xml MIME body in the outgoing SIP MESSAGE request;

C) set the <alert-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body to a value of "false"; and

D)	send the SIP MESSAGE request according to 3GPP TS 24.229 [4];

ii)	if the received SIP re-INVITE request contains an <originated-by> element in the application/vnd.3gpp.mcptt-info+xml MIME body, shall clear the cache of the MCPTT ID of the MCPTT user identified by the <originated-by> element as havings an outstanding MCPTT emergency alert; and

iii)	if the received SIP re-INVITE request does not contain an <originated-by> element in the application/vnd.3gpp.mcptt-info+xml MIME body, shall clear the cache of the MCPTT ID of the MCPTT ID of the sender of the SIP re-INVITE request as having an outstanding MCPTT emergency alert; and

iv)	shall reject the SIP re-INVITE request with a SIP 403 (Forbidden) response to the SIP re-INVITE request with the following clarifications:

A)	include in the SIP 403 (Forbidden) response an application/vnd.3gpp.mcptt-info+xml MIME body as specified in clause F.1 with the <mcpttinfo> element containing the <mcptt-Params> element with the <emergency-ind> element set to a value of "true";

B) include in the application/vnd.3gpp.mcptt-info+xml MIME body and <alert-ind> element set to a value of "false"; and

C)	shall send the SIP 403 (Forbidden) response as specified in 3GPP TS 24.229 [4] and skip the rest of the steps; and

b)	if an <alert-ind> element of the mcpttinfo MIME body is included and set to "false" and is an unauthorised request for an MCPTT emergency alert cancellation as determined by subclause 6.3.3.1.13.3 and there is an outstanding MCPTT emergency alert for this MCPTT user:

i)	shall reject the SIP re-INVITE request with a SIP 403 (Forbidden) response to the SIP re-INVITE request with the following clarifications:

A)	include in the SIP 403 (Forbidden) response an application/vnd.3gpp.mcptt-info+xml MIME body as specified in clause F.1 with the <mcpttinfo> element containing the <mcptt-Params> element with the <emergency-ind> element set to a value of "true"; and

B) include in the application/vnd.3gpp.mcptt-info+xml MIME body and <alert-ind> element set to a value of "true"; and

ii)	shall send the SIP 403 (Forbidden) response as specified in 3GPP TS 24.229 [4] and skip the rest of the steps;

6)	if the SIP re-INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <emergency-ind> element set to a value of "false" and is determined to be an authorised request for an MCPTT emergency call cancellation as specified in subclause 6.3.3.1.13.4 and the in-progress emergency state of the group to is set to a value of "true" the controlling MCPTT function shall:

a)	validate that the SIP INVITE request includes a Resource-Priority header field is populated correctly for a normal priority MCPTT group call as specified in subclause 6.3.3.1.19, and if not:

i)	shall perform the actions specified in subclause 6.3.3.1.8; and

ii)	upon receiving a SIP 200 (OK) response to the SIP UPDATE request sent in subclause 6.3.3.1.8 shall proceed with the rest of the steps;

NOTE 3:	Verify that the Resource-Priority header is included and properly populated for an in-progress emergency state cancellation of the specified group.

b)	shall set the in-progress emergency group state of the group to a value of "false";

c)	shall clear the cache of the MCPTT ID of the MCPTT user identified by the <originated-by> element as having an outstanding MCPTT emergency group call;

d)	if an <alert-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body is included and set to "false" and is determined to be an authorised request for an MCPTT emergency alert cancellation as specified in subclause 6.3.3.1.13.3 and there is an outstanding MCPTT emergency alert for this MCPTT user shall:

i)	if the received SIP re-INVITE request contains an <originated-by> element in the application/vnd.3gpp.mcptt-info+xml MIME body, clear the cache of the MCPTT ID of the MCPTT user identified by the <originated-by> element as having an outstanding MCPTT emergency alert; and

ii)	if the received SIP re-INVITE request does not contain an <originated-by> element in the application/vnd.3gpp.mcptt-info+xml MIME body, clear the cache of the MCPTT ID of the sender of the SIP re-INVITE request as having an outstanding MCPTT emergency alert;

e)	shall generate SIP re-INVITE requests to the other affiliated and joined members of the MCPTT group as specified in subclause 6.3.3.1.6. The MCPTT controlling function:

i)	for each affiliated and joined member shall send the SIP re-INVITE request towards the MCPTT client as specified in 3GPP TS 24.229 [4]; and

ii)	Upon receiving a SIP 200 (OK) response to the SIP re-INVITE request the controlling MCPTT function shall interact with the media plane as specified in 3GPP TS 24.380 [5]; and

NOTE 4:	Subclause 6.3.3.1.6 will inform the affiliated and joined members of the cancellation of the MCPTT group's in-progress emergency state and the cancellation of the MCPTT emergency alert if applicable.

f)	for each of the affiliated but not joined members of the group shall:

i)	generate a SIP MESSAGE request notification of the cancellation of the MCPTT user's emergency call as specified in subclause 6.3.3.1.11;

ii)	set the <emergency-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body to a value of "false";

iii)	if indicated above in step d), set the <alert-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body to a value of "false"; and

iv)	send the SIP MESSAGE request according to 3GPP TS 24.229 [4];

7)	if a Resource-Priority header field is included in the SIP INVITE request:

a)	if the Resource-Priority header field is set to the value indicated for emergency calls and the received SIP re-INVITE request does not contain an authorised request for an MCPTT emergency call as determined in step 4) above and the in-progress emergency state of the group is set to a value of "false", shall reject the SIP INVITE request with a SIP 403 (Forbidden) response and skip the remaining steps; or

b)	if the Resource-Priority header field is set to the value indicated for imminent peril calls and the received SIP re-INVITE request does not contain an authorised request for an MCPTT imminent peril call as determined by the procedures of subclause 6.3.3.1.13.5 and the in-progress imminent peril state of the group is set to a value of "false", shall reject the SIP INVITE request with a SIP 403 (Forbidden) response and skip the remaining steps;

8)	if the received SIP re-INVITE request contains an imminent peril indication, shall perform the procedures specified in subclause 10.1.2.4.1.3 and skip the rest of the steps;

9)	shall include in the SIP 200 (OK) response an SDP answer according to 3GPP TS 24.229 [4] with the clarifications specified in subclause 6.3.3.2.1 unless the procedures of subclause 6.3.3.1.8 were performed in step 5) a) i) above;

10)	shall include the "tdialog" option tag in a Supported header field according to IETF RFC 4538 [23];

11)	if the received SIP re-INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <alert-ind> element set to a value of "true" and if this is an unauthorised request for an MCPTT emergency alert as determined by subclause 6.3.3.1.13.1, shall include in the SIP 200 (OK) response the warning text set to "149 SIP INFO request pending" in a Warning header field as specified in subclause 4.4;

12)	if the received SIP re-INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <alert-ind> element set to a value of "false" and if this is an unauthorised request for an MCPTT emergency alert cancellation as determined by subclause 6.3.3.1.13.3, shall include in the SIP 200 (OK) response the warning text set to "149 SIP INFO request pending" in a Warning header field as specified in subclause 4.4;

13)	if the received SIP re-INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <imminentperil-ind> element set to a value of "true", this is an authorised request for an MCPTT imminent peril group call and if the in-progress emergency state of the group is set to a value of "true", shall include in the SIP 200 (OK) response the warning text set to "149 SIP INFO request pending" in a Warning header field as specified in subclause 4.4;

NOTE 5:	In this case, the request was for an imminent peril call but a higher priority MCPTT emergency call was already in progress on the group. Hence, the imminent peril call request aspect of the request is denied but the request is granted with emergency level priority.

14)	shall interact with media plane as specified in 3GPP TS 24.380 [5]; and

15)	shall send the SIP 200 (OK) response towards the MCPTT client according to 3GPP TS 24.229 [4].

Upon receiving a SIP ACK to the SIP 200 (OK) response sent towards the inviting MCPTT client, and the SIP 200 (OK) response was sent with the warning text set to "149 SIP INFO request pending" in a Warning header field as specified in subclause 4.4, the controlling MCPTT function shall follow the procedures in subclause 6.3.3.1.18.

###### 10.1.2.4.1.3	Handling of a SIP re-INVITE request for imminent peril session

In the procedures in this subclause:

1)	imminent peril indication in an incoming SIP INVITE request refers to the <imminentperil-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body.

When the controlling function receives a SIP re-INVITE request with and imminent peril indication, the controlling function:

1)	if the SIP re-INVITE request contains an unauthorised request for an MCPTT imminent peril group call as determined by subclause 6.3.3.1.13.5, shall reject the SIP re-INVITE request with a SIP 403 (Forbidden) response with the following clarifications:

a)	shall include in the SIP 403 (Forbidden) response an application/vnd.3gpp.mcptt-info+xml MIME body as specified in Annex F.1 with the <mcpttinfo> element containing the <mcptt-Params> element with the <imminentperil-ind> element set to a value of "false"; and

b)	shall send the SIP 403 (Forbidden) response as specified in 3GPP TS 24.229 [4] and skip the rest of the steps;

2)	if the in-progress emergency group state of the group is set to a value of "false" and if the SIP re-INVITE request contains an imminent peril indication set to a value of "true" or the in-progress imminent peril state of the group to "true", the controlling MCPTT function shall:

a)	validate that the SIP re-INVITE request includes a Resource-Priority header field with the namespace set to the MCPTT-specific namespace specified in draft-holmberg-dispatch-mcptt-rp-namespace [48] and the priority set to the priority designated for imminent peril calls and if not:

i)	perform the actions specified in subclause 6.3.3.1.8;

ii)	send the SIP UPDATE request generated in subclause 6.3.3.1.8 towards the initiator of the SIP re-INVITE request according to 3GPP TS 24.229 [4]; and

iii)	upon receiving a SIP 200 (OK) response to the SIP UPDATE request sent in subclause 6.3.3.1.8 proceed with the rest of the steps.

NOTE 3:	Verify that the Resource-Priority header is included and properly populated for both ongoing and newly- entered in-progress imminent peril states of the specified group.

b)	if the in-progress imminent peril state of the group is set to a value of "true" and this MCPTT user is indicating a new imminent peril indication:

i)	for each of the other affiliated member of the group generate a SIP MESSAGE request notification of the MCPTT user's imminent peril indication as specified in subclause 6.3.3.1.11 with the following clarifications;

A)	set the <imminentperil-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body to a value of "true"; and

B)	send the SIP MESSAGE request as specified in 3GPP TS 24.229 [4]; and

ii)	cache the information that this MCPTT user has initiated an MCPTT imminent peril call; and

c)	if the in-progress imminent peril state of the group is set to a value of "false":

i)	shall set the value of the in-progress imminent peril state of the group to "true";

ii)	shall generate SIP re-INVITE requests for the MCPTT imminent peril group call to the other affiliated and joined participants of the chat MCPTT group as specified in subclause 6.3.3.1.15;

iii)	shall generate SIP INVITE requests for the MCPTT imminent peril group call to the affiliated but not joined members of the chat MCPTT group as specified in subclause 6.3.3.1.7;

A)	for each affiliated but not joined member shall send the SIP INVITE request towards the MCPTT client as specified in 3GPP TS 24.229 [4]; and

B)	Upon receiving a SIP 200 (OK) response to the SIP INVITE request the controlling MCPTT function shall interact with the media plane as specified in 3GPP TS 24.380 [5]; and

iv)	shall cache the information that this MCPTT user has initiated an MCPTT imminent peril call;

3)	if the SIP re-INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <imminentperil-ind> element set to a value of "false" and is an unauthorised request for an MCPTT imminent peril group call cancellation as determined by subclause 6.3.3.1.13.6 shall:

a)	reject the SIP re-INVITE request with a SIP 403 (Forbidden) response to the SIP re-INVITE request; and

b)	include in the SIP 403 (Forbidden) response:

i)	include in the SIP 403 (Forbidden) response an application/vnd.3gpp.mcptt-info+xml MIME body as specified in Annex F.1 with the <mcpttinfo> element containing the <mcptt-Params> element with the <imminentperil-ind> element set to a value of "false";

ii) send the SIP 403 (Forbidden) response as specified in 3GPP TS 24.229 [4]; and

iii) skip the rest of the steps;

4)	if the SIP re-INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <imminentperil-ind> element set to a value of "false" and is determined to be an authorised request for an MCPTT imminent peril call cancellation as specified in subclause 6.3.3.1.13.6 and the in-progress imminent peril state of the group to is set to a value of "true" the controlling MCPTT function shall:

a)	validate that the SIP re-INVITE request includes a Resource-Priority header field with the namespace set to the MCPTT-specific namespace specified in draft-holmberg-dispatch-mcptt-rp-namespace [48], and the priority set to the priority level designated for a normal priority MCPTT group call, and if not:

i)	shall perform the actions specified in subclause 6.3.3.1.8; and

ii)	upon receiving a SIP 200 (OK) response to the SIP UPDATE request sent in subclause 6.3.3.1.8 shall proceed with the rest of the steps;

NOTE 3:	verify that the Resource-Priority header is included and properly populated for an in-progress emergency group state cancellation of the specified group.

b)	shall set the in-progress imminent peril state of the group to a value of "false";

c)	shall cache the information that this MCPTT user no longer has an outstanding MCPTT imminent peril group call;

d)	shall generate SIP re-INVITES requests to the other affiliated and joined members of the MCPTT group as specified in subclause 6.3.3.1.15. The MCPTT controlling function:

i)	for each affiliated and joined member shall send the SIP re-INVITE request towards the MCPTT client as specified in 3GPP TS 24.229 [4]; and

ii)	Upon receiving a SIP 200 (OK) response to the SIP re-INVITE request the controlling MCPTT function shall interact with the media plane as specified in 3GPP TS 24.380 [5]; and

NOTE 4:	subclause 6.3.3.1.15 will inform the affiliated and joined members of the cancellation of the MCPTT group's in-progress emergency group state and the cancellation of the MCPTT emergency alert if applicable.

e)	for each of the affiliated but not joined members of the group shall:

i)	generate a SIP MESSAGE request notification of the cancellation of the MCPTT user's imminent peril call as specified in subclause 6.3.3.1.11;

ii)	set the <imminentperil-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body to a value of "false"; and

iii)	send the SIP MESSAGE request according to 3GPP TS 24.229 [4];

5)	shall include in the SIP 200 (OK) response an SDP answer according to 3GPP TS 24.229 [4] with the clarifications specified in subclause 6.3.3.2.1 unless the procedures of subclause 6.3.3.1.8 were performed in step 2) or 4) above;

6)	shall include the "norefersub" option tag in a Supported header field according to IETF RFC 4488 [22];

7)	shall include the "tdialog" option tag in a Supported header field according to IETF RFC 4538 [23];

8)	shall interact with media plane as specified in 3GPP TS 24.380 [5]; and

9)	shall send the SIP 200 (OK) response towards the MCPTT client according to 3GPP TS 24.229 [4].

#### 10.1.2.5	Non-controlling function of an MCPTT group procedures

##### 10.1.2.5.1	Terminating procedures

###### 10.1.2.5.1.1	General

When receiving the "SIP INVITE request for non-controlling MCPTT function of an MCPTT group" the MCPTT server can be acting as a controller MCPTT function in an ongoing chat group call or, if a chat group call is not ongoing, be initiated as an non-controlling MCPTT function and invite MCPTT users.

If a chat group call is not ongoing the MCPTT server shall perform the actions specified in subclause 10.1.2.5.1.2.

If the "SIP INVITE request for non-controlling MCPTT function of an MCPTT group" is received when a chat group call is ongoing, the controlling MCPTT function may switch from operating in a controlling MCPTT function mode to operate in a non-controlling MCPTT function mode as specified in subclause 10.1.2.5.1.3.

When operating in the non-controlling mode and a SIP BYE request is received from the controlling MCPTT function, the non-controlling MCPTT function shall change from operating in the non-controlling mode to operating in the controlling mode as specified in subclause 10.1.2.5.1.4.

###### 10.1.2.5.1.2	Initiating a chat group session

Upon receipt of a "SIP INVITE request for non-controlling MCPTT function of an MCPTT group" and if a chat group call is not ongoing, the non-controlling MCPTT function of an MCPTT group:

NOTE 1:	The Contact header field of the SIP INVITE request contains the "isfocus" feature media tag.

1)	if unable to process the request due to a lack of resources or a risk of congestion exists, may reject the SIP INVITE request with a SIP 500 (Server Internal Error) response. The controlling MCPTT function may include a Retry-After header field to the SIP 500 (Server Internal Error) response as specified in IETF RFC 3261 [24]. Otherwise, continue with the rest of the steps;

2)	shall determine if the media parameters are acceptable and the MCPTT speech codec is offered in the SDP offer and if not, reject the request with a SIP 488 (Not Acceptable Here) response. Otherwise, continue with the rest of the steps;

3)	shall reject the SIP request with a SIP 403 (Forbidden) response and not process the remaining steps if:

a)	an Accept-Contact header field does not include the g.3gpp.mcptt media feature tag; or

b)	an Accept-Contact header field does not include the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt";

4)	if the partner MCPTT system does not have a mutual aid relationship with the primary MCPTT system identified by the contents of the P-Asserted-Identity, shall reject the "SIP INVITE request for non-controlling MCPTT function of an MCPTT group" with a SIP 403 (Forbidden) response, with warning text set to "128 isfocus already assigned" in a Warning header field as specified in subclause 4.4, and shall not process the remaining steps;

5)	shall check if a Resource-Priority header field is included in the incoming SIP INVITE request and may apply any preferential treatment to the SIP request as specified in 3GPP TS 24.229 [4];

6)	shall generate SIP 200 (OK) response to the SIP INVITE request as specified in the subclause 6.3.4.2.2.2 before continuing with the rest of the steps;

7)	shall include in the SIP 200 (OK) response an SDP answer to the SDP offer in the incoming SIP INVITE request as specified in the subclause 6.3.4.2.1;

8)	shall interact with the media plane as specified in 3GPP TS 24.380 [5] subclause 6.3.5; and

NOTE 2:	Resulting media plane processing is completed before the next step is performed.

9)	shall send a SIP 200 (OK) response to the controlling MCPTT function according to 3GPP TS 24.229 [4].

###### 10.1.2.5.1.3	Joining an ongoing chat group call

Upon receipt of a "SIP INVITE request for non-controlling MCPTT function of an MCPTT group" and if a chat group call is already ongoing, the non-controlling MCPTT function of an MCPTT group:

NOTE 1:	The Contact header field of the SIP INVITE request contains the "isfocus" feature media tag.

1)	shall determine if the media parameters are acceptable and the MCPTT speech codec is offered in the SDP offer and if not reject the request with a SIP 488 (Not Acceptable Here) response. Otherwise, continue with the rest of the steps;

2)	shall reject the SIP request with a SIP 403 (Forbidden) response and not process the remaining steps if:

a)	an Accept-Contact header field does not include the g.3gpp.mcptt media feature tag; or

b)	an Accept-Contact header field does not include the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt";

3)	if the partner MCPTT system does not have a mutual aid relationship with the primary MCPTT system identified by the contents of the P-Asserted-Identity, shall reject the "SIP INVITE request for non-controlling MCPTT function of an MCPTT group" with a SIP 403 (Forbidden) response, with warning text set to "128 isfocus already assigned" in a Warning header field as specified in subclause 4.4, and shall not process the remaining steps;

4)	shall cache the content of the SIP INVITE request, if received in the Contact header field and if the specific feature tags are supported;

5)	shall check if a Resource-Priority header field is included in the incoming SIP INVITE request and may apply any preferential treatment to the SIP request as specified in 3GPP TS 24.229 [4];

6)	shall generate SIP 200 (OK) response to the SIP INVITE request as specified in the subclause 6.3.4.2.2.2 before continuing with the rest of the steps;

7)	shall include in the SIP 200 (OK) response an SDP answer to the SDP offer in the incoming SIP INVITE request as specified in the subclause 6.3.4.2.1;

8)	shall instruct the media plane to initialise the switch to the non-controlling mode as specified in 3GPP TS 24.380 [5] subclause 6.5.2.3;

NOTE 2:	Resulting media plane processing is completed before the next step is performed. The media plane indicates the state of the floor and if the state is "floor-taken", information about the current speaker(s).

9)	if the media plane provided information about the current speaker(s), cache the information about the current speaker(s); and

10)	shall send a SIP 200 (OK) response to the controlling MCPTT function according to 3GPP TS 24.229 [4].

Upon receipt of the SIP ACK request, the non-controlling MCPTT function of an MCPTT group:

1)	if information about a current speaker(s) is cached:

a)	shall generate a SIP INFO request as specified in subclause 6.3.4.1.3; and

b)	shall send the SIP INFO request to the controlling MCPTT function as specified in 3GPP TS 24.229 [4];

2)	shall instruct the media plane to finalise the switch to the non-controlling mode as specified in 3GPP TS 24.380 [5] subclause 6.3.5.3; and

3)	if at least one of the MCPTT clients in the chat group session has a subscription to the conference event package, shall subscribe to the conference event package from the controlling MCPTT function as specified in subclause 10.1.3.5.3.

###### 10.1.2.5.1.4	Splitting an ongoing chat group call

Upon receipt of a SIP BYE request, the non-controlling MCPTT function of an MCPTT group:

1)	if keeping the chat group call active is according to the release policy in subclause 6.3.8.1, shall request media plane to switch to controlling mode as specified in 3GPP TS 24.380 [5] subclause 6.3.5;

NOTE 1:	Resulting media plane processing is completed before the next step is performed.

2)	shall send a SIP 200 (OK) response to the SIP BYE request; and

3)	if at least one MCPTT client has subscribed to the conference package, shall send a NOTIFY request to all participants with a subscription to the conference event package as specified in subclause 10.1.3.5.2.

NOTE 2:	The SIP NOTIFY request will indicate that all participants, with the exception of the MCPTT users belonging to the constituent MCPTT group hosted by the non-controlling MCPTT function, have left the group session.

###### 10.1.2.5.1.5	MCPTT client joining the temporary group chat session

When acting in the non-controlling connection mode when receiving of a "SIP INVITE request for controlling MCPTT function of an MCPTT group" containing a group identity identifying a constituent chat MCPTT group being part of the temporary group call, the non-controlling MCPTT function shall act as a controlling MCPTT function towards the MCPTT client and shall perform the actions in the subclause 10.1.2.4.1.1 with the following clarifications:

1)	the MCPTT session identity in the Contact header field of the SIP 200 (OK) response shall be the MCPTT session identity generated by the non-controlling MCPTT function; and

2)	the subclause 10.1.3.5.2 shall be used when sending the SIP NOTIFY request for subscriptions to the conference event package.

###### 10.1.2.5.1.6	Receipt of a SIP re-INVITE request from an MCPTT client

Upon receipt of a SIP re-INVITE request from an MCPTT client the non-controlling MCPTT function shall act as the controlling MCPTT function and shall perform the actions in subclause 10.1.2.4.1.2.

###### 10.1.2.5.1.7	SIP OPTIONS request authorization procedure

Upon receipt of an SIP OPTIONS request containing a P-Asserted-Identity header field containing the public service identity of a MCPTT server authorized to send the OPTIONS request, the non-controlling MCPTT function shall perform the actions in subclause 10.1.1.5.4 otherwise the non-controlling MCPTT function shall send a SIP 403 (Forbidden) response as specified in 3GPP TS 24.229.

###### 10.1.2.5.1.8	Initiating a temporary group session

Upon receiving a "SIP INVITE request "SIP INVITE request for controlling MCPTT function of an MCPTT group" when a chat group session is not ongoing, the non-controlling MCPTT-function shall:

NOTE 1:	The difference between a "SIP INVITE request "SIP INVITE request for controlling MCPTT function of an MCPTT group" and a "SIP INVITE request for non-controlling MCPTT function of an MCPTT group" is that the latter SIP INVITE request contains the isfocus media feature tag in the Contact header field.

1)	if unable to process the request due to a lack of resources or a risk of congestion exists, may reject the SIP INVITE request with a SIP 500 (Server Internal Error) response. The non-controlling MCPTT function may include a Retry-After header field to the SIP 500 (Server Internal Error) response as specified in IETF RFC 3261 [24]. Otherwise, continue with the rest of the steps;

2)	shall determine if the media parameters are acceptable and the MCPTT speech codec is offered in the SDP offer and if not reject the request with a SIP 488 (Not Acceptable Here) response. Otherwise, continue with the rest of the steps;

3)	shall reject the SIP request with a SIP 403 (Forbidden) response and not process the remaining steps if:

a)	an Accept-Contact header field does not include the g.3gpp.mcptt media feature tag; or

b)	an Accept-Contact header field does not include the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt";

4)	shall retrieve the group document from the group management server for the MCPTT group ID contained in the <mcptt-request-uri> element of the application/vnd.3gpp.mcptt-info+xml MIME body of the SIP INVITE request and carry out initial processing as specified in subclause 6.3.5.2 and continue with the rest of the steps if the checks in subclause 6.3.5.2 succeed;

NOTE 2:	If the checks are not succesful, the SIP response to the "SIP INVITE request "SIP INVITE request for controlling MCPTT function of an MCPTT group" is already sent in the subclause 6.3.5.2.

5)	shall cache the content of the SIP INVITE request;

6)	shall check if a Resource-Priority header field is included in the incoming SIP INVITE request and may apply any preferential treatment to the SIP request as specified in 3GPP TS 24.229 [4];

7)	shall authorize the MCPTT user in the <mcptt-calling-user-id> element in the application/vnd.3gpp.mcptt-info+xml MIME body of the "SIP INVITE request for controlling MCPTT function of an MCPTT group" as specified in subclause 6.3.5.2, if the MCPTT user is unauthorized to join a chat group session, the non-controlling MCPTT function shall send a SIP 403 (Forbidden) response with the warning text set to "106 user not authorised to join chat group" in a Warning header field as specified in subclause 4.4.

8)	shall generate a SIP INVITE request to the controlling MCPTT function as specified in subclause 6.3.4.1.4; and

9)	shall send the SIP INVITE request to the controlling MCPTT function as specified in 3GPP TS 24.229 [4].

Upon receipt of a SIP 2xx response to the SIP INVITE request sent to the controlling MCPTT function as specified above, the non-controlling MCPTT function:

1)	shall send the SIP ACK request to the controlling MCPTT function as specified in 3GPP TS 24.229 [4];

2)	shall generate a SIP 200 (OK) to the "SIP INVITE request for controlling MCPTT function of an MCPTT group" as specified in 3GPP TS 24.229 populated as follows:

a)	shall include an SDP answer as specified in subclause 6.3.4.2.1 based on the SDP answer in the SIP 200 (OK) response;

b)	shall include the public service identifier of the non-controlling MCPTT function in the P-Asserted-Identity header field; and

c)	shall include the warning text set to "148 MCPTT group is regrouped" in a Warning header field as specified in subclause 4.4; and

NOTE 3:	As long as the MCPTT group is regrouped the floor control messages in the media plane includes a grouped regrouped indication as specified in 3GPP TS 24.380 [5].

3)	shall start acting as a non-controlling MCPTT function and interact with the media plane as specified in 3GPP TS 24.380 [5] subclause 6.5.

Upon receipt of other final SIP responses with the exception of the SIP 2xx response to the INVITE request sent to the controlling MCPTT function as specified above, the non-controlling MCPTT function:

1)	shall send the SIP ACK response to the controlling MCPTT function as specified in 3GPP TS 24.229 [4]; and

2)	perform the actions in the subclause 10.1.1.5.2.4.

NOTE 4:	Regardless if the controlling MCPTT function accepts or rejects the SIP INVITE request sent above the prearranged group session continues to be initiated with only the members of the group homed on the non-controlling MCPTT function of the group being invited to the group call.

### 10.1.3	Subscription to the conference event package

#### 10.1.3.1	General

The IETF RFC 4575 [30] defines a conference state event package that shall be used to obtain the status of participants in group sessions.

The MCPTT client may subscribe to the conference state event package at any time in a group session that the MCPTT client participates in. The subclause 10.1.3.2 specifies the procedures in the MCPTT client when subscribing to the conference events.

The participating MCPTT function shall forward conference state subscriptions and notifications as specified in subclause 10.1.3.3.

The controlling MCPTT function shall handle subscriptions and notification of conference state events as specified in subclause 10.1.3.4.

The non-controlling MCPTT function shall handle subscriptions and notification of conference state events as specified in subclause 10.1.3.4.

When the non-controlling MCPTT function connection model is used, the controlling MCPTT function subscribes to the conference state event package from the non-controlling MCPTT function as specified in subclause 10.1.3.4.3 and the non-controlling MCPTT function subscribes to the conference state event package from the controlling MCPTT function as specified in subclause 10.1.3.5.3.

#### 10.1.3.2	MCPTT client

A MCPTT client may subscribe to the conference state event package when a group call is ongoing and the ongoing group call is not initiated as a broadcast group call by sending a SIP SUBSCRIBE request to obtain information of the status of a group session.

When subscribing to the conference state event package, the MCPTT client:

1)	shall generate a SIP SUBSCRIBE request and use a new SIP-dialog according to IETF RFC 6665 [26], IETF RFC 4575 [30] and 3GPP TS 24.229 [4];

2)	shall set the Request-URI of the SIP SUBSCRIBE request to the MCPTT session identity of the group session;

3)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Preferred-Service header field according to IETF RFC 6050 [9];

4)	shall include an Accept-Contact header with the media feature tag g.3gpp.icsi-ref with the value "urn:urn-7:3gpp-service.ims.icsi.mcptt" along with "require" and "explicit" header field parameters according to IETF RFC 3841 [6];

5)	if the MCPTT client wants to receive the current status and later notification, shall set the Expires header field according to IETF RFC 6665 [26], to 4294967295;

NOTE 1:	4294967295, which is equal to 232-1, is the highest value defined for Expires header field in IETF RFC 3261 [24].

6)	if the MCPTT client wants to fetch the current state only, shall set the Expires header field according to IETF RFC 6665 [26], to zero;

7)	shall include an Accept header field containing the application/conference-info+xml"MIME type;

8)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcptt-request-uri> element set to the MCPTT group ID of the group session; and

9)	shall send the SIP SUBSCRIBE request using a new SIP dialog according to 3GPP TS 24.229 [4].

The responses to the SIP SUBSCRIBE request shall be handled according to IETF RFC 6665 [26], IETF RFC 4575 [30] and TS 24.229 [4].

Upon receiving a SIP NOTIFY requests to the previously sent SIP SUBSCRIBE request the MCPTT client:

1)	shall handle the request according to IETF RFC 6665 [26] and IETF RFC 4575 [30]; and

2)	may display the current state information to the MCPTT client based on the information in the SIP NOTIFY request body.

When needed the MCPTT client shall terminate the subscription and indicate it terminated according to IETF RFC 6665 [26].

NOTE 2:	The contents of the received SIP NOTIFY request body is specified in subclause 6.3.3.4.

#### 10.1.3.3	Participating MCPTT function

Upon receipt of a SIP SUBSCRIBE request for conference event status subscription from a MCPTT user served by the participating MCPTT function and if the SIP SUBSCRIBE request contains:

1)	the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Asserted-Service header field according to IETF RFC 6050 [9];

2)	an Accept header field containing the application/conference-info+xml"MIME type; and

3)	an application/vnd.3gpp.mcptt-info+xml MIME body containing the <mcptt-request-uri> set to a MCPTT group ID;

then the participating MCPTT function:

1)	shall attempt to resolve the received Request-URI to an existing MCPTT session identity;

2)	if the participating MCPTT function could not resolve the received Request-URI to an existing MCPTT session identity, shall reject the SIP SUBSCRIBE response with a SIP 404 (Not Found) response with a warning text set to "137 the indicated group call does not exists" as specified in subclause 4.4 and shall skip the rest of the steps

3)	shall generate a SUBSCRIBE request as specified in TS 24.229 [4]

4)	shall set the SIP URI in the Request-URI with the MCPTT session identity that is mapped to the MCPTT session identity in the received Request-URI;

5)	shall include in the application/vnd.3gpp.mcptt-info+xml MIME body the <mcptt-calling-user-id> element set to the MCPTT ID of the served user: and

6)	shall insert a Record-Route header containing a URI identifying its own address; and

7)	shall send the SIP SUBSCRIBE request according to 3GPP TS 24.229 [4].

Upon receiving a SIP response to the SIP SUBSCRIBE request the participating MCPTT function:

1)	shall copy the content of the incoming SIP response to an outgoing SIP response;

2)	if a SIP 200 (OK) response, shall include in the Contact header field of the outgoing SIP response an MCPTT session identity mapped to the MCPTT session identity provided in the Contact header field of the received SIP 200 (OK) response in the outgoing SIP response; and

3)	shall forward the SIP response according to 3GPP TS 24.229 [4].

Upon receiving a SIP NOTIFY request within the dialog created by the SIP SUBSCRIBE request destined to a served MCPTT client, the participating MCPTT function:

1)	shall include the public service identity of the MCPTT user in the Request-URI;

2)	shall copy the content of the incoming SIP NOTIFY request to the outgoing SIP NOTIFY request; and

3)	shall send the SIP NOTIFY request according to 3GPP TS 24.229 [4].

Upon receiving a SIP response to the SIP NOTIFY request the participating MCPTT function:

1)	shall copy the content of the incoming SIP response to an outgoing SIP response;

2)	if a SIP 200 (OK) response, shall include an MCPTT session identity constructed from the MCPTT session identity provided in the Contact header field of the received SIP 200 (OK) response in the outgoing SIP response; and

3)	shall forward the SIP response according to 3GPP TS 24.229 [4].

#### 10.1.3.4	Controlling MCPTT function

##### 10.1.3.4.1	Receiving a subscription to the conference event package

Upon receipt of a SIP SUBSCRIBE request for event package subscription in the controlling MCPTT function and the SIP SUBSCRIBE request:

1)	contains an application/vnd.3gpp.mcptt-info+xml MIME body with

a)	the <mcptt-request-uri> element set to the group identity of the group session and the <mcptt-calling-user-id> element set to either:

i)	the MCPTT ID of a participant in the group session; or

ii)	a constituent MCPTT group ID of a non-controlling MCPTT function in a temporary group session;

2)	contains the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Asserted-Service header field according to IETF RFC 6050 [9];

3)	contains an Accept header field containing the application/conference-info+xml MIME type; and

4)	is not received in a group call initiated as a broadcast group call;

then the controlling MCPTT function:

1)	shall check if the <on-network-allow-conference-state> element in the group document in 3GPP TS 24.381 [31] allows the MCPTT ID or the constituent MCPTT group ID in the <mcptt-calling-user-id> element to subscribe to the conference event package and if not allowed:

a)	shall reject the "SIP SUBSCRIBE request for event status subscription in the controlling MCPTT function" with a SIP 403 (Forbidden) response to the SIP SUBSCRIBE request, with warning text set to "138 subscription of conference events not allowed" as specified in subclause 4.4; and

b)	shall not continue with the remaining steps;

2)	shall handle the request according to IETF RFC 6665 [26] and IETF RFC 4575 [30];

3)	shall cache information about the subscription;

4)	shall send a conference state notification as specified in subclause 10.1.3.4.2; and

5)	if the SIP SUBSCRIBE request is the first SUBSCRIBE request from a participant in a temporary group session, shall subscribe to the conference event package from all non-controlling MCPTT functions in the group session as specified in subclause 10.1.3.4.3.

Upon receipt of a SIP SUBSCRIBE request for event package subscription in the controlling MCPTT function in an group call initiated as a broadcast group call, the controlling MCPTT function:

1)	shall generate a SIP 480 (Temporarily Unavailable) response to the SIP SUBSCRIBE request as specified in 3GPP TS 24.229 [4];

2)	shall include a Warning header field with the warning text set to "105 subscription not allowed in a broadcast group call" as specified in subclause 4.4; and

3)	send the SIP 480 (Temporarily Unavailable) response according to 3GPP TS 24.229 [4].

##### 10.1.3.4.2	Sending notifications to the conference event package

The procedures in this subclause is triggered by:

1)	the receipt of a SIP SUBSCRIBE request as specified in subclause 10.1.3.4.1;

2)	the receipt of a SIP BYE request from one of the participants in a pre-arranged or a chat group session; or

3)	when a new participant is added in a pre-arranged or chat group session.

When sending a conference state event notification, the controlling MCPTT function:

1)	shall generate a notification package as specified in subclause 6.3.3.4 to all MCPTT clients which have subscribed to the conference state event package; and

NOTE:	As a group document can potentially have a large content, the controlling MCPTT function can notify using content-indirection as defined in IETF RFC 4483 [32].

2)	shall send a SIP NOTIFY request to all participants which have subscribed to the conference state event package as specified in 3GPP TS 24.229 [4].

##### 10.1.3.4.3	Sending subscriptions to the conference event package

The procedure in this subclause is triggered by:

1)	the receipt of a SIP 200 (OK) response to a SIP INVITE request for non-controlling MCPTT function of an MCPTT group and if at least one participant already has subscribed to the conference event package in the controlling MCPTT function as specified in subclause 10.1.3.4.1; or

2)	the receipt of the first SIP SUBSCRIBE request as specified in subclause 10.1.3.4.1 and one or more participant in the group session is a non-controlling MCPTT function;

then, for each non-controlling MCPTT function from where a SIP 200 (OK) response to a SIP INVITE request for non-controlling MCPTT function of an MCPTT group has been received and where a SIP SUBSCRIBE request is not already sent, the controlling MCPTT function:

1)	shall generate a SIP SUBSCRIBE request and use a new SIP-dialog according to IETF RFC 6665 [26], IETF RFC 4575 [30] and 3GPP TS 24.229 [4];

2)	shall set the Request-URI of the SIP SUBSCRIBE request to the public service identity of the non-controlling MCPTT function serving the group identity of the MCPTT group owned by the partner MCPTT system;

3)	shall include the same P-Asserted-Identity header field as included in the SIP INVITE request for non-controlling MCPTT function of an MCPTT group;

4)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Preferred-Service header field according to IETF RFC 6050 [9];

5)	shall include an Accept-Contact header with the g.3gpp.mcptt along with "require" and "explicit" header field parameters according to IETF RFC 3841 [6];

6)	shall set the Expires header field according to IETF RFC 6665 [26], to 4294967295;

NOTE:	4294967295, which is equal to 232-1, is the highest value defined for Expires header field in IETF RFC 3261 [24].

7)	shall include an Accept header field containing the application/conference-info+xml MIME type;

8)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body with:

a)	the <mcptt-request-uri> element set to the constituent MCPTT group ID; and

b)	the <mcptt-calling-group-id> set to the temporary MCPTT group ID; and

9)	shall send the SIP SUBSCRIBE request using a new SIP dialog according to 3GPP TS 24.229 [4].

The responses to the SIP SUBSCRIBE request shall be handled according to IETF RFC 6665 [26], IETF RFC 4575 [30] and TS 24.229 [4].

Upon receiving an incoming SIP NOTIFY requests to the previously sent SIP SUBSCRIBE request, the controlling MCPTT function:

1)	shall handle the request according to IETF RFC 6665 [26] and IETF RFC 4575 [30];

2)	shall modify the SIP NOTIFY request as specified in subclause 6.3.3.4; and

3)	shall forward the modified SIP NOTIFY request according to 3GPP TS 24.229 [4] to all other participants with a subscription to the conference event package.

NOTE:	A non-controlling MCPTT function of an MCPTT group is regarded as a participant in a temporary group session.

##### 10.1.3.4.4	Terminating a subscription

Upon receipt of a SIP SUBSCRIBE request for event status subscription in the controlling MCPTT function that terminates the subscription of the conference event package as specified in IETF RFC 6665 [26], the controlling MCPTT function:

1)	shall send a SIP 200 (OK) response as specified in IETF RFC 6665 [26]; and

2)	if there are no remaining subscriptions to the event package in the ongoing MCPTT call in a temporary group session, shall terminate the subscriptions to the conference event package as specified in IETF RFC 6665 [26] in all non-controlling MCPTT functions in the temporary group session.

Upon expiry of the subscription timer and if there are no remaining subscriptions to the event package in the ongoing MCPTT call in a temporary group session, the controlling MCPTT function shall terminate the subscriptions to the conference event package as specified in IETF RFC 6665 [26] in all non-controlling MCPTT functions in the temporary group session.

#### 10.1.3.5	Non-controlling MCPTT function

##### 10.1.3.5.1	Receiving subscriptions to the conference event package

Upon receipt of SIP SUBSCRIBE request for event package subscription in the non-controlling MCPTT function and the SIP SUBSCRIBE request:

1)	contains an application/vnd.3gpp.mcptt-info+xml MIME body with

a)	the <mcptt-request-uri> element set to the constituent MCPTT group ID; and

b)	the <mcptt-calling-user-id> element is set to:

i)	a participant in the group session; or

ii)	the temporary MCPTT group ID;

2)	contains the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Asserted-Service header field according to IETF RFC 6050 [9];

3)	contains an Accept header field containing the application/conference-info+xml MIME type; and

4)	is not received in a group call initiated as a broadcast group call;

then the non-controlling MCPTT function:

1)	shall check if the <on-network-allow-conference-state> element in the group document in 3GPP TS 24.381 [31] of the constituent group allows the MCPTT ID in the <mcptt-calling-user-id> element to subscribe to the conference event package and if not allowed:

a)	shall reject the "SIP SUBSCRIBE request for event status subscription in the controlling MCPTT function" with a SIP 403 (Forbidden) response to the SIP SUBSCRIBE request, with warning text set to "138 subscription of conference events not allowed" as specified in subclause 4.4; and

b)	shall not continue with the remaining steps;

2)	shall handle the request according to IETF RFC 6665 [26] and IETF RFC 4575 [30];

3)	shall cache information about the subscription;

4)	shall generate a notification package as specified in subclause 6.3.4.3 and send a SIP NOTIFY request according to 3GPP TS 24.229 [4] to the MCPTT client which have subscribed to the conference state event; and

5)	if the SIP SUBSCRIBE request is the first SIP SUBSCRIBE request from a MCPTT client, shall subscribe to the conference event package from the controlling MCPTT functions in the group session as specified in subclause 10.1.3.5.3.

Upon receipt of a SIP SUBSCRIBE request for event package subscription in the controlling MCPTT function in a group call initiated as a broadcast group call, the controlling MCPTT function:

1)	shall generate a SIP 480 (Temporarily Unavailable) response to the SIP SUBSCRIBE request as specified in 3GPP TS 24.229 [4];

2)	shall include a Warning header field with the warning text set to "105 subscription not allowed in a broadcast group call" as specified in subclause 4.4; and

3)	send the SIP 480 (Temporarily Unavailable) response according to 3GPP TS 24.229 [4].

##### 10.1.3.5.2	Sending notifications to the conference event package

The procedures in this subclause is triggered by:

1)	the receipt of a receipt of a SIP BYE request from one of the participants in a pre-arranged or a chat group session; or

2)	when a new participant is added in a pre-arranged or chat group session.

When sending a conference state event notification, the non-controlling MCPTT function:

1)	shall generate a notification package as specified in subclause 6.3.4.3 to all participants which have subscribed to the conference state event package; and

NOTE:	As a group document can potentially have a large content, the controlling MCPTT function can notify using content-indirection as defined in IETF RFC 4483 [32].

2)	shall send a SIP NOTIFY request to all participants which have subscribed to the conference state event package as specified in 3GPP TS 24.229 [4].

##### 10.1.3.5.3	Sending a subscription to the conference event package

Upon receipt of the first subscription to the conference event package from an MCPTT client, the non-controlling MCPTT function:

1)	shall generate a SIP SUBSCRIBE request and use a new SIP-dialog according to IETF RFC 6665 [26], IETF RFC 4575 [30] and 3GPP TS 24.229 [4];

2)	shall set the Request-URI of the SIP SUBSCRIBE request to the temporary MCPTT session identity;

NOTE:	The SIP URI received in the Contact header field of the SIP INVITE request for non-controlling MCPTT function of an MCPTT group is the temporary MCPTT session identity. Towards MCPTT clients the non-controlling MCPTT function uses an internal generated MCPTT session identity.

3)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Asserted-Service header field according to IETF RFC 6050 [9];

4)	shall include an Accept-Contact header with the media feature tag g.3gpp.icsi-ref with the value "urn:urn-7:3gpp-service.ims.icsi.mcptt" along with "require" and "explicit" header field parameters according to IETF RFC 3841 [6];

5)	shall set the Expires header field according to IETF RFC 6665 [26], to 4294967295;

NOTE 2:	4294967295, which is equal to 232-1, is the highest value defined for Expires header field in IETF RFC 3261 [24].

6)	shall include an Accept header field containing the application/conference-info+xml MIME type;

7)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body with:

a)	the <mcptt-request-uri> element set to the temporary MCPTT group ID: and

b)	the <mcptt-calling-group-id> set to the constituent MCPTT group ID; and

8)	shall send the SIP SUBSCRIBE request using a new SIP dialog according to 3GPP TS 24.229 [4].

The 2xx response to the SIP SUBSCRIBE request shall be handled according to IETF RFC 6665 [26], IETF RFC 4575 [30] and TS 24.229 [4].

Upon receiving an incoming SIP NOTIFY requests to the previously sent SIP SUBSCRIBE request the non-controlling MCPTT function:

1)	shall handle the request according to IETF RFC 6665 [26] and IETF RFC 4575 [30];

2)	shall store conference information based on the SIP NOTIFY request content;

3)	shall modify the SIP NOTIFY request as specified in subclause 6.3.4.3; and

4)	forward the modified SIP NOTIFY request according to 3GPP TS 24.229 [4] to all MCPTT clients with a subscription to the conference event package.

##### 10.1.3.5.4	Terminating a subscription

Upon receipt of a SIP SUBSCRIBE request for event status subscription in the non-controlling MCPTT function that terminates the subscription of the conference event package as specified in IETF RFC 6665 [26], the non-controlling MCPTT function:

1)	shall send a SIP 200 (OK) response as specified in IETF RFC 6665 [26]; and

2)	if there are no remaining subscriptions to the event package (excluding any subscriptions to the event package made by the controlling MCPTT function), shall terminate the subscriptions to the conference event package in the controlling MCPTT function as specified in IETF RFC 6665 [26].

Upon expiry of the subscription timer and if there are no remaining subscriptions to the event package (excluding any subscriptions to the event package made by the controlling MCPTT function), the non-controlling MCPTT function shall terminate the subscriptions to the conference event package in the controlling MCPTT function as specified in IETF RFC 6665 [26].

NOTE:	The subscription to the event package made by the controlling MCPTT function will be terminated by the controlling MCPTT function when the last subscription to the event package is terminated in the controlling MCPTT function.

## 10.2	Off-network group call

### 10.2.1	General

#### 10.2.1.1	Common Procedures

##### 10.2.1.1.1	MONP message transport

In order to participate in a call of an MCPTT group, the MCPTT client:

1)	shall send the MONP message as a UDP message to the multicast IP address of the MCPTT group, on UDP port TBD, with an IP time-to-live set to 255; and

Editor's note [CT1#95-bis, C1-160770]: Port number for the message is FFS.

2)	shall treat UDP messages received on the multicast IP address of the MCPTT group and on port TBD as received MONP messages.

The MONP message is the entire payload of the UDP message.

##### 10.2.1.1.2	Session description

For an off-network MCPTT session, only MCPTT speech is used.

One off-network MCPTT session includes one media-floor control entity.

The MCPTT client shall generate an SDP body for a group call in accordance with rules and procedures of RFC4566 [12].

The MCPTT client:

1)	shall include in the session-level section:

a)	the "o=" field with the <username> portion set to a dash;

b)	the "s=" field with the <session name> portion set to a dash; and

c)	the "c=" field with the <nettype> portion set to "IN", the <addrtype> portion set to the IP version of a multicast IP address of the MCPTT group and the <connection-address> portions set to the multicast IP address of the MCPTT group;

2)	shall include the media-level section for MCPTT speech consisting of:

a)	the "m=" field with the <media> portion set to "audio", the <port> portion set to a port number for MCPTT speech of the MCPTT group, the <proto> field set to "RTP/AVP" and <fmt> portion set indicating RTP payload type numbers;

b)	the "i=" field with the <session description> portion set to "speech";

c)	the "a=fmtp:" attribute(s), the "a=rtpmap:" attribute(s) or both, indicating the codec(s) and media parameters of the MCPTT speech with the following clarification:

i)	if the "/<x>/<x>/Common/PreferredVoiceCodec" leaf node is present in the group document configured on the group management client as specified in 3GPP TS 24.383 [45] containing an RTP payload format name as specified in IETF RFC 4566 [12], indicating a preferred voice codec for an MCPTT group; and

ii)	if the MCPTT client supports the encoding name indicated in the value of the "name" attribute;

then the MCPTT client:

i)	shall insert the value of the "/<x>/<x>/Common/PreferredVoiceCodec" leaf node in the <encoding name> field of the "a=rtpmap" attribute as defined in IETF RFC 4566 [12]; and

d)	the "a=rtcp:" attribute indicating port number to be used for RTCP at the MCPTT client selected according to the rules and procedures of IETF RFC 3605 [13], if the media steam uses other than the default IP address; and

3)	shall include the media-level section for media-floor control entity consisting of:

a)	an "m=" line, with the <media> portion set to "application", the <port> portion set to a port number for media-floor control entity of the MCPTT group, the <proto> field set to "udp" and <fmt> portion set to "MCPTT"; and

b)	the "a=fmtp:MCPTT" attribute indicating the parameters of the media-floor control entity as specified 3GPP TS 24.380 [5].

### 10.2.2	Basic call control

#### 10.2.2.1	General

In this release of specification, media streams of off-network group call cannot be modified and the SDP is the same for the entire duration of the call.

The maximum number of simultaneous off-network group calls is limited by the value of "/<x>/Common/MCPTTGroupCall/MaxCallN4" leaf node present in the UE configuration as specified in 3GPP TS 24.383 [45].

#### 10.2.2.2	Basic call control state machine

The Figure 10.2.2.2-1 gives an overview of the main states and transitions on the UE for call control.

Each call control state machine is per MCPTT group ID.

Figure 10.2.2.2-1: Basic call control state machine

The following pieces of information are associated with the basic call control state machine:

a)	the stored call identifier of the call;

b)	the probe response value of the call;

c)	the stored refresh interval of the call;

d)	the stored SDP body of the call;

e)	the stored originating MCPTT user ID of the call; and

f)	the stored MCPTT group ID of the call.

The basic call control state machine has a related call type control state machine described in subclause 10.2.3.2.

When sending the message, MCPTT client indicates the stored current ProSe per-packet priority associated with the call type control state machine to the lower layers.

#### 10.2.2.3	Call Control states

##### 10.2.2.3.1	S1: start-stop

This state exists for UE, when the UE is not part of an ongoing call.

This state is the start state of this state machine.

This state is the stop state of this state machine.

##### 10.2.2.3.2	S2: waiting for call announcement

This state exists for UE, when the UE has sent a GROUP CALL PROBE message and is waiting for a GROUP CALL ANNOUNCEMENT message.

##### 10.2.2.3.3	S3: part of ongoing call

This state exists for UE, when the UE is part of an ongoing group call.

##### 10.2.2.3.4	S4: pending user action without confirm indication

This state exists for UE, when the UE has presented a notification to the MCPTT user for the received GROUP CALL ANNOUNCEMENT message, is waiting for a response and is not expected to send confirm indication.

##### 10.2.2.3.5	S5: pending user action with confirm indication

This state exists for UE, when the UE has presented a notification to the MCPTT user for the received GROUP CALL ANNOUNCEMENT message, is waiting for a response and is expected to send confirm indication.

##### 10.2.2.3.6	S6: ignoring incoming call announcements

This state exists for UE, when the group call was rejected or released, GROUP CALL ANNOUNCEMENT message was sent or received and GROUP CALL ANNOUNCEMENT messages continue being received.

##### 10.2.2.3.7	S7: waiting for call announcement after call release

This state exists for UE, when the group call was released, GROUP CALL ANNOUNCEMENT message was neither sent nor received and GROUP CALL PROBE was sent.

#### 10.2.2.4	Procedures

##### 10.2.2.4.1	General

###### 10.2.2.4.1.1	Call announcement timer calculation

10.2.2.4.1.1.1	Periodic call announcement timer calculation

The MCPTT client:

1)	shall generate a random number, X, with uniform distribution between 0 and 1; and

2)	shall set the TFG2 (periodic announcement) timer as follows:

-	TFG2 (periodic announcement) = the refresh interval of the call * (2/3 + 2/3*X) seconds.

10.2.2.4.1.1.2	Call announcement timer calculation after CALL PROBE

The MCPTT client:

1)	shall generate a random number, X, with uniform distribution between 0 and 1; and

2)	shall set the TFG2 (periodic announcement) timer as follows:

-	TFG2 (periodic announcement) = 1/12*X seconds.

###### 10.2.2.4.1.2	Max duration timer calculation

The MCPTT client shall set the TFG6 (max duration) timer as follows:

-	TFG6 (max duration) = X – (Y – Z) seconds, where:

-	X = value of "/<x>/<x>/OffNetwork/MaxDuration" leaf node present in group configuration as specified in 3GPP TS 24.383 [45];

-	Y = current UTC time, in seconds since midnight UTC of January 1, 1970 (not counting leap seconds);

-	Z = Call start time IE of the GROUP CALL ANNOUNCEMENT message.

##### 10.2.2.4.2	Call Probe

###### 10.2.2.4.2.1	Originating UE call probe initiation

When in the "S1: start-stop" state, upon an indication from an MCPTT user to initiate a group call for an MCPTT group ID, the MCPTT client:

1)	shall store the MCPTT group ID as the MCPTT group ID of the call;

2)	shall create a call type control state machine as described in subclause 10.2.3.2;

3)	shall generate a GROUP CALL PROBE message as specified in subclause 15.1.2. In the GROUP CALL PROBE message, the MCPTT client:

a)	shall set the MCPTT group ID IE to the stored MCPTT group ID of the call;

4)	shall send the GROUP CALL PROBE message as specified in subclause 10.2.1.1.1;

5)	shall start timer TFG3 (call probe retransmission);

6)	shall start timer TFG1 (wait for call announcement); and

7)	shall enter the "S2: waiting for call announcement" state.

###### 10.2.2.4.2.2	Originating UE call probe retransmission

When in the "S2: waiting for call announcement" state, upon expiration of TFG3 (call probe retransmission), the MCPTT client:

1)	shall generate a GROUP CALL PROBE message as specified in subclause 15.1.2. In the GROUP CALL PROBE message, the MCPTT client:

a)	shall set the MCPTT group ID IE to the stored MCPTT group ID of the call;

2)	shall send the GROUP CALL PROBE message as specified in subclause 10.2.1.1.1;

3)	shall start timer TFG3 (call probe retransmission); and

4)	shall remain in the "S2: waiting for call announcement" state.

###### 10.2.2.4.2.3	Terminating UE receiving GROUP CALL PROBE message when participating in the ongoing call

When in the "S3: part of ongoing call" state, upon receiving a GROUP CALL PROBE message with the MCPTT group ID IE matching the stored MCPTT group ID of the call, the MCPTT client:

1)	if the stored probe response value of the call is set to "false":

a)	shall stop timer TFG2 (call announcement);

b)	shall start timer TFG2 (call announcement) with value as specified in subclause 10.2.2.4.1.1.2; and

c)	shall set the stored probe response of the call to "true"; and

2)	shall remain in the "S3: part of ongoing call" state.

##### 10.2.2.4.3	Call setup

###### 10.2.2.4.3.1	UE not receiving any response to GROUP CALL PROBE message

When in the "S2: waiting for call announcement" state, upon expiry of timer TFG1 (wait for call announcement), the MCPTT client:

1)	shall stop timer TFG3 (call probe retransmission), if running;

2)	shall generate an SDP body as specified in subclause 10.2.1.1.2 and store it as the SDP body of the call;

3)	shall generate a random number with uniform distribution between 0 and 65535 and store it as the call identifier of the call;

4)	shall select refresh interval value and store it as the refresh interval of the call;

5)	shall store own MCPTT user ID as the originating MCPTT user ID of the call;

6)	shall store the current UTC time as the call start time of the call;

7)	shall generate a GROUP CALL ANNOUNCEMENT message as specified in subclause 15.1.3. In the GROUP CALL ANNOUNCEMENT message, the MCPTT client:

a)	shall set the Call identifier IE to the stored call identifier of the call;

b)	shall set the Call type IE to the stored current call type associated with the call type control state machine;

c)	shall set the Refresh interval IE to the stored refresh interval of the call;

d)	shall set the SDP IE to the stored SDP body of the call;

e)	shall set the Originating MCPTT user ID IE to the stored originating MCPTT user ID of the call;

f)	shall set the MCPTT group ID IE to the stored MCPTT group ID of the call;

g)	shall set the Call start time IE to the stored call start time of the call;

h)	shall set the Last call type change time IE to the stored last call type change time of the call associated with call type control state machine;

i)	shall set the Last user to change call type IE to last user to change call type associated with call type control state machine; and

j)	may include the Confirm mode indication IE;

8)	shall send the GROUP CALL ANNOUNCEMENT message as specified in subclause 10.2.1.1.1;

9)	shall establish a media session based on the stored SDP body of the call;

10)	shall start floor control as originating floor participant as specified in subclause 7.2 in 3GPP TS 24.380 [5];

11)	shall start timer TFG6 (max duration) with value as specified in subclause 10.2.2.4.1.2;

12)	shall start timer TFG2 (call announcement) with value as specified in subclause 10.2.2.4.1.1.1; and

13)	shall enter the "S3: part of ongoing call" state.

Note: In this release of the specification, the refresh interval of the call is fixed to 10 seconds.

###### 10.2.2.4.3.2	Originating UE receiving a GROUP CALL ANNOUNCEMENT message

When in the "S2: waiting for call announcement" state, upon receiving a GROUP CALL ANNOUNCEMENT message with the MCPTT group ID IE matching the stored MCPTT group ID of the call, the MCPTT client:

1)	shall stop timer TFG3 (call probe retransmission);

2)	shall stop timer TFG1 (wait for call announcement);

3)	shall store the value of the SDP IE of the GROUP CALL ANNOUNCEMENT message as the SDP body of the call;

4)	shall store the value of the Call identifier IE of the GROUP CALL ANNOUNCEMENT message as the call identifier of the call;

5)	shall store the value of the originating MCPTT user ID IE of the GROUP CALL ANNOUNCEMENT message as the Originating MCPTT user ID of the call;

6)	shall store the value of the Refresh interval IE of the GROUP CALL ANNOUNCEMENT message as the refresh interval of the call;

7)	shall store the value of the Call start time IE of the GROUP CALL ANNOUNCEMENT message as the call start time of the call;

8)	shall establish a media session based on the stored SDP body of the call;

9)	shall start floor control as terminating floor participant as specified in subclause 7.2 in 3GPP TS 24.380 [5];

10)	shall start timer TFG6 (max duration) with value as specified in subclause 10.2.2.4.1.2;

11)	shall start timer TFG2 (call announcement) with value as specified in subclause 10.2.2.4.1.1.1; and

12)	shall enter the "S3: part of ongoing call" state.

###### 10.2.2.4.3.3	Terminating UE receiving a GROUP CALL ANNOUNCEMENT message when not participating in the ongoing call

When in the "S1: start-stop" state, upon receiving a GROUP CALL ANNOUNCEMENT message with the MCPTT group ID IE not matching MCPTT group ID of the call stored for other state machines, the MCPTT client:

1)	shall store the value of the SDP IE of the GROUP CALL ANNOUNCEMENT message as the SDP body of the call;

2)	shall store the value of the Call identifier IE of the GROUP CALL ANNOUNCEMENT message as the call identifier of the call;

3)	shall store the value of the Originating MCPTT user ID IE of the GROUP CALL ANNOUNCEMENT message as the originating MCPTT user ID of the call;

4)	shall store the value of the Refresh interval IE of the GROUP CALL ANNOUNCEMENT message as the refresh interval of the call;

5)	shall store the value of the MCPTT group ID IE of the GROUP CALL ANNOUNCEMENT message as the MCPTT group ID of the call;

6)	shall store the value of the Call start time IE of the GROUP CALL ANNOUNCEMENT message as the call start time of the call;

7)	shall create a call type control state machine as described in subclause 10.2.3.2;

8)	if the terminating UE is configured that the terminating MCPTT user acknowledgement is required upon a terminating call request reception:

a)	shall start timer TFG4 (waiting for the user);

b)	if the GROUP CALL ANNOUNCEMENT message contains the Confirm mode indication IE, shall enter the "S5: pending user action with confirm indication" state; and

c)	if the GROUP CALL ANNOUNCEMENT message does not contains the Confirm mode indication IE, shall enter the "S4: pending user action without confirm indication" state; and

9)	if the terminating UE is configured that the terminating MCPTT user acknowledgement is not required upon a terminating call request reception:

a)	shall establish a media session based on the stored SDP body of the call;

b)	shall start floor control as terminating floor participant as specified in subclause 7.2 in 3GPP TS 24.380 [5];

c)	if the GROUP CALL ANNOUNCEMENT message contains the Confirm mode indication IE:

i)	shall generate a GROUP CALL ACCEPT message as specified in subclause 15.1.4. In the GROUP CALL ACCEPT message, the MCPTT client:

A)	shall set the Call identifier IE to the stored call identifier of the call;

B)	shall set the Sending MCPTT user ID IE to own MCPTT user id;

C)	shall set the Call type IE to the stored current call type associated with the call type control state machine; and

D)	shall set the MCPTT group ID IE to the stored MCPTT group ID of the call; and

ii)	shall send the GROUP CALL ACCEPT message as specified in subclause 10.2.1.1.1;

d)	shall start timer TFG6 (max duration) with value as specified in subclause 10.2.2.4.1.2;

e)	shall start timer TFG2 (call announcement) with value as specified in subclause 10.2.2.4.1.1.1; and

f)	shall enter the "S3: part of ongoing call" state.

###### 10.2.2.4.3.4	MCPTT user accepts the terminating call with confirm indication

When in the "S5: pending user action with confirm indication" state, upon indication from the MCPTT user to accept the incoming group call, the MCPTT client:

1)	shall establish a media session based on the stored SDP body of the call;

2)	shall start floor control as terminating floor participant as specified in subclause 7.2 in 3GPP TS 24.380 [5];

3)	shall generate a GROUP CALL ACCEPT message as specified in subclause 15.1.4. In the GROUP CALL ACCEPT message, the MCPTT client:

a)	shall set the Call identifier IE to the stored call identifier of the call;

b)	shall set the Sending MCPTT user ID IE to own MCPTT user id;

c)	shall set the Call type IE to the stored current call type associated with the call type control state machine; and

d)	shall set the MCPTT group ID IE to the stored MCPTT group ID of the call; and

4)	shall send the GROUP CALL ACCEPT message as specified in subclause 10.2.1.1.1;

5)	shall start timer TFG6 (max duration) with value as specified in subclause 10.2.2.4.1.2;

6)	shall start timer TFG2 (call announcement) with value as specified in subclause 10.2.2.4.1.1.1; and

7)	shall enter the "S3: part of ongoing call" state.

###### 10.2.2.4.3.5	MCPTT user accepts the terminating call without confirm indication

When in the "S4: pending user action without confirm indication" state, upon an indication from the MCPTT user to accept the incoming group call, the MCPTT client:

1)	shall establish a media session based on the stored SDP body of the call;

2)	shall start floor control as terminating floor participant as specified in subclause 7.2 in 3GPP TS 24.380 [5];

3)	shall start timer TFG6 (max duration) with value as specified in subclause 10.2.2.4.1.2;

4)	shall start timer TFG2 (call announcement) with value as specified in subclause 10.2.2.4.1.1.1; and

5)	shall enter the "S3: part of ongoing call" state.

###### 10.2.2.4.3.6	Originating UE receiving GROUP CALL ACCEPT message

When in the "S3: part of ongoing call" state, upon receiving a GROUP CALL ACCEPT message with the MCPTT group ID IE matching the stored MCPTT group ID of the call, the MCPTT client:

1)	can inform the MCPTT user about the call acceptance; and

2)	shall remain in the "S3: part of ongoing call" state.

###### 10.2.2.4.3.7	MCPTT user rejects the terminating call

When in the "S5: pending user action with confirm indication" state or the "S4: pending user action without confirm indication" state, upon an indication from the MCPTT user to reject the incoming group call, the MCPTT client:

1)	shall stop timer TFG4 (waiting for the user);

2)	shall start timer TFG5 (not present incoming call announcements); and

3)	shall enter the "S6: ignoring incoming call announcements" state.

###### 10.2.2.4.3.8	MCPTT user does not act on terminating call

When in the "S5: pending user action with confirm indication" state or the "S4: pending user action without confirm indication" state, upon expiration of timer TFG4 (waiting for the user), the MCPTT client:

1)	shall start timer TFG5 (not present incoming call announcements); and

2)	shall enter the "S6: ignoring incoming call announcements" state.

##### 10.2.2.4.4	Periodic group call announcement

###### 10.2.2.4.4.1	Originating UE sending periodic call announcement

When in the "S3: part of ongoing call" state, upon expiry of timer TFG2 (call announcement), the MCPTT client:

1)	shall generate a GROUP CALL ANNOUNCEMENT message as specified in subclause 15.1.3. In the GROUP CALL ANNOUNCEMENT message, the MCPTT client:

a)	shall set the Call identifier IE to the stored call identifier of the call;

b)	shall set the Call type IE to the stored current call type associated with the call type control state machine;

c)	shall set the Refresh interval IE to the stored refresh interval of the call;

d)	shall set the SDP IE to the stored SDP body of the call;

e)	shall set the Originating MCPTT user ID IE to the stored originating MCPTT user ID of the call;

f)	shall set the MCPTT group ID IE to the stored MCPTT group ID of the call;

g)	shall set the Last call type change time IE to the stored last call type change time of the call associated with call type control state machine;

h)	shall set the Last user to change call type IE to last user to change call type associated with call type control state machine;

i)	shall set the Call start time IE to the stored call start time of the call;

j)	if the stored probe response value of the call is set to "true", shall include Probe response IE;

2)	 shall send the GROUP CALL ANNOUNCEMENT message as specified in subclause 10.2.2.1.1.1;

3)	if the stored probe response value of the call is set to "true", shall set the stored probe response value of the call to "false";

4)	shall start timer TFG2 (call announcement) with value as specified in subclause 10.2.2.4.1.1.1; and

5)	shall remain in the "S3: part of ongoing call" state.

###### 10.2.2.4.4.2	Terminating UE receiving periodic call announcement

When in the "S3: part of ongoing call" state, upon receiving a GROUP CALL ANNOUNCEMENT message with the MCPTT group ID IE matching the stored MCPTT group ID of the call, the Call start time IE being the same as the stored call start time of the call, the Last call type change time IE being the same as the stored last call type change time of the call associated with the call type control state machine, the Last user to change call type IE being the same as the stored last user to change call type of the call associated with the call type control state machine and the Call identifier IE being the same as the stored call identifier of the call and Call type IE same as the stored current call type associated with the call type control state machine and:

1)	if the stored probe response value of the call is set to "true" and GROUP CALL ANNOUNCEMENT message contains Probe response IE; or

2)	if the stored probe response value of the call is set to "false":

the MCPTT client,

1)	shall stop timer TFG2 (call announcement);

2)	shall start timer TFG2 (call announcement) with value as specified in subclause 10.2.2.4.1.1.1;

3)	shall set the stored probe response of the call to "false", if set to "true"; and

4)	shall remain in the "S3: part of ongoing call" state.

##### 10.2.2.4.5	Call release

###### 10.2.2.4.5.1	MCPTT user leaves the call when GROUP CALL ANNOUNCEMENT was sent or received

When in the "S3: part of ongoing call" state, the "S5: pending user action with confirm indication" state, or the "S4: pending user action without confirm indication" state, upon an indication from the MCPTT user to release the group call, the MCPTT client:

1)	shall release the media session, if established;

2)	shall stop timer TFG4 (waiting for the user), if running;

3)	shall stop timer TFG2 (call announcement), if running;

4)	shall start timer TFG5 (not present incoming call announcements); and

5)	shall enter the "S6: ignoring incoming call announcements" state.

###### 10.2.2.4.5.2	Receiving GROUP CALL ANNOUNCEMENT message for rejected or released call

When in the "S6: ignoring incoming call announcements" state, upon receiving a GROUP CALL ANNOUNCEMENT message with the MCPTT group ID IE matching the stored MCPTT group ID of the call, the MCPTT client:

1)	shall store the value of the SDP IE of the GROUP CALL ANNOUNCEMENT message as the SDP body of the call;

2)	shall store the value of the Call identifier IE of the GROUP CALL ANNOUNCEMENT message as the call identifier of the call;

3)	shall store the value of the Originating MCPTT user ID IE of the GROUP CALL ANNOUNCEMENT message as the originating MCPTT user ID of the call;

4)	shall store the value of the Refresh interval IE of the GROUP CALL ANNOUNCEMENT message as the refresh interval of the call;

5)	shall store the value of the Call start time IE of the GROUP CALL ANNOUNCEMENT message as the call start time of the call;

6)	shall stop timer TFG5 (not present incoming call announcements);

7)	shall start timer TFG5 (not present incoming call announcements); and

8)	shall remain in the "S6: ignoring incoming call announcements" state.

###### 10.2.2.4.5.3	MCPTT user initiates originating call for rejected or released call

When in the "S6: ignoring incoming call announcements" state, upon an indication from the MCPTT user to initiate a group call for an MCPTT group ID matching the stored MCPTT group ID of the call, the MCPTT client:

1)	stop timer TFG5 (not present incoming call announcements);

2)	shall establish a media session based on the stored SDP body of the call;

3)	shall start floor control as terminating floor participant as specified in subclause 7.2 in 3GPP TS 24.380 [5];

4)	shall start timer TFG6 (max duration) with value as specified in subclause 10.2.2.4.1.2;

5)	shall start timer TFG2 (call announcement) with value as specified in subclause 10.2.2.4.1.1.1; and

6)	shall enter the "S3: part of ongoing call" state.

###### 10.2.2.4.5.4	No GROUP CALL ANNOUNCEMENT messages for rejected or released call

When in the "S6: ignoring incoming call announcements" state, upon expiration of timer TFG5 (not present incoming call announcements), the MCPTT client:

1)	shall release the stored SDP body of the call;

2)	shall release the stored call identifier of the call;

3)	shall release the stored originating MCPTT user ID of the call;

4)	shall release the stored refresh interval of the call;

5)	shall release the stored MCPTT group ID of the call;

6)	shall release the call start time of the call;

7)	shall destroy the call type control state machine; and

8)	shall enter the "S1: start-stop" state.

###### 10.2.2.4.5.5	MCPTT user leaves the call when GROUP CALL PROBE was sent

When in the "S2: waiting for call announcement" state, upon an indication from the MCPTT user to release the group call, the MCPTT client:

1)	shall stop timer TFG3 (call probe retransmission); and

2)	shall enter the "S7: Waiting for call announcement after call release" state.

###### 10.2.2.4.5.6	MCPTT user initiates originating call for released call

When in the "S7: Waiting for call announcement after call release" state, upon an indication from the MCPTT user to initiate a group call for an MCPTT group ID matching the stored MCPTT group ID of the call, the MCPTT client:

1)	shall stop timer TFG1 (wait for call announcement);

2)	shall generate a GROUP CALL PROBE message as specified in subclause 15.1.2. In the GROUP CALL PROBE message, the MCPTT client:

a)	shall set the MCPTT group ID IE to the stored MCPTT group ID of the call; and

3)	shall send the GROUP CALL PROBE message as specified in subclause 10.2.1.1.1;

4)	shall start timer TFG3 (call probe retransmission);

5)	shall start timer TFG1 (wait for call announcement); and

6)	shall enter the "S2: waiting for call announcement" state.

###### 10.2.2.4.5.7	Receiving GROUP CALL ANNOUNCEMENT message for released call

When in the "S7: Waiting for call announcement after call release" state, upon receiving a GROUP CALL ANNOUNCEMENT message with the MCPTT group ID IE matching the stored MCPTT group ID of the call, the MCPTT client:

1)	shall store the value of the SDP IE of the GROUP CALL ANNOUNCEMENT message as the SDP body of the call;

2)	shall store the value of the Call identifier IE of the GROUP CALL ANNOUNCEMENT message as the call identifier of the call;

3)	shall store the value of the Originating MCPTT user ID IE of the GROUP CALL ANNOUNCEMENT message as the originating MCPTT user ID of the call;

4)	shall store the value of the Refresh interval IE of the GROUP CALL ANNOUNCEMENT message as the refresh interval of the call;

5)	shall store the value of the Call start time IE of the GROUP CALL ANNOUNCEMENT message as the call start time of the call;

6)	shall stop timer TFG1 (wait for call announcement);

7)	shall start timer TFG5 (not present incoming call announcements); and

8)	shall enter the "S6: ignoring incoming call announcements" state.

###### 10.2.2.4.5.8	No GROUP CALL ANNOUNCEMENT messages for released call

When in the "S7: Waiting for call announcement after call release" state, upon expiration of timer TFG1 (wait for call announcement), the MCPTT client:

1)	shall release the stored MCPTT group ID of the call;

2)	shall destroy the call type control state machine; and

3)	shall enter the "S1: start-stop" state.

###### 10.2.2.4.5.9	Max duration reached

When in the "S3: part of ongoing call" state, upon expiration of timer TFG6 (max duration), the MCPTT client:

1)	shall release the media session;

2)	shall stop timer TFG2 (call announcement), if running;

3)	shall start timer TFG5 (not present incoming call announcements); and

4)	shall enter the "S6: ignoring incoming call announcements" state.

##### 10.2.2.4.6	Merge of calls

###### 10.2.2.4.6.1	Merge of two calls

When in the "S3: part of ongoing call" state, upon receiving a GROUP CALL ANNOUNCEMENT message with the MCPTT group ID IE matching the stored MCPTT group ID of the call and:

1)	the Originating MCPTT user ID IE is different from the stored originating MCPTT user ID of the call; or

2)	the Call identifier IE is different from the stored call identifier of the call;

then:

1)	if the stored current call type associated with the call type control state machine is "BASIC GROUP CALL" and the value of the Call type IE of GROUP CALL ANNOUNCEMENT message is either "IMMINENT PERIL GROUP CALL" or "EMERGENCY GROUP CALL";

2)	if the stored current call type associated with the call type control state machine is "IMMINENT PERIL GROUP CALL" and the value of the Call type IE of GROUP CALL ANNOUNCEMENT message is "EMERGENCY GROUP CALL";

3)	if the stored current call type associated with the call type control state machine being equal to the Call type IE of the GROUP CALL ANNOUNCEMENT message and the Call start time IE of the GROUP CALL ANNOUNCEMENT message being lower than the stored call start time of the call; or

4)	if the stored current call type associated with the call type control state machine being equal to the Call type IE of the GROUP CALL ANNOUNCEMENT message and the Call start time IE of the GROUP CALL ANNOUNCEMENT message being equal to the stored call start time of the call and the Call identifier IE of the GROUP CALL ANNOUNCEMENT message being lower than the stored call identifier of the call;

the MCPTT client:

1)	shall store the value of the SDP IE of the GROUP CALL ANNOUNCEMENT message as the SDP body of the call;

2)	shall store the value of the Call identifier IE of the GROUP CALL ANNOUNCEMENT message as the call identifier of the call;

3)	shall store the value of the Originating MCPTT user ID IE of the GROUP CALL ANNOUNCEMENT message as the originating MCPTT user ID of the call;

4)	shall store the value of the Refresh interval IE of the GROUP CALL ANNOUNCEMENT message as the refresh interval of the call;

5)	shall store the value of the Call start time IE of the GROUP CALL ANNOUNCEMENT message as the call start time of the call;

6)	shall adjust the media session based on the stored SDP body of the call and restart floor control as terminating floor participant as specified in subclause 7.2 in 3GPP TS 24.380 [5];

7)	shall stop timer TFG6 (max duration);

8)	shall start timer TFG6 (max duration) with value as specified in subclause 10.2.2.4.1.2;

9)	shall stop timer TFG2 (call announcement); and

10)	shall start timer TFG2 (call announcement) with value according to rules and procedures as specified in subclause 10.2.2.4.1.1.1; and

11)	shall remain in the "S3: part of ongoing call" state.

##### 10.2.2.4.7	Error handling

###### 10.2.2.4.7.1	Unexpected MONP message received

Upon receiving a MONP message in a state where there is no handling specified for the MONP message, the MCPTT client shall discard the MONP message.

###### 10.2.2.4.7.2	Unexpected indication from MCPTT user

Upon receiving an indication from the MCPTT user in a state where there is no handling specified for the indication, the MCPTT client shall ignore the indication.

###### 10.2.2.4.7.3	Unexpected expiration of a timer

Upon expiration of a timer in a state where there is no handling specified for expiration of the timer, the MCPTT client shall ignore the expiration of the timer.

### 10.2.3.	Call type control

#### 10.2.3.1	General

This state machine exists when the UE is part of an ongoing off-network group call.

#### 10.2.3.2	Call type control state machine

The Figure 10.2.3.2-1 gives an overview of the states and transitions of the state machine.

Figure 10.2.3.2-1: Call type control state machine

The following pieces of information are associated with the call type control state machine:

a)	the stored current call type;

b)	the stored current ProSe per-packet priority;

c)	the stored last call type change time of the call; and

d)	the stored last user to change call type of the call.

When sending the message, MCPTT client indicates the stored current ProSe per-packet priority (as described in 3GPP TS 24.383 [45]) associated with the call type control state machine to the lower layers.

#### 10.2.3.3	Call Control states

##### 10.2.3.3.1	T0: waiting for call to establish

This state is the start state of this state machine.

##### 10.2.3.3.2	T1: in-progress emergency group call

This state exists for UE, when the UE is part of an in-progress emergency group call.

##### 10.2.3.3.3	T2: in-progress basic group call

This state exists for UE, when the UE is part of an in-progress basic group call.

##### 10.2.3.3.4	T3: in-progress imminent peril group call

This state exists for UE, when the UE is part of an in-progress imminent peril group call.

#### 10.2.3.4	Procedures

##### 10.2.3.4.1	General

##### 10.2.3.4.2	User initiated the call probe

When in the "T0: waiting for the call to establish " state, upon an indication from an MCPTT user to initiate a group call probe for an MCPTT group, the MCPTT client:

1)	if the stored emergency state associated with emergency alert state machine described in 12.2.2.2 is set to "true" and the value of "/<x>/<x>/Common/AllowedEmergencyCall" leaf node present in group configuration as specified in 3GPP TS 24.383 [45] is set to "true":

a)	shall set the stored current call type to "EMERGENCY GROUP CALL"; and

b)	shall set the stored current ProSe per-packet priority to value corresponding to MCPTT off-network emergency group call as described in 3GPP TS 24.383 [45];

2)	if the stored emergency state associated with emergency alert state machine described in 12.2.2.2 is set to "false", and:

a)	if the user initiates an MCPTT emergency call and the values of "/<x>/<x>/Common/MCPTTGroupCall/EmergencyCall/Enabled" leaf node present in the user profile and "/<x>/<x>/Common/AllowedEmergencyCall" leaf node present in group configuration as specified in 3GPP TS 24.383 [45] are set to "true":

i)	shall set the stored current call type to "EMERGENCY GROUP CALL"; and

ii)	shall set the stored current ProSe per-packet priority to value corresponding to MCPTT off-network emergency group call as described in 3GPP TS 24.383 [45];

b)	if the user initiates an MCPTT imminent peril group call and the values of "/<x>/<x>/Common/MCPTTGroupCall/ImminentPerilCall/Authorised" leaf node present in the user profile "/<x>/<x>/Common/AllowedImminentPerilCall " leaf node present in group configuration as specified in 3GPP TS 24.383 [45] are set to "true":

i)	shall set the stored current call type to "IMMINENT PERIL GROUP CALL"; and

ii)	shall set the stored current ProSe per-packet priority to value corresponding to MCPTT off-network imminent peril group call as described in 3GPP TS 24.383 [45]; and

c)	if the user initiates an MCPTT group call which is not an MCPTT emergency call and which is not an MCPTT imminent peril group call:

i)	shall set the stored current call type to "BASIC GROUP CALL"; and

ii)	shall set the stored current ProSe per-packet priority to value corresponding to MCPTT off-network basic group call as described in 3GPP TS 24.383 [45];

3)	shall set the stored last call type change time to current UTC time;

4)	shall set the last user to change call type to own MCPTT user ID; and

5)	shall remain in "T0: waiting for the call to establish" state.

##### 10.2.3.4.3	Received GROUP CALL ANNOUNCEMENT message as a response to GROUP CALL PROBE message

When in the "T0: waiting for the call to establish " state, upon receipt of a GROUP CALL ANNOUNCEMENT message as a response to GROUP CALL PROBE message, the MCPTT client:

1)	if the Call type IE of the received GROUP CALL ANNOUNCEMENT message is set to "EMERGENCY GROUP CALL":

a)	shall set the stored current call type to "EMERGENCY GROUP CALL";

b)	shall set the stored current ProSe per-packet priority to value corresponding to MCPTT off-network emergency group call as described in 3GPP TS 24.383 [45];

c)	shall start timer TFG13 (implicit downgrade); and

d)	shall enter "T1: in-progress emergency group call" state;

2)	if the Call type IE of the received GROUP CALL ANNOUNCEMENT message is set to "IMMINENT PERIL GROUP CALL", and if the stored current call type is other than "EMERGENCY GROUP CALL":

a)	shall set the stored current call type to "IMMINENT PERIL GROUP CALL";

b)	shall set the stored current ProSe per-packet priority to value corresponding to MCPTT off-network imminent peril group call as described in 3GPP TS 24.383 [45];

c)	shall start timer TFG13 (implicit downgrade); and

d)	shall enter "T3: in-progress imminent peril group call" state; and

3)	if the Call type IE of the received GROUP CALL ANNOUNCEMENT message is set to "BASIC GROUP CALL", and if the stored current call type is "BASIC GROUP CALL":

a)	shall enter "T2: in-progress basic group call" state.

##### 10.2.3.4.4	Received GROUP CALL ANNOUNCEMENT with MCPTT user acknowledgement required

When in the "T0: waiting for the call to establish" state, upon receipt of a GROUP CALL ANNOUNCEMENT message by an idle MCPTT client when MCPTT user acknowledgement is required, the MCPTT client:

1)	if the Call type IE of the received GROUP CALL ANNOUNCEMENT message is set to "EMERGENCY GROUP CALL":

a)	shall set the stored current call type to "EMERGENCY GROUP CALL"; and

b)	shall set the stored current ProSe per-packet priority to value corresponding to MCPTT off-network emergency group call as described in 3GPP TS 24.383 [45];

2)	if the Call type IE of the received GROUP CALL ANNOUNCEMENT message is set to "IMMINENT PERIL GROUP CALL":

a)	shall set the stored current call type to "IMMINENT PERIL GROUP CALL";

b)	shall start timer TFG13 (implicit downgrade); and

c)	shall set the stored current ProSe per-packet priority to value corresponding to MCPTT off-network imminent peril group call as described in 3GPP TS 24.383 [45];

3)	if the Call type IE of the received GROUP CALL ANNOUNCEMENT message is set to "BASIC GROUP CALL":

a)	shall set the stored current call type to "BASIC GROUP CALL";

b)	shall set the stored current ProSe per-packet priority to value corresponding to MCPTT off-network basic group call as described in 3GPP TS 24.383 [45];

4)	shall set the stored last call type change time to the Last call type change time IE of the GROUP CALL ANNOUNCEMENT message;

5)	shall set the last user to change call type to the Last user to change call type IE of the GROUP CALL ANNOUNCEMENT message; and

6)	shall remain in "T0: waiting for the call to establish" state.

##### 10.2.3.4.5	Received GROUP CALL ANNOUNCEMENT without MCPTT user acknowledgement required

When in the "T0: waiting for the call to establish" state, upon receipt of a GROUP CALL ANNOUNCEMENT message by an idle MCPTT client when MCPTT user acknowledgement is not required, the MCPTT client:

1)	shall set the stored last call type change time to the Last call type change time IE of the GROUP CALL ANNOUNCEMENT message;

2)	shall set the last user to change call type to the Last user to change call type IE of the GROUP CALL ANNOUNCEMENT message;

3)	if the Call type IE of the received GROUP CALL ANNOUNCEMENT message is set to "EMERGENCY GROUP CALL":

a)	shall set the stored current call type to "EMERGENCY GROUP CALL";

b)	shall set the stored current ProSe per-packet priority to value corresponding to MCPTT off-network emergency group call as described in 3GPP TS 24.383 [45]; and

c)	shall enter "T1: in-progress emergency group call" state;

4)	if the Call type IE of the received GROUP CALL ANNOUNCEMENT message is set to "IMMINENT PERIL GROUP CALL":

a)	shall set the stored current call type to "IMMINENT PERIL GROUP CALL";

b)	shall set the stored current ProSe per-packet priority to value corresponding to MCPTT off-network imminent peril group call as described in3GPP TS 24.383 [45];

c)	shall start timer TFG13 (implicit downgrade); and

d)	shall enter "T3: in-progress imminent peril group call" state; and

5)	if the Call type IE of the received GROUP CALL ANNOUNCEMENT message is set to "BASIC GROUP CALL":

a)	shall set the stored current call type to "BASIC GROUP CALL";

b)	shall set the stored current ProSe per-packet priority to value corresponding to MCPTT off-network basic group call as described in 3GPP TS 24.383 [45]; and

c)	shall enter "T2: in-progress basic group call" state.

##### 10.2.3.4.6	Call started

When in state "T0: waiting for the call to establish", if:

a)	the MCPTT user accepts the call when MCPTT user acknowledgement is required; or

b)	the MCPTT client sends a GROUP CALL ANNOUNCEMENT message on expiry of timer TFG1 (wait for call announcement) associated with the basic call control state machine;

the MCPTT client:

1)	if the stored current call type is set to "EMERGENCY GROUP CALL", shall enter "T1: in-progress emergency group call" state;

2)	if the stored current call type is set to "IMMINENT PERIL GROUP CALL", shall enter "T3: in-progress imminent peril group call" state; or

3)	if the stored current call type is set to "BASIC GROUP CALL", shall enter "T2: in-progress basic group call" state.

##### 10.2.3.4.7	Upgrade call

###### 10.2.3.4.7.1	Originating user upgrading the call

When in the "T2: in-progress basic group call" state, upon receiving an indication from the user to upgrade the call to "IMMINENT PERIL GROUP CALL" or "EMERGENCY GROUP CALL" or when in the "T3: in-progress imminent peril group call" state, upon receiving an indication from the user to upgrade the call to "EMERGENCY GROUP CALL", the MCPTT client:

1)	if the user request is to upgrade the call to "EMERGENCY GROUP CALL" and the value of "/<x>/<x>/OffNetwork/EmergencyCallChange" leaf node present in the user profile as specified in 3GPP TS 24.383 [45] set to "true":

a)	shall set the stored current call type to "EMERGENCY GROUP CALL";

b)	shall set the stored current ProSe per-packet priority to value corresponding to MCPTT off-network emergency group call as described in 3GPP TS 24.383 [45]; and

c)	shall enter "T1: in-progress emergency group call" state;

2)	if the user request is to upgrade the call to "IMMINENT PERIL GROUP CALL" and the value of "/<x>/<x>/OffNetwork/ ImminentPerilCallChange" leaf node present in the user profile as specified in 3GPP TS 24.383 [45] set to "true":

a)	shall set the stored current call type to "IMMINENT PERIL GROUP CALL";

b)	shall set the stored current ProSe per-packet priority to value corresponding to MCPTT off-network imminent peril group call as described in 3GPP TS 24.383 [45]; and

c)	shall enter "T3: in-progress imminent peril group call" state;

3)	shall store the current UTC time as last call type change time of the call;

4)	shall store own MCPTT user ID as last user to change call type of the call;

5)	shall generate a GROUP CALL ANNOUNCEMENT message as specified in subclause 15.1.3. In the GROUP CALL ANNOUNCEMENT message, the MCPTT client:

a)	shall set the Call identifier IE to the stored call identifier of the call associated with the basic call control state machine;

b)	shall set the Call type IE to the stored current call type;

c)	shall set the Refresh interval IE to the stored refresh interval of the call associated with the basic call control state machine;

d)	shall set the SDP IE to the stored SDP body of the call associated with the basic call control state machine;

e)	shall set the Originating MCPTT user ID IE to the stored originating MCPTT user ID of the call associated with the basic call control state machine;

f)	shall set the MCPTT group ID IE to the stored MCPTT group ID of the call associated with the basic call control state machine;

g)	shall set the call start time IE to the stored call start time of the call;

h)	shall set the Last call type change time IE to the stored last call type change time of the call; and

i)	shall set the Last user to change call type IE to the stored last user to change call type of the call; and

6)	 shall send the GROUP CALL ANNOUNCEMENT message as specified in subclause 10.2.1.1.1;

###### 10.2.3.4.7.2	Terminating UE receiving a GROUP CALL ANNOUNCEMENT message when participating in the ongoing call

When in the "T1: in-progress emergency group call" state or "T2: in-progress basic group call" state or "T3: in-progress imminent peril group call" state, upon receiving a GROUP CALL ANNOUNCEMENT message with the MCPTT group ID IE matching with MCPTT group ID of the ongoing call and the Call Identifier IE being the same as the stored call identifier of the call, the MCPTT client:

1)	if the stored last user to change call type of the call is same as the Last user to change call type IE of the GROUP CALL ANNOUNCEMENT message and the stored last call type change time is smaller than Last call type change time IE of the GROUP CALL ANNOUNCEMENT message:

a)	shall set the stored last call type change time of the call to Last call type change time IE of the GROUP CALL ANNOUNCEMENT message;

b)	if the Call type IE of the received GROUP CALL ANNOUNCEMENT message is set to "EMERGENCY GROUP CALL" and the stored call type is other than "EMERGENCY GROUP CALL":

i)	shall set the stored current call type to "EMERGENCY GROUP CALL";

ii)	shall set the stored current ProSe per-packet priority to value corresponding to MCPTT off-network emergency group call as described in 3GPP TS 24.383 [45];

iii)	shall start timer TFG13 (implicit downgrade); and

iv)	shall enter "T1: in-progress emergency group call" state;

c)	if the Call type IE of the received GROUP CALL ANNOUNCEMENT message is set to "IMMINENT PERIL GROUP CALL" and the stored call type is other than "IMMINENT PERIL GROUP CALL":

i)	shall set the stored current call type to "IMMINENT PERIL GROUP CALL";

ii)	shall set the stored current ProSe per-packet priority to value corresponding to MCPTT off-network imminent peril group call as described in 3GPP TS 24.383 [45];

iii)	shall start timer TFG13 (implicit downgrade); and

iv)	shall enter "T3: in-progress imminent peril group call" state; and

d)	if the Call type IE of the received GROUP CALL ANNOUNCEMENT message is set to "BASIC GROUP CALL" and the stored call type is other than "BASIC GROUP CALL":

i)	shall set the stored current call type to "BASIC GROUP CALL";

ii)	shall set the stored current ProSe per-packet priority to value corresponding to MCPTT off-network basic group call as described in 3GPP TS 24.383 [45];

iii)	shall stop timer TFG13 (implicit downgrade), if running; and

iv)	shall enter "T2: in-progress basic group call" state; and

2)	if the stored last user to change call type of the call is different from the Last user to change call type IE of the GROUP CALL ANNOUNCEMENT message and:

a) if the stored call type is same as Call type IE in the received GROUP CALL ANNOUNCEMENT message and the stored last call type change time is smaller than Last call type change time IE of the GROUP CALL ANNOUNCEMENT message:

i)	shall set the stored last call type change time of the call to Last call type change time IE of the GROUP CALL ANNOUNCEMENT message; and

ii)	shall set the stored last user to change call type of the call to Last user to change call type IE of the GROUP CALL ANNOUNCEMENT message;

b)	if the Call type IE of the received GROUP CALL ANNOUNCEMENT message is set to "EMERGENCY GROUP CALL" and the stored call type is other than "EMERGENCY GROUP CALL":

i)	shall set the stored last call type change time of the call to Last call type change time IE of the GROUP CALL ANNOUNCEMENT message;

ii)	shall set the stored last user to change call type of the call to Last user to change call type IE of the GROUP CALL ANNOUNCEMENT message;

iii)	shall set the stored current call type to "EMERGENCY GROUP CALL";

iv)	shall set the stored current ProSe per-packet priority to value corresponding to MCPTT off-network emergency group call as described in 3GPP TS 24.383 [45];

v)	shall start timer TFG13 (implicit downgrade); and

vi)	shall enter "T1: in-progress emergency group call" state; and

c)	if the Call type IE of the received GROUP CALL ANNOUNCEMENT message is set to "IMMINENT PERIL GROUP CALL" and the stored call type is "BASIC GROUP CALL":

i)	shall set the stored last call type change time of the call to Last call type change time IE of the GROUP CALL ANNOUNCEMENT message;

ii)	shall set the stored last user to change call type of the call to Last user to change call type IE of the GROUP CALL ANNOUNCEMENT message;

iii)	shall set the stored current call type to "IMMINENT PERIL GROUP CALL ";

iv)	shall set the stored current ProSe per-packet priority to value corresponding to MCPTT off-network imminent peril group call as described in 3GPP TS 24.383 [45];

v)	shall start timer TFG13 (implicit downgrade); and

vi)	shall enter "T3: in-progress imminent peril group call" state.

##### 10.2.3.4.8	Downgrade call

###### 10.2.3.4.8.1	Originating user downgrading emergency group call

When in the "T1: in-progress emergency group call" state, upon receiving an indication from:

1)	the MCPTT user who upgraded the MCPTT group call; or

2.	an authorized MCPTT user with the value of "/<x>/<x>/Common/MCPTTGroupCall/EmergencyCall/CancelMCPTTGroup" leaf node present in the user profile as specified in 3GPP TS 24.383 [45] is set to "true",

to downgrade "EMERGENCY GROUP CALL", the MCPTT client:

1)	shall set the stored current call type to "BASIC GROUP CALL";

2)	shall set the stored current ProSe per-packet priority to value corresponding to MCPTT off-network basic group call as described in 3GPP TS 24.383 [45];

3)	shall set current UTC time as last call type change time of the call;

4)	shall store own MCPTT user ID as last user to change call type of the call;

5)	shall generate a GROUP CALL EMERGENCY END message as specified in subclause 15.1.15. In the GROUP CALL EMERGENCY END message, the MCPTT client:

a)	shall set the Call identifier IE to the stored call identifier of the call associated with the basic call control state machine;

b)	shall set the Originating MCPTT user ID IE to the stored originating MCPTT user ID of the call associated with the basic call control state machine;

c)	shall set the MCPTT group ID IE to the stored MCPTT group ID of the call associated with the basic call control state machine;

d)	shall set the Last call type change time IE to the stored last call type change time of the call; and

e)	shall set the Last user to change call type IE to the stored last user to change call type of the call;

6)	 shall send the GROUP CALL EMERGENCY END message as specified in subclause 10.2.1.1.1;

7)	shall initialize the counter CFG11 (emergency end retransmission) with value set to 1;

8)	shall start timer TFG11 (emergency end retransmission); and

9)	shall enter the "T2: in-progress basic group call" state.

###### 10.2.3.4.8.2	Retransmitting GROUP CALL EMERGENCY END

When in the "T2: in-progress basic group call" state, upon expiry of timer TFG11 (emergency end retransmission), the MCPTT client:

1)	shall generate a GROUP CALL EMERGENCY END message as specified in subclause 15.1.15. In the GROUP CALL EMERGENCY END message, the MCPTT client:

a)	shall set the Call identifier IE to the stored call identifier of the call associated with the basic call control state machine;

b)	shall set the Originating MCPTT user ID IE to the stored originating MCPTT user ID of the call associated with the basic call control state machine;

c)	shall set the MCPTT group ID IE to the stored MCPTT group ID of the call associated with the basic call control state machine;

d)	shall set the Last call type change time IE to the stored last call type change time of the call; and

e)	shall set the Last user to change call type IE to the stored last user to change call type of the call;

2)	 shall send the GROUP CALL EMERGENCY END message as specified in subclause 10.2.1.1.1;

3)	shall increment the value of the counter CFG11 (emergency end retransmission) by 1;

4)	shall start timer TFG11 (emergency end retransmission) if the value of the associated counter CFG11 (emergency end retransmission) is less than the upper limit; and

5)	shall remain in "T2: in-progress basic group call" state.

###### 10.2.3.4.8.3	Terminating user downgrading emergency group call

When in the "T1: in-progress emergency group call" state, upon receiving GROUP CALL EMERGENCY END message, the MCPTT client:

1)	shall set the stored last call type change time to the Last call type change time IE of the received GROUP CALL EMERGENCY END message;

2)	shall set the stored last user to change call type to the Last user to change call type IE of the received GROUP CALL EMERGENCY END message;

3)	shall set the stored current call type to "BASIC GROUP CALL";

4)	shall set the stored current ProSe per-packet priority to value corresponding to MCPTT off-network basic group call as described in 3GPP TS 24.383 [45];

5)	shall stop timer TFG13 (implicit downgrade); and

6)	shall enter the "T2: in-progress basic group call" state.

###### 10.2.3.4.8.4	Originating user downgrading imminent peril group call

When in the "T3: in-progress imminent peril group call" state, upon receiving an indication from:

1)	the MCPTT user who upgraded the call; or

2.	an authorized with the value of "/<x>/<x>/Common/MCPTTGroupCall/ImminentPerilCall/Cancel" leaf node present in the user profile as specified in 3GPP TS 24.383 [45] is set to "true",

to downgrade "IMMINENT PERIL GROUP CALL", the MCPTT client:

1)	shall set the stored current call type to "BASIC GROUP CALL";

2)	shall set the stored current ProSe per-packet priority to value corresponding to MCPTT off-network basic group call as described in 3GPP TS 24.383 [45];

3)	shall set current UTC time as last call type change time of the call;

4)	shall store own MCPTT user ID as last user to change call type of the call;

5)	shall generate a GROUP CALL IMMINENT PERIL END message as specified in subclause 15.1.14. In the GROUP CALL IMMINENT PERIL END message, the MCPTT client:

a)	shall set the Call identifier IE to the stored call identifier of the call associated with the basic call control state machine;

b)	shall set the Originating MCPTT user ID IE to the stored originating MCPTT user ID of the call associated with the basic call control state machine;

c)	shall set the MCPTT group ID IE to the stored MCPTT group ID of the call associated with the basic call control state machine;

d)	shall set the Last call type change time IE to the stored last call type change time of the call; and

e)	shall set the Last user to change call type IE to the stored last user to change call type of the call;

6)	shall send the GROUP CALL IMMINENT PERIL END message as specified in subclause 10.2.1.1.1;

7)	shall initialize the counter CFG12 (imminent peril end retransmission) with value set to 1;

8)	shall start timer TFG12 (imminent peril end retransmission); and

9)	shall enter the "T2: in-progress basic group call" state.

###### 10.2.3.4.8.5	Retransmitting GROUP CALL IMMINENT PERIL END

When in the "T2: in-progress basic group call" state, upon expiry of timer TFG12 (imminent peril end retransmission), the MCPTT client:

1)	shall generate a GROUP CALL IMMINENT PERIL END message as specified in subclause 15.1.14. In the GROUP CALL IMMINENT PERIL END message, the MCPTT client:

a)	shall set the Call identifier IE to the stored call identifier of the call associated with the basic call control state machine;

b)	shall set the Originating MCPTT user ID IE to the stored originating MCPTT user ID of the call associated with the basic call control state machine;

c)	shall set the MCPTT group ID IE to the stored MCPTT group ID of the call associated with the basic call control state machine;

d)	shall set the Last call type change time IE to the stored last call type change time of the call; and

e)	shall set the Last user to change call type IE to the stored last user to change call type of the call;

2)	 shall send the GROUP CALL IMMINENT PERIL END message as specified in subclause 10.2.1.1.1;

3)	shall increment the value of the counter CFG12 (imminent peril end retransmission) by 1;

4)	shall start the timer TFG12 (imminent peril end retransmission) if the value of the associated counter CFG12 (imminent peril end retransmission) is less than the upper limit; and

5)	shall remain in "T2: in-progress basic group call" state.

###### 10.2.3.4.8.6	Terminating user downgrading imminent peril group call

When in the "T3: in-progress imminent peril group call" state, upon receiving GROUP CALL IMMINENT PERIL END message, the MCPTT client:

1)	shall set the stored last call type change time to the Last call type change time IE of the received GROUP CALL IMMINENT PERIL END message;

2)	shall set the stored last user to change call type to the Last user to change call type IE of the received GROUP CALL IMMINENT PERIL END message;

3)	shall set the stored current call type to "BASIC GROUP CALL";

4)	shall set the stored current ProSe per-packet priority to value corresponding to MCPTT off-network basic group call as described in 3GPP TS 24.383 [45];

5)	shall stop timer TFG13 (implicit downgrade); and

6)	shall enter the "T2: in-progress basic group call" state.

###### 10.2.3.4.8.7	Reset TFG13 (implicit downgrade) timer

When in the "T1: in-progress emergency group call" state or "T3: in-progress imminent peril group call" state, upon receiving a GROUP CALL ANNOUNCEMENT message with Last user to change call type IE same as stored last user to change call type, the MCPTT client:

1)	shall restart timer TFG13 (implicit downgrade); and

2)	shall remain in the current state.

###### 10.2.3.4.8.8	Implicit priority end

When in the "T1: in-progress emergency group call" state or "T3: in-progress imminent peril call" state, upon expiry of timer TFG13 (implicit downgrade), the MCPTT client:

1)	shall store the current UTC time as the stored last call type change time of the call;

2)	shall store the originating MCPTT user ID as the stored last user to change call type of the call;

3)	shall set the stored current call type to "BASIC GROUP CALL";

4)	shall set the stored current ProSe per-packet priority to value corresponding to MCPTT off-network basic group call as described in 3GPP TS 24.383 [45]; and

5)	shall enter the "T2: in-progress basic group call" state.

##### 10.2.3.4.9	Merge of two calls

When in the "T1: in-progress emergency group call" state or "T2: in-progress basic group call" state or "T3: in-progress imminent peril group call" state, upon receiving a GROUP CALL ANNOUNCEMENT message with the MCPTT group ID IE matching the stored MCPTT group ID of the call and:

1)	the Originating MCPTT user ID IE is different from the stored originating MCPTT user ID of the call; or

2)	the Call identifier IE is different from the stored call identifier of the call;

then:

1)	if the stored current call type is "BASIC GROUP CALL" and the value of the Call type IE of GROUP CALL ANNOUNCEMENT message is either "IMMINENT PERIL GROUP CALL" or "EMERGENCY GROUP CALL"; or

2)	if the stored current call type is "IMMINENT PERIL GROUP CALL" and the value of the Call type IE of GROUP CALL ANNOUNCEMENT message is "EMERGENCY GROUP CALL"; or

3)	if the stored current call type being equal to the Call type IE of the GROUP CALL ANNOUNCEMENT message and the Call start time IE of the GROUP CALL ANNOUNCEMENT message being lower than the stored call start time of the call; or

4)	if the stored current call type being equal to the Call type IE of the GROUP CALL ANNOUNCEMENT message and the Call start time IE of the GROUP CALL ANNOUNCEMENT message being equal to the stored call start time of the call and the Call identifier IE of the GROUP CALL ANNOUNCEMENT message being lower than the stored call identifier of the call;

the MCPTT client:

1)	shall store the value of the Last call type change time IE of the received GROUP CALL ANNOUNCEMENT message as the last call type change time of the call;

2)	shall store the value of the Last user to change call type IE of the GROUP CALL ANNOUNCEMENT message as the last user to change call type of the call;

3)	shall store the value of the Call type IE of the GROUP CALL ANNOUNCEMENT message as the current call type of the call;

4)	if the Call type IE of GROUP CALL ANNOUNCEMENT message is set to "EMERGENCY GROUP CALL":

a)	shall set the stored current ProSe per-packet priority to value corresponding to MCPTT off-network emergency group call as described in 3GPP TS 24.383 [45]; and

b)	shall enter "T1: in-progress emergency group call" state; and

5)	if the Call type IE of GROUP CALL ANNOUNCEMENT message is set to "IMMINENT PERIL GROUP CALL":

a)	shall set the stored current ProSe per-packet priority to value corresponding to MCPTT off-network imminent peril group call as described in 3GPP TS 24.383 [45]; and

b)	shall enter "T3: in-progress imminent peril group call" state.

##### 10.2.3.4.10	Call release after call establishment

When in state T1: in-progress emergency group call" or "T2: in-progress basic group call" or "T3: in-progress imminent peril group call" or upon receiving an indication from MCPTT user to release the call, the MCPTT client:

1)	shall release stored current call type;

2)	shall release stored ProSe per-packet priority;

3)	shall release Last call type change time;

4)	shall release Last user to change call type; and

5)	shall enter "T0: waiting for the call to establish" state.

##### 10.2.3.4.11	Call release or reject before call establishment

When in state "T0: waiting for the call to establish", upon receiving an indication from MCPTT user to release or reject the call, the MCPTT client:

1)	shall release stored current call type;

2)	shall release stored ProSe per-packet priority;

3)	shall release Last call type change time;

4)	shall release Last user to change call type;

5)	shall remain in "T0: waiting for the call to establish" state.

##### 10.2.3.4.12	Error handling

###### 10.2.3.4.12.1	Unexpected MONP message received

Upon receiving a MONP message in a state where there is no handling specified for the MONP message, the MCPTT client shall discard the MONP message.

###### 10.2.3.4.12.2	Unexpected indication from MCPTT user

Upon receiving an indication from the MCPTT user in a state where there is no handling specified for the indication, the MCPTT client shall ignore the indication.

###### 10.2.3.4.12.3	Unexpected expiration of a timer

Upon expiration of a timer in a state where there is no handling specified for expiration of the timer, the MCPTT client shall ignore the expiration of the timer.

## 10.3	Off-network Broadcast group call

### 10.3.1	General

### 10.3.2	Basic call control

#### 10.3.2.1	General

#### 10.3.2.2	Call control state machine

The figure 10.3.2.2-1 gives an overview of the main states and transitions on the UE for broadcast group call call control.

Figure 10.3.2.2-1: Broadcast group call state machine

#### 10.3.2.3	Broadcast group call Control states

##### 10.3.2.3.1	B1: start-stop

This state exists for UE, when the UE is not part of an ongoing broadcast group call.

##### 10.3.2.3.2	B2: in-progress broadcast group call

This state exists for UE, when the UE is part of an ongoing broadcast group call.

##### 10.3.2.3.3	B3: pending user action

This state exists for the UE, when the UE has presented a notification to the MCPTT user for the received GROUP CALL BROADCAST message, is waiting for a response and is not expected to send confirm indication.

##### 10.3.2.3.4	B4: ignoring same call ID

This state exists for UE, when the group call was rejected or released and GROUP CALL BROADCAST messages continue being received.

#### 10.3.2.4	Procedures

##### 10.3.2.4.1	User initiating a broadcast group call

When in the "B1: start-stop" state, upon the indication from MCPTT user to initiate the broadcast group call, the MCPTT client:

1)	shall generate an SDP body as specified in subclause 10.2.1.1.2 and store it as the SDP body of the call;

2)	shall generate a random number with uniform distribution between 0 and 65535 and store it as the call identifier of the call;

3)	shall store own MCPTT user ID as the originating MCPTT user ID of the call;

4)	shall store "BROADCAST GROUP CALL" as the current call type;

5)	shall generate a GROUP CALL BROADCAST message as specified in subclause 15.1.20. In the GROUP CALL BROADCAST message, the MCPTT client:

a)	shall set the Call identifier IE to the stored call identifier of the call;

b)	shall set the Call type IE to the stored current call type;

c)	shall set the Originating MCPTT user ID IE to the stored originating MCPTT user ID of the call;

d)	shall set the MCPTT group ID IE to the stored MCPTT group ID of the call; and

e)	shall set the SDP IE to the stored SDP body of the call;

6)	shall set the ProSe per-packet priority to the value corresponding to MCPTT off-network broadcast callas described in 3GPP TS 24.383 [45];

7)	shall start floor control as originating floor participant as described specified in subclause 7.2 in 3GPP TS 24.380 [5];

8)	shall send the GROUP CALL BROADCAST message as specified in subclause 10.2.1.1.1;

9)	shall establish a media session based on the stored SDP body of the call;

10)	shall start timer TFB2 (broadcast retransmission); and

11)	shall enter the "B2: in-progress broadcast group call" state.

##### 10.3.2.4.2	Terminating UE receiving a GROUP CALL BROADCAST message when not participating in the in-progress broadcast group call

When in the "B1: start-stop" state, upon receiving a GROUP CALL BROADCAST message with the Call identifier IE not matching any in-progress broadcast group call, the MCPTT client:

1)	shall store the value of the Call identifier IE of the GROUP CALL BROADCAST message as the call identifier of the call;

2)	shall store the value of the Call type IE of the GROUP CALL BROADCAST message as the received current call type;

3)	shall store the value of the SDP IE of the GROUP CALL BROADCAST message as the SDP body of the call;

4)	shall store the value of the Originating MCPTT user ID IE of the GROUP CALL BROADCAST message as the originating MCPTT user ID of the call;

5)	shall store the value of the MCPTT group ID IE of the GROUP CALL BROADCAST message as the MCPTT group ID of the call;

6)	if the terminating UE is configured that the terminating MCPTT user acknowledgement is required upon a terminating call request reception:

i)	shall start timer TFB3 (waiting for the user); and

ii)	shall enter the "B3: pending user action" state; and

7)	if the terminating UE is configured that the terminating MCPTT user acknowledgement is not required upon a terminating call request reception:

i)	shall establish a media session based on the stored SDP body of the call;

ii) shall start floor control as terminating floor participant as specified in subclause 7.2 in 3GPP TS 24.380 [5];

iii)	shall start timer TFB1 (max duration); and

iv)	shall enter the "B2: in-progress broadcast group call" state.

##### 10.3.2.4.3	MCPTT user accepts the terminating call

When in the "B3: pending user action" state, upon indication from the MCPTT user to accept the incoming broadcast group call, the MCPTT client:

1)	shall establish a media session based on the stored SDP body of the call;

2)	shall start floor control as terminating floor participant as described specified in subclause 7.2 in 3GPP TS 24.380 [5];

3)	shall stop timer TFB3 (waiting for the user);

4)	shall start timer TFB1 (max duration); and

5)	shall enter the "B2: in-progress broadcast group call" state.

##### 10.3.2.4.4	MCPTT user rejects the terminating call

When in the "B3: pending user action" state, upon an indication from the MCPTT user to reject the incoming broadcast group call, the MCPTT client:

1)	shall stop timer TFB3 (waiting for the user); and

2)	shall enter the "B4: ignoring same call ID" state.

##### 10.3.2.4.5	MCPTT user does not act on terminating call

When in the "B3: pending user action" state, upon expiration of timer TFB3 (waiting for the user), the MCPTT client:

1)	shall enter the "B4: ignoring same call ID" state.

##### 10.3.2.4.6	Terminating user releasing the call

When in the "B2: in-progress broadcast group call" state, upon an indication from the terminating MCPTT user to release the in-progress broadcast group call, the MCPTT client:

1)	shall release the media session;

2)	shall stop floor control; and

3)	shall enter the "B4: ignoring same call ID" state.

##### 10.3.2.4.7	Originating user releasing the call

When in the "B2: in-progress broadcast group call" state, upon an indication from the originating MCPTT user to release the in-progress broadcast group call, the MCPTT client:

1)	shall release the media session;

2)	shall generate a GROUP CALL BROADCAST END message as specified in subclause 15.1.21. In the GROUP CALL BROADCAST END message, the MCPTT client:

a)	shall set the Call identifier IE to the stored call identifier of the call;

b)	shall set the Originating MCPTT user ID IE to the stored originating MCPTT user ID of the call; and

c)	shall set the MCPTT group ID IE to the stored MCPTT group ID of the call;

3)	shall send the GROUP CALL BROADCAST END message as specified in subclause 10.2.1.1.1;

4)	shall stop timer TFB2 (broadcast retransmission);

5)	shall stop floor control; and

6)	shall enter the "B1: start-stop" state.

##### 10.3.2.4.8	Receiving GROUP CALL BROADCAST END message

When in the "B2: in-progress broadcast group call" state or "B4: ignoring same call ID" state, upon receiving GROUP CALL BROADCAST END message with the same Call identifier IE as the stored call identifier, the MCPTT client:

1)	shall release media session;

2)	shall stop floor control, if running; and

3)	shall enter the "B1: start-stop" state.

##### 10.3.2.4.9	Originating UE retransmitting GROUP CALL BROADCAST message

When in the "B2: in-progress broadcast group call" state, upon expiry of timer TFB2 (broadcast retransmission), the MCPTT client:

1)	shall generate a GROUP CALL BROADCAST message as specified in subclause 15.1.20. In the GROUP CALL BROADCAST message, the MCPTT client:

a)	shall set the Call identifier IE to the stored call identifier of the call;

b)	shall set the Call type IE to the stored current call type;

c)	shall set the Originating MCPTT user ID IE to the stored originating MCPTT user ID of the call;

d)	shall set the MCPTT group ID IE to the stored MCPTT group ID of the call; and

e)	shall set the SDP IE to the stored SDP body of the call;

2)	shall send the GROUP CALL BROADCAST message as specified in subclause 10.2.1.1.1;

3)	shall restart timer TFB2 (broadcast retransmission); and

4)	shall remain in the "B2: in-progress broadcast group call" state.

##### 10.3.2.4.10	Ignoring same call ID

When in the "B4: ignoring same call ID" state, upon receiving GROUP CALL BROADCAST message and if the call identifier in GROUP CALL BROADCAST message matches with the stored call identifier the MCPTT client:

1)	shall restart timer TFB1 (max duration); and

2)	shall remain in "B4: ignoring same call ID" state.

##### 10.3.2.4.11	Releasing the call

When in the "B2: in-progress broadcast group call" state or "B4: ignoring same call ID" state, upon expiry of timer TFB1 (max duration) the MCPTT client:

1)	shall release the media session;

2)	shall clear the stored call identifier;

3)	shall stop floor control, if running; and

4)	shall enter the "B1: start-stop" state.

# 11	Private call

## 11.0	General

This subclause describes the private call procedures between two MCPTT clients for on-network and off-network.

For on-network, private call procedures with floor control are specified in subclause 11.1.1 and without floor control are specified in subclause 11.1.2.

For on-network, private call procedures are specified for the MCPTT client, the participating MCPTT function and the controlling MCPTT function on the originating side and terminating side.

For off-network, only private call procedures with floor control are specified in subclause 11.2.

For off-network, private call procedures are specified for the MCPTT client on the originating side and terminating side.

For both on-network and off-network private calls, the use of automatic commencement mode and manual commencement mode are specified.

## 11.1	On-network private call

### 11.1.1	Private call with floor control

#### 11.1.1.1	General

Subclause 11.1.1 specifies the MCPTT client procedures, participating MCPTT function procedures and controlling MCPTT function procedures for on-network private calls with floor control. The procedures cover both on-demand and pre-established session establishment. The procedures also cover emergency private call initiation, upgrade and cancellation.

#### 11.1.1.2	MCPTT client procedures

##### 11.1.1.2.1	On-demand private call

###### 11.1.1.2.1.1	Client originating procedures

Upon receiving a request from an MCPTT user to establish an MCPTT private call the MCPTT client shall generate an initial SIP INVITE request by following the UE originating session procedures specified in 3GPP TS 24.229 [4], with the clarifications given below.

The MCPTT client:

1)	shall set the Request-URI of the SIP INVITE request to a public service identity of the participating MCPTT function serving the MCPTT user;

2)	if the MCPTT user has requested the origination of an MCPTT emergency private call or is originating an MCPTT private call and the MCPTT emergency state is already set, the MCPTT client:

a)	shall, if this is an authorised request for an MCPTT emergency private call as determined by the procedures of subclause 6.2.8.3.1.1, comply with the procedures in subclause 6.2.8.3.2; and

b)	should, if this is an unauthorised request for an MCPTT emergency private call as determined in step a) above, indicate to the MCPTT user that they are not authorised to initiate an MCPTT emergency private call;

3)	may include a P-Preferred-Identity header field in the SIP INVITE request containing a public user identity as specified in 3GPP TS 24.229 [4];

4)	shall include the g.3gpp.mcptt media feature tag and the g.3gpp.icsi-ref media feature tag with the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" in the Contact header field of the SIP INVITE request according to IETF RFC 3840 [16];

5)	shall include an Accept-Contact header field containing the g.3gpp.mcptt media feature tag along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [6];

6)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Preferred-Service header field according to IETF RFC 6050 [9] in the SIP INVITE request;

7)	shall include an Accept-Contact header field with the media feature tag g.3gpp.icsi-ref contain with the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" along with parameters "require" and "explicit" according to IETF RFC 3841 [6];

8)	shall insert in the SIP INVITE request a MIME resource-lists body with the MCPTT ID of the invited MCPTT user, according to rules and procedures of IETF RFC 5366 [20];

9)	if an end-to-end security context needs to be established then:

a)	if necessary, shall instruct the key management client to request keying material from the key management server as described in 3GPP TS 33.179 [46];

b)	shall use the keying material to generate a PCK as described in 3GPP TS 33.179 [46];

c)	shall use the PCK to generate a PCK-ID with the four most significant bits set to "0101" to indicate that the purpose of the PCK is to protect private call communications and with the remaining twenty eight bits being randomly generated as described in 3GPP TS 33.179 [46];

d)	shall encrypt the PCK to a UID associated to the MCPTT client using the MCPTT ID of the invited user and a time related parameter as described in 3GPP TS 33.179 [46];

e)	shall generate a MIKEY-SAKKE I_MESSAGE using the encapsulated PCK and PCK-ID as specified in 3GPP TS 33.179 [46]; and

g)	shall add the MCPTT ID of the originating MCPTT to the initiator field (IDRi) of the I_MESSAGE as described in 3GPP TS 33.179 [46]; and

f)	shall sign the MIKEY-SAKKE I_MESSAGE using the originating MCPTT user's signing key provided in the keying material together with a time related parameter, and add this to the MIKEY-SAKKE payload, as described in 3GPP TS 33.179 [46].

10)	shall include an SDP offer according to 3GPP TS 24.229 [4] with the clarification given in subclause 6.2.1 and with a media stream of the offered media-floor control entity;

11)	if implicit floor control is required, shall comply with the conditions specified in subclause 6.4;

12)	if force of automatic commencement mode at the invited MCPTT client is requested by the MCPTT user, shall include in the SIP INVITE request a Priv-Answer-Mode header field with the value "Auto" according to the rules and procedures of IETF RFC 5373 [18];

13)	if force of automatic commencement mode at the invited MCPTT client is not requested by the MCPTT user and:

a)	if automatic commencement mode at the invited MCPTT client is requested by the MCPTT user, shall include in the SIP INVITE request an Answer-Mode header field with the value "Auto" according to the rules and procedures of IETF RFC 5373 [18]; and

b)	if manual commencement mode at the invited MCPTT client is requested by the MCPTT user, shall include in the SIP INVITE request an Answer-Mode header field with the value "Manual" according to the rules and procedures of IETF RFC 5373 [18];

14)	shall contain an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcpttinfo> element containing the <mcptt-Params> element with the <session-type> element set to a value of "private";

15)	if the MCPTT emergency private call state is set to either "MEPC 2: emergency-pc-requested" or "MEPC 3: emergency-pc-granted" or the MCPTT emergency private priority state for this private call is set to "MEPP 2: in-progress", the MCPTT client shall comply with the procedures in subclause 6.2.8.3.3; and

16)	shall send SIP INVITE request towards the MCPTT server according to 3GPP TS 24.229 [4].

Upon receiving a SIP 183(Session Progress) response to the SIP INVITE request the MCPTT client:

1)	may indicate the progress of the session establishment to the inviting MCPTT user.

Upon receiving a SIP 200 (OK) response to the SIP INVITE request the MCPTT client:

1)	shall interact with the media plane as specified in 3GPP TS 24.380 [5];

2)	if the MCPTT emergency private call state is set to "MEPC 2: emergency-pc-requested" or "MEPC 3: emergency-pc-granted", shall perform the actions specified in subclause 6.2.8.3.4; and

3)	shall notify the user that the call has been successfully established.

On receiving a SIP 4xx response, a SIP 5xx response or a SIP 6xx response to the SIP INVITE request:

1)	if the MCPTT emergency private call state is set to "MEPC 2: emergency-pc-requested"; or

2)	if the MCPTT emergency private call state is set to "MEPC 3: emergency-pc-granted";

the MCPTT client shall perform the actions specified in subclause 6.2.8.3.5.

On receiving a SIP INFO request where the Request-URI contains an MCPTT session ID identifying an ongoing group session, the MCPTT client shall follow the actions specified in subclause 6.2.8.3.7.

###### 11.1.1.2.1.2	Client terminating procedures

Upon receipt of an initial SIP INVITE request, the MCPTT client shall follow the procedures for termination of multimedia sessions in the IM CN subsystem as specified in 3GPP TS 24.229 [4] with the clarifications below.

The MCPTT client:

1)	may reject the SIP INVITE request if any of the following conditions are met:

a)	MCPTT client is already occupied in another session and the number of simultaneous sessions exceeds <MaxCall>, the maximum simultaneous MCPTT session for private call, as specified in TS 24.384 [50];

b)	MCPTT client does not have enough resources to handle the call; or

c)	any other reason outside the scope of this specification;

otherwise, continue with the rest of the steps.

NOTE:	If the SIP INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <emergency-ind> element set to a value of "true", the participating MCPTT function can choose to accept the request.

2)	if the SIP INVITE request is rejected in step 1), shall respond toward participating MCPTT function either with appropriate reject code as specified in 3GPP TS 24.229 [4] and warning texts as specified in subclause 4.4.2 or with SIP 480 (Temporarily unavailable) response not including warning texts if the user is authorised to restrict the reason for failure according to <allow-failure-restriction> as specified in 3GPP TS 24.384 [50] and skip the rest of the steps of this subclause;

3)	if the SIP INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcpttinfo> element containing the <mcptt-Params> element with the <emergency-ind> element set to a value of "true":

a)	should display to the MCPTT user an indication that this is a SIP INVITE request for an MCPTT emergency private call and:

i)	should display the MCPTT ID of the originator of the MCPTT emergency private call contained in the <mcptt-calling-user-id> element of the application/vnd.3gpp.mcptt-info+xml MIME body; and

ii)	if the <alert-ind> element is set to "true", should display to the MCPTT user an indication of the MCPTT emergency alert and associated information; and

b)	shall set the MCPTT emergency private priority state to "MEPP 2: in-progress" for this private call;

4)	if the SDP offer of the SIP INVITE request contains an "a=key-mgmt" attribute field with a "mikey" attribute value containing a MIKEY-SAKKE I_MESSAGE:

a)	shall extract the MCPTT ID of the originating MCPTT from the initiator field (IDRi) of the I_MESSAGE as described in 3GPP TS 33.179 [46];

b)	shall convert the MCPTT ID to a UID as described in 3GPP TS 33.179 [46];

c)	shall use the UID to validate the signature of the MIKEY-SAKKE I_MESSAGE as described in 3GPP TS 33.179 [46];

d)	if authentication verification of the MIKEY-SAKKE I_MESSAGE fails, shall reject the SIP INVITE request with a SIP 488 (Not Acceptable Here) response as specified in IETF RFC 4567 [47], and include warning text set to "136 authentication of the MIKEY-SAKE I_MESSAGE failed" in a Warning header field as specified in subclause 4.4; and

e)	if the signature of the MIKEY-SAKKE I_MESSAGE was successfully validated:

i)	shall extract and decrypt the encapsulated PCK using the terminating user's (KMS provisioned) UID key as described in 3GPP TS 33.179 [46]; and

ii)	shall extract the PCK-ID, from the payload as specified in 3GPP TS 33.179 [46];

NOTE:	With the PCK successfully shared between the originating MCPTT client and the terminating MCPTT client, both clients are able to use SRTP/SRTCP to create an end-to-end secure session.

5)	may check if a Resource-Priority header field is included in the incoming SIP INVITE request and may perform further actions outside the scope of this specification to act upon an included Resource-Priority header field as specified in 3GPP TS 24.229 [4];

6)	may display to the MCPTT user the MCPTT ID of the inviting MCPTT user;

7)	shall perform the automatic commencement procedures specified in subclause 6.2.3.1.1 if one of the following conditions are met:

a)	SIP INVITE request contains an Answer-Mode header field with the value "Auto" and the MCPTT service setting at the invited MCPTT client for answering the call is set to automatic commencement mode;

b)	SIP INVITE request contains an Answer-Mode header field with the value "Auto" and the MCPTT service setting at the invited MCPTT client for answering the call is set to manual commencement mode, yet the invited MCPTT client is willing to answer the call with automatic commencement mode; or

c)	SIP INVITE request contains a Priv-Answer-Mode header field with the value of "Auto"; and

8)	shall perform the manual commencement procedures specified in subclause 6.2.3.2.1 if either of the following conditions are met:

a)	SIP INVITE request contains an Answer-Mode header field with the value "Manual" and the MCPTT service setting at the invited MCPTT client for answering the call is set to manual commencement mode; or

b)	SIP INVITE request contains an Answer-Mode header field with the value "Manual" and the MCPTT service setting at the invited MCPTT client for answering the call is set to automatic commencement mode, yet the invited MCPTT client allows the call to be answered with manual commencement mode.

###### 11.1.1.2.1.3	Client terminating procedures for reception of SIP re-INVITE request

This subclause covers both on-demand session and pre-established sessions.

Upon receipt of a SIP re-INVITE request for an existing private call session, the MCPTT client shall:

1)	if the SIP re-INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcpttinfo> element containing the <mcptt-Params> element with the <emergency-ind> element set to a value of "true":

a)	should display to the MCPTT user an indication that this is a SIP re-INVITE request to upgrade this call to an MCPTT emergency private call and:

i)	should display the MCPTT ID of the originator of the MCPTT emergency private call contained in the <mcptt-calling-user-id> element of the application/vnd.3gpp.mcptt-info+xml MIME body; and

ii)	if the <alert-ind> element is set to "true", should display to the MCPTT user an indication of the MCPTT emergency alert and associated information; and

b)	shall set the MCPTT emergency private priority state to "MEPP 2: in-progress" for this private call;

2)	if the SIP re-INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcpttinfo> element containing the <mcptt-Params> element with the <emergency-ind> element set to a value of "false":

a)	should display to the MCPTT user an indication that this is a SIP re-INVITE request to downgrade this emergency private call to a normal priority private call and:

i)	should display the MCPTT ID of the sender of the SIP re-INVITE request contained in the <mcptt-calling-user-id> element of the application/vnd.3gpp.mcptt-info+xml MIME body; and

ii)	if the <alert-ind> element is set to "false" should display to the MCPTT user an indication that the MCPTT emergency alert is cancelled;

iii)	if the SIP re-INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body including an <originated-by> element:

A)	should display to the MCPTT user the MCPTT ID contained in the <originated-by> element of the MCPTT user that originated the MCPTT emergency alert; and

B)	if the MCPTT ID contained in the <originated-by> element is the MCPTT ID of the receiving MCPTT user, shall set the MCPTT emergency alert state to "MPEA 1: no-alert";

b)	shall set the MCPTT emergency private priority state to "MEPP 1: no-emergency" for this private call; and

c)	if the MCPTT emergency private call state of the call is set to "MEPC 3: emergency-call-granted", shall set the MCPTT emergency private call state of the call to "MEPC 1: emergency-pc-capable";

3)	may check if a Resource-Priority header field is included in the incoming SIP INVITE request and may perform further actions outside the scope of this specification to act upon an included Resource-Priority header field as specified in 3GPP TS 24.229 [4];

4)	may display to the MCPTT user the MCPTT ID of the inviting MCPTT user if not done so in step 1 or step 2 above;

NOTE 1:	As this is a re-INVITE for an existing MCPTT private call session, there is no attempt made to change the answer-mode from its current state.

5)	shall accept the SIP re-INVITE request and generate a SIP 200 (OK) response according to rules and procedures of 3GPP TS 24.229 [4];

6)	if the SIP re-INVITE request was received within an on-demand session, shall include an SDP answer in the SIP 200 (OK) response to the SDP offer in the incoming SIP INVITE request according to 3GPP TS 24.229 [4] with the clarifications given in subclause 6.2.2;

7)	if the SIP re-INVITE request was received within a pre-established session, shall include an SDP answer in the SIP 200 (OK) response to the SDP offer in the incoming SIP re-INVITE request according to 3GPP TS 24.229 [4], based upon the parameters already negotiated for the pre-established session;

NOTE 2:	The SIP re-INVITE request can be received within an on-demand session or a pre-established session. If the SIP re-INVITE request is received within a pre-established session, the media-level section for the MCPTT speech media stream and the media-level section of the media-floor control entity are expected to be the same as was negotiated in the existing pre-established session.

8)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcptt-called-party-id> element set to the MCPTT ID of the called MCPTT user, in the SIP 200 (OK) response;

9)	shall send the SIP 200 (OK) response towards the MCPTT server according to rules and procedures of 3GPP TS 24.229 [4]; and

10)	shall interact with the media plane as specified in 3GPP TS 24.380 [5].

###### 11.1.1.2.1.4	MCPTT in-progress emergency cancel

This subclause covers both on-demand session and pre-established sessions.

Upon receiving a request from an MCPTT user to cancel the in-progress emergency condition on an MCPTT emergency private call, the MCPTT client shall generate a SIP re-INVITE request by following the UE session procedures specified in 3GPP TS 24.229 [4], with the clarifications given below.

The MCPTT client:

1)	if the MCPTT user is not authorised to cancel the in-progress emergency condition on an MCPTT emergency private call as determined by the procedures of subclause 6.2.8.3.1.2, the MCPTT client:

a)	should indicate to the MCPTT user that they are not authorised to cancel the in-progress emergency condition on an MCPTT emergency private call; and

b)	shall skip the remaining steps of the current subclause;

2)	shall, if the MCPTT user is cancelling an in-progress emergency condition and optionally an MCPTT emergency alert originated by the MCPTT user,  include an application/vnd.3gpp.mcptt-info+xml MIME body populated as specified in subclause 6.2.8.3.6;

3)	shall, if the MCPTT user is cancelling an in-progress emergency condition and optionally an MCPTT emergency alert originated by another MCPTT user, include an application/vnd.3gpp.mcptt-info+xml MIME body populated as specified in subclause 6.2.8.3.8;

4)	shall include a Resource-Priority header field and comply with the procedures in subclause 6.2.8.3.3;

5)	shall include in the SIP re-INVITE request an SDP offer the media parameters as currently established;

NOTE 1:	The SIP re-INVITE request can be sent within an on-demand session or a pre-established session associated with an MCPTT group session. If the SIP re-INVITE request is sent within a pre-established session, the media-level section for the offered MCPTT speech media stream and the media-level section of the offered media-floor control entity are expected to be the same as was negotiated in the existing pre-established session.

6)	if an implicit floor request is required, shall indicate this as specified in subclause 6.4; and

7)	shall send the SIP re-INVITE request according to 3GPP TS 24.229 [4].

On receiving a SIP 2xx response to the SIP re-INVITE request, the MCPTT client:

1)	shall interact with the user plane as specified in 3GPP TS 24.380 [5];

2)	shall set the MCPTT emergency private priority state of the MCPTT private call to "MEPP 1: no-emergency";

3)	shall set the MCPTT emergency private call state of the call to "MEPC 1: emergency-pc-capable"; and

4)	if the MCPTT emergency alert state is set to "MPEA 4: Emergency-alert-cancel-pending", the sent SIP re-INVITE request did not contain an <originated-by> element in the application/vnd.3gpp.mcptt-info+xml MIME body and the SIP 2xx response to the SIP request for a priority group call does not contain a Warning header field as specified in subclause 4.4 with the warning text containing the mcptt-warn-code set to "149", shall set the MCPTT emergency alert state to "MPEA 1: no-alert".

On receiving a SIP 4xx response, SIP 5xx response or SIP 6xx response to the SIP re-INVITE request:

1)	if the SIP 4xx response, SIP 5xx response or SIP 6xx response contains an application/vnd.3gpp.mcptt-info+xml MIME body with an <emergency-ind> element set to a value of "true", the MCPTT client shall set the MCPTT emergency private priority state as "MEPP 2: in-progress";

2)	if the SIP 4xx response, SIP 5xx response or SIP 6xx response contains an application/vnd.3gpp.mcptt-info+xml MIME body with an <alert-ind> element set to a value of "false" and the sent SIP re-INVITE request did not contain an <originated-by> element in the application/vnd.3gpp.mcptt-info+xml MIME body, the MCPTT client shall set the MCPTT emergency alert state to "MPEA 1: no-alert"; and

NOTE 2:	This is the case where the MCPTT client requested the cancellation of the MCPTT emergency alert in addition to the MCPTT emergency alert and the cancellation of the MCPTT emergency alert was successful.

3)	if the SIP 4xx response, SIP 5xx response or SIP 6xx response contains an application/vnd.3gpp.mcptt-info+xml MIME body with an <alert-ind> element set to a value of "true" and the sent SIP re-INVITE request did not contain an <originated-by> element in the application/vnd.3gpp.mcptt-info+xml MIME body, the MCPTT client shall set the MCPTT emergency alert state to "MPEA 3: emergency-alert-initiated"; and

NOTE 3:	This is the case where the MCPTT client requested the cancellation of the MCPTT emergency alert in addition to the MCPTT emergency alert and the cancellation of the MCPTT emergency alert was not successful.

4)	if the SIP 4xx response, SIP 5xx response or SIP 6xx response did not contain an application/vnd.3gpp.mcptt-info+xml MIME body, shall set the MCPTT emergency private priority state as "MEPP 2: in-progress" and the MCPTT emergency alert (MPEA) state shall revert to its value prior to entering the current procedure.

NOTE 4:	If the in-progress emergency private priority state cancel request is rejected, the state of the session does not change, i.e. continues with MCPTT emergency private call level priority.

On receiving a SIP INFO request where the Request-URI contains an MCPTT session ID identifying an ongoing group session, the MCPTT client shall follow the actions specified in subclause 6.2.8.3.7.

###### 11.1.1.2.1.5	Upgrade to MCPTT emergency private call

This subclause covers both on-demand session and pre-established sessions.

Upon receiving a request from an MCPTT user to upgrade the ongoing MCPTT private call to an MCPTT emergency private call, the MCPTT client shall generate a SIP re-INVITE request as specified in 3GPP TS 24.229 [4], with the clarifications given below.

1)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body populated as specified in subclause 6.2.8.3.2;

2)	shall include a Resource-Priority header field and comply with the procedures in subclause 6.2.8.3.3.

3)	shall include an SDP offer with the media parameters as currently established according to 3GPP TS 24.229 [4];

NOTE:	The SIP re-INVITE request can be sent within an on-demand session or a pre-established session associated with an MCPTT private call. If the SIP re-INVITE request is sent within a pre-established session, the media-level section for the offered MCPTT speech media stream and the media-level section of the offered media-floor control entity are expected to be the same as was negotiated in the existing pre-established session.

4)	if an implicit floor request is required, shall indicate this as specified in subclause 6.4; and

5)	shall send the SIP re-INVITE request according to 3GPP TS 24.229 [4].

On receiving a SIP 2xx response to the SIP re-INVITE request the MCPTT client:

1)	shall interact with the user plane as specified in 3GPP TS 24.380 [5]; and

2)	shall perform the actions specified in subclause 6.2.8.3.4.

On receiving a SIP 4xx response, SIP 5xx response or SIP 6xx response to the SIP re-INVITE request, the MCPTT client shall perform the actions specified in subclause 6.2.8.3.5.

On receiving a SIP INFO request where the Request-URI contains an MCPTT session ID identifying an ongoing group session, the MCPTT client shall follow the actions specified in subclause 6.2.8.3.7

##### 11.1.1.2.2	Private call using pre-established session

###### 11.1.1.2.2.1	Client originating procedures

Upon receiving a request from an MCPTT user to establish an MCPTT private call within a pre-established session the MCPTT client shall generate a SIP REFER request outside a dialog in accordance with the procedures specified in 3GPP TS 24.229 [4], IETF RFC 4488 [22] and IETF RFC 3515 [25] as updated by IETF RFC 6665 [26] and IETF RFC 7647 [27], with the clarifications given below.

If an end-to-end security context needs to be established the MCPTT client:

1)	if necessary, shall instruct the key management client to request keying material from the key management server as described in 3GPP TS 33.179 [46];

2)	shall use the keying material to generate a PCK as described in 3GPP TS 33.179 [46];

3)	shall use the PCK to generate a PCK-ID with the four most significant bits set to "0100" to indicate that the purpose of the PCK is to protect private call communications and with the remaining twenty eight bits being randomly generated as described in 3GPP TS 33.179 [46];

4)	shall encrypt the PCK to a UID associated to the MCPTT client using the MCPTT ID of the invited user and a time related parameter as described in 3GPP TS 33.179 [46];

5)	shall generate a MIKEY-SAKKE I_MESSAGE using the encapsulated PCK and PCK-ID as specified in 3GPP TS 33.179 [46];

6)	shall add the MCPTT ID of the originating MCPTT to the initiator field (IDRi) of the I_MESSAGE as described in 3GPP TS 33.179 [46]; and

7)	shall sign the MIKEY-SAKKE I_MESSAGE using the originating MCPTT user's signing key provided in the keying material together with a time related parameter, and add this to the MIKEY-SAKKE payload, as described in 3GPP TS 33.179 [46].

The MCPTT client populates the SIP REFER request as follows:

1)	shall include the Request-URI set to the public service identity identifying the pre-established session on the MCPTT server serving the MCPTT user;

2)	shall include the Refer-Sub header field with value "false" according to rules and procedures of IETF RFC 4488 [22];

3)	shall include the Supported header field with value "norefersub" according to rules and procedures of IETF RFC 4488 [22];

4)	shall include the option tag "multiple-refer" in the Require header field;

5)	may include a P-Preferred-Identity header field in the SIP REFER request containing a public user identity as specified in 3GPP TS 24.229 [4];

6)	shall include a P-Preferred-Service header field set to the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), according to IETF RFC 6050 [9];

7)	shall set the Refer-To header field of the SIP REFER request as specified in IETF RFC 3515 [25] with a Content-ID ("cid") Uniform Resource Locator (URL) as specified in IETF RFC 2392 [62] that points to an application/resource-lists MIME body as specified in IETF RFC 5366 [20], and with the Content-ID header field set to this "cid" URL.

8)	shall include in the application/resource-lists MIME body a single <entry> element containing a "uri" attribute set to the MCPTT ID of the called user, extended with the following URI header fields:

NOTE:	Characters that are not formatted as ASCII characters are escaped in the following URI header fields

a)	if force of automatic commencement mode at the invited MCPTT client is requested by the MCPTT user, shall include a Priv-Answer-Mode header field with the value "Auto" according to the rules and procedures of IETF RFC 5373 [18];

b)	if force of automatic commencement mode at the invited MCPTT client is not requested by the MCPTT user and:

i)	if automatic commencement mode at the invited MCPTT client is requested by the MCPTT user, shall include an Answer-Mode header field with the value "Automatic" according to rules and procedures of IETF RFC 5373 [18]; and

ii)	if manual commencement mode at the invited MCPTT client is requested by the MCPTT user, shall include an Answer-Mode header field with the value "Manual" according to rules and procedures of IETF RFC 5373 [18]; and

c)	shall include in a hname "body" URI header field:

i)	if the SDP parameters of the pre-established session do not contain a media-level section of a media-floor control entity or if end-to-end security is required for the private call, an application/sdp MIME body containing the SDP parameters of the pre-established session according to 3GPP TS 24.229 [4] with the clarification given in subclause 6.2.1;

ii)	an application/vnd.3gpp.mcptt-info MIME body with the <session-type> element set to "private"; and

iii)	an application/resources-list MIME body with an <entry> element containing a "uri" attribute set to the MCPTT ID of the called user;

9)	may include a Resource-Priority header field according to rules and procedures of IETF RFC 4412 [29] in the SIP REFER request as well as the Refer-to URI parameter; and

Editor's Note [CT1#94, C1-153765]: The values of Resource-Priority header field and the detailed conditions for inclusion are FFS.

10)	shall include a Target-Dialog header field as specified in IETF RFC 4538 [23] identifying the pre-established session.

The MCPTT client shall send the SIP REFER request towards the MCPTT server according to 3GPP TS 24.229 [4].

Upon receiving a final SIP 2xx response to the SIP REFER request the MCPTT client shall interact with media plane as specified in 3GPP TS 24.380 [5].

###### 11.1.1.2.2.2	Client terminating procedures

The MCPTT client shall follow the procedures for termination of multimedia sessions as specified in subclause 11.1.1.2.1.2 with the following clarifications:

1)	if the MCPTT client is targeted for a new MCPTT emergency private call, the MCPTT client receives a SIP INVITE with an application/vnd.3gpp.mcptt-info+xml MIME body with an <emergency-ind> set to a value of "true"; or

2)	if the MCPTT client is targeted for a new normal priority private call, the MCPTT client receives a SIP re-INVITE request rather than a SIP INVITE request.

#### 11.1.1.3	Participating MCPTT function procedures

##### 11.1.1.3.1	Originating procedures

###### 11.1.1.3.1.1	On-demand private call

Upon receipt of a "SIP INVITE request for originating participating MCPTT function" containing an application/vnd.3gpp.mcptt-info+xml MIME body with the <session-type> element set to a value of "private", the participating MCPTT function:

1)	may reject the SIP INVITE request depending on the value of the Resource-Priority header field if the Resource-Priority header field is included in the received SIP INVITE request according to rules and procedures specified in IETF RFC 4412 [29] and shall not continue with the rest of the steps;

2)	if unable to process the request due to a lack of resources or a risk of congestion exists, may reject the "SIP INVITE request for originating participating MCPTT function" with a SIP 500 (Server Internal Error) response. The participating MCPTT function may include a Retry-After header field to the SIP 500 (Server Internal Error) response as specified in IETF RFC 3261 [24] and shall not continue with the rest of the steps;

NOTE 1:	If the received SIP INVITE request contains an emergency indication set to a value of "true", the participating MCPTT function can choose to accept the request.

NOTE 2:	If the received SIP INVITE request contains an emergency indication set to a value of "true", the participating MCPTT function can choose to allow an exception to the limit on the number of private calls and accept the request.

3)	shall determine the MCPTT ID of the calling user from public user identity in the P-Asserted-Identity header field of the SIP INVITE request and shall authorise the user;

NOTE 3:	The MCPTT ID of the calling user is bound to the public user identity at the time of service authorisation, as documented in subclause 7.3.

4)	if the participating MCPTT function cannot find a binding between the public user identity and an MCPTT ID or if the validity period of an existing binding has expired, then the participating MCPTT function shall reject the SIP INVITE request with a SIP 404 (Not Found) response with the warning text set to "141 user unknown to the participating function" in a Warning header field as specified in subclause 4.4, and shall not continue with any of the remaining steps;

5)	shall determine the public service identity of the controlling MCPTT function for the private call service associated with the originating user's MCPTT ID identity. If the participating MCPTT function is unable to identify the controlling MCPTT function for the private call service associated with the originating user's MCPTT ID identity, it shall reject the SIP INVITE request with a SIP 404 (Not Found) response with the warning text "142 unable to determine the controlling function" in a Warning header field as specified in subclause 4.4, and shall not continue with any of the remaining steps;

6)	if the incoming SIP INVITE request does not contain an application/resource-lists MIME body or contains an application/resource-lists MIME body with more than one <entry> element, shall reject the "SIP INVITE request for originating participating MCPTT function" with a SIP 403 (Forbidden) response including warning text set to "145 unable to determine called party" in a Warning header field as specified in subclause 4.4, and shall not continue with the rest of the steps;

7)	if the <allow-private-call> element of the <ruleset> element is not present in the MCPTT user profile document on the participating MCPTT function or is present with the value "false" (see the MCPTT user profile document in 3GPP TS 24.384 [50]), indicating that the user identified by the MCPTT ID is not authorised to initiate private calls, shall reject the "SIP INVITE request for originating participating MCPTT function" with a SIP 403 (Forbidden) response, with warning text set to "107 user not authorised to make private calls" in a Warning header field as specified in subclause 4.4, and shall not continue with the rest of the steps;

8)	if the received SIP INVITE request includes an Answer-Mode header field as specified in IETF RFC 5373 [18] with the value "Auto" and the <allow-automatic-commencement> element of the <ruleset> element is not present in the MCPTT user profile document on the participating MCPTT function or is present with the value "false" (see the MCPTT user profile document in 3GPP TS 24.384 [50]) indicating that the user identified by the MCPTT ID is not authorised to initiate private call with automatic commencement, shall reject the "SIP INVITE request for originating participating MCPTT function" with a SIP 403 (Forbidden) response including warning text set to "125 user not authorised to make private call with automatic commencement" in a Warning header field as specified in subclause 4.4, and shall not continue with the rest of the steps;

9)	if the received SIP INVITE request includes an Answer-Mode header field as specified in IETF RFC 5373 [18] with the value "Manual" and the <allow-manual-commencement> element of the <ruleset> element is not present in the MCPTT user profile document on the participating MCPTT function or is present with the value "false" (see the MCPTT user profile document in 3GPP TS 24.384 [50]), indicating that the user identified by the MCPTT ID is not authorised to initiate private call with manual commencement, shall reject the "SIP INVITE request for originating participating MCPTT function" with a SIP 403 (Forbidden) response including warning text set to "126 user not authorised to make private call with manual commencement" in a Warning header field as specified in subclause 4.4, and shall not continue with the rest of the steps;

10)	if the <PrivateCall> element exists in the MCPTT user profile document with one more <entry> elements (see the MCPTT user profile document in 3GPP TS 24.384 [50]) and:

a)	if the "uri" attribute of the <entry> element of the application/resource-lists MIME body does not match with one of the <entry> elements of the <PrivateCall> element of the MCPTT user profile document (see the MCPTT user profile document in 3GPP TS 24.384 [50]); and

b)	if configuration is not set in the MCPTT user profile document (see the MCPTT user profile document in 3GPP TS 24.384 [50]) that allows the MCPTT user to make a private call to users not contained within the <entry> elements of the <PrivateCall> element;

then:

a)	shall reject the "SIP INVITE request for originating participating MCPTT function" with a SIP 403 (Forbidden) response including warning text set to "144 user not authorised to call this particular user" in a Warning header field as specified in subclause 4.4 and shall not continue with the rest of the steps;

11)	shall validate the media parameters and if the MCPTT speech codec is not offered in the "SIP INVITE request for originating participating MCPTT function" shall reject the request with a SIP 488 (Not Acceptable Here) response. Otherwise, continue with the rest of the steps;

12)	shall, using the MCPTT ID of the calling user, determine the public service identity of the controlling MCPTT function hosting the private call service;

13)	shall generate a SIP INVITE request as specified in subclause 6.3.2.1.3;

14)	shall set the Request-URI to the public service identity of the controlling MCPTT function hosting the private call service;

15)	shall set the <mcptt-calling-user-identity> element in an application/vnd.3gpp.mcptt-info+xml MIME body of the SIP INVITE request to the MCPTT ID of the calling user;

16)	if a Priv-Answer-Mode header field specified in IETF RFC 5373 [18] was received in the incoming SIP INVITE request with a value of "Manual", shall not include a Priv-Answer-Mode header field in the outgoing SIP INVITE request;

17)	if the <allow-force-auto-answer> element of the <ruleset> element is not present in the MCPTT user profile document on the participating MCPTT function or is present with the value "false" (see the MCPTT user profile document in 3GPP TS 24.384 [50]), and the Priv-Answer-Mode header field specified in IETF RFC 5373 [18] was received in the incoming SIP INVITE request with a value of "Auto", shall reject the "SIP INVITE request for originating participating MCPTT function" with a SIP 403 (Forbidden) response including warning text set to "143 not authorised to force auto answer" in a Warning header field as specified in subclause 4.4, and shall not continue with the rest of the steps;

18)	if the <allow-force-auto-answer> element of the <ruleset> element is present in the MCPTT user profile document with the value "true" (see the MCPTT user profile document in 3GPP TS 24.384 [50]) on the participating MCPTT function, and the Priv-Answer-Mode header field specified in IETF RFC 5373 [18] was received in the incoming SIP INVITE request with a value of "Auto", shall include the Priv-Answer-Mode header field set to a value of "Auto" in the outgoing SIP INVITE request;

19)	if a Priv-Answer-Mode header field containing the value of "Auto" has not been included in the outgoing SIP INVITE request as specified in step 17) above and the incoming "SIP INVITE request for originating participating MCPTT function" contained an Answer-Mode header field as specified in IETF RFC 5373 [18], then shall populate the Answer-Mode header field of the outgoing SIP INVITE request with the contents of the Answer-Mode header field from the incoming "SIP INVITE request for originating participating MCPTT function";

20) shall include in the SIP INVITE request an SDP offer based on the SDP offer in the received "SIP INVITE request for originating participating MCPTT function", as specified in subclause 6.3.2.1.1.1;

21) shall include a Resource-Priority header field according to rules and procedures of 3GPP TS 24.229 [4] set to the value indicated in the Resource-Priority header field if included in the SIP INVITE request from the MCPTT client; and

22) shall forward the SIP INVITE request, according to 3GPP TS 24.229 [4].

Upon receiving a SIP 180 (Ringing) response, the participating MCPTT function:

1)	shall generate a SIP 180 (Ringing) response to the SIP INVITE request as specified in the subclause 6.3.2.1.5.1;

2)	shall include the P-Asserted-Identity header field as received in the incoming SIP 180 (Ringing) response;

3)	shall include Warning header field(s) received in the incoming SIP 180 (Ringing) response; and

4)	shall forward the SIP 180 (Ringing) response to the MCPTT client according to 3GPP TS 24.229 [4].

Upon receiving a SIP 200 (OK) response, the participating MCPTT function:

1)	shall generate a SIP 200 (OK) response as specified in the subclause 6.3.2.1.5.2;

2)	shall include in the SIP 200 (OK) response an SDP answer as specified in the subclause 6.3.2.1.2.1;

3)	shall include Warning header field(s) received in the incoming SIP 200 (OK) response;

4)	shall include the P-Asserted-Identity header field received in the incoming SIP 200 (OK) response into the outgoing SIP 200 (OK) response;

5)	shall include an MCPTT session identity mapped to the MCPTT session identity provided in the Contact header field of the received SIP 200 (OK) response;

6)	shall send the SIP 200 (OK) response to the MCPTT client according to 3GPP TS 24.229 [4];

7)	shall interact with the media plane as specified in 3GPP TS 24.380 [5]; and

8)	shall start the SIP session timer according to rules and procedures of IETF RFC 4028 [7].

The participating MCPTT function shall forward any other SIP response that does not contain SDP, including any MIME bodies contained therein, along the signalling path to the originating network according to 3GPP TS 24.229 [4].

###### 11.1.1.3.1.2	Private call initiation using pre-established session

Upon receipt of a "SIP REFER request for a pre-established session", with:

1)	the Refer-To header field containing a Content-ID ("cid") Uniform Resource Locator (URL) as specified in IETF RFC 2392 [62] that points to an application/resource-lists MIME body as specified in IETF RFC 5366 [20] containing an <entry> element with a "uri" attribute containing a SIP-URI set to the MCPTT ID of the called user;

2)	a body" URI header field of the SIP-URI specified above containing an application/vnd.3gpp.mcptt-info MIME body with the <session-type> element set to "private"; and

3)	a Content-ID header field set to the "cid" URL;

the participating function:

1)	if unable to process the request due to a lack of resources or a risk of congestion exists, may reject the SIP INVITE request with a SIP 500 (Server Internal Error) response. The participating MCPTT function may include a Retry-After header field to the SIP 500 (Server Internal Error) response as specified in IETF RFC 3261 [24] and shall not continue with the rest of the steps;

Editor's Note [CT1#94, C1-153764]: The priority mechanisms are still to be specified for MCPTT. The priority mechanisms may be a factor in determining whether to reject or allow the request.

2)	shall determine the MCPTT ID of the calling user from public user identity in the P-Asserted-Identity header field of the SIP REFER request;

3)	if the participating MCPTT function cannot find a binding between the public user identity and an MCPTT ID or if the validity period of an existing binding has expired, then the participating MCPTT function shall reject the SIP REFER request with a SIP 404 (Not Found) response with the warning text set to "141 user unknown to the participating function" in a Warning header field as specified in subclause 4.4, and shall not continue with any of the remaining steps;

4)	shall determine the public service identity of the controlling MCPTT function for the private call service associated with the originating user's MCPTT ID identity. If the participating MCPTT function is unable to identify the controlling MCPTT function for the private call service associated with the originating user's MCPTT ID identity, it shall reject the REFER request with a SIP 404 (Not Found) response with the warning text "142 unable to determine the controlling function" in a Warning header field as specified in subclause 4.4, and shall not continue with any of the remaining steps;

5)	if the <allow-private-call> element of the <ruleset> element is not present in the MCPTT user profile document (see the MCPTT user profile document in 3GPP TS 24.384 [50]) on the participating MCPTT function or is present with the value "false", indicating that the user identified by the MCPTT ID is not authorised to initiate private calls, shall reject the "SIP REFER request for pre-established session" with a SIP 403 (Forbidden) response to the SIP INVITE request, with warning text set to "107 user not authorised to make private calls" in a Warning header field as specified in subclause 4.4;

6)	if the received SIP REFER request does not contain an application/resource-lists MIME body referenced by a "cid" URL in the Refer-To header field, or contains an application/resource-lists MIME body with more than one <entry> element, shall reject the "SIP REFER request for pre-established session" with a SIP 403 (Forbidden) response including warning text set to "145 unable to determine called party" in a Warning header field as specified in subclause 4.4, and shall not continue with the rest of the steps;

7)	if the received SIP REFER request includes an Answer-Mode header field as specified in IETF RFC 5373 [18] set to "Auto" contained in the header portion of the SIP URI present in the application/resource-lists MIME body referenced by a "cid" URL in the Refer-To header field, and the <allow-automatic-commencement> element of the <ruleset> element is not present in the MCPTT user profile document (see the MCPTT user profile document in 3GPP TS 24.384 [50]) on the participating MCPTT function or is present with the value "false" (indicating that the user identified by the MCPTT ID is not authorised to initiate private call with automatic commencement), shall reject the "SIP REFER request for pre-established session' with a SIP 403 (Forbidden) response including warning text set to "125 user not authorised to make private call with automatic commencement" in a Warning header field as specified in subclause 4.4, and shall not continue with the rest of the steps;

8)	if the received SIP REFER request includes an Answer-Mode header field as specified in IETF RFC 5373 [18] set to "Manual" contained in the header portion of the SIP URI present in the application/resource-lists MIME body referenced by a "cid" URL in the Refer-To header field, and the <allow-manual-commencement> element of the <ruleset> element is not present in the MCPTT user profile document (see the MCPTT user profile document in 3GPP TS 24.384 [50]) on the participating MCPTT function or is present with the value "false" (indicating that the user identified by the MCPTT ID is not authorised to initiate private call with manual commencement), shall reject the "SIP REFER request for pre-established session' with a SIP 403 (Forbidden) response including warning text set to "126 user not authorised to make private call with manual commencement" in a Warning header field as specified in subclause 4.4 and shall not continue with the rest of the steps;

9)	if the <allow-force-auto-answer> element of the <ruleset> element is not present in the MCPTT user profile document (see the MCPTT user profile document in 3GPP TS 24.384 [50]) on the participating MCPTT function or is present with the value "false", and the SIP REFER request contained a Priv-Answer-Mode header field as specified in IETF RFC 5373 [18] set to "Auto" in the header portion of the SIP URI in the application/resource-lists MIME body referenced by a "cid" URL in the Refer-To header field, shall reject the "SIP INVITE request for originating participating MCPTT function" with a SIP 403 (Forbidden) response including warning text set to "143 not authorised to force auto answer" in a Warning header field as specified in subclause 4.4, and shall not continue with the rest of the steps;

10)	if the <PrivateCall> element exists in the MCPTT user profile document with one more <entry> elements (see the MCPTT user profile document in 3GPP TS 24.384 [50]) and:

a)	if the SIP-URI in the application/resource-lists MIME body referenced by a "cid" URL in the Refer-To header field not match with one of the <entry> elements of the <PrivateCall> element of the MCPTT user profile document (see the MCPTT user profile document in 3GPP TS 24.384 [50]); and

b)	if configuration is not set in the MCPTT user profile document (see the MCPTT user profile document in 3GPP TS 24.384 [50]) that allows the MCPTT user to make a private call to users not contained within the <entry> elements of the <PrivateCall> element;

then:

a)	shall reject the "SIP INVITE request for originating participating MCPTT function" with a SIP 403 (Forbidden) response including warning text set to "144 user not authorised to call this particular user" in a Warning header field as specified in subclause 4.4 and shall not continue with the rest of the steps;

11)	shall using the MCPTT ID of the calling user, determine the public service identity of the controlling MCPTT function associated with the private call service;

NOTE 1:	How the participating MCPTT server discovers the public service identity of the controlling MCPTT function associated with the private call service of the calling user is out of scope of the current document.

12)	if the "SIP REFER request for a pre-established session" contained a Refer-Sub header field containing "false" value and a Supported header field containing "norefersub" value, shall handle the SIP REFER request as specified in 3GPP TS 24.229 [4], IETF RFC 3515 [25] as updated by IETF RFC 6665 [26], and IETF RFC 4488 [22] without establishing an implicit subscription;

13)	shall generate a final SIP 200 (OK) response to the "SIP REFER request for a pre-established session" according to 3GPP TS 24.229 [4];

NOTE 2:	In accordance with IETF RFC 4488 [22], the participating MCPTT function inserts the Refer-Sub header field containing the value "false" in the SIP 200 (OK) response to the SIP REFER request to indicate that it has not created an implicit subscription.

14)	shall send the response to the "SIP REFER request for a pre-established session" towards the MCPTT client according to 3GPP TS 24.229 [4];

15)	shall generate a SIP INVITE request as specified in subclause 6.3.2.1.4;

16)	shall set the Request-URI of the SIP INVITE request to the public service identity of the controlling MCPTT function hosting the private call service for the calling MCPTT user;

17)	if the SIP REFER request contained a Priv-Answer-Mode header field as specified in IETF RFC 5373 [18] set to "Manual" in the header portion of the SIP URI in the application/resource-lists MIME body referenced by a "cid" URL in the Refer-To header field, shall copy the Priv-Answer-Mode header field from the incoming SIP REFER request to the outgoing SIP INVITE request;

18)	if the <allow-force-auto-answer> element of the <ruleset> element is present in the MCPTT user profile document with the value "true" (see the MCPTT user profile document in 3GPP TS 24.384 [50]) on the participating MCPTT function, and the Priv-Answer-Mode header field specified in IETF RFC 5373 [18] was received in the header portion of the SIP URI in the application/resource-lists MIME body referenced by a "cid" URL in the Refer-To header field, with a value set to "Auto", shall copy the Priv-Answer-Mode header field to the outgoing SIP INVITE request;

19)	if a Priv-Answer-Mode header field containing the value of "Auto" has not been copied to the outgoing SIP INVITE request as specified in step 16) above, and the incoming SIP REFER request contained an Answer-Mode header field in the headers portion of the SIP URI in the application/resource-lists referenced by a "cid" URL in the Refer-To header field, then copy the Answer-Mode header field to the outgoing SIP INVITE request; and

20)	shall forward the SIP INVITE request according to 3GPP TS 24.229 [4].

Upon receiving SIP provisional responses for the SIP INVITE request the participating MCPTT function:

1)	shall discard the received SIP responses without forwarding them.

Upon receiving a SIP 200 (OK) response for the SIP INVITE request the participating MCPTT function:

1)	shall interact with the media plane as specified in 3GPP TS 24.380 [5].

###### 11.1.1.3.1.3	Receipt of SIP re-INVITE for MCPTT private call from the served user

This subclause covers both on-demand session and pre-established sessions.

Upon receipt of a SIP re-INVITE request for an existing MCPTT private call session the participating MCPTT function:

1)	may reject the SIP re-INVITE request depending on the value of the Resource-Priority header field if the Resource-Priority header field is included in the received SIP re-INVITE request according to rules and procedures specified in IETF RFC 4412 [29] and skip the rest of the steps;

2)	if unable to process the request due to a lack of resources or a risk of congestion exists, may reject the "SIP INVITE request for originating participating MCPTT function" with a SIP 500 (Server Internal Error) response. The participating MCPTT function may include a Retry-After header field to the SIP 500 (Server Internal Error) response as specified in IETF RFC 3261 [24];

NOTE 1:	If the SIP re-INVITE request contains an emergency indication, the participating MCPTT function can choose to accept the request.

3)	shall determine the MCPTT ID of the calling user from public user identity in the P-Asserted-Identity header field of the SIP INVITE request and shall authorise the user;

NOTE 2:	The MCPTT ID of the calling user is bound to the public user identity at the time of service authorisation, as documented in subclause 7.3.

4)	shall validate the media parameters and if the MCPTT speech codec is not offered in the SIP re-INVITE request shall reject the request with a SIP 488 (Not Acceptable Here) response and skip the rest of the steps;

NOTE 3:	If the received SIP re-INVITE request is received within a pre-established session, associated with an MCPTT private call, the media-level section for the offered MCPTT speech media stream and the media-level section of the offered media-floor control entity are expected to be the same as was negotiated in the existing pre-established session.

5)	shall generate a SIP re-INVITE request as specified in subclause 6.3.2.1.9;

6)	shall set the <mcptt-calling-user-identity> element in an application/vnd.3gpp.mcptt-info+xml MIME body of the SIP re-INVITE request to the MCPTT ID of the calling user;

7)	shall, if the SIP re-INVITE request was received within an on-demand session include in the SIP re-INVITE request an SDP containing the current media parameters used by the existing session;

8)	shall, if the SIP re-INVITE request was received within a pre-established session, include in the SIP re-INVITE request an SDP offer based upon the previously negotiated SDP for the pre-established session as specified in subclause 6.3.2.1.1.2;

9)	shall include a Resource-Priority header field according to rules and procedures of 3GPP TS 24.229 [4] set to the value indicated in the Resource-Priority header field if included in the SIP re-INVITE request from the MCPTT client; and

10)	shall forward the SIP re-INVITE request, according to 3GPP TS 24.229 [4].

Upon receiving a SIP 200 (OK) response, the participating MCPTT function:

1)	shall generate a SIP 200 (OK) response as specified in the subclause 6.3.2.1.5.2;

2)	if the SIP 200 (OK) response is to be sent within an on-demand session shall include in the SIP 200 (OK) response an SDP answer as specified in the subclause 6.3.2.1.2.1;

3)	if the SIP 200 (OK) response is to be sent within a pre-established session shall include in the SIP 200 (OK) response an SDP answer based upon the previously negotiated SDP for the pre-established session;

4)	shall include Warning header field(s) received in the incoming SIP 200 (OK) response;

5)	shall include the P-Asserted-Identity header field received in the incoming SIP 200 (OK) response into the outgoing SIP 200 (OK) response;

6)	shall send the SIP 200 (OK) response to the MCPTT client according to 3GPP TS 24.229 [4]; and

7)	shall interact with the media plane as specified in 3GPP TS 24.380 [5].

The participating MCPTT function shall forward any other SIP response that does not contain SDP, including any MIME bodies contained therein, along the signalling path to the originating network according to 3GPP TS 24.229 [4].

##### 11.1.1.3.2	Terminating procedures

NOTE:	This subclause covers both on demand session and pre-established session.

Upon receipt of a "SIP INVITE request for terminating participating MCPTT function", the participating MCPTT function:

1)	if unable to process the request due to a lack of resources or a risk of congestion exists, may reject the "SIP INVITE request for terminating participating MCPTT function" with a SIP 500 (Server Internal Error) response. The participating MCPTT function may include a Retry-After header field to the SIP 500 (Server Internal Error) response as specified in IETF RFC 3261 [24], and shall not continue with the rest of the steps;

NOTE:	If the received SIP INVITE request contains an emergency indication set to a value of "true", the participating MCPTT function can choose to accept the request.

2)	shall check the presence of the isfocus media feature tag in the URI of the Contact header field and if it is not present then the participating MCPTT function shall reject the request with a SIP 403 (Forbidden) response with the warning text set to "104 isfocus not assigned" in a Warning header field as specified in subclause 4.4, and shall not continue with the rest of the steps;

3)	if the Answer-Mode Indication in the application/poc-settings+xml MIME body has not yet been received from the invited MCPTT client as defined in subclause 7.3.3 or subclause 7.3.4, shall reject the request with a SIP 480 (Temporarily Unavailable) response with the warning text set to "146 T-PF unable to determine the service settings for the called user" in a Warning header field as specified in subclause 4.4 and shall not continue with the rest of the steps;

4)	shall use the MCPTT ID present in the <mcptt-request-uri> element of the application/vnd.3gpp.mcptt-info+xml MIME body of the incoming SIP INVITE request to retrieve the binding between the MCPTT ID and public user identity;

5)	if the binding between the MCPTT ID and public user identity does not exist, then the participating MCPTT function shall reject the SIP INVITE request with a SIP 404 (Not Found) response. Otherwise, continue with the rest of the steps;

6)	when the called user identified by the MCPTT ID is unable to participate in private calls as identified in the called user's MCPTT user profile document (see the MCPTT user profile document in 3GPP TS 24.384 [50]) on the terminating participating MCPTT function, shall reject the "SIP INVITE request for terminating participating MCPTT function" with a SIP 403 (Forbidden) response including warning text set to "127 user not authorised to be called in private call" in a Warning header field as specified in subclause 4.4;

7)	shall perform the automatic commencement procedures specified in subclause 6.3.2.2.5.1 and according to IETF RFC 5373 [18] if one of the following conditions are met:

a)	"SIP INVITE request for terminating participating MCPTT function" contains an Answer-Mode header field with the value "Auto";

b)	"SIP INVITE request for terminating participating MCPTT function" does not contain an Answer-Mode header field and the Answer-Mode Indication received in the application/poc-settings+xml MIME body received from the invited MCPTT client as per subclause 7.3.3 or subclause 7.3.4 is set to "auto-answer"; or

c)	"SIP INVITE request for terminating participating MCPTT function" contains a Priv-Answer-Mode header field with the value "Auto"; and

7)	shall perform the manual commencement procedures specified in subclause 6.3.2.2.6.1 and according to IETF RFC 5373 [18] if either of the following conditions are met:

a)	"SIP INVITE request for terminating participating MCPTT function" contains an Answer-Mode header field with the value "Manual"; or

b)	"SIP INVITE request for terminating participating MCPTT function" does not contain an Answer-Mode header field and Answer-Mode Indication received in the application/poc-settings+xml MIME body received from the invited MCPTT client as per subclause 7.3.3 or subclause 7.3.4 is set to "manual-answer".

##### 11.1.1.3.3	Receipt of SIP re-INVITE request by terminating participating function

This subclause covers the on-demand session case only.

Upon receipt of a SIP re-INVITE request for an existing MCPTT private call session the participating MCPTT function:

1)	if unable to process the request due to a lack of resources or a risk of congestion exists, may reject the SIP re-INVITE with a SIP 500 (Server Internal Error) response. The participating MCPTT function may include a Retry-After header field to the SIP 500 (Server Internal Error) response as specified in IETF RFC 3261 [24] and skip the rest of the steps;

NOTE 1:	If the SIP re-INVITE request contains an emergency indication, the participating MCPTT function can choose to accept the request.

2)	shall use the MCPTT ID present in the <mcptt-request-uri> element of the application/vnd.3gpp.mcptt-info+xml MIME body of the incoming SIP re-INVITE request to retrieve the binding between the MCPTT ID and public user identity;

3)	if the binding between the MCPTT ID and public user identity does not exist, then the participating MCPTT function shall reject the SIP re-INVITE request with a SIP 404 (Not Found) response and skip the rest of the steps;

4)	shall generate a SIP re-INVITE as specified in subclause 6.3.2.2.10;

NOTE 2:	As this is the modification of an in-progress MCPTT private call, this procedure does not attempt modification of the existing answer-mode of the call.

5)	shall include in the SIP re-INVITE request an SDP offer containing the current media parameters used by the existing session; and

6)	shall send the SIP re-INVITE request towards the MCPTT client according to 3GPP TS 24.229 [4].

Upon receiving the SIP 200 (OK) response to the SIP re-INVITE request, the participating MCPTT function:

1)	shall generate a SIP 200 (OK) response as described in the subclause 6.3.2.2.4.2;

2)	shall include in the SIP 200 (OK) response an SDP answer based on the SDP answer in the received SIP 200 (OK) response as specified in subclause 6.3.2.2.2.1;

3)	shall copy the P-Asserted-Identity header field from the incoming SIP 200 (OK) response to the outgoing SIP 200 (OK) response;

4)	shall interact with the media plane as specified in 3GPP TS 24.380 [5]; and

5)	shall forward the SIP 200 (OK) response according to 3GPP TS 24.229 [4].

The participating MCPTT function shall forward any other SIP response that does not contain SDP along the signalling path to the originating network according to 3GPP TS 24.229 [4].

#### 11.1.1.4	Controlling MCPTT function procedures

##### 11.1.1.4.1	Originating procedures

This subclause describes the procedures for inviting an MCPTT user to an MCPTT session. The procedure is initiated by the controlling MCPTT function as the result of an action in subclause 11.1.1.4.2

The controlling MCPTT function:

1)	shall generate a SIP INVITE request as specified in subclause 6.3.3.1.2;

NOTE 1:	As a result of calling subclause 6.3.3.1.2, the <mcptt-calling-user-id> containing the calling user's MCPTT ID is copied into the outgoing SIP INVITE.

2)	if the received SIP INVITE request contains an authorised request for an MCPTT emergency private call as determined by subclause 6.3.3.1.13.2:

a)	shall set the <emergency-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body to a value of "true";

b)	if the received SIP INVITE request contains an alert indication set to a value of "true" and this is an authorised request for an MCPTT emergency alert meeting the conditions specified in subclause 6.3.3.1.13.1, perform the procedures specified in subclause 6.3.3.1.12; and

c)	if the received SIP INVITE request did not contain an alert indication or contains an alert indication set to a value of "true" and is not an authorised request for an MCPTT emergency alert meeting the conditions specified in subclause 6.3.3.1.13.1, shall set the <alert-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body to a value of "false";

3)	shall copy the MCPTT ID of the MCPTT user listed in the MIME resources body of the incoming SIP INVITE request, into the <mcptt-request-uri> element in the application/vnd.3gpp.mcptt-info+xml MIME body of the outgoing SIP INVITE request;

4)	shall set the Request-URI to the public service identity of the terminating participating MCPTT function associated to the MCPTT user to be invited;

NOTE 2:	How the controlling MCPTT function finds the address of the terminating MCPTT participating function is out of the scope of the current release.

NOTE 3:	If the terminating MCPTT user is part of a partner MCPTT system, then the public service identity can identify an entry point in the partner network that is able to identify the terminating participating MCPTT function.

5)	shall copy the public user identity of the calling MCPTT user from the P-Asserted-Identity header field of the incoming SIP INVITE request into the P-Asserted-Identity header field of the SIP INVITE request;

6)	shall include a Resource-Priority header field populated with the values for an MCPTT emergency private call as specified in subclause 6.3.3.1.19, if either of the following conditions is met:

a)	if the received SIP INVITE request contains an authorised request for an MCPTT emergency private call as determined in step 2 above; or

b)	the originating MCPTT user is in an in-progress emergency private call state with the targeted MCPTT user;

7)	shall include in the SIP INVITE request an SDP offer based on the SDP offer in the received SIP INVITE request from the originating network according to the procedures specified in subclause 6.3.3.1.1;

8)	shall send the SIP INVITE request towards the core network according to 3GPP TS 24.229 [4]; and

9)	shall start a private call timer with a value set to the configured max private call duration for the user.

Upon receiving SIP 200 (OK) response for the SIP INVITE request the controlling MCPTT function:

1)	shall cache the contact received in the Contact header field; and

2)	shall interact with the media plane as specified in 3GPP TS 24.380 [5].

Upon expiry of the private call timer, the controlling MCPTT function shall follow the procedure for releasing private call session as specified in subclause 11.1.4.4.

##### 11.1.1.4.2	Terminating procedures

In the procedures in this subclause:

1)	emergency indication in an incoming SIP INVITE request refers to the <emergency-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body; and

2)	alert indication in an incoming SIP INVITE request refers to the <alert-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body.

Upon receiving of a "SIP INVITE request for controlling MCPTT function of a private call" the controlling MCPTT function:

1)	shall check whether the public service identity contained in the Request-URI is allocated for private call and perform the actions specified in subclause 6.3.7.1 if it is not allocated and skip the rest of the steps;

2)	shall perform actions to verify the MCPTT ID of the inviting MCPTT user in the <mcptt-calling-user-id> element of the application/vnd.3gpp.mcptt-info+xml MIME body of the SIP INVITE request, and authorise the request according to local policy, and if it is not authorised the controlling MCPTT function shall return a SIP 403 (Forbidden) response with the warning text as specified in "Warning header field" and skip the rest of the steps;

3)	shall validate that the received SDP offer includes at least one media stream for which the media parameters and at least one codec or media format is acceptable by the controlling MCPTT function and if not, reject the request with a SIP 488 (Not Acceptable Here) response and skip the rest of the steps;

4)	if received SIP INVITE request includes an application/vnd.3gpp.mcptt-info+xml MIME body with an <emergency-ind> element included, shall validate the request as described in subclause 6.3.3.1.17;

5)	if the received SIP INVITE request contains an unauthorised request for an MCPTT emergency private call as determined by subclause 6.3.3.1.13.2; and

a)	if the received SIP INVITE request contains an alert indication set to "true" and this is an authorised request for an MCPTT emergency alert meeting the conditions specified in subclause 6.3.3.1.13.1, shall:

i)	cache the information that this MCPTT user has initiated an MCPTT emergency alert to the targeted MCPTT user; and

ii)	reject the SIP INVITE request with a SIP 403 (Forbidden) response to the SIP INVITE request, with the following clarifications:

A)	shall generate the SIP 403 (Forbidden) response as specified in subclause 6.3.3.1.14; and

B)	shall send the SIP 403 (Forbidden) response as specified in 3GPP TS 24.229 [4] and skip the rest of the steps;

b)	if the SIP INVITE request contains an alert indication set to "true" and this is an unauthorised request for an MCPTT emergency alert as specified in subclause 6.3.3.1.13.1, shall reject the SIP INVITE request with the following clarifications:

i)	shall generate the SIP 403 (Forbidden) response as specified in subclause 6.3.3.1.14; and

ii)	shall send the SIP 403 (Forbidden) response as specified in 3GPP TS 24.229 [4] and skip the rest of the steps; and

c)	if the SIP INVITE request contains an alert indication set to "false" shall reject the SIP INVITE request with a SIP 403 (Forbidden) response with the following clarifications:

i)	shall generate the SIP 403 (Forbidden) response as specified in subclause 6.3.3.1.14; and

ii)	shall send the SIP 403 (Forbidden) response as specified in 3GPP TS 24.229 [4] and skip the rest of the steps;

6)	if a Resource-Priority header field is included in the received SIP INVITE request and if the Resource-Priority header field is set to the value indicated for emergency calls, shall reject the SIP INVITE request with a SIP 403 (Forbidden) response and skip the remaining steps if neither one of the following conditions are true:

a)	the SIP INVITE request does not contain an authorised request for an MCPTT emergency call as determined in step 4 above; or

b)	the originating MCPTT user is not in an in-progress emergency private call state with the targeted MCPTT user;

7)	if the SIP INVITE request contains an emergency indication set to a value of "true" and the originating MCPTT user is not in an in-progress emergency private call state with the targeted MCPTT user:

a)	shall cache the information that this MCPTT user has initiated an MCPTT emergency private call to the targeted user; and

b)	shall cache the information that this MCPTT user is in an in-progress emergency private call state with the targeted MCPTT user;

8)	shall perform actions as described in subclause 6.3.3.2.2;

9)	shall allocate an MCPTT session identity for the MCPTT private call session; and

10)	shall invite the MCPTT user listed in the MIME resource-lists body of received SIP INVITE request as specified in subclause 11.1.1.4.1.

Upon receiving a SIP 180 (Ringing) response and if the SIP 180 (Ringing) response or the SIP final response has not yet been sent to the inviting MCPTT client, the controlling MCPTT function:

1)	shall generate a SIP 180 (Ringing) response to the SIP INVITE request and send the SIP 180 (Ringing) response towards the inviting MCPTT client according to 3GPP TS 24.229 [4].

Upon receiving a SIP 200 (OK) response for the SIP INVITE request and if the SIP final response has not yet been sent to the inviting MCPTT client the, controlling MCPTT function:

1)	shall generate a SIP 200 (OK) response to the SIP INVITE request as specified in the subclause 6.3.3.2.3.2 before continuing with the rest of the steps;

2)	shall include in the SIP 200 (OK) response an SDP answer to the SDP offer in the incoming SIP INVITE request as specified in the subclause 6.3.3.2.2;

3)	if the received SIP INVITE request contains an alert indication set to a value of "true" and this is an unauthorised request for an MCPTT emergency alert as specified in subclause 6.3.3.1.13.1, shall include in the SIP 200 (OK) response the warning text set to "149 SIP INFO request pending" in a Warning header field as specified in subclause 4.4;

NOTE 1:	This is the case when the MCPTT user's request for an MCPTT emergency private call was granted but the request for the MCPTT emergency alert was denied.

4)	shall interact with the media plane as specified in 3GPP TS 24.380 [5]; and

NOTE 2:	Resulting media plane processing is completed before the next step is performed.

5)	shall send a SIP 200 (OK) response towards the inviting MCPTT client according to 3GPP TS 24.229 [4].

Upon receiving a SIP ACK to the SIP 200 (OK) response sent towards the inviting MCPTT client, and the SIP 200 (OK) response was sent with the warning text set to "149 SIP INFO request pending" in a Warning header field as specified in subclause 4.4, the controlling MCPTT function shall follow the procedures in subclause 6.3.3.1.18.

The controlling MCPTT function shall forward any other SIP response that does not contain SDP, including any MIME bodies contained therein, along the signalling path to the originating network according to 3GPP TS 24.229 [4].

##### 11.1.1.4.3	Receiving a SIP re-INVITE for upgrade to emergency private call

In the procedures in this subclause:

1)	emergency indication in an incoming SIP re-INVITE request refers to the <emergency-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body; and

2)	alert indication in an incoming SIP re-INVITE request refers to the <alert-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body.

Upon receiving a SIP re-INVITE request with an emergency indication set to a value of "true", the controlling MCPTT function:

1)	shall validate that the received SDP offer includes at least one media stream for which the media parameters and at least one codec or media format is acceptable by the controlling MCPTT function and if not, reject the request with a SIP 488 (Not Acceptable Here) response and skip the rest of the steps;

2)	shall validate the request as described in subclause 6.3.3.1.17;

3)	if the SIP re-INVITE request contains an unauthorised request for an MCPTT emergency private call as determined by subclause 6.3.3.1.13.2; and

a)	if the SIP re-INVITE request contains an alert indication set to "true" and this is an authorised request for an MCPTT emergency alert meeting the conditions specified in subclause 6.3.3.1.13.1:

i)	shall cache the information that this MCPTT user has initiated an MCPTT emergency alert to the targeted user; and

ii)	shall reject the SIP re-INVITE request with a SIP 403 (Forbidden) response to the SIP re-INVITE request, with the following clarifications:

A)	shall generate the SIP 403 (Forbidden) response as specified in subclause 6.3.3.1.14; and

B)	shall send the SIP 403 (Forbidden) response as specified in 3GPP TS 24.229 [4] and skip the rest of the steps;

b)	if the SIP re-INVITE request contains an alert indication set to "true" and this is an unauthorised request for an MCPTT emergency alert as specified in subclause 6.3.3.1.13.1, shall reject the SIP re-INVITE request with the following clarifications:

i)	shall generate the SIP 403 (Forbidden) response as specified in subclause 6.3.3.1.14; and

ii)	shall send the SIP 403 (Forbidden) response as specified in 3GPP TS 24.229 [4] and skip the rest of the steps; and

c)	if the SIP re-INVITE request contains an alert indication set to "false" shall reject the SIP re-INVITE request with a SIP 403 (Forbidden) response with the following clarifications:

i)	shall generate the SIP 403 (Forbidden) response as specified in subclause 6.3.3.1.14; and

ii)	shall send the SIP 403 (Forbidden) response as specified in 3GPP TS 24.229 [4] and skip the rest of the steps;

4)	if a Resource-Priority header field is included in the received SIP re-INVITE request and if the Resource-Priority header field is set to the value indicated for emergency calls, shall reject the SIP re-INVITE request with a SIP 403 (Forbidden) response and skip the remaining steps if neither of the following conditions are true:

a)	the SIP re-INVITE request does contains an authorised request for an MCPTT emergency call as determined in step 2 above; or

b)	the originating MCPTT user is in an in-progress emergency private call state with the targeted MCPTT user;

5)	if the SIP re-INVITE request contains an emergency indication set to a value of "true" and the originating MCPTT user is not in an in-progress emergency private call state with the targeted MCPTT user:

a)	shall cache the information that this MCPTT user is in an in-progress emergency private call state with the targeted MCPTT user; and

b)	if the SIP re-INVITE request contains an alert indication set to "true" and this is an authorised request for an MCPTT emergency alert as specified in subclause 6.3.3.1.13.1, shall cache the information that this MCPTT user has sent an MCPTT emergency alert to the targeted user; and

6)	shall send a SIP re-INVITE invite towards the MCPTT user listed in the MIME resource-lists body of received SIP re-INVITE request as specified in subclause 11.1.1.4.5.

Upon receiving a SIP 200 (OK) response for the SIP re-INVITE request and if the SIP response has not yet been sent to the inviting MCPTT client, the controlling MCPTT function:

1)	shall generate a SIP 200 (OK) response to the SIP re-INVITE request as specified in the subclause 6.3.3.2.3 before continuing with the rest of the steps;

2)	shall include in the SIP 200 (OK) response an SDP answer to the SDP offer in the incoming SIP re-INVITE request containing the current media parameters used by the existing session;

3)	if the received SIP re-INVITE request contains an alert indication set to a value of "true" and this is an unauthorised request for an MCPTT emergency alert as specified in subclause 6.3.3.1.13.1, shall include in the SIP 200 (OK) response the warning text set to "149 SIP INFO request pending" in a Warning header field as specified in subclause 4.4.

NOTE:	When a SIP 200 (OK) response sent to the originator as a response to a SIP INVITE request that contained authorised request(s) for an MCPTT emergency private call and optionally an MCPTT emergency alert the originator will consider a SIP 200 (OK) response populated in this manner as confirmation that its request(s) for an upgrade to an MCPTT emergency private call and optionally an MCPTT emergency alert were accepted by the controlling function.

4)	shall interact with the media plane as specified in 3GPP TS 24.380 [5]; and

5)	shall send the SIP 200 (OK) response towards the inviting MCPTT client according to 3GPP TS 24.229 [4].

Upon receiving a SIP ACK to the SIP 200 (OK) response sent towards the inviting MCPTT client, and the SIP 200 (OK) response was sent with the warning text set to "149 SIP INFO request pending" in a Warning header field as specified in subclause 4.4, the controlling MCPTT function shall follow the procedures in subclause 6.3.3.1.18:

The controlling MCPTT function shall forward any other SIP response that does not contain SDP, including any MIME bodies contained therein, along the signalling path to the originating network according to 3GPP TS 24.229 [4].

##### 11.1.1.4.4	Receiving a SIP re-INVITE for cancellation of emergency private call

In the procedures in this subclause:

1)	emergency indication in an incoming SIP INVITE request refers to the <emergency-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body; and

2)	alert indication in an incoming SIP INVITE request refers to the <alert-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body.

Upon receiving a SIP re-INVITE request with an emergency indication set to a value of "false", the controlling MCPTT function:

1)	shall validate that the received SDP offer includes at least one media stream for which the media parameters and at least one codec or media format is acceptable by the controlling MCPTT function and if not, reject the request with a SIP 488 (Not Acceptable Here) response and skip the rest of the steps;

2)	shall validate the request as described in subclause 6.3.3.1.17;

3)	if the SIP re-INVITE request contains an unauthorised request for an MCPTT emergency private call cancellation as determined by subclause 6.3.3.1.13.4:

a)	if the SIP re-INVITE request contains an alert indication set to "false" and this is an authorised request for of an MCPTT emergency alert cancellation meeting the conditions specified in subclause 6.3.3.1.13.3, shall:

i)	if the received SIP re-INVITE request contains an <originated-by> element in the application/vnd.3gpp.mcptt-info+xml MIME body, clear the cache of the MCPTT ID of the MCPTT user identified by the <originated-by> element as having an outstanding MCPTT emergency alert; and

ii)	if the received SIP re-INVITE request does not contain an <originated-by> element in the application/vnd.3gpp.mcptt-info+xml MIME body, clear the cache of the MCPTT ID of the sender of the SIP re-INVITE request as having an outstanding MCPTT emergency alert; and

iii)	reject the SIP re-INVITE request with a SIP 403 (Forbidden) response to the SIP INVITE request, with the following clarifications:

A)	shall generate the SIP 403 (Forbidden) response according to 3GPP TS 24.229 [4];

B)	shall include in the SIP 403 (Forbidden) response an application/vnd.3gpp.mcptt-info+xml MIME body with an <emergency-ind> element set to "true";

C)	shall include in the SIP 403 (Forbidden) response and application/vnd.3gpp.mcptt-info+xml MIME body with an <alert-ind> element set to "false"; and

D)	shall send the SIP 403 (Forbidden) response as specified in 3GPP TS 24.229 [4] and skip the rest of the steps;

b)	if the SIP re-INVITE request contains an alert indication set to "false" and this is an unauthorised request for an MCPTT emergency alert cancellation as specified in subclause 6.3.3.1.13.3, shall reject the SIP re-INVITE request with the following clarifications:

i)	shall generate the SIP 403 (Forbidden) response according to 3GPP TS 24.229 [4];

ii)	shall include in the SIP 403 (Forbidden) response an application/vnd.3gpp.mcptt-info+xml MIME body with an <emergency-ind> element set to "true";

iii)	shall include in the SIP 403 (Forbidden) response and application/vnd.3gpp.mcptt-info+xml MIME body with an <alert-ind> element set to "true"; and

iv)	shall send the SIP 403 (Forbidden) response as specified in 3GPP TS 24.229 [4] and skip the rest of the steps;

4)	shall reject the SIP re-INVITE request with a SIP 403 (Forbidden) response if a Resource-Priority header field is included in the received SIP re-INVITE request set to the value configured for emergency calls, and skip the remaining steps; and

5	if the SIP re-INVITE request contains an authorised request for an MCPTT emergency private call cancellation as determined by subclause 6.3.3.1.13.4:

a)	shall clear the cache of the MCPTT ID of the originator of the MCPTT emergency private call that is no longer in an in-progress emergency private call state with the targeted MCPTT user; and

b)	if the SIP re-INVITE request contains an alert indication set to "false" and this is an authorised request for an MCPTT emergency alert cancellation meeting the conditions specified in subclause 6.3.3.1.13.3:

i)	if the received SIP re-INVITE request contains an <originated-by> element in the application/vnd.3gpp.mcptt-info+xml MIME body,, shall clear the cache of the MCPTT ID of MCPTT user identified by the <originated-by> element as having an outstandingMCPTT emergency alert; and

ii)	if the received SIP re-INVITE request does not contain an <originated-by> element in the application/vnd.3gpp.mcptt-info+xml MIME body, clear the cache of the MCPTT ID of the sender of the SIP re-INVITE request as having an outstanding MCPTT emergency alert;

6)	shall send a SIP re-INVITE request towards the MCPTT user listed in the MIME resource-lists body of received SIP re-INVITE request as specified in subclause 11.1.1.4.6.

Upon receiving a SIP 200 (OK) response for the SIP re-INVITE request and if the SIP response has not yet been sent to the inviting MCPTT client, the controlling MCPTT function:

1)	shall generate a SIP 200 (OK) response to the SIP re-INVITE request as specified in the subclause 6.3.3.2.3 before continuing with the rest of the steps;

2)	shall include in the SIP 200 (OK) response an SDP answer to the SDP offer in the incoming SIP re-INVITE request as specified in the subclause 6.3.3.2.2;

3)	if the received SIP re-INVITE request contains an alert indication set to a value of "false" and this is an unauthorised request for an MCPTT emergency alert cancellation as specified in subclause 6.3.3.1.13.3, shall include in the SIP 200 (OK) response the warning text set to "149 SIP INFO request pending" in a Warning header field as specified in subclause 4.4.

NOTE:	When a SIP 200 (OK) response sent to the originator as a response to a SIP INVITE request that contained authorised request(s) for an MCPTT emergency private call cancellation and optionally an MCPTT emergency alert cancellation, the originator will consider a SIP 200 (OK) response populated in this manner as confirmation that its request(s) for cancellation of an MCPTT emergency private call and optionally an MCPTT emergency alert were accepted by the controlling function.

4)	shall interact with the media plane as specified in 3GPP TS 24.380 [5]; and

5)	shall send the SIP 200 (OK) response towards the inviting MCPTT client according to 3GPP TS 24.229 [4].

Upon receiving a SIP ACK to the SIP 200 (OK) response sent towards the inviting MCPTT client, and the SIP 200 (OK) response was sent with the warning text set to "149 SIP INFO request pending" in a Warning header field as specified in subclause 4.4, the controlling MCPTT function shall follow the procedures in subclause 6.3.3.1.18.

The controlling MCPTT function shall forward any other SIP response that does not contain SDP, including any MIME bodies contained therein, along the signalling path to the originating network according to 3GPP TS 24.229 [4].

##### 11.1.1.4.5	Sending a SIP re-INVITE for upgrade to emergency private call

This subclause describes the procedures for sending a re-INVITE request to an MCPTT user in an MCPTT private call for the purpose of upgrading the session to an emergency private call session. The procedure is initiated by the controlling MCPTT function as the result of an action in subclause 11.1.1.4.3.

The controlling MCPTT function:

1)	shall generate a SIP re-INVITE request as specified in subclause 6.3.3.1.9;

2)	if the received SIP re-INVITE request contained an application/vnd.3gpp.mcptt-info+xml MIME body, shall copy the application/vnd.3gpp.mcptt-info+xml MIME body to the outgoing re-INVITE request;

3)	if the received SIP re-INVITE request contains an authorised request for an MCPTT emergency private call as determined by subclause 6.3.3.1.13.2:

a)	shall set the <emergency-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body to a value of "true";

b)	if the received SIP INVITE request contains an alert indication set to a value of "true" and this is an authorised request for an MCPTT emergency alert meeting the conditions specified in subclause 6.3.3.1.13.1, perform the procedures specified in subclause 6.3.3.1.12; and

c)	if the received SIP INVITE request did not contain an alert indication or contains an alert indication set to a value of "true" and is not an authorised request for an MCPTT emergency alert meeting the conditions specified in subclause 6.3.3.1.13.1, shall set the <alert-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body to a value of "false";

4)	shall include a Resource-Priority header field populated with the values for an MCPTT emergency private call as specified in subclause 6.3.3.1.19, if the received SIP re-INVITE request contains an authorised request for an MCPTT emergency private call as determined in step 2 above; and

5)	shall send the SIP re-INVITE request towards the core network according to 3GPP TS 24.229 [4].

Upon receiving SIP 200 (OK) response for the SIP re-INVITE request the controlling MCPTT function:

1)	shall cache the contact received in the Contact header field.

##### 11.1.1.4.6	Sending a SIP re-INVITE for cancellation of emergency private call

This subclause describes the procedures for sending a re-INVITE request to an MCPTT user in an MCPTT emergency private call for the purpose of downgrading the session to a normal priority private call session. The procedure is initiated by the controlling MCPTT function as the result of an action in subclause 11.1.1.4.4.

The controlling MCPTT function:

1)	shall generate a SIP re-INVITE request as specified in subclause 6.3.3.1.9;

2)	if the received SIP re-INVITE request contained an application/vnd.3gpp.mcptt-info+xml MIME body, shall copy the application/vnd.3gpp.mcptt-info+xml MIME body to the outgoing re-INVITE request.

3)	if the received SIP re-INVITE request contains an authorised request for an MCPTT emergency private call cancellation as determined by subclause 6.3.3.1.13.4:

a)	shall set the <emergency-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body to a value of "false";

b)	if the received SIP INVITE request contains an alert indication set to a value of "false" and this is an authorised request for an MCPTT emergency alert cancellation meeting the conditions specified in subclause 6.3.3.1.13.3:

i)	shall set the <alert-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body to a value of "false"; and

ii)	if the received SIP request contains an <originated-by> element in the application/vnd.3gpp.mcptt-info+xml MIME body, copy the contents of the received <originated-by> element to an <originated-by> element in the application/vnd.3gpp.mcptt-info+xml MIME body in the outgoing SIP re-INVITE request;

c)	if the received SIP INVITE request contains an alert indication set to a value of "false" and is not an authorised request for an MCPTT emergency alert cancellation meeting the conditions specified in subclause 6.3.3.1.13.3, shall set the <alert-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body to a value of "true";

4)	shall include a Resource-Priority header field populated with the values for a normal MCPTT private call as specified in subclause 6.3.3.1.19, if the received SIP re-INVITE request contains an authorised request for an MCPTT emergency private call cancellation as determined in step 3 above; and

5)	shall send the SIP re-INVITE request towards the core network according to 3GPP TS 24.229 [4].

Upon receiving SIP 200 (OK) response for the SIP re-INVITE request the controlling MCPTT function:

1)	shall cache the contact received in the Contact header field.

### 11.1.2	Private call without floor control

#### 11.1.2.1	General

Subclause 11.1.2 specifies the MCPTT client procedures, participating MCPTT function procedures and controlling MCPTT function procedures for on-network private calls without floor control. The procedures in subclause 11.1.2 refer to the on-network private calls without floor control procedures in subclause 11.1.1 citing the differences.

#### 11.1.2.2	MCPTT client procedures

When the MCPTT user wants to make an on-demand private call without floor control, the MCPTT client shall follow the procedures in subclause 11.1.1.2.1.1 with the following exceptions:

1)	in step 11) of subclause 11.1.1.2.1.1, the MCPTT client shall not offer a media-level section for a media-floor control entity; and

2)	step 12) of subclause 11.1.1.2.1.1 shall be ignored.

When the MCPTT user wants to make a private call without floor control using a pre-established session, the MCPTT client shall follow the procedures in subclause 11.1.1.2.2.1 with the following exceptions:

1)	in step 5d) of subclause 11.1.1.2.2.1, if the MCPTT client includes an SDP offer in the request, it shall not include a media-level section for a media-floor control entity; and

2)	step 6) in the second numbered list of subclause 11.1.1.2.2.1 shall be ignored.

Upon receipt of an initial SIP INVITE request for the private call with an SDP offer not including a media-level section for a media-floor control entity, the MCPTT client shall consider it as the request for private call without floor control and shall follow the procedures as specified in subclause 11.1.1.2.1.2 for on-demand session and subclause 11.1.1.2.2.2 for pre-established session.

#### 11.1.2.3	Participating MCPTT function procedures

##### 11.1.2.3.1	Originating procedures

Upon receipt of a "SIP INVITE request for originating participating MCPTT function" or "SIP REFER request for a pre-established session" for the private call with SDP offer not including media-level section for media-floor control entity, the participating MCPTT function shall consider it as the request for the private call without floor control and shall follow the procedures as specified in subclause 11.1.1.3.1.1 for an on-demand session and shall follow the procedures as specified in subclause 11.1.1.3.1.2 for initiation using a pre-established session.

##### 11.1.2.3.2	Terminating procedures

Upon receipt of a "SIP INVITE request for terminating participating MCPTT function" for the private call with SDP offer not including media-level section for media-floor control entity, the participating MCPTT shall consider it as the request for the private call without floor control and shall follow the procedures as specified in subclause 11.1.1.3.2.

#### 11.1.2.4	Controlling MCPTT function procedures

##### 11.1.2.4.1	Originating procedures

The controlling MCPTT function shall follow the procedures as specified in subclause 11.1.1.4.1.

##### 11.1.2.4.2	Terminating procedures

Upon receiving of a "SIP INVITE request for controlling MCPTT function of a private call" with SDP offer not including media-level section for media-floor control entity, the controlling MCPTT function shall consider it as the request for the private call without floor control and shall follow the procedures as specified in subclause 11.1.1.4.2.

### 11.1.3	Ending the private call initiated by MCPTT client

#### 11.1.3.1	MCPTT client procedures

##### 11.1.3.1.1	On-demand private call

###### 11.1.3.1.1.1	Client originating procedures

Upon receiving a request from an MCPTT user to release an MCPTT private call session established using on-demand session signalling, the MCPTT client shall follow the procedures as specified in subclause 6.2.5.1.

###### 11.1.3.1.1.2	Client terminating procedures

Upon receiving a SIP BYE request for private call session, the MCPTT client shall follow the procedures as specified in subclause 6.2.6.

##### 11.1.3.1.2	Private call using pre-established session

###### 11.1.3.1.2.1	Client originating procedures

Upon receiving a request from an MCPTT user to release an MCPTT private call within a pre-established session, the MCPTT client shall follow the procedures as specified in subclause 6.2.5.2.

###### 11.1.3.1.2.2	Client terminating procedures

The MCPTT client shall follow the procedures for terminating of request for MCPTT private call release as specified in subclause 6.2.6.

#### 11.1.3.2	Participating MCPTT function procedures

##### 11.1.3.2.1	Originating procedures

###### 11.1.3.2.1.1	Receipt of SIP BYE request for on-demand private call

Upon receiving from the MCPTT client a SIP BYE request the participating MCPTT function shall follow the procedures as specified in subclause 6.3.2.1.6.

###### 11.1.3.2.1.2	Receipt of REFER "BYE" request for private call using pre-established session

Upon receiving from the MCPTT client a SIP REFER request when using a pre-established session with the "method" SIP URI parameter set to value "BYE" in the URI in the Refer-To header field the participating MCPTT function shall follow the procedures as specified in subclause 6.3.2.1.7.

##### 11.1.3.2.2	Terminating procedures

###### 11.1.3.2.2.1	Receipt of SIP BYE request for private call on-demand

Upon receiving a SIP BYE request from the controlling MCPTT function, the participating MCPTT function shall follow the procedures as specified in subclause 6.3.2.2.8.1.

###### 11.1.3.2.2.2	Receipt of SIP BYE request when ongoing pre-established session

Upon receiving a SIP BYE request from the controlling MCPTT function and if the MCPTT session id refers to an MCPTT user that has a pre-established session with the participating MCPTT function, the participating MCPTT function:

1)	shall interact with the media plane as specified in subclause 9.3 in 3GPP TS 24.380 [5] for disconnecting the media plane resources towards the controlling MCPTT function;

2)	shall send a SIP 200 (OK) response to the controlling MCPTT function;

3)	shall interact with the media plane as specified in subclause 9.3 in 3GPP TS 24.380 [5] for disconnecting media plane resources towards the MCPTT client from the media plane resources towards the controlling MCPTT function; and

4)	shall maintain the pre-established session towards the MCPTT client.

#### 11.1.3.3	Controlling MCPTT function procedures

##### 11.1.3.3.1	Terminating procedures

Upon receiving a SIP BYE request the controlling MCPTT function shall follow the procedures as specified in subclause 6.3.3.2.4.

### 11.1.4	Ending the private call initiated by the MCPTT server

#### 11.1.4.1	General

This subclause describes the procedures of each functional entity for ending the private call initiated by the MCPTT server.

NOTE:	For private call without floor control, ending the private call is initiated only by the MCPTT client.

#### 11.1.4.2	MCPTT client procedures

Upon receiving a SIP BYE request for private call session, the MCPTT client shall follow the procedures as specified in subclause 6.2.6.

#### 11.1.4.3	Participating MCPTT function procedures

##### 11.1.4.3.1	Originating procedures

When the MCPTT session for private call needs to be released as specified in subclause 6.3.8.2, the participating MCPTT function shall follow the procedures in subclause 6.3.3.1.5.

##### 11.1.4.3.2	Terminating procedures

###### 11.1.4.3.2.1	Receipt of SIP BYE request for private call on-demand

Upon receiving a SIP BYE request from the controlling MCPTT function, the participating MCPTT function shall follow the procedures as specified in subclause 6.3.2.2.8.1.

###### 11.1.4.3.2.2	Receipt of SIP BYE request when ongoing pre-established session

Upon receiving a SIP BYE request from the controlling MCPTT function and if the MCPTT session id refers to an MCPTT user that has a pre-established session with the participating MCPTT function, the participating MCPTT function shall follow the procedures in subclause 11.1.3.2.2.2.

#### 11.1.4.4	Controlling MCPTT function procedures

When the MCPTT session for private call needs to be released as specified in subclause 6.3.8.2, the controlling MCPTT function shall follow the procedures in subclause 6.3.3.1.5.

## 11.2	Off-network private call

### 11.2.1	General

#### 11.2.1.1	Common procedures

##### 11.2.1.1.1	Sending/Receiving a message

In order to participate in a private call, the MCPTT client:

1)	shall send the MONP message as a UDP message to the local IP address of the MCPTT user, on UDP port TBD, with an IP time-to-live set to 255; and

Editor's note [CT1#95, C1-160392]: Port number for the message is FFS.

2)	shall treat UDP messages received on the port TBD as received MONP messages.

NOTE:	An MCPTT client that supports IPv6 shall listen to the IPv6 addresses.

##### 11.2.1.1.2	Session description

For an off-network MCPTT session, only MCPTT speech is used.

One off-network MCPTT session includes one media-floor control entity.

The MCPTT client shall generate an SDP body for a group call in accordance with rules and procedures of IETF RFC 4566 [12] and IETF RFC 3264 [44].

The MCPTT client:

1)	shall include in the session-level section:

a)	the "o=" field with the <username> portion set to a dash;

b)	the "s=" field with the <session name> portion set to a dash; and

c)	the "c=" field with the <nettype> portion set to "IN", the <addrtype> portion set to the IP version of the unicast IP address of the MCPTT client and the <connection-address> portion set to the unicast IP address of the MCPTT client;

2)	shall include the media-level section for MCPTT speech consisting of:

a)	the "m=" field with the <media> portion set to "audio", the <port> portion set to a port number for MCPTT speech of the MCPTT group, the <proto> field set to "RTP/AVP" and <fmt> portion set indicating RTP payload type numbers;

b)	the "i=" field with the <session description> portion set to "speech";

c)	the "a=fmtp:" attribute(s), the "a=rtpmap:" attribute(s) or both, indicating the codec(s) and media parameters of the MCPTT speech; and

d)	the "a=rtcp:" attribute indicating port number to be used for RTCP at the MCPTT client selected according to the rules and procedures of IETF RFC 3605 [13], if the media steam uses other than the default IP address;

3)	shall include the media-level section for media-floor control entity consisting of:

a)	an "m=" line, with the <media> portion set to "application", the <port> portion set to a port number for media-floor control entity of the MCPTT group, the <proto> field set to "udp" and <fmt> portion set to "MCPTT"; and

b)	the "a=fmtp:MCPTT" attribute indicating the parameters of the media-floor control entity as specified 3GPP TS 24.380 [5]; and

4)	shall include the MIKEY-SAKKE I_MESSAGE, if generated by the MCPTT client, in an "a=key-mgmt" attribute as a "mikey" attribute value in the SDP offer as specified in IETF RFC 4567 [47].

### 11.2.2	Basic call control

#### 11.2.2.1	General

The maximum number of simultaneous off-network private calls is limited by the value of "/<x>/Common/PrivateCall/MaxCallN10" leaf node present in the UE configuration as specified in 3GPP TS 24.383 [45].

#### 11.2.2.2	Private call control state machine

The figure 11.2.2.2-1 gives an overview of the main states and transitions on the UE for private call control.

Each private call control state machine is per MCPTT user ID.

Figure 11.2.2.2-1: Private Call Control state machine

#### 11.2.2.3	Call Control states

##### 11.2.2.3.1	P0: start-stop

In this state, no private call control entity exists.

##### 11.2.2.3.2	P1: ignoring same call id

This state exists for UE, when the UE is not part of an ongoing private call.

##### 11.2.2.3.3	P2: waiting for call response

This state exists for UE, when the UE has sent a PRIVATE CALL SETUP REQUEST message and is waiting for a response, PRIVATE CALL ACCEPT or PRIVATE CALL REJECT message.

##### 11.2.2.3.4	P3: waiting for release response

This state exists for UE, when the UE has sent a PRIVATE CALL RELEASE message and is waiting for a PRIVATE CALL RELEASE ACK message.

##### 11.2.2.3.5	P4: part of ongoing call

This state exists for UE, when the UE is part of an ongoing private call.

##### 11.2.2.3.6	P5: pending

This state exists for UE, when the UE has presented a notification to the user for the received PRIVATE CALL SETUP REQUEST message and is waiting for a user indication.

#### 11.2.2.4	Procedures

##### 11.2.2.4.1	General

##### 11.2.2.4.2	Private call setup

###### 11.2.2.4.2.1	Initiating a private call

When in the "P0: start-stop" state or "P1: ignoring same call id", upon an indication from MCPTT User to initiate a private call and the value of "/<x>/<x>/Common/PrivateCall/Authorised" leaf node present in the user profile as specified in 3GPP TS 24.383 [45] is set to "true", the MCPTT client:

1)	shall generate and store the call identifier as a random number uniformly distributed between (0, 65536);

2)	shall store own MCPTT user ID as caller ID;

3)	shall store MCPTT user ID of the callee as callee ID;

4)	shall store "AUTOMATIC COMMENCEMENT MODE" as commencement mode, if requested and the value of "/<x>/<x>/Common/PrivateCall/AutoCommence" leaf node present in the user profile as specified in 3GPP TS 24.383 [45] is set to "true". Otherwise if the value of "/<x>/<x>/Common/PrivateCall/ManualCommence" leaf node present in the user profile as specified in 3GPP TS 24.383 [45] is set to "true", store "MANUAL COMMENCEMENT MODE" as commencement mode;

5)	shall create a call type control state machine as described in subclause 11.2.3.2;

6)	if an end-to-end security context needs to be established then:

a)	shall use keying material provided by the key management server to generate a PCK as described in 3GPP TS 33.179 [46];

b)	shall use the PCK to generate a PCK-ID with the four most significant bits set to "0011" to indicate that the purpose of the PCK is to protect private call communications and with the remaining twenty eight bits being randomly generated as described in 3GPP TS 33.179 [46];

c)	shall encrypt the PCK to a UID associated to the MCPTT client using the MCPTT ID of the invited user and a time related parameter as described in 3GPP TS 33.179 [46];

d)	shall generate a MIKEY-SAKKE I_MESSAGE using the encapsulated PCK and PCK-ID as specified in 3GPP TS 33.179 [46];

e)	shall add the MCPTT ID of the originating MCPTT to the initiator field (IDRi) of the I_MESSAGE as described in 3GPP TS 33.179 [46];

f)	shall sign the MIKEY-SAKKE I_MESSAGE using the originating MCPTT user's signing key provided in the keying material together with a time related parameter, and add this to the MIKEY-SAKKE payload, as described in 3GPP TS 33.179 [46] and;

g)	shall store the MIKEY-SAKKE I_MESSAGE for later inclusion in an SDP body;

7)	may store current user location as user location;

8)	shall generate and store offer SDP, as defined in subclause 11.2.1.1.2;

9)	shall generate a PRIVATE CALL SETUP REQUEST message as specified in subclause 15.1.5. In the PRIVATE CALL SETUP REQUEST message, the MCPTT client:

a)	shall set the Call identifier IE with the stored call identifier;

b)	shall set the MCPTT user ID of the caller IE with the stored caller ID;

c)	shall set the MCPTT user ID of the callee IE with the stored callee ID;

d)	shall set the Commencement mode IE with the stored commencement mode;

e)	shall set the Call type IE with the stored current call type associated with the call type control state machine;

f)	shall set the SDP offer IE with the stored offer SDP; and

g)	may set the User location IE with the stored user location if the stored current call type associated with the call type control state machine is "EMERGENCY PRIVATE CALL".

10)	shall send the PRIVATE CALL SETUP REQUEST message towards other MCPTT client according to rules and procedures as specified in subclause 11.2.1.1.1;

11)	shall initialize the counter CFP1 (private call request retransmission) with the value set to 1;

12)	shall start timer TFP1 (private call request retransmission); and

13)	shall enter the "P2: waiting for call response" state.

###### 11.2.2.4.2.2	Private call setup request retransmission

When in the "P2: waiting for call response" state, upon expiry of timer TFP1 (private call request retransmission), the MCPTT client:

1)	may update the stored user location with current user location;

2)	shall increment the value of counter CFP1 (private call request retransmission) by 1;

3)	shall generate a PRIVATE CALL SETUP REQUEST message as specified in subclause 15.1.5. In the PRIVATE CALL SETUP REQUEST message, the MCPTT client:

a)	shall set the Call identifier IE with the stored call identifier;

b)	shall set the MCPTT user ID of the caller IE with the stored caller ID;

c)	shall set the MCPTT user ID of the callee IE with the stored callee ID;

d)	shall set the Commencement mode IE with the stored commencement mode;

e)	shall set the Call type IE with the stored current call type associated with the call type control state machine;

f)	shall set the SDP offer IE with the stored offer SDP; and

g)	may set the User location IE with stored user location if the stored current call type is "EMERGENCY PRIVATE CALL" associated with the call type control state machine.

4)	shall send the PRIVATE CALL SETUP REQUEST message towards other MCPTT client according to rules and procedures as specified in subclause 11.2.1.1.1;

5)	shall start timer TFP1 (private call request retransmission); and

6)	shall remain in the "P2: waiting for call response" state.

###### 11.2.2.4.2.3	Ringing notification to the user

When in the "P2: waiting for call response" state, upon receiving a PRIVATE CALL RINGING message, the MCPTT client:

1)	shall remain in the "P2: waiting for call response" state.

###### 11.2.2.4.2.4	No response to private call setup request with automatic commencement mode

In the "P2: waiting for call response" state, when timer TFP1 (private call request retransmission) expires and the value of the counter CFP1 (private call request retransmission) is equal to the upper limit and the stored commencement mode is "AUTOMATIC COMMENCEMENT MODE", the MCPTT client:

1)	shall start timer TFP7 (waiting for any message with same call identifier); and

2)	shall enter the "P1: ignoring same call id" state.

###### 11.2.2.4.2.5	No response to private call setup request with manual commencement mode

When in the "P2: waiting for call response" state when timer TFP1 (private call request retransmission) expires and the value of the counter CFP1 (private call request retransmission) is equal to the upper limit and the stored commencement mode is "MANUAL COMMENCEMENT MODE", the MCPTT client:

1)	shall start timer TFP2 (waiting for call response message); and

2)	shall remain in the "P2: waiting for call response" state.

###### 11.2.2.4.2.6	No response to private call setup request after waiting for user acknowledgement

When in the "P2: waiting for call response" state, upon expiry of timer TFP2 (waiting for call response message), the MCPTT client:

1)	shall start timer TFP7 (waiting for any message with same call identifier);

2)	shall release the call control state machine; and

3)	shall enter the "P1: ignoring same call id" state.

###### 11.2.2.4.2.7	Private call setup request rejected

When in the "P2: waiting for call response" state, upon receiving a PRIVATE CALL REJECT message in response to PRIVATE CALL SETUP REQUEST message with Call identifier IE same as the stored call identifier, the MCPTT client:

1)	shall stop timer TFP1 (call setup retransmission), if running;

2)	shall stop timer TFP2 (waiting for call response message), if running;

3)	shall start timer TFP7 (waiting for any message with same call identifier);

4)	shall release the call control state machine; and

5)	shall enter the "P1: ignoring same call id" state.

###### 11.2.2.4.2.8	Private call setup request accepted

When in the "P2: waiting for call response" state, upon receiving a PRIVATE CALL ACCEPT message response to PRIVATE CALL SETUP REQUEST message with the same call identifier, the MCPTT client:

1)	shall store the SDP answer IE received in the PRIVATE CALL ACCEPT message as answer SDP;

2)	shall generate a PRIVATE CALL ACCEPT ACK message as specified in subclause 15.1.11:

a)	shall set the Call identifier IE to the stored call identifier;

b)	shall set the MCPTT user ID of the caller IE with the stored caller ID; and

c)	shall set the MCPTT user ID of the callee IE with the stored callee ID.

3)	shall send the PRIVATE CALL ACCEPT ACK message in response to the request message according to rules and procedures as specified in subclause 11.2.1.1.1;

4)	shall stop timer TFP1 (call setup retransmission), if running;

5)	shall stop timer TFP2 (waiting for call response message), if running;

6)	shall establish a media session based on the SDP body of the stored answer SDP;

7)	shall start floor control as terminating floor participant as specified in subclause 7.2 in 3GPP TS 24.380 [5];

8)	shall start timer TFP5 (max duration); and

9)	shall enter the "P4: part of ongoing call" state.

###### 11.2.2.4.2.9	User cancels the private call setup request

When in the "P2: waiting for call response" state, upon an indication from MCPTT User to cancel the private call request, the MCPTT client:

1)	shall generate a PRIVATE CALL RELEASE message as specified in subclause 15.1.9;

a)	shall set the Call identifier IE to the stored call identifier;

b)	shall set the MCPTT user ID of the caller IE with the stored caller ID; and

c)	shall set the MCPTT user ID of the callee IE with the stored callee ID;

2)	shall send the PRIVATE CALL RELEASE message in response to the request message according to rules and procedures as specified in subclause 11.2.1.1.1;

3)	shall start timer TFP3 (private call release retransmission); and

4)	shall enter the "P3: waiting for release response" state.

##### 11.2.2.4.3	Private call setup in automatic commencement mode

###### 11.2.2.4.3.1	Unable to establish media

When in the "P0: start-stop" or "P1: ignoring same call id" state, upon receiving a PRIVATE CALL SETUP REQUEST message with Call identifier IE different than stored call identifier and media session declared in SDP body of PRIVATE CALL SETUP REQUEST message cannot be established, the MCPTT client:

1)	shall store the Call identifier IE in the received message as call identifier;

2)	shall store the MCPTT user ID of the caller IE in the received PRIVATE CALL SETUP message as caller ID;

3)	shall store own MCPTT user ID as callee ID;

4)	shall generate a PRIVATE CALL REJECT message as specified in subclause 15.1.8. In the PRIVATE CALL REJECT message, the MCPTT client:

a)	shall set the Call identifier IE to the cached call identifier;

b)	shall set the MCPTT user ID of the caller IE with the stored caller ID;

c)	shall set the MCPTT user ID of the callee IE with stored callee ID; and

d)	shall set the Reason IE as "FAILED", if requested to restrict notification of call failure and the value of "/<x>/<x>/Common/PrivateCall/FailRestrict" leaf node present in the user profile as specified in 3GPP TS 24.383 [45] is set to "true". Otherwise, shall set the Reason IE as "MEDIA FAILURE".

5)	shall send the PRIVATE CALL REJECT message in response to the request message according to rules and procedures as specified in subclause 11.2.1.1.1;

6)	shall start timer TFP7 (waiting for any message with same call identifier); and

7)	shall enter the "P1: ignoring same call id" state if current state is the "P0: start-stop" state.

###### 11.2.2.4.3.2	Responding to private call setup request when not participating in the ongoing call

When in the "P0: start-stop" or "P1: ignoring same call id" state, upon receiving a PRIVATE CALL SETUP REQUEST message with Commencement mode IE set to "AUTOMATIC COMMENCEMENT MODE" and Call identifier IE different than stored call identifier and media session declared in SDP body of PRIVATE CALL SETUP REQUEST message can be established, the MCPTT client:

1)	shall store the Call identifier IE in the received message as call identifier;

2)	shall create the call type control state machine as described in subclause 11.2.3.2;

3)	shall store the MCPTT user ID of the caller IE in the received PRIVATE CALL SETUP REQUEST message as caller ID;

4)	shall store own MCPTT user ID as callee ID;

5)	if the SDP offer contains an "a=key-mgmt" attribute field with a "mikey" attribute value containing a MIKEY-SAKKE I_MESSAGE:

a)	shall extract the MCPTT ID of the originating MCPTT user from the initiator field (IDRi) of the I_MESSAGE as described in 3GPP TS 33.179 [46];

b)	shall convert the MCPTT ID to a UID as described in 3GPP TS 33.179 [46];

c)	shall use the UID to validate the signature of the MIKEY-SAKKE I_MESSAGE as described in 3GPP TS 33.179 [46];

d)	if the validation of the signature failed, shall generate a PRIVATE CALL REJECT message as specified in subclause 15.1.8. In the PRIVATE CALL REJECT message, the MCPTT client:

i)	shall set the call identifier IE to the stored call identifier;

ii)	shall set the MCPTT user ID of the caller IE with the stored caller ID;

iii)	shall set the MCPTT user ID of the callee IE with the stored callee ID;

iv)	shall set the Reason IE as "FAILED", if requested to restrict notification of call failure and the value of "/<x>/<x>/Common/PrivateCall/FailRestrict" leaf node present in the user profile as specified in 3GPP TS 24.383 [45] is set to "true". Otherwise, shall set the reason IE as "E2E SECURITY CONTEXT FAILURE";

v)	shall send the PRIVATE CALL REJECT message in response to the request message according to rules and procedures as specified in subclause 11.2.1.1.1; and

vi)	shall remain in the current state;

e)	if the validation of the signature was successful:

i)	shall extract and decrypt the encapsulated PCK using the terminating user's (KMS provisioned) UID key as described in 3GPP TS 33.179 [46];

ii)	shall extract the PCK-ID, from the payload as specified in 3GPP TS 33.179 [46];

iii)	shall generate and store answer SDP based on received SDP offer IE in PRIVATE CALL SETUP REQUEST message, as defined in subclause 11.2.1.1.2;

iv)	shall generate a PRIVATE CALL ACCEPT message as specified in subclause 15.1.7. In the PRIVATE CALL ACCEPT message, the MCPTT client:

A)	shall set the Call identifier IE to the stored call identifier; and

B)	shall set the MCPTT user ID of the caller IE with stored caller ID.

C)	shall set the MCPTT user ID of the callee IE with stored callee ID; and

D)	shall set the SDP answer IE with the stored answer SDP;

v)	shall send PRIVATE CALL ACCEPT message in response to the request message according to rules and procedures as specified in subclause 11.2.1.1.1;

vi)	shall establish a media session based on the SDP body of the stored answer SDP;

vii)	shall initialize the counter CFP4 with value set to 1;

viii)	shall start timer TFP4 (private call accept retransmission); and

ix)	shall enter the "P5: pending" state; and

NOTE:	With the PCK successfully shared between the originating MCPTT client and the terminating MCPTT client, both clients are able to use SRTP/SRTCP to create an end-to-end secure session.

6)	if the SDP offer does not contain an "a=key-mgmt" attribute, the MCPTT client:

a)	shall generate and store answer SDP based on received SDP offer IE in PRIVATE CALL SETUP REQUEST message, as defined in subclause 11.2.1.1.2;

b)	shall generate a PRIVATE CALL ACCEPT message as specified in subclause 15.1.7:

i)	shall set the Call identifier IE to the stored call identifier;

ii)	shall set the MCPTT user ID of the caller IE with stored caller ID.

iii)	shall set the MCPTT user ID of the callee IE with stored callee ID; and

iv)	shall set the SDP answer IE with the stored answer SDP;

c)	shall send PRIVATE CALL ACCEPT message in response to the request message according to rules and procedures as specified in subclause 11.2.1.1.1;

d)	shall establish a media session based on the SDP body of the stored answer SDP;

e)	shall initialize the counter CFP4 with value set to 1;

f)	shall start timer TFP4 (private call accept retransmission); and

g)	shall enter the "P5: pending" state.

###### 11.2.2.4.3.3	Private call accept retransmission

When in the "P5: pending" state, upon expiry of timer TFP4 (private call accept retransmission), the MCPTT client:

1)	shall generate a PRIVATE CALL ACCEPT message as specified in subclause 15.1.7. In the PRIVATE CALL ACCEPT message, the MCPTT client:

a)	shall set the Call identifier IE to the stored call identifier;

b)	shall set the MCPTT user ID of the caller IE with the stored caller ID;

c)	shall set the MCPTT user ID of the callee IE with the stored callee ID; and

d)	shall set the SDP answer IE with the stored answer SDP;

2)	shall send PRIVATE CALL ACCEPT message in response to the request message according to rules and procedures as specified in subclause 11.2.1.1.1;

3)	shall increment the value of the counter CFP4 (private call accept retransmission) by 1;

4)	shall start timer TFP4 (private call accept retransmission); and

5)	shall remain in the "P5: pending" state.

###### 11.2.2.4.3.4	Establishing the call

When in the "P5: pending" state, upon receiving a PRIVATE CALL ACCEPT ACK message or RTP media from originating user, the MCPTT client:

1)	shall stop timer TFP4(private call accept retransmission);

2)	shall start floor control as terminating MCPTT client as specified in subclause 7.2 in 3GPP TS 24.380 [5];

3)	shall start timer TFP5 (max duration); and

4)	shall enter the "P4: part of ongoing call" state.

###### 11.2.2.4.3.5	Call failure

In the "P5: pending" state, when timer TFP4 (private call accept retransmission) expires and the value of the counter CFP4 (private call accept retransmission) is equal to the upper limit, the MCPTT client:

1)	shall start timer TFP7 (waiting for any message with same call identifier);

2)	shall release the call type control state machine; and

3)	shall enter the "P1: ignoring same call id" state.

##### 11.2.2.4.4	Private call setup in manual commencement mode

###### 11.2.2.4.4.1	Incoming private call

When in the "P0: start-stop" or "P1: ignoring same call id" state, upon receiving a PRIVATE CALL SETUP REQUEST message with Commencement mode IE set to "MANUAL COMMENCEMENT MODE" and Call identifier IE different from stored call identifier, the MCPTT client:

1)	shall store the Call identifier IE in the received message as call identifier;

2)	shall create the call type control state machine as described in subclause 11.2.3.2;

3)	shall store the MCPTT user ID of the caller IE as received in the PRIVATE CALL SETUP REQUEST as caller ID;

4)	shall store own MCPTT user ID as callee ID;

5)	shall generate a PRIVATE CALL RINGING message as specified in subclause 15.1.6;

a)	shall set the Call identifier IE to the stored call identifier;

b)	shall set the MCPTT user ID of the caller IE with the stored caller ID; and

c)	shall set the MCPTT user ID of the callee IE with the stored callee ID;

6)	shall send PRIVATE CALL RINGING message in response to the request message according to rules and procedures as specified in subclause 11.2.1.1.1;

7)	shall start timer TFP2 (waiting for call response message); and

8)	shall enter the "P5: pending" state.

###### 11.2.2.4.4.2	No response from the user

When in the "P5: pending" state, upon expiry of timer TFP2 (waiting for call response message), the MCPTT client:

1)	shall generate a PRIVATE CALL REJECT message as specified in subclause 15.1.8:

a)	shall set the Call identifier IE to the stored call identifier;

b)	shall set the MCPTT user ID of the caller IE with the stored caller ID;

c)	shall set the MCPTT user ID of the callee IE with the stored callee ID; and

d)	shall set the Reason IE as "FAILED".

2)	shall send the PRIVATE CALL REJECT message in response to the request message according to rules and procedures as specified in subclause 11.2.1.1.1;

3)	shall start timer TFP7 (waiting for any message with same call identifier);

4)	shall release the call type control state machine; and

5)	shall enter the "P1: ignoring same call id" state.

###### 11.2.2.4.4.3	User accepts the private call setup request

When in the "P5: pending" state, upon an indication from MCPTT User to accept the incoming private call, the MCPTT client:

1)	if the SDP offer contains an "a=key-mgmt" attribute field with a "mikey" attribute value containing a MIKEY-SAKKE I_MESSAGE:

a)	shall extract the MCPTT ID of the originating MCPTT user from the initiator field (IDRi) of the I_MESSAGE as described in 3GPP TS 33.179 [46];

b)	shall convert the MCPTT ID to a UID as described in 3GPP TS 33.179 [46];

c)	shall use the UID to validate the signature of the MIKEY-SAKKE I_MESSAGE as described in 3GPP TS 33.179 [46];

d)	if the validation of the signature failed, shall generate a PRIVATE CALL REJECT message as specified in subclause 15.1.8. In the PRIVATE CALL REJECT message, the MCPTT client:

i)	shall set the call identifier IE to the stored call identifier;

ii)	shall set the MCPTT user ID of the caller IE with the stored caller ID;

iii)	shall set the MCPTT user ID of the callee IE with the stored callee ID;

iv)	shall set the Reason IE as "FAILED", if requested to restrict notification of call failure and the value of "/<x>/<x>/Common/PrivateCall/FailRestrict" leaf node present in the user profile as specified in 3GPP TS 24.383 [45] is set to "true". Otherwise, shall set the reason IE as "E2E SECURITY CONTEXT FAILURE";

v)	shall send the PRIVATE CALL REJECT message in response to the request message according to rules and procedures as specified in subclause 11.2.1.1.1; and

vi)	shall enter the "P1: ignoring same call id" state;

e)	if the validation of the signature was successful:

i)	shall extract and decrypt the encapsulated PCK using the terminating user's (KMS provisioned) UID key as described in 3GPP TS 33.179 [46];

ii)	shall extract the PCK-ID, from the payload as specified in 3GPP TS 33.179 [46];

iii)	shall generate and store answer SDP based on received SDP offer IE in PRIVATE CALL SETUP REQUEST message, as defined in subclause 11.2.1.1.2;

iv)	shall generate a PRIVATE CALL ACCEPT message as specified in subclause 15.1.7. In the PRIVATE CALL ACCEPT message, the MCPTT client:

A)	shall set the Call identifier IE to the stored call identifier;

B)	shall set the MCPTT user ID of the caller IE with the stored caller ID;

C)	shall set the MCPTT user ID of the callee IE with the stored callee ID; and

D)	shall set the SDP answer IE with the stored answer SDP;

v)	shall send the PRIVATE CALL ACCEPT message in response to the request message according to rules and procedures as specified in subclause 11.2.1.1.1;

vi)	shall establish a media session based on the SDP body of the private call;

vii)	shall stop timer TFP2 (waiting for call response message);

viii)	shall initialize the counter CFP4 with value set to 1;

ix)	shall start timer TFP4 (private call accept retransmission); and

x)	shall remain in the "P5: pending" state; and

NOTE:	With the PCK successfully shared between the originating MCPTT client and the terminating MCPTT client, both clients are able to use SRTP/SRTCP to create an end-to-end secure session.

2)	if the SDP offer does not contain an "a=key-mgmt" attribute, the MCPTT client:

a)	shall generate and store answer SDP based on received SDP offer IE in PRIVATE CALL SETUP REQUEST message, as defined in subclause 11.2.1.1.2;

b)	shall generate a PRIVATE CALL ACCEPT message as specified in subclause 15.1.7. In the PRIVATE CALL ACCEPT message, the MCPTT client:

i)	shall set the Call identifier IE to the stored call identifier;

ii)	shall set the MCPTT user ID of the caller IE with the stored caller ID;

iii)	shall set the MCPTT user ID of the callee IE with the stored callee ID; and

iv)	shall set the SDP answer IE with the stored answer SDP;

c)	shall send the PRIVATE CALL ACCEPT message in response to the request message according to rules and procedures as specified in subclause 11.2.1.1.1;

d)	shall establish a media session based on the SDP body of the private call;

e)	shall stop timer TFP2 (waiting for call response message);

f)	shall initialize the counter CFP4 with value set to 1;

g)	shall start timer TFP4 (private call accept retransmission); and

h)	shall remain in the "P5: pending" state.

###### 11.2.2.4.4.4	Private call accept retransmission

When in the "P5: pending" state, upon expiry of timer TFP4 (private call accept retransmission), the MCPTT client:

1)	shall generate a PRIVATE CALL ACCEPT message as specified in subclause 15.1.7. In the PRIVATE CALL ACCEPT message, the MCPTT client:

a)	shall set the Call identifier IE to the stored call identifier;

b)	shall set the MCPTT user ID of the caller IE the stored caller ID;

c)	shall set the MCPTT user ID of the callee IE with the stored callee ID; and

d)	shall set the SDP answer IE with the stored answer SDP;

2)	shall send PRIVATE CALL ACCEPT message in response to the request message according to rules and procedures as specified in subclause 11.2.1.1.1;

3)	shall increment the value of the (counter CFP4 private call accept retransmission) by 1;

4)	shall start timer TFP4 (private call accept retransmission); and

5)	shall remain in the "P5: pending" state.

###### 11.2.2.4.4.5	Establishing the call

When in the "P5: pending" state, upon receiving a PRIVATE CALL ACCEPT ACK message or RTP media from originating user, the MCPTT client:

1)	shall stop timer TFP4 (private call accept retransmission);

2)	shall start floor control as terminating MCPTT client as specified in subclause 7.2 in 3GPP TS 24.380 [5];

3)	shall start timer TFP5 (max duration); and

4)	shall enter the "P4: part of ongoing call" state.

###### 11.2.2.4.4.6	Call failure

In the "P5: pending" state, when timer TFP4 (private call accept retransmission) expires and the value of the counter CFP4 (private call accept retransmission) is equal to the upper limit, the MCPTT client:

1)	shall start timer TFP7 (waiting for any message with same call identifier);

2)	shall release the call type control state machine; and

3)	shall enter the "P1: ignoring same call id" state.

###### 11.2.2.4.4.7	User rejects the private call setup request

When in the "P5: pending" state, upon an indication from MCPTT User to reject the incoming private call, the MCPTT client:

1)	shall generate a PRIVATE CALL REJECT message as specified in subclause 15.1.8:

a)	shall set the Call identifier IE to the stored call identifier;

b)	shall set the MCPTT user ID of the caller IE with the stored caller ID;

c)	shall set the MCPTT user ID of the callee IE with stored callee ID; and

d)	shall set the Reason IE as "FAILED", if requested to restrict notification of call failure and the value of "/<x>/<x>/Common/PrivateCall/FailRestrict" leaf node present in the user profile as specified in 3GPP TS 24.383 [45] is set to "true". Otherwise, shall set the Reason IE as "REJECT";

2)	shall send the PRIVATE CALL REJECT message in response to the request message according to rules and procedures as specified in subclause 11.2.1.1.1;

3)	shall start timer TFP7 (waiting for any message with same call identifier);

4)	shall release the call type control state machine; and

5)	shall enter the "P1: ignoring same call id" state.

###### 11.2.2.4.4.8	Caller cancels the private call setup request before call establishment

When in the "P5: pending" state or "P1: ignoring same call id" state, upon receiving a PRIVATE CALL RELEASE message, the MCPTT client:

1)	shall generate a PRIVATE CALL RELEASE ACK message as specified in subclause 15.1.10. In the PRIVATE CALL RELEASE ACK message, the MCPTT client:

a)	shall set the Call identifier IE to the stored call identifier;

b)	shall set the MCPTT user ID of the caller IE with the stored caller ID; and.

c)	shall set the MCPTT user ID of the callee IE with the stored callee ID.

2)	shall send the PRIVATE CALL RELEASE ACK message in response to the request message according to rules and procedures as specified in subclause 11.2.1.1.1;

3)	shall start timer TFP7 (waiting for any message with same call identifier);

4)	shall stop timer TFP4 (private call accept retransmission) if running;

5)	shall release the call type control state machine, if the current state is "P5: pending" state; and

6)	shall enter the "P1: ignoring same call id" state, if the current state is "P5: pending" state.

##### 11.2.2.4.5	Private call release

###### 11.2.2.4.5.1	Releasing a private call

When in the "P4: part of ongoing call" state, upon an indication from MCPTT User to release a private call, the MCPTT client:

1)	shall generate a PRIVATE CALL RELEASE message as specified in subclause 15.1.9. In the PRIVATE CALL RELEASE message, the MCPTT client:

a)	shall set the Call identifier IE to the stored call identifier;

b)	shall set the MCPTT user ID of the caller IE with stored caller ID; and

c)	shall set the MCPTT user ID of the callee IE with stored callee ID.

2)	shall send the PRIVATE CALL RELEASE message in response to the request message according to rules and procedures as specified in subclause 11.2.1.1.1;

3)	shall initialize the counter CFP3 (private call release retransmission) with the value set to 1;

4)	shall start timer TFP3 (private call release retransmission); and

5)	shall enter the "P3: waiting for release response" state.

###### 11.2.2.4.5.2	Private call release retransmission

When in the "P3: waiting for release response" state, upon expiry of timer TFP3 (private call release retransmission), the MCPTT client:

1)	shall generate a PRIVATE CALL RELEASE message as specified in subclause 15.1.9. In the PRIVATE CALL RELEASE message, the MCPTT client:

a)	shall set the Call identifier IE to the stored call identifier;

b)	shall set the MCPTT user ID of the caller IE with stored caller ID; and

c)	shall set the MCPTT user ID of the callee IE with the stored callee ID.

2)	shall send the PRIVATE CALL RELEASE message in response to the request message according to rules and procedures as specified in subclause 11.2.1.1.1;

3)	shall increment the value of timer CFP3 by 1;

4)	shall start timer TFP3 (private call release retransmission); and

5)	shall remain in the "P3: waiting for release response" state.

###### 11.2.2.4.5.3	No response to private call release

In the "P3: waiting for release response" state, when timer TFP3 (private call request retransmission) expires and the value of the counter CFP3 (private call release retransmission) is equal to the upper limit, the MCPTT client:

1)	shall terminate the media session;

2)	shall start timer TFP7 (waiting for any message with same call identifier);

3)	shall release the call type control state machine; and

4)	shall enter the "P1: ignoring same call id" state.

###### 11.2.2.4.5.4	Acknowledging private call release after call establishment

When in the "P4: part of ongoing call" state, upon receiving a PRIVATE CALL RELEASE message, the MCPTT client:

1)	shall generate a PRIVATE CALL RELEASE ACK message as specified in subclause 15.1.10;

a)	shall set the Call identifier IE to the stored call identifier;

b)	shall set the MCPTT user ID of the caller IE the stored caller ID; and

c)	shall set the MCPTT user ID of the callee IE with the stored callee ID.

2)	shall send the PRIVATE CALL RELEASE ACK message in response to the request message according to rules and procedures as specified in subclause 11.2.1.1.1;

3)	shall terminate the media session for private call;

4)	shall start timer TFP7 (waiting for any message with same call identifier);

5)	shall release the call type control state machine; and

6)	shall enter the "P1: ignoring same call id" state.

###### 11.2.2.4.5.5	Private call release acknowledged

When in the "P3: waiting for release response" state, upon receiving a PRIVATE CALL RELEASE ACK to PRIVATE CALL RELEASE message, the MCPTT client:

1)	shall stop timer TFP3 (private call release retransmission), if running;

2)	shall terminate the media session;

3)	shall start timer TFP7 (waiting for any message with same call identifier);

4)	shall release the call type control state machine; and

5)	shall enter the "P1: ignoring same call id" state.

###### 11.2.2.4.5.6	Max duration reached

When in the "P4: part of ongoing call" state, upon expiry of timer TFP5 (max duration), the MCPTT client:

1)	shall terminate the media session;

2)	shall start timer TFP7 (waiting for any message with same call identifier);

3)	shall release the call type control state machine; and

4)	shall enter the "P1: ignoring same call id" state.

###### 11.2.2.4.5.7	Stop ignoring same call id

When in the "P1: ignoring same call id" state, upon expiry of timer TFP7 (waiting for any message with same call identifier) the MCPTT client:

1)	shall clear the stored call identifier; and

2)	shall enter the "P0: start-stop" state.

###### 11.2.2.4.5.8	No response to emergency private call setup request

In the "P4: part of ongoing call" state, when timer TFP1 (private call request retransmission) expires and the value of the counter CFP1 (private call request retransmission) is equal to the upper limit, the MCPTT client:

1)	shall start timer TFP7 (waiting for any message with same call identifier);

2)	shall release the call type control state machine; and

3)	shall enter the "P1: ignoring same call id" state.

###### 11.2.2.4.5.9	No response to emergency private call cancel

In the "P4: part of ongoing call" state, when timer TFP6 (emergency private call cancel retransmission) expires and the value of the counter CFP6 (emergency private call cancel retransmission) is equal to the upper limit, the MCPTT client:

1)	shall start timer TFP7 (waiting for any message with same call identifier);

2)	shall release the call type control state machine; and

3)	shall enter the "P1: ignoring same call id" state.

##### 11.2.2.4.6	Error handling

###### 11.2.2.4.6.1	Unexpected MONP message received

Upon receiving a MONP message in a state where there is no handling specified for the MONP message, the MCPTT client shall discard the MONP message.

###### 11.2.2.4.6.2	Unexpected indication from MCPTT user

Upon receiving an indication from the MCPTT user in a state where there is no handling specified for the indication, the MCPTT client shall ignore the indication.

###### 11.2.2.4.6.3	Unexpected expiration of a timer

Upon expiration of a timer in a state where there is no handling specified for expiration of the timer, the MCPTT client shall ignore the expiration of the timer.

### 11.2.3	Call type control

#### 11.2.3.1	General

This state machine exists when the UE is part of an ongoing off-network private call.

#### 11.2.3.2	Call type control state machine

The Figure 11.2.3.2-1 gives an overview of the states and transitions of the state machine.

Figure 11.2.3.2-1: Call type state machine

When sending the message, MCPTT client indicates the stored current ProSe per-packet priority associated with the call type control state machine to the lower layers.

#### 11.2.3.3	Call Control states

###### 11.2.3.3.1	Q0: waiting for the call to be established

This state is the start state of this state machine.

###### 11.2.3.3.2	Q1: in-progress private call

This state exists for UE, when the UE is part of an in-progress private call.

###### 11.2.3.3.3	Q2: in-progress emergency private call

This state exists for UE, when the UE is part of an in-progress emergency private call.

#### 11.2.3.4	Procedures

##### 11.2.3.4.1	General

##### 11.2.3.4.2	Outgoing call initiated

When in "Q0: waiting for the call to be established" state, upon an indication from the MCPTT user to initiate a call, the MCPTT client:

1)	if the stored emergency state associated with emergency alert state machine described in 12.2.2.2 is set to "true" and the value of "/<x>/<x>/Common/PrivateCall/EmergencyCall/Authorised" leaf node present in the user profile as specified in 3GPP TS 24.383 [45] is set to "true":

a)	shall set the stored current call type to "EMERGENCY PRIVATE CALL"; and

b)	shall set the stored current ProSe per-packet priority to value corresponding to MCPTT off-network emergency private call as described in 3GPP TS 24.383 [45]; or

2)	if the stored emergency state associated with emergency alert state machine described in 12.2.2.2 is set to "false":

a)	if the user initiates an MCPTT emergency private call and the value of "/<x>/<x>/Common/PrivateCall/EmergencyCall/Authorised" leaf node present in the user profile as specified in 3GPP TS 24.383 [45] is set to "true":

i)	shall set the stored current call type to "EMERGENCY PRIVATE CALL"; and

ii)	shall set the stored current ProSe per-packet priority to value corresponding to MCPTT off-network emergency private call as described in 3GPP TS 24.383 [45]; or

b)	if the user initiates an MCPTT private call:

i)	shall set the stored current call type to "PRIVATE CALL"; and

ii)	shall set the stored current ProSe per-packet priority to value corresponding to MCPTT off-network private call as described in 3GPP TS 24.383 [45].

##### 11.2.3.4.3	Received incoming call

When in "Q0: waiting for the call to be established" state, upon receipt of a PRIVATE CALL SETUP REQUEST message by an idle MCPTT client, the MCPTT client:

1)	if the Call type IE of the received PRIVATE CALL SETUP REQUEST message is set to "EMERGENCY PRIVATE CALL":

a)	shall set the stored current call type to "EMERGENCY PRIVATE CALL"; and

b)	shall set the stored current ProSe per-packet priority to value corresponding to MCPTT off-network emergency private call as described in 3GPP TS 24.383 [45];

2)	if the Call type IE of the received PRIVATE CALL SETUP REQUEST message is set to "PRIVATE CALL":

a)	shall set the stored current call type to "PRIVATE CALL"; and

b)	shall set the stored current ProSe per-packet priority to value corresponding to MCPTT off-network private call as described in 3GPP TS 24.383 [45].

##### 11.2.3.4.4	Establishing the private call

When in "Q0: waiting for the call to be established" state, upon receiving PRIVATE CALL ACCEPT message or PRIVATE CALL ACCEPT ACK message, the MCPTT client:

1)	if the stored current call type is set to "EMERGENCY PRIVATE CALL":

a)	shall enter "Q2: in-progress emergency private call" state.

2)	if the stored current call type is set to "PRIVATE CALL":

a)	shall enter "Q1: in-progress private call" state.

##### 11.2.3.4.5	Upgrade call

###### 11.2.3.4.5.1	User upgrades private call to emergency private call

When in the "Q1: in-progress private call" state, upon an indication from MCPTT User to upgrade the call to emergency and the value of "/<x>/<x>/Common/PrivateCall/EmergencyCall/Authorised" leaf node present in the user profile as specified in 3GPP TS 24.383 [45] is set to "true", the MCPTT client:

1)	shall generate and store emergency offer SDP as defined in subclause 11.2.1.1.2;

2)	shall update caller ID as own MCPTT user ID;

3)	shall update callee ID as MCPTT user ID of the other user;

4)	shall store current user location as user location;

5)	shall generate a PRIVATE CALL SETUP REQUEST message as specified in subclause 15.1.5. In the PRIVATE SETUP REQUEST message, the MCPTT client:

a)	shall set the Call identifier IE with the stored call identifier;

b)	shall set the MCPTT user ID of the caller IE with stored caller ID;

c)	shall set the MCPTT user ID of the callee IE with the stored callee ID;

d)	shall set the Commencement mode IE as "AUTOMATIC COMMENCEMENT MODE";

e)	shall set the Call type IE as "EMERGENCY PRIVATE CALL";

f)	shall set the SDP offer IE with emergency offer SDP; and

g)	may set the User location IE with user location.

6)	shall set the ProSe per-packet priority to the value corresponding to MCPTT off-network emergency private call as described in 3GPP TS 24.383 [45];

7)	shall send the PRIVATE CALL SETUP REQUEST message towards other MCPTT client according to rules and procedures as specified in subclause 11.2.1.1.1;

8)	shall initialize the counter CFP1 (private call request retransmission) with value set to 1;

9)	shall start timer TFP1 (private call request retransmission); and

10)	shall enter the "Q2: in-progress emergency private call" state.

###### 11.2.3.4.5.2	Emergency private call setup request retransmission

When in the "Q2: in-progress emergency private call" state, upon expiry of timer TFP1 (private call request retransmission), the MCPTT client:

1)	may update the stored user location with current user location;

2)	shall increment the value of the counter CFP1 (private call request retransmission) by 1;

3)	shall generate a PRIVATE CALL SETUP REQUEST message as specified in subclause 15.1.5. In the PRIVATE CALL SETUP REQUEST message, the MCPTT client:

a)	shall set the Call identifier IE with the stored call identifier;

b)	shall set the MCPTT user ID of the caller IE with stored caller ID;

c)	shall set the MCPTT user ID of the callee IE with stored callee ID;

d)	shall set the Commencement mode IE as "AUTOMATIC COMMENCEMENT MODE";

e)	shall set the Call type IE as "EMERGENCY PRIVATE CALL";

f)	shall set the SDP offer IE with the stored emergency offer SDP; and

g)	may set the User location IE with stored user location;

4)	shall send the PRIVATE CALL SETUP REQUEST message towards other MCPTT client according to rules and procedures as specified in subclause 11.2.1.1.1;

5)	shall start timer TFP1 (private call request retransmission); and

6)	shall remain in the "Q2: in-progress emergency private call" state.

###### 11.2.3.4.5.3	Emergency private call setup request accepted

When in the "Q2: in-progress emergency private call" state or in the "Q1: in-progress private call" state, upon receiving a PRIVATE CALL ACCEPT message response to PRIVATE CALL SETUP REQUEST message with the same call identifier, the MCPTT client:

1)	shall store the SDP answer IE received in the PRIVATE CALL ACCEPT message as emergency answer SDP;

2)	shall generate a PRIVATE CALL ACCEPT ACK message as specified in subclause 15.1.11:

a)	shall set the Call identifier IE to the stored call identifier;

b)	shall set the MCPTT user ID of the caller IE with stored caller ID; and

c)	shall set the MCPTT user ID of the callee IE with the stored callee ID;

3)	shall send the PRIVATE CALL ACCEPT ACK message in response to the request message according to rules and procedures as specified in subclause 11.2.1.1.1;

4)	shall stop timer TFP1 (call setup retransmission), if running;

5)	shall stop timer TFP2 (waiting for call response message) , if running;

6)	shall establish a media session based on the SDP body of the stored emergency answer SDP; and

7)	shall remain in the current state.

NOTE:	PRIVATE CALL ACCEPT ACK message is retransmitted as described in this subclause, every time a PRIVATE CALL ACCEPT message is received.

###### 11.2.3.4.5.4	Emergency private call setup request rejected

When in the "Q2: in-progress emergency private call" state, upon receiving a PRIVATE CALL REJECT message in response to PRIVATE CALL SETUP REQUEST message with Call identifier IE same as stored call identifier, the MCPTT client:

1)	shall stop timer TFP1 (call setup retransmission), if running;

2)	shall set the ProSe per-packet priority to the value corresponding to the MCPTT off-network private call as described in 3GPP TS 24.383 [45]; and

3)	shall enter the "Q1: in-progress private call" state.

###### 11.2.3.4.5.5	No response to emergency private call setup request

In the "Q2: in-progress emergency private call" state, when timer TFP1 (private call request retransmission) expires and the value of the counter CFP1 (private call request retransmission) is equal to the upper, the MCPTT client:

1)	shall release the stored current call type;

2)	shall release the stored Prose per-packet priority; and

3)	shall enter "Q0: waiting for the call to be established".

###### 11.2.3.4.5.6	Responding to emergency private call setup request when participating in the ongoing call

When in the "Q1: in-progress private call" or "Q2: in-progress emergency private call", upon receiving a PRIVATE CALL SETUP REQUEST message with the Call identifier IE same as the stored call identifier of the call, the Call type IE set as "EMERGENCY PRIVATE CALL", the MCPTT client:

1)	if the media session declared in SDP body of PRIVATE CALL SETUP REQUEST message can be established:

a)	shall generate and store emergency answer SDP based on received SDP offer IE in PRIVATE CALL SETUP REQUEST message, as defined in subclause 11.2.1.1.2;

b)	shall update the caller ID with the MCPTT user ID of the caller IE as received in the PRIVATE CALL SETUP REQUEST message;

c)	shall update the callee ID with own MCPTT user ID;

d)	shall generate a PRIVATE CALL ACCEPT message as specified in subclause 15.1.7:

i)	shall set the Call identifier IE to the stored call identifier;

ii)	shall set the MCPTT user ID of the callee IE with stored callee ID;

iii)	shall set the MCPTT user ID of the caller IE with stored caller ID; and

iv)	shall set the SDP answer IE with the stored emergency answer SDP;

e)	shall set the ProSe per-packet priority to the value corresponding to MCPTT off-network emergency private call as described in 3GPP TS 24.383 [45];

f)	shall send PRIVATE CALL ACCEPT message in response to the request message according to rules and procedures as specified in subclause 11.2.1.1.1; and

g)	shall enter the "Q2: in-progress emergency private call" state.

2)	if the media session declared in SDP body of PRIVATE CALL SETUP REQUEST message cannot be established:

a)	shall generate a PRIVATE CALL REJECT message as specified in subclause 15.1.8;

b)	shall set the call identifier IE with the call identifier in the received message;

c)	shall set the MCPTT user ID of the caller IE with the caller ID in the received message;

d)	shall set the MCPTT user ID of the callee IE with the callee ID in the received message;

e)	shall set the Reason IE as "FAILED", if requested to restrict notification of call failure and the value of "/<x>/<x>/Common/PrivateCall/FailRestrict" leaf node present in the user profile as specified in 3GPP TS 24.383 [45] is set to "true". Otherwise, shall set the reason IE as "MEDIA FAILURE";

f)	shall send a PRIVATE CALL REJECT message in response to the request message according to rules and procedures as specified in subclause 11.2.1.1.1; and

g)	shall remain in the current state.

##### 11.2.3.4.6	Downgrade call

###### 11.2.3.4.6.1	User cancels the emergency private call

When in the "Q2: in-progress emergency private call" state, upon an indication from:

1)	the caller of the emergency private call; or

2)	the recipient of the emergency private call with the value of "/<x>/<x>/Common/PrivateCall/EmergencyCall/CancelPriority" leaf node present in the user profile as specified in 3GPP TS 24.383 [45] set to "true",

to cancel the emergency private call, the MCPTT client:

1)	shall generate a PRIVATE CALL EMERGENCY CANCEL message as specified in subclause 15.1.12. In the PRIVATE CALL EMERGENCY CANCEL message, the MCPTT client:

a)	shall set the Call identifier IE to the stored call identifier;

b)	shall set the MCPTT user ID of the caller IE with the stored caller; and

c)	shall set the MCPTT user ID of the callee IE with the stored callee.

2)	shall send the PRIVATE CALL EMERGENCY CANCEL message according to rules and procedures as specified in subclause 11.2.1.1.1;

3)	shall initialize the counter CFP6 (emergency private call cancel retransmission) with the value set to 1;

4)	shall start timer TFP6 (emergency private call cancel retransmission); and

5)	shall enter the "Q1: in-progress private call" state.

###### 11.2.3.4.6.2	Emergency private call cancel retransmission

When in the "Q1: in-progress private call" state, upon expiry of timer TFP6 (emergency private call cancel retransmission), the MCPTT client:

1)	shall generate a PRIVATE CALL EMERGENCY CANCEL message as specified in subclause 15.1.12. In the PRIVATE CALL EMERGENCY CANCEL message, the MCPTT client:

a)	shall set the Call identifier IE to the stored call identifier;

b)	shall set the MCPTT user ID of the caller IE with the stored caller ID; and

c)	shall set the MCPTT user ID of the callee IE with store callee ID.

2)	shall send the PRIVATE CALL EMERGENCY CANCEL message according to rules and procedures as specified in subclause 11.2.1.1.1;

3)	shall increment the value of the timer CFP6 (emergency private call cancel retransmission) by 1;

4)	shall start timer TFP6 (emergency private call cancel retransmission); and

5)	shall remain in the "Q1: in-progress private call" state.

###### 11.2.3.4.6.3	Emergency private call cancel accepted

When in the "Q1: in-progress private call" state, upon receiving a PRIVATE CALL EMERGENCY CANCEL ACK message response to PRIVATE CALL EMERGENCY CANCEL message with the same "call identifier", the MCPTT client:

1)	shall stop timer TFP6 (emergency private call cancel retransmission), if running;

2)	shall establish a media session based on the SDP body of the stored answer SDP;

3)	shall set the ProSe per-packet priority to the value corresponding to MCPTT off-network private call as described in 3GPP TS 24.383 [45]; and

4)	shall remain in the "Q1: in-progress private call" state.

###### 11.2.3.4.6.4	No response to emergency private call cancel

In the "Q1: in-progress private call" state, when timer TFP6 (emergency private call cancel retransmission) expires and the value of the counter CFP6 (emergency private call cancel retransmission) is equal to the upper limit, the MCPTT client:

1)	shall release the stored current call type;

2)	shall release the stored Prose per-packet priority; and

3)	shall enter "Q0: waiting for the call to be established".

###### 11.2.3.4.6.5	Responding to emergency private call cancel

When in the "Q1: in-progress private call" or "Q2: in-progress emergency private call", upon receiving a PRIVATE CALL EMERGENCY CANCEL message with the same "call identifier" IE, the MCPTT client:

1)	shall generate a PRIVATE CALL EMERGENCY CANCEL ACK as specified in subclause 15.1.13:

a)	shall set the Call identifier IE to the stored call identifier;

b)	shall set the MCPTT user ID of the callee IE with own MCPTT user ID; and

c)	shall set the MCPTT user ID of the caller IE with MCPTT user ID of the caller IE in received message;

2)	shall send PRIVATE CALL EMERGENCY CANCEL ACK message according to rules and procedures as specified in subclause 11.2.1.1.1;

3)	shall establish a media session based on the SDP body of the stored answer SDP;

4)	shall set the ProSe per-packet priority to the value corresponding to MCPTT off-network private call as described in 3GPP TS 24.383 [45]; and

5)	shall enter the "Q1: in-progress private call" state if current state is the "Q2: in-progress emergency private call" state.

##### 11.2.3.4.7	Call Release

When in state "Q1: in-progress private call" or "Q2: in-progress emergency private call", upon receiving an indication from MCPTT user to release the call or upon receiving PRIVATE CALL RELEASE message, the MCPTT client:

1)	shall release the stored current call type;

2)	shall release the stored Prose per-packet priority; and

3)	shall enter "Q0: waiting for the call to be established".

##### 11.2.3.4.8	Error handling

###### 11.2.3.4.8.1	Unexpected MONP message received

Upon receiving a MONP message in a state where there is no handling specified for the MONP message, the MCPTT client shall discard the MONP message.

###### 11.2.3.4.8.2	Unexpected indication from MCPTT user

Upon receiving an indication from the MCPTT user in a state where there is no handling specified for the indication, the MCPTT client shall ignore the indication.

###### 11.2.3.4.8.3	Unexpected expiration of a timer

Upon expiration of a timer in a state where there is no handling specified for expiration of the timer, the MCPTT client shall ignore the expiration of the timer.

# 12	Emergency alert

## 12.0	General

This subclause describes the emergency alert procedures for on-network and off-network.

For on-network emergency alert, the procedures for originating and terminating MCPTT clients, participating MCPTT functions and controlling MCPTT function are specified in subclause 12.1. MCPTT emergency call procedures that have emergency alerts as an optional capability shall be performed as defined in subclause 10.1 for on-network group call and defined in subclause 11.1 for on-network private call.

For off-network emergency alert, the procedures for each functional entity is specified in subclause 12.2.

## 12.1	On-network emergency alert

### 12.1.1	Client procedures

#### 12.1.1.1	Emergency alert origination

Upon receiving a request from the MCPTT user to send an MCPTT emergency alert to the indicated MCPTT group and this is an authorised request for an MCPTT emergency alert as determined by subclause 6.2.8.1.6, the MCPTT client shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [4] and IETF RFC 3428 [33] with the clarifications given below.

NOTE 1:	this SIP MESSAGE request is assumed to be sent out-of-dialog.

The MCPTT client:

1)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Preferred-Service header field according to IETF RFC 6050 [9] in the SIP MESSAGE request;

2)	shall include an Accept-Contact header field with the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [6];

3)	may include a P-Preferred-Identity header field in the SIP MESSAGE request containing a public user identity as specified in 3GPP TS 24.229 [4];

4)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body as specified in clause F.1 with the <mcpttinfo> element containing the <mcptt-Params> element with:

a)	the <mcptt-request-uri> element set to the group identity; and

b)	the <alert-ind> element set to a value of "true";

5)	shall include an application/vnd.3gpp.location-info+xml MIME body as specified in Annex F.3 with a <Report> element included in the <location-info> root element;

6)	shall include in the <Report> element the specific location information configured for the MCPTT emergency alert location trigger;

7)	shall set the MCPTT emergency state if not already set;

8)	shall set the MCPTT emergency alert state to "MEA 2: emergency-alert-confirm-pending";

9)	shall set the Request-URI to the public service identity identifying the participating MCPTT function serving the group identity; and

10)	shall send the SIP MESSAGE request according to rules and procedures of 3GPP TS 24.229 [4].

On receiving a SIP 2xx response to the SIP MESSAGE request, the MCPTT client shall set the MCPTT emergency alert state to "MEA 3: emergency-alert-initiated".

On receiving a SIP 4xx response a SIP 5xx response or a SIP 6xx response to the SIP MESSAGE request, the MCPTT client shall set the MCPTT emergency alert state to "MEA 1: no-alert".

NOTE 2:	the MCPTT emergency state is left set in this case as the MCPTT user presumably is in the best position to determine whether or not they are in a life-threatening condition. The assumption is that the MCPTT user can clear the MCPTT emergency state manually if need be.

#### 12.1.1.2	Emergency alert cancellation

Upon receiving a request from the MCPTT user to send an MCPTT emergency alert cancellation to the indicated MCPTT group and this is an authorised request for an MCPTT emergency alert cancellation as determined by subclause 6.2.8.1.6, the MCPTT client shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [4] and IETF RFC 3428 [33] with the clarifications given below.

NOTE 1:	This SIP MESSAGE request is assumed to be sent out-of-dialog.

The MCPTT client:

1)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Preferred-Service header field according to IETF RFC 6050 [9] in the SIP MESSAGE request;

2)	shall include an Accept-Contact header field with the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [6];

3)	may include a P-Preferred-Identity header field in the SIP MESSAGE request containing the public user identity of the originator as specified in 3GPP TS 24.229 [4];

4)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body as specified in clause F.1 with the <mcpttinfo> element containing the <mcptt-Params> element with:

a)	the <mcptt-request-uri> element set to the MCPTT group identity;

b)	the <alert-ind> element set to a value of "false"; and

c)	if the MCPTT user is cancelling an MCPTT emergency alert originated by another MCPTT user, include the <originated-by> element set to the MCPTT ID of the MCPTT user who originated the MCPTT emergency alert;

5)	if the MCPTT user has additionally requested the cancellation of the in-progress emergency state of the MCPTT group and this is an authorised request for an in-progress emergency group state cancellation as determined by subclause 6.2.8.1.7, shall include an <emergency-ind> element set to a value of "false" in the <mcpttinfo> element containing the <mcptt-Params> element;

6)	shall set the Request-URI to the public service identity identifying the participating MCPTT function serving the group identity;

7)	shall set the MCPTT emergency alert state to "MEA 4: Emergency-alert-cancel-pending"; and

8)	shall send the SIP MESSAGE request according to rules and procedures of 3GPP TS 24.229 [4].

On receiving a SIP 2xx response to the SIP MESSAGE request, the MCPTT client:

1)	if the received SIP 2xx response contains an application/vnd.3gpp.mcptt-info+xml MIME body as specified in clause F.1 with the <mcpttinfo> element containing the <mcptt-Params> element with the <alert-ind> element:

a)	if the <alert-ind> element is set to a value of "false" and the sent SIP MESSAGE request did not contain an <originated-by> element in the application/vnd.3gpp.mcptt-info+xml MIME body, shall:

i)	set the MCPTT emergency alert state to "MEA 1: no-alert"; and

ii)	clear the MCPTT emergency state if not already cleared;

b)	if the <alert-ind> element is set to a value of "true" and if the MCPTT emergency alert state is set to "MEA 4: Emergency-alert-cancel-pending" and the sent SIP MESSAGE request did not contain an <originated-by> element in the application/vnd.3gpp.mcptt-info+xml MIME body, shall set the MCPTT emergency alert state to "MEA 3: emergency-alert-initiated"; and

NOTE 2:	It would appear to be an unusual situation for the initiator of an MCPTT emergency alert to not be able to clear their own alert. Nevertheless, an MCPTT user can be configured to be authorised to initiate MCPTT emergency alerts but not have the authority to clear them. Hence, the case is covered here.

2)	if an <emergency-ind> element is present in the received SIP 200 (OK) response and is set to a value of "false":

a)	shall set the MCPTT emergency group call state of the group to "MEGC 1: emergency-gc-capable"; and

b)	shall set the MCPTT emergency group state of the group to "MEG 1: no-emergency".

NOTE 3:	The case where an <emergency-ind> element is set to true is possible but not handled specifically above as it results in no state changes.

On receiving a SIP 4xx response, SIP 5xx response or SIP 6xx response to the SIP MESSAGE request:

1)	if the received SIP 4xx response, SIP 5xx response or SIP 6xx response contains an application/vnd.3gpp.mcptt-info+xml MIME body as specified in clause F.1 with the <mcpttinfo> element containing the <mcptt-Params> element with the <alert-ind> element set to a value of "true", the sent SIP MESSAGE request did not contain an <originated-by> element in the application/vnd.3gpp.mcptt-info+xml MIME body and the MCPTT emergency alert state is set to "MEA 4: Emergency-alert-cancel-pending", shall set the MCPTT emergency alert state to "MEA 3: emergency-alert-initiated"; and

NOTE 4:	In this case, an <emergency-ind> element would either not be present or would be set to true. In either case, no change in state would result. Hence, this case is not specified above.

2)	 if the received SIP 4xx response, SIP 5xx response or a SIP 6xx response to the SIP MESSAGE request does not contain an application/vnd.3gpp.mcptt-info+xml MIME body with an <alert-ind> element, the sent SIP MESSAGE request does not contain an <originated-by> element in the application/vnd.3gpp.mcptt-info+xml MIME body and the MCPTT emergency alert state is set to "MEA 4: Emergency-alert-cancel-pending", shall set the MCPTT emergency alert state to "MEA 3: emergency-alert-initiated".

#### 12.1.1.3	MCPTT client receives an MCPTT emergency alert or call notification

Upon receipt of a "SIP MESSAGE request for emergency notification", the MCPTT client:

1)	if the received SIP MESSAGE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <alert-ind> element set to a value of "true", should display to the MCPTT user an indication of the MCPTT emergency alert and associated information, including:

a)	the MCPTT group identity contained in <mcptt-calling-group-id> element application/vnd.3gpp.mcptt-info+xml MIME body;

b)	the originator of the MCPTT emergency alert contained in the <mcptt-calling-user-id> element of the application/vnd.3gpp.mcptt-info+xml MIME body; and

c)	the mission critical organization of the MCPTT emergency alert originator contained in the <mc-org> element of the application/vnd.3gpp.mcptt-info+xml MIME body;

NOTE 1:	This is the case of the MCPTT client receiving the notification of another MCPTT user's emergency alert.

2)	if the received SIP MESSAGE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <alert-ind> element set to a value of "false":

a)	should display to the MCPTT user an indication of the MCPTT emergency alert cancellation and associated information, including:

i)	the MCPTT group identity contained in the <mcptt-calling-group-id> element application/vnd.3gpp.mcptt-info+xml MIME body;

ii)	the originator of the MCPTT emergency alert contained in:

A)	if present, the <originated-by> element of the application/vnd.3gpp.mcptt-info+xml MIME body; or

B)	the <mcptt-calling-user-id> element of the application/vnd.3gpp.mcptt-info+xml MIME body; and

b)	if the MCPTT ID contained in the <originated-by> element is the MCPTT ID of the receiving MCPTT user, shall set the MCPTT emergency alert state to "MEA 1: no-alert"; and

c)	if the received SIP MESSAGE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <emergency-ind> element is set to a value of "false", shall set the MCPTT emergency group state to "MEG 1: no-emergency";

NOTE 2:	This is the case of the MCPTT client receiving the notification of the cancellation by a third party of an MCPTT emergency alert. This can be the MCPTT emergency alert of another MCPTT user or the MCPTT emergency alert of the recipient, as determined by the contents of the <originated-by> element. Optionally, notification of the cancellation of the in-progress emergency state of the MCPTT group can be included.

3)	if the received SIP MESSAGE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <emergency-ind> element set to a value of "true":

a)	should display to the MCPTT user an indication of the additional emergency MCPTT user participating in the MCPTT emergency group call including the following if not already displayed as part of step 1):

i)	the MCPTT group identity contained in the <mcptt-calling-group-id> element application/vnd.3gpp.mcptt-info+xml MIME body; and

ii)	the <mcptt-calling-user-id> element of the application/vnd.3gpp.mcptt-info+xml MIME body;

b)	shall set the MCPTT emergency group state to "MEG 2: in-progress" if not already set to that value;

NOTE 3:	This is the case of the MCPTT client receiving notification of an additional MCPTT user in an MCPTT emergency state (i.e., not the MCPTT user that originally triggered the in-progress emergency state of the group) joining the in-progress emergency group call. An emergency alert indication, if included, is handled in step 1).

4)	if the received SIP MESSAGE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <emergency-ind> element set to a value of "false":

a)	should display to the MCPTT user an indication of the cancellation of the in-progress emergency state of the MCPTT group call including the following if not already displayed as part of step 2):

i)	the MCPTT group identity contained in the <mcptt-calling-group-id> element application/vnd.3gpp.mcptt-info+xml MIME body; and

ii)	the <mcptt-calling-user-id> element of the application/vnd.3gpp.mcptt-info+xml MIME body; and

b)	shall set the MCPTT emergency group state to "MEG 1: no-emergency";

NOTE 4:	This is the case of the MCPTT client receiving the notification of the cancellation of the in-progress emergency state of the MCPTT group. In this case, the receiving MCPTT client is affiliated with the MCPTT group but not participating in the session. An emergency cancellation, if included, is handled in step 2).

5)	if the received SIP MESSAGE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <imminentperil-ind> element set to a value of "true":

a)	should display to the MCPTT user an indication of the MCPTT user participating in the MCPTT imminent peril group call including the following if not already displayed as part of step 1):

i)	the MCPTT group identity contained in the <mcptt-calling-group-id> element application/vnd.3gpp.mcptt-info+xml MIME body; and

ii)	the <mcptt-calling-user-id> element of the application/vnd.3gpp.mcptt-info+xml MIME body; and

b)	shall set the MCPTT imminent peril group state to "MIG 2: in-progress" if not already set to that value;

NOTE 5:	This is the case of the MCPTT client receiving notification of an additional MCPTT user initiating an imminent peril group call when there is already an in-progress imminent peril state in effect on the group.

6)	if the received SIP MESSAGE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <imminentperil-ind> element set to a value of "false":

a)	should display to the MCPTT user an indication of the cancellation of the in-progress imminent peril state of the MCPTT group including the following if not already displayed as part of step 2):

i)	the MCPTT group identity contained in the <mcptt-calling-group-id> element application/vnd.3gpp.mcptt-info+xml MIME body; and

ii)	the <mcptt-calling-user-id> element of the application/vnd.3gpp.mcptt-info+xml MIME body; and

b)	shall set the MCPTT imminent peril group state to "MIG 1: no-imminent-peril";

NOTE 6:	This is the case of the MCPTT client receiving notification of the cancellation of the in-progress imminent peril state of the group.

7)	shall generate a SIP 200 (OK) response according to rules and procedures of 3GPP TS 24.229 [4];

8)	shall include the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" in the Contact header field of the SIP 200 (OK) response;

9)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcptt-called-party-id> element set to the MCPTT ID of the called MCPTT user, in the SIP 200 (OK) response; and

10)	shall send the SIP 200 (OK) response towards the MCPTT server according to rules and procedures of 3GPP TS 24.229 [4].

### 12.1.2	Participating MCPTT function procedures

#### 12.1.2.1	Receipt of a SIP MESSAGE request for emergency notification from the served MCPTT client

Upon receipt of a "SIP MESSAGE request for emergency notification for originating participating MCPTT function", the participating MCPTT function:

1)	if unable to process the request due to a lack of resources or a risk of congestion exists, may reject the SIP MESSAGE request with a SIP 500 (Server Internal Error) response. The participating MCPTT function may include a Retry-After header field to the SIP 500 (Server Internal Error) response as specified in IETF RFC 3261 [24] and skip the rest of the steps;

NOTE 1:	if the SIP MESSAGE request contains an emergency indication set to a value of "true" or an alert indication set to a value of "true", the participating MCPTT function can by means beyond the scope of this specification choose to accept the request.

2)	shall determine the MCPTT ID of the calling user from the public user identity in the P-Asserted-Identity header field of the SIP MESSAGE request, and shall authorise the calling user;

NOTE 2:	The MCPTT ID of the calling user is bound to the public user identity at the time of service authorisation, as documented in subclause 7.3.

3)	if the MCPTT user is not affiliated with the MCPTT group and already has N2 simultaneous affiliations, shall reject the SIP MESSAGE request with a SIP 486 (Busy Here) response with the warning text set to "102 too many simultaneous affiliations" in a Warning header field as specified in subclause 4.4. and skip the remaining steps;

NOTE 3:	As this is a request for MCPTT emergency services, the participating MCPTT function can choose to accept the request.

4)	shall determine the public service identity of the controlling MCPTT function associated with the group identity in the received SIP MESSAGE request;

5)	shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [4] and IETF RFC 3428 [33];

6)	shall set the Request-URI of the outgoing SIP MESSAGE request to the public service identity of the controlling MCPTT function associated with the group identified by the <mcptt-request-uri> element contained in the <mcpttinfo> element containing the <mcptt-Params> element of the application/vnd.3gpp.mcptt-info+xml MIME body in the incoming SIP MESSAGE request;

7)	shall copy the contents of the application/vnd.3gpp. mcptt-info+xml MIME body in the received SIP MESSAGE request into an application/vnd.3gpp.mcptt-info+xml MIME body as specified in clause F.1 included in the outgoing SIP MESSAGE request;

8)	shall set the <mcptt-calling-user-id> element of the <mcpttinfo> element containing the <mcptt-Params> element to the MCPTT ID determined in step 2) above;

9)	if the received SIP MESSAGE request contains an application/vnd.3gpp.location-info+xml MIME body as specified in clause F.3 shall copy the contents of the application/vnd.3gpp.location-info+xml MIME body in the received SIP MESSAGE request into an application/vnd.3gpp.location-info+xml MIME body included in the outgoing SIP MESSAGE request;

10)	shall set the P-Asserted-Identity in the outgoing SIP MESSAGE request to the public user identity in the P-Asserted-Identity header field contained in the received SIP MESSAGE request; and

11)	shall send the SIP MESSAGE request as specified to 3GPP TS 24.229 [4].

Upon receipt of a SIP 2xx response in response to the SIP MESSAGE request sent in step 10):

1)	shall generate a SIP 200 (OK) response as specified in 3GPP TS 24.229 [4] with the follow clarifications:

a) shall include in the Contact header field the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt";

b)	if the incoming SIP 200 (OK) response contains an application/vnd.3gpp.mcptt-info+xml MIME body, shall copy the application/vnd.3gpp.mcptt-info+xml MIME body to the outgoing SIP 200 (OK) response; and

c)	shall include the public service identity received in the P-Asserted-Identity header field of the incoming SIP 200 (OK) response into the P-Asserted-Identity header field of the outgoing SIP 200 (OK) response; and

2)	shall send the SIP 200 (OK) response to the MCPTT client according to 3GPP TS 24.229 [4].

#### 12.1.2.2	Receipt of a SIP MESSAGE request for emergency notification for terminating MCPTT client

In the procedures in this subclause:

1)	emergency indication in an incoming SIP MESSAGE request refers to the <emergency-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body; and

2)	alert indication in an incoming SIP MESSAGE request refers to the <alert-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body.

Upon receipt of a "SIP MESSAGE requests for emergency notification for terminating participating MCPTT function", the participating MCPTT function:

1)	if unable to process the request due to a lack of resources or a risk of congestion exists, may reject the SIP MESSAGE request with a SIP 500 (Server Internal Error) response. The participating MCPTT function may include a Retry-After header field to the SIP 500 (Server Internal Error) response as specified in IETF RFC 3261 [24] and skip the rest of the steps;

NOTE 1:	if the SIP MESSAGE request contains an emergency indication set to a value of "true" or an alert indication set to a value of "true", the participating MCPTT function can by means beyond the scope of this specification choose to accept the request.

2)	shall use the MCPTT ID present in the <mcptt-request-uri> element of the application/vnd.3gpp.mcptt-info+xml MIME body of the incoming SIP MESSAGE request to retrieve the binding between the MCPTT ID and public user identity;

3)	if the binding between the MCPTT ID and public user identity does not exist, then the participating MCPTT function shall reject the SIP MESSAGE request with a SIP 404 (Not Found) response. Otherwise, continue with the rest of the steps;

4)	shall generate an outgoing SIP MESSAGE request as specified in subclause 6.3.2.2.11;

5)	if the received SIP MESSAGE request contains a Resource-Priority header field, shall include a Resource-Priority header field with the contents set as in the received Resource-Priority header field; and

6)	shall send the SIP MESSAGE request as specified in 3GPP TS 24.229 [4].

Upon receipt of SIP 2xx responses to the outgoing SIP MESSAGE requests, the participating MCPTT function shall follow the procedures specified in 3GPP TS 24.229 [4].

### 12.1.3	Controlling MCPTT function procedures

#### 12.1.3.1	Handling of a SIP MESSAGE request for emergency notification

Upon receipt of a "SIP MESSAGE request for emergency notification for controlling MCPTT function", the controlling MCPTT function:

1)	if unable to process the request due to a lack of resources or a risk of congestion exists, may reject the SIP MESSAGE request with a SIP 500 (Server Internal Error) response. The controlling MCPTT function may include a Retry-After header field to the SIP 500 (Server Internal Error) response as specified in IETF RFC 3261 [24]. Otherwise, continue with the rest of the steps;

NOTE 1:	If the SIP MESSAGE request contains an alert indication set to a value of "true", the controlling MCPTT function can by means beyond the scope of this specification choose to accept the request.

2)	shall reject the SIP request with a SIP 403 (Forbidden) response and not process the remaining steps if an Accept-Contact header field does not include the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt";

3)	if the received SIP MESSAGE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <alert-ind> element set to a value of "false", shall perform the procedures specified in subclause 12.1.3.2 and skip the rest of the steps;

4)	if the received SIP MESSAGE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <alert-ind> element set to a value of "true":

a)	if the received SIP MESSAGE request is an unauthorised request for an MCPTT emergency alert as specified in subclause 6.3.3.1.13.1 shall reject the SIP MESSAGE request with a SIP 403 (Forbidden) response to the SIP MESSAGE request as specified in 3GPP TS 24.229 [4] with the following clarifications:

i)	shall include in the SIP 403 (Forbidden) response an application/vnd.3gpp.mcptt-info+xml MIME body as specified in clause F.1 with the <mcpttinfo> element containing the <mcptt-Params> element with the <alert-ind> element set to a value of "false"; and

ii)	shall send the SIP 403 (Forbidden) response as specified in 3GPP TS 24.229 [4] and skip the rest of the steps; and

b)	if the received SIP MESSAGE request is an authorised request for an MCPTT emergency alert as specified in subclause 6.3.3.1.13.1:

i)	shall determine if the sending MCPTT user identified by the <mcptt-calling-user-id> element included in the application/vnd.3gpp.mcptt-info+xml MIME body is affiliated with the group identified by the <mcptt-request-uri> element of the MIME body by performing the procedures of subclause 6.3.6;

ii)	if the MCPTT user identified by the MCPTT ID in the SIP MESSAGE request is not affiliated with the MCPTT group as determined by step i) above, the controlling MCPTT function shall affiliate the MCPTT user with the MCPTT chat; and

iii)	for each of the other affiliated members of the group:

A)	generate an outgoing SIP MESSAGE request notification of the MCPTT user's emergency alert indication as specified in subclause 6.3.3.1.11 with the clarifications of subclause 6.3.3.1.12;

B)	shall include in the application/vnd.3gpp.mcptt-info+xml MIME body with the <mcpttinfo> element containing the <mcptt-Params> element with the <mcptt-calling-user-id> element set to the value of the <mcptt-calling-user-id> element in the received SIP MESSAGE request; and

C)	send the SIP MESSAGE request according to according to rules and procedures of 3GPP TS 24.229 [4];

iv)	shall generate a SIP 200 (OK) response to the received SIP MESSAGE request as specified in 3GPP TS 24.229 [4] with the following clarifications:

A)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body as specified in clause F.1 with the <mcpttinfo> element containing the <mcptt-Params> element with the <alert-ind> element set to a value of "true";

B)	shall include the g.3gpp.mcptt media feature tag in the Contact header field;

C)	shall include the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt"; and

D)	shall cache the information that this MCPTT user has initiated an MCPTT emergency alert; and

v)	shall send the SIP 200 (OK) response to the received SIP MESSAGE according to rules and procedures of 3GPP TS 24.229 [4].

Upon receipt of SIP 2xx responses to the outgoing SIP MESSAGE requests, the controlling MCPTT function shall follow the procedures specified in 3GPP TS 24.229 [4].

#### 12.1.3.2	Handling of a SIP MESSAGE request for emergency alert cancellation

Upon receipt of a "SIP MESSAGE request for emergency notification for controlling MCPTT function" containing an application/vnd.3gpp.mcptt-info+xml MIME body with the <alert-ind> element set to a value of "false", the controlling MCPTT function:

1)	if the received SIP MESSAGE request is an unauthorised request for an MCPTT emergency alert cancellation as specified in subclause 6.3.3.1.13.1:

a)	and if the received SIP MESSAGE request does not contain an <emergency-ind> element or is an unauthorised request for an MCPTT emergency call cancellation as specified in subclause 6.3.3.1.13.4, shall reject the SIP MESSAGE request with a SIP 403 (Forbidden) response to the SIP MESSAGE request as specified in 3GPP TS 24.229 [4] with the following clarifications:

i)	shall include in the SIP 403 (Forbidden) response an application/vnd.3gpp.mcptt-info+xml MIME body as specified in clause F.1 with the <mcpttinfo> element containing the <mcptt-Params> element with the <alert-ind> element set to a value of "true";

ii)	if the received SIP MESSAGE request contains an <emergency-ind> element of the <mcpttinfo> element set to a value of "false" and if the in-progress emergency state of the group is set to a value of "true" and this is an unauthorised request for an MCPTT emergency call cancellation as determined in step i) above, shall include an <emergency-ind> element set to a value of "true" in the application/vnd.3gpp.mcptt-info+xml MIME body in the SIP 403 (Forbidden) response; and

iii)	shall send the SIP 403 (Forbidden) response according to rules and procedures of 3GPP TS 24.229 [4] and skip the rest of the steps; and

b)	and if the received SIP MESSAGE request contains an <emergency-ind> element and is an authorised request for an MCPTT emergency call cancellation as specified in subclause 6.3.3.1.13.4 and the in-progress emergency state of the MCPTT group is set to a value of "true":

i)	shall set the in-progress emergency state of the group to a value of "false";

ii)	shall clear the cache of the MCPTT ID of the MCPTT user that triggered the setting of the in-progress emergency state of this MCPTT group to "true";

iii)	shall generate SIP re-INVITE requests to the other affiliated and joined members of the MCPTT group as specified in subclause 6.3.3.1.6. The MCPTT controlling function:

A)	for each affiliated and joined member shall send the SIP re-INVITE request towards the MCPTT client as specified in 3GPP TS 24.229 [4]; and

B)	Upon receiving a SIP 200 (OK) response to the SIP re-INVITE request the controlling MCPTT function shall interact with the media plane as specified in 3GPP TS 24.380 [5];

iv)	for each of the affiliated but not joined members of the group shall:

A)	generate a SIP MESSAGE request notification of the cancellation of the MCPTT user's emergency call as specified in subclause 6.3.3.1.11;

B)	shall include in the application/vnd.3gpp.mcptt-info+xml MIME body with the <mcpttinfo> element containing the <mcptt-Params> element with the <mcptt-calling-user-id> element set to the value of the <mcptt-calling-user-id> element in the received SIP MESSAGE request; and

C)	shall include an <emergency-ind> element set to a value of "false" in the application/vnd.3gpp.mcptt-info+xml MIME body in the outgoing SIP MESSAGE request;

v)	shall generate a SIP 200 (OK) response to the received SIP MESSAGE request as specified in 3GPP TS 24.229 [4] with the following clarifications:

A)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body as specified in clause F.1 with the <mcpttinfo> element containing the <mcptt-Params> element with the <alert-ind> element set to a value of "true";

B)	shall include an <emergency-ind> element set to a value of "false" in the application/vnd.3gpp.mcptt-info+xml MIME body in the outgoing SIP 200 (OK) response;

C)	shall include the g.3gpp.mcptt media feature tag in the Contact header field; and

D)	shall include the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt"; and

vi)	shall send the SIP 200 (OK) response to the received SIP MESSAGE as specified in 3GPP TS 24.229 [4] and skip the rest of the steps; and

2)	if the received SIP MESSAGE request is an authorised request for an MCPTT emergency alert cancellation as specified in subclause 6.3.3.1.13.1:

a)	if the received SIP MESSAGE request contains an <originated-by> element in the application/vnd.3gpp.mcptt-info+xml MIME body, shall clear the cache of the MCPTT ID of the MCPTT user identified by the <originated-by> element as having an outstanding MCPTT emergency alert;

b)	if the received SIP MESSAGE request does not contain an <originated-by> element in the application/vnd.3gpp.mcptt-info+xml MIME body, clear the cache of the MCPTT ID of the sender of the SIP MESSAGE request as having an outstanding MCPTT emergency alert;

c)	if the received SIP MESSAGE request does not contain an <emergency-ind> element or is an unauthorised request for an MCPTT emergency call cancellation as specified in subclause 6.3.3.1.13.4, for each of the affiliated but not joined members of the group shall:

i)	generate a SIP MESSAGE request notification of the cancellation of the MCPTT user's emergency alert as specified in subclause 6.3.3.1.11;

ii)	shall include in the application/vnd.3gpp.mcptt-info+xml MIME body with the <mcpttinfo> element containing the <mcptt-Params> element with the <mcptt-calling-user-id> element set to the value of the <mcptt-calling-user-id> element in the received SIP MESSAGE request;

iii)	if the received SIP MESSAGE request contains an <originated-by> element in the application/vnd.3gpp.mcptt-info+xml MIME body, copy the contents of the received <originated-by> element to an <originated-by> element in the application/vnd.3gpp.mcptt-info+xml MIME body in the outgoing SIP MESSAGE request;

iv)	shall include an <alert-ind> element set to a value of "false" in the application/vnd.3gpp.mcptt-info+xml MIME body in the outgoing SIP MESSAGE request; and

v)	send the SIP MESSAGE request as specified in 3GPP TS 24.229 [4];

d)	if the received SIP MESSAGE request contains an <emergency-ind> element and is an authorised request for an MCPTT emergency call cancellation as specified in subclause 6.3.3.1.13.4 and the in-progress emergency state of the MCPTT group is set to a value of "true":

i)	shall set the in-progress emergency state of the group to a value of "false";

ii)	cache the information that this MCPTT user has cancelled the outstanding in-progress emergency state of the group;

iii)	shall generate SIP re-INVITES requests to the other affiliated and joined members of the MCPTT group as specified in subclause 6.3.3.1.6. The MCPTT controlling function:

A)	for each affiliated and joined member shall send the SIP re-INVITE request towards the MCPTT client as specified in 3GPP TS 24.229 [4]; and

B)	Upon receiving a SIP 200 (OK) response to the SIP re-INVITE request the controlling MCPTT function shall interact with the media plane as specified in 3GPP TS 24.380 [5]; and

iv)	for each of the affiliated but not joined members of the group shall:

A)	generate a SIP MESSAGE request notification of the cancellation of the MCPTT user's emergency call as specified in subclause 6.3.3.1.11;

B)	include in the application/vnd.3gpp.mcptt-info+xml MIME body with the <mcpttinfo> element containing the <mcptt-Params> element with the <mcptt-calling-user-id> element set to the value of the <mcptt-calling-user-id> element in the received SIP MESSAGE request;

C)	if the received SIP MESSAGE request contains an <originated-by> element in the application/vnd.3gpp.mcptt-info+xml MIME body, copy the contents of the received <originated-by> element to an <originated-by> element in the application/vnd.3gpp.mcptt-info+xml MIME body in the outgoing SIP MESSAGE request;

D)	include in the application/vnd.3gpp.mcptt-info+xml MIME body an <alert-ind> element set to a value of "false"; and

E)	shall include an <emergency-ind> element set to a value of "false" in the application/vnd.3gpp.mcptt-info+xml MIME body in the outgoing SIP MESSAGE request;

e)	shall generate a SIP 200 (OK) response to the received SIP MESSAGE request as specified in 3GPP TS 24.229 [4] with the following clarifications:

i)	shall include an application/vnd.3gpp.mcptt-info+xml" MIME body as specified in clause F.1 with the <mcpttinfo> element containing the <mcptt-Params> element with the <alert-ind> element set to a value of "false";

ii)	if the received SIP MESSAGE request contains an <emergency-ind> element of the <mcpttinfo> element set to a value of "false":

A)	if this is an authorised request for an MCPTT emergency call cancellation as specified in subclause 6.3.3.1.13.4, shall include an <emergency-ind> element set to a value of "false" in the application/vnd.3gpp.mcptt-info+xml MIME body in the outgoing SIP 200 (OK) response; and

B)	otherwise, if this is an unauthorised request for an MCPTT emergency call cancellation as specified in subclause 6.3.3.1.13.4, and the in-progress emergency state of the group is set to a value of "true", shall include an <emergency-ind> element set to a value of "true" in the application/vnd.3gpp.mcptt-info+xml MIME body in the outgoing SIP 200 (OK) response;

iii)	shall include the g.3gpp.mcptt media feature tag in the Contact header field; and

iv)	shall include the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt"; and

f)	shall send the SIP 200 (OK) response to the received SIP MESSAGE as specified in 3GPP TS 24.229 [4].

Upon receipt of SIP 2xx responses to the outgoing SIP MESSAGE requests, the controlling MCPTT function shall follow the procedures specified in 3GPP TS 24.229 [4].

## 12.2	Off-network emergency alert

### 12.2.1	General

### 12.2.2	Basic state machine

#### 12.2.2.1	General

#### 12.2.2.2	Emergency alert state machine

The figure 12.2.2.2-1 gives an overview of the main states and transitions on the UE for emergency alert.

Each emergency alert state machine is per MCPTT group.

Figure 12.2.2.2-1: Emergency alert state machine

The following piece of information is associated with the emergency alert state machine:

a)	the stored emergency state of the MCPTT group.

NOTE:	The emergency alert state machine is referred by the MCPTT off-network group call and MCPTT off-network private call procedures.

#### 12.2.2.3	Emergency alert states

##### 12.2.2.3.1	E1: Not in emergency state

This state is the start state of this state machine.

The UE stays in this state while not in emergency state.

##### 12.2.2.3.2	E2: Emergency state

This state exists for UE, when the UE has sent a GROUP EMERGENCY ALERT message.

### 12.2.3	Procedures

#### 12.2.3.1	Originating user sending emergency alert

When in state "E1: Not in emergency state", upon receiving an indication from the MCPTT user to transmit an emergency alert for an MCPTT group ID, and the values of "/<x>/<x>/Common/MCPTTGroupCall/EmergencyAlert/Authorised" leaf node present in the user profile and "/<x>/<x>/Common/AllowedEmergencyAlert" present in group configuration as specified in 3GPP TS 24.383 [45] are set to "true", the MCPTT client:

1)	shall set the stored emergency state as "true";

2)	shall set the stored MCPTT group ID to the indicated MCPTT group ID;

3)	shall generate a GROUP EMERGENCY ALERT message as specified in subclause 15.1.16. In the GROUP EMERGENCY ALERT message, the MCPTT client:

a)	shall set the MCPTT group ID IE to the stored MCPTT group ID;

b)	shall set the Originating MCPTT user ID IE to own MCPTT user ID;

c)	shall set the Organization name IE to own organization name; and

d)	may set the User location IE with client's current location, if requested;

4)	 shall send the GROUP EMERGENCY ALERT message as specified in subclause 10.2.1.1.1;

5)	shall start timer TFE2 (emergency alert retransmission); and

6)	shall enter "E2: Emergency state" state.

#### 12.2.3.2	Emergency alert retransmission

When in state "E2: Emergency state", upon expiry of timer TFE2 (emergency alert retransmission), the MCPTT client:

1)	shall generate a GROUP EMERGENCY ALERT message as specified in subclause 15.1.16. In the GROUP EMERGENCY ALERT message, the MCPTT client:

a)	shall set the MCPTT group ID IE to the stored MCPTT group ID;

b)	shall set the originating MCPTT user ID IE to own MCPTT user ID;

c)	shall set the Organization name IE to own organization name; and

d)	may set the Location IE with client's current location, if requested; and

2)	 shall send the GROUP EMERGENCY ALERT message as specified in subclause 10.2.1.1.1;

3)	shall start the timer TFE2 (emergency alert retransmission); and

4)	shall remain in the current state.

#### 12.2.3.3	Terminating user receiving emergency alert

When in state "E1: Not in emergency state" or in "E2: Emergency state", upon receiving a GROUP EMERGENCY ALERT message with the Originating MCPTT user ID IE not stored in the list of users in emergency, the MCPTT client:

1)	shall store the Originating MCPTT user ID IE and location IE in the list of users in emergency;

2)	shall generate a GROUP EMERGENCY ALERT ACK message as specified in subclause 15.1.17. In the GROUP EMERGENCY ALERT ACK message, the MCPTT client:

a)	shall set the MCPTT group ID IE to the MCPTT group ID IE of the received GROUP EMERGENCY ALERT message;

b)	shall set the Sending MCPTT user ID IE to own MCPTT user ID; and

c)	shall set the Originating MCPTT user ID IE to the Originating MCPTT user ID IE of the received GROUP EMERGENCY ALERT message; and

3)	 shall send the GROUP EMERGENCY ALERT ACK message as specified in subclause 10.2.1.1.1;

4)	shall start timer TFE1 (Emergency Alert); and

5)	shall remain in the current state.

NOTE:	Each instance of timer TFE1 is per MCPTT user ID.

#### 12.2.3.4	Terminating user receiving retransmitted emergency alert

When in state "E1: Not in emergency state" or in "E2: Emergency state", upon receiving a GROUP EMERGENCY ALERT message with the Originating MCPTT user ID IE stored in the list of users in emergency and Location IE different than the stored location of the user, the MCPTT client:

1)	may update the stored location of the user with the received Location IE;

2)	shall restart the associated timer TFE1 (Emergency Alert); and

3)	shall remain in the current state.

#### 12.2.3.5	Originating user cancels emergency alert

When in "E2: Emergency state", upon receiving an indication from the MCPTT user to cancel an emergency alert and the value of "/<x>/<x>/Common/MCPTTGroupCall/EmergencyAlert/Cancel" leaf node present in the user profile as specified in 3GPP TS 24.383 [45] set to "true", the MCPTT client:

1)	shall set the stored emergency state as "false";

2)	shall generate a GROUP EMERGENCY ALERT CANCEL message as specified in subclause 15.1.18. In the GROUP EMERGENCY ALERT CANCEL message, the MCPTT client:

a)	shall set the MCPTT group ID IE to the stored MCPTT group ID; and

b)	shall set the Originating MCPTT user ID IE to own MCPTT user ID; and

3)	 shall send the GROUP EMERGENCY ALERT CANCEL message as specified in subclause 10.2.1.1.1; and

4)	shall enter "E1: Not in emergency state" state.

#### 12.2.3.6	Terminating user receives GROUP EMERGENCY ALERT CANCEL message

When in state "E1: Not in emergency state" or in "E2: Emergency state", upon receiving a GROUP EMERGENCY ALERT CANCEL message with the Originating MCPTT user ID IE stored in the list of users in emergency, the MCPTT client:

1)	shall remove the MCPTT user ID and associated location information from the stored list of users in emergency;

2)	shall generate a GROUP EMERGENCY ALERT CANCEL ACK message as specified in subclause 15.1.19. In the GROUP EMERGENCY ALERT CANCEL ACK message, the MCPTT client:

a)	shall set the MCPTT group ID IE to the MCPTT group ID IE of the received GROUP EMERGENCY ALERT CANCEL message; and

b)	shall set the Sending MCPTT user ID IE to own MCPTT user ID; and

c)	shall set the Originating MCPTT user ID IE to the Originating MCPTT user ID IE of the received GROUP EMERGENCY ALERT message;

3)	 shall send the GROUP EMERGENCY ALERT CANCEL ACK message as specified in subclause 10.2.1.1.1; and

4)	shall remain in the current state.

#### 12.2.3.7	Implicit emergency alert cancel

When in state "E1: Not in emergency state" or in "E2: Emergency state", upon expiry of timer TFE1 (Emergency Alert) associated with a stored MCPTT user ID, the MCPTT client:

1)	shall remove the MCPTT user ID and associated location information from the stored list of users in emergency; and

2)	shall remain in the current state.

# 13	Location procedures

## 13.1	General

If the participating MCPTT function needs to obtain location information, the participating MCPTT function configures the MCPTT client when the participating MCPTT function receives a third-party REGISTER request where the MCPTT client SIP URI is in the To header field. The configuration contains information the MCPTT client uses to set up filter criteria for when the MCPTT client shall send location reports to the participating MCPTT function.

The participating MCPTT function can also explicitly request the MCPTT client to send a location report.

The MCPTT client will, based on the received configuration or when explicitly requested, send location reports.

The location information is used by the participating MCPTT function to determine whether to use MBMS bearers or not as described in clause 14.

## 13.2	Participating MCPTT function location procedures

### 13.2.1	General

The participating MCPTT function has procedures to:

-	configure the location reporting at the UE;

-	request the UE to report the location of the UE; and

-	receive a location information report from the UE.

### 13.2.2	Location reporting configuration

Upon receipt of a third-party SIP REGISTER request for an MCPTT client, the participating MCPTT function may configure the location reporting in the MCPTT client by generating a SIP MESSAGE request in accordance with 3GPP TS 24.229 [4] and IETF RFC 3428 [33]. The participating MCPTT function:

1)	shall include a Request-URI set to the URI received in the To header field in the third-party SIP REGISTER request;

2)	shall include an Accept-Contact header field with the media feature tag g.3gpp.icsi-ref set to the value "urn:urn-7:3gpp-service.ims.icsi.mcptt" along with parameters "require" and "explicit" in accordance with IETF RFC 3841 [6];

3)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body with an <mcptt-request-uri> element containing the MCPTT ID of the MCPTT user to receive the configuration;

4)	shall include an application/vnd.3gpp.mcptt-location-info+xml MIME body with the <Configuration> element contained in the <location-info> root element set to the desired configuration;

5)	shall include the TriggerId attribute where defined for the sub-elements defining the trigger criterion ;

6)	shall include the public service identity of the participating MCPTT function in the P-Asserted-Identity header field;

7)	shall include a P-Asserted-Service header field with the value "urn:urn-7:3gpp-service.ims.icsi.mcptt"; and

8)	shall send the SIP MESSAGE request as specified in 3GPP TS 24.229 [4].

### 13.2.3	Location information request

If the participating MCPTT function needs to request the MCPTT client to report its location, the participating MCPTT functions shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [4] and IETF RFC 3428 [33]. The participating MCPTT function:

1)	shall include a Request-URI set to the URI received in the To header field in the third-party SIP REGISTER request;

2)	shall include an Accept-Contact header field with the media feature tag g.3gpp.icsi-ref set to the value "urn:urn-7:3gpp-service.ims.icsi.mcptt" along with parameters "require" and "explicit" in accordance with IETF RFC 3841 [6];

3)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body with an <mcptt-request-uri> element containing the MCPTT ID of the MCPTT user;

4)	shall include an application/vnd.3gpp.mcptt-location-info+xml MIME body with a <Request> element contained in the <location-info> root element;

5)	shall include a P-Asserted-Service header field with the value "urn:urn-7:3gpp-service.ims.icsi.mcptt"; and

6)	shall send the SIP MESSAGE request as specified in 3GPP TS 24.229 [4].

### 13.2.4	Location information report

If the participating MCPTT function receives a SIP request containing:

1)	a Content-Type header field set to "application/vnd.3gpp.mcptt-location-info+xml"; and

2)	an application/vnd.3gpp.mcptt-location-info+xml MIME body with a <Report> element included in the <location-info> root element;

then the participating MCPTT function shall authorise the location report based on the MCPTT ID received. If the MCPTT user is authorised to send a location report the participating MCPTT function:

1)	shall use the location information as needed.

NOTE:	The <Report> element contains the event triggering identity in the location information report from the UE, and can contain location information.

### 13.2.5	Abnormal cases

Upon receipt of a SIP request:

1)	where the P-Asserted-Identity identifies a public user identity not associated with an MCPTT user served by the participating MCPTT function; or

2)	with a MIME body with Content-Type header field set to "application/vnd.3gpp.mcptt-info+xml" and with a <mcptt-request-URI> element containing an MCPTT ID that identifies an MCPTT user served by the participating MCPTT function;

then, when the SIP request contains:

1)	an Accept-Contact header field with the g.3gpp.mcptt media feature tag;

2)	an Accept-Contact header field with the g.3gpp.icsi-ref media-feature tag with the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt"; and

3)	an application/vnd.3gpp.location-info+xml MIME body containing a <Request> element or a <Configuration> element;

the participating MCPTT function shall remove the application/vnd.3gpp.location-info+xml MIME body from the outgoing SIP request.

## 13.3	MCPTT client location procedures

### 13.3.1	General

The MCPTT client sends a location report when one of the trigger criteria is fulfilled or when it receives a request from the participating MCPTT function to send a location report. To send the location report the MCPTT client can use an appropriate SIP message that it needs to send for other reasons, or it can include the location report in a SIP MESSAGE request.

To send a location report, the MCPTT client includes in the SIP MESSAGE request an application/vnd.3gpp.mcptt-location-info+xml MIME body as specified in clause F.3. The MCPTT client populates the elements in accordance with its reporting configuration. Further location information may also be included in the P-Access-Network-Info header field.

### 13.3.2	Location reporting configuration

Upon receiving a SIP MESSAGE request containing:

1)	an Accept-Contact header field with the media feature tag g.3gpp.icsi-ref set to the value "urn:urn-7:3gpp-service.ims.icsi.mcptt";

2)	a Content-Type header field set to "application/vnd.3gpp.mcptt-location-info+xml"; and

3)	 an application/vnd.3gpp.mcptt-location-info+xml MIME body with a <Configuration> root element included in the <location-info> root element;

then the MCPTT client:

1)	shall store the contents of the <Configuration> elements;

2)	shall set the location reporting triggers accordingly; and

3)	shall start the minimumReportInterval timer.

### 13.3.3	Location information request

Upon receiving a SIP MESSAGE request containing

1)	an Accept-Contact header field with the media feature tag g.3gpp.icsi-ref set to the value "urn:urn-7:3gpp-service.ims.icsi.mcptt";

2)	a Content-Type header field set to "application/vnd.3gpp.mcptt-location-info+xml"; and

3)	 an application/vnd.3gpp.mcptt-location-info+xml MIME body with a <Request> element included in the <location-info> root element;

then the MCPTT client:

1)	shall send a location report as specified in subclause 13.3.4; and

2)	shall reset the minimumReportInterval timer.

### 13.3.4	Location information report

#### 13.3.4.1	Report triggering

If a location reporting trigger fires the MCPTT client checks if the minimumReportInterval timer is running. If the timer is running the MCPTT client waits until the timer expires. When the minimumReportInterval timer fires, the MCPTT client:

1)	shall, if any of the reporting triggers are still true, send a location information report as specified in subclause 13.3.4.2.

If the MCPTT client receives a location information request as specified in subclause 13.3.3, the MCPTT client shall send a location report as specified in subclause 13.3.4.2.

#### 13.3.4.2	Sending location information report

If the MCPTT client needs to send a SIP request for other reasons (e.g. a SIP MESSAGE request containing an MBMS listening report as described in clause 14), the MCPTT client:

1)	 shall include an application/vnd.3gpp.mcptt-location-info+xml MIME body and in the <location-info> root element the MCPTT client shall include:

a)	a <Report> element and if the Report was triggered by a location request include the <ReportID> attribute set to the value of the <RequestID> attribute in the received Request;

b)	<TriggerId> child elements, where each element is set to the value of the <Trigger-Id> attribute associated with the trigger that have fired; and

c)	the location reporting elements corresponding to the triggers that have fired;

2)	shall set the minimumReportInterval timer to the minimumReportInterval time and start the timer; and

3)	shall reset all triggers.

If the MCPTT client does not need to send a SIP request for other reasons, the MCPTT client shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [4] and IETF RFC 3428 [33]. The MCPTT client;

1)	shall include in the Request-URI, the SIP URI received in the P-Asserted-Identity header field in the received SIP MESSAGE request for location report configuration;

2)	shall include a Content-Type header field set to "application/vnd.3gpp.mcptt-location-info+xml";

3)	shall include an application/vnd.3gpp.mcptt-location-info+xml MIME body and in the <location-info> root element include:

a)	a <Report> element and if the Report was triggered by a location request include the <ReportID> attribute set to the value of the <RequestID> attribute in the received Request;

b)	a <TriggerId> child element set to the value of each <Trigger-Id> value of the triggers that have fired; and

c)	the location reporting elements corresponding to the triggers that have fired;

4)	shall include an Accept-Contact header field with the media feature tag g.3gpp.mcptt along with parameters "require" and "explicit" in accordance with IETF RFC 3841 [6];

5)	shall set the minimumReportInterval timer to the minimumReportInterval time and start the timer;

6)	shall reset all triggers; and

7)	shall send the SIP MESSAGE request as specified in 3GPP TS 24.229 [4].

# 14	MBMS transmission usage procedure

## 14.1	General

This clause describes the participating MCPTT function and the MCPTT client procedure for:

1)	MBMS bearer announcements; and

2)	MBMS bearer listening status.

## 14.2	Participating MCPTT function MBMS usage procedures

### 14.2.1	General

This subclause describes the procedures in the participating MCPTT function for:

1)	sending an MBMS bearer announcements to the MCPTT client; and

2)	receiving an MBMS bearer listening status from the MCPTT client.

### 14.2.2	Sending MBMS bearer announcement procedures

#### 14.2.2.1	General

The availability of a pre-activated MBMS bearer is announced to MCPTT clients by means of an MBMS bearer announcement. One or more MBMS bearer announcements are included in an application/vnd.3gpp.mcptt-mbms-usage-info+xml MIME body.

An MBMS bearer announcement message can contain new MBMS bearer announcements, updated MBMS bearer announcements or cancelled MBMS bearer announcements or a mix of all of them at the same time in an application/vnd.3gpp.mcptt-mbms-usage-info+xml MIME body.

NOTE:	A new MBMS bearer announcement does not implicitly remove previously sent MBMS bearer announcements if the previously sent MBMS bearer announcement is not included in an MBMS bearer announcement message.

When and to whom the participating MCPTT function sends the MBMS bearer announcement is based on local policy in the participating MCPTT function.

The following subclauses describe how the participating MCPTT function:

1.	sends an initial MBMS bearer announcement;

2.	updates a previously sent announcement; and

3.	cancels a previously sent announcement.

#### 14.2.2.2	Sending an initial MBMS bearer announcement procedure

For each MCPTT client that the participating MCPTT function is sending an MBMS bearer announcement to, the participating MCPTT function:

1)	shall generate an SIP MESSAGE request in accordance with 3GPP TS 24.229 [4] and IETF RFC 3428 [33];

2)	shall set the Request-URI to the public service identity identifying the participating MCPTT function serving the MCPTT user;

3)	shall include an Accept-Contact header field containing the g.3gpp.mcptt media feature tag along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [6];

4)	shall include an Accept-Contact header field with the g.3gpp.icsi-ref media-feature tag with the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" along with parameters "require" and "explicit" according to IETF RFC 3841 [6];

5)	shall include a P-Asserted-Service header field with the value "urn:urn-7:3gpp-service.ims.icsi.mcptt";

6)	shall include one application/sdp MIME body conforming to 3GPP TS 24.229 [4] where the application/sdp MIME body and:

a)	shall include the Content-Disposition header field with the value "render"; and

b)	should include one or more" m=audio" media lines and media line attributes as defined in 3GPP TS 24.380 [5] to be used as the MBMS subchannel for audio and media control. Additional the participating MCPTT function:

i)	shall set c-line to the unspecified address (0.0.0.0), if IPv4, or to a domain name within the ".invalid" DNS top-level domain, if IPv6;

ii)	shall set port number of the media line to 9;

ii)	shall include the "a=rtp-mux" attribute as specified in IETF RFC 5761 [39]; and

iii)	shall include the "a=rtcp:9" as specified in IETF RFC 5761 [39].

c)	should include one or more" m=audio" media lines and media line attributes as defined in 3GPP TS 24.380 [5] to be used as the MBMS subchannel for audio only. Additional the participating MCPTT function:

i)	shall set the c-line to the unspecified address (0.0.0.0), if IPv4, or to a domain name within the ".invalid" DNS top-level domain, if IPv6; and

ii)	shall set the port number of the media line to 9;

NOTE 1:	If an MBMS subchannel for audio only is included, the "a=rtp-mux" and "a=rtcp:" attributes are not included in the media line.

d)	shall include one "m=application" media line as defined in 3GPP TS 24.380 [5] to be used as the general purpose MBMS subchannel. The media line shall include a valid IP address and a valid port number; and

NOTE 2:	The media parameters to be used by the MBMS subchannel for media is included in the Map Group To Bearer message defined in 3GPP TS 24.380 [5] and not included in this application/sdp MIME body.

e)	if "m=audio" media lines to be used in an MBMS subchannel for audio only are included above, shall include one or more "m=application" media line as defined in 3GPP TS 24.380 [5] to be used as the MBMS subchannel for floor control messages. The media line:

i)	shall set c-line to the unspecified address (0.0.0.0), if IPv4, or to a domain name within the ".invalid" DNS top-level domain, if IPv6; and

ii)	shall set the port number of the media line to 9;

NOTE 3:	The use of a separate MBMS subchannel for floor control is optional. When a separate MBMS subchannel for floor control is not used, floor control messages are sent in the MBMS subchannel for media.

7)	shall include one or more <announcement> elements associated with the pre-activated MBMS bearers in the application/vnd.3gpp.mcptt-mbms-usage-info+xml MIME body as defined in clause F.2. Each set of an <announcement> element:

a)	shall include a TMGI value in the <TMGI> element;

NOTE 4:	The same TMGI value can only appear in one <announcement> element. The TMGI value is also used to identify the <announcement> when updating or cancelling the <announcement> element.

b)	shall include the QCI value in the <QCI> element;

c)	if multiple carriers are supported, shall include the frequency to be used in the <frequency> element;

NOTE 5:	In the current release if the <frequency> element is included, the frequency in the <frequency> element is the same as the frequency used for unicast.

d)	shall include one or more MBMS service area IDs in the <mbms-service-areas> element; and

e)	shall include the number of "m=application" media line to be used as the general purpose MBMS subchannel;

8)	shall include one <GPMS> element giving the number of "m=application" media line in the application/sdp MIME body generated in step 7 above to be used as the general purpose MBMS subchannel;

9)	shall include the MBMS public service identity of the participating MCPTT function in the P-Asserted-Identity header field;

10)	shall include in a MIME body with Content-Type header field set to "application/vnd.3gpp.mcptt-info+xml", the <mcptt-request-uri> element set to the MCPTT ID of the user; and

11)	shall send the SIP MESSAGE request towards the MCPTT client according to 3GPP TS 24.229 [4].

#### 14.2.2.3	Updating an announcement

When the participating MCPTT function wants to update a previously sent announcement, the participating MCPTT function sends an MBMS bearer announcement in an SIP MESSAGE request as specified in subclause 14.2.2.2 where the participating MCPTT function in the <announcement> element to be updated:

1)	shall include the same TMGI value as in the MBMS bearer announcement to be updated in the <TMGI> element;

NOTE 1:	TMGI value is used to identify the <announcement> when updating or cancelling the <announcement> element and can't be changed.

2)	shall include the same or an updated value of the QCI in the <QCI> element;

3)	if a frequency was included in the previously sent announcement, shall include the same value in the <frequency> element;

NOTE 2:	In the current release if the <frequency> element is included, the frequency in the <frequency> element is the same as the frequency used for unicast.

4)	shall include the same list of MBMS service area IDs or an updated list of MBMS service area IDs in the <mbms-service-areas> element;

5)	shall include the <GPMS> element with the same value as in the initial <announcement> element; and

6)	shall include the same application/sdp MIME body as included in the initial MBMS announcement.

#### 14.2.2.4	Cancelling an MBMS bearer announcement

When the participating MCPTT function wants to cancel an MBMS bearer announcement associated with an <announcement> element, the participating MCPTT function sends an MBMS bearer announcement as specified in subclause 14.2.2.2 where the participating MCPTT function in the <announcement> element to be cancelled:

1)	shall include the same TMGI value as in the <announcement> element to be cancelled in the <TMGI> element;

2)	shall include the same value of the QCI in the <QCI> element;

3)	shall include one <mbms-service-area> element set to "0";

4)	if the application/vnd.3gpp.mcptt-mbms-usage-info+xml MIME body only contains <announcement> elements that are to be cancelled, shall not include an <GPMS> element; and

5)	if the application/vnd.3gpp.mcptt-mbms-usage-info+xml MIME body only contains <announcement> elements that are to be cancelled, shall not include an application/sdp MIME body.

### 14.2.3	Receiving an MBMS bearer listening status from an MCPTT client

Upon receiving a "SIP MESSAGE request for an MBMS listening status update", the participating MCPTT function shall handle the request in accordance with 3GPP TS 24.229 [4] and IETF RFC 3428 [33].

If the SIP MESSAGE request contains:

1)	an application/vnd.3gpp.mcptt-mbms-usage-info+xml MIME body with an <mbms-listening-status> element; and

2)	an application/vnd.3gpp.mcptt-info+xml MIME body containing an MCPTT ID in the <mcptt-request-uri> served by the participating MCPTT function;

then the participating MCPTT function:

1)	shall verify that the public user identity in the P-Asserted-Identity header field is bound to the MCPTT ID in the <mcptt-request-uri> element in the application/vnd.3gpp.mcptt-info+xml MIME body, and if that is the case:

a)	if the <mbms-listening-status> element is set to "listening":

i)	if <session-identifier> elements are included, shall indicate to the media plane that the MCPTT client in the session identified by the <session-identifier> element is now listening to the MBMS subchannel; and

ii)	if <general-purpose> element is included with the value "true", shall indicate to the media plane that the MCPTT client is now listening to the general purpose MBMS subchannel; and

b)	if the <mbms-listening-status> element is set to "not-listening":

i)	if <session-identifier> elements are included, shall indicate to the media plane that the MCPTT client in the sessions identified by the <session-identifier> elements is not listening to the MBMS subchannel;

ii)	if <general-purpose> element is included with the value "false", shall indicate to the media plane that the MCPTT client is no longer listening to the general purpose MBMS bearer; and

iii)	shall interact with the media plane as specified in 3GPP TS 24.380 [5].

NOTE:	If the MCPTT client reports that the MCPTT client is no longer listening to the general purpose MBMS subchannel it is implicitly understood that the MCPTT client no longer listens to any MBMS subchannel in ongoing conversations that the MCPTT client previously reported status "listening".

### 14.2.4	Abnormal cases

Upon receipt of a SIP MESSAGE request:

1)	where the P-Asserted-Identity identifies a public user identity not associated with MCPTT user served by the participating MCPTT function; or

2)	with an application/vnd.3gpp.mcptt-info+xml and with a <mcptt-request-uri> element containing an MCPTT ID that identifies an MCPTT user served by the participating MCPTT function;

then, when the SIP MESSAGE request contains:

1)	an application/vnd.3gpp.mcptt-mbms-usage-info+xml MIME body containing an <mbms-usage-info> element;

the participating MCPTT function shall send a SIP 403 (Forbidden) response as specified in 3GPP TS 24.229 [4].

## 14.3	MCPTT client MBMS usage procedures

### 14.3.1	General

This subclause describes the procedures in the MCPTT client for:

1)	receiving an MBMS bearer announcement from the participating MCPTT function; and

2)	sending an MBMS bearer listening status report to the participating MCPTT function.

### 14.3.2	Receiving an MBMS bearer announcement

When the MCPTT client receives a SIP MESSAGE request containing:

1)	a P-Asserted-Service header field containing the "urn:urn-7:3gpp-service.ims.icsi.mcptt"; and

2)	an application/vnd.3gpp.mcptt-mbms-usage-info+xml MIME body containing an <mbms-usage-info> element;

then the MCPTT client for each <announcement> element in the application/vnd.3gpp.mcptt-mbms-usage-info+xml MIME body:

1)	if the <mbms-service-areas> element contains a value different from "0":

a)	if an <announcement> element with the same value of the <TMGI> element is already stored:

i)	shall replace the old <announcement> element with the <announcement> element received in the application/vnd.3gpp.mcptt-mbms-usage-info+xml MIME body;

b)	if there is no <announcement> element with the same value of the <TMGI> element stored:

i)	shall store the received <announcement> element;

c)	shall associate the received announcement with the received application/sdp MIME body;

d)	shall associate the received announcement with the received <GPMS> element;

e)	shall store the MBMS public service identity of the participating MCPTT function received in the P-Asserted-Identity header field and associate the MBMS public service identity with the new <announcement> element;

f)	shall listen to the general purpose MBMS subchannel defined in the "m=application" media line in the application/sdp MIME body in the received SIP MESSAGE request when entering an MBMS service area where the announced MBMS bearer is available; and

g)	shall check the condition for sending a listening status report as specified in the subclause 14.3.3; and

2)	if the <mbms-service-areas> element contains a value equal to "0":

a)	shall discard a previously stored <announcement> element identified by the value of the <TMGI>;

b)	shall remove the association with the stored application/sdp MIME body and stop listening to the general purpose MBMS subchannel;

c)	 if no more <announcement> elements associated with the stored application/sdp MIME body are stored in the MCPTT client, shall remove the stored application/sdp MIME body; and

d)	check the condition for sending a listening status report as specified in the subclause 14.3.3.

### 14.3.3	The MBMS bearer listening status report procedure

#### 14.3.3.1	Conditions for sending an MBMS listening status report

If one of the following conditions is fulfilled:

1)	if the MCPTT client:

a)	receives a Map Group To Bearer message over the general purpose MBMS channel;

b)	participates in a group session identified by the Map Group To Bearer message; and

c)	the status "listening" is not already reported; or

2)	if the MCPTT client:

a)	receives an announcement as described in subclause 14.3.2;

b)	enters an MBMS service area where a general purpose MBMS is available; and

c)	experiences good MBMS bearer radio condition;

then the MCPTT client shall report that the MCPTT client is listening to the MBMS bearer as specified in subclause 14.3.3.2.

If one of the following conditions is fulfilled:

1)	if the MCPTT client:

a)	receives an MBMS bearer announcement as described in the subclause 14.3.2;

b)	the MBMS bearer announcement contains a cancellation of an <announcement> element identified by the same TGMI value as received in a Map Group To Bearer message in an ongoing conversation; and

c)	the status "not-listening" is not already reported;

2)	if the MCPTT client:

a)	receives an MBMS bearer announcement as described in the subclause 14.3.2;

b)	the MBMS bearer announcement contains a cancellation of an <announcement> element;

c)	does not participate in an ongoing conversation;

d)	the MCPTT client has reported the "listening" status due to the availability of the general purpose MBMS subchannel in the <announcement> element; and

e)	the status "not-listening" is not already reported; or

3.	if the MCPTT client:

a)	suffers from bad MBMS bearer radio condition, then the MCPTT client shall report that the MCPTT client is not listening to the MBMS subchannels as specified in subclause 14.3.3.2.

#### 14.3.3.2	Sending the MBMS bearer listening status report

When the MCPTT client wants to report the MBMS bearer listening status, the MCPTT client:

NOTE 1:	The application/vnd.3gpp.mcptt-mbms-usage-info+xml can contain both the listening status "listening" and "not listening" at the same time.

1)	shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [4] and IETF RFC 3428 [33]; and

2)	the SIP MESSAGE request:

a)	shall include in the Request-URI the MBMS public service identity of the participating MCPTT function received in the P-Asserted-Identity header field of the announcement message;

b)	shall include an Accept-Contact header field with the g.3gpp.icsi-ref media-feature tag with the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" along with parameters "require" and "explicit" according to IETF RFC 3841 [6];

c)	should include a public user identity in the P-Preferred-Identity header field as specified in 3GPP TS 24.229 [4];

d)	shall include a P-Preferred-Service header field with the value "urn:urn-7:3gpp-service.ims.icsi.mcptt";

e)	if the MCPTT client is listening to the MBMS bearer, the application/vnd.3gpp.mcptt-mbms-usage-info+xml MIME body:

i)	shall include an <mbms-listening-status> element set to "listening";

ii)	if the intention is to report that the MCPTT client is listening to the MBMS subchannel for an ongoing conversation in a session (e.g. as the response to the Map Group To Bearer message), shall include the MCPTT session identity of the ongoing conversation in <session-identity> element;

iii)	shall include one or more <TGMI> elements for which the listening status applies; and

iv)	if the intention is to report that the MCPTT client is listening to the general purpose MBMS subchannel, shall include the <general-purpose> element set to "true";

f)	if the MCPTT client is not listening, the application/vnd.3gpp.mcptt-mbms-usage-info+xml MIME body:

i)	shall include an <mbms-listening-status> element set to "not-listening";

iii)	shall include one or more <TGMI> elements for which the listening status applies;

iii)	if the intention is to report that the MCPTT client is no longer listening to the MBMS subchannel in an ongoing session (e.g. as the response to Unmap Group to Bearer message), shall include the MCPTT session identity in <session-identity> elements; and

iv)	if the intention is to report that the MCPTT client is no longer listening to general purpose MBMS subchannel, shall include the <general-purpose> element set to "false"; and

NOTE 2:	If the MCPTT client reports that the MCPTT client is no longer listening to the general purpose MBMS subchannel, it is implicitly understood that the MCPTT client no longer listens to any MBMS subchannel in ongoing conversations that the MCPTT client previously reported status "listening".

g)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcptt-request-uri> set to the MCPTT ID; and

3)	shall send the SIP MESSAGE request according to 3GPP TS 24.229 [4].

# 14A	MCPTT Service Continuity

## 14A.1	General

This clause describes the procedures for service continuity of an ongoing SIP session supporting an MCPTT private call or MCPTT group call when:

-	the MCPTT UE (referred to as the remote UE) is connected to the network via E-UTRAN and decides to connect to a UE-to-network relay, e.g. because it realises that it is losing connection to the network and wants to ensure seamless service; and

-	the remote UE is connected to the network via the UE-to-network-relay and decides to disconnect from the UE-to-network relay, e.g. because the remote UE realises that it is losing connection to UE-to-network relay or because the LTE-Uu link quality goes above a certain threshold, and decides to connect to the network via E-UTRAN for seamless service.

MCPTT service continuity follows the principles of 3GPP TS 24.237 [58] for PS-PS service continuity. In particular:

1)	the SIP session is anchored at a Service Centralisation and Continuity Application Server (SCC AS) before and after the handover. This requires that initial filter criteria is configured to ensure that the SCC AS is in the registration path, is the first application server in the path of an originating session, and the last AS in the path of a terminating session;

2)	the remote UE is an SC UE that supports PS-PS access transfer as per 3GPP TS 24.237 [58]; and

3)	the remote UE is either configured with a static PS to PS STI as specified in 3GPP TS 24.216 [66] that it uses when initiating the session transfer request, or it uses a dynamic PS to PS STI which is the URI contained in the Contact header field returned at the creation of the dialog over the Source Access Leg, as specified in 3GPP TS 24.237 [58].

## 14A.2	Service continuity from on-network MCPTT service to UE-to-network relay MCPTT service

### 14A.2.1	Remote UE

When performing service continuity from on-network MCPTT service to UE-to-network relay MCPTT service, the remote UE:

1)	shall perform ProSe UE-to-network relay discovery over PC5 as specified in clause 10A of 3GPP TS 24.334 [28];

NOTE 1:	Depending on the model (A or B) used for discovery as specified in 3GPP TS 24.334 [28], the remote UE can perform UE-to-network relay discovery while still in coverage (when model A is used), or while still in coverage if the LTE-Uu link quality drops below a certain threshold (when model B is used).

NOTE 2:	As part of the discovery process, service authorisation is performed as specified in 3GPP TS 24.334 [28]. The UE-to-network relay is provisioned with relay service code(s) associated with allowed MCPTT group(s) as specified in 3GPP TS 24.383 [45] and 3GPP TS 24.384 [50]. To find a permitted UE-to-network relay for group communications, a remote UE is provisioned with the relay service code(s) associated with the MCPTT group(s) which the MCPTT user is part of, in the MCPTT group configuration MO as specified in 3GPP TS 24.383 [45].

2)	shall select a suitable UE-to-network relay by performing the UE-to-network relay selection procedure specified in subclause 10A.2.12 of 3GPP TS 24.334 [28];3)	shall establish a direct link to the relay as specified in subclause 10.4.2 of 3GPP TS 24.334 [28];

NOTE 3:	As part of this process the remote UE is assigned a /64 IPv6 Prefix by the relay.

4)	shall initiate IMS registration over the UE-to-network relay target access leg by following the procedures in subclause 10.2.0 of 3GPP TS 24.237 [58];

NOTE 4:	As part of this process the remote UE needs to discover the P-CSCF address to connect to via the UE-to-network relay. The remote UE either uses mechanism I or mechanism III of subclause 9.2.1 in 3GPP TS 24.229 [4] to discover the P-CSCF address. The details of how mechanism I or mechanism III are used to discover the P-CSCF address are not covered by the present document.

5)	shall initiate session transfer by following the procedures specified in subclause 10.2.1 of 3GPP TS 24.237 [58];

6)	after successful session transfer if MCPTT content is being distributed on the target side using MBMS bearers, shall send a MBMS bearer listening status report procedure to the participating MCPTT function by performing the procedures in subclause 14.3.3; and

NOTE 5:	Upon receiving the MBMS bearer listening status from an MCPTT client indicating that the MCPTT UE is now listening to a MBMS subchannel, the participating MCPTT function performs the procedures in subclause 14.2.3 to switch to MBMS bearer.

7)	after successful session transfer if the remote UE still has an connection in the source access, may perform IMS de-registration of the contact address of the IMS public user identity registered on the source access leg by following the procedures in 3GPP TS 24.229 [4];

### 14A.2.2	SCC AS

The SCC AS follows the procedures in subclause 10.3.2 of 3GPP TS 24.237 [58].

## 14A.3	Service continuity from UE-to-network relay MCPTT service to on-network MCPTT service

### 14A.3.1	Remote UE

When performing access transfer between UE-to-network relay MCPTT service and on-network MCPTT service, the remote UE:

1)	shall initiate IMS registration over the on-network target access leg by following the procedures in subclause 10.2.0 of 3GPP TS 24.237 [58]; and

NOTE:	The remote UE uses option II procedures for P-CSCF discovery as defined in subclause L.2.2.1 of 3GPP TS 24.229 [4] to discover the P-CSCF address when connecting to EPC.

2)	follows the procedures in steps 5), 6) and 7) of subclause 14.A.2.1.

### 14A.3.2	SCC AS

The SCC AS follows the procedures in subclause 14A.2.2.

# 15 Off-network message formats

### 15.1	MONP message functional definitions and contents

### 15.1.1	General

The following subclauses describe the MONP message functional definitions and contents. Each message consist of a series of information elements. Annex I describes the standard format of a MONP message and the encoding rules for each type of information element.

### 15.1.2	GROUP CALL PROBE message

#### 15.1.2.1	Message definition

This message is sent by the UE to other UEs to check for an ongoing group call. For contents of the message see Table 15.1.2.1-1.

Message type:	GROUP CALL PROBE

Direction:			UE to other UEs

Table 15.1.2.1-1: GROUP CALL PROBE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Group call probe message identity | Message type 15.2.2 | M | V | 1 |
|  | MCPTT group ID | MCPTT group ID 15.2.5 | M | LV-E | 3-x |

### 15.1.3	GROUP CALL ANNOUNCEMENT message

#### 15.1.3.1	Message definition

This message is sent by the UE to other UEs to announce an ongoing group call to other UEs. For contents of the message see Table 15.1.3.1-1.

Message type:	GROUP CALL ANNOUNCEMENT

Direction:			UE to other UEs

Table 15.1.3.1-1: GROUP CALL ANNOUNCEMENT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Group call announcement message identity | Message type 15.2.2 | M | V | 1 |
|  | Call identifier | Call identifier 15.2.3 | M | V | 2 |
|  | Call type | Call type 15.2.11 | M | V | 1 |
|  | Refresh interval | Refresh interval 15.2.4 | M | V | 2 |
|  | Call start time | Call start time 15.2.14 | M | V | 5 |
|  | Last call type change time | Last call type change time 15.2.15 | M | V | 5 |
|  | MCPTT group ID | MCPTT group ID 15.2.5 | M | LV-E | 3-x |
|  | SDP | SDP 15.2.6 | M | LV-E | 3-x |
|  | Originating MCPTT user ID | MCPTT user ID 15.2.10 | M | LV-E | 3-x |
|  | Last user to change call type | MCPTT User ID 15.2.10 | M | LV-E | 3-x |
| 80 | Confirm mode indication | Confirm mode indication 15.2.9 | O | T | 1 |
| 81 | Probe response | Probe response 15.2.16 | O | T | 1 |

### 15.1.4	GROUP CALL ACCEPT message

#### 15.1.4.1	Message definition

This message is sent by the UE to other UEs to indicate acceptance of a group call. For contents of the message see Table 15.1.4.1-1.

Message type:	GROUP CALL ACCEPT

Direction:			UE to other UEs

Table 15.1.4.1-1: GROUP CALL ACCEPT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Group call accept message identity | Message type 15.2.2 | M | V | 1 |
|  | Call identifier | Call identifier 15.2.3 | M | V | 2 |
|  | Call type | Call type 15.2.11 | M | V | 1 |
|  | MCPTT group ID | MCPTT group ID 15.2.5 | M | LV-E | 3-x |
|  | Sending MCPTT user ID | MCPTT user ID 15.2.10 | M | LV-E | 3-x |

### 15.1.5	PRIVATE CALL SETUP REQUEST message

#### 15.1.5.1	Message definition

This message is sent by a UE to another UE to request setup of a private call. For contents of the message see Table 15.1.5.1-1.

Message type:	PRIVATE CALL SETUP REQUEST

Direction:			UE to another UE

Table 15.1.5.1-1: PRIVATE CALL SETUP REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Private call setup request message identity | Message type 15.2.2 | M | V | 1 |
|  | Call identifier | Call identifier 15.2.3 | M | V | 2 |
|  | Commencement mode | Commencement mode 15.2.7 | M | V | 1 |
|  | Call type | Call type 15.2.11 | M | V | 1 |
|  | MCPTT user ID of the caller | MCPTT user ID 15.2.10 | M | LV-E | 3-x |
|  | MCPTT user ID of the callee | MCPTT user ID 15.2.10 | M | LV-E | 3-x |
|  | SDP offer | SDP 15.2.6 | M | LV-E | 3-x |
| 78 | User location | User location 15.2.12 | O | TLV-E | 3-x |

### 15.1.6	PRIVATE CALL RINGING message

#### 15.1.6.1	Message definition

This message is automatically sent by a UE to another UE in response to a PRIVATE CALL SETUP REQUEST message. This message indicates that the UE has presented the incoming call notification to the user and is awaiting user response. For contents of the message see Table 15.1.6.1-1.

Message type:	PRIVATE CALL RINGING

Direction:			UE to another UE

Table 15.1.6.1-1: PRIVATE CALL RINGING message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Private call ringing message identity | Message type 15.2.2 | M | V | 1 |
|  | Call identifier | Call identifier 15.2.3 | M | V | 2 |
|  | MCPTT user ID of the caller | MCPTT user ID 15.2.10 | M | LV-E | 3-x |
|  | MCPTT user ID of the callee | MCPTT user ID 15.2.10 | M | LV-E | 3-x |

### 15.1.7	PRIVATE CALL ACCEPT message

#### 15.1.7.1	Message definition

This message is sent by a UE to another UE in response to a PRIVATE CALL SETUP REQUEST message when user accepts the call. This message indicates that the UE accepts the call setup request. For contents of the message see Table 15.1.7.1-1.

Message type:	PRIVATE CALL ACCEPT

Direction:			UE to another UE

Table 15.1.7.1-1: PRIVATE CALL ACCEPT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Private call accept message identity | Message type 15.2.2 | M | V | 1 |
|  | Call identifier | Call identifier 15.2.3 | M | V | 2 |
|  | MCPTT user ID of the caller | MCPTT user ID 15.2.10 | M | LV-E | 3-x |
|  | MCPTT user ID of the callee | MCPTT user ID 15.2.10 | M | LV-E | 3-x |
|  | SDP answer | SDP 15.2.6 | M | LV-E | 3-x |

### 15.1.8	PRIVATE CALL REJECT message

#### 15.1.8.1	Message definition

This message is sent by a UE to another UE in response to a PRIVATE CALL SETUP REQUEST message when user rejects the call. This message indicates that the UE rejects the call setup request. For contents of the message see Table 15.1.8.1-1.

Message type:	PRIVATE CALL REJECT

Direction:			UE to another UE

Table 15.1.8.1-1: PRIVATE CALL REJECT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Private call reject message identity | Message type 15.2.2 | M | V | 1 |
|  | Call identifier | Call identifier 15.2.3 | M | V | 2 |
|  | Reason | Reason 15.2.8 | M | V | 1 |
|  | MCPTT user ID of the caller | MCPTT user ID 15.2.10 | M | LV-E | 3-x |
|  | MCPTT user ID of the callee | MCPTT user ID 15.2.10 | M | LV-E | 3-x |

### 15.1.9	PRIVATE CALL RELEASE message

#### 15.1.9.1	Message definition

This message is sent by a UE to another UE to terminate an ongoing private call. For contents of the message see Table 15.1.9.1-1.

Message type:	PRIVATE CALL RELEASE

Direction:			UE to another UE

Table 15.1.9.1-1: PRIVATE CALL RELEASE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Private call release message identity | Message type 15.2.2 | M | V | 1 |
|  | Call identifier | Call identifier 15.2.3 | M | V | 2 |
|  | MCPTT user ID of the caller | MCPTT user id 15.2.10 | M | LV-E | 3-x |
|  | MCPTT user ID of the callee | MCPTT user id 15.2.10 | M | LV-E | 3-x |

### 15.1.10	PRIVATE CALL RELEASE ACK message

#### 15.1.10.1	Message definition

This message is sent by a UE to another UE in response to a PRIVATE CALL RELEASE message. This message indicates that the UE has terminated the call. For contents of the message see Table 15.1.10.1-1.

Message type:	PRIVATE CALL RELEASE ACK

Direction:			UE to another UE

Table 15.1.10.1-1: PRIVATE CALL RELEASE ACK message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Private call release ack message identity | Message type 15.2.2 | M | V | 1 |
|  | Call identifier | Call identifier 15.2.3 | M | V | 2 |
|  | MCPTT user ID of the caller | MCPTT user id 15.2.10 | M | LV-E | 3-x |
|  | MCPTT user ID of the callee | MCPTT user ID 15.2.10 | M | LV-E | 3-x |

### 15.1.11	PRIVATE CALL ACCEPT ACK message

#### 15.1.11.1	Message definition

This message is sent by a UE to another UE in response to a PRIVATE CALL ACCEPT message. This message acknowledges the receipt of PRIVATE CALL ACCEPT message. For contents of the message see Table 15.1.11.1-1.

Message type:	PRIVATE CALL ACCEPT ACK

Direction:			UE to another UE

Table 15.1.11.1-1: PRIVATE CALL ACCEPT ACK message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Private call accept ack message identity | Message type 15.2.2 | M | V | 1 |
|  | Call identifier | Call identifier 15.2.3 | M | V | 2 |
|  | MCPTT user ID of the caller | MCPTT user ID 15.2.10 | M | LV-E | 3-x |
|  | MCPTT user ID of the callee | MCPTT user ID 15.2.10 | M | LV-E | 3-x |

### 15.1.12	PRIVATE CALL EMERGENCY CANCEL message

#### 15.1.12.1	Message definition

This message is sent by a UE to another UE to indicate termination of emergency mode in private call. For contents of the message see Table 15.1.12.1-1.

Message type:	PRIVATE CALL EMERGENCY CANCEL

Direction:			UE to another UE

Table 15.1.12.1-1: PRIVATE CALL EMERGENCY CANCEL message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Private call emergency cancel message identity | Message type 15.2.2 | M | V | 1 |
|  | Call identifier | Call identifier 15.2.3 | M | V | 2 |
|  | MCPTT user ID of the caller | MCPTT user ID 15.2.10 | M | LV-E | 3-x |
|  | MCPTT user ID of the callee | MCPTT user ID 15.2.10 | M | LV-E | 3-x |

### 15.1.13	PRIVATE CALL EMERGENCY CANCEL ACK message

#### 15.1.13.1	Message definition

This message is sent by a UE to another UE to indicate receipt of PRIVATE CALL EMERGENCY CANCEL message. For contents of the message see Table 15.1.13.1-1.

Message type:	PRIVATE CALL EMERGENCY CANCEL ACK

Direction:			UE to another UE

Table 15.1.13.1-1: PRIVATE CALL EMERGENCY CANCEL ACK message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Private call emergency cancel ack message identity | Message type 15.2.2 | M | V | 1 |
|  | Call identifier | Call identifier 15.2.3 | M | V | 2 |
|  | MCPTT user ID of the caller | MCPTT user ID 15.2.10 | M | LV-E | 3-x |
|  | MCPTT user ID of the callee | MCPTT user ID 15.2.10 | M | LV-E | 3-x |

### 15.1.14	GROUP CALL IMMINENT PERIL END message

#### 15.1.14.1	Message definition

This message is sent by the UE to other UEs to indicate termination of imminent peril mode in the group call. For contents of the message see Table 15.1.14.1-1.

Message type:	GROUP CALL IMMINENT PERIL END

Direction:			UE to other UEs

Table 15.1.14.1-1: GROUP CALL IMMINENT PERIL END message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Group call imminent peril end message identity | Message type 15.2.2 | M | V | 1 |
|  | Call identifier | Call identifier 15.2.3 | M | V | 2 |
|  | Last call type change time | Last call type change time 15.2.16 | M | V | 5 |
|  | Last user to change call type | MCPTT User ID 10.2.4.10 | M | LV-E | 3-x |
|  | MCPTT group ID | MCPTT group ID 15.2.5 | M | LV-E | 3-x |
|  | Originating MCPTT user ID | MCPTT user ID 15.2.10 | M | LV-E | 3-x |

### 15.1.15	GROUP CALL EMERGENCY END message

#### 15.1.15.1	Message definition

This message is sent by the UE to other UEs to indicate termination of emergency mode in the group call. For contents of the message see Table 15.1.15.1-1.

Message type:	GROUP CALL EMERGENCY END

Direction:			UE to other UEs

Table 15.1.15.1-1: GROUP CALL EMERGENCY END message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Group call emergency end message identity | Message type 15.2.2 | M | V | 1 |
|  | Call identifier | Call identifier 15.2.3 | M | V | 2 |
|  | Last call type change time | Last call type change time 15.2.16 | M | V | 5 |
|  | Last user to change call type | MCPTT User ID 10.2.4.10 | M | LV-E | 3-x |
|  | MCPTT group ID | MCPTT group ID 15.2.5 | M | LV-E | 3-x |
|  | Originating MCPTT user ID | MCPTT user ID 15.2.10 | M | LV-E | 3-x |

### 15.1.16	GROUP EMERGENCY ALERT message

#### 15.1.16.1	Message definition

This message is sent by the UE to other UEs to indicate an emergency situation. For contents of the message see Table 15.1.16.1-1.

Message type:	GROUP EMERGENCY ALERT

Direction:			UE to other UEs

Table 15.1.16.1-1: GROUP EMERGENCY ALERT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Group emergency alert message identity | Message type 15.2.2 | M | V | 1 |
|  | MCPTT group ID | MCPTT group ID 15.2.5 | M | LV-E | 3-x |
|  | Originating MCPTT user ID | MCPTT user ID 15.2.10 | M | LV-E | 3-x |
|  | Organization name | Organization name 15.2.13 | M | LV-E | 3-x |
| 78 | User location | User location 15.2.12 | O | TLV-E | 4-x |

### 15.1.17	GROUP EMERGENCY ALERT ACK message

#### 15.1.17.1	Message definition

This message is sent by the UE to other UEs to indicate receipt of emergency alert. For contents of the message see Table 15.1.17.1-1.

Message type:	GROUP EMERGENCY ALERT ACK

Direction:			UE to other UEs

Table 15.1.17.1-1: GROUP EMERGENCY ALERT ACK message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Group emergency alert ack message identity | Message type 15.2.2 | M | V | 1 |
|  | MCPTT group ID | MCPTT group ID 15.2.5 | M | LV-E | 3-x |
|  | Originating MCPTT user ID | MCPTT user ID 15.2.10 | M | LV-E | 3-x |
|  | Sending MCPTT user ID | MCPTT user ID 15.2.10 | M | LV-E | 3-x |

### 15.1.18	GROUP EMERGENCY ALERT CANCEL message

#### 15.1.18.1	Message definition

This message is sent by the UE to other UEs to indicate end of emergency situation. For contents of the message see Table 15.1.18.1-1.

Message type:	GROUP EMERGENCY ALERT CANCEL

Direction:			UE to other UEs

Table 15.1.18.1-1: GROUP EMERGENCY ALERT CANCEL message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Group emergency alert cancel 0message identity | Message type 15.2.2 | M | V | 1 |
|  | MCPTT group ID | MCPTT group ID 15.2.5 | M | LV-E | 3-x |
|  | Originating MCPTT user ID | MCPTT User ID 15.2.10 | M | LV-E | 3-x |
|  | Sending MCPTT user ID | MCPTT user ID 15.2.10 | M | LV-E | 3-x |

### 15.1.19	GROUP EMERGENCY ALERT CANCEL ACK message

#### 15.1.19.1	Message definition

This message is sent by the UE to other UEs to indicate receipt of emergency alert cancel. For contents of the message see Table 15.1.19.1-1.

Message type:	GROUP EMERGENCY ALERT CANCEL ACK

Direction:			UE to other UEs

Table 15.1.19.1-1: GROUP EMERGENCY ALERT CANCEL ACK message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Group emergency alert cancel ack message identity | Message type 15.2.2 | M | V | 1 |
|  | MCPTT group ID | MCPTT group ID 15.2.5 | M | LV-E | 3-x |
|  | Originating MCPTT user ID | MCPTT User ID 15.2.10 | M | LV-E | 3-x |
|  | Sending MCPTT user ID | MCPTT user ID 15.2.10 | M | LV-E | 3-x |

### 15.1.20	GROUP CALL BROADCAST message

#### 15.1.20.1	Message definition

This message is sent by the UE to other UEs to announce a broadcast group call to other UEs. For contents of the message see Table 15.1.20.1-1.

Message type:	GROUP CALL BROADCAST

Direction:			UE to other UEs

Table 15.1.20.1-1: GROUP CALL BROADCAST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Group call broadcast message identity | Message type 15.2.2 | M | V | 1 |
|  | Call identifier | Call identifier 15.2.3 | M | V | 2 |
|  | Call type | Call type 15.2.11 | M | V | 1 |
|  | Originating MCPTT user ID | MCPTT user ID 15.2.10 | M | LV-E | 3-x |
|  | MCPTT group ID | Group ID 15.2.5 | M | LV-E | 3-x |
|  | SDP | SDP 15.2.6 | M | LV-E | 3-x |

### 15.1.21	GROUP CALL BROADCAST END message

#### 15.1.21.1	Message definition

This message is sent by the UE to other UEs to indicate termination of a broadcast group call. For contents of the message see Table 15.1.21.1-1.

Message type:	GROUP CALL BROADCAST END

Direction:			UE to other UEs

Table 15.1.21.1-1: GROUP CALL BROADCAST END message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Group call broadcast end message identity | Message type 15.2.2 | M | V | 1 |
|  | Call identifier | Call identifier 15.2.3 | M | V | 2 |
|  | MCPTT group ID | MCPTT group ID 15.2.5 | M | LV-E | 3-x |
|  | Originating MCPTT user ID | MCPTT user ID 15.2.10 | M | LV-E | 3-x |

## 15.2	General message format and information elements coding

### 15.2.1	General

The least significant bit of a field is represented by the lowest numbered bit of the highest numbered octet of the field. When the field extends over more than one octet, the order of bit values progressively decreases as the octet number increases.

Figure 15.2.1-1 shows an example of a field where the most significant bit of the field is marked MSB and the least significant bit of the field is marked LSB.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MSB | x | x | x | x | x | x | x | octet 1 |
| x | x | x | x | x | x | x | x |  |
| x | x | x | x | x | x | x | LSB | octet N |

Figure 15.2.1-1: Example of bit ordering of a field

Within the protocols defined in the present document, the message consists of the following parts:

a)	message type information element; and

b)	other information elements, as required.

The organization of a message is illustrated in the example shown in Figure 15.2.1-2.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message type |  |  |  |  |  |  |  | octet 1 |
|  |  |  |  |  |  |  |  | octet 2 |
| Other information elements as required |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | octet n |

Figure 15.2.1-2: General message organization example

Unless specified otherwise in the message descriptions of subclause 15.1, a particular information element shall not be present more than once in a given message.

The sending entity shall set value of a spare bit to zero. The receiving entity shall ignore value of a spare bit

The sending entity shall not set a value of an information element to a reserved value. The receiving entity shall discard message containing an information element set to a reserved value.

### 15.2.2	Message type

The purpose of the Message type information element is to identify the type of the message.

The value part of the Message type information element is coded as shown in Table 15.2.2-1.

The Message type information element is a type 3 information element with a length of 1 octet.

Table 15.2.2-1: Message types

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |  | GROUP CALL PROBE |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |  | GROUP CALL ANNOUNCEMENT |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |  | GROUP CALL ACCEPT |
| 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 |  | GROUP CALL EMERGENCY END |
| 0 | 0 | 0 | 0 | 0 | 1 | 0 | 1 |  | GROUP CALL IMMINENT PERIL END |
| 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 |  | GROUP CALL BROADCAST |
| 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 |  | GROUP CALL BROADCAST END |
| 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 |  | PRIVATE CALL SETUP REQUEST |
| 0 | 0 | 0 | 0 | 1 | 0 | 0 | 1 |  | PRIVATE CALL RINGING |
| 0 | 0 | 0 | 0 | 1 | 0 | 1 | 0 |  | PRIVATE CALL ACCEPT |
| 0 | 0 | 0 | 0 | 1 | 0 | 1 | 1 |  | PRIVATE CALL REJECT |
| 0 | 0 | 0 | 0 | 1 | 1 | 0 | 0 |  | PRIVATE CALL RELEASE |
| 0 | 0 | 0 | 0 | 1 | 1 | 0 | 1 |  | PRIVATE CALL RELEASE ACK |
| 0 | 0 | 0 | 0 | 1 | 1 | 1 | 0 |  | PRIVATE CALL ACCEPT ACK |
| 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |  | PRIVATE EMERGENCY CALL CANCEL |
| 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 |  | PRIVATE EMERGENCY CALL CANCEL ACK |
| 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 |  | GROUP EMERGENCY ALERT |
| 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 |  | GROUP EMERGENCY ALERT ACK |
| 0 | 0 | 0 | 1 | 0 | 0 | 1 | 1 |  | GROUP EMERGENCY ALERT CANCEL |
| 0 | 0 | 0 | 1 | 0 | 1 | 0 | 0 |  | GROUP EMERGENCY ALERT CANCEL ACK |
| All other values are reserved. |  |  |  |  |  |  |  |  |  |

### 15.2.3	Call identifier

The purpose of the Call identifier information element is to uniquely identify the call.

The Call identifier information element is coded as shown in Figure 15.2.3-1 and Table 15.2.3-1.

The Call identifier information element is a type 3 information element with a length of 2 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Call identifier |  |  |  |  |  |  |  | octet 1 |
|  |  |  |  |  |  |  |  | octet 2 |

Figure 15.2.3-1: Call identifier information element

Table 15.2.3-1: Call identifier information element

Call identifier value (octet 1 to 2) The Call identifier contains a number uniquely identifying the call.

### 15.2.4	Refresh interval

The refresh interval information identifier is used to indicate the minimum time period between successive periodic messages;

The Refresh interval information element is coded as shown in Figure 15.2.4-1 and Table 15.2.4-1.

The Refresh interval information element is a type 3 information element with a length of 2 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Refresh interval value |  |  |  |  |  |  |  | Octet 1 Octet 2 |

Figure 15.2.4-1: Refresh interval

Table 15.2.4-1: Refresh interval information element

Refresh interval value (octet 1 to 2) The Refresh interval contains a number denoting the minimum time interval (milliseconds) between two successive periodic announcements.

### 15.2.5	MCPTT group ID

The MCPTT group ID information element is used to indicate the destination MCPTT group identifier;

The MCPTT group ID information element is coded as shown in Figure 15.2.5-1 and Table 15.2.5-1.

The MCPTT group ID information element is a type 6 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of MCPTT group ID contents |  |  |  |  |  |  |  | octet 1 |
|  |  |  |  |  |  |  |  | octet 2 |
|  |  |  |  |  |  |  |  | octet 3 |
| MCPTT group ID contents |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | octet n |

Figure 15.2.5-1: MCPTT group ID information element

Table 15.2.5-1: MCPTT group ID information element

MCPTT group ID is contained in octet 3 to octet n; Max value of 65535 octets.

### 15.2.6	SDP

The purpose of the SDP information element is to contain SDP message.

The SDP information element is coded as shown in Figure 15.2.6-1 and Table 15.2.6-1.

The SDP information element is a type 6 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of SDP contents |  |  |  |  |  |  |  | octet 1 |
|  |  |  |  |  |  |  |  | octet 2 |
|  |  |  |  |  |  |  |  | octet 3 |
| SDP contents |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | octet n |

Figure 15.2.6-1: SDP information element

Table 15.2.6-1: SDP information element

SDP message container contents (octet 3 to octet n); Max value of 65535 octets.

This information element contains SDP message as defined in Section 10.2.1.1.2 or 11.2.1.1.2.

### 15.2.7	Commencement mode

The purpose of the Commencement mode information element is to identify the type of the commencement mode of the private call.

The value part of the Commencement mode information element is coded as shown in Table 15.2.7-1.

The Commence mode information element is a type 3 information element with a length of TBD octet.

Table 15.2.7-1: Commencement mode

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | AUTOMATIC COMMENCEMENT MODE |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |  | MANUAL COMMENCEMENT MODE |
| All other values are reserved. |  |  |  |  |  |  |  |  |  |

### 15.2.8	Reason

The purpose of the Reason information element is to indicate the reason of the reject.

The Reason information element is coded as shown in figure 15.2.8-1.

The Reason information element is a type 3 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reason IEI |  |  |  |  |  |  |  | octet 1 |

Figure 15.2.8-1: Reason information element

Table 15.2.8-1: Reason type

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | REJECT |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |  | MEDIA FAILURE |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |  | BUSY |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |  | E2E SECURITY CONTEXT FAILURE |
| 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 |  | FAILED |
| All other values are reserved. |  |  |  |  |  |  |  |  |  |

### 15.2.9	Confirm mode indication

The purpose of the Confirm mode indication information element is to indicate that the terminating MCPTT client is expected to confirm call acceptance.

The Confirm mode indication information element is coded as shown in figure 15.2.9-1.

The Confirm mode indication information element is a type 2 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Confirm mode indication IEI |  |  |  |  |  |  |  | octet 1 |

Figure 15.2.9-1: Confirm mode indication information element

### 15.2.10	MCPTT user ID

The MCPTT user ID information element is used to indicate an MCPTT user ID.

The MCPTT user ID information element is coded as shown in Figure 15.2.10-1 and Table 15.2.10-1.

The MCPTT user ID information element is a type 6 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of MCPTT user ID contents |  |  |  |  |  |  |  | octet 1 |
|  |  |  |  |  |  |  |  | octet 2 |
|  |  |  |  |  |  |  |  | octet 3 |
| MCPTT user ID contents |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | octet n |

Figure 15.2.10-1: MCPTT user ID information element

Table 15.2.10-1: MCPTT user ID information element

MCPTT user ID is contained in octet 3 to octet n; Max value of 65535 octets.

### 15.2.11	Call type

The purpose of the Call type information element is to identify the type of the call.

The value part of the Call type information element is coded as shown in Table 15.2.11-1.

The Call type information element is a type 3 information element with a length of 1 octet.

Table 15.2.11-1: Call type

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |  | BASIC GROUP CALL |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |  | BROADCAST GROUP CALL |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |  | EMERGENCY GROUP CALL |
| 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 |  | IMMINENT PERIL GROUP CALL |
| 0 | 0 | 0 | 0 | 0 | 1 | 0 | 1 |  | PRIVATE CALL |
| 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 |  | EMERGENCY PRIVATE CALL |
| All other values are reserved. |  |  |  |  |  |  |  |  |  |

### 15.2.12	User location

The User location information element is used to indicate the current location of the MCPTT client;

The User location information element is coded as shown in Figure 15.2.12-1 and Table 15.2.12-1.

The User location information element is a type 6 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| User location IEI |  |  |  |  |  |  |  | octet 1 |
| Length of User location contents |  |  |  |  |  |  |  | octet 2 |
|  |  |  |  |  |  |  |  | octet 3 |
|  |  |  |  |  |  |  |  | octet 4 |
| User location contents |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | octet n |

Figure 15.2.12-1: User location information element

Table 15.2.12-1: User location information element

User location is contained in octet 4 to octet n; Max value of 65535 octets.

The User location information element contains the LocationInfo structure defined in subclause 7.4 of 3GPP TS 29.199-9 [59].

### 15.2.13	Organization name

The Organization name information element is used to indicate the name of the organization to which the user belongs.

The Organization name information element is coded as shown in Figure 15.2.13-1 and Table 15.2.13-1.

The Organization name information element is a type 6 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of Organization name contents |  |  |  |  |  |  |  | octet 1 |
|  |  |  |  |  |  |  |  | octet 2 |
|  |  |  |  |  |  |  |  | octet 3 |
| Organization name contents |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | octet n |

Figure 15.2.13-1: Organization name information element

Table 15.2.13-1: Organization name information element

Organization name is contained in octet 3 to octet n; Max value of 65535 octets.

### 15.2.14	Call start time

The Call start time information element is used to indicate the UTC time when a call was started.

The Call start time information element is coded as shown in Figure 15.2.14-1 and Table 15.2.14-1.

The Call start time information element is a type 3 information element with a length of 5 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Call start time value |  |  |  |  |  |  |  | Octet 1 Octet 5 |

Figure 15.2.14-1: Call start time value

Table 15.2.14-1: Call start time value

Call start time value (octet 1 to 5) The Call start time value is an unsigned integer containing UTC time of the time when a call was started, in seconds since midnight UTC of January 1, 1970 (not counting leap seconds).

### 15.2.15	Last call type change time

The Last call type change time information identifier is used to indicate the last UTC time when a call priority was changed.

The Last call type change time information element is coded as shown in Figure 15.2.15-1 and Table 15.2.15-1.

The Last call type change time information element is a type 3 information element with a length of 5 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Call start time value |  |  |  |  |  |  |  | Octet 1 Octet 5 |

Figure 15.2.15-1: Call start time value

Table 15.2.15-1: Call start time value

Last call type change time (octet 1 to 5) The Last call type change time value is an unsigned integer containing UTC time of the time when a call priority was changed, in seconds since midnight UTC of January 1, 1970 (not counting leap seconds).

### 15.2.16	Probe response

The purpose of the probe response information element is to indicate that the GROUP CALL ANNOUNCEMENT message was sent in response of a GROUP CALL PROBE message.

The probe response information element is coded as shown in figure 15.2.16-1.

The probe response is a type 2 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Probe Response IEI |  |  |  |  |  |  |  | octet 1 |

Figure 15.2.16-1: Probe response information element

Annex A (informative):
Signalling flows

# A.1	Group regrouping flow

## A.1.1	General

This clause describes how a temporary group call as the result of a group regroup operation is established.

## A.1.2	Use case description

The police and a private security company cooperate during a fair. A temporary prearranged group is created as specified in TS 24.381 [31] to allow the policemen and security personal to communicate.

The temporary group consists of two groups of policemen and a group of security personal from the private security company. One of the groups of policemen is called in from another town. A dispatcher from the police creates the temporary group. The dispatcher is added to the group of policemen hosted by the controlling MCPTT function A.

Table A.1.2-1 shows the group coding details.

Table A.1.2-1:	Group coding details

| MCPTT group identity | Hosted by | Group type | MCPTT group members |
| --- | --- | --- | --- |
| mcptt-group-A-B@mcptt-op.gov | controlling MCPTT function A | Temporary group | mcptt-group-A1@mcptt-op.gov |
|  |  |  | mcptt-group-B@mcptt-city1.net |
|  |  |  | mcptt-group-A2@mcptt-op.gov |
| mcptt-group-A1@mcptt-op.gov | controlling MCPTT function A | Prearranged group | mcptt-id-A2@mcptt-op.gov |
| mcptt-group-B@mcptt-city1.net | non-controlling MCPTT function B | Prearranged group | mcptt-id-B@mcptt-city1.net |
| mcptt-group-A2@mcptt-op.gov | non-controlling MCPTT function A | Prearranged group | mcptt-id-A3@mcptt-op.gov |

For readability reasons all prearranged groups have only one member. Table A.1.2-1 shows the group member coding details.

Table A.1.2-2:	Group member coding details

| MCPTT ID of group member |  | Registered public user identity | MCPTT client name | IP address audio / floor control port number (NOTE 2) |
| --- | --- | --- | --- | --- |
| mcptt-id-A1@mcptt-op.gov |  | sip:userA1@ims-op.net | MCPTT client A1 | 5555::aaa:bbb:ccc:eee 3456 / 3457 |
| mcptt-id-A2@mcptt-op.gov |  | sip:userA2@ims-op.net | MCPTT client A2 | 5555::aaa:ccc:aaa:bbb 26456 / 26457 (NOTE 1) |
| mcptt-id-A3@mcptt-op.gov |  | sip:userA3@ims-op.net | MCPTT client A3 | 5555::bbb:aaa:ccc:aaa 25644 / 25645 |
| mcptt-id-B@mcptt-city1.net |  | sip:userB@ims-op.net | MCPTT client B | 5555::baa:abb:ddd:ccc 62122 / 62122 |
| NOTE 1: A pre-established session is used and the IP address and port numbers of the participating MCPTT function A2 is used. NOTE 2: The IP address is the same as for the registered contact. |  |  |  |  |

Each MCPTT user is served by a different MCPTT function. Table A.1.2-3 shows the Participating MCPTT functions coding details.

Table A.1.2-3:	Participating MCPTT functions coding details

| Functional entity | Public service identity | IP address audio / floor control port number |
| --- | --- | --- |
| participating MCPTT function A1 | sip:pf-A1.ims-op.net | 5555::aaa:bbb:ccc:eef 7890 / 7891 |
| participating MCPTT function A2 | sip:pf-A2.ims-op.net | 5555::aaa:ccc:aaa:bbb 26456 / 26457 |
| participating MCPTT function A3 | sip:pf-A3.ims-op.net | 5555::aaa:ccc:aaa:ccc 18412 / 18413 |
| participating MCPTT function B | sip:pf-B.ims-op.net | 5555::aaa:bbb:ccc:eef 16412 / 16413 |

Each group is hosted by a different MCPTT server. Table A.1.2-4 shows the Controlling and non-controlling MCPTT functions coding details.

Table A.1.2-4:	Controlling and non-controlling MCPTT functions coding details

| Functional entity | Public service identity | MCPTT session identifier | IP address audio / floor control port number |
| --- | --- | --- | --- |
| controlling MCPTT function A | sip:cf-A.ims-op.net | session@cf-A@ims-op.net | 5555::aaa:bbb:ddd:aaa 23124 / 23125 |
| non-controlling MCPTT function A | sip:cf-A2.ims-op.net | sessionA@cf-A2@ims-op.net | 5555::aaa:bbb:ddd:eee 12344 / 12345 |
| non-controlling MCPTT function B | sip:cf-B.ims-op.net | sessionB@cf-B@ims-op.net | 5555::aaa:bbb:ddd:ddd 34456 / 34457 |

## A.1.3	Signalling flow

The temporary group consists of a group A1 (police), group A2 (police in other town) and a group B (security company in a partner system).

For the mcptt-id-B@mcptt-city1.net the controlling MCPTT function is using the connection model in figure 5.3.2-5.

For the mcptt-id-A3@mcptt-op.gov the controlling MCPTT function is using the connection model in figure 5.3.2-2.

Preconditions:

1)	the temporary group mcptt-group-A-B is already created and all members are affiliated to the group;

2)	this is not an emergency or imminent peril call;

3)	MCPTT client A2 has a pre-established session and the MCPTT services setting for the commencement mode is auto-answer;

4)	MCPTT client A3 has no pre-established session and the MCPTT services setting for the commencement mode is manual-answer;

5)	MCPTT client has no pre-established session and the MCPTT services setting for queueing the commencement mode is auto-answer;

6)	the IMS service provider (ims-op.net) is the same for all groups;

7)	the flow does not consider confidentiality and integrity protection; and

8)	non-controlling model used between the Primary and the partner is the "untrusted" model.

Figure A.1.3-1 shows the signalling flow when the controlling MCPTT function invites the prearranged groups consisting in the temporary group.

Figure A.1.3-1: Temporary prearranged group call setup

The steps of the flow are as follows:

NOTE 1:	The parameters in the coding parts are only those specified in the normative part of the present document. For information about other parameters, see 3GPP TS 24.229 [4].

1)	SIP INVITE request (MCPTT client A1 to participating MCPTT function A1) - see example in table A.1.3-1

Upon receiving a request from an MCPTT user A1 to establish an MCPTT temporary group session the MCPTT client A1 sends a SIP INVITE request towards the participating MCPTT function A1 according to 3GPP TS 24.229 [4].

Table A.1.3-1: SIP INVITE request (MCPTT client A1 to participating MCPTT function A1)

INVITE sip: pf-A1.ims-op.net SIP/2.0

Accept-Contact: *;+g.3gpp.mcptt;require;explicit,+g.3gpp.icsi-ref="urn%3Aurn-7%3A3gpp-service.ims.icsi.mcptt;require;explicit

P-Preferred-Service:urn:urn-7:3gpp-service.ims.icsi.mcptt

Supported: timer

Session-Expires: 3600;refresher=uac

Contact: <sip:[5555::aaa:bbb:ccc:eee]>;g.3gpp.mcptt;g.3gpp.icsi-ref=urn%3Aurn-7%3A3gpp-service.ims.icsi.mcptt

P-Preferred-Identity:<sip:userA1@ims-op.net>

Content-Type: multipart/mixed;boundary="boundary1"

--boundary1

Content-Type: application/sdp

Content-Length: (…)

c=IN IP6 5555::aaa:bbb:ccc:eee

m=audio 3456 RTP/AVP 97

a=rtpmap:97 AMR

a=fmtp:97 mode-set=0,2,5,7; mode-change-period=2

a=maxptime:20

i=speech

m=application 3457 udp mcptt fmtp

a=fmtp:MCPTT mc_queueing;mc_implicit_request

--boundary1

Content-Type: application/vnd.3gpp.mcptt-info+xml

Content-Length: (…)

<?xml version="1.0"?>

<mcpttinfo>

<mcptt-Params>

<session-type>prearranged</>

<mcptt-request-uri>mcptt-group-T@mcptt-op.gov</>

</mcptt-Params>

</mcpttinfo>

--boundary1—

Request-URI:	Contains the public service identity of the participating MCPTT function serving the MCPTT user.

Accept-Contact:	Contains the g.3gpp.mcptt feature tag.

P-Preferred-Service:	Contains the ICSI for MCPTT.

Contact:	Contains the registered contact and the g.3gpp.mcptt feature tag.

P-preferred-Identity:	Contains the public user identity of the MCPTT user.

mcptt-info:	The application/vnd.3gpp.mcptt-info+xml contains the <mcptt-Params> element with the <session-type> element set to "prearranged" and the <mcptt-request-uri> element set to "mcptt-group-T@mcptt-op.gov".

SDP offer:	Contains a media-level section for the MCPTT speech media stream and a media-level section for the media-floor control entity indicating that  of floor requests is supported ("mc:queueing") and that the SIP INVITE request is an implicit floor request ("mc_implicit_request").

2)	SIP INVITE request (participating MCPTT function A1 to controlling MCPTT function A) - see example in table A.1.3-2

Upon receiving of a "SIP INVITE request for originating participating MCPTT function" containing an application/vnd.3gpp.mcptt-info+xml MIME body with the <session-type> element set to a value of "prearranged", the participating MCPTT function authorizes the MCPTT user to initiate the prearranged group call and since the MCPTT user is authorized, the participating MCPTT function A1 forwards the SIP INVITE request according to 3GPP TS 24.229 [4].

Table A.1.3-2: SIP INVITE request (participating MCPTT function A to controlling MCPTT function A)

INVITE sip: cf-A.ims-op.net SIP/2.0

Accept-Contact:

P-Asserted-Service:

Supported:

Session-Expires:

Contact:

P-Asserted-Identity:<sip:userA1@ims-op.net>

Content-Type: multipart/mixed;boundary="boundary1"

--boundary1

Content-Type: application/sdp

Content-Length: (…)

c=IN IP6 5555::aaa:bbb:ccc:eef

m=audio 7890 RTP/AVP 97

a=

a=

i=

m=application 7891 udp mcptt

a=

--boundary1

Content-Type: application/vnd.3gpp.mcptt-info+xml

Content-Length: (…)

<?xml version="1.0"?>

<mcpttinfo>

<mcptt-Params>

<session-type>prearranged</>

<mcptt-request-uri>mcptt-group-T@mcptt-city1.gov</>

<mcptt-calling-user-id>mcptt-id-A1@mcptt-op.gov</>

</mcptt-Params>

</mcpttinfo>

--boundary1--

Request-URI:	Updated to include the public service identifier of the MCPTT server owning the mcptt-group-T@mcptt-city1.gov.

mcptt-info:	The application/vnd.3gpp.mcptt-info+xml is updated to include the MCPTT ID of the MCPTT user in the <mcptt-calling-user-id> element. The participating MCPTT function A1 determines the MCPTT-ID using the public user identity to locate the binding formed during service authorisation.

SDP offer:	The SDP offer is updated to include the IP address and port numbers of the participating MCPTT function.

3)	HTTP GET request (controlling MCPTT function A to GMS A)

Upon receipt of the "SIP INVITE request for controlling MCPTT function of an MCPTT group" the controlling MCPTT functions fetches the group document mcptt-group-T@mcptt-op.gov from the GMS A.

4)	HTTP 200 response (GMS A to controlling MCPTT function A)

The GMS A returns the mcptt-group-T@mcptt-op.gov group document and the document indicates that this is a temporary group consisting of mcptt-group-A1@mcptt-op.gov, mcptt-group-B@mcptt-city1.net and mcptt-group-A2@mcptt-op.gov.

5)	HTTP GET request (controlling MCPTT function A to GMS A)

Since the mcptt-id-A1@mcptt-op.gov is affiliated to the mcptt-group-A1@mcptt-op.gov the controlling MCPTT function fetches the group document from GMS A by means of a HTPP GET request as described in 3GPP TS 24.381 [31].

6)	HTTP 200 response (GMS A to controlling MCPTT function B)

The GMS A returns the mcptt-group-A1@mcptt-op.gov group document with the list of group members in a HTTP 200 response as described in 3GPP TS 24.381 [31].

The controlling MCPTT function verifies that the mcptt-id-A1@mcptt-op.gov is authorized to initiate a prearranged group call.

The following steps describe the invitation of the group member mcptt-id-A@mcptt-op.gov using a pre-established session between the participating MCPTT function A2 and MCPTT client A2. Other members can be invited using the same method or using on-demand session signalling.

7)	SIP INVITE request (controlling MCPTT function A to participating MCPTT function A2) - see example in table A.1.3-7

The controlling MCPTT function sends an initial SIP INVITE request towards the participating MCPTT function A2 according to 3GPP TS 24.229 [4].

Table A.1.3-7: SIP INVITE request (controlling MCPTT function A to participating MCPTT function A2)

INVITE sip: pf-A2.ims-op.net SIP/2.0

Accept-Contact: *;+g.3gpp.mcptt;require;explicit,+g.3gpp.icsi-ref="urn%3Aurn-7%3A3gpp-service.ims.icsi.mcptt;require;explicit

P-Asserted-Service: urn:urn-7:3gpp-service.ims.icsi.mcptt

Supported:timer

Session-Expires:3600

Contact: <sip:session@cf-A@ims-op.net:66350>;g.3gpp.mcptt;isfocus;g.3gpp.icsi-ref=urn%3Aurn-7%3A3gpp-service.ims.icsi.mcptt

P-Asserted-Identity:<sip:cf-a@ims-op.net>

Content-Type: multipart/mixed;boundary="boundary1"

--boundary1

Content-Type: application/sdp

Content-Length: (…)

c=IN IP6 5555::aaa:bbb:ddd:aaa

m=audio 23124 RTP/AVP 97

a=rtpmap:97 AMR

a=fmtp:97 mode-set=0,2,5,7; mode-change-period=2

a=maxptime:20

i=speech

m=application 23125 udp mcptt fmtp

a=fmtp:MCPTT mc_queueing;

--boundary1

Content-Type: application/vnd.3gpp.mcptt-info+xml

Content-Length: (…)

<?xml version="1.0"?>

<mcpttinfo>

<mcptt-Params>

<session-type>prearranged</>

<mcptt-request-uri>mcptt-id-A2@mcptt-op.gov</>

<mcptt-calling-group-id>mcptt-group-T@mcptt-op.gov</>

<mcptt-calling-user-id>userA1@mcptt-op.gov</>

</mcptt-Params>

</mcpttinfo>

--boundary1--

Request-URI:	Contains the public service identity of the participating MCPTT function A2 associated to the MCPTT user mcptt-id-A2@mcptt-op.gov.

Contact:	Contains the MCPTT session identity.

P-Asserted-Identity:	Contains the public service identity of the controlling MCPTT function A.

mcptt-info	Is copied from the received INVITE request and updated to include the "mcptt-id-A2@mcptt-op.gov" in the <mcptt-request-uri> element and mcptt-group-T@mcptt-op.gov in the <mcptt-calling-group-id> element.

SDP offer:	The SDP offer is based on the received SDP offer where the IP address and port numbers are replaced with the IP address and port numbers of the controlling MCPTT function. The media-level section for the media-floor control entity is indicating that queueing of floor requests is supported ("mc:queueing").

8)	SIP 200 (OK) response (participating MCPTT function A2 to controlling MCPTT function A) - see example in table A.1.3-8

The participating MCPTT function detects that there is a pre-established session to the MCPTT client A2 where negotiated MCPTT speech media stream parameters and media-floor control entity 'fmtp' attributes matches the incoming SIP INVITE request and that the MCPTT service setting for commencement mode is set to "auto-answer".

The participating MCPTT function A2 sends a SIP 200 (OK) response to the controlling MCPTT function A according to 3GPP TS 24.229 [4].

Table A.1.3-8: SIP 200 (OK) response (participating MCPTT function A2 to controlling MCPTT function A)

SIP/2.0 200 OK

Session-Expires:3600;refresher=uac

Require: timer

P-Asserted-Identity:<sip:userA2@ims-op.net>

Contact: <sip:[5555::aaa:ccc:aaa:bbbb]>; g.3gpp.mcptt;

Supported: tdialog, norefersub, explicitsub, nosub

P-Answer-State:Unconfirmed

Content-Type: multipart/mixed;boundary="boundary1"

--boundary1

Content-Type: application/sdp

Content-Length: (…)

c=IN IP6 5555::aaa:ccc:aaa:bbbb

m=audio 26456 RTP/AVP 97

a=rtpmap:97 AMR

a=fmtp:97 mode-set=0,2,5,7; maxframes=2

a=maxptime:20

i=speech

m=application 26457 udp mcptt

a=fmtp:MCPTT mc_queueing

--boundary1

Content-Type: application/vnd.3gpp.mcptt-info+xml

Content-Length: (…)

<?xml version="1.0"?>

<mcpttinfo>

<mcptt-Params>

<session-type>prearranged</>

<mcptt-called-party-id>mcptt-id-A2@mcptt-op.gov</>

</mcptt-Params>

</mcpttinfo>

--boundary1—

P-Asserted-Identity:	Contains a public user identity of the sip: userA2@ims-op.net.

Contact:	Contains a URI that identifies the session in the participating MCPTT function A2.

P-Answer-State:	Contains the value "Unconfirmed" to indicate that the participating MCPTT function A2 sent this on behalf of the MCPTT A2.

SDP answer:	The SDP answer is based on the received SDP offer and is updated to IP address and port numbers of the participating MCPTT function A2. The media-floor control entity 'fmtp' attributes includes the "mc_queueing" to indicate that queueing is supported.

9)	SIP ACK request (controlling MCPTT function A to participating MCPTT function A2)

The controlling MCPTT function A acknowledge the SIP 200 (OK) response by means of the SIP ACK request.

10)	MCPC Connect message (participating MCPTT function to MCPTT client B)

Since the MCPTT client B has a pre-established session the participating MCPTT function B uses the pre-established session to invite the MCPTT client B by means of a MCPC Connect messages as described in 3GPP TS 24.380 [5] annex A.

11)	MCPC Acknowledgement message (MCPTT client B to participating MCPTT B)

The MCPTT client B acceptes the invitation and sends an MCCP Acknowledge message as as described in 3GPP TS 24.380 [5] annex A.

12)	SIP 200 (OK) response (controlling MCPTT function A to participating MCPTT function A1) - see example in table A.1.3-12

Upon receiving the SIP 200 (OK) response from the participating MCPTT function A2, the controlling MCPTT function A interacts with the media plane as specified in 3GPP TS 24.380 [5] subclause 6.3 and sends the SIP 200 (OK) towards the participating MCPTT function A1 according to 3GPP TS 24.229 [4].

NOTE 2:	When more than one MCPTT user is invited (not done in this example) then when receiving additional SIP 200 (OK) responses from other participating MCPTT functions, the controlling MCPTT function sends the SIP ACK request towards the other participating MCPTT functions and interacts with the media plane as specified in subclause 6.3 i.e. there is no SIP signalling sent towards the MCPTT client A1.

NOTE 3:	If there more than one user in the group, the 200 (OK) response is not sent if the acknowledged timeout value for the group is running if some members of the group are marked as <on-network-required> as specified in 3GPP TS 24.381 [31], and responses are not received from those members.

Table A.1.3-12: SIP 200 (OK) response (controlling MCPTT function A to participating MCPTT function A1)

SIP/2.0 200 OK

Session-Expires:3600;refresher=uac

Require: timer

P-Asserted-Identity:<sip:cf-A.ims-op.net>

Contact: <sip:[5555::aaa:bbb:ddd:bbbb]>; +g.3gpp.mcptt;+g.3gpp.icsi-ref; isfocus

Supported: tdialog, norefersub, explicitsub, nosub

Content-Type: application/sdp

Content-Length: (…)

c=IN IP6 5555::aaa:bbb:ddd:bbbb

m=audio 23124 RTP/AVP 97

a=rtpmap:97 AMR

a=fmtp:97 mode-set=0,2,5,7; mode-change-period=2

a=maxptime:20

i=speech

m=application 23125 udp mcptt fmtp

a=fmtp:MCPTT mc_queueing;mc_implicit_request

P-Asserted-Identity:	Contains the public user identifier identifying the controlling MCPTT function.

Contact:	Contains the MCPTT session identifier and the g.3gpp.icsi-ref and isfocus media feature tags.

SDP answer:	Contains the SDP answer to the SDP offer received from the participating MCPTT function A2 updated with the IP address and port numbers of the controlling MCPTT function. The media-level section for the media-floor control entity acknowledges that queueing is supported ("mc_queueing") and that the SIP INVITE request is accepted as an implicit floor request.

13)	SIP 200 (OK) response (participating MCPTT function A to MCPTT client A) - see example in table A.1.3-13

The participating MCPTT function A1 interacts with the media plane as specified in 3GPP TS 24.380 [5] subclause 6.4 and sends the SIP 200 (OK) response towards the MCPTT client A1.

Table A.1.3-13: SIP 200 (OK) response (participating MCPTT function A to MCPTT client A1)

SIP/2.0 200 OK

Session-Expires:

Require:

P-Asserted-Identity:

Contact:

Resouce-Share: media-sharing; session-initiator; rules="k1::UL"; timestamp=55688

Priority-Share:allowed

Supported: tdialog

Content-Type: application/sdp

Content-Length: (…)

c=IN IP6 5555::aaa:ccc:aaa:bbb

m=audio 7890 RTP/AVP 97

a=

a=

a=

m=application 7891 udp mcptt

a=

Resource-Share:	Contains a new sharing key along with indication that MCPTT speech media can be shared in the uplink (UL) direction. The value "k1" is the key that P-CSCF can use to identify media streams towards MCPTT client A1 that can share media.

Priority-Share:	The Priority-Share header field is set to "allowed" indicating that priority sharing can be applied by IMS.

SDP answer:	Contains the SDP answer received from the controlling MCPTT function updated with the IP address and port numbers of the participating MCPTT function.

14)-15)	SIP ACK request (MCPTT client A1 to controlling MCPTT function A via participating MCPTT function A1)

The MCPTT client A1 acknowledges the receipt of the SIP 200 (OK) response by means of a SIP ACK request. The ACK request is forwarded by the participating MCPTT function A1 to the controlling MCPTT function A.

16)	SIP INVITE request (controlling MCPTT function A to non-Controlling MCPTT function C) - see example in table A.1.3-16

Since the GMS storing the group document of the "mcptt-group-B@mcptt-city1.net" is not available to the controlling MCPTT function A, the controlling MCPTT function A decides to use the non-controlling MCPTT function connectivity model (see figure 5.3.2-5) and sends a SIP INVITE request towards the MCPTT server hosting "mcptt-group-B@mcptt-city1.net" according to 3GPP TS 24.229 [4].

Table A.1.3-16: SIP INVITE request (controlling MCPTT function A to non-Controlling MCPTT function B)

INVITE sip:cf-B.ims-op.net SIP/2.0

Accept-Contact: *;+g.3gpp.mcptt;require;explicit,+g.3gpp.icsi-ref="urn%3Aurn-7%3A3gpp-service.ims.icsi.mcptt;require;explicit

P-Asserted-Service:urn:urn-7:3gpp-service.ims.icsi.mcptt

Supported:timer

Session-Expires:3600

Contact: <sip:session@cf-A@ims-op.net:66350>;g.3gpp.mcptt;isfocus;g.3gpp.icsi-ref=urn%3Aurn-7%3A3gpp-service.ims.icsi.mcptt

P-Asserted-Identity:<sip:cf-a@ims-op.net>

Content-Type: multipart/mixed;boundary="boundary1"

--boundary1

Content-Type: application/sdp

Content-Length: (…)

c=IN IP6 5555::aaa:bbb:ddd:aaa

m=audio 23124 RTP/AVP 97

a=rtpmap:97 AMR

a=fmtp:97 mode-set=0,2,5,7; mode-change-period=2

a=maxptime:20

i=speech

m=application 23125 udp mcptt fmtp

a=fmtp:MCPTT mc_queueing

--boundary1

Content-Type: application/vnd.3gpp.mcptt-info+xml

Content-Length: (…)

<?xml version="1.0"?>

<mcpttinfo>

<mcptt-Params>

<session-type>prearranged</>

<mcptt-request-uri>mcptt-group-B@mcptt-city1.net</>

<mcptt-calling-user-id>userA1@mcptt-op.gov</>

<mcptt-calling-group-id>mcptt-group-T@mcptt-op.gov</>

</mcptt-Params>

</mcpttinfo>

--boundary1--

Request-URI:	Contains the public service identity of the MCPTT server hosting the "mcptt-group-B@mcptt-city1.net".

Contact:	Contains the MCPTT session identity.

mcptt-info:	Is copied from the received INVITE request and updated to include the "mcptt-group-B@mcptt-city1.net" in the <mcptt-request-uri> element and mcptt-group-T@mcptt-op.gov in the <mcptt-calling-group-id> element.

SDP offer:	Is based on the received SDP offer where the IP address and port numbers are replaced with the IP address and port numbers of the controlling MCPTT function. The media-level section for the media-floor control entity is indicating that queueing is supported ("mc:queueing").

17)	HTTP GET request (controlling MCPTT function A to GMS B)

Since mcptt-group-B@mcptt-city1.net is hosted by the non-controlling MCPTT function B the group document is fetched from GMS B.

The group document are fetched by means of a HTPP GET request as described in 3GPP TS 24.381 [31].

18)	HTTP 200 response (GMS B to controlling MCPTT function B)

The GMS B returns the mcptt-group-B@mcptt-city1.net group document with the list of group members in a HTTP 200 response as described in 3GPP TS 24.381 [31].

The following steps describes the invitation of one group member, mcptt-id-B@mcptt-city1.net, using an on-demand session between the participating MCPTT function B and MCPTT client B. Other members can be invited using the same method or using pre-established session.

19)	SIP INVITE request (non-controlling MCPTT function B to participating MCPTT function B) - see example in table A.1.3-19

The non-controlling MCPTT function B invites the MCPTT user "mcptt-id-B@mcptt-city1.net" to the session by sending and SIP INVITE request towards the terminating network in accordance with 3GPP TS 24.229 [4].

Table A.1.3-19: SIP INVITE request (non-Controlling MCPTT function B to participating MCPTT function B)

INVITE sip: pf-B.ims-op.net SIP/2.0

Accept-Contact:

P-Asserted-Service:

Supported:

Session-Expires:

Contact: <sip:sessionB@cf-B@ims-op.net:45678>;g.3gpp.mcptt; isfocus; g.3gpp.icsi-ref=urn%3Aurn-7%3A3gpp-service.ims.icsi.mcptt

P-Asserted-Identity:<sip:cf-b@ims-op.net>

Content-Type: multipart/mixed;boundary="boundary1"

--boundary1

Content-Type: application/sdp

Content-Length: (…)

c=IN IP6 5555::aaa:bbb:ddd:ddd

m=audio 34456 RTP/AVP 97

a=

a=

a=

i=

m=application 34457 udp mcptt

a=fmtp:MCPTT mc_queueing

--boundary1

Content-Type: application/vnd.3gpp.mcptt-info+xml

Content-Length: (…)

<?xml version="1.0"?>

<mcpttinfo>

<mcptt-Params>

<session-type>prearranged</>

<mcptt-request-uri>mcptt-id-B@mcptt-city1.net</>

<mcptt-calling-user-id>mcptt-id-A@mcptt-op.gov</>

<mcptt-calling-group-id>mcptt-group-T@mcptt-op.gov</>

</mcptt-Params>

</mcpttinfo>

--boundary1--

Contact:	Contains a new MCPTT session identity and can be used by the members to rejoin the session.

NOTE 4:	The MCPTT session identity received from the controlling MCPTT function A cannot be used since when a member of group mcptt-group-B@mcptt-city1.net rejoins the session, the member shall rejoin the session in the non-controlling MCPTT function B and not the session in the controlling MCPTT function A.

P-Asserted-Identity:	Contains the public service identity of the non-controlling MCPTT function B.

mcptt-info:	The identity of the invited MCPTT user mcptt-id-B@mcptt-city1.net is added.

SDP offer:	The SDP offer is a copy of the received SDP offer updated with the IP address and port numbers of the non-controlling MCPTT function B.

20)	SIP 183 (Session Progress) response (participating MCPTT function B to non-Controlling MCPTT function B) - see example in table A.1.3-20

The participating MCPTT function B sends a SIP 183 (Session Progress) response towards the non-controlling MCPTT function B according to 3GPP TS 24.229 [4].

Table A.1.3-20: SIP 183 (Session Progress) response (participating MCPTT function B non-Controlling MCPTT function B)

SIP/2.0 183 Session Progress

P-Asserted-Identity:

Contact: <sip:[5555::aaa:bbb:ccc:eef]>;+g.3gpp.mcptt;,+g.3gpp.icsi-ref="urn%3Aurn-7%3A3gpp-service.ims.icsi.mcptt

P-Answer-State:Unconfirmed

P-Answer-State:	Contains the "Unconfirmed" indication acknowledging the automatic commencement mode.

21)	SIP INVITE request (participating MCPTT function B to MCPTT client B) - see example in table A.1.3-21

The participating MCPTT function B sends the SIP INVITE request towards the MCPTT client B according to 3GPP TS 24.229 [4].

Table A.1.3-21: SIP INVITE request (participating MCPTT function B to MCPTT client B)

INVITE sip: userB@ims-op.net SIP/2.0

Accept-Contact:

P-Asserted-Service:

Supported:timer;tdialog

Session-Expires:refresher;uac

Contact:

Resouce-Share: media-sharing; session-receiver; rules="k2::UL"; timestamp=55750

Priority-Share:allowed

P-Asserted-Identity:<sip:cf-b@ims-op.net>

Answer-Mode:Auto

Content-Type: multipart/mixed;boundary="boundary1"

--boundary1

Content-Type: application/sdp

Content-Length: (…)

c=IN IP6 5555::aaa:bbb:ccc:eef

m=audio 16412 RTP/AVP 97

a=

a=

a=

i=

m=application 16413 udp mcptt

a=fmtp:MCPTT mc_queueing

--boundary1

Content-Type: application/vnd.3gpp.mcptt-info+xml

Content-Length: (…)

<?xml version="1.0"?>

<mcpttinfo>

<mcptt-Params>

<session-type>prearranged</>

<mcptt-request-uri>mcptt-id-B@mcptt-city1.net</>

<mcptt-calling-user-id>userA1@mcptt-op.gov</>

<mcptt-calling-group-id>mcptt-group-T@mcptt-op.gov</>

</mcptt-Params>

</mcpttinfo>

--boundary1--

Request-URI:	Contains the public user identity of the mcptt-id-B@mcptt-city1.net.

Resource-Share:	Contains a new sharing key along with indication that MCPTT speech media can be shared in the uplink (UL) direction. The value "k2" is the key that P-CSCF can use to identify media streams towards MCPTT client A2 that can share media.

Priority-Share:	The Priority-Share header field is set to "allowed" indicating that priority sharing can be applied by IMS.

Answer-Mode:	The MCPTT service setting is "auto-answer" so the participating MCPTT function includes the value "auto" in the Answer-Mode header field.

SDP offer:	The SDP offer is a copy of the received SDP offer updated with the IP address and port numbers of the participating MCPTT function B.

22)	SIP 200 (OK) response (non-controlling MCPTT function B to controlling MCPTT function A) - see example in table A.1.3-22

Upon receipt of the first SIP 200 (OK) response from the participating MCPTT function B serving the invited MCPTT user B in the prearranged group mcptt-group-C@mcptt-city1.net, the non-controlling MCPTT function interact with the media plane as specified in 3GPP TS 24.380 [5] and sends the SIP 200 (OK) response towards the controlling MCPTT function A in accordance with 3GPP TS 24.229 [4].

NOTE 6:	The non-controlling function translates the SIP 183 (Session Progress) response with a P-Answer-State header field set to "Auto" to a SIP 200 (OK) response only if the non-controlling function supports media-buffering otherwise a SIP 183 (Session Progress) response is sent.

Table A.1.3-22: SIP 200 (OK) response (non-controlling MCPTT function B to controlling MCPTT function A)

SIP/2.0 200 OK

Session-Expires:3600;refresher=uac

Require: timer

P-Asserted-Identity:<sip:cf-B.op.net>

Contact: <sip:[5555::aaa:bbb:ddd:ddd]>; g.3gpp.mcptt;+g.3gpp.icsi-ref="urn%3Aurn-7%3A3gpp-service.ims.icsi.mcptt

Supported: tdialog, norefersub, explicitsub, nosub

Content-Type: application/sdp

Content-Length: (…)

c=IN IP6 5555::aaa:bbb:ddd:ddd

m=audio 34456 RTP/AVP 97

a=rtpmap:97 AMR

a=fmtp:97 mode-set=0,2,5,7; maxframes=2

a=maxptime:20

i=speech

m=application 34457 udp mcptt

a=fmtp:MCPTT mc_queueing

P-Asserted-Identity	Contains a public service identity identifying the non-controlling MCPTT function B.

Contact	Contains a contact address and the media feature tags g.3gpp.mcptt and g.3gpp.icsi-ref.

SDP	Contains the SDP answer to the SDP offer received from the controlling MCPTT function A. The media-level section for the media-floor control entity acknowledges that queueing is supported ("mc_queueing").

NOTE 5:	The SDP answer is based on the capabilities of the non-controlling MCPTT function and not based on the SIP 200 (OK) response received from the participating MCPTT function.

23)	SIP ACK request (controlling MCPTT function A to MCPTT client B via non-controlling MCPTT function B and participating MCPTT function B)

The controlling MCPTT function acknowledges the receipt of the SIP 200 (OK) response by means of a SIP ACK request.

24)-25)	SIP 200 (OK) response (MCPTT client B to non-controlling MCPTT function B via participating MCPTT function B) - see example in table A.1.3-24

Upon receiving the SIP INVITE request the MCPTT client B accepts the invitation, interacts with the media plane as specified in 3GPP TS 24.380 subclause 6.2 and sends the SIP 200 (OK) response towards the MCPTT server according to rules and procedures of 3GPP TS 24.229 [4].

Table A.1.3-24: SIP 200 (OK) response (MCPTT client B to non-controlling MCPTT function B via participating MCPTT function B)

SIP/2.0 200 OK

Session-Expires:3600;refresher=uas

Require: timer

Contact:<sip:[5555::baa:abb:ddd:cccc]>;+g.3gpp.mcptt;+g.3gpp.icsi-ref="urn%3Aurn-7%3A3gpp-service.ims.icsi.mcptt"

P-Asserted-Identity:<sip:userB@ims-op.net>

Supported: tdialog, norefersub, explicitsub, nosub

P-Answer-State:Confirmed

Content-Type: multipart/mixed;boundary="boundary1"

--boundary1

Content-Type: application/sdp

Content-Length: (…)

c=IN IP6 5555::baa:abb:ddd:cccc

m=audio 62122 RTP/AVP 97

a=rtpmap:97 AMR

a=fmtp:97 mode-set=0,2,5,7; maxframes=2

a=maxptime:20

i=speech

m=application 62123 udp mcptt

a=fmtp:MCPTT mc_queueing

--boundary1

Content-Type: application/vnd.3gpp.mcptt-info+xml

Content-Length: (…)

<?xml version="1.0"?>

<mcpttinfo>

<mcptt-Params>

<session-type>prearranged</>

<mcptt-called-party-id>mcptt-id-B@mcptt-city1.net</>

</mcptt-Params>

</mcpttinfo>

--boundary1—

Contact:	Contains the registered contact and the g.3gpp.mcptt feature tag.

P-Answer-State:	Contains the value "Confirmed" to indicate that the MCPTT client B sent the response.

SDP answer:	The received SDP offer is accepted and updated with the IP address and port numbers of the MCPTT client B. The MCPTT client B confirms that queueing of floor requests are supported in the 'fmtp' attribute "mc_queueing".

The participating MCPTT function B updates the IP address and port numbers with the IP address and port numbers of the participating MCPTT function B.

26)-27)	SIP ACK request (non-controlling MCPTT function B to MCPTT client B via participating MCPTT function B)

The non-controlling MCPTT function acknowledges the receipt of the SIP 200 (OK) response by means of a SIP ACK request.

28)	SIP INVITE request (controlling MCPTT function A to non-controlling MCPTT function A) – see example in table A.1.3-28

Since the GMS storing the group document of the "mcptt-group-A2@mcptt-op.gov" is not available to the controlling MCPTT function A, the controlling MCPTT function A decides to use the non-controlling MCPTT function connectivity model (see figure 5.3.2-2) and sends a SIP INVITE request towards the MCPTT server hosting "mcptt-group-A2@mcptt-op.gov" in accordance with 3GPP TS 24.229 [4].

Table A.1.3-28: SIP INVITE request (controlling MCPTT function A to non-Controlling MCPTT function A)

INVITE sip: cf-A2.ims-op.net SIP/2.0

Accept-Contact: *;+g.3gpp.mcptt;require;explicit,+g.3gpp.icsi-ref="urn%3Aurn-7%3A3gpp-service.ims.icsi.mcptt;require;explicit

P-Asserted-Service:urn:urn-7:3gpp-service.ims.icsi.mcptt

Supported:timer

Session-Expires:3600

Contact: <sip:session@cf-A@ims-op.net:66350>;g.3gpp.mcptt;isfocus;g.3gpp.icsi-ref=urn%3Aurn-7%3A3gpp-service.ims.icsi.mcptt

P-Asserted-Identity:<sipcf-a@ims-op.net>

Content-Type: multipart/mixed;boundary="boundary1"

--boundary1

Content-Type: application/sdp

Content-Length: (…)

c=IN IP6 5555::aaa:bbb:ddd:aaa

m=audio 23124 RTP/AVP 97

a=rtpmap:97 AMR

a=fmtp:97 mode-set=0,2,5,7; mode-change-period=2

a=maxptime:20

i=speech

m=application 23125 udp mcptt fmtp

a=fmtp:MCPTT mc_queueing

--boundary1

Content-Type: application/vnd.3gpp.mcptt-info+xml

Content-Length: (…)

<?xml version="1.0"?>

<mcpttinfo>

<mcptt-Params>

<session-type>prearranged</>

<mcptt-request-uri>mcptt-group-A2@mcptt-op.gov</>

<mcptt-calling-user-id>userA1@mcptt-op.gov</>

<mcptt-calling-group-id>mcptt-group-T@mcptt-op.gov</>

</mcptt-Params>

</mcpttinfo>

--boundary1--

Request-URI:	Contains the public service identity of the MCPTT server hosting the "mcptt-group-A2@mcptt-op.gov".

Contact:	Contains the MCPTT session identity.

mcptt-info:	Is copied from the received INVITE request and updated to include the "mcptt-group-A2@mcptt-op.gov" in the <mcptt-request-uri> element and mcptt-group-T@mcptt-op.gov in the <mcptt-calling-group-id> element.

SDP offer:	Is based on the received SDP offer where the IP address and port numbers are replaced with the IP address and port numbers of the controlling MCPTT function. The media-level section for the media-floor control entity is indicating that queueing is supported ("mc:queueing").

29)	HTTP GET request (controlling MCPTT function A to GMS B)

Since mcptt-group-A@mcptt-city1.net is hosted by the non-controlling MCPTT function A the group document is fetched from GMS A2.

The group document is fetched by means of a HTPP GET request as described in 3GPP TS 24.381 [31].

30)	HTTP 200 response (GMS B to controlling MCPTT function B)

The GMS B returns the mcptt-group-A2@mcptt-op.gov group document with the list of group members in a HTTP 200 response as described in 3GPP TS 24.381 [31].

The following steps describes the invitation of one group member, mcptt-id-A@mcptt-op.gov, using an on-demand session between the participating MCPTT function B and MCPTT client B. Other members can be invited using the same method or using pre-established session.

31)	SIP INVITE request (non-controlling MCPTT function A to participating MCPTT function A3) – see example in table A.1.3-31

The non-controlling MCPTT function A invites the MCPTT user mcptt-id-A3@mcptt-op.gov to the session by sending and SIP INVITE request towards the terminating network in accordance with 3GPP TS 24.229 [4].

Table A.1.3-31: SIP INVITE request (non-Controlling MCPTT function A to participating MCPTT function A3)

INVITE sip: pf-A3.ims-op.net SIP/2.0

Accept-Contact:

P-Asserted-Service:

Supported:

Session-Expires:

Contact: <sessionA@cf-A2@ims-op.net61234>;g.3gpp.mcptt; isfocus; g.3gpp.icsi-ref=urn%3Aurn-7%3A3gpp-service.ims.icsi.mcptt

P-Asserted-Identity:<sip:cf-A2.ims-op.net>

Content-Type: multipart/mixed;boundary="boundary1"

--boundary1

Content-Type: application/sdp

Content-Length: (…)

c=IN IP6 5555::aaa:bbb:ddd:eee

m=audio 12344 RTP/AVP 97

a=

a=

a=

i=

m=application 12345 udp mcptt

a=fmtp:MCPTT mc_queueing

--boundary1

Content-Type: application/vnd.3gpp.mcptt-info+xml

Content-Length: (…)

<?xml version="1.0"?>

<mcpttinfo>

<mcptt-Params>

<session-type>prearranged</>

<mcptt-request-uri>mcptt-id-A3@mcptt-op.gov</>

<mcptt-calling-user-id>mcptt-id-A@mcptt-op.gov</>

<mcptt-calling-group-id>mcptt-group-T@mcptt-op.gov</>

</mcptt-Params>

</mcpttinfo>

--boundary1--

Contact:	Contains a new MCPTT session identity and can be used by the members to rejoin the session.

NOTE 7:	The MCPTT session identity received from the controlling MCPTT function A cannot be used since when a member of group mcptt-group-A2@mcptt-op.gov rejoins the session, the member shall rejoin the session in the non-controlling MCPTT function A and not the session in the controlling MCPTT function A.

P-Asserted-Identity:	Contains the public service identity of the non-controlling MCPTT function A.

mcptt-info:	The identity of the invited MCPTT user mcptt-id-A3@mcptt-op.gov is added.

SDP offer:	The SDP offer is a copy of the received SDP offer updated with the IP address and port numbers of the non-controlling MCPTT function A.

32)	SIP INVITE request (participating MCPTT function A3 to MCPTT client A3) – see example in table A.1.3-32

The participating MCPTT function A3 sends the SIP INVITE request towards the MCPTT client A3 according to 3GPP TS 24.229 [4].

Table A.1.3-32: SIP INVITE request (participating MCPTT function A3 to MCPTT client A3)

INVITE sip: userA3@ims-op.net SIP/2.0

Accept-Contact:

P-Asserted-Service:

Supported:timer;tdialog

Session-Expires:refresher;uac

Contact:

Resouce-Share: media-sharing; session-receiver; rules="k3::UL"; timestamp=32651

Priority-Share:allowed

P-Asserted-Identity:<sip:cf-A2.ims-op.net>

Answer-Mode:Manual

Content-Type: multipart/mixed;boundary="boundary1"

--boundary1

Content-Type: application/sdp

Content-Length: (…)

c=IN IP6 5555::aaa:ccc:aaa:ccc

m=audio 18412 RTP/AVP 97

a=

a=

a=

i=

m=application 18413 udp mcptt

a=fmtp:MCPTT mc_queueing

--boundary1

Content-Type: application/vnd.3gpp.mcptt-info+xml

Content-Length: (…)

<?xml version="1.0"?>

<mcpttinfo>

<mcptt-Params>

<session-type>prearranged</>

<mcptt-request-uri>mcptt-id-A3@mcptt-op.gov</>

<mcptt-calling-user-id>mcptt-id-A@mcptt-op.gov</>

<mcptt-calling-group-id>mcptt-group-T@mcptt-op.gov</>

</mcptt-Params>

</mcpttinfo>

--boundary1--

Request-URI:	Contains the public user identity of the userA3@ims-op.net.

Resource-Share:	Contains a new sharing key along with indication that MCPTT speech media can be shared in the uplink (UL) direction. The value "k3" is the key that P-CSCF can use to identify media streams towards MCPTT client A3 that can share media.

Priority-Share:	The Priority-Share header field is set to "allowed" indicating that priority sharing can be applied by IMS.

Answer-Mode:	The MCPTT service setting is "manual-answer" so the participating MCPTT function includes the value "Manual" in the Answer-Mode header field.

SDP offer:	The SDP offer is a copy of the received SDP offer updated with the IP address and port numbers of the participating MCPTT function B.

33)-34)	SIP 200 (OK) response to the SIP INVITE request (MCPTT client A3 to non-controlling MCPTT function A) – see example in table A.1.3-33

Upon receiving the SIP INVITE request the MCPTT client A3 accepts the invitation, interacts with the media plane as specified in 3GPP TS 24.380 subclause 6.2 and sends the SIP 200 (OK) response towards the MCPTT server according to rules and procedures of 3GPP TS 24.229 [4].

Table A.1.3-33: SIP 200 (OK) response (MCPTT client A3 to non-controlling MCPTT function via participating MCPTT function A3)

SIP/2.0 200 OK

Session-Expires:3600;refresher=uas

Require: timer

Contact:<sip:[5555::bbb:aaa:ccc:aaa]>;+g.3gpp.mcptt;+g.3gpp.icsi-ref="urn%3Aurn-7%3A3gpp-service.ims.icsi.mcptt"

P-Asserted-Identity:<sip:userA3@ims-op.net>

Supported: tdialog, norefersub, explicitsub, nosub

P-Answer-State:Confirmed

Content-Type: multipart/mixed;boundary="boundary1"

--boundary1

Content-Type: application/sdp

Content-Length: (…)

c=IN IP6 5555::baa:abb:ddd:cccc

m=audio 25644 RTP/AVP 97

a=rtpmap:97 AMR

a=fmtp:97 mode-set=0,2,5,7; maxframes=2

a=maxptime:20

i=speech

m=application 25644 udp mcptt

a=fmtp:MCPTT mc_queueing

--boundary1

Content-Type: application/vnd.3gpp.mcptt-info+xml

Content-Length: (…)

<?xml version="1.0"?>

<mcpttinfo>

<mcptt-Params>

<session-type>prearranged</>

<mcptt-called-party-id>mcptt-id-A3@mcptt-op.gov</>

</mcptt-Params>

</mcpttinfo>

--boundary1—

Contact:	Contains the registered contact, the g.3gpp.mcptt feature tag and the g.3gpp.icsi-ref media feature tag with the IMS communication service MCPTT.

P-Answer-State:	Contains the value "Confirmed" to indicate that the MCPTT client sent the response.

SDP answer:	The received SDP offer is accepted and updated with the IP address and port numbers of the MCPTT client A3. The MCPTT client A3 confirms that queueing of floor requests are supported in the 'fmtp' attribute "mc_queueing".

The participating MCPTT function A3 updates the IP address and port numbers with the IP address and port numbers of the participating MCPTT function A3.

35)-36)	SIP ACK request (non-controlling MCPTT function A to MCPTT client A3 via participating MCPTT function A3.

The non-controlling MCPTT function A acknowledges the receipt of the SIP 200 (OK) response to the INVITE request by means of a SIP ACK request.

37)	SIP 200 (OK) response to the SIP INVITE request (non-controlling MCPTT function A to controlling MCPTT function A) – see example in table A.1.3-37

Upon receipt of the first SIP 200 (OK) response from the participating MCPTT function A3 serving the invited MCPTT user A3 in the prearranged group mcptt-id-A3@mcptt-op.gov, the non-controlling MCPTT function A interact with the media plane as specified in 3GPP TS 24.380 [5] and sends the SIP 200 (OK) towards the controlling MCPTT function A in accordance with 3GPP TS 24.229 [4].

Table A.1.3-37: SIP 200 (OK) response (non-controlling MCPTT function A to controlling MCPTT function A)

SIP/2.0 200 OK

Session-Expires:3600;refresher=uac

Require: timer

P-Asserted-Identity:<sip:cf-B.ims-op.net>

Contact: <sip:[5555::aaa:bbb:ddd:eee]>;+g.3gpp.mcptt;+g.3gpp.icsi-ref="urn%3Aurn-7%3A3gpp-service.ims.icsi.mcptt

Supported: tdialog, norefersub, explicitsub, nosub

Content-Type: application/sdp

Content-Length: (…)

c=IN IP6 5555::aaa:bbb:ddd:eee

m=audio 12344 RTP/AVP 97

a=rtpmap:97 AMR

a=fmtp:97 mode-set=0,2,5,7; maxframes=2

a=maxptime:20

i=speech

m=application 12345 udp mcptt

a=fmtp:MCPTT mc_queueing

P-Asserted-Identity	Contains a public service identity identifying the non-controlling MCPTT function A3.

Contact	Contains a contact address and the media feature tags g.3gpp.mcptt and g.3gpp.icsi-ref.

SDP	Contains the SDP answer to the SDP offer received from the controlling MCPTT function A. The media-level section for the media-floor control entity acknowledged that queueing is supported ("mc_queueing").

NOTE 8:	The SDP answer is based on the capabilities of the non-controlling MCPTT function and not based on the SIP 200 (OK) response received from the participating MCPTT function.

38)	SIP ACK request (controlling MCPTT function A to non-controlling MCPTT function A.

The controlling MCPTT function A acknowledges the receipt of the SIP 200 (OK) response to the INVITE request by means of a SIP ACK request.

Annex B (normative):
Timers

# B.1	General

The following tables give a brief description of the timers used in the present document.

For the on-network timers described in the present document, the following timer families are used:

-	TNGx: Timer oN-network Group call x

For the off-network timers described in the present document, the following timer families are used:

-	TFGx:	Timer oFf-network Group call x

-	TFPy:	Timer oFf-network Private call y

-	TFBz:	Timer oFf-network Broadcast group call z

where x, y and z represent numbers.

# B.2	On-network timers

## B.2.1	Timers in the controlling MCPTT function

The table B.2.1-1 provides a description of the timers used by the controlling MCPTT function, specifies the timer values, describes the reason for starting of the timer, normal stop and the action on expiry.

Table B.2.1-1: controlling MCPTT function timers

| Timer | Timer value | Cause of start | Normal stop | On expiry |
| --- | --- | --- | --- | --- |
| TNG1 (acknowledged call setup timer) (NOTE 1) | Obtained from the group document in the <on-network-timeout-for-acknowledgement-of-required-members> element as specified in 3GPP TS 24.381 [31]. | On reception of a SIP INVITE request to start a group session where the group document contains <on-network-required> group members as specified in 3GPP TS 24.381 [31]. | On receipt of all SIP 200 (OK) responses to all SIP INVITE requests for <on-network-required> group members as specified in 3GPP TS 24.381 [31]. | Either proceed with the set-up of the call or abandon the call. |
| TNG2 (in-progress emergency group call timer) (NOTE 2) | Obtained from the <group-time-limit> element of the <emergency-call> element of the <on-network> element of the service configuration document as specified in 3GPP TS 24.384 [50]. | On reception of a SIP INVITE request or SIP re-INVITE request that initiates an MCPTT emergency group call. | On acceptance of a request to cancel the in-progress emergency state of a group. | Cancels the in-progress emergency state of the group and return the session and/or call to normal priority level. |
| TNG3 (group call timer). (NOTE 1). | Set to the value of the <on-network-maximum-duration> element from the group document. | On reception of a SIP INVITE request to start a group session after retrieval of the group document from the group management server. For a temporary group call, when merging active group calls into a temporary group call. When splitting a temporary group all into independent active calls | When the last MCPTT client has left the session. When a temporary group call is split into independent active calls. For active group calls, when merging them into a temporary group call. | Release the group call. |
| NOTE 1: More than one instance of this timer can be running in the controlling MCPTT function, each instance associated with a specific group call. NOTE 2: More than one instance of this timer can be running in the controlling MCPTT function, each instance associated with a specific in-progress emergency state of a single group |  |  |  |  |

# B.3	Off-network timers

## B.3.1	Timers in off-network group call

### B.3.1.1	Basic call control

The table B.3.1.1-1 lists the timers used in basic call control, their start values, their limits, describes the cause of the start, and the action to take on normal stop and on expiry.

Table B.3.1.1-1: Timers in basic call control

| Timer | Timer value | Cause of start | Normal stop | On expiry |
| --- | --- | --- | --- | --- |
| TFG1 (wait for call announcement) | Default value: 150 millisecond Configurable. Set to the value of "/<x>/OffNetwork/Timers/TFG1" leaf node present in the UE initial configuration as specified in 3GPP TS 24.383 [45]. | When the client sends a CALL PROBE message. | Reception of a CALL ANNOUNCEMENT message. | Send a CALL ANNOUNCEMENT message. |
| TFG2 (call announcement) | Calculated. Refer to section 10.2.2.4.1.1. | Commencement of group call. Restarted every time a CALL PROBE message is received OR CALL ANNOUNCEMENT message is sent or received. | Termination of group call. When the client Receives a CALL PROBE message or CALL ANNOUNCEMENT message, Re-calculate timer value and restart. | Send a CALL ANNOUNCEMENT message. Re-calculate timer value and restart. |
| TFG3 (call probe retransmission) | Default value: 40 millisecond Depends on the characteristic of the D2D (D2D Sidelink period) Configurable. Set to the value of "/<x>/OffNetwork/Timers/TFG3" leaf node present in the UE initial configuration as specified in 3GPP TS 24.383 [45]. | When the client sends a CALL PROBE message. | Reception of a CALL ANNOUNCEMENT message. Or TFG1 Expires. Or User releases the call. | Send a CALL PROBE message. |
| TFG4 (waiting for the user) | Default value: 30 seconds Maximum value: 60 seconds Configurable. Set to the value of "/<x>/OffNetwork/Timers/TFG4" leaf node present in the UE initial configuration as specified in 3GPP TS 24.383 [45]. | Reception of CALL ANNOUNCEMENT message when not participating in the ongoing call. | Reception of User action (Accept or Reject). | Stop incoming call notification. |
| TFG5 (not present incoming call announcements) | Default value: 30 seconds Configurable. Set to the value of "/<x>/OffNetwork/Timers/TFG5" leaf node present in the UE initial configuration as specified in 3GPP TS 24.383 [45]. | Expiration of TFG4 Or User rejects the call. Or User releases the call. | - | Reset group call state machine. |
| TFG6 (Max duration) | Calculated. Refer to section 10.2.2.4.1.2. | Commencement of group call | Termination of group call | Release the group call |

### B.3.1.2	Call type control

The table B.3.1.2-1 lists the timers used in call type control, their star values, describes the cause of start, and the action to take on normal stop and on expiry.

Table B.3.1.2-1: Call type control

| Timer | Timer value | Cause of start | Normal stop | On expiry |
| --- | --- | --- | --- | --- |
| TFG11 (emergency end retransmission) | Default value: 1 second Configurable. Set to the value of "/<x>/OffNetwork/Timers/TFG11" leaf node present in the UE initial configuration as specified in 3GPP TS 24.383 [45]. | When the client sends a GROUP CALL EMERGENCY END message. |  | Send a GROUP CALL EMERGENCY END message Increment associated counter by 1. If counter has reached limit, stop the timer. |
| TFG12 (imminent peril end retransmission) | Default value: 1 second Configurable. Set to the value of "/<x>/OffNetwork/Timers/TFG12" leaf node present in the UE initial configuration as specified in 3GPP TS 24.383 [45]. | When the client sends a GROUP CALL IMMINENT PERIL END message. |  | Send a GROUP CALL IMMINENT PERIL END message Increment associated counter by 1. If counter has reached limit, stop the timer. |
| TFG13 (implicit downgrade) | Default value : 600 seconds Configurable. Set to the value of "/<x>/OffNetwork/Timers/TFG13" leaf node present in the UE initial configuration as specified in 3GPP TS 24.383 [45]. | Upgrade of the call. | Downgrade of the call. | Downgrade the call. |

## B.3.2	Timers in off-network private call

The table B.3.2-1 lists the timers used in off-network private call, their start values, their limits, describes the cause of start, and the action to take on normal stop and on expiry.

Table B.3.2-1: Timers in off-network private call

| Timer | Timer value | Cause of start | Normal stop | On expiry |
| --- | --- | --- | --- | --- |
| TFP1 (private call request retransmission) | Default value: 40 millisecond Depends on the characteristic of the D2D (D2D Sidelink period) Configurable. Set to the value of "/<x>/OffNetwork/Timers/TFP1" leaf node present in the UE initial configuration as specified in 3GPP TS 24.383 [45]. | When the client sends a PRIVATE CALL SETUP REQUEST message. | Reception of a PRIVATE CALL ACCEPT or PRIVATE CALL REJECT message. | Resend PRIVATE CALL SETUP REQUEST message. Increment associated counter by 1. If counter has reached limit, assume the called client is not available. Terminate call setup. |
| TFP2 (waiting for call response message) | Default value: 30 seconds Maximum value: 60 seconds Configurable. Set to the value of "/<x>/OffNetwork/Timers/TFP2" leaf node present in the UE initial configuration as specified in 3GPP TS 24.383 [45]. | Reception of a PRIVATE CALL SETUP REQUEST message. | User responds to the incoming call notification. | Start TFP7 timer. Send a PRIVATE CALL REJECT message |
| TFP3 (private call release retransmission) | Default value: 40 millisecond Depends on the characteristic of the D2D (D2D Sidelink period) Configurable. Set to the value of "/<x>/OffNetwork/Timers/TFP3" leaf node present in the UE initial configuration as specified in 3GPP TS 24.383 [45]. | When the client sends a PRIVATE CALL RELEASE message. | Reception of PRIVATE CALL RELEASE ACK message. | Resend PRIVATE CALL RELEASE message. Increment associated counter by 1. If counter has reached limit, assume the receiving client is not available anymore. Release the call. |
| TFP4 (private call accept retransmission) | Default value: 40 millisecond Depends on the characteristic of the D2D (D2D Sidelink period) Configurable. Set to the value of "/<x>/OffNetwork/Timers/TFP4" leaf node present in the UE initial configuration as specified in 3GPP TS 24.383 [45]. | When the client sends a PRIVATE CALL ACCEPT message. | Reception of a PRIVATE CALL ACCEPT ACK message or RTP media. | Resend PRIVATE CALL ACCEPT message. Increment associated counter by 1. If counter has reached limit, assume the receiving client is not available anymore Notify call setup failure. |
| TFP5 (max duration) | Configurable. Set to the value of "/<x>/OffNetwork/PrivateCall/MaxDuration" leaf node present in the service configuration as specified in 3GPP TS 24.383 [45]. | Call establishment. | Call termination. | Terminate the call. |
| TFP6 (private emergency call cancel retransmission) | Default value: 40 millisecond Depends on the characteristic of the D2D. (D2D Sidelink period) Configurable. Set to the value of "/<x>/OffNetwork/Timers/TFP6" leaf node present in the UE initial configuration as specified in 3GPP TS 24.383 [45]. | When the client sends a PRIVATE EMERGENCY CALL CANCEL message. | Reception of a PRIVATE EMERGENCY CALL CANCEL ACK message. | Resend PRIVATE EMERGENCY CALL CANCEL message. Increment associated counter by 1. If counter has reached limit, assume the receiving client is not available anymore Notify call setup failure. |
| TFP7 (waiting for any message with same call identifier) | Default value: 1 second Configurable. Set to the value of "/<x>/OffNetwork/Timers/TFP7" leaf node present in the UE initial configuration as specified in 3GPP TS 24.383 [45]. | Rejection of a call OR Termination of a call OR Call Failure. | - | Reset the call control state machine. |

## B.3.3	Timers in off-network broadcast call

The table B.3.3-1 lists the timers used in off-network broadcast call, their start values, their limits, describes the cause of start, and the action to take on normal stop and on expiry.

Table B.3.3-1: Timers in off-network broadcast call

| Timer | Timer value | Cause of start | Normal stop | On expiry |
| --- | --- | --- | --- | --- |
| TFB1 (max duration) | Default value: 300 seconds Maximum value: 600 seconds Configurable. Set to the value of "/<x>/OffNetwork/Timers/TFB1" leaf node present in the UE initial configuration as specified in 3GPP TS 24.383 [45]. | Start of the broadcast call. | - | Terminate the broadcast call. |
| TFB2 (broadcast retransmission) | Default value: 3 seconds Maximum value: 10 seconds Configurable. Set to the value of "/<x>/OffNetwork/Timers/TFB2" leaf node present in the UE initial configuration as specified in 3GPP TS 24.383 [45]. | Start of the broadcast call. | Broadcast call termination. | Send GROUP CALL BROADCAST message. |
| TFB3 (waiting for the user) | Default value: 30 seconds Maximum value: 60 seconds Configurable. Set to the value of "/<x>/OffNetwork/Timers/TFB3" leaf node present in the UE initial configuration as specified in 3GPP TS 24.383 [45]. | Receipt of GROUP CALL BROADCAST message when user response is required. | Response from user. | Terminate incoming call notification. |

## B.3.4	Timers in off-network emergency alert

The table B.3.4-1 lists the timers used in off-network emergency alert, their start values, their limits, describes the cause of start, and the action to take on normal stop and on expiry.

Table B.3.4-1: Timers in off-network emergency alert

| Timer | Timer value | Cause of start | Normal stop | On expiry |
| --- | --- | --- | --- | --- |
| TFE1 (Emergency Alert) | Default value: 30 seconds Maximum value: 60 seconds Configurable. Set to the value of "/<x>/OffNetwork/Timers/TFE1" leaf node present in the UE initial configuration as specified in 3GPP TS 24.383 [45]. | Receipt of GROUP EMERGENCY ALERT. | Receipt of GROUP EMERGENCY ALERT CANCEL. | Assume end of emergency state, remove associated user from the list. |
| TFE2 (emergency alert retransmission) | Default value: 5 seconds Maximum value: 10 seconds Configurable. Set to the value of "/<x>/OffNetwork/Timers/TFE2" leaf node present in the UE initial configuration as specified in 3GPP TS 24.383 [45]. | Transmission of GROUP EMERGENCY ALERT. | Transmission of GROUP EMERGENCY ALERT CANCEL. | Transmit GROUP EMERGENCY ALERT. |

Annex C (normative):
Counters

# C.1	General

The following tables give a brief description of the counters used in the present document.

# C.2	Off-network counters

## C.2.1	Counters in off-network group call

The table C.2.1-1 lists the counters used in off-network group call, their default upper limits and the action to take upon reaching the upper limit. The counters start at 1.

Table C.2.1-1: Counters in off-network group call

| Counter | Upper Limit | Associated timer | Upon reaching the upper limit |
| --- | --- | --- | --- |
| CFG11 (emergency end retransmission) | Default value: 5 Configurable. Set to the value of "/<x>/OffNetwork/Counters/CFG11" leaf node present in the UE initial configuration as specified in 3GPP TS 24.383 [45]. | TFG11 | Stop timer TFG11. |
| CFG12 (imminent peril end retransmission) | Default value: 5 Configurable. Set to the value of "/<x>/OffNetwork/Counters/CFG12" leaf node present in the UE initial configuration as specified in 3GPP TS 24.383 [45]. | TFG12 | Stop timer TFG12. |

## C.2.2	Counters in off-network private call

The table C.2.2-1 lists the counters used in off-network private call, their default upper limits and the action to take upon reaching the upper limit. The counters start at 1.

Table C.2.2-1: Counters in off-network private call

| Counter | Upper Limit | Associated timer | Upon reaching the upper limit |
| --- | --- | --- | --- |
| CFP1 (private call request retransmission) | Default value: 3 Configurable. Set to the value of "/<x>/OffNetwork/Counters/CFP1" leaf node present in the UE initial configuration as specified in 3GPP TS 24.383 [45]. | TFP1 | Assume the called client is not available. Terminate call setup. |
| CFP3 (private call release retransmission) | Default value: 3 Configurable. Set to the value of "/<x>/OffNetwork/Counters/CFP3" leaf node present in the UE initial configuration as specified in 3GPP TS 24.383 [45]. | TFP3 | Assume the receiving client is not available anymore. Release the call. |
| CFP4 (private call accept retransmission) | Default value: 3 Configurable. Set to the value of "/<x>/OffNetwork/Counters/CFP4" leaf node present in the UE initial configuration as specified in 3GPP TS 24.383 [45]. | TFP4 | Notify call setup failure. |
| CFP6 (emergency private call cancel retransmission) | Default value: 3 Configurable. Set to the value of "/<x>/OffNetwork/Counters/CFP6" leaf node present in the UE initial configuration as specified in 3GPP TS 24.383 [45]. | TFP6 | Notify emergency call release failure. |

Annex D (normative):
Media feature tags within the current document

# D.1	General

This subclause describes the media feature tag definitions that are applicable for the 3GPP IM CN Subsystem for the realisation of the Mission Critical Push To Talk (MCPTT) service.

# D.2	Definition of media feature tag g.3gpp.mcptt

Media feature tag name: g.3gpp.mcptt

Editor's Note: this media feature tag needs to be registered with IANA when the release 13 is completed.

ASN.1 Identifier: 1.3.6.1.8.2.x

Summary of the media feature indicated by this media feature tag: This media feature tag when used in a SIP request or a SIP response indicates that the function sending the SIP message supports Mission Critical Push To Talk (MCPTT) communication.

Values appropriate for use with this media feature tag: Boolean

The media feature tag is intended primarily for use in the following applications, protocols, services, or negotiation mechanisms: This media feature tag is most useful in a communications application, for describing the capabilities of a device, such as a phone or PDA.

Examples of typical use: Indicating that a mobile phone supports the Mission Critical Push To Talk (MCPTT) communication.

Related standards or documents: 3GPP TS 24.379: "Mission Critical Push To Talk (MCPTT) call control Protocol specification"

Security Considerations: Security considerations for this media feature tag are discussed in subclause 11.1 of IETF RFC 3840 [16].

Annex E (normative):
ICSI values defined within the current document

# E.1		General

This subclause describes the IMS communications service identifier definitions that are applicable for the 3GPP IM CN Subsystem for the realisation of the Mission Critical Push To Talk (MCPTT) service.

NOTE:	The template has been created using the headers of the table in http://www.3gpp.org/specifications-groups/34-uniform-resource-name-urn-list

# E.2		Definition of ICSI value for MCPTT service

## E.2.1	URN

urn:urn-7:3gpp-service.ims.icsi.mcptt

## E.2.2	Description

This URN indicates that the device has the capabilities to support the mission critical push to talk (MCPTT) service.

## E.2.3	Reference

3GPP TS 24 379: "Mission Critical Push To Talk (MCPTT) call control Protocol specification"

## E.2.3	Contact

Name:	<MCC name>

Email:	<MCC email address>

## E.2.4	Registration of subtype

Yes

## E.2.5	Remarks

None

Annex F (normative):
XML schemas

# F.1	XML schema for MCPTT Information

## F.1.1	General

This subclause defines XML schema and MIME type for MCPTT information.

## F.1.2	XML schema

<?xml version="1.0" encoding="UTF-8"?>

<xs:schema

xmlns:xs="http://www.w3.org/2001/XMLSchema"

targetNamespace="urn:3gpp:ns:mcpttInfo:1.0"

xmlns:mcpttinfo="urn:3gpp:ns:mcpttInfo:1.0"

elementFormDefault="qualified"

attributeFormDefault="unqualified"

xmlns:xenc="http://www.w3.org/2001/04/xmlenc#">

<xs:import namespace="http://www.w3.org/2001/04/xmlenc#"/>

<!-- root XML element -->

<xs:element name="mpcttinfo" type="mcpttinfo:mcpttinfo-Type" id="info"/>

<xs:complexType name="mcpttinfo-Type">

<xs:sequence>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="mcpttinfo:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:element name="mcptt-Params" type="mcpttinfo:mcptt-ParamsType"/>

<xs:complexType name="mcptt-ParamsType">

<xs:sequence>

<xs:element name="mcptt-access-token" type="mcpttinfo:contentType" minOccurs="0" maxOccurs="1"/>

<xs:element name="session-type" type="xs:string" minOccurs="0" maxOccurs="1"/>

<xs:element name="mcptt-request-uri" type="mcpttinfo:contentType" minOccurs="0" maxOccurs="1"/>

<xs:element name="mcptt-calling-user-id" type="mcpttinfo:contentType" minOccurs="0" maxOccurs="1"/>

<xs:element name="mcptt-called-party-id" type="mcpttinfo:contentType" minOccurs="0" maxOccurs="1"/>

<xs:element name="mcptt-calling-group-id" type="mcpttinfo:contentType" minOccurs="0" maxOccurs="1"/>

<xs:element name="required" type="mcpttinfo:contentType" minOccurs="0" maxOccurs="1"/>

<xs:element name="emergency-ind" type="mcpttinfo:contentType" minOccurs="0" maxOccurs="1"/>

<xs:element name="alert-ind" type="mcpttinfo:contentType" minOccurs="0" maxOccurs="1"/>

<xs:element name="imminentperil-ind" type="xs:boolean" minOccurs="0" maxOccurs="1"/>

<xs:element name="broadcast-ind" type="xs:boolean" minOccurs="0" maxOccurs="1"/>

<xs:element name="mc-org" type="xs:string" minOccurs="0" maxOccurs="1"/>

<xs:element name="floor-state" type="xs:string" minOccurs="0" maxOccurs="1"/>

<xs:element name="associated-group-id" type="xs:string" minOccurs="0" maxOccurs="1"/>

<xs:element name="originated-by" type="xs:string" minOccurs="0" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="mcpttinfo:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:simpleType name="protectionType">

<xs:restriction base="xs:string">

<xs:enumeration value="Normal"/>

<xs:enumeration value="Encrypted"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="contentType">

<xs:choice minOccurs="1" maxOccurs="1">

<xs:element name="mcpttURI" type="xs:anyURI"/>

<xs:element name="mcpttString" type="xs:string"/>

<xs:element name="mcpttBoolean" type="xs:boolean"/>

<xs:any namespace="##other" processContents="lax"/>

<xs:element name="anyExt" type="mcpttinfo:anyExtType" minOccurs="0"/>

</xs:choice>

<xs:attribute name="type" type="protectionType"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="anyExtType">

<xs:sequence>

<xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:schema>

## F.1.3	Semantic

The <mcpttinfo> element is the root element of the XML document. The <mcpttinfo> element can contain subelements.

NOTE 1:	The subelements of the <mcptt-info> are validated by the <xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> particle of the <mcptt-info> element

If the <mcpttinfo> contains the <mcptt-Params> element then:

1)	the <mcptt-access-token>, <mcptt-request-uri>, <mcptt-calling-user-id>, <mcptt-called-party-id>, <mcptt-calling-group-id>, <emergency-ind>, <alert-ind> and <imminentperil-ind> can be included with encrypted content;

2)	for each element in 1) that is included with content that is not encrypted:

a)	the element has the "type" attribute set to "Normal";

b)	if the element is the <mcptt-request-uri>, <mcptt-calling-user-id>, <mcptt-called-party-id> or <mcptt-calling-group-id> then the <mcpttURI> element is included;

c)	if the element is the <mcptt-access-token>, then the <mcpttString> element is included; and

d)	if the element is <emergency-ind>, <alert-ind> or <imminentperil-ind> elements then the <mcpttBoolean> element is included;

3)	for each element in 1) that is included with content that is encrypted:

a)	the element has the "type" attribute set to "Encrypted";

b)	the <xenc:EncryptedData> element from the "http://www.w3.org/2001/04/xmlenc#" namespace is included and:

i)	can have a "Type" attribute can be included with a value of "http://www.w3.org/2001/04/xmlenc#Content";

ii)	can include an <EncryptionMethod> element with the "Algorithm" attribute set to value of "http://www.w3.org/2009/xmlenc11#aes128-gcm";

iii)	can include a <KeyInfo> element with a <KeyName> element containing the base 64 encoded XPK-ID; and

iv)	includes a <CipherData> element with a <CipherValue> element containing the encrypted data.

NOTE 2:	When the optional attributes and elements are not included within the <xenc:EncryptedData> element, the information they contain is known to sender and the receiver by other means.

If the <mcpttinfo> contains the <mcptt-Params> element then:

1)	the <mcptt-access-token> can be included with the access token received during authentication procedure as described in 3GPP TS 24.382 [49];

2)	the <session-type> can be included with:

a)	a value of "chat" to indicate that the MCPTT client wants to join a chat group call

b)	a value of "prearranged" to indicate the MCPTT client wants to make a prearranged group call; or

c)	a value of "private" to indicate the MCPTT client wants to make a private call;

3)	the <mcptt-request-uri> can be included with:

a)	a value set to an MCPTT group ID or temporary MCPTT group ID when the <session-type> is set to a value of "prearranged" or "chat"; and

b)	a value set to the MCPTT ID of the called MCPTT user when the <session-type> is set to a value of "private";

4)	the <mcptt-calling-user-id> can be included, set to MCPTT ID of the originating user;

5)	the <mcptt-called-party-id> can be included, set to the MCPTT ID of the terminating user;

6)	the <mcptt-calling-group-id> can be included to indicate the MCPTT group identity to the terminating user;

7)	the <required> can be included in a SIP 183 (Session Progress) from a non-controlling MCPTT function of an MCPTT group to inform the controlling MCPTT function that the group on the non-controlling MCPTT function has group members in the group document which are marked as <on-network-required>, as specified in 3GPP TS 24.381 [31];

8)	the <emergency-ind> can be:

a)	set to "true" to indicate that the call that the MCPTT client is initiating is an emergency MCPTT call; or

b)	set to "false" to indicate that the MCPTT client is cancelling an emergency MCPTT call (i.e. converting it back to a non-emergency call)

9)	the <alert-ind> can be:

a)	set to "true" in an emergency call initiation to indicate that an alert to be sent; or

b)	set to "false" when cancelling an emergency call which requires an alert to be cancelled also

10)	if the <session-type> is set to "chat" or "prearranged":

a)	the <imminentperil-ind> can be set to "true" to indicate that the call that the MCPTT client is initiating is an imminent peril group MCPTT call;

11)	the <broadcast-ind> can be:

a)	set to "true" indicates that the MCPTT client is initiating a broadcast group call; or

b)	set to "false" indicates that the MCPTT client is initiating a non-broadcast group call;

12)	the <mc-org> can be:

a)	set to the MCPTT user's Mission Critical Organization in an emergency alert sent by the MCPTT server to terminating MCPTT clients;

13)	the <floor-state> can be:

a)	set to "floor-idle", if the floor is idle in a non-controlling MCPTT function; or

b)	set to "floor-taken" if the floor state in a non-controlling MCPTT function is taken;

14)	the <associated-group-id>:

a)	if the <mcptt-request-uri> element contains a group identity then this element can include an MCPTT group ID associated with the group identity in the <mcptt-request-uri> element. E.g. if the <mcptt-request-uri> element contains a temporary group identity (TGI), then the <associated-group-id> element can contain the constituent MCPTT group ID; and

15)	the <originated-by>:

a)	can be included, set to the MCPTT ID of the originating user of an MCPTT emergency alert when being cancelled by another authorised MCPTT user.

Absence of the <emergency-ind>, <alert-ind> and <imminentperil-ind> in a SIP INVITE request indicates that the MCPTT client is initiating a non-emergency private call or non-emergency group call.

Absence of the <broadcast-ind> in a SIP INVITE request indicates that the MCPTT client is initiating a non-broadcast group call.

Absence of the <floor-state> in a SIP 200 (OK) response from the non-controlling MCPTT function indicates that the floor is idle.

The recipient of the XML ignores any unknown element and any unknown attribute.

## F.1.4	IANA registration template

Editor"s note [CT1#95, C1-154478]: The MIME type application/vnd.3gpp.mcptt-info +xml as defined in this subclause is to be registered in the IANA registry for Application Media Types based upon the following template. The registration is to be started when work on the MCPTT-CT WID completes.

Your Name:

<MCC name>

Your Email Address:

<MCC email address>

Media Type Name:

Application

Subtype name:

vnd.3gpp.mcptt-info+xml

Required parameters:

# None

Optional parameters:

"charset"	the parameter has identical semantics to the charset parameter of the "application/xml" media type as specified in section 9.1 of IETF RFC 7303.

Encoding considerations:

binary.

Security considerations:

Same as general security considerations for application/xml media type as specified in section 9.1 of IETF RFC 7303. In addition, this media type provides a format for exchanging information in SIP, so the security considerations from IETF RFC 3261 apply.

The information transported in this media type does not include active or executable content.

Mechanisms for privacy and integrity protection of protocol parameters exist. Those mechanisms as well as authentication and further security mechanisms are described in 3GPP TS 24.229.

This media type does not include provisions for directives that institute actions on a recipient's files or other resources.

This media type does not include provisions for directives that institute actions that, while not directly harmful to the recipient, may result in disclosure of information that either facilitates a subsequent attack or else violates a recipient's privacy in any way.

This media type does not employ compression.

Interoperability considerations:

Same as general interoperability considerations for application/xml media type as specified in section 9.1 of IETF RFC 7303. Any unknown XML elements and any unknown XML attributes are to be ignored by recipient of the MIME body.

Published specification:

3GPP TS 24.379 "Mission Critical Push To Talk (MCPTT) call control" version 13.0.0, available via http://www.3gpp.org/specs/numbering.htm.

Applications which use this media type:

Applications supporting the service continuity as described in the published specification.

Fragment identifier considerations:

The handling in section 5 of IETF RFC 7303 applies.

Restrictions on usage:

None

Provisional registration? (standards tree only):

N/A

Additional information:

1.	Deprecated alias names for this type: none

2.	Magic number(s): none

3.	File extension(s): none

4.	Macintosh File Type Code(s): none

5.	Object Identifier(s) or OID(s): none

Intended usage:

Common

Person to contact for further information:

-	Name: <MCC name>

-	Email: <MCC email address>

-	Author/Change controller:

i)	Author: 3GPP CT1 Working Group/3GPP_TSG_CT_WG1@LIST.ETSI.ORG

ii)	Change controller: <MCC name>/<MCC email address>

# F.2	XML schema for MBMS usage information

## F.2.1	General

This subclause defines XML schema and MIME type for application/vnd.3gpp.mcptt-mbms-usage-info+xml.

## F.2.2	XML schema

<?xml version="1.0" encoding="UTF-8"?>

<xs:schema attributeFormDefault="unqualified" elementFormDefault="qualified"

xmlns:xs="http://www.w3.org/2001/XMLSchema"

targetNamespace="urn:3gpp:ns:mcpttMbmsUsage:1.0"

xmlns:mcpttmbms="urn:3gpp:ns:mcpttMbmsUsage:1.0">

<!-- the root element -->

<xs:element name="mcptt-mbms-usage-info" type="mcpttmbms:mcptt-mbms-usage-info-Type" id="mbms"/>

<xs:complexType name="mcptt-mbms-usage-info-Type">

<xs:sequence>

<xs:element name="mbms-listening-status" type="mcpttmbms:mbms-listening-statusType" 
			minOccurs="0"/>

<xs:element name="announcement" type="mcpttmbms:announcementTypeParams" minOccurs="0"/>

<xs:element name="GPMS" type="xs:positiveInteger" minOccurs="0" maxOccurs="1"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="mcpttmbms:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="mbms-listening-statusType">

<xs:sequence>

<xs:element name="mbms-listening-status" type="xs:string"/>

<xs:element name="session-id" type="xs:anyURI" minOccurs="0"/>

<xs:element name="general-purpose" type="xs:boolean" minOccurs="0"/>

<xs:element name="version" type="xs:integer"/>

<xs:element name="TMGI" type="xs:hexBinary" maxOccurs="unbounded"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="mcpttmbms:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="announcementTypeParams">

<xs:sequence>

<xs:element name="TMGI" type="xs:hexBinary" minOccurs="1"/>

<xs:element name="QCI" type="xs:integer" minOccurs="1"/>

<xs:element name="frequency" type="xs:unsignedLong" minOccurs="0"/>

<xs:element name="mbms-service-areas" type="xs:hexBinary" minOccurs="1" maxOccurs="unbounded"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="mcpttmbms:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="anyExtType">

<xs:sequence>

<xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:schema>

## F.2.3	Semantic

This subclause describes the elements of the MBMS usage information XML Schema.

<mbms-listening-status>:	The <mbms-listening-status> element is used to indicate the MCPTT listening status.

-	The value "listening" indicates that the MCPTT client now is receiving RTP media packets and floor control messages over the MBMS subchannel in the session identified by the <session-id> element or if the <general-purpose> element is set to "true", that the MCPTT client is now listening to the general purpose MBMS subchannel.

-	The value "not-listening" indicates that the MCPTT client has stopped listening to the MBMS subchannel in the session identified by the <session-id> element or, if the <general-purpose> element is set to "false", that the MCPTT client no longer listens to the general purpose MBMS subchannel.

Table F.2.3-1 shows the ABNF of the <mbms-listening-status> element.

Table F.2.3-1: ABNF syntax of values of the <mbms-listening-status> element

mbms-listening-status = listening-value / not-listening-value

listening-value = %x6c.69.73.74.65.6e.69.6e.67 ; "listening"

not-listening-value = %x6e.6f.74.2d.6c.69.73.74.65.6e.69.6e.67 ; "not-listening"

<session-id>:	contains the value of the URI received in the Contact header field received from the controlling MCPTT function when on on-demand session was established, or from the participating MCPTT function in the Connect message when the session was established over a pre-established session.

<TMGI>:	Contains the TMGI. The <TMGI> element is coded as described in 3GPP TS 24.008 [43] subclause 10.5.6.13 excluding the Temporary Mobile Group Identity IEI and Length of Temporary Mobile Group Identity contents (octet 1 and octet 2 in 3GPP TS 24.008 [43] subclause 10.5.6.13).

<QCI>:	QCI information used by the ProSe UE-Network Relay to determine the ProSe Per-Packet Priority value to be applied for the multicast packets relayed to Remote UE over PC5. QCI values are defined in 3GPP TS 23.203 [41].

<mbms-service-areas>:	A list of MBMS service area IDs for the applicable MBMS broadcast area as specified in 3GPP TS 23.003 [40] for Service Area Identifier (SAI).

<Frequency>:	Identification of frequency in case of multi carrier support. The <Frequency> element is coded as specified in 3GPP TS 29.468 [42].

NOTE:	In the current release the frequency in the <frequency> element is the same as the frequency used for unicast.

<SDP-ref>:	A URL with a cid url as specified in IETF RFC 5368 [38] referring to a SDP MIME body in the SIP MESSAGE request.

<general-purpose>	An integer referring to which media line in the SDP where the general purpose MBMS subchannel is declared.

<GPMS>	A positive integer that gives the number of the media line containing the general purpose MBMS subchannel in the application/sdp MIME body attached to the SIP MESSAGE request containing the MBMS announcements.

<version>	this element indicates the version of the application/vnd.3gpp.mbms-usage-info MIME body. In this version the <version element> indicates "1".

The recipient of the XML ignores any unknown element and any unknown attribute.

## F.2.4	IANA registration template

Editor"s note [CT1#95-bis, C1-160397]: The MIME type application/vnd.3gpp.mcptt-mbms-usage-info+xml as defined in this subclause is to be registered in the IANA registry for Application Media Types based upon the following template. The registration is to be started when work on the MCPTT-CT WID completes.

Your Name:

<MCC name>

Your Email Address:

<MCC email address>

Media Type Name:

Application

Subtype name:

vnd.3gpp.mcptt-mbms-usage-info+xml

Required parameters:

None

Optional parameters:

"charset"	the parameter has identical semantics to the charset parameter of the "application/xml" media type as specified in section 9.1 of IETF RFC 7303.

Encoding considerations:

binary.

Security considerations:

Same as general security considerations for application/xml media type as specified in section 9.1 of IETF RFC 7303. In addition, this media type provides a format for exchanging information in SIP, so the security considerations from IETF RFC 3261 apply.

The information transported in this media type does not include active or executable content.

Mechanisms for privacy and integrity protection of protocol parameters exist. Those mechanisms as well as authentication and further security mechanisms are described in 3GPP TS 24.229.

This media type does not include provisions for directives that institute actions on a recipient's files or other resources.

This media type does not include provisions for directives that institute actions that, while not directly harmful to the recipient, may result in disclosure of information that either facilitates a subsequent attack or else violates a recipient's privacy in any way.

This media type does not employ compression.

Interoperability considerations:

Same as general interoperability considerations for application/xml media type as specified in section 9.1 of IETF RFC 7303. Any unknown XML elements and any unknown XML attributes are to be ignored by recipient of the MIME body.

Published specification:

3GPP TS 24.379 "Mission Critical Push To Talk (MCPTT) call control" version 13.0.0, available via http://www.3gpp.org/specs/numbering.htm.

Applications which use this media type:

Applications supporting the service continuity as described in the published specification.

Fragment identifier considerations:

The handling in section 5 of IETF RFC 7303 applies.

Restrictions on usage:

None

Provisional registration? (standards tree only):

N/A

Additional information:

1.	Deprecated alias names for this type: none

2.	Magic number(s): none

3.	File extension(s): none

4.	Macintosh File Type Code(s): none

5.	Object Identifier(s) or OID(s): none

Intended usage:

Common

Person to contact for further information:

-	Name: <MCC name>

-	Email: <MCC email address>

-	Author/Change controller:

i)	Author: 3GPP CT1 Working Group/3GPP_TSG_CT_WG1@LIST.ETSI.ORG

ii)	Change controller: <MCC name>/<MCC email address>

# F.3	XML schema for MCPTT location information

## F.3.1	General

This subclause defines the XML schema and the MIME type for location information.

## F.3.2	XML schema

<?xml version="1.0" encoding="UTF-8"?>

<xs:schema xmlns:xs="http://www.w3.org/2001/XMLSchema" xmlns:mcpttloc="urn:3gpp:ns:mcpttLocationInfo:1.0" targetNamespace="urn:3gpp:ns:mcpttLocationInfo:1.0" elementFormDefault="qualified" attributeFormDefault="unqualified"

xmlns:xenc="http://www.w3.org/2001/04/xmlenc#">

<xs:import namespace="http://www.w3.org/2001/04/xmlenc#"/>

<xs:element name="location-info" id="loc">

<xs:annotation>

<xs:documentation>Root element, contains all information related to location configuration, location request and location reporting for the MCPTT service</xs:documentation>

</xs:annotation>

<xs:complexType>

<xs:choice>

<xs:element name="Configuration" type="mcpttloc:tConfigurationType"/>

<xs:element name="Request" type="mcpttloc:tRequestType"/>

<xs:element name="Report" type="mcpttloc:tReportType"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="mcpttloc:anyExtType" minOccurs="0"/>

</xs:choice>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

</xs:element>

<xs:complexType name="tConfigurationType">

<xs:sequence>

<xs:element name="NonEmergencyLocationInformation" type="mcpttloc:tRequestedLocationType" minOccurs="0"/>

<xs:element name="EmergencyLocationInformation" type="mcpttloc:tRequestedLocationType" minOccurs="0"/>

<xs:element name="TriggeringCriteria" type="mcpttloc:TriggeringCriteriaType"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="mcpttloc:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:attribute name="ConfigScope">

<xs:simpleType>

<xs:restriction base="xs:string">

<xs:enumeration value="Full"/>

<xs:enumeration value="Update"/>

</xs:restriction>

</xs:simpleType>

</xs:attribute>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tRequestType">

<xs:complexContent>

<xs:extension base="mcpttloc:tEmptyType">

<xs:attribute name="RequestId" type="xs:string" use="required"/>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="tReportType">

<xs:sequence>

<xs:element name="TriggerId" type="xs:string" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="CurrentLocation" type="mcpttloc:tCurrentLocationType"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="mcpttloc:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:attribute name="ReportID" type="xs:string" use="optional"/>

<xs:attribute name="ReportType" use="required">

<xs:simpleType>

<xs:restriction base="xs:string">

<xs:enumeration value="Emergency"/>

<xs:enumeration value="NonEmergency"/>

</xs:restriction>

</xs:simpleType>

</xs:attribute>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="TriggeringCriteriaType">

<xs:sequence>

<xs:element name="CellChange" type="mcpttloc:tCellChange" minOccurs="0"/>

<xs:element name="TrackingAreaChange" type="mcpttloc:tTrackingAreaChangeType" minOccurs="0"/>

<xs:element name="PlmnChange" type="mcpttloc:tPlmnChangeType" minOccurs="0"/>

<xs:element name="MbmsSaChange" type="mcpttloc:tMbmsSaChangeType" minOccurs="0"/>

<xs:element name="MbsfnAreaChange" type="mcpttloc:tMbsfnAreaChangeType" minOccurs="0"/>

<xs:element name="PeriodicReport" type="mcpttloc:tIntegerAttributeType" minOccurs="0"/>

<xs:element name="TravelledDistance" type="mcpttloc:tIntegerAttributeType" minOccurs="0"/>

<xs:element name="McpttSignallingEvent" type="mcpttloc:tSignallingEventType" minOccurs="0"/>

<xs:element name="GeographicalAreaChange" type="mcpttloc:tGeographicalAreaChange"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="mcpttloc:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tCellChange">

<xs:sequence>

<xs:element name="AnyCellChange" type="mcpttloc:tEmptyTypeAttribute" minOccurs="0"/>

<xs:element name="EnterSpecificCell" type="mcpttloc:tSpecificCellType" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="ExitSpecificCell" type="mcpttloc:tSpecificCellType" minOccurs="0" maxOccurs="unbounded"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="mcpttloc:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tEmptyType"/>

<xs:simpleType name="tEcgi">

<xs:restriction base="xs:string">

<xs:pattern value="\d{3}\d{3}[0-1]{28}"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="tSpecificCellType">

<xs:simpleContent>

<xs:extension base="mcpttloc:tEcgi">

<xs:attribute name="TriggerId" type="xs:string" use="required"/>

</xs:extension>

</xs:simpleContent>

</xs:complexType>

<xs:complexType name="tEmptyTypeAttribute">

<xs:complexContent>

<xs:extension base="mcpttloc:tEmptyType">

<xs:attribute name="TriggerId" type="xs:string" use="required"/>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<xs:complexType name="tTrackingAreaChangeType">

<xs:sequence>

<xs:element name="AnyTrackingAreaChange" type="mcpttloc:tEmptyTypeAttribute" minOccurs="0"/>

<xs:element name="EnterSpecificTrackingArea" type="mcpttloc:tTrackingAreaIdentity" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="ExitSpecificTrackingArea" type="mcpttloc:tTrackingAreaIdentity" minOccurs="0" maxOccurs="unbounded"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="mcpttloc:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:simpleType name="tTrackingAreaIdentityFormat">

<xs:restriction base="xs:string">

<xs:pattern value="\d{3}\d{3}[0-1]{16}"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="tTrackingAreaIdentity">

<xs:simpleContent>

<xs:extension base="mcpttloc:tTrackingAreaIdentityFormat">

<xs:attribute name="TriggerId" type="xs:string" use="required"/>

</xs:extension>

</xs:simpleContent>

</xs:complexType>

<xs:complexType name="tPlmnChangeType">

<xs:sequence>

<xs:element name="AnyPlmnChange" type="mcpttloc:tEmptyTypeAttribute" minOccurs="0"/>

<xs:element name="EnterSpecificPlmn" type="mcpttloc:tPlmnIdentity" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="ExitSpecificPlmn" type="mcpttloc:tPlmnIdentity" minOccurs="0" maxOccurs="unbounded"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="mcpttloc:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:simpleType name="tPlmnIdentityFormat">

<xs:restriction base="xs:string">

<xs:pattern value="\d{3}\d{3}"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="tPlmnIdentity">

<xs:simpleContent>

<xs:extension base="mcpttloc:tPlmnIdentityFormat">

<xs:attribute name="TriggerId" type="xs:string" use="required"/>

</xs:extension>

</xs:simpleContent>

</xs:complexType>

<xs:complexType name="tMbmsSaChangeType">

<xs:sequence>

<xs:element name="AnyMbmsSaChange" type="mcpttloc:tEmptyTypeAttribute" minOccurs="0"/>

<xs:element name="EnterSpecificMbmsSa" type="mcpttloc:tMbmsSaIdentity" minOccurs="0"/>

<xs:element name="ExitSpecificMbmsSa" type="mcpttloc:tMbmsSaIdentity" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="mcpttloc:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:simpleType name="tMbmsSaIdentityFormat">

<xs:restriction base="xs:integer">

<xs:minInclusive value="0"/>

<xs:maxInclusive value="65535"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="tMbmsSaIdentity">

<xs:simpleContent>

<xs:extension base="mcpttloc:tMbmsSaIdentityFormat">

<xs:attribute name="TriggerId" type="xs:string" use="required"/>

</xs:extension>

</xs:simpleContent>

</xs:complexType>

<xs:complexType name="tMbsfnAreaChangeType">

<xs:sequence>

<xs:element name="EnterSpecificMbsfnArea" type="mcpttloc:tMbsfnAreaIdentity" minOccurs="0"/>

<xs:element name="ExitSpecificMbsfnArea" type="mcpttloc:tMbsfnAreaIdentity" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="mcpttloc:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:simpleType name="tMbsfnAreaIdentityFormat">

<xs:restriction base="xs:integer">

<xs:minInclusive value="0"/>

<xs:maxInclusive value="255"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="tMbsfnAreaIdentity">

<xs:simpleContent>

<xs:extension base="mcpttloc:tMbsfnAreaIdentityFormat">

<xs:attribute name="TriggerId" type="xs:string" use="required"/>

</xs:extension>

</xs:simpleContent>

</xs:complexType>

<xs:complexType name="tIntegerAttributeType">

<xs:simpleContent>

<xs:extension base="xs:integer">

<xs:attribute name="TriggerId" type="xs:string" use="required"/>

</xs:extension>

</xs:simpleContent>

</xs:complexType>

<xs:complexType name="tTravelledDistanceType">

<xs:sequence>

<xs:element name="TravelledDistance" type="xs:positiveInteger"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="mcpttloc:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tSignallingEventType">

<xs:sequence>

<xs:element name="InitialLogOn" type="mcpttloc:tEmptyTypeAttribute" minOccurs="0"/>

<xs:element name="GroupCallNonEmergency" type="mcpttloc:tEmptyTypeAttribute" minOccurs="0"/>

<xs:element name="PrivateCallNonEmergency" type="mcpttloc:tEmptyTypeAttribute" minOccurs="0"/>

<xs:element name="LocationConfigurationReceived" type="mcpttloc:tEmptyTypeAttribute" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type=" mcpttloc:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tEmergencyEventType">

<xs:sequence>

<xs:element name="GroupCallEmergency" type="mcpttloc:tEmptyTypeAttribute" minOccurs="0"/>

<xs:element name="GroupCallImminentPeril" type="mcpttloc:tEmptyTypeAttribute" minOccurs="0"/>

<xs:element name="PrivateCallEmergency" type="mcpttloc:tEmptyTypeAttribute" minOccurs="0"/>

<xs:element name="InitiateEmergencyAlert" type="mcpttloc:tEmptyTypeAttribute" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="mcpttloc:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tRequestedLocationType">

<xs:sequence>

<xs:element name="ServingEcgi" type="mcpttloc:tEmptyType" minOccurs="0"/>

<xs:element name="NeighbouringEcgi" type="mcpttloc:tEmptyType" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="MbmsSaId" type="mcpttloc:tEmptyType" minOccurs="0"/>

<xs:element name="MbsfnArea" type="mcpttloc:tEmptyType" minOccurs="0"/>

<xs:element name="GeographicalCordinate" type="mcpttloc:tEmptyType" minOccurs="0"/>

<xs:element name="minimumIntervalLength" type="xs:positiveInteger"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="mcpttloc:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tCurrentLocationType">

<xs:sequence>

<xs:element name="CurrentServingEcgi" type="mcpttloc:tLocationType" minOccurs="0"/>

<xs:element name="NeighbouringEcgi" type="mcpttloc:tLocationType" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="MbmsSaId" type="mcpttloc:tLocationType" minOccurs="0"/>

<xs:element name="MbsfnArea" type="mcpttloc:tLocationType" minOccurs="0"/>

<xs:element name="CurrentCoordinate" type="mcpttloc:tPointCoordinate" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="mcpttloc:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:simpleType name="protectionType">

<xs:restriction base="xs:string">

<xs:enumeration value="Normal"/>

<xs:enumeration value="Encrypted"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="tLocationType">

<xs:choice minOccurs="1" maxOccurs="1">

<xs:element name="Ecgi" type="mcpttloc:tEcgi" minOccurs="0"/>

<xs:element name="SaId" type="mcpttloc:tMbmsSaIdentity" minOccurs="0"/>

<xs:element name="MbsfnAreaId" type="mcpttloc:tMbsfnAreaIdentity" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax"/>

<xs:element name="anyExt" type="mcpttinfo:anyExtType" minOccurs="0"/>

</xs:choice>

<xs:attribute name="type" type="protectionType"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tGeographicalAreaChange">

<xs:sequence>

<xs:element name="AnyAreaChange" type="mcpttloc:tEmptyTypeAttribute" minOccurs="0"/>

<xs:element name="EnterSpecificAreaType" type="mcpttloc:tSpecificAreaType" minOccurs="0"/>

<xs:element name="ExitSpecificAreaType" type="mcpttloc:tSpecificAreaType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="mcpttloc:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tSpecificAreaType">

<xs:sequence>

<xs:element name="GeographicalArea" type="mcpttloc:tGeographicalAreaDef"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="mcpttloc:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:attribute name="TriggerId" type="xs:string" use="required"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tPointCoordinate">

<xs:sequence>

<xs:element name="longitude" type="mcpttloc:tCoordinate"/>

<xs:element name="latitude" type="mcpttloc:tCoordinate"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="mcpttloc:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tCoordinateType">

<xs:choice minOccurs="1" maxOccurs="1">

<xs:element name="threebytes" type="mcpttloc:tThreeByteType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax"/>

<xs:element name="anyExt" type="mcpttinfo:anyExtType" minOccurs="0"/>

</xs:choice>

<xs:attribute name="type" type="protectionType"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:simpleType name="tThreeByteType">

<xs:restriction base="xs:integer">

<xs:minInclusive value="0"/>

<xs:maxInclusive value="16777215"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="tGeographicalAreaDef">

<xs:sequence>

<xs:element name="PolygonArea" type="mcpttloc:tPolygonAreaType" minOccurs="0"/>

<xs:element name="EllipsoidArcArea" type="mcpttloc:tEllipsoidArcType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="mcpttloc:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tPolygonAreaType">

<xs:sequence>

<xs:element name="Corner" type="mcpttloc:tPointCoordinate" minOccurs="3" maxOccurs="15"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="mcpttloc:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="tEllipsoidArcType">

<xs:sequence>

<xs:element name="Center" type="mcpttloc:tPointCoordinate"/>

<xs:element name="Radius" type="xs:nonNegativeInteger"/>

<xs:element name="OffsetAngle" type="xs:unsignedByte"/>

<xs:element name="IncludedAngle" type="xs:unsignedByte"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="mcpttloc:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="anyExtType">

<xs:sequence>

<xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:schema>

## F.3.3	Semantic

The <location-info> element is the root element of the XML document. The <location-info> element contains the <Configuration>, <Request> and <Report> subelements, of which only one can be present.

<Configuration> element has a <ConfigScope> attribute that can assume the values "Full" and "Update". The value "Full" means that the Configuration> element contains the full location configuration which replaces any previous location configuration. The value "Update" means that the location configuration is in addition to any previous location configuration. To remove configuration elements a "Full" configuration is needed. The <Configuration> element contains the following child elements:

1)	<NonEmergencyLocationInformation>, an optional element that specifies the location information requested in non-emergency situations. The <NonEmergencyLocationInformation> has the subelements:

a)	<ServingEcgi>, an optional element specifying that the serving E-UTRAN Cell Global Identity (ECGI) needs to be reported;

b)	<NeighbouringEcgi>, an optional element that can occur multiple times, specifying that neighbouring ECGIs need to be reported;

c)	<MbmsSaId>, an optional element specifying that the serving MBMS Service Area Id needs to be reported;

d)	<MbsfnArea>, an optional element specifying that the MBSFN area Id needs to be reported;

e)	<GeographicalCoordinate>, an optional element specifying that the geographical coordinate specified in subclause 6.1 in 3GPP TS 23.032 [54] needs to be reported; and

f)	<minimumIntervalLength>, a mandatory element specifying the minimum time the MCPTT client needs to wait between sending location reports. The value is given in seconds;

2)	<EmergencyLocationInformation>, an optional element that specifies the location information requested in emergency situations. The <EmergencyLocationInformation> has the subelements:

a)	<ServingEcgi>, an optional element specifying that the serving ECGI needs to be reported;

b)	<NeighbouringEcgi>, an optional element that can occur multiple times, specifying that neighbouring ECGIs need to be reported;

c)	<MbmsSaId>, an optional element specifying that the serving MBMS Service Area Id needs to be reported;

d)	<MbsfnArea>, an optional element specifying that the MBSFN area Id needs to be reported;

e)	<GeographicalCoordinate>, an optional element specifying that the geographical coordinate specified in subclause 6.1 in 3GPP TS 23.032 [54] needs to be reported; and

f)	<minimumIntervalLength>, a mandatory element specifying the minimum time the MCPTT client needs to wait between sending location reports. The value is given in seconds; and

3)	<TriggeringCriteria>, a mandatory element specifying the triggers for the MCPTT client to perform reporting. The <TriggeringCriteria> element contains the following sub-elements:

a)	<CellChange>, an optional element specifying what cell changes trigger location reporting. Consists of the following sub-elements:

I)	<AnyCellChange>, an optional element. The presence of this element specifies that any cell change is a trigger. Contains a mandatory <TriggerId> attribute that shall be set to a unique string;

II)	<EnterSpecificCell>, an optional element specifying an ECGI which when entered triggers a location report. Contains a mandatory <TriggerId> attribute that shall be set to a unique string; and

III)	<ExitSpecificCell>, an optional element specifying an ECGI which when exited triggers a location report. Contains a mandatory <TriggerId> attribute that shall be set to a unique string;

b)	<TrackingAreaChange>, an optional element specifying what tracking area changes trigger location reporting. Consists of the following sub-elements:

I)	<AnyTrackingAreaChange>, an optional element. The presence of this element specifies that any tracking area change is a trigger. Contains a mandatory <TriggerId> attribute that shall be set to a unique string;

II)	<EnterSpecificTrackingArea>, an optional element specifying a Tracking Area Id which when entered triggers a location report. Contains a mandatory <TriggerId> attribute that shall be set to a unique string; and

III)	<ExitSpecificTrackingArea>, an optional element specifying a Tracking Area Id which when exited triggers a location report. Contains a mandatory <TriggerId> attribute that shall be set to a unique string;

c)	<PlmnChange>, an optional element specifying what PLMN changes trigger location reporting. Consists of the following sub-elements:

I)	<AnyPlmnChange>, an optional element. The presence of this element specifies that any PLMN change is a trigger. Contains a mandatory <TriggerId> attribute that shall be set to a unique string;

II)	<EnterSpecificPlmn>, an optional element specifying a PLMN Id which when entered triggers a location report. Contains a mandatory <TriggerId> attribute that shall be set to a unique string; and

III)	<ExitSpecificPlmn>, an optional element specifying a PLMN Id which when exited triggers a location report. Contains a mandatory <TriggerId> attribute that shall be set to a unique string;

d)	<MbmsSaChange>, an optional element specifying what MBMS changes trigger location reporting. Consists of the following sub-elements:

I)	<AnyMbmsSaChange>, an optional element. The presence of this element specifies that any MBMS SA change is a trigger. Contains a mandatory <TriggerId> attribute that shall be set to a unique string;

II)	<EnterSpecificMbmsSa>, an optional element specifying an MBMS Service Area Id which when entered triggers a location report. Contains a mandatory <TriggerId> attribute that shall be set to a unique string; and

III)	<ExitSpecificMbmsSa>, an optional element specifying an MBMS Service Area Id which when exited triggers a location report. Contains a mandatory <TriggerId> attribute that shall be set to a unique string;

e)	<MbsfnAreaChange>, an optional element specifying what MBSFN changes trigger location reporting. Consists of the following sub-elements:

I)	<AnyMbsfnAreaChange>, an optional element. The presence of this element specifies that any MBSFN area change is a trigger. Contains a mandatory <TriggerId> attribute that shall be set to a unique string;

II)	<EnterSpecificMbsfnArea>, an optional element specifying an MBSFN area which when entered triggers a location report. Contains a mandatory <TriggerId> attribute that shall be set to a unique string; and

III)	<ExitSpecificMbsfnArea>, an optional element specifying an MBSFN area which when exited triggers a location report. Contains a mandatory <TriggerId> attribute that shall be set to a unique string;

f)	<PeriodicReport>, an optional element specifying that periodic location reports shall be sent. The value in seconds specifies the reporting interval. Contains a mandatory <TriggerId> attribute that shall be set to a unique string;

g)	<TravelledDistance>, an optional element specifying that the travelled distance shall trigger a report. The value in metres specified the travelled distance. Contains a mandatory <TriggerId> attribute that shall be set to a unique string;

h)	<McpttSignallingEvent>, an optional element specifying what signalling events triggers a location report. The <McpttSignallingEvent> element has the following sub-elements:

I)	<InitialLogOn>, an optional element specifying that an initial log on triggers a location report. Contains a mandatory <TriggerId> attribute that shall be set to a unique string;

II)	<GroupCallNonEmergency>, an optional element specifying that a non-emergency group call triggers a location report. Contains a mandatory <TriggerId> attribute that shall be set to a unique string;

III)	<PrivateCallNonEmergency>, an optional element specifying that a non-emergency private call triggers a location report. Contains a mandatory <TriggerId> attribute that shall be set to a unique string; and

IV)	<LocationConfigurationReceived>, an optional element specifying that a received location configuration triggers a location report. Contains a mandatory <TriggerId> attribute that shall be set to a unique string; and

i)	<GeographicalAreaChange>, an optional element specifying what geographical are changes trigger location reporting. Consists of the following sub-elements:

I)	<AnyAreaChange>, an optional element. The presence of this element specifies that any geographical area change is a trigger. Contains a mandatory <TriggerId> attribute that shall be set to a unique string;

II)	<EnterSpecificArea>, an optional element specifying a geographical area which when entered triggers a location report. Contains a mandatory <TriggerId> attribute that shall be set to a unique string. The <EnterSpecificArea> element has the following sub-elements:

A)	<GeographicalArea>, an optional element containing a <TriggerId> attribute and the following two subelements:

x1)	<PolygonArea>, an optional element specifying the area as a polygon specified in subclause 5.2 in 3GPP TS 23.032 [54]; and

x2)	<EllipsoidArcArea>, an optional element specifying the area as an Ellipsoid Arc specified in subclause 5.7 in 3GPP TS 23.032 [54]; and

III)	<ExitSpecificAreaType>, an optional element specifying a geographical area which when exited triggers a location report. Contains a mandatory <TriggerId> attribute that shall be set to a unique string.

<Request> is an element with a <RequestId> attribute. The <Request> element is used to request a location report. The value of the <RequestId> attribute is returned in the corresponding <ReportId> attribute in order to correlate the request and the report.

<Report> is an element used to include the location report. It contains a <ReportId> attribute and a <ReportType> attribute. The <ReportId> attribute is used to return the value in the <RequestId> attribute in the <Request> element. The <ReportType> attribute has two values "Emergency" and "NonEmergency" used to inform whether the client is sending the report in an emergency situation or not. The <Report> element contains the following sub-elements:

1)	<TriggerId>, an optional element which can occur multiple times that contain the value of the <TriggerId> attribute associated with a trigger that has fired; and

2)	<CurrentLocation>, a mandatory element that contains the location information. The <CurrentLocation> element contains the following sub-elements:

a)	<CurrentServingEcgi>, an optional element containing the ECGI of the serving cell;

b)	<NeighbouringEcgi>, an optional element that can occur multiple times. It contains the ECGI of any neighbouring cell the MCPTT client can detect;

c)	<MbmsSaId>, an optional element containing the MBMS Service Area Id the MCPTT client is using;

d)	<MbsfnArea>, an optional element containing the MBSFN area the MCPTT is located in; and

e)	<CurrentCoordinate>, an optional element containing the longitude and latitude coded as in subclause 6.1 in 3GPP TS 23.032 [54].

The contents of the subelements in the <CurrentLocation> subelement of the <Report> element can be encrypted. The following rules are applied when any of these elements are included:

1)	if confidentiality protection is not required, then:

a)	the "type" attributes associated with the <CurrentServingEcgi>, <NeighbouringEcgi>, <MbmsSaId>, and <MbsfnArea> elements of the <Report> element have the value "Normal" and

ii)	the <Ecgi> subelement of the <CurrentServingEcgi> element contains the unencrypted value of the ECGI of the serving cell;

iii)	the <Ecgi> subelement of the <NeighbouringEcgi> element contains the unencrypted value of the ECGI of any neighbouring cell;

iv)	the <SaId> subelement of the <MbmsSaId> element contains the unencrypted value of the MBMS Service Area Id the MCPTT client is using; and

v)	the <MbsfnAreaId> subelement of the <MbsfnArea>, element contains the unencrypted value of the MBSFN area the MCPTT is located in;

b)	the "type" attributes associated with the <longitude> and <latitude> subelements of the <CurrentCoordinate> element have the value "Normal" and the <three-bytes> subelements of <longitude> and <latitude> subelements contain the unencrypted value of longitude and latitude.

2)	if confidentiality protection is required, then:

a)	the "type" attributes associated with the <CurrentServingEcgi>, <NeighbouringEcgi>, <MbmsSaId>, and <MbsfnArea> elements have the value "Encrypted";

b)	the "type" attributes associated with the <longitude> and <latitude> subelements of the <CurrentCoordinate> element have the value "Encrypted";

c)	for each of the elements described in 2a) and subelements described in 2b) above, the <xenc:EncryptedData> element from the "http://www.w3.org/2001/04/xmlenc#" namespace is included and:

i)	can have a "Type" attribute can be included with a value of "http://www.w3.org/2001/04/xmlenc#Content";

ii)	can include an <EncryptionMethod> element with the "Algorithm" attribute set to value of "http://www.w3.org/2009/xmlenc11#aes128-gcm";

iii)	can include a <KeyInfo> element with a <KeyName> element containing the base 64 encoded XPK-ID; and

iv)	includes a <CipherData> element with a <CipherValue> element containing the encrypted data.

NOTE:	When the optional attributes and elements are not included within the <xenc:EncryptedData> element, the information they contain is known to sender and the receiver by other means.

The recipient of the XML ignores any unknown element and any unknown attribute.

## F.3.4	IANA registration template

Editor"s note [CT1#95-bis, C1-160453]: The application/vnd.3gpp.mcptt-location-info+xml MIME type as defined in this subclause needs to be registered at completion of Release-13.

Your Name:

<MCC name>

Your Email Address:

<MCC email address>

Media Type Name:

Application

Subtype name:

vnd.3gpp. mcptt-location-info+xml

Required parameters:

None

Optional parameters:

"charset"	the parameter has identical semantics to the charset parameter of the "application/xml" media type as specified in section 9.1 of IETF RFC 7303.

Encoding considerations:

binary.

Security considerations:

Same as general security considerations for application/xml media type as specified in section 9.1 of IETF RFC 7303. In addition, this media type provides a format for exchanging information in SIP, so the security considerations from IETF RFC 3261 apply.

The information transported in this media type does not include active or executable content.

Mechanisms for privacy and integrity protection of protocol parameters exist. Those mechanisms as well as authentication and further security mechanisms are described in 3GPP TS 24.229.

This media type does not include provisions for directives that institute actions on a recipient's files or other resources.

This media type does not include provisions for directives that institute actions that, while not directly harmful to the recipient, may result in disclosure of information that either facilitates a subsequent attack or else violates a recipient's privacy in any way.

This media type does not employ compression.

Interoperability considerations:

Same as general interoperability considerations for application/xml media type as specified in section 9.1 of IETF RFC 7303. Any unknown XML elements and any unknown XML attributes are to be ignored by recipient of the MIME body.

Published specification:

3GPP TS 24.379 "Mission Critical Push To Talk (MCPTT) call control" version 13.0.0, available via http://www.3gpp.org/specs/numbering.htm.

Applications which use this media type:

Applications supporting the service continuity as described in the published specification.

Fragment identifier considerations:

The handling in section 5 of IETF RFC 7303 applies.

Restrictions on usage:

None

Provisional registration? (standards tree only):

N/A

Additional information:

1.	Deprecated alias names for this type: none

2.	Magic number(s): none

3.	File extension(s): none

4.	Macintosh File Type Code(s): none

5.	Object Identifier(s) or OID(s): none

Intended usage:

Common

Person to contact for further information:

-	Name: <MCC name>

-	Email: <MCC email address>

-	Author/Change controller:

i)	Author: 3GPP CT1 Working Group/3GPP_TSG_CT_WG1@LIST.ETSI.ORG

ii)	Change controller: <MCC name>/<MCC email address>

# F.4	XML schema for MCPTT (de)-affiliation requests

## F.4.1	General

This subclause defines XML schema and MIME type for MCPTT (de)-affiliation requests.

## F.4.2	XML schema

<?xml version="1.0" encoding="UTF-8"?>

<xs:schema xmlns:xs="http://www.w3.org/2001/XMLSchema"

targetNamespace="urn:3gpp:ns:affiliationCommand:1.0"

xmlns:mcpttaff="urn:3gpp:ns:affiliationCommand:1.0"

attributeFormDefault="unqualified" elementFormDefault="qualified">

<xs:complexType name="affiliate-command"> id="affil">

<xs:sequence>

<xs:element type="xs:anyURI" name="group" minOccurs="1" maxOccurs="unbounded"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="mcpttaff:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="de-affiliate-command">

<xs:sequence>

<xs:element type="xs:anyURI" name="group" minOccurs="1" maxOccurs="unbounded"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="mcpttaff:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:element name="command-list">

<xs:complexType>

<xs:sequence>

<xs:element name="affiliate" type="mcpttaff:affiliate-command" minOccurs="0" maxOccurs="1"/>

<xs:element name="de-affiliate" type="mcpttaff:de-affiliate-command" minOccurs="0" maxOccurs="1"/>

<xs:element name="anyExt" type="mcpttaff:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:element>

<xs:complexType name="anyExtType">

<xs:sequence>

<xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:schema>

## F.4.3	Semantic

The <command-list> element is the root element of the XML document. The <command-list> element may contain <affiliate-command>, or <de-affiliate-command> subelements or both.

If the <command-list> contains the <affiliate-command> element then:

1)	the <affiliate-command> element contains a list of <group> subelements having at least one subelement. The recipient shall perform an affiliation for all the MCPTT groups contained in the list for the clients for which the <command-list> applies.

If the <command-list> contains the <de-affiliate-command> element then:

1)	the <de-affiliate-command> element contains a list of <group> subelements having at least one subelement. The recipient shall perform a de-affiliation for all the MCPTT groups contained in the list for the clients for which the <command-list> applies.

The recipient of the XML ignores any unknown element and any unknown attribute.

## F.4.4	IANA registration template

Editor"s note [CT1onMCPTT, C1ah-160012]: The MIME type application/ vnd.3gpp.mcptt-affiliation-command+xml as defined in this subclause is to be registered in the IANA registry for Application Media Types based upon the following template. The registration is to be started when work on the MCPTT-CT WID completes.

Your Name:

<MCC name>

Your Email Address:

<MCC email address>

Media Type Name:

Application

Subtype name:

vnd.3gpp.mcptt-affiliation-command+xml

Required parameters:

# None

Optional parameters:

"charset"	the parameter has identical semantics to the charset parameter of the "application/xml" media type as specified in section 9.1 of IETF RFC 7303.

Encoding considerations:

binary.

Security considerations:

Same as general security considerations for application/xml media type as specified in section 9.1 of IETF RFC 7303. In addition, this media type provides a format for exchanging information in SIP, so the security considerations from IETF RFC 3261 apply.

The information transported in this media type does not include active or executable content.

Mechanisms for privacy and integrity protection of protocol parameters exist. Those mechanisms as well as authentication and further security mechanisms are described in 3GPP TS 24.229.

This media type does not include provisions for directives that institute actions on a recipient's files or other resources.

This media type does not include provisions for directives that institute actions that, while not directly harmful to the recipient, may result in disclosure of information that either facilitates a subsequent attack or else violates a recipient's privacy in any way.

This media type does not employ compression.

Interoperability considerations:

Same as general interoperability considerations for application/xml media type as specified in section 9.1 of IETF RFC 7303. Any unknown XML elements and any unknown XML attributes are to be ignored by recipient of the MIME body.

Published specification:

3GPP TS 24.379 "Mission Critical Push To Talk (MCPTT) call control" version 13.0.0, available via http://www.3gpp.org/specs/numbering.htm.

Applications which use this media type:

Applications supporting the service continuity as described in the published specification.

Fragment identifier considerations:

The handling in section 5 of IETF RFC 7303 applies.

Restrictions on usage:

None

Provisional registration? (standards tree only):

N/A

Additional information:

1.	Deprecated alias names for this type: none

2.	Magic number(s): none

3.	File extension(s): none

4.	Macintosh File Type Code(s): none

5.	Object Identifier(s) or OID(s): none

Intended usage:

Common

Person to contact for further information:

-	Name: <MCC name>

-	Email: <MCC email address>

-	Author/Change controller:

i)	Author: 3GPP CT1 Working Group/3GPP_TSG_CT_WG1@LIST.ETSI.ORG

ii)	Change controller: <MCC name>/<MCC email address>

# F.5	XML schema for the floor request

## F.5.1	General

This subclause defines XML schema and MIME type for application/vnd.3gpp.mcptt-floor-request+xml.

## F.5.2	XML schema

<?xml version="1.0" encoding="UTF-8"?>

<xs:schema attributeFormDefault="unqualified" elementFormDefault="qualified" xmlns:xs="http://www.w3.org/2001/XMLSchema" targetNamespace="urn:3gpp:ns:mcpttFloorRequest:1.0" xmlns:mcpttfloor="urn:3gpp:ns:mcpttFloorRequest:1.0">

<!-- the root element -->

<xs:element name="mcptt-floor-request" type="mcpttfloor:mcptt-floor-request-Type" minOccurs="1" maxOccurs="2"/>

<xs:complexType name="mcptt-floor-request-Type">

<xs:sequence>

<xs:element name="floor-type" type="xs:string"/>

<xs:element name="ssrc" type="xs:unsignedLong"/>

<xs:element name="floor-priority" type="xs:unsignedByte"/>

<xs:element name="user-id" type="xs:anyURI"/>

<xs:element name="track-info" type="mcpttfloor:track-info-Type"/>

<xs:element name="floor-indicator" type="xs:unsignedLong"/>

<xs:element name="anyExt" type="mcpttfloor:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="anyExtType">

<xs:sequence>

<xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<xs:complexType name="track-info-Type">

<xs:sequence>

<xs:element name="queueing-capability" type="xs:byte"/>

<xs:element name="participant-type" type="xs:string"/>

<xs:element name="floor-participant-reference" type="xs:unsignedLong" minOccurs="1" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="mcpttfloor:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

</xs:schema>

## F.5.3	Semantic

This subclause describes the elements of the floor request XML Schema.

<floor-type>	Contains information about which floor the current speaker was using. The value "general is selected when the current speaker has permission to speak in on the general floor as specified in 3GPP TS 24.380 [5] subclause 6.3.4. The value "dual" is selected when the current speaker has permission to speak in on the dual floor as specified in 3GPP TS 24.380 [5] subclause 6.3.6.

Table F.5.3-1 shows the ABNF of the <floor-type> element.

Table F.5.3-1: ABNF syntax of values of the <floor-type> element

floor-type-value = general-value / dual-value

general-value = %x67.65.6e.65.72.62.6c ; "general"

dual-value = %x64.75.61.6c ; "dual"

<ssrc>:	Contains the SSRC of the floor participant. The content of the SSRC field shall be coded as specified in IETF RFC 3550 [10].

<floor-priority>:	Contains the level of priority of the floor request. The <floor-priority> element is coded as specified in 3GPP TS 24.380 [5].

<user-id>:	Contains the MCPTT ID of the MCPTT user requesting the permission to send media.

<track-info>:	Contains the <queueing-capability> element, the <participant-type> element and the <floor-participant-reference>.

<floor-indicator>:	Contains additional information. The <floor-indicator> element is coded as specified in 3GPP TS 24.380 [5].

<participant-type>:	Contains the participant type assigned to the MCPTT user identified by the <user-id> element. The <participant-type> element is coded as specified in3GPP TS 24.380 [5].

NOTE:	The reference to the floor participant is a value only understandable by the floor control server interface in the non-Controlling function of an MCPTT group.

<queueing-capability>:	Contains the queueing capability of the MCPTT client. The <queueing-capability> element is coded as specified in 3GPP TS 24.380 [5].

The recipient of the XML ignores any unknown element and any unknown attribute.

## F.5.4	IANA registration template

Editor"s note [CT1#95-bis, CR 0100]: The MIME type "application/vnd.3gpp.mcptt-floor-request+xml" as defined in this subclause is to be registered in the IANA registry for Application Media Types based upon the following template. The registration is to be started when work on the MCPTT-CT WID completes.

Your Name:

<MCC name>

Your Email Address:

<MCC email address>

Media Type Name:

Application

Subtype name:

vnd.3gpp.mcptt-floor-request+xml

Required parameters:

None

Optional parameters:

"charset"	the parameter has identical semantics to the charset parameter of the "application/xml" media type as specified in section 9.1 of IETF RFC 7303.

Encoding considerations:

binary.

Security considerations:

Same as general security considerations for application/xml media type as specified in section 9.1 of IETF RFC 7303. In addition, this media type provides a format for exchanging information in SIP, so the security considerations from IETF RFC 3261 apply.

The information transported in this media type does not include active or executable content.

Mechanisms for privacy and integrity protection of protocol parameters exist. Those mechanisms as well as authentication and further security mechanisms are described in 3GPP TS 24.229.

This media type does not include provisions for directives that institute actions on a recipient's files or other resources.

This media type does not include provisions for directives that institute actions that, while not directly harmful to the recipient, may result in disclosure of information that either facilitates a subsequent attack or else violates a recipient's privacy in any way.

This media type does not employ compression.

Interoperability considerations:

Same as general interoperability considerations for application/xml media type as specified in section 9.1 of IETF RFC 7303. Any unknown XML elements and any unknown XML attributes are to be ignored by recipient of the MIME body.

Published specification:

3GPP TS 24.379 "Mission Critical Push To Talk (MCPTT) call control" version 13.0.0, available via http://www.3gpp.org/specs/numbering.htm.

Applications which use this media type:

Applications supporting the service continuity as described in the published specification.

Fragment identifier considerations:

The handling in section 5 of IETF RFC 7303 applies.

Restrictions on usage:

None

Provisional registration? (standards tree only):

N/A

Additional information:

1.	Deprecated alias names for this type: none

2.	Magic number(s): none

3.	File extension(s): none

4.	Macintosh File Type Code(s): none

5.	Object Identifier(s) or OID(s): none

Intended usage:

Common

Person to contact for further information:

-	Name: <MCC name>

-	Email: <MCC email address>

-	Author/Change controller:

i)	Author: 3GPP CT1 Working Group/3GPP_TSG_CT_WG1@LIST.ETSI.ORG

ii)	Change controller: <MCC name>/<MCC email address>

# F.6	XML schema for integrity protection of MIME bodies

## F.6.1	General

This subclause defines the XML schema and the MIME type vnd.3gpp.mcptt-signed+xml, for integrity protection of MIME bodies used in the present document.

## F.6.2	XML schema

<?xml version="1.0" encoding="UTF-8"?>

<xs:schema

xmlns:xs="http://www.w3.org/2001/XMLSchema"

targetNamespace="urn:3gpp:ns:mcpttSignedDoc:1.0"

xmlns:mcpttsigneddoc="urn:3gpp:ns:mcpttSignedDoc:1.0"

elementFormDefault="qualified"

attributeFormDefault="unqualified"

xmlns:xmldsig="http://www.w3.org/2000/09/xmldsig#">

<xs:import namespace="http://www.w3.org/2000/09/xmldsig#"/>

<xs:complexType name="signaturesType">

<xs:sequence>      <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="mcpttsigneddoc:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<!-- root XML element when creating a signed XML document -->

<xs:element name="signatures" type="mcpttsigneddoc:signaturesType"/>

<xs:complexType name="anyExtType">

<xs:sequence>

<xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:schema>

## F.6.3	Semantic

When integrity protection is turned on, the vnd.3gpp.mcptt-signed+xml MIME body is included when sending MIME bodies containing XML content in SIP requests and SIP responses.

The <signatures> element is the root element of the XML document.

NOTE 1:	The subelements of the <signatures> element are validated by the <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> particle of the <signatures> element

An instance of the <signatures> element contains one or more instances of the <xmldsig:Signature> element from the http://www.w3.org/2000/09/xmldsig# namespace.

Each <xmldsig:Signature> element contains the following child elements:

1)	a <xmldsig:SignatureValue> element is included with a signature value;

2)	a <xmldsig:SignedInfo> element can be included and can contain the following child elements:

a)	a <xmldsig:CanonicalizationMethod> element can be included with an "Algorithm" attribute containing an appropriate cannonicalisation method to be applied to the signed information;

b)	a <xmldsig:SignatureMethod> element can be included with an "Algorithm" attribute containing an appropriate algorithm for the signature; and

NOTE 2:	For signatures, it is assumed that HMAC-SHA256 is supported by the sender and the receiver.

c)	a <xmldsig:Reference> element can be included and can contain the following child elements:

i)	a "URI" attribute can be included with a "cid-URL" referring to an XML MIME body containing a Content-ID set to the "cid-URL"

ii)	a <xmldsig:DigestMethod> element can be included referring to an appropriate method for hashing the content; and

NOTE 3:	For hashing of the content, it is assumed that SHA-256 is supported by the sender and the receiver.

iii)	a <xmldsig:DigestValue> element can be included containing the hashed content; and

3)	a <xmldsig:KeyInfo> element can be included with a <xmldsig:KeyName> element containing the base 64 encoded XPK-ID.

NOTE 4:	When the optional attributes and elements are not included within the <xmldsig:Signature> element, the information they contain is known to sender and the receiver by other means.

## F.6.4	IANA registration template

Editor"s note [CT1#95, C1-154478]: The MIME type application/vnd.3gpp.mcptt-signed+xml as defined in this subclause is to be registered in the IANA registry for Application Media Types based upon the following template. The registration is to be started when work on the MCPTT-CT WID completes.

Your Name:

<MCC name>

Your Email Address:

<MCC email address>

Media Type Name:

Application

Subtype name:

vnd.3gpp.mcptt-signed+xml

Required parameters:

# None

Optional parameters:

"charset"	the parameter has identical semantics to the charset parameter of the "application/xml" media type as specified in section 9.1 of IETF RFC 7303.

Encoding considerations:

binary.

Security considerations:

Same as general security considerations for application/xml media type as specified in section 9.1 of IETF RFC 7303. In addition, this media type provides a format for exchanging information in SIP, so the security considerations from IETF RFC 3261 apply.

The information transported in this media type does not include active or executable content.

Mechanisms for privacy and integrity protection of protocol parameters exist. Those mechanisms as well as authentication and further security mechanisms are described in 3GPP TS 24.229.

This media type does not include provisions for directives that institute actions on a recipient's files or other resources.

This media type does not include provisions for directives that institute actions that, while not directly harmful to the recipient, may result in disclosure of information that either facilitates a subsequent attack or else violates a recipient's privacy in any way.

This media type does not employ compression.

Interoperability considerations:

Same as general interoperability considerations for application/xml media type as specified in section 9.1 of IETF RFC 7303. Any unknown XML elements and any unknown XML attributes are to be ignored by recipient of the MIME body.

Published specification:

3GPP TS 24.379 "Mission Critical Push To Talk (MCPTT) call control" version 13.1.0, available via http://www.3gpp.org/specs/numbering.htm.

Applications which use this media type:

Applications supporting the service continuity as described in the published specification.

Fragment identifier considerations:

The handling in section 5 of IETF RFC 7303 applies.

Restrictions on usage:

None

Provisional registration? (standards tree only):

N/A

Additional information:

1.	Deprecated alias names for this type: none

2.	Magic number(s): none

3.	File extension(s): none

4.	Macintosh File Type Code(s): none

5.	Object Identifier(s) or OID(s): none

Intended usage:

Common

Person to contact for further information:

-	Name: <MCC name>

-	Email: <MCC email address>

-	Author/Change controller:

i)	Author: 3GPP CT1 Working Group/3GPP_TSG_CT_WG1@LIST.ETSI.ORG

ii)	Change controller: <MCC name>/<MCC email address>

Annex G (informative):
On-network emergency and imminent peril related states

# G.1	MCPTT emergency state

The MCPTT emergency state is managed by the MCPTT client and MCPTT user. High-level characteristics of this state are captured in table G-1.1.

Table G.1-1: MCPTT emergency state

| MCPTT emergency state | State-setting events | State-clearing events | Comments |
| --- | --- | --- | --- |
| Values: "set": MCPTT user is in a life-threatening situation "clear": MCPTT user is not in a life-threatening situation Managed by: MCPTT client and MCPTT user | MCPTT emergency alert initiated MCPTT emergency group call initiated MCPTT emergency private call initiated | MCPTT emergency alert cancelled (by initiator) MCPTT emergency alert cancelled (by authorised-user) MCPTT emergency call cancelled by initiator (if there is no outstanding MCPTT emergency alert) MCPTT user manually clears the state | While the MCPTT client is in the MCPTT emergency state, all group calls it makes will be MCPTT emergency group calls, providing the group is authorised for MCPTT emergency group calls. While in an emergency group call while in the MCPTT emergency state, the MCPTT user is an "emergency talker" and will have pre-emptive priority over non-emergency talkers in the emergency group call. |

# G.2	In-progress emergency group state

This state is described in both 3GPP TS 22.179 [2] and 3GPP TS 23.179 [3]. It is managed by the controlling MCPTT function. High-level characteristics of this state are captured in table G.2-1.

Table G.2-1: in-progress emergency group state

| In-progress emergency group state values | State-entering events | Comments |
| --- | --- | --- |
| "true" | acceptance by the controlling MCPTT function of an MCPTT emergency group call request (as per subclause 10.6.2.6.1.1 of 3GPP TS 23.179 [3]). |  |
| "false" | initial state prior to any call activity acceptance by the controlling MCPTT function of an MCPTT emergency group cancel request (as per subclause 10.6.2.6.1.3 of 3GPP TS 23.179 [3]). |  |

# G.3	MCPTT emergency group state

The MCPTT emergency group state is the MCPTT client's perspective of the in-progress emergency group state which is managed by the controlling MCPTT function. The MCPTT emergency group (MEG) state is managed by the MCPTT client to enable the requesting of MCPTT emergency-level priority as early as possible in the origination of an MCPTT emergency group call. High-level characteristics of this state are captured in table G.3-1.

Table G.3-1: MCPTT emergency group state

| MCPTT emergency group state values | State-entering events | Comments |
| --- | --- | --- |
| MEG 1: no-emergency | initial state prior to any call activity Emergency group call cancel request received on behalf of another user from the MCPTT server Emergency group call cancel response (success) in response to initiator's request |  |
| MEG 2: in-progress | Emergency group call response received (confirm) to initiator's emergency group call request Emergency group call request received (on behalf of another user) | In this state, all participants in calls on this group will receive emergency level priority whether or not they are in the MCPTT emergency state themselves. |
| MEG 3: cancel-pending | Emergency group call cancel request sent by initiator | The controlling MCPTT server may not grant the cancel request for various reasons, e.g., other users in an MCPTT emergency state remain in the call. |
| MEG 4: confirm-pending | Emergency group call request sent by initiator | The controlling MCPTT server may not grant the call request for various reasons, e.g., the MCPTT group is not configured as being emergency-capable so it can't be assumed that the group will enter the in-progress state. |

# G.4	MCPTT emergency group call state

Table G.4-1 provides the semantics of the MCPTT emergency group call (MEGC) state values. This internal state of the MCPTT client and is managed by the MCPTT client. These states aid in the managing of the information elements of MCPTT emergency group calls and MCPTT emergency alerts and their cancellations.

Table G.4-1: MCPTT emergency group call state

| MCPTT emergency group call state values | Semantics | Comments |
| --- | --- | --- |
| MEGC 1: emergency-gc-capable | MCPTT client emergency-capable client is not currently in an MCPTT emergency group call that it has originated, nor is it in the process of initiating one. | MCPTT emergency state: may or may not be set in this state, depending upon the MCPTT client's MEA state |
| MEGC 2: emergency-call-requested | MCPTT client has initiated an MCPTT emergency group call request. | MCPTT emergency state: is set |
| MEGC 3: emergency-call-granted | MCPTT client has received an MCPTT emergency group call grant. | If the MCPTT user initiates a call while the MCPTT emergency state is still set, that call will be an MCPTT emergency group call, assuming that group is authorised for the client to initiate emergency group calls on. MCPTT emergency state: is set |

# G.5	MCPTT emergency alert state

Table G.5-1provides the semantics of the MCPTT emergency alert (MEA) state values. This is an internal state of the MCPTT client and is managed by the MCPTT client. These states aid in the managing of the information elements of MCPTT emergency group calls and MCPTT emergency alerts and their cancellations.

Table G.5-1: MCPTT emergency alert state

| MCPTT emergency alert state values | State-entering events | Comments |
| --- | --- | --- |
| MEA 1: no-alert | initial state emergency alert cancelled emergency alert request denied | emergency alerts can be cancelled in several ways: MCPTT emergency alert cancel request with <alert-ind> set to "false" (by initiator) MCPTT emergency alert cancel request with <alert-ind> set to "false" (by authorised user) MCPTT emergency group call cancel request with <alert-ind> set to "false" MCPTT emergency state: may be set or clear, depending on MCPTT emergency call status |
| MEA 2: emergency-alert-confirm-pending | emergency alert request sent | emergency alerts can be requested in several ways: MCPTT emergency alert request with <alert-ind> set to "true" MCPTT emergency group call request with <alert-ind> set to "true" MCPTT emergency state: is set |
| MEA 3: emergency-alert -initiated | emergency alert response (success) received | MCPTT emergency state: is set |
| MEA 4: emergency-alert-cancel-pending | emergency alert cancellation request sent by alert originator | MCPTT emergency state: is clear |

# G.6	In-progress imminent peril group state

This state is described in both 3GPP TS 22.179 [2] and 3GPP TS 23.179 [3]. It is managed by the controlling MCPTT function. High-level characteristics of this state are captured in table G.6-1.

Table G.6-1: in-progress imminent peril group state

| In-progress imminent peril group state values | State-entering events | Comments |
| --- | --- | --- |
| "true" | acceptance by the controlling MCPTT function of an MCPTT imminent peril group call request (as per subclause 10.6.2.6.2.1 and subclause 10.6.2.6.2.2 of 3GPP TS 23.179 [3]). |  |
| "false" | initial state prior to any call activity acceptance by the controlling MCPTT function of an MCPTT imminent peril group cancel request (as per subclause 10.6.2.6.2.3 of 3GPP TS 23.179 [3]). |  |

# G.7	MCPTT imminent peril group state

The MCPTT imminent peril group state is the MCPTT client's perspective of the in-progress imminent peril group state which is managed by the controlling MCPTT function. The MCPTT imminent peril group (MIG) state is managed by the MCPTT client to enable the requesting of MCPTT imminent peril-level priority as early as possible in the origination of an MCPTT imminent peril group call. High-level characteristics of this state are captured in table G.7-1.

Table G.7-1: MCPTT imminent peril group state

| MCPTT imminent peril group state values | State-entering events | Comments |
| --- | --- | --- |
| MIG 1: no-imminent peril | initial state prior to any call activity Imminent peril group call cancel request received on behalf of another user from the MCPTT server Imminent peril group call cancel response (success) in response to initiator's request |  |
| MIG 2: in-progress | Imminent peril group call response received (confirm) to initiator's imminent peril group call request Imminent peril group call request received (on behalf of another user) | In this state, all participants in calls on this group will receive imminent peril level priority whether or not they initiated an MCPTT imminent peril group call themselves. |
| MIG 3: cancel-pending | Imminent peril group call cancel request sent by initiator | The controlling MCPTT server may not grant the cancel request for various reasons, e.g., other users in an MCPTT imminent peril state remain in the call. |
| MIG 4: confirm-pending | Imminent peril group call request sent by initiator | The controlling MCPTT server may not grant the call request for various reasons, e.g., the MCPTT group is not configured as being imminent peril-capable so it can't be assumed that the group will enter the in-progress state. |

# G.8	MCPTT imminent peril group call state

Table G.8-1 provides the semantics of the MCPTT imminent peril group call (MIGC) state values. This internal state of the MCPTT client and is managed by the MCPTT client. These states aid in the managing of the information elements of MCPTT imminent peril group calls and their cancellations.

Table G.8-1: MCPTT imminent peril group call state

| MCPTT imminent peril group call state values | Semantics | Comments |
| --- | --- | --- |
| MIGC 1: imminent peril-gc-capable | MCPTT client imminent peril-capable client is not currently in an MCPTT imminent peril group call that it has originated, nor is it in the process of initiating one. | In this state, the MCPTT imminent peril group state will be set to either "MIG 1: no-imminent-peril" or "MIG 2 in-progress" state. |
| MIGC 2: imminent peril-call-requested | MCPTT client has initiated an MCPTT imminent peril group call request. | In this state, the MCPTT imminent peril group state will be set to "MIG 4: confirm-pending" if not already in the "MIG 2 in-progress" state. |
| MIGC 3: imminent peril-call-granted | MCPTT client has received an MCPTT imminent peril group call grant. | In this state, the MCPTT imminent peril group state will be set to "MIG2 in-progress". |

# G.9	In-progress emergency private call state

This state is managed by the controlling MCPTT function. All private calls originated between an initiator and the target MCPTT user when they are in an in-progress emergency private call state are MCPTT emergency private calls until this state is cancelled, whether or not the originator of the private call is in an MCPTT emergency state.

Table G.9-1: in-progress emergency private call state

| In-progress emergency private call state values | State-entering events | Comments |
| --- | --- | --- |
| "true" | acceptance by the controlling MCPTT function of an MCPTT emergency private call request (as per subclause 10.7.2.4.1 of 3GPP TS 23.179 [3]). | The in-progress emergency private call state applies to the call and the two MCPTT users in the call. |
| "false" | initial state prior to any private call activity acceptance by the controlling MCPTT function of the cancellation of an MCPTT emergency private call (as per subclause 10.7.2.4.1 of 3GPP TS 23.179 [3]). |  |

# G.10	MCPTT emergency private priority state

The MCPTT emergency private priority state is the MCPTT client's perspective of the in-progress emergency private call state which is managed by the controlling MCPTT function. The MCPTT emergency private priority (MEPP) state is managed by the MCPTT client to enable the requesting of MCPTT emergency-level priority as early as possible in the origination of an MCPTT emergency private call. High-level characteristics of this state are captured in table G.10-1.

Table G.10-1: MCPTT emergency private priority state

| MCPTT emergency private priority state values | State-entering events | Comments |
| --- | --- | --- |
| MEPP 1: no-emergency | initial state prior to any call activity Emergency private call cancel request received on behalf of another user from the MCPTT server Emergency private call cancel response (success) in response to initiator's request |  |
| MEPP 2: in-progress | Emergency private call response received (confirm) to initiator's emergency private call request Emergency private call request received (on behalf of another user) | In this state, both participants in calls to each other will request emergency level priority whether or not they are in the MCPTT emergency state themselves. |
| MEPP 3: cancel-pending | Emergency private call cancel request sent by initiator | The controlling MCPTT server may not grant the cancel request for various reasons, e.g., the other user in the call is in an MCPTT emergency state. |
| MEPP 4: confirm-pending | Emergency private call request sent by initiator | The controlling MCPTT server may not grant the call request for various reasons, e.g., the MCPTT user is not configured as being authorised to originate an emergency private call so it can't be assumed that the call (originator and target users) will enter the in-progress state. |

# G.11	MCPTT emergency private call state

Table G.11-1 provides the semantics of the MCPTT emergency private call (MEPC) state values. This is an internal state of the MCPTT client and is managed by the MCPTT client. This state aids in the managing of the information elements of MCPTT emergency private calls and MCPTT emergency alerts and their cancellations.

Table G.11-1: MCPTT emergency private call state

| MCPTT emergency private call state values | Semantics | Comments |
| --- | --- | --- |
| MEPC 1: emergency-pc-capable | MCPTT client emergency-capable client is not currently in an MCPTT emergency private call that it has originated, nor is it in the process of initiating one. | MCPTT emergency state: may or may not be set in this state, depending upon the MCPTT client's MPEA state and the emergency states related to MCPTT emergency group calls. |
| MEPC 2: emergency-pc-requested | MCPTT client has initiated an MCPTT emergency private call request. | MCPTT emergency state: is set |
| MEPC 3: emergency-pc-granted | MCPTT client has received an MCPTT emergency private call grant. | If the MCPTT user initiates a call while the MCPTT emergency state is still set, that call will be an MCPTT emergency private call, assuming that the initiating MCPTT user is authorised to initiate an MCPTT emergency private call to the targeted MCPTT user. MCPTT emergency state: is set |

# G.12	MCPTT private emergency alert state

Table G.5-1provides the semantics of the MCPTT private emergency alert (MPEA) state values. This is an internal state of the MCPTT client and is managed by the MCPTT client. These states aid in the managing of the information elements of MCPTT emergency private calls and MCPTT emergency alerts and their cancellations. MCPTT private emergency alerts are targeted to an MCPTT user.

Table G.12-1: MCPTT private emergency alert state

| MCPTT emergency alert state values | State-entering events | Comments |
| --- | --- | --- |
| MPEA 1: no-alert | initial state emergency alert cancelled emergency alert request denied | emergency alerts targeted to an MCPTT user can be cancelled in several ways: MCPTT emergency private call cancel request with <alert-ind> set to "false" timeout of private call inactivity timer end of call (if system policy) MCPTT emergency state: may be set or clear, depending on MCPTT emergency call status |
| MPEA 2: emergency-alert-confirm-pending | emergency alert request sent | emergency alerts can be requested as an optional part of a MCPTT client's request to initiate an MCPTT emergency private call, in which case the request has an <alert-ind> element set to "true". MCPTT emergency state: is set |
| MPEA 3: emergency-alert -initiated | emergency alert response (success) received | MCPTT emergency state: is set |
| MPEA 4: emergency-alert-cancel-pending | emergency alert cancellation request sent by alert originator | MCPTT emergency state: is clear |

Annex H (informative):
On-network routing considerations

# H.1	General

The following subclauses summarise the identities placed into SIP headers and SIP bodies during session establishment.

# H.2	Group Call

Table H.2-1 describes the contents of the SIP headers and SIP bodies inserted by MCPTT clients and MCPTT servers involved in a group call.

Table H.2-1: Routing considerations for group call

| Interface | Content of SIP headers | Content of "mcptt-info" MIME body | Notes |
| --- | --- | --- | --- |
| originating MCPTT client to originating participating MCPTT function (O-PF). | Request-URI contains PSI of O-PF. P-Preferred-Identity may contain IMPU of originating user. | "mcptt-request-uri" contains the group identity. | PSI of O-PF configured for each client. MCPTT-id of each client is never sent in session initiation. |
| O-PF to controlling MCPTT function (CF). | Request-URI contains PSI of CF. P-Asserted-Identity contains IMPU of originating user. | "mcptt-request-uri" contains the group identity. "mcptt-calling-user-id" contains MCPTT ID of originating user. | CF finds the MCPTT ID of the originating user from the stored IMPU-MCPTT ID binding and locates the PSI of the controller that serves the group identity. O-PF contains configuration of the PSIs of the CFs. |
| CF to terminating participating MCPTT function (T-PF). | Request-URI contains the address of the T-PF. P-Asserted-Identity contains the address of the CF. | "mcptt-request-uri" contains the MCPTT ID of the terminating user. "mcptt-calling-user-id" contains MCPTT ID of originating user. "mcptt-calling-group-id" contains the group identity. | For each client in the group, CF maps the MCPTT-ID of the terminator to the address of the T-PF. If the terminator is in another domain, the CF can map the MCPTT ID of the terminator to a PSI identifying a interrogating function in the partner network that is able to find the T-PF using the MCPTT ID. |
| CF to non-controlling MCPTT function of an MCPTT group (NCF). | Request-URI contains the PSI of the NCF. P-Asserted-Identity contains the PSI of the CF. | "mcptt-request-uri" contains the group identity. "mcptt-calling-user-id" contains MCPTT ID of originating user. | - |
| T-PF to terminating MCPTT client. | Request-URI contains the IMPU of the terminating user. P-Asserted-Identity contains the address of the CF. | "mcptt-request-uri" contains the MCPTT ID of the terminating user. "mcptt-calling-user-id" contains MCPTT ID of originating user. "mcptt-calling-group-id" contains the group identity. | T-PF finds the IMPU of the terminating user from the stored IMPU-MCPTT ID binding at the time of registration. |
| terminating MCPTT client to T-PF (response). | as in TS 24.229. | "mcptt-called-party-id" contains contacted client's MCPTT ID. | - |
| T-PF to NCF (response) | as in TS 24.229 | "mcptt-called-party-id" contains contacted client's MCPTT ID. | - |
| T-PF to CF (response). | as in TS 24.229. | "mcptt-called-user" contains contacted client's MCPTT ID. | - |
| NCF to CF (response) | as in TS 24.229. | - | In the case of trusted mutual aid, the NCF returns the identities of the group in a "resource-lists" MIME body. |
| CF to O-PF (response) | as in TS 24.229. | - | - |
| O-PF to originating MCPTT client (response) | as in TS 24.229. | - | - |

# H.3	Private Call

Table H.3-1 describes the contents of the SIP headers and SIP bodies inserted by MCPTT clients and MCPTT servers involved in a private call.

Table H.3-1: Routing considerations for private call

| Interface | Content of SIP headers | Content of SIP bodies (body in brackets) | Notes |
| --- | --- | --- | --- |
| originating MCPTT client to originating participating MCPTT function (O-PF) | Request-URI contains the PSI for the private call service. P-Preferred-Identity may contain IMPU of originating user | MCPTT ID of called user (resource-lists) "mcptt-calling-user-id" (mcptt-info) | PSI for private call is configured on the client. |
| O-PF to controlling MCPTT function (CF) | Request-URI contains the PSI for the private call service. P-Asserted-Identity contains IMPU of originating user. | MCPTT ID of called user (resource-lists) MCPTT ID of calling user contained in "mcptt-calling-user-id" (mcptt-info) | - |
| CF to terminating participating MCPTT function (T-PF) | Request-URI contains the address of the T-PF. P-Asserted-Identity contains IMPU of originating user. | MCPTT ID of calling user contained in "mcptt-calling-user-id" (mcptt-info). MCPTT ID of called user contained in "mcptt-called-party-id" (mcptt-info). | If the terminator is in another domain, the CF can map the MCPTT ID of the terminator to a PSI identifying an interrogating function in the partner network that is able to find the T-PF using the MCPTT ID. |
| T-PF to terminating MCPTT client | Request-URI contains the IMPU of the terminating user. P-Asserted-Identity contains IMPU of originating user. | MCPTT ID of calling user contained in "mcptt-calling-user-id" (mcptt-info). MCPTT ID of called user contained in "mcptt-called-party-id" (mcptt-info). | - |
| terminating MCPTT client to T-PF (response) | as in TS 24.229 | "mcptt-called-party-id" contains contacted client's MCPTT ID. | - |
| T-PF to CF (response) | as in TS 24.229 | "mcptt-called-user" contains contacted client's MCPTT ID. | - |
| CF to O-PF (response) | as in TS 24.229 | "mcptt-called-party-id" contains contacted client's MCPTT ID. | - |
| O-PF to originating MCPTT client (response) | as in TS 24.229 | "mcptt-called-party-id" contains contacted client's MCPTT ID. | - |

Annex I (normative):
MCPTT Off-Network Protocol (MONP) message coding rules

# I.1	General

The following subclauses describe the message coding rules for the MCPTT Off-Network Protocol (MONP).

# I.2	MONP messages

## I.2.1	Components of a MONP message

A standard MONP message consists of an imperative part, itself composed of a header and the rest of imperative part, followed by a non-imperative part. Both the non-header part of the imperative part and the non-imperative part are composed of successive parts referred as standard information elements.

## I.2.2	Format of standard information elements

A standard IE may have the following parts, in that order:

-	an information element identifier (IEI);

-	a length indicator (LI);

-	a value part.

A standard IE has one of the formats shown in table I.2.2-1:

Table I.2.2-1: Formats of information elements

| Format | Meaning | IEI present | LI present | Value part present |
| --- | --- | --- | --- | --- |
| T | Type only | yes | no | no |
| V | Value only | no | no | yes |
| TV | Type and Value | yes | no | yes |
| LV | Length and Value | no | yes | yes |
| TLV | Type, Length and Value | yes | yes | yes |
| LV-E | Length and Value | no | yes | yes |
| TLV-E | Type, Length and Value | yes | yes | yes |

Some IEs may appear in the structure, but not in all instances of messages. An IE is then said to be present or not present in the message instance. If an IE is not present in a message instance, none of the three parts is present. Otherwise, parts must be present according to the IE format.

In the message structure, an IE that is allowed not to be present in all message instances is said not to be mandatory. Other IEs are said to be mandatory.

### I.2.2.1	Information element type and value part

Every standard IE has an information element type which determines the values possible for the value part of the IE, and the basic meaning of the information. The information element type describes only the value part. Standard IEs of the same information element type may appear with different formats. The format used for a given standard IE in a given message is specified within the description of the message.

The value part of a standard IE either consists of a half octet or one or more octets; the value part of a standard IE with format LV or TLV consists of an integral number of octets, between 0 and 255 inclusive; it then may be empty, i.e., consist of zero octets; if it consists of a half octet and has format TV, its IEI consists of a half octet, too. For LV-E and TLV-E, the value part of a standard IE consists of an integral number of octets, between 0 and 65535 inclusive. The value part of a standard IE may be further structured into parts, called fields.

### I.2.2.2	Length indicator

For LV or TLV, the length indicator (LI) of a standard IE consists of one octet. For LV-E and TLV-E, the LI of a standard IE consists of two octets where bit 8 of octet n contains the most significant bit and bit 1 of octet n+1 contains the least significant bit (refer to figure I.2.2.4-9 in subclause I.2.2.4 for the relative ordering of the 2 octets). The LI contains the binary encoding of the number of octets of the IE value part. The LI of a standard IE with empty value part indicates 0 octets. Standard IE of an information element type such that the possible values may have different values must be formatted with a length field, i.e., LV, TLV, LV-E or TLV-E.

### I.2.2.3	Information element identifier

When present, the IEI of a standard IE consists of a half octet or one octet. A standard IE with IEI consisting of a half octet has format TV, and its value part consists of a half octet. The value of the IEI depends on the standard IE, not on its information element type. The IEI, if any, of a given standard IE in a given message is specified within the description of the message. In some protocol specifications, default IEI values can be indicated. They are to be used if not indicated in the message specification. Non mandatory standard IE in a given message, i.e., IE which may be not be present (formally, for which the null string is acceptable in the message), must be formatted with an IEI, i.e., with format T, TV, TLV or TLV-E.

### I.2.2.4	Categories of IEs; order of occurrence of IEI, LI, and value part

The following categories of standard information elements are defined:

-	information elements of format V or TV with value part consisting of 1/2 octet (type 1);

-	information elements of format T with value part consisting of 0 octets (type 2);

-	information elements of format V or TV with value part that has fixed length of at least one octet (type 3);

-	information elements of format LV or TLV with value part consisting of zero, one or more octets (type 4)

-	information elements of format LV-E or TLV-E with value part consisting of zero, one or more octets and a maximum of 65535 octets (type 6).

NOTE:	The "types" and "formats" used for MONP match those defined in 3GPP TS 24.007 [56];

Type 1 standard information elements of format V provide the value in bit positions 8, 7, 6, 5 of an octet (see figure I.2.2.4-1) or bits 4, 3, 2, 1 of an octet (see figure I.2.2.4-2).

Figure I.2.2.4-1: Type 1 IE of format V

Figure I.2.2.4-2: Type 1 IE of format V

Type 1 standard information elements of format TV have an IEI of a half octet length; they provide the IEI in bit positions 8, 7, 6, 5 of an octet and the value part in bit positions 4, 3, 2, 1 of the same octet, see figure I.2.2.4-3.

Figure I.2.2.4-3: Type 1 IE of format TV

A type 2 standard IE has format T; its IEI consists of one octet, its value part is empty, see figure I.2.2.3-4.

Figure I.2.2.4-4: Type 2 IE

A type 3 standard information element has format V or TV; if it has format TV, its IEI consists of one octet and proceeds the value part in the IE. The value part consists of at least one octet. See figure I.2.2.4-5 and figure I.2.2.4-6.

Figure I.2.2.4-5: Type 3 IE of format V (k = 0, 1, 2, ...)

Figure I.2.2.4-6: Type 3 IE of format TV (k = 1, 2, ...)

A type 4 standard information element has format LV or TLV. Its LI precedes the value part, which consists of zero, one, or more octets; if present, its IEI has one octet length and precedes the LI. See figure I.2.2.4-7 and figure I.2.2.4-8.

Figure I.2.2.4-7: Type 4 IE of format LV (k = 0, 1, 2, ...)

Figure I.2.2.4-8: Type 4 IE of format TLV (k = 1, 2, ...)

A type 6 standard information element has format TLV-E. The IEI has one octet length and precedes the LI of 2 octets and the value part which consists of zero, one or up to 65535 octets. See figure I.2.2.4-9 and figure I.2.2.4-10.

Figure I.2.2.4-9: Type 6 IE of format LV-E (k = 1, 2, ...)

Figure I.2.2.4-10: Type 6 IE of format TLV-E (k = 1, 2, ...)

### I.2.2.5	Method for IE structure

Standard IEs can be further structured in parts called fields and represented in a tabular form.

The IE is presented in its maximum format, i.e., T, TV, TLV or TLV-E, in a picture representing the bits in a table, each line representing an octet. Bits appear in the occidental order, i.e., from left of the page to right of the page, and from top of the page to bottom of the page.

Boxes so delimited contains typically the field name, possibly an indication of which bits in the field are in the box, and possibly a value (e.g., for spare bits).

A specific method can be used in the IE description to describe a branching structure, i.e., a structure variable according to the value of particular fields in the IE. This design is unusual outside type 4 and type 6 IEs, and as, a design rule, should be used only in type 4 and type 6 IEs.

a)	The octet number of an octet within the IE is defined typically in the table. It consists of a positive integer, possibly of an additional letter, and possibly of an additional asterisk, see clause f). The positive integer identifies one octet or a group of octets.

b)	Each octet group is a self-contained entity. The internal structure of an octet group may be defined in alternative ways.

c)	An octet group is formed by using some extension mechanism. The preferred extension mechanism is to extend an octet (N) through the next octet(s) (Na, Nb, etc.) by using bit 8 in each octet as an extension bit.

-	The bit value "0" indicates that the octet group continues through to the next octet. The bit value "1" indicates that this octet is the last octet of the group. If one octet (Nb) is present, the preceding octets (N and Na) shall also be present.

-	In the format descriptions of the individual information elements, bit 8 is marked "0/1 ext" if another octet follows. Bit 8 is marked "1 ext" if this is the last octet in the extension domain.

-	Additional octets may be defined in later versions of the protocols ("1 ext" changed to "0/1 ext") and equipments shall be prepared to receive such additional octets; the contents of these octets shall be ignored. However the length indicated in the formal description of the messages and of the individual information elements only takes into account this version of the protocols.

d)	In addition to the extension mechanism defined above, an octet (N) may be extended through the next octet(s) (N+1, N+2 etc.) by indications in bits 7-1 (of octet N).

e)	The mechanisms in c) and d) may be combined.

f)	Optional octets are marked with asterisks (*). As a design rule, the presence or absence of an optional octet should be determinable from information in the IE and preceding the optional octet. Care should be taken not to introduce ambiguities with optional octets.

g)	At the end of the IE, additional octets may be added in later versions of the protocols also without using the mechanisms defined in c) and d). Equipments shall be prepared to receive such additional octets; the contents of these octets shall be ignored. However the length indicated in the formal description of the messages and of the individual information elements only takes into account this version of the protocols.

### I.2.2.6	Imperative part of a standard MONP message

The imperative part of a standard MONP message is composed of a header possibly followed by mandatory standard IEs having the format V, LV or LV-E.

The header consists of a message type information element as specified in subclause 15.2.1.

#### I.2.2.6.1	Standard information elements of the imperative part

The message type octet of a standard MONP message may be followed by mandatory standard IEs having the format V, LV or LV-E as specified in the message description in the relevant protocol specification.

As a design rule, octet boundaries must be respected. This implies that half-octet standard IEs (i.e., V formatted type 1 standard IEs) must appear by pair.

If message is received as a standard MONP message, and that is too short to contain the complete imperative part as specified in the relevant protocol specification, an imperative message part error is diagnosed. (The same error may be diagnosed at detection of certain contents of the imperative part of a message; this is defined in the relevant protocol specification.) The treatment of an imperative message part error is defined in the relevant protocol specification.

### I.2.2.7	Non-imperative part of a standard MONP message

The imperative part of a standard MONP message may be followed by the non-imperative part. The MONP protocol specification defines where the imperative part of a standard MONP message ends. The non-imperative part of a standard MONP message is composed of (zero, one, or several) standard IEs having the format T, TV, TLV or TLV-E. The receiver of a standard MONP message shall analyse the non-imperative part as a succession of standard IEs each containing an IEI, and shall be prepared for the non-imperative part of the message to contain standard IEs that are not specified in the relevant protocol specification.

An IEI may be known in a message or unknown in a message.

An IEI that is known in a message designates the IE type of the IE the first part of which the IEI is, as well as the use of the information. Which IE type it designates is specified in the relevant protocol specification. Within a message, different IEIs may designate the same IE type if that is defined in the relevant protocol specification.

Whether the second part of an IE with IEI known in a message is the length or not (in other words, whether the IEI is the first part of an IE formatted as TLV, TLV-E or not) is specified in the relevant protocol specification.

Unless otherwise specified in the protocol specification, the receiver shall assume that IE with unknown IEI are TV formatted type 1, T formatted type 2, TLV formatted type 4 or TLV-E formatted type 6 standard IEs. The IEI of unknown IEs together with, when applicable, the length indicator, enable the receiver to determine the total length of the IE, and then to skip unknown IEs. The receiver shall assume the following rule for IEs with unknown IEI:

Bit 8 of the IEI octet is set to "1" indicates a TV formatted type 1 standard IE or a T formatted type 2 IEs. Hence, a 1 valued bit 8 indicates that the whole IE is one octet long.

Furthermore:

Bit 8 of the IEI octet set to "0" and bits 7 to 4 set to "1" indicates a TLV-E formatted type 6 IE, i.e. the following two octets are length octets. Bit 8 of the IEI octet set to "0" and bit 7 to 4 set to any other bit combination indicates a TLV formatted type 4 IE, i.e. the following octet is a length octet.

As a design rule, it is recommended that IEIs of any TV formatted type 1, T formatted type 2, TLV formatted type 4 or TLV-E formatted type 6 IE follow the rule, even if assumed to be known by all potential receivers.

A message may contain two or more IEs with equal IEI. Two IEs with the same IEI in a same message must have the same format, and, when of type 3, the same length. More generally, care should be taken not to introduce ambiguities by using an IEI for two purposes. Ambiguities appear in particular when two IEs potentially immediately successive have the same IEI but different meanings and when both are non-mandatory. As a recommended design rule, messages should contain a single IE of a given IEI.

Each protocol specification may put specific rules for the order of IEs in the non-imperative part. An IE known in the message, but at a position non-compliant with these rules is said to be out of sequence. An out of sequence IE is decoded according to the format, and, when of type 3 the length, as defined in the message for its IEI.

### I.2.2.8	Presence requirements of information elements

The relevant protocol specification may define three different presence requirements (M, C, or O) for a standard IE within a given standard MONP message:

-	M ("Mandatory") means that the IE shall be included by the sending side, and that the receiver diagnoses a "missing mandatory IE" error when detecting that the IE is not present. An IE belonging to the imperative part of a message has presence requirement M. An IE belonging to the non-imperative part of a message may have presence requirement M;

-	C ("Conditional") means:

*	that inclusion of the IE by the sender depends on conditions specified in the relevant protocol specification;

*	that there are conditions for the receiver to expect that the IE is present and/or conditions for the receiver to expect that the IE is not present in a received message of a given message type; these conditions depend only on the content of the message itself, and not for instance on the state in which the message was received, or on the receiver characteristics; they are known as static conditions;

*	that the receiver detecting that the IE is not present when sufficient static conditions are fulfilled for its presence, shall diagnose a "missing conditional IE" error;

*	that the receiver detecting that the IE is present when sufficient static conditions are fulfilled for its non-presence, shall diagnose an "unexpected conditional IE" error.

-	Only IEs belonging to the non-imperative part of a message may have presence requirement C;

-	O ("Optional") means that the receiver shall never diagnose a "missing mandatory IE" error, a "missing conditional IE" error, or an "unexpected conditional IE" error because it detects that the IE is present or that the IE is not present. (There may however be conditions depending on the states, resources, etc. of the receiver to diagnose other errors.) Only IEs belonging to the non-imperative part of a message may have presence requirement O.

Unless otherwise specified the presence of an IE of unknown IEI or of an out of sequence IE shall not lead by itself to an error.

### I.2.2.9	Description of standard MONP messages

This subclause describes a generic description method for MONP messages, the tabular description.

A MONP message is described by a table listing the header elements and the standard IEs in the message. For each element is given:

-	if applicable the IEI, in hexadecimal representation (one digit followed by and hyphen for TV formatted type 1, and two digits for the other cases);

-	the name of the IE (this is used in particular for the description of conditional presence rules);

-	the type of the information element, with a reference of where the internal structure of the value part is specified;

-	the format of the standard IE (T, V, TV, LV, TLV, LV-E or TLV-E); and

-	the length, or the range of lengths, of the whole standard IE, including when applicable the T and L parts.

The list of elements is given in the table in the order they appear in the resulting bit string, with the exception of half-octet elements in the imperative part: half octets in a pair are inverted.

Annex J (informative):
INFO packages defined in the present document

# J.1	Info package for transfer of floor requests

## J.1.1	Scope

This subclause contains the information required for the IANA registration of info package g.3gpp.mcptt-floor-request in accordance with IETF RFC 6086 [64].

## J.1.2	g.3gpp.mcptt-floor-request info package

### J.1.2.1	Overall description

When a temporary group call includes constituent MCPTT groups in partner systems where a MCPTT call is ongoing and if the floor is occupied by a participant with a permission to speak, the non-controlling MCPTT function of a MCPTT group needs to transfer information of the current speaker(s) to the controlling MCPTT function hosting the temporary group. The information is transferred in the form of a floor request.The controlling MCPTT function will then determine if the participant will be permitted to continue to speak or if the permission to speak is revoked.

### J.1.2.2	Applicability

This package is used to transport a floor request from the non-controlling MCPTT function of an MCPTT group to the controlling MCPTT function hosting the temporary group.

### J.1.2.3	Appropriateness of INFO Package Usage

A number of solutions were discussed for the transportation of the floor request to the controlling MCPTT function hosting the temporary MCPTT group. The solutions were:

1)	Use of the session related methods (e.g. SIP 200 (OK) response to the SIP INVITE request).

2)	Use of the SIP MESSAGE method.

3)	Use of the SIP INFO method as described in IETF RFC 6086, by defining a new info package.

The result of the evaluation of the above solutions were:

1)	To include such a large amount of data in a SIP 200 (OK) response to an SIP INVITE request could cause problems with the size of the SIP 200 (OK) response resulting in packet fragmentation.

2)	The use of the SIP MESSAGE request would result in that the recommended value of size of the information transferred by the SIP MESSAGE request would be exceeded.

3)	The use of SIP INFO request was found as the most appropriate solution since the SIP INFO request could be sent in the existing SIP session.

### J.1.2.4	Info package name

g.3gpp.mcptt-floor-request

### J.1.2.5	Info package parameters

None defined

### J.1.2.6	SIP options tags

None defined

### J.1.2.7	INFO message body parts

The MIME type of the message body carrying participant identities is application/vnd.3gpp.floor-request+xml. The application/vnd.3gpp.floor-request+xml MIME type is defined in 3GPP TS 24.379.

When associated with the g.3gpp.mcptt-floor-request info package, the Content-Disposition value of the message body carrying the floor request is "info-package".

### J.1.2.8	Info package usage restrictions

None defined.

### J.1.2.9	Rate of INFO Requests

Single INFO request generated after session set up.

### J.1.2.10	Info package security considerations

The security is based on the generic security mechanism provided for the underlying SIP signalling. No additional security mechanism is defined.

### J.1.2.11	Implementation details and examples

UAC generation of INFO requests: See 3GPP TS 24.379: "Mission Critical Push To Talk (MCPTT) call control; Protocol specification".

UAS processing of INFO requests: See 3GPP TS 24.379: ""Mission Critical Push To Talk (MCPTT) call control; Protocol specification"

EXAMPLE:	A controlling MCPTT function hosting a temporary MCPTT group inviting a constituent MCPTT group hosted by a non-controlling MCPTT function of a MCPTT group in a partner system where an MCPTT call is ongoing with one or two of the participants granted to speak. Then the non-controlling MCPTT function of the an MCPTT group sends a SIP INFO request carrying a floor request in an application/vnd.3gpp.mcptt-floor-request+xml MIME body using the g.3gpp.mcptt-floor-request info package.

# J.2	Info package for transfer of MCPTT information

## J.2.1	Scope

This subclause contains the information required for the IANA registration of info package g.3gpp.mcptt-info in accordance with IETF RFC 6086 [64].

## J.2.2	g.3gpp.mcptt-info info package

### J.2.2.1	Overall description

The MCPTT client request for MCPTT emergency call origination can also optionally request the origination of an MCPTT emergency alert.  Similarly, the MCPTT client request for MCPTT emergency call cancellation can also optionally request the cancellation of an MCPTT emergency alert. A mechanism to inform the MCPTT client that one of the requested actions has been rejected by the controlling MCPTT function is needed to both inform the MCPTT user that one of their requested actions has been rejected and to keep the emergency and imminent peril related state machines maintained by the MCPTT client updated appropriately. Note that a SIP 200 OK has to be sent in the case where the MCPTT emergency call origination request or cancellation request is accepted by the controller to allow the MCPTT user to initiate the MCPTT emergency call and receive updated priority even if the accompanying MCPTT alert request is rejected.

An MCPTT client request for an MCPTT imminent peril call when the targeted MCPTT group is in an in-progress emergency state also needs special handling, as in this case, the call request will be accepted but the MCPTT client needs to be informed that the MCPTT user will be joined to an in-progress MCPTT emergency group call instead of the requested MCPTT imminent peril group call to keep the emergency and imminent peril related state machines maintained by the MCPTT client updated appropriately.

### J.2.2.2	Applicability

This package is used to transport emergency call, imminent peril and emergency alert indications from the controlling function to the MCPTT client

### J.2.2.3	Appropriateness of INFO Package Usage

A number of solutions were discussed for the transportation of the emergency call, imminent peril and emergency alert indications from the controlling function to the MCPTT client. The solutions were:

1)	Use of the session related methods (e.g. SIP 200 (OK) response to the SIP INVITE request).

2)	Use of the SIP MESSAGE method.

3)	Use of the SIP INFO method as described in IETF RFC 6086, by defining a new info package.

The result of the evaluation of the above solutions were:

1)	To include such a large amount of data in a SIP 200 (OK) response to an SIP INVITE request could cause problems with the size of the SIP 200 (OK) response resulting in packet fragmentation.

2)	The use of the SIP MESSAGE request would result in that the recommended value of size of the information transferred by the SIP MESSAGE request would be exceeded.

3)	The use of SIP INFO request was found as the most appropriate solution since the SIP INFO request could be sent in the existing SIP session.

### J.2.2.4	Info package name

g.3gpp.mcptt-info

### J.2.2.5	Info package parameters

None defined

### J.2.2.6	SIP options tags

None defined

### J.2.2.7	INFO message body parts

The MIME type of the message body carrying participant identities is application/vnd.3gpp.mcptt-info+xml. The application/vnd.3gpp.mcptt-info+xml MIME type is defined in 3GPP TS 24.379.

When associated with the g.3gpp.mcptt-info info package, the Content-Disposition value of the message body carrying mcptt information is "info-package".

### J.2.2.8	Info package usage restrictions

None defined.

### J.2.2.9	Rate of INFO Requests

Single INFO request generated after session set up.

### J.2.2.10	Info package security considerations

The security is based on the generic security mechanism provided for the underlying SIP signalling. No additional security mechanism is defined.

### J.2.2.11	Implementation details and examples

UAC generation of INFO requests: See 3GPP TS 24.379: "Mission Critical Push To Talk (MCPTT) call control; Protocol specification".

UAS processing of INFO requests: See 3GPP TS 24.379: "Mission Critical Push To Talk (MCPTT) call control; Protocol specification"

EXAMPLE:	A controlling MCPTT function will receive a SIP INVITE request or SIP (re-)INVITE request containing a request for an emergency call (with or without an alert) or an imminent peril call. When an emergency call has been authorised but an optional request for an emergency alert has been determined to be unauthorised, the controller will respond with a SIP 200 (OK) response to indicate acceptance of the call request and return an indication of the rejection of the emergency alert request in a SIP INFO request carrying the application/vnd.3gpp.mcptt-info+xml MIME body using the g.3gpp.mcptt-info info package.

Annex K (informative):
IANA UDP port registration form

This annex contains information to be provided to IANA for MCPTT Off-Network Protocol (MONP) UDP port registration. The following information is to be used to register MONP user port number and service name in the "IANA Service Name and Transport Protocol Port Number Registry" and specifically "Service Name and Transport Protocol Port Number Registry". This registration form can be found at: https://www.iana.org/form/ports-services.

| Assignee Name | <MCC name> |
| --- | --- |
| Assignee E-mail | <MCC email address> |
| Contact Person | <MCC name> |
| Contact E-mail | <MCC email address> |
| Resources required | Port number and service name |
| Transport Protocols | UDP |
| Service Code |  |
| Service Name | MONP |
| Desired Port Number |  |
| Description | Mission Critical Push To Talk over LTE (MCPTT) Off-Network Protocol (MONP) is a 3GPP control protocol used by a MCPTT client hosted on a User Equipment (UE). MONP facilitates the MCPTT public safety service between MCPTT clients hosted on UEs communicating using IP using a single physical network segment, separated from Internet and any other IP network. The network segment is wireless network segment and UEs are mobile devices. The MCPTT public safety service offers half-duplex voice communication. |
| Reference | 3GPP TS 24.379 |
| Defined TXT keys | N/A |
| If broadcast/multicast is used, how and what for? | When performing off-network group calls, the MCPTT client initiates the group call to an MCPTT Group by sending a group call announcement message. The group call announcement message is a MONP message which is sent as a UDP message to a multicast IP address of the MCPTT group so that it is ensured that the MONP messages sent for the corresponding MCPTT group are only received by the MCPTT group's members. |
| If UDP is requested, please explain how traffic is limited, and whether the protocol reacts to congestion. | The number of MONP messages that need to be sent between MCPTT clients depends upon the number of members of the MCPTT group. MONP employs a back-off mechanism to defer transmission of another MONP message once a MONP message is received. MONP controls the number of messages transmitted within a certain, configurable amount of time, thus averting congestion. At maximum a few MONP messages per second are expected in communication between MCPTT clients. MONP does not support any reaction to congestion. |
| If UDP is requested, please indicate whether the service is solely for the discovery of hosts supporting this protocol. | MONP is not used solely for discovery of hosts supporting this protocol. |
| Please explain how your protocol supports versioning. | MONP does not support versioning. |
| If your request is for more than one transport, please explain in detail how the protocol differs over each transport. | N/A |
| Please describe how your protocol supports security. Note that presently there is no IETF consensus on when it is appropriate to use a second port for an insecure version of a protocol. | MONP does not support security. MONP relies on the security mechanisms of the lower layers. |
| Please explain why a unique port assignment is necessary as opposed to a port in range (49152-65535) or existing port. | As a general principle, 3GPP protocols use assigned User Ports, e.g. GTP-C uses UDP port number 2123, GTP-U uses UDP port number 2152, S1AP uses SCTP port number 36412, X2AP uses SCTP port number 36422, WLCP uses 36411. A dynamic port number (i.e. 49152 to 65535) cannot be used for the MONP because of the nature of communication on a single physical network segment, separated from Internet and any other IP network. The requirement of MONP to continuously listen for incoming messages needs an always active listener port. There is no local server that is administering the use of emphemeral ports in the MONParchitecture, so there would be no way for one MCPTT client to know that a port is already being used by another MCPTT client. Communication can potentially be long-lived and MCPTT clients could leave and re-join the calls. |
| Please explain the state of development of your protocol. | Protocol Standard definition. No implementation exists yet. |
| If SCTP is requested, is there an existing TCP and/or UDP service name or port number assignment? If yes, provide the existing service name and port number. | N/A |
| What specific SCTP capability is used by the application such that a user who has the choice of both TCP (and/or UDP) and SCTP ports for this application would choose SCTP? See RFC 4960 section 7.1. | N/A |
| Please provide any other information that would be helpful in understanding how this protocol differs from existing assigned services | This protocol is between the UEs communicating using IP over a single physical network segment, separated from Internet and any other IP network. An MCPTT public safety service offered by the MCPTT clients hosted by the UEs is for public safety. The MCPTT public safety service offers half-duplex voice communication. This differs from existing protocols in 3GPP where UDP ports have been requested, as those protocols have been either between the UE and network or between network elements. |

Annex L (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 2015-07 |  |  |  |  | Initial version. | - | 0.0.0 |
| 2015-08 |  |  |  |  | Implementation of the following P-CRs from CT1#92-bis MCPTT: C1ah-150007; C1ah-150042; C1ah-150046 | 0.0.0 | 0.1.0 |
| 2015-08 |  |  |  |  | Implementation of the following P-CRs from CT1#93: C1-152952; C1-152956; C1-153125; C1-153177; C1-153178; C1-153179; C1-153180; C1-153220; C1-153229 ;changes by Rapporteur | 0.1.0 | 0.2.0 |
| 2015-08 |  |  |  |  | Minor fixes from the rapporteur | 0.2.0 | 0.2.1 |
| 2015-09 |  |  |  |  | Modifying the cover sheet to add the TS number and replacing MCPTT TS references with their allocated numbers. Changes by the rapporteur. | 0.2.1 | 0.2.2 |
| 2015-10 |  |  |  |  | Implementation of the following P-CRs from CT1#94: C1-153730; C1-153736; C1-153739; C1-153740; C1-154748; C1-153749; C1-153750; C1-153751; C1-153762;C1-153763; C1-153764; C1-153765; C1-153766; C1-153802; C1-153804; C1-153805; C1-153926; C1-153975; | 0.2.2 | 0.3.0 |
| 2015-11 |  |  |  |  | Implementation of the following P-CRs from CT1#95: C1-154472; C1-154473; C1-154326; C1-154480; C1-154103; C1-154477; C1-154478; C1-154479; C1-154858; C1-154355; C1-154398; C1-154535; C1-154536; C1-154537; C1-154539; C1-154540; C1-154542; C1-154544; C1-154548; C1-154549; C1-154550; C1-154552; C1-154553; C1-154712; C1-154715; C1-154716; C1-154717; C1-154731; C1-154732;C1-154399; C1-154401; Editorial changes by rapporteur. | 0.3.0 | 0.4.0 |
| 2015-11 |  |  |  |  | Minor editorial fixes from the rapporteur | 0.4.0 | 0.4.1 |
| 2015-12 | CT-70 | CP-150733 |  |  | Version 1.0.0 created for presentation for information | 0.4.1 | 1.0.0 |
| 2016-01 |  |  |  |  | Implementation of the following P-CRs from CT1#95-bis: C1-160322; C1-160323; C1-160326; C1-160380; C1-160392; C1-160393; C1-160394; C1-160395; C1-160396, C1-160397; C1-160400; C1-160414; C1-160415; C1-160416; C1-160417; C1-160418; C1-160419; C1-160420; C1-160421; C1-160422; C1-160423; C1-160453, C1-160455; C1-160456; C1-160457; C1-160458; C1-160466; C1-160489; C1-160490; C1-160491; C1-160612; C1-160617; C1-160770; Editorial changes by rapporteur. | 1.0.0 | 1.1.0 |
| 2016-02 |  |  |  |  | (1) Implementation of the following P-CRs from CT1 MCPTT-adhoc: C1ah-160039; C1ah-160074; C1ah-160075; C1-ah160080; C1ah-160083; C1ah-160100; C1ah-160101; C1ah-160102; C1ah-160107; C1ah-160108; (2) Implementation of the following P-CRs from CT1#96: C1-161042; C1-161045; C1-161047; C1-161048; C1-161050; C1-161058; C1-161108; C1-161109; C1-161139; C1-161206; C1-161208; C1-161209; C1-161212; C1-161213; C1-161214; C1-161234; C1-161302; C1-161304; C1-161305; C1-161308; C1-161312; C1-161357; C1-161358; C1-161359; C1-161360; C1-161361; C1-161364; C1-161366; C1-161367; C1-161368; C1-161369; C1-161389; C1-161390; C1-161391; C1-161393; C1-161394; C1-161395; C1-161396; C1-161438; C1-161439; C1-161440; C1-161441; C1-161505; C1-161507; C1-161508; C1-161512; C1-161520; C1-161521. (3) Editorial changes by rapporteur. | 1.1.0 | 1.2.0 |
| 2016-02 |  |  |  |  | Further corrections by rapporteur | 1.2.0 | 1.2.1 |
| 2016-03 | CT-71 | CP-160059 |  |  | Version 2.0.0 created for presentation for approval | 1.2.1 | 2.0.0 |
| 2016-03 | CT-71 |  |  |  | Version 13.0.0 created after approval | 2.0.0 | 13.0.0 |
| 2016-03 |  |  |  |  | Minor editorial changes from TS rapporteur | 13.0.0 | 13.0.1 |
| 2016-06 | CT-72 | CP-160322 | 0001 | 1 | Subscription to the conference package | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0011 |  | Removing editor's note about MBMS and broadcast group call | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0002 | 1 | Triggering conference event subscription – controlling MCPTT function | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0003 | 1 | Triggering conference event subscription – non-controlling MCPTT function | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0004 | 1 | Triggering conference event subscription – MCPTT client | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0005 | 1 | Rejoin, join or late entry procedures added in non-controlling MCPTT function | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0006 | 1 | Clarifying SDP procedures | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0007 | 1 | Cleaning the usage of multipart/mixed content type | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0008 | 1 | URIs in SIP responses clarification | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0010 | 3 | Automatic commencement for On-Demand session clarifications | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0012 | 1 | Automatic commencement for pre-established session clarification | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0013 |  | Removing text about REFER in controlling MCPTT function | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0014 | 3 | Manual Commencement Mode corrections | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0015 |  | Correcting the use of <mcptt-calling-group-id> | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0016 |  | Removing isfocus in responses from non-controlling MCPTT function | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0017 | 4 | Temporary group flows as a result of group regrouping | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0018 | 1 | Corrections in affiliation procedures | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0019 | 5 | Corrections to call control | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0020 | 3 | 24.379 rel-13 Off-network Call Merge | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0021 | 2 | 24.379 rel-13 Off-network Emergency alert | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0022 |  | Off-network implicit downgrade timer | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0023 | 2 | 24.379 rel-13 Off-network mandatory floor control in private calls | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0024 | 2 | 24.379 rel-13 Off-network message format alignment | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0025 | 3 | 24.379 rel-13 Off-network editor's notes | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0026 | 2 | Corrections to clauses 4 to 12 | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0028 | 2 | Location procedures edits and cross reference fix | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0030 | 2 | MBMS corrections | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0032 | 2 | Annex B corrections | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0035 | 1 | Alignment of XML schema for extensibility | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0034 | 1 | The meaning of the word, see, when used in F.3.3 | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0036 | 1 | MIME body corrections | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0037 | 2 | Annex H corrections and modifications | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | C1-162029 | 0038 | 1 | MCPTT Session Identity clarification | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0039 |  | Replace the use of "this document" with "the present document" | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0040 | 2 | Correct the specification of the acknowledged call setup timer | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0041 | 2 | Correct the specification of the in-progress emergency group call timer. | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0042 |  | Compliance Statement | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160342 | 0043 | 3 | Correct authorisation to join a group call or initiate a group call. | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0044 |  | Affiliation Check | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0045 | 2 | Modifications to use of non-controlling function | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0046 |  | Private Call Corrections | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0047 | 1 | Setting the P-Asserted-Identity in responses from the terminating participating MCPTT function. | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0048 |  | Remove privacy requirements using SIP Privacy header field | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0049 | 4 | Out of dialog REFER and no implicit subscription. | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160343 | 0050 | 3 | MCPTT identities used in pre-established sessions | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0051 |  | Correct the encoding of the "purpose tag" in the PCK. | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0052 |  | Functional description of MONP | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0053 | 5 | General procedure for confidentiality protection of specific XML elements. | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0054 | 4 | General procedure for integrity protecting XML documents | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0055 | 3 | Corrections to SIP PUBLISH | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0056 | 5 | Descriptive text on MCPTT emergency alert subclause 4.6.3 | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0057 | 5 | Emergency alert client procedures | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0058 | 3 | Emergency alert participating and controlling function procedures | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0059 | 4 | Descriptive text on MCPTT imminent peril call Subclause 4.6.4 | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0060 | 4 | Client originating procedures for on-demand priority calls | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0061 | 4 | Partcipating MCPTT functions modifications for on-demand priority calls and alerts | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0062 | 5 | Chat controlling function emergency and imminent peril updates | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0063 | 4 | MCPTT prearranged controlling function updates for emergency and imminent peril origination | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0064 | 4 | Teminating MCPTT client receives a SIP INVITE request or SIP re-INVITE for an MCPTT group call | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0065 | 3 | Origination MCPTT client SIP REFER updates for MCPTT emergency and imminent peril group calls | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0067 | 4 | Descriptive text on MCPTT emergency private call - subclause 4.6.2 | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0068 | 4 | Description of imminent peril states | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0070 | 2 | Definition of content for the User location IE and removal of EN | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0071 |  | Removing several editor"s note | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0073 | 4 | Alignment of commencement mode determination with answer mode settings delivered by the MCPTT client | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0075 | 1 | Correcting MCPTT address to MCPTT ID | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160359 | 0076 | 3 | P-Asserted-Identity added in the Referred-By header field | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0077 | 1 | mcptt-group-id changed to mcptt-calling-group-id in annex H | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0078 | 1 | Changing from 2xx to 200 OK when sending a response | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0079 | 1 | Hiding of the MCPTT ID in MBMS listening status report | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0080 | 1 | MBMS usage procedure error corrections | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0082 | 2 | O-PF private call authorisations for on-demand call | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0083 | 2 | Alignment of XML schemas in annex F of 24.379 | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0084 | 1 | O-PF group call authorisations commencement mode actions | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0085 | 1 | T-PF private call commencement mode for on-demand call | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0086 | 1 | T-PF group call commencement mode for on-demand call | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0087 | 1 | Originating client private call modifications for on-demand call | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0088 | 1 | Terminating client private call modifications for on-demand call | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0089 | 1 | Terminating client group call modifications for on-demand call | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0090 | 1 | Maximum number of simultaneously received group calls (N6) | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0091 |  | Referring to on-network-max-participant-count in TS 24.381 | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0092 |  | Correcting names of XML elements "disabled" and "invite-members" | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0093 |  | Correcting name of "required" XML element | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0094 | 1 | Missing procedures of originating PF of authorized MCPTT user | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0095 | 1 | The g.3gpp.icsi-ref media feature tag included in the Contact header field | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0096 | 2 | Clarifications on calling a temporary group | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0097 | 1 | Adding use of "preferred-voice-encodings" to SDP procedures | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0098 |  | Adding use of "on-network-minimum-number-to-start" element to group procedures | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0099 | 1 | Implementing conditions for maximum duration of on-network group call | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0100 | 1 | Handling the current speaker during group regrouping | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0101 | 1 | MBSFN Area Id: Alt-1 (Removal from call control specification) | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0103 | 1 | MCPTT client origination procedures for on-demand private emergency call | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0104 | 1 | MCPTT client termination procedures for on-demand private emergency call | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0105 | 1 | MCPTT participating function procedures for private emergency call | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0106 | 1 | MCPTT controlling function procedures for private emergency call | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0107 |  | Correcting subsequent request procedures | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0108 | 1 | Switching from MBMS bearer to unicast bearer based on MBMS listening status | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0110 | 2 | Completion of Pre-established Session General | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0111 | 2 | Completion of Pre-established Session Establishment | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0112 | 3 | Completion of Pre-established Session Modification | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0122 |  | MBMS Service Area ID alignment | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0114 | 2 | Pre-established Session Release | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0115 | 1 | Off-network - Corrections to private call security context | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0116 | 1 | Off-network - Correction to state machines | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0117 | 1 | Off-network - Corrections to timers and counters. | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0118 |  | Removal of unused warning texts from subclause 4.4.2 | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0119 |  | Description of emergency private call states | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0120 |  | MCPTT server procedures for prearranged priority call re-INVITEs | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 | CP-160322 | 0123 |  | Reference number not implemented | 13.0.1 | 13.1.0 |
| 2016-06 | CT-72 |  |  |  | Various corrections | 13.1.0 | 13.1.1 |
| 2016-09 | CT-73 | CP-160439 | 0125 | 2 | Emergency group call cancel procedure updates for authorised users | 13.1.1 | 13.2.0 |
| 2016-09 | CT-73 | CP-160556 | 0126 | 2 | MCPTT state management and authorisation corrections for priority calls | 13.1.1 | 13.2.0 |
| 2016-09 | CT-73 | CP-160500 | 0127 | 1 | Introducing priority sharing in participating MCPTT function | 13.1.1 | 13.2.0 |
| 2016-09 | CT-73 | CP-160497 | 0128 | 1 | Resolving an editor's note about broadcast group call | 13.1.1 | 13.2.0 |
| 2016-09 | CT-73 | CP-160497 | 0129 | 1 | Terminating subscription to the event package | 13.1.1 | 13.2.0 |
| 2016-09 | CT-73 | CP-160497 | 0130 | 1 | Removing errors in re-INVITE and UPDATE procedures | 13.1.1 | 13.2.0 |
| 2016-09 | CT-73 | CP-160496 | 0131 |  | Authorizing MCPTT clients in constituent groups in non-controllers that do not expose group members, when initiating a temporary group session | 13.1.1 | 13.2.0 |
| 2016-09 | CT-73 | CP-160497 | 0132 | 1 | Clarifying the sending of SIP NOTIFY request during release of temporary group call | 13.1.1 | 13.2.0 |
| 2016-09 | CT-73 | CP-160497 | 0133 | 1 | Updating the release policy for chat group | 13.1.1 | 13.2.0 |
| 2016-09 | CT-73 | CP-160496 | 0134 |  | Moving overriding speaker - call control part | 13.1.1 | 13.2.0 |
| 2016-09 | CT-73 | CP-160500 | 0135 | 2 | Pre-established session functionality for emergency and imminent peril calls. | 13.1.1 | 13.2.0 |
| 2016-09 | CT-73 | CP-160496 | 0136 |  | MCPTT client ID | 13.1.1 | 13.2.0 |
| 2016-09 | CT-73 | CP-160498 | 0137 | 2 | Confidentiality of URIs in XML attributes | 13.1.1 | 13.2.0 |
| 2016-09 | CT-73 | CP-160557 | 0139 | 2 | Removal of Editor's Notes related to emergency functionality and possible optimizations | 13.1.1 | 13.2.0 |
| 2016-09 | CT-73 | CP-160496 | 0140 |  | Resolution of Editor's Notes related to priority mechanisms | 13.1.1 | 13.2.0 |
| 2016-09 | CT-73 | CP-160504 | 0141 | 2 | Assignment of Resource Priority header field values | 13.1.1 | 13.2.0 |
| 2016-09 | CT-73 | CP-160496 | 0143 |  | EN: Coding of <Configuration> element | 13.1.1 | 13.2.0 |
| 2016-09 | CT-73 | CP-160499 | 0144 | 1 | EN: Reference regarding mission critical organization. | 13.1.1 | 13.2.0 |
| 2016-09 | CT-73 | CP-160497 | 0145 | 1 | EN: Resource-Share and INVITE without SDP | 13.1.1 | 13.2.0 |
| 2016-09 | CT-73 | CP-160514 | 0146 |  | Reference update draft-holmberg-dispatch-mcptt-rp-namespace | 13.1.1 | 13.2.0 |
| 2016-09 | CT-73 | CP-160496 | 0147 |  | Update the use of configuration for signalling protection | 13.1.1 | 13.2.0 |
| 2016-09 | CT-73 | CP-160497 | 0148 | 1 | Description of encryption of elements and attributes in pre-established sessions | 13.1.1 | 13.2.0 |
| 2016-09 | CT-73 | CP-160496 | 0149 |  | Admitting a higher priority user to a call that has reached its max users | 13.1.1 | 13.2.0 |
| 2016-09 | CT-73 | CP-160440 | 0150 | 1 | Emergency Indications currently in SIP 200 (OK) to SIP INVITE requests need to be in SIP INFO requests | 13.1.1 | 13.2.0 |
| 2016-09 | CT-73 | CP-160496 | 0151 |  | Corrections for max simultaneous sessions | 13.1.1 | 13.2.0 |
| 2016-09 | CT-73 | CP-160500 | 0152 | 1 | Solving general editor's notes and removing solved editor's notes | 13.1.1 | 13.2.0 |
| 2016-09 | CT-73 | CP-160496 | 0153 |  | Removal of editor's notes related to id hiding | 13.1.1 | 13.2.0 |
| 2016-09 | CT-73 | CP-160496 | 0154 |  | Management of media resource allocation for pre-established sessions | 13.1.1 | 13.2.0 |
| 2016-09 | CT-73 | CP-160496 | 0155 |  | Remove privacy requirements from Rel-13 MCPTT | 13.1.1 | 13.2.0 |
| 2016-09 | CT-73 | CP-160496 | 0156 |  | XML schema changes to accommodate confidentiality protection | 13.1.1 | 13.2.0 |
| 2016-09 | CT-73 | CP-160498 | 0157 | 2 | XML schema changes to accommodate integrity protection | 13.1.1 | 13.2.0 |
| 2016-09 | CT-73 | CP-160542 | 0158 | 5 | MCPTT Service Continuity | 13.1.1 | 13.2.0 |
| 2016-09 | CT-73 | CP-160500 | 0160 | 2 | IP address and port number for media stream | 13.1.1 | 13.2.0 |
| 2016-09 | CT-73 | CP-160497 | 0162 | 1 | Authorization checks | 13.1.1 | 13.2.0 |
| 2016-09 | CT-73 | CP-160496 | 0163 |  | Removal of editor's note on call probe | 13.1.1 | 13.2.0 |
| 2016-09 | CT-73 | CP-160500 | 0165 | 1 | Max duration of the calls | 13.1.1 | 13.2.0 |
| 2016-09 | CT-73 | CP-160497 | 0166 |  | Restrict notification of call failure reason for private call | 13.1.1 | 13.2.0 |
| 2016-09 | CT-73 | CP-160500 | 0167 | 1 | Timer TFB3 | 13.1.1 | 13.2.0 |
| 2016-09 | CT-73 | CP-160500 | 0168 | 1 | IANA Registration form for UDP Port number for MONP messages | 13.1.1 | 13.2.0 |
| 2016-09 | CT-73 | CP-160497 | 0169 |  | Error in authorization reference | 13.1.1 | 13.2.0 |
| 2016-09 | CT-73 | CP-160500 | 0170 | 2 | MCPTT session identity and subscription to conference events | 13.1.1 | 13.2.0 |
| 2016-09 | CT-73 | CP-160500 | 0171 | 3 | Completion of Pre-established session | 13.1.1 | 13.2.0 |
| 2016-09 | CT-73 | CP-160500 | 0172 | 1 | Resolution of Editor's Note on NATs | 13.1.1 | 13.2.0 |
| 2016-09 | CT-73 | CP-160500 | 0176 |  | Downloading MCPTT user profile when receiving the service settings | 13.1.1 | 13.2.0 |
| 2016-09 | CT-73 | CP-160558 | 0177 |  | Alignment with MCPTT user profile document in TS 24.384 | 13.1.1 | 13.2.0 |

# History

| Document history |  |  |
| --- | --- | --- |
| V13.0.1 | May 2016 | Publication |
| V13.1.1 | July 2016 | Publication |
| V13.2.0 | October 2016 | Publication |