Annex to ETSI EN 300 395-2

Terrestrial Trunked Radio (TETRA);

Speech codec for full-rate traffic channel;

Part 2: TETRA codec

Annex E.1

ANSI C code for the TETRA speech codec

This is an unpublished work the copyright in which vests in the European Telecommunications Standards Institute. All rights reserved.

The information contained herein is the property of ETSI and no part may be reproduced or used except as authorised by contract or other written permission. The copyright and the foregoing restriction on reproduction and use extend to all media in which the information may be embodied.

Reference

DTS/TCCE-06211

Keywords

Security, TETRA V+D, DMO, air interface

Important notice

The present document can be downloaded from:
https://www.etsi.org/standards-search

The present document may be made available in electronic versions and/or in print. The content of any electronic and/or print versions of the present document shall not be modified without the prior written authorization of ETSI. In case of any existing or perceived difference in contents between such versions and/or in print, the prevailing version of an ETSI deliverable is the one made publicly available in PDF format at www.etsi.org/deliver.

Users of the present document should be aware that the document may be subject to revision or change of status. Information on the current status of this and other ETSI documents is available at https://portal.etsi.org/TB/ETSIDeliverableStatus.aspx

If you find errors in the present document, please send your comment to one of the following services:
https://portal.etsi.org/People/CommiteeSupportStaff.aspx

If you find a security vulnerability in the present document, please report it through our

Coordinated Vulnerability Disclosure Program:

https://www.etsi.org/standards/coordinated-vulnerability-disclosure

Notice of disclaimer & limitation of liability

The information provided in the present deliverable is directed solely to professionals who have the appropriate degree of experience to understand and interpret its content in accordance with generally accepted engineering or

other professional standard and applicable regulations.

No recommendation as to products and services or vendors is made or should be implied.

No representation or warranty is made that this deliverable is technically accurate or sufficient or conforms to any law and/or governmental rule and/or regulation and further, no representation or warranty is made of merchantability or fitness for any particular purpose or against infringement of intellectual property rights.

In no event shall ETSI be held liable for loss of profits or any other incidental or consequential damages.

Any software contained in this deliverable is provided "AS IS" with no warranties, express or implied, including but not limited to, the warranties of merchantability, fitness for a particular purpose and non-infringement of intellectual property rights and ETSI shall not be held liable in any event for any damages whatsoever (including, without limitation, damages for loss of profits, business interruption, loss of information, or any other pecuniary loss) arising out of or related to the use of or inability to use the software.

Copyright Notification

No part may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm except as authorized by written permission of ETSI.
The content of the PDF version shall not be modified without the written authorization of ETSI.
The copyright and the foregoing restriction extend to reproduction in all media.

© ETSI 2024.

All rights reserved.

ETSI

650 Route des Lucioles

F-06921 Sophia Antipolis Cedex - FRANCE

Tel.: +33 4 92 94 42 00   Fax: +33 4 93 65 47 16

Siret N° 348 623 562 00017 - APE 7112B

Association à but non lucratif enregistrée à la

Sous-préfecture de Grasse (06) N° w061004871

Contents

Scope	8

A	C Code for Full Rate Speech Source Codec	9

A.1	Main Program for Source Coder	9

A.2	Main Program for Source Decoder	25

A.3	Library of Source Coder/Decoder Subroutines	36

A.4	Library of Signal Processing Related Subroutines	91

A.5	Library of Basic and Mathematic Functions	116

A.5.1	Basic Functions	116

A.5.2	Extended Precision Functions	127

A.5.3	Mathematic Functions	134

A.6	Tables and Constants for Source Coding	141

B	C Code for the TETRA Basic Operators	176

C	C Code for Speech Channel Coding	211

C.1	Main Program for Speech Channel Coding	211

C.2	Library of Speech Channel Coding Subroutines	218

C.3	Tables and Constants for Speech Channel	237

D	C Code for Speech Channel Decoding	246

D.1	Main Program for Speech Channel Decoding	246

D.2	Library of Speech Channel Decoding Subroutines	253

E	Makefile	276

F 	Indexes	277

F.1	Index of Routines	277

F.2	Index of Files	280

History	282

# Intellectual Property Rights

Essential patents

IPRs essential or potentially essential to normative deliverables may have been declared to ETSI. The declarations pertaining to these essential IPRs, if any, are publicly available for ETSI members and non-members, and can be found in ETSI SR 000 314: "Intellectual Property Rights (IPRs); Essential, or potentially Essential, IPRs notified to ETSI in respect of ETSI standards", which is available from the ETSI Secretariat. Latest updates are available on the ETSI Web server (https://ipr.etsi.org).

Pursuant to the ETSI Directives including the ETSI IPR Policy, no investigation regarding the essentiality of IPRs, including IPR searches, has been carried out by ETSI. No guarantee can be given as to the existence of other IPRs not referenced in ETSI SR 000 314 (or the updates on the ETSI Web server) which are, or may be, or may become, essential to the present document.

Trademarks

The present document may include trademarks and/or tradenames which are asserted and/or registered by their owners. ETSI claims no ownership of these except for any which are indicated as being the property of ETSI, and conveys no right to use or reproduce any trademark and/or tradename. Mention of those trademarks in the present document does not constitute an endorsement by ETSI of products, services or organizations associated with those trademarks.

DECT™, PLUGTESTS™, UMTS™ and the ETSI logo are trademarks of ETSI registered for the benefit of its Members. 3GPP™ and LTE™ are trademarks of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners. oneM2M™ logo is a trademark of ETSI registered for the benefit of its Members and of the oneM2M Partners. GSM® and the GSM logo are trademarks registered and owned by the GSM Association.

# Foreword

This Unpublished Annex has been produced to replace the original version which was drafted in MS Word 2 and is now unreadable.

# Modal verbs terminology

In the present document "shall", "shall not", "should", "should not", "may", "need not", "will", "will not", "can" and "cannot" are to be interpreted as described in clause 3.2 of the ETSI Drafting Rules (Verbal forms for the expression of provisions).

"must" and "must not" are NOT allowed in ETSI deliverables except when used in direct citation.

# 1	Scope

This document contains a printed version of the C code associated with ETS 300 395 part 2.

In order to increase its readability, this document has been split in a few sections. Also, a different font is used for the headers and for the executable code.

Indexes of all the routines and files included in the document are also provided

# 2	References

## 2.1	Normative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

Referenced documents which are not found to be publicly available in the expected location might be found at https://docbox.etsi.org/Reference/.

NOTE:	While any hyperlinks included in this clause were valid at the time of publication, ETSI cannot guarantee their long term validity.

The following referenced documents are necessary for the application of the present document.

[1]

## 2.2	Informative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

NOTE:	While any hyperlinks included in this clause were valid at the time of publication, ETSI cannot guarantee their long-term validity.

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1]

# 3	C Code for Full Rate Speech Source Codec

## A.1	Main Program for Source Coder

/************************************************************************

*

*	FILENAME		:	scoder					.c

*

*	DESCRIPTION		:	Main program for speech source encoding

*

************************************************************************

*

*	USAGE			:	scoder  speech_file  serial_file  synth_file

*					(executable_file input1 output1 output2)

*

*	INPUT FILE(S)		:

*

*		INPUT1	:	- Description : speech file to be analyzed

*					- Format : binary file 16 bit-samples

*					  240 samples per frame

*

*

*	OUTPUT FILE(S)	:

*

*		OUTPUT1	:	- Description : serial stream output file

*					- Format : binary file 16 bit-samples

*					  each 16 bit-sample represents one encoded bit

*					  138 (= 1 + 137) bits per frame

*

*		OUTPUT2	:	- Description : local synthesis output file

*					- Format : binary file 16 bit-samples

*

*	COMMENTS		:	First sample of each OUTPUT1 frame is forced to zero

*

************************************************************************

*

*	INCLUDED FILES	:	source.h

*					stdio.h

*					stdlib.h

*

************************************************************************/

#include <stdio.h>

#include <stdlib.h>

#include "source.h"

/*-----------------*

* Constants       *

*-----------------*/

#define L_frame     240

#define serial_size 138

#define ana_size     23

int main(int argc, char *argv[] )

{

Word16 frame;

extern Word16 *new_speech;		/* Pointer on new_speech. */

Word16 syn[L_frame];			/* Local synthesis.       */

Word16 ana[ana_size];			/* Analysis parameters.   */

Word16 serial[serial_size];		/* Serial stream.         */

FILE  *f_speech, *f_syn, *f_serial;

/* Passed arguments */

if ( argc != 4 )

{

printf("Usage : scoder  speech_file  serial_file  synth_file\n");

printf("\n");

printf("Format for speech_file:\n");

printf("  Speech is read form a binary file of 16 bits data.\n");

printf("\n");

printf("Format for serial_file:\n");

printf("  Serial stream output is written to a binary file\n");

printf("  where each 16-bit word represents 1 encoded bit.\n");

printf("  BFI(=0) + 137 bits by frame\n");

printf("\n");

printf("Format for synth_file:\n");

printf("  Local synthesis is written to a binary file of 16 bits data.\n");

exit( 1 );

}

/* Open files for speech, serial stream and local synthesis */

if( (f_speech = fopen(argv[1],"rb") ) == NULL )

{

printf("Input file '%s' does not exist !!\n", argv[1]);

exit(0);

}

if( (f_serial = fopen(argv[2], "wb") ) == NULL )

{

printf("Cannot open file '%s' !!\n", argv[2]);

exit(0);

}

if( (f_syn = fopen(argv[3], "wb") ) == NULL )

{

printf("Cannot open file '%s' !!\n", argv[3]);

exit(0);

}

/* Initialization of the coder */

Init_Pre_Process();

Init_Coder_Tetra();

/* Loop for each "L_frame" speech data. */

frame =0;

while( fread(new_speech, sizeof(Word16), L_frame, f_speech) == L_frame)

{

printf("frame=%d\n", ++frame);

Pre_Process(new_speech, (Word16)L_frame);	/* Pre processing of input

speech */

Coder_Tetra(ana, syn);              /* Find speech parameters         */

Post_Process(syn, (Word16)L_frame); /* Post processing of synthesis   */

Prm2bits_Tetra(ana, serial);        /* Parameters to serial bits      */

fwrite(syn,    sizeof(Word16), L_frame    ,  f_syn);

fwrite(serial, sizeof(Word16), serial_size,  f_serial);

}

return (EXIT_SUCCESS);

}

/************************************************************************

*

*	FILENAME		:	scod_tet.c

*

*	DESCRIPTION		:	Main routines for speech source encoding

*

************************************************************************

*

*	SUB-ROUTINES	:	- Init_Coder_Tetra()

*					- Coder_Tetra()

*

************************************************************************

*

*	INCLUDED FILES	:	source.h

*

************************************************************************/

#include "source.h"

/*----------------------------------------------------------------------*

*         Coder constant parameters.                                   *

*                                                                      *

*   L_window    : LPC analysis window size.                            *

*   L_next      : Samples of next frame needed for autocor.            *

*   L_frame     : Frame size.                                          *

*   L_subfr     : Sub-frame size.                                      *

*   p           : LPC order.                                           *

*   pp1         : LPC order+1                                          *

*   L_total     : Total speech size.                                   *

*   dim_rr      : Dimension of matrix rr[][].                          *

*   pit_min     : Minimum pitch lag.                                   *

*   pit_max     : Maximum pitch lag.                                   *

*   L_inter     : Length of filter for interpolation                   *

*----------------------------------------------------------------------*/

#define  L_window (Word16)256

#define  L_next   (Word16)40

#define  L_frame  (Word16)240

#define  L_subfr  (Word16)60

#define  p        (Word16)10

#define  pp1      (Word16)11

#define  L_total  (Word16)(L_frame+L_next+p)

#define  dim_rr   (Word16)32

#define  pit_min  (Word16)20

#define  pit_max  (Word16)143

#define  L_inter  (Word16)15

/*--------------------------------------------------------*

*   LPC bandwidth expansion factors.                     *

*      In Q15 = 0.95, 0.60, 0.75, 0.85                   *

*--------------------------------------------------------*/

#define gamma1  (Word16)31130

#define gamma2  (Word16)19661

#define gamma3  (Word16)24576

#define gamma4  (Word16)27853

/*--------------------------------------------------------*

*         Static memory allocation.                      *

*--------------------------------------------------------*/

/* Speech vector */

static Word16 old_speech[L_total];

static Word16 *speech, *p_window;

Word16 *new_speech;                    /* Global variable */

/* Weighted speech vector */

static Word16 old_wsp[L_frame+pit_max];

static Word16 *wsp;

/* Excitation vector */

static Word16 old_exc[L_frame+pit_max+L_inter];

static Word16 *exc;

/* All-zero vector */

static Word16 ai_zero[L_subfr+pp1];

static Word16 *zero;

/* Spectral expansion factors */

static Word16 F_gamma1[p];

static Word16 F_gamma2[p];

static Word16 F_gamma3[p];

static Word16 F_gamma4[p];

/* Lsp (Line Spectral Pairs in the cosine domain) */

static Word16 lspold[p]={

30000, 26000, 21000, 15000, 8000, 0,

-8000,-15000,-21000,-26000};

static Word16 lspnew[p];

static Word16 lspnew_q[p], lspold_q[p];

/* Initial lsp values used after each time */

/* a reset is executed */

static Word16 lspold_init[p]={

30000, 26000, 21000, 15000, 8000, 0,

-8000,-15000,-21000,-26000};

/* Filters memories */

static Word16 mem_syn[p], mem_w0[p], mem_w[p];

/* Matrix rr[dim_rr][dim_rr] */

static Word16 rr[dim_rr][dim_rr];

/* Global definition */

Word16 last_ener_cod;

Word16 last_ener_pit;

/**************************************************************************

*

*	ROUTINE				:	Init_Coder_Tetra

*

*	DESCRIPTION			:	Initialization of variables for the speech encoder

*

**************************************************************************

*

*	USAGE				:	Init_Coder_Tetra()

*

*	INPUT ARGUMENT(S)		:	None

*

*	OUTPUT ARGUMENT(S)		:	None

*

*	RETURNED VALUE		:	None

*

**************************************************************************/

void Init_Coder_Tetra(void)

{

Word16 i,j;

/*-----------------------------------------------------------------------*

*      Initialize pointers to speech vector.                            *

*                                                                       *

*                                                                       *

*   |--------------------|--------|--------|......|-------|-------|     *

*     previous speech       sf1      sf2             sf4    L_next      *

*                                                                       *

*   <----------------  Total speech vector (L_total)   ----------->     *

*   |           <----  LPC analysis window (L_window)  ----------->     *

*   |           |        <---- present frame (L_frame) --->             *

*  old_speech   |        |       <----- new speech (L_frame) ----->     *

*            p_window    |       |                                      *

*                     speech     |                                      *

*                             new_speech                                *

*-----------------------------------------------------------------------*/

new_speech = old_speech + L_total - L_frame;	/* New speech     */

speech     = new_speech - L_next;			/* Present frame  */

p_window   = old_speech + L_total - L_window;	/* For LPC window */

/* Initialize global variables */

last_ener_cod = 0;

last_ener_pit = 0;

/* Initialize static pointers */

wsp    = old_wsp + pit_max;

exc    = old_exc + pit_max + L_inter;

zero   = ai_zero + pp1;

/* Static vectors to zero */

for(i=0; i<L_total; i++)

old_speech[i] = 0;

for(i=0; i<pit_max + L_inter; i++)

old_exc[i] = old_wsp[i] = 0;

for(i=0; i<p; i++)

mem_syn[i] = mem_w[i] = mem_w0[i] = 0;

for(i=0; i<L_subfr; i++)

zero[i] = 0;

for(i=0; i< dim_rr; i++)

for(j=0; j< dim_rr; j++)

rr[i][j] = 0;

/* Initialisation of lsp values for first */

/* frame lsp interpolation */

for(i=0; i<p; i++)

lspold_q[i] = lspold[i] = lspold_init[i];

/* Compute LPC spectral expansion factors */

Fac_Pond(gamma1, F_gamma1);

Fac_Pond(gamma2, F_gamma2);

Fac_Pond(gamma3, F_gamma3);

Fac_Pond(gamma4, F_gamma4);

return;

}

/**************************************************************************

*

*	ROUTINE				:	Coder_Tetra

*

*	DESCRIPTION			:	Main speech coder function

*

**************************************************************************

*

*	USAGE				:	Coder_Tetra(ana,synth)

*							(Routine_Name(output1,output2))

*

*	INPUT ARGUMENT(S)		:	None

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : Analysis parameters

*							- Format : 23 * 16 bit-samples

*

*		OUTPUT2			:	- Description : Local synthesis

*							- Format : 240 * 16 bit-samples

*

*	RETURNED VALUE		:	None

*

*	COMMENTS			:	- 240 speech data should have been copied to vector

*						new_speech[].  This vector is global and is declared in

*						this function.

*						- Output2 is for debugging only

*

**************************************************************************/

void Coder_Tetra(Word16 ana[], Word16 synth[])

{

/* LPC coefficients */

Word16 r_l[pp1], r_h[pp1];	/* Autocorrelations low and high        */

Word16 A_t[(pp1)*4];		/* A(z) unquantized for the 4 subframes */

Word16 Aq_t[(pp1)*4];		/* A(z)   quantized for the 4 subframes */

Word16 Ap1[pp1];		/* A(z) with spectral expansion         */

Word16 Ap2[pp1];		/* A(z) with spectral expansion         */

Word16 Ap3[pp1];		/* A(z) with spectral expansion         */

Word16 Ap4[pp1];		/* A(z) with spectral expansion         */

Word16 *A, *Aq;			/* Pointer on A_t and Aq_t              */

/* Other vectors */

Word16 h1[L_subfr];

Word16 zero_h2[L_subfr+64], *h2;

Word16 zero_F[L_subfr+64],  *F;

Word16 res[L_subfr];

Word16 xn[L_subfr];

Word16 xn2[L_subfr];

Word16 dn[L_subfr+4];

Word16 code[L_subfr+4];

Word16 y1[L_subfr];

Word16 y2[L_subfr];

/* Scalars */

Word16 i, i_subfr;

Word16 T0, T0_min, T0_max, T0_frac;

Word16 gain_pit, gain_code, index;

Word16 sign_code, shift_code;

Word16 temp;

Word32 L_temp;

/* Initialization of F and h2 */

F  = &zero_F[64];

h2 = &zero_h2[64];

for(i=0; i<64; i++)

zero_F[i] = zero_h2[i] = 0;

/*------------------------------------------------------------------------*

*  - Perform LPC analysis:                                               *

*       * autocorrelation + lag windowing                                *

*       * Levinson-Durbin algorithm to find a[]                          *

*       * convert a[] to lsp[]                                           *

*       * quantize and code the LSPs                                     *

*       * find the interpolated LSPs and convert to a[] for all          *

*         subframes (both quantized and unquantized)                     *

*------------------------------------------------------------------------*/

Autocorr(p_window, p, r_h, r_l);		/* Autocorrelations */

Lag_Window(p, r_h, r_l);			/* Lag windowing    */

Levin_32(r_h, r_l, A_t);			/* Levinson-Durbin  */

Az_Lsp(A_t, lspnew, lspold);		/* From A(z) to lsp */

Clsp_334(lspnew, lspnew_q, ana);		/* Lsp quantization */

ana += 3;				/* Increment analysis parameters pointer */

/* Interpolation of LPC for the 4 subframes */

Int_Lpc4(lspold,   lspnew,   A_t);

Int_Lpc4(lspold_q, lspnew_q, Aq_t);

/* update the LSPs for the next frame */

for(i=0; i<p; i++)

{

lspold[i]   = lspnew[i];

lspold_q[i] = lspnew_q[i];

}

/*----------------------------------------------------------------------*

* - Find the weighted input speech wsp[] for the whole speech frame    *

* - Find open-loop pitch delay                                         *

* - Set the range for searching closed-loop pitch                      *

*----------------------------------------------------------------------*/

A = A_t;

for (i = 0; i < L_frame; i += L_subfr)

{

Pond_Ai(A, F_gamma1, Ap1);

Pond_Ai(A, F_gamma2, Ap2);

Residu(Ap1, &speech[i], &wsp[i], L_subfr);

Syn_Filt(Ap2, &wsp[i], &wsp[i], L_subfr, mem_w, (Word16)1);

A += pp1;

}

/* Find open loop pitch delay */

T0 = Pitch_Ol_Dec(wsp, L_frame);

/* range for closed loop pitch search */

T0_min = sub(T0, (Word16)2);

if (T0_min < pit_min) T0_min = pit_min;

T0_max = add(T0_min, (Word16)4);

if (T0_max > pit_max)

{

T0_max = pit_max;

T0_min = sub(T0_max, (Word16)4);

}

/*------------------------------------------------------------------------*

*          Loop for every subframe in the analysis frame                 *

*------------------------------------------------------------------------*

*  To find the pitch and innovation parameters. The subframe size is     *

*  L_subfr and the loop is repeated L_frame/L_subfr times.               *

*     - find the weighted LPC coefficients                               *

*     - find the LPC residual signal res[]                               *

*     - compute the target signal for pitch search                       *

*     - compute impulse response of weighted synthesis filter (h1[])     *

*     - find the closed-loop pitch parameters                            *

*     - encode the pitch delay                                           *

*     - update the impulse response h1[] by including fixed-gain pitch   *

*     - find the autocorrelations of h1[] (rr[][])                       *

*     - find target vector for codebook search                           *

*     - backward filtering of target vector                              *

*     - codebook search                                                  *

*     - encode codebook address                                          *

*     - VQ of pitch and codebook gains                                   *

*     - find synthesis speech                                            *

*     - update states of weighting filter                                *

*------------------------------------------------------------------------*/

Aq = Aq_t;	/* pointer to interpolated quantized LPC parameters */

for (i_subfr = 0;  i_subfr < L_frame; i_subfr += L_subfr)

{

/*---------------------------------------------------------------*

* Find the weighted LPC coefficients for the weighting filter.  *

*---------------------------------------------------------------*/

Pond_Ai(Aq, F_gamma3, Ap3);

Pond_Ai(Aq, F_gamma4, Ap4);

/*---------------------------------------------------------------*

* Compute impulse response, h1[], of weighted synthesis filter  *

*---------------------------------------------------------------*/

ai_zero[0] = 4096;				/* 1 in Q12 */

for (i = 1; i <= p; i++) ai_zero[i] = 0;

Syn_Filt(Ap4, ai_zero, h1, L_subfr, zero, (Word16)0);

/*---------------------------------------------------------------*

* Compute LPC residual and copy it to exc[i_subfr]              *

*---------------------------------------------------------------*/

Residu(Aq, &speech[i_subfr], res, L_subfr);

for(i=0; i<L_subfr; i++) exc[i_subfr+i] = res[i];

/*---------------------------------------------------------------*

* Find the target vector for pitch search:  ->xn[]              *

*---------------------------------------------------------------*/

Syn_Filt(Ap4, res, xn, L_subfr, mem_w0, (Word16)0);

/*----------------------------------------------------------------------*

*                 Closed-loop fractional pitch search                  *

*----------------------------------------------------------------------*

* The pitch range for the first subframe is divided as follows:        *

*   19 1/3  to   84 2/3   resolution 1/3                               *

*   85      to   143      resolution 1                                 *

*                                                                      *

* The period in the first subframe is encoded with 8 bits.             *

* For the range with fractions:                                        *

*   code = (T0-19)*3 + frac - 1;   where T0=[19..85] and frac=[-1,0,1] *

* and for the integer only range                                       *

*   code = (T0 - 85) + 197;        where T0=[86..143]                  *

*----------------------------------------------------------------------*

* For other subframes: if t0 is the period in the first subframe then  *

* T0_min=t0-5   and  T0_max=T0_min+9   and  the range is given by      *

*      T0_min-1 + 1/3   to  T0_max + 2/3                               *

*                                                                      *

* The period in the 2nd,3rd,4th subframe is encoded with 5 bits:       *

*  code = (T0-(T0_min-1))*3 + frac - 1;  where T0[T0_min-1 .. T0_max+1]*

*---------------------------------------------------------------------*/

T0 = Pitch_Fr(&exc[i_subfr], xn, h1, L_subfr, T0_min, T0_max,

i_subfr, &T0_frac);

if (i_subfr == 0)

{

/* encode pitch delay (with fraction) */

if (T0 <= 85)

{

/* index = T0*3 - 58 + T0_frac; */

index = add(T0, add(T0, T0));

index = sub(index, (Word16)58);

index = add(index, T0_frac);

}

else

index = add(T0, (Word16)112);

/* find T0_min and T0_max for other subframes */

T0_min = sub(T0, (Word16)5);

if (T0_min < pit_min) T0_min = pit_min;

T0_max = add(T0_min, (Word16)9);

if (T0_max > pit_max)

{

T0_max = pit_max;

T0_min = sub(T0_max, (Word16)9);

}

}

else						/* other subframes */

{

i = sub(T0, T0_min);

/* index = i*3 + 2 + T0_frac;  */

index = add(i, add(i, i));

index = add(index, (Word16)2);

index = add(index, T0_frac);

}

*ana++ = index;

/*-----------------------------------------------------------------*

*   - find unity gain pitch excitation (adaptive codebook entry)  *

*     with fractional interpolation.                              *

*   - find filtered pitch exc. y1[]=exc[] filtered by 1/Ap4(z)    *

*   - compute pitch gain and limit between 0 and 1.2              *

*   - update target vector for codebook search                    *

*-----------------------------------------------------------------*/

Pred_Lt(&exc[i_subfr], T0, T0_frac, L_subfr);

Syn_Filt(Ap4, &exc[i_subfr], y1, L_subfr, zero, (Word16)0);

gain_pit = G_Pitch(xn, y1, L_subfr);

/* xn2[i] = xn[i] - y1[i]*gain_pit */

for (i = 0; i < L_subfr; i++)

{

L_temp = L_mult(y1[i], gain_pit);

L_temp = L_shl(L_temp, (Word16)3);	/* gain_pit in Q12 */

L_temp = L_sub( Load_sh16(xn[i]), L_temp);

xn2[i] = extract_h(L_temp);

}

/*----------------------------------------------------------------*

* -Compute impulse response F[] and h2[] for innovation codebook *

* -Find correlations of h2[];  rr[i][j] = sum h2[n-i]*h2[n-j]    *

*----------------------------------------------------------------*/

for (i = 0; i <= p; i++) ai_zero[i] = Ap3[i];

Syn_Filt(Ap4, ai_zero, F, L_subfr, zero, (Word16)0);

/* Introduce pitch contribution with fixe gain of 0.8 to F[] */

for (i = T0; i < L_subfr; i++)

{

temp = mult(F[i-T0], (Word16)26216);

F[i] = add(F[i], temp);

}

/* Compute h2[]; -> F[] filtered by 1/Ap4(z) */

Syn_Filt(Ap4, F, h2, L_subfr, zero, (Word16)0);

Cal_Rr2(h2, (Word16*)rr);

/*-----------------------------------------------------------------*

* - Backward filtering of target vector (find dn[] from xn2[])    *

* - Innovative codebook search (find index and gain)              *

*-----------------------------------------------------------------*/

Back_Fil(xn2, h2, dn, L_subfr);	/* backward filtered target vector dn */

*ana++ =D4i60_16(dn,F,h2, rr, code, y2, &sign_code, &shift_code);

*ana++ = sign_code;

*ana++ = shift_code;

gain_code = G_Code(xn2, y2, L_subfr);

/*-----------------------------------------------------------------*

* - Quantization of gains.                                        *

*-----------------------------------------------------------------*/

*ana++ = Ener_Qua(Aq,&exc[i_subfr],code, L_subfr, &gain_pit, &gain_code);

/*-------------------------------------------------------*

* - Find the total excitation                           *

* - Update filter memory mem_w0 for finding the target  *

*   vector in the next subframe.                        *

*   The filter state mem_w0[] is found by filtering by  *

*   1/Ap4(z) the error between res[i] and exc[i]        *

*-------------------------------------------------------*/

for (i = 0; i < L_subfr;  i++)

{

/* exc[i] = gain_pit*exc[i] + gain_code*code[i]; */

/* exc[i]  in Q0   gain_pit in Q12               */

/* code[i] in Q12  gain_cod in Q0                */

L_temp = L_mult0(exc[i+i_subfr], gain_pit);

L_temp = L_mac0(L_temp, code[i], gain_code);

exc[i+i_subfr] = L_shr_r(L_temp, (Word16)12);

}

for(i=0; i<L_subfr; i++)

res[i] = sub(res[i], exc[i_subfr+i]);

Syn_Filt(Ap4, res, code, L_subfr, mem_w0, (Word16)1);

/* Note: we use vector code[] as output only as temporary vector */

/*-------------------------------------------------------*

* - find synthesis speech corresponding to exc[]        *

*   This filter is to help debug only.                  *

*-------------------------------------------------------*/

Syn_Filt(Aq, &exc[i_subfr], &synth[i_subfr], L_subfr, mem_syn,

(Word16)1);

Aq += pp1;

}

/*--------------------------------------------------*

* Update signal for next frame.                    *

* -> shift to the left by L_frame:                 *

*     speech[], wsp[] and  exc[]                   *

*--------------------------------------------------*/

for(i=0; i< L_total-L_frame; i++)

old_speech[i] = old_speech[i+L_frame];

for(i=0; i<pit_max; i++)

old_wsp[i] = old_wsp[i+L_frame];

for(i=0; i<pit_max+L_inter; i++)

old_exc[i] = old_exc[i+L_frame];

return;

}

## A.2	Main Program for Source Decoder

/************************************************************************

*

*	FILENAME		:	sdecoder			.c

*

*	DESCRIPTION		:	Main program for speech source decoding

*

************************************************************************

*

*	USAGE			:	sdecoder  serial_file  synth_file

*					(executable_file input1 output1)

*

*	INPUT FILE(S)		:

*

*		INPUT1	:	- Description : serial stream input file

*					- Format : binary file 16 bit-samples

*					  138 (= 1 + 137) samples per frame

*

*	OUTPUT FILE(S)	:

*

*		OUTPUT1	:	- Description : synthesis output file

*					- Format : binary file 16 bit-samples

*					  240 bits per frame

*

*	COMMENTS		:	First sample of each INPUT1 frame is the bad frame

*					indicator (BFI), BFI = 0 corresponds to a valid frame,

*					BFI = 1 to corrupted data

*

************************************************************************

*

*	INCLUDED FILES	:	source.h

*					stdio.h

*					stdlib.h

*

************************************************************************/

#include <stdio.h>

#include <stdlib.h>

#include "source.h"

/*-----------------*

* Constants       *

*-----------------*/

#define L_frame     240

#define serial_size 138

#define prm_size    24

int main(int argc, char *argv[] )

{

Word16 frame;

Word16 synth[L_frame];		/* Synthesis              */

Word16 parm[prm_size];		/* Synthesis parameters   */

Word16 serial[serial_size];		/* Serial stream          */

FILE   *f_syn, *f_serial;

/* Passed arguments */

if ( argc != 3 )

{

printf("Usage : sdecoder  serial_file  synth_file\n");

printf("\n");

printf("Format for serial_file:\n");

printf("  Serial stream input is read from a binary file\n");

printf("  where each 16-bit word represents 1 encoded bit.\n");

printf("  BFI + 137 bits by frame\n");

printf("\n");

printf("Format for synth_file:\n");

printf("  Synthesis is written to a binary file of 16 bits data.\n");

exit( 1 );

}

/* Open file for synthesis and serial stream */

if( (f_serial = fopen(argv[1],"rb") ) == NULL )

{

printf("Input file '%s' does not exist !!\n",argv[1]);

exit(0);

}

if( (f_syn = fopen(argv[2], "wb") ) == NULL )

{

printf("Cannot open file '%s' !!\n", argv[2]);

exit(0);

}

/* Initialization of decoder  */

Init_Decod_Tetra();

/* Loop for each "L_frame" speech data. */

frame =0;

while( fread(serial, sizeof(Word16), serial_size, f_serial) == serial_size)

{

printf("frame=%d\n", ++frame);

Bits2prm_Tetra(serial, parm);	/* serial to parameters */

Decod_Tetra(parm, synth);		/* decoder */

Post_Process(synth, (Word16)L_frame);	/* Post processing of synthesis  */

fwrite(synth, sizeof(Word16), L_frame,  f_syn);   /* Write output file */

}

return (EXIT_SUCCESS);

}

/************************************************************************

*

*	FILENAME		:	sdec_tet.c

*

*	DESCRIPTION		:	Main routines for speech source decoding

*

************************************************************************

*

*	SUB-ROUTINES	:	- Init_Decod_Tetra()

*					- Decod_Tetra()

*

************************************************************************

*

*	INCLUDED FILES	:	source.h

*

************************************************************************/

#include "source.h"

/*--------------------------------------------------------*

*       Decoder constants parameters.                    *

*                                                        *

*   L_frame     : Frame size.                            *

*   L_subfr     : Sub-frame size.                        *

*   p           : LPC order.                             *

*   pp1         : LPC order+1                            *

*   pit_min     : Minimum pitch lag.                     *

*   pit_max     : Maximum pitch lag.                     *

*   L_inter     : Length of filter for interpolation     *

*   parm_size   : Lenght of vector parm[]                *

*--------------------------------------------------------*/

#define  L_frame  (Word16)240

#define  L_subfr  (Word16)60

#define  p        (Word16)10

#define  pp1      (Word16)11

#define  pit_min  (Word16)20

#define  pit_max  (Word16)143

#define  L_inter  (Word16)15

#define  parm_size (Word16)23

/*--------------------------------------------------------*

*   LPC bandwidth expansion factors for noise filter.    *

*      In Q15 = 0.75, 0.85                               *

*--------------------------------------------------------*/

#define gamma3  (Word16)24576

#define gamma4  (Word16)27853

/*--------------------------------------------------------*

*         Static memory allocation.                      *

*--------------------------------------------------------*/

/* Excitation vector */

static Word16 old_exc[L_frame+pit_max+L_inter];

static Word16 *exc;

/* Spectral expansion factors */

static Word16 F_gamma3[p];

static Word16 F_gamma4[p];

/* Lsp (Line spectral pairs in the cosine domain) */

static Word16 lspold[p]={

30000, 26000, 21000, 15000, 8000, 0,

-8000,-15000,-21000,

-26000};

static Word16 lspnew[p];

/* Initial lsp values used after each time */

/* a reset is executed */

static Word16 lspold_init[p]={

30000, 26000, 21000, 15000, 8000, 0,

-8000,-15000,-21000,-26000};

/* Filter's memory */

static Word16 mem_syn[p];

/* Default parameters */

static Word16 old_parm[parm_size], old_T0;

/* Global definition */

Word16 last_ener_cod;

Word16 last_ener_pit;

/**************************************************************************

*

*	ROUTINE				:	Init_Decod_Tetra

*

*	DESCRIPTION			:	Initialization of variables for the speech decoder

*

**************************************************************************

*

*	USAGE				:	Init_Decod_Tetra()

*

*	INPUT ARGUMENT(S)		:	None

*

*	OUTPUT ARGUMENT(S)		:	None

*

*	RETURNED VALUE		:	None

*

**************************************************************************/

void Init_Decod_Tetra(void)

{

Word16 i;

old_T0 = 60;

for(i=0; i<23; i++)

old_parm[i] = 0;

/* Initialize static pointer */

exc    = old_exc + pit_max + L_inter;

/* Initialize global variables */

last_ener_cod = 0;

last_ener_pit = 0;

/* Static vectors to zero */

for(i=0; i<pit_max + L_inter; i++)

old_exc[i] = 0;

for(i=0; i<p; i++)

mem_syn[i] = 0;

/* Initialisation of lsp values for first */

/* frame lsp interpolation */

for(i=0; i<p; i++)

lspold[i] = lspold_init[i];

/* Compute LPC spectral expansion factors */

Fac_Pond(gamma3, F_gamma3);

Fac_Pond(gamma4, F_gamma4);

return;

}

/**************************************************************************

*

*	ROUTINE				:	Decod_Tetra

*

*	DESCRIPTION			:	Main speech decoder function

*

**************************************************************************

*

*	USAGE				:	Decod_Tetra(parm,synth)

*							(Routine_Name(input1,output1))

*

*	INPUT ARGUMENT(S)		:

*

*		INPUT1			:	- Description : Synthesis parameters

*							- Format : 24 * 16 bit-samples

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : Synthesis

*							- Format : 240 * 16 bit-samples

*

*	RETURNED VALUE		:	None

*

**************************************************************************/

void Decod_Tetra(Word16 parm[], Word16 synth[])

{

/* LPC coefficients */

Word16 A_t[(pp1)*4];		/* A(z) unquantized for the 4 subframes */

Word16 Ap3[pp1];		/* A(z) with spectral expansion         */

Word16 Ap4[pp1];		/* A(z) with spectral expansion         */

Word16 *A;			/* Pointer on A_t                       */

/* Other vectors */

Word16 zero_F[L_subfr+64],  *F;

Word16 code[L_subfr+4];

/* Scalars */

Word16 i, i_subfr;

Word16 T0, T0_min, T0_max, T0_frac;

Word16 gain_pit, gain_code, index;

Word16 sign_code, shift_code;

Word16 bfi, temp;

Word32 L_temp;

/* Initialization of F */

F  = &zero_F[64];

for(i=0; i<64; i++)

zero_F[i] = 0;

/* Test bfi */

bfi = *parm++;

if(bfi == 0)

{

D_Lsp334(&parm[0], lspnew, lspold);	/* lsp decoding   */

for(i=0; i< parm_size; i++)		/* keep parm[] as old_parm */

old_parm[i] = parm[i];

}

else

{

for(i=1; i<p; i++)

lspnew[i] = lspold[i];

for(i=0; i< parm_size; i++)		/* use old parm[] */

parm[i] = old_parm[i];

}

parm += 3;			/* Advance synthesis parameters pointer */

/* Interpolation of LPC for the 4 subframes */

Int_Lpc4(lspold,   lspnew,   A_t);

/* update the LSPs for the next frame */

for(i=0; i<p; i++)

lspold[i]   = lspnew[i];

/*------------------------------------------------------------------------*

*          Loop for every subframe in the analysis frame                 *

*------------------------------------------------------------------------*

* The subframe size is L_subfr and the loop is repeated L_frame/L_subfr  *

*  times                                                                 *

*     - decode the pitch delay                                           *

*     - decode algebraic code                                            *

*     - decode pitch and codebook gains                                  *

*     - find the excitation and compute synthesis speech                 *

*------------------------------------------------------------------------*/

A = A_t;				/* pointer to interpolated LPC parameters */

for (i_subfr = 0; i_subfr < L_frame; i_subfr += L_subfr)

{

index = *parm++;				/* pitch index */

if (i_subfr == 0)				/* if first subframe */

{

if (bfi == 0)

{						/* if bfi == 0 decode pitch */

if (index < 197)

{

/* T0 = (index+2)/3 + 19; T0_frac = index - T0*3 + 58; */

i = add(index, (Word16)2);

i = mult(i, (Word16)10923);	/* 10923 = 1/3 in Q15 */

T0 = add(i, (Word16)19);

i = add(T0, add(T0, T0) );	/* T0*3 */

i = sub((Word16)58, (Word16)i);

T0_frac = add(index, (Word16)i);

}

else

{

T0 = sub(index, (Word16)112);

T0_frac = 0;

}

}

else   /* bfi == 1 */

{

T0 = old_T0;

T0_frac = 0;

}

/* find T0_min and T0_max for other subframes */

T0_min = sub(T0, (Word16)5);

if (T0_min < pit_min) T0_min = pit_min;

T0_max = add(T0_min, (Word16)9);

if (T0_max > pit_max)

{

T0_max = pit_max;

T0_min = sub(T0_max, (Word16)9);

}

}

else  /* other subframes */

{

if (bfi == 0)				/* if bfi == 0 decode pitch */

{

/* T0 = (index+2)/3 - 1 + T0_min; */

i = add(index, (Word16)2);

i = mult(i, (Word16)10923);	/* 10923 = 1/3 in Q15 */

i = sub(i, (Word16)1);

T0 = add(T0_min, i);

/* T0_frac = index - 2 - i*3; */

i = add(i, add(i,i) );		/* i*3 */

T0_frac = sub( index , add(i, (Word16)2) );

}

}

/*-------------------------------------------------*

* - Find the adaptive codebook vector.            *

*-------------------------------------------------*/

Pred_Lt(&exc[i_subfr], T0, T0_frac, L_subfr);

/*-----------------------------------------------------*

* - Compute noise filter F[].                         *

* - Decode codebook sign and index.                   *

* - Find the algebraic codeword.                      *

*-----------------------------------------------------*/

Pond_Ai(A, F_gamma3, Ap3);

Pond_Ai(A, F_gamma4, Ap4);

for (i = 0;   i <= p;      i++) F[i] = Ap3[i];

for (i = pp1; i < L_subfr; i++) F[i] = 0;

Syn_Filt(Ap4, F, F, L_subfr, &F[pp1], (Word16)0);

/* Introduce pitch contribution with fixed gain of 0.8 to F[] */

for (i = T0; i < L_subfr; i++)

{

temp = mult(F[i-T0], (Word16)26216);

F[i] = add(F[i], temp);

}

index = *parm++;

sign_code  = *parm++;

shift_code = *parm++;

D_D4i60(index, sign_code, shift_code, F, code);

/*-------------------------------------------------*

* - Decode pitch and codebook gains.              *

*-------------------------------------------------*/

index = *parm++;        /* index of energy VQ */

Dec_Ener(index,bfi,A,&exc[i_subfr],code, L_subfr, &gain_pit, &gain_code);

/*-------------------------------------------------------*

* - Find the total excitation.                          *

* - Find synthesis speech corresponding to exc[].       *

*-------------------------------------------------------*/

for (i = 0; i < L_subfr;  i++)

{

/* exc[i] = gain_pit*exc[i] + gain_code*code[i]; */

/* exc[i]  in Q0   gain_pit in Q12               */

/* code[i] in Q12  gain_cod in Q0                */

L_temp = L_mult0(exc[i+i_subfr], gain_pit);

L_temp = L_mac0(L_temp, code[i], gain_code);

exc[i+i_subfr] = L_shr_r(L_temp, (Word16)12);

}

Syn_Filt(A, &exc[i_subfr], &synth[i_subfr], L_subfr, mem_syn, (Word16)1);

A  += pp1;    /* interpolated LPC parameters for next subframe */

}

/*--------------------------------------------------*

* Update signal for next frame.                    *

* -> shift to the left by L_frame  exc[]           *

*--------------------------------------------------*/

for(i=0; i<pit_max+L_inter; i++)

old_exc[i] = old_exc[i+L_frame];

old_T0 = T0;

return;

}

## A.3	Library of Source Coder/Decoder Subroutines

/************************************************************************

*

*	FILENAME		:	sub_sc_d.c

*

*	DESCRIPTION		:	Source coder/decoder sub-routines used in the

*					TETRA speech codec

*

************************************************************************

*

*	SUB-ROUTINES	:	- Bits2prm_Tetra()

*					- Cal_Rr2()

*					- Clsp_334()

*					- Dec_Ener()

*					- D4i60_16()

*					- D_D4i60()

*					- D_Lsp334()

*					- Ener_Qua()

*					- G_Code()

*					- G_Pitch()

*					- Inter8_M1_3()

*					- Inter8_1_3()

*					- Inter32_M1_3()

*					- Inter32_1_3()

*					- Lag_Max()

*					- Norm_Corr()

*					- Pitch_Fr()

*					- Pitch_Ol_Dec()

*					- Post_Process()

*					- Pred_Lt()

*					- Pre_Process() (including Init_Pre_Process)

*					- Prm2bits_Tetra()

*

************************************************************************

*

*	INCLUDED FILES	:	source.h

*

*					clsp_334.tab

*					ener_qua.tab

*

************************************************************************/

#include "source.h"

#include "clsp_334.tab"

#include "ener_qua.tab"

/**************************************************************************

*

*	ROUTINE				:	Bits2prm_Tetra

*

*	DESCRIPTION			:	Convert serial received bits to the encoder

*							parameter vector

*

**************************************************************************

*

*	USAGE				:	Bits2prm_Tetra(buffer_in,buffer_out)

*							(Routine_Name(input1,output1))

*

*	INPUT ARGUMENT(S)		:

*

*		INPUT1			:	- Description : Serial bits (137 + bfi)

*							- Format : Word16 - .. * 16 bit-samples

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : Encoded parameters

*								         (23 parameters + bfi)

*							- Format : Word16 - .. * 16 bit-samples

*

*	RETURNED VALUE		:	None

*

*	COMMENTS			:	The received parameters are :

*

*						- BFI	bad frame indicator		1 bit

*

*						- LSP	1st codebook 			8 bits

*							2nd codebook			9 bits

*							3nd codebook			9 bits

*

*						- for the 4 subframes :

*							pitch delay			8 bits (first)

*											5 bits (others)

*							codebook index		14 bits

*							pulse global sign		1 bit

*							pulse shift			1 bit

*							pitch and innovation gains	6 bits

*

**************************************************************************/

#define PRM_NO 23

void Bits2prm_Tetra(Word16 *bits, Word16 prm[])

{

Word16 i;

static Word16 bitno[PRM_NO] = {8, 9, 9,            /* split VQ LSP  */

8, 14, 1, 1, 6,     /* subframe 1    */

5, 14, 1, 1, 6,     /* subframe 2    */

5, 14, 1, 1, 6,     /* subframe 3    */

5, 14, 1, 1, 6};    /* subframe 4    */

*prm++ = *bits++;     /* read BFI */

for (i = 0; i < PRM_NO; i++)

{

prm[i] = bin2int(bitno[i], bits);

bits  += bitno[i];

}

}

/**************************************************************************

*

*	ROUTINE				:	Cal_Rr2

*

*	DESCRIPTION			:	Compute the autocorrelation matrix of

*						impulse response h

*							Only the even elements are stored in the matrix

*

**************************************************************************

*

*	USAGE				:	Cal_Rr2(buffer_1,buffer_2)

*							(Routine_Name(input1,input2))

*

*	INPUT ARGUMENT(S)		:

*

*		INPUT1			:	- Description : Impulse response

*						- Format : Word16

*

*		INPUT2			:	- Description : Autocorrelation matrix

*								          (passed as a vector)

*							- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:	None

*

*	RETURNED VALUE		:	None

*

*	COMMENTS			:	Matrice rr[dim_rr][dim_rr]

*

*							<------- dim_rr ------->

*							<------ L/2 ---->

*

*						^	x x x x x x ... x 0 ...  0	^

*						|	x x x x x x ... x 0 ...  0	|

*						|	x x x x x x ... x 0 ...  0	|

*						|	x x x x x x ... x 0 ...  0	|

*						L/2	x x x x x x ... x 0 ...  0	|

*						|	x x x x x x ... x 0 ...  0	dim_rr

*						|	| | | | | |       | |      |	|

*						v	x x x x x x ... x 0 ...  0	|

*							0 0 0 0 0 0 ... 0 0 ... 0	|

*							| | | | | |     | |     |		|

*							0 0 0 0 0 0 ... 0 0 ... 0	v

*

*						Firstly, rr[L/2-1][L/2-1] is computed, with

*						the main diagonal step up. Then, the same

*						is done for the other diagonals.

*

**************************************************************************/

/* Length of subframe = 60 */

#define L      60

/* Dimension of matrix rr[dim_rr][dim_rr]  = 32 */

#define dim_rr 32

/* Increment on diagonal */

#define diag   33

/* Index of last element to compute = rr[L/2-1][L/2-1] = rr[29*32+29] */

#define last   957

void Cal_Rr2(Word16 h[], Word16 *rr)

{

Word16 i, k, dec;

Word32 s;

Word16 *ph, *phd;

Word16 *rr_fin_c;

Word16 *rr_fin_r;

Word16 *rr_ij;

Word16 *rr_ji;

Word16 hs[L];

/* Scaling for maximum precision */

s = 0;

for(i=0; i<L; i++)

s = L_mac0(s, h[i], h[i]);

k = norm_l(s);

k = shr(sub(k, (Word16)1), (Word16)1);

for(i=0; i<L; i++)

hs[i] = shl(h[i], k);

/* Compute rr[][] */

rr_fin_r = &rr[last];                  /* pointer on rr[L/2-1][L/2-1]*/

rr_fin_c = rr_fin_r;

for (dec = 0 ;dec < L; dec+=2)

{

ph  = hs;                            /* pointer on hs[i]      */

phd = &hs[dec];                      /* pointer on hs[dec+i]  */

rr_ij = rr_fin_c;                    /* end of column j       */

rr_ji = rr_fin_r;                    /* end of row j          */

s = 0;

for (k = 0; k < L-dec; k+=2)

{

s = L_mac(s, *ph++, *phd++);

s = L_mac(s, *ph++, *phd++);

*rr_ij = extract_h(s);

*rr_ji = extract_h(s);

rr_ij -= diag;                     /* decrement pointers */

rr_ji -= diag;

}

rr_fin_c--;                          /* decrement column */

rr_fin_r -= dim_rr;                  /* decrement row    */

}

return;

}

/**************************************************************************

*

*	ROUTINE				:	Clsp_334

*

*	DESCRIPTION			:	Split vector quantization of LSP parameters

*							Use a split-3 VQ in the cosine lsp domain (-1,1),

*							without weighting

*

**************************************************************************

*

*	USAGE				:	Clsp_334(buffer_in,buffer_out1,buffer_out2)

*							(Routine_Name(input1,output1,output2))

*

*	INPUT ARGUMENT(S)		:

*

*		INPUT1			:	- Description : LSPs in the cosine domain (-1,1)

*						- Format : Word16 - Q15

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : Quantized LSPs in the cosine domain

*						- Format : Word16 - Q15

*

*		OUTPUT2			:	- Description : Indices of the 3 selected

*								          codebook entries

*							- Format : Word16

*

*	RETURNED VALUE		:	None

*

*

**************************************************************************/

void Clsp_334(Word16 *lsp, Word16 *lsp_q, Word16 *indice)

{

Word16 i, j, ind, temp;

Word16 *p_dico;

Word32 min, dist;

static Word16 lsp_old[10]={

30000, 26000, 21000, 15000, 8000, 0,

-8000,-15000,-21000,-26000};

/* Search dico1  lsp[0]-lsp[2] */

p_dico = dico1_clsp;

min    = MAX_32;

for (i = 0; i< taille_dic1; i++)

{

temp  = sub(lsp[0], *p_dico++);

dist = L_mult0(temp,temp);

for(j=1; j<3; j++)

{

temp  = sub(lsp[j], *p_dico++);

dist  = L_mac0(dist,temp,temp);

}

if (L_sub(dist, min) < 0) { min = dist; ind = i; }

}

indice[0] = ind;

p_dico    = &dico1_clsp[ind * 3];

lsp_q[0]  = *p_dico++ ;

lsp_q[1]  = *p_dico++ ;

lsp_q[2]  = *p_dico++ ;

/* Search dico2 lsp[3]-lsp[5] */

p_dico = dico2_clsp;

min    = MAX_32;

for (i = 0; i< taille_dic2; i++)

{

temp  = sub(lsp[3], *p_dico++);

dist  = L_mult0(temp,temp);

for(j=4; j<6; j++)

{

temp  = sub(lsp[j], *p_dico++);

dist  = L_mac0(dist,temp,temp);

}

if (L_sub(dist, min) < 0) { min = dist; ind = i; }

}

indice[1] = ind;

p_dico    = &dico2_clsp[ind * 3];

lsp_q[3]  = *p_dico++ ;

lsp_q[4]  = *p_dico++ ;

lsp_q[5]  = *p_dico++ ;

/* Search dico3 lsp[6]-lsp[9] */

p_dico = dico3_clsp;

min    = MAX_32;

for (i = 0; i< taille_dic3; i++)

{

temp  = sub(lsp[6], *p_dico++);

dist  = L_mult0(temp,temp);

for(j=7; j<10; j++)

{

temp  = sub(lsp[j], *p_dico++);

dist  = L_mac0(dist,temp,temp);

}

if (L_sub(dist, min) < 0) { min = dist; ind = i; }

}

indice[2] = ind;

p_dico    = &dico3_clsp[ind * 4];

lsp_q[6]  = *p_dico++ ;

lsp_q[7]  = *p_dico++ ;

lsp_q[8]  = *p_dico++ ;

lsp_q[9]  = *p_dico++ ;

/* Minimum distance between lsp_q[2] and lsp_q[3] */

temp = 917;                    /* 917 = 0.028 in Q15 = 50hz around 1000hz */

temp = sub(temp, lsp_q[2]);

temp = add(temp, lsp_q[3]);    /* temp = 0.028 - (lsp_q[2]-lsp_q[3])      */

if (temp > 0)

{

temp = shr(temp, (Word16)1);

lsp_q[2] = add(lsp_q[2], temp);

lsp_q[3] = sub(lsp_q[3], temp);

}

/* Minimum distance between lsp_q[5] and lsp_q[6] */

temp = 1245;                   /* 1245= 0.038 in Q15 = 50hz around 1600hz */

temp = sub(temp, lsp_q[5]);

temp = add(temp, lsp_q[6]);    /* temp = 0.038 - (lsp_q[5]-lsp_q[6])      */

if (temp > 0)

{

temp = shr(temp, (Word16)1);

lsp_q[5] = add(lsp_q[5], temp);

lsp_q[6] = sub(lsp_q[6], temp);

}

/* Verify if lsp_q[] are still in order */

temp = 0;

for(i=0; i<9; i++)

{

if(sub(lsp_q[i],lsp_q[i+1]) <= 0 )

{

temp = 1;

}

}

/* If lsp_q[] are not in order keep old lsp_q[] */

if(temp != 0)

{

for(i=0; i<10; i++)

lsp_q[i] = lsp_old[i];

}

else

{

for(i=0; i<10; i++)

lsp_old[i] = lsp_q[i];

}

return;

}

/**************************************************************************

*

*	ROUTINE				:	Dec_Ener

*

*	DESCRIPTION			:	Decode gains of pitch and innovative codebook

*

**************************************************************************

*

*	USAGE				:	Dec_Ener(index,bfi,buffer_in1,buffer_in2,buffer_in3,

*							L_subfr,gain_p,gain_c)

*							(Routine_Name(input1,input2,input3,input4,input5,

*							input6,output1,output2))

*

*	INPUT ARGUMENT(S)		:

*

*	INPUT1			:	- Description : Index of energy quantizer

*						- Format : Word16

*

*	INPUT2			:	- Description : Bad frame indicator

*						- Format : Word16

*

*	INPUT3			:	- Description : LPC filter

*						- Format : Word16

*

*	INPUT4			:	- Description : Adaptive codebook

*						- Format : Word16

*

*		INPUT5			:	- Description : Innovation codebook

*							- Format : Word16

*

*		INPUT6			:	- Description : Subframe length

*							- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : Quantized pitch gain

*							- Format : Word16

*

*		OUTPUT2			:	- Description : Quantized code gain

*							- Format : Word16

*

*	RETURNED VALUE		:	Index of energy quantizer

*

**************************************************************************/

extern Word16 last_ener_pit;

extern Word16 last_ener_cod;

Word16 Dec_Ener(Word16 index, Word16 bfi, Word16 A[], Word16 prd_lt[],

Word16 code[], Word16 L_subfr, Word16 *gain_pit, Word16 *gain_cod)

{

Word16  i, j;

Word16  exp, frac;

Word16  exp_lpc, ener_lpc;

Word16  exp_plt, ener_plt, pred_pit;

Word16  ener_c, pred_cod;

Word32  L_tmp;

/*------------------------------------------------------*

* Energy of impulse response of 1/A(z) for length = 60 *

*------------------------------------------------------*/

L_tmp = Lpc_Gain(A);

exp_lpc  = norm_l(L_tmp);

ener_lpc = extract_h( L_shl(L_tmp, exp_lpc) );

/*-------------------------------*

* Energy of adaptive codebook   *

*-------------------------------*/

L_tmp = 1;				/* Avoid case of all-zeros */

for(i=0; i<L_subfr; i++)

L_tmp = L_mac0(L_tmp, prd_lt[i], prd_lt[i]);

exp_plt  = norm_l(L_tmp);

ener_plt = extract_h( L_shl(L_tmp, exp_plt) );

/* ener_plt = Log2(ener_plt * ener_lpc) */

L_tmp = L_mult0(ener_plt, ener_lpc);

exp_plt = add(exp_plt, exp_lpc);

Log2(L_tmp, &exp, &frac);

L_tmp = Load_sh16(exp);

L_tmp = add_sh(L_tmp, frac, (Word16)1);	/* Log2(ener_plt*ener_lpc) in Q16*/

L_tmp = sub_sh16(L_tmp, exp_plt);		/* subtract exponant of ener_plt */

/* Input on 15 bits */

L_tmp = add_sh(L_tmp, (Word16)1710, (Word16)8); /* +6.68 in Q16

for other scaling    */

L_tmp = L_shr(L_tmp, (Word16)8);		/* Convert from Q16 to Q8        */

ener_plt = extract_l(L_tmp);

/*-------------------------------*

* Energy coming from code       *

*-------------------------------*/

L_tmp = 0;

for(i=0; i<L_subfr; i++)

L_tmp = L_mac0(L_tmp, code[i], code[i]);

ener_c = extract_h(L_tmp);

/* ener_c = Log2(ener_c * ener_lpc) */

L_tmp = L_mult0(ener_c, ener_lpc);

Log2(L_tmp, &exp, &frac);

L_tmp = Load_sh16(exp);

L_tmp = add_sh(L_tmp, frac, (Word16)1);	/* Log2(ener_plt*ener_lpc) in Q16*/

L_tmp = sub_sh16(L_tmp, exp_lpc);		/* subtract exponant of ener_lpc */

/* Input on 15 bits */

L_tmp = sub_sh(L_tmp, (Word16)4434, (Word16)8); /*-17.32 in Q16

for other scaling    */

L_tmp = L_shr(L_tmp, (Word16)8);		/* Convert from Q16 to Q8        */

ener_c = extract_l(L_tmp);

/*-----------------------------------------------*

* Test for bfi.                                 *

*                                               *

*  if (bfi != 0)                                *

*    ->last_ener_pit -= 0.5                     *

*    ->last_ener_cod -= 0.5                     *

*  else                                         *

*  {                                            *

*    decode new last_ener_pit et last_ener_cod  *

*  }                                            *

*-----------------------------------------------*/

if(bfi != 0)

{

last_ener_pit = sub(last_ener_pit, (Word16)128);	/* -0.5 in Q8 */

if(last_ener_pit < 0) last_ener_pit = 0;

last_ener_cod = sub(last_ener_cod, (Word16)128);	/* -0.5 in Q8 */

if(last_ener_cod < 0) last_ener_cod = 0;

}

else

{

/*-----------------------------------------------------------------*

* Prediction on pitch energy.                                     *

*  pred_pit = 0.50 * last_ener_pit + 0.25 * last_ener_cod - 3.0   *

*  if(pred_pit < 0.0) pred_pit = 0.0;                             *

*-----------------------------------------------------------------*/

L_tmp = Load_sh(last_ener_pit, (Word16)8);	/* .5 last_ener_pit in Q9  */

L_tmp = add_sh(L_tmp,last_ener_cod, (Word16)7); /*+.25 last_ener_code

in Q9    */

L_tmp = sub_sh(L_tmp, (Word16)768, (Word16)9); /* -3.0 in Q9          */

if(L_tmp < 0) L_tmp = 0;

pred_pit = store_hi(L_tmp, (Word16)7);	/* result in Q8 		   */

/*-----------------------------------------------------------------*

* Prediction on code energy.                                      *

*  pred_cod = 0.50 * last_ener_cod + 0.25 * last_ener_pit - 3.0   *

*  if(pred_cod < 0.0) pred_cod = 0.0;                             *

*-----------------------------------------------------------------*/

L_tmp = Load_sh(last_ener_cod, (Word16)8);	/* .5 last_ener_cod in Q9  */

L_tmp = add_sh(L_tmp, last_ener_pit, (Word16)7); /*+.25 last_ener_pit

in Q9    */

L_tmp = sub_sh(L_tmp, (Word16)768, (Word16)9); /* -3.0 in Q9          */

if(L_tmp < 0) L_tmp = 0;

pred_cod = store_hi(L_tmp, (Word16)7);	/* result in Q8		   */

/*-----------------------------------------------------------------*

* Read quantized values.                                          *

*-----------------------------------------------------------------*/

j = shl(index, (Word16)1);

last_ener_pit = add(t_qua_ener[j],   pred_pit);

last_ener_cod = add(t_qua_ener[j+1], pred_cod);

/* Limit energies ->for transmission errors */

if(sub(last_ener_pit, (Word16)6912)>0)last_ener_pit = 6912;

/* 6912 = 27 in Q8 */

if(sub(last_ener_cod, (Word16)6400)>0)last_ener_cod = 6400;

/* 6400 = 25 in Q8 */

}

/*---------------------------------------------------*

*                                                   *

*  Compute the quantized pitch gain.                *

*                                                   *

*     temp = 0.5 * ( last_ener_pit - ener_plt );    *

*     temp = pow(2.0, temp);                        *

*     if( temp > 1.2) temp = 1.2;                   *

*     *gain_pit = temp;                             *

*                                                   *

*---------------------------------------------------*/

L_tmp = Load_sh(last_ener_pit, (Word16)6);	 /* last_ener_pit/2 in Q15 */

L_tmp = sub_sh(L_tmp, ener_plt, (Word16)6);	 /* - ener_plt/2    in Q15 */

L_tmp = add_sh(L_tmp, (Word16)12, (Word16)15); /* to have gain in Q12    */

L_extract(L_tmp, &exp, &frac);

L_tmp = pow2(exp, frac);

if( L_sub(L_tmp, (Word16)4915) > 0) L_tmp = 4915; /* 4915 = 1.2 in Q12   */

*gain_pit = extract_l(L_tmp);

/*---------------------------------------------------*

*                                                   *

*  Compute the innovative code gain.                *

*                                                   *

*     temp = 0.5 * ( last_ener_cod - ener_c );      *

*     *gain_cod = pow(2.0, temp);                   *

*                                                   *

*---------------------------------------------------*/

L_tmp = Load_sh(last_ener_cod, (Word16)6);	/* last_ener_cod/2 in Q15 */

L_tmp = sub_sh(L_tmp, ener_c, (Word16)6);	/* - ener_c/2      in Q15 */

L_extract(L_tmp, &exp, &frac);

L_tmp = pow2(exp, frac);

*gain_cod = extract_l(L_tmp);

return index;

}

/**************************************************************************

*

*	ROUTINE				:	D4i60_16

*

*	DESCRIPTION			:	Innovation codebook search

*

**************************************************************************

*

*	USAGE				:	D4i60_16(buffer_in1,buffer_in2,buffer_in3,buffer_in4,

*							buffer_out1, buffer_out2,sign,shift)

*							(Routine_Name(input1,input2,input3,input4,

*							output1,output2,output3,output4))

*

*	INPUT ARGUMENT(S)		:

*

*	INPUT1			:	- Description : Result of backward-filtering

*						- Format : Word16

*

*	INPUT2			:	- Description : Impulse response of noise filter

*							          (vector f[])

*						- Format : Word16 - (must be preceeded by 64 zeros)

*

*	INPUT3			:	- Description : Total impulse response (vector h[])

*						- Format : Word16 - (must be preceeded by 64 zeros)

*

*	INPUT4			:	- Description : Autocorrelations of h[]

*						- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : Innovation code found

*							  by convolving with f[]

*							- Format : Word16

*

*		OUTPUT2			:	- Description : Innovation code found

*							  by convolving with h[]

*							- Format : Word16

*

*		OUTPUT3			:	- Description : Innovation code gain sign

*							- Format : Word16

*

*		OUTPUT4			:	- Description : Shift of innovation code

*							- Format : Word16

*

*	RETURNED VALUE		:	Index of innovation code

*

**************************************************************************

**************************************************************************

*

*	COMMENTS			:	The code length is 60, containing 4 nonzero pulses

*

*						i0, i1, i2, i3, with fixed signs.

*						i0 can take 30 positions.

*						i1, i2 and i3 can have 8 possible positions :

*

*						i0 (+)	: 0,  2, 4.  .... 58

*						i1 (-)	: 2, 10, 18, 26, 34, 42, 50, 58

*						i2 (+)	: 4, 12, 20, 28, 36, 44, 52, (60)

*						i3 (-)	: 6, 14, 22, 30, 38, 46, 54, (62)

*

*						Positions 60 and 62 correspond, respectively,

*						to pulses i2 and i3 not used.

*						The code can be shifted 1 position to the right.

*

*	NOTES ON COMPLEXITY		:

*

*	1 :	In this routine all accesses to matrix rr[][] use indices. This was done for clarity and

*		readability of the program. In the actual real-time implementation all these

*		accesses use pointers.  Parallel autoincrement of pointers on dn[] and rr[][]

*		should be used, especially for the two most inner loops in the search which are very

*		critical.

*

*		As the number of addressing registers and the possible modes of autoincrement

*		can vary with each DSP, the addressing of matrix rr[][] should be optimized

*		for the DSP used.

*

*	2 :	Because of the thresholds it is difficult to estimate the average complexity.

*		Usually, we mesure it with the real-time implementation. The worst

*		case corresponds to "time <= 0".  The decrements of 3 and 4 on variable

*		"time" come from the relative number of operations when "seuil2" is exceeded and

*		the number of operations when "seuil1" is exceeded, excluding the operations when

*		"seuil2" is exceeded. We supose that a new maximum is found 1 out of 8 new tries.

*		The numbers 3 and 4 correspond to the real-time implementation and not to this

*		C code.

*

**************************************************************************/

#define lcode 60

/* Gain for impulse 0 (i0) = sqrt(2) = 1.4142                             */

/* We use different values to be compatible with the implementation       */

/* which uses 3 different Qx formats for this number to simplify scaling. */

#define Q11_gain_i0  2896

#define Q13_gain_i0 11585

#define Q14_gain_i0 23170

/*------------------------------------------------------------------*

* The next 3 parameters control the time taken by the innovative   *

* codebook search.                                                 *

* The first two parameters control if a section of the innovative  *

* codebook should be search or not. The last parameter controls    *

* the absolute maximum time used by the innovative codebook search.*

*------------------------------------------------------------------*/

/* thresholds = 0.586 in Q15 */

#define threshold1 19200

#define threshold2 19200

#define max_time   350

Word16 D4i60_16(Word16 dn[], Word16 f[], Word16 h[], Word16 rr[][32],

Word16 cod[], Word16 y[], Word16 *sign, Word16 *shift_code)

{

Word16 ip0, ip1, ip2, ip3;

Word16 ii0, ii1, ii2, ii3;

Word16 *p0, *p1, *p2, *p3;

Word16 i,j;

Word16 shif, shift;

Word16 time, index;

Word16 ps0,  ps1,  ps2,  ps3;

Word16 ps0a, ps1a, ps2a;

Word16 ps, ps3c, psc;

Word16 alpha, alp1, alp2, alp3;

Word16 seuil1,seuil2;

Word16 max0, max1, max2, min0, min1, min2;

Word32 L_tmp;

Word32 alp0_32, alp1_32, alp2_32, ps2_32;

/* set dn[60] - dn[63] to zero */

dn[60]=dn[61]=dn[62]=dn[63] = 0;

/* Find min. and max. of dn[] for positions of i0, i1, i2 */

min0=max0=0;

for(i = 0; i<60; i += 2)

{

j = add(dn[i], dn[i+1]);

if      (sub(j,max0) > 0) max0 = j;

else if (sub(j,min0) < 0) min0 = j;

}

max0 = shr(max0, (Word16)1);

min0 = shr(min0, (Word16)1);

/* Multiply max0 and min0 by gain_i0 in Q11 */

max0 = store_hi(L_mult0(max0, (Word16)Q11_gain_i0), (Word16)5);

min0 = store_hi(L_mult0(min0, (Word16)Q11_gain_i0), (Word16)5);

min1=max1=0;

for(i = 2; i<60; i += 8)

{

j = add(dn[i], dn[i+1]);

if      (sub(j,max1) > 0) max1 = j;

else if (sub(j,min1) < 0) min1 = j;

}

max1 = shr(max1, (Word16)1);

min1 = shr(min1, (Word16)1);

min2=max2=0;

for(i = 4; i<60; i += 8)

{

j = add(dn[i], dn[i+1]);

if      (sub(j,max2) > 0) max2 = j;

else if (sub(j,min2) < 0) min2 = j;

}

max2 = shr(max2, (Word16)1);

min2 = shr(min2, (Word16)1);

/*----------------------------------------------------------*

* Find absolute maximum for combination dn[i0] and dn[i1]  *

*      and for combination dn[i0], dn[i1] and dn[i2]       *

*   max1 = max ( max0-min1 , max1-min0 )                   *

*   max2 = max ( max0-min1+max2 , max1-min0-min2 )         *

*----------------------------------------------------------*/

max0 = sub(max0, min1);

max2 = add(max0, max2);

max1 = sub(max1, min0);

j    = sub(max1, min2);

if(sub(max0,max1) > 0) max1 = max0;

if(sub(j,max2)    > 0) max2 = j;

/* Set thresholds */

seuil1 = mult(max1, (Word16)threshold1);

seuil2 = mult(max2, (Word16)threshold2);

/* Default values */

ip0    = 0;

ip1    = 2;

ip2    = 4;

ip3    = 6;

shift  = 0;

ps     = 0;

psc    = 0;

alpha  = 255;

time   = max_time;

/* Four loops to search innovation code. */

for (ii0=0; ii0<30; ii0++)

{

ps0  = store_hi(L_mult0((Word16)Q11_gain_i0, dn[2*ii0]), (Word16)5);

ps0a = store_hi(L_mult0((Word16)Q11_gain_i0, dn[2*ii0+1]), (Word16)5);

alp0_32 = Load_sh(rr[ii0][ii0], (Word16)14);

for (ii1=1; ii1<30; ii1+=4)

{

ps1  = sub(ps0, dn[2*ii1]);

ps1a = sub(ps0a, dn[2*ii1+1]);

/* alp1 = alp0*2 + rr[ii1][ii1] - 2.0*gain_i0*rr[ii0][ii1]  */

/* The result is divided by 8 to avoid overflow.            */

L_tmp = add_sh(alp0_32, rr[ii1][ii1], (Word16)13);

L_tmp = L_msu0(L_tmp, (Word16)Q14_gain_i0, rr[ii0][ii1]);

alp1  = extract_h(L_tmp);

alp1_32 = Load_sh(alp1, (Word16)15);

/* If( abs( (ps1+ps1a)/2 ) > seuil1 */

L_tmp = Load_sh(ps1, (Word16)15);

L_tmp = L_abs( add_sh(L_tmp, ps1a, (Word16)15) );

L_tmp = sub_sh16( L_tmp, seuil1);

if( L_tmp > 0)

{

for (ii2=2; ii2<31; ii2+=4)

{

ps2  = add(ps1, dn[2*ii2]);

ps2a = add(ps1a, dn[2*ii2+1]);

/* alp2=alp1+rr[ii2][ii2]+2.0*(gain_i0*rr[ii0][ii2]-rr[ii1][ii2]) */

/* The result is divided by 16 to avoid overflow.                 */

L_tmp = add_sh(alp1_32, rr[ii2][ii2], (Word16)12);

L_tmp = L_mac0(L_tmp, (Word16)Q13_gain_i0, rr[ii0][ii2]);

L_tmp = sub_sh(L_tmp, rr[ii1][ii2] , (Word16)13);

alp2  = extract_h(L_tmp);

alp2_32 = Load_sh16(alp2);

/* If( abs( (ps2+ps2a)/2 ) > seuil2 */

L_tmp = Load_sh(ps2, (Word16)15);

L_tmp = L_abs( add_sh(L_tmp, ps2a, (Word16)15) );

L_tmp = sub_sh16( L_tmp, seuil2);

if( L_tmp > 0)

{

shif = 0;

if( sub(abs_s(ps2a),abs_s(ps2)) > 0)

{

ps2  = ps2a;

shif = 1;

}

ps2_32 = Load_sh(ps2, (Word16)15);

for (ii3=3; ii3<32; ii3+=4)

{

/* ps3 = (ps2-dn[2*ii3+shift]) / 2                       */

/* Need to work on 32 bits to avoid possible overflow */

L_tmp = sub_sh(ps2_32, dn[2*ii3+shif], (Word16)15);

ps3  = extract_h(L_tmp);

/* alp3 = alp2 + rr[ii3][ii3] +                               */

/*   2.0*(rr[ii1][ii3] - gain_i0*rr[ii0][ii3] - rr[ii2][ii3]) */

/*                                                            */

/* This part is the most critical part in all the coder       */

L_tmp = add_sh(alp2_32, rr[ii3][ii3], (Word16)12);

L_tmp = add_sh(L_tmp, rr[ii1][ii3], (Word16)13);

L_tmp = L_msu0(L_tmp, (Word16)Q13_gain_i0, rr[ii0][ii3] );

L_tmp = sub_sh(L_tmp, rr[ii2][ii3], (Word16)13);

alp3  = extract_h(L_tmp);

/* if( (ps3**2 * alpha) - (psc *alp3)  > 0 ) */

/*    ->new maximum                          */

ps3c = mult(ps3, ps3);

L_tmp  = L_mult(ps3c, alpha);

if( L_msu(L_tmp,psc,alp3) > 0)

{

ps    = ps3;

psc   = ps3c;

alpha = alp3;

ip0 = 2*ii0;

ip1 = 2*ii1;

ip2 = 2*ii2;

ip3 = 2*ii3;

shift = shif;

} /*  end of if(.. > 0) */

} /* end of for ii3 = */

time = sub(time, (Word16)3);	/* See note on complexity */

if(time <= 0 ) goto fin;		/* Maximum time finish    */

} /* end if >seuil2 */

} /* end of for ii2 = */

time = sub(time, (Word16)4);		/* See note on complexity */

if(time <= 0 ) goto fin;		/* Maximum time finish    */

} /* end if >seuil1 */

} /* end of for ii1 = */

} /* end of for ii0 = */

fin:

/* Convolve code with f[] */

f -= shift;            /* Operations on pointers !!! */

p0 = f - ip0;

p1 = f - ip1;

p2 = f - ip2;

p3 = f - ip3;

/* cod[i] = p0[i] * gain_i0  - p1[i] + p2[i] - p3[i]; */

if(ps >= 0)

{

*sign = 0;

for (i = 0; i < lcode; i++)

{

L_tmp  = L_mult0(p0[i], (Word16)Q11_gain_i0);

L_tmp  = sub_sh(L_tmp, p1[i], (Word16)11);

L_tmp  = add_sh(L_tmp, p2[i], (Word16)11);

L_tmp  = sub_sh(L_tmp, p3[i], (Word16)11);

cod[i] = store_hi(L_tmp, (Word16)5);

}

}

else

{

*sign = 1;

for (i = 0; i < lcode; i++)

{

L_tmp  = L_mult0(p0[i], (Word16)Q11_gain_i0);

L_tmp  = sub_sh(L_tmp, p1[i], (Word16)11);

L_tmp  = add_sh(L_tmp, p2[i], (Word16)11);

L_tmp  = sub_sh(L_tmp, p3[i], (Word16)11);

L_tmp  = L_negate(L_tmp);

cod[i] = store_hi(L_tmp, (Word16)5);

}

}

/* Convolve code with h[] */

h -= shift;            /* Operations on pointers !!! */

p0 = h - ip0;

p1 = h - ip1;

p2 = h - ip2;

p3 = h - ip3;

/* y[i] = p0[i] * gain_i0  - p1[i] + p2[i] - p3[i]; */

if(ps >= 0)

{

for (i = 0; i < lcode; i++)

{

L_tmp = L_mult0(p0[i], (Word16)Q11_gain_i0);

L_tmp = sub_sh(L_tmp, p1[i], (Word16)11);

L_tmp = add_sh(L_tmp, p2[i], (Word16)11);

L_tmp = sub_sh(L_tmp, p3[i], (Word16)11);

y[i]  = store_hi(L_tmp, (Word16)5);

}

}

else

{

for (i = 0; i < lcode; i++)

{

L_tmp = L_mult0(p0[i], (Word16)Q11_gain_i0);

L_tmp = sub_sh(L_tmp, p1[i], (Word16)11);

L_tmp = add_sh(L_tmp, p2[i], (Word16)11);

L_tmp = sub_sh(L_tmp, p3[i], (Word16)11);

L_tmp = L_negate(L_tmp);

y[i]  = store_hi(L_tmp, (Word16)5);

}

}

/* Return parameters */

*shift_code = shift;

/* index = (ip0>>1) + ( (ip1>>3)<<5 ) + ( (ip2>>3)<<8 ) + ( (ip3>>3)<<11 )*/

index = shr(ip0, (Word16)1);

index = add(index, shl( shr( ip1,(Word16)3 ),(Word16)5 ));

index = add(index, shl( shr( ip2,(Word16)3 ),(Word16)8 ));

index = add(index, shl( shr( ip3,(Word16)3 ),(Word16)11 ));

return index;

}

/**************************************************************************

*

*	ROUTINE				:	D_D4i60

*

*	DESCRIPTION			:	Decode innovative codebook d4i60_16

*

**************************************************************************

*

*	USAGE				:	D_D4i60(index,sign, shift,buffer_in,buffer_out)

*							(Routine_Name(input1,input2,input3,input4,output1))

*

*	INPUT ARGUMENT(S)		:

*

*	INPUT1			:	- Description : Index of codebook

*						- Format : Word16

*

*	INPUT2			:	- Description : Sign of codebook

*						- Format : Word16

*

*	INPUT3			:	- Description : Shift of codebook

*						- Format : Word16

*

*	INPUT4			:	- Description : Noise filter

*						- Format : Word16 - (must be preceeded by 64 zeros)

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : Innovative vector

*							- Format : Word16

*

*	RETURNED VALUE		:	None

*

*

**************************************************************************/

#define lcode 60

/* Gain for impulse 0 = sqrt(2) = 1.4142 = 2896 in Q11 */

#define Q11_gain_i0  2896

void D_D4i60(Word16 index, Word16 sign, Word16 shift, Word16 F[], Word16 cod[])

{

Word16 i, pos0, pos1, pos2, pos3;

Word16 *p0, *p1, *p2, *p3;

Word32 L_tmp;

/* Position of the 4 impulses */

pos0 = shl( (Word16)(index & 31), (Word16)1);

pos1 = shr( (Word16)(index & 224), (Word16)2);

pos1 = add(pos1, (Word16)2);

pos2 = shr( (Word16)(index & 1792), (Word16)5);

pos2 = add(pos2, (Word16)4);

pos3 = shr( (Word16)(index & 14336), (Word16)8);

pos3 = add(pos3, (Word16)6);

/* Convolve code with F[]                             */

/* cod[i] = p0[i] * gain_i0  - p1[i] + p2[i] - p3[i]; */

F -= shift;            /* Operations on pointers !!!  */

p0 = F - pos0;

p1 = F - pos1;

p2 = F - pos2;

p3 = F - pos3;

if(sign == 0)

{

for (i = 0; i < lcode; i++)

{

L_tmp  = L_mult0(p0[i], (Word16)Q11_gain_i0);

L_tmp  = sub_sh(L_tmp, p1[i], (Word16)11);

L_tmp  = add_sh(L_tmp, p2[i], (Word16)11);

L_tmp  = sub_sh(L_tmp, p3[i], (Word16)11);

cod[i] = store_hi(L_tmp, (Word16)5);

}

}

else

{

for (i = 0; i < lcode; i++)

{

L_tmp  = L_mult0(p0[i], (Word16)Q11_gain_i0);

L_tmp  = sub_sh(L_tmp, p1[i], (Word16)11);

L_tmp  = add_sh(L_tmp, p2[i], (Word16)11);

L_tmp  = sub_sh(L_tmp, p3[i], (Word16)11);

L_tmp  = L_negate(L_tmp);

cod[i] = store_hi(L_tmp, (Word16)5);

}

}

return;

}

/**************************************************************************

*

*	ROUTINE				:	D_Lsp334

*

*	DESCRIPTION			:	Decoding: Split vector quantization of

*							LSP parameters

*

**************************************************************************

*

*	USAGE				:	D_Lsp334(buffer_in1,buffer_out1,buffer_in2)

*							(Routine_Name(input1,output1,input2))

*

*	INPUT ARGUMENT(S)		:

*

*	INPUT1			:	- Description : indices of the three selected

*							          codebook entries

*						- Format : Word16

*

*		INPUT2			:	- Description : Previous LSP values

*								          (in the cosine domain)

*							- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : Quantized LSPs (in the cosine domain)

*							- Format : Word16

*

*	RETURNED VALUE		:	None

*

*

**************************************************************************/

void D_Lsp334(Word16 indice[], Word16 lsp[], Word16 lsp_old[])

{

Word16 i, *p, temp;

p      = &dico1_clsp[ indice[0] * 3];

lsp[0] = *p++ ;

lsp[1] = *p++ ;

lsp[2] = *p++ ;

p      = &dico2_clsp[ indice[1] * 3];

lsp[3] = *p++ ;

lsp[4] = *p++ ;

lsp[5] = *p++ ;

p      = &dico3_clsp[ indice[2] * 4];

lsp[6] = *p++ ;

lsp[7] = *p++ ;

lsp[8] = *p++ ;

lsp[9] = *p++ ;

/* Minimum distance between lsp[2] and lsp[3] */

temp = 917;			/* 917 = 0.028 in Q15 = 50hz around 1000hz */

temp = sub(temp, lsp[2]);

temp = add(temp, lsp[3]);	/* temp = 0.028 - (lsp[2]-lsp[3])      */

if (temp > 0)

{

temp = shr(temp, (Word16)1);

lsp[2] = add(lsp[2], temp);

lsp[3] = sub(lsp[3], temp);

}

/* Minimum distance between lsp[5] and lsp[6] */

temp = 1245;			/* 1245= 0.038 in Q15 = 50hz around 1600hz */

temp = sub(temp, lsp[5]);

temp = add(temp, lsp[6]);	/* temp = 0.038 - (lsp[5]-lsp[6])      */

if (temp > 0)

{

temp = shr( temp,(Word16)1 );

lsp[5] = add(lsp[5], temp);

lsp[6] = sub(lsp[6], temp);

}

/* Verify if lsp[] are still in order */

temp = 0;

for(i=0; i<9; i++)

{

if(sub(lsp[i],lsp[i+1]) <= 0 )

{

temp = 1;

}

}

/* If lsp[] are not in order keep old lsp[] */

if(temp != 0)

{

for(i=0; i<10; i++)

lsp[i] = lsp_old[i];

}

return;

}

/**************************************************************************

*

*	ROUTINE				:	Ener_Qua

*

*	DESCRIPTION			:	Compute quantized gains of pitch and innovative

*						 	codebook resulting from the quantization of energies

*							of adaptive and innovation codebook

*

**************************************************************************

*

*	USAGE				:	Ener_Qua(buffer_in1,buffer_in2,buffer_in3,L_subfr,

*							gain_p,gain_c)

*							(Routine_Name(input1,input2,input3,input4,

*							arg5,arg6))

*

*	INPUT ARGUMENT(S)		:

*

*	INPUT1			:	- Description : LPC filter

*						- Format : Word16

*

*	INPUT2			:	- Description : Adaptive codebook

*						- Format : Word16

*

*	INPUT3			:	- Description : Innovation codebook

*						- Format : Word16

*

*	INPUT4			:	- Description : Subframe length

*						- Format : Word16

*

*		ARG5				:	- Description : Pitch gain

*							- Format : Word16

*

*		ARG6				:	- Description : Code gain

*							- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:

*

*		ARG5				:	- Description : Quantized pitch gain

*							- Format : Word16

*

*		ARG6				:	- Description : Quantized code gain

*							- Format : Word16

*

*	RETURNED VALUE		:	Index of quantization

*

**************************************************************************/

extern Word16 last_ener_pit;

extern Word16 last_ener_cod;

Word16 Ener_Qua(Word16 A[], Word16 prd_lt[], Word16 code[], Word16 L_subfr,

Word16 *gain_pit, Word16 *gain_cod)

{

Word16  i, j, index, tmp, *p;

Word16  exp, frac;

Word16  exp_lpc, ener_lpc;

Word16  exp_plt, ener_plt, ener_pit, err_pit, pred_pit;

Word16  ener_c, ener_cod, err_cod, pred_cod;

Word32  L_tmp, dist, dist_min;

/*------------------------------------------------------*

* Energy of impulse response of 1/A(z) for length = 60 *

*------------------------------------------------------*/

L_tmp = Lpc_Gain(A);

exp_lpc  = norm_l(L_tmp);

ener_lpc = extract_h( L_shl(L_tmp, exp_lpc) );

/*-------------------------------*

* Energy coming from pitch      *

*-------------------------------*/

L_tmp = 1;            /* Avoid case of all-zeros */

for(i=0; i<L_subfr; i++)

L_tmp = L_mac0(L_tmp, prd_lt[i], prd_lt[i]);

exp_plt  = norm_l(L_tmp);

ener_plt = extract_h( L_shl(L_tmp, exp_plt) );

/* ener_plt = Log2(ener_plt * ener_lpc) */

L_tmp = L_mult0(ener_plt, ener_lpc);

exp_plt = add(exp_plt, exp_lpc);

Log2(L_tmp, &exp, &frac);

L_tmp = Load_sh16(exp);

L_tmp = add_sh(L_tmp, frac, (Word16)1);	/*Log2(ener_plt*ener_lpc) in Q16 */

L_tmp = sub_sh16(L_tmp, exp_plt);		/* subtract exponant of ener_plt */

/* Input on 15 bits */

L_tmp = add_sh(L_tmp, (Word16)1710, (Word16)8);	/* +6.68 in Q16

for other scaling    */

/* The scaling factor +6.68 includes many scalings, due to the fact that

impulse response of the LP filter is in Q10 (see routine Lpc_Gain), and

that the original quantization gain table was designed using -26 dB data

reduced to 13 bits while the final coder works with -22dB signal reduced

to 15 bits */

L_tmp = L_shr(L_tmp, (Word16)8);		/* Convert from Q16 to Q8        */

ener_plt = extract_l(L_tmp);

/* ener_pit = Log2(gain_pit**2) + ener_plt */

L_tmp = 1;					/* Avoid Log2(0) 			   */

L_tmp = L_mac0(L_tmp, *gain_pit, *gain_pit);

Log2(L_tmp, &exp, &frac);

L_tmp = Load_sh16(exp);

L_tmp = add_sh(L_tmp, frac, (Word16)1);	/* Log2(gain_pit**2)    in Q16   */

L_tmp = sub_sh16(L_tmp, (Word16)24);	/* -24.0 -> gain_pit**2 is in Q24*/

/* Pitch gain (variable "gain_pit")is in Q12, so its square is in Q24,

which means that the real value is multplied by 2**24. Thus, in applying

the function Log2(), a value of 24 is being added. A scaling factor of

-24 is then needed to compensate  */

L_tmp = L_shr(L_tmp, (Word16)8);		/* Convert from Q16 to Q8        */

ener_pit = extract_l(L_tmp);

ener_pit = add(ener_pit, ener_plt);

/*-------------------------------*

* Energy coming from code       *

*-------------------------------*/

L_tmp = 0;

for(i=0; i<L_subfr; i++)

L_tmp = L_mac0(L_tmp, code[i], code[i]);

ener_c = extract_h(L_tmp);

/* ener_c = Log2(ener_c * ener_lpc) */

L_tmp = L_mult0(ener_c, ener_lpc);

Log2(L_tmp, &exp, &frac);

L_tmp = Load_sh16(exp);

L_tmp = add_sh(L_tmp, frac, (Word16)1);	/* Log2(ener_plt*ener_lpc) in Q16*/

L_tmp = sub_sh16(L_tmp, exp_lpc);		/* subtract exponant of ener_lpc */

/* Input on 15 bits */

L_tmp = sub_sh(L_tmp, (Word16)4434, (Word16)8);	/*-17.32 in Q16

for other scaling    */

/* The vector "code[i]" is in Q12, so its square (variable "ener_c") is in

Q24, which means that the real value is multplied by 2**24. Thus, in

applying the function Log2(), a value of 24 is being added. A scaling

factor of -24 is then needed to compensate, but as the other scaling

factor of +6.68 (as explained above) has also to be applied, the final

resulting factor is -17.32 */

L_tmp = L_shr(L_tmp, (Word16)8);		/* Convert from Q16 to Q8        */

ener_c = extract_l(L_tmp);

/* ener_cod = Log2(gain_code**2) + ener_c */

L_tmp = 1;					/* Avoid Log2(0) 			   */

L_tmp = L_mac0(L_tmp, *gain_cod, *gain_cod);

Log2(L_tmp, &exp, &frac);

L_tmp = Load_sh16(exp);

L_tmp = add_sh(L_tmp, frac, (Word16)1);	/* Log2(gain_code**2) in Q16	   */

L_tmp = L_shr(L_tmp, (Word16)8);		/* Convert from Q16 to Q8        */

ener_cod = extract_l(L_tmp);

ener_cod = add(ener_cod, ener_c);

/*-----------------------------------------------------------------*

* Prediction on pitch energy and prediction error                 *

*  pred_pit = 0.50 * last_ener_pit + 0.25 * last_ener_cod - 3.0   *

*  if(pred_pit < 0.0) pred_pit = 0.0;                             *

*  err_pit = ener_pit - pred_pit;                                 *

*-----------------------------------------------------------------*/

L_tmp = Load_sh(last_ener_pit, (Word16)8); /* .5 last_ener_pit in Q9     */

L_tmp = add_sh(L_tmp, last_ener_cod, (Word16)7); /*+.25 last_ener_code

in Q9    */

L_tmp = sub_sh(L_tmp, (Word16)768, (Word16)9); /* -3.0 in Q9             */

if(L_tmp < 0) L_tmp = 0;

pred_pit = store_hi(L_tmp, (Word16)7);	/* result in Q8 			   */

err_pit  = sub(ener_pit, pred_pit);

/*-----------------------------------------------------------------*

* Prediction on code energy and prediction error                  *

*  pred_cod = 0.50 * last_ener_cod + 0.25 * last_ener_pit - 3.0   *

*  if(pred_cod < 0.0) pred_cod = 0.0;                             *

*  err_cod = ener_cod - pred_cod;                                 *

*-----------------------------------------------------------------*/

L_tmp = Load_sh(last_ener_cod, (Word16)8); /* .5 last_ener_cod  in Q9    */

L_tmp = add_sh(L_tmp, last_ener_pit, (Word16)7); /*+.25 last_ener_pit

in Q9    */

L_tmp = sub_sh(L_tmp, (Word16)768, (Word16)9); /* -3.0 in Q9             */

if(L_tmp < 0) L_tmp = 0;

pred_cod = store_hi(L_tmp, (Word16)7);	/* result in Q8 			   */

err_cod  = sub(ener_cod, pred_cod);

/*-----------------------------------------------------------------*

* Codebook search.                                                *

* Find vector which minimizes:                                    *

*                                                                 *

*   Min k :  (err_pit - dico[0,k])**2 +  (err_cod - dico[1,k])**2 *

*                                                                 *

*-----------------------------------------------------------------*/

dist_min = MAX_32;

p = t_qua_ener;

for (i = 0; i< nb_qua_ener; i++)

{

tmp  = sub(*p++, err_pit);

dist = L_mult0(tmp, tmp);

tmp  = sub(*p++, err_cod);

dist = L_mac0(dist, tmp, tmp);

if (L_sub(dist, dist_min) < 0) { dist_min = dist; index = i; }

}

j = shl(index, (Word16)1);

last_ener_pit = add(t_qua_ener[j],   pred_pit);

last_ener_cod = add(t_qua_ener[j+1], pred_cod);

/* Limit energies ->for transmission errors */

if(sub(last_ener_pit, (Word16)6912) > 0) last_ener_pit = 6912;

/* 6912 = 27 in Q8 */

if(sub(last_ener_cod, (Word16)6400) > 0) last_ener_cod = 6400;

/* 6400 = 25 in Q8 */

/*---------------------------------------------------*

*                                                   *

*  Compute the quantized pitch gain.                *

*                                                   *

*     temp = 0.5 * ( last_ener_pit - ener_plt );    *

*     temp = pow(2.0, temp);                        *

*     if( temp > 1.2) temp = 1.2;                   *

*     *gain_pit = temp;                             *

*                                                   *

*---------------------------------------------------*/

L_tmp = Load_sh(last_ener_pit, (Word16)6);	/* last_ener_pit/2 in Q15 */

L_tmp = sub_sh(L_tmp, ener_plt, (Word16)6);	/* - ener_plt/2    in Q15 */

L_tmp = add_sh(L_tmp, (Word16)12, (Word16)15); /* to have gain in Q12   */

L_extract(L_tmp, &exp, &frac);

L_tmp = pow2(exp, frac);

if( L_sub(L_tmp, (Word32)4915) > 0) L_tmp = 4915; /* 4915 = 1.2 in Q12  */

*gain_pit = extract_l(L_tmp);

/*---------------------------------------------------*

*                                                   *

*  Compute the innovative code gain.                *

*                                                   *

*     temp = 0.5 * ( last_ener_cod - ener_c );      *

*     *gain_cod = pow(2.0, temp);                   *

*                                                   *

*---------------------------------------------------*/

L_tmp = Load_sh(last_ener_cod, (Word16)6);	/* last_ener_cod/2 in Q15 */

L_tmp = sub_sh(L_tmp, ener_c, (Word16)6);	/* - ener_c/2      in Q15 */

L_extract(L_tmp, &exp, &frac);

L_tmp = pow2(exp, frac);

*gain_cod = extract_l(L_tmp);

return index;

}

/**************************************************************************

*

*	ROUTINE				:	G_Code

*

*	DESCRIPTION			:	Compute the gain of innovative code

*

**************************************************************************

*

*	USAGE				:	G_Code(buffer_in1,buffer_in2,L_subfr)

*							(Routine_Name(input1,input2,input3))

*

*	INPUT ARGUMENT(S)		:

*

*	INPUT1			:	- Description : Code target

*						- Format : Word16

*

*		INPUT2			:	- Description : Filtered innovation code (with sign)

*							- Format : Word16

*

*	INPUT3			:	- Description : Length of subframe

*						- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:	None

*

*	RETURNED VALUE		:	Gain of innovation code

*

**************************************************************************/

Word16 G_Code(Word16 xn2[], Word16 y2[], Word16 L_subfr)

{

Word16 i;

Word16 xy, yy, exp_xy, exp_yy, gain;

Word32 s, L_tmp;

Overflow = 0;

/* Compute scalar product <xn2[],y2[]> */

s = 1;					/* Avoid case of all zeros */

for(i=0; i<L_subfr; i++)

s = L_mac0(s, xn2[i], y2[i]);

exp_xy = norm_l(s);

xy     = extract_h( L_shl(s, exp_xy) );

/* Compute scalar product <y2[],y2[]> */

s = 1;					/* Avoid case of all zeros */

for(i=0; i<L_subfr; i++)

s = L_mac0(s, y2[i], y2[i]);

exp_yy = norm_l(s);

yy     = extract_h( L_shl(s, exp_yy) );

/* Test if Overflow */

if(Overflow == 1)

{

/* Compute scalar product <xn2[],y2[]> */

s = 1;				/* Avoid case of all zeros */

for(i=0; i<L_subfr; i++)

{

L_tmp = L_mult0(xn2[i], y2[i]);

L_tmp = L_shr(L_tmp, (Word16)6);

s = L_add(s, L_tmp);

}

exp_xy = norm_l(s);

xy     = extract_h( L_shl(s, exp_xy) );

/* Compute scalar product <y2[],y2[]> */

s = 1;				/* Avoid case of all zeros */

for(i=0; i<L_subfr; i++)

{

L_tmp = L_mult0(y2[i], y2[i]);

L_tmp = L_shr(L_tmp, (Word16)6);

s = L_add(s, L_tmp);

}

exp_yy = norm_l(s);

yy     = extract_h( L_shl(s, exp_yy) );

}

/* If (xy < 0) gain = 0  */

if( xy <= 0) return( (Word16) 0);

/* compute gain = xy/yy */

xy = shr(xy, (Word16)1);			/* Be sure xy < yy */

gain = div_s( xy, yy);

i = add(exp_xy, (Word16)2);		/* Denormalization of division */

i = sub(i, exp_yy);

gain = shr(gain, i);

return(gain);

}

/**************************************************************************

*

*	ROUTINE				:	G_Pitch

*

*	DESCRIPTION			:	Compute the gain of pitch. Result in Q12

*							if (gain < 0)  gain =0

*								if (gain >1.2) gain =1.2

*

**************************************************************************

*

*	USAGE				:	G_Pitch(buffer_in1,buffer_in2,L_subfr)

*							(Routine_Name(input1,input2,input3))

*

*	INPUT ARGUMENT(S)		:

*

*	INPUT1			:	- Description : Pitch target

*						- Format : Word16

*

*		INPUT2			:	- Description : Filtered adaptive codebook

*							- Format : Word16

*

*	INPUT3			:	- Description : Length of subframe

*						- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:	None

*

*	RETURNED VALUE		:	Gain of pitch lag in Q12 saturated to 1.2

*

**************************************************************************/

Word16 G_Pitch(Word16 xn[], Word16 y1[], Word16 L_subfr)

{

Word16 i;

Word16 xy, yy, exp_xy, exp_yy, gain;

Word32 s, L_tmp;

Overflow = 0;

/* Compute scalar product <xn[],y1[]> */

s = 1;					/* Avoid case of all zeros */

for(i=0; i<L_subfr; i++)

s = L_mac0(s, xn[i], y1[i]);

exp_xy = norm_l(s);

xy     = extract_h( L_shl(s, exp_xy) );

/* Compute scalar product <y1[],y1[]> */

s = 1;					/* Avoid case of all zeros */

for(i=0; i<L_subfr; i++)

s = L_mac0(s, y1[i], y1[i]);

exp_yy = norm_l(s);

yy     = extract_h( L_shl(s, exp_yy) );

/* Test if Overflow */

if(Overflow != 0)

{

/* Compute scalar product <xn[],y1[]> */

s = 1;				/* Avoid case of all zeros */

for(i=0; i<L_subfr; i++)

{

L_tmp = L_mult0(xn[i], y1[i]);

L_tmp = L_shr(L_tmp, (Word16)6);

s = L_add(s, L_tmp);

}

exp_xy = norm_l(s);

xy     = extract_h( L_shl(s, exp_xy) );

/* Compute scalar product <y1[],y1[]> */

s = 1;				/* Avoid case of all zeros */

for(i=0; i<L_subfr; i++)

{

L_tmp = L_mult0(y1[i], y1[i]);

L_tmp = L_shr(L_tmp, (Word16)6);

s = L_add(s, L_tmp);

}

exp_yy = norm_l(s);

yy     = extract_h( L_shl(s, exp_yy) );

}

/* If (xy < 4) gain = 0 */

if( sub(xy, (Word16)4) < 0) return( (Word16) 0);

/* compute gain = xy/yy */

xy = shr(xy, (Word16)1);			/* Be sure xy < yy */

gain = div_s( xy, yy);

i = add(exp_xy, (Word16)2);		/* Denormalization of division */

i = sub(i, exp_yy);

gain = shr(gain, i);

/* if(gain >1.2) gain = 1.2  in Q12 */

if( sub(gain, (Word16)4915) > 0) gain = 4915;

return(gain);

}

/**************************************************************************

*

*	ROUTINE				:	Inter8_M1_3

*

*	DESCRIPTION			:	Fractional interpolation -1/3 with 8 coefficients

*

**************************************************************************

*

*	USAGE				:	Inter8_M1_3(buffer_in)

*							(Routine_Name(input1))

*

*	INPUT ARGUMENT(S)		:

*

*		INPUT1			:	- Description : Buffer containing the function

*								          to be interpolated

*						- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:	None

*

*	RETURNED VALUE		:	- Description : Interpolated value

*						- Format : Word32

*

**************************************************************************/

Word32 Inter8_M1_3(Word16 x[])

{

Word16 i;

Word32 s;

static Word16 coef[8] = {-236,1050,-3572,12714,26674,-5217,1630,-384};

s = 0;

for(i=0; i<8; i++)

s = L_mac0(s, x[i-4], coef[i]);

return(s);

}

/**************************************************************************

*

*	ROUTINE				:	Inter8_1_3

*

*	DESCRIPTION			:	Fractional interpolation 1/3 with 8 coefficients

*

**************************************************************************

*

*	USAGE				:	Inter8_1_3(buffer_in)

*							(Routine_Name(input1))

*

*	INPUT ARGUMENT(S)		:

*

*		INPUT1			:	- Description : Buffer containing the function

*								          to be interpolated

*						- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:	None

*

*	RETURNED VALUE		:	- Description : Interpolated value

*						- Format : Word32

*

**************************************************************************/

Word32 Inter8_1_3(Word16 x[])

{

Word16 i;

Word32 s;

static Word16 coef[8] = {-384,1630,-5217,26674,12714,-3572,1050,-236};

s = 0;

for(i=0; i<8; i++)

s = L_mac0(s, x[i-3], coef[i]);

return(s);

}

/**************************************************************************

*

*	ROUTINE				:	Inter32_M1_3

*

*	DESCRIPTION			:	Fractional interpolation -1/3 with 32 coefficients

*

**************************************************************************

*

*	USAGE				:	Inter32_M1_3(buffer_in)

*							(Routine_Name(input1))

*

*	INPUT ARGUMENT(S)		:

*

*		INPUT1			:	- Description : Buffer containing the function

*								          to be interpolated

*						- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:	None

*

*	RETURNED VALUE		:	- Description : Interpolated value

*						- Format : Word16

*

*	COMMENTS			:	For long term prediction, it must be noted that

*							exc[-(T0-1/3)] corresponds to exc[-T0+1/3]

*

**************************************************************************/

Word16 Inter32_M1_3(Word16 x[])

{

Word16 i;

Word32 s;

static Word16 coef[32] = {

-49,    66,   -96,   142,  -207,   294,  -407,   553,  -739,

981, -1304,  1758, -2452,  3688, -6669, 27072, 13496, -5287,  3179,

-2182,  1587, -1185,   893,  -672,   500,  -366,   263,  -183,   125,

-84,    59,   -47 };

s = 0;

for(i=0; i<32; i++)

s = L_mac0(s, x[i-15], coef[i]);

s = L_add(s, s);

i = round(s);

return(i);

}

/**************************************************************************

*

*	ROUTINE				:	Inter32_1_3

*

*	DESCRIPTION			:	Fractional interpolation 1/3 with 32 coefficients

*

**************************************************************************

*

*	USAGE				:	Inter32_1_3(buffer_in)

*							(Routine_Name(input1))

*

*	INPUT ARGUMENT(S)		:

*

*		INPUT1			:	- Description : Buffer containing the function

*								          to be interpolated

*						- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:	None

*

*	RETURNED VALUE		:	- Description : Interpolated value

*						- Format : Word16

*

*	COMMENTS			:	For long term prediction, it must be noted that

*							exc[-(T0+1/3)] corresponds to exc[-T0-1/3]

*

**************************************************************************/

Word16 Inter32_1_3(Word16 x[])

{

Word16 i;

Word32 s;

static Word16 coef[32] = {

-47,    59,   -84,   125,  -183,   263,  -366,   500,  -672,   893,

-1185,  1587, -2182,  3179, -5287, 13496, 27072, -6669,  3688, -2452,

1758, -1304,   981,  -739,   553,  -407,   294,  -207,   142,   -96,

66,   -49};

s = 0;

for(i=0; i<32; i++)

s = L_mac0(s, x[i-16], coef[i]);

s = L_add(s, s);

i = round(s);

return(i);

}

/**************************************************************************

*

*	ROUTINE				:	Lag_Max

*

*	DESCRIPTION			:	Find the lag that has maximum correlation with

*							the input buffer sig_dec[]

*

**************************************************************************

*

*	USAGE				:	Lag_Max(buffer_in1,buffer_in2,

*							L_frame, lag_max,lag_min,cor_max)

*							(Routine_Name(input1,input2,

*							input3,input4,input5,output1))

*

*	INPUT ARGUMENT(S)		:

*

*	INPUT1			:	- Description :	- Signal used to compute

*								the open loop pitch

*								- Buffer_in1[-142] to buffer_in1[-1]

*								should be known

*						- Format : Word16

*

*	INPUT2			:	- Description : Decimated signal (buffer sig_dec[])

*						- Format : Word16

*

*	INPUT3			:	- Description : Length of frame to compute pitch

*						- Format : Word16

*

*	INPUT4			:	- Description : Maximum lag

*						- Format : Word16

*

*		INPUT5			:	- Description : Minimum lag

*							- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : Maximum of normalized correlation

*								         of lag found

*							- Format : Word16

*

*	RETURNED VALUE		:	- Description : Lag found

*							- Format : Word16

*

**************************************************************************/

Word16 Lag_Max(Word16 signal[], Word16 sig_dec[], Word16 L_frame,

Word16 lag_max, Word16 lag_min, Word16 *cor_max)

{

Word16  i, j;

Word16  *p, *p1;

Word32  max, t0;

Word16  max_h, max_l, ener_h, ener_l;

Word16  p_max;

max = MIN_32;

for (i = lag_max; i >= lag_min; i--)

{

p  = sig_dec;

p1 = &signal[-i];

t0 = 0;

for (j=0; j<L_frame; j+=2, p++, p1+=2)

t0 = L_mac0(t0, *p, *p1);

if (L_sub(t0,max) >= 0)

{

max    = t0;

p_max = i;

}

}

max = L_shr(max, (Word16)1);	/* for special double precision format */

L_extract(max, &max_h, &max_l);

/* compute energie */

t0 = 0;

p = &signal[-p_max];

for(i=0; i<L_frame; i+=2, p+=2)

t0 = L_mac0(t0, *p, *p);

/* 1/sqrt(energie),    result in Q30 */

t0 = inv_sqrt(t0);

L_extract(t0, &ener_h, &ener_l);

/* max = max/sqrt(energie)                  */

/* This result will always be on 16 bits !! */

t0 = mpy_32(max_h, max_l, ener_h, ener_l);

*cor_max = extract_l(t0);

return(p_max);

}

/**************************************************************************

*

*	ROUTINE				:	Norm_Corr

*

*	DESCRIPTION			:	Find the normalized correlation

*						(correlation between the target vector and

*						the filtered past excitation divided by the square root

*							of  energy of filtered excitation)

*

**************************************************************************

*

*	USAGE				:	Norm_Corr(buffer_in1,buffer_in2,buffer_in3,

*							L_subfr,t_min,t_max,corr_norm)

*							(Routine_Name(input1,input2,input3,

*							input4,input5,input6,output1))

*

*	INPUT ARGUMENT(S)		:

*

*	INPUT1			:	- Description : Excitation buffer

*						- Format : Word16

*

*	INPUT2			:	- Description : Target vector

*						- Format : Word16

*

*	INPUT3			:	- Description : Impulse response of synthesis

*							          and weighting filters

*						- Format : Word16 - Q12

*

*	INPUT4			:	- Description : Length of subframe

*						- Format : Word16

*

*		INPUT5			:	- Description : Minimum value of pitch lag

*							- Format : Word16

*

*		INPUT6			:	- Description : Maximum value of pitch lag

*							- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : Normalized correlation

*							- Format : Word16

*

*	RETURNED VALUE		:	None

*

**************************************************************************/

void Norm_Corr(Word16 exc[], Word16 xn[], Word16 h[], Word16 L_subfr,

Word16 t_min, Word16 t_max, Word16 corr_norm[])

{

Word16 i,j,k;

Word16 corr_h, corr_l, norm_hi, norm_lo;

Word32 s;

Word16 excf[80];			/* Usally dynamic allocation of (L_subfr) */

k = - t_min;

/* compute the filtered excitation for the first delay t_min */

Convolve(&exc[k], h, excf, L_subfr);

/* loop for every possible period */

for (i = t_min; i <= t_max; ++i)

{

/* Compute correlation between xn[] and excf[] */

s = 0;

for (j = 0; j < L_subfr; j++)

s = L_mac0(s, xn[j], excf[j]);

s = L_shr(s, (Word16)1);		/* Special double precision format */

L_extract(s, &corr_h, &corr_l);

/* Compute 1/sqrt(energie of excf[]) */

s = 0;

for (j = 0; j < L_subfr; j++)

s = L_mac0(s, excf[j], excf[j]);

s = inv_sqrt(s);			/* Result in Q30 */

L_extract(s, &norm_hi, &norm_lo);

/* Normalize correlation = correlation * (1/sqrt(energie)) */

s = mpy_32(corr_h, corr_l, norm_hi, norm_lo);

corr_norm[i] = extract_l(s);	/* Result is on 16 bits */

/* modify the filtered excitation excf[] for the next iteration */

if( sub(i, t_max) != 0)

{

k--;

for (j = L_subfr-1; j > 0; j--)

{

s = L_mult0(exc[k], h[j]);

s = add_sh(s, excf[j-1], (Word16)12);

excf[j] = store_hi(s, (Word16)4);

}

excf[0] = exc[k];

}

}

return;

}

/**************************************************************************

*

*	ROUTINE				:	Pitch_Fr

*

*	DESCRIPTION			:	Find the pitch period with 1/3 subsample resolution

*

**************************************************************************

*

*	USAGE				:	Pitch_Fr(buffer_in1,buffer_in2,buffer_in3,

*							L_subfr,t0_min,t0_max,i_subfr,pit_frac)

*							(Routine_Name(input1,input2,input3,

*							input4,input5,input6,input7,output1))

*

*	INPUT ARGUMENT(S)		:

*

*	INPUT1			:	- Description : Excitation buffer

*						- Format : Word16

*

*	INPUT2			:	- Description : Target vector

*						- Format : Word16

*

*	INPUT3			:	- Description : Impulse response of synthesis

*							          and weighting filters

*						- Format : Word16 - Q12

*

*	INPUT4			:	- Description : Length of subframe

*						- Format : Word16

*

*		INPUT5			:	- Description : Minimum value in the searched range

*							- Format : Word16

*

*		INPUT6			:	- Description : Maximum value in the searched range

*							- Format : Word16

*

*		INPUT7			:	- Description : Indicator for first subframe

*							- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : Chosen fraction : (-1, 0, 1) / 3

*							- Format : Word16

*

*	RETURNED VALUE		:	- Description : Pitch period

*							- Format : Word16

*

**************************************************************************/

/* Lg_inter = Length for fractionnal interpolation = nb.coeff/2 */

#define Lg_inter 4

Word16 Pitch_Fr(Word16 exc[], Word16 xn[], Word16 h[], Word16 L_subfr,

Word16 t0_min, Word16 t0_max, Word16 i_subfr,

Word16 *pit_frac)

{

Word16 i;

Word16 t_min, t_max;

Word16 max, lag, frac;

Word16 *corr;

Word32 corr_int, L_max;

Word16 corr_v[40];	/* Total length = t0_max-t0_min+1+2*Lg_inter */

/* Find interval to compute normalized correlation */

t_min = sub(t0_min, (Word16)Lg_inter);

t_max = add(t0_max, (Word16)Lg_inter);

corr = &corr_v[-t_min];

/* Compute normalized correlation between target and filtered excitation */

Norm_Corr(exc, xn, h, L_subfr, t_min, t_max, corr);

/* Find integer pitch */

max = corr[t0_min];

lag = t0_min;

for(i= t0_min+1; i<=t0_max; i++)

{

if( sub(corr[i],max)>= 0)

{

max = corr[i];

lag = i;

}

}

/* If first subframe and lag > 84 do not search fractionnal pitch */

if( (i_subfr == 0) && (sub(lag, (Word16)84) > 0) )

{

*pit_frac = 0;

return(lag);

}

/* Test the fractions around T0 and choose the one which maximizes   */

/* the interpolated normalized correlation.                          */

frac  = 0;

L_max = Load_sh(max, (Word16)15);

/* test +1/3 */

corr_int = Inter8_1_3(&corr[lag]);

if(L_sub(corr_int, L_max) >= 0)

{ L_max=corr_int; frac= 1;}

/* test +2/3  = 1-1/3 */

corr_int = Inter8_M1_3(&corr[lag+1]);

if(L_sub(corr_int, L_max) >= 0)

{ L_max=corr_int; frac= 2;}

/* test -2/3 = 1/3 -1 */

corr_int = Inter8_1_3(&corr[lag-1]);

if(L_sub(corr_int, L_max) >= 0)

{ L_max=corr_int; frac= -2;}

/* test -1/3 */

corr_int = Inter8_M1_3(&corr[lag]);

if(L_sub(corr_int, L_max) >= 0)

{ L_max=corr_int; frac= -1;}

if(sub(frac, (Word16)2) == 0)

{ frac = -1; lag = add(lag, (Word16)1);};

if(sub(frac, (Word16)-2) == 0)

{ frac = 1; lag = sub(lag, (Word16)1);};

*pit_frac = frac;

return(lag);

}

/**************************************************************************

*

*	ROUTINE				:	Pitch_Ol_Dec

*

*	DESCRIPTION			:	Compute the open loop pitch lag

*							(includes decimation of signal)

*

**************************************************************************

*

*	USAGE				:	Pitch_Ol_Dec(buffer_in,L_frame)

*							(Routine_Name(input1,input2))

*

*	INPUT ARGUMENT(S)		:

*

*	INPUT1			:	- Description :	- Signal used to compute

*								the open loop pitch

*								- Buffer_in[-142] to buffer_in[-1]

*								should be known

*						- Format : Word16

*

*	INPUT2			:	- Description : Length of frame to compute pitch

*						- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:	None

*

*	RETURNED VALUE		:	- Description : Open pitch lag as found

*							- Format : Word16

*

**************************************************************************/

/* Minimum and maximum pitch lag */

#define pit_min 20

#define pit_max 142

/* Threshold to favor small pitch = 0.85 in Q15 */

#define seuil 27856

Word16 Pitch_Ol_Dec(Word16 signal[], Word16 L_frame)

{

Word16  i, j;

Word16  max1, max2, max3;

Word16  p_max1, p_max2, p_max3;

Word32  t0;

/* Decimated signal                                       */

/* Can be allocated with memory allocation of(L_frame/2)  */

Word16  sig_dec[120];

/*--------------------------------------------------------*

*  Verification for risk of overflow.                    *

*--------------------------------------------------------*/

Overflow = 0;

t0 = 0;

for(i= -pit_max; i< L_frame; i+=2)

t0 = L_mac0(t0, signal[i], signal[i]);

/*--------------------------------------------------------*

* Decimation of signal and scaling.                      *

*                                                        *

*   if Overflow        -> sig_dec[i] = signal[i*2]>>6    *

*   else if t0 < 1<<22 -> sig_dec[i] = signal[i*2]<<4    *

*   else               -> sig_dec[i] = signal[i*2]       *

*--------------------------------------------------------*/

if(Overflow == 1)

{

for(i=0, j=0; i<L_frame; i+=2, j++)

sig_dec[j] = shr(signal[i], (Word16)6);

}

else if (L_sub(t0, L_shl( (Word32)1, (Word16)22)) < 0 )

{

for(i=0, j=0; i<L_frame; i+=2, j++)

sig_dec[j] = shl(signal[i], (Word16)4);

}

else

{

for(i=0, j=0; i<L_frame; i+=2, j++)

sig_dec[j] = signal[i];

}

/*--------------------------------------------------------------------*

*  The pitch lag search is divided in three sections.                *

*  Each section cannot have a pitch multiple.                        *

*  A maximum is found for each section.                              *

*  The maximum of each section is compared to favor small lag.       *

*                                                                    *

*  First section:  lag delay = pit_max to 80                         *

*  Second section: lag delay = 79 to 40                              *

*  Third section:  lag delay = 39 to 20                              *

*--------------------------------------------------------------------*/

p_max1 = Lag_Max(signal, sig_dec, L_frame, (Word16)pit_max,

(Word16)80 , &max1);

p_max2 = Lag_Max(signal, sig_dec, L_frame, (Word16)79     ,

(Word16)40 , &max2);

p_max3 = Lag_Max(signal, sig_dec, L_frame, (Word16)39     ,

(Word16)20 , &max3);

/*--------------------------------------------------------------------*

* Compare the 3 sections maximum, and favor small lag.               *

*--------------------------------------------------------------------*/

if( sub(mult((Word16)max1, (Word16)seuil), (Word16)max2)  < 0)

{ max1 = max2;  p_max1 = p_max2;}

if( sub(mult((Word16)max1, (Word16)seuil), (Word16)max3)  < 0)

p_max1 = p_max3;

return (p_max1);

}

/**************************************************************************

*

*	ROUTINE				:	Post_Process

*

*	DESCRIPTION			:	Post-processing of output speech

*							Multiplication by two of output speech

*							with saturation

*

**************************************************************************

*

*	USAGE				:	Post_Process(buffer,Length)

*							(Routine_Name(arg1,input2))

*

*	INPUT ARGUMENT(S)		:

*

*	ARG1				:	- Description : Input speech signal buffer

*						- Format : Word16

*

*		INPUT2			:	- Description : Length of signal

*							- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:

*

*		ARG1				:	- Description : Output speech signal buffer

*							- Format : Word16

*

*	RETURNED VALUE		:	None

*

**************************************************************************/

void Post_Process(Word16 signal[], Word16 lg)

{

Word16 i;

for(i=0; i<lg; i++)

signal[i] = add(signal[i], signal[i]);

}

/**************************************************************************

*

*	ROUTINE				:	Pred_Lt

*

*	DESCRIPTION			:	Compute the result of long term prediction with

*							fractional interpolation

*

**************************************************************************

*

*	USAGE				:	Pred_Lt(buffer,T0,frac,L_subfr)

*							(Routine_Name(arg1,input2,input3,input4))

*

*	INPUT ARGUMENT(S)		:

*

*	ARG1				:	- Description : Excitation vector

*						- Format : Word16

*

*	INPUT2			:	- Description : Pitch lag

*						- Format : Word16

*

*	INPUT3			:	- Description : Fraction of pitch lag : (-1, 0, 1)  / 3

*						- Format : Word16 - Q12

*

*	INPUT4			:	- Description : Length of subframe

*						- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:

*

*		ARG1				:	- Description : Interpolated signal contained in

*								         buffer[0..L_subfr-1]

*							- Format : Word16

*

*	RETURNED VALUE		:	None

*

**************************************************************************/

void Pred_Lt(Word16 exc[], Word16 T0, Word16 frac, Word16 L_subfr)

{

Word16 i;

if(frac == 0)

{

for (i = 0; i < L_subfr; i++)

exc[i] = exc[i-T0];

}

else if( sub(frac, (Word16)1) == 0)

{

for (i = 0; i < L_subfr; i++)

exc[i] = Inter32_1_3(&exc[i-T0]);

}

else if( sub(frac, (Word16)-1) == 0)

{

for (i = 0; i < L_subfr; i++)

exc[i] = Inter32_M1_3(&exc[i-T0]);

}

return;

}

/**************************************************************************

*

*	ROUTINE				:	Pre_Process

*							(include Init_Pre_Process)

*

*	DESCRIPTION			:	Preprocessing of input speech

*							- Offset compensation

*								- Divide input by two

*

**************************************************************************

*

*	USAGE				:	Pre_Process(buffer,Length)

*							(Routine_Name(arg1,input2))

*

*	INPUT ARGUMENT(S)		:

*

*	ARG1				:	- Description : Input speech signal buffer

*						- Format : Word16

*

*		INPUT2			:	- Description : Length of signal

*							- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:

*

*		ARG1				:	- Description : Output speech signal buffer

*							- Format : Word16

*

*	RETURNED VALUE		:	None

*

*	COMMENTS			:	Algorithm :

*

*						y[i] = x[i]/2 - x[i-1]/2 + alpha * y[i-1]

*

*  						alpha = 32735 / 2**15

*  						y[i-1] is keep in double precision

*

*  						It is the same algorithm than in GSM except that :

*  						- Input is divided by two in the filtering process

*  						- No rounding

*

**************************************************************************/

/* Static values to be preserved between calls */

static Word16 y_hi, y_lo, x0;

/* Initialization of static values */

void Init_Pre_Process(void)

{

y_hi = 0;

y_lo = 0;

x0   = 0;

}

/* Offset compensation and divide by 2 */

void Pre_Process(Word16 signal[], Word16 lg)

{

Word16 i, x1;

Word32 L_tmp;

for(i=0; i<lg; i++)

{

x1 = x0;

x0 = signal[i];

L_tmp     = Load_sh(x0, (Word16)15);

L_tmp     = sub_sh(L_tmp, x1, (Word16)15);

L_tmp     = L_mac(L_tmp, y_hi, (Word16)32735);

L_tmp     = add_sh(L_tmp, mult(y_lo, (Word16)32735), (Word16)1);

signal[i] = extract_h(L_tmp);

y_hi      = extract_h(L_tmp);

y_lo      = extract_l(sub_sh(L_shr( L_tmp,(Word16)1 ), y_hi, (Word16)15));

}

return;

}

/**************************************************************************

*

*	ROUTINE				:	Prm2bits_Tetra

*

*	DESCRIPTION			:	Convert the encoder parameter vector into

*							a vector of serial bits

*

**************************************************************************

*

*	USAGE				:	Prm2bits_Tetra(buffer_in,buffer_out)

*							(Routine_Name(input1,output1))

*

*	INPUT ARGUMENT(S)		:

*

*		INPUT1			:	- Description : Encoded parameters

*								         (23 parameters)

*							- Format : Word16 - .. * 16 bit-samples

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : Serial bits (137 + bfi)

*							- Format : Word16 - .. * 16 bit-samples

*

*	RETURNED VALUE		:	None

*

*	COMMENTS			:	The transmitted parameters are :

*

*						- LSP	1st codebook 			8 bits

*							2nd codebook			9 bits

*							3nd codebook			9 bits

*

*						- for the 4 subframes :

*							pitch delay			8 bits (first)

*											5 bits (others)

*							codebook index		14 bits

*							pulse global sign		1 bit

*							pulse shift			1 bit

*							pitch and innovation gains	6 bits

*

**************************************************************************/

#define PRM_NO    23

void Prm2bits_Tetra(Word16 prm[], Word16 bits[])

{

Word16 i;

static Word16 bitno[PRM_NO] = {8, 9, 9,            /* split VQ LSP  */

8, 14, 1, 1, 6,     /* subframe 1    */

5, 14, 1, 1, 6,     /* subframe 2    */

5, 14, 1, 1, 6,     /* subframe 3    */

5, 14, 1, 1, 6};    /* subframe 4    */

*bits++ = 0;	/* bit[0] = 0, at receiver this bits indicate BFI */

for (i = 0; i < PRM_NO; i++)

{

int2bin(prm[i], bitno[i], bits);

bits += bitno[i];

}

return;

}

## A.4	Library of Signal Processing Related Subroutines

/************************************************************************

*

*	FILENAME		:	sub_dsp.c

*

*	DESCRIPTION		:	General Purpose Signal Processing Library

*

************************************************************************

*

*	SUB-ROUTINES	:	- Autocorr()

*					- Az_Lsp()

*					- Back_Fil()

*					- Chebps()

*					- Convolve()

*					- Fac_Pond()

*					- Get_Lsp_Pol()

*					- Int_Lpc4()

*					- Lag_Window()

*					- Levin_32()

*					- LPC_Gain()

*					- Lsp_Az()

*					- Pond_Ai()

*					- Residu()

*					- Syn_Filt()

*

************************************************************************

*

*	INCLUDED FILES	:	source.h

*					stdio.h

*					stdlib.h

*

*					grid .tab

*					lag_wind.tab

*					window.tab

*

************************************************************************/

#include <stdio.h>

#include <stdlib.h>

#include "source.h"

#include "grid.tab"

#include "lag_wind.tab"

#include "window.tab"

/* pp = local LPC order, nc =pp/2 */

#define pp (Word16)10

#define nc (Word16)5

/* Length for local impulse response */

#define  llg     (Word16)60

/**************************************************************************

*

*	ROUTINE				:	Autocorr

*

*	DESCRIPTION			:	Compute autocorrelations

*

**************************************************************************

*

*	USAGE				:	Autocorr(buffer_in,p,buffer_out1,buffer_out2)

*							(Routine_Name(input1,input2,output1,output2))

*

*	INPUT ARGUMENT(S)		:

*

*	INPUT1			:	- Description : Input signal buffer

*						- Format : Word16

*

*	INPUT2			:	- Description : LPC order

*						- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : Autocorrelations buffer (msb)

*							- Format : Word16

*

*		OUTPUT2			:	- Description : Autocorrelations buffer (lsb)

*							- Format : Word16

*

*	RETURNED VALUE		:	None

*

**************************************************************************/

void Autocorr(Word16 x[], Word16 p, Word16 r_h[], Word16 r_l[])

{

Word16 i, j, norm;

Word16 y[L_window];

Word32 sum;

extern Flag Overflow;

/* Windowing of signal */

for(i=0; i<L_window; i++)

y[i] = mult_r(x[i], window[i]);

/* Compute r[0] and test for overflow */

do {

Overflow = 0;

sum = 1;				/* Avoid case of all zeros */

for(i=0; i<L_window; i++)

sum = L_mac0(sum, y[i], y[i]);

/* If overflow divide y[] by 4 */

if(Overflow != 0)

{

for(i=0; i<L_window; i++)

y[i] = shr(y[i], (Word16)2);

}

}while(Overflow != 0);

/* Normalization of r[0] */

norm = norm_l(sum);

sum  = L_shl(sum, norm);

sum  = L_shr(sum, (Word16)1);		/* For special double format */

L_extract(sum, &r_h[0], &r_l[0]);

/* r[1] to r[p] */

for (i = 1; i <= p; i++)

{

sum = 0;

for(j=0; j<L_window-i; j++)

sum = L_mac0(sum, y[j], y[j+i]);

sum = L_shr(sum, (Word16)1);		/* Special double format */

sum = L_shl(sum, norm);

L_extract(sum, &r_h[i], &r_l[i]);

}

return;

}

/**************************************************************************

*

*	ROUTINE				:	Az_Lsp

*

*	DESCRIPTION			:	Compute the LSPs  in the cosine domain

*							from the LPC coefficients  (order=10)

*

**************************************************************************

*

*	USAGE				:	Az_Lsp(buffer_in1,buffer_out,buffer_in2)

*							(Routine_Name(input1,output1,input2))

*

*	INPUT ARGUMENT(S)		:

*

*	INPUT1			:	- Description : Predictor coefficients

*						- Format : Word16 - Q12

*

*	INPUT2			:	- Description : Previous LSP values

*							          (in case not 10 roots are found)

*						- Format : Word16 - Q15

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : Line spectral pairs in the

*								          cosine domain

*							- Format : Word16 - Q15

*

*	RETURNED VALUE		:	None

*

**************************************************************************/

void Az_Lsp(Word16 a[], Word16 lsp[], Word16 old_lsp[])

{

Word16 i, j, nf, ip;

Word16 xlow, ylow, xhigh, yhigh, xmid, ymid, xint;

Word16 x, y, sign, exp;

Word16 *coef;

Word16 f1[pp/2+1], f2[pp/2+1];

Word32 t0;

/*-------------------------------------------------------------*

*  find the sum and diff. pol. F1(z) and F2(z)                *

*    F1(z) <--- F1(z)/(1+z**-1) & F2(z) <--- F2(z)/(1-z**-1)  *

*                                                             *

* f1[0] = 1.0;                                                *

* f2[0] = 1.0;                                                *

*                                                             *

* for (i = 0; i< nc; i++)                                     *

* {                                                           *

*   f1[i+1] = a[i+1] + a[pp-i] - f1[i] ;                       *

*   f2[i+1] = a[i+1] - a[pp-i] + f2[i] ;                       *

* }                                                           *

*-------------------------------------------------------------*/

f1[0] = 2048;				/* f1[0] = 1.0 is in Q11 */

f2[0] = 2048;				/* f2[0] = 1.0 is in Q11 */

for (i = 0; i< nc; i++)

{

t0 = Load_sh(a[i+1], (Word16)15);	/* a[i+1]  in Q27 */

t0 = add_sh(t0, a[pp-i], (Word16)15);	/* +a[pp-i] in Q27 */

t0 = sub_sh16(t0, f1[i]);			/* -f1[i]  in Q27 */

f1[i+1] = extract_h(t0);		/* f1[i+1] = a[i+1] + a[pp-i] - f1[i] */

/* result in Q11  */

t0 = Load_sh(a[i+1], (Word16)15);	/* a[i+1]  in Q27   */

t0 = sub_sh(t0, a[pp-i], (Word16)15);	/* -a[pp-i] in Q27 */

t0 = add_sh16(t0, f2[i]);			/* +f2[i] in Q27  */

f2[i+1] = extract_h(t0);		/* f2[i+1] = a[i+1] - a[pp-i] + f2[i] */

/* result in Q11  */

}

/*-------------------------------------------------------------*

* find the LSPs using the Chebichev pol. evaluation           *

*-------------------------------------------------------------*/

nf=0;          /* number of found frequencies */

ip=0;          /* indicator for f1 or f2      */

coef = f1;

xlow = grid[0];

ylow = Chebps(xlow, coef, nc);

j = 0;

while ( (nf < pp) && (j < grid_points) )

{

j++;

xhigh = xlow;

yhigh = ylow;

xlow  = grid[j];

ylow  = Chebps(xlow,coef,nc);

if ( L_mult0(ylow,yhigh) <= (Word32)0)

{

/* divide 4 times the interval */

for (i = 0; i < 4; i++)

{

t0   = Load_sh(xlow, (Word16)15);	/* xmid = 0.5*(xlow + xhigh) */

t0   = add_sh(t0, xhigh, (Word16)15);

xmid = extract_h(t0);

ymid = Chebps(xmid,coef,nc);

if ( L_mult0(ylow,ymid) <= (Word32)0)

{

yhigh = ymid;

xhigh = xmid;

}

else

{

ylow = ymid;

xlow = xmid;

}

}

/*-------------------------------------------------------------*

* Linear interpolation                                        *

*    xint = xlow - ylow*(xhigh-xlow)/(yhigh-ylow);            *

*-------------------------------------------------------------*/

x   = sub(xhigh, xlow);

y   = sub(yhigh, ylow);

if(y == 0)

{

xint = xlow;

}

else

{

sign= y;

y   = abs_s(y);

exp = norm_s(y);

y   = shl(y, exp);

y   = div_s( (Word16)16383, y);

t0  = L_mult0( x, y);

t0  = L_shr(t0, sub((Word16)19,exp) );

y    = extract_l(t0);		/* y= (xhigh-xlow)/(yhigh-ylow) in Q10 */

if(sign < 0) y = negate(y);

t0   = Load_sh(xlow, (Word16)10);	/* xint = xlow - ylow*y */

t0   = L_msu0(t0, ylow, y);

xint = store_hi(t0, (Word16)6);

}

lsp[nf] = xint;

xlow    = xint;

nf++;

if(ip == 0)

{

ip = 1;

coef = f2;

}

else

{

ip = 0;

coef = f1;

}

ylow = Chebps(xlow,coef,nc);

}

}

/* Check if pp roots found */

if( sub(nf, pp) < 0)

{

for(i=0; i<pp; i++)

lsp[i] = old_lsp[i];

printf("\n !!Not 10 roots found in Az_Lsp()!!!n");

}

return;

}

/**************************************************************************

*

*	ROUTINE				:	Back_Fil

*

*	DESCRIPTION			:	Perform the Backward filtering of input vector

*						buffer_in1 with buffer_in2 and write the result

*						in output vector buffer_out.

*							All vectors are of length L

*

**************************************************************************

*

*	USAGE				:	Back_Fil(buffer_in1,buffer_in2,buffer_out,L)

*							(Routine_Name(input1,input2,output1,input3))

*

*	INPUT ARGUMENT(S)		:

*

*	INPUT1			:	- Description : Input vector

*						- Format : Word16

*

*	INPUT2			:	- Description : Impulse response

*						- Format : Word16 - Q12

*

*	INPUT3			:	- Description : Vector size

*						- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : Backward filtering result

*							- Format : Word16

*

*	RETURNED VALUE		:	None

*

**************************************************************************/

void Back_Fil(Word16 x[], Word16 h[], Word16 y[], Word16 L)

{

Word16 i, j;

Word32 s, max;

Word32 y32[60];		/* Usually, dynamic allocation of L */

/* first keep the result on 32 bits and find absolute maximum */

max = 0;

for (i = 0; i < L; i++)

{

s = 0;

for (j = i; j <  L; j++)

s = L_mac0(s, x[j], h[j-i]);

y32[i] = s;

s = L_abs(s);

if(L_sub(s, max) > 0) max = s;

}

/* Find the number of right shifts to do on y32[]  */

/* so that maximum is on 13 bits                   */

j = norm_l(max);

if( sub(j,(Word16)16) > 0) j = 16;

j = sub((Word16)18, j);

for(i=0; i<L; i++)

y[i] = extract_l( L_shr(y32[i], j) );

return;

}

/**************************************************************************

*

*	ROUTINE				:	Chebps

*

*	DESCRIPTION			:	Evaluate the Chebishev polynomial series

*

**************************************************************************

*

*	USAGE				:	Chebps(x,buffer_in,n)

*							(Routine_Name(input1,input2,input3))

*

*	INPUT ARGUMENT(S)		:

*

*	INPUT1			:	- Description : Input value of evaluation

*							          x = cos(frequency)

*						- Format : Word16 - Q15

*

*	INPUT2			:	- Description : Coefficients of the polynomial series

*						- Format : Word16 - Q11

*

*	INPUT3			:	- Description : Polynomial order

*						- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:	None

*

*	RETURNED VALUE		:	- Description : Value of the polynomial C(x)

*							- Format : Word16 - Q14

*							     (saturated to +/- 1.99)

*

*	COMMENTS			:	- The polynomial order is :

*						n = p/2   (p is the prediction order)

*

*						- The polynomial is given by :

*							C(x)=T_n(x)+f(1)T_n-1(x)+...+f(n-1)T_1(x)+ f(n)/2

*

**************************************************************************/

Word16 Chebps(Word16 x, Word16 f[], Word16 n)

{

Word16 i, cheb;

Word16 b0_h, b0_l, b1_h, b1_l, b2_h, b2_l;

Word32 t0;

/* Note: All computation are done in Q24. */

b2_h = 512;					/* b2 = 1.0 in Q24 DPF */

b2_l = 0;

t0 = Load_sh( x,(Word16)10);		/* 2*x in Q24          */

t0 = add_sh( t0, f[1], (Word16)13);	/* + f[1] in Q24       */

L_extract(t0, &b1_h, &b1_l);		/* b1 = 2*x + f[1]     */

for (i = 2; i<n; i++)

{

t0 = mpy_mix(b1_h, b1_l, x);		/* t0 = x*b1                  */

t0 = L_shl(t0,(Word16)1);			/* t0 = 2.0*x*b1              */

t0 = sub_sh(t0, b2_l, (Word16)0);	/* t0 = 2.0*x*b1 - b2         */

t0 = sub_sh(t0, b2_h, (Word16)15);

t0 = add_sh(t0, f[i], (Word16)13);	/* t0 = 2.0*x*b1 - b2 + f[i]; */

L_extract(t0, &b0_h, &b0_l);		/* b0 = 2.0*x*b1 - b2 + f[i]; */

b2_l = b1_l;					/* b2 = b1; */

b2_h = b1_h;

b1_l = b0_l;					/* b1 = b0; */

b1_h = b0_h;

}

t0 = mpy_mix(b1_h, b1_l, x);		/* t0    = x*b1;     */

t0 = sub_sh(t0, b2_l, (Word16)0);		/* t0   -= b2;       */

t0 = sub_sh(t0, b2_h, (Word16)15);

t0 = add_sh(t0, f[n], (Word16)12);	/* t0   += 0.5*f[n]; */

t0 = L_shl(t0, (Word16)6);			/* Q24 to Q30 with saturation */

cheb = extract_h(t0);				/* Result in Q14           */

return(cheb);

}

/**************************************************************************

*

*	ROUTINE				:	Convolve

*

*	DESCRIPTION			:	Perform the convolution between two input vectors

*						buffer_in1 and buffer_in2 and write the result in

*						output vector buffer_out.

*							All vectors are of length L

*

**************************************************************************

*

*	USAGE				:	Convolve(buffer_in1,buffer_in2,buffer_out,L)

*							(Routine_Name(input1,input2,output1,input3))

*

*	INPUT ARGUMENT(S)		:

*

*	INPUT1			:	- Description : Input vector

*						- Format : Word16

*

*	INPUT2			:	- Description : Impulse response

*						- Format : Word16 - Q12

*

*	INPUT3			:	- Description : Vector size

*						- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : Output vector

*							- Format : Word16

*

*	RETURNED VALUE		:	None

*

**************************************************************************/

void Convolve(Word16 x[], Word16 h[], Word16 y[], Word16 L)

{

Word16 i, n;

Word32 s;

for (n = 0; n < L; n++)

{

s = 0;

for (i = 0; i <= n; i++)

s = L_mac0(s, x[i], h[n-i]);

y[n] = store_hi(s, (Word16)4);		/* h is in Q12 */

}

return;

}

/**************************************************************************

*

*	ROUTINE				:	Fac_Pond

*

*	DESCRIPTION			:	Compute LPC spectral expansion factors (fac[])

*							with the LPC order fixed to 10

*

**************************************************************************

*

*	USAGE				:	Fac_Pond(gamma,buffer_out)

*							(Routine_Name(input1,output1))

*

*	INPUT ARGUMENT(S)		:

*

*	INPUT1			:	- Description : Spectral expansion

*						- Format : Word16 - Q15

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : Computed expansion factors

*							- Format : Word16 - Q15

*

*	RETURNED VALUE		:	None

*

*	COMMENTS			:	- fac[0] = gamma

*							- fac[i] = fac[i-1] * gamma	i=1,9

*

**************************************************************************/

void Fac_Pond(Word16 gamma, Word16 fac[])

{

Word16 i;

fac[0] = gamma;

for(i=1; i<pp; i++)

fac[i] = round( L_mult(fac[i-1], gamma) );

return;

}

/**************************************************************************

*

*	ROUTINE				:	Get_Lsp_Pol

*

*	DESCRIPTION			:	Find the polynomial F1(z) or F2(z) from the LSPs

*

**************************************************************************

*

*	USAGE				:	Get_Lsp_Pol(buffer_in,buffer_out)

*							(Routine_Name(input1,output1))

*

*	INPUT ARGUMENT(S)		:

*

*	INPUT1			:	- Description : line spectral pairs

*							          (cosine domaine)

*						- Format : Word16 - Q15

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : Coefficients of F1 or F2

*							- Format : Word32 - Q24

*

*	RETURNED VALUE		:	None

*

**************************************************************************/

void Get_Lsp_Pol(Word16 *lsp, Word32 *f)

{

Word16 i,j, hi, lo;

Word32 t0;

/* Computation in Q24 */

*f = Load_sh((Word16)4096,(Word16)12);	/* f[0] = 1.0;           in Q24  */

f++;

*f = 0;

*f = sub_sh(*f, *lsp, (Word16)10);	/* f[1] = -2.0 * lsp[0]; in Q24  */

f++;

lsp += 2;					/* Advance lsp pointer           */

for(i=2; i<=5; i++)

{

*f = f[-2];

for(j=1; j<i; j++, f--)

{

L_extract(f[-1] ,&hi, &lo);

t0 = mpy_mix(hi, lo, *lsp);		/* t0 = f[-1] * lsp    */

t0 = L_shl(t0, (Word16)1);

*f = L_add(*f, f[-2]);			/* *f += f[-2]         */

*f = L_sub(*f, t0);			/* *f -= t0            */

}

*f   = sub_sh(*f, *lsp, (Word16)10);	/* *f -= lsp<<10       */

f   += i;					/* Advance f pointer   */

lsp += 2;					/* Advance lsp pointer */

}

return;

}

/**************************************************************************

*

*	ROUTINE				:	Int_Lpc4

*

*	DESCRIPTION			:	Perform the LPC interpolation for the 4 sub-frames.

*							The interpolation is done on the LSP computed

*							in the cosine domain

*

**************************************************************************

*

*	USAGE				:	Int_Lpc4(buffer_in1,buffer_in2,buffer_out)

*							(Routine_Name(input1,input2,output1))

*

*	INPUT ARGUMENT(S)		:

*

*	INPUT1			:	- Description : LSP of previous frame

*						- Format : Word16

*

*	INPUT2			:	- Description : LSP of current frame

*						- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : LPC coeff. vector for the 4 sub-frames

*							- Format : Word16

*

*	RETURNED VALUE		:	None

*

**************************************************************************/

void Int_Lpc4(Word16 lsp_old[], Word16 lsp_new[], Word16 a[])

{

Word16 i, j, fac_new, fac_old;

Word16 lsp[pp];

Word32 t0;

fac_new = 8192;       /* 1/4 in Q15 */

fac_old = 24576;      /* 3/4 in Q15 */

for(j=0; j<33; j+=11)

{

for(i=0; i<pp; i++)

{

t0 = L_mult(lsp_old[i], fac_old);

t0 = L_mac(t0, lsp_new[i], fac_new);

lsp[i] = extract_h(t0);

}

Lsp_Az(lsp, &a[j]);

fac_old = sub(fac_old, (Word16)8192);

fac_new = add(fac_new, (Word16)8192);

}

Lsp_Az(lsp_new, &a[33]);

return;

}

/**************************************************************************

*

*	ROUTINE				:	Lag_Window

*

*	DESCRIPTION			:	Lag_window on autocorrelations

*

**************************************************************************

*

*	USAGE				:	Lag_Window(p,buffer1,buffer2)

*							(Routine_Name(input1,arg2,arg3))

*

*	INPUT ARGUMENT(S)		:

*

*	INPUT1			:	- Description :  LPC order

*						- Format : Word16

*

*	ARG2				:	- Description : Autocorrelations (msb)

*						- Format : Word16

*

*	ARG3				:	- Description : Autocorrelations (lsb)

*						- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:

*

*	ARG2				:	- Description : Lag_Windowed autocorrelations (msb)

*						- Format : Word16

*

*	ARG3				:	- Description : Lag_Windowed autocorrelations (lsb)

*						- Format : Word16

*

*	RETURNED VALUE		:	None

*

*	COMMENTS			:	r[i] *= lag_wind[i]

*

*						r[i] and lag_wind[i] are in special extended precision

*						(for details on this format, see "comments"

*						in the header of the file fexp_tet.c - Section A.5.2)

*

**************************************************************************/

void Lag_Window(Word16 p, Word16 r_h[], Word16 r_l[])

{

Word16 i;

Word32 x;

for(i=1; i<=p; i++)

{

x  = mpy_32(r_h[i], r_l[i], lag_h[i-1], lag_l[i-1]);

L_extract(x, &r_h[i], &r_l[i]);

}

return;

}

/**************************************************************************

*

*	ROUTINE				:	Levin_32

*

*	DESCRIPTION			:	Computation of 10 LPC coefficients

*							based on the Levison-Durbin algorithm

*							in double precision

*

**************************************************************************

*

*	USAGE				:	Levin_32(buffer_in1,buffer_in2,buffer_out)

*							(Routine_Name(input1,input2,output1))

*

*	INPUT ARGUMENT(S)		:

*

*	INPUT1			:	- Description : Vector of autocorrelations (msb)

*						- Format : Word16 - 11 values

*

*	INPUT2			:	- Description : Vector of autocorrelations (lsb)

*						- Format : Word16 - 11 values

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : LPC coefficients

*							- Format : Word16 - Q12

*

*	RETURNED VALUE		:	None

*

*	COMMENTS			:	Algorithm :

*

*							R[i]	autocorrelations

*							A[i]	filter coefficients

*							K 	reflection coefficients

*							Alpha	prediction gain.

*

*						Initialisations :

*

*							A[0] = 1

*							K    = -R[1]/R[0]

*							A[1] = K

*							Alpha = R[0] * (1-K**2]

*

*						DO for  i = 2 to pp

*

*							S =  SUM ( R[j]*A[i-j] ,j=1,i-1 ) +  R[i]

*							K = -S / Alpha

* 							An[j] = A[j] + K*A[i-j]   for j=1 to i-1

*								where   An[i] = new A[i]

*							An[i]=K

*							Alpha=Alpha * (1-K**2)

*

*						END

*

**************************************************************************

**************************************************************************

*

*	NOTES ON THE DYNAMICS OF THE COMPUTATIONS	:

*

*	The numbers used are in double precision with the following format :

*

*	A = AH <<15 + AL.  AH and AL are 16 bit signed integers. Since the LSB's also contain *	a sign bit, this format does not correspond to standard 32 bit integers.  This format is used *	since it allows fast execution of multiplications and divisions.

*

*	"DPF" will refer to this special format in the following text.

*	(for details on this format, see "comments" in the header of file fexp_tet.c - Section A.5.2)

*

*	The R[i] were normalized in routine Autocorr (hence, R[i] < 1.0).

*	The K[i] and Alpha are theoretically < 1.0.

*	The A[i], for a sampling frequency of 8 kHz, are in practice always inferior to 16.0.

*

*	These characteristics allow straigthforward fixed-point implementation.  The parameters are *	represented as follows :

*

*	R[i]    Q30   +- .99..

*	K[i]    Q30   +- .99..

*	Alpha   Normalised -> mantissa in Q30 plus exponant

*	A[i]    Q26   +- 15.999..

*

*	The additions are performed in 32 bit.  For the summation used to compute the K[i],

*	numbers in Q30 are multiplied by numbers in Q26, with the results of the multiplications in *	Q26, resulting in a dynamic of +/- 32.  This is sufficient to avoid overflow, since the final

*	result of the summation is necessarily < 1.0 as both the K[i] and Alpha are

*	theoretically < 1.0.

*

**************************************************************************/

/* Last A(z) for case of unstable filter */

static Word16 old_A[pp+1]={4096,0,0,0,0,0,0,0,0,0,0};

void Levin_32(Word16 Rh[], Word16 Rl[], Word16 A[])

{

Word16 i, j;

Word16 hi, lo;

Word16 Kh, Kl;              	/* reflexion coefficient; hi and lo          */

Word16 alp_h, alp_l, alp_e;	/* Prediction gain; hi lo and exponant       */

Word16 Ah[pp+1], Al[pp+1];	/* LPC coef. in double prec.                 */

Word16 Anh[pp+1], Anl[pp+1];	/* LPC coef.for next iterat. in double prec. */

Word32 t0, t1, t2;		/* temporary variable                        */

/* K = A[1] = -R[1] / R[0] */

t1  = L_comp(Rh[1], Rl[1]);			/* R[1] in Q30      */

t2  = L_abs(t1);				/* abs R[1]         */

t0  = div_32(t2, Rh[0], Rl[0]);		/* R[1]/R[0] in Q30 */

if(t1 > 0) t0= L_negate(t0);		/* -R[1]/R[0]       */

L_extract(t0, &Kh, &Kl);			/* K in DPF         */

t0 = L_shr(t0,(Word16)4);			/* A[1] in Q26      */

L_extract(t0, &Ah[1], &Al[1]);		/* A[1] in DPF      */

/*  Alpha = R[0] * (1-K**2) */

t0 = mpy_32(Kh ,Kl, Kh, Kl);		/* K*K      in Q30 */

t0 = L_abs(t0);					/* Some case <0 !! */

t0 = L_sub( (Word32)0x3fffffff, t0 );	/* 1 - K*K  in Q30 */

L_extract(t0, &hi, &lo);			/* DPF format      */

t0 = mpy_32(Rh[0] ,Rl[0], hi, lo);	/* Alpha in Q30    */

/* Normalize Alpha */

t0 = norm_v(t0, (Word16)12, &i);

t0 = L_shr(t0, (Word16)1);

L_extract(t0, &alp_h, &alp_l);		/* DPF format    */

alp_e = i-1;					/* t0 was in Q30 */

/*--------------------------------------*

* ITERATIONS  I=2 to pp                *

*--------------------------------------*/

for(i= 2; i<=pp; i++)

{

/* t0 = SUM ( R[j]*A[i-j] ,j=1,i-1 ) +  R[i] */

t0 = 0;

for(j=1; j<i; j++)

t0 = L_add(t0, mpy_32(Rh[j], Rl[j], Ah[i-j], Al[i-j]));

t0 = L_shl(t0,(Word16)4);			/* result in Q26->convert to Q30 */

/* No overflow possible          */

t1 = L_comp(Rh[i],Rl[i]);

t0 = L_add(t0, t1);				/* add R[i] in Q30               */

/* K = -t0 / Alpha */

t1 = L_abs(t0);

t2 = div_32(t1, alp_h, alp_l);		/* abs(t0)/Alpha                 */

if(t0 > 0) t2= L_negate(t2);		/* K =-t0/Alpha                  */

t2 = L_shl(t2, alp_e);			/* denormalize; compare to Alpha */

L_extract(t2, &Kh, &Kl);			/* K in DPF                      */

/* Test for unstable filter, if unstable keep old A(z) */

if ( abs_s(Kh) > 32750)

{

for(j=0; j<=pp; j++) A[j] = old_A[j];

return;

}

/*------------------------------------------*

*  Compute new LPC coeff. -> An[i]         *

*  An[j]= A[j] + K*A[i-j]     , j=1 to i-1 *

*  An[i]= K                                *

*------------------------------------------*/

for(j=1; j<i; j++)

{

t0 = mpy_32(Kh, Kl, Ah[i-j], Al[i-j]);

t0 = add_sh(t0, Ah[j], (Word16)15);

t0 = add_sh(t0, Al[j], (Word16)0);

L_extract(t0, &Anh[j], &Anl[j]);

}

t2 = L_shr(t2, (Word16)4);		/* t2 = K in Q30->convert to Q26 */

L_extract(t2, &Anh[i], &Anl[i]);	/* An[i] in Q26                  */

/*  Alpha = Alpha * (1-K**2) */

t0 = mpy_32(Kh ,Kl, Kh, Kl);		/* K*K      in Q30 */

t0 = L_abs(t0);				/* Some case <0 !! */

t0 = L_sub( (Word32)0x3fffffff, t0 );	/* 1 - K*K  in Q30 */

L_extract(t0, &hi, &lo);			/* DPF format      */

t0 = mpy_32(alp_h , alp_l, hi, lo);	/* Alpha in Q30    */

/* Normalize Alpha */

t0 = norm_v(t0, (Word16)12, &j);

t0 = L_shr(t0, (Word16)1);

L_extract(t0, &alp_h, &alp_l);		/* DPF format    */

alp_e += j-1;					/* t0 was in Q30 */

/* A[j] = An[j]   Note: can be done with pointers */

for(j=1; j<=i; j++)

{

Ah[j] =Anh[j];

Al[j] =Anl[j];

}

}

/* Troncate A[i] in Q26 to Q12 with rounding */

A[0] = 4096;

for(i=1; i<=pp; i++)

{

t0   = L_comp(Ah[i], Al[i]);

t0   = add_sh(t0, (Word16)1, (Word16)13);	/* rounding */

old_A[i] = A[i] = store_hi(t0,(Word16)2);

}

return;

}

/**************************************************************************

*

*	ROUTINE				:	Lpc_Gain

*

*	DESCRIPTION			:	Compute energy of impulse response of 1/A(z)

*							on 60 points

*

**************************************************************************

*

*	USAGE				:	Lpc_Gain(buffer_in)

*							(Routine_Name(input1))

*

*	INPUT ARGUMENT(S)		:

*

*	INPUT1			:	- Description : LPC coefficients

*						- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:	None

*

*	RETURNED VALUE		:	- Description : Energy of impulse response of 1/A(z)

*							- Format : Word32 - Q20

*

**************************************************************************/

Word32 Lpc_Gain(Word16 a[])

{

Word16 i;

Word32 ener;

Word16 h[llg];

/* Compute the impulse response of A(z) */

h[0] = 1024;				/* 1.0 in Q10 */

for(i=1; i<llg; i++) h[i]=0;

Syn_Filt(a, h, h, llg, &h[1], (Word16)0);

/* Compute the energy of the impulse response */

ener = 0;

for(i=0; i<llg; i++)

ener = L_mac0(ener, h[i], h[i]);

return(ener);

}

/**************************************************************************

*

*	ROUTINE				:	Lsp_Az

*

*	DESCRIPTION			:	Compute the LPC coefficients

*							from the LSPs in the cosine domain

*							(order=10)

*

**************************************************************************

*

*	USAGE				:	Lsp_Az(buffer_in,buffer_out)

*							(Routine_Name(input1,output1))

*

*	INPUT ARGUMENT(S)		:

*

*		INPUT1			:	- Description : Line spectral pairs in the

*								          cosine domain

*						- Format : Word16 - Q15

*

*	OUTPUT ARGUMENT(S)		:

*

*	OUTPUT1			:	- Description : Predictor coefficients

*							- Format : Word16 - Q12

*

*	RETURNED VALUE		:	None

*

**************************************************************************/

void Lsp_Az(Word16 lsp[], Word16 a[])

{

Word16 i, j;

Word32 f1[6], f2[6];

Word32 t0;

Get_Lsp_Pol(&lsp[0],f1);

Get_Lsp_Pol(&lsp[1],f2);

for (i = 5; i > 0; i--)

{

f1[i] = L_add(f1[i], f1[i-1]);			/* f1[i] += f1[i-1]; */

f2[i] = L_sub(f2[i], f2[i-1]);			/* f2[i] -= f2[i-1]; */

}

a[0] = 4096;

for (i = 1, j = 10; i <= 5; i++, j--)

{

t0   = L_add(f1[i], f2[i]);			/* f1[i] + f2[i] */

a[i] = extract_l( L_shr_r(t0,(Word16)13) );	/*from Q24 to Q12 and * 0.5*/

t0   = L_sub(f1[i], f2[i]);			/* f1[i] - f2[i] */

a[j] = extract_l( L_shr_r(t0,(Word16)13) );	/*from Q24 to Q12 and * 0.5*/

}

return;

}

/**************************************************************************

*

*	ROUTINE				:	Pond_Ai

*

*	DESCRIPTION			:	Compute spectral expansion (a_exp[]) of LPC

*							coefficients (a[]) using spectral expansion

*							factors (fac[]) with the LPC order fixed to 10

*

**************************************************************************

*

*	USAGE				:	Pond_Ai(buffer_in1,buffer_in2,buffer_out)

*							(Routine_Name(input1,input2,output1))

*

*	INPUT ARGUMENT(S)		:

*

*	INPUT1			:	- Description : LPC coefficients

*						- Format : Word16

*

*	INPUT2			:	- Description : Spectral expansion factors

*						- Format : Word16 - Q15

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : Spectral expanded LPC coefficients

*							- Format : Word16

*

*	RETURNED VALUE		:	None

*

*	COMMENTS			:	- a_exp[i] = a[i] * fac[i-1]	i=1,10

*

**************************************************************************/

void Pond_Ai(Word16 a[], Word16 fac[], Word16 a_exp[])

{

Word16 i;

a_exp[0] = a[0];

for(i=1; i<=pp; i++)

a_exp[i] = round( L_mult(a[i], fac[i-1]) );

return;

}

/**************************************************************************

*

*	ROUTINE				:	Residu

*

*	DESCRIPTION			:	Compute the LPC residual  by filtering the input

*							speech through A(z)

*

**************************************************************************

*

*	USAGE				:	Residu(buffer_in1,buffer_in2,buffer_out,lg)

*							(Routine_Name(input1,input2,output1,input3))

*

*	INPUT ARGUMENT(S)		:

*

*	INPUT1			:	- Description : Prediction coefficients

*						- Format : Word16 - Q12

*

*	INPUT2			:	- Description : Input speech - values of buffer_in2[]

*							          from -p to -1 are needed

*						- Format : Word16

*

*	INPUT3			:	- Description : Size of filtering

*						- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : Residual signal

*							- Format : Word16

*

*	RETURNED VALUE		:	None

*

**************************************************************************/

void Residu(Word16 a[], Word16 x[], Word16 y[], Word16 lg)

{

Word16 i, j;

Word32 s;

for (i = 0; i < lg; i++)

{

s = Load_sh(x[i], (Word16)12);

for (j = 1; j <= pp; j++)

s = L_mac0(s, a[j], x[i-j]);

s = add_sh(s, (Word16)1, (Word16)11);		/* Rounding */

s = L_shl(s, (Word16)4);				/* Saturation */

y[i] = extract_h(s);

}

return;

}

/**************************************************************************

*

*	ROUTINE				:	Syn_Filt

*

*	DESCRIPTION			:	Perform the synthesis filtering 1/A(z)

*

**************************************************************************

*

*	USAGE				:	Syn_Filt(buffer_in1,buffer_in2,buffer_out,

*							lg,buffer,flag)

*							(Routine_Name(input1,input2,output1,

*							input3,arg4,input5))

*

*	INPUT ARGUMENT(S)		:

*

*	INPUT1			:	- Description : Prediction coefficients

*						- Format : Word16 - Q12

*

*	INPUT2			:	- Description : Input signal

*						- Format : Word16

*

*	INPUT3			:	- Description : Size of filtering

*						- Format : Word16

*

*	ARG4				:	- Description : Memory associated with this filtering

*						- Format : Word16

*

*		INPUT5			:	- Description :	- flag = 0  -> no update of memory

*									- flag = 1  -> update of memory

*							- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : Output signal

*							- Format : Word16

*

*	RETURNED VALUE		:	None

*

**************************************************************************/

void Syn_Filt(Word16 a[], Word16 x[], Word16 y[], Word16 lg, Word16 mem[],

Word16 update)

{

Word16 i, j;

Word32 s;

Word16 tmp[80];	/* This is usually done by memory allocation (lg+pp) */

Word16 *yy;

/* Copy mem[] to yy[] */

yy = tmp;

for(i=0; i<pp; i++)

*yy++ = mem[i];

/* Do the filtering. */

for (i = 0; i < lg; i++)

{

s = Load_sh(x[i], (Word16)12);				/* a[] is in Q12 */

for (j = 1; j <= pp; j++)

s = L_msu0(s, a[j], yy[-j]);

s     = add_sh(s, (Word16)1, (Word16)11);		/* Rounding */

*yy++ = extract_h( L_shl(s, (Word16)4) );

}

for(i=0; i<lg; i++) y[i] = tmp[i+pp];

/* Update of memory if update==1 */

if(update != 0)

for (i = 0; i < pp; i++) mem[i] = y[lg-pp+i];

return;

}

## A.5	Library of Basic and Mathematic Functions

### A.5.1	Basic Functions

/************************************************************************

*

*	FILENAME		:	fbas_tet.c

*

*	DESCRIPTION		:	Library of basic functions used in the TETRA speech

*					codec, other than accepted operators

*

************************************************************************

*

*	FUNCTIONS		:	- add_sh()

*					- add_sh16()

*					- bin2int()

*					- int2bin()

*					- Load_sh()

*					- Load_sh16()

*					- norm_v()

*					- store _hi()

*					- sub_sh()

*					- sub_sh16()

*

************************************************************************

*

*	INCLUDED FILES	:	source.h

*

************************************************************************/

#include "source.h"

static Word16 POW2[16] = { -1, -2, -4, -8, -16, -32, -64, -128, -256, -512,

-1024, -2048, -4096, -8192, -16384, -32768};

/************************************************************************

*

*	Function Name : add_sh

*

*	Purpose :

*

*		Add var1 with a left shift(0-15) to L_var2.

*		Control saturation and set overflow flag

*

*	Complexity Weight : 1

*

*	Inputs :

*

*		L_var2

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var2 <= 0x7fff ffff.

*

*		var1

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var1 <= 0x0000 7fff.

*

*		shift

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0 <= shift <= 15.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_var_out

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var_out <= 0x7fff ffff.

*

************************************************************************/

Word32 add_sh(Word32 L_var2, Word16 var1, Word16 shift)

{

return( L_msu0(L_var2, var1, POW2[shift]));

}

/************************************************************************

*

*	Function Name : add_sh16

*

*	Purpose :

*

*		Add var1 with a left shift of 16 to L_var2.

*		Control saturation and set overflow flag

*

*	Complexity Weight : 1

*

*	Inputs :

*

*		L_var2

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var2 <= 0x7fff ffff.

*

*		var1

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var1 <= 0x0000 7fff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_var_out

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var_out <= 0x7fff ffff.

*

************************************************************************/

Word32 add_sh16(Word32 L_var2, Word16 var1)

{

return( L_msu(L_var2, var1, (Word16)-32768));

}

/************************************************************************

*

*	Function Name : bin2int

*

*	Purpose :

*

*		Read "no_of_bits" bits from the array bitstream[] and convert to integer

*

*	Inputs :

*

*		no_of_bits

*			16 bit

*

*		*bitstream

*			16 bit

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		value

*			32 bit

*

************************************************************************/

#define BIT_1  1

Word16 bin2int(Word16 no_of_bits, Word16 *bitstream)

{

Word16 value, i, bit;

value = 0;

for (i = 0; i < no_of_bits; i++)

{

value = shl( value,(Word16)1 );

bit = *bitstream++;

if (bit == BIT_1)  value += 1;

}

return(value);

}

/************************************************************************

*

*	Function Name : int2bin

*

*	Purpose :

*

*		Convert integer to binary and write the bits to the array bitstream []

*

*	Inputs :

*

*		no_of_bits

*			16 bit

*

*		*bitstream

*			16 bit

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		value

*			32 bit

*

************************************************************************/

#define BIT_0     0

#define BIT_1     1

#define MASK      1

void int2bin(Word16 value, Word16 no_of_bits, Word16 *bitstream)

{

Word16 *pt_bitstream, i, bit;

pt_bitstream = bitstream + no_of_bits;

for (i = 0; i < no_of_bits; i++)

{

bit = value & MASK;

if (bit == 0)

*--pt_bitstream = BIT_0;

else

*--pt_bitstream = BIT_1;

value = shr( value,(Word16)1 );

}

}

/************************************************************************

*

*	Function Name : Load_sh

*

*	Purpose :

*

*		Load the 16 bit var1 left shift(0-15) into the 32 bit output.

*		MS bits of the output are sign extended.

*

*	Complexity Weight : 1

*

*	Inputs :

*

*		var1

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var1 <= 0x0000 7fff.

*

*		shift

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0 <= var1 <= 15.

*

*	Outputs :

*

*		none.

*

*	Returned Value :

*

*		L_var_out

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var_out <= 0x7fff ffff.

*

************************************************************************/

Word32 Load_sh(Word16 var1, Word16 shift)

{

return( L_msu0( (Word32)0, var1, POW2[shift]));

}

/************************************************************************

*

*	Function Name : Load_sh16

*

*	Purpose :

*

*		Load the 16 bit var1 with a left shift of 16 into the 32 bit output.

*

*	Complexity Weight : 1

*

*	Inputs :

*

*		var1

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var1 <= 0x0000 7fff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_var_out

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var_out <= 0x7fff ffff.

*

************************************************************************/

Word32 Load_sh16(Word16 var1)

{

return( L_msu( (Word32)0, var1, (Word16)-32768));

}

/************************************************************************

*

*	Function Name : norm_v

*

*	Purpose :

*

*		Variable normalisation of a 32 bit integer (L_var3).

*		var1 gives the maximum number of left shift to be done

*		*var2 returns the actual number of left shift

*

*	Complexity Weight : 37

*

*	Inputs :

*

*		L_var3

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var3 <= 0x7fff ffff.

*

*		var1

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0 <= var1 <= 15.

*

*	Outputs :

*

*		*var2

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0 <= *var2 <= 15.

*

*	Returned Value :

*

*		L_var_out

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var_out <= 0x7fff ffff.

*

************************************************************************/

Word32 norm_v(Word32 L_var3, Word16 var1, Word16 *var2)

{

Word16 shift;

shift = norm_l(L_var3);

if(sub(shift, var1) > 0) shift = var1;

*var2 = shift;

return(L_shl(L_var3, shift));

}

/************************************************************************

*

*	Function Name : store_hi

*

*	Purpose :

*

*		Store high part of a L_var1 with a left shift of var2.

*

*	Complexity Weight : 3

*

*	Inputs :

*

*		L_var1

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var1 <= 0x7fff ffff.

*

*		var2

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0 <= var2 <= 7.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		var_out

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var_out <= 0x0000 7fff.

*

************************************************************************/

Word16 store_hi(Word32 L_var1, Word16 var2)

{

static Word16 SHR[8]={16, 15, 14, 13, 12, 11, 10, 9};

return(extract_l( L_shr(L_var1, SHR[var2])));

}

/************************************************************************

*

*	Function Name : sub_sh

*

*	Purpose :

*

*		Subtract var1 with a left shift(0-15) to L_var2.

*		Control saturation and set overflow_flag.

*

*	Complexity Weight : 1

*

*	Inputs :

*

*		L_var2

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var2 <= 0x7fff ffff.

*

*		var1

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var1 <= 0x0000 7fff.

*

*		shift

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0 <= shift <= 15.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_var_out

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var_out <= 0x7fff ffff.

*

************************************************************************/

Word32 sub_sh(Word32 L_var2, Word16 var1, Word16 shift)

{

return( L_mac0(L_var2, var1, POW2[shift]));

}

/************************************************************************

*

*	Function Name : sub_sh16

*

*	Purpose :

*

*		Subtract var1 with a left shift of 16 to L_var2.

*		Control saturation and set overflow flag.

*

*	Complexity Weight : 1

*

*	Inputs :

*

*		L_var2

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var2 <= 0x7fff ffff.

*

*		var1

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var1 <= 0x0000 7fff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_var_out

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var_out <= 0x7fff ffff.

************************************************************************/

Word32 sub_sh16(Word32 L_var2, Word16 var1)

{

return( L_mac(L_var2, var1, (Word16)-32768));

}

### A.5.2	Extended Precision Functions

/************************************************************************

*

*	FILENAME		:	fexp_tet.c

*

*	DESCRIPTION		:	Library of special functions for operations in

*					extended precision used in the TETRA speech codec

*

************************************************************************

*

*	FUNCTIONS		:	- L_comp()

*					- L_extract()

*					- mpy_mix()

*					- mpy_32()

*					- div_32()

*

************************************************************************

*

*	INCLUDED FILES	:	source.h

*

************************************************************************

*

*	COMMENTS		:	This subsection contains operations in double precision.

*					These operations are non standard double precision

*					operations.

*

*					They are used where single precision is not enough but the

*					full 32 bits precision is not necessary. For example, the

*					function div_32() has a 24 bits precision.

*

*					The double precision numbers use a special representation :

*

*					L_32 = hi<<15 + lo

*

*					L_32 is a 32 bit integer with b30 == b31.

*					hi and lo are 16 bit signed integers.

*					As the low part also contains the sign, this allows fast

*					multiplication.

*

*					0xc000 0000 <= L_32 <= 0x3fff ffff.

*

*					In general, DPF is used to specify this special

*					format.

*

************************************************************************/

#include "source.h"

/************************************************************************

*

*	Function Name : L_comp

*

*	Purpose :

*

*		Compose from two 16 bit DPF a normal 32 bit integer.

*		L_32 = hi<<15 + lo

*

*	Complexity Weight : 2

*

*	Inputs :

*

*		hi

*			msb

*

*		lo

*			lsb (with sign)

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_32

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0xc000 0000 <= L_32 <= 0x3fff ffff.

*

************************************************************************/

Word32 L_comp(Word16 hi, Word16 lo)

{

return(add_sh( Load_sh( lo,(Word16)0 ), hi, (Word16)15 ));

}

/************************************************************************

*

*	Function Name : L_extract

*

*	Purpose :

*

*		Extract from a 31 bit integer two 16 bit DPF.

*

*	Complexity Weight : 5

*

*	Inputs :

*

*		L_32

*			32 bit long signed integer (Word32) with b30 == b31

*			whose value falls in the range : 0xc000 0000 <= L_32 <= 0x3fff ffff.

*

*	Outputs :

*

*		hi

*			b15 to b30 of L_32

*

*		lo

*			L_32 - hi<<15

*

*	Returned Value :

*

*		none

*

************************************************************************/

void L_extract(Word32 L_32, Word16 *hi, Word16 *lo)

{

*hi  = extract_h( L_shl( L_32,(Word16)1 ) );

*lo  = extract_l( sub_sh( L_32, *hi, (Word16)15 ) );

return;

}

/************************************************************************

*

*	Function Name : mpy_mix

*

*	Purpose :

*

*		Multiply a 16 bit integer by a 32 bit (DPF).

*		The result is divided by 2**16

*		L_32 = hi1*lo2 + (lo1*lo2)>>15

*

*	Complexity Weight : 4

*

*	Inputs :

*

*		hi1

*			hi part of 32 bit number

*

*		lo1

*			lo part of 32 bit number

*

*		lo2

*			16 bit number

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_var_out

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x0000 0000 <= L_var_out <= 0x7fff ffff.

*

************************************************************************/

Word32 mpy_mix(Word16 hi1, Word16 lo1, Word16 lo2)

{

Word16 p1;

Word32 L_32;

p1   = extract_h(L_mult0(lo1, lo2));

L_32 = L_mult0(hi1,lo2 );

return(add_sh( L_32, p1, (Word16)1 ));

}

/************************************************************************

*

*	Function Name : mpy_32

*

*	Purpose :

*

*		Multiply two 32 bit integers (DPF). The result is divided by 2**32

*		L_32 = hi1*hi2 + (hi1*lo2)>>15 + (lo1*hi2)>>15)

*

*	Complexity Weight : 7

*

*	Inputs :

*

*		hi1

*			hi part of first number

*

*		lo1

*			lo part of first number

*

*		hi2

*			hi part of second number

*

*		lo2

*			lo part of second number

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_var_out

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x0000 0000 <= L_var_out <= 0x7fff ffff.

*

*************************************************************************/

Word32 mpy_32(Word16 hi1, Word16 lo1, Word16 hi2, Word16 lo2)

{

Word16 p1, p2;

Word32 L_32;

p1   = extract_h(L_mult0(hi1, lo2));

p2   = extract_h(L_mult0(lo1, hi2));

L_32 = L_mult0(hi1, hi2);

L_32 = add_sh( L_32, p1, (Word16)1 );

return(add_sh( L_32, p2, (Word16)1 ));

}

/************************************************************************

*

*	Function Name : div_32

*

*	Purpose :

*

*		Fractionnal integer division of two 32 bit numbers.

*		L_num / L_denom

*		L_num and L_denom must be positive and L_num < L_denom

*		L_denom = denom_hi<<15 + denom_lo

*		denom_hi is a normalized number

*		The result is in Q30

*

*	Complexity Weight : 52

*

*	Inputs :

*

*		L_num

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x0000 0000 <= L_num <= L_denom.

*

*		(L_denom = denom_hi<<15 + denom_lo)

*

*		denom_hi

*			16 bit normalized integer whose value falls in the

*			range : 0x4000000 < hi < 0x7fff ffff.

*

*		denom_lo

*			16 bit positive integer whose value falls in the

*			range : 0 < lo < 0x7fff ffff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_div

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x0000 0000 <= L_div <= 0x3fff ffff.

*			L_div is a Q30 value (point between b30 and b29)

*

*	Algorithm :

*

*		 - find = 1/L_denom

*			First approximation: approx = 1 / denom_hi

*			1/L_denom = approx * (2.0 - L_denom * approx )

*		-  result = L_num * (1/L_denom)

*

************************************************************************/

Word32 div_32(Word32 L_num, Word16 denom_hi, Word16 denom_lo)

{

Word16 approx, hi, lo, n_hi, n_lo;

Word32 t0;

/* First approximation: 1 / L_denom = 1/denom_hi */

approx = div_s( (Word16)0x3fff, denom_hi);	/* result in Q15 */

/* 1/L_denom = approx * (2.0 - L_denom * approx) */

t0 = mpy_mix(denom_hi, denom_lo, approx);	/* result in Q29 */

t0 = L_sub( (Word32)0x40000000, t0);		/* result in Q29 */

L_extract(t0, &hi, &lo);

t0 = mpy_mix(hi, lo, approx);			/* = 1/L_denom in Q28 */

/* L_num * (1/L_denom) */

L_extract(t0, &hi, &lo);

L_extract(L_num, &n_hi, &n_lo);

t0 = mpy_32(n_hi, n_lo, hi, lo);

return( L_shl( t0,(Word16)2) );			/* From Q28 to Q30 */

}

### A.5.3	Mathematic Functions

/************************************************************************

*

*	FILENAME		:	fmat_tet.c

*

*	DESCRIPTION		:	Library of mathematic functions used in the TETRA codec

*

************************************************************************

*

*	FUNCTIONS		:	- inv_sqrt()

*					- Log2()

*					- pow2()

*

************************************************************************

*

*	INCLUDED FILES	:	source.h

*

*					inv_sqrt.tab

*					log2.tab

*					pow2.tab

*

************************************************************************/

#include "source.h"

#include "inv_sqrt.tab"         /* Table for inv_sqrt() */

#include "log2.tab"             /* Table for Log2() */

#include "pow2.tab"             /* Table for pow2() */

/************************************************************************

*

*	Function Name : inv_sqrt

*

*	Purpose :

*

*		Compute 1/sqrt(L_x).

*		L_x is positive. The result is in Q30.

*

*	Complexity Weight : 56

*

*	Inputs :

*

*		L_x

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x0000 0000 <= L_x <= 0x7fff ffff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_y

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x0000 0000 <= L_y <= 0x3fff ffff.

*			L_y is a Q30 value (point between b30 and b29)

*

*	Algorithm :

*

*		The function 1/sqrt(L_x) is approximated by a table (tab_inv_sqrt)

*		and linear interpolation :

*

*			1 - Normalization of L_x

*			2 - If (30-exponant) is even then shift right once

*			3 - exponant = (30-exponant)/2  +1

*			4 - i = bit25-b31 of L_x,    16 <= i <= 63  ->because of normalization

*			5 - a = bit10-b24

*			6 - i -=16

*			7 - L_y = tab_inv_sqrt[i]<<16 - (tab_inv_sqrt[i] - tab_inv_sqrt[i+1]) * a * 2

*			8 - L_y >>= exponant

*

************************************************************************/

Word32 inv_sqrt(Word32 L_x)

{

Word16 exp, i, a, tmp;

Word32 L_y;

if( L_x <= (Word32)0) return ( (Word32)0x3fffffff);

exp = norm_l(L_x);

L_x = L_shl(L_x, exp );		/* L_x is normalized */

exp = sub( (Word16)30, exp );

if( (exp & 1) == 0 )			/* If exponant even -> shift right */

L_x = L_shr( L_x, (Word16)1 );

exp = shr( exp, (Word16)1 );

exp = add( exp, (Word16)1 );

L_x = L_shr( L_x, (Word16)9 );

i   = extract_h(L_x);			/* Extract b25-b31 */

L_x = L_shr( L_x, (Word16)1 );

a   = extract_l(L_x);			/* Extract b10-b24 */

a   = a & (Word16)0x7fff;

i   = sub( i, (Word16)16 );

L_y = L_deposit_h(tab_inv_sqrt[i]);	/* tab_inv_sqrt[i] << 16 */

tmp = sub(tab_inv_sqrt[i], tab_inv_sqrt[i+1]);

/* tab_inv_sqrt[i] - tab_inv_sqrt[i+1])*/

L_y = L_msu(L_y, tmp, a);			/* L_y -=  tmp*a*2 */

L_y = L_shr(L_y, exp);		/* denormalization */

return(L_y);

}

/************************************************************************

*

*	Function Name : Log2

*

*	Purpose :

*

*		Compute Log2(L_x).

*		L_x is positive.

*

*	Complexity Weight : 48

*

*	Inputs :

*

*		L_x

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x0000 0000 <= L_x <= 0x7fff ffff.

*

*	Outputs :

*

*		exponant

*			Integer part of Log2()

*			16 bit  signed integer (Word16) whose value falls in the

*			range :   0 <= exponant <= 30

*

*		fraction

*			Fractional part of Log2()

*			16 bit signed integer (Word16) whose value falls in the

*			range : 0x0000 0000 <= fraction <= 0x7fff.

*			It's a Q15 value (point between b15 and b16).

*

*	Returned Value :

*

*		none

*

*	Algorithm :

*

*		The function Log2(L_x) is approximated by a table (tab_log2)

*		and linear interpolation :

*

*			1 - Normalization of L_x

*			2 - exponant = 30-exponant

*			3 - i = bit25-b31 of L_x,    32 <= i <= 63  ->because of normalization

*			4 - a = bit10-b24

*			5 - i -=32

*			6 - fraction = tab_log2[i]<<16 - (tab_log2[i] - tab_log2[i+1]) * a * 2

*

************************************************************************/

void Log2(Word32 L_x, Word16 *exponant, Word16 *fraction)

{

Word16 exp, i, a, tmp;

Word32 L_y;

if( L_x <= (Word32)0 )

{

*exponant = 0;

*fraction = 0;

return;

}

exp = norm_l(L_x);

L_x = L_shl(L_x, exp );			/* L_x is normalized */

*exponant = sub( (Word16)30, exp );

L_x = L_shr( L_x, (Word16)9 );

i   = extract_h(L_x);				/* Extract b25-b31 */

L_x = L_shr( L_x, (Word16)1 );

a   = extract_l(L_x);				/* Extract b10-b24 of fraction */

a   = a & (Word16)0x7fff;

i   = sub( i, (Word16)32 );

L_y = L_deposit_h(tab_log2[i]);		/* tab_log2[i] << 16 */

tmp = sub(tab_log2[i], tab_log2[i+1]);	/* tab_log2[i] - tab_log2[i+1] */

L_y = L_msu(L_y, tmp, a);			/* L_y -= tmp*a*2 */

*fraction = extract_h( L_y);

return;

}

/************************************************************************

*

*	Function Name : pow2

*

*	Purpose :

*

*		L_x = pow(2.0, exponant.fraction).

*

*	Complexity Weight : 17

*

*	Inputs :

*

*		exponant

*			Integer part

*			16 bit  signed integer (Word16) whose value falls in the

*			range :   0 <= exponant <= 30

*

*		fraction

*			Fractional part

*			16 bit signed integer (Word16) whose value falls in the

*			range : 0x0000 0000 <= fraction <= 0x7fff.

*			It's a Q15 value (point between b15 and b16).

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_x

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x0000 0000 <= L_x <= 0x7fff ffff.

*

*	Algorithm :

*

*		The function pow2(L_x) is approximated by a table (tab_pow2)

*		and linear interpolation :

*

*			1 - i = bit11-b16 of fraction,   0 <= i <= 31

*			2 - a = bit0-b10  of fraction

*			3 - L_x = tab_pow2[i]<<16 - (tab_pow2[i] - tab_pow2[i+1]) * a * 2

*			4 - L_x = L_x >> (30-exponant)     (with rounding)

*

************************************************************************/

Word32 pow2(Word16 exponant, Word16 fraction)

{

Word16 exp, i, a, tmp;

Word32 L_x;

L_x = L_deposit_l(fraction);

L_x = L_shl( L_x, (Word16)6 );

i   = extract_h(L_x);				/* Extract b10-b16 of fraction */

L_x = L_shr( L_x, (Word16)1 );

a   = extract_l(L_x);				/* Extract b0-b9   of fraction */

a   = a & (Word16)0x7fff;

L_x = L_deposit_h(tab_pow2[i]);		/* tab_pow2[i] << 16 */

tmp = sub(tab_pow2[i], tab_pow2[i+1]);	/* tab_pow2[i] - tab_pow2[i+1] */

L_x = L_msu(L_x, tmp, a);			/* L_x -= tmp*a*2  */

exp = sub( (Word16)30, exponant );

L_x = L_shr_r(L_x, exp);

return(L_x);

}

## A.6	Tables and Constants for Source Coding

/************************************************************************

*

*	FILENAME		:	source.h

*

*	DESCRIPTION		:	DEFINITIONS

*

*					FUNCTION PROTOTYPES for the TETRA

*					speech source coder and decoder

*

*					OPERATOR PROTOTYPES for TETRA codec

*

************************************************************************/

/************************************************************************

*

*      DEFINITIONS

*

************************************************************************/

#ifndef TYPEDEF_H

#define TYPEDEF_H

typedef short Word16;

typedef long  Word32;

typedef int   Flag;

#endif

/************************************************************************

*

*	DESCRIPTION		:	FUNCTION PROTOTYPES for the TETRA

*					speech source coder and decoder

*

************************************************************************/

#ifndef TETRA_H

#define TETRA_H

/* Basic functions */

Word32 add_sh(Word32 L_var, Word16 var1, Word16 shift);

Word32 add_sh16(Word32 L_var, Word16 var1);

Word16 bin2int(Word16 no_of_bits, Word16 *bitstream);

void   int2bin(Word16 value, Word16 no_of_bits, Word16 *bitstream);

Word32 Load_sh(Word16 var1, Word16 shift);

Word32 Load_sh16(Word16 var1);

Word32 norm_v(Word32 L_var3, Word16 var1, Word16 *var2);

Word16 store_hi(Word32 L_var1, Word16 var2);

Word32 sub_sh(Word32 L_var, Word16 var1, Word16 shift);

Word32 sub_sh16(Word32 L_var, Word16 var1);

/* Extended precision functions */

Word32 div_32(Word32 L_num, Word16 hi, Word16 lo);

Word32 L_comp(Word16 hi, Word16 lo);

void   L_extract(Word32 L_32, Word16 *hi, Word16 *lo);

Word32 mpy_mix(Word16 hi1, Word16 lo1, Word16 lo2);

Word32 mpy_32(Word16 hi1, Word16 lo1, Word16 hi2, Word16 lo2);

/* Mathematic functions  */

Word32 inv_sqrt(Word32 L_x);

void   Log2(Word32 L_x, Word16 *exponant, Word16 *fraction);

Word32 pow2(Word16 exponant, Word16 fraction);

/* General signal processing */

void   Autocorr(Word16 x[], Word16 p, Word16 r_h[], Word16 r_l[]);

void   Az_Lsp(Word16 a[], Word16 lsp[], Word16 old_lsp[]);

void   Back_Fil(Word16 x[], Word16 h[], Word16 y[], Word16 L);

Word16 Chebps(Word16 x, Word16 f[], Word16 n);

void   Convolve(Word16 x[], Word16 h[], Word16 y[], Word16 L);

void   Fac_Pond(Word16 gamma, Word16 fac[]);

void   Get_Lsp_Pol(Word16 *lsp, Word32 *f);

void   Int_Lpc4(Word16 lsp_old[], Word16 lsp_new[], Word16 a_4[]);

void   Lag_Window(Word16 p, Word16 r_h[], Word16 r_l[]);

void   Levin_32(Word16 Rh[], Word16 Rl[], Word16 A[]);

Word32 Lpc_Gain(Word16 a[]);

void   Lsp_Az(Word16 lsp[], Word16 a[]);

void   Pond_Ai(Word16 a[], Word16 fac[], Word16 a_exp[]);

void   Residu(Word16 a[], Word16 x[], Word16 y[], Word16 lg);

void   Syn_Filt(Word16 a[], Word16 x[], Word16 y[], Word16 lg, Word16 mem[],

Word16 update);

/* Specific coder functions */

void   Init_Coder_Tetra(void);

void   Coder_Tetra(Word16 ana[], Word16 synth[]);

void   Cal_Rr2(Word16 h[], Word16 *rr);

void   Clsp_334(Word16 *lsp, Word16 *lsp_q, Word16 *indice);

Word16 D4i60_16(Word16 dn[], Word16 f[], Word16 h[], Word16 rr[][32],

Word16 cod[], Word16 y[], Word16 *sign, Word16 *shift_code);

Word16 Ener_Qua(Word16 A[], Word16 prd_lt[], Word16 code[], Word16 L_subfr,

Word16 *gain_pit, Word16 *gain_cod);

Word16 G_Code(Word16 xn2[], Word16 y2[], Word16 L_subfr);

Word16 G_Pitch(Word16 xn[], Word16 y1[], Word16 L_subfr);

void   Init_Pre_Process(void);

Word16 Lag_Max(Word16 signal[], Word16 sig_dec[], Word16 L_frame,

Word16 lag_max, Word16 lag_min, Word16 *cor_max);

Word16 Pitch_Fr(Word16 exc[], Word16 xn[], Word16 h[], Word16 L_subfr,

Word16 t0_min, Word16 t0_max, Word16 i_subfr,

Word16 *pit_frac);

Word16 Pitch_Ol_Dec(Word16 signal[], Word16 L_frame);

void   Pred_Lt(Word16 exc[], Word16 T0, Word16 frac, Word16 L_subfr);

void   Pre_Process(Word16 signal[], Word16 lg);

void   Prm2bits_Tetra(Word16 prm[], Word16 bits[]);

/* Specific decoder functions */

void   Init_Decod_Tetra(void);

void   Decod_Tetra(Word16 parm[], Word16 synth[]);

void   Bits2prm_Tetra(Word16 bits[], Word16 prm[]);

Word16 Dec_Ener(Word16 index, Word16 bfi, Word16 A[], Word16 prd_lt[],

Word16 code[], Word16 L_subfr, Word16 *gain_pit, Word16 *gain_cod);

void   D_D4i60(Word16 index,Word16 sign,Word16 shift, Word16 F[],

Word16 cod[]);

void   D_Lsp334(Word16 indice[], Word16 lsp[], Word16 old_lsp[]);

void   Post_Process(Word16 signal[], Word16 lg);

#endif

/************************************************************************

*

*	DESCRIPTION		:     OPERATOR PROTOTYPES for TETRA codec

*

************************************************************************/

#ifndef TETRA_OP_H

#define TETRA_OP_H

/*-----------------------*

* Constants and Globals *

*-----------------------*/

extern Flag Overflow;

extern Flag Carry;

#define MAX_32 (Word32)0x7fffffff

#define MIN_32 (Word32)0x80000000

#define MAX_16 (Word16)0x7fff

#define MIN_16 (Word16)0x8000

/*-----------------------*

* Operators prototypes  *

*-----------------------*/

Word16 abs_s(Word16 var1);                /* Short abs,           1 */

Word16 add(Word16 var1, Word16 var2);     /* Short add,           1 */

Word16 div_s(Word16 var1, Word16 var2);   /* Short division,     18 */

Word16 extract_h(Word32 L_var1);          /* Extract high,        1 */

Word16 extract_l(Word32 L_var1);          /* Extract low,         1 */

Word16 mult(Word16 var1, Word16 var2);    /* Short mult,          1 */

Word16 mult_r(Word16 var1, Word16 var2);  /* Mult with round,     2 */

Word16 negate(Word16 var1);               /* Short negate,        1 */

Word16 norm_l(Word32 L_var1);             /* Long norm,          30 */

Word16 norm_s(Word16 var1);               /* Short norm,         15 */

Word16 round(Word32 L_var1);              /* Round,               1 */

Word16 shl(Word16 var1, Word16 var2);     /* Short shift left,    1 */

Word16 shr(Word16 var1, Word16 var2);     /* Short shift right,   1 */

Word16 sub(Word16 var1, Word16 var2);     /* Short sub,           1 */

Word32 L_abs(Word32 L_var1);              /* Long abs,            3 */

Word32 L_add(Word32 L_var1, Word32 L_var2);  /* Long add,         2 */

Word32 L_deposit_h(Word16 var1);          /* 16 bit var1 -> MSB   2 */

Word32 L_deposit_l(Word16 var1);          /* 16 bit var1 -> LSB,  2 */

Word32 L_mac(Word32 L_var3, Word16 var1, Word16 var2);  /* Mac,   1 */

Word32 L_mac0(Word32 L_var3, Word16 var1, Word16 var2); /* no shi 1 */

Word32 L_msu(Word32 L_var3, Word16 var1, Word16 var2);  /* Msu,   1 */

Word32 L_msu0(Word32 L_var3, Word16 var1, Word16 var2); /* no shi 1 */

Word32 L_mult(Word16 var1, Word16 var2);  /* Long mult,           1 */

Word32 L_mult0(Word16 var1, Word16 var2); /* Long mult no shift,  1 */

Word32 L_negate(Word32 L_var1);           /* Long negate,         2 */

Word32 L_shl(Word32 L_var1, Word16 var2); /* Long shift left,     2 */

Word32 L_shr(Word32 L_var1, Word16 var2); /* Long shift right,    2 */

Word32 L_shr_r(Word32 L_var1, Word16 var2);  /* L_shr with round, 3 */

Word32 L_sub(Word32 L_var1, Word32 L_var2);  /* Long sub,         2 */

#endif

/************************************************************************

*

*	FILENAME		:	clsp_334.tab

*

*	DESCRIPTION		:	LSP codebooks:  split VQ with 3 codebooks.

*					The LSPs are in the cosine domain (-1, 1) in Q15

*

* 					codebook    	vector dimension        number of vectors

*					~~~~~~~~   	~~~~~~~~~~~~~         ~~~~~~~~~~~~~~

*					        1			3		256	(8 bits)

*					        2			3		512	(9 bits)

*					        3			4		512	(9 bits)

*

************************************************************************/

#define dim_dic1		3

#define dim_dic2		3

#define dim_dic3		4

#define taille_dic1	256

#define taille_dic2	512

#define taille_dic3	512

/*-----------------------------------------*

* 1st codebook:   lsp0 to lsp2            *

*-----------------------------------------*/

static Word16 dico1_clsp[taille_dic1*dim_dic1] = {

16231 * 2,     16091 * 2,     15682 * 2,

16198 * 2,     16032 * 2,     15577 * 2,

16142 * 2,     15986 * 2,     15536 * 2,

16190 * 2,     15922 * 2,     15515 * 2,

16188 * 2,     16002 * 2,     15314 * 2,

16099 * 2,     15919 * 2,     15368 * 2,

16149 * 2,     15941 * 2,     15254 * 2,

16228 * 2,     16079 * 2,     15245 * 2,

16127 * 2,     15802 * 2,     15316 * 2,

16151 * 2,     15855 * 2,     15122 * 2,

16056 * 2,     15878 * 2,     15137 * 2,

16092 * 2,     15782 * 2,     15027 * 2,

16047 * 2,     15719 * 2,     15263 * 2,

16070 * 2,     15640 * 2,     15074 * 2,

16104 * 2,     15877 * 2,     14778 * 2,

16118 * 2,     15740 * 2,     14805 * 2,

15978 * 2,     15756 * 2,     14910 * 2,

16058 * 2,     15653 * 2,     14737 * 2,

16023 * 2,     15562 * 2,     14896 * 2,

15923 * 2,     15643 * 2,     15103 * 2,

15963 * 2,     15463 * 2,     15048 * 2,

15964 * 2,     15472 * 2,     14741 * 2,

16027 * 2,     15539 * 2,     14592 * 2,

15822 * 2,     15577 * 2,     14759 * 2,

15894 * 2,     15657 * 2,     14525 * 2,

15897 * 2,     15322 * 2,     14813 * 2,

15768 * 2,     15447 * 2,     14793 * 2,

15970 * 2,     15379 * 2,     14477 * 2,

15827 * 2,     15527 * 2,     14303 * 2,

15798 * 2,     15189 * 2,     14552 * 2,

15716 * 2,     15424 * 2,     14318 * 2,

15684 * 2,     15270 * 2,     14723 * 2,

15862 * 2,     15324 * 2,     14172 * 2,

15606 * 2,     15259 * 2,     14402 * 2,

15878 * 2,     15108 * 2,     14312 * 2,

15675 * 2,     15173 * 2,     14035 * 2,

15568 * 2,     15011 * 2,     14381 * 2,

15475 * 2,     15137 * 2,     14183 * 2,

15702 * 2,     15347 * 2,     13870 * 2,

15791 * 2,     15147 * 2,     13806 * 2,

15568 * 2,     15002 * 2,     13834 * 2,

15558 * 2,     15249 * 2,     13725 * 2,

15802 * 2,     14842 * 2,     14041 * 2,

15747 * 2,     14957 * 2,     13522 * 2,

15368 * 2,     14888 * 2,     13942 * 2,

15537 * 2,     14783 * 2,     13594 * 2,

15476 * 2,     15115 * 2,     13393 * 2,

15563 * 2,     14924 * 2,     13195 * 2,

15383 * 2,     14829 * 2,     13377 * 2,

15328 * 2,     15000 * 2,     13392 * 2,

15505 * 2,     14658 * 2,     13177 * 2,

15221 * 2,     14686 * 2,     13400 * 2,

15501 * 2,     14450 * 2,     13469 * 2,

15338 * 2,     14510 * 2,     13020 * 2,

15232 * 2,     14720 * 2,     12775 * 2,

15349 * 2,     14936 * 2,     12698 * 2,

15085 * 2,     14466 * 2,     13087 * 2,

15572 * 2,     14715 * 2,     12716 * 2,

15259 * 2,     14332 * 2,     12698 * 2,

15380 * 2,     14205 * 2,     13160 * 2,

15383 * 2,     14518 * 2,     12244 * 2,

15641 * 2,     14328 * 2,     12633 * 2,

15501 * 2,     14832 * 2,     12172 * 2,

15081 * 2,     14564 * 2,     12175 * 2,

15063 * 2,     14095 * 2,     12747 * 2,

15528 * 2,     14047 * 2,     12215 * 2,

15315 * 2,     14226 * 2,     11850 * 2,

14982 * 2,     14294 * 2,     12094 * 2,

15123 * 2,     13908 * 2,     12244 * 2,

15280 * 2,     14519 * 2,     11473 * 2,

15225 * 2,     14844 * 2,     11802 * 2,

15093 * 2,     13935 * 2,     11431 * 2,

15482 * 2,     13924 * 2,     11356 * 2,

15475 * 2,     14577 * 2,     11183 * 2,

15218 * 2,     14226 * 2,     10791 * 2,

14737 * 2,     14154 * 2,     11457 * 2,

14916 * 2,     14278 * 2,     10548 * 2,

15167 * 2,     14584 * 2,     10383 * 2,

15441 * 2,     14281 * 2,     10338 * 2,

15217 * 2,     13701 * 2,     10458 * 2,

14864 * 2,     13740 * 2,     10632 * 2,

15158 * 2,     13956 * 2,      9868 * 2,

15160 * 2,     14387 * 2,      9550 * 2,

14820 * 2,     13827 * 2,      9553 * 2,

15307 * 2,     14034 * 2,      9137 * 2,

15561 * 2,     13729 * 2,      9617 * 2,

15121 * 2,     13366 * 2,      9420 * 2,

14834 * 2,     13347 * 2,      9964 * 2,

15075 * 2,     13592 * 2,      8793 * 2,

15032 * 2,     14126 * 2,      8529 * 2,

14716 * 2,     13670 * 2,      8579 * 2,

15158 * 2,     13141 * 2,      8223 * 2,

14584 * 2,     13155 * 2,      8860 * 2,

14934 * 2,     12714 * 2,      8774 * 2,

14725 * 2,     12847 * 2,      9653 * 2,

14797 * 2,     12944 * 2,      7872 * 2,

15022 * 2,     13701 * 2,      7746 * 2,

14460 * 2,     12456 * 2,      8752 * 2,

14271 * 2,     12719 * 2,      7966 * 2,

14616 * 2,     12296 * 2,      7929 * 2,

14837 * 2,     11931 * 2,      9091 * 2,

15162 * 2,     12854 * 2,      7074 * 2,

14781 * 2,     13408 * 2,      7008 * 2,

14513 * 2,     12747 * 2,      6582 * 2,

14328 * 2,     11984 * 2,      6857 * 2,

14732 * 2,     11567 * 2,      7593 * 2,

14754 * 2,     11677 * 2,      6455 * 2,

14795 * 2,     12447 * 2,      5975 * 2,

13850 * 2,     11888 * 2,      7609 * 2,

14927 * 2,     13097 * 2,      5461 * 2,

14269 * 2,     11440 * 2,      5252 * 2,

14752 * 2,     11489 * 2,      4625 * 2,

15290 * 2,     11792 * 2,      5357 * 2,

14609 * 2,     12271 * 2,      4241 * 2,

14387 * 2,     11193 * 2,      3749 * 2,

14900 * 2,     12992 * 2,      3558 * 2,

14865 * 2,     12377 * 2,      2905 * 2,

14600 * 2,     11232 * 2,      2715 * 2,

15425 * 2,     10659 * 2,      3564 * 2,

14985 * 2,     13418 * 2,      4349 * 2,

15424 * 2,     13710 * 2,      2694 * 2,

15248 * 2,     13980 * 2,      4419 * 2,

15139 * 2,     13555 * 2,      6077 * 2,

15878 * 2,     13355 * 2,      4567 * 2,

15740 * 2,     13445 * 2,      6142 * 2,

15483 * 2,     14469 * 2,      5488 * 2,

15404 * 2,     13842 * 2,      7138 * 2,

15091 * 2,     14172 * 2,      6855 * 2,

15514 * 2,     14415 * 2,      7464 * 2,

15828 * 2,     14810 * 2,      6056 * 2,

15905 * 2,     14299 * 2,      7695 * 2,

15616 * 2,     13743 * 2,      8439 * 2,

15349 * 2,     14546 * 2,      8611 * 2,

15768 * 2,     15204 * 2,      7352 * 2,

15718 * 2,     14835 * 2,      8847 * 2,

15715 * 2,     14298 * 2,      9159 * 2,

15922 * 2,     15121 * 2,      8926 * 2,

15659 * 2,     15242 * 2,      9593 * 2,

15864 * 2,     14260 * 2,      9965 * 2,

15598 * 2,     14749 * 2,     10202 * 2,

15347 * 2,     14862 * 2,      9903 * 2,

15832 * 2,     14938 * 2,     10314 * 2,

16020 * 2,     15210 * 2,     10351 * 2,

15772 * 2,     14601 * 2,     11044 * 2,

15781 * 2,     15501 * 2,     10547 * 2,

15812 * 2,     15191 * 2,     11258 * 2,

15637 * 2,     14955 * 2,     11529 * 2,

15658 * 2,     15382 * 2,     11479 * 2,

15534 * 2,     15204 * 2,     11639 * 2,

15943 * 2,     14965 * 2,     11691 * 2,

15955 * 2,     15373 * 2,     11636 * 2,

15413 * 2,     15029 * 2,     11640 * 2,

15761 * 2,     15509 * 2,     12201 * 2,

15834 * 2,     15090 * 2,     12389 * 2,

15737 * 2,     15311 * 2,     12584 * 2,

15857 * 2,     15616 * 2,     11897 * 2,

15889 * 2,     15436 * 2,     12566 * 2,

16008 * 2,     15590 * 2,     12292 * 2,

16017 * 2,     15262 * 2,     12780 * 2,

15540 * 2,     15106 * 2,     12672 * 2,

15929 * 2,     15639 * 2,     12941 * 2,

15699 * 2,     15448 * 2,     13140 * 2,

15955 * 2,     15459 * 2,     13281 * 2,

15984 * 2,     15782 * 2,     12858 * 2,

15826 * 2,     15311 * 2,     13396 * 2,

15608 * 2,     15291 * 2,     13082 * 2,

15725 * 2,     15089 * 2,     13155 * 2,

15790 * 2,     15525 * 2,     13586 * 2,

15861 * 2,     15622 * 2,     13596 * 2,

15960 * 2,     15103 * 2,     13294 * 2,

15950 * 2,     15421 * 2,     13857 * 2,

15938 * 2,     15709 * 2,     13665 * 2,

16045 * 2,     15641 * 2,     13517 * 2,

16113 * 2,     15435 * 2,     13783 * 2,

15950 * 2,     15579 * 2,     14127 * 2,

16032 * 2,     15763 * 2,     13984 * 2,

16056 * 2,     15597 * 2,     14271 * 2,

16071 * 2,     15282 * 2,     14159 * 2,

15973 * 2,     15723 * 2,     14379 * 2,

16086 * 2,     15755 * 2,     14429 * 2,

16119 * 2,     15881 * 2,     14090 * 2,

16205 * 2,     15680 * 2,     14372 * 2,

16028 * 2,     15842 * 2,     14528 * 2,

16199 * 2,     15761 * 2,     13813 * 2,

16141 * 2,     15959 * 2,     14662 * 2,

16198 * 2,     16013 * 2,     14080 * 2,

16233 * 2,     16079 * 2,     14465 * 2,

16203 * 2,     15948 * 2,     14878 * 2,

16271 * 2,     16051 * 2,     13866 * 2,

16287 * 2,     16154 * 2,     14482 * 2,

16167 * 2,     15960 * 2,     13393 * 2,

16267 * 2,     15940 * 2,     13407 * 2,

16076 * 2,     15830 * 2,     13540 * 2,

16237 * 2,     15561 * 2,     13288 * 2,

16200 * 2,     15808 * 2,     12800 * 2,

16126 * 2,     15534 * 2,     13042 * 2,

16082 * 2,     15783 * 2,     12455 * 2,

16139 * 2,     15482 * 2,     12064 * 2,

16155 * 2,     15840 * 2,     11644 * 2,

16238 * 2,     16026 * 2,     11633 * 2,

16071 * 2,     15639 * 2,     11117 * 2,

16186 * 2,     15311 * 2,     11146 * 2,

15938 * 2,     15700 * 2,     10741 * 2,

15948 * 2,     15495 * 2,     10329 * 2,

16108 * 2,     15736 * 2,      9800 * 2,

16056 * 2,     14735 * 2,     11245 * 2,

15830 * 2,     15467 * 2,      9241 * 2,

16167 * 2,     15951 * 2,      8972 * 2,

16051 * 2,     15570 * 2,      8007 * 2,

15888 * 2,     15592 * 2,      7559 * 2,

15978 * 2,     15368 * 2,      7032 * 2,

15846 * 2,     15467 * 2,      5711 * 2,

16014 * 2,     15082 * 2,      4645 * 2,

15831 * 2,     14913 * 2,      3458 * 2,

15755 * 2,     13822 * 2,     10633 * 2,

15883 * 2,     14259 * 2,     11598 * 2,

15463 * 2,     13031 * 2,     10169 * 2,

15821 * 2,     12195 * 2,      8137 * 2,

15582 * 2,     12516 * 2,      9433 * 2,

14821 * 2,     12969 * 2,     10527 * 2,

15732 * 2,     13268 * 2,     11498 * 2,

15146 * 2,     13327 * 2,     11267 * 2,

14842 * 2,     12289 * 2,     10422 * 2,

14567 * 2,     13505 * 2,     10780 * 2,

14390 * 2,     12759 * 2,     11170 * 2,

14858 * 2,     13614 * 2,     11657 * 2,

14237 * 2,     13218 * 2,     11461 * 2,

14608 * 2,     13504 * 2,     12080 * 2,

14674 * 2,     13974 * 2,     12184 * 2,

14866 * 2,     13720 * 2,     12773 * 2,

14896 * 2,     14332 * 2,     12782 * 2,

14840 * 2,     14067 * 2,     13132 * 2,

15435 * 2,     13786 * 2,     12823 * 2,

15121 * 2,     14302 * 2,     13483 * 2,

15333 * 2,     14603 * 2,     13872 * 2,

15772 * 2,     14524 * 2,     13052 * 2,

15708 * 2,     14584 * 2,     12009 * 2,

15797 * 2,     14669 * 2,     13637 * 2,

15814 * 2,     14858 * 2,     12904 * 2,

15962 * 2,     14568 * 2,     12497 * 2,

15969 * 2,     15014 * 2,     13792 * 2,

14039 * 2,     12511 * 2,      9773 * 2,

14251 * 2,     11878 * 2,      9621 * 2,

14100 * 2,     11691 * 2,      8583 * 2,

14814 * 2,     10471 * 2,      7911 * 2,

13965 * 2,     10725 * 2,      7012 * 2,

15689 * 2,      9981 * 2,      6000 * 2,

14388 * 2,      9740 * 2,      5769 * 2,

14384 * 2,      9736 * 2,      4341 * 2,

13970 * 2,      9084 * 2,      5028 * 2,

15298 * 2,      8857 * 2,      4205 * 2,

14071 * 2,      7942 * 2,      4472 * 2,

13980 * 2,      9604 * 2,      3273 * 2,

13383 * 2,      8140 * 2,      3828 * 2,

13005 * 2,      7620 * 2,      4742 * 2,

16310 * 2,     16203 * 2,     15451 * 2 };

/*-----------------------------------------*

* 2nd codebook:   lsp3 to lsp5            *

*-----------------------------------------*/

static Word16 dico2_clsp[taille_dic2*dim_dic2] = {

7564 * 2,      -293 * 2,     -2543 * 2,

7820 * 2,      -150 * 2,     -1813 * 2,

6913 * 2,     -1006 * 2,     -2297 * 2,

7099 * 2,      -498 * 2,     -3337 * 2,

6526 * 2,       308 * 2,     -2075 * 2,

6953 * 2,       958 * 2,     -2388 * 2,

6811 * 2,       758 * 2,     -3159 * 2,

6448 * 2,        93 * 2,     -1294 * 2,

7572 * 2,       843 * 2,     -1485 * 2,

5803 * 2,        76 * 2,     -2785 * 2,

5791 * 2,      -566 * 2,     -1850 * 2,

5655 * 2,       944 * 2,     -1852 * 2,

6035 * 2,      1575 * 2,     -2728 * 2,

5226 * 2,       672 * 2,     -2515 * 2,

6131 * 2,      1800 * 2,     -1816 * 2,

6104 * 2,      1092 * 2,      -955 * 2,

4868 * 2,       150 * 2,     -1704 * 2,

4540 * 2,       928 * 2,     -2198 * 2,

4954 * 2,      1589 * 2,     -1403 * 2,

5133 * 2,      1825 * 2,     -2481 * 2,

5266 * 2,      2505 * 2,     -2018 * 2,

4140 * 2,      1163 * 2,     -1467 * 2,

5312 * 2,      2208 * 2,      -846 * 2,

4412 * 2,      2344 * 2,     -1137 * 2,

4016 * 2,      1694 * 2,     -1020 * 2,

3937 * 2,      2428 * 2,     -2025 * 2,

4706 * 2,      3059 * 2,     -1381 * 2,

3469 * 2,      1623 * 2,     -1956 * 2,

4611 * 2,      2818 * 2,      -273 * 2,

3830 * 2,      2126 * 2,      -140 * 2,

5232 * 2,      1681 * 2,      -440 * 2,

4523 * 2,      1482 * 2,       106 * 2,

4461 * 2,       950 * 2,      -458 * 2,

3228 * 2,      1395 * 2,      -718 * 2,

4792 * 2,      2283 * 2,       392 * 2,

4198 * 2,       318 * 2,      -990 * 2,

5397 * 2,      3081 * 2,      -622 * 2,

6034 * 2,      1867 * 2,       379 * 2,

6276 * 2,      1978 * 2,      -806 * 2,

5881 * 2,      2678 * 2,       452 * 2,

6308 * 2,      2449 * 2,      -240 * 2,

5782 * 2,      2367 * 2,      1051 * 2,

6319 * 2,      3243 * 2,       153 * 2,

6434 * 2,      2971 * 2,      -955 * 2,

7053 * 2,      1856 * 2,      -216 * 2,

7385 * 2,      2650 * 2,        64 * 2,

6774 * 2,      2930 * 2,      1008 * 2,

7416 * 2,      2162 * 2,       703 * 2,

7282 * 2,      2723 * 2,      -833 * 2,

7452 * 2,      3624 * 2,      -298 * 2,

8031 * 2,      3206 * 2,       530 * 2,

6889 * 2,      4018 * 2,       470 * 2,

7485 * 2,      3735 * 2,      1191 * 2,

7008 * 2,      2808 * 2,      1663 * 2,

8040 * 2,      3276 * 2,      1625 * 2,

7841 * 2,      4506 * 2,       984 * 2,

8460 * 2,      4070 * 2,       391 * 2,

6611 * 2,      4137 * 2,      1514 * 2,

7487 * 2,      4459 * 2,      2017 * 2,

7549 * 2,      3763 * 2,      2384 * 2,

6420 * 2,      3454 * 2,      1907 * 2,

6694 * 2,      4869 * 2,      1313 * 2,

7852 * 2,      5242 * 2,      1730 * 2,

6991 * 2,      5489 * 2,      1418 * 2,

6873 * 2,      4940 * 2,      2548 * 2,

6389 * 2,      4265 * 2,      2477 * 2,

7703 * 2,      4357 * 2,      2885 * 2,

7318 * 2,      5438 * 2,      2788 * 2,

7257 * 2,      4987 * 2,      3443 * 2,

8222 * 2,      5222 * 2,      2614 * 2,

7645 * 2,      6104 * 2,      2139 * 2,

8148 * 2,      5663 * 2,      3485 * 2,

8607 * 2,      4884 * 2,      3527 * 2,

7512 * 2,      5913 * 2,      4110 * 2,

8568 * 2,      6064 * 2,      2396 * 2,

8113 * 2,      6670 * 2,      2267 * 2,

8790 * 2,      6611 * 2,      3338 * 2,

9197 * 2,      5692 * 2,      3316 * 2,

8272 * 2,      6549 * 2,      4051 * 2,

9059 * 2,      6038 * 2,      4412 * 2,

9554 * 2,      6359 * 2,      3624 * 2,

9264 * 2,      5327 * 2,      4105 * 2,

9149 * 2,      6926 * 2,      4494 * 2,

9916 * 2,      6778 * 2,      4197 * 2,

9600 * 2,      7338 * 2,      3435 * 2,

10366 * 2,      5712 * 2,      3980 * 2,

10362 * 2,      6304 * 2,      4858 * 2,

10383 * 2,      7083 * 2,      4912 * 2,

10467 * 2,      7485 * 2,      4027 * 2,

10573 * 2,      6057 * 2,      3302 * 2,

10367 * 2,      6987 * 2,      3015 * 2,

11091 * 2,      6561 * 2,      3914 * 2,

11279 * 2,      7390 * 2,      4289 * 2,

11378 * 2,      5806 * 2,      4560 * 2,

11504 * 2,      6657 * 2,      5030 * 2,

11919 * 2,      6407 * 2,      3700 * 2,

11338 * 2,      5343 * 2,      3847 * 2,

11333 * 2,      5308 * 2,      3155 * 2,

11051 * 2,      6450 * 2,      2535 * 2,

11518 * 2,      7251 * 2,      2774 * 2,

12044 * 2,      6329 * 2,      2477 * 2,

12482 * 2,      5821 * 2,      4097 * 2,

12478 * 2,      7102 * 2,      3587 * 2,

12676 * 2,      5586 * 2,      3333 * 2,

12171 * 2,      4918 * 2,      2877 * 2,

12751 * 2,      6844 * 2,      2736 * 2,

13176 * 2,      5963 * 2,      3744 * 2,

12802 * 2,      6393 * 2,      4533 * 2,

12692 * 2,      4994 * 2,      4094 * 2,

13017 * 2,      4667 * 2,      3408 * 2,

13343 * 2,      5710 * 2,      2375 * 2,

12868 * 2,      5296 * 2,      2008 * 2,

13364 * 2,      4239 * 2,      2361 * 2,

13843 * 2,      5743 * 2,      4255 * 2,

14049 * 2,      5097 * 2,      3468 * 2,

14033 * 2,      4939 * 2,      2835 * 2,

13898 * 2,      6536 * 2,      3115 * 2,

14169 * 2,      4961 * 2,      1705 * 2,

13710 * 2,      4832 * 2,      1148 * 2,

14596 * 2,      5508 * 2,      2662 * 2,

13187 * 2,      6223 * 2,       960 * 2,

13822 * 2,      6583 * 2,       583 * 2,

14436 * 2,      7264 * 2,      1534 * 2,

13770 * 2,      7598 * 2,      1965 * 2,

13170 * 2,      7340 * 2,      1891 * 2,

13941 * 2,      4561 * 2,       129 * 2,

13431 * 2,      5471 * 2,      -449 * 2,

14335 * 2,      5782 * 2,      -545 * 2,

12866 * 2,      5691 * 2,      -477 * 2,

12509 * 2,      6194 * 2,      1001 * 2,

12692 * 2,      7437 * 2,       704 * 2,

12722 * 2,      4956 * 2,       901 * 2,

12151 * 2,      6291 * 2,       -75 * 2,

12353 * 2,      4481 * 2,       -63 * 2,

11761 * 2,      6052 * 2,       985 * 2,

12284 * 2,      4617 * 2,      1797 * 2,

11734 * 2,      4431 * 2,       388 * 2,

11295 * 2,      5975 * 2,      -390 * 2,

11032 * 2,      4944 * 2,       743 * 2,

11399 * 2,      5089 * 2,      2116 * 2,

10964 * 2,      6187 * 2,      1187 * 2,

10863 * 2,      4166 * 2,      1606 * 2,

10205 * 2,      5235 * 2,      1461 * 2,

10772 * 2,      4436 * 2,      -223 * 2,

10305 * 2,      5642 * 2,        70 * 2,

10393 * 2,      6369 * 2,      1675 * 2,

10597 * 2,      4319 * 2,      2393 * 2,

10600 * 2,      5163 * 2,      2801 * 2,

9741 * 2,      4539 * 2,       336 * 2,

9347 * 2,      4692 * 2,      1139 * 2,

9785 * 2,      3625 * 2,      1401 * 2,

9006 * 2,      5480 * 2,      1621 * 2,

9538 * 2,      5265 * 2,      2600 * 2,

8992 * 2,      4694 * 2,      2420 * 2,

8956 * 2,      4140 * 2,      1972 * 2,

9472 * 2,      3917 * 2,      2709 * 2,

9301 * 2,      4477 * 2,      3132 * 2,

8874 * 2,      3348 * 2,      2170 * 2,

9827 * 2,      5042 * 2,      3445 * 2,

9890 * 2,      6059 * 2,      2463 * 2,

10518 * 2,      3522 * 2,      2283 * 2,

10797 * 2,      4366 * 2,      3190 * 2,

10728 * 2,      4775 * 2,      3680 * 2,

9809 * 2,      2897 * 2,      1746 * 2,

11591 * 2,      3608 * 2,      1929 * 2,

10834 * 2,      3491 * 2,       761 * 2,

11979 * 2,      3948 * 2,      2794 * 2,

11799 * 2,      2977 * 2,      1849 * 2,

11804 * 2,      3224 * 2,      1025 * 2,

10578 * 2,      2760 * 2,       924 * 2,

12052 * 2,      2472 * 2,      1517 * 2,

11763 * 2,      1897 * 2,       874 * 2,

10935 * 2,      2556 * 2,       -10 * 2,

12339 * 2,      2548 * 2,       178 * 2,

11812 * 2,      2374 * 2,      -613 * 2,

10869 * 2,      1311 * 2,       203 * 2,

11242 * 2,      3714 * 2,      -618 * 2,

10672 * 2,      1941 * 2,     -1043 * 2,

11974 * 2,       768 * 2,      -371 * 2,

10582 * 2,      1020 * 2,      -737 * 2,

11222 * 2,      1190 * 2,     -1751 * 2,

11652 * 2,      3167 * 2,     -1686 * 2,

11771 * 2,      1803 * 2,     -2377 * 2,

12157 * 2,       276 * 2,     -1261 * 2,

11281 * 2,      1351 * 2,     -3052 * 2,

11760 * 2,        77 * 2,     -1978 * 2,

10461 * 2,       599 * 2,     -2519 * 2,

12320 * 2,      -312 * 2,     -2603 * 2,

12908 * 2,      1043 * 2,     -1899 * 2,

12826 * 2,       931 * 2,     -3215 * 2,

12987 * 2,      1269 * 2,     -1053 * 2,

13424 * 2,       538 * 2,     -2342 * 2,

13102 * 2,       -57 * 2,     -4076 * 2,

12263 * 2,      2454 * 2,     -3882 * 2,

13816 * 2,       796 * 2,     -3719 * 2,

13973 * 2,      1713 * 2,     -2871 * 2,

13869 * 2,      1446 * 2,     -1618 * 2,

13292 * 2,      2675 * 2,     -1898 * 2,

13868 * 2,      2841 * 2,     -1281 * 2,

14411 * 2,      2364 * 2,     -3252 * 2,

13521 * 2,      3517 * 2,     -3271 * 2,

14425 * 2,      2348 * 2,     -1729 * 2,

14732 * 2,      2891 * 2,     -2389 * 2,

14526 * 2,      3821 * 2,     -1287 * 2,

14039 * 2,      4544 * 2,     -2746 * 2,

13926 * 2,      5111 * 2,     -1292 * 2,

13529 * 2,      3616 * 2,      -239 * 2,

14613 * 2,      5736 * 2,     -2230 * 2,

14968 * 2,      5253 * 2,     -1021 * 2,

14559 * 2,      3776 * 2,       465 * 2,

14617 * 2,      4378 * 2,     -4224 * 2,

13368 * 2,      6485 * 2,     -1784 * 2,

12599 * 2,      4340 * 2,     -1435 * 2,

12694 * 2,      3951 * 2,     -2784 * 2,

13073 * 2,      3789 * 2,       588 * 2,

12852 * 2,      2836 * 2,      -620 * 2,

13700 * 2,      1906 * 2,       -33 * 2,

13417 * 2,      2505 * 2,       809 * 2,

12998 * 2,      3554 * 2,      1467 * 2,

14015 * 2,      3141 * 2,      1469 * 2,

12867 * 2,      3536 * 2,      2271 * 2,

13756 * 2,       929 * 2,      -384 * 2,

10188 * 2,      1745 * 2,       592 * 2,

11419 * 2,      5535 * 2,     -1711 * 2,

9637 * 2,      3260 * 2,       332 * 2,

10133 * 2,      4611 * 2,     -1013 * 2,

9707 * 2,      2996 * 2,      -871 * 2,

10284 * 2,      3161 * 2,     -2270 * 2,

9710 * 2,      1748 * 2,      -461 * 2,

9184 * 2,      2352 * 2,       138 * 2,

9166 * 2,      3710 * 2,     -1743 * 2,

9163 * 2,      4407 * 2,      -475 * 2,

8561 * 2,      3500 * 2,      -614 * 2,

8676 * 2,      2539 * 2,     -1572 * 2,

8520 * 2,      2158 * 2,      -676 * 2,

7797 * 2,      3393 * 2,     -1237 * 2,

7991 * 2,      1776 * 2,     -1668 * 2,

8159 * 2,      3239 * 2,     -2484 * 2,

8140 * 2,      4597 * 2,      -982 * 2,

6990 * 2,      2510 * 2,     -1870 * 2,

7026 * 2,      4004 * 2,     -1953 * 2,

7136 * 2,      1609 * 2,     -1203 * 2,

7217 * 2,      1904 * 2,     -2725 * 2,

6773 * 2,      3042 * 2,     -2878 * 2,

5944 * 2,      2926 * 2,     -1591 * 2,

6396 * 2,      3845 * 2,      -761 * 2,

5666 * 2,      3021 * 2,     -3053 * 2,

5164 * 2,      3788 * 2,     -1911 * 2,

5680 * 2,      4283 * 2,     -1046 * 2,

6350 * 2,      4968 * 2,     -1518 * 2,

5489 * 2,      3743 * 2,       -28 * 2,

6183 * 2,      4396 * 2,       173 * 2,

7036 * 2,      4742 * 2,      -310 * 2,

5005 * 2,      3366 * 2,       513 * 2,

5862 * 2,      3686 * 2,      1244 * 2,

5779 * 2,      4527 * 2,      1341 * 2,

5432 * 2,      3100 * 2,      1205 * 2,

7416 * 2,      5187 * 2,       465 * 2,

7287 * 2,      5658 * 2,      -643 * 2,

8414 * 2,      5026 * 2,       586 * 2,

7991 * 2,      5959 * 2,       759 * 2,

8790 * 2,      2939 * 2,      1176 * 2,

9149 * 2,      5825 * 2,       175 * 2,

8623 * 2,      6672 * 2,      1016 * 2,

8299 * 2,      6034 * 2,     -1339 * 2,

9158 * 2,      6865 * 2,      -646 * 2,

9716 * 2,      6566 * 2,       979 * 2,

9119 * 2,      7508 * 2,      1103 * 2,

9216 * 2,      6732 * 2,      2187 * 2,

9767 * 2,      7791 * 2,       192 * 2,

10117 * 2,      7427 * 2,      1579 * 2,

9646 * 2,      8099 * 2,      2184 * 2,

10789 * 2,      7148 * 2,      -115 * 2,

10538 * 2,      8395 * 2,       577 * 2,

10523 * 2,      7909 * 2,      2683 * 2,

11293 * 2,      7494 * 2,      1440 * 2,

11121 * 2,      8682 * 2,      1598 * 2,

10387 * 2,      8722 * 2,      2843 * 2,

11092 * 2,      7990 * 2,      3176 * 2,

11790 * 2,      8511 * 2,      2266 * 2,

10995 * 2,      9415 * 2,      2452 * 2,

11666 * 2,      8212 * 2,      3760 * 2,

11999 * 2,      9089 * 2,      3774 * 2,

12268 * 2,      9210 * 2,      2028 * 2,

11821 * 2,      9800 * 2,      3288 * 2,

11424 * 2,      9183 * 2,      4356 * 2,

12664 * 2,      9221 * 2,      3093 * 2,

12343 * 2,      9929 * 2,      3844 * 2,

11318 * 2,     10061 * 2,      4382 * 2,

12362 * 2,      9114 * 2,      4908 * 2,

12753 * 2,     10503 * 2,      3698 * 2,

12985 * 2,      9731 * 2,      4733 * 2,

11892 * 2,      9626 * 2,      5588 * 2,

12458 * 2,     10370 * 2,      5601 * 2,

11854 * 2,     10622 * 2,      5262 * 2,

12878 * 2,      8886 * 2,      5806 * 2,

12559 * 2,      9585 * 2,      6659 * 2,

13068 * 2,     10386 * 2,      6203 * 2,

12633 * 2,     11087 * 2,      6026 * 2,

12186 * 2,     10894 * 2,      6938 * 2,

11920 * 2,      9993 * 2,      7144 * 2,

13104 * 2,     11211 * 2,      6967 * 2,

13164 * 2,     10142 * 2,      7665 * 2,

12367 * 2,      9323 * 2,      7804 * 2,

12585 * 2,     10879 * 2,      8219 * 2,

12317 * 2,     10251 * 2,      8449 * 2,

12720 * 2,     11567 * 2,      8102 * 2,

13234 * 2,      9369 * 2,      8463 * 2,

11548 * 2,      9421 * 2,      8254 * 2,

13249 * 2,      8973 * 2,      7039 * 2,

12176 * 2,      8597 * 2,      6758 * 2,

11461 * 2,      9342 * 2,      6748 * 2,

10742 * 2,      8788 * 2,      7518 * 2,

10806 * 2,      9124 * 2,      6235 * 2,

10997 * 2,      8275 * 2,      6159 * 2,

11560 * 2,      8613 * 2,      5437 * 2,

12283 * 2,      8066 * 2,      5739 * 2,

11068 * 2,      7556 * 2,      5368 * 2,

10110 * 2,      8304 * 2,      6289 * 2,

10311 * 2,      7972 * 2,      5163 * 2,

10441 * 2,      7206 * 2,      5823 * 2,

10880 * 2,      8468 * 2,      4478 * 2,

11908 * 2,      7316 * 2,      5254 * 2,

10635 * 2,      9226 * 2,      4414 * 2,

9984 * 2,      8256 * 2,      4205 * 2,

9275 * 2,      7592 * 2,      5010 * 2,

12135 * 2,      8005 * 2,      4117 * 2,

12553 * 2,      7346 * 2,      4798 * 2,

12699 * 2,      8151 * 2,      2816 * 2,

13136 * 2,      8345 * 2,      4614 * 2,

12192 * 2,      7482 * 2,      1678 * 2,

13413 * 2,      7358 * 2,      3703 * 2,

13403 * 2,      6763 * 2,      5157 * 2,

13300 * 2,      7614 * 2,      5971 * 2,

13940 * 2,      8404 * 2,      3784 * 2,

13777 * 2,      8074 * 2,      5441 * 2,

13465 * 2,      9081 * 2,      2751 * 2,

13567 * 2,      9647 * 2,      4283 * 2,

14428 * 2,      7463 * 2,      4258 * 2,

14332 * 2,      9351 * 2,      4182 * 2,

14393 * 2,      8218 * 2,      2867 * 2,

14276 * 2,      9080 * 2,      6060 * 2,

14076 * 2,      9711 * 2,      2470 * 2,

13630 * 2,      9696 * 2,      5808 * 2,

14679 * 2,      9709 * 2,      2242 * 2,

14160 * 2,     10997 * 2,      4409 * 2,

13309 * 2,     10327 * 2,      2581 * 2,

13224 * 2,     10865 * 2,      4126 * 2,

14552 * 2,     11321 * 2,      4526 * 2,

13808 * 2,     11509 * 2,      4970 * 2,

13475 * 2,     10927 * 2,      5633 * 2,

13018 * 2,     11618 * 2,      4262 * 2,

13931 * 2,     12153 * 2,      3241 * 2,

13376 * 2,     12029 * 2,      5980 * 2,

12334 * 2,     10978 * 2,      3108 * 2,

13728 * 2,     10999 * 2,      1834 * 2,

14883 * 2,     11658 * 2,      3750 * 2,

13940 * 2,     10644 * 2,      6944 * 2,

14216 * 2,     12662 * 2,      6727 * 2,

13652 * 2,     12582 * 2,      7202 * 2,

13647 * 2,     11561 * 2,      8042 * 2,

13073 * 2,     12210 * 2,      7825 * 2,

13021 * 2,     11362 * 2,      9396 * 2,

13850 * 2,     12600 * 2,      9870 * 2,

13321 * 2,     12442 * 2,     10862 * 2,

12641 * 2,     10336 * 2,      1486 * 2,

12993 * 2,     11301 * 2,      1024 * 2,

12026 * 2,     10336 * 2,      1353 * 2,

14249 * 2,     10493 * 2,      1174 * 2,

12910 * 2,      8766 * 2,      1401 * 2,

13576 * 2,      9065 * 2,       929 * 2,

13237 * 2,     10168 * 2,        45 * 2,

12659 * 2,      9572 * 2,      -237 * 2,

12200 * 2,      8811 * 2,       539 * 2,

11676 * 2,      9627 * 2,       701 * 2,

13175 * 2,      7994 * 2,      -647 * 2,

14274 * 2,      8614 * 2,      1030 * 2,

11770 * 2,      7827 * 2,       402 * 2,

12397 * 2,      7494 * 2,      -904 * 2,

11242 * 2,      8488 * 2,      -714 * 2,

11883 * 2,      8572 * 2,     -2307 * 2,

12942 * 2,      8137 * 2,     -2821 * 2,

14047 * 2,      7675 * 2,     -1923 * 2,

12011 * 2,      6420 * 2,     -3262 * 2,

10184 * 2,      7669 * 2,     -1818 * 2,

10155 * 2,      6096 * 2,     -1676 * 2,

13048 * 2,      5362 * 2,     -4666 * 2,

13653 * 2,      6231 * 2,     -5460 * 2,

11925 * 2,      4015 * 2,     -5430 * 2,

10735 * 2,      3346 * 2,     -3524 * 2,

9349 * 2,      4598 * 2,     -3399 * 2,

11031 * 2,      2014 * 2,     -5157 * 2,

9330 * 2,      2646 * 2,     -5465 * 2,

9374 * 2,      1306 * 2,     -3874 * 2,

11556 * 2,       299 * 2,     -4747 * 2,

9264 * 2,      1787 * 2,     -2708 * 2,

10938 * 2,       -42 * 2,     -3854 * 2,

9077 * 2,       249 * 2,     -2965 * 2,

9951 * 2,      -643 * 2,     -4696 * 2,

8695 * 2,       -70 * 2,     -3650 * 2,

8193 * 2,       664 * 2,     -4738 * 2,

8925 * 2,      -879 * 2,     -2873 * 2,

8632 * 2,       644 * 2,     -2251 * 2,

10049 * 2,      -345 * 2,     -1940 * 2,

8812 * 2,     -1699 * 2,     -4282 * 2,

7585 * 2,     -1492 * 2,     -3374 * 2,

7321 * 2,      -662 * 2,     -4259 * 2,

8513 * 2,      -862 * 2,     -5444 * 2,

6425 * 2,       426 * 2,     -4076 * 2,

7536 * 2,      1957 * 2,     -3948 * 2,

6558 * 2,      1461 * 2,     -4873 * 2,

5999 * 2,      -265 * 2,     -5358 * 2,

5657 * 2,       348 * 2,     -4658 * 2,

5488 * 2,      1123 * 2,     -3537 * 2,

5150 * 2,        51 * 2,     -3476 * 2,

5222 * 2,      -943 * 2,     -4507 * 2,

4581 * 2,       362 * 2,     -4293 * 2,

4868 * 2,      1676 * 2,     -5157 * 2,

4431 * 2,      1543 * 2,     -4063 * 2,

3850 * 2,       845 * 2,     -4686 * 2,

4059 * 2,      -107 * 2,     -3395 * 2,

4527 * 2,      1273 * 2,     -2949 * 2,

3735 * 2,       744 * 2,     -3093 * 2,

3082 * 2,      1200 * 2,     -4127 * 2,

3405 * 2,      1879 * 2,     -3231 * 2,

4594 * 2,      2463 * 2,     -3711 * 2,

2999 * 2,       103 * 2,     -3506 * 2,

2380 * 2,       690 * 2,     -3330 * 2,

3339 * 2,       658 * 2,     -2070 * 2,

2691 * 2,      1302 * 2,     -1971 * 2,

3346 * 2,       -52 * 2,     -1960 * 2,

2814 * 2,      -501 * 2,     -3150 * 2,

2052 * 2,        79 * 2,     -2475 * 2,

2320 * 2,      -560 * 2,     -2143 * 2,

2676 * 2,       627 * 2,     -1273 * 2,

2770 * 2,     -1345 * 2,     -2640 * 2,

4487 * 2,       171 * 2,     -2513 * 2,

4378 * 2,      -681 * 2,     -2521 * 2,

3230 * 2,     -1229 * 2,     -3412 * 2,

2693 * 2,     -2022 * 2,     -3359 * 2,

4176 * 2,      -888 * 2,     -3993 * 2,

3523 * 2,     -2141 * 2,     -3983 * 2,

3004 * 2,      -293 * 2,     -4569 * 2,

3375 * 2,     -1360 * 2,     -4767 * 2,

4100 * 2,     -1735 * 2,     -5277 * 2,

4204 * 2,      -302 * 2,     -5248 * 2,

2085 * 2,      -979 * 2,     -4878 * 2,

2646 * 2,     -1469 * 2,     -5896 * 2,

3291 * 2,      -357 * 2,     -5984 * 2,

2098 * 2,       152 * 2,     -5496 * 2,

3851 * 2,     -1336 * 2,     -6741 * 2,

2411 * 2,     -1002 * 2,     -6998 * 2,

4221 * 2,     -2346 * 2,     -6013 * 2,

1591 * 2,     -1869 * 2,     -6561 * 2,

1203 * 2,      -796 * 2,     -6075 * 2,

1744 * 2,     -2522 * 2,     -5654 * 2,

2310 * 2,     -3164 * 2,     -6344 * 2,

1385 * 2,     -1923 * 2,     -4877 * 2,

1045 * 2,     -2988 * 2,     -6886 * 2,

1018 * 2,     -3544 * 2,     -5868 * 2,

2581 * 2,     -3527 * 2,     -5340 * 2,

441 * 2,     -2748 * 2,     -5208 * 2,

1814 * 2,     -3242 * 2,     -4644 * 2,

1316 * 2,     -3962 * 2,     -6696 * 2,

1858 * 2,     -2301 * 2,     -4189 * 2,

-49 * 2,     -3715 * 2,     -6801 * 2,

-16 * 2,     -4707 * 2,     -6171 * 2,

1248 * 2,     -4592 * 2,     -7268 * 2,

1512 * 2,     -3646 * 2,     -7795 * 2,

-1013 * 2,     -2955 * 2,     -6031 * 2,

-92 * 2,     -2204 * 2,     -7396 * 2,

255 * 2,     -4976 * 2,     -7893 * 2,

3177 * 2,     -4114 * 2,     -6632 * 2,

1920 * 2,     -5053 * 2,     -9114 * 2,

3872 * 2,     -3048 * 2,     -7641 * 2,

4455 * 2,     -3457 * 2,     -6789 * 2,

3806 * 2,     -2934 * 2,     -4924 * 2,

5509 * 2,     -2502 * 2,     -6742 * 2,

5692 * 2,     -1390 * 2,     -5591 * 2,

6772 * 2,     -2446 * 2,     -4848 * 2,

5667 * 2,     -1925 * 2,     -3815 * 2,

4441 * 2,       625 * 2,     -6528 * 2,

5532 * 2,     -1052 * 2,     -3049 * 2,

7978 * 2,      -858 * 2,     -6705 * 2,

3643 * 2,      1692 * 2,     -5218 * 2,

5975 * 2,      2763 * 2,     -4375 * 2,

2361 * 2,       891 * 2,     -5213 * 2,

1718 * 2,      -991 * 2,     -4008 * 2,

1651 * 2,       -61 * 2,     -3633 * 2,

4489 * 2,      3255 * 2,     -2982 * 2,

1208 * 2,      -990 * 2,     -2970 * 2,

-338 * 2,     -2055 * 2,     -4560 * 2,

6498 * 2,      1132 * 2,      -164 * 2,

7336 * 2,       403 * 2,      -675 * 2,

8818 * 2,      1050 * 2,      -967 * 2,

8652 * 2,      1072 * 2,      -293 * 2,

8291 * 2,      1571 * 2,       303 * 2,

9525 * 2,       235 * 2,     -1134 * 2,

9652 * 2,      1049 * 2,     -1668 * 2,

9069 * 2,      2296 * 2,       828 * 2,

9526 * 2,      2265 * 2,      1359 * 2,

10787 * 2,     -1109 * 2,     -3223 * 2,

7767 * 2,      5486 * 2,     -3086 * 2,

11690 * 2,     -1931 * 2,     -4657 * 2,

8997 * 2,     -3150 * 2,     -5482 * 2,

12107 * 2,      -782 * 2,     -5986 * 2,

13013 * 2,       511 * 2,     -5164 * 2,

14172 * 2,       840 * 2,     -4841 * 2,

14794 * 2,      7792 * 2,     -3028 * 2,

14596 * 2,      7581 * 2,      -813 * 2,

8808 * 2,      7531 * 2,      2912 * 2,

13790 * 2,     10971 * 2,     -1294 * 2 };

/*-----------------------------------------*

* 3nd codebook:   lsp6 to lsp9            *

*-----------------------------------------*/

static Word16 dico3_clsp[taille_dic3*dim_dic3] = {

2258 * 2,     -3027 * 2,    -14154 * 2,    -15275 * 2,

2617 * 2,     -2421 * 2,    -13423 * 2,    -14666 * 2,

1491 * 2,     -4636 * 2,    -13353 * 2,    -15066 * 2,

4111 * 2,     -2037 * 2,    -13313 * 2,    -14464 * 2,

2450 * 2,     -2406 * 2,    -12501 * 2,    -14033 * 2,

2373 * 2,     -4492 * 2,    -11954 * 2,    -15115 * 2,

3913 * 2,     -3857 * 2,    -11915 * 2,    -15152 * 2,

3423 * 2,     -1758 * 2,    -11741 * 2,    -15053 * 2,

3765 * 2,     -3882 * 2,    -12788 * 2,    -13519 * 2,

1562 * 2,     -1771 * 2,    -11644 * 2,    -14135 * 2,

1075 * 2,     -4880 * 2,    -12649 * 2,    -14058 * 2,

3638 * 2,     -3092 * 2,    -11893 * 2,    -13136 * 2,

1839 * 2,     -3454 * 2,    -12190 * 2,    -13087 * 2,

541 * 2,     -3451 * 2,    -11602 * 2,    -13865 * 2,

1557 * 2,     -4501 * 2,    -11314 * 2,    -12939 * 2,

872 * 2,     -5493 * 2,    -11547 * 2,    -14155 * 2,

-297 * 2,     -5352 * 2,    -12385 * 2,    -13504 * 2,

123 * 2,     -5666 * 2,    -11671 * 2,    -12907 * 2,

618 * 2,     -5376 * 2,    -10600 * 2,    -13291 * 2,

-1370 * 2,     -3979 * 2,    -11880 * 2,    -13663 * 2,

-1024 * 2,     -5949 * 2,    -10829 * 2,    -13536 * 2,

-1721 * 2,     -5448 * 2,    -11365 * 2,    -13776 * 2,

-641 * 2,     -6511 * 2,    -11878 * 2,    -14002 * 2,

-1514 * 2,     -6290 * 2,    -12143 * 2,    -13142 * 2,

-1583 * 2,     -4767 * 2,    -11486 * 2,    -12786 * 2,

-1183 * 2,     -5072 * 2,    -10358 * 2,    -13940 * 2,

-2687 * 2,     -4541 * 2,    -10928 * 2,    -13507 * 2,

-2919 * 2,     -5785 * 2,    -11747 * 2,    -13494 * 2,

-2538 * 2,     -6619 * 2,    -11027 * 2,    -13192 * 2,

-2446 * 2,     -5705 * 2,    -11156 * 2,    -12698 * 2,

-2043 * 2,     -5217 * 2,    -10126 * 2,    -13152 * 2,

-3314 * 2,     -5992 * 2,    -10494 * 2,    -13277 * 2,

-3380 * 2,     -4989 * 2,    -10032 * 2,    -13357 * 2,

-4071 * 2,     -5683 * 2,    -11128 * 2,    -13624 * 2,

-3464 * 2,     -4918 * 2,    -11331 * 2,    -13973 * 2,

-3331 * 2,     -6676 * 2,    -11238 * 2,    -13913 * 2,

-3686 * 2,     -6545 * 2,    -11198 * 2,    -12740 * 2,

-4633 * 2,     -5939 * 2,    -10395 * 2,    -13663 * 2,

-4443 * 2,     -6846 * 2,    -11017 * 2,    -13196 * 2,

-4066 * 2,     -6849 * 2,    -10301 * 2,    -13913 * 2,

-4291 * 2,     -6407 * 2,    -11398 * 2,    -14119 * 2,

-5002 * 2,     -6434 * 2,    -11583 * 2,    -13789 * 2,

-5310 * 2,     -6544 * 2,    -10539 * 2,    -13746 * 2,

-5112 * 2,     -7394 * 2,    -11164 * 2,    -13567 * 2,

-4827 * 2,     -6971 * 2,    -10001 * 2,    -13369 * 2,

-5657 * 2,     -7140 * 2,    -10365 * 2,    -13493 * 2,

-5190 * 2,     -8027 * 2,    -10599 * 2,    -13415 * 2,

-5907 * 2,     -7080 * 2,    -11187 * 2,    -14166 * 2,

-6284 * 2,     -7633 * 2,    -10747 * 2,    -13860 * 2,

-6051 * 2,     -7745 * 2,    -11537 * 2,    -13737 * 2,

-5341 * 2,     -8328 * 2,    -11248 * 2,    -13067 * 2,

-6310 * 2,     -8473 * 2,    -11362 * 2,    -13596 * 2,

-6467 * 2,     -8073 * 2,    -10692 * 2,    -12970 * 2,

-7180 * 2,     -8492 * 2,    -10931 * 2,    -13343 * 2,

-6977 * 2,     -8046 * 2,    -11613 * 2,    -14006 * 2,

-5705 * 2,     -7955 * 2,    -12189 * 2,    -13287 * 2,

-6190 * 2,     -8455 * 2,    -11824 * 2,    -12820 * 2,

-6802 * 2,     -8892 * 2,    -12057 * 2,    -13555 * 2,

-5609 * 2,     -9078 * 2,    -11130 * 2,    -13067 * 2,

-7282 * 2,     -9224 * 2,    -11146 * 2,    -13507 * 2,

-5587 * 2,     -8876 * 2,    -12187 * 2,    -13602 * 2,

-6157 * 2,     -9509 * 2,    -12117 * 2,    -13104 * 2,

-5840 * 2,     -9545 * 2,    -11672 * 2,    -13785 * 2,

-5068 * 2,     -8786 * 2,    -11683 * 2,    -13401 * 2,

-6344 * 2,     -9137 * 2,    -12671 * 2,    -13555 * 2,

-6655 * 2,    -10179 * 2,    -11980 * 2,    -13370 * 2,

-6242 * 2,    -10092 * 2,    -12522 * 2,    -13432 * 2,

-5821 * 2,     -9271 * 2,    -12552 * 2,    -14012 * 2,

-7085 * 2,     -9616 * 2,    -12276 * 2,    -13968 * 2,

-7061 * 2,     -9362 * 2,    -12863 * 2,    -13880 * 2,

-6592 * 2,     -8328 * 2,    -12460 * 2,    -13845 * 2,

-6286 * 2,     -8804 * 2,    -12945 * 2,    -14148 * 2,

-6710 * 2,    -10050 * 2,    -13020 * 2,    -14176 * 2,

-6713 * 2,    -10374 * 2,    -12927 * 2,    -13653 * 2,

-6869 * 2,     -8974 * 2,    -13313 * 2,    -14071 * 2,

-6771 * 2,     -9578 * 2,    -13010 * 2,    -14529 * 2,

-7836 * 2,     -9434 * 2,    -13091 * 2,    -14286 * 2,

-7582 * 2,    -10082 * 2,    -13311 * 2,    -14192 * 2,

-7685 * 2,     -8768 * 2,    -12554 * 2,    -14190 * 2,

-6660 * 2,     -9185 * 2,    -13614 * 2,    -14547 * 2,

-7287 * 2,     -9994 * 2,    -13761 * 2,    -14424 * 2,

-6119 * 2,    -10003 * 2,    -13519 * 2,    -14326 * 2,

-7610 * 2,     -8870 * 2,    -13578 * 2,    -14780 * 2,

-7944 * 2,     -9765 * 2,    -13469 * 2,    -14817 * 2,

-7068 * 2,    -10465 * 2,    -13490 * 2,    -14913 * 2,

-6437 * 2,     -9902 * 2,    -13996 * 2,    -14643 * 2,

-6860 * 2,     -9320 * 2,    -14192 * 2,    -14997 * 2,

-6033 * 2,     -9723 * 2,    -13600 * 2,    -15112 * 2,

-7312 * 2,    -10482 * 2,    -14253 * 2,    -15107 * 2,

-6026 * 2,    -10794 * 2,    -13901 * 2,    -14777 * 2,

-7911 * 2,    -10718 * 2,    -14030 * 2,    -14709 * 2,

-6908 * 2,    -11331 * 2,    -13939 * 2,    -15039 * 2,

-5948 * 2,    -10946 * 2,    -13395 * 2,    -14592 * 2,

-5903 * 2,     -9715 * 2,    -14491 * 2,    -15166 * 2,

-7071 * 2,    -11312 * 2,    -14446 * 2,    -15360 * 2,

-6732 * 2,    -10351 * 2,    -13546 * 2,    -15582 * 2,

-7547 * 2,    -11517 * 2,    -13733 * 2,    -14499 * 2,

-8135 * 2,    -11430 * 2,    -14394 * 2,    -14974 * 2,

-8436 * 2,    -11182 * 2,    -13843 * 2,    -14806 * 2,

-7833 * 2,    -12031 * 2,    -14002 * 2,    -15207 * 2,

-8091 * 2,    -10713 * 2,    -13635 * 2,    -14465 * 2,

-8509 * 2,    -11642 * 2,    -13383 * 2,    -15333 * 2,

-9007 * 2,    -11657 * 2,    -14100 * 2,    -14737 * 2,

-8815 * 2,    -11159 * 2,    -13312 * 2,    -14700 * 2,

-9207 * 2,    -11913 * 2,    -13759 * 2,    -14560 * 2,

-9285 * 2,    -10831 * 2,    -13848 * 2,    -14694 * 2,

-8597 * 2,    -11303 * 2,    -13516 * 2,    -14163 * 2,

-9264 * 2,    -11873 * 2,    -14171 * 2,    -15153 * 2,

-10039 * 2,    -11402 * 2,    -13745 * 2,    -14729 * 2,

-9459 * 2,    -11244 * 2,    -14384 * 2,    -14989 * 2,

-9566 * 2,    -10760 * 2,    -13370 * 2,    -14409 * 2,

-8917 * 2,    -10214 * 2,    -13810 * 2,    -14916 * 2,

-9932 * 2,    -11413 * 2,    -13003 * 2,    -14482 * 2,

-8704 * 2,    -10367 * 2,    -13309 * 2,    -14295 * 2,

-10370 * 2,    -11827 * 2,    -13525 * 2,    -14295 * 2,

-9682 * 2,    -11721 * 2,    -13258 * 2,    -13944 * 2,

-8606 * 2,    -11043 * 2,    -13115 * 2,    -13931 * 2,

-9323 * 2,    -10436 * 2,    -12611 * 2,    -14175 * 2,

-8799 * 2,    -10894 * 2,    -12675 * 2,    -13938 * 2,

-8745 * 2,    -11134 * 2,    -12565 * 2,    -14705 * 2,

-8699 * 2,    -11343 * 2,    -12686 * 2,    -13532 * 2,

-8615 * 2,    -10184 * 2,    -12313 * 2,    -13908 * 2,

-8013 * 2,    -10163 * 2,    -12923 * 2,    -13918 * 2,

-7827 * 2,    -10632 * 2,    -12342 * 2,    -13684 * 2,

-8801 * 2,    -10562 * 2,    -12012 * 2,    -13314 * 2,

-7782 * 2,     -9941 * 2,    -12520 * 2,    -13508 * 2,

-7184 * 2,    -10374 * 2,    -12445 * 2,    -14056 * 2,

-8329 * 2,     -9450 * 2,    -12339 * 2,    -14009 * 2,

-8129 * 2,     -9827 * 2,    -11637 * 2,    -13458 * 2,

-7572 * 2,     -9148 * 2,    -12059 * 2,    -13390 * 2,

-8677 * 2,     -9708 * 2,    -12866 * 2,    -14477 * 2,

-7594 * 2,    -10662 * 2,    -12969 * 2,    -14495 * 2,

-6901 * 2,     -8991 * 2,    -12128 * 2,    -14424 * 2,

-7217 * 2,     -8242 * 2,    -12871 * 2,    -14508 * 2,

-7034 * 2,    -10885 * 2,    -13420 * 2,    -14158 * 2,

-6348 * 2,     -9423 * 2,    -12826 * 2,    -14969 * 2,

-6111 * 2,    -10844 * 2,    -12297 * 2,    -14546 * 2,

-5493 * 2,    -10258 * 2,    -12720 * 2,    -14164 * 2,

-5372 * 2,     -9350 * 2,    -12931 * 2,    -14455 * 2,

-5654 * 2,     -9935 * 2,    -12744 * 2,    -15081 * 2,

-5464 * 2,     -8605 * 2,    -13009 * 2,    -14723 * 2,

-5012 * 2,    -10523 * 2,    -13054 * 2,    -14874 * 2,

-4898 * 2,     -9333 * 2,    -13567 * 2,    -14618 * 2,

-5347 * 2,     -9040 * 2,    -12207 * 2,    -15051 * 2,

-4353 * 2,     -9886 * 2,    -12281 * 2,    -14405 * 2,

-4458 * 2,     -8878 * 2,    -12945 * 2,    -14333 * 2,

-4073 * 2,     -8797 * 2,    -12507 * 2,    -14647 * 2,

-3601 * 2,     -8943 * 2,    -13112 * 2,    -14775 * 2,

-4518 * 2,     -9080 * 2,    -13530 * 2,    -15080 * 2,

-4159 * 2,     -9772 * 2,    -12836 * 2,    -14063 * 2,

-4909 * 2,     -8973 * 2,    -12211 * 2,    -14094 * 2,

-4961 * 2,     -8262 * 2,    -12126 * 2,    -14627 * 2,

-4409 * 2,     -8219 * 2,    -12531 * 2,    -14028 * 2,

-4435 * 2,     -7684 * 2,    -12834 * 2,    -14629 * 2,

-3529 * 2,     -7962 * 2,    -12876 * 2,    -14255 * 2,

-3751 * 2,     -8655 * 2,    -13494 * 2,    -14204 * 2,

-5246 * 2,     -8294 * 2,    -13381 * 2,    -14163 * 2,

-5690 * 2,     -8131 * 2,    -12666 * 2,    -14052 * 2,

-4438 * 2,     -7338 * 2,    -13222 * 2,    -13975 * 2,

-5584 * 2,     -7365 * 2,    -13205 * 2,    -14332 * 2,

-4946 * 2,     -7365 * 2,    -12549 * 2,    -13708 * 2,

-3934 * 2,     -7160 * 2,    -12882 * 2,    -13679 * 2,

-5238 * 2,     -8579 * 2,    -12846 * 2,    -13617 * 2,

-4910 * 2,     -6631 * 2,    -12587 * 2,    -14270 * 2,

-5542 * 2,     -7117 * 2,    -12123 * 2,    -13862 * 2,

-4012 * 2,     -7223 * 2,    -12212 * 2,    -13973 * 2,

-5125 * 2,     -8049 * 2,    -11943 * 2,    -13886 * 2,

-3879 * 2,     -7351 * 2,    -12375 * 2,    -13382 * 2,

-4600 * 2,     -6946 * 2,    -12021 * 2,    -13168 * 2,

-4605 * 2,     -7897 * 2,    -11739 * 2,    -13337 * 2,

-4082 * 2,     -8445 * 2,    -12068 * 2,    -13597 * 2,

-3665 * 2,     -7463 * 2,    -11557 * 2,    -13688 * 2,

-3867 * 2,     -7581 * 2,    -11625 * 2,    -12775 * 2,

-3722 * 2,     -7888 * 2,    -10925 * 2,    -13470 * 2,

-3031 * 2,     -8207 * 2,    -12287 * 2,    -13101 * 2,

-2639 * 2,     -7377 * 2,    -11750 * 2,    -13247 * 2,

-3291 * 2,     -8811 * 2,    -11872 * 2,    -12968 * 2,

-3759 * 2,     -8500 * 2,    -10902 * 2,    -12955 * 2,

-4233 * 2,     -8614 * 2,    -11788 * 2,    -12483 * 2,

-3832 * 2,     -8933 * 2,    -11332 * 2,    -12544 * 2,

-2269 * 2,     -8265 * 2,    -11190 * 2,    -12700 * 2,

-2151 * 2,     -8973 * 2,    -11367 * 2,    -13237 * 2,

-3700 * 2,     -8547 * 2,    -10965 * 2,    -12117 * 2,

-2425 * 2,     -8480 * 2,    -10385 * 2,    -13061 * 2,

-4135 * 2,     -7675 * 2,    -10670 * 2,    -12717 * 2,

-2463 * 2,     -7452 * 2,    -10692 * 2,    -12495 * 2,

-4116 * 2,     -7373 * 2,    -11309 * 2,    -12239 * 2,

-3509 * 2,     -7888 * 2,    -10270 * 2,    -11879 * 2,

-3150 * 2,     -7234 * 2,    -10169 * 2,    -13063 * 2,

-3005 * 2,     -7089 * 2,    -10930 * 2,    -11839 * 2,

-3384 * 2,     -7758 * 2,     -9501 * 2,    -12527 * 2,

-4448 * 2,     -7776 * 2,    -10006 * 2,    -12915 * 2,

-4547 * 2,     -6600 * 2,    -10270 * 2,    -12426 * 2,

-3027 * 2,     -6337 * 2,    -10287 * 2,    -12059 * 2,

-3216 * 2,     -6586 * 2,     -9655 * 2,    -12890 * 2,

-4121 * 2,     -6902 * 2,     -9225 * 2,    -12883 * 2,

-2915 * 2,     -5839 * 2,     -9437 * 2,    -12387 * 2,

-4060 * 2,     -5770 * 2,     -9461 * 2,    -13002 * 2,

-2924 * 2,     -7160 * 2,     -8755 * 2,    -12284 * 2,

-5042 * 2,     -6390 * 2,     -9057 * 2,    -13100 * 2,

-4201 * 2,     -5887 * 2,     -8410 * 2,    -13116 * 2,

-2889 * 2,     -6473 * 2,     -8294 * 2,    -13110 * 2,

-3461 * 2,     -5187 * 2,     -8362 * 2,    -12949 * 2,

-4651 * 2,     -6493 * 2,     -8009 * 2,    -12209 * 2,

-5323 * 2,     -7101 * 2,     -9166 * 2,    -12660 * 2,

-3232 * 2,     -5772 * 2,     -7693 * 2,    -11727 * 2,

-3954 * 2,     -6435 * 2,     -8949 * 2,    -11244 * 2,

-2691 * 2,     -6185 * 2,     -8260 * 2,    -11380 * 2,

-2648 * 2,     -5363 * 2,     -8576 * 2,    -11475 * 2,

-5102 * 2,     -6875 * 2,     -8511 * 2,    -11190 * 2,

-3824 * 2,     -6717 * 2,     -9987 * 2,    -11387 * 2,

-2425 * 2,     -6443 * 2,     -9186 * 2,    -10955 * 2,

-2216 * 2,     -7118 * 2,     -9656 * 2,    -11662 * 2,

-2027 * 2,     -5643 * 2,     -9985 * 2,    -11669 * 2,

-1582 * 2,     -6236 * 2,     -9060 * 2,    -12331 * 2,

-2068 * 2,     -7313 * 2,    -10351 * 2,    -11582 * 2,

-755 * 2,     -6844 * 2,     -9710 * 2,    -11707 * 2,

-1518 * 2,     -7717 * 2,    -10100 * 2,    -12380 * 2,

-744 * 2,     -6089 * 2,    -10481 * 2,    -11823 * 2,

-1635 * 2,     -5886 * 2,    -10856 * 2,    -12158 * 2,

-404 * 2,     -7526 * 2,    -10553 * 2,    -12547 * 2,

-1336 * 2,     -7077 * 2,    -11289 * 2,    -12512 * 2,

500 * 2,     -6955 * 2,    -10065 * 2,    -12438 * 2,

-17 * 2,     -5673 * 2,    -10843 * 2,    -12286 * 2,

-815 * 2,     -6924 * 2,     -9235 * 2,    -12999 * 2,

88 * 2,     -7143 * 2,    -11346 * 2,    -13241 * 2,

-1661 * 2,     -7418 * 2,    -10376 * 2,    -13501 * 2,

-1128 * 2,     -8022 * 2,    -11858 * 2,    -12875 * 2,

-1097 * 2,     -8331 * 2,    -11141 * 2,    -13565 * 2,

-1839 * 2,     -7119 * 2,    -11451 * 2,    -13819 * 2,

-331 * 2,     -7501 * 2,     -9970 * 2,    -13978 * 2,

-2238 * 2,     -6683 * 2,    -10486 * 2,    -13947 * 2,

-583 * 2,     -7350 * 2,    -11045 * 2,    -14570 * 2,

-2354 * 2,     -8320 * 2,    -11714 * 2,    -13950 * 2,

-1573 * 2,     -8210 * 2,    -12170 * 2,    -13626 * 2,

-2298 * 2,     -7840 * 2,    -11162 * 2,    -14578 * 2,

-2478 * 2,     -6535 * 2,    -11432 * 2,    -14457 * 2,

-2071 * 2,     -6924 * 2,    -12417 * 2,    -14293 * 2,

-1423 * 2,     -6187 * 2,    -11723 * 2,    -14768 * 2,

-2987 * 2,     -6901 * 2,    -12229 * 2,    -13837 * 2,

-2254 * 2,     -8289 * 2,    -12343 * 2,    -14663 * 2,

-3354 * 2,     -7942 * 2,    -12155 * 2,    -14284 * 2,

-3437 * 2,     -6531 * 2,    -12137 * 2,    -14527 * 2,

-2882 * 2,     -7340 * 2,    -11837 * 2,    -15079 * 2,

-3021 * 2,     -7084 * 2,    -12860 * 2,    -14905 * 2,

-2649 * 2,     -5888 * 2,    -12910 * 2,    -14579 * 2,

-1922 * 2,     -6363 * 2,    -12567 * 2,    -15093 * 2,

-3917 * 2,     -7895 * 2,    -12288 * 2,    -15009 * 2,

-4268 * 2,     -7384 * 2,    -11802 * 2,    -14440 * 2,

-3114 * 2,     -5648 * 2,    -12388 * 2,    -15124 * 2,

-4192 * 2,     -6273 * 2,    -12999 * 2,    -14330 * 2,

-3604 * 2,     -7229 * 2,    -13430 * 2,    -14509 * 2,

-4506 * 2,     -5775 * 2,    -12538 * 2,    -14447 * 2,

-4797 * 2,     -6306 * 2,    -13096 * 2,    -14957 * 2,

-3669 * 2,     -5302 * 2,    -12349 * 2,    -14277 * 2,

-3340 * 2,     -5796 * 2,    -13124 * 2,    -14068 * 2,

-3591 * 2,     -5380 * 2,    -13440 * 2,    -14707 * 2,

-2836 * 2,     -6184 * 2,    -13714 * 2,    -14435 * 2,

-4821 * 2,     -6920 * 2,    -13606 * 2,    -14505 * 2,

-3866 * 2,     -6360 * 2,    -14015 * 2,    -14772 * 2,

-4129 * 2,     -6015 * 2,    -12428 * 2,    -13774 * 2,

-2525 * 2,     -7214 * 2,    -13270 * 2,    -14143 * 2,

-2688 * 2,     -5151 * 2,    -12312 * 2,    -14060 * 2,

-2841 * 2,     -4505 * 2,    -12737 * 2,    -14704 * 2,

-2167 * 2,     -4432 * 2,    -13236 * 2,    -14351 * 2,

-1674 * 2,     -5285 * 2,    -13542 * 2,    -14729 * 2,

-1682 * 2,     -5312 * 2,    -12819 * 2,    -13912 * 2,

-2689 * 2,     -5766 * 2,    -12461 * 2,    -13395 * 2,

-522 * 2,     -5970 * 2,    -12628 * 2,    -14436 * 2,

-37 * 2,     -6079 * 2,    -13267 * 2,    -14165 * 2,

-1251 * 2,     -3344 * 2,    -12940 * 2,    -14242 * 2,

-2042 * 2,     -3653 * 2,    -12979 * 2,    -14880 * 2,

-1563 * 2,     -4470 * 2,    -11756 * 2,    -14774 * 2,

-252 * 2,     -4094 * 2,    -12046 * 2,    -14881 * 2,

-2590 * 2,     -4088 * 2,    -11559 * 2,    -14093 * 2,

-1817 * 2,     -3406 * 2,    -11146 * 2,    -14214 * 2,

-2347 * 2,     -4840 * 2,    -10723 * 2,    -14719 * 2,

-333 * 2,     -4997 * 2,    -10524 * 2,    -14779 * 2,

-1048 * 2,     -3769 * 2,    -10195 * 2,    -14339 * 2,

-2502 * 2,     -5724 * 2,    -10197 * 2,    -14570 * 2,

-2069 * 2,     -4009 * 2,     -9460 * 2,    -14313 * 2,

-3075 * 2,     -4500 * 2,     -9738 * 2,    -14703 * 2,

-3665 * 2,     -5650 * 2,    -10968 * 2,    -14696 * 2,

-3038 * 2,     -5592 * 2,     -9134 * 2,    -14096 * 2,

-1683 * 2,     -5635 * 2,     -8743 * 2,    -14555 * 2,

-3478 * 2,     -6306 * 2,     -9632 * 2,    -14834 * 2,

-2310 * 2,     -6941 * 2,     -9412 * 2,    -14113 * 2,

-1789 * 2,     -7180 * 2,    -10026 * 2,    -14917 * 2,

-3649 * 2,     -7174 * 2,    -10798 * 2,    -14964 * 2,

-4884 * 2,     -6782 * 2,     -9331 * 2,    -14539 * 2,

-4130 * 2,     -7783 * 2,     -9704 * 2,    -14296 * 2,

-3209 * 2,     -8267 * 2,    -10448 * 2,    -14139 * 2,

-1523 * 2,     -8503 * 2,    -10050 * 2,    -14744 * 2,

-4967 * 2,     -7233 * 2,    -10980 * 2,    -14622 * 2,

-4405 * 2,     -8268 * 2,    -11101 * 2,    -14164 * 2,

-4444 * 2,     -7988 * 2,    -11414 * 2,    -15169 * 2,

-5731 * 2,     -7986 * 2,    -10506 * 2,    -14819 * 2,

-3533 * 2,     -8866 * 2,    -11731 * 2,    -14582 * 2,

-3904 * 2,     -9181 * 2,    -11170 * 2,    -13855 * 2,

-2560 * 2,     -9499 * 2,    -11078 * 2,    -14844 * 2,

-5102 * 2,     -9641 * 2,    -11597 * 2,    -14571 * 2,

-5762 * 2,     -8671 * 2,    -11671 * 2,    -14272 * 2,

-5555 * 2,     -7699 * 2,    -12183 * 2,    -14734 * 2,

-3257 * 2,     -8774 * 2,    -12130 * 2,    -15150 * 2,

-6385 * 2,     -8877 * 2,    -10879 * 2,    -14952 * 2,

-2639 * 2,     -9484 * 2,    -12495 * 2,    -14502 * 2,

-4573 * 2,    -10038 * 2,    -12044 * 2,    -13615 * 2,

-2839 * 2,     -9657 * 2,    -11759 * 2,    -13773 * 2,

-4563 * 2,     -8769 * 2,    -12475 * 2,    -13275 * 2,

-3931 * 2,     -9965 * 2,    -12686 * 2,    -13508 * 2,

-2446 * 2,     -9300 * 2,    -12342 * 2,    -13708 * 2,

-2678 * 2,     -8421 * 2,    -12910 * 2,    -13811 * 2,

-4683 * 2,     -9297 * 2,    -12269 * 2,    -12973 * 2,

-2442 * 2,    -10261 * 2,    -13015 * 2,    -14122 * 2,

-4892 * 2,     -9479 * 2,    -11754 * 2,    -12898 * 2,

-5478 * 2,     -9556 * 2,    -13200 * 2,    -13844 * 2,

-4707 * 2,    -10400 * 2,    -13392 * 2,    -14169 * 2,

-3466 * 2,     -9494 * 2,    -13865 * 2,    -14327 * 2,

-3353 * 2,     -8233 * 2,    -13869 * 2,    -14575 * 2,

-5703 * 2,     -8553 * 2,    -13673 * 2,    -14414 * 2,

-4650 * 2,     -9988 * 2,    -14209 * 2,    -14789 * 2,

-5839 * 2,     -8786 * 2,    -13931 * 2,    -14949 * 2,

-5016 * 2,     -7751 * 2,    -13859 * 2,    -14823 * 2,

-3606 * 2,     -8092 * 2,    -14241 * 2,    -14949 * 2,

-4620 * 2,     -7842 * 2,    -14347 * 2,    -15262 * 2,

-3504 * 2,     -7468 * 2,    -13683 * 2,    -15120 * 2,

-4662 * 2,     -8042 * 2,    -12990 * 2,    -15281 * 2,

-5991 * 2,     -7721 * 2,    -13275 * 2,    -15019 * 2,

-5360 * 2,     -7044 * 2,    -13474 * 2,    -15424 * 2,

-4058 * 2,     -8833 * 2,    -12783 * 2,    -15391 * 2,

-3911 * 2,     -8840 * 2,    -13626 * 2,    -15721 * 2,

-4785 * 2,     -9441 * 2,    -13014 * 2,    -15382 * 2,

-5789 * 2,     -8844 * 2,    -12818 * 2,    -15475 * 2,

-3056 * 2,     -8107 * 2,    -12624 * 2,    -15369 * 2,

-3757 * 2,     -6875 * 2,    -12637 * 2,    -15623 * 2,

-2557 * 2,     -8635 * 2,    -13284 * 2,    -15087 * 2,

-3702 * 2,     -9042 * 2,    -12656 * 2,    -15986 * 2,

-4066 * 2,    -10051 * 2,    -12271 * 2,    -15442 * 2,

-2533 * 2,    -10409 * 2,    -13255 * 2,    -15462 * 2,

-5357 * 2,    -10545 * 2,    -13401 * 2,    -15642 * 2,

-4009 * 2,    -10553 * 2,    -13868 * 2,    -15118 * 2,

-3004 * 2,     -9401 * 2,    -14284 * 2,    -15227 * 2,

-3917 * 2,    -10672 * 2,    -14563 * 2,    -15613 * 2,

-1729 * 2,     -9887 * 2,    -13941 * 2,    -14722 * 2,

-5558 * 2,    -10041 * 2,    -14600 * 2,    -15447 * 2,

-4949 * 2,    -11441 * 2,    -14321 * 2,    -14933 * 2,

-3688 * 2,    -11414 * 2,    -13850 * 2,    -14526 * 2,

-5148 * 2,    -12634 * 2,    -14279 * 2,    -15609 * 2,

-5913 * 2,    -11034 * 2,    -14790 * 2,    -15975 * 2,

-6840 * 2,    -12206 * 2,    -14079 * 2,    -15646 * 2,

-7445 * 2,    -11687 * 2,    -14823 * 2,    -15405 * 2,

-7657 * 2,    -11124 * 2,    -14105 * 2,    -15748 * 2,

-7460 * 2,    -11012 * 2,    -13241 * 2,    -15590 * 2,

-7816 * 2,    -11903 * 2,    -15113 * 2,    -15635 * 2,

-7283 * 2,    -12023 * 2,    -14984 * 2,    -16015 * 2,

-8586 * 2,    -12702 * 2,    -14420 * 2,    -15665 * 2,

-8768 * 2,    -11668 * 2,    -14871 * 2,    -16066 * 2,

-8938 * 2,    -11004 * 2,    -14763 * 2,    -15378 * 2,

-8517 * 2,    -10255 * 2,    -14626 * 2,    -15736 * 2,

-9509 * 2,    -10924 * 2,    -14371 * 2,    -15669 * 2,

-9560 * 2,    -12148 * 2,    -14681 * 2,    -15573 * 2,

-9524 * 2,    -11637 * 2,    -13931 * 2,    -15687 * 2,

-9952 * 2,    -11446 * 2,    -14749 * 2,    -16022 * 2,

-10589 * 2,    -11733 * 2,    -14404 * 2,    -15410 * 2,

-10599 * 2,    -12143 * 2,    -14897 * 2,    -15877 * 2,

-9402 * 2,    -12839 * 2,    -14943 * 2,    -15516 * 2,

-8972 * 2,    -12682 * 2,    -14666 * 2,    -15233 * 2,

-10771 * 2,    -13173 * 2,    -14355 * 2,    -15621 * 2,

-10297 * 2,    -13564 * 2,    -14733 * 2,    -15734 * 2,

-10822 * 2,    -12937 * 2,    -14477 * 2,    -15042 * 2,

-11142 * 2,    -12785 * 2,    -15139 * 2,    -15905 * 2,

-10101 * 2,    -12649 * 2,    -14283 * 2,    -14883 * 2,

-10035 * 2,    -12902 * 2,    -13731 * 2,    -15264 * 2,

-11322 * 2,    -13805 * 2,    -15031 * 2,    -15382 * 2,

-12119 * 2,    -13184 * 2,    -14533 * 2,    -15272 * 2,

-10453 * 2,    -12308 * 2,    -14055 * 2,    -14623 * 2,

-10909 * 2,    -12027 * 2,    -13531 * 2,    -15149 * 2,

-10569 * 2,    -12502 * 2,    -13783 * 2,    -14390 * 2,

-12295 * 2,    -13452 * 2,    -14519 * 2,    -16014 * 2,

-9070 * 2,    -13916 * 2,    -15100 * 2,    -15778 * 2,

-9173 * 2,    -13031 * 2,    -15055 * 2,    -16027 * 2,

-10759 * 2,    -13738 * 2,    -15398 * 2,    -16195 * 2,

-10454 * 2,    -11762 * 2,    -15233 * 2,    -16185 * 2,

-8931 * 2,    -13657 * 2,    -15532 * 2,    -15997 * 2,

-10846 * 2,    -14286 * 2,    -15519 * 2,    -16176 * 2,

-7775 * 2,    -10326 * 2,    -15224 * 2,    -15794 * 2,

-8874 * 2,    -10860 * 2,    -13493 * 2,    -15558 * 2,

-7842 * 2,    -10236 * 2,    -14047 * 2,    -15982 * 2,

-8241 * 2,     -9518 * 2,    -14118 * 2,    -15253 * 2,

-7417 * 2,     -8839 * 2,    -14502 * 2,    -15988 * 2,

-6511 * 2,     -8764 * 2,    -14745 * 2,    -15662 * 2,

-6592 * 2,     -9218 * 2,    -13452 * 2,    -15816 * 2,

-8666 * 2,    -10330 * 2,    -12824 * 2,    -15364 * 2,

-6878 * 2,     -8214 * 2,    -14008 * 2,    -15242 * 2,

-6472 * 2,     -9944 * 2,    -12580 * 2,    -15630 * 2,

-6093 * 2,     -7786 * 2,    -14146 * 2,    -15705 * 2,

-7140 * 2,     -9175 * 2,    -12134 * 2,    -15479 * 2,

-7465 * 2,    -10291 * 2,    -12240 * 2,    -15235 * 2,

-6556 * 2,     -8071 * 2,    -13251 * 2,    -14474 * 2,

-6529 * 2,     -8031 * 2,    -12129 * 2,    -14681 * 2,

-5876 * 2,    -10227 * 2,    -11466 * 2,    -15307 * 2,

-5654 * 2,     -6943 * 2,    -12564 * 2,    -14736 * 2,

-6353 * 2,     -7469 * 2,    -12201 * 2,    -14350 * 2,

-5627 * 2,     -7728 * 2,    -11689 * 2,    -15788 * 2,

-5197 * 2,     -6409 * 2,    -11655 * 2,    -14597 * 2,

-4239 * 2,     -6118 * 2,    -11432 * 2,    -15123 * 2,

-4436 * 2,     -9115 * 2,    -10839 * 2,    -15679 * 2,

-3548 * 2,     -8408 * 2,    -10345 * 2,    -15621 * 2,

-4227 * 2,     -7315 * 2,     -9749 * 2,    -15819 * 2,

-3051 * 2,     -6359 * 2,    -10763 * 2,    -15801 * 2,

-1744 * 2,     -8688 * 2,    -11352 * 2,    -15519 * 2,

-1507 * 2,     -7789 * 2,    -12284 * 2,    -15213 * 2,

-898 * 2,     -8081 * 2,    -12437 * 2,    -15816 * 2,

-1027 * 2,    -10079 * 2,    -11965 * 2,    -15213 * 2,

-734 * 2,     -7974 * 2,    -12549 * 2,    -14605 * 2,

827 * 2,     -7472 * 2,    -12192 * 2,    -15324 * 2,

-511 * 2,     -9088 * 2,    -11661 * 2,    -14363 * 2,

-1296 * 2,     -8962 * 2,    -13040 * 2,    -14289 * 2,

-1798 * 2,     -7790 * 2,    -13720 * 2,    -14889 * 2,

-1027 * 2,     -7855 * 2,    -13842 * 2,    -15336 * 2,

-438 * 2,     -6447 * 2,    -13350 * 2,    -15000 * 2,

-1645 * 2,     -7871 * 2,    -13618 * 2,    -14276 * 2,

-1796 * 2,     -7857 * 2,    -14187 * 2,    -14647 * 2,

1419 * 2,     -7430 * 2,    -13326 * 2,    -14604 * 2,

968 * 2,     -8714 * 2,    -14064 * 2,    -14746 * 2,

-1389 * 2,     -8443 * 2,    -14591 * 2,    -15035 * 2,

-2376 * 2,     -6547 * 2,    -14289 * 2,    -15131 * 2,

681 * 2,     -6039 * 2,    -14174 * 2,    -15261 * 2,

-1278 * 2,     -7426 * 2,    -14995 * 2,    -15460 * 2,

-1650 * 2,     -5306 * 2,    -14072 * 2,    -15417 * 2,

1179 * 2,     -5908 * 2,    -12475 * 2,    -14644 * 2,

156 * 2,     -7873 * 2,    -12697 * 2,    -13694 * 2,

-624 * 2,     -4770 * 2,    -12873 * 2,    -15838 * 2,

-3129 * 2,     -4907 * 2,    -13467 * 2,    -15778 * 2,

-4247 * 2,     -5883 * 2,    -14196 * 2,    -15500 * 2,

-4086 * 2,     -7081 * 2,    -14616 * 2,    -15684 * 2,

-2201 * 2,     -7651 * 2,    -12663 * 2,    -13344 * 2,

-3453 * 2,     -6630 * 2,    -12004 * 2,    -12864 * 2,

856 * 2,     -8323 * 2,    -11736 * 2,    -13622 * 2,

-2646 * 2,     -7110 * 2,    -11587 * 2,    -12505 * 2,

-5336 * 2,     -7532 * 2,    -11484 * 2,    -12670 * 2,

-5033 * 2,     -8705 * 2,    -10678 * 2,    -12334 * 2,

-4970 * 2,     -7667 * 2,    -10717 * 2,    -11807 * 2,

-5795 * 2,     -8993 * 2,    -11290 * 2,    -12255 * 2,

-4249 * 2,     -8531 * 2,     -9938 * 2,    -12187 * 2,

-5502 * 2,     -7806 * 2,     -9865 * 2,    -12731 * 2,

-5921 * 2,     -8823 * 2,    -10290 * 2,    -13093 * 2,

-6168 * 2,     -8325 * 2,    -10247 * 2,    -11776 * 2,

-6218 * 2,     -7581 * 2,     -9573 * 2,    -12938 * 2,

-4793 * 2,     -7741 * 2,     -9545 * 2,    -11334 * 2,

-6263 * 2,     -9720 * 2,    -11069 * 2,    -12886 * 2,

-6931 * 2,     -9690 * 2,    -11618 * 2,    -12771 * 2,

-4180 * 2,     -6051 * 2,     -8002 * 2,    -10238 * 2,

-2494 * 2,     -5755 * 2,     -7791 * 2,    -13517 * 2,

-2327 * 2,     -4509 * 2,     -8991 * 2,    -12941 * 2,

-2882 * 2,     -5563 * 2,     -7443 * 2,    -10727 * 2,

-1012 * 2,     -5319 * 2,     -9297 * 2,    -12505 * 2,

-1512 * 2,     -5510 * 2,     -7537 * 2,    -11602 * 2,

-1865 * 2,     -4284 * 2,     -8937 * 2,    -11243 * 2,

-675 * 2,     -5828 * 2,     -9165 * 2,    -11035 * 2,

121 * 2,     -5394 * 2,     -8187 * 2,    -12358 * 2,

-1828 * 2,     -3798 * 2,     -7161 * 2,    -12607 * 2,

-1250 * 2,     -3403 * 2,     -8971 * 2,    -12892 * 2,

82 * 2,     -3228 * 2,     -8165 * 2,    -12383 * 2,

-205 * 2,     -4263 * 2,     -9511 * 2,    -13198 * 2,

-970 * 2,     -4326 * 2,    -10376 * 2,    -12376 * 2,

771 * 2,     -5818 * 2,     -9306 * 2,    -12364 * 2,

1200 * 2,     -4560 * 2,     -9731 * 2,    -11691 * 2,

373 * 2,     -3855 * 2,    -10589 * 2,    -11908 * 2,

-806 * 2,     -2391 * 2,     -9914 * 2,    -12909 * 2,

-340 * 2,     -5531 * 2,    -10039 * 2,    -11121 * 2,

-29 * 2,     -4047 * 2,     -9090 * 2,    -10202 * 2,

-118 * 2,     -6292 * 2,     -9474 * 2,    -13728 * 2,

2151 * 2,     -6644 * 2,    -10546 * 2,    -13000 * 2,

3581 * 2,     -3940 * 2,    -10803 * 2,    -12157 * 2,

281 * 2,     -3753 * 2,     -9785 * 2,    -14179 * 2,

-970 * 2,     -4782 * 2,     -7680 * 2,    -13979 * 2,

616 * 2,     -2056 * 2,     -9585 * 2,    -14312 * 2,

301 * 2,     -1319 * 2,     -7377 * 2,    -12987 * 2,

-2860 * 2,     -4579 * 2,     -7377 * 2,    -13190 * 2,

1579 * 2,     -3254 * 2,     -5673 * 2,    -12155 * 2,

-670 * 2,     -3002 * 2,     -6917 * 2,    -10909 * 2,

1148 * 2,      -112 * 2,     -6676 * 2,    -13294 * 2,

-1456 * 2,     -4559 * 2,     -6359 * 2,    -11251 * 2,

2315 * 2,     -4495 * 2,     -6938 * 2,    -10622 * 2,

-910 * 2,     -4965 * 2,     -7196 * 2,     -9729 * 2,

-3313 * 2,     -5078 * 2,     -7349 * 2,     -9257 * 2,

2995 * 2,        30 * 2,     -6398 * 2,    -14337 * 2,

2728 * 2,      1352 * 2,     -6451 * 2,    -14372 * 2,

4050 * 2,     -5770 * 2,    -10681 * 2,    -14211 * 2,

3555 * 2,      2365 * 2,     -8304 * 2,    -14656 * 2,

3378 * 2,       470 * 2,    -11318 * 2,    -14972 * 2,

4295 * 2,       920 * 2,    -11156 * 2,    -14996 * 2,

4690 * 2,      2206 * 2,    -10216 * 2,    -14946 * 2,

5240 * 2,       661 * 2,    -11119 * 2,    -14998 * 2,

6099 * 2,      1695 * 2,     -9999 * 2,    -14947 * 2,

4890 * 2,     -1618 * 2,    -11589 * 2,    -15054 * 2,

6603 * 2,     -2072 * 2,    -11463 * 2,    -15067 * 2,

5215 * 2,      3325 * 2,     -9190 * 2,    -14925 * 2,

4800 * 2,     -1018 * 2,    -12645 * 2,    -13591 * 2,

5953 * 2,      3144 * 2,     -7673 * 2,    -14850 * 2,

4467 * 2,      3391 * 2,     -6908 * 2,    -14827 * 2,

6908 * 2,      4412 * 2,     -7205 * 2,    -14816 * 2,

5090 * 2,      3105 * 2,     -5817 * 2,    -14749 * 2,

5482 * 2,      3606 * 2,     -3761 * 2,    -14629 * 2,

4139 * 2,      2850 * 2,     -1782 * 2,    -14239 * 2,

5611 * 2,      4106 * 2,      -669 * 2,    -14497 * 2,

7300 * 2,      3950 * 2,       736 * 2,    -13497 * 2,

5376 * 2,     -5185 * 2,    -11826 * 2,    -15194 * 2,

2924 * 2,     -6522 * 2,    -12052 * 2,    -15281 * 2,

-6084 * 2,    -11147 * 2,    -12964 * 2,    -14025 * 2,

-7054 * 2,    -11706 * 2,    -13153 * 2,    -14339 * 2 };

/************************************************************************

*

*	FILENAME		:	ener_qua.tab

*

*	DESCRIPTION		:	Energy codebook in Q8.

*

************************************************************************/

#define nb_qua_ener 64

static Word16 t_qua_ener[64*2] = {

12,       48,

69,      287,

98,      589,

103,      865,

406,      561,

95,     1161,

418,      913,

465,     1288,

705,     1056,

822,      699,

1036,     1016,

842,     1353,

555,     1646,

681,     2119,

891,     1758,

1071,     1529,

1236,     1281,

1455,      962,

1661,     1279,

1416,     1514,

1243,     1768,

1110,     2088,

1349,     2406,

1480,     2035,

1558,     1762,

1770,     1554,

2027,     1319,

1952,     1030,

2281,      914,

2337,     1209,

2451,     1441,

2234,     1441,

2033,     1592,

2279,     1644,

2109,     1808,

2319,     1859,

2469,     1656,

2533,     1857,

2414,     2084,

2114,     2087,

1834,     1887,

1769,     2242,

1977,     2560,

2278,     2344,

2343,     2749,

2661,     2416,

2857,     2037,

3239,     2405,

2848,     2836,

3426,     2863,

3475,     3416,

2831,     3409,

2410,     3322,

2033,     3303,

1609,     3514,

1625,     2836,

880,     2775,

1122,     3500,

605,     3517,

88,     3532,

111,     2956,

89,     2436,

88,     1938,

74,     1526};

/************************************************************************

*

*	FILENAME		:	grid.tab

*

*	DESCRIPTION		:	Table for routine Az_Lsp().

*

*					Vector grid[] is in Q15

*

*						grid[0] = 1.0

*						grid[grid_points+1] = -1.0

*						for (i = 1; i < grid_points; i++)

*						grid[i] = cos((6.283185307*i)/(2.0*grid_points))

*

************************************************************************/

#define grid_points 60

static Word16 grid[grid_points+1] ={

32760,     32723,     32588,     32364,     32051,     31651,

31164,     30591,     29935,     29196,     28377,     27481,

26509,     25465,     24351,     23170,     21926,     20621,

19260,     17846,     16384,     14876,     13327,     11743,

10125,      8480,      6812,      5126,      3425,      1714,

-1,     -1715,     -3426,     -5127,     -6813,     -8481,

-10126,    -11744,    -13328,    -14877,    -16385,    -17847,

-19261,    -20622,    -21927,    -23171,    -24352,    -25466,

-26510,    -27482,    -28378,    -29197,    -29936,    -30592,

-31165,    -31652,    -32052,    -32365,    -32589,    -32724,

-32760};

/************************************************************************

*

*	FILENAME		:	inv_sqrt.tab

*

*	DESCRIPTION		:	Table for routine inv_sqrt().

*

************************************************************************/

static Word16 tab_inv_sqrt[49] = {

32767, 31790, 30894, 30070, 29309, 28602, 27945, 27330, 26755, 26214,

25705, 25225, 24770, 24339, 23930, 23541, 23170, 22817, 22479, 22155,

21845, 21548, 21263, 20988, 20724, 20470, 20225, 19988, 19760, 19539,

19326, 19119, 18919, 18725, 18536, 18354, 18176, 18004, 17837, 17674,

17515, 17361, 17211, 17064, 16921, 16782, 16646, 16514, 16384 };

/************************************************************************

*

*	FILENAME		:	lag_wind.tab

*

*	DESCRIPTION		:	Table of lag_window coefficents for the autocorrelation.

*

*						lag_wind[0] = 1.00000000 	(*)

*						lag_wind[1] = 0.99884027	(**)

*						lag_wind[2] = 0.99551868

*						lag_wind[3] = 0.99000722

*						lag_wind[4] = 0.98234236

*						lag_wind[5] = 0.97257471

*						lag_wind[6] = 0.96076828

*						lag_wind[7] = 0.94699991

*						lag_wind[8] = 0.93135828

*						lag_wind[9] = 0.91394323

*						lag_wind[10]= 0.89486438

*

*	COMMENTS		:	- (*) The first coefficient whose value = 1 is just

*					  mentioned for information, but not included in the code

*					- (**) All the other coefficents incorporate a scaling factor *					  of 1,00005 corresponding to a noise floor of -43 dB

*					- This table uses a special extended precision format

*					  (see "comments" in the header of

*					   the file fexp_tet.c - Section A.5.2)

*

************************************************************************/

static Word16 lag_h[10] = {

32729,

32621,

32440,

32189,

31869,

31482,

31031,

30518,

29948,

29322};

static Word16 lag_l[10] = {

32704,

5120,

18240,

12928,

10752,

14912,

9600,

24512,

3008,

30016};

/************************************************************************

*

*	FILENAME		:	log2.tab

*

*	DESCRIPTION		:	Table for routine Log2().

*

************************************************************************/

static Word16 tab_log2[33] = {

0,  1455,  2866,  4236,  5568,  6863,  8124,  9352, 10549, 11716,

12855, 13967, 15054, 16117, 17156, 18172, 19167, 20142, 21097, 22033,

22951, 23852, 24735, 25603, 26455, 27291, 28113, 28922, 29716, 30497,

31266, 32023, 32767 };

/************************************************************************

*

*	FILENAME		:	pow2.tab

*

*	DESCRIPTION		:	Table for routine pow2().

*

************************************************************************/

static Word16 tab_pow2[33] = {

16384, 16743, 17109, 17484, 17867, 18258, 18658, 19066, 19484, 19911,

20347, 20792, 21247, 21713, 22188, 22674, 23170, 23678, 24196, 24726,

25268, 25821, 26386, 26964, 27554, 28158, 28774, 29405, 30048, 30706,

31379, 32066, 32767 };

/************************************************************************

*

*	FILENAME		:	window.tab

*

*	DESCRIPTION		:	Asymetric Hamming window for LPC analysis.

*

************************************************************************/

#define L_window 256

static Word16 window[L_window] = {

2621,  2623,  2628,  2636,  2647,  2662,  2679,  2700,  2724,  2752,

2782,  2816,  2852,  2892,  2936,  2982,  3031,  3084,  3140,  3199,

3260,  3325,  3393,  3465,  3539,  3616,  3696,  3779,  3865,  3954,

4047,  4141,  4239,  4340,  4444,  4550,  4659,  4771,  4886,  5003,

5123,  5246,  5372,  5500,  5631,  5764,  5900,  6038,  6179,  6323,

6468,  6617,  6767,  6920,  7075,  7233,  7392,  7554,  7718,  7884,

8052,  8223,  8395,  8569,  8746,  8924,  9104,  9286,  9470,  9655,

9842, 10031, 10221, 10413, 10607, 10802, 10999, 11197, 11396, 11597,

11799, 12002, 12207, 12413, 12619, 12827, 13036, 13246, 13457, 13669,

13882, 14095, 14309, 14524, 14740, 14956, 15173, 15391, 15608, 15827,

16046, 16265, 16484, 16704, 16924, 17144, 17364, 17584, 17804, 18024,

18245, 18465, 18684, 18904, 19124, 19343, 19561, 19780, 19998, 20215,

20432, 20648, 20864, 21079, 21293, 21506, 21719, 21931, 22142, 22352,

22561, 22769, 22976, 23181, 23386, 23589, 23791, 23992, 24191, 24389,

24586, 24781, 24975, 25167, 25357, 25546, 25733, 25919, 26102, 26284,

26464, 26642, 26819, 26993, 27165, 27336, 27504, 27670, 27834, 27996,

28156, 28313, 28468, 28621, 28772, 28920, 29066, 29209, 29350, 29488,

29624, 29757, 29888, 30016, 30142, 30265, 30385, 30502, 30617, 30729,

30838, 30945, 31048, 31149, 31247, 31342, 31434, 31523, 31609, 31692,

31772, 31850, 31924, 31995, 32063, 32128, 32190, 32249, 32304, 32357,

32406, 32453, 32496, 32536, 32573, 32606, 32637, 32664, 32688, 32709,

32727, 32741, 32753, 32761, 32765, 32767, 32767, 32718, 32572, 32329,

31991, 31561, 31041, 30434, 29744, 28977, 28136, 27227, 26257, 25231,

24156, 23039, 21888, 20709, 19511, 18301, 17088, 15878, 14680, 13501,

12350, 11233, 10158,  9132,  8162,  7253,  6412,  5645,  4955,  4348,

3828,  3397,  3059,  2817,  2670,  2621 };

# B	C Code for the TETRA Basic Operators

/***********************************************************************

*

*	FILENAME		:	tetra_op.c

*

*	DESCRIPTION		:	TETRA  basic operators

*

************************************************************************

*

*	FUNCTIONS		:	- abs_s()

*					- add()

*					- div_s()

*					- extract_h()

*					- extract_l()

*					- L_abs()

*					- L_add()

*					- L_deposit_h()

*					- L_deposit_l()

*					- L_mac()

*					- L_mac0()

*					- L_msu()

*					- L_msu0()

*					- L_mult()

*					- L_mult0()

*					- L_negate()

*					- L_shl()

*					- L_shr()

*					- L_shr_r()

*					- L_sub()

*					- mult()

*					- mult_r()

*					- negate()

*					- norm_l()

*					- norm_s()

*					- round()

*					- sature()

*					- shl()

*					- shr()

*					- sub()

*

*	COMMENTS		:	Only the operators used in the actual version

*					of the TETRA codec are included in this file

*

************************************************************************

*

*	INCLUDED FILES	:	source.h

*					stdio.h

*					stdlib.h

*

************************************************************************/

#include <stdio.h>

#include <stdlib.h>

#include "source.h"

/*-----------------*

* Local Functions *

*-----------------*/

Word16 sature(Word32 L_var1);

/*-----------------------*

* Constants and Globals *

*-----------------------*/

Flag Overflow =0;

Flag Carry =0;

/************************************************************************

*

*	Function Name : abs_s

*

*	Purpose :

*

*		Absolute value of var1; abs_s(-32768) = 32767.

*

*	Complexity Weight : 1

*

*	Inputs :

*

*		var1

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var1 <= 0x0000 7fff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		var_out

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0x0000 0000 <= var_out <= 0x0000 7fff.

*

************************************************************************/

Word16 abs_s(Word16 var1)

{

Word16 var_out;

if (var1 == (Word16)0X8000 )

{

var_out = MAX_16;

}

else

{

if (var1 < 0)

{

var_out = -var1;

}

else

{

var_out = var1;

}

}

return(var_out);

}

/************************************************************************

*

*	Function Name : add

*

*	Purpose :

*

*		Performs the addition (var1+var2) with overflow control and saturation;|

*		the 16 bit result is set at +32767 when overflow occurs or at -32768

*		when underflow occurs.

*

*	Complexity Weight : 1

*

*	Inputs :

*

*		var1

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var1 <= 0x0000 7fff.

*

*		var2

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var2 <= 0x0000 7fff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		var_out

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var_out <= 0x0000 7fff.

*

************************************************************************/

Word16 add(Word16 var1,Word16 var2)

{

Word16 var_out;

Word32 L_somme;

L_somme = (Word32) var1 + var2;

var_out = sature(L_somme);

return(var_out);

}

/************************************************************************

*

*	Function Name : div_s

*

*	Purpose :

*

*		Produces a result which is the fractionnal integer division of var1 by

*		var2; var1 and var2 must be positive and var2 must be greater or equal

*		to var1; the result is positive (leading bit equal to 0) and truncated

*		to 16 bits.

*		If var1 = var2 then div(var1,var2) = 32767.

*

*	Complexity Weight : 18

*

*	Inputs :

*

*		var1

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0x0000 0000 <= var1 <= var2 and var2 != 0.

*

*		var2

*			16 bit short signed integer (Word16) whose value falls in the

*			range : var1 <= var2 <= 0x0000 7fff and var2 != 0.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		var_out

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0x0000 0000 <= var_out <= 0x0000 7fff.

*			It's a Q15 value (point between b15 and b14).

*

************************************************************************/

Word16 div_s(Word16 var1, Word16 var2)

{

Word16 var_out = 0;

Word16 iteration;

Word32 L_num;

Word32 L_denom;

if ((var1 > var2) || (var1 < 0) || (var2 < 0))

{

printf("Division Error\n");

exit(0);

}

if (var2 == 0)

{

printf("Division by 0, Fatal error \n");

exit(0);

}

if (var1 == 0)

{

var_out = 0;

}

else

{

if (var1 == var2)

{

var_out = MAX_16;

}

else

{

L_num = L_deposit_l(var1);

L_denom = L_deposit_l(var2);

for(iteration=0;iteration<15;iteration++)

{

var_out <<=1;

L_num <<= 1;

if (L_num >= L_denom)

{

L_num = L_sub(L_num,L_denom);

var_out = add( var_out,(Word16)1 );

}

}

}

}

return(var_out);

}

/************************************************************************

*

*	Function Name : extract_h

*

*	Purpose :

*

*		Return the 16 MSB of L_var1.

*

*	Complexity Weight : 1

*

*	Inputs :

*

*		L_var1

*			32 bit long signed integer (Word32 ) whose value falls in the

*			range : 0x8000 0000 <= L_var1 <= 0x7fff ffff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		var_out

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var_out <= 0x0000 7fff.

*

************************************************************************/

Word16 extract_h(Word32 L_var1)

{

Word16 var_out;

var_out = (Word16) (L_var1 >> 16);

return(var_out);

}

/************************************************************************

*

*	Function Name : extract_l

*

*	Purpose :

*

*		Return the 16 LSB of L_var1.

*

*	Complexity Weight : 1

*

*	Inputs :

*

*		L_var1

*			32 bit long signed integer (Word32 ) whose value falls in the

*			range : 0x8000 0000 <= L_var1 <= 0x7fff ffff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		var_out

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var_out <= 0x0000 7fff.

*

************************************************************************/

Word16 extract_l(Word32 L_var1)

{

Word16 var_out;

var_out = (Word16) L_var1;

return(var_out);

}

/************************************************************************

*

*	Function Name : L_abs

*

*	Purpose :

*

*		Absolute value of L_var1; Saturate in case where the input is -214783648

*

*	Complexity Weight : 3

*

*	Inputs :

*

*		L_var1

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var1 <= 0x7fff ffff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_var_out

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x0000 0000 <= L_var_out <= 0x7fff ffff.

*

************************************************************************/

Word32 L_abs(Word32 L_var1)

{

Word32 L_var_out;

if (L_var1 == MIN_32)

{

L_var_out = MAX_32;

}

else

{

if (L_var1 < 0)

{

L_var_out = -L_var1;

}

else

{

L_var_out = L_var1;

}

}

return(L_var_out);

}

/************************************************************************

*

*	Function Name : L_add

*

*	Purpose :

*

*		32 bits addition of the two 32 bits variables (L_var1+L_var2) with

*		overflow control and saturation; the result is set at +214783647 when

*		overflow occurs or at -214783648 when underflow occurs.

*

*	Complexity Weight : 2

*

*	Inputs :

*

*		L_var1

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var1 <= 0x7fff ffff.

*

*		L_var2

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var2 <= 0x7fff ffff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_var_out

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var_out <= 0x7fff ffff.

*

************************************************************************/

Word32 L_add(Word32 L_var1, Word32 L_var2)

{

Word32 L_var_out;

L_var_out = L_var1 + L_var2;

if (((L_var1 ^ L_var2) & MIN_32) == 0)

{

if ((L_var_out ^ L_var1) & MIN_32)

{

L_var_out = (L_var1 < 0) ? MIN_32 : MAX_32;

Overflow = 1;

}

}

return(L_var_out);

}

/************************************************************************

*

*	Function Name : L_deposit_h

*

*	Purpose :

*

*		Deposit the 16 bit var1 into the 16 MS bits of the 32 bit output. The

*		16 LS bits of the output are zeroed.

*

*	Complexity Weight : 2

*

*	Inputs :

*

*		var1

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var1 <= 0x0000 7fff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_var_out

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= var_out <= 0x7fff 0000.

*

************************************************************************/

Word32 L_deposit_h(Word16 var1)

{

Word32 L_var_out;

L_var_out = (Word32) var1 << 16;

return(L_var_out);

}

/************************************************************************

*

*	Function Name : L_deposit_l

*

*	Purpose :

*

*		Deposit the 16 bit var1 into the 16 LS bits of the 32 bit output. The

*		16 MS bits of the output are sign extended.

*

*	Complexity Weight : 2

*

*	Inputs :

*

*		var1

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var1 <= 0x0000 7fff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_var_out

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0xFFFF 8000 <= L_var_out <= 0x0000 7fff.

*

************************************************************************/

Word32 L_deposit_l(Word16 var1)

{

Word32 L_var_out;

L_var_out = (Word32) var1;

return(L_var_out);

}

/************************************************************************

*

*	Function Name : L_mac

*

*	Purpose :

*

*		Multiply var1 by var2 and shift the result left by 1. Add the 32 bit

*		result to L_var3 with saturation, return a 32 bit result:

*			L_mac(L_var3,var1,var2) = L_add(L_var3,(L_mult(var1,var2)).

*

*	Complexity Weight : 1

*

*	Inputs :

*

*		L_var3

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var3 <= 0x7fff ffff.

*

*		var1

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var1 <= 0x0000 7fff.

*

*		var2

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var2 <= 0x0000 7fff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_var_out

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var_out <= 0x7fff ffff.

*

************************************************************************/

Word32 L_mac(Word32 L_var3, Word16 var1, Word16 var2)

{

Word32 L_var_out;

Word32 L_produit;

L_produit = L_mult(var1,var2);

L_var_out = L_add(L_var3,L_produit);

return(L_var_out);

}

/************************************************************************

*

*	Function Name : L_mac0

*

*	Purpose :

*

*		Multiply var1 by var2. Add the 32 bit result to L_var3.

*		Control saturation and set overflow_flag.

*		Same as L_mac without left shift.

*

*	Complexity Weight : 1

*

*	Inputs :

*

*		L_var3

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var3 <= 0x7fff ffff.

*

*		var1

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var1 <= 0x0000 7fff.

*

*		var2

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var2 <= 0x0000 7fff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_var_out

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var_out <= 0x7fff ffff.

*

************************************************************************/

Word32 L_mac0(Word32 L_var3, Word16 var1, Word16 var2)

{

Word32 L_var_out;

Word32 L_produit;

L_produit = L_mult0(var1,var2);

L_var_out = L_add(L_var3,L_produit);

return(L_var_out);

}

/************************************************************************

*

*	Function Name : L_msu

*

*	Purpose :

*

*		Multiply var1 by var2 and shift the result left by 1. Subtract the 32

*		bit result to L_var3 with saturation, return a 32 bit result:

*			L_msu(L_var3,var1,var2) = L_sub(L_var3,(L_mult(var1,var2)).

*

*	Complexity Weight : 1

*

*	Inputs :

*

*		L_var3

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var3 <= 0x7fff ffff.

*

*		var1

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var1 <= 0x0000 7fff.

*

*		var2

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var2 <= 0x0000 7fff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_var_out

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var_out <= 0x7fff ffff.

*

************************************************************************/

Word32 L_msu(Word32 L_var3, Word16 var1, Word16 var2)

{

Word32 L_var_out;

Word32 L_produit;

L_produit = L_mult(var1,var2);

L_var_out = L_sub(L_var3,L_produit);

return(L_var_out);

}

/************************************************************************

*

*	Function Name : L_msu0

*

*	Purpose :

*

*		Multiply var1 by var2. Subtract the 32 bit result to L_var3.

*		Control saturation and set overflow_flag.

*		Same as L_msu without left shift.

*

*	Complexity Weight : 1

*

*	Inputs :

*

*		L_var3

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var3 <= 0x7fff ffff.

*

*		var1

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var1 <= 0x0000 7fff.

*

*		var2

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var2 <= 0x0000 7fff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_var_out

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var_out <= 0x7fff ffff.

*

************************************************************************/

Word32 L_msu0(Word32 L_var3, Word16 var1, Word16 var2)

{

Word32 L_var_out;

Word32 L_produit;

L_produit = L_mult0(var1,var2);

L_var_out = L_sub(L_var3,L_produit);

return(L_var_out);

}

/************************************************************************

*

*	Function Name : L_mult

*

*	Purpose :

*

*		L_mult is the 32 bit result of the multiplication of var1 times var2

*		with one shift left i.e.:

*			L_mult(var1,var2) = shl((var1 times var2),1) and

*			L_mult(-32768,-32768) = 2147483647.

*

*	Complexity Weight : 1

*

*	Inputs :

*

*		var1

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var1 <= 0x0000 7fff.

*

*		var2

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var2 <= 0x0000 7fff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_var_out

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var_out <= 0x7fff ffff.

*

************************************************************************/

Word32 L_mult(Word16 var1,Word16 var2)

{

Word32 L_var_out;

L_var_out = (Word32)var1 * (Word32)var2;

if (L_var_out != (Word32)0x40000000)

{

L_var_out *= 2;

}

else

{

Overflow = 1;

L_var_out = MAX_32;

}

return(L_var_out);

}

/************************************************************************

*

*	Function Name : L_mult0

*

*	Purpose :

*

*		L_mult is the 32 bit result of the multiplication of var1 times var2.

*		Same as L_mult without left shift.

*

*	Complexity Weight : 1

*

*	Inputs :

*

*		var1

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var1 <= 0x0000 7fff.

*

*		var2

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var2 <= 0x0000 7fff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_var_out

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var_out <= 0x7fff ffff.

*

************************************************************************/

Word32 L_mult0(Word16 var1, Word16 var2)

{

Word32 L_var_out;

L_var_out = (Word32)var1 * (Word32)var2;

return(L_var_out);

}

/************************************************************************

*

*	Function Name : L_negate

*

*	Purpose :

*

*		Negate the 32 bit variable L_var1 with saturation; saturate in the case

*		where input is -2147483648 (0x8000 0000).

*

*	Complexity Weight : 2

*

*	Inputs :

*

*		L_var1

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var1 <= 0x7fff ffff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_var_out

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var_out <= 0x7fff ffff.

*

************************************************************************/

Word32 L_negate(Word32 L_var1)

{

Word32 L_var_out;

L_var_out = (L_var1 == MIN_32) ? MAX_32 : -L_var1;

return(L_var_out);

}

/************************************************************************

*

*	Function Name : L_shl

*

*	Purpose :

*

*		Arithmetically shift the 32 bit input L_var1 left var2 positions. Zero fill the var2 LSB *		of the result. If var2 is negative, L_var1 right by -var2 arithmetically shift with sign

*		extension. Saturate the result in case of underflows or overflows.

*

*	Complexity Weight : 2

*

*	Inputs :

*

*		L_var1

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var1 <= 0x7fff ffff.

*

*		var2

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var2 <= 0x0000 7fff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_var_out

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var_out <= 0x7fff ffff.

*

************************************************************************/

Word32 L_shl(Word32 L_var1, Word16 var2)

{

Word32 L_var_out;

if (var2 <= 0)

{

L_var_out = L_shr( L_var1, (Word16)(-var2) );

}

else

{

for(;var2>0;var2--)

{

if (L_var1 > (Word32) 0X3fffffff)

{

Overflow = 1;

L_var_out = MAX_32;

break;

}

else

{

if (L_var1 < (Word32) 0xc0000000)

{

Overflow = 1;

L_var_out = MIN_32;

break;

}

}

L_var1 *= 2;

L_var_out = L_var1;

}

}

return(L_var_out);

}

/************************************************************************

*

*	Function Name : L_shr

*

*	Purpose :

*

*		Arithmetically shift the 32 bit input L_var1 right var2 positions with

*		sign extension. If var2 is negative, arithmetically shift L_var1 left

*		by -var2 and zero fill the var2 LSB of the result. Saturate the result

*		in case of underflows or overflows.

*

*	Complexity Weight : 2

*

*	Inputs :

*

*		L_var1

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var1 <= 0x7fff ffff.

*

*		var2

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var2 <= 0x0000 7fff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_var_out

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var_out <= 0x7fff ffff.

*

************************************************************************/

Word32 L_shr(Word32 L_var1, Word16 var2)

{

Word32 L_var_out;

if (var2 < 0)

{

L_var_out = L_shl( L_var1,(Word16)(-var2) );

}

else

{

if (var2 >= 31)

{

L_var_out = (L_var1 < 0L) ? -1 : 0;

}

else

{

if (L_var1<0)

{

L_var_out = ~((~L_var1) >> var2);

}

else

{

L_var_out = L_var1 >> var2;

}

}

}

return(L_var_out);

}

/************************************************************************

*

*	Function Name : L_shr_r

*

*	Purpose :

*

*		Same as L_shr(L_var1,var2)but with rounding. Saturate the result in case|

*		of underflows or overflows :

*		If var2 is greater than zero :

*			L_shr_r(var1,var2) = L_shr(L_add(L_var1,2**(var2-1)),var2)

*		If var2 is less than zero :

*			L_shr_r(var1,var2) = L_shr(L_var1,var2).

*

*	Complexity Weight : 3

*

*	Inputs :

*

*		L_var1

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var1 <= 0x7fff ffff.

*

*		var2

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var2 <= 0x0000 7fff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_var_out

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var_out <= 0x7fff ffff.

*

************************************************************************/

Word32 L_shr_r(Word32 L_var1,Word16 var2)

{

Word32 L_var_out;

if (var2 > 31)

{

L_var_out = 0;

}

else

{

L_var_out = L_shr(L_var1,var2);

if (var2 > 0)

{

if ( (L_var1 & ( (Word32)1 << (var2-1) )) != 0)

{

L_var_out++;

}

}

}

return(L_var_out);

}

/************************************************************************

*

*	Function Name : L_sub

*

*	Purpose :

*

*		32 bits subtraction of the two 32 bits variables (L_var1-L_var2) with

*		overflow control and saturation; the result is set at +214783647 when

*		overflow occurs or at -214783648 when underflow occurs.

*

*	Complexity Weight : 2

*

*	Inputs :

*

*		L_var1

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var1 <= 0x7fff ffff.

*

*		L_var2

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var2 <= 0x7fff ffff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_var_out

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var_out <= 0x7fff ffff.

*

************************************************************************/

Word32 L_sub(Word32 L_var1, Word32 L_var2)

{

Word32 L_var_out;

L_var_out = L_var1 - L_var2;

if (((L_var1 ^ L_var2) & MIN_32) != 0)

{

if ((L_var_out ^ L_var1) & MIN_32)

{

L_var_out = (L_var1 < 0L) ? MIN_32 : MAX_32;

Overflow = 1;

}

}

return(L_var_out);

}

/************************************************************************

*

*	Function Name : mult

*

*	Purpose :

*

*		Performs the multiplication of var1 by var2 and gives a 16 bit result

*		which is scaled i.e.:

*			mult(var1,var2) = shr((var1 times var2),15) and

*			mult(-32768,-32768) = 32767.

*

*	Complexity Weight : 1

*

*	Inputs :

*

*		var1

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var1 <= 0x0000 7fff.

*

*		var2

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var2 <= 0x0000 7fff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		var_out

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var_out <= 0x0000 7fff.

*

************************************************************************/

Word16 mult(Word16 var1, Word16 var2)

{

Word16 var_out;

Word32 L_produit;

L_produit = (Word32)var1 * (Word32)var2;

L_produit = (L_produit & (Word32) 0xffff8000) >> 15;

if (L_produit & (Word32) 0x00010000)

L_produit = L_produit | (Word32) 0xffff0000;

var_out = sature(L_produit);

return(var_out);

}

/************************************************************************

*

*	Function Name : mult_r

*

*	Purpose :

*

*		Same as mult with rounding, i.e.:

*			mult_r(var1,var2) = shr(((var1*var2) + 16384),15) and

*			mult_r(-32768,-32768) = 32767.

*

*	Complexity Weight : 2

*

*	Inputs :

*

*		var1

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var1 <= 0x0000 7fff.

*

*		var2

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var2 <= 0x0000 7fff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		var_out

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var_out <= 0x0000 7fff.

*

************************************************************************/

Word16 mult_r(Word16 var1, Word16 var2)

{

Word16 var_out;

Word32 L_produit_arr;

L_produit_arr = (Word32)var1 * (Word32)var2; /* product */

L_produit_arr += (Word32) 0x00004000;        /* round */

L_produit_arr &= (Word32) 0xffff8000;

L_produit_arr >>= 15;                        /* shift */

if (L_produit_arr & (Word32) 0x00010000)   /*sign extend when necessary*/

{

L_produit_arr |= (Word32) 0xffff0000;

}

var_out = sature(L_produit_arr);

return(var_out);

}

/************************************************************************

*

*	Function Name : negate

*

*	Purpose :

*

*		Negate var1 with saturation, saturate in the case where input is -32768:

*			negate(var1) = sub(0,var1).

*

*	Complexity Weight : 1

*

*	Inputs :

*

*		var1

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var1 <= 0x0000 7fff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		var_out

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var_out <= 0x0000 7fff.

*

************************************************************************/

Word16 negate(Word16 var1)

{

Word16 var_out;

var_out = (var1 == MIN_16) ? MAX_16 : -var1;

return(var_out);

}

/************************************************************************

*

*	Function Name : norm_l

*

*	Purpose :

*

*		Produces the number of left shift needed to normalize the 32 bit variable

*		L_var1 for positive values on the interval with minimum of

*		1073741824 and maximum of 2147483647, and for negative values on the

*		interval with minimum of -2147483648 and maximum of -1073741824; in order

*		to normalize the result, the following operation must be done :

*			norm_L_var1 = L_shl(L_var1,norm_l(L_var1)).

*

*	Complexity Weight : 30

*

*	Inputs :

*

*		L_var1

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var1 <= 0x7fff ffff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		var_out

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0x0000 0000 <= var_out <= 0x0000 001f.

*

************************************************************************/

Word16 norm_l(Word32 L_var1)

{

Word16 var_out;

if (L_var1 == 0)

{

var_out = 0;

}

else

{

if (L_var1 == (Word32)0xffffffff)

{

var_out = 31;

}

else

{

if (L_var1 < 0)

{

L_var1 = ~L_var1;

}

for(var_out = 0;L_var1 < (Word32)0x40000000;var_out++)

{

L_var1 <<= 1;

}

}

}

return(var_out);

}

/************************************************************************

*

*	Function Name : norm_s

*

*	Purpose :

*

*		Produces the number of left shift needed to normalize the 16 bit variable

*		var1 for positive values on the interval with minimum of 16384 and

*		maximum of 32767, and for negative values on the interval with minimum

*		of -32768 and maximum of -16384; in order to normalize the result, the

*		following operation must be done :

*			norm_var1 = shl(var1,norm_s(var1)).

*

*	Complexity Weight : 15

*

*	Inputs :

*

*		var1

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var1 <= 0x0000 7fff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		var_out

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0x0000 0000 <= var_out <= 0x0000 000f.

*

************************************************************************/

Word16 norm_s(Word16 var1)

{

Word16 var_out;

if (var1 == 0)

{

var_out = 0;

}

else

{

if (var1 == (Word16) 0xffff)

{

var_out = 15;

}

else

{

if (var1 < 0)

{

var1 = ~var1;

}

for(var_out = 0; var1 < 0x4000; var_out++)

{

var1 <<= 1;

}

}

}

return(var_out);

}

/************************************************************************

*

*	Function Name : round

*

*	Purpose :

*

*		Round the lower 16 bits of the 32 bit input number into its MS 16 bits

*		with saturation. Shift the resulting bits right by 16 and return the 16

*		bit number:

*			round(L_var1) = extract_h(L_add(L_var1,32768))

*

*	Complexity Weight : 1

*

*	Inputs :

*

*		L_var1

*			32 bit long signed integer (Word32 ) whose value falls in the

*			range : 0x8000 0000 <= L_var1 <= 0x7fff ffff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		var_out

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var_out <= 0x0000 7fff.

*

************************************************************************/

Word16 round(Word32 L_var1)

{

Word16 var_out;

Word32 L_arrondi;

L_arrondi = L_add(L_var1, (Word32)0x00008000);

var_out = extract_h(L_arrondi);

return(var_out);

}

/************************************************************************

*

*	Function Name : sature

*

*	Purpose :

*

*		Limit the 32 bit input to the range of a 16 bit word.

*

*	Inputs :

*

*		L_var1

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var1 <= 0x7fff ffff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		var_out

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var_out <= 0x0000 7fff.

*

************************************************************************/

Word16 sature(Word32 L_var1)

{

Word16 var_out;

if (L_var1 > 0X00007fffL)

{

Overflow = 1;

var_out = MAX_16;

}

else if (L_var1 < (Word32)0xffff8000L)

{

Overflow = 1;

var_out = MIN_16;

}

else

{

Overflow = 0;

var_out = extract_l(L_var1);

}

return(var_out);

}

/************************************************************************

*

*	Function Name : shl

*

*	Purpose :

*

*		Arithmetically shift the 16 bit input var1 left var2 positions.Zero fill|

*		the var2 LSB of the result. If var2 is negative, arithmetically shift

*		var1 right by -var2 with sign extension. Saturate the result in case of

*		underflows or overflows.

*

*	Complexity Weight : 1

*

*	Inputs :

*

*		var1

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var1 <= 0x0000 7fff.

*

*		var2

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var2 <= 0x0000 7fff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		var_out

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var_out <= 0x0000 7fff.

*

************************************************************************/

Word16 shl(Word16 var1,Word16 var2)

{

Word16 var_out;

Word32 resultat;

if (var2 < 0)

{

var_out = shr( var1,(Word16)(-var2) );

}

else

{

resultat = (Word32) var1 * ((Word32) 1 << var2);

if ((var2 > 15 && var1 != 0) || (resultat != (Word32)((Word16) resultat)))

{

Overflow = 1;

var_out = (var1 > 0) ? MAX_16 : MIN_16;

}

else

{

var_out = extract_l(resultat);

}

}

return(var_out);

}

/************************************************************************

*

*	Function Name : shr

*

*	Purpose :

*

*		Arithmetically shift the 16 bit input var1 right var2 positions with

*		sign extension. If var2 is negative, arithmetically shift var1 left by

*		-var2 with sign extension. Saturate the result in case of underflows or

*		overflows.

*

*	Complexity Weight : 1

*

*	Inputs :

*

*		var1

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var1 <= 0x0000 7fff.

*

*		var2

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var2 <= 0x0000 7fff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		var_out

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var_out <= 0x0000 7fff.

*

************************************************************************/

Word16 shr(Word16 var1,Word16 var2)

{

Word16 var_out;

if (var2 < 0)

{

var_out = shl( var1,(Word16)(-var2) );

}

else

{

if (var2 >= 15)

{

var_out = (var1 < 0) ? -1 : 0;

}

else

{

if (var1 < 0)

{

var_out = ~(( ~var1) >> var2 );

}

else

{

var_out = var1 >> var2;

}

}

}

return(var_out);

}

/************************************************************************

*

*	Function Name : sub

*

*	Purpose :

*

*			Performs the subtraction (var1+var2) with overflow control and satu-

*			ration; the 16 bit result is set at +32767 when overflow occurs or at

*			-32768 when underflow occurs.

*

*	Complexity Weight : 1

*

*	Inputs :

*

*		var1

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var1 <= 0x0000 7fff.

*

*		var2

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var2 <= 0x0000 7fff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		var_out

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var_out <= 0x0000 7fff.

*

************************************************************************/

Word16 sub(Word16 var1,Word16 var2)

{

Word16 var_out;

Word32 L_diff;

L_diff = (Word32) var1 - var2;

var_out = sature(L_diff);

return(var_out);

}

# C	C Code for Speech Channel Coding

## C.1	Main Program for Speech Channel Coding

/************************************************************************

*

*	FILENAME		:	ccoder.					c

*

*	DESCRIPTION		:	Main program for speech channel encoding

*

************************************************************************

*

*	USAGE			:	ccoder serial_file cenc_file

*					(Executable_File Input1 Output1)

*

*	INPUT FILE(S)		:

*

*		INPUT1	:	- Description : serial stream input file

*					- Format : binary file 16 bit-samples

*					  each 16 bit-sample represents one encoded bit

*					  138 (= 1 + 137) bits per frame

*

*	OUTPUT FILE(S)	:

*

*		OUTPUT1	:	- Description : channel encoded serial stream

*					- Format : binary file 16 bit-samples

*					  each 16 bit-sample represents one encoded bit

*

*	COMMENTS		:	- Values of channel encoded samples are

*					either -127 or +127

*					- 2 input frames are encoded together to produce

*					1 single output frame of 432 samples written in

*					the ouput1 file according to a specific format

*					compatible with the error insertion device

*

************************************************************************

*

*	INCLUDED FILES	:	channel.h

*					stdlib.h

*

************************************************************************/

/* LIBRARIES USED */

#include <stdlib.h>

#include "channel.h"

#ifndef TRUE

# define TRUE   (1==1)

# define FALSE  (1==0)

#endif

Word16  FS_Flag = 0;	/* Frame Stealing Flag :

0 = no stealing in the time-slot,

!0 = stealing of the first speech frame

in the time-slot

This flag is set/reset by external world */

int    main( int argc, char *argv[] )

{

FILE    *fin, *fout;

Word32  Loop_counter = 0;

short   first_pass = TRUE;

Word16  i;

Word16  Vocod_array[274];       /* Input Buffer : 2 vocoder frames */

Word16  Coded_array[432];

Word16  Interleaved_coded_array[432]; /* Output Buffer */

/* Parse arguments */

if ( argc != 3 )

{

puts( "usage : ccoder infile outfile" );

puts( "Channel coding time-slot per time-slot" );

puts( "format for input file : 138 (BFI + 137) bits frame" );

puts( "format for output :  $6B21...114 bits...$6B22...114..." );

puts( "        ...$6B26...114...$6B21");

exit( 1 );

}

if ( (fin = fopen( argv[1], "rb" )) == NULL )

{

puts("chanlcod: can't open input file" );

exit( 1 );

}

if ( (fout = fopen( argv[2], "wb" )) == NULL )

{

puts("chanlcod: can't open output file" );

exit( 1 );

}

while( 1 )

{

/* Skip over bfi bit */

if( fread( Vocod_array, sizeof(short), 1, fin ) != 1 ) {

puts( "chanlcod: reached end of file" );

break;

}

/* 1st speech frame */

if( fread( Vocod_array, sizeof(short), 137, fin ) != 137 ) {

puts( "chanlcod: reached end of file" );

break;

}

/* Skip over bfi bit */

if( fread( Vocod_array+137, sizeof(short), 1, fin ) != 1 ) {

puts( "chanlcod: reached end of file" );

break;

}

/* 2nd speech frame */

if( fread( Vocod_array+137, sizeof(short), 137, fin ) != 137 ) {

puts( "chanlcod: reached end of file" );

break;

}

/* Channel Encoding */

Channel_Encoding(first_pass,FS_Flag,Vocod_array,Coded_array);

first_pass = FALSE;

/* Interleaving */

if (!FS_Flag) 	Interleaving_Speech(Coded_array,

Interleaved_coded_array);

else	{

Interleaving_Signalling(	Coded_array + 216,

Interleaved_coded_array + 216);

for (i = 0; i < 216; i++)	Interleaved_coded_array[i] =

Coded_array[i];

}

/* Increment Loop counter */

Loop_counter++;

/* write Output_array (1 TETRA frame = 2 speech frames) to output file */

if (Write_Tetra_File (fout, Interleaved_coded_array) == -1)

{

puts ("chanlcod: cannot write to output file");

break;

}

}

printf("%ld Speech Frames processed\n",2*Loop_counter);

printf("ie %ld Channel Frames\n",Loop_counter);

/* closing files */

fclose( fin );

fclose( fout );

return (EXIT_SUCCESS);

}

/************************************************************************

*	FILENAME		:	ccod_tet.c

*

*	DESCRIPTION		:	Main routine for speech channel encoding

*

************************************************************************

*

*	SUB-ROUTINES	:	- Channel_Encoding()

*

************************************************************************

*

*	INCLUDED FILES	:	channel.h

*					const.tab

*

************************************************************************/

#include "channel.h"

#include "const.tab"

/**************************************************************************

*

*	ROUTINE			:	Channel_Encoding

*

*	DESCRIPTION			:	Main speech channel coding function

*

**************************************************************************

*

*	USAGE				:	Channel_Encoding(first,flag,buffer_in, buffer_out)

*						(Routine_Name(input1,input2,input3,output1))

*

*	INPUT ARGUMENT(S)		:

*

*			INPUT1		:	- Description :	- True only if first call to this routine

*						- Format : Short

*

*			INPUT2		:	- Description :	- (flag = 0) : standard mode

*								- (flag  0) : frame stealing activated

*						- Format : Word16

*

*		INPUT3		:	- Description : two concatened speech frames

*						- Format : 274 * 16 bit-samples

*							   In case of frame stealing, only the second

*							  	   frame is encoded

*

*	OUTPUT ARGUMENT(S)	:

*

*		OUTPUT1		:	- Description : encoded frame

*							- Format : 432 * 16 bit-samples

*								   In case of frame stealing , only the second

*							   half is filled

*

*	RETURNED VALUE		:	None

*

**************************************************************************/

void Channel_Encoding(short first_pass, Word16 Frame_Stealing, Word16 Input_Frame[], Word16 Output_Frame[])

{

Word16  Ordered_array[286];     /* 2 vocoder frames + 8 + 4 */

Word16  i, Nber_Info_Bits;

/* Init for channel encoder */

if (!Frame_Stealing)

Nber_Info_Bits = N0_2 + N1_2 + N2_2 + SIZE_CRC;

else

Nber_Info_Bits = N0 + N1 + N2 + Fs_SIZE_CRC;

if ( first_pass ) Init_Rcpc_Coding(Frame_Stealing,

Ordered_array);

else {  /* Zero the (K - 1) last bits of Ordered_array */

for (i = 0; i < K - 1; i++)

Ordered_array[Nber_Info_Bits++] = 0;

}

if (!Frame_Stealing)

/* Ordering of the two speech frames in three classes */

Build_Sensitivity_Classes(Frame_Stealing, Input_Frame,

Ordered_array);

else

/* Ordering of the second speech frame in three classes */

Build_Sensitivity_Classes(Frame_Stealing,

Input_Frame + 137, Ordered_array);

/* "Coding" for non-protected class (class 0) : 0 --> 127, 1 --> -127 */

Transform_Class_0(Frame_Stealing, Ordered_array);

/* CRC computation on the last class (most sensitive bits) */

Build_Crc(Frame_Stealing, Ordered_array);

if (!Frame_Stealing)

/* Channel encoding of the 2 frames */

Rcpc_Coding(Frame_Stealing, Ordered_array, Output_Frame);

else

/* Channel encoding of 2nd frame */

Rcpc_Coding(Frame_Stealing, Ordered_array,

Output_Frame + 216);

return;

}

## C.2	Library of Speech Channel Coding Subroutines

/************************************************************************

*	FILENAME		:	sub_cc.c

*

*	DESCRIPTION		:	Sub-routines for speech channel encoding

*

************************************************************************

*

*	SUBROUTINES		:	- Build_Crc()

*					- Build_Sensitivity_Classes()

*					- Combination()

*					- Init_Rcpc_Coding()

*					- Interleaving_Signalling()

*					- Interleaving_Speech()

*					- Rcpc_Coding()

*					- Transform_Class_0()

*					- Write_Tetra_File()

*

************************************************************************

*

*	INCLUDED FILES	:	arrays.tab

*					channel.h

*					const.tab

*					stdlib.h

*

************************************************************************/

#include <stdlib.h>

#include "channel.h"

#include "const.tab" /* contains constants for channel coding/decoding */

#include "arrays.tab" /* contains arrays for channel coding/decoding */

/* #define DEBUG */	   /*	This is a compilation directive eventually used

in the routine Rcpc_Coding to check that the

puncturing matrices A1 and A2 are correctly set

The results given by this software shall be the

same, independently of the definition of DEBUG

The complexity of the channel encoding

component has been evaluated with the

assumption that DEBUG is NOT defined */

/**************************************************************************

*

*	ROUTINE				:	Build_Crc

*

*	DESCRIPTION			:	Computation of a 8 bit CRC and concatenation

*							to the input frame

*

**************************************************************************

*

*	USAGE				:	Build_Crc(flag,buffer)

*							(Routine_Name(arg1,arg2))

*

*	ARGUMENT(S)			:

*

*		ARG1				:	- Description :	- (flag = 0) : standard mode

*								- (flag  0) : frame stealing activated

*						- Format : Word16

*

*		ARG2				:	- Description : input serial stream containing

*						parameters of 2 speech frames re-ordered in 3

*						sensitivity classes

*							- Format :

*

*	RETURNED VALUE		:	None

*

*	COMMENTS			:	8 bit CRC is computed on the third sensitivity class

*

**************************************************************************/

Word16     Build_Crc(Word16 FS_Flag, Word16 Input_Frame[])

{

/* Variables */

Word16             i, temp;

if (!FS_Flag) {

/* The class protected by the CRC (Class 2) starts at index N0_2 + N1_2 */

/* First Bit of the CRC : */

temp = 0;

for (i = 0; i < SIZE_TAB_CRC1; i++)

temp = add(temp,Input_Frame[N0_2 + N1_2 + TAB_CRC1[i] - 1]);

/* The CRC bit is the LSB of temp : */

temp = temp & 1;

/* Concatenation of the CRC bit to Input_Frame : */

Input_Frame[N0_2 + N1_2 + N2_2] = temp;

/* Second Bit of the CRC : */

temp = 0;

for (i = 0; i < SIZE_TAB_CRC2; i++)

temp = add(temp,Input_Frame[N0_2 + N1_2 + TAB_CRC2[i] - 1]);

temp = temp & 1;

Input_Frame[N0_2 + N1_2 + N2_2 + 1] = temp;

/* Third Bit of the CRC : */

temp = 0;

for (i = 0; i < SIZE_TAB_CRC3; i++)

temp = add(temp,Input_Frame[N0_2 + N1_2 + TAB_CRC3[i] - 1]);

temp = temp & 1;

Input_Frame[N0_2 + N1_2 + N2_2 + 2] = temp;

/* Fourth Bit of the CRC : */

temp = 0;

for (i = 0; i < SIZE_TAB_CRC4; i++)

temp = add(temp,Input_Frame[N0_2 + N1_2 + TAB_CRC4[i] - 1]);

temp = temp & 1;

Input_Frame[N0_2 + N1_2 + N2_2 + 3] = temp;

/* Fifth Bit of the CRC : */

temp = 0;

for (i = 0; i < SIZE_TAB_CRC5; i++)

temp = add(temp,Input_Frame[N0_2 + N1_2 + TAB_CRC5[i] - 1]);

temp = temp & 1;

Input_Frame[N0_2 + N1_2 + N2_2 + 4] = temp;

/* Sixth Bit of the CRC : */

temp = 0;

for (i = 0; i < SIZE_TAB_CRC6; i++)

temp = add(temp,Input_Frame[N0_2 + N1_2 + TAB_CRC6[i] - 1]);

temp = temp & 1;

Input_Frame[N0_2 + N1_2 + N2_2 + 5] = temp;

/* Seventh Bit of the CRC : */

temp = 0;

for (i = 0; i < SIZE_TAB_CRC7; i++)

temp = add(temp,Input_Frame[N0_2 + N1_2 + TAB_CRC7[i] - 1]);

temp = temp & 1;

Input_Frame[N0_2 + N1_2 + N2_2 + 6] = temp;

/* Eighth Bit of the CRC : */

temp = 0;

for (i = 0; i < SIZE_TAB_CRC8; i++)

temp = add(temp,Input_Frame[N0_2 + N1_2 + TAB_CRC8[i] - 1]);

temp = temp & 1;

Input_Frame[N0_2 + N1_2 + N2_2 + 7] = temp;

return(0);

} /* If !FS_Flag */

else {

/* The class protected by the CRC (Class 2) starts at index N0 + N1 */

/* First Bit of the CRC : */

temp = 0;

for (i = 0; i < Fs_SIZE_TAB_CRC1; i++)

temp = add(temp,Input_Frame[N0 + N1 + Fs_TAB_CRC1[i] - 1]);

/* The CRC bit is the LSB of temp : */

temp = temp & 1;

/* Concatenation of the CRC bit to Input_Frame : */

Input_Frame[N0 + N1 + N2] = temp;

/* Second Bit of the CRC : */

temp = 0;

for (i = 0; i < Fs_SIZE_TAB_CRC2; i++)

temp = add(temp,Input_Frame[N0 + N1 + Fs_TAB_CRC2[i] - 1]);

temp = temp & 1;

Input_Frame[N0 + N1 + N2 + 1] = temp;

/* Third Bit of the CRC : */

temp = 0;

for (i = 0; i < Fs_SIZE_TAB_CRC3; i++)

temp = add(temp,Input_Frame[N0 + N1 + Fs_TAB_CRC3[i] - 1]);

temp = temp & 1;

Input_Frame[N0 + N1 + N2 + 2] = temp;

/* Fourth Bit of the CRC : */

temp = 0;

for (i = 0; i < Fs_SIZE_TAB_CRC4; i++)

temp = add(temp,Input_Frame[N0 + N1 + Fs_TAB_CRC4[i] - 1]);

temp = temp & 1;

Input_Frame[N0 + N1 + N2 + 3] = temp;

return(0);

}

}
/**************************************************************************

*

*	ROUTINE				:	Build_Sensitivity_Classes

*

*	DESCRIPTION			:	Reorders two concatened speech frames into a single

*						frame ordered in three sensitivity classes

*						(from least, ie Class 0, to most sensitive bits,

*						ie Class 2)

*

**************************************************************************

*

*	USAGE				:	Build_Sensitivity_Classes(flag,buffer_in, buffer_out)

*							(Routine_Name(input1,input2,output1))

*

*	INPUT ARGUMENT(S)		:

*

*		INPUT1			:	- Description :	- (flag = 0) : standard mode

*								- (flag  0) : frame stealing activated

*							- Format : Word16

*

*		INPUT2			:	- Description : 2 concatenated speech frames

*							- Format : 274 * 16 bit-samples

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : 1 reordered frame

*							- Format : 274 * 16 bit-samples

*

*	RETURNED VALUE		:	None

*

**************************************************************************/

Word16     Build_Sensitivity_Classes(Word16 FS_Flag, Word16 Input_Frame[], Word16 Output_Frame[])

{

/* Variables */

Word16             i;

if (!FS_Flag) {

/* Class 0 : */

for (i = 0; i < N0; i++) {

Output_Frame[2*i] = Input_Frame[TAB0[i] - 1];

Output_Frame[2*i + 1] = Input_Frame[Length_vocoder_frame + TAB0[i] - 1];

}

/* Building and Concatenation of Class 1 : */

for (i = 0; i < N1; i++) {

Output_Frame[2*(N0 + i)] = Input_Frame[TAB1[i] - 1];

Output_Frame[2*(N0 + i) + 1] = Input_Frame[Length_vocoder_frame + TAB1[i] - 1];

}

/* Building and Concatenation of Class 2 : */

for (i = 0; i < N2; i++) {

Output_Frame[2*(N0 + N1 + i)] = Input_Frame[TAB2[i] - 1];

Output_Frame[2*(N0 + N1 + i) + 1] = Input_Frame[Length_vocoder_frame + TAB2[i] - 1];

}

return(0);

} /* If !FS_Flag */

else {

/* Class 0 : */

for (i = 0; i < N0; i++)

Output_Frame[i] = Input_Frame[TAB0[i] - 1];

/* Building and Concatenation of Class 1 : */

for (i = 0; i < N1; i++)

Output_Frame[N0 + i] = Input_Frame[TAB1[i] - 1];

/* Building and Concatenation of Class 2 : */

for (i = 0; i < N2; i++)

Output_Frame[N0 + N1 + i] = Input_Frame[TAB2[i] - 1];

return(0);

}

}

/**************************************************************************

*

*	ROUTINE				:	Combination

*

*	DESCRIPTION			:	Computes the (convolutional) coded bit for a given

*							polynomial generator and a given state of the encoder

*

**************************************************************************

*

*	USAGE				:	Combination(gene,state)

*							(Routine_Name(input1,input2))

*

*	INPUT ARGUMENT(S)		:

*

*		INPUT1			:	- Description : Polynomial generator

*							- Format : Word16

*

*		INPUT2			:	- Description : State of the encoder

*							- Format : Word16

*

*	RETURNED VALUE		:	1 coded bit

*

**************************************************************************/

Word16     Combination(Word16 A, Word16 B)

{

Word16             Comb;

Word16             i, temp1, temp2, temp3;

Comb = -1;

temp1 = A & B;

for (i = 0;i <= (K - 1);i++)

{

temp2 = shl( (Word16)1,i );

temp3 = temp1 & temp2;

if (temp3 != 0) Comb = negate(Comb);

}

return(Comb);

}

/**************************************************************************

*

*	ROUTINE				:	Init_Rcpc_Coding

*

*	DESCRIPTION			:	Initialization for channel encoding

*

**************************************************************************

*

*	USAGE				:	Init_Rcpc_Coding(flag,buffer)

*							(Routine_Name(arg1,arg2))

*

*	ARGUMENT(S)			:

*

*		ARG1				:	- Description :	- (flag = 0) : standard mode

*								- (flag  0) : frame stealing activated

*						- Format : Word16

*

*		ARG2				:	- Description : Frame to be encoded

*							- Format : 286 * 16 bit-samples

*

*	RETURNED VALUE		:	None

*

*	COMMENTS			:	4 zeroes are concatenated to the input buffer

*							to clear the encoder

*

**************************************************************************/

void	Init_Rcpc_Coding(Word16 FS_Flag, Word16 Input_Frame[])

{

/* Variables */

Word16             i, M;

Word16             Arrival_state;  /* index for Loop on the Lattice States */

Word16             Starting_state;

Word16             Msb;    /* Value used for computation of coded bit */

Word16             Lsb;    /* Value used for computation of coded bit */

Word16             Lsb_bits;

Word16             Msbs_starting_state;

Word16             Involved_bits;

if (!FS_Flag) {

/* Initialization of Nber_Info_Bits, number of bits to be encoded :

-       Class 1, N1_2 bits,

-       Class 2, N2_2 bits,

-       CRC bits

-       (K - 1) zeros to empty the encoder */

Nber_Info_Bits = N1_2 + N2_2 + SIZE_CRC;

/* Zero the (K - 1) last bits of the Input_Frame */

for (i = 0; i < K - 1; i++) {

Input_Frame[N0_2 + Nber_Info_Bits] = 0;

Nber_Info_Bits++;

}

/* Nber_Info_Bits = N1_2 + N2_2 + SIZE_CRC + (K - 1) */

} /* If FS_Flag */

else {

/* Initialization of Nber_Info_Bits, number of bits to be encoded :

-       Class 1, N1 bits,

-       Class 2, N2 bits,

-       Fs_CRC bits

-       (K - 1) zeros to empty the encoder */

Nber_Info_Bits = N1 + N2 + Fs_SIZE_CRC;

/* Zero the (K - 1) last bits of the Input_Frame */

for (i = 0; i < K - 1; i++) {

Input_Frame[N0 + Nber_Info_Bits] = 0;

Nber_Info_Bits++;

}

/* Nber_Info_Bits = N1 + N2 + Fs_SIZE_CRC + (K - 1) */

}

/* Number of states in the Viterbi Lattice : */

M = shl( (Word16)1,(Word16)(K - 1) );

/* Last State of the Viterbi Lattice */

M_1 = sub( M,(Word16)1 );

Msb_bit = shl( (Word16)1,(Word16)(K - 2) );

Lsb_bits = Msb_bit - 1;

/* Description of the Lattice : Loop on Arrival_State */

for (Arrival_state = 0; Arrival_state <= M_1; Arrival_state++) {

/* Computation of the MSB for the Arrival State */

Msb = Arrival_state & Msb_bit;

/* Computation of the (K - 1)MSBs for the Starting State */

Msbs_starting_state = Arrival_state & Lsb_bits;

/* Loop on Lsb, LSB of the Starting State */

for (Lsb = 0; Lsb <= 1; Lsb++) {

Starting_state = add( shl( Msbs_starting_state,(Word16)1 ),Lsb );

Previous[Arrival_state][Lsb] = Starting_state;

/*   TRANSITION BITS T1, T2, T3   */

Involved_bits = add(shl( Msb,(Word16)1 ),Starting_state);

T1[Arrival_state][Lsb] = Combination( Involved_bits,(Word16)G1 );

T2[Arrival_state][Lsb] = Combination( Involved_bits,(Word16)G2 );

T3[Arrival_state][Lsb] = Combination( Involved_bits,(Word16)G3 );

} /* End Loop on Lsb  */

} /* End Loop on Arrival_state */

}

/**************************************************************************

*

*	ROUTINE				:	Interleaving_Signalling

*

*	DESCRIPTION			:	Signalling channel-type interleaving

*							of a single frame (216 bits)

*

**************************************************************************

*

*	USAGE				:	Interleaving_Signalling(buffer_in,buffer_out)

*							(Routine_Name(input1,output1))

*

*	INPUT ARGUMENT(S)		:

*

*		INPUT1			:	- Description : Input buffer

*							- Format : 216 * 16 bit-samples

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : Interleaved buffer

*							- Format : 216 * 16 bit-samples

*

*	RETURNED VALUE		:	None

*

**************************************************************************/

Word16     Interleaving_Signalling(Word16 Input_Frame[],

Word16 Output_frame[])

{

static Word16   K3_const = 216;

static Word16   K_const = 216;

static Word16   a_const = 101;

Word16  i, k;

for (i = 0; i < K3_const; i++) {

k = (Word16)((Word32)((Word32)a_const * (Word32)(i+1)) % K_const);

Output_frame[k] = Input_Frame[i];

}

return(0);

}

/**************************************************************************

*

*	ROUTINE				:	Interleaving_Speech

*

*	DESCRIPTION			:	Matrix interleaving of a  432 bits frame

*

**************************************************************************

*

*	USAGE				:	Interleaving_Speech(buffer_in,buffer_out)

*							(Routine_Name(input1,output1))

*

*	INPUT ARGUMENT(S)		:

*

*		INPUT1			:	- Description : Input buffer

*							- Format : 432 * 16 bit-samples

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : Interleaved buffer

*							- Format : 432 * 16 bit-samples

*

*	RETURNED VALUE		:	None

*

**************************************************************************/

Word16     Interleaving_Speech(Word16 Input_Frame[],

Word16 Output_frame[])

{

Word16             index_lines, index_columns;

for (index_columns = 0; index_columns < COLUMNS; index_columns++) {

for (index_lines = 0; index_lines < LINES; index_lines++)

Output_frame[index_columns * LINES + index_lines] =

Input_Frame[index_lines * COLUMNS + index_columns];

} /* End Loop COLUMNS */

return(0);

}

/**************************************************************************

*

*	ROUTINE				:	Rcpc_Coding

*

*	DESCRIPTION			:	Routine for channel encoding

*

**************************************************************************

*

*	USAGE				:	Rcpc_Coding(flag,buffer_in,buffer_out)

*							(Routine_Name(input1,input2,output1))

*

*	INPUT ARGUMENT(S)		:

*

*		INPUT1			:	- Description :	- (flag = 0) : standard mode

*								- (flag  0) : frame stealing activated

*						- Format : Word16

*

*		INPUT2			:	- Description : Frame to be encoded

*							- Format : 286 * 16 bit-samples

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : Encoded frame

*							- Format : 432 * 16 bit-samples

*

*	RETURNED VALUE		:	None

*

*	COMMENTS			:	- Buffer_in shall be ordered by

* 						sensitivity classes :

*							Class 0

*							Class 1

*							Class 2

*							8 CRC bits

*							4 zeroes

*						- Buffer_out :

*							0 --> +127

*							1 --> -127

*							Class 0 (not encoded)

*							Class 1 (8/12 encoded)

*							Class 2 (8/18 encoded)

*							8 CRC bits (8/18 encoded)

*							4 zeroes (8/18 encoded)

*

**************************************************************************/

void	Rcpc_Coding(Word16 FS_Flag, Word16 Input_Frame[],

Word16 Output_Frame[])

{

/* Variables */

Word16             i, temp, Msb_bit_div2;

Word16             Nber_coded_bits;

Word16             Index_puncturing;

Word16             Coder_state;

Word16             Lsb;

Word16             Size_Class0, Size_Class1, Size_Class2, Size_Error_Control;

if (!FS_Flag) {

Size_Class0 = N0_2;

Size_Class1 = N1_2;

Size_Class2 = N2_2;

Size_Error_Control = SIZE_CRC;

} /* If FS_Flag */

else {

Size_Class0 = N0;

Size_Class1 = N1;

Size_Class2 = N2;

Size_Error_Control = Fs_SIZE_CRC;

}

/* Recopy of Class 0 (unprotected) */

for (i = 0; i < Size_Class0; i++)

Output_Frame[i] = Input_Frame[i];

/* Init of the Starting State of the encoder at zero */

Coder_state = 0;

/* Init Number of Coded Bits */

Nber_coded_bits = 0;

Msb_bit_div2 = shr( Msb_bit,(Word16)1 );

/*-------------------------------------------------------------------------*/

/* Coding of Class 1 */

/* Init of Index for Puncturing: 0 <= Index_puncturing < Period_pct */

Index_puncturing = 0;

for (i = 0; i < Size_Class1; i++) {   /* Loop on class 1 */

/* Output Bit (LSB of the previous state), to define the transition number */

Lsb = Coder_state & 1;

/* Change of state : New state of the encoder */

temp = extract_l(L_mult(Msb_bit_div2,Input_Frame[Size_Class0 + i]));

Coder_state = add( temp,shr( Coder_state,(Word16)1 ) );

/* According to the Puncturing matrix, computation of a coded bit */

/* First coded bit, if any, per info bit */

#ifdef DEBUG

if ((A1[Index_puncturing]) != 0)

{

#endif

Output_Frame[Size_Class0 + Nber_coded_bits] = (T1[Coder_state][Lsb] > 0) ? -127 : 127;

Nber_coded_bits++;

#ifdef DEBUG

}

#endif

/* Second coded bit, if any, per info bit */

if ((A1[Index_puncturing + Period_pct]) != 0)

{

Output_Frame[Size_Class0 + Nber_coded_bits] = (T2[Coder_state][Lsb] > 0) ? -127 : 127;

Nber_coded_bits++;

}

#ifdef DEBUG

/* Third coded bit, if any, per info bit */

if ((A1[Index_puncturing + 2*Period_pct]) != 0)

{

Output_Frame[Size_Class0 + Nber_coded_bits] = (T3[Coder_state][Lsb] > 0) ? -127 : 127;

Nber_coded_bits++;

}

#endif

Index_puncturing++;

if (sub( Index_puncturing,(Word16)Period_pct ) == 0)

Index_puncturing = 0;

} /* End Loop on class 1 */

/* End coding class 1 */

/*-------------------------------------------------------------------------*/
/*-------------------------------------------------------------------------*/

/* Coding of Class 2 */

/* Init of Index for Puncturing: 0 <= Index_puncturing < Period_pct */

Index_puncturing = 0;

for (i = 0; i < Size_Class2 + Size_Error_Control + (K - 1); i++) {   /* Loop on class 2 */

/* Output Bit (LSB of the previous state), to define the transition number */

Lsb = Coder_state & 1;

/* Change of state : New state of the encoder */

temp = extract_l(L_mult(Msb_bit_div2,Input_Frame[Size_Class0 + Size_Class1 + i]));

Coder_state = add( temp,shr( Coder_state,(Word16)1 ) );

/* According to the Puncturing matrix, computation of a coded bit */

/* First coded bit, if any, per info bit */

#ifdef DEBUG

if ((A2[Index_puncturing]) != 0)

{

#endif

Output_Frame[Size_Class0 + Nber_coded_bits] = (T1[Coder_state][Lsb] > 0) ? -127 : 127;

Nber_coded_bits++;

#ifdef DEBUG

}

#endif

/* Second coded bit, if any, per info bit */

#ifdef DEBUG

if ((A2[Index_puncturing + Period_pct]) != 0)

{

#endif

Output_Frame[Size_Class0 + Nber_coded_bits] = (T2[Coder_state][Lsb] > 0) ? -127 : 127;

Nber_coded_bits++;

#ifdef DEBUG

}

#endif

if (!FS_Flag) {

if ((A2[Index_puncturing + 2*Period_pct]) != 0)

/* Third coded bit, if any, per info bit */

{

Output_Frame[Size_Class0 + Nber_coded_bits] = (T3[Coder_state][Lsb] > 0) ? -127 : 127;

Nber_coded_bits++;

}

} /* If FS_Flag */

else {

if ((Fs_A2[Index_puncturing + 2*Period_pct]) != 0)

/* Third coded bit, if any, per info bit */

{

Output_Frame[Size_Class0 + Nber_coded_bits] = (T3[Coder_state][Lsb] > 0) ? -127 : 127;

Nber_coded_bits++;

}

}

Index_puncturing++;

if (sub( Index_puncturing,(Word16)Period_pct ) == 0)

Index_puncturing = 0;

} /* End Loop on class 2 */

/* End coding class 2 */

/*-------------------------------------------------------------------------*/

}

/**************************************************************************

*

*	ROUTINE				:	Transform_Class_0

*

*	DESCRIPTION			:	Transformation ("encoding") of class 0 of the frame

*						0 --> +127

*						1 --> -127

*						the remaining of the frame is not modified

*

**************************************************************************

*

*	USAGE				:	Transform_Class_0(flag,buffer)

*							(Routine_Name(arg1,arg2))

*

*	ARGUMENT(S)			:

*

*		ARG1				:	- Description :	- (flag = 0) : standard mode

*								- (flag  0) : frame stealing activated

*						- Format : Word16

*

*		ARG2				:	- Description : Frame to be encoded

*							- Format : 286 * 16 bit-samples

*

*	RETURNED VALUE		:	None

*

*	COMMENTS			:	Class 0 corresponds to the N0_2 first bits

*						of the buffer (N0 in case of stealing)

*

**************************************************************************/

void	Transform_Class_0(Word16 FS_Flag, Word16 Input_Frame[])

{

/* Variables */

Word16          i;

Word16          Size_Class0;

if (!FS_Flag)

Size_Class0 = N0_2;

else

Size_Class0 = N0;

for (i = 0; i < Size_Class0; i++) {

if (Input_Frame[i] == 0)

Input_Frame[i] = 127;

else

Input_Frame[i] = -127;

} /* End Loop Size_Class0 */

}

/**************************************************************************

*

*	ROUTINE				:	Write_Tetra_File

*

*	DESCRIPTION			:	Writes a file in the TETRA hardware test

*							frame format

*

**************************************************************************

*

*	USAGE				:	Write_Tetra_File(file_pointer,buffer)

*							(Routine_Name(input1,input2))

*

*	INPUT ARGUMENT(S)		:

*

*		INPUT1			:	- Description : File pointer on output file

*							- Format : FILE

*

*		INPUT2			:	- Description : buffer containing the TETRA frame

*							- Format : 432 * 16 bit-samples

*

*	RETURNED VALUE		:	0 if process correct, -1 if EOF

*

*	COMMENTS			:	Data written in the output file are short integers

*

**************************************************************************/

short           Write_Tetra_File (FILE *fout, short *array)

{

static short    initial = 0;

static short    block[690];    /* 960 if FFFF filling is included */

short          *ptr_block;

short i;

/* Return value: -1  EOF

*                0  array TETRA frame written to 'fout'

*/

/* if first call to this routine, then set up output block */

if (initial == 0)

{

initial = 1;

ptr_block = block;

*ptr_block++ = 0x6b21;

for (i = 0; i < 114; i++)

*ptr_block++ = 0;

/* for (i = 0; i < 45; i++)

*ptr_block++ = 0xFFFF;  */

*ptr_block++ = 0x6b22;

for (i = 0; i < 114; i++)

*ptr_block++ = 0;

/* for (i = 0; i < 45; i++)

*ptr_block++ = 0xFFFF;  */

*ptr_block++ = 0x6b23;

for (i = 0; i < 114; i++)

*ptr_block++ = 0;

/* for (i = 0; i < 45; i++)

*ptr_block++ = 0xFFFF;  */

*ptr_block++ = 0x6b24;

for (i = 0; i < 114; i++)

*ptr_block++ = 0;

/* for (i = 0; i < 45; i++)

*ptr_block++ = 0xFFFF;  */

*ptr_block++ = 0x6b25;

for (i = 0; i < 114; i++)

*ptr_block++ = 0;

/* for (i = 0; i < 45; i++)

*ptr_block++ = 0xFFFF;  */

*ptr_block++ = 0x6b26;

for (i = 0; i < 114; i++)

*ptr_block++ = 0;

/* for (i = 0; i < 45; i++)

*ptr_block++ = 0xFFFF;  */

}

/* Fill first valid block */

ptr_block = block+1;

for (i = 0; i < 114; i++)

*ptr_block++ = *array++ & 0x00FF;

/* Fill second valid block */

ptr_block = block + 161 - 45;

for (i = 0; i < 114; i++)

*ptr_block++ = *array++ & 0x00FF;

/* Fill third valid block */

ptr_block = block + 321 - 45 - 45;

for (i = 0; i < 114; i++)

*ptr_block++ = *array++ & 0x00FF;

/* Fill fourth valid block */

ptr_block = block + 481 - 45 - 45 - 45;

for (i = 0; i < 90; i++)

*ptr_block++ = *array++ & 0x00FF;

/* Write out TETRA frame */

if (fwrite (block, sizeof (short), 690, fout) != 690)

return -1;

return 0;

}

## C.3	Tables and Constants for Speech Channel

/************************************************************************

*

*	FILENAME		:		channel.h

*

*	DESCRIPTION	:		DEFINITIONS

*

*						FUNCTION PROTOTYPES for the TETRA

*						speech channel coder and

*						speech channel decoder

*

*						OPERATOR PROTOTYPES for TETRA codec

*

************************************************************************/

/***********************************************************************

*

*      DEFINITIONS

*

************************************************************************/

#ifndef TYPEDEF_H

#define TYPEDEF_H

typedef short Word16;

typedef long  Word32;

typedef int   Flag;

#endif

/************************************************************************

*

*	DESCRIPTION		:	FUNCTION PROTOTYPES for the TETRA

*						speech channel coder

*

************************************************************************/

#ifndef SUB_CC_H

#define SUB_CC_H

#include <stdio.h>

Word16	Build_Crc(Word16 FS_Flag, Word16 Input_Frame[]);

Word16	Build_Sensitivity_Classes(Word16 FS_Flag, Word16 Input_Frame[], 			Word16 Output_Frame[]);

Word16	Combination(Word16 A, Word16 B);

void		Channel_Encoding(short first_pass, Word16 Frame_Stealing,

Word16 Input_Frame[], Word16 Output_Frame[]);

void		Init_Rcpc_Coding(Word16 FS_Flag, Word16 Input_Frame[]);

Word16	Interleaving_Signalling(Word16 Input_Frame[],

Word16 Output_frame[]);

Word16	Interleaving_Speech(Word16 Input_Frame[], Word16 Output_frame[]);

void		Rcpc_Coding(Word16 FS_Flag, Word16 Input_Frame[],

Word16 Output_Frame[]);

void		Transform_Class_0(Word16 FS_Flag, Word16 Input_Frame[]);

short		Write_Tetra_File (FILE *fout, short *array);

#endif

/************************************************************************

*

*	DESCRIPTION		:	FUNCTION PROTOTYPES for the TETRA

*						speech channel decoder

*

************************************************************************/

#ifndef SUB_CD_H

#define SUB_CD_H

Word16	Bfi(Word16 FS_Flag, Word16 Input_Frame[]);

Word16	Channel_Decoding(short first_pass, Word16 Frame_Stealing,

Word16 Input_Frame[], Word16 Output_Frame[]);

Word16	Combination(Word16 A, Word16 B);

Word16	Desinterleaving_Signalling(Word16 Input_Frame[],

Word16 Output_frame[]);

Word16	Desinterleaving_Speech(Word16 Input_Frame[],

Word16 Output_frame[]);

void		Init_Rcpc_Decoding(void);

void		Rcpc_Decoding(Word16 FS_Flag, Word16 Input_Frame[],

Word16 Output_Frame[]);

short		Read_Tetra_File (FILE *fin, short *array);

Word16	Unbuild_Sensitivity_Classes(Word16 FS_Flag, Word16 Input_Frame[],

Word16 Output_Frame[]);

Word16	Untransform_Class_0(Word16 FS_Flag, Word16 Input_Frame[]);

#endif

/************************************************************************

*

*	DESCRIPTION		:	OPERATOR PROTOTYPES for TETRA codec

*

************************************************************************/

#ifndef TETRA_OP_H

#define TETRA_OP_H

/*-----------------------*

* Constants and Globals *

*-----------------------*/

extern Flag Overflow;

extern Flag Carry;

#define MAX_32 (Word32)0x7fffffff

#define MIN_32 (Word32)0x80000000

#define MAX_16 (Word16)0x7fff

#define MIN_16 (Word16)0x8000

/*-----------------------*

* Operators prototypes  *

*-----------------------*/

Word16 abs_s(Word16 var1);                /* Short abs,           1 */

Word16 add(Word16 var1, Word16 var2);     /* Short add,           1 */

Word16 div_s(Word16 var1, Word16 var2);   /* Short division,     18 */

Word16 extract_h(Word32 L_var1);          /* Extract high,        1 */

Word16 extract_l(Word32 L_var1);          /* Extract low,         1 */

Word16 mult(Word16 var1, Word16 var2);    /* Short mult,          1 */

Word16 mult_r(Word16 var1, Word16 var2);  /* Mult with round,     2 */

Word16 negate(Word16 var1);               /* Short negate,        1 */

Word16 norm_l(Word32 L_var1);             /* Long norm,          30 */

Word16 norm_s(Word16 var1);               /* Short norm,         15 */

Word16 round(Word32 L_var1);              /* Round,               1 */

Word16 shl(Word16 var1, Word16 var2);     /* Short shift left,    1 */

Word16 shr(Word16 var1, Word16 var2);     /* Short shift right,   1 */

Word16 sub(Word16 var1, Word16 var2);     /* Short sub,           1 */

Word32 L_abs(Word32 L_var1);              /* Long abs,            3 */

Word32 L_add(Word32 L_var1, Word32 L_var2);  /* Long add,         2 */

Word32 L_deposit_h(Word16 var1);          /* 16 bit var1 -> MSB   2 */

Word32 L_deposit_l(Word16 var1);          /* 16 bit var1 -> LSB,  2 */

Word32 L_mac(Word32 L_var3, Word16 var1, Word16 var2);  /* Mac,   1 */

Word32 L_mac0(Word32 L_var3, Word16 var1, Word16 var2); /* no shi 1 */

Word32 L_msu(Word32 L_var3, Word16 var1, Word16 var2);  /* Msu,   1 */

Word32 L_msu0(Word32 L_var3, Word16 var1, Word16 var2); /* no shi 1 */

Word32 L_mult(Word16 var1, Word16 var2);  /* Long mult,           1 */

Word32 L_mult0(Word16 var1, Word16 var2); /* Long mult no shift,  1 */

Word32 L_negate(Word32 L_var1);           /* Long negate,         2 */

Word32 L_shl(Word32 L_var1, Word16 var2); /* Long shift left,     2 */

Word32 L_shr(Word32 L_var1, Word16 var2); /* Long shift right,    2 */

Word32 L_shr_r(Word32 L_var1, Word16 var2);  /* L_shr with round, 3 */

Word32 L_sub(Word32 L_var1, Word32 L_var2);  /* Long sub,         2 */

#endif

/**************************************************************************

*

*	FILENAME				:	arrays.tab

*

*	DESCRIPTION			:	Arrays for speech channel coding and decoding

*

**************************************************************************/

/* ARRAYS FOR INITIALIZATION OF THE CHANNEL CODING */

/* Definition of the sensitivity classes : 3 classes */

const Word16	TAB0[N0] =	{35,36,37,38,39,40,41,42,43,  47,48,

56,  61,62,63,64,65,66,67,68,69,70,

74,75,  83,  88,89,90,91,92,93,94,95,

96,97,  101,102,  110,115,116,117,118,

119,120,121,122,123,124,  128,129,

137};

/* Class 0

51 terms : Non protected class

Least sensitive bits */

const Word16	TAB1[N1] =	{58,85,112,  54,81,108,135,  50,77,

104,131,  45,72,99,126,  55,82,109,

136,  5,13,34,  8,16,17,  22,23,24,

25,26,  6,14,7,15,  60,87,114,  46,

73,100,127,  44,71,98,125,  33,49,

76,103,130,  59,86,113,  57,84,111};

/* Class 1

56 terms : Protected by Rate 8/12

Ranked with the most important bits at the boundaries,

and least important at the middle */

const Word16	TAB2[N2] =		{18,19,20,21,	/* LSF3 */

31,32,		/* Pitch0 */

53,80,107,134,	/* Energie 3 */

1,2,3,4,		/* LSF1 */

9,10,11,12,		/* LSF2 */

27,28,29,30,	/* Pitch0 */

52,79,106,133,	/* Energie 4 */

51,78,105,132};/* Energie 5 */

/* Class 2

30 terms : Protected by Rate 8/18

Ranked from least to most important bits */

/* PUNCTURING CHARACTERISTICS: */

/* Puncturing Tables for the two protected classes (Class 1 and 2) */

const Word16	A1[Period_pct*3]		=

{1, 1, 1, 1, 1, 1, 1, 1,

1, 0, 1, 0, 1, 0, 1, 0,

0, 0, 0, 0, 0, 0, 0, 0};

/* Puncturing Table for Class 1 : Rate 8/12 (2/3) */

const Word16	A2[Period_pct*3]		=

{1, 1, 1, 1, 1, 1, 1, 1,

1, 1, 1, 1, 1, 1, 1, 1,

1, 0, 0, 0, 1, 0, 0, 0};

/* Puncturing Table for Class 2 : Rate 8/18 */

const Word16	Fs_A2[Period_pct*3]	=

{1, 1, 1, 1, 1, 1, 1, 1,

1, 1, 1, 1, 1, 1, 1, 1,

1, 0, 0, 0, 0, 0, 0, 0};

/* In case of Frame Stealing */

/* Puncturing Table for Class 2 : Rate 8/17 */

/* BAD FRAME INDICATOR : CRC (Cyclic Redundant Check) */

/* Definition of the 8 CRC bits */

const Word16	TAB_CRC1[SIZE_TAB_CRC1] =

{1,  5,  8,9,  13,  15,16,

17,  19,  21,22,  24,25,  31,

32,  35,36,  38,  40,  43,44,

45,  48,49,50,51,  53,54,  56};

/* Ranks of the bits in Class2 protected by the First bit of the CRC */

const Word16	TAB_CRC2[SIZE_TAB_CRC2] =

{2,  6,  9,10,  14,

16,17,18,  20,  22,23,  25,26,

32,33,  36,37,  39,  41,  44,45,

46,  49,50,51,52,  54,55,  57};

/* Ranks of the bits in Class 2 protected by the Second bit of the CRC */

const Word16	TAB_CRC3[SIZE_TAB_CRC3] =

{3,  7,  10,11,  15,

17,18,19,  21,  23,24,  26,27,

33,34,  37,38,  40,  42,  45,

46,47,  50,51,52,53,  55,56,  58};

/* Ranks of the bits in Class 2 protected by the Third bit of the CRC */

const Word16	TAB_CRC4[SIZE_TAB_CRC4] =

{1,  4,5,  9,  11,12,13,  15,

17,18,  20,21,  27,28,  31,

32,  34,  36,  39,40,41,  44,45,

46,47,  49,50,  52,  57,  59};

/* Ranks of the bits in Class 2 protected by the Fourth bit of the CRC */

const Word16	TAB_CRC5[SIZE_TAB_CRC5] =

{2,  5,6,  10,  12,13,14,

16,  18,19,  21,22,  28,29,

32,33,  35,  37,  40,41,42,  45,

46,47,48,  50,51,  53,  58,  60};

/* Ranks of the bits in Class 2 protected by the Fifth bit of the CRC */

const Word16	TAB_CRC6[SIZE_TAB_CRC6] =

{3,  6,7,  11,  13,14,15,

17,  19,20,  22,23,  29,30,

33,34,  36,  38,  41,42,43,

46,47,48,49,  51,52,  54,  59};

/* Ranks of the bits in Class 2 protected by the Sixth bit of the CRC */

const Word16	TAB_CRC7[SIZE_TAB_CRC7] =

{4,  7,8,  12,  14,15,

16,  18,  20,21,  23,24,  30,

31,  34,35,  37,  39,  42,43,44,

47,48,49,50,  52,53,  55,  60};

/* Ranks of the bits in Class 2 protected by the Seventh bit of the CRC */

const Word16	TAB_CRC8[SIZE_TAB_CRC8] =

{1,2,3,4,  8,  13,14,  16,  19,

20,  22,23,  25,26,27,28,29,30,

32,33,34,  36,37,  40,41,42,  44,

48,  50,  53,  56,57,58,59,60};

/* Ranks of the bits in Class 2 protected by the Eighth bit of the CRC */

/* IN CASE OF FRAME STEALING :

BAD FRAME INDICATOR : CRC (Cyclic Redundant Check)

Definition of the 4 CRC bits */

#define	Fs_SIZE_TAB_CRC1	16

#define	Fs_SIZE_TAB_CRC2	16

#define	Fs_SIZE_TAB_CRC3	16

#define	Fs_SIZE_TAB_CRC4	16

const Word16	Fs_TAB_CRC1[Fs_SIZE_TAB_CRC1] =

{1,  4,5,  7,  9,10,11,12,

16,  19,20,  22,  24,25,26,27  };

/* Ranks of the bits in Class2 protected by the First bit of the CRC */

const Word16	Fs_TAB_CRC2[Fs_SIZE_TAB_CRC2] =

{1,2,  4,  6,7,8,9,  13,

16,17,  19,  21,22,23,24,  28  };

/* Ranks of the bits in Class 2 protected by the Second bit of the CRC */

const Word16	Fs_TAB_CRC3[Fs_SIZE_TAB_CRC3] =

{  2,3,  5,  7,8,9,10,  14,

17,18,  20,  22,23,24,25,  29  };

/* Ranks of the bits in Class 2 protected by the Third bit of the CRC */

const Word16	Fs_TAB_CRC4[Fs_SIZE_TAB_CRC4] =

{  3,4,  6,  8,9,10,11,  15,

18,19,  21,  23,24,25,26,  30};

/* Ranks of the bits in Class 2 protected by the Fourth bit of the CRC */

#define	N1_coded	84	/* Size of Class1 (one speech frame) after coding 8/12 */

#define	N2_coded	81	/* Size of Class2 (one speech frame) after coding 8/17 */

Word16	Previous[(1 << (K - 1))][2];

Word16	Best_previous[(1 << (K - 1))][Decoding_delay];

Word16	T1[(1 << (K - 1))][2], T2[(1 << (K - 1))][2], T3[(1 << (K - 1))][2];

Word16	Score[(1 << (K - 1))];

Word16	Ex_score[(1 << (K - 1))];

Word16	Received[3];

Word16	Initialization;

Word16	Nber_Info_Bits;

Word16	Msb_bit;

Word16	M_1;

Word16	Min_value_allowed;

Word16	Max_value_allowed;

/* END GLOBAL VARIABLES ***********************************/

/**************************************************************************

*

*	FILENAME				:	const.tab

*

*	DESCRIPTION			:	Constants for speech channel coding and decoding

*

**************************************************************************/

/* CONSTANTS FOR CONVOLUTIONAL CODING */

#define	K				5 /* Constraint Length */

#define	Nber_bits_quantif		8 /* Quantization */

#define	Decoding_delay		30 /* Decoding Delay */

/* Polynomial generators : these polynomials should be under the format

1+...+X**4 to match the decoding procedure : */

#define	G1				0x01F /* First Polynomial */

#define	G2				0x01B /* Second Polynomial */

#define	G3				0x015 /* Third Polynomial */

/* FRAME FORMAT : */

/* Size of the three sensitivity classes  */

/* these sizes correspond to the coding of two speech frames at a time */

/*                                         ---                         */

#define	N0		51 /* One Frame : Less sensitive class (class 0) */

#define	N1		56 /* One Frame : Moderate sensitive (class 1) */

#define	N2		30 /* One Frame : Most sensitive (class2) */

#define	N0_2		102 /* Two Frames : Less sensitive class (class 0) */

#define	N1_2		112 /* Two Frames : Moderate sensitive (class 1) */

#define	N2_2		60 /* Two Frames : Most sensitive (class 2) */

#define	N1_2_coded	168 /* Two Frames after coding : size of class 1 after coding */

#define	N2_2_coded	162 /* Two Frames after coding : size of class 2 after coding including CRC bits, (K-1) bits to empty the encoder */

/* N0_2 + N1_2_coded + N2_2_coded	=	size of a coded frame (60 ms  at 7200b/s)

=	432

=	Long_time_slot */

#define	Length_vocoder_frame	137 /* 30 ms speech frame at 4567 b/s */

#define	Length_2_frames		274 /* Two 30 ms speech frames */

#define	Length_time_slot		432 /* 60 ms frame at 7200 b/s */

/* PUNCTURING CHARACTERISTICS: */

#define	Period_pct			8 /* Puncturing Period */

#define	Ai_j_SIZE		Period_pct*6 /* Size of Puncturing Arrays */

/* BAD FRAME INDICATOR: CRC */

#define	SIZE_CRC			8 /* Number of bits of the CRC (Cyclic Redundant Check) */

#define	Fs_SIZE_CRC			4 /* Number of bits of the CRC (Frame Stealing Active) */

#define	SIZE_TAB_CRC1		29	/* Amount of bits taken into account */

#define	SIZE_TAB_CRC2		29	/* In every of the 8 CRC bits */

#define	SIZE_TAB_CRC3		29

#define	SIZE_TAB_CRC4		30

#define	SIZE_TAB_CRC5		30

#define	SIZE_TAB_CRC6		29

#define	SIZE_TAB_CRC7		29

#define	SIZE_TAB_CRC8		35

/* CONSTANTS FOR MATRIX INTERLEAVING */

#define	LINES				24

#define	COLUMNS			18

/* LINES * COLUMNS = Length_time_slot  = 432 */

# D	C Code for Speech Channel Decoding

## D.1	Main Program for Speech Channel Decoding

/************************************************************************

*

*	FILENAME		:	cdecoder.			c

*

*	DESCRIPTION		:	Main program for speech channel decoding

*

************************************************************************

*

*	USAGE			:	cdecoder input_file output_file

*					(Executable_File Input1 Output1)

*

*	INPUT FILE(S)		:

*

*		INPUT1	:	- Description : channel encoded serial stream

*					- Format : binary file 16 bit-samples

*					  each 16 bit-sample represents one encoded bit

*					- 1 channel frame = 432 bits

*

*	OUTPUT FILE(S)	:

*

*		OUTPUT1	:	- Description :  serial stream output file

*					- Format : binary file 16 bit-samples

*					  each 16 bit-sample represents one encoded bit

*					- 1 output frame includes 2 speech frames with BFI

*					   = 2 * (137 + 1) = 276 bits

*

*	COMMENTS		:	- Values of channel encoded samples are

*					either -127 or +127

*

************************************************************************

*

*	INCLUDED FILES	:	channel.h

*					stdlib.h

*

************************************************************************/

/* LIBRARIES USED */

#include <stdlib.h>

#include "channel.h"

#ifndef TRUE

# define TRUE   (1==1)

# define FALSE  (1==0)

#endif

Word16  Frame_stealing = 0;	/* Frame Stealing Flag :

0 = Inactive,

!0 = First Frame in time-slot stolen

This flag is set/reset by external world */

int	main( int argc, char *argv[] )

{

FILE    *fin, *fout;

Word32  Loop_counter = 0;

short   first_pass = TRUE;

Word16  i;

Word16  bfi1 = 0;

Word16  bfi2 = 0;               /* Reset Bad Frame Indicator :

0 = correct data, 1 = Corrupted frame */

Word16  Reordered_array[286];   /* 2 frames vocoder + 8 + 4 */

Word16  Interleaved_coded_array[432]; /*time-slot length at 7.2 kb/s*/

Word16  Coded_array[432];

/* Parse arguments */

if ( argc != 3 )

{

puts( "usage : cdecoder input_file output_file" );

puts( "format for input_file  : $6B21...114 bits");

puts( "       ...$6B22...114..." );

puts( "       ...$6B26...114...$6B21");

puts( "format for output_file : two 138 (BFI + 137) bit frames");

exit( 1 );

}

if ( (fin = fopen( argv[1], "rb" )) == NULL )

{

puts("cdecoder: can't open input_file" );

exit( 1 );

}

if ( (fout = fopen( argv[2], "wb" )) == NULL )

{

puts("cdecoder: can't open output_file" );

exit( 1 );

}

while( 1 )

{

/* read Input_array (1 TETRA frame = 2 speech frames) from input file */

if (Read_Tetra_File (fin, Interleaved_coded_array) == -1)

{

puts ("cdecoder: reached end of input_file");

break;

}

if (Frame_stealing)

{

Desinterleaving_Signalling(Interleaved_coded_array + 216,

Coded_array + 216);

/* When Frame Stealing occurs, recopy first half slot : */

for (i = 0; i < 216; i++) Coded_array[i] =

Interleaved_coded_array[i];

}

else

Desinterleaving_Speech(Interleaved_coded_array, Coded_array);

bfi1 = Frame_stealing;

/* "Interleaved_coded_array" has been desinterleaved and result put

in "Coded_array" */

/* Message in case the Frame was stolen */

if (bfi1) printf("Frame Nb %ld was stolen\n",Loop_counter+1);

/* Channel Decoding */

bfi2 = Channel_Decoding(first_pass,Frame_stealing,

Coded_array,Reordered_array);

first_pass = FALSE;

if ((Frame_stealing==0) && (bfi2==1)) bfi1=1;

/* Increment Loop counter */

Loop_counter++;

/* Message in case the Bad Frame Indicator was set */

if (bfi2) printf("Frame Nb %ld Bfi active\n\n",Loop_counter);

/* writing  Reordered_array to output file */

/* bfi bit */

if( fwrite( &bfi1, sizeof(short), 1, fout ) != 1 ) {

puts( "cdecoder: can't write to output_file" );

break;

}

/* 1st speech frame */

if( fwrite( Reordered_array, sizeof(short), 137, fout ) != 137 )

{

puts( "cdecoder: can't write to output_file" );

break;

}

/* bfi bit */

if( fwrite( &bfi2, sizeof(short), 1, fout ) != 1 ) {

puts( "cdecoder: can't write to output_file" );

break;

}

/* 2nd speech frame */

if( fwrite( Reordered_array+137, sizeof(short), 137, fout )

!= 137 ) {

puts( "cdecoder: can't write to output_file" );

break;

}

}

printf("%ld Channel Frames processed\n",Loop_counter);

printf("ie %ld Speech Frames\n",2*Loop_counter);

/* closing files */

fclose( fin );

fclose( fout );

return (EXIT_SUCCESS);

}

/************************************************************************

*	FILENAME		:	cdec_tet.c

*

*	DESCRIPTION		:	Main routine for speech channel decoding

*

************************************************************************

*

*	SUB-ROUTINES	:	- Channel_Decoding()

*

************************************************************************

*

*	INCLUDED FILES	:	channel.h

*

************************************************************************/

#include "channel.h"

/**************************************************************************

*

*	ROUTINE			:	Channel_Decoding

*

*	DESCRIPTION			:	Main speech channel decoding function

*

**************************************************************************

*

*	USAGE				:	Channel_Decoding(first,flag,buffer_in, buffer_out)

*						(Routine_Name(input1,input2,input3,output1))

*

*	INPUT ARGUMENT(S)		:

*

*			INPUT1		:	- Description :	- True only if first call to this routine

*						- Format : Short

*

*			INPUT2		:	- Description :	- (flag = 0) : standard mode

*								- (flag  0) : frame stealing activated

*						- Format : Word16

*

*			INPUT3		:	- Description : Frame to be decoded

*							- Format : 432 * 16 bit-samples

*								   In case of frame stealing , only the second

*							   half is decoded

*

*	OUTPUT ARGUMENT(S)	:

*

*		OUTPUT1		:	- Description : Two concatened speech frames

*						- Format : 274 * 16 bit-samples

*							   In case of frame stealing, only the second

*							   frame is useful

*

*	RETURNED VALUE		:	BFI = 0 --> OK

*						BFI = 1 --> Bad Frame Flag set

*

**************************************************************************/

Word16    Channel_Decoding(short first_pass, Word16 Frame_Stealing,

Word16 Input_Frame[], Word16 Output_Frame[])

{

Word16  Decoded_array[286];

Word16  badframeindicator;

/* Init for channel-decoder */

if ( first_pass ) Init_Rcpc_Decoding();

/* Decoding */

if (!Frame_Stealing)

Rcpc_Decoding(Frame_Stealing, Input_Frame, Decoded_array);

else

Rcpc_Decoding(Frame_Stealing, Input_Frame + 216, Decoded_array);

badframeindicator = Bfi(Frame_Stealing,Decoded_array);

/* "Decoding" for non-protected class (class 0) */

Untransform_Class_0(Frame_Stealing,Decoded_array);

if (!Frame_Stealing)

/* Reordering of the three classes in two speech frames */

Unbuild_Sensitivity_Classes(Frame_Stealing,Decoded_array,

Output_Frame);

else

/* Reordering of the three classes in one speech frame */

Unbuild_Sensitivity_Classes(Frame_Stealing,Decoded_array,

Output_Frame + 137);

return(badframeindicator);

}

## D.2	Library of Speech Channel Decoding Subroutines

/************************************************************************

*	FILENAME		:	sub_cd.c

*

*	DESCRIPTION		:	Sub-routines for speech channel decoding

*

************************************************************************

*

*	SUBROUTINES		:	- Bfi()

*					- Combination()

*					- Desinterleaving_Signalling()

*					- Desinterleaving_Speech()

*					- Init_Rcpc_Decoding()

*					- Rcpc_Decoding()

*					- Read_Tetra_File()

*					- Unbuild_Sensitivity_Classes()

*					- Untransform_Class_0()

*

************************************************************************

*

*	INCLUDED FILES	:	arrays.tab

*					channel.h

*					const.tab

*					stdlib.h

*

************************************************************************/

#include <stdlib.h>

#include "channel.h"

#include "const.tab" /* contains constants for channel coding/decoding */

#include "arrays.tab" /* contains arrays for channel coding/decoding */

/* #define DEBUG */	   /*	This is a compilation directive eventually used

in the routine Rcpc_Decoding to check that the

puncturing matrices A1 and A2 are correctly set

The results given by this software shall be the

same, independently of the definition of DEBUG

The complexity of the channel decoding

component has been evaluated with the

assumption that DEBUG is NOT defined */

/**************************************************************************

*

*	ROUTINE				:	Bfi

*

*	DESCRIPTION			:	Computation of the Bad Frame Indicator

*							(CRC based) of a frame

*

**************************************************************************

*

*	USAGE				:	Bfi(flag,buffer)

*							(Routine_Name(arg1,arg2))

*

*	ARGUMENT(S)			:

*

*		ARG1				:	- Description :	- (flag = 0) : standard mode

*								- (flag  0) : frame stealing activated

*						- Format : Word16

*

*	ARG2				:	- Description : One ordered frame after decoding

*							- Format : Word16

*

*	RETURNED VALUE		:	BFI = 0 --> OK

*						BFI = 1 --> Bad Frame Flag set

*

*	COMMENTS			:	8 Crc bits are located at the end  of the Input_Frame

*						CRC computation is carried out on Class 2 only

*						as defined by arrays TAB_CRC1

*						(length SIZE_TAB_CRC1)

*						to TAB_CRC8 (length SIZE_TAB_CRC8)

*

**************************************************************************/

Word16     Bfi(Word16 FS_Flag, Word16 Input_Frame[])

{

/* Variables */

Word16             i, temp;

Word16             Bad_Frame_Indicator;

/* Init of the Flag */

Bad_Frame_Indicator = 0;

if (!FS_Flag) {

/* The class protected by the CRC (Class 2) starts at index N0_2 + N1_2 */

/* First Bit of the CRC : */

temp = 0;

for (i = 0; i < SIZE_TAB_CRC1; i++)

temp = add(temp,Input_Frame[N0_2 + N1_2 + TAB_CRC1[i] - 1]);

/* The CRC bit is the LSB of temp : */

temp = temp & 1;

/* Comparison of the CRC bit just computed to the one contained in

the Input_Frame : */

if (sub(temp,Input_Frame[N0_2 + N1_2 + N2_2]) != 0)

Bad_Frame_Indicator = 1;

/* Second Bit of the CRC : */

temp = 0;

for (i = 0; i < SIZE_TAB_CRC2; i++)

temp = add(temp,Input_Frame[N0_2 + N1_2 + TAB_CRC2[i] - 1]);

temp = temp & 1;

if (sub(temp,Input_Frame[N0_2 + N1_2 + N2_2 + 1]) != 0)

Bad_Frame_Indicator = 1;

/* Third Bit of the CRC : */

temp = 0;

for (i = 0; i < SIZE_TAB_CRC3; i++)

temp = add(temp,Input_Frame[N0_2 + N1_2 + TAB_CRC3[i] - 1]);

temp = temp & 1;

if (sub(temp,Input_Frame[N0_2 + N1_2 + N2_2 + 2]) != 0)

Bad_Frame_Indicator = 1;

/* Fourth Bit of the CRC : */

temp = 0;

for (i = 0; i < SIZE_TAB_CRC4; i++)

temp = add(temp,Input_Frame[N0_2 + N1_2 + TAB_CRC4[i] - 1]);

temp = temp & 1;

if (sub(temp,Input_Frame[N0_2 + N1_2 + N2_2 + 3]) != 0)

Bad_Frame_Indicator = 1;

/* Fifth Bit of the CRC : */

temp = 0;

for (i = 0; i < SIZE_TAB_CRC5; i++)

temp = add(temp,Input_Frame[N0_2 + N1_2 + TAB_CRC5[i] - 1]);

temp = temp & 1;

if (sub(temp,Input_Frame[N0_2 + N1_2 + N2_2 + 4]) != 0)

Bad_Frame_Indicator = 1;

/* Sixth Bit of the CRC : */

temp = 0;

for (i = 0; i < SIZE_TAB_CRC6; i++)

temp = add(temp,Input_Frame[N0_2 + N1_2 + TAB_CRC6[i] - 1]);

temp = temp & 1;

if (sub(temp,Input_Frame[N0_2 + N1_2 + N2_2 + 5]) != 0)

Bad_Frame_Indicator = 1;

/* Seventh Bit of the CRC : */

temp = 0;

for (i = 0; i < SIZE_TAB_CRC7; i++)

temp = add(temp,Input_Frame[N0_2 + N1_2 + TAB_CRC7[i] - 1]);

temp = temp & 1;

if (sub(temp,Input_Frame[N0_2 + N1_2 + N2_2 + 6]) != 0)

Bad_Frame_Indicator = 1;

/* Eighth Bit of the CRC : */

temp = 0;

for (i = 0; i < SIZE_TAB_CRC8; i++)

temp = add(temp,Input_Frame[N0_2 + N1_2 + TAB_CRC8[i] - 1]);

temp = temp & 1;

if (sub(temp,Input_Frame[N0_2 + N1_2 + N2_2 + 7]) != 0)

Bad_Frame_Indicator = 1;

} /* If FS_Flag */

else {

/* The class protected by the CRC (Class 2) starts at index N0 + N1 */

/* First Bit of the CRC : */

temp = 0;

for (i = 0; i < Fs_SIZE_TAB_CRC1; i++)

temp = add(temp,Input_Frame[N0 + N1 + Fs_TAB_CRC1[i] - 1]);

/* The CRC bit is the LSB of temp : */

temp = temp & 1;

/* Comparison of the CRC bit just computed to the one contained in

the Input_Frame : */

if (sub(temp,Input_Frame[N0 + N1 + N2]) != 0)

Bad_Frame_Indicator = 1;

/* Second Bit of the CRC : */

temp = 0;

for (i = 0; i < Fs_SIZE_TAB_CRC2; i++)

temp = add(temp,Input_Frame[N0 + N1 + Fs_TAB_CRC2[i] - 1]);

temp = temp & 1;

if (sub(temp,Input_Frame[N0 + N1 + N2 + 1]) != 0)

Bad_Frame_Indicator = 1;

/* Third Bit of the CRC : */

temp = 0;

for (i = 0; i < Fs_SIZE_TAB_CRC3; i++)

temp = add(temp,Input_Frame[N0 + N1 + Fs_TAB_CRC3[i] - 1]);

temp = temp & 1;

if (sub(temp,Input_Frame[N0 + N1 + N2 + 2]) != 0)

Bad_Frame_Indicator = 1;

/* Fourth Bit of the CRC : */

temp = 0;

for (i = 0; i < Fs_SIZE_TAB_CRC4; i++)

temp = add(temp,Input_Frame[N0 + N1 + Fs_TAB_CRC4[i] - 1]);

temp = temp & 1;

if (sub(temp,Input_Frame[N0 + N1 + N2 + 3]) != 0)

Bad_Frame_Indicator = 1;

}

return(Bad_Frame_Indicator);

}

/**************************************************************************

*

*	ROUTINE				:	Combination

*

*	DESCRIPTION			:	Computes the (convolutional) coded bit for a given

*							polynomial generator and a given state of the encoder

*

**************************************************************************

*

*	USAGE				:	Combination(gene,state)

*							(Routine_Name(input1,input2))

*

*	INPUT ARGUMENT(S)		:

*

*		INPUT1			:	- Description : Polynomial generator

*							- Format : Word16

*

*		INPUT2			:	- Description : State of the encoder

*							- Format : Word16

*

*	RETURNED VALUE		:	1 coded bit

*

**************************************************************************/

Word16     Combination(Word16 A, Word16 B)

{

Word16             Comb;

Word16             i, temp1, temp2, temp3;

Comb = -1;

temp1 = A & B;

for (i = 0;i <= (K - 1);i++)

{

temp2 = shl( (Word16)1,i );

temp3 = temp1 & temp2;

if (temp3 != 0) Comb = negate(Comb);

}

return(Comb);

}

/**************************************************************************

*

*	ROUTINE				:	Desinterleaving_Signalling

*

*	DESCRIPTION			:	Signalling Channel-type Desinterleaving

*						of a single frame (216 bits)

*

**************************************************************************

*

*	USAGE				:	Desinterleaving_Signalling(buffer_in,buffer_out)

*							(Routine_Name(input1,output1))

*

*	INPUT ARGUMENT(S)		:

*

*		INPUT1			:	- Description : One interleaved frame

*							- Format : 216 * 16 bit-samples

*

*	OUTPUT ARGUMENT(S)		:

*

*	OUTPUT1			:	- Description : One matrix desinterleaved frame

*							- Format : 216 * 16 bit-samples

*

*	RETURNED VALUE		:	None

*

**************************************************************************/

Word16     Desinterleaving_Signalling(Word16 Input_Frame[],

Word16 Output_frame[])

{

static Word16   K3_const = 216;

static Word16   K_const = 216;

static Word16   a_const = 101;

Word16  i, k;

for (i = 0; i < K3_const; i++) {

k = (Word16)((Word32)((Word32)a_const * (Word32)(i+1)) % K_const);

Output_frame[i] = Input_Frame[k];

}

return(0);

}

/**************************************************************************

*

*	ROUTINE				:	Desinterleaving_Speech

*

*	DESCRIPTION			:	Desinterleaving of an interleaved frame (432 bits)

*

**************************************************************************

*

*	USAGE				:	Desinterleaving_Speech(buffer_in,buffer_out)

*							(Routine_Name(input1,output1))

*

*	INPUT ARGUMENT(S)		:

*

*		INPUT1			:	- Description : One interleaved frame

*							- Format : 432 * 16 bit-samples

*

*	OUTPUT ARGUMENT(S)		:

*

*	OUTPUT1			:	- Description : One matrix desinterleaved frame

*							- Format : 432 * 16 bit-samples

*

*	RETURNED VALUE		:	None

*

**************************************************************************/

Word16     Desinterleaving_Speech(Word16 Input_Frame[],

Word16 Output_frame[])

{

Word16             index_lines, index_columns;

for (index_columns = 0; index_columns < COLUMNS; index_columns++) {

for (index_lines = 0; index_lines < LINES; index_lines++)

Output_frame[index_lines * COLUMNS + index_columns] =

Input_Frame[index_columns * LINES + index_lines];

} /* End Loop COLUMNS */

return(0);

}

/**************************************************************************

*

*	ROUTINE				:	Init_Rcpc_Decoding

*

*	DESCRIPTION			:	Initialization for convolutional rate compatible

*							punctured decoding

*

**************************************************************************

*

*	USAGE				:	Init_Rcpc_Decoding()

*							(Routine_Name)

*

*	ARGUMENT(S)			:	None

*

*	RETURNED VALUE		:	None

*

**************************************************************************/

void	Init_Rcpc_Decoding(void)

{

/* Variables */

Word16             M;

Word16             Arrival_state;  /* index for Loop on the Lattice States */

Word16             Starting_state;

Word16             Msb;    /* Value used for computation of coded bit */

Word16             Lsb;    /* Value used for computation of coded bit */

Word16             Lsb_bits;

Word16             Msbs_starting_state;

Word16             Involved_bits;

/* Number of states in the Viterbi Lattice : */

M = shl( (Word16)1,(Word16)(K - 1) );

/* Last State of the Viterbi Lattice */

M_1 = sub( M,(Word16)1 );

Msb_bit = shl( (Word16)1,(Word16)(K - 2) );

Lsb_bits = Msb_bit - 1;

/* Description of the Lattice : Loop on Arrival_State */

for (Arrival_state = 0; Arrival_state <= M_1; Arrival_state++) {

/* Computation of the MSB for the Arrival State */

Msb = Arrival_state & Msb_bit;

/* Computation of the (K - 1)MSBs for the Starting State */

Msbs_starting_state = Arrival_state & Lsb_bits;

/* Loop on Lsb, LSB of the Starting State */

for (Lsb = 0; Lsb <= 1; Lsb++) {

Starting_state = add(shl( Msbs_starting_state,(Word16)1 ),Lsb);

Previous[Arrival_state][Lsb] = Starting_state;

/*   TRANSITION BITS T1, T2, T3   */

Involved_bits = add( shl( Msb,(Word16)1 ),Starting_state );

T1[Arrival_state][Lsb] = Combination( Involved_bits,(Word16)G1 );

T2[Arrival_state][Lsb] = Combination( Involved_bits,(Word16)G2 );

T3[Arrival_state][Lsb] = Combination( Involved_bits,(Word16)G3 );

} /* End Loop on Lsb  */

} /* End Loop on Arrival_state */

}

/**************************************************************************

*

*	ROUTINE				:	Rcpc_Decoding

*

*	DESCRIPTION			:	Convolutional rate compatible punctured decoding

*							of a frame

*

**************************************************************************

*

*	USAGE				:	Rcpc_Decoding(flag,buffer_in,buffer_out)

*							(Routine_Name(input1,input2,output1))

*

*	INPUT ARGUMENT(S)		:

*

*		INPUT1			:	- Description :	- (flag = 0) : standard mode

*								- (flag  0) : frame stealing activated

*						- Format : Word16

*

*		INPUT2			:	- Description : Frame to be decoded

*							- Format : 432 * 16 bit-samples

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : Decoded frame

*							- Format : 286 * 16 bit-samples

*

*	RETURNED VALUE		:	None

*

*	COMMENTS			:	- Decoding of one frame : [-127 ... +127]

*							-127 : 1 almost sure

*							0    : 50% probability

*							+127 : 0 almost sure

*						- For the first bits, classical

*						  (punctured) Viterbi decoding ie one bit

*						  is decoded at a time,

*						- For the very last bits, block decoding.

*

**************************************************************************/

void	Rcpc_Decoding(Word16 FS_Flag, Word16 Input_Frame[],

Word16 Output_Frame[])

{

/* Variables */

Word16          index, j;

Word16          Index_puncturing;

Word16          Nber_decoded_bits;

Word16          Step, Pointer; /* rank of the bit being decoded :

0 <= Step_bis < Nber_decoded_bits */

Word16          Step_bis, Pointer_bis; /* Step is counted modulo 'Decoding_delay' */

Word16          Decoder_ready; /* Flag for starting decoding */

Word16          Essai;

Word16          Lsb;

Word16          Chosen_score, Chosen_lsb;

Word16          Maximum; /* Best score of the current Step */

Word16          Best;

Word16          Arrival_state,  Arrival_state_sym;

Word32          L_temp;

Word16          Size_Class0, Size_Class1, Size_Class2;

Word16          Size_Coded_Class1, Size_Coded_Class2, Size_Error_Control;

if (!FS_Flag) {

Size_Class0 = N0_2;

Size_Class1 = N1_2;

Size_Class2 = N2_2;

Size_Coded_Class1 = N1_2_coded;

Size_Coded_Class2 = N2_2_coded;

Size_Error_Control = SIZE_CRC;

} /* If FS_Flag */

else {

Size_Class0 = N0;

Size_Class1 = N1;

Size_Class2 = N2;

Size_Coded_Class1 = N1_coded;

Size_Coded_Class2 = N2_coded;

Size_Error_Control = Fs_SIZE_CRC;

}

/* Recopy of Class 0 (unprotected) */

for (j = 0; j < Size_Class0; j++)

Output_Frame[j] = Input_Frame[j];

/* Init of Nber_decoded_bits, number of bits decoded */

Nber_decoded_bits = 0;

/* Init of Step  */

Step = -1;         /* Step < Decoding_delay */

Step_bis = -1;

Decoder_ready = 0; /* Decoder not ready yet (Step_bis < Decoding_delay) */

/* Init of the scores (scores of the current Step) */

/* Starting State 0 is favoured */

Score[0] = 0;

for (j = 1; j <= M_1; j++)

Score[j] = - 16000;   /* M states */

/*-------------------------------------------------------------------------*/

/* Decoding of Class 1 */

/* Init of Index for Puncturing: 0 <= Index_puncturing < Period_pct */

Index_puncturing = 0;

for (index = 0; index < Size_Coded_Class1;) {   /* Loop on coded class 1 */

/* Loading of array Received with the received data after symetrization */

/* If Punctured data Received[data] = 0 else Received[data] = -(2*data - 1) */

#ifdef DEBUG

if ((A1[Index_puncturing]) != 0)

{

#endif

Received[0] = negate( sub( shl( Input_Frame[Size_Class0 +

index],(Word16)1 ),(Word16)1 ) );

index++;

#ifdef DEBUG

}

else

Received[0] = 0;

#endif

if ((A1[Index_puncturing + Period_pct]) != 0)

{

Received[1] = negate( sub( shl (Input_Frame[Size_Class0 +

index],(Word16)1 ),(Word16)1 ) );

index++;

}

else

Received[1] = 0;

#ifdef DEBUG

if ((A1[Index_puncturing + 2*Period_pct]) != 0)

{

Received[2] = negate( sub( shl (Input_Frame[Size_Class0 +

index],(Word16)1 ),(Word16)1 ) );

index++;

}

else

#endif

Received[2] = 0;

Index_puncturing++;

if (sub( Index_puncturing,(Word16)Period_pct ) == 0)

Index_puncturing = 0;

/* Determination of the state of highest score in the current Step */

Step++;

Step_bis++;

if (sub( Step,(Word16)Decoding_delay ) == 0) Step = 0;

/*   Computation of the best predecessor of each state and determination

of the best state */

Maximum = -32768;

for (j = 0; j <= M_1; j++)

Ex_score[j] = Score[j];

/* Loop on the states */

for (Arrival_state_sym = ((M_1 + 1)/2); Arrival_state_sym <= M_1; Arrival_state_sym++) {

for (Lsb = 0; Lsb <= 1; Lsb++) {    /* Loop on LSB */

/* For every state : Estimation of the best predecessor between

the one of LSB=0 and the one of LSB=1 */

Essai = Ex_score[Previous[Arrival_state_sym][Lsb]];

/* Accumulation of the transition due to the received data */

Essai = add( Essai,(Word16)((T1[Arrival_state_sym][Lsb] >

0) ? Received[0] : negate(Received[0])) );

Essai = add( Essai,(Word16)((T2[Arrival_state_sym][Lsb] >

0) ? Received[1] : negate(Received[1])) );

Essai = add( Essai,(Word16)((T3[Arrival_state_sym][Lsb] >

0) ? Received[2] : negate(Received[2])) );

/*  Search of the best predecessor (predecessor with the highest score) */

if (Lsb == 0) {

Chosen_score = Essai;

Chosen_lsb = Lsb;

}

else {

if (sub(Essai,Chosen_score) > 0) {

Chosen_score = Essai;

Chosen_lsb = Lsb;

}

}

} /* Loop on Lsb */

/* The score of the current state is the best among the two (Lsb=0,1) : */

Score[Arrival_state_sym] = Chosen_score;

Best_previous[Arrival_state_sym][Step] =

Previous[Arrival_state_sym][Chosen_lsb];

/*     Search of the best arrival state */

if (sub(Chosen_score,Maximum) > 0) {

Best = Arrival_state_sym;

Maximum = Chosen_score;

}

Arrival_state = sub( Arrival_state_sym, (Word16)((M_1 + 1)/2) );

for (Lsb = 0; Lsb <= 1; Lsb++) {    /* Loop on LSB */

/* For every state : Estimation of the best predecessor between

the one of LSB=0 and the one of LSB=1 */

Essai = Ex_score[Previous[Arrival_state][Lsb]];

/* Accumulation of the transition due to the received data */

Essai = add( Essai,(Word16)((T1[Arrival_state][Lsb]

> 0) ? Received[0] : negate(Received[0])) );

Essai = add( Essai,(Word16)((T2[Arrival_state][Lsb]

> 0) ? Received[1] : negate(Received[1])) );

Essai = add( Essai,(Word16)((T3[Arrival_state][Lsb]

> 0) ? Received[2] : negate(Received[2])) );

/*  Search of the best predecessor (predecessor with the highest score) */

if (Lsb == 0) {

Chosen_score = Essai;

Chosen_lsb = Lsb;

}

else {

if (sub(Essai,Chosen_score) > 0) {

Chosen_score = Essai;

Chosen_lsb = Lsb;

}

}

} /* Loop on Lsb */

/* The score of the current state is the best among the two (Lsb=0,1) : */

Score[Arrival_state] = Chosen_score;

Best_previous[Arrival_state][Step] =

Previous[Arrival_state][Chosen_lsb];

/*     Search of the best arrival state */

if(sub(Chosen_score,Maximum) > 0) {

Best = Arrival_state;

Maximum = Chosen_score;

}

} /* Loop on the states */

/* To avoid overflow, substraction of the highest score */

for (j = 0; j <= M_1; j++)

Score[j] = sub(Score[j],Maximum);

/* Check that the decoder is ready */

if (sub( Step,(Word16)(Decoding_delay - 1) ) == 0) Decoder_ready = 1;

if (sub( Decoder_ready, (Word16)1 ) == 0) {

/* Normal procedure : decoding of one bit at a time */

Pointer = Step;

for (j = 1; j <= Decoding_delay - 1; j++) {

Best =

Best_previous[Best][Pointer];

Pointer--;

if (Pointer < 0)

Pointer = Decoding_delay - 1;

} /* End Decoding of one bit */

L_temp = L_deposit_l(Best);

L_temp = L_shl( L_temp,(Word16)13 );

Output_Frame[Size_Class0 + Nber_decoded_bits] =

extract_h(L_temp);

Nber_decoded_bits++;

/* End Normal Decoding procedure */

} /* End if Decoder_ready */

} /* End Loop on class 1 */

/* End Decoding class 1 */

/*-------------------------------------------------------------------------*/

/*-------------------------------------------------------------------------*/

/* Decoding of Class 2 */

/* Init of Index for Puncturing: 0 <= Index_puncturing < Period_pct */

Index_puncturing = 0;

for (index = 0; index < Size_Coded_Class2; ) {   /* Loop on coded class 2 */

/* Loading of array Received with the received data after symetrization */

/* If Punctured data Received[data] = 0 else Received[data] = -(2*data - 1) */

#ifdef DEBUG

if ((A2[Index_puncturing]) != 0)

{

#endif

Received[0] = negate( sub( shl( Input_Frame[Size_Class0 +

Size_Coded_Class1 + index],(Word16)1 ),(Word16)1 ) );

index++;

#ifdef DEBUG

}

else

Received[0] = 0;

#endif

#ifdef DEBUG

if ((A2[Index_puncturing + Period_pct]) != 0)

{

#endif

Received[1] = negate( sub( shl( Input_Frame[Size_Class0 +

Size_Coded_Class1 + index],(Word16)1 ),(Word16)1 ) );

index++;

#ifdef DEBUG

}

else

Received[1] = 0;

#endif

if (!FS_Flag) {

if ((A2[Index_puncturing + 2*Period_pct]) != 0)

{

Received[2] = negate( sub( shl( Input_Frame[Size_Class0 +

Size_Coded_Class1 + index],(Word16)1 ),(Word16)1 ) );

index++;

}

else

Received[2] = 0;

}

else {

if ((Fs_A2[Index_puncturing + 2*Period_pct]) != 0)

{

Received[2] = negate( sub( shl( Input_Frame[Size_Class0 +

Size_Coded_Class1 + index],(Word16)1 ),(Word16)1 ) );

index++;

}

else

Received[2] = 0;

}

Index_puncturing++;

if (sub( Index_puncturing,(Word16)Period_pct ) == 0)

Index_puncturing = 0;

/* Determination of the state of highest score in the current Step */

Step++;

Step_bis++;

if (sub( Step,(Word16)Decoding_delay ) == 0) Step = 0;

/*   Computation of the best predecessor of each state and determination

of the best state */

Maximum = -32768;

for (j = 0; j <= M_1; j++)

Ex_score[j] = Score[j];

/* Loop on the states */

for (Arrival_state_sym = ((M_1 + 1)/2); Arrival_state_sym <= M_1; Arrival_state_sym++) {

/* Arrival_state_sym */

for (Lsb = 0; Lsb <= 1; Lsb++) {    /* Loop on LSB */

/* For every state : Estimation of the best predecessor between

the one of LSB=0 and the one of LSB=1 */

Essai = Ex_score[Previous[Arrival_state_sym][Lsb]];

/* Accumulation of the transition due to the received data */

Essai = add( Essai,(Word16)((T1[Arrival_state_sym][Lsb] >

0) ? Received[0] : negate(Received[0])) );

Essai = add( Essai,(Word16)((T2[Arrival_state_sym][Lsb] >

0) ? Received[1] : negate(Received[1])) );

Essai = add( Essai,(Word16)((T3[Arrival_state_sym][Lsb] >

0) ? Received[2] : negate(Received[2])) );

/*  Search of the best predecessor (predecessor with the highest score) */

if (Lsb == 0) {

Chosen_score = Essai;

Chosen_lsb = Lsb;

}

else {

if (sub(Essai,Chosen_score) > 0) {

Chosen_score = Essai;

Chosen_lsb = Lsb;

}

}

} /* Loop on Lsb */

/* The score of the current state is the best among the two (Lsb=0,1) : */

Score[Arrival_state_sym] = Chosen_score;

Best_previous[Arrival_state_sym][Step] =

Previous[Arrival_state_sym][Chosen_lsb];

if (sub(Chosen_score,Maximum) > 0) {

Best = Arrival_state_sym;

Maximum = Chosen_score;

}

Arrival_state = sub( Arrival_state_sym,(Word16)((M_1 + 1)/2) );

for (Lsb = 0; Lsb <= 1; Lsb++) {    /* Loop on LSB */

/* For every state : Estimation of the best predecessor between

the one of LSB=0 and the one of LSB=1 */

Essai = Ex_score[Previous[Arrival_state][Lsb]];

/* Accumulation of the transition due to the received data */

Essai = add( Essai,(Word16)((T1[Arrival_state][Lsb] > 0) ?

Received[0] : negate(Received[0])) );

Essai = add( Essai,(Word16)((T2[Arrival_state][Lsb] > 0) ?

Received[1] : negate(Received[1])) );

Essai = add( Essai,(Word16)((T3[Arrival_state][Lsb] > 0) ?

Received[2] : negate(Received[2])) );

/*  Search of the best predecessor (predecessor with the highest score) */

if (Lsb == 0) {

Chosen_score = Essai;

Chosen_lsb = Lsb;

}

else {

if (sub(Essai,Chosen_score) > 0) {

Chosen_score = Essai;

Chosen_lsb = Lsb;

}

}

} /* Loop on Lsb */

/* The score of the current state is the best among the two (Lsb=0,1) : */

Score[Arrival_state] = Chosen_score;

Best_previous[Arrival_state][Step] =

Previous[Arrival_state][Chosen_lsb];

/*     Search of the best arrival state */

if (sub(Chosen_score,Maximum) > 0) {

Best = Arrival_state;

Maximum = Chosen_score;

}

} /* Loop on the states */

/* To avoid overflow, substraction of the highest score */

for (j = 0; j <= M_1; j++)

Score[j] = sub(Score[j],Maximum);

/* Check that the decoder is ready */

/*------------------------------------------------------------------------*/

/* For the very last bits of the frame, we make a block decoding :

Decoding_delay bits are decoded at a time */

/*-------------------------------------------------------------------------*/

if (sub (Step_bis,(Word16)(Size_Class1 + Size_Class2 +

Size_Error_Control + (K - 1) - 1) ) == 0) {

/* At the end, we use the fact that the last state of the encoder is zero */

Best = 0;

Pointer_bis = Step_bis;

Pointer = Step;

/* Block decoding : */

for (j = 1; j <= Decoding_delay; j++) {

L_temp = L_deposit_l(Best);

L_temp = L_shl( L_temp,(Word16)13 );

Output_Frame[Size_Class0 + Pointer_bis] =

extract_h(L_temp);

Best =

Best_previous[Best][Pointer];

Nber_decoded_bits++;

Pointer_bis--;

Pointer--;

if (Pointer < 0)

Pointer = Decoding_delay - 1;

}

Nber_decoded_bits = sub( Nber_decoded_bits,(Word16)(K - 1) );

return;

} /* End block decoding */

/*-------------------------------------------------------------------------*/

else {

/* Normal procedure : decoding of one bit at a time */

Pointer = Step;

for (j = 1; j <= Decoding_delay - 1; j++) {

Best =

Best_previous[Best][Pointer];

Pointer--;

if (Pointer < 0)

Pointer = Decoding_delay - 1;

} /* End Decoding of one bit */

L_temp = L_deposit_l(Best);

L_temp = L_shl( L_temp,(Word16)13 );

Output_Frame[Size_Class0 + Nber_decoded_bits] =

extract_h(L_temp);

Nber_decoded_bits++;

}

/* End Normal Decoding procedure */

/* End if Decoder_ready */

} /* End Loop on class 2 */

/* End Decoding class 2 */

/*-------------------------------------------------------------------------*/

}

/**************************************************************************

*

*	ROUTINE				:	Read_Tetra_File

*

*	DESCRIPTION			:	Read a file in the TETRA hardware test

*							frame format

*

**************************************************************************

*

*	USAGE				:	Read_Tetra_File(file_pointer,buffer)

*							(Routine_Name(input1,output1))

*

*	INPUT ARGUMENT(S)		:

*

*		INPUT1			:	- Description : File pointer for the TETRA format file

*							- Format : FILE

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : buffer containing the TETRA frame

*							- Format : 432 * 16 bit-samples

*

*	RETURNED VALUE		:	0 if process correct, -1 if EOF

*

*	COMMENTS			:	Data read from the TETRA file are short integers

*

**************************************************************************/

short           Read_Tetra_File (FILE *fin, short *array)

{

static short    initial = 0;

static short    block[690];

short          *ptr_block;

short i;

short *start_of_array;

/* Return value: -1  EOF

*                0  array filled with TETRA frame

*/

start_of_array = array;

/* if first call to this routine, then skip past any header */

if (initial == 0)

{

while (*block != 0x6b21)

{

if (fread (block, sizeof (short), 1, fin) != 1)

return -1;

}

initial = 1;

if (fread (block+1, sizeof (short), 689, fin) != 689)

return -1;

} else    /* Read in TETRA frame */

{

if (fread (block, sizeof (short), 690, fin) != 690)

return -1;

}

/* Copy first valid block */

ptr_block = block+1;

for (i = 0; i < 114; i++)

*array++ = *ptr_block++;

/* Copy second valid block */

ptr_block = block + 161 - 45;

for (i = 0; i < 114; i++)

*array++ = *ptr_block++;

/* Copy third valid block */

ptr_block = block + 321 - 45 - 45;

for (i = 0; i < 114; i++)

*array++ = *ptr_block++;

/* Copy fourth valid block */

ptr_block = block + 481 - 45 - 45 - 45;

for (i = 0; i < 90; i++)

*array++ = *ptr_block++;

array = start_of_array;

for (i=0; i<432; i++)

{

if ((array[i] & 0x0080) == 0x0080)  array[i]

= array[i] | 0xFF00;

if ((array[i] > 127) || (array[i] < -127 ))

printf("Input soft bit out of range\n");

}

return 0;

}

/**************************************************************************

*

*	ROUTINE				:	Unbuild_Sensitivity_Classes

*

*

*	DESCRIPTION			:	Rebuilds two concatened speech frames from a single

*							frame ordered in three sensitivity classes

*

**************************************************************************

*

*	USAGE				:	Unbuild_Sensitivity_Classes(flag,buffer_in, buffer_out)

*							(Routine_Name(input1,input2,output1))

*

*	INPUT ARGUMENT(S)		:

*

*		INPUT1			:	- Description :	- (flag = 0) : standard mode

*								- (flag  0) : frame stealing activated

*							- Format : Word16

*

*		INPUT2			:	- Description : One ordered frame

*							- Format : 274 * 16 bit-samples

*

*	OUTPUT ARGUMENT(S)		:

*

*	OUTPUT1			:	- Description : Two concatened speech frames

*							          (length of one speech frame

*							          given by Length_vocoder_frame)

*						- Format : 274 * 16 bit-samples

*

*	RETURNED VALUE		:	None

*

*	COMMENTS			:	The way reordering is carried out is defined by

*						arrays TAB0 (length N0), TAB1 (length N1) and

*						TAB2 (length N2)

*

************************************************************************/

Word16     Unbuild_Sensitivity_Classes(Word16 FS_Flag, Word16 Input_Frame[], Word16 Output_Frame[])

{

/* Variables */

Word16     i;

if (!FS_Flag) {

/* Class 0 : */

for (i = 0; i < N0; i++) {

Output_Frame[TAB0[i] - 1] = Input_Frame[2*i];

Output_Frame[Length_vocoder_frame + TAB0[i] - 1] =

Input_Frame[2*i + 1];

}

/* Class 1 : */

for (i = 0; i < N1; i++) {

Output_Frame[TAB1[i] - 1] = Input_Frame[N0_2 + 2*i];

Output_Frame[Length_vocoder_frame + TAB1[i] - 1] =

Input_Frame[N0_2 + 2*i + 1];

}

/* Class 2 : */

for (i = 0; i < N2; i++) {

Output_Frame[TAB2[i] - 1] = Input_Frame[N0_2 + N1_2 + 2*i];

Output_Frame[Length_vocoder_frame + TAB2[i] - 1] =

Input_Frame[N0_2 + N1_2 + 2*i + 1];

}

} /* If FS_Flag */

else {

/* Class 0 : */

for (i = 0; i < N0; i++) {

Output_Frame[TAB0[i] - 1] = Input_Frame[i];

}

/* Class 1 : */

for (i = 0; i < N1; i++) {

Output_Frame[TAB1[i] - 1] = Input_Frame[N0 + i];

}

/* Class 2 : */

for (i = 0; i < N2; i++) {

Output_Frame[TAB2[i] - 1] = Input_Frame[N0 + N1 + i];

}

}

return(0);

}

/**************************************************************************

*

*	ROUTINE				:	Untransform_Class_0

*

*	DESCRIPTION			:	Transformation ("decoding") of class 0 of the frame

*						127 >= x >= 0 --> 0

*						0 > x >= -127 --> 1

*						the remaining of the frame is not modified

*

**************************************************************************

*

*	USAGE				:	Untransform_Class_0(flag,buffer)

*							(Routine_Name(arg1,arg2))

*

*	ARGUMENT(S)			:

*

*		ARG1				:	- Description :	- (flag = 0) : standard mode

*								- (flag  0) : frame stealing activated

*						- Format : Word16

*

*		ARG2				:	- Description : Ordered Frame (sensitivity classes)

*							          before or after decoding

*							          First bits  = class 0

*									= Unprotected bits

*						- Format : 286 * 16 bit-samples

*

*	RETURNED VALUE		:	None

*

**************************************************************************/

Word16     Untransform_Class_0(Word16 FS_Flag, Word16 Input_Frame[])

{

/* Variables */

Word16          i;

Word16          Size_Class0;

if (!FS_Flag)

Size_Class0 = N0_2;

else

Size_Class0 = N0;

for (i = 0; i < Size_Class0; i++) {

if (Input_Frame[i] >= 0)

Input_Frame[i] = 0;

else

Input_Frame[i] = 1;

} /* End Loop Size_Class0 */

return(0);  /* Normal completion */

}

# E	Makefile

This example of "makefile" written for an ANSI C compiler running within a UNIX environment can be used to compile the C code associated with the TETRA speech codec.

Similar "makefile" could be derived for other environments.

########################################################################

#

#	FILENAME		:	makefile

#

#	DESCRIPTION		:	Usable for the compilation of the C code

#					(UNIX environment)

#

#	LANGUAGE		:	ANSI-C

#

########################################################################

#

#	COMMENTS		:	Command line to be used (without the brackets)

#

#					- "make scoder"	: to compile the source coder

#					- "make ccoder"	: to compile the channel coder

#					- "make sdecoder"	: to compile the source decoder

#					- "make cdecoder"	: to compile the channel decoder

#					- "make"		: to compile all

#

########################################################################

# macro definitions

CC=acc

CFLAGS	=	-I.

SRCS1		=	scoder.c		scod_tet.c		sub_sc_d.c \

sub_dsp.c		fbas_tet.c		fexp_tet.c \

fmat_tet.c		tetra_op.c

SRCS2		=	ccoder.c		ccod_tet.c		sub_cc.c \

tetra_op.c

SRCS3		=	sdecoder.c		sdec_tet.c		sub_sc_d.c \

sub_dsp.c		fbas_tet.c		fexp_tet.c \

fmat_tet.c		tetra_op.c

SRCS4		=	cdecoder.c		cdec_tet.c		sub_cd.c \

tetra_op.c

# target definitions

all:		scoder	ccoder	sdecoder	cdecoder

scoder:

$(CC) $(SRCS1) $(CFLAGS) -o scoder

ccoder:

$(CC) $(SRCS2) $(CFLAGS) -o ccoder

sdecoder:

$(CC) $(SRCS3) $(CFLAGS) -o sdecoder

cdecoder:

$(CC) $(SRCS4) $(CFLAGS) -o cdecoder

clean:

rm -f core *.o

# F 	Indexes

## F.1	Index of Routines

In this index are listed all the routines included in the C code.

Routines are classified according to their names. The second column relates to the source file in which they are located, the third one to the section number and the fourth one to the page number in this document.

All source files have the extension ".c".

A

abs_s			tetra_op	B.1		159

add			tetra_op	B.1		160

add_sh			fbas_tet	A.5.1		102

add_sh16		fbas_tet	A.5.1		103

Autocorr		sub_dsp	A.4		77

Az_Lsp			sub_dsp	A.4		79

B

Back_Fil		sub_dsp	A.4		82

Bfi			sub_cd		D.2		225

bin2int			fbas_tet	A.5.1		104

Bits2prm_Tetra		sub_sc_d	A.3		29

Build_Crc		sub_cc		C.2		197

Build_Sensitivity_Cl...	sub_cc		C.2		199

C

Cal_Rr2		sub_sc_d	A.3		30

ccoder			ccoder		C.1		190

cdecoder		cdecoder	D.1		219

Channel_Decoding	cdec_tet	D.1		223

Channel_Encoding	ccod_tet	C.1		194

Chebps			sub_dsp	A.4		84

Clsp_334		sub_sc_d	A.3		32

Coder_Tetra		scod_tet	A.1		13

Combination		sub_cc		C.2		201

Combination		sub_cd		D.2		228

Convolve		sub_dsp	A.4		86

D

D_D4i60		sub_sc_d	A.3		46

D_Lsp334		sub_sc_d	A.3		48

D4i60_16		sub_sc_d	A.3		39

Dec_Ener		sub_sc_d	A.3		35

Decod_Tetra		sdec_tet	A.2		24

Desinterleaving_Sig...	sub_cd		D.2		229

Desinterleaving_Spe..	sub_cd		D.2		230

div_32			fexp_tet	A.5.2		117

div_s			tetra_op	B.1		161

E

Ener_Qua		sub_sc_d	A.3		50

extract_h		tetra_op	B.1		163

extract_l		tetra_op	B.1		164

F

Fac_Pond		sub_dsp	A.4		87

G

G_Code		sub_sc_d	A.3		55

G_Pitch			sub_sc_d	A.3		57

Get_Lsp_Pol		sub_dsp	A.4		88

I

Init_Coder_Tetra	scod_tet	A.1		11

Init_Decod_Tetra	sdec_tet	A.2		23

Init_Rcpc_Coding	sub_cc		C.2		202

Init_Rcpc_Decoding	sub_cd		D.2		231

Int_Lpc4		sub_dsp	A.4		89

int2bin			fbas_tet	A.5.1		105

Inter32_1_3		sub_sc_d	A.3		62

Inter32_M1_3		sub_sc_d	A.3		61

Inter8_1_3		sub_sc_d	A.3		60

Inter8_M1_3		sub_sc_d	A.3		59

Interleaving_Signal...	sub_cc		C.2		204

Interleaving_Speech	sub_cc		C.2		205

inv_sqrt		fmat_tet	A.5.3		120

L

L_abs			tetra_op	B.1		165

L_add			tetra_op	B.1		166

L_comp		fexp_tet	A.5.2		113

L_deposit_h		tetra_op	B.1		167

L_deposit_l		tetra_op	B.1		168

L_extract		fexp_tet	A.5.2		114

L_mac			tetra_op	B.1		169

L_mac0		tetra_op	B.1		170

L_msu			tetra_op	B.1		171

L_msu0		tetra_op	B.1		172

L_mult			tetra_op	B.1		173

L_mult0		tetra_op	B.1		174

L_negate		tetra_op	B.1		175

L_shl			tetra_op	B.1		176

L_shr			tetra_op	B.1		177

L_shr_r			tetra_op	B.1		178

L_sub			tetra_op	B.1		179

Lag_Max		sub_sc_d	A.3		63

Lag_Window		sub_dsp	A.4		90

Levin_32		sub_dsp	A.4		91

Load_sh		fbas_tet	A.5.1		106

Load_sh16		fbas_tet	A.5.1		107

Log2			fmat_tet	A.5.3		122

Lpc_Gain		sub_dsp	A.4		95

Lsp_Az			sub_dsp	A.4		96

M

mpy_32		fexp_tet	A.5.2		116

mpy_mix		fexp_tet	A.5.2		115

mult			tetra_op	B.1		180

mult_r			tetra_op	B.1		181

N

negate			tetra_op	B.1		182

Norm_Corr		sub_sc_d	A.3		65

norm_l			tetra_op	B.1		183

norm_s			tetra_op	B.1		184

norm_v			fbas_tet	A.5.1		108

P

Pitch_Fr		sub_sc_d	A.3		67

Pitch_Ol_Dec		sub_sc_d	A.3		69

Pond_Ai		sub_dsp	A.4		97

Post_Process		sub_sc_d	A.3		71

pow2			fmat_tet	A.5.3		124

Pre_Process		sub_sc_d	A.3		73

Pred_Lt			sub_sc_d	A.3		72

Prm2bits_Tetra		sub_sc_d	A.3		75

R

Rcpc_Coding		sub_cc		C.2		206

Rcpc_Decoding		sub_cd		D.2		232

Read_Tetra_File	sub_cd		D.2		239

Residu			sub_dsp	A.4		98

round			tetra_op	B.1		185

S

sature			tetra_op	B.1		186

scoder			scoder		A.1		6

sdecoder		sdecoder	A.2		19

shl			tetra_op	B.1		187

shr			tetra_op	B.1		188

store_hi		fbas_tet	A.5.1		109

sub			tetra_op	B.1		189

sub_sh			fbas_tet	A.5.1		110

sub_sh16		fbas_tet	A.5.1		111

Syn_Filt		sub_dsp	A.4		99

T

Transform_Class_0	sub_cc		C.2		209

U

Unbuild_Sensitivity...	sub_cd		D.2		241

Untransform_Class_0	sub_cd		D.2		243

W

Write_Tetra_File	sub_cc		C.2		210

## F.2	Index of Files

In this index are listed all the computer files containing C code for the TETRA speech codec.

Files are classified according to their names. The second column relates to the section number and the third one to the page number in this document.

File extensions are given. By convention, extension "tab" is used for files containing only numerical data and extension "h" for files containing definitions and prototypes.

A

arrays.tab		C.3		215

C

ccoder.c		C.1		190

ccod_tet.c		C.1		193

cdecoder.c		D.1		219

cdec_tet.c		D.1		222

channel.h		C.3		212

clsp_334.tab		A.6		129

const.tab		C.3		218

E

ener_qua.tab		A.6		149

F

fbas_tet.c		A.5.1		101

fexp_tet.c		A.5.2		112

fmat_tet.c		A.5.3		119

G

grid.tab			A.6		151

I

inv_sqrt.tab		A.6		152

L

lag_wind.tab		A.6		153

log2.tab		A.6		154

M

makefile		E		244

P

pow2.tab		A.6		155

S

scoder.c		A.1		6

scod_tet.c		A.1		9

sdecoder.c		A.2		19

sdec_tet.c		A.2		21

source.h		A.6		126

sub_cc.c		C.2		196

sub_cd.c		D.2		224

sub_sc_d.c		A.3		28

sub_dsp.c		A.4		76

T

tetra_op.c		B.1		157

W

window.tab		A.6		156

# History

| Document history |  |  |
| --- | --- | --- |
| Date | Status | Comment |
| June 1995 |  | 1st printing of C code coming from TETRA documents 06.20, 05.11 and 05.12 merged together |
| March 1996 |  | Comments from PE92 taken into account according to results of Resolution Meetings |
| March 2024 |  | New MS Word format |