# A	C Code for Full Rate Speech Source Codec

## A.1	Main Program for Source Coder

/************************************************************************

*

*	FILENAME		:	scoder					.c

*

*	DESCRIPTION		:	Main program for speech source encoding

*

************************************************************************

*

*	USAGE			:	scoder  speech_file  serial_file  synth_file

*					(executable_file input1 output1 output2)

*

*	INPUT FILE(S)		:

*

*		INPUT1	:	- Description : speech file to be analyzed

*					- Format : binary file 16 bit-samples

*					  240 samples per frame

*

*

*	OUTPUT FILE(S)	:

*

*		OUTPUT1	:	- Description : serial stream output file

*					- Format : binary file 16 bit-samples

*					  each 16 bit-sample represents one encoded bit

*					  138 (= 1 + 137) bits per frame

*

*		OUTPUT2	:	- Description : local synthesis output file

*					- Format : binary file 16 bit-samples

*

*	COMMENTS		:	First sample of each OUTPUT1 frame is forced to zero

*

************************************************************************

*

*	INCLUDED FILES	:	source.h

*					stdio.h

*					stdlib.h

*

************************************************************************/

#include <stdio.h>

#include <stdlib.h>

#include "source.h"

/*-----------------*

* Constants       *

*-----------------*/

#define L_frame     240

#define serial_size 138

#define ana_size     23

int main(int argc, char *argv[] )

{

Word16 frame;

extern Word16 *new_speech;		/* Pointer on new_speech. */

Word16 syn[L_frame];			/* Local synthesis.       */

Word16 ana[ana_size];			/* Analysis parameters.   */

Word16 serial[serial_size];		/* Serial stream.         */

FILE  *f_speech, *f_syn, *f_serial;

/* Passed arguments */

if ( argc != 4 )

{

printf("Usage : scoder  speech_file  serial_file  synth_file\n");

printf("\n");

printf("Format for speech_file:\n");

printf("  Speech is read form a binary file of 16 bits data.\n");

printf("\n");

printf("Format for serial_file:\n");

printf("  Serial stream output is written to a binary file\n");

printf("  where each 16-bit word represents 1 encoded bit.\n");

printf("  BFI(=0) + 137 bits by frame\n");

printf("\n");

printf("Format for synth_file:\n");

printf("  Local synthesis is written to a binary file of 16 bits data.\n");

exit( 1 );

}

/* Open files for speech, serial stream and local synthesis */

if( (f_speech = fopen(argv[1],"rb") ) == NULL )

{

printf("Input file '%s' does not exist !!\n", argv[1]);

exit(0);

}

if( (f_serial = fopen(argv[2], "wb") ) == NULL )

{

printf("Cannot open file '%s' !!\n", argv[2]);

exit(0);

}

if( (f_syn = fopen(argv[3], "wb") ) == NULL )

{

printf("Cannot open file '%s' !!\n", argv[3]);

exit(0);

}

/* Initialization of the coder */

Init_Pre_Process();

Init_Coder_Tetra();

/* Loop for each "L_frame" speech data. */

frame =0;

while( fread(new_speech, sizeof(Word16), L_frame, f_speech) == L_frame)

{

printf("frame=%d\n", ++frame);

Pre_Process(new_speech, (Word16)L_frame);	/* Pre processing of input

speech */

Coder_Tetra(ana, syn);              /* Find speech parameters         */

Post_Process(syn, (Word16)L_frame); /* Post processing of synthesis   */

Prm2bits_Tetra(ana, serial);        /* Parameters to serial bits      */

fwrite(syn,    sizeof(Word16), L_frame    ,  f_syn);

fwrite(serial, sizeof(Word16), serial_size,  f_serial);

}

return (EXIT_SUCCESS);

}

/************************************************************************

*

*	FILENAME		:	scod_tet.c

*

*	DESCRIPTION		:	Main routines for speech source encoding

*

************************************************************************

*

*	SUB-ROUTINES	:	- Init_Coder_Tetra()

*					- Coder_Tetra()

*

************************************************************************

*

*	INCLUDED FILES	:	source.h

*

************************************************************************/

#include "source.h"

/*----------------------------------------------------------------------*

*         Coder constant parameters.                                   *

*                                                                      *

*   L_window    : LPC analysis window size.                            *

*   L_next      : Samples of next frame needed for autocor.            *

*   L_frame     : Frame size.                                          *

*   L_subfr     : Sub-frame size.                                      *

*   p           : LPC order.                                           *

*   pp1         : LPC order+1                                          *

*   L_total     : Total speech size.                                   *

*   dim_rr      : Dimension of matrix rr[][].                          *

*   pit_min     : Minimum pitch lag.                                   *

*   pit_max     : Maximum pitch lag.                                   *

*   L_inter     : Length of filter for interpolation                   *

*----------------------------------------------------------------------*/

#define  L_window (Word16)256

#define  L_next   (Word16)40

#define  L_frame  (Word16)240

#define  L_subfr  (Word16)60

#define  p        (Word16)10

#define  pp1      (Word16)11

#define  L_total  (Word16)(L_frame+L_next+p)

#define  dim_rr   (Word16)32

#define  pit_min  (Word16)20

#define  pit_max  (Word16)143

#define  L_inter  (Word16)15

/*--------------------------------------------------------*

*   LPC bandwidth expansion factors.                     *

*      In Q15 = 0.95, 0.60, 0.75, 0.85                   *

*--------------------------------------------------------*/

#define gamma1  (Word16)31130

#define gamma2  (Word16)19661

#define gamma3  (Word16)24576

#define gamma4  (Word16)27853

/*--------------------------------------------------------*

*         Static memory allocation.                      *

*--------------------------------------------------------*/

/* Speech vector */

static Word16 old_speech[L_total];

static Word16 *speech, *p_window;

Word16 *new_speech;                    /* Global variable */

/* Weighted speech vector */

static Word16 old_wsp[L_frame+pit_max];

static Word16 *wsp;

/* Excitation vector */

static Word16 old_exc[L_frame+pit_max+L_inter];

static Word16 *exc;

/* All-zero vector */

static Word16 ai_zero[L_subfr+pp1];

static Word16 *zero;

/* Spectral expansion factors */

static Word16 F_gamma1[p];

static Word16 F_gamma2[p];

static Word16 F_gamma3[p];

static Word16 F_gamma4[p];

/* Lsp (Line Spectral Pairs in the cosine domain) */

static Word16 lspold[p]={

30000, 26000, 21000, 15000, 8000, 0,

-8000,-15000,-21000,-26000};

static Word16 lspnew[p];

static Word16 lspnew_q[p], lspold_q[p];

/* Initial lsp values used after each time */

/* a reset is executed */

static Word16 lspold_init[p]={

30000, 26000, 21000, 15000, 8000, 0,

-8000,-15000,-21000,-26000};

/* Filters memories */

static Word16 mem_syn[p], mem_w0[p], mem_w[p];

/* Matrix rr[dim_rr][dim_rr] */

static Word16 rr[dim_rr][dim_rr];

/* Global definition */

Word16 last_ener_cod;

Word16 last_ener_pit;

/**************************************************************************

*

*	ROUTINE				:	Init_Coder_Tetra

*

*	DESCRIPTION			:	Initialization of variables for the speech encoder

*

**************************************************************************

*

*	USAGE				:	Init_Coder_Tetra()

*

*	INPUT ARGUMENT(S)		:	None

*

*	OUTPUT ARGUMENT(S)		:	None

*

*	RETURNED VALUE		:	None

*

**************************************************************************/

void Init_Coder_Tetra(void)

{

Word16 i,j;

/*-----------------------------------------------------------------------*

*      Initialize pointers to speech vector.                            *

*                                                                       *

*                                                                       *

*   |--------------------|--------|--------|......|-------|-------|     *

*     previous speech       sf1      sf2             sf4    L_next      *

*                                                                       *

*   <----------------  Total speech vector (L_total)   ----------->     *

*   |           <----  LPC analysis window (L_window)  ----------->     *

*   |           |        <---- present frame (L_frame) --->             *

*  old_speech   |        |       <----- new speech (L_frame) ----->     *

*            p_window    |       |                                      *

*                     speech     |                                      *

*                             new_speech                                *

*-----------------------------------------------------------------------*/

new_speech = old_speech + L_total - L_frame;	/* New speech     */

speech     = new_speech - L_next;			/* Present frame  */

p_window   = old_speech + L_total - L_window;	/* For LPC window */

/* Initialize global variables */

last_ener_cod = 0;

last_ener_pit = 0;

/* Initialize static pointers */

wsp    = old_wsp + pit_max;

exc    = old_exc + pit_max + L_inter;

zero   = ai_zero + pp1;

/* Static vectors to zero */

for(i=0; i<L_total; i++)

old_speech[i] = 0;

for(i=0; i<pit_max + L_inter; i++)

old_exc[i] = old_wsp[i] = 0;

for(i=0; i<p; i++)

mem_syn[i] = mem_w[i] = mem_w0[i] = 0;

for(i=0; i<L_subfr; i++)

zero[i] = 0;

for(i=0; i< dim_rr; i++)

for(j=0; j< dim_rr; j++)

rr[i][j] = 0;

/* Initialisation of lsp values for first */

/* frame lsp interpolation */

for(i=0; i<p; i++)

lspold_q[i] = lspold[i] = lspold_init[i];

/* Compute LPC spectral expansion factors */

Fac_Pond(gamma1, F_gamma1);

Fac_Pond(gamma2, F_gamma2);

Fac_Pond(gamma3, F_gamma3);

Fac_Pond(gamma4, F_gamma4);

return;

}

/**************************************************************************

*

*	ROUTINE				:	Coder_Tetra

*

*	DESCRIPTION			:	Main speech coder function

*

**************************************************************************

*

*	USAGE				:	Coder_Tetra(ana,synth)

*							(Routine_Name(output1,output2))

*

*	INPUT ARGUMENT(S)		:	None

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : Analysis parameters

*							- Format : 23 * 16 bit-samples

*

*		OUTPUT2			:	- Description : Local synthesis

*							- Format : 240 * 16 bit-samples

*

*	RETURNED VALUE		:	None

*

*	COMMENTS			:	- 240 speech data should have been copied to vector

*						new_speech[].  This vector is global and is declared in

*						this function.

*						- Output2 is for debugging only

*

**************************************************************************/

void Coder_Tetra(Word16 ana[], Word16 synth[])

{

/* LPC coefficients */

Word16 r_l[pp1], r_h[pp1];	/* Autocorrelations low and high        */

Word16 A_t[(pp1)*4];		/* A(z) unquantized for the 4 subframes */

Word16 Aq_t[(pp1)*4];		/* A(z)   quantized for the 4 subframes */

Word16 Ap1[pp1];		/* A(z) with spectral expansion         */

Word16 Ap2[pp1];		/* A(z) with spectral expansion         */

Word16 Ap3[pp1];		/* A(z) with spectral expansion         */

Word16 Ap4[pp1];		/* A(z) with spectral expansion         */

Word16 *A, *Aq;			/* Pointer on A_t and Aq_t              */

/* Other vectors */

Word16 h1[L_subfr];

Word16 zero_h2[L_subfr+64], *h2;

Word16 zero_F[L_subfr+64],  *F;

Word16 res[L_subfr];

Word16 xn[L_subfr];

Word16 xn2[L_subfr];

Word16 dn[L_subfr+4];

Word16 code[L_subfr+4];

Word16 y1[L_subfr];

Word16 y2[L_subfr];

/* Scalars */

Word16 i, i_subfr;

Word16 T0, T0_min, T0_max, T0_frac;

Word16 gain_pit, gain_code, index;

Word16 sign_code, shift_code;

Word16 temp;

Word32 L_temp;

/* Initialization of F and h2 */

F  = &zero_F[64];

h2 = &zero_h2[64];

for(i=0; i<64; i++)

zero_F[i] = zero_h2[i] = 0;

/*------------------------------------------------------------------------*

*  - Perform LPC analysis:                                               *

*       * autocorrelation + lag windowing                                *

*       * Levinson-Durbin algorithm to find a[]                          *

*       * convert a[] to lsp[]                                           *

*       * quantize and code the LSPs                                     *

*       * find the interpolated LSPs and convert to a[] for all          *

*         subframes (both quantized and unquantized)                     *

*------------------------------------------------------------------------*/

Autocorr(p_window, p, r_h, r_l);		/* Autocorrelations */

Lag_Window(p, r_h, r_l);			/* Lag windowing    */

Levin_32(r_h, r_l, A_t);			/* Levinson-Durbin  */

Az_Lsp(A_t, lspnew, lspold);		/* From A(z) to lsp */

Clsp_334(lspnew, lspnew_q, ana);		/* Lsp quantization */

ana += 3;				/* Increment analysis parameters pointer */

/* Interpolation of LPC for the 4 subframes */

Int_Lpc4(lspold,   lspnew,   A_t);

Int_Lpc4(lspold_q, lspnew_q, Aq_t);

/* update the LSPs for the next frame */

for(i=0; i<p; i++)

{

lspold[i]   = lspnew[i];

lspold_q[i] = lspnew_q[i];

}

/*----------------------------------------------------------------------*

* - Find the weighted input speech wsp[] for the whole speech frame    *

* - Find open-loop pitch delay                                         *

* - Set the range for searching closed-loop pitch                      *

*----------------------------------------------------------------------*/

A = A_t;

for (i = 0; i < L_frame; i += L_subfr)

{

Pond_Ai(A, F_gamma1, Ap1);

Pond_Ai(A, F_gamma2, Ap2);

Residu(Ap1, &speech[i], &wsp[i], L_subfr);

Syn_Filt(Ap2, &wsp[i], &wsp[i], L_subfr, mem_w, (Word16)1);

A += pp1;

}

/* Find open loop pitch delay */

T0 = Pitch_Ol_Dec(wsp, L_frame);

/* range for closed loop pitch search */

T0_min = sub(T0, (Word16)2);

if (T0_min < pit_min) T0_min = pit_min;

T0_max = add(T0_min, (Word16)4);

if (T0_max > pit_max)

{

T0_max = pit_max;

T0_min = sub(T0_max, (Word16)4);

}

/*------------------------------------------------------------------------*

*          Loop for every subframe in the analysis frame                 *

*------------------------------------------------------------------------*

*  To find the pitch and innovation parameters. The subframe size is     *

*  L_subfr and the loop is repeated L_frame/L_subfr times.               *

*     - find the weighted LPC coefficients                               *

*     - find the LPC residual signal res[]                               *

*     - compute the target signal for pitch search                       *

*     - compute impulse response of weighted synthesis filter (h1[])     *

*     - find the closed-loop pitch parameters                            *

*     - encode the pitch delay                                           *

*     - update the impulse response h1[] by including fixed-gain pitch   *

*     - find the autocorrelations of h1[] (rr[][])                       *

*     - find target vector for codebook search                           *

*     - backward filtering of target vector                              *

*     - codebook search                                                  *

*     - encode codebook address                                          *

*     - VQ of pitch and codebook gains                                   *

*     - find synthesis speech                                            *

*     - update states of weighting filter                                *

*------------------------------------------------------------------------*/

Aq = Aq_t;	/* pointer to interpolated quantized LPC parameters */

for (i_subfr = 0;  i_subfr < L_frame; i_subfr += L_subfr)

{

/*---------------------------------------------------------------*

* Find the weighted LPC coefficients for the weighting filter.  *

*---------------------------------------------------------------*/

Pond_Ai(Aq, F_gamma3, Ap3);

Pond_Ai(Aq, F_gamma4, Ap4);

/*---------------------------------------------------------------*

* Compute impulse response, h1[], of weighted synthesis filter  *

*---------------------------------------------------------------*/

ai_zero[0] = 4096;				/* 1 in Q12 */

for (i = 1; i <= p; i++) ai_zero[i] = 0;

Syn_Filt(Ap4, ai_zero, h1, L_subfr, zero, (Word16)0);

/*---------------------------------------------------------------*

* Compute LPC residual and copy it to exc[i_subfr]              *

*---------------------------------------------------------------*/

Residu(Aq, &speech[i_subfr], res, L_subfr);

for(i=0; i<L_subfr; i++) exc[i_subfr+i] = res[i];

/*---------------------------------------------------------------*

* Find the target vector for pitch search:  ->xn[]              *

*---------------------------------------------------------------*/

Syn_Filt(Ap4, res, xn, L_subfr, mem_w0, (Word16)0);

/*----------------------------------------------------------------------*

*                 Closed-loop fractional pitch search                  *

*----------------------------------------------------------------------*

* The pitch range for the first subframe is divided as follows:        *

*   19 1/3  to   84 2/3   resolution 1/3                               *

*   85      to   143      resolution 1                                 *

*                                                                      *

* The period in the first subframe is encoded with 8 bits.             *

* For the range with fractions:                                        *

*   code = (T0-19)*3 + frac - 1;   where T0=[19..85] and frac=[-1,0,1] *

* and for the integer only range                                       *

*   code = (T0 - 85) + 197;        where T0=[86..143]                  *

*----------------------------------------------------------------------*

* For other subframes: if t0 is the period in the first subframe then  *

* T0_min=t0-5   and  T0_max=T0_min+9   and  the range is given by      *

*      T0_min-1 + 1/3   to  T0_max + 2/3                               *

*                                                                      *

* The period in the 2nd,3rd,4th subframe is encoded with 5 bits:       *

*  code = (T0-(T0_min-1))*3 + frac - 1;  where T0[T0_min-1 .. T0_max+1]*

*---------------------------------------------------------------------*/

T0 = Pitch_Fr(&exc[i_subfr], xn, h1, L_subfr, T0_min, T0_max,

i_subfr, &T0_frac);

if (i_subfr == 0)

{

/* encode pitch delay (with fraction) */

if (T0 <= 85)

{

/* index = T0*3 - 58 + T0_frac; */

index = add(T0, add(T0, T0));

index = sub(index, (Word16)58);

index = add(index, T0_frac);

}

else

index = add(T0, (Word16)112);

/* find T0_min and T0_max for other subframes */

T0_min = sub(T0, (Word16)5);

if (T0_min < pit_min) T0_min = pit_min;

T0_max = add(T0_min, (Word16)9);

if (T0_max > pit_max)

{

T0_max = pit_max;

T0_min = sub(T0_max, (Word16)9);

}

}

else						/* other subframes */

{

i = sub(T0, T0_min);

/* index = i*3 + 2 + T0_frac;  */

index = add(i, add(i, i));

index = add(index, (Word16)2);

index = add(index, T0_frac);

}

*ana++ = index;

/*-----------------------------------------------------------------*

*   - find unity gain pitch excitation (adaptive codebook entry)  *

*     with fractional interpolation.                              *

*   - find filtered pitch exc. y1[]=exc[] filtered by 1/Ap4(z)    *

*   - compute pitch gain and limit between 0 and 1.2              *

*   - update target vector for codebook search                    *

*-----------------------------------------------------------------*/

Pred_Lt(&exc[i_subfr], T0, T0_frac, L_subfr);

Syn_Filt(Ap4, &exc[i_subfr], y1, L_subfr, zero, (Word16)0);

gain_pit = G_Pitch(xn, y1, L_subfr);

/* xn2[i] = xn[i] - y1[i]*gain_pit */

for (i = 0; i < L_subfr; i++)

{

L_temp = L_mult(y1[i], gain_pit);

L_temp = L_shl(L_temp, (Word16)3);	/* gain_pit in Q12 */

L_temp = L_sub( Load_sh16(xn[i]), L_temp);

xn2[i] = extract_h(L_temp);

}

/*----------------------------------------------------------------*

* -Compute impulse response F[] and h2[] for innovation codebook *

* -Find correlations of h2[];  rr[i][j] = sum h2[n-i]*h2[n-j]    *

*----------------------------------------------------------------*/

for (i = 0; i <= p; i++) ai_zero[i] = Ap3[i];

Syn_Filt(Ap4, ai_zero, F, L_subfr, zero, (Word16)0);

/* Introduce pitch contribution with fixe gain of 0.8 to F[] */

for (i = T0; i < L_subfr; i++)

{

temp = mult(F[i-T0], (Word16)26216);

F[i] = add(F[i], temp);

}

/* Compute h2[]; -> F[] filtered by 1/Ap4(z) */

Syn_Filt(Ap4, F, h2, L_subfr, zero, (Word16)0);

Cal_Rr2(h2, (Word16*)rr);

/*-----------------------------------------------------------------*

* - Backward filtering of target vector (find dn[] from xn2[])    *

* - Innovative codebook search (find index and gain)              *

*-----------------------------------------------------------------*/

Back_Fil(xn2, h2, dn, L_subfr);	/* backward filtered target vector dn */

*ana++ =D4i60_16(dn,F,h2, rr, code, y2, &sign_code, &shift_code);

*ana++ = sign_code;

*ana++ = shift_code;

gain_code = G_Code(xn2, y2, L_subfr);

/*-----------------------------------------------------------------*

* - Quantization of gains.                                        *

*-----------------------------------------------------------------*/

*ana++ = Ener_Qua(Aq,&exc[i_subfr],code, L_subfr, &gain_pit, &gain_code);

/*-------------------------------------------------------*

* - Find the total excitation                           *

* - Update filter memory mem_w0 for finding the target  *

*   vector in the next subframe.                        *

*   The filter state mem_w0[] is found by filtering by  *

*   1/Ap4(z) the error between res[i] and exc[i]        *

*-------------------------------------------------------*/

for (i = 0; i < L_subfr;  i++)

{

/* exc[i] = gain_pit*exc[i] + gain_code*code[i]; */

/* exc[i]  in Q0   gain_pit in Q12               */

/* code[i] in Q12  gain_cod in Q0                */

L_temp = L_mult0(exc[i+i_subfr], gain_pit);

L_temp = L_mac0(L_temp, code[i], gain_code);

exc[i+i_subfr] = L_shr_r(L_temp, (Word16)12);

}

for(i=0; i<L_subfr; i++)

res[i] = sub(res[i], exc[i_subfr+i]);

Syn_Filt(Ap4, res, code, L_subfr, mem_w0, (Word16)1);

/* Note: we use vector code[] as output only as temporary vector */

/*-------------------------------------------------------*

* - find synthesis speech corresponding to exc[]        *

*   This filter is to help debug only.                  *

*-------------------------------------------------------*/

Syn_Filt(Aq, &exc[i_subfr], &synth[i_subfr], L_subfr, mem_syn,

(Word16)1);

Aq += pp1;

}

/*--------------------------------------------------*

* Update signal for next frame.                    *

* -> shift to the left by L_frame:                 *

*     speech[], wsp[] and  exc[]                   *

*--------------------------------------------------*/

for(i=0; i< L_total-L_frame; i++)

old_speech[i] = old_speech[i+L_frame];

for(i=0; i<pit_max; i++)

old_wsp[i] = old_wsp[i+L_frame];

for(i=0; i<pit_max+L_inter; i++)

old_exc[i] = old_exc[i+L_frame];

return;

}

## A.2	Main Program for Source Decoder

/************************************************************************

*

*	FILENAME		:	sdecoder			.c

*

*	DESCRIPTION		:	Main program for speech source decoding

*

************************************************************************

*

*	USAGE			:	sdecoder  serial_file  synth_file

*					(executable_file input1 output1)

*

*	INPUT FILE(S)		:

*

*		INPUT1	:	- Description : serial stream input file

*					- Format : binary file 16 bit-samples

*					  138 (= 1 + 137) samples per frame

*

*	OUTPUT FILE(S)	:

*

*		OUTPUT1	:	- Description : synthesis output file

*					- Format : binary file 16 bit-samples

*					  240 bits per frame

*

*	COMMENTS		:	First sample of each INPUT1 frame is the bad frame

*					indicator (BFI), BFI = 0 corresponds to a valid frame,

*					BFI = 1 to corrupted data

*

************************************************************************

*

*	INCLUDED FILES	:	source.h

*					stdio.h

*					stdlib.h

*

************************************************************************/

#include <stdio.h>

#include <stdlib.h>

#include "source.h"

/*-----------------*

* Constants       *

*-----------------*/

#define L_frame     240

#define serial_size 138

#define prm_size    24

int main(int argc, char *argv[] )

{

Word16 frame;

Word16 synth[L_frame];		/* Synthesis              */

Word16 parm[prm_size];		/* Synthesis parameters   */

Word16 serial[serial_size];		/* Serial stream          */

FILE   *f_syn, *f_serial;

/* Passed arguments */

if ( argc != 3 )

{

printf("Usage : sdecoder  serial_file  synth_file\n");

printf("\n");

printf("Format for serial_file:\n");

printf("  Serial stream input is read from a binary file\n");

printf("  where each 16-bit word represents 1 encoded bit.\n");

printf("  BFI + 137 bits by frame\n");

printf("\n");

printf("Format for synth_file:\n");

printf("  Synthesis is written to a binary file of 16 bits data.\n");

exit( 1 );

}

/* Open file for synthesis and serial stream */

if( (f_serial = fopen(argv[1],"rb") ) == NULL )

{

printf("Input file '%s' does not exist !!\n",argv[1]);

exit(0);

}

if( (f_syn = fopen(argv[2], "wb") ) == NULL )

{

printf("Cannot open file '%s' !!\n", argv[2]);

exit(0);

}

/* Initialization of decoder  */

Init_Decod_Tetra();

/* Loop for each "L_frame" speech data. */

frame =0;

while( fread(serial, sizeof(Word16), serial_size, f_serial) == serial_size)

{

printf("frame=%d\n", ++frame);

Bits2prm_Tetra(serial, parm);	/* serial to parameters */

Decod_Tetra(parm, synth);		/* decoder */

Post_Process(synth, (Word16)L_frame);	/* Post processing of synthesis  */

fwrite(synth, sizeof(Word16), L_frame,  f_syn);   /* Write output file */

}

return (EXIT_SUCCESS);

}

/************************************************************************

*

*	FILENAME		:	sdec_tet.c

*

*	DESCRIPTION		:	Main routines for speech source decoding

*

************************************************************************

*

*	SUB-ROUTINES	:	- Init_Decod_Tetra()

*					- Decod_Tetra()

*

************************************************************************

*

*	INCLUDED FILES	:	source.h

*

************************************************************************/

#include "source.h"

/*--------------------------------------------------------*

*       Decoder constants parameters.                    *

*                                                        *

*   L_frame     : Frame size.                            *

*   L_subfr     : Sub-frame size.                        *

*   p           : LPC order.                             *

*   pp1         : LPC order+1                            *

*   pit_min     : Minimum pitch lag.                     *

*   pit_max     : Maximum pitch lag.                     *

*   L_inter     : Length of filter for interpolation     *

*   parm_size   : Lenght of vector parm[]                *

*--------------------------------------------------------*/

#define  L_frame  (Word16)240

#define  L_subfr  (Word16)60

#define  p        (Word16)10

#define  pp1      (Word16)11

#define  pit_min  (Word16)20

#define  pit_max  (Word16)143

#define  L_inter  (Word16)15

#define  parm_size (Word16)23

/*--------------------------------------------------------*

*   LPC bandwidth expansion factors for noise filter.    *

*      In Q15 = 0.75, 0.85                               *

*--------------------------------------------------------*/

#define gamma3  (Word16)24576

#define gamma4  (Word16)27853

/*--------------------------------------------------------*

*         Static memory allocation.                      *

*--------------------------------------------------------*/

/* Excitation vector */

static Word16 old_exc[L_frame+pit_max+L_inter];

static Word16 *exc;

/* Spectral expansion factors */

static Word16 F_gamma3[p];

static Word16 F_gamma4[p];

/* Lsp (Line spectral pairs in the cosine domain) */

static Word16 lspold[p]={

30000, 26000, 21000, 15000, 8000, 0,

-8000,-15000,-21000,

-26000};

static Word16 lspnew[p];

/* Initial lsp values used after each time */

/* a reset is executed */

static Word16 lspold_init[p]={

30000, 26000, 21000, 15000, 8000, 0,

-8000,-15000,-21000,-26000};

/* Filter's memory */

static Word16 mem_syn[p];

/* Default parameters */

static Word16 old_parm[parm_size], old_T0;

/* Global definition */

Word16 last_ener_cod;

Word16 last_ener_pit;

/**************************************************************************

*

*	ROUTINE				:	Init_Decod_Tetra

*

*	DESCRIPTION			:	Initialization of variables for the speech decoder

*

**************************************************************************

*

*	USAGE				:	Init_Decod_Tetra()

*

*	INPUT ARGUMENT(S)		:	None

*

*	OUTPUT ARGUMENT(S)		:	None

*

*	RETURNED VALUE		:	None

*

**************************************************************************/

void Init_Decod_Tetra(void)

{

Word16 i;

old_T0 = 60;

for(i=0; i<23; i++)

old_parm[i] = 0;

/* Initialize static pointer */

exc    = old_exc + pit_max + L_inter;

/* Initialize global variables */

last_ener_cod = 0;

last_ener_pit = 0;

/* Static vectors to zero */

for(i=0; i<pit_max + L_inter; i++)

old_exc[i] = 0;

for(i=0; i<p; i++)

mem_syn[i] = 0;

/* Initialisation of lsp values for first */

/* frame lsp interpolation */

for(i=0; i<p; i++)

lspold[i] = lspold_init[i];

/* Compute LPC spectral expansion factors */

Fac_Pond(gamma3, F_gamma3);

Fac_Pond(gamma4, F_gamma4);

return;

}

/**************************************************************************

*

*	ROUTINE				:	Decod_Tetra

*

*	DESCRIPTION			:	Main speech decoder function

*

**************************************************************************

*

*	USAGE				:	Decod_Tetra(parm,synth)

*							(Routine_Name(input1,output1))

*

*	INPUT ARGUMENT(S)		:

*

*		INPUT1			:	- Description : Synthesis parameters

*							- Format : 24 * 16 bit-samples

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : Synthesis

*							- Format : 240 * 16 bit-samples

*

*	RETURNED VALUE		:	None

*

**************************************************************************/

void Decod_Tetra(Word16 parm[], Word16 synth[])

{

/* LPC coefficients */

Word16 A_t[(pp1)*4];		/* A(z) unquantized for the 4 subframes */

Word16 Ap3[pp1];		/* A(z) with spectral expansion         */

Word16 Ap4[pp1];		/* A(z) with spectral expansion         */

Word16 *A;			/* Pointer on A_t                       */

/* Other vectors */

Word16 zero_F[L_subfr+64],  *F;

Word16 code[L_subfr+4];

/* Scalars */

Word16 i, i_subfr;

Word16 T0, T0_min, T0_max, T0_frac;

Word16 gain_pit, gain_code, index;

Word16 sign_code, shift_code;

Word16 bfi, temp;

Word32 L_temp;

/* Initialization of F */

F  = &zero_F[64];

for(i=0; i<64; i++)

zero_F[i] = 0;

/* Test bfi */

bfi = *parm++;

if(bfi == 0)

{

D_Lsp334(&parm[0], lspnew, lspold);	/* lsp decoding   */

for(i=0; i< parm_size; i++)		/* keep parm[] as old_parm */

old_parm[i] = parm[i];

}

else

{

for(i=1; i<p; i++)

lspnew[i] = lspold[i];

for(i=0; i< parm_size; i++)		/* use old parm[] */

parm[i] = old_parm[i];

}

parm += 3;			/* Advance synthesis parameters pointer */

/* Interpolation of LPC for the 4 subframes */

Int_Lpc4(lspold,   lspnew,   A_t);

/* update the LSPs for the next frame */

for(i=0; i<p; i++)

lspold[i]   = lspnew[i];

/*------------------------------------------------------------------------*

*          Loop for every subframe in the analysis frame                 *

*------------------------------------------------------------------------*

* The subframe size is L_subfr and the loop is repeated L_frame/L_subfr  *

*  times                                                                 *

*     - decode the pitch delay                                           *

*     - decode algebraic code                                            *

*     - decode pitch and codebook gains                                  *

*     - find the excitation and compute synthesis speech                 *

*------------------------------------------------------------------------*/

A = A_t;				/* pointer to interpolated LPC parameters */

for (i_subfr = 0; i_subfr < L_frame; i_subfr += L_subfr)

{

index = *parm++;				/* pitch index */

if (i_subfr == 0)				/* if first subframe */

{

if (bfi == 0)

{						/* if bfi == 0 decode pitch */

if (index < 197)

{

/* T0 = (index+2)/3 + 19; T0_frac = index - T0*3 + 58; */

i = add(index, (Word16)2);

i = mult(i, (Word16)10923);	/* 10923 = 1/3 in Q15 */

T0 = add(i, (Word16)19);

i = add(T0, add(T0, T0) );	/* T0*3 */

i = sub((Word16)58, (Word16)i);

T0_frac = add(index, (Word16)i);

}

else

{

T0 = sub(index, (Word16)112);

T0_frac = 0;

}

}

else   /* bfi == 1 */

{

T0 = old_T0;

T0_frac = 0;

}

/* find T0_min and T0_max for other subframes */

T0_min = sub(T0, (Word16)5);

if (T0_min < pit_min) T0_min = pit_min;

T0_max = add(T0_min, (Word16)9);

if (T0_max > pit_max)

{

T0_max = pit_max;

T0_min = sub(T0_max, (Word16)9);

}

}

else  /* other subframes */

{

if (bfi == 0)				/* if bfi == 0 decode pitch */

{

/* T0 = (index+2)/3 - 1 + T0_min; */

i = add(index, (Word16)2);

i = mult(i, (Word16)10923);	/* 10923 = 1/3 in Q15 */

i = sub(i, (Word16)1);

T0 = add(T0_min, i);

/* T0_frac = index - 2 - i*3; */

i = add(i, add(i,i) );		/* i*3 */

T0_frac = sub( index , add(i, (Word16)2) );

}

}

/*-------------------------------------------------*

* - Find the adaptive codebook vector.            *

*-------------------------------------------------*/

Pred_Lt(&exc[i_subfr], T0, T0_frac, L_subfr);

/*-----------------------------------------------------*

* - Compute noise filter F[].                         *

* - Decode codebook sign and index.                   *

* - Find the algebraic codeword.                      *

*-----------------------------------------------------*/

Pond_Ai(A, F_gamma3, Ap3);

Pond_Ai(A, F_gamma4, Ap4);

for (i = 0;   i <= p;      i++) F[i] = Ap3[i];

for (i = pp1; i < L_subfr; i++) F[i] = 0;

Syn_Filt(Ap4, F, F, L_subfr, &F[pp1], (Word16)0);

/* Introduce pitch contribution with fixed gain of 0.8 to F[] */

for (i = T0; i < L_subfr; i++)

{

temp = mult(F[i-T0], (Word16)26216);

F[i] = add(F[i], temp);

}

index = *parm++;

sign_code  = *parm++;

shift_code = *parm++;

D_D4i60(index, sign_code, shift_code, F, code);

/*-------------------------------------------------*

* - Decode pitch and codebook gains.              *

*-------------------------------------------------*/

index = *parm++;        /* index of energy VQ */

Dec_Ener(index,bfi,A,&exc[i_subfr],code, L_subfr, &gain_pit, &gain_code);

/*-------------------------------------------------------*

* - Find the total excitation.                          *

* - Find synthesis speech corresponding to exc[].       *

*-------------------------------------------------------*/

for (i = 0; i < L_subfr;  i++)

{

/* exc[i] = gain_pit*exc[i] + gain_code*code[i]; */

/* exc[i]  in Q0   gain_pit in Q12               */

/* code[i] in Q12  gain_cod in Q0                */

L_temp = L_mult0(exc[i+i_subfr], gain_pit);

L_temp = L_mac0(L_temp, code[i], gain_code);

exc[i+i_subfr] = L_shr_r(L_temp, (Word16)12);

}

Syn_Filt(A, &exc[i_subfr], &synth[i_subfr], L_subfr, mem_syn, (Word16)1);

A  += pp1;    /* interpolated LPC parameters for next subframe */

}

/*--------------------------------------------------*

* Update signal for next frame.                    *

* -> shift to the left by L_frame  exc[]           *

*--------------------------------------------------*/

for(i=0; i<pit_max+L_inter; i++)

old_exc[i] = old_exc[i+L_frame];

old_T0 = T0;

return;

}

## A.3	Library of Source Coder/Decoder Subroutines

/************************************************************************

*

*	FILENAME		:	sub_sc_d.c

*

*	DESCRIPTION		:	Source coder/decoder sub-routines used in the

*					TETRA speech codec

*

************************************************************************

*

*	SUB-ROUTINES	:	- Bits2prm_Tetra()

*					- Cal_Rr2()

*					- Clsp_334()

*					- Dec_Ener()

*					- D4i60_16()

*					- D_D4i60()

*					- D_Lsp334()

*					- Ener_Qua()

*					- G_Code()

*					- G_Pitch()

*					- Inter8_M1_3()

*					- Inter8_1_3()

*					- Inter32_M1_3()

*					- Inter32_1_3()

*					- Lag_Max()

*					- Norm_Corr()

*					- Pitch_Fr()

*					- Pitch_Ol_Dec()

*					- Post_Process()

*					- Pred_Lt()

*					- Pre_Process() (including Init_Pre_Process)

*					- Prm2bits_Tetra()

*

************************************************************************

*

*	INCLUDED FILES	:	source.h

*

*					clsp_334.tab

*					ener_qua.tab

*

************************************************************************/

#include "source.h"

#include "clsp_334.tab"

#include "ener_qua.tab"

/**************************************************************************

*

*	ROUTINE				:	Bits2prm_Tetra

*

*	DESCRIPTION			:	Convert serial received bits to the encoder

*							parameter vector

*

**************************************************************************

*

*	USAGE				:	Bits2prm_Tetra(buffer_in,buffer_out)

*							(Routine_Name(input1,output1))

*

*	INPUT ARGUMENT(S)		:

*

*		INPUT1			:	- Description : Serial bits (137 + bfi)

*							- Format : Word16 - .. * 16 bit-samples

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : Encoded parameters

*								         (23 parameters + bfi)

*							- Format : Word16 - .. * 16 bit-samples

*

*	RETURNED VALUE		:	None

*

*	COMMENTS			:	The received parameters are :

*

*						- BFI	bad frame indicator		1 bit

*

*						- LSP	1st codebook 			8 bits

*							2nd codebook			9 bits

*							3nd codebook			9 bits

*

*						- for the 4 subframes :

*							pitch delay			8 bits (first)

*											5 bits (others)

*							codebook index		14 bits

*							pulse global sign		1 bit

*							pulse shift			1 bit

*							pitch and innovation gains	6 bits

*

**************************************************************************/

#define PRM_NO 23

void Bits2prm_Tetra(Word16 *bits, Word16 prm[])

{

Word16 i;

static Word16 bitno[PRM_NO] = {8, 9, 9,            /* split VQ LSP  */

8, 14, 1, 1, 6,     /* subframe 1    */

5, 14, 1, 1, 6,     /* subframe 2    */

5, 14, 1, 1, 6,     /* subframe 3    */

5, 14, 1, 1, 6};    /* subframe 4    */

*prm++ = *bits++;     /* read BFI */

for (i = 0; i < PRM_NO; i++)

{

prm[i] = bin2int(bitno[i], bits);

bits  += bitno[i];

}

}

/**************************************************************************

*

*	ROUTINE				:	Cal_Rr2

*

*	DESCRIPTION			:	Compute the autocorrelation matrix of

*						impulse response h

*							Only the even elements are stored in the matrix

*

**************************************************************************

*

*	USAGE				:	Cal_Rr2(buffer_1,buffer_2)

*							(Routine_Name(input1,input2))

*

*	INPUT ARGUMENT(S)		:

*

*		INPUT1			:	- Description : Impulse response

*						- Format : Word16

*

*		INPUT2			:	- Description : Autocorrelation matrix

*								          (passed as a vector)

*							- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:	None

*

*	RETURNED VALUE		:	None

*

*	COMMENTS			:	Matrice rr[dim_rr][dim_rr]

*

*							<------- dim_rr ------->

*							<------ L/2 ---->

*

*						^	x x x x x x ... x 0 ...  0	^

*						|	x x x x x x ... x 0 ...  0	|

*						|	x x x x x x ... x 0 ...  0	|

*						|	x x x x x x ... x 0 ...  0	|

*						L/2	x x x x x x ... x 0 ...  0	|

*						|	x x x x x x ... x 0 ...  0	dim_rr

*						|	| | | | | |       | |      |	|

*						v	x x x x x x ... x 0 ...  0	|

*							0 0 0 0 0 0 ... 0 0 ... 0	|

*							| | | | | |     | |     |		|

*							0 0 0 0 0 0 ... 0 0 ... 0	v

*

*						Firstly, rr[L/2-1][L/2-1] is computed, with

*						the main diagonal step up. Then, the same

*						is done for the other diagonals.

*

**************************************************************************/

/* Length of subframe = 60 */

#define L      60

/* Dimension of matrix rr[dim_rr][dim_rr]  = 32 */

#define dim_rr 32

/* Increment on diagonal */

#define diag   33

/* Index of last element to compute = rr[L/2-1][L/2-1] = rr[29*32+29] */

#define last   957

void Cal_Rr2(Word16 h[], Word16 *rr)

{

Word16 i, k, dec;

Word32 s;

Word16 *ph, *phd;

Word16 *rr_fin_c;

Word16 *rr_fin_r;

Word16 *rr_ij;

Word16 *rr_ji;

Word16 hs[L];

/* Scaling for maximum precision */

s = 0;

for(i=0; i<L; i++)

s = L_mac0(s, h[i], h[i]);

k = norm_l(s);

k = shr(sub(k, (Word16)1), (Word16)1);

for(i=0; i<L; i++)

hs[i] = shl(h[i], k);

/* Compute rr[][] */

rr_fin_r = &rr[last];                  /* pointer on rr[L/2-1][L/2-1]*/

rr_fin_c = rr_fin_r;

for (dec = 0 ;dec < L; dec+=2)

{

ph  = hs;                            /* pointer on hs[i]      */

phd = &hs[dec];                      /* pointer on hs[dec+i]  */

rr_ij = rr_fin_c;                    /* end of column j       */

rr_ji = rr_fin_r;                    /* end of row j          */

s = 0;

for (k = 0; k < L-dec; k+=2)

{

s = L_mac(s, *ph++, *phd++);

s = L_mac(s, *ph++, *phd++);

*rr_ij = extract_h(s);

*rr_ji = extract_h(s);

rr_ij -= diag;                     /* decrement pointers */

rr_ji -= diag;

}

rr_fin_c--;                          /* decrement column */

rr_fin_r -= dim_rr;                  /* decrement row    */

}

return;

}

/**************************************************************************

*

*	ROUTINE				:	Clsp_334

*

*	DESCRIPTION			:	Split vector quantization of LSP parameters

*							Use a split-3 VQ in the cosine lsp domain (-1,1),

*							without weighting

*

**************************************************************************

*

*	USAGE				:	Clsp_334(buffer_in,buffer_out1,buffer_out2)

*							(Routine_Name(input1,output1,output2))

*

*	INPUT ARGUMENT(S)		:

*

*		INPUT1			:	- Description : LSPs in the cosine domain (-1,1)

*						- Format : Word16 - Q15

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : Quantized LSPs in the cosine domain

*						- Format : Word16 - Q15

*

*		OUTPUT2			:	- Description : Indices of the 3 selected

*								          codebook entries

*							- Format : Word16

*

*	RETURNED VALUE		:	None

*

*

**************************************************************************/

void Clsp_334(Word16 *lsp, Word16 *lsp_q, Word16 *indice)

{

Word16 i, j, ind, temp;

Word16 *p_dico;

Word32 min, dist;

static Word16 lsp_old[10]={

30000, 26000, 21000, 15000, 8000, 0,

-8000,-15000,-21000,-26000};

/* Search dico1  lsp[0]-lsp[2] */

p_dico = dico1_clsp;

min    = MAX_32;

for (i = 0; i< taille_dic1; i++)

{

temp  = sub(lsp[0], *p_dico++);

dist = L_mult0(temp,temp);

for(j=1; j<3; j++)

{

temp  = sub(lsp[j], *p_dico++);

dist  = L_mac0(dist,temp,temp);

}

if (L_sub(dist, min) < 0) { min = dist; ind = i; }

}

indice[0] = ind;

p_dico    = &dico1_clsp[ind * 3];

lsp_q[0]  = *p_dico++ ;

lsp_q[1]  = *p_dico++ ;

lsp_q[2]  = *p_dico++ ;

/* Search dico2 lsp[3]-lsp[5] */

p_dico = dico2_clsp;

min    = MAX_32;

for (i = 0; i< taille_dic2; i++)

{

temp  = sub(lsp[3], *p_dico++);

dist  = L_mult0(temp,temp);

for(j=4; j<6; j++)

{

temp  = sub(lsp[j], *p_dico++);

dist  = L_mac0(dist,temp,temp);

}

if (L_sub(dist, min) < 0) { min = dist; ind = i; }

}

indice[1] = ind;

p_dico    = &dico2_clsp[ind * 3];

lsp_q[3]  = *p_dico++ ;

lsp_q[4]  = *p_dico++ ;

lsp_q[5]  = *p_dico++ ;

/* Search dico3 lsp[6]-lsp[9] */

p_dico = dico3_clsp;

min    = MAX_32;

for (i = 0; i< taille_dic3; i++)

{

temp  = sub(lsp[6], *p_dico++);

dist  = L_mult0(temp,temp);

for(j=7; j<10; j++)

{

temp  = sub(lsp[j], *p_dico++);

dist  = L_mac0(dist,temp,temp);

}

if (L_sub(dist, min) < 0) { min = dist; ind = i; }

}

indice[2] = ind;

p_dico    = &dico3_clsp[ind * 4];

lsp_q[6]  = *p_dico++ ;

lsp_q[7]  = *p_dico++ ;

lsp_q[8]  = *p_dico++ ;

lsp_q[9]  = *p_dico++ ;

/* Minimum distance between lsp_q[2] and lsp_q[3] */

temp = 917;                    /* 917 = 0.028 in Q15 = 50hz around 1000hz */

temp = sub(temp, lsp_q[2]);

temp = add(temp, lsp_q[3]);    /* temp = 0.028 - (lsp_q[2]-lsp_q[3])      */

if (temp > 0)

{

temp = shr(temp, (Word16)1);

lsp_q[2] = add(lsp_q[2], temp);

lsp_q[3] = sub(lsp_q[3], temp);

}

/* Minimum distance between lsp_q[5] and lsp_q[6] */

temp = 1245;                   /* 1245= 0.038 in Q15 = 50hz around 1600hz */

temp = sub(temp, lsp_q[5]);

temp = add(temp, lsp_q[6]);    /* temp = 0.038 - (lsp_q[5]-lsp_q[6])      */

if (temp > 0)

{

temp = shr(temp, (Word16)1);

lsp_q[5] = add(lsp_q[5], temp);

lsp_q[6] = sub(lsp_q[6], temp);

}

/* Verify if lsp_q[] are still in order */

temp = 0;

for(i=0; i<9; i++)

{

if(sub(lsp_q[i],lsp_q[i+1]) <= 0 )

{

temp = 1;

}

}

/* If lsp_q[] are not in order keep old lsp_q[] */

if(temp != 0)

{

for(i=0; i<10; i++)

lsp_q[i] = lsp_old[i];

}

else

{

for(i=0; i<10; i++)

lsp_old[i] = lsp_q[i];

}

return;

}

/**************************************************************************

*

*	ROUTINE				:	Dec_Ener

*

*	DESCRIPTION			:	Decode gains of pitch and innovative codebook

*

**************************************************************************

*

*	USAGE				:	Dec_Ener(index,bfi,buffer_in1,buffer_in2,buffer_in3,

*							L_subfr,gain_p,gain_c)

*							(Routine_Name(input1,input2,input3,input4,input5,

*							input6,output1,output2))

*

*	INPUT ARGUMENT(S)		:

*

*	INPUT1			:	- Description : Index of energy quantizer

*						- Format : Word16

*

*	INPUT2			:	- Description : Bad frame indicator

*						- Format : Word16

*

*	INPUT3			:	- Description : LPC filter

*						- Format : Word16

*

*	INPUT4			:	- Description : Adaptive codebook

*						- Format : Word16

*

*		INPUT5			:	- Description : Innovation codebook

*							- Format : Word16

*

*		INPUT6			:	- Description : Subframe length

*							- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : Quantized pitch gain

*							- Format : Word16

*

*		OUTPUT2			:	- Description : Quantized code gain

*							- Format : Word16

*

*	RETURNED VALUE		:	Index of energy quantizer

*

**************************************************************************/

extern Word16 last_ener_pit;

extern Word16 last_ener_cod;

Word16 Dec_Ener(Word16 index, Word16 bfi, Word16 A[], Word16 prd_lt[],

Word16 code[], Word16 L_subfr, Word16 *gain_pit, Word16 *gain_cod)

{

Word16  i, j;

Word16  exp, frac;

Word16  exp_lpc, ener_lpc;

Word16  exp_plt, ener_plt, pred_pit;

Word16  ener_c, pred_cod;

Word32  L_tmp;

/*------------------------------------------------------*

* Energy of impulse response of 1/A(z) for length = 60 *

*------------------------------------------------------*/

L_tmp = Lpc_Gain(A);

exp_lpc  = norm_l(L_tmp);

ener_lpc = extract_h( L_shl(L_tmp, exp_lpc) );

/*-------------------------------*

* Energy of adaptive codebook   *

*-------------------------------*/

L_tmp = 1;				/* Avoid case of all-zeros */

for(i=0; i<L_subfr; i++)

L_tmp = L_mac0(L_tmp, prd_lt[i], prd_lt[i]);

exp_plt  = norm_l(L_tmp);

ener_plt = extract_h( L_shl(L_tmp, exp_plt) );

/* ener_plt = Log2(ener_plt * ener_lpc) */

L_tmp = L_mult0(ener_plt, ener_lpc);

exp_plt = add(exp_plt, exp_lpc);

Log2(L_tmp, &exp, &frac);

L_tmp = Load_sh16(exp);

L_tmp = add_sh(L_tmp, frac, (Word16)1);	/* Log2(ener_plt*ener_lpc) in Q16*/

L_tmp = sub_sh16(L_tmp, exp_plt);		/* subtract exponant of ener_plt */

/* Input on 15 bits */

L_tmp = add_sh(L_tmp, (Word16)1710, (Word16)8); /* +6.68 in Q16

for other scaling    */

L_tmp = L_shr(L_tmp, (Word16)8);		/* Convert from Q16 to Q8        */

ener_plt = extract_l(L_tmp);

/*-------------------------------*

* Energy coming from code       *

*-------------------------------*/

L_tmp = 0;

for(i=0; i<L_subfr; i++)

L_tmp = L_mac0(L_tmp, code[i], code[i]);

ener_c = extract_h(L_tmp);

/* ener_c = Log2(ener_c * ener_lpc) */

L_tmp = L_mult0(ener_c, ener_lpc);

Log2(L_tmp, &exp, &frac);

L_tmp = Load_sh16(exp);

L_tmp = add_sh(L_tmp, frac, (Word16)1);	/* Log2(ener_plt*ener_lpc) in Q16*/

L_tmp = sub_sh16(L_tmp, exp_lpc);		/* subtract exponant of ener_lpc */

/* Input on 15 bits */

L_tmp = sub_sh(L_tmp, (Word16)4434, (Word16)8); /*-17.32 in Q16

for other scaling    */

L_tmp = L_shr(L_tmp, (Word16)8);		/* Convert from Q16 to Q8        */

ener_c = extract_l(L_tmp);

/*-----------------------------------------------*

* Test for bfi.                                 *

*                                               *

*  if (bfi != 0)                                *

*    ->last_ener_pit -= 0.5                     *

*    ->last_ener_cod -= 0.5                     *

*  else                                         *

*  {                                            *

*    decode new last_ener_pit et last_ener_cod  *

*  }                                            *

*-----------------------------------------------*/

if(bfi != 0)

{

last_ener_pit = sub(last_ener_pit, (Word16)128);	/* -0.5 in Q8 */

if(last_ener_pit < 0) last_ener_pit = 0;

last_ener_cod = sub(last_ener_cod, (Word16)128);	/* -0.5 in Q8 */

if(last_ener_cod < 0) last_ener_cod = 0;

}

else

{

/*-----------------------------------------------------------------*

* Prediction on pitch energy.                                     *

*  pred_pit = 0.50 * last_ener_pit + 0.25 * last_ener_cod - 3.0   *

*  if(pred_pit < 0.0) pred_pit = 0.0;                             *

*-----------------------------------------------------------------*/

L_tmp = Load_sh(last_ener_pit, (Word16)8);	/* .5 last_ener_pit in Q9  */

L_tmp = add_sh(L_tmp,last_ener_cod, (Word16)7); /*+.25 last_ener_code

in Q9    */

L_tmp = sub_sh(L_tmp, (Word16)768, (Word16)9); /* -3.0 in Q9          */

if(L_tmp < 0) L_tmp = 0;

pred_pit = store_hi(L_tmp, (Word16)7);	/* result in Q8 		   */

/*-----------------------------------------------------------------*

* Prediction on code energy.                                      *

*  pred_cod = 0.50 * last_ener_cod + 0.25 * last_ener_pit - 3.0   *

*  if(pred_cod < 0.0) pred_cod = 0.0;                             *

*-----------------------------------------------------------------*/

L_tmp = Load_sh(last_ener_cod, (Word16)8);	/* .5 last_ener_cod in Q9  */

L_tmp = add_sh(L_tmp, last_ener_pit, (Word16)7); /*+.25 last_ener_pit

in Q9    */

L_tmp = sub_sh(L_tmp, (Word16)768, (Word16)9); /* -3.0 in Q9          */

if(L_tmp < 0) L_tmp = 0;

pred_cod = store_hi(L_tmp, (Word16)7);	/* result in Q8		   */

/*-----------------------------------------------------------------*

* Read quantized values.                                          *

*-----------------------------------------------------------------*/

j = shl(index, (Word16)1);

last_ener_pit = add(t_qua_ener[j],   pred_pit);

last_ener_cod = add(t_qua_ener[j+1], pred_cod);

/* Limit energies ->for transmission errors */

if(sub(last_ener_pit, (Word16)6912)>0)last_ener_pit = 6912;

/* 6912 = 27 in Q8 */

if(sub(last_ener_cod, (Word16)6400)>0)last_ener_cod = 6400;

/* 6400 = 25 in Q8 */

}

/*---------------------------------------------------*

*                                                   *

*  Compute the quantized pitch gain.                *

*                                                   *

*     temp = 0.5 * ( last_ener_pit - ener_plt );    *

*     temp = pow(2.0, temp);                        *

*     if( temp > 1.2) temp = 1.2;                   *

*     *gain_pit = temp;                             *

*                                                   *

*---------------------------------------------------*/

L_tmp = Load_sh(last_ener_pit, (Word16)6);	 /* last_ener_pit/2 in Q15 */

L_tmp = sub_sh(L_tmp, ener_plt, (Word16)6);	 /* - ener_plt/2    in Q15 */

L_tmp = add_sh(L_tmp, (Word16)12, (Word16)15); /* to have gain in Q12    */

L_extract(L_tmp, &exp, &frac);

L_tmp = pow2(exp, frac);

if( L_sub(L_tmp, (Word16)4915) > 0) L_tmp = 4915; /* 4915 = 1.2 in Q12   */

*gain_pit = extract_l(L_tmp);

/*---------------------------------------------------*

*                                                   *

*  Compute the innovative code gain.                *

*                                                   *

*     temp = 0.5 * ( last_ener_cod - ener_c );      *

*     *gain_cod = pow(2.0, temp);                   *

*                                                   *

*---------------------------------------------------*/

L_tmp = Load_sh(last_ener_cod, (Word16)6);	/* last_ener_cod/2 in Q15 */

L_tmp = sub_sh(L_tmp, ener_c, (Word16)6);	/* - ener_c/2      in Q15 */

L_extract(L_tmp, &exp, &frac);

L_tmp = pow2(exp, frac);

*gain_cod = extract_l(L_tmp);

return index;

}

/**************************************************************************

*

*	ROUTINE				:	D4i60_16

*

*	DESCRIPTION			:	Innovation codebook search

*

**************************************************************************

*

*	USAGE				:	D4i60_16(buffer_in1,buffer_in2,buffer_in3,buffer_in4,

*							buffer_out1, buffer_out2,sign,shift)

*							(Routine_Name(input1,input2,input3,input4,

*							output1,output2,output3,output4))

*

*	INPUT ARGUMENT(S)		:

*

*	INPUT1			:	- Description : Result of backward-filtering

*						- Format : Word16

*

*	INPUT2			:	- Description : Impulse response of noise filter

*							          (vector f[])

*						- Format : Word16 - (must be preceeded by 64 zeros)

*

*	INPUT3			:	- Description : Total impulse response (vector h[])

*						- Format : Word16 - (must be preceeded by 64 zeros)

*

*	INPUT4			:	- Description : Autocorrelations of h[]

*						- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : Innovation code found

*							  by convolving with f[]

*							- Format : Word16

*

*		OUTPUT2			:	- Description : Innovation code found

*							  by convolving with h[]

*							- Format : Word16

*

*		OUTPUT3			:	- Description : Innovation code gain sign

*							- Format : Word16

*

*		OUTPUT4			:	- Description : Shift of innovation code

*							- Format : Word16

*

*	RETURNED VALUE		:	Index of innovation code

*

**************************************************************************

**************************************************************************

*

*	COMMENTS			:	The code length is 60, containing 4 nonzero pulses

*

*						i0, i1, i2, i3, with fixed signs.

*						i0 can take 30 positions.

*						i1, i2 and i3 can have 8 possible positions :

*

*						i0 (+)	: 0,  2, 4.  .... 58

*						i1 (-)	: 2, 10, 18, 26, 34, 42, 50, 58

*						i2 (+)	: 4, 12, 20, 28, 36, 44, 52, (60)

*						i3 (-)	: 6, 14, 22, 30, 38, 46, 54, (62)

*

*						Positions 60 and 62 correspond, respectively,

*						to pulses i2 and i3 not used.

*						The code can be shifted 1 position to the right.

*

*	NOTES ON COMPLEXITY		:

*

*	1 :	In this routine all accesses to matrix rr[][] use indices. This was done for clarity and

*		readability of the program. In the actual real-time implementation all these

*		accesses use pointers.  Parallel autoincrement of pointers on dn[] and rr[][]

*		should be used, especially for the two most inner loops in the search which are very

*		critical.

*

*		As the number of addressing registers and the possible modes of autoincrement

*		can vary with each DSP, the addressing of matrix rr[][] should be optimized

*		for the DSP used.

*

*	2 :	Because of the thresholds it is difficult to estimate the average complexity.

*		Usually, we mesure it with the real-time implementation. The worst

*		case corresponds to "time <= 0".  The decrements of 3 and 4 on variable

*		"time" come from the relative number of operations when "seuil2" is exceeded and

*		the number of operations when "seuil1" is exceeded, excluding the operations when

*		"seuil2" is exceeded. We supose that a new maximum is found 1 out of 8 new tries.

*		The numbers 3 and 4 correspond to the real-time implementation and not to this

*		C code.

*

**************************************************************************/

#define lcode 60

/* Gain for impulse 0 (i0) = sqrt(2) = 1.4142                             */

/* We use different values to be compatible with the implementation       */

/* which uses 3 different Qx formats for this number to simplify scaling. */

#define Q11_gain_i0  2896

#define Q13_gain_i0 11585

#define Q14_gain_i0 23170

/*------------------------------------------------------------------*

* The next 3 parameters control the time taken by the innovative   *

* codebook search.                                                 *

* The first two parameters control if a section of the innovative  *

* codebook should be search or not. The last parameter controls    *

* the absolute maximum time used by the innovative codebook search.*

*------------------------------------------------------------------*/

/* thresholds = 0.586 in Q15 */

#define threshold1 19200

#define threshold2 19200

#define max_time   350

Word16 D4i60_16(Word16 dn[], Word16 f[], Word16 h[], Word16 rr[][32],

Word16 cod[], Word16 y[], Word16 *sign, Word16 *shift_code)

{

Word16 ip0, ip1, ip2, ip3;

Word16 ii0, ii1, ii2, ii3;

Word16 *p0, *p1, *p2, *p3;

Word16 i,j;

Word16 shif, shift;

Word16 time, index;

Word16 ps0,  ps1,  ps2,  ps3;

Word16 ps0a, ps1a, ps2a;

Word16 ps, ps3c, psc;

Word16 alpha, alp1, alp2, alp3;

Word16 seuil1,seuil2;

Word16 max0, max1, max2, min0, min1, min2;

Word32 L_tmp;

Word32 alp0_32, alp1_32, alp2_32, ps2_32;

/* set dn[60] - dn[63] to zero */

dn[60]=dn[61]=dn[62]=dn[63] = 0;

/* Find min. and max. of dn[] for positions of i0, i1, i2 */

min0=max0=0;

for(i = 0; i<60; i += 2)

{

j = add(dn[i], dn[i+1]);

if      (sub(j,max0) > 0) max0 = j;

else if (sub(j,min0) < 0) min0 = j;

}

max0 = shr(max0, (Word16)1);

min0 = shr(min0, (Word16)1);

/* Multiply max0 and min0 by gain_i0 in Q11 */

max0 = store_hi(L_mult0(max0, (Word16)Q11_gain_i0), (Word16)5);

min0 = store_hi(L_mult0(min0, (Word16)Q11_gain_i0), (Word16)5);

min1=max1=0;

for(i = 2; i<60; i += 8)

{

j = add(dn[i], dn[i+1]);

if      (sub(j,max1) > 0) max1 = j;

else if (sub(j,min1) < 0) min1 = j;

}

max1 = shr(max1, (Word16)1);

min1 = shr(min1, (Word16)1);

min2=max2=0;

for(i = 4; i<60; i += 8)

{

j = add(dn[i], dn[i+1]);

if      (sub(j,max2) > 0) max2 = j;

else if (sub(j,min2) < 0) min2 = j;

}

max2 = shr(max2, (Word16)1);

min2 = shr(min2, (Word16)1);

/*----------------------------------------------------------*

* Find absolute maximum for combination dn[i0] and dn[i1]  *

*      and for combination dn[i0], dn[i1] and dn[i2]       *

*   max1 = max ( max0-min1 , max1-min0 )                   *

*   max2 = max ( max0-min1+max2 , max1-min0-min2 )         *

*----------------------------------------------------------*/

max0 = sub(max0, min1);

max2 = add(max0, max2);

max1 = sub(max1, min0);

j    = sub(max1, min2);

if(sub(max0,max1) > 0) max1 = max0;

if(sub(j,max2)    > 0) max2 = j;

/* Set thresholds */

seuil1 = mult(max1, (Word16)threshold1);

seuil2 = mult(max2, (Word16)threshold2);

/* Default values */

ip0    = 0;

ip1    = 2;

ip2    = 4;

ip3    = 6;

shift  = 0;

ps     = 0;

psc    = 0;

alpha  = 255;

time   = max_time;

/* Four loops to search innovation code. */

for (ii0=0; ii0<30; ii0++)

{

ps0  = store_hi(L_mult0((Word16)Q11_gain_i0, dn[2*ii0]), (Word16)5);

ps0a = store_hi(L_mult0((Word16)Q11_gain_i0, dn[2*ii0+1]), (Word16)5);

alp0_32 = Load_sh(rr[ii0][ii0], (Word16)14);

for (ii1=1; ii1<30; ii1+=4)

{

ps1  = sub(ps0, dn[2*ii1]);

ps1a = sub(ps0a, dn[2*ii1+1]);

/* alp1 = alp0*2 + rr[ii1][ii1] - 2.0*gain_i0*rr[ii0][ii1]  */

/* The result is divided by 8 to avoid overflow.            */

L_tmp = add_sh(alp0_32, rr[ii1][ii1], (Word16)13);

L_tmp = L_msu0(L_tmp, (Word16)Q14_gain_i0, rr[ii0][ii1]);

alp1  = extract_h(L_tmp);

alp1_32 = Load_sh(alp1, (Word16)15);

/* If( abs( (ps1+ps1a)/2 ) > seuil1 */

L_tmp = Load_sh(ps1, (Word16)15);

L_tmp = L_abs( add_sh(L_tmp, ps1a, (Word16)15) );

L_tmp = sub_sh16( L_tmp, seuil1);

if( L_tmp > 0)

{

for (ii2=2; ii2<31; ii2+=4)

{

ps2  = add(ps1, dn[2*ii2]);

ps2a = add(ps1a, dn[2*ii2+1]);

/* alp2=alp1+rr[ii2][ii2]+2.0*(gain_i0*rr[ii0][ii2]-rr[ii1][ii2]) */

/* The result is divided by 16 to avoid overflow.                 */

L_tmp = add_sh(alp1_32, rr[ii2][ii2], (Word16)12);

L_tmp = L_mac0(L_tmp, (Word16)Q13_gain_i0, rr[ii0][ii2]);

L_tmp = sub_sh(L_tmp, rr[ii1][ii2] , (Word16)13);

alp2  = extract_h(L_tmp);

alp2_32 = Load_sh16(alp2);

/* If( abs( (ps2+ps2a)/2 ) > seuil2 */

L_tmp = Load_sh(ps2, (Word16)15);

L_tmp = L_abs( add_sh(L_tmp, ps2a, (Word16)15) );

L_tmp = sub_sh16( L_tmp, seuil2);

if( L_tmp > 0)

{

shif = 0;

if( sub(abs_s(ps2a),abs_s(ps2)) > 0)

{

ps2  = ps2a;

shif = 1;

}

ps2_32 = Load_sh(ps2, (Word16)15);

for (ii3=3; ii3<32; ii3+=4)

{

/* ps3 = (ps2-dn[2*ii3+shift]) / 2                       */

/* Need to work on 32 bits to avoid possible overflow */

L_tmp = sub_sh(ps2_32, dn[2*ii3+shif], (Word16)15);

ps3  = extract_h(L_tmp);

/* alp3 = alp2 + rr[ii3][ii3] +                               */

/*   2.0*(rr[ii1][ii3] - gain_i0*rr[ii0][ii3] - rr[ii2][ii3]) */

/*                                                            */

/* This part is the most critical part in all the coder       */

L_tmp = add_sh(alp2_32, rr[ii3][ii3], (Word16)12);

L_tmp = add_sh(L_tmp, rr[ii1][ii3], (Word16)13);

L_tmp = L_msu0(L_tmp, (Word16)Q13_gain_i0, rr[ii0][ii3] );

L_tmp = sub_sh(L_tmp, rr[ii2][ii3], (Word16)13);

alp3  = extract_h(L_tmp);

/* if( (ps3**2 * alpha) - (psc *alp3)  > 0 ) */

/*    ->new maximum                          */

ps3c = mult(ps3, ps3);

L_tmp  = L_mult(ps3c, alpha);

if( L_msu(L_tmp,psc,alp3) > 0)

{

ps    = ps3;

psc   = ps3c;

alpha = alp3;

ip0 = 2*ii0;

ip1 = 2*ii1;

ip2 = 2*ii2;

ip3 = 2*ii3;

shift = shif;

} /*  end of if(.. > 0) */

} /* end of for ii3 = */

time = sub(time, (Word16)3);	/* See note on complexity */

if(time <= 0 ) goto fin;		/* Maximum time finish    */

} /* end if >seuil2 */

} /* end of for ii2 = */

time = sub(time, (Word16)4);		/* See note on complexity */

if(time <= 0 ) goto fin;		/* Maximum time finish    */

} /* end if >seuil1 */

} /* end of for ii1 = */

} /* end of for ii0 = */

fin:

/* Convolve code with f[] */

f -= shift;            /* Operations on pointers !!! */

p0 = f - ip0;

p1 = f - ip1;

p2 = f - ip2;

p3 = f - ip3;

/* cod[i] = p0[i] * gain_i0  - p1[i] + p2[i] - p3[i]; */

if(ps >= 0)

{

*sign = 0;

for (i = 0; i < lcode; i++)

{

L_tmp  = L_mult0(p0[i], (Word16)Q11_gain_i0);

L_tmp  = sub_sh(L_tmp, p1[i], (Word16)11);

L_tmp  = add_sh(L_tmp, p2[i], (Word16)11);

L_tmp  = sub_sh(L_tmp, p3[i], (Word16)11);

cod[i] = store_hi(L_tmp, (Word16)5);

}

}

else

{

*sign = 1;

for (i = 0; i < lcode; i++)

{

L_tmp  = L_mult0(p0[i], (Word16)Q11_gain_i0);

L_tmp  = sub_sh(L_tmp, p1[i], (Word16)11);

L_tmp  = add_sh(L_tmp, p2[i], (Word16)11);

L_tmp  = sub_sh(L_tmp, p3[i], (Word16)11);

L_tmp  = L_negate(L_tmp);

cod[i] = store_hi(L_tmp, (Word16)5);

}

}

/* Convolve code with h[] */

h -= shift;            /* Operations on pointers !!! */

p0 = h - ip0;

p1 = h - ip1;

p2 = h - ip2;

p3 = h - ip3;

/* y[i] = p0[i] * gain_i0  - p1[i] + p2[i] - p3[i]; */

if(ps >= 0)

{

for (i = 0; i < lcode; i++)

{

L_tmp = L_mult0(p0[i], (Word16)Q11_gain_i0);

L_tmp = sub_sh(L_tmp, p1[i], (Word16)11);

L_tmp = add_sh(L_tmp, p2[i], (Word16)11);

L_tmp = sub_sh(L_tmp, p3[i], (Word16)11);

y[i]  = store_hi(L_tmp, (Word16)5);

}

}

else

{

for (i = 0; i < lcode; i++)

{

L_tmp = L_mult0(p0[i], (Word16)Q11_gain_i0);

L_tmp = sub_sh(L_tmp, p1[i], (Word16)11);

L_tmp = add_sh(L_tmp, p2[i], (Word16)11);

L_tmp = sub_sh(L_tmp, p3[i], (Word16)11);

L_tmp = L_negate(L_tmp);

y[i]  = store_hi(L_tmp, (Word16)5);

}

}

/* Return parameters */

*shift_code = shift;

/* index = (ip0>>1) + ( (ip1>>3)<<5 ) + ( (ip2>>3)<<8 ) + ( (ip3>>3)<<11 )*/

index = shr(ip0, (Word16)1);

index = add(index, shl( shr( ip1,(Word16)3 ),(Word16)5 ));

index = add(index, shl( shr( ip2,(Word16)3 ),(Word16)8 ));

index = add(index, shl( shr( ip3,(Word16)3 ),(Word16)11 ));

return index;

}

/**************************************************************************

*

*	ROUTINE				:	D_D4i60

*

*	DESCRIPTION			:	Decode innovative codebook d4i60_16

*

**************************************************************************

*

*	USAGE				:	D_D4i60(index,sign, shift,buffer_in,buffer_out)

*							(Routine_Name(input1,input2,input3,input4,output1))

*

*	INPUT ARGUMENT(S)		:

*

*	INPUT1			:	- Description : Index of codebook

*						- Format : Word16

*

*	INPUT2			:	- Description : Sign of codebook

*						- Format : Word16

*

*	INPUT3			:	- Description : Shift of codebook

*						- Format : Word16

*

*	INPUT4			:	- Description : Noise filter

*						- Format : Word16 - (must be preceeded by 64 zeros)

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : Innovative vector

*							- Format : Word16

*

*	RETURNED VALUE		:	None

*

*

**************************************************************************/

#define lcode 60

/* Gain for impulse 0 = sqrt(2) = 1.4142 = 2896 in Q11 */

#define Q11_gain_i0  2896

void D_D4i60(Word16 index, Word16 sign, Word16 shift, Word16 F[], Word16 cod[])

{

Word16 i, pos0, pos1, pos2, pos3;

Word16 *p0, *p1, *p2, *p3;

Word32 L_tmp;

/* Position of the 4 impulses */

pos0 = shl( (Word16)(index & 31), (Word16)1);

pos1 = shr( (Word16)(index & 224), (Word16)2);

pos1 = add(pos1, (Word16)2);

pos2 = shr( (Word16)(index & 1792), (Word16)5);

pos2 = add(pos2, (Word16)4);

pos3 = shr( (Word16)(index & 14336), (Word16)8);

pos3 = add(pos3, (Word16)6);

/* Convolve code with F[]                             */

/* cod[i] = p0[i] * gain_i0  - p1[i] + p2[i] - p3[i]; */

F -= shift;            /* Operations on pointers !!!  */

p0 = F - pos0;

p1 = F - pos1;

p2 = F - pos2;

p3 = F - pos3;

if(sign == 0)

{

for (i = 0; i < lcode; i++)

{

L_tmp  = L_mult0(p0[i], (Word16)Q11_gain_i0);

L_tmp  = sub_sh(L_tmp, p1[i], (Word16)11);

L_tmp  = add_sh(L_tmp, p2[i], (Word16)11);

L_tmp  = sub_sh(L_tmp, p3[i], (Word16)11);

cod[i] = store_hi(L_tmp, (Word16)5);

}

}

else

{

for (i = 0; i < lcode; i++)

{

L_tmp  = L_mult0(p0[i], (Word16)Q11_gain_i0);

L_tmp  = sub_sh(L_tmp, p1[i], (Word16)11);

L_tmp  = add_sh(L_tmp, p2[i], (Word16)11);

L_tmp  = sub_sh(L_tmp, p3[i], (Word16)11);

L_tmp  = L_negate(L_tmp);

cod[i] = store_hi(L_tmp, (Word16)5);

}

}

return;

}

/**************************************************************************

*

*	ROUTINE				:	D_Lsp334

*

*	DESCRIPTION			:	Decoding: Split vector quantization of

*							LSP parameters

*

**************************************************************************

*

*	USAGE				:	D_Lsp334(buffer_in1,buffer_out1,buffer_in2)

*							(Routine_Name(input1,output1,input2))

*

*	INPUT ARGUMENT(S)		:

*

*	INPUT1			:	- Description : indices of the three selected

*							          codebook entries

*						- Format : Word16

*

*		INPUT2			:	- Description : Previous LSP values

*								          (in the cosine domain)

*							- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : Quantized LSPs (in the cosine domain)

*							- Format : Word16

*

*	RETURNED VALUE		:	None

*

*

**************************************************************************/

void D_Lsp334(Word16 indice[], Word16 lsp[], Word16 lsp_old[])

{

Word16 i, *p, temp;

p      = &dico1_clsp[ indice[0] * 3];

lsp[0] = *p++ ;

lsp[1] = *p++ ;

lsp[2] = *p++ ;

p      = &dico2_clsp[ indice[1] * 3];

lsp[3] = *p++ ;

lsp[4] = *p++ ;

lsp[5] = *p++ ;

p      = &dico3_clsp[ indice[2] * 4];

lsp[6] = *p++ ;

lsp[7] = *p++ ;

lsp[8] = *p++ ;

lsp[9] = *p++ ;

/* Minimum distance between lsp[2] and lsp[3] */

temp = 917;			/* 917 = 0.028 in Q15 = 50hz around 1000hz */

temp = sub(temp, lsp[2]);

temp = add(temp, lsp[3]);	/* temp = 0.028 - (lsp[2]-lsp[3])      */

if (temp > 0)

{

temp = shr(temp, (Word16)1);

lsp[2] = add(lsp[2], temp);

lsp[3] = sub(lsp[3], temp);

}

/* Minimum distance between lsp[5] and lsp[6] */

temp = 1245;			/* 1245= 0.038 in Q15 = 50hz around 1600hz */

temp = sub(temp, lsp[5]);

temp = add(temp, lsp[6]);	/* temp = 0.038 - (lsp[5]-lsp[6])      */

if (temp > 0)

{

temp = shr( temp,(Word16)1 );

lsp[5] = add(lsp[5], temp);

lsp[6] = sub(lsp[6], temp);

}

/* Verify if lsp[] are still in order */

temp = 0;

for(i=0; i<9; i++)

{

if(sub(lsp[i],lsp[i+1]) <= 0 )

{

temp = 1;

}

}

/* If lsp[] are not in order keep old lsp[] */

if(temp != 0)

{

for(i=0; i<10; i++)

lsp[i] = lsp_old[i];

}

return;

}

/**************************************************************************

*

*	ROUTINE				:	Ener_Qua

*

*	DESCRIPTION			:	Compute quantized gains of pitch and innovative

*						 	codebook resulting from the quantization of energies

*							of adaptive and innovation codebook

*

**************************************************************************

*

*	USAGE				:	Ener_Qua(buffer_in1,buffer_in2,buffer_in3,L_subfr,

*							gain_p,gain_c)

*							(Routine_Name(input1,input2,input3,input4,

*							arg5,arg6))

*

*	INPUT ARGUMENT(S)		:

*

*	INPUT1			:	- Description : LPC filter

*						- Format : Word16

*

*	INPUT2			:	- Description : Adaptive codebook

*						- Format : Word16

*

*	INPUT3			:	- Description : Innovation codebook

*						- Format : Word16

*

*	INPUT4			:	- Description : Subframe length

*						- Format : Word16

*

*		ARG5				:	- Description : Pitch gain

*							- Format : Word16

*

*		ARG6				:	- Description : Code gain

*							- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:

*

*		ARG5				:	- Description : Quantized pitch gain

*							- Format : Word16

*

*		ARG6				:	- Description : Quantized code gain

*							- Format : Word16

*

*	RETURNED VALUE		:	Index of quantization

*

**************************************************************************/

extern Word16 last_ener_pit;

extern Word16 last_ener_cod;

Word16 Ener_Qua(Word16 A[], Word16 prd_lt[], Word16 code[], Word16 L_subfr,

Word16 *gain_pit, Word16 *gain_cod)

{

Word16  i, j, index, tmp, *p;

Word16  exp, frac;

Word16  exp_lpc, ener_lpc;

Word16  exp_plt, ener_plt, ener_pit, err_pit, pred_pit;

Word16  ener_c, ener_cod, err_cod, pred_cod;

Word32  L_tmp, dist, dist_min;

/*------------------------------------------------------*

* Energy of impulse response of 1/A(z) for length = 60 *

*------------------------------------------------------*/

L_tmp = Lpc_Gain(A);

exp_lpc  = norm_l(L_tmp);

ener_lpc = extract_h( L_shl(L_tmp, exp_lpc) );

/*-------------------------------*

* Energy coming from pitch      *

*-------------------------------*/

L_tmp = 1;            /* Avoid case of all-zeros */

for(i=0; i<L_subfr; i++)

L_tmp = L_mac0(L_tmp, prd_lt[i], prd_lt[i]);

exp_plt  = norm_l(L_tmp);

ener_plt = extract_h( L_shl(L_tmp, exp_plt) );

/* ener_plt = Log2(ener_plt * ener_lpc) */

L_tmp = L_mult0(ener_plt, ener_lpc);

exp_plt = add(exp_plt, exp_lpc);

Log2(L_tmp, &exp, &frac);

L_tmp = Load_sh16(exp);

L_tmp = add_sh(L_tmp, frac, (Word16)1);	/*Log2(ener_plt*ener_lpc) in Q16 */

L_tmp = sub_sh16(L_tmp, exp_plt);		/* subtract exponant of ener_plt */

/* Input on 15 bits */

L_tmp = add_sh(L_tmp, (Word16)1710, (Word16)8);	/* +6.68 in Q16

for other scaling    */

/* The scaling factor +6.68 includes many scalings, due to the fact that

impulse response of the LP filter is in Q10 (see routine Lpc_Gain), and

that the original quantization gain table was designed using -26 dB data

reduced to 13 bits while the final coder works with -22dB signal reduced

to 15 bits */

L_tmp = L_shr(L_tmp, (Word16)8);		/* Convert from Q16 to Q8        */

ener_plt = extract_l(L_tmp);

/* ener_pit = Log2(gain_pit**2) + ener_plt */

L_tmp = 1;					/* Avoid Log2(0) 			   */

L_tmp = L_mac0(L_tmp, *gain_pit, *gain_pit);

Log2(L_tmp, &exp, &frac);

L_tmp = Load_sh16(exp);

L_tmp = add_sh(L_tmp, frac, (Word16)1);	/* Log2(gain_pit**2)    in Q16   */

L_tmp = sub_sh16(L_tmp, (Word16)24);	/* -24.0 -> gain_pit**2 is in Q24*/

/* Pitch gain (variable "gain_pit")is in Q12, so its square is in Q24,

which means that the real value is multplied by 2**24. Thus, in applying

the function Log2(), a value of 24 is being added. A scaling factor of

-24 is then needed to compensate  */

L_tmp = L_shr(L_tmp, (Word16)8);		/* Convert from Q16 to Q8        */

ener_pit = extract_l(L_tmp);

ener_pit = add(ener_pit, ener_plt);

/*-------------------------------*

* Energy coming from code       *

*-------------------------------*/

L_tmp = 0;

for(i=0; i<L_subfr; i++)

L_tmp = L_mac0(L_tmp, code[i], code[i]);

ener_c = extract_h(L_tmp);

/* ener_c = Log2(ener_c * ener_lpc) */

L_tmp = L_mult0(ener_c, ener_lpc);

Log2(L_tmp, &exp, &frac);

L_tmp = Load_sh16(exp);

L_tmp = add_sh(L_tmp, frac, (Word16)1);	/* Log2(ener_plt*ener_lpc) in Q16*/

L_tmp = sub_sh16(L_tmp, exp_lpc);		/* subtract exponant of ener_lpc */

/* Input on 15 bits */

L_tmp = sub_sh(L_tmp, (Word16)4434, (Word16)8);	/*-17.32 in Q16

for other scaling    */

/* The vector "code[i]" is in Q12, so its square (variable "ener_c") is in

Q24, which means that the real value is multplied by 2**24. Thus, in

applying the function Log2(), a value of 24 is being added. A scaling

factor of -24 is then needed to compensate, but as the other scaling

factor of +6.68 (as explained above) has also to be applied, the final

resulting factor is -17.32 */

L_tmp = L_shr(L_tmp, (Word16)8);		/* Convert from Q16 to Q8        */

ener_c = extract_l(L_tmp);

/* ener_cod = Log2(gain_code**2) + ener_c */

L_tmp = 1;					/* Avoid Log2(0) 			   */

L_tmp = L_mac0(L_tmp, *gain_cod, *gain_cod);

Log2(L_tmp, &exp, &frac);

L_tmp = Load_sh16(exp);

L_tmp = add_sh(L_tmp, frac, (Word16)1);	/* Log2(gain_code**2) in Q16	   */

L_tmp = L_shr(L_tmp, (Word16)8);		/* Convert from Q16 to Q8        */

ener_cod = extract_l(L_tmp);

ener_cod = add(ener_cod, ener_c);

/*-----------------------------------------------------------------*

* Prediction on pitch energy and prediction error                 *

*  pred_pit = 0.50 * last_ener_pit + 0.25 * last_ener_cod - 3.0   *

*  if(pred_pit < 0.0) pred_pit = 0.0;                             *

*  err_pit = ener_pit - pred_pit;                                 *

*-----------------------------------------------------------------*/

L_tmp = Load_sh(last_ener_pit, (Word16)8); /* .5 last_ener_pit in Q9     */

L_tmp = add_sh(L_tmp, last_ener_cod, (Word16)7); /*+.25 last_ener_code

in Q9    */

L_tmp = sub_sh(L_tmp, (Word16)768, (Word16)9); /* -3.0 in Q9             */

if(L_tmp < 0) L_tmp = 0;

pred_pit = store_hi(L_tmp, (Word16)7);	/* result in Q8 			   */

err_pit  = sub(ener_pit, pred_pit);

/*-----------------------------------------------------------------*

* Prediction on code energy and prediction error                  *

*  pred_cod = 0.50 * last_ener_cod + 0.25 * last_ener_pit - 3.0   *

*  if(pred_cod < 0.0) pred_cod = 0.0;                             *

*  err_cod = ener_cod - pred_cod;                                 *

*-----------------------------------------------------------------*/

L_tmp = Load_sh(last_ener_cod, (Word16)8); /* .5 last_ener_cod  in Q9    */

L_tmp = add_sh(L_tmp, last_ener_pit, (Word16)7); /*+.25 last_ener_pit

in Q9    */

L_tmp = sub_sh(L_tmp, (Word16)768, (Word16)9); /* -3.0 in Q9             */

if(L_tmp < 0) L_tmp = 0;

pred_cod = store_hi(L_tmp, (Word16)7);	/* result in Q8 			   */

err_cod  = sub(ener_cod, pred_cod);

/*-----------------------------------------------------------------*

* Codebook search.                                                *

* Find vector which minimizes:                                    *

*                                                                 *

*   Min k :  (err_pit - dico[0,k])**2 +  (err_cod - dico[1,k])**2 *

*                                                                 *

*-----------------------------------------------------------------*/

dist_min = MAX_32;

p = t_qua_ener;

for (i = 0; i< nb_qua_ener; i++)

{

tmp  = sub(*p++, err_pit);

dist = L_mult0(tmp, tmp);

tmp  = sub(*p++, err_cod);

dist = L_mac0(dist, tmp, tmp);

if (L_sub(dist, dist_min) < 0) { dist_min = dist; index = i; }

}

j = shl(index, (Word16)1);

last_ener_pit = add(t_qua_ener[j],   pred_pit);

last_ener_cod = add(t_qua_ener[j+1], pred_cod);

/* Limit energies ->for transmission errors */

if(sub(last_ener_pit, (Word16)6912) > 0) last_ener_pit = 6912;

/* 6912 = 27 in Q8 */

if(sub(last_ener_cod, (Word16)6400) > 0) last_ener_cod = 6400;

/* 6400 = 25 in Q8 */

/*---------------------------------------------------*

*                                                   *

*  Compute the quantized pitch gain.                *

*                                                   *

*     temp = 0.5 * ( last_ener_pit - ener_plt );    *

*     temp = pow(2.0, temp);                        *

*     if( temp > 1.2) temp = 1.2;                   *

*     *gain_pit = temp;                             *

*                                                   *

*---------------------------------------------------*/

L_tmp = Load_sh(last_ener_pit, (Word16)6);	/* last_ener_pit/2 in Q15 */

L_tmp = sub_sh(L_tmp, ener_plt, (Word16)6);	/* - ener_plt/2    in Q15 */

L_tmp = add_sh(L_tmp, (Word16)12, (Word16)15); /* to have gain in Q12   */

L_extract(L_tmp, &exp, &frac);

L_tmp = pow2(exp, frac);

if( L_sub(L_tmp, (Word32)4915) > 0) L_tmp = 4915; /* 4915 = 1.2 in Q12  */

*gain_pit = extract_l(L_tmp);

/*---------------------------------------------------*

*                                                   *

*  Compute the innovative code gain.                *

*                                                   *

*     temp = 0.5 * ( last_ener_cod - ener_c );      *

*     *gain_cod = pow(2.0, temp);                   *

*                                                   *

*---------------------------------------------------*/

L_tmp = Load_sh(last_ener_cod, (Word16)6);	/* last_ener_cod/2 in Q15 */

L_tmp = sub_sh(L_tmp, ener_c, (Word16)6);	/* - ener_c/2      in Q15 */

L_extract(L_tmp, &exp, &frac);

L_tmp = pow2(exp, frac);

*gain_cod = extract_l(L_tmp);

return index;

}

/**************************************************************************

*

*	ROUTINE				:	G_Code

*

*	DESCRIPTION			:	Compute the gain of innovative code

*

**************************************************************************

*

*	USAGE				:	G_Code(buffer_in1,buffer_in2,L_subfr)

*							(Routine_Name(input1,input2,input3))

*

*	INPUT ARGUMENT(S)		:

*

*	INPUT1			:	- Description : Code target

*						- Format : Word16

*

*		INPUT2			:	- Description : Filtered innovation code (with sign)

*							- Format : Word16

*

*	INPUT3			:	- Description : Length of subframe

*						- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:	None

*

*	RETURNED VALUE		:	Gain of innovation code

*

**************************************************************************/

Word16 G_Code(Word16 xn2[], Word16 y2[], Word16 L_subfr)

{

Word16 i;

Word16 xy, yy, exp_xy, exp_yy, gain;

Word32 s, L_tmp;

Overflow = 0;

/* Compute scalar product <xn2[],y2[]> */

s = 1;					/* Avoid case of all zeros */

for(i=0; i<L_subfr; i++)

s = L_mac0(s, xn2[i], y2[i]);

exp_xy = norm_l(s);

xy     = extract_h( L_shl(s, exp_xy) );

/* Compute scalar product <y2[],y2[]> */

s = 1;					/* Avoid case of all zeros */

for(i=0; i<L_subfr; i++)

s = L_mac0(s, y2[i], y2[i]);

exp_yy = norm_l(s);

yy     = extract_h( L_shl(s, exp_yy) );

/* Test if Overflow */

if(Overflow == 1)

{

/* Compute scalar product <xn2[],y2[]> */

s = 1;				/* Avoid case of all zeros */

for(i=0; i<L_subfr; i++)

{

L_tmp = L_mult0(xn2[i], y2[i]);

L_tmp = L_shr(L_tmp, (Word16)6);

s = L_add(s, L_tmp);

}

exp_xy = norm_l(s);

xy     = extract_h( L_shl(s, exp_xy) );

/* Compute scalar product <y2[],y2[]> */

s = 1;				/* Avoid case of all zeros */

for(i=0; i<L_subfr; i++)

{

L_tmp = L_mult0(y2[i], y2[i]);

L_tmp = L_shr(L_tmp, (Word16)6);

s = L_add(s, L_tmp);

}

exp_yy = norm_l(s);

yy     = extract_h( L_shl(s, exp_yy) );

}

/* If (xy < 0) gain = 0  */

if( xy <= 0) return( (Word16) 0);

/* compute gain = xy/yy */

xy = shr(xy, (Word16)1);			/* Be sure xy < yy */

gain = div_s( xy, yy);

i = add(exp_xy, (Word16)2);		/* Denormalization of division */

i = sub(i, exp_yy);

gain = shr(gain, i);

return(gain);

}

/**************************************************************************

*

*	ROUTINE				:	G_Pitch

*

*	DESCRIPTION			:	Compute the gain of pitch. Result in Q12

*							if (gain < 0)  gain =0

*								if (gain >1.2) gain =1.2

*

**************************************************************************

*

*	USAGE				:	G_Pitch(buffer_in1,buffer_in2,L_subfr)

*							(Routine_Name(input1,input2,input3))

*

*	INPUT ARGUMENT(S)		:

*

*	INPUT1			:	- Description : Pitch target

*						- Format : Word16

*

*		INPUT2			:	- Description : Filtered adaptive codebook

*							- Format : Word16

*

*	INPUT3			:	- Description : Length of subframe

*						- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:	None

*

*	RETURNED VALUE		:	Gain of pitch lag in Q12 saturated to 1.2

*

**************************************************************************/

Word16 G_Pitch(Word16 xn[], Word16 y1[], Word16 L_subfr)

{

Word16 i;

Word16 xy, yy, exp_xy, exp_yy, gain;

Word32 s, L_tmp;

Overflow = 0;

/* Compute scalar product <xn[],y1[]> */

s = 1;					/* Avoid case of all zeros */

for(i=0; i<L_subfr; i++)

s = L_mac0(s, xn[i], y1[i]);

exp_xy = norm_l(s);

xy     = extract_h( L_shl(s, exp_xy) );

/* Compute scalar product <y1[],y1[]> */

s = 1;					/* Avoid case of all zeros */

for(i=0; i<L_subfr; i++)

s = L_mac0(s, y1[i], y1[i]);

exp_yy = norm_l(s);

yy     = extract_h( L_shl(s, exp_yy) );

/* Test if Overflow */

if(Overflow != 0)

{

/* Compute scalar product <xn[],y1[]> */

s = 1;				/* Avoid case of all zeros */

for(i=0; i<L_subfr; i++)

{

L_tmp = L_mult0(xn[i], y1[i]);

L_tmp = L_shr(L_tmp, (Word16)6);

s = L_add(s, L_tmp);

}

exp_xy = norm_l(s);

xy     = extract_h( L_shl(s, exp_xy) );

/* Compute scalar product <y1[],y1[]> */

s = 1;				/* Avoid case of all zeros */

for(i=0; i<L_subfr; i++)

{

L_tmp = L_mult0(y1[i], y1[i]);

L_tmp = L_shr(L_tmp, (Word16)6);

s = L_add(s, L_tmp);

}

exp_yy = norm_l(s);

yy     = extract_h( L_shl(s, exp_yy) );

}

/* If (xy < 4) gain = 0 */

if( sub(xy, (Word16)4) < 0) return( (Word16) 0);

/* compute gain = xy/yy */

xy = shr(xy, (Word16)1);			/* Be sure xy < yy */

gain = div_s( xy, yy);

i = add(exp_xy, (Word16)2);		/* Denormalization of division */

i = sub(i, exp_yy);

gain = shr(gain, i);

/* if(gain >1.2) gain = 1.2  in Q12 */

if( sub(gain, (Word16)4915) > 0) gain = 4915;

return(gain);

}

/**************************************************************************

*

*	ROUTINE				:	Inter8_M1_3

*

*	DESCRIPTION			:	Fractional interpolation -1/3 with 8 coefficients

*

**************************************************************************

*

*	USAGE				:	Inter8_M1_3(buffer_in)

*							(Routine_Name(input1))

*

*	INPUT ARGUMENT(S)		:

*

*		INPUT1			:	- Description : Buffer containing the function

*								          to be interpolated

*						- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:	None

*

*	RETURNED VALUE		:	- Description : Interpolated value

*						- Format : Word32

*

**************************************************************************/

Word32 Inter8_M1_3(Word16 x[])

{

Word16 i;

Word32 s;

static Word16 coef[8] = {-236,1050,-3572,12714,26674,-5217,1630,-384};

s = 0;

for(i=0; i<8; i++)

s = L_mac0(s, x[i-4], coef[i]);

return(s);

}

/**************************************************************************

*

*	ROUTINE				:	Inter8_1_3

*

*	DESCRIPTION			:	Fractional interpolation 1/3 with 8 coefficients

*

**************************************************************************

*

*	USAGE				:	Inter8_1_3(buffer_in)

*							(Routine_Name(input1))

*

*	INPUT ARGUMENT(S)		:

*

*		INPUT1			:	- Description : Buffer containing the function

*								          to be interpolated

*						- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:	None

*

*	RETURNED VALUE		:	- Description : Interpolated value

*						- Format : Word32

*

**************************************************************************/

Word32 Inter8_1_3(Word16 x[])

{

Word16 i;

Word32 s;

static Word16 coef[8] = {-384,1630,-5217,26674,12714,-3572,1050,-236};

s = 0;

for(i=0; i<8; i++)

s = L_mac0(s, x[i-3], coef[i]);

return(s);

}

/**************************************************************************

*

*	ROUTINE				:	Inter32_M1_3

*

*	DESCRIPTION			:	Fractional interpolation -1/3 with 32 coefficients

*

**************************************************************************

*

*	USAGE				:	Inter32_M1_3(buffer_in)

*							(Routine_Name(input1))

*

*	INPUT ARGUMENT(S)		:

*

*		INPUT1			:	- Description : Buffer containing the function

*								          to be interpolated

*						- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:	None

*

*	RETURNED VALUE		:	- Description : Interpolated value

*						- Format : Word16

*

*	COMMENTS			:	For long term prediction, it must be noted that

*							exc[-(T0-1/3)] corresponds to exc[-T0+1/3]

*

**************************************************************************/

Word16 Inter32_M1_3(Word16 x[])

{

Word16 i;

Word32 s;

static Word16 coef[32] = {

-49,    66,   -96,   142,  -207,   294,  -407,   553,  -739,

981, -1304,  1758, -2452,  3688, -6669, 27072, 13496, -5287,  3179,

-2182,  1587, -1185,   893,  -672,   500,  -366,   263,  -183,   125,

-84,    59,   -47 };

s = 0;

for(i=0; i<32; i++)

s = L_mac0(s, x[i-15], coef[i]);

s = L_add(s, s);

i = round(s);

return(i);

}

/**************************************************************************

*

*	ROUTINE				:	Inter32_1_3

*

*	DESCRIPTION			:	Fractional interpolation 1/3 with 32 coefficients

*

**************************************************************************

*

*	USAGE				:	Inter32_1_3(buffer_in)

*							(Routine_Name(input1))

*

*	INPUT ARGUMENT(S)		:

*

*		INPUT1			:	- Description : Buffer containing the function

*								          to be interpolated

*						- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:	None

*

*	RETURNED VALUE		:	- Description : Interpolated value

*						- Format : Word16

*

*	COMMENTS			:	For long term prediction, it must be noted that

*							exc[-(T0+1/3)] corresponds to exc[-T0-1/3]

*

**************************************************************************/

Word16 Inter32_1_3(Word16 x[])

{

Word16 i;

Word32 s;

static Word16 coef[32] = {

-47,    59,   -84,   125,  -183,   263,  -366,   500,  -672,   893,

-1185,  1587, -2182,  3179, -5287, 13496, 27072, -6669,  3688, -2452,

1758, -1304,   981,  -739,   553,  -407,   294,  -207,   142,   -96,

66,   -49};

s = 0;

for(i=0; i<32; i++)

s = L_mac0(s, x[i-16], coef[i]);

s = L_add(s, s);

i = round(s);

return(i);

}

/**************************************************************************

*

*	ROUTINE				:	Lag_Max

*

*	DESCRIPTION			:	Find the lag that has maximum correlation with

*							the input buffer sig_dec[]

*

**************************************************************************

*

*	USAGE				:	Lag_Max(buffer_in1,buffer_in2,

*							L_frame, lag_max,lag_min,cor_max)

*							(Routine_Name(input1,input2,

*							input3,input4,input5,output1))

*

*	INPUT ARGUMENT(S)		:

*

*	INPUT1			:	- Description :	- Signal used to compute

*								the open loop pitch

*								- Buffer_in1[-142] to buffer_in1[-1]

*								should be known

*						- Format : Word16

*

*	INPUT2			:	- Description : Decimated signal (buffer sig_dec[])

*						- Format : Word16

*

*	INPUT3			:	- Description : Length of frame to compute pitch

*						- Format : Word16

*

*	INPUT4			:	- Description : Maximum lag

*						- Format : Word16

*

*		INPUT5			:	- Description : Minimum lag

*							- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : Maximum of normalized correlation

*								         of lag found

*							- Format : Word16

*

*	RETURNED VALUE		:	- Description : Lag found

*							- Format : Word16

*

**************************************************************************/

Word16 Lag_Max(Word16 signal[], Word16 sig_dec[], Word16 L_frame,

Word16 lag_max, Word16 lag_min, Word16 *cor_max)

{

Word16  i, j;

Word16  *p, *p1;

Word32  max, t0;

Word16  max_h, max_l, ener_h, ener_l;

Word16  p_max;

max = MIN_32;

for (i = lag_max; i >= lag_min; i--)

{

p  = sig_dec;

p1 = &signal[-i];

t0 = 0;

for (j=0; j<L_frame; j+=2, p++, p1+=2)

t0 = L_mac0(t0, *p, *p1);

if (L_sub(t0,max) >= 0)

{

max    = t0;

p_max = i;

}

}

max = L_shr(max, (Word16)1);	/* for special double precision format */

L_extract(max, &max_h, &max_l);

/* compute energie */

t0 = 0;

p = &signal[-p_max];

for(i=0; i<L_frame; i+=2, p+=2)

t0 = L_mac0(t0, *p, *p);

/* 1/sqrt(energie),    result in Q30 */

t0 = inv_sqrt(t0);

L_extract(t0, &ener_h, &ener_l);

/* max = max/sqrt(energie)                  */

/* This result will always be on 16 bits !! */

t0 = mpy_32(max_h, max_l, ener_h, ener_l);

*cor_max = extract_l(t0);

return(p_max);

}

/**************************************************************************

*

*	ROUTINE				:	Norm_Corr

*

*	DESCRIPTION			:	Find the normalized correlation

*						(correlation between the target vector and

*						the filtered past excitation divided by the square root

*							of  energy of filtered excitation)

*

**************************************************************************

*

*	USAGE				:	Norm_Corr(buffer_in1,buffer_in2,buffer_in3,

*							L_subfr,t_min,t_max,corr_norm)

*							(Routine_Name(input1,input2,input3,

*							input4,input5,input6,output1))

*

*	INPUT ARGUMENT(S)		:

*

*	INPUT1			:	- Description : Excitation buffer

*						- Format : Word16

*

*	INPUT2			:	- Description : Target vector

*						- Format : Word16

*

*	INPUT3			:	- Description : Impulse response of synthesis

*							          and weighting filters

*						- Format : Word16 - Q12

*

*	INPUT4			:	- Description : Length of subframe

*						- Format : Word16

*

*		INPUT5			:	- Description : Minimum value of pitch lag

*							- Format : Word16

*

*		INPUT6			:	- Description : Maximum value of pitch lag

*							- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : Normalized correlation

*							- Format : Word16

*

*	RETURNED VALUE		:	None

*

**************************************************************************/

void Norm_Corr(Word16 exc[], Word16 xn[], Word16 h[], Word16 L_subfr,

Word16 t_min, Word16 t_max, Word16 corr_norm[])

{

Word16 i,j,k;

Word16 corr_h, corr_l, norm_hi, norm_lo;

Word32 s;

Word16 excf[80];			/* Usally dynamic allocation of (L_subfr) */

k = - t_min;

/* compute the filtered excitation for the first delay t_min */

Convolve(&exc[k], h, excf, L_subfr);

/* loop for every possible period */

for (i = t_min; i <= t_max; ++i)

{

/* Compute correlation between xn[] and excf[] */

s = 0;

for (j = 0; j < L_subfr; j++)

s = L_mac0(s, xn[j], excf[j]);

s = L_shr(s, (Word16)1);		/* Special double precision format */

L_extract(s, &corr_h, &corr_l);

/* Compute 1/sqrt(energie of excf[]) */

s = 0;

for (j = 0; j < L_subfr; j++)

s = L_mac0(s, excf[j], excf[j]);

s = inv_sqrt(s);			/* Result in Q30 */

L_extract(s, &norm_hi, &norm_lo);

/* Normalize correlation = correlation * (1/sqrt(energie)) */

s = mpy_32(corr_h, corr_l, norm_hi, norm_lo);

corr_norm[i] = extract_l(s);	/* Result is on 16 bits */

/* modify the filtered excitation excf[] for the next iteration */

if( sub(i, t_max) != 0)

{

k--;

for (j = L_subfr-1; j > 0; j--)

{

s = L_mult0(exc[k], h[j]);

s = add_sh(s, excf[j-1], (Word16)12);

excf[j] = store_hi(s, (Word16)4);

}

excf[0] = exc[k];

}

}

return;

}

/**************************************************************************

*

*	ROUTINE				:	Pitch_Fr

*

*	DESCRIPTION			:	Find the pitch period with 1/3 subsample resolution

*

**************************************************************************

*

*	USAGE				:	Pitch_Fr(buffer_in1,buffer_in2,buffer_in3,

*							L_subfr,t0_min,t0_max,i_subfr,pit_frac)

*							(Routine_Name(input1,input2,input3,

*							input4,input5,input6,input7,output1))

*

*	INPUT ARGUMENT(S)		:

*

*	INPUT1			:	- Description : Excitation buffer

*						- Format : Word16

*

*	INPUT2			:	- Description : Target vector

*						- Format : Word16

*

*	INPUT3			:	- Description : Impulse response of synthesis

*							          and weighting filters

*						- Format : Word16 - Q12

*

*	INPUT4			:	- Description : Length of subframe

*						- Format : Word16

*

*		INPUT5			:	- Description : Minimum value in the searched range

*							- Format : Word16

*

*		INPUT6			:	- Description : Maximum value in the searched range

*							- Format : Word16

*

*		INPUT7			:	- Description : Indicator for first subframe

*							- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : Chosen fraction : (-1, 0, 1) / 3

*							- Format : Word16

*

*	RETURNED VALUE		:	- Description : Pitch period

*							- Format : Word16

*

**************************************************************************/

/* Lg_inter = Length for fractionnal interpolation = nb.coeff/2 */

#define Lg_inter 4

Word16 Pitch_Fr(Word16 exc[], Word16 xn[], Word16 h[], Word16 L_subfr,

Word16 t0_min, Word16 t0_max, Word16 i_subfr,

Word16 *pit_frac)

{

Word16 i;

Word16 t_min, t_max;

Word16 max, lag, frac;

Word16 *corr;

Word32 corr_int, L_max;

Word16 corr_v[40];	/* Total length = t0_max-t0_min+1+2*Lg_inter */

/* Find interval to compute normalized correlation */

t_min = sub(t0_min, (Word16)Lg_inter);

t_max = add(t0_max, (Word16)Lg_inter);

corr = &corr_v[-t_min];

/* Compute normalized correlation between target and filtered excitation */

Norm_Corr(exc, xn, h, L_subfr, t_min, t_max, corr);

/* Find integer pitch */

max = corr[t0_min];

lag = t0_min;

for(i= t0_min+1; i<=t0_max; i++)

{

if( sub(corr[i],max)>= 0)

{

max = corr[i];

lag = i;

}

}

/* If first subframe and lag > 84 do not search fractionnal pitch */

if( (i_subfr == 0) && (sub(lag, (Word16)84) > 0) )

{

*pit_frac = 0;

return(lag);

}

/* Test the fractions around T0 and choose the one which maximizes   */

/* the interpolated normalized correlation.                          */

frac  = 0;

L_max = Load_sh(max, (Word16)15);

/* test +1/3 */

corr_int = Inter8_1_3(&corr[lag]);

if(L_sub(corr_int, L_max) >= 0)

{ L_max=corr_int; frac= 1;}

/* test +2/3  = 1-1/3 */

corr_int = Inter8_M1_3(&corr[lag+1]);

if(L_sub(corr_int, L_max) >= 0)

{ L_max=corr_int; frac= 2;}

/* test -2/3 = 1/3 -1 */

corr_int = Inter8_1_3(&corr[lag-1]);

if(L_sub(corr_int, L_max) >= 0)

{ L_max=corr_int; frac= -2;}

/* test -1/3 */

corr_int = Inter8_M1_3(&corr[lag]);

if(L_sub(corr_int, L_max) >= 0)

{ L_max=corr_int; frac= -1;}

if(sub(frac, (Word16)2) == 0)

{ frac = -1; lag = add(lag, (Word16)1);};

if(sub(frac, (Word16)-2) == 0)

{ frac = 1; lag = sub(lag, (Word16)1);};

*pit_frac = frac;

return(lag);

}

/**************************************************************************

*

*	ROUTINE				:	Pitch_Ol_Dec

*

*	DESCRIPTION			:	Compute the open loop pitch lag

*							(includes decimation of signal)

*

**************************************************************************

*

*	USAGE				:	Pitch_Ol_Dec(buffer_in,L_frame)

*							(Routine_Name(input1,input2))

*

*	INPUT ARGUMENT(S)		:

*

*	INPUT1			:	- Description :	- Signal used to compute

*								the open loop pitch

*								- Buffer_in[-142] to buffer_in[-1]

*								should be known

*						- Format : Word16

*

*	INPUT2			:	- Description : Length of frame to compute pitch

*						- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:	None

*

*	RETURNED VALUE		:	- Description : Open pitch lag as found

*							- Format : Word16

*

**************************************************************************/

/* Minimum and maximum pitch lag */

#define pit_min 20

#define pit_max 142

/* Threshold to favor small pitch = 0.85 in Q15 */

#define seuil 27856

Word16 Pitch_Ol_Dec(Word16 signal[], Word16 L_frame)

{

Word16  i, j;

Word16  max1, max2, max3;

Word16  p_max1, p_max2, p_max3;

Word32  t0;

/* Decimated signal                                       */

/* Can be allocated with memory allocation of(L_frame/2)  */

Word16  sig_dec[120];

/*--------------------------------------------------------*

*  Verification for risk of overflow.                    *

*--------------------------------------------------------*/

Overflow = 0;

t0 = 0;

for(i= -pit_max; i< L_frame; i+=2)

t0 = L_mac0(t0, signal[i], signal[i]);

/*--------------------------------------------------------*

* Decimation of signal and scaling.                      *

*                                                        *

*   if Overflow        -> sig_dec[i] = signal[i*2]>>6    *

*   else if t0 < 1<<22 -> sig_dec[i] = signal[i*2]<<4    *

*   else               -> sig_dec[i] = signal[i*2]       *

*--------------------------------------------------------*/

if(Overflow == 1)

{

for(i=0, j=0; i<L_frame; i+=2, j++)

sig_dec[j] = shr(signal[i], (Word16)6);

}

else if (L_sub(t0, L_shl( (Word32)1, (Word16)22)) < 0 )

{

for(i=0, j=0; i<L_frame; i+=2, j++)

sig_dec[j] = shl(signal[i], (Word16)4);

}

else

{

for(i=0, j=0; i<L_frame; i+=2, j++)

sig_dec[j] = signal[i];

}

/*--------------------------------------------------------------------*

*  The pitch lag search is divided in three sections.                *

*  Each section cannot have a pitch multiple.                        *

*  A maximum is found for each section.                              *

*  The maximum of each section is compared to favor small lag.       *

*                                                                    *

*  First section:  lag delay = pit_max to 80                         *

*  Second section: lag delay = 79 to 40                              *

*  Third section:  lag delay = 39 to 20                              *

*--------------------------------------------------------------------*/

p_max1 = Lag_Max(signal, sig_dec, L_frame, (Word16)pit_max,

(Word16)80 , &max1);

p_max2 = Lag_Max(signal, sig_dec, L_frame, (Word16)79     ,

(Word16)40 , &max2);

p_max3 = Lag_Max(signal, sig_dec, L_frame, (Word16)39     ,

(Word16)20 , &max3);

/*--------------------------------------------------------------------*

* Compare the 3 sections maximum, and favor small lag.               *

*--------------------------------------------------------------------*/

if( sub(mult((Word16)max1, (Word16)seuil), (Word16)max2)  < 0)

{ max1 = max2;  p_max1 = p_max2;}

if( sub(mult((Word16)max1, (Word16)seuil), (Word16)max3)  < 0)

p_max1 = p_max3;

return (p_max1);

}

/**************************************************************************

*

*	ROUTINE				:	Post_Process

*

*	DESCRIPTION			:	Post-processing of output speech

*							Multiplication by two of output speech

*							with saturation

*

**************************************************************************

*

*	USAGE				:	Post_Process(buffer,Length)

*							(Routine_Name(arg1,input2))

*

*	INPUT ARGUMENT(S)		:

*

*	ARG1				:	- Description : Input speech signal buffer

*						- Format : Word16

*

*		INPUT2			:	- Description : Length of signal

*							- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:

*

*		ARG1				:	- Description : Output speech signal buffer

*							- Format : Word16

*

*	RETURNED VALUE		:	None

*

**************************************************************************/

void Post_Process(Word16 signal[], Word16 lg)

{

Word16 i;

for(i=0; i<lg; i++)

signal[i] = add(signal[i], signal[i]);

}

/**************************************************************************

*

*	ROUTINE				:	Pred_Lt

*

*	DESCRIPTION			:	Compute the result of long term prediction with

*							fractional interpolation

*

**************************************************************************

*

*	USAGE				:	Pred_Lt(buffer,T0,frac,L_subfr)

*							(Routine_Name(arg1,input2,input3,input4))

*

*	INPUT ARGUMENT(S)		:

*

*	ARG1				:	- Description : Excitation vector

*						- Format : Word16

*

*	INPUT2			:	- Description : Pitch lag

*						- Format : Word16

*

*	INPUT3			:	- Description : Fraction of pitch lag : (-1, 0, 1)  / 3

*						- Format : Word16 - Q12

*

*	INPUT4			:	- Description : Length of subframe

*						- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:

*

*		ARG1				:	- Description : Interpolated signal contained in

*								         buffer[0..L_subfr-1]

*							- Format : Word16

*

*	RETURNED VALUE		:	None

*

**************************************************************************/

void Pred_Lt(Word16 exc[], Word16 T0, Word16 frac, Word16 L_subfr)

{

Word16 i;

if(frac == 0)

{

for (i = 0; i < L_subfr; i++)

exc[i] = exc[i-T0];

}

else if( sub(frac, (Word16)1) == 0)

{

for (i = 0; i < L_subfr; i++)

exc[i] = Inter32_1_3(&exc[i-T0]);

}

else if( sub(frac, (Word16)-1) == 0)

{

for (i = 0; i < L_subfr; i++)

exc[i] = Inter32_M1_3(&exc[i-T0]);

}

return;

}

/**************************************************************************

*

*	ROUTINE				:	Pre_Process

*							(include Init_Pre_Process)

*

*	DESCRIPTION			:	Preprocessing of input speech

*							- Offset compensation

*								- Divide input by two

*

**************************************************************************

*

*	USAGE				:	Pre_Process(buffer,Length)

*							(Routine_Name(arg1,input2))

*

*	INPUT ARGUMENT(S)		:

*

*	ARG1				:	- Description : Input speech signal buffer

*						- Format : Word16

*

*		INPUT2			:	- Description : Length of signal

*							- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:

*

*		ARG1				:	- Description : Output speech signal buffer

*							- Format : Word16

*

*	RETURNED VALUE		:	None

*

*	COMMENTS			:	Algorithm :

*

*						y[i] = x[i]/2 - x[i-1]/2 + alpha * y[i-1]

*

*  						alpha = 32735 / 2**15

*  						y[i-1] is keep in double precision

*

*  						It is the same algorithm than in GSM except that :

*  						- Input is divided by two in the filtering process

*  						- No rounding

*

**************************************************************************/

/* Static values to be preserved between calls */

static Word16 y_hi, y_lo, x0;

/* Initialization of static values */

void Init_Pre_Process(void)

{

y_hi = 0;

y_lo = 0;

x0   = 0;

}

/* Offset compensation and divide by 2 */

void Pre_Process(Word16 signal[], Word16 lg)

{

Word16 i, x1;

Word32 L_tmp;

for(i=0; i<lg; i++)

{

x1 = x0;

x0 = signal[i];

L_tmp     = Load_sh(x0, (Word16)15);

L_tmp     = sub_sh(L_tmp, x1, (Word16)15);

L_tmp     = L_mac(L_tmp, y_hi, (Word16)32735);

L_tmp     = add_sh(L_tmp, mult(y_lo, (Word16)32735), (Word16)1);

signal[i] = extract_h(L_tmp);

y_hi      = extract_h(L_tmp);

y_lo      = extract_l(sub_sh(L_shr( L_tmp,(Word16)1 ), y_hi, (Word16)15));

}

return;

}

/**************************************************************************

*

*	ROUTINE				:	Prm2bits_Tetra

*

*	DESCRIPTION			:	Convert the encoder parameter vector into

*							a vector of serial bits

*

**************************************************************************

*

*	USAGE				:	Prm2bits_Tetra(buffer_in,buffer_out)

*							(Routine_Name(input1,output1))

*

*	INPUT ARGUMENT(S)		:

*

*		INPUT1			:	- Description : Encoded parameters

*								         (23 parameters)

*							- Format : Word16 - .. * 16 bit-samples

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : Serial bits (137 + bfi)

*							- Format : Word16 - .. * 16 bit-samples

*

*	RETURNED VALUE		:	None

*

*	COMMENTS			:	The transmitted parameters are :

*

*						- LSP	1st codebook 			8 bits

*							2nd codebook			9 bits

*							3nd codebook			9 bits

*

*						- for the 4 subframes :

*							pitch delay			8 bits (first)

*											5 bits (others)

*							codebook index		14 bits

*							pulse global sign		1 bit

*							pulse shift			1 bit

*							pitch and innovation gains	6 bits

*

**************************************************************************/

#define PRM_NO    23

void Prm2bits_Tetra(Word16 prm[], Word16 bits[])

{

Word16 i;

static Word16 bitno[PRM_NO] = {8, 9, 9,            /* split VQ LSP  */

8, 14, 1, 1, 6,     /* subframe 1    */

5, 14, 1, 1, 6,     /* subframe 2    */

5, 14, 1, 1, 6,     /* subframe 3    */

5, 14, 1, 1, 6};    /* subframe 4    */

*bits++ = 0;	/* bit[0] = 0, at receiver this bits indicate BFI */

for (i = 0; i < PRM_NO; i++)

{

int2bin(prm[i], bitno[i], bits);

bits += bitno[i];

}

return;

}

## A.4	Library of Signal Processing Related Subroutines

/************************************************************************

*

*	FILENAME		:	sub_dsp.c

*

*	DESCRIPTION		:	General Purpose Signal Processing Library

*

************************************************************************

*

*	SUB-ROUTINES	:	- Autocorr()

*					- Az_Lsp()

*					- Back_Fil()

*					- Chebps()

*					- Convolve()

*					- Fac_Pond()

*					- Get_Lsp_Pol()

*					- Int_Lpc4()

*					- Lag_Window()

*					- Levin_32()

*					- LPC_Gain()

*					- Lsp_Az()

*					- Pond_Ai()

*					- Residu()

*					- Syn_Filt()

*

************************************************************************

*

*	INCLUDED FILES	:	source.h

*					stdio.h

*					stdlib.h

*

*					grid .tab

*					lag_wind.tab

*					window.tab

*

************************************************************************/

#include <stdio.h>

#include <stdlib.h>

#include "source.h"

#include "grid.tab"

#include "lag_wind.tab"

#include "window.tab"

/* pp = local LPC order, nc =pp/2 */

#define pp (Word16)10

#define nc (Word16)5

/* Length for local impulse response */

#define  llg     (Word16)60

/**************************************************************************

*

*	ROUTINE				:	Autocorr

*

*	DESCRIPTION			:	Compute autocorrelations

*

**************************************************************************

*

*	USAGE				:	Autocorr(buffer_in,p,buffer_out1,buffer_out2)

*							(Routine_Name(input1,input2,output1,output2))

*

*	INPUT ARGUMENT(S)		:

*

*	INPUT1			:	- Description : Input signal buffer

*						- Format : Word16

*

*	INPUT2			:	- Description : LPC order

*						- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : Autocorrelations buffer (msb)

*							- Format : Word16

*

*		OUTPUT2			:	- Description : Autocorrelations buffer (lsb)

*							- Format : Word16

*

*	RETURNED VALUE		:	None

*

**************************************************************************/

void Autocorr(Word16 x[], Word16 p, Word16 r_h[], Word16 r_l[])

{

Word16 i, j, norm;

Word16 y[L_window];

Word32 sum;

extern Flag Overflow;

/* Windowing of signal */

for(i=0; i<L_window; i++)

y[i] = mult_r(x[i], window[i]);

/* Compute r[0] and test for overflow */

do {

Overflow = 0;

sum = 1;				/* Avoid case of all zeros */

for(i=0; i<L_window; i++)

sum = L_mac0(sum, y[i], y[i]);

/* If overflow divide y[] by 4 */

if(Overflow != 0)

{

for(i=0; i<L_window; i++)

y[i] = shr(y[i], (Word16)2);

}

}while(Overflow != 0);

/* Normalization of r[0] */

norm = norm_l(sum);

sum  = L_shl(sum, norm);

sum  = L_shr(sum, (Word16)1);		/* For special double format */

L_extract(sum, &r_h[0], &r_l[0]);

/* r[1] to r[p] */

for (i = 1; i <= p; i++)

{

sum = 0;

for(j=0; j<L_window-i; j++)

sum = L_mac0(sum, y[j], y[j+i]);

sum = L_shr(sum, (Word16)1);		/* Special double format */

sum = L_shl(sum, norm);

L_extract(sum, &r_h[i], &r_l[i]);

}

return;

}

/**************************************************************************

*

*	ROUTINE				:	Az_Lsp

*

*	DESCRIPTION			:	Compute the LSPs  in the cosine domain

*							from the LPC coefficients  (order=10)

*

**************************************************************************

*

*	USAGE				:	Az_Lsp(buffer_in1,buffer_out,buffer_in2)

*							(Routine_Name(input1,output1,input2))

*

*	INPUT ARGUMENT(S)		:

*

*	INPUT1			:	- Description : Predictor coefficients

*						- Format : Word16 - Q12

*

*	INPUT2			:	- Description : Previous LSP values

*							          (in case not 10 roots are found)

*						- Format : Word16 - Q15

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : Line spectral pairs in the

*								          cosine domain

*							- Format : Word16 - Q15

*

*	RETURNED VALUE		:	None

*

**************************************************************************/

void Az_Lsp(Word16 a[], Word16 lsp[], Word16 old_lsp[])

{

Word16 i, j, nf, ip;

Word16 xlow, ylow, xhigh, yhigh, xmid, ymid, xint;

Word16 x, y, sign, exp;

Word16 *coef;

Word16 f1[pp/2+1], f2[pp/2+1];

Word32 t0;

/*-------------------------------------------------------------*

*  find the sum and diff. pol. F1(z) and F2(z)                *

*    F1(z) <--- F1(z)/(1+z**-1) & F2(z) <--- F2(z)/(1-z**-1)  *

*                                                             *

* f1[0] = 1.0;                                                *

* f2[0] = 1.0;                                                *

*                                                             *

* for (i = 0; i< nc; i++)                                     *

* {                                                           *

*   f1[i+1] = a[i+1] + a[pp-i] - f1[i] ;                       *

*   f2[i+1] = a[i+1] - a[pp-i] + f2[i] ;                       *

* }                                                           *

*-------------------------------------------------------------*/

f1[0] = 2048;				/* f1[0] = 1.0 is in Q11 */

f2[0] = 2048;				/* f2[0] = 1.0 is in Q11 */

for (i = 0; i< nc; i++)

{

t0 = Load_sh(a[i+1], (Word16)15);	/* a[i+1]  in Q27 */

t0 = add_sh(t0, a[pp-i], (Word16)15);	/* +a[pp-i] in Q27 */

t0 = sub_sh16(t0, f1[i]);			/* -f1[i]  in Q27 */

f1[i+1] = extract_h(t0);		/* f1[i+1] = a[i+1] + a[pp-i] - f1[i] */

/* result in Q11  */

t0 = Load_sh(a[i+1], (Word16)15);	/* a[i+1]  in Q27   */

t0 = sub_sh(t0, a[pp-i], (Word16)15);	/* -a[pp-i] in Q27 */

t0 = add_sh16(t0, f2[i]);			/* +f2[i] in Q27  */

f2[i+1] = extract_h(t0);		/* f2[i+1] = a[i+1] - a[pp-i] + f2[i] */

/* result in Q11  */

}

/*-------------------------------------------------------------*

* find the LSPs using the Chebichev pol. evaluation           *

*-------------------------------------------------------------*/

nf=0;          /* number of found frequencies */

ip=0;          /* indicator for f1 or f2      */

coef = f1;

xlow = grid[0];

ylow = Chebps(xlow, coef, nc);

j = 0;

while ( (nf < pp) && (j < grid_points) )

{

j++;

xhigh = xlow;

yhigh = ylow;

xlow  = grid[j];

ylow  = Chebps(xlow,coef,nc);

if ( L_mult0(ylow,yhigh) <= (Word32)0)

{

/* divide 4 times the interval */

for (i = 0; i < 4; i++)

{

t0   = Load_sh(xlow, (Word16)15);	/* xmid = 0.5*(xlow + xhigh) */

t0   = add_sh(t0, xhigh, (Word16)15);

xmid = extract_h(t0);

ymid = Chebps(xmid,coef,nc);

if ( L_mult0(ylow,ymid) <= (Word32)0)

{

yhigh = ymid;

xhigh = xmid;

}

else

{

ylow = ymid;

xlow = xmid;

}

}

/*-------------------------------------------------------------*

* Linear interpolation                                        *

*    xint = xlow - ylow*(xhigh-xlow)/(yhigh-ylow);            *

*-------------------------------------------------------------*/

x   = sub(xhigh, xlow);

y   = sub(yhigh, ylow);

if(y == 0)

{

xint = xlow;

}

else

{

sign= y;

y   = abs_s(y);

exp = norm_s(y);

y   = shl(y, exp);

y   = div_s( (Word16)16383, y);

t0  = L_mult0( x, y);

t0  = L_shr(t0, sub((Word16)19,exp) );

y    = extract_l(t0);		/* y= (xhigh-xlow)/(yhigh-ylow) in Q10 */

if(sign < 0) y = negate(y);

t0   = Load_sh(xlow, (Word16)10);	/* xint = xlow - ylow*y */

t0   = L_msu0(t0, ylow, y);

xint = store_hi(t0, (Word16)6);

}

lsp[nf] = xint;

xlow    = xint;

nf++;

if(ip == 0)

{

ip = 1;

coef = f2;

}

else

{

ip = 0;

coef = f1;

}

ylow = Chebps(xlow,coef,nc);

}

}

/* Check if pp roots found */

if( sub(nf, pp) < 0)

{

for(i=0; i<pp; i++)

lsp[i] = old_lsp[i];

printf("\n !!Not 10 roots found in Az_Lsp()!!!n");

}

return;

}

/**************************************************************************

*

*	ROUTINE				:	Back_Fil

*

*	DESCRIPTION			:	Perform the Backward filtering of input vector

*						buffer_in1 with buffer_in2 and write the result

*						in output vector buffer_out.

*							All vectors are of length L

*

**************************************************************************

*

*	USAGE				:	Back_Fil(buffer_in1,buffer_in2,buffer_out,L)

*							(Routine_Name(input1,input2,output1,input3))

*

*	INPUT ARGUMENT(S)		:

*

*	INPUT1			:	- Description : Input vector

*						- Format : Word16

*

*	INPUT2			:	- Description : Impulse response

*						- Format : Word16 - Q12

*

*	INPUT3			:	- Description : Vector size

*						- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : Backward filtering result

*							- Format : Word16

*

*	RETURNED VALUE		:	None

*

**************************************************************************/

void Back_Fil(Word16 x[], Word16 h[], Word16 y[], Word16 L)

{

Word16 i, j;

Word32 s, max;

Word32 y32[60];		/* Usually, dynamic allocation of L */

/* first keep the result on 32 bits and find absolute maximum */

max = 0;

for (i = 0; i < L; i++)

{

s = 0;

for (j = i; j <  L; j++)

s = L_mac0(s, x[j], h[j-i]);

y32[i] = s;

s = L_abs(s);

if(L_sub(s, max) > 0) max = s;

}

/* Find the number of right shifts to do on y32[]  */

/* so that maximum is on 13 bits                   */

j = norm_l(max);

if( sub(j,(Word16)16) > 0) j = 16;

j = sub((Word16)18, j);

for(i=0; i<L; i++)

y[i] = extract_l( L_shr(y32[i], j) );

return;

}

/**************************************************************************

*

*	ROUTINE				:	Chebps

*

*	DESCRIPTION			:	Evaluate the Chebishev polynomial series

*

**************************************************************************

*

*	USAGE				:	Chebps(x,buffer_in,n)

*							(Routine_Name(input1,input2,input3))

*

*	INPUT ARGUMENT(S)		:

*

*	INPUT1			:	- Description : Input value of evaluation

*							          x = cos(frequency)

*						- Format : Word16 - Q15

*

*	INPUT2			:	- Description : Coefficients of the polynomial series

*						- Format : Word16 - Q11

*

*	INPUT3			:	- Description : Polynomial order

*						- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:	None

*

*	RETURNED VALUE		:	- Description : Value of the polynomial C(x)

*							- Format : Word16 - Q14

*							     (saturated to +/- 1.99)

*

*	COMMENTS			:	- The polynomial order is :

*						n = p/2   (p is the prediction order)

*

*						- The polynomial is given by :

*							C(x)=T_n(x)+f(1)T_n-1(x)+...+f(n-1)T_1(x)+ f(n)/2

*

**************************************************************************/

Word16 Chebps(Word16 x, Word16 f[], Word16 n)

{

Word16 i, cheb;

Word16 b0_h, b0_l, b1_h, b1_l, b2_h, b2_l;

Word32 t0;

/* Note: All computation are done in Q24. */

b2_h = 512;					/* b2 = 1.0 in Q24 DPF */

b2_l = 0;

t0 = Load_sh( x,(Word16)10);		/* 2*x in Q24          */

t0 = add_sh( t0, f[1], (Word16)13);	/* + f[1] in Q24       */

L_extract(t0, &b1_h, &b1_l);		/* b1 = 2*x + f[1]     */

for (i = 2; i<n; i++)

{

t0 = mpy_mix(b1_h, b1_l, x);		/* t0 = x*b1                  */

t0 = L_shl(t0,(Word16)1);			/* t0 = 2.0*x*b1              */

t0 = sub_sh(t0, b2_l, (Word16)0);	/* t0 = 2.0*x*b1 - b2         */

t0 = sub_sh(t0, b2_h, (Word16)15);

t0 = add_sh(t0, f[i], (Word16)13);	/* t0 = 2.0*x*b1 - b2 + f[i]; */

L_extract(t0, &b0_h, &b0_l);		/* b0 = 2.0*x*b1 - b2 + f[i]; */

b2_l = b1_l;					/* b2 = b1; */

b2_h = b1_h;

b1_l = b0_l;					/* b1 = b0; */

b1_h = b0_h;

}

t0 = mpy_mix(b1_h, b1_l, x);		/* t0    = x*b1;     */

t0 = sub_sh(t0, b2_l, (Word16)0);		/* t0   -= b2;       */

t0 = sub_sh(t0, b2_h, (Word16)15);

t0 = add_sh(t0, f[n], (Word16)12);	/* t0   += 0.5*f[n]; */

t0 = L_shl(t0, (Word16)6);			/* Q24 to Q30 with saturation */

cheb = extract_h(t0);				/* Result in Q14           */

return(cheb);

}

/**************************************************************************

*

*	ROUTINE				:	Convolve

*

*	DESCRIPTION			:	Perform the convolution between two input vectors

*						buffer_in1 and buffer_in2 and write the result in

*						output vector buffer_out.

*							All vectors are of length L

*

**************************************************************************

*

*	USAGE				:	Convolve(buffer_in1,buffer_in2,buffer_out,L)

*							(Routine_Name(input1,input2,output1,input3))

*

*	INPUT ARGUMENT(S)		:

*

*	INPUT1			:	- Description : Input vector

*						- Format : Word16

*

*	INPUT2			:	- Description : Impulse response

*						- Format : Word16 - Q12

*

*	INPUT3			:	- Description : Vector size

*						- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : Output vector

*							- Format : Word16

*

*	RETURNED VALUE		:	None

*

**************************************************************************/

void Convolve(Word16 x[], Word16 h[], Word16 y[], Word16 L)

{

Word16 i, n;

Word32 s;

for (n = 0; n < L; n++)

{

s = 0;

for (i = 0; i <= n; i++)

s = L_mac0(s, x[i], h[n-i]);

y[n] = store_hi(s, (Word16)4);		/* h is in Q12 */

}

return;

}

/**************************************************************************

*

*	ROUTINE				:	Fac_Pond

*

*	DESCRIPTION			:	Compute LPC spectral expansion factors (fac[])

*							with the LPC order fixed to 10

*

**************************************************************************

*

*	USAGE				:	Fac_Pond(gamma,buffer_out)

*							(Routine_Name(input1,output1))

*

*	INPUT ARGUMENT(S)		:

*

*	INPUT1			:	- Description : Spectral expansion

*						- Format : Word16 - Q15

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : Computed expansion factors

*							- Format : Word16 - Q15

*

*	RETURNED VALUE		:	None

*

*	COMMENTS			:	- fac[0] = gamma

*							- fac[i] = fac[i-1] * gamma	i=1,9

*

**************************************************************************/

void Fac_Pond(Word16 gamma, Word16 fac[])

{

Word16 i;

fac[0] = gamma;

for(i=1; i<pp; i++)

fac[i] = round( L_mult(fac[i-1], gamma) );

return;

}

/**************************************************************************

*

*	ROUTINE				:	Get_Lsp_Pol

*

*	DESCRIPTION			:	Find the polynomial F1(z) or F2(z) from the LSPs

*

**************************************************************************

*

*	USAGE				:	Get_Lsp_Pol(buffer_in,buffer_out)

*							(Routine_Name(input1,output1))

*

*	INPUT ARGUMENT(S)		:

*

*	INPUT1			:	- Description : line spectral pairs

*							          (cosine domaine)

*						- Format : Word16 - Q15

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : Coefficients of F1 or F2

*							- Format : Word32 - Q24

*

*	RETURNED VALUE		:	None

*

**************************************************************************/

void Get_Lsp_Pol(Word16 *lsp, Word32 *f)

{

Word16 i,j, hi, lo;

Word32 t0;

/* Computation in Q24 */

*f = Load_sh((Word16)4096,(Word16)12);	/* f[0] = 1.0;           in Q24  */

f++;

*f = 0;

*f = sub_sh(*f, *lsp, (Word16)10);	/* f[1] = -2.0 * lsp[0]; in Q24  */

f++;

lsp += 2;					/* Advance lsp pointer           */

for(i=2; i<=5; i++)

{

*f = f[-2];

for(j=1; j<i; j++, f--)

{

L_extract(f[-1] ,&hi, &lo);

t0 = mpy_mix(hi, lo, *lsp);		/* t0 = f[-1] * lsp    */

t0 = L_shl(t0, (Word16)1);

*f = L_add(*f, f[-2]);			/* *f += f[-2]         */

*f = L_sub(*f, t0);			/* *f -= t0            */

}

*f   = sub_sh(*f, *lsp, (Word16)10);	/* *f -= lsp<<10       */

f   += i;					/* Advance f pointer   */

lsp += 2;					/* Advance lsp pointer */

}

return;

}

/**************************************************************************

*

*	ROUTINE				:	Int_Lpc4

*

*	DESCRIPTION			:	Perform the LPC interpolation for the 4 sub-frames.

*							The interpolation is done on the LSP computed

*							in the cosine domain

*

**************************************************************************

*

*	USAGE				:	Int_Lpc4(buffer_in1,buffer_in2,buffer_out)

*							(Routine_Name(input1,input2,output1))

*

*	INPUT ARGUMENT(S)		:

*

*	INPUT1			:	- Description : LSP of previous frame

*						- Format : Word16

*

*	INPUT2			:	- Description : LSP of current frame

*						- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : LPC coeff. vector for the 4 sub-frames

*							- Format : Word16

*

*	RETURNED VALUE		:	None

*

**************************************************************************/

void Int_Lpc4(Word16 lsp_old[], Word16 lsp_new[], Word16 a[])

{

Word16 i, j, fac_new, fac_old;

Word16 lsp[pp];

Word32 t0;

fac_new = 8192;       /* 1/4 in Q15 */

fac_old = 24576;      /* 3/4 in Q15 */

for(j=0; j<33; j+=11)

{

for(i=0; i<pp; i++)

{

t0 = L_mult(lsp_old[i], fac_old);

t0 = L_mac(t0, lsp_new[i], fac_new);

lsp[i] = extract_h(t0);

}

Lsp_Az(lsp, &a[j]);

fac_old = sub(fac_old, (Word16)8192);

fac_new = add(fac_new, (Word16)8192);

}

Lsp_Az(lsp_new, &a[33]);

return;

}

/**************************************************************************

*

*	ROUTINE				:	Lag_Window

*

*	DESCRIPTION			:	Lag_window on autocorrelations

*

**************************************************************************

*

*	USAGE				:	Lag_Window(p,buffer1,buffer2)

*							(Routine_Name(input1,arg2,arg3))

*

*	INPUT ARGUMENT(S)		:

*

*	INPUT1			:	- Description :  LPC order

*						- Format : Word16

*

*	ARG2				:	- Description : Autocorrelations (msb)

*						- Format : Word16

*

*	ARG3				:	- Description : Autocorrelations (lsb)

*						- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:

*

*	ARG2				:	- Description : Lag_Windowed autocorrelations (msb)

*						- Format : Word16

*

*	ARG3				:	- Description : Lag_Windowed autocorrelations (lsb)

*						- Format : Word16

*

*	RETURNED VALUE		:	None

*

*	COMMENTS			:	r[i] *= lag_wind[i]

*

*						r[i] and lag_wind[i] are in special extended precision

*						(for details on this format, see "comments"

*						in the header of the file fexp_tet.c - Section A.5.2)

*

**************************************************************************/

void Lag_Window(Word16 p, Word16 r_h[], Word16 r_l[])

{

Word16 i;

Word32 x;

for(i=1; i<=p; i++)

{

x  = mpy_32(r_h[i], r_l[i], lag_h[i-1], lag_l[i-1]);

L_extract(x, &r_h[i], &r_l[i]);

}

return;

}

/**************************************************************************

*

*	ROUTINE				:	Levin_32

*

*	DESCRIPTION			:	Computation of 10 LPC coefficients

*							based on the Levison-Durbin algorithm

*							in double precision

*

**************************************************************************

*

*	USAGE				:	Levin_32(buffer_in1,buffer_in2,buffer_out)

*							(Routine_Name(input1,input2,output1))

*

*	INPUT ARGUMENT(S)		:

*

*	INPUT1			:	- Description : Vector of autocorrelations (msb)

*						- Format : Word16 - 11 values

*

*	INPUT2			:	- Description : Vector of autocorrelations (lsb)

*						- Format : Word16 - 11 values

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : LPC coefficients

*							- Format : Word16 - Q12

*

*	RETURNED VALUE		:	None

*

*	COMMENTS			:	Algorithm :

*

*							R[i]	autocorrelations

*							A[i]	filter coefficients

*							K 	reflection coefficients

*							Alpha	prediction gain.

*

*						Initialisations :

*

*							A[0] = 1

*							K    = -R[1]/R[0]

*							A[1] = K

*							Alpha = R[0] * (1-K**2]

*

*						DO for  i = 2 to pp

*

*							S =  SUM ( R[j]*A[i-j] ,j=1,i-1 ) +  R[i]

*							K = -S / Alpha

* 							An[j] = A[j] + K*A[i-j]   for j=1 to i-1

*								where   An[i] = new A[i]

*							An[i]=K

*							Alpha=Alpha * (1-K**2)

*

*						END

*

**************************************************************************

**************************************************************************

*

*	NOTES ON THE DYNAMICS OF THE COMPUTATIONS	:

*

*	The numbers used are in double precision with the following format :

*

*	A = AH <<15 + AL.  AH and AL are 16 bit signed integers. Since the LSB's also contain *	a sign bit, this format does not correspond to standard 32 bit integers.  This format is used *	since it allows fast execution of multiplications and divisions.

*

*	"DPF" will refer to this special format in the following text.

*	(for details on this format, see "comments" in the header of file fexp_tet.c - Section A.5.2)

*

*	The R[i] were normalized in routine Autocorr (hence, R[i] < 1.0).

*	The K[i] and Alpha are theoretically < 1.0.

*	The A[i], for a sampling frequency of 8 kHz, are in practice always inferior to 16.0.

*

*	These characteristics allow straigthforward fixed-point implementation.  The parameters are *	represented as follows :

*

*	R[i]    Q30   +- .99..

*	K[i]    Q30   +- .99..

*	Alpha   Normalised -> mantissa in Q30 plus exponant

*	A[i]    Q26   +- 15.999..

*

*	The additions are performed in 32 bit.  For the summation used to compute the K[i],

*	numbers in Q30 are multiplied by numbers in Q26, with the results of the multiplications in *	Q26, resulting in a dynamic of +/- 32.  This is sufficient to avoid overflow, since the final

*	result of the summation is necessarily < 1.0 as both the K[i] and Alpha are

*	theoretically < 1.0.

*

**************************************************************************/

/* Last A(z) for case of unstable filter */

static Word16 old_A[pp+1]={4096,0,0,0,0,0,0,0,0,0,0};

void Levin_32(Word16 Rh[], Word16 Rl[], Word16 A[])

{

Word16 i, j;

Word16 hi, lo;

Word16 Kh, Kl;              	/* reflexion coefficient; hi and lo          */

Word16 alp_h, alp_l, alp_e;	/* Prediction gain; hi lo and exponant       */

Word16 Ah[pp+1], Al[pp+1];	/* LPC coef. in double prec.                 */

Word16 Anh[pp+1], Anl[pp+1];	/* LPC coef.for next iterat. in double prec. */

Word32 t0, t1, t2;		/* temporary variable                        */

/* K = A[1] = -R[1] / R[0] */

t1  = L_comp(Rh[1], Rl[1]);			/* R[1] in Q30      */

t2  = L_abs(t1);				/* abs R[1]         */

t0  = div_32(t2, Rh[0], Rl[0]);		/* R[1]/R[0] in Q30 */

if(t1 > 0) t0= L_negate(t0);		/* -R[1]/R[0]       */

L_extract(t0, &Kh, &Kl);			/* K in DPF         */

t0 = L_shr(t0,(Word16)4);			/* A[1] in Q26      */

L_extract(t0, &Ah[1], &Al[1]);		/* A[1] in DPF      */

/*  Alpha = R[0] * (1-K**2) */

t0 = mpy_32(Kh ,Kl, Kh, Kl);		/* K*K      in Q30 */

t0 = L_abs(t0);					/* Some case <0 !! */

t0 = L_sub( (Word32)0x3fffffff, t0 );	/* 1 - K*K  in Q30 */

L_extract(t0, &hi, &lo);			/* DPF format      */

t0 = mpy_32(Rh[0] ,Rl[0], hi, lo);	/* Alpha in Q30    */

/* Normalize Alpha */

t0 = norm_v(t0, (Word16)12, &i);

t0 = L_shr(t0, (Word16)1);

L_extract(t0, &alp_h, &alp_l);		/* DPF format    */

alp_e = i-1;					/* t0 was in Q30 */

/*--------------------------------------*

* ITERATIONS  I=2 to pp                *

*--------------------------------------*/

for(i= 2; i<=pp; i++)

{

/* t0 = SUM ( R[j]*A[i-j] ,j=1,i-1 ) +  R[i] */

t0 = 0;

for(j=1; j<i; j++)

t0 = L_add(t0, mpy_32(Rh[j], Rl[j], Ah[i-j], Al[i-j]));

t0 = L_shl(t0,(Word16)4);			/* result in Q26->convert to Q30 */

/* No overflow possible          */

t1 = L_comp(Rh[i],Rl[i]);

t0 = L_add(t0, t1);				/* add R[i] in Q30               */

/* K = -t0 / Alpha */

t1 = L_abs(t0);

t2 = div_32(t1, alp_h, alp_l);		/* abs(t0)/Alpha                 */

if(t0 > 0) t2= L_negate(t2);		/* K =-t0/Alpha                  */

t2 = L_shl(t2, alp_e);			/* denormalize; compare to Alpha */

L_extract(t2, &Kh, &Kl);			/* K in DPF                      */

/* Test for unstable filter, if unstable keep old A(z) */

if ( abs_s(Kh) > 32750)

{

for(j=0; j<=pp; j++) A[j] = old_A[j];

return;

}

/*------------------------------------------*

*  Compute new LPC coeff. -> An[i]         *

*  An[j]= A[j] + K*A[i-j]     , j=1 to i-1 *

*  An[i]= K                                *

*------------------------------------------*/

for(j=1; j<i; j++)

{

t0 = mpy_32(Kh, Kl, Ah[i-j], Al[i-j]);

t0 = add_sh(t0, Ah[j], (Word16)15);

t0 = add_sh(t0, Al[j], (Word16)0);

L_extract(t0, &Anh[j], &Anl[j]);

}

t2 = L_shr(t2, (Word16)4);		/* t2 = K in Q30->convert to Q26 */

L_extract(t2, &Anh[i], &Anl[i]);	/* An[i] in Q26                  */

/*  Alpha = Alpha * (1-K**2) */

t0 = mpy_32(Kh ,Kl, Kh, Kl);		/* K*K      in Q30 */

t0 = L_abs(t0);				/* Some case <0 !! */

t0 = L_sub( (Word32)0x3fffffff, t0 );	/* 1 - K*K  in Q30 */

L_extract(t0, &hi, &lo);			/* DPF format      */

t0 = mpy_32(alp_h , alp_l, hi, lo);	/* Alpha in Q30    */

/* Normalize Alpha */

t0 = norm_v(t0, (Word16)12, &j);

t0 = L_shr(t0, (Word16)1);

L_extract(t0, &alp_h, &alp_l);		/* DPF format    */

alp_e += j-1;					/* t0 was in Q30 */

/* A[j] = An[j]   Note: can be done with pointers */

for(j=1; j<=i; j++)

{

Ah[j] =Anh[j];

Al[j] =Anl[j];

}

}

/* Troncate A[i] in Q26 to Q12 with rounding */

A[0] = 4096;

for(i=1; i<=pp; i++)

{

t0   = L_comp(Ah[i], Al[i]);

t0   = add_sh(t0, (Word16)1, (Word16)13);	/* rounding */

old_A[i] = A[i] = store_hi(t0,(Word16)2);

}

return;

}

/**************************************************************************

*

*	ROUTINE				:	Lpc_Gain

*

*	DESCRIPTION			:	Compute energy of impulse response of 1/A(z)

*							on 60 points

*

**************************************************************************

*

*	USAGE				:	Lpc_Gain(buffer_in)

*							(Routine_Name(input1))

*

*	INPUT ARGUMENT(S)		:

*

*	INPUT1			:	- Description : LPC coefficients

*						- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:	None

*

*	RETURNED VALUE		:	- Description : Energy of impulse response of 1/A(z)

*							- Format : Word32 - Q20

*

**************************************************************************/

Word32 Lpc_Gain(Word16 a[])

{

Word16 i;

Word32 ener;

Word16 h[llg];

/* Compute the impulse response of A(z) */

h[0] = 1024;				/* 1.0 in Q10 */

for(i=1; i<llg; i++) h[i]=0;

Syn_Filt(a, h, h, llg, &h[1], (Word16)0);

/* Compute the energy of the impulse response */

ener = 0;

for(i=0; i<llg; i++)

ener = L_mac0(ener, h[i], h[i]);

return(ener);

}

/**************************************************************************

*

*	ROUTINE				:	Lsp_Az

*

*	DESCRIPTION			:	Compute the LPC coefficients

*							from the LSPs in the cosine domain

*							(order=10)

*

**************************************************************************

*

*	USAGE				:	Lsp_Az(buffer_in,buffer_out)

*							(Routine_Name(input1,output1))

*

*	INPUT ARGUMENT(S)		:

*

*		INPUT1			:	- Description : Line spectral pairs in the

*								          cosine domain

*						- Format : Word16 - Q15

*

*	OUTPUT ARGUMENT(S)		:

*

*	OUTPUT1			:	- Description : Predictor coefficients

*							- Format : Word16 - Q12

*

*	RETURNED VALUE		:	None

*

**************************************************************************/

void Lsp_Az(Word16 lsp[], Word16 a[])

{

Word16 i, j;

Word32 f1[6], f2[6];

Word32 t0;

Get_Lsp_Pol(&lsp[0],f1);

Get_Lsp_Pol(&lsp[1],f2);

for (i = 5; i > 0; i--)

{

f1[i] = L_add(f1[i], f1[i-1]);			/* f1[i] += f1[i-1]; */

f2[i] = L_sub(f2[i], f2[i-1]);			/* f2[i] -= f2[i-1]; */

}

a[0] = 4096;

for (i = 1, j = 10; i <= 5; i++, j--)

{

t0   = L_add(f1[i], f2[i]);			/* f1[i] + f2[i] */

a[i] = extract_l( L_shr_r(t0,(Word16)13) );	/*from Q24 to Q12 and * 0.5*/

t0   = L_sub(f1[i], f2[i]);			/* f1[i] - f2[i] */

a[j] = extract_l( L_shr_r(t0,(Word16)13) );	/*from Q24 to Q12 and * 0.5*/

}

return;

}

/**************************************************************************

*

*	ROUTINE				:	Pond_Ai

*

*	DESCRIPTION			:	Compute spectral expansion (a_exp[]) of LPC

*							coefficients (a[]) using spectral expansion

*							factors (fac[]) with the LPC order fixed to 10

*

**************************************************************************

*

*	USAGE				:	Pond_Ai(buffer_in1,buffer_in2,buffer_out)

*							(Routine_Name(input1,input2,output1))

*

*	INPUT ARGUMENT(S)		:

*

*	INPUT1			:	- Description : LPC coefficients

*						- Format : Word16

*

*	INPUT2			:	- Description : Spectral expansion factors

*						- Format : Word16 - Q15

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : Spectral expanded LPC coefficients

*							- Format : Word16

*

*	RETURNED VALUE		:	None

*

*	COMMENTS			:	- a_exp[i] = a[i] * fac[i-1]	i=1,10

*

**************************************************************************/

void Pond_Ai(Word16 a[], Word16 fac[], Word16 a_exp[])

{

Word16 i;

a_exp[0] = a[0];

for(i=1; i<=pp; i++)

a_exp[i] = round( L_mult(a[i], fac[i-1]) );

return;

}

/**************************************************************************

*

*	ROUTINE				:	Residu

*

*	DESCRIPTION			:	Compute the LPC residual  by filtering the input

*							speech through A(z)

*

**************************************************************************

*

*	USAGE				:	Residu(buffer_in1,buffer_in2,buffer_out,lg)

*							(Routine_Name(input1,input2,output1,input3))

*

*	INPUT ARGUMENT(S)		:

*

*	INPUT1			:	- Description : Prediction coefficients

*						- Format : Word16 - Q12

*

*	INPUT2			:	- Description : Input speech - values of buffer_in2[]

*							          from -p to -1 are needed

*						- Format : Word16

*

*	INPUT3			:	- Description : Size of filtering

*						- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : Residual signal

*							- Format : Word16

*

*	RETURNED VALUE		:	None

*

**************************************************************************/

void Residu(Word16 a[], Word16 x[], Word16 y[], Word16 lg)

{

Word16 i, j;

Word32 s;

for (i = 0; i < lg; i++)

{

s = Load_sh(x[i], (Word16)12);

for (j = 1; j <= pp; j++)

s = L_mac0(s, a[j], x[i-j]);

s = add_sh(s, (Word16)1, (Word16)11);		/* Rounding */

s = L_shl(s, (Word16)4);				/* Saturation */

y[i] = extract_h(s);

}

return;

}

/**************************************************************************

*

*	ROUTINE				:	Syn_Filt

*

*	DESCRIPTION			:	Perform the synthesis filtering 1/A(z)

*

**************************************************************************

*

*	USAGE				:	Syn_Filt(buffer_in1,buffer_in2,buffer_out,

*							lg,buffer,flag)

*							(Routine_Name(input1,input2,output1,

*							input3,arg4,input5))

*

*	INPUT ARGUMENT(S)		:

*

*	INPUT1			:	- Description : Prediction coefficients

*						- Format : Word16 - Q12

*

*	INPUT2			:	- Description : Input signal

*						- Format : Word16

*

*	INPUT3			:	- Description : Size of filtering

*						- Format : Word16

*

*	ARG4				:	- Description : Memory associated with this filtering

*						- Format : Word16

*

*		INPUT5			:	- Description :	- flag = 0  -> no update of memory

*									- flag = 1  -> update of memory

*							- Format : Word16

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : Output signal

*							- Format : Word16

*

*	RETURNED VALUE		:	None

*

**************************************************************************/

void Syn_Filt(Word16 a[], Word16 x[], Word16 y[], Word16 lg, Word16 mem[],

Word16 update)

{

Word16 i, j;

Word32 s;

Word16 tmp[80];	/* This is usually done by memory allocation (lg+pp) */

Word16 *yy;

/* Copy mem[] to yy[] */

yy = tmp;

for(i=0; i<pp; i++)

*yy++ = mem[i];

/* Do the filtering. */

for (i = 0; i < lg; i++)

{

s = Load_sh(x[i], (Word16)12);				/* a[] is in Q12 */

for (j = 1; j <= pp; j++)

s = L_msu0(s, a[j], yy[-j]);

s     = add_sh(s, (Word16)1, (Word16)11);		/* Rounding */

*yy++ = extract_h( L_shl(s, (Word16)4) );

}

for(i=0; i<lg; i++) y[i] = tmp[i+pp];

/* Update of memory if update==1 */

if(update != 0)

for (i = 0; i < pp; i++) mem[i] = y[lg-pp+i];

return;

}

## A.5	Library of Basic and Mathematic Functions

### A.5.1	Basic Functions

/************************************************************************

*

*	FILENAME		:	fbas_tet.c

*

*	DESCRIPTION		:	Library of basic functions used in the TETRA speech

*					codec, other than accepted operators

*

************************************************************************

*

*	FUNCTIONS		:	- add_sh()

*					- add_sh16()

*					- bin2int()

*					- int2bin()

*					- Load_sh()

*					- Load_sh16()

*					- norm_v()

*					- store _hi()

*					- sub_sh()

*					- sub_sh16()

*

************************************************************************

*

*	INCLUDED FILES	:	source.h

*

************************************************************************/

#include "source.h"

static Word16 POW2[16] = { -1, -2, -4, -8, -16, -32, -64, -128, -256, -512,

-1024, -2048, -4096, -8192, -16384, -32768};

/************************************************************************

*

*	Function Name : add_sh

*

*	Purpose :

*

*		Add var1 with a left shift(0-15) to L_var2.

*		Control saturation and set overflow flag

*

*	Complexity Weight : 1

*

*	Inputs :

*

*		L_var2

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var2 <= 0x7fff ffff.

*

*		var1

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var1 <= 0x0000 7fff.

*

*		shift

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0 <= shift <= 15.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_var_out

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var_out <= 0x7fff ffff.

*

************************************************************************/

Word32 add_sh(Word32 L_var2, Word16 var1, Word16 shift)

{

return( L_msu0(L_var2, var1, POW2[shift]));

}

/************************************************************************

*

*	Function Name : add_sh16

*

*	Purpose :

*

*		Add var1 with a left shift of 16 to L_var2.

*		Control saturation and set overflow flag

*

*	Complexity Weight : 1

*

*	Inputs :

*

*		L_var2

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var2 <= 0x7fff ffff.

*

*		var1

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var1 <= 0x0000 7fff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_var_out

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var_out <= 0x7fff ffff.

*

************************************************************************/

Word32 add_sh16(Word32 L_var2, Word16 var1)

{

return( L_msu(L_var2, var1, (Word16)-32768));

}

/************************************************************************

*

*	Function Name : bin2int

*

*	Purpose :

*

*		Read "no_of_bits" bits from the array bitstream[] and convert to integer

*

*	Inputs :

*

*		no_of_bits

*			16 bit

*

*		*bitstream

*			16 bit

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		value

*			32 bit

*

************************************************************************/

#define BIT_1  1

Word16 bin2int(Word16 no_of_bits, Word16 *bitstream)

{

Word16 value, i, bit;

value = 0;

for (i = 0; i < no_of_bits; i++)

{

value = shl( value,(Word16)1 );

bit = *bitstream++;

if (bit == BIT_1)  value += 1;

}

return(value);

}

/************************************************************************

*

*	Function Name : int2bin

*

*	Purpose :

*

*		Convert integer to binary and write the bits to the array bitstream []

*

*	Inputs :

*

*		no_of_bits

*			16 bit

*

*		*bitstream

*			16 bit

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		value

*			32 bit

*

************************************************************************/

#define BIT_0     0

#define BIT_1     1

#define MASK      1

void int2bin(Word16 value, Word16 no_of_bits, Word16 *bitstream)

{

Word16 *pt_bitstream, i, bit;

pt_bitstream = bitstream + no_of_bits;

for (i = 0; i < no_of_bits; i++)

{

bit = value & MASK;

if (bit == 0)

*--pt_bitstream = BIT_0;

else

*--pt_bitstream = BIT_1;

value = shr( value,(Word16)1 );

}

}

/************************************************************************

*

*	Function Name : Load_sh

*

*	Purpose :

*

*		Load the 16 bit var1 left shift(0-15) into the 32 bit output.

*		MS bits of the output are sign extended.

*

*	Complexity Weight : 1

*

*	Inputs :

*

*		var1

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var1 <= 0x0000 7fff.

*

*		shift

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0 <= var1 <= 15.

*

*	Outputs :

*

*		none.

*

*	Returned Value :

*

*		L_var_out

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var_out <= 0x7fff ffff.

*

************************************************************************/

Word32 Load_sh(Word16 var1, Word16 shift)

{

return( L_msu0( (Word32)0, var1, POW2[shift]));

}

/************************************************************************

*

*	Function Name : Load_sh16

*

*	Purpose :

*

*		Load the 16 bit var1 with a left shift of 16 into the 32 bit output.

*

*	Complexity Weight : 1

*

*	Inputs :

*

*		var1

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var1 <= 0x0000 7fff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_var_out

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var_out <= 0x7fff ffff.

*

************************************************************************/

Word32 Load_sh16(Word16 var1)

{

return( L_msu( (Word32)0, var1, (Word16)-32768));

}

/************************************************************************

*

*	Function Name : norm_v

*

*	Purpose :

*

*		Variable normalisation of a 32 bit integer (L_var3).

*		var1 gives the maximum number of left shift to be done

*		*var2 returns the actual number of left shift

*

*	Complexity Weight : 37

*

*	Inputs :

*

*		L_var3

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var3 <= 0x7fff ffff.

*

*		var1

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0 <= var1 <= 15.

*

*	Outputs :

*

*		*var2

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0 <= *var2 <= 15.

*

*	Returned Value :

*

*		L_var_out

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var_out <= 0x7fff ffff.

*

************************************************************************/

Word32 norm_v(Word32 L_var3, Word16 var1, Word16 *var2)

{

Word16 shift;

shift = norm_l(L_var3);

if(sub(shift, var1) > 0) shift = var1;

*var2 = shift;

return(L_shl(L_var3, shift));

}

/************************************************************************

*

*	Function Name : store_hi

*

*	Purpose :

*

*		Store high part of a L_var1 with a left shift of var2.

*

*	Complexity Weight : 3

*

*	Inputs :

*

*		L_var1

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var1 <= 0x7fff ffff.

*

*		var2

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0 <= var2 <= 7.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		var_out

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var_out <= 0x0000 7fff.

*

************************************************************************/

Word16 store_hi(Word32 L_var1, Word16 var2)

{

static Word16 SHR[8]={16, 15, 14, 13, 12, 11, 10, 9};

return(extract_l( L_shr(L_var1, SHR[var2])));

}

/************************************************************************

*

*	Function Name : sub_sh

*

*	Purpose :

*

*		Subtract var1 with a left shift(0-15) to L_var2.

*		Control saturation and set overflow_flag.

*

*	Complexity Weight : 1

*

*	Inputs :

*

*		L_var2

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var2 <= 0x7fff ffff.

*

*		var1

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var1 <= 0x0000 7fff.

*

*		shift

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0 <= shift <= 15.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_var_out

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var_out <= 0x7fff ffff.

*

************************************************************************/

Word32 sub_sh(Word32 L_var2, Word16 var1, Word16 shift)

{

return( L_mac0(L_var2, var1, POW2[shift]));

}

/************************************************************************

*

*	Function Name : sub_sh16

*

*	Purpose :

*

*		Subtract var1 with a left shift of 16 to L_var2.

*		Control saturation and set overflow flag.

*

*	Complexity Weight : 1

*

*	Inputs :

*

*		L_var2

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var2 <= 0x7fff ffff.

*

*		var1

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var1 <= 0x0000 7fff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_var_out

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var_out <= 0x7fff ffff.

************************************************************************/

Word32 sub_sh16(Word32 L_var2, Word16 var1)

{

return( L_mac(L_var2, var1, (Word16)-32768));

}

### A.5.2	Extended Precision Functions

/************************************************************************

*

*	FILENAME		:	fexp_tet.c

*

*	DESCRIPTION		:	Library of special functions for operations in

*					extended precision used in the TETRA speech codec

*

************************************************************************

*

*	FUNCTIONS		:	- L_comp()

*					- L_extract()

*					- mpy_mix()

*					- mpy_32()

*					- div_32()

*

************************************************************************

*

*	INCLUDED FILES	:	source.h

*

************************************************************************

*

*	COMMENTS		:	This subsection contains operations in double precision.

*					These operations are non standard double precision

*					operations.

*

*					They are used where single precision is not enough but the

*					full 32 bits precision is not necessary. For example, the

*					function div_32() has a 24 bits precision.

*

*					The double precision numbers use a special representation :

*

*					L_32 = hi<<15 + lo

*

*					L_32 is a 32 bit integer with b30 == b31.

*					hi and lo are 16 bit signed integers.

*					As the low part also contains the sign, this allows fast

*					multiplication.

*

*					0xc000 0000 <= L_32 <= 0x3fff ffff.

*

*					In general, DPF is used to specify this special

*					format.

*

************************************************************************/

#include "source.h"

/************************************************************************

*

*	Function Name : L_comp

*

*	Purpose :

*

*		Compose from two 16 bit DPF a normal 32 bit integer.

*		L_32 = hi<<15 + lo

*

*	Complexity Weight : 2

*

*	Inputs :

*

*		hi

*			msb

*

*		lo

*			lsb (with sign)

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_32

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0xc000 0000 <= L_32 <= 0x3fff ffff.

*

************************************************************************/

Word32 L_comp(Word16 hi, Word16 lo)

{

return(add_sh( Load_sh( lo,(Word16)0 ), hi, (Word16)15 ));

}

/************************************************************************

*

*	Function Name : L_extract

*

*	Purpose :

*

*		Extract from a 31 bit integer two 16 bit DPF.

*

*	Complexity Weight : 5

*

*	Inputs :

*

*		L_32

*			32 bit long signed integer (Word32) with b30 == b31

*			whose value falls in the range : 0xc000 0000 <= L_32 <= 0x3fff ffff.

*

*	Outputs :

*

*		hi

*			b15 to b30 of L_32

*

*		lo

*			L_32 - hi<<15

*

*	Returned Value :

*

*		none

*

************************************************************************/

void L_extract(Word32 L_32, Word16 *hi, Word16 *lo)

{

*hi  = extract_h( L_shl( L_32,(Word16)1 ) );

*lo  = extract_l( sub_sh( L_32, *hi, (Word16)15 ) );

return;

}

/************************************************************************

*

*	Function Name : mpy_mix

*

*	Purpose :

*

*		Multiply a 16 bit integer by a 32 bit (DPF).

*		The result is divided by 2**16

*		L_32 = hi1*lo2 + (lo1*lo2)>>15

*

*	Complexity Weight : 4

*

*	Inputs :

*

*		hi1

*			hi part of 32 bit number

*

*		lo1

*			lo part of 32 bit number

*

*		lo2

*			16 bit number

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_var_out

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x0000 0000 <= L_var_out <= 0x7fff ffff.

*

************************************************************************/

Word32 mpy_mix(Word16 hi1, Word16 lo1, Word16 lo2)

{

Word16 p1;

Word32 L_32;

p1   = extract_h(L_mult0(lo1, lo2));

L_32 = L_mult0(hi1,lo2 );

return(add_sh( L_32, p1, (Word16)1 ));

}

/************************************************************************

*

*	Function Name : mpy_32

*

*	Purpose :

*

*		Multiply two 32 bit integers (DPF). The result is divided by 2**32

*		L_32 = hi1*hi2 + (hi1*lo2)>>15 + (lo1*hi2)>>15)

*

*	Complexity Weight : 7

*

*	Inputs :

*

*		hi1

*			hi part of first number

*

*		lo1

*			lo part of first number

*

*		hi2

*			hi part of second number

*

*		lo2

*			lo part of second number

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_var_out

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x0000 0000 <= L_var_out <= 0x7fff ffff.

*

*************************************************************************/

Word32 mpy_32(Word16 hi1, Word16 lo1, Word16 hi2, Word16 lo2)

{

Word16 p1, p2;

Word32 L_32;

p1   = extract_h(L_mult0(hi1, lo2));

p2   = extract_h(L_mult0(lo1, hi2));

L_32 = L_mult0(hi1, hi2);

L_32 = add_sh( L_32, p1, (Word16)1 );

return(add_sh( L_32, p2, (Word16)1 ));

}

/************************************************************************

*

*	Function Name : div_32

*

*	Purpose :

*

*		Fractionnal integer division of two 32 bit numbers.

*		L_num / L_denom

*		L_num and L_denom must be positive and L_num < L_denom

*		L_denom = denom_hi<<15 + denom_lo

*		denom_hi is a normalized number

*		The result is in Q30

*

*	Complexity Weight : 52

*

*	Inputs :

*

*		L_num

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x0000 0000 <= L_num <= L_denom.

*

*		(L_denom = denom_hi<<15 + denom_lo)

*

*		denom_hi

*			16 bit normalized integer whose value falls in the

*			range : 0x4000000 < hi < 0x7fff ffff.

*

*		denom_lo

*			16 bit positive integer whose value falls in the

*			range : 0 < lo < 0x7fff ffff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_div

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x0000 0000 <= L_div <= 0x3fff ffff.

*			L_div is a Q30 value (point between b30 and b29)

*

*	Algorithm :

*

*		 - find = 1/L_denom

*			First approximation: approx = 1 / denom_hi

*			1/L_denom = approx * (2.0 - L_denom * approx )

*		-  result = L_num * (1/L_denom)

*

************************************************************************/

Word32 div_32(Word32 L_num, Word16 denom_hi, Word16 denom_lo)

{

Word16 approx, hi, lo, n_hi, n_lo;

Word32 t0;

/* First approximation: 1 / L_denom = 1/denom_hi */

approx = div_s( (Word16)0x3fff, denom_hi);	/* result in Q15 */

/* 1/L_denom = approx * (2.0 - L_denom * approx) */

t0 = mpy_mix(denom_hi, denom_lo, approx);	/* result in Q29 */

t0 = L_sub( (Word32)0x40000000, t0);		/* result in Q29 */

L_extract(t0, &hi, &lo);

t0 = mpy_mix(hi, lo, approx);			/* = 1/L_denom in Q28 */

/* L_num * (1/L_denom) */

L_extract(t0, &hi, &lo);

L_extract(L_num, &n_hi, &n_lo);

t0 = mpy_32(n_hi, n_lo, hi, lo);

return( L_shl( t0,(Word16)2) );			/* From Q28 to Q30 */

}

### A.5.3	Mathematic Functions

/************************************************************************

*

*	FILENAME		:	fmat_tet.c

*

*	DESCRIPTION		:	Library of mathematic functions used in the TETRA codec

*

************************************************************************

*

*	FUNCTIONS		:	- inv_sqrt()

*					- Log2()

*					- pow2()

*

************************************************************************

*

*	INCLUDED FILES	:	source.h

*

*					inv_sqrt.tab

*					log2.tab

*					pow2.tab

*

************************************************************************/

#include "source.h"

#include "inv_sqrt.tab"         /* Table for inv_sqrt() */

#include "log2.tab"             /* Table for Log2() */

#include "pow2.tab"             /* Table for pow2() */

/************************************************************************

*

*	Function Name : inv_sqrt

*

*	Purpose :

*

*		Compute 1/sqrt(L_x).

*		L_x is positive. The result is in Q30.

*

*	Complexity Weight : 56

*

*	Inputs :

*

*		L_x

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x0000 0000 <= L_x <= 0x7fff ffff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_y

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x0000 0000 <= L_y <= 0x3fff ffff.

*			L_y is a Q30 value (point between b30 and b29)

*

*	Algorithm :

*

*		The function 1/sqrt(L_x) is approximated by a table (tab_inv_sqrt)

*		and linear interpolation :

*

*			1 - Normalization of L_x

*			2 - If (30-exponant) is even then shift right once

*			3 - exponant = (30-exponant)/2  +1

*			4 - i = bit25-b31 of L_x,    16 <= i <= 63  ->because of normalization

*			5 - a = bit10-b24

*			6 - i -=16

*			7 - L_y = tab_inv_sqrt[i]<<16 - (tab_inv_sqrt[i] - tab_inv_sqrt[i+1]) * a * 2

*			8 - L_y >>= exponant

*

************************************************************************/

Word32 inv_sqrt(Word32 L_x)

{

Word16 exp, i, a, tmp;

Word32 L_y;

if( L_x <= (Word32)0) return ( (Word32)0x3fffffff);

exp = norm_l(L_x);

L_x = L_shl(L_x, exp );		/* L_x is normalized */

exp = sub( (Word16)30, exp );

if( (exp & 1) == 0 )			/* If exponant even -> shift right */

L_x = L_shr( L_x, (Word16)1 );

exp = shr( exp, (Word16)1 );

exp = add( exp, (Word16)1 );

L_x = L_shr( L_x, (Word16)9 );

i   = extract_h(L_x);			/* Extract b25-b31 */

L_x = L_shr( L_x, (Word16)1 );

a   = extract_l(L_x);			/* Extract b10-b24 */

a   = a & (Word16)0x7fff;

i   = sub( i, (Word16)16 );

L_y = L_deposit_h(tab_inv_sqrt[i]);	/* tab_inv_sqrt[i] << 16 */

tmp = sub(tab_inv_sqrt[i], tab_inv_sqrt[i+1]);

/* tab_inv_sqrt[i] - tab_inv_sqrt[i+1])*/

L_y = L_msu(L_y, tmp, a);			/* L_y -=  tmp*a*2 */

L_y = L_shr(L_y, exp);		/* denormalization */

return(L_y);

}

/************************************************************************

*

*	Function Name : Log2

*

*	Purpose :

*

*		Compute Log2(L_x).

*		L_x is positive.

*

*	Complexity Weight : 48

*

*	Inputs :

*

*		L_x

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x0000 0000 <= L_x <= 0x7fff ffff.

*

*	Outputs :

*

*		exponant

*			Integer part of Log2()

*			16 bit  signed integer (Word16) whose value falls in the

*			range :   0 <= exponant <= 30

*

*		fraction

*			Fractional part of Log2()

*			16 bit signed integer (Word16) whose value falls in the

*			range : 0x0000 0000 <= fraction <= 0x7fff.

*			It's a Q15 value (point between b15 and b16).

*

*	Returned Value :

*

*		none

*

*	Algorithm :

*

*		The function Log2(L_x) is approximated by a table (tab_log2)

*		and linear interpolation :

*

*			1 - Normalization of L_x

*			2 - exponant = 30-exponant

*			3 - i = bit25-b31 of L_x,    32 <= i <= 63  ->because of normalization

*			4 - a = bit10-b24

*			5 - i -=32

*			6 - fraction = tab_log2[i]<<16 - (tab_log2[i] - tab_log2[i+1]) * a * 2

*

************************************************************************/

void Log2(Word32 L_x, Word16 *exponant, Word16 *fraction)

{

Word16 exp, i, a, tmp;

Word32 L_y;

if( L_x <= (Word32)0 )

{

*exponant = 0;

*fraction = 0;

return;

}

exp = norm_l(L_x);

L_x = L_shl(L_x, exp );			/* L_x is normalized */

*exponant = sub( (Word16)30, exp );

L_x = L_shr( L_x, (Word16)9 );

i   = extract_h(L_x);				/* Extract b25-b31 */

L_x = L_shr( L_x, (Word16)1 );

a   = extract_l(L_x);				/* Extract b10-b24 of fraction */

a   = a & (Word16)0x7fff;

i   = sub( i, (Word16)32 );

L_y = L_deposit_h(tab_log2[i]);		/* tab_log2[i] << 16 */

tmp = sub(tab_log2[i], tab_log2[i+1]);	/* tab_log2[i] - tab_log2[i+1] */

L_y = L_msu(L_y, tmp, a);			/* L_y -= tmp*a*2 */

*fraction = extract_h( L_y);

return;

}

/************************************************************************

*

*	Function Name : pow2

*

*	Purpose :

*

*		L_x = pow(2.0, exponant.fraction).

*

*	Complexity Weight : 17

*

*	Inputs :

*

*		exponant

*			Integer part

*			16 bit  signed integer (Word16) whose value falls in the

*			range :   0 <= exponant <= 30

*

*		fraction

*			Fractional part

*			16 bit signed integer (Word16) whose value falls in the

*			range : 0x0000 0000 <= fraction <= 0x7fff.

*			It's a Q15 value (point between b15 and b16).

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_x

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x0000 0000 <= L_x <= 0x7fff ffff.

*

*	Algorithm :

*

*		The function pow2(L_x) is approximated by a table (tab_pow2)

*		and linear interpolation :

*

*			1 - i = bit11-b16 of fraction,   0 <= i <= 31

*			2 - a = bit0-b10  of fraction

*			3 - L_x = tab_pow2[i]<<16 - (tab_pow2[i] - tab_pow2[i+1]) * a * 2

*			4 - L_x = L_x >> (30-exponant)     (with rounding)

*

************************************************************************/

Word32 pow2(Word16 exponant, Word16 fraction)

{

Word16 exp, i, a, tmp;

Word32 L_x;

L_x = L_deposit_l(fraction);

L_x = L_shl( L_x, (Word16)6 );

i   = extract_h(L_x);				/* Extract b10-b16 of fraction */

L_x = L_shr( L_x, (Word16)1 );

a   = extract_l(L_x);				/* Extract b0-b9   of fraction */

a   = a & (Word16)0x7fff;

L_x = L_deposit_h(tab_pow2[i]);		/* tab_pow2[i] << 16 */

tmp = sub(tab_pow2[i], tab_pow2[i+1]);	/* tab_pow2[i] - tab_pow2[i+1] */

L_x = L_msu(L_x, tmp, a);			/* L_x -= tmp*a*2  */

exp = sub( (Word16)30, exponant );

L_x = L_shr_r(L_x, exp);

return(L_x);

}

## A.6	Tables and Constants for Source Coding

/************************************************************************

*

*	FILENAME		:	source.h

*

*	DESCRIPTION		:	DEFINITIONS

*

*					FUNCTION PROTOTYPES for the TETRA

*					speech source coder and decoder

*

*					OPERATOR PROTOTYPES for TETRA codec

*

************************************************************************/

/************************************************************************

*

*      DEFINITIONS

*

************************************************************************/

#ifndef TYPEDEF_H

#define TYPEDEF_H

typedef short Word16;

typedef long  Word32;

typedef int   Flag;

#endif

/************************************************************************

*

*	DESCRIPTION		:	FUNCTION PROTOTYPES for the TETRA

*					speech source coder and decoder

*

************************************************************************/

#ifndef TETRA_H

#define TETRA_H

/* Basic functions */

Word32 add_sh(Word32 L_var, Word16 var1, Word16 shift);

Word32 add_sh16(Word32 L_var, Word16 var1);

Word16 bin2int(Word16 no_of_bits, Word16 *bitstream);

void   int2bin(Word16 value, Word16 no_of_bits, Word16 *bitstream);

Word32 Load_sh(Word16 var1, Word16 shift);

Word32 Load_sh16(Word16 var1);

Word32 norm_v(Word32 L_var3, Word16 var1, Word16 *var2);

Word16 store_hi(Word32 L_var1, Word16 var2);

Word32 sub_sh(Word32 L_var, Word16 var1, Word16 shift);

Word32 sub_sh16(Word32 L_var, Word16 var1);

/* Extended precision functions */

Word32 div_32(Word32 L_num, Word16 hi, Word16 lo);

Word32 L_comp(Word16 hi, Word16 lo);

void   L_extract(Word32 L_32, Word16 *hi, Word16 *lo);

Word32 mpy_mix(Word16 hi1, Word16 lo1, Word16 lo2);

Word32 mpy_32(Word16 hi1, Word16 lo1, Word16 hi2, Word16 lo2);

/* Mathematic functions  */

Word32 inv_sqrt(Word32 L_x);

void   Log2(Word32 L_x, Word16 *exponant, Word16 *fraction);

Word32 pow2(Word16 exponant, Word16 fraction);

/* General signal processing */

void   Autocorr(Word16 x[], Word16 p, Word16 r_h[], Word16 r_l[]);

void   Az_Lsp(Word16 a[], Word16 lsp[], Word16 old_lsp[]);

void   Back_Fil(Word16 x[], Word16 h[], Word16 y[], Word16 L);

Word16 Chebps(Word16 x, Word16 f[], Word16 n);

void   Convolve(Word16 x[], Word16 h[], Word16 y[], Word16 L);

void   Fac_Pond(Word16 gamma, Word16 fac[]);

void   Get_Lsp_Pol(Word16 *lsp, Word32 *f);

void   Int_Lpc4(Word16 lsp_old[], Word16 lsp_new[], Word16 a_4[]);

void   Lag_Window(Word16 p, Word16 r_h[], Word16 r_l[]);

void   Levin_32(Word16 Rh[], Word16 Rl[], Word16 A[]);

Word32 Lpc_Gain(Word16 a[]);

void   Lsp_Az(Word16 lsp[], Word16 a[]);

void   Pond_Ai(Word16 a[], Word16 fac[], Word16 a_exp[]);

void   Residu(Word16 a[], Word16 x[], Word16 y[], Word16 lg);

void   Syn_Filt(Word16 a[], Word16 x[], Word16 y[], Word16 lg, Word16 mem[],

Word16 update);

/* Specific coder functions */

void   Init_Coder_Tetra(void);

void   Coder_Tetra(Word16 ana[], Word16 synth[]);

void   Cal_Rr2(Word16 h[], Word16 *rr);

void   Clsp_334(Word16 *lsp, Word16 *lsp_q, Word16 *indice);

Word16 D4i60_16(Word16 dn[], Word16 f[], Word16 h[], Word16 rr[][32],

Word16 cod[], Word16 y[], Word16 *sign, Word16 *shift_code);

Word16 Ener_Qua(Word16 A[], Word16 prd_lt[], Word16 code[], Word16 L_subfr,

Word16 *gain_pit, Word16 *gain_cod);

Word16 G_Code(Word16 xn2[], Word16 y2[], Word16 L_subfr);

Word16 G_Pitch(Word16 xn[], Word16 y1[], Word16 L_subfr);

void   Init_Pre_Process(void);

Word16 Lag_Max(Word16 signal[], Word16 sig_dec[], Word16 L_frame,

Word16 lag_max, Word16 lag_min, Word16 *cor_max);

Word16 Pitch_Fr(Word16 exc[], Word16 xn[], Word16 h[], Word16 L_subfr,

Word16 t0_min, Word16 t0_max, Word16 i_subfr,

Word16 *pit_frac);

Word16 Pitch_Ol_Dec(Word16 signal[], Word16 L_frame);

void   Pred_Lt(Word16 exc[], Word16 T0, Word16 frac, Word16 L_subfr);

void   Pre_Process(Word16 signal[], Word16 lg);

void   Prm2bits_Tetra(Word16 prm[], Word16 bits[]);

/* Specific decoder functions */

void   Init_Decod_Tetra(void);

void   Decod_Tetra(Word16 parm[], Word16 synth[]);

void   Bits2prm_Tetra(Word16 bits[], Word16 prm[]);

Word16 Dec_Ener(Word16 index, Word16 bfi, Word16 A[], Word16 prd_lt[],

Word16 code[], Word16 L_subfr, Word16 *gain_pit, Word16 *gain_cod);

void   D_D4i60(Word16 index,Word16 sign,Word16 shift, Word16 F[],

Word16 cod[]);

void   D_Lsp334(Word16 indice[], Word16 lsp[], Word16 old_lsp[]);

void   Post_Process(Word16 signal[], Word16 lg);

#endif

/************************************************************************

*

*	DESCRIPTION		:     OPERATOR PROTOTYPES for TETRA codec

*

************************************************************************/

#ifndef TETRA_OP_H

#define TETRA_OP_H

/*-----------------------*

* Constants and Globals *

*-----------------------*/

extern Flag Overflow;

extern Flag Carry;

#define MAX_32 (Word32)0x7fffffff

#define MIN_32 (Word32)0x80000000

#define MAX_16 (Word16)0x7fff

#define MIN_16 (Word16)0x8000

/*-----------------------*

* Operators prototypes  *

*-----------------------*/

Word16 abs_s(Word16 var1);                /* Short abs,           1 */

Word16 add(Word16 var1, Word16 var2);     /* Short add,           1 */

Word16 div_s(Word16 var1, Word16 var2);   /* Short division,     18 */

Word16 extract_h(Word32 L_var1);          /* Extract high,        1 */

Word16 extract_l(Word32 L_var1);          /* Extract low,         1 */

Word16 mult(Word16 var1, Word16 var2);    /* Short mult,          1 */

Word16 mult_r(Word16 var1, Word16 var2);  /* Mult with round,     2 */

Word16 negate(Word16 var1);               /* Short negate,        1 */

Word16 norm_l(Word32 L_var1);             /* Long norm,          30 */

Word16 norm_s(Word16 var1);               /* Short norm,         15 */

Word16 round(Word32 L_var1);              /* Round,               1 */

Word16 shl(Word16 var1, Word16 var2);     /* Short shift left,    1 */

Word16 shr(Word16 var1, Word16 var2);     /* Short shift right,   1 */

Word16 sub(Word16 var1, Word16 var2);     /* Short sub,           1 */

Word32 L_abs(Word32 L_var1);              /* Long abs,            3 */

Word32 L_add(Word32 L_var1, Word32 L_var2);  /* Long add,         2 */

Word32 L_deposit_h(Word16 var1);          /* 16 bit var1 -> MSB   2 */

Word32 L_deposit_l(Word16 var1);          /* 16 bit var1 -> LSB,  2 */

Word32 L_mac(Word32 L_var3, Word16 var1, Word16 var2);  /* Mac,   1 */

Word32 L_mac0(Word32 L_var3, Word16 var1, Word16 var2); /* no shi 1 */

Word32 L_msu(Word32 L_var3, Word16 var1, Word16 var2);  /* Msu,   1 */

Word32 L_msu0(Word32 L_var3, Word16 var1, Word16 var2); /* no shi 1 */

Word32 L_mult(Word16 var1, Word16 var2);  /* Long mult,           1 */

Word32 L_mult0(Word16 var1, Word16 var2); /* Long mult no shift,  1 */

Word32 L_negate(Word32 L_var1);           /* Long negate,         2 */

Word32 L_shl(Word32 L_var1, Word16 var2); /* Long shift left,     2 */

Word32 L_shr(Word32 L_var1, Word16 var2); /* Long shift right,    2 */

Word32 L_shr_r(Word32 L_var1, Word16 var2);  /* L_shr with round, 3 */

Word32 L_sub(Word32 L_var1, Word32 L_var2);  /* Long sub,         2 */

#endif

/************************************************************************

*

*	FILENAME		:	clsp_334.tab

*

*	DESCRIPTION		:	LSP codebooks:  split VQ with 3 codebooks.

*					The LSPs are in the cosine domain (-1, 1) in Q15

*

* 					codebook    	vector dimension        number of vectors

*					~~~~~~~~   	~~~~~~~~~~~~~         ~~~~~~~~~~~~~~

*					        1			3		256	(8 bits)

*					        2			3		512	(9 bits)

*					        3			4		512	(9 bits)

*

************************************************************************/

#define dim_dic1		3

#define dim_dic2		3

#define dim_dic3		4

#define taille_dic1	256

#define taille_dic2	512

#define taille_dic3	512

/*-----------------------------------------*

* 1st codebook:   lsp0 to lsp2            *

*-----------------------------------------*/

static Word16 dico1_clsp[taille_dic1*dim_dic1] = {

16231 * 2,     16091 * 2,     15682 * 2,

16198 * 2,     16032 * 2,     15577 * 2,

16142 * 2,     15986 * 2,     15536 * 2,

16190 * 2,     15922 * 2,     15515 * 2,

16188 * 2,     16002 * 2,     15314 * 2,

16099 * 2,     15919 * 2,     15368 * 2,

16149 * 2,     15941 * 2,     15254 * 2,

16228 * 2,     16079 * 2,     15245 * 2,

16127 * 2,     15802 * 2,     15316 * 2,

16151 * 2,     15855 * 2,     15122 * 2,

16056 * 2,     15878 * 2,     15137 * 2,

16092 * 2,     15782 * 2,     15027 * 2,

16047 * 2,     15719 * 2,     15263 * 2,

16070 * 2,     15640 * 2,     15074 * 2,

16104 * 2,     15877 * 2,     14778 * 2,

16118 * 2,     15740 * 2,     14805 * 2,

15978 * 2,     15756 * 2,     14910 * 2,

16058 * 2,     15653 * 2,     14737 * 2,

16023 * 2,     15562 * 2,     14896 * 2,

15923 * 2,     15643 * 2,     15103 * 2,

15963 * 2,     15463 * 2,     15048 * 2,

15964 * 2,     15472 * 2,     14741 * 2,

16027 * 2,     15539 * 2,     14592 * 2,

15822 * 2,     15577 * 2,     14759 * 2,

15894 * 2,     15657 * 2,     14525 * 2,

15897 * 2,     15322 * 2,     14813 * 2,

15768 * 2,     15447 * 2,     14793 * 2,

15970 * 2,     15379 * 2,     14477 * 2,

15827 * 2,     15527 * 2,     14303 * 2,

15798 * 2,     15189 * 2,     14552 * 2,

15716 * 2,     15424 * 2,     14318 * 2,

15684 * 2,     15270 * 2,     14723 * 2,

15862 * 2,     15324 * 2,     14172 * 2,

15606 * 2,     15259 * 2,     14402 * 2,

15878 * 2,     15108 * 2,     14312 * 2,

15675 * 2,     15173 * 2,     14035 * 2,

15568 * 2,     15011 * 2,     14381 * 2,

15475 * 2,     15137 * 2,     14183 * 2,

15702 * 2,     15347 * 2,     13870 * 2,

15791 * 2,     15147 * 2,     13806 * 2,

15568 * 2,     15002 * 2,     13834 * 2,

15558 * 2,     15249 * 2,     13725 * 2,

15802 * 2,     14842 * 2,     14041 * 2,

15747 * 2,     14957 * 2,     13522 * 2,

15368 * 2,     14888 * 2,     13942 * 2,

15537 * 2,     14783 * 2,     13594 * 2,

15476 * 2,     15115 * 2,     13393 * 2,

15563 * 2,     14924 * 2,     13195 * 2,

15383 * 2,     14829 * 2,     13377 * 2,

15328 * 2,     15000 * 2,     13392 * 2,

15505 * 2,     14658 * 2,     13177 * 2,

15221 * 2,     14686 * 2,     13400 * 2,

15501 * 2,     14450 * 2,     13469 * 2,

15338 * 2,     14510 * 2,     13020 * 2,

15232 * 2,     14720 * 2,     12775 * 2,

15349 * 2,     14936 * 2,     12698 * 2,

15085 * 2,     14466 * 2,     13087 * 2,

15572 * 2,     14715 * 2,     12716 * 2,

15259 * 2,     14332 * 2,     12698 * 2,

15380 * 2,     14205 * 2,     13160 * 2,

15383 * 2,     14518 * 2,     12244 * 2,

15641 * 2,     14328 * 2,     12633 * 2,

15501 * 2,     14832 * 2,     12172 * 2,

15081 * 2,     14564 * 2,     12175 * 2,

15063 * 2,     14095 * 2,     12747 * 2,

15528 * 2,     14047 * 2,     12215 * 2,

15315 * 2,     14226 * 2,     11850 * 2,

14982 * 2,     14294 * 2,     12094 * 2,

15123 * 2,     13908 * 2,     12244 * 2,

15280 * 2,     14519 * 2,     11473 * 2,

15225 * 2,     14844 * 2,     11802 * 2,

15093 * 2,     13935 * 2,     11431 * 2,

15482 * 2,     13924 * 2,     11356 * 2,

15475 * 2,     14577 * 2,     11183 * 2,

15218 * 2,     14226 * 2,     10791 * 2,

14737 * 2,     14154 * 2,     11457 * 2,

14916 * 2,     14278 * 2,     10548 * 2,

15167 * 2,     14584 * 2,     10383 * 2,

15441 * 2,     14281 * 2,     10338 * 2,

15217 * 2,     13701 * 2,     10458 * 2,

14864 * 2,     13740 * 2,     10632 * 2,

15158 * 2,     13956 * 2,      9868 * 2,

15160 * 2,     14387 * 2,      9550 * 2,

14820 * 2,     13827 * 2,      9553 * 2,

15307 * 2,     14034 * 2,      9137 * 2,

15561 * 2,     13729 * 2,      9617 * 2,

15121 * 2,     13366 * 2,      9420 * 2,

14834 * 2,     13347 * 2,      9964 * 2,

15075 * 2,     13592 * 2,      8793 * 2,

15032 * 2,     14126 * 2,      8529 * 2,

14716 * 2,     13670 * 2,      8579 * 2,

15158 * 2,     13141 * 2,      8223 * 2,

14584 * 2,     13155 * 2,      8860 * 2,

14934 * 2,     12714 * 2,      8774 * 2,

14725 * 2,     12847 * 2,      9653 * 2,

14797 * 2,     12944 * 2,      7872 * 2,

15022 * 2,     13701 * 2,      7746 * 2,

14460 * 2,     12456 * 2,      8752 * 2,

14271 * 2,     12719 * 2,      7966 * 2,

14616 * 2,     12296 * 2,      7929 * 2,

14837 * 2,     11931 * 2,      9091 * 2,

15162 * 2,     12854 * 2,      7074 * 2,

14781 * 2,     13408 * 2,      7008 * 2,

14513 * 2,     12747 * 2,      6582 * 2,

14328 * 2,     11984 * 2,      6857 * 2,

14732 * 2,     11567 * 2,      7593 * 2,

14754 * 2,     11677 * 2,      6455 * 2,

14795 * 2,     12447 * 2,      5975 * 2,

13850 * 2,     11888 * 2,      7609 * 2,

14927 * 2,     13097 * 2,      5461 * 2,

14269 * 2,     11440 * 2,      5252 * 2,

14752 * 2,     11489 * 2,      4625 * 2,

15290 * 2,     11792 * 2,      5357 * 2,

14609 * 2,     12271 * 2,      4241 * 2,

14387 * 2,     11193 * 2,      3749 * 2,

14900 * 2,     12992 * 2,      3558 * 2,

14865 * 2,     12377 * 2,      2905 * 2,

14600 * 2,     11232 * 2,      2715 * 2,

15425 * 2,     10659 * 2,      3564 * 2,

14985 * 2,     13418 * 2,      4349 * 2,

15424 * 2,     13710 * 2,      2694 * 2,

15248 * 2,     13980 * 2,      4419 * 2,

15139 * 2,     13555 * 2,      6077 * 2,

15878 * 2,     13355 * 2,      4567 * 2,

15740 * 2,     13445 * 2,      6142 * 2,

15483 * 2,     14469 * 2,      5488 * 2,

15404 * 2,     13842 * 2,      7138 * 2,

15091 * 2,     14172 * 2,      6855 * 2,

15514 * 2,     14415 * 2,      7464 * 2,

15828 * 2,     14810 * 2,      6056 * 2,

15905 * 2,     14299 * 2,      7695 * 2,

15616 * 2,     13743 * 2,      8439 * 2,

15349 * 2,     14546 * 2,      8611 * 2,

15768 * 2,     15204 * 2,      7352 * 2,

15718 * 2,     14835 * 2,      8847 * 2,

15715 * 2,     14298 * 2,      9159 * 2,

15922 * 2,     15121 * 2,      8926 * 2,

15659 * 2,     15242 * 2,      9593 * 2,

15864 * 2,     14260 * 2,      9965 * 2,

15598 * 2,     14749 * 2,     10202 * 2,

15347 * 2,     14862 * 2,      9903 * 2,

15832 * 2,     14938 * 2,     10314 * 2,

16020 * 2,     15210 * 2,     10351 * 2,

15772 * 2,     14601 * 2,     11044 * 2,

15781 * 2,     15501 * 2,     10547 * 2,

15812 * 2,     15191 * 2,     11258 * 2,

15637 * 2,     14955 * 2,     11529 * 2,

15658 * 2,     15382 * 2,     11479 * 2,

15534 * 2,     15204 * 2,     11639 * 2,

15943 * 2,     14965 * 2,     11691 * 2,

15955 * 2,     15373 * 2,     11636 * 2,

15413 * 2,     15029 * 2,     11640 * 2,

15761 * 2,     15509 * 2,     12201 * 2,

15834 * 2,     15090 * 2,     12389 * 2,

15737 * 2,     15311 * 2,     12584 * 2,

15857 * 2,     15616 * 2,     11897 * 2,

15889 * 2,     15436 * 2,     12566 * 2,

16008 * 2,     15590 * 2,     12292 * 2,

16017 * 2,     15262 * 2,     12780 * 2,

15540 * 2,     15106 * 2,     12672 * 2,

15929 * 2,     15639 * 2,     12941 * 2,

15699 * 2,     15448 * 2,     13140 * 2,

15955 * 2,     15459 * 2,     13281 * 2,

15984 * 2,     15782 * 2,     12858 * 2,

15826 * 2,     15311 * 2,     13396 * 2,

15608 * 2,     15291 * 2,     13082 * 2,

15725 * 2,     15089 * 2,     13155 * 2,

15790 * 2,     15525 * 2,     13586 * 2,

15861 * 2,     15622 * 2,     13596 * 2,

15960 * 2,     15103 * 2,     13294 * 2,

15950 * 2,     15421 * 2,     13857 * 2,

15938 * 2,     15709 * 2,     13665 * 2,

16045 * 2,     15641 * 2,     13517 * 2,

16113 * 2,     15435 * 2,     13783 * 2,

15950 * 2,     15579 * 2,     14127 * 2,

16032 * 2,     15763 * 2,     13984 * 2,

16056 * 2,     15597 * 2,     14271 * 2,

16071 * 2,     15282 * 2,     14159 * 2,

15973 * 2,     15723 * 2,     14379 * 2,

16086 * 2,     15755 * 2,     14429 * 2,

16119 * 2,     15881 * 2,     14090 * 2,

16205 * 2,     15680 * 2,     14372 * 2,

16028 * 2,     15842 * 2,     14528 * 2,

16199 * 2,     15761 * 2,     13813 * 2,

16141 * 2,     15959 * 2,     14662 * 2,

16198 * 2,     16013 * 2,     14080 * 2,

16233 * 2,     16079 * 2,     14465 * 2,

16203 * 2,     15948 * 2,     14878 * 2,

16271 * 2,     16051 * 2,     13866 * 2,

16287 * 2,     16154 * 2,     14482 * 2,

16167 * 2,     15960 * 2,     13393 * 2,

16267 * 2,     15940 * 2,     13407 * 2,

16076 * 2,     15830 * 2,     13540 * 2,

16237 * 2,     15561 * 2,     13288 * 2,

16200 * 2,     15808 * 2,     12800 * 2,

16126 * 2,     15534 * 2,     13042 * 2,

16082 * 2,     15783 * 2,     12455 * 2,

16139 * 2,     15482 * 2,     12064 * 2,

16155 * 2,     15840 * 2,     11644 * 2,

16238 * 2,     16026 * 2,     11633 * 2,

16071 * 2,     15639 * 2,     11117 * 2,

16186 * 2,     15311 * 2,     11146 * 2,

15938 * 2,     15700 * 2,     10741 * 2,

15948 * 2,     15495 * 2,     10329 * 2,

16108 * 2,     15736 * 2,      9800 * 2,

16056 * 2,     14735 * 2,     11245 * 2,

15830 * 2,     15467 * 2,      9241 * 2,

16167 * 2,     15951 * 2,      8972 * 2,

16051 * 2,     15570 * 2,      8007 * 2,

15888 * 2,     15592 * 2,      7559 * 2,

15978 * 2,     15368 * 2,      7032 * 2,

15846 * 2,     15467 * 2,      5711 * 2,

16014 * 2,     15082 * 2,      4645 * 2,

15831 * 2,     14913 * 2,      3458 * 2,

15755 * 2,     13822 * 2,     10633 * 2,

15883 * 2,     14259 * 2,     11598 * 2,

15463 * 2,     13031 * 2,     10169 * 2,

15821 * 2,     12195 * 2,      8137 * 2,

15582 * 2,     12516 * 2,      9433 * 2,

14821 * 2,     12969 * 2,     10527 * 2,

15732 * 2,     13268 * 2,     11498 * 2,

15146 * 2,     13327 * 2,     11267 * 2,

14842 * 2,     12289 * 2,     10422 * 2,

14567 * 2,     13505 * 2,     10780 * 2,

14390 * 2,     12759 * 2,     11170 * 2,

14858 * 2,     13614 * 2,     11657 * 2,

14237 * 2,     13218 * 2,     11461 * 2,

14608 * 2,     13504 * 2,     12080 * 2,

14674 * 2,     13974 * 2,     12184 * 2,

14866 * 2,     13720 * 2,     12773 * 2,

14896 * 2,     14332 * 2,     12782 * 2,

14840 * 2,     14067 * 2,     13132 * 2,

15435 * 2,     13786 * 2,     12823 * 2,

15121 * 2,     14302 * 2,     13483 * 2,

15333 * 2,     14603 * 2,     13872 * 2,

15772 * 2,     14524 * 2,     13052 * 2,

15708 * 2,     14584 * 2,     12009 * 2,

15797 * 2,     14669 * 2,     13637 * 2,

15814 * 2,     14858 * 2,     12904 * 2,

15962 * 2,     14568 * 2,     12497 * 2,

15969 * 2,     15014 * 2,     13792 * 2,

14039 * 2,     12511 * 2,      9773 * 2,

14251 * 2,     11878 * 2,      9621 * 2,

14100 * 2,     11691 * 2,      8583 * 2,

14814 * 2,     10471 * 2,      7911 * 2,

13965 * 2,     10725 * 2,      7012 * 2,

15689 * 2,      9981 * 2,      6000 * 2,

14388 * 2,      9740 * 2,      5769 * 2,

14384 * 2,      9736 * 2,      4341 * 2,

13970 * 2,      9084 * 2,      5028 * 2,

15298 * 2,      8857 * 2,      4205 * 2,

14071 * 2,      7942 * 2,      4472 * 2,

13980 * 2,      9604 * 2,      3273 * 2,

13383 * 2,      8140 * 2,      3828 * 2,

13005 * 2,      7620 * 2,      4742 * 2,

16310 * 2,     16203 * 2,     15451 * 2 };

/*-----------------------------------------*

* 2nd codebook:   lsp3 to lsp5            *

*-----------------------------------------*/

static Word16 dico2_clsp[taille_dic2*dim_dic2] = {

7564 * 2,      -293 * 2,     -2543 * 2,

7820 * 2,      -150 * 2,     -1813 * 2,

6913 * 2,     -1006 * 2,     -2297 * 2,

7099 * 2,      -498 * 2,     -3337 * 2,

6526 * 2,       308 * 2,     -2075 * 2,

6953 * 2,       958 * 2,     -2388 * 2,

6811 * 2,       758 * 2,     -3159 * 2,

6448 * 2,        93 * 2,     -1294 * 2,

7572 * 2,       843 * 2,     -1485 * 2,

5803 * 2,        76 * 2,     -2785 * 2,

5791 * 2,      -566 * 2,     -1850 * 2,

5655 * 2,       944 * 2,     -1852 * 2,

6035 * 2,      1575 * 2,     -2728 * 2,

5226 * 2,       672 * 2,     -2515 * 2,

6131 * 2,      1800 * 2,     -1816 * 2,

6104 * 2,      1092 * 2,      -955 * 2,

4868 * 2,       150 * 2,     -1704 * 2,

4540 * 2,       928 * 2,     -2198 * 2,

4954 * 2,      1589 * 2,     -1403 * 2,

5133 * 2,      1825 * 2,     -2481 * 2,

5266 * 2,      2505 * 2,     -2018 * 2,

4140 * 2,      1163 * 2,     -1467 * 2,

5312 * 2,      2208 * 2,      -846 * 2,

4412 * 2,      2344 * 2,     -1137 * 2,

4016 * 2,      1694 * 2,     -1020 * 2,

3937 * 2,      2428 * 2,     -2025 * 2,

4706 * 2,      3059 * 2,     -1381 * 2,

3469 * 2,      1623 * 2,     -1956 * 2,

4611 * 2,      2818 * 2,      -273 * 2,

3830 * 2,      2126 * 2,      -140 * 2,

5232 * 2,      1681 * 2,      -440 * 2,

4523 * 2,      1482 * 2,       106 * 2,

4461 * 2,       950 * 2,      -458 * 2,

3228 * 2,      1395 * 2,      -718 * 2,

4792 * 2,      2283 * 2,       392 * 2,

4198 * 2,       318 * 2,      -990 * 2,

5397 * 2,      3081 * 2,      -622 * 2,

6034 * 2,      1867 * 2,       379 * 2,

6276 * 2,      1978 * 2,      -806 * 2,

5881 * 2,      2678 * 2,       452 * 2,

6308 * 2,      2449 * 2,      -240 * 2,

5782 * 2,      2367 * 2,      1051 * 2,

6319 * 2,      3243 * 2,       153 * 2,

6434 * 2,      2971 * 2,      -955 * 2,

7053 * 2,      1856 * 2,      -216 * 2,

7385 * 2,      2650 * 2,        64 * 2,

6774 * 2,      2930 * 2,      1008 * 2,

7416 * 2,      2162 * 2,       703 * 2,

7282 * 2,      2723 * 2,      -833 * 2,

7452 * 2,      3624 * 2,      -298 * 2,

8031 * 2,      3206 * 2,       530 * 2,

6889 * 2,      4018 * 2,       470 * 2,

7485 * 2,      3735 * 2,      1191 * 2,

7008 * 2,      2808 * 2,      1663 * 2,

8040 * 2,      3276 * 2,      1625 * 2,

7841 * 2,      4506 * 2,       984 * 2,

8460 * 2,      4070 * 2,       391 * 2,

6611 * 2,      4137 * 2,      1514 * 2,

7487 * 2,      4459 * 2,      2017 * 2,

7549 * 2,      3763 * 2,      2384 * 2,

6420 * 2,      3454 * 2,      1907 * 2,

6694 * 2,      4869 * 2,      1313 * 2,

7852 * 2,      5242 * 2,      1730 * 2,

6991 * 2,      5489 * 2,      1418 * 2,

6873 * 2,      4940 * 2,      2548 * 2,

6389 * 2,      4265 * 2,      2477 * 2,

7703 * 2,      4357 * 2,      2885 * 2,

7318 * 2,      5438 * 2,      2788 * 2,

7257 * 2,      4987 * 2,      3443 * 2,

8222 * 2,      5222 * 2,      2614 * 2,

7645 * 2,      6104 * 2,      2139 * 2,

8148 * 2,      5663 * 2,      3485 * 2,

8607 * 2,      4884 * 2,      3527 * 2,

7512 * 2,      5913 * 2,      4110 * 2,

8568 * 2,      6064 * 2,      2396 * 2,

8113 * 2,      6670 * 2,      2267 * 2,

8790 * 2,      6611 * 2,      3338 * 2,

9197 * 2,      5692 * 2,      3316 * 2,

8272 * 2,      6549 * 2,      4051 * 2,

9059 * 2,      6038 * 2,      4412 * 2,

9554 * 2,      6359 * 2,      3624 * 2,

9264 * 2,      5327 * 2,      4105 * 2,

9149 * 2,      6926 * 2,      4494 * 2,

9916 * 2,      6778 * 2,      4197 * 2,

9600 * 2,      7338 * 2,      3435 * 2,

10366 * 2,      5712 * 2,      3980 * 2,

10362 * 2,      6304 * 2,      4858 * 2,

10383 * 2,      7083 * 2,      4912 * 2,

10467 * 2,      7485 * 2,      4027 * 2,

10573 * 2,      6057 * 2,      3302 * 2,

10367 * 2,      6987 * 2,      3015 * 2,

11091 * 2,      6561 * 2,      3914 * 2,

11279 * 2,      7390 * 2,      4289 * 2,

11378 * 2,      5806 * 2,      4560 * 2,

11504 * 2,      6657 * 2,      5030 * 2,

11919 * 2,      6407 * 2,      3700 * 2,

11338 * 2,      5343 * 2,      3847 * 2,

11333 * 2,      5308 * 2,      3155 * 2,

11051 * 2,      6450 * 2,      2535 * 2,

11518 * 2,      7251 * 2,      2774 * 2,

12044 * 2,      6329 * 2,      2477 * 2,

12482 * 2,      5821 * 2,      4097 * 2,

12478 * 2,      7102 * 2,      3587 * 2,

12676 * 2,      5586 * 2,      3333 * 2,

12171 * 2,      4918 * 2,      2877 * 2,

12751 * 2,      6844 * 2,      2736 * 2,

13176 * 2,      5963 * 2,      3744 * 2,

12802 * 2,      6393 * 2,      4533 * 2,

12692 * 2,      4994 * 2,      4094 * 2,

13017 * 2,      4667 * 2,      3408 * 2,

13343 * 2,      5710 * 2,      2375 * 2,

12868 * 2,      5296 * 2,      2008 * 2,

13364 * 2,      4239 * 2,      2361 * 2,

13843 * 2,      5743 * 2,      4255 * 2,

14049 * 2,      5097 * 2,      3468 * 2,

14033 * 2,      4939 * 2,      2835 * 2,

13898 * 2,      6536 * 2,      3115 * 2,

14169 * 2,      4961 * 2,      1705 * 2,

13710 * 2,      4832 * 2,      1148 * 2,

14596 * 2,      5508 * 2,      2662 * 2,

13187 * 2,      6223 * 2,       960 * 2,

13822 * 2,      6583 * 2,       583 * 2,

14436 * 2,      7264 * 2,      1534 * 2,

13770 * 2,      7598 * 2,      1965 * 2,

13170 * 2,      7340 * 2,      1891 * 2,

13941 * 2,      4561 * 2,       129 * 2,

13431 * 2,      5471 * 2,      -449 * 2,

14335 * 2,      5782 * 2,      -545 * 2,

12866 * 2,      5691 * 2,      -477 * 2,

12509 * 2,      6194 * 2,      1001 * 2,

12692 * 2,      7437 * 2,       704 * 2,

12722 * 2,      4956 * 2,       901 * 2,

12151 * 2,      6291 * 2,       -75 * 2,

12353 * 2,      4481 * 2,       -63 * 2,

11761 * 2,      6052 * 2,       985 * 2,

12284 * 2,      4617 * 2,      1797 * 2,

11734 * 2,      4431 * 2,       388 * 2,

11295 * 2,      5975 * 2,      -390 * 2,

11032 * 2,      4944 * 2,       743 * 2,

11399 * 2,      5089 * 2,      2116 * 2,

10964 * 2,      6187 * 2,      1187 * 2,

10863 * 2,      4166 * 2,      1606 * 2,

10205 * 2,      5235 * 2,      1461 * 2,

10772 * 2,      4436 * 2,      -223 * 2,

10305 * 2,      5642 * 2,        70 * 2,

10393 * 2,      6369 * 2,      1675 * 2,

10597 * 2,      4319 * 2,      2393 * 2,

10600 * 2,      5163 * 2,      2801 * 2,

9741 * 2,      4539 * 2,       336 * 2,

9347 * 2,      4692 * 2,      1139 * 2,

9785 * 2,      3625 * 2,      1401 * 2,

9006 * 2,      5480 * 2,      1621 * 2,

9538 * 2,      5265 * 2,      2600 * 2,

8992 * 2,      4694 * 2,      2420 * 2,

8956 * 2,      4140 * 2,      1972 * 2,

9472 * 2,      3917 * 2,      2709 * 2,

9301 * 2,      4477 * 2,      3132 * 2,

8874 * 2,      3348 * 2,      2170 * 2,

9827 * 2,      5042 * 2,      3445 * 2,

9890 * 2,      6059 * 2,      2463 * 2,

10518 * 2,      3522 * 2,      2283 * 2,

10797 * 2,      4366 * 2,      3190 * 2,

10728 * 2,      4775 * 2,      3680 * 2,

9809 * 2,      2897 * 2,      1746 * 2,

11591 * 2,      3608 * 2,      1929 * 2,

10834 * 2,      3491 * 2,       761 * 2,

11979 * 2,      3948 * 2,      2794 * 2,

11799 * 2,      2977 * 2,      1849 * 2,

11804 * 2,      3224 * 2,      1025 * 2,

10578 * 2,      2760 * 2,       924 * 2,

12052 * 2,      2472 * 2,      1517 * 2,

11763 * 2,      1897 * 2,       874 * 2,

10935 * 2,      2556 * 2,       -10 * 2,

12339 * 2,      2548 * 2,       178 * 2,

11812 * 2,      2374 * 2,      -613 * 2,

10869 * 2,      1311 * 2,       203 * 2,

11242 * 2,      3714 * 2,      -618 * 2,

10672 * 2,      1941 * 2,     -1043 * 2,

11974 * 2,       768 * 2,      -371 * 2,

10582 * 2,      1020 * 2,      -737 * 2,

11222 * 2,      1190 * 2,     -1751 * 2,

11652 * 2,      3167 * 2,     -1686 * 2,

11771 * 2,      1803 * 2,     -2377 * 2,

12157 * 2,       276 * 2,     -1261 * 2,

11281 * 2,      1351 * 2,     -3052 * 2,

11760 * 2,        77 * 2,     -1978 * 2,

10461 * 2,       599 * 2,     -2519 * 2,

12320 * 2,      -312 * 2,     -2603 * 2,

12908 * 2,      1043 * 2,     -1899 * 2,

12826 * 2,       931 * 2,     -3215 * 2,

12987 * 2,      1269 * 2,     -1053 * 2,

13424 * 2,       538 * 2,     -2342 * 2,

13102 * 2,       -57 * 2,     -4076 * 2,

12263 * 2,      2454 * 2,     -3882 * 2,

13816 * 2,       796 * 2,     -3719 * 2,

13973 * 2,      1713 * 2,     -2871 * 2,

13869 * 2,      1446 * 2,     -1618 * 2,

13292 * 2,      2675 * 2,     -1898 * 2,

13868 * 2,      2841 * 2,     -1281 * 2,

14411 * 2,      2364 * 2,     -3252 * 2,

13521 * 2,      3517 * 2,     -3271 * 2,

14425 * 2,      2348 * 2,     -1729 * 2,

14732 * 2,      2891 * 2,     -2389 * 2,

14526 * 2,      3821 * 2,     -1287 * 2,

14039 * 2,      4544 * 2,     -2746 * 2,

13926 * 2,      5111 * 2,     -1292 * 2,

13529 * 2,      3616 * 2,      -239 * 2,

14613 * 2,      5736 * 2,     -2230 * 2,

14968 * 2,      5253 * 2,     -1021 * 2,

14559 * 2,      3776 * 2,       465 * 2,

14617 * 2,      4378 * 2,     -4224 * 2,

13368 * 2,      6485 * 2,     -1784 * 2,

12599 * 2,      4340 * 2,     -1435 * 2,

12694 * 2,      3951 * 2,     -2784 * 2,

13073 * 2,      3789 * 2,       588 * 2,

12852 * 2,      2836 * 2,      -620 * 2,

13700 * 2,      1906 * 2,       -33 * 2,

13417 * 2,      2505 * 2,       809 * 2,

12998 * 2,      3554 * 2,      1467 * 2,

14015 * 2,      3141 * 2,      1469 * 2,

12867 * 2,      3536 * 2,      2271 * 2,

13756 * 2,       929 * 2,      -384 * 2,

10188 * 2,      1745 * 2,       592 * 2,

11419 * 2,      5535 * 2,     -1711 * 2,

9637 * 2,      3260 * 2,       332 * 2,

10133 * 2,      4611 * 2,     -1013 * 2,

9707 * 2,      2996 * 2,      -871 * 2,

10284 * 2,      3161 * 2,     -2270 * 2,

9710 * 2,      1748 * 2,      -461 * 2,

9184 * 2,      2352 * 2,       138 * 2,

9166 * 2,      3710 * 2,     -1743 * 2,

9163 * 2,      4407 * 2,      -475 * 2,

8561 * 2,      3500 * 2,      -614 * 2,

8676 * 2,      2539 * 2,     -1572 * 2,

8520 * 2,      2158 * 2,      -676 * 2,

7797 * 2,      3393 * 2,     -1237 * 2,

7991 * 2,      1776 * 2,     -1668 * 2,

8159 * 2,      3239 * 2,     -2484 * 2,

8140 * 2,      4597 * 2,      -982 * 2,

6990 * 2,      2510 * 2,     -1870 * 2,

7026 * 2,      4004 * 2,     -1953 * 2,

7136 * 2,      1609 * 2,     -1203 * 2,

7217 * 2,      1904 * 2,     -2725 * 2,

6773 * 2,      3042 * 2,     -2878 * 2,

5944 * 2,      2926 * 2,     -1591 * 2,

6396 * 2,      3845 * 2,      -761 * 2,

5666 * 2,      3021 * 2,     -3053 * 2,

5164 * 2,      3788 * 2,     -1911 * 2,

5680 * 2,      4283 * 2,     -1046 * 2,

6350 * 2,      4968 * 2,     -1518 * 2,

5489 * 2,      3743 * 2,       -28 * 2,

6183 * 2,      4396 * 2,       173 * 2,

7036 * 2,      4742 * 2,      -310 * 2,

5005 * 2,      3366 * 2,       513 * 2,

5862 * 2,      3686 * 2,      1244 * 2,

5779 * 2,      4527 * 2,      1341 * 2,

5432 * 2,      3100 * 2,      1205 * 2,

7416 * 2,      5187 * 2,       465 * 2,

7287 * 2,      5658 * 2,      -643 * 2,

8414 * 2,      5026 * 2,       586 * 2,

7991 * 2,      5959 * 2,       759 * 2,

8790 * 2,      2939 * 2,      1176 * 2,

9149 * 2,      5825 * 2,       175 * 2,

8623 * 2,      6672 * 2,      1016 * 2,

8299 * 2,      6034 * 2,     -1339 * 2,

9158 * 2,      6865 * 2,      -646 * 2,

9716 * 2,      6566 * 2,       979 * 2,

9119 * 2,      7508 * 2,      1103 * 2,

9216 * 2,      6732 * 2,      2187 * 2,

9767 * 2,      7791 * 2,       192 * 2,

10117 * 2,      7427 * 2,      1579 * 2,

9646 * 2,      8099 * 2,      2184 * 2,

10789 * 2,      7148 * 2,      -115 * 2,

10538 * 2,      8395 * 2,       577 * 2,

10523 * 2,      7909 * 2,      2683 * 2,

11293 * 2,      7494 * 2,      1440 * 2,

11121 * 2,      8682 * 2,      1598 * 2,

10387 * 2,      8722 * 2,      2843 * 2,

11092 * 2,      7990 * 2,      3176 * 2,

11790 * 2,      8511 * 2,      2266 * 2,

10995 * 2,      9415 * 2,      2452 * 2,

11666 * 2,      8212 * 2,      3760 * 2,

11999 * 2,      9089 * 2,      3774 * 2,

12268 * 2,      9210 * 2,      2028 * 2,

11821 * 2,      9800 * 2,      3288 * 2,

11424 * 2,      9183 * 2,      4356 * 2,

12664 * 2,      9221 * 2,      3093 * 2,

12343 * 2,      9929 * 2,      3844 * 2,

11318 * 2,     10061 * 2,      4382 * 2,

12362 * 2,      9114 * 2,      4908 * 2,

12753 * 2,     10503 * 2,      3698 * 2,

12985 * 2,      9731 * 2,      4733 * 2,

11892 * 2,      9626 * 2,      5588 * 2,

12458 * 2,     10370 * 2,      5601 * 2,

11854 * 2,     10622 * 2,      5262 * 2,

12878 * 2,      8886 * 2,      5806 * 2,

12559 * 2,      9585 * 2,      6659 * 2,

13068 * 2,     10386 * 2,      6203 * 2,

12633 * 2,     11087 * 2,      6026 * 2,

12186 * 2,     10894 * 2,      6938 * 2,

11920 * 2,      9993 * 2,      7144 * 2,

13104 * 2,     11211 * 2,      6967 * 2,

13164 * 2,     10142 * 2,      7665 * 2,

12367 * 2,      9323 * 2,      7804 * 2,

12585 * 2,     10879 * 2,      8219 * 2,

12317 * 2,     10251 * 2,      8449 * 2,

12720 * 2,     11567 * 2,      8102 * 2,

13234 * 2,      9369 * 2,      8463 * 2,

11548 * 2,      9421 * 2,      8254 * 2,

13249 * 2,      8973 * 2,      7039 * 2,

12176 * 2,      8597 * 2,      6758 * 2,

11461 * 2,      9342 * 2,      6748 * 2,

10742 * 2,      8788 * 2,      7518 * 2,

10806 * 2,      9124 * 2,      6235 * 2,

10997 * 2,      8275 * 2,      6159 * 2,

11560 * 2,      8613 * 2,      5437 * 2,

12283 * 2,      8066 * 2,      5739 * 2,

11068 * 2,      7556 * 2,      5368 * 2,

10110 * 2,      8304 * 2,      6289 * 2,

10311 * 2,      7972 * 2,      5163 * 2,

10441 * 2,      7206 * 2,      5823 * 2,

10880 * 2,      8468 * 2,      4478 * 2,

11908 * 2,      7316 * 2,      5254 * 2,

10635 * 2,      9226 * 2,      4414 * 2,

9984 * 2,      8256 * 2,      4205 * 2,

9275 * 2,      7592 * 2,      5010 * 2,

12135 * 2,      8005 * 2,      4117 * 2,

12553 * 2,      7346 * 2,      4798 * 2,

12699 * 2,      8151 * 2,      2816 * 2,

13136 * 2,      8345 * 2,      4614 * 2,

12192 * 2,      7482 * 2,      1678 * 2,

13413 * 2,      7358 * 2,      3703 * 2,

13403 * 2,      6763 * 2,      5157 * 2,

13300 * 2,      7614 * 2,      5971 * 2,

13940 * 2,      8404 * 2,      3784 * 2,

13777 * 2,      8074 * 2,      5441 * 2,

13465 * 2,      9081 * 2,      2751 * 2,

13567 * 2,      9647 * 2,      4283 * 2,

14428 * 2,      7463 * 2,      4258 * 2,

14332 * 2,      9351 * 2,      4182 * 2,

14393 * 2,      8218 * 2,      2867 * 2,

14276 * 2,      9080 * 2,      6060 * 2,

14076 * 2,      9711 * 2,      2470 * 2,

13630 * 2,      9696 * 2,      5808 * 2,

14679 * 2,      9709 * 2,      2242 * 2,

14160 * 2,     10997 * 2,      4409 * 2,

13309 * 2,     10327 * 2,      2581 * 2,

13224 * 2,     10865 * 2,      4126 * 2,

14552 * 2,     11321 * 2,      4526 * 2,

13808 * 2,     11509 * 2,      4970 * 2,

13475 * 2,     10927 * 2,      5633 * 2,

13018 * 2,     11618 * 2,      4262 * 2,

13931 * 2,     12153 * 2,      3241 * 2,

13376 * 2,     12029 * 2,      5980 * 2,

12334 * 2,     10978 * 2,      3108 * 2,

13728 * 2,     10999 * 2,      1834 * 2,

14883 * 2,     11658 * 2,      3750 * 2,

13940 * 2,     10644 * 2,      6944 * 2,

14216 * 2,     12662 * 2,      6727 * 2,

13652 * 2,     12582 * 2,      7202 * 2,

13647 * 2,     11561 * 2,      8042 * 2,

13073 * 2,     12210 * 2,      7825 * 2,

13021 * 2,     11362 * 2,      9396 * 2,

13850 * 2,     12600 * 2,      9870 * 2,

13321 * 2,     12442 * 2,     10862 * 2,

12641 * 2,     10336 * 2,      1486 * 2,

12993 * 2,     11301 * 2,      1024 * 2,

12026 * 2,     10336 * 2,      1353 * 2,

14249 * 2,     10493 * 2,      1174 * 2,

12910 * 2,      8766 * 2,      1401 * 2,

13576 * 2,      9065 * 2,       929 * 2,

13237 * 2,     10168 * 2,        45 * 2,

12659 * 2,      9572 * 2,      -237 * 2,

12200 * 2,      8811 * 2,       539 * 2,

11676 * 2,      9627 * 2,       701 * 2,

13175 * 2,      7994 * 2,      -647 * 2,

14274 * 2,      8614 * 2,      1030 * 2,

11770 * 2,      7827 * 2,       402 * 2,

12397 * 2,      7494 * 2,      -904 * 2,

11242 * 2,      8488 * 2,      -714 * 2,

11883 * 2,      8572 * 2,     -2307 * 2,

12942 * 2,      8137 * 2,     -2821 * 2,

14047 * 2,      7675 * 2,     -1923 * 2,

12011 * 2,      6420 * 2,     -3262 * 2,

10184 * 2,      7669 * 2,     -1818 * 2,

10155 * 2,      6096 * 2,     -1676 * 2,

13048 * 2,      5362 * 2,     -4666 * 2,

13653 * 2,      6231 * 2,     -5460 * 2,

11925 * 2,      4015 * 2,     -5430 * 2,

10735 * 2,      3346 * 2,     -3524 * 2,

9349 * 2,      4598 * 2,     -3399 * 2,

11031 * 2,      2014 * 2,     -5157 * 2,

9330 * 2,      2646 * 2,     -5465 * 2,

9374 * 2,      1306 * 2,     -3874 * 2,

11556 * 2,       299 * 2,     -4747 * 2,

9264 * 2,      1787 * 2,     -2708 * 2,

10938 * 2,       -42 * 2,     -3854 * 2,

9077 * 2,       249 * 2,     -2965 * 2,

9951 * 2,      -643 * 2,     -4696 * 2,

8695 * 2,       -70 * 2,     -3650 * 2,

8193 * 2,       664 * 2,     -4738 * 2,

8925 * 2,      -879 * 2,     -2873 * 2,

8632 * 2,       644 * 2,     -2251 * 2,

10049 * 2,      -345 * 2,     -1940 * 2,

8812 * 2,     -1699 * 2,     -4282 * 2,

7585 * 2,     -1492 * 2,     -3374 * 2,

7321 * 2,      -662 * 2,     -4259 * 2,

8513 * 2,      -862 * 2,     -5444 * 2,

6425 * 2,       426 * 2,     -4076 * 2,

7536 * 2,      1957 * 2,     -3948 * 2,

6558 * 2,      1461 * 2,     -4873 * 2,

5999 * 2,      -265 * 2,     -5358 * 2,

5657 * 2,       348 * 2,     -4658 * 2,

5488 * 2,      1123 * 2,     -3537 * 2,

5150 * 2,        51 * 2,     -3476 * 2,

5222 * 2,      -943 * 2,     -4507 * 2,

4581 * 2,       362 * 2,     -4293 * 2,

4868 * 2,      1676 * 2,     -5157 * 2,

4431 * 2,      1543 * 2,     -4063 * 2,

3850 * 2,       845 * 2,     -4686 * 2,

4059 * 2,      -107 * 2,     -3395 * 2,

4527 * 2,      1273 * 2,     -2949 * 2,

3735 * 2,       744 * 2,     -3093 * 2,

3082 * 2,      1200 * 2,     -4127 * 2,

3405 * 2,      1879 * 2,     -3231 * 2,

4594 * 2,      2463 * 2,     -3711 * 2,

2999 * 2,       103 * 2,     -3506 * 2,

2380 * 2,       690 * 2,     -3330 * 2,

3339 * 2,       658 * 2,     -2070 * 2,

2691 * 2,      1302 * 2,     -1971 * 2,

3346 * 2,       -52 * 2,     -1960 * 2,

2814 * 2,      -501 * 2,     -3150 * 2,

2052 * 2,        79 * 2,     -2475 * 2,

2320 * 2,      -560 * 2,     -2143 * 2,

2676 * 2,       627 * 2,     -1273 * 2,

2770 * 2,     -1345 * 2,     -2640 * 2,

4487 * 2,       171 * 2,     -2513 * 2,

4378 * 2,      -681 * 2,     -2521 * 2,

3230 * 2,     -1229 * 2,     -3412 * 2,

2693 * 2,     -2022 * 2,     -3359 * 2,

4176 * 2,      -888 * 2,     -3993 * 2,

3523 * 2,     -2141 * 2,     -3983 * 2,

3004 * 2,      -293 * 2,     -4569 * 2,

3375 * 2,     -1360 * 2,     -4767 * 2,

4100 * 2,     -1735 * 2,     -5277 * 2,

4204 * 2,      -302 * 2,     -5248 * 2,

2085 * 2,      -979 * 2,     -4878 * 2,

2646 * 2,     -1469 * 2,     -5896 * 2,

3291 * 2,      -357 * 2,     -5984 * 2,

2098 * 2,       152 * 2,     -5496 * 2,

3851 * 2,     -1336 * 2,     -6741 * 2,

2411 * 2,     -1002 * 2,     -6998 * 2,

4221 * 2,     -2346 * 2,     -6013 * 2,

1591 * 2,     -1869 * 2,     -6561 * 2,

1203 * 2,      -796 * 2,     -6075 * 2,

1744 * 2,     -2522 * 2,     -5654 * 2,

2310 * 2,     -3164 * 2,     -6344 * 2,

1385 * 2,     -1923 * 2,     -4877 * 2,

1045 * 2,     -2988 * 2,     -6886 * 2,

1018 * 2,     -3544 * 2,     -5868 * 2,

2581 * 2,     -3527 * 2,     -5340 * 2,

441 * 2,     -2748 * 2,     -5208 * 2,

1814 * 2,     -3242 * 2,     -4644 * 2,

1316 * 2,     -3962 * 2,     -6696 * 2,

1858 * 2,     -2301 * 2,     -4189 * 2,

-49 * 2,     -3715 * 2,     -6801 * 2,

-16 * 2,     -4707 * 2,     -6171 * 2,

1248 * 2,     -4592 * 2,     -7268 * 2,

1512 * 2,     -3646 * 2,     -7795 * 2,

-1013 * 2,     -2955 * 2,     -6031 * 2,

-92 * 2,     -2204 * 2,     -7396 * 2,

255 * 2,     -4976 * 2,     -7893 * 2,

3177 * 2,     -4114 * 2,     -6632 * 2,

1920 * 2,     -5053 * 2,     -9114 * 2,

3872 * 2,     -3048 * 2,     -7641 * 2,

4455 * 2,     -3457 * 2,     -6789 * 2,

3806 * 2,     -2934 * 2,     -4924 * 2,

5509 * 2,     -2502 * 2,     -6742 * 2,

5692 * 2,     -1390 * 2,     -5591 * 2,

6772 * 2,     -2446 * 2,     -4848 * 2,

5667 * 2,     -1925 * 2,     -3815 * 2,

4441 * 2,       625 * 2,     -6528 * 2,

5532 * 2,     -1052 * 2,     -3049 * 2,

7978 * 2,      -858 * 2,     -6705 * 2,

3643 * 2,      1692 * 2,     -5218 * 2,

5975 * 2,      2763 * 2,     -4375 * 2,

2361 * 2,       891 * 2,     -5213 * 2,

1718 * 2,      -991 * 2,     -4008 * 2,

1651 * 2,       -61 * 2,     -3633 * 2,

4489 * 2,      3255 * 2,     -2982 * 2,

1208 * 2,      -990 * 2,     -2970 * 2,

-338 * 2,     -2055 * 2,     -4560 * 2,

6498 * 2,      1132 * 2,      -164 * 2,

7336 * 2,       403 * 2,      -675 * 2,

8818 * 2,      1050 * 2,      -967 * 2,

8652 * 2,      1072 * 2,      -293 * 2,

8291 * 2,      1571 * 2,       303 * 2,

9525 * 2,       235 * 2,     -1134 * 2,

9652 * 2,      1049 * 2,     -1668 * 2,

9069 * 2,      2296 * 2,       828 * 2,

9526 * 2,      2265 * 2,      1359 * 2,

10787 * 2,     -1109 * 2,     -3223 * 2,

7767 * 2,      5486 * 2,     -3086 * 2,

11690 * 2,     -1931 * 2,     -4657 * 2,

8997 * 2,     -3150 * 2,     -5482 * 2,

12107 * 2,      -782 * 2,     -5986 * 2,

13013 * 2,       511 * 2,     -5164 * 2,

14172 * 2,       840 * 2,     -4841 * 2,

14794 * 2,      7792 * 2,     -3028 * 2,

14596 * 2,      7581 * 2,      -813 * 2,

8808 * 2,      7531 * 2,      2912 * 2,

13790 * 2,     10971 * 2,     -1294 * 2 };

/*-----------------------------------------*

* 3nd codebook:   lsp6 to lsp9            *

*-----------------------------------------*/

static Word16 dico3_clsp[taille_dic3*dim_dic3] = {

2258 * 2,     -3027 * 2,    -14154 * 2,    -15275 * 2,

2617 * 2,     -2421 * 2,    -13423 * 2,    -14666 * 2,

1491 * 2,     -4636 * 2,    -13353 * 2,    -15066 * 2,

4111 * 2,     -2037 * 2,    -13313 * 2,    -14464 * 2,

2450 * 2,     -2406 * 2,    -12501 * 2,    -14033 * 2,

2373 * 2,     -4492 * 2,    -11954 * 2,    -15115 * 2,

3913 * 2,     -3857 * 2,    -11915 * 2,    -15152 * 2,

3423 * 2,     -1758 * 2,    -11741 * 2,    -15053 * 2,

3765 * 2,     -3882 * 2,    -12788 * 2,    -13519 * 2,

1562 * 2,     -1771 * 2,    -11644 * 2,    -14135 * 2,

1075 * 2,     -4880 * 2,    -12649 * 2,    -14058 * 2,

3638 * 2,     -3092 * 2,    -11893 * 2,    -13136 * 2,

1839 * 2,     -3454 * 2,    -12190 * 2,    -13087 * 2,

541 * 2,     -3451 * 2,    -11602 * 2,    -13865 * 2,

1557 * 2,     -4501 * 2,    -11314 * 2,    -12939 * 2,

872 * 2,     -5493 * 2,    -11547 * 2,    -14155 * 2,

-297 * 2,     -5352 * 2,    -12385 * 2,    -13504 * 2,

123 * 2,     -5666 * 2,    -11671 * 2,    -12907 * 2,

618 * 2,     -5376 * 2,    -10600 * 2,    -13291 * 2,

-1370 * 2,     -3979 * 2,    -11880 * 2,    -13663 * 2,

-1024 * 2,     -5949 * 2,    -10829 * 2,    -13536 * 2,

-1721 * 2,     -5448 * 2,    -11365 * 2,    -13776 * 2,

-641 * 2,     -6511 * 2,    -11878 * 2,    -14002 * 2,

-1514 * 2,     -6290 * 2,    -12143 * 2,    -13142 * 2,

-1583 * 2,     -4767 * 2,    -11486 * 2,    -12786 * 2,

-1183 * 2,     -5072 * 2,    -10358 * 2,    -13940 * 2,

-2687 * 2,     -4541 * 2,    -10928 * 2,    -13507 * 2,

-2919 * 2,     -5785 * 2,    -11747 * 2,    -13494 * 2,

-2538 * 2,     -6619 * 2,    -11027 * 2,    -13192 * 2,

-2446 * 2,     -5705 * 2,    -11156 * 2,    -12698 * 2,

-2043 * 2,     -5217 * 2,    -10126 * 2,    -13152 * 2,

-3314 * 2,     -5992 * 2,    -10494 * 2,    -13277 * 2,

-3380 * 2,     -4989 * 2,    -10032 * 2,    -13357 * 2,

-4071 * 2,     -5683 * 2,    -11128 * 2,    -13624 * 2,

-3464 * 2,     -4918 * 2,    -11331 * 2,    -13973 * 2,

-3331 * 2,     -6676 * 2,    -11238 * 2,    -13913 * 2,

-3686 * 2,     -6545 * 2,    -11198 * 2,    -12740 * 2,

-4633 * 2,     -5939 * 2,    -10395 * 2,    -13663 * 2,

-4443 * 2,     -6846 * 2,    -11017 * 2,    -13196 * 2,

-4066 * 2,     -6849 * 2,    -10301 * 2,    -13913 * 2,

-4291 * 2,     -6407 * 2,    -11398 * 2,    -14119 * 2,

-5002 * 2,     -6434 * 2,    -11583 * 2,    -13789 * 2,

-5310 * 2,     -6544 * 2,    -10539 * 2,    -13746 * 2,

-5112 * 2,     -7394 * 2,    -11164 * 2,    -13567 * 2,

-4827 * 2,     -6971 * 2,    -10001 * 2,    -13369 * 2,

-5657 * 2,     -7140 * 2,    -10365 * 2,    -13493 * 2,

-5190 * 2,     -8027 * 2,    -10599 * 2,    -13415 * 2,

-5907 * 2,     -7080 * 2,    -11187 * 2,    -14166 * 2,

-6284 * 2,     -7633 * 2,    -10747 * 2,    -13860 * 2,

-6051 * 2,     -7745 * 2,    -11537 * 2,    -13737 * 2,

-5341 * 2,     -8328 * 2,    -11248 * 2,    -13067 * 2,

-6310 * 2,     -8473 * 2,    -11362 * 2,    -13596 * 2,

-6467 * 2,     -8073 * 2,    -10692 * 2,    -12970 * 2,

-7180 * 2,     -8492 * 2,    -10931 * 2,    -13343 * 2,

-6977 * 2,     -8046 * 2,    -11613 * 2,    -14006 * 2,

-5705 * 2,     -7955 * 2,    -12189 * 2,    -13287 * 2,

-6190 * 2,     -8455 * 2,    -11824 * 2,    -12820 * 2,

-6802 * 2,     -8892 * 2,    -12057 * 2,    -13555 * 2,

-5609 * 2,     -9078 * 2,    -11130 * 2,    -13067 * 2,

-7282 * 2,     -9224 * 2,    -11146 * 2,    -13507 * 2,

-5587 * 2,     -8876 * 2,    -12187 * 2,    -13602 * 2,

-6157 * 2,     -9509 * 2,    -12117 * 2,    -13104 * 2,

-5840 * 2,     -9545 * 2,    -11672 * 2,    -13785 * 2,

-5068 * 2,     -8786 * 2,    -11683 * 2,    -13401 * 2,

-6344 * 2,     -9137 * 2,    -12671 * 2,    -13555 * 2,

-6655 * 2,    -10179 * 2,    -11980 * 2,    -13370 * 2,

-6242 * 2,    -10092 * 2,    -12522 * 2,    -13432 * 2,

-5821 * 2,     -9271 * 2,    -12552 * 2,    -14012 * 2,

-7085 * 2,     -9616 * 2,    -12276 * 2,    -13968 * 2,

-7061 * 2,     -9362 * 2,    -12863 * 2,    -13880 * 2,

-6592 * 2,     -8328 * 2,    -12460 * 2,    -13845 * 2,

-6286 * 2,     -8804 * 2,    -12945 * 2,    -14148 * 2,

-6710 * 2,    -10050 * 2,    -13020 * 2,    -14176 * 2,

-6713 * 2,    -10374 * 2,    -12927 * 2,    -13653 * 2,

-6869 * 2,     -8974 * 2,    -13313 * 2,    -14071 * 2,

-6771 * 2,     -9578 * 2,    -13010 * 2,    -14529 * 2,

-7836 * 2,     -9434 * 2,    -13091 * 2,    -14286 * 2,

-7582 * 2,    -10082 * 2,    -13311 * 2,    -14192 * 2,

-7685 * 2,     -8768 * 2,    -12554 * 2,    -14190 * 2,

-6660 * 2,     -9185 * 2,    -13614 * 2,    -14547 * 2,

-7287 * 2,     -9994 * 2,    -13761 * 2,    -14424 * 2,

-6119 * 2,    -10003 * 2,    -13519 * 2,    -14326 * 2,

-7610 * 2,     -8870 * 2,    -13578 * 2,    -14780 * 2,

-7944 * 2,     -9765 * 2,    -13469 * 2,    -14817 * 2,

-7068 * 2,    -10465 * 2,    -13490 * 2,    -14913 * 2,

-6437 * 2,     -9902 * 2,    -13996 * 2,    -14643 * 2,

-6860 * 2,     -9320 * 2,    -14192 * 2,    -14997 * 2,

-6033 * 2,     -9723 * 2,    -13600 * 2,    -15112 * 2,

-7312 * 2,    -10482 * 2,    -14253 * 2,    -15107 * 2,

-6026 * 2,    -10794 * 2,    -13901 * 2,    -14777 * 2,

-7911 * 2,    -10718 * 2,    -14030 * 2,    -14709 * 2,

-6908 * 2,    -11331 * 2,    -13939 * 2,    -15039 * 2,

-5948 * 2,    -10946 * 2,    -13395 * 2,    -14592 * 2,

-5903 * 2,     -9715 * 2,    -14491 * 2,    -15166 * 2,

-7071 * 2,    -11312 * 2,    -14446 * 2,    -15360 * 2,

-6732 * 2,    -10351 * 2,    -13546 * 2,    -15582 * 2,

-7547 * 2,    -11517 * 2,    -13733 * 2,    -14499 * 2,

-8135 * 2,    -11430 * 2,    -14394 * 2,    -14974 * 2,

-8436 * 2,    -11182 * 2,    -13843 * 2,    -14806 * 2,

-7833 * 2,    -12031 * 2,    -14002 * 2,    -15207 * 2,

-8091 * 2,    -10713 * 2,    -13635 * 2,    -14465 * 2,

-8509 * 2,    -11642 * 2,    -13383 * 2,    -15333 * 2,

-9007 * 2,    -11657 * 2,    -14100 * 2,    -14737 * 2,

-8815 * 2,    -11159 * 2,    -13312 * 2,    -14700 * 2,

-9207 * 2,    -11913 * 2,    -13759 * 2,    -14560 * 2,

-9285 * 2,    -10831 * 2,    -13848 * 2,    -14694 * 2,

-8597 * 2,    -11303 * 2,    -13516 * 2,    -14163 * 2,

-9264 * 2,    -11873 * 2,    -14171 * 2,    -15153 * 2,

-10039 * 2,    -11402 * 2,    -13745 * 2,    -14729 * 2,

-9459 * 2,    -11244 * 2,    -14384 * 2,    -14989 * 2,

-9566 * 2,    -10760 * 2,    -13370 * 2,    -14409 * 2,

-8917 * 2,    -10214 * 2,    -13810 * 2,    -14916 * 2,

-9932 * 2,    -11413 * 2,    -13003 * 2,    -14482 * 2,

-8704 * 2,    -10367 * 2,    -13309 * 2,    -14295 * 2,

-10370 * 2,    -11827 * 2,    -13525 * 2,    -14295 * 2,

-9682 * 2,    -11721 * 2,    -13258 * 2,    -13944 * 2,

-8606 * 2,    -11043 * 2,    -13115 * 2,    -13931 * 2,

-9323 * 2,    -10436 * 2,    -12611 * 2,    -14175 * 2,

-8799 * 2,    -10894 * 2,    -12675 * 2,    -13938 * 2,

-8745 * 2,    -11134 * 2,    -12565 * 2,    -14705 * 2,

-8699 * 2,    -11343 * 2,    -12686 * 2,    -13532 * 2,

-8615 * 2,    -10184 * 2,    -12313 * 2,    -13908 * 2,

-8013 * 2,    -10163 * 2,    -12923 * 2,    -13918 * 2,

-7827 * 2,    -10632 * 2,    -12342 * 2,    -13684 * 2,

-8801 * 2,    -10562 * 2,    -12012 * 2,    -13314 * 2,

-7782 * 2,     -9941 * 2,    -12520 * 2,    -13508 * 2,

-7184 * 2,    -10374 * 2,    -12445 * 2,    -14056 * 2,

-8329 * 2,     -9450 * 2,    -12339 * 2,    -14009 * 2,

-8129 * 2,     -9827 * 2,    -11637 * 2,    -13458 * 2,

-7572 * 2,     -9148 * 2,    -12059 * 2,    -13390 * 2,

-8677 * 2,     -9708 * 2,    -12866 * 2,    -14477 * 2,

-7594 * 2,    -10662 * 2,    -12969 * 2,    -14495 * 2,

-6901 * 2,     -8991 * 2,    -12128 * 2,    -14424 * 2,

-7217 * 2,     -8242 * 2,    -12871 * 2,    -14508 * 2,

-7034 * 2,    -10885 * 2,    -13420 * 2,    -14158 * 2,

-6348 * 2,     -9423 * 2,    -12826 * 2,    -14969 * 2,

-6111 * 2,    -10844 * 2,    -12297 * 2,    -14546 * 2,

-5493 * 2,    -10258 * 2,    -12720 * 2,    -14164 * 2,

-5372 * 2,     -9350 * 2,    -12931 * 2,    -14455 * 2,

-5654 * 2,     -9935 * 2,    -12744 * 2,    -15081 * 2,

-5464 * 2,     -8605 * 2,    -13009 * 2,    -14723 * 2,

-5012 * 2,    -10523 * 2,    -13054 * 2,    -14874 * 2,

-4898 * 2,     -9333 * 2,    -13567 * 2,    -14618 * 2,

-5347 * 2,     -9040 * 2,    -12207 * 2,    -15051 * 2,

-4353 * 2,     -9886 * 2,    -12281 * 2,    -14405 * 2,

-4458 * 2,     -8878 * 2,    -12945 * 2,    -14333 * 2,

-4073 * 2,     -8797 * 2,    -12507 * 2,    -14647 * 2,

-3601 * 2,     -8943 * 2,    -13112 * 2,    -14775 * 2,

-4518 * 2,     -9080 * 2,    -13530 * 2,    -15080 * 2,

-4159 * 2,     -9772 * 2,    -12836 * 2,    -14063 * 2,

-4909 * 2,     -8973 * 2,    -12211 * 2,    -14094 * 2,

-4961 * 2,     -8262 * 2,    -12126 * 2,    -14627 * 2,

-4409 * 2,     -8219 * 2,    -12531 * 2,    -14028 * 2,

-4435 * 2,     -7684 * 2,    -12834 * 2,    -14629 * 2,

-3529 * 2,     -7962 * 2,    -12876 * 2,    -14255 * 2,

-3751 * 2,     -8655 * 2,    -13494 * 2,    -14204 * 2,

-5246 * 2,     -8294 * 2,    -13381 * 2,    -14163 * 2,

-5690 * 2,     -8131 * 2,    -12666 * 2,    -14052 * 2,

-4438 * 2,     -7338 * 2,    -13222 * 2,    -13975 * 2,

-5584 * 2,     -7365 * 2,    -13205 * 2,    -14332 * 2,

-4946 * 2,     -7365 * 2,    -12549 * 2,    -13708 * 2,

-3934 * 2,     -7160 * 2,    -12882 * 2,    -13679 * 2,

-5238 * 2,     -8579 * 2,    -12846 * 2,    -13617 * 2,

-4910 * 2,     -6631 * 2,    -12587 * 2,    -14270 * 2,

-5542 * 2,     -7117 * 2,    -12123 * 2,    -13862 * 2,

-4012 * 2,     -7223 * 2,    -12212 * 2,    -13973 * 2,

-5125 * 2,     -8049 * 2,    -11943 * 2,    -13886 * 2,

-3879 * 2,     -7351 * 2,    -12375 * 2,    -13382 * 2,

-4600 * 2,     -6946 * 2,    -12021 * 2,    -13168 * 2,

-4605 * 2,     -7897 * 2,    -11739 * 2,    -13337 * 2,

-4082 * 2,     -8445 * 2,    -12068 * 2,    -13597 * 2,

-3665 * 2,     -7463 * 2,    -11557 * 2,    -13688 * 2,

-3867 * 2,     -7581 * 2,    -11625 * 2,    -12775 * 2,

-3722 * 2,     -7888 * 2,    -10925 * 2,    -13470 * 2,

-3031 * 2,     -8207 * 2,    -12287 * 2,    -13101 * 2,

-2639 * 2,     -7377 * 2,    -11750 * 2,    -13247 * 2,

-3291 * 2,     -8811 * 2,    -11872 * 2,    -12968 * 2,

-3759 * 2,     -8500 * 2,    -10902 * 2,    -12955 * 2,

-4233 * 2,     -8614 * 2,    -11788 * 2,    -12483 * 2,

-3832 * 2,     -8933 * 2,    -11332 * 2,    -12544 * 2,

-2269 * 2,     -8265 * 2,    -11190 * 2,    -12700 * 2,

-2151 * 2,     -8973 * 2,    -11367 * 2,    -13237 * 2,

-3700 * 2,     -8547 * 2,    -10965 * 2,    -12117 * 2,

-2425 * 2,     -8480 * 2,    -10385 * 2,    -13061 * 2,

-4135 * 2,     -7675 * 2,    -10670 * 2,    -12717 * 2,

-2463 * 2,     -7452 * 2,    -10692 * 2,    -12495 * 2,

-4116 * 2,     -7373 * 2,    -11309 * 2,    -12239 * 2,

-3509 * 2,     -7888 * 2,    -10270 * 2,    -11879 * 2,

-3150 * 2,     -7234 * 2,    -10169 * 2,    -13063 * 2,

-3005 * 2,     -7089 * 2,    -10930 * 2,    -11839 * 2,

-3384 * 2,     -7758 * 2,     -9501 * 2,    -12527 * 2,

-4448 * 2,     -7776 * 2,    -10006 * 2,    -12915 * 2,

-4547 * 2,     -6600 * 2,    -10270 * 2,    -12426 * 2,

-3027 * 2,     -6337 * 2,    -10287 * 2,    -12059 * 2,

-3216 * 2,     -6586 * 2,     -9655 * 2,    -12890 * 2,

-4121 * 2,     -6902 * 2,     -9225 * 2,    -12883 * 2,

-2915 * 2,     -5839 * 2,     -9437 * 2,    -12387 * 2,

-4060 * 2,     -5770 * 2,     -9461 * 2,    -13002 * 2,

-2924 * 2,     -7160 * 2,     -8755 * 2,    -12284 * 2,

-5042 * 2,     -6390 * 2,     -9057 * 2,    -13100 * 2,

-4201 * 2,     -5887 * 2,     -8410 * 2,    -13116 * 2,

-2889 * 2,     -6473 * 2,     -8294 * 2,    -13110 * 2,

-3461 * 2,     -5187 * 2,     -8362 * 2,    -12949 * 2,

-4651 * 2,     -6493 * 2,     -8009 * 2,    -12209 * 2,

-5323 * 2,     -7101 * 2,     -9166 * 2,    -12660 * 2,

-3232 * 2,     -5772 * 2,     -7693 * 2,    -11727 * 2,

-3954 * 2,     -6435 * 2,     -8949 * 2,    -11244 * 2,

-2691 * 2,     -6185 * 2,     -8260 * 2,    -11380 * 2,

-2648 * 2,     -5363 * 2,     -8576 * 2,    -11475 * 2,

-5102 * 2,     -6875 * 2,     -8511 * 2,    -11190 * 2,

-3824 * 2,     -6717 * 2,     -9987 * 2,    -11387 * 2,

-2425 * 2,     -6443 * 2,     -9186 * 2,    -10955 * 2,

-2216 * 2,     -7118 * 2,     -9656 * 2,    -11662 * 2,

-2027 * 2,     -5643 * 2,     -9985 * 2,    -11669 * 2,

-1582 * 2,     -6236 * 2,     -9060 * 2,    -12331 * 2,

-2068 * 2,     -7313 * 2,    -10351 * 2,    -11582 * 2,

-755 * 2,     -6844 * 2,     -9710 * 2,    -11707 * 2,

-1518 * 2,     -7717 * 2,    -10100 * 2,    -12380 * 2,

-744 * 2,     -6089 * 2,    -10481 * 2,    -11823 * 2,

-1635 * 2,     -5886 * 2,    -10856 * 2,    -12158 * 2,

-404 * 2,     -7526 * 2,    -10553 * 2,    -12547 * 2,

-1336 * 2,     -7077 * 2,    -11289 * 2,    -12512 * 2,

500 * 2,     -6955 * 2,    -10065 * 2,    -12438 * 2,

-17 * 2,     -5673 * 2,    -10843 * 2,    -12286 * 2,

-815 * 2,     -6924 * 2,     -9235 * 2,    -12999 * 2,

88 * 2,     -7143 * 2,    -11346 * 2,    -13241 * 2,

-1661 * 2,     -7418 * 2,    -10376 * 2,    -13501 * 2,

-1128 * 2,     -8022 * 2,    -11858 * 2,    -12875 * 2,

-1097 * 2,     -8331 * 2,    -11141 * 2,    -13565 * 2,

-1839 * 2,     -7119 * 2,    -11451 * 2,    -13819 * 2,

-331 * 2,     -7501 * 2,     -9970 * 2,    -13978 * 2,

-2238 * 2,     -6683 * 2,    -10486 * 2,    -13947 * 2,

-583 * 2,     -7350 * 2,    -11045 * 2,    -14570 * 2,

-2354 * 2,     -8320 * 2,    -11714 * 2,    -13950 * 2,

-1573 * 2,     -8210 * 2,    -12170 * 2,    -13626 * 2,

-2298 * 2,     -7840 * 2,    -11162 * 2,    -14578 * 2,

-2478 * 2,     -6535 * 2,    -11432 * 2,    -14457 * 2,

-2071 * 2,     -6924 * 2,    -12417 * 2,    -14293 * 2,

-1423 * 2,     -6187 * 2,    -11723 * 2,    -14768 * 2,

-2987 * 2,     -6901 * 2,    -12229 * 2,    -13837 * 2,

-2254 * 2,     -8289 * 2,    -12343 * 2,    -14663 * 2,

-3354 * 2,     -7942 * 2,    -12155 * 2,    -14284 * 2,

-3437 * 2,     -6531 * 2,    -12137 * 2,    -14527 * 2,

-2882 * 2,     -7340 * 2,    -11837 * 2,    -15079 * 2,

-3021 * 2,     -7084 * 2,    -12860 * 2,    -14905 * 2,

-2649 * 2,     -5888 * 2,    -12910 * 2,    -14579 * 2,

-1922 * 2,     -6363 * 2,    -12567 * 2,    -15093 * 2,

-3917 * 2,     -7895 * 2,    -12288 * 2,    -15009 * 2,

-4268 * 2,     -7384 * 2,    -11802 * 2,    -14440 * 2,

-3114 * 2,     -5648 * 2,    -12388 * 2,    -15124 * 2,

-4192 * 2,     -6273 * 2,    -12999 * 2,    -14330 * 2,

-3604 * 2,     -7229 * 2,    -13430 * 2,    -14509 * 2,

-4506 * 2,     -5775 * 2,    -12538 * 2,    -14447 * 2,

-4797 * 2,     -6306 * 2,    -13096 * 2,    -14957 * 2,

-3669 * 2,     -5302 * 2,    -12349 * 2,    -14277 * 2,

-3340 * 2,     -5796 * 2,    -13124 * 2,    -14068 * 2,

-3591 * 2,     -5380 * 2,    -13440 * 2,    -14707 * 2,

-2836 * 2,     -6184 * 2,    -13714 * 2,    -14435 * 2,

-4821 * 2,     -6920 * 2,    -13606 * 2,    -14505 * 2,

-3866 * 2,     -6360 * 2,    -14015 * 2,    -14772 * 2,

-4129 * 2,     -6015 * 2,    -12428 * 2,    -13774 * 2,

-2525 * 2,     -7214 * 2,    -13270 * 2,    -14143 * 2,

-2688 * 2,     -5151 * 2,    -12312 * 2,    -14060 * 2,

-2841 * 2,     -4505 * 2,    -12737 * 2,    -14704 * 2,

-2167 * 2,     -4432 * 2,    -13236 * 2,    -14351 * 2,

-1674 * 2,     -5285 * 2,    -13542 * 2,    -14729 * 2,

-1682 * 2,     -5312 * 2,    -12819 * 2,    -13912 * 2,

-2689 * 2,     -5766 * 2,    -12461 * 2,    -13395 * 2,

-522 * 2,     -5970 * 2,    -12628 * 2,    -14436 * 2,

-37 * 2,     -6079 * 2,    -13267 * 2,    -14165 * 2,

-1251 * 2,     -3344 * 2,    -12940 * 2,    -14242 * 2,

-2042 * 2,     -3653 * 2,    -12979 * 2,    -14880 * 2,

-1563 * 2,     -4470 * 2,    -11756 * 2,    -14774 * 2,

-252 * 2,     -4094 * 2,    -12046 * 2,    -14881 * 2,

-2590 * 2,     -4088 * 2,    -11559 * 2,    -14093 * 2,

-1817 * 2,     -3406 * 2,    -11146 * 2,    -14214 * 2,

-2347 * 2,     -4840 * 2,    -10723 * 2,    -14719 * 2,

-333 * 2,     -4997 * 2,    -10524 * 2,    -14779 * 2,

-1048 * 2,     -3769 * 2,    -10195 * 2,    -14339 * 2,

-2502 * 2,     -5724 * 2,    -10197 * 2,    -14570 * 2,

-2069 * 2,     -4009 * 2,     -9460 * 2,    -14313 * 2,

-3075 * 2,     -4500 * 2,     -9738 * 2,    -14703 * 2,

-3665 * 2,     -5650 * 2,    -10968 * 2,    -14696 * 2,

-3038 * 2,     -5592 * 2,     -9134 * 2,    -14096 * 2,

-1683 * 2,     -5635 * 2,     -8743 * 2,    -14555 * 2,

-3478 * 2,     -6306 * 2,     -9632 * 2,    -14834 * 2,

-2310 * 2,     -6941 * 2,     -9412 * 2,    -14113 * 2,

-1789 * 2,     -7180 * 2,    -10026 * 2,    -14917 * 2,

-3649 * 2,     -7174 * 2,    -10798 * 2,    -14964 * 2,

-4884 * 2,     -6782 * 2,     -9331 * 2,    -14539 * 2,

-4130 * 2,     -7783 * 2,     -9704 * 2,    -14296 * 2,

-3209 * 2,     -8267 * 2,    -10448 * 2,    -14139 * 2,

-1523 * 2,     -8503 * 2,    -10050 * 2,    -14744 * 2,

-4967 * 2,     -7233 * 2,    -10980 * 2,    -14622 * 2,

-4405 * 2,     -8268 * 2,    -11101 * 2,    -14164 * 2,

-4444 * 2,     -7988 * 2,    -11414 * 2,    -15169 * 2,

-5731 * 2,     -7986 * 2,    -10506 * 2,    -14819 * 2,

-3533 * 2,     -8866 * 2,    -11731 * 2,    -14582 * 2,

-3904 * 2,     -9181 * 2,    -11170 * 2,    -13855 * 2,

-2560 * 2,     -9499 * 2,    -11078 * 2,    -14844 * 2,

-5102 * 2,     -9641 * 2,    -11597 * 2,    -14571 * 2,

-5762 * 2,     -8671 * 2,    -11671 * 2,    -14272 * 2,

-5555 * 2,     -7699 * 2,    -12183 * 2,    -14734 * 2,

-3257 * 2,     -8774 * 2,    -12130 * 2,    -15150 * 2,

-6385 * 2,     -8877 * 2,    -10879 * 2,    -14952 * 2,

-2639 * 2,     -9484 * 2,    -12495 * 2,    -14502 * 2,

-4573 * 2,    -10038 * 2,    -12044 * 2,    -13615 * 2,

-2839 * 2,     -9657 * 2,    -11759 * 2,    -13773 * 2,

-4563 * 2,     -8769 * 2,    -12475 * 2,    -13275 * 2,

-3931 * 2,     -9965 * 2,    -12686 * 2,    -13508 * 2,

-2446 * 2,     -9300 * 2,    -12342 * 2,    -13708 * 2,

-2678 * 2,     -8421 * 2,    -12910 * 2,    -13811 * 2,

-4683 * 2,     -9297 * 2,    -12269 * 2,    -12973 * 2,

-2442 * 2,    -10261 * 2,    -13015 * 2,    -14122 * 2,

-4892 * 2,     -9479 * 2,    -11754 * 2,    -12898 * 2,

-5478 * 2,     -9556 * 2,    -13200 * 2,    -13844 * 2,

-4707 * 2,    -10400 * 2,    -13392 * 2,    -14169 * 2,

-3466 * 2,     -9494 * 2,    -13865 * 2,    -14327 * 2,

-3353 * 2,     -8233 * 2,    -13869 * 2,    -14575 * 2,

-5703 * 2,     -8553 * 2,    -13673 * 2,    -14414 * 2,

-4650 * 2,     -9988 * 2,    -14209 * 2,    -14789 * 2,

-5839 * 2,     -8786 * 2,    -13931 * 2,    -14949 * 2,

-5016 * 2,     -7751 * 2,    -13859 * 2,    -14823 * 2,

-3606 * 2,     -8092 * 2,    -14241 * 2,    -14949 * 2,

-4620 * 2,     -7842 * 2,    -14347 * 2,    -15262 * 2,

-3504 * 2,     -7468 * 2,    -13683 * 2,    -15120 * 2,

-4662 * 2,     -8042 * 2,    -12990 * 2,    -15281 * 2,

-5991 * 2,     -7721 * 2,    -13275 * 2,    -15019 * 2,

-5360 * 2,     -7044 * 2,    -13474 * 2,    -15424 * 2,

-4058 * 2,     -8833 * 2,    -12783 * 2,    -15391 * 2,

-3911 * 2,     -8840 * 2,    -13626 * 2,    -15721 * 2,

-4785 * 2,     -9441 * 2,    -13014 * 2,    -15382 * 2,

-5789 * 2,     -8844 * 2,    -12818 * 2,    -15475 * 2,

-3056 * 2,     -8107 * 2,    -12624 * 2,    -15369 * 2,

-3757 * 2,     -6875 * 2,    -12637 * 2,    -15623 * 2,

-2557 * 2,     -8635 * 2,    -13284 * 2,    -15087 * 2,

-3702 * 2,     -9042 * 2,    -12656 * 2,    -15986 * 2,

-4066 * 2,    -10051 * 2,    -12271 * 2,    -15442 * 2,

-2533 * 2,    -10409 * 2,    -13255 * 2,    -15462 * 2,

-5357 * 2,    -10545 * 2,    -13401 * 2,    -15642 * 2,

-4009 * 2,    -10553 * 2,    -13868 * 2,    -15118 * 2,

-3004 * 2,     -9401 * 2,    -14284 * 2,    -15227 * 2,

-3917 * 2,    -10672 * 2,    -14563 * 2,    -15613 * 2,

-1729 * 2,     -9887 * 2,    -13941 * 2,    -14722 * 2,

-5558 * 2,    -10041 * 2,    -14600 * 2,    -15447 * 2,

-4949 * 2,    -11441 * 2,    -14321 * 2,    -14933 * 2,

-3688 * 2,    -11414 * 2,    -13850 * 2,    -14526 * 2,

-5148 * 2,    -12634 * 2,    -14279 * 2,    -15609 * 2,

-5913 * 2,    -11034 * 2,    -14790 * 2,    -15975 * 2,

-6840 * 2,    -12206 * 2,    -14079 * 2,    -15646 * 2,

-7445 * 2,    -11687 * 2,    -14823 * 2,    -15405 * 2,

-7657 * 2,    -11124 * 2,    -14105 * 2,    -15748 * 2,

-7460 * 2,    -11012 * 2,    -13241 * 2,    -15590 * 2,

-7816 * 2,    -11903 * 2,    -15113 * 2,    -15635 * 2,

-7283 * 2,    -12023 * 2,    -14984 * 2,    -16015 * 2,

-8586 * 2,    -12702 * 2,    -14420 * 2,    -15665 * 2,

-8768 * 2,    -11668 * 2,    -14871 * 2,    -16066 * 2,

-8938 * 2,    -11004 * 2,    -14763 * 2,    -15378 * 2,

-8517 * 2,    -10255 * 2,    -14626 * 2,    -15736 * 2,

-9509 * 2,    -10924 * 2,    -14371 * 2,    -15669 * 2,

-9560 * 2,    -12148 * 2,    -14681 * 2,    -15573 * 2,

-9524 * 2,    -11637 * 2,    -13931 * 2,    -15687 * 2,

-9952 * 2,    -11446 * 2,    -14749 * 2,    -16022 * 2,

-10589 * 2,    -11733 * 2,    -14404 * 2,    -15410 * 2,

-10599 * 2,    -12143 * 2,    -14897 * 2,    -15877 * 2,

-9402 * 2,    -12839 * 2,    -14943 * 2,    -15516 * 2,

-8972 * 2,    -12682 * 2,    -14666 * 2,    -15233 * 2,

-10771 * 2,    -13173 * 2,    -14355 * 2,    -15621 * 2,

-10297 * 2,    -13564 * 2,    -14733 * 2,    -15734 * 2,

-10822 * 2,    -12937 * 2,    -14477 * 2,    -15042 * 2,

-11142 * 2,    -12785 * 2,    -15139 * 2,    -15905 * 2,

-10101 * 2,    -12649 * 2,    -14283 * 2,    -14883 * 2,

-10035 * 2,    -12902 * 2,    -13731 * 2,    -15264 * 2,

-11322 * 2,    -13805 * 2,    -15031 * 2,    -15382 * 2,

-12119 * 2,    -13184 * 2,    -14533 * 2,    -15272 * 2,

-10453 * 2,    -12308 * 2,    -14055 * 2,    -14623 * 2,

-10909 * 2,    -12027 * 2,    -13531 * 2,    -15149 * 2,

-10569 * 2,    -12502 * 2,    -13783 * 2,    -14390 * 2,

-12295 * 2,    -13452 * 2,    -14519 * 2,    -16014 * 2,

-9070 * 2,    -13916 * 2,    -15100 * 2,    -15778 * 2,

-9173 * 2,    -13031 * 2,    -15055 * 2,    -16027 * 2,

-10759 * 2,    -13738 * 2,    -15398 * 2,    -16195 * 2,

-10454 * 2,    -11762 * 2,    -15233 * 2,    -16185 * 2,

-8931 * 2,    -13657 * 2,    -15532 * 2,    -15997 * 2,

-10846 * 2,    -14286 * 2,    -15519 * 2,    -16176 * 2,

-7775 * 2,    -10326 * 2,    -15224 * 2,    -15794 * 2,

-8874 * 2,    -10860 * 2,    -13493 * 2,    -15558 * 2,

-7842 * 2,    -10236 * 2,    -14047 * 2,    -15982 * 2,

-8241 * 2,     -9518 * 2,    -14118 * 2,    -15253 * 2,

-7417 * 2,     -8839 * 2,    -14502 * 2,    -15988 * 2,

-6511 * 2,     -8764 * 2,    -14745 * 2,    -15662 * 2,

-6592 * 2,     -9218 * 2,    -13452 * 2,    -15816 * 2,

-8666 * 2,    -10330 * 2,    -12824 * 2,    -15364 * 2,

-6878 * 2,     -8214 * 2,    -14008 * 2,    -15242 * 2,

-6472 * 2,     -9944 * 2,    -12580 * 2,    -15630 * 2,

-6093 * 2,     -7786 * 2,    -14146 * 2,    -15705 * 2,

-7140 * 2,     -9175 * 2,    -12134 * 2,    -15479 * 2,

-7465 * 2,    -10291 * 2,    -12240 * 2,    -15235 * 2,

-6556 * 2,     -8071 * 2,    -13251 * 2,    -14474 * 2,

-6529 * 2,     -8031 * 2,    -12129 * 2,    -14681 * 2,

-5876 * 2,    -10227 * 2,    -11466 * 2,    -15307 * 2,

-5654 * 2,     -6943 * 2,    -12564 * 2,    -14736 * 2,

-6353 * 2,     -7469 * 2,    -12201 * 2,    -14350 * 2,

-5627 * 2,     -7728 * 2,    -11689 * 2,    -15788 * 2,

-5197 * 2,     -6409 * 2,    -11655 * 2,    -14597 * 2,

-4239 * 2,     -6118 * 2,    -11432 * 2,    -15123 * 2,

-4436 * 2,     -9115 * 2,    -10839 * 2,    -15679 * 2,

-3548 * 2,     -8408 * 2,    -10345 * 2,    -15621 * 2,

-4227 * 2,     -7315 * 2,     -9749 * 2,    -15819 * 2,

-3051 * 2,     -6359 * 2,    -10763 * 2,    -15801 * 2,

-1744 * 2,     -8688 * 2,    -11352 * 2,    -15519 * 2,

-1507 * 2,     -7789 * 2,    -12284 * 2,    -15213 * 2,

-898 * 2,     -8081 * 2,    -12437 * 2,    -15816 * 2,

-1027 * 2,    -10079 * 2,    -11965 * 2,    -15213 * 2,

-734 * 2,     -7974 * 2,    -12549 * 2,    -14605 * 2,

827 * 2,     -7472 * 2,    -12192 * 2,    -15324 * 2,

-511 * 2,     -9088 * 2,    -11661 * 2,    -14363 * 2,

-1296 * 2,     -8962 * 2,    -13040 * 2,    -14289 * 2,

-1798 * 2,     -7790 * 2,    -13720 * 2,    -14889 * 2,

-1027 * 2,     -7855 * 2,    -13842 * 2,    -15336 * 2,

-438 * 2,     -6447 * 2,    -13350 * 2,    -15000 * 2,

-1645 * 2,     -7871 * 2,    -13618 * 2,    -14276 * 2,

-1796 * 2,     -7857 * 2,    -14187 * 2,    -14647 * 2,

1419 * 2,     -7430 * 2,    -13326 * 2,    -14604 * 2,

968 * 2,     -8714 * 2,    -14064 * 2,    -14746 * 2,

-1389 * 2,     -8443 * 2,    -14591 * 2,    -15035 * 2,

-2376 * 2,     -6547 * 2,    -14289 * 2,    -15131 * 2,

681 * 2,     -6039 * 2,    -14174 * 2,    -15261 * 2,

-1278 * 2,     -7426 * 2,    -14995 * 2,    -15460 * 2,

-1650 * 2,     -5306 * 2,    -14072 * 2,    -15417 * 2,

1179 * 2,     -5908 * 2,    -12475 * 2,    -14644 * 2,

156 * 2,     -7873 * 2,    -12697 * 2,    -13694 * 2,

-624 * 2,     -4770 * 2,    -12873 * 2,    -15838 * 2,

-3129 * 2,     -4907 * 2,    -13467 * 2,    -15778 * 2,

-4247 * 2,     -5883 * 2,    -14196 * 2,    -15500 * 2,

-4086 * 2,     -7081 * 2,    -14616 * 2,    -15684 * 2,

-2201 * 2,     -7651 * 2,    -12663 * 2,    -13344 * 2,

-3453 * 2,     -6630 * 2,    -12004 * 2,    -12864 * 2,

856 * 2,     -8323 * 2,    -11736 * 2,    -13622 * 2,

-2646 * 2,     -7110 * 2,    -11587 * 2,    -12505 * 2,

-5336 * 2,     -7532 * 2,    -11484 * 2,    -12670 * 2,

-5033 * 2,     -8705 * 2,    -10678 * 2,    -12334 * 2,

-4970 * 2,     -7667 * 2,    -10717 * 2,    -11807 * 2,

-5795 * 2,     -8993 * 2,    -11290 * 2,    -12255 * 2,

-4249 * 2,     -8531 * 2,     -9938 * 2,    -12187 * 2,

-5502 * 2,     -7806 * 2,     -9865 * 2,    -12731 * 2,

-5921 * 2,     -8823 * 2,    -10290 * 2,    -13093 * 2,

-6168 * 2,     -8325 * 2,    -10247 * 2,    -11776 * 2,

-6218 * 2,     -7581 * 2,     -9573 * 2,    -12938 * 2,

-4793 * 2,     -7741 * 2,     -9545 * 2,    -11334 * 2,

-6263 * 2,     -9720 * 2,    -11069 * 2,    -12886 * 2,

-6931 * 2,     -9690 * 2,    -11618 * 2,    -12771 * 2,

-4180 * 2,     -6051 * 2,     -8002 * 2,    -10238 * 2,

-2494 * 2,     -5755 * 2,     -7791 * 2,    -13517 * 2,

-2327 * 2,     -4509 * 2,     -8991 * 2,    -12941 * 2,

-2882 * 2,     -5563 * 2,     -7443 * 2,    -10727 * 2,

-1012 * 2,     -5319 * 2,     -9297 * 2,    -12505 * 2,

-1512 * 2,     -5510 * 2,     -7537 * 2,    -11602 * 2,

-1865 * 2,     -4284 * 2,     -8937 * 2,    -11243 * 2,

-675 * 2,     -5828 * 2,     -9165 * 2,    -11035 * 2,

121 * 2,     -5394 * 2,     -8187 * 2,    -12358 * 2,

-1828 * 2,     -3798 * 2,     -7161 * 2,    -12607 * 2,

-1250 * 2,     -3403 * 2,     -8971 * 2,    -12892 * 2,

82 * 2,     -3228 * 2,     -8165 * 2,    -12383 * 2,

-205 * 2,     -4263 * 2,     -9511 * 2,    -13198 * 2,

-970 * 2,     -4326 * 2,    -10376 * 2,    -12376 * 2,

771 * 2,     -5818 * 2,     -9306 * 2,    -12364 * 2,

1200 * 2,     -4560 * 2,     -9731 * 2,    -11691 * 2,

373 * 2,     -3855 * 2,    -10589 * 2,    -11908 * 2,

-806 * 2,     -2391 * 2,     -9914 * 2,    -12909 * 2,

-340 * 2,     -5531 * 2,    -10039 * 2,    -11121 * 2,

-29 * 2,     -4047 * 2,     -9090 * 2,    -10202 * 2,

-118 * 2,     -6292 * 2,     -9474 * 2,    -13728 * 2,

2151 * 2,     -6644 * 2,    -10546 * 2,    -13000 * 2,

3581 * 2,     -3940 * 2,    -10803 * 2,    -12157 * 2,

281 * 2,     -3753 * 2,     -9785 * 2,    -14179 * 2,

-970 * 2,     -4782 * 2,     -7680 * 2,    -13979 * 2,

616 * 2,     -2056 * 2,     -9585 * 2,    -14312 * 2,

301 * 2,     -1319 * 2,     -7377 * 2,    -12987 * 2,

-2860 * 2,     -4579 * 2,     -7377 * 2,    -13190 * 2,

1579 * 2,     -3254 * 2,     -5673 * 2,    -12155 * 2,

-670 * 2,     -3002 * 2,     -6917 * 2,    -10909 * 2,

1148 * 2,      -112 * 2,     -6676 * 2,    -13294 * 2,

-1456 * 2,     -4559 * 2,     -6359 * 2,    -11251 * 2,

2315 * 2,     -4495 * 2,     -6938 * 2,    -10622 * 2,

-910 * 2,     -4965 * 2,     -7196 * 2,     -9729 * 2,

-3313 * 2,     -5078 * 2,     -7349 * 2,     -9257 * 2,

2995 * 2,        30 * 2,     -6398 * 2,    -14337 * 2,

2728 * 2,      1352 * 2,     -6451 * 2,    -14372 * 2,

4050 * 2,     -5770 * 2,    -10681 * 2,    -14211 * 2,

3555 * 2,      2365 * 2,     -8304 * 2,    -14656 * 2,

3378 * 2,       470 * 2,    -11318 * 2,    -14972 * 2,

4295 * 2,       920 * 2,    -11156 * 2,    -14996 * 2,

4690 * 2,      2206 * 2,    -10216 * 2,    -14946 * 2,

5240 * 2,       661 * 2,    -11119 * 2,    -14998 * 2,

6099 * 2,      1695 * 2,     -9999 * 2,    -14947 * 2,

4890 * 2,     -1618 * 2,    -11589 * 2,    -15054 * 2,

6603 * 2,     -2072 * 2,    -11463 * 2,    -15067 * 2,

5215 * 2,      3325 * 2,     -9190 * 2,    -14925 * 2,

4800 * 2,     -1018 * 2,    -12645 * 2,    -13591 * 2,

5953 * 2,      3144 * 2,     -7673 * 2,    -14850 * 2,

4467 * 2,      3391 * 2,     -6908 * 2,    -14827 * 2,

6908 * 2,      4412 * 2,     -7205 * 2,    -14816 * 2,

5090 * 2,      3105 * 2,     -5817 * 2,    -14749 * 2,

5482 * 2,      3606 * 2,     -3761 * 2,    -14629 * 2,

4139 * 2,      2850 * 2,     -1782 * 2,    -14239 * 2,

5611 * 2,      4106 * 2,      -669 * 2,    -14497 * 2,

7300 * 2,      3950 * 2,       736 * 2,    -13497 * 2,

5376 * 2,     -5185 * 2,    -11826 * 2,    -15194 * 2,

2924 * 2,     -6522 * 2,    -12052 * 2,    -15281 * 2,

-6084 * 2,    -11147 * 2,    -12964 * 2,    -14025 * 2,

-7054 * 2,    -11706 * 2,    -13153 * 2,    -14339 * 2 };

/************************************************************************

*

*	FILENAME		:	ener_qua.tab

*

*	DESCRIPTION		:	Energy codebook in Q8.

*

************************************************************************/

#define nb_qua_ener 64

static Word16 t_qua_ener[64*2] = {

12,       48,

69,      287,

98,      589,

103,      865,

406,      561,

95,     1161,

418,      913,

465,     1288,

705,     1056,

822,      699,

1036,     1016,

842,     1353,

555,     1646,

681,     2119,

891,     1758,

1071,     1529,

1236,     1281,

1455,      962,

1661,     1279,

1416,     1514,

1243,     1768,

1110,     2088,

1349,     2406,

1480,     2035,

1558,     1762,

1770,     1554,

2027,     1319,

1952,     1030,

2281,      914,

2337,     1209,

2451,     1441,

2234,     1441,

2033,     1592,

2279,     1644,

2109,     1808,

2319,     1859,

2469,     1656,

2533,     1857,

2414,     2084,

2114,     2087,

1834,     1887,

1769,     2242,

1977,     2560,

2278,     2344,

2343,     2749,

2661,     2416,

2857,     2037,

3239,     2405,

2848,     2836,

3426,     2863,

3475,     3416,

2831,     3409,

2410,     3322,

2033,     3303,

1609,     3514,

1625,     2836,

880,     2775,

1122,     3500,

605,     3517,

88,     3532,

111,     2956,

89,     2436,

88,     1938,

74,     1526};

/************************************************************************

*

*	FILENAME		:	grid.tab

*

*	DESCRIPTION		:	Table for routine Az_Lsp().

*

*					Vector grid[] is in Q15

*

*						grid[0] = 1.0

*						grid[grid_points+1] = -1.0

*						for (i = 1; i < grid_points; i++)

*						grid[i] = cos((6.283185307*i)/(2.0*grid_points))

*

************************************************************************/

#define grid_points 60

static Word16 grid[grid_points+1] ={

32760,     32723,     32588,     32364,     32051,     31651,

31164,     30591,     29935,     29196,     28377,     27481,

26509,     25465,     24351,     23170,     21926,     20621,

19260,     17846,     16384,     14876,     13327,     11743,

10125,      8480,      6812,      5126,      3425,      1714,

-1,     -1715,     -3426,     -5127,     -6813,     -8481,

-10126,    -11744,    -13328,    -14877,    -16385,    -17847,

-19261,    -20622,    -21927,    -23171,    -24352,    -25466,

-26510,    -27482,    -28378,    -29197,    -29936,    -30592,

-31165,    -31652,    -32052,    -32365,    -32589,    -32724,

-32760};

/************************************************************************

*

*	FILENAME		:	inv_sqrt.tab

*

*	DESCRIPTION		:	Table for routine inv_sqrt().

*

************************************************************************/

static Word16 tab_inv_sqrt[49] = {

32767, 31790, 30894, 30070, 29309, 28602, 27945, 27330, 26755, 26214,

25705, 25225, 24770, 24339, 23930, 23541, 23170, 22817, 22479, 22155,

21845, 21548, 21263, 20988, 20724, 20470, 20225, 19988, 19760, 19539,

19326, 19119, 18919, 18725, 18536, 18354, 18176, 18004, 17837, 17674,

17515, 17361, 17211, 17064, 16921, 16782, 16646, 16514, 16384 };

/************************************************************************

*

*	FILENAME		:	lag_wind.tab

*

*	DESCRIPTION		:	Table of lag_window coefficents for the autocorrelation.

*

*						lag_wind[0] = 1.00000000 	(*)

*						lag_wind[1] = 0.99884027	(**)

*						lag_wind[2] = 0.99551868

*						lag_wind[3] = 0.99000722

*						lag_wind[4] = 0.98234236

*						lag_wind[5] = 0.97257471

*						lag_wind[6] = 0.96076828

*						lag_wind[7] = 0.94699991

*						lag_wind[8] = 0.93135828

*						lag_wind[9] = 0.91394323

*						lag_wind[10]= 0.89486438

*

*	COMMENTS		:	- (*) The first coefficient whose value = 1 is just

*					  mentioned for information, but not included in the code

*					- (**) All the other coefficents incorporate a scaling factor *					  of 1,00005 corresponding to a noise floor of -43 dB

*					- This table uses a special extended precision format

*					  (see "comments" in the header of

*					   the file fexp_tet.c - Section A.5.2)

*

************************************************************************/

static Word16 lag_h[10] = {

32729,

32621,

32440,

32189,

31869,

31482,

31031,

30518,

29948,

29322};

static Word16 lag_l[10] = {

32704,

5120,

18240,

12928,

10752,

14912,

9600,

24512,

3008,

30016};

/************************************************************************

*

*	FILENAME		:	log2.tab

*

*	DESCRIPTION		:	Table for routine Log2().

*

************************************************************************/

static Word16 tab_log2[33] = {

0,  1455,  2866,  4236,  5568,  6863,  8124,  9352, 10549, 11716,

12855, 13967, 15054, 16117, 17156, 18172, 19167, 20142, 21097, 22033,

22951, 23852, 24735, 25603, 26455, 27291, 28113, 28922, 29716, 30497,

31266, 32023, 32767 };

/************************************************************************

*

*	FILENAME		:	pow2.tab

*

*	DESCRIPTION		:	Table for routine pow2().

*

************************************************************************/

static Word16 tab_pow2[33] = {

16384, 16743, 17109, 17484, 17867, 18258, 18658, 19066, 19484, 19911,

20347, 20792, 21247, 21713, 22188, 22674, 23170, 23678, 24196, 24726,

25268, 25821, 26386, 26964, 27554, 28158, 28774, 29405, 30048, 30706,

31379, 32066, 32767 };

/************************************************************************

*

*	FILENAME		:	window.tab

*

*	DESCRIPTION		:	Asymetric Hamming window for LPC analysis.

*

************************************************************************/

#define L_window 256

static Word16 window[L_window] = {

2621,  2623,  2628,  2636,  2647,  2662,  2679,  2700,  2724,  2752,

2782,  2816,  2852,  2892,  2936,  2982,  3031,  3084,  3140,  3199,

3260,  3325,  3393,  3465,  3539,  3616,  3696,  3779,  3865,  3954,

4047,  4141,  4239,  4340,  4444,  4550,  4659,  4771,  4886,  5003,

5123,  5246,  5372,  5500,  5631,  5764,  5900,  6038,  6179,  6323,

6468,  6617,  6767,  6920,  7075,  7233,  7392,  7554,  7718,  7884,

8052,  8223,  8395,  8569,  8746,  8924,  9104,  9286,  9470,  9655,

9842, 10031, 10221, 10413, 10607, 10802, 10999, 11197, 11396, 11597,

11799, 12002, 12207, 12413, 12619, 12827, 13036, 13246, 13457, 13669,

13882, 14095, 14309, 14524, 14740, 14956, 15173, 15391, 15608, 15827,

16046, 16265, 16484, 16704, 16924, 17144, 17364, 17584, 17804, 18024,

18245, 18465, 18684, 18904, 19124, 19343, 19561, 19780, 19998, 20215,

20432, 20648, 20864, 21079, 21293, 21506, 21719, 21931, 22142, 22352,

22561, 22769, 22976, 23181, 23386, 23589, 23791, 23992, 24191, 24389,

24586, 24781, 24975, 25167, 25357, 25546, 25733, 25919, 26102, 26284,

26464, 26642, 26819, 26993, 27165, 27336, 27504, 27670, 27834, 27996,

28156, 28313, 28468, 28621, 28772, 28920, 29066, 29209, 29350, 29488,

29624, 29757, 29888, 30016, 30142, 30265, 30385, 30502, 30617, 30729,

30838, 30945, 31048, 31149, 31247, 31342, 31434, 31523, 31609, 31692,

31772, 31850, 31924, 31995, 32063, 32128, 32190, 32249, 32304, 32357,

32406, 32453, 32496, 32536, 32573, 32606, 32637, 32664, 32688, 32709,

32727, 32741, 32753, 32761, 32765, 32767, 32767, 32718, 32572, 32329,

31991, 31561, 31041, 30434, 29744, 28977, 28136, 27227, 26257, 25231,

24156, 23039, 21888, 20709, 19511, 18301, 17088, 15878, 14680, 13501,

12350, 11233, 10158,  9132,  8162,  7253,  6412,  5645,  4955,  4348,

3828,  3397,  3059,  2817,  2670,  2621 };