# B	C Code for the TETRA Basic Operators

/***********************************************************************

*

*	FILENAME		:	tetra_op.c

*

*	DESCRIPTION		:	TETRA  basic operators

*

************************************************************************

*

*	FUNCTIONS		:	- abs_s()

*					- add()

*					- div_s()

*					- extract_h()

*					- extract_l()

*					- L_abs()

*					- L_add()

*					- L_deposit_h()

*					- L_deposit_l()

*					- L_mac()

*					- L_mac0()

*					- L_msu()

*					- L_msu0()

*					- L_mult()

*					- L_mult0()

*					- L_negate()

*					- L_shl()

*					- L_shr()

*					- L_shr_r()

*					- L_sub()

*					- mult()

*					- mult_r()

*					- negate()

*					- norm_l()

*					- norm_s()

*					- round()

*					- sature()

*					- shl()

*					- shr()

*					- sub()

*

*	COMMENTS		:	Only the operators used in the actual version

*					of the TETRA codec are included in this file

*

************************************************************************

*

*	INCLUDED FILES	:	source.h

*					stdio.h

*					stdlib.h

*

************************************************************************/

#include <stdio.h>

#include <stdlib.h>

#include "source.h"

/*-----------------*

* Local Functions *

*-----------------*/

Word16 sature(Word32 L_var1);

/*-----------------------*

* Constants and Globals *

*-----------------------*/

Flag Overflow =0;

Flag Carry =0;

/************************************************************************

*

*	Function Name : abs_s

*

*	Purpose :

*

*		Absolute value of var1; abs_s(-32768) = 32767.

*

*	Complexity Weight : 1

*

*	Inputs :

*

*		var1

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var1 <= 0x0000 7fff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		var_out

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0x0000 0000 <= var_out <= 0x0000 7fff.

*

************************************************************************/

Word16 abs_s(Word16 var1)

{

Word16 var_out;

if (var1 == (Word16)0X8000 )

{

var_out = MAX_16;

}

else

{

if (var1 < 0)

{

var_out = -var1;

}

else

{

var_out = var1;

}

}

return(var_out);

}

/************************************************************************

*

*	Function Name : add

*

*	Purpose :

*

*		Performs the addition (var1+var2) with overflow control and saturation;|

*		the 16 bit result is set at +32767 when overflow occurs or at -32768

*		when underflow occurs.

*

*	Complexity Weight : 1

*

*	Inputs :

*

*		var1

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var1 <= 0x0000 7fff.

*

*		var2

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var2 <= 0x0000 7fff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		var_out

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var_out <= 0x0000 7fff.

*

************************************************************************/

Word16 add(Word16 var1,Word16 var2)

{

Word16 var_out;

Word32 L_somme;

L_somme = (Word32) var1 + var2;

var_out = sature(L_somme);

return(var_out);

}

/************************************************************************

*

*	Function Name : div_s

*

*	Purpose :

*

*		Produces a result which is the fractionnal integer division of var1 by

*		var2; var1 and var2 must be positive and var2 must be greater or equal

*		to var1; the result is positive (leading bit equal to 0) and truncated

*		to 16 bits.

*		If var1 = var2 then div(var1,var2) = 32767.

*

*	Complexity Weight : 18

*

*	Inputs :

*

*		var1

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0x0000 0000 <= var1 <= var2 and var2 != 0.

*

*		var2

*			16 bit short signed integer (Word16) whose value falls in the

*			range : var1 <= var2 <= 0x0000 7fff and var2 != 0.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		var_out

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0x0000 0000 <= var_out <= 0x0000 7fff.

*			It's a Q15 value (point between b15 and b14).

*

************************************************************************/

Word16 div_s(Word16 var1, Word16 var2)

{

Word16 var_out = 0;

Word16 iteration;

Word32 L_num;

Word32 L_denom;

if ((var1 > var2) || (var1 < 0) || (var2 < 0))

{

printf("Division Error\n");

exit(0);

}

if (var2 == 0)

{

printf("Division by 0, Fatal error \n");

exit(0);

}

if (var1 == 0)

{

var_out = 0;

}

else

{

if (var1 == var2)

{

var_out = MAX_16;

}

else

{

L_num = L_deposit_l(var1);

L_denom = L_deposit_l(var2);

for(iteration=0;iteration<15;iteration++)

{

var_out <<=1;

L_num <<= 1;

if (L_num >= L_denom)

{

L_num = L_sub(L_num,L_denom);

var_out = add( var_out,(Word16)1 );

}

}

}

}

return(var_out);

}

/************************************************************************

*

*	Function Name : extract_h

*

*	Purpose :

*

*		Return the 16 MSB of L_var1.

*

*	Complexity Weight : 1

*

*	Inputs :

*

*		L_var1

*			32 bit long signed integer (Word32 ) whose value falls in the

*			range : 0x8000 0000 <= L_var1 <= 0x7fff ffff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		var_out

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var_out <= 0x0000 7fff.

*

************************************************************************/

Word16 extract_h(Word32 L_var1)

{

Word16 var_out;

var_out = (Word16) (L_var1 >> 16);

return(var_out);

}

/************************************************************************

*

*	Function Name : extract_l

*

*	Purpose :

*

*		Return the 16 LSB of L_var1.

*

*	Complexity Weight : 1

*

*	Inputs :

*

*		L_var1

*			32 bit long signed integer (Word32 ) whose value falls in the

*			range : 0x8000 0000 <= L_var1 <= 0x7fff ffff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		var_out

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var_out <= 0x0000 7fff.

*

************************************************************************/

Word16 extract_l(Word32 L_var1)

{

Word16 var_out;

var_out = (Word16) L_var1;

return(var_out);

}

/************************************************************************

*

*	Function Name : L_abs

*

*	Purpose :

*

*		Absolute value of L_var1; Saturate in case where the input is -214783648

*

*	Complexity Weight : 3

*

*	Inputs :

*

*		L_var1

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var1 <= 0x7fff ffff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_var_out

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x0000 0000 <= L_var_out <= 0x7fff ffff.

*

************************************************************************/

Word32 L_abs(Word32 L_var1)

{

Word32 L_var_out;

if (L_var1 == MIN_32)

{

L_var_out = MAX_32;

}

else

{

if (L_var1 < 0)

{

L_var_out = -L_var1;

}

else

{

L_var_out = L_var1;

}

}

return(L_var_out);

}

/************************************************************************

*

*	Function Name : L_add

*

*	Purpose :

*

*		32 bits addition of the two 32 bits variables (L_var1+L_var2) with

*		overflow control and saturation; the result is set at +214783647 when

*		overflow occurs or at -214783648 when underflow occurs.

*

*	Complexity Weight : 2

*

*	Inputs :

*

*		L_var1

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var1 <= 0x7fff ffff.

*

*		L_var2

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var2 <= 0x7fff ffff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_var_out

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var_out <= 0x7fff ffff.

*

************************************************************************/

Word32 L_add(Word32 L_var1, Word32 L_var2)

{

Word32 L_var_out;

L_var_out = L_var1 + L_var2;

if (((L_var1 ^ L_var2) & MIN_32) == 0)

{

if ((L_var_out ^ L_var1) & MIN_32)

{

L_var_out = (L_var1 < 0) ? MIN_32 : MAX_32;

Overflow = 1;

}

}

return(L_var_out);

}

/************************************************************************

*

*	Function Name : L_deposit_h

*

*	Purpose :

*

*		Deposit the 16 bit var1 into the 16 MS bits of the 32 bit output. The

*		16 LS bits of the output are zeroed.

*

*	Complexity Weight : 2

*

*	Inputs :

*

*		var1

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var1 <= 0x0000 7fff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_var_out

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= var_out <= 0x7fff 0000.

*

************************************************************************/

Word32 L_deposit_h(Word16 var1)

{

Word32 L_var_out;

L_var_out = (Word32) var1 << 16;

return(L_var_out);

}

/************************************************************************

*

*	Function Name : L_deposit_l

*

*	Purpose :

*

*		Deposit the 16 bit var1 into the 16 LS bits of the 32 bit output. The

*		16 MS bits of the output are sign extended.

*

*	Complexity Weight : 2

*

*	Inputs :

*

*		var1

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var1 <= 0x0000 7fff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_var_out

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0xFFFF 8000 <= L_var_out <= 0x0000 7fff.

*

************************************************************************/

Word32 L_deposit_l(Word16 var1)

{

Word32 L_var_out;

L_var_out = (Word32) var1;

return(L_var_out);

}

/************************************************************************

*

*	Function Name : L_mac

*

*	Purpose :

*

*		Multiply var1 by var2 and shift the result left by 1. Add the 32 bit

*		result to L_var3 with saturation, return a 32 bit result:

*			L_mac(L_var3,var1,var2) = L_add(L_var3,(L_mult(var1,var2)).

*

*	Complexity Weight : 1

*

*	Inputs :

*

*		L_var3

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var3 <= 0x7fff ffff.

*

*		var1

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var1 <= 0x0000 7fff.

*

*		var2

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var2 <= 0x0000 7fff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_var_out

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var_out <= 0x7fff ffff.

*

************************************************************************/

Word32 L_mac(Word32 L_var3, Word16 var1, Word16 var2)

{

Word32 L_var_out;

Word32 L_produit;

L_produit = L_mult(var1,var2);

L_var_out = L_add(L_var3,L_produit);

return(L_var_out);

}

/************************************************************************

*

*	Function Name : L_mac0

*

*	Purpose :

*

*		Multiply var1 by var2. Add the 32 bit result to L_var3.

*		Control saturation and set overflow_flag.

*		Same as L_mac without left shift.

*

*	Complexity Weight : 1

*

*	Inputs :

*

*		L_var3

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var3 <= 0x7fff ffff.

*

*		var1

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var1 <= 0x0000 7fff.

*

*		var2

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var2 <= 0x0000 7fff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_var_out

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var_out <= 0x7fff ffff.

*

************************************************************************/

Word32 L_mac0(Word32 L_var3, Word16 var1, Word16 var2)

{

Word32 L_var_out;

Word32 L_produit;

L_produit = L_mult0(var1,var2);

L_var_out = L_add(L_var3,L_produit);

return(L_var_out);

}

/************************************************************************

*

*	Function Name : L_msu

*

*	Purpose :

*

*		Multiply var1 by var2 and shift the result left by 1. Subtract the 32

*		bit result to L_var3 with saturation, return a 32 bit result:

*			L_msu(L_var3,var1,var2) = L_sub(L_var3,(L_mult(var1,var2)).

*

*	Complexity Weight : 1

*

*	Inputs :

*

*		L_var3

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var3 <= 0x7fff ffff.

*

*		var1

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var1 <= 0x0000 7fff.

*

*		var2

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var2 <= 0x0000 7fff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_var_out

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var_out <= 0x7fff ffff.

*

************************************************************************/

Word32 L_msu(Word32 L_var3, Word16 var1, Word16 var2)

{

Word32 L_var_out;

Word32 L_produit;

L_produit = L_mult(var1,var2);

L_var_out = L_sub(L_var3,L_produit);

return(L_var_out);

}

/************************************************************************

*

*	Function Name : L_msu0

*

*	Purpose :

*

*		Multiply var1 by var2. Subtract the 32 bit result to L_var3.

*		Control saturation and set overflow_flag.

*		Same as L_msu without left shift.

*

*	Complexity Weight : 1

*

*	Inputs :

*

*		L_var3

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var3 <= 0x7fff ffff.

*

*		var1

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var1 <= 0x0000 7fff.

*

*		var2

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var2 <= 0x0000 7fff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_var_out

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var_out <= 0x7fff ffff.

*

************************************************************************/

Word32 L_msu0(Word32 L_var3, Word16 var1, Word16 var2)

{

Word32 L_var_out;

Word32 L_produit;

L_produit = L_mult0(var1,var2);

L_var_out = L_sub(L_var3,L_produit);

return(L_var_out);

}

/************************************************************************

*

*	Function Name : L_mult

*

*	Purpose :

*

*		L_mult is the 32 bit result of the multiplication of var1 times var2

*		with one shift left i.e.:

*			L_mult(var1,var2) = shl((var1 times var2),1) and

*			L_mult(-32768,-32768) = 2147483647.

*

*	Complexity Weight : 1

*

*	Inputs :

*

*		var1

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var1 <= 0x0000 7fff.

*

*		var2

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var2 <= 0x0000 7fff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_var_out

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var_out <= 0x7fff ffff.

*

************************************************************************/

Word32 L_mult(Word16 var1,Word16 var2)

{

Word32 L_var_out;

L_var_out = (Word32)var1 * (Word32)var2;

if (L_var_out != (Word32)0x40000000)

{

L_var_out *= 2;

}

else

{

Overflow = 1;

L_var_out = MAX_32;

}

return(L_var_out);

}

/************************************************************************

*

*	Function Name : L_mult0

*

*	Purpose :

*

*		L_mult is the 32 bit result of the multiplication of var1 times var2.

*		Same as L_mult without left shift.

*

*	Complexity Weight : 1

*

*	Inputs :

*

*		var1

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var1 <= 0x0000 7fff.

*

*		var2

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var2 <= 0x0000 7fff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_var_out

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var_out <= 0x7fff ffff.

*

************************************************************************/

Word32 L_mult0(Word16 var1, Word16 var2)

{

Word32 L_var_out;

L_var_out = (Word32)var1 * (Word32)var2;

return(L_var_out);

}

/************************************************************************

*

*	Function Name : L_negate

*

*	Purpose :

*

*		Negate the 32 bit variable L_var1 with saturation; saturate in the case

*		where input is -2147483648 (0x8000 0000).

*

*	Complexity Weight : 2

*

*	Inputs :

*

*		L_var1

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var1 <= 0x7fff ffff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_var_out

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var_out <= 0x7fff ffff.

*

************************************************************************/

Word32 L_negate(Word32 L_var1)

{

Word32 L_var_out;

L_var_out = (L_var1 == MIN_32) ? MAX_32 : -L_var1;

return(L_var_out);

}

/************************************************************************

*

*	Function Name : L_shl

*

*	Purpose :

*

*		Arithmetically shift the 32 bit input L_var1 left var2 positions. Zero fill the var2 LSB *		of the result. If var2 is negative, L_var1 right by -var2 arithmetically shift with sign

*		extension. Saturate the result in case of underflows or overflows.

*

*	Complexity Weight : 2

*

*	Inputs :

*

*		L_var1

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var1 <= 0x7fff ffff.

*

*		var2

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var2 <= 0x0000 7fff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_var_out

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var_out <= 0x7fff ffff.

*

************************************************************************/

Word32 L_shl(Word32 L_var1, Word16 var2)

{

Word32 L_var_out;

if (var2 <= 0)

{

L_var_out = L_shr( L_var1, (Word16)(-var2) );

}

else

{

for(;var2>0;var2--)

{

if (L_var1 > (Word32) 0X3fffffff)

{

Overflow = 1;

L_var_out = MAX_32;

break;

}

else

{

if (L_var1 < (Word32) 0xc0000000)

{

Overflow = 1;

L_var_out = MIN_32;

break;

}

}

L_var1 *= 2;

L_var_out = L_var1;

}

}

return(L_var_out);

}

/************************************************************************

*

*	Function Name : L_shr

*

*	Purpose :

*

*		Arithmetically shift the 32 bit input L_var1 right var2 positions with

*		sign extension. If var2 is negative, arithmetically shift L_var1 left

*		by -var2 and zero fill the var2 LSB of the result. Saturate the result

*		in case of underflows or overflows.

*

*	Complexity Weight : 2

*

*	Inputs :

*

*		L_var1

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var1 <= 0x7fff ffff.

*

*		var2

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var2 <= 0x0000 7fff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_var_out

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var_out <= 0x7fff ffff.

*

************************************************************************/

Word32 L_shr(Word32 L_var1, Word16 var2)

{

Word32 L_var_out;

if (var2 < 0)

{

L_var_out = L_shl( L_var1,(Word16)(-var2) );

}

else

{

if (var2 >= 31)

{

L_var_out = (L_var1 < 0L) ? -1 : 0;

}

else

{

if (L_var1<0)

{

L_var_out = ~((~L_var1) >> var2);

}

else

{

L_var_out = L_var1 >> var2;

}

}

}

return(L_var_out);

}

/************************************************************************

*

*	Function Name : L_shr_r

*

*	Purpose :

*

*		Same as L_shr(L_var1,var2)but with rounding. Saturate the result in case|

*		of underflows or overflows :

*		If var2 is greater than zero :

*			L_shr_r(var1,var2) = L_shr(L_add(L_var1,2**(var2-1)),var2)

*		If var2 is less than zero :

*			L_shr_r(var1,var2) = L_shr(L_var1,var2).

*

*	Complexity Weight : 3

*

*	Inputs :

*

*		L_var1

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var1 <= 0x7fff ffff.

*

*		var2

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var2 <= 0x0000 7fff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_var_out

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var_out <= 0x7fff ffff.

*

************************************************************************/

Word32 L_shr_r(Word32 L_var1,Word16 var2)

{

Word32 L_var_out;

if (var2 > 31)

{

L_var_out = 0;

}

else

{

L_var_out = L_shr(L_var1,var2);

if (var2 > 0)

{

if ( (L_var1 & ( (Word32)1 << (var2-1) )) != 0)

{

L_var_out++;

}

}

}

return(L_var_out);

}

/************************************************************************

*

*	Function Name : L_sub

*

*	Purpose :

*

*		32 bits subtraction of the two 32 bits variables (L_var1-L_var2) with

*		overflow control and saturation; the result is set at +214783647 when

*		overflow occurs or at -214783648 when underflow occurs.

*

*	Complexity Weight : 2

*

*	Inputs :

*

*		L_var1

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var1 <= 0x7fff ffff.

*

*		L_var2

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var2 <= 0x7fff ffff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		L_var_out

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var_out <= 0x7fff ffff.

*

************************************************************************/

Word32 L_sub(Word32 L_var1, Word32 L_var2)

{

Word32 L_var_out;

L_var_out = L_var1 - L_var2;

if (((L_var1 ^ L_var2) & MIN_32) != 0)

{

if ((L_var_out ^ L_var1) & MIN_32)

{

L_var_out = (L_var1 < 0L) ? MIN_32 : MAX_32;

Overflow = 1;

}

}

return(L_var_out);

}

/************************************************************************

*

*	Function Name : mult

*

*	Purpose :

*

*		Performs the multiplication of var1 by var2 and gives a 16 bit result

*		which is scaled i.e.:

*			mult(var1,var2) = shr((var1 times var2),15) and

*			mult(-32768,-32768) = 32767.

*

*	Complexity Weight : 1

*

*	Inputs :

*

*		var1

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var1 <= 0x0000 7fff.

*

*		var2

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var2 <= 0x0000 7fff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		var_out

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var_out <= 0x0000 7fff.

*

************************************************************************/

Word16 mult(Word16 var1, Word16 var2)

{

Word16 var_out;

Word32 L_produit;

L_produit = (Word32)var1 * (Word32)var2;

L_produit = (L_produit & (Word32) 0xffff8000) >> 15;

if (L_produit & (Word32) 0x00010000)

L_produit = L_produit | (Word32) 0xffff0000;

var_out = sature(L_produit);

return(var_out);

}

/************************************************************************

*

*	Function Name : mult_r

*

*	Purpose :

*

*		Same as mult with rounding, i.e.:

*			mult_r(var1,var2) = shr(((var1*var2) + 16384),15) and

*			mult_r(-32768,-32768) = 32767.

*

*	Complexity Weight : 2

*

*	Inputs :

*

*		var1

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var1 <= 0x0000 7fff.

*

*		var2

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var2 <= 0x0000 7fff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		var_out

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var_out <= 0x0000 7fff.

*

************************************************************************/

Word16 mult_r(Word16 var1, Word16 var2)

{

Word16 var_out;

Word32 L_produit_arr;

L_produit_arr = (Word32)var1 * (Word32)var2; /* product */

L_produit_arr += (Word32) 0x00004000;        /* round */

L_produit_arr &= (Word32) 0xffff8000;

L_produit_arr >>= 15;                        /* shift */

if (L_produit_arr & (Word32) 0x00010000)   /*sign extend when necessary*/

{

L_produit_arr |= (Word32) 0xffff0000;

}

var_out = sature(L_produit_arr);

return(var_out);

}

/************************************************************************

*

*	Function Name : negate

*

*	Purpose :

*

*		Negate var1 with saturation, saturate in the case where input is -32768:

*			negate(var1) = sub(0,var1).

*

*	Complexity Weight : 1

*

*	Inputs :

*

*		var1

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var1 <= 0x0000 7fff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		var_out

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var_out <= 0x0000 7fff.

*

************************************************************************/

Word16 negate(Word16 var1)

{

Word16 var_out;

var_out = (var1 == MIN_16) ? MAX_16 : -var1;

return(var_out);

}

/************************************************************************

*

*	Function Name : norm_l

*

*	Purpose :

*

*		Produces the number of left shift needed to normalize the 32 bit variable

*		L_var1 for positive values on the interval with minimum of

*		1073741824 and maximum of 2147483647, and for negative values on the

*		interval with minimum of -2147483648 and maximum of -1073741824; in order

*		to normalize the result, the following operation must be done :

*			norm_L_var1 = L_shl(L_var1,norm_l(L_var1)).

*

*	Complexity Weight : 30

*

*	Inputs :

*

*		L_var1

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var1 <= 0x7fff ffff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		var_out

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0x0000 0000 <= var_out <= 0x0000 001f.

*

************************************************************************/

Word16 norm_l(Word32 L_var1)

{

Word16 var_out;

if (L_var1 == 0)

{

var_out = 0;

}

else

{

if (L_var1 == (Word32)0xffffffff)

{

var_out = 31;

}

else

{

if (L_var1 < 0)

{

L_var1 = ~L_var1;

}

for(var_out = 0;L_var1 < (Word32)0x40000000;var_out++)

{

L_var1 <<= 1;

}

}

}

return(var_out);

}

/************************************************************************

*

*	Function Name : norm_s

*

*	Purpose :

*

*		Produces the number of left shift needed to normalize the 16 bit variable

*		var1 for positive values on the interval with minimum of 16384 and

*		maximum of 32767, and for negative values on the interval with minimum

*		of -32768 and maximum of -16384; in order to normalize the result, the

*		following operation must be done :

*			norm_var1 = shl(var1,norm_s(var1)).

*

*	Complexity Weight : 15

*

*	Inputs :

*

*		var1

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var1 <= 0x0000 7fff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		var_out

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0x0000 0000 <= var_out <= 0x0000 000f.

*

************************************************************************/

Word16 norm_s(Word16 var1)

{

Word16 var_out;

if (var1 == 0)

{

var_out = 0;

}

else

{

if (var1 == (Word16) 0xffff)

{

var_out = 15;

}

else

{

if (var1 < 0)

{

var1 = ~var1;

}

for(var_out = 0; var1 < 0x4000; var_out++)

{

var1 <<= 1;

}

}

}

return(var_out);

}

/************************************************************************

*

*	Function Name : round

*

*	Purpose :

*

*		Round the lower 16 bits of the 32 bit input number into its MS 16 bits

*		with saturation. Shift the resulting bits right by 16 and return the 16

*		bit number:

*			round(L_var1) = extract_h(L_add(L_var1,32768))

*

*	Complexity Weight : 1

*

*	Inputs :

*

*		L_var1

*			32 bit long signed integer (Word32 ) whose value falls in the

*			range : 0x8000 0000 <= L_var1 <= 0x7fff ffff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		var_out

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var_out <= 0x0000 7fff.

*

************************************************************************/

Word16 round(Word32 L_var1)

{

Word16 var_out;

Word32 L_arrondi;

L_arrondi = L_add(L_var1, (Word32)0x00008000);

var_out = extract_h(L_arrondi);

return(var_out);

}

/************************************************************************

*

*	Function Name : sature

*

*	Purpose :

*

*		Limit the 32 bit input to the range of a 16 bit word.

*

*	Inputs :

*

*		L_var1

*			32 bit long signed integer (Word32) whose value falls in the

*			range : 0x8000 0000 <= L_var1 <= 0x7fff ffff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		var_out

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var_out <= 0x0000 7fff.

*

************************************************************************/

Word16 sature(Word32 L_var1)

{

Word16 var_out;

if (L_var1 > 0X00007fffL)

{

Overflow = 1;

var_out = MAX_16;

}

else if (L_var1 < (Word32)0xffff8000L)

{

Overflow = 1;

var_out = MIN_16;

}

else

{

Overflow = 0;

var_out = extract_l(L_var1);

}

return(var_out);

}

/************************************************************************

*

*	Function Name : shl

*

*	Purpose :

*

*		Arithmetically shift the 16 bit input var1 left var2 positions.Zero fill|

*		the var2 LSB of the result. If var2 is negative, arithmetically shift

*		var1 right by -var2 with sign extension. Saturate the result in case of

*		underflows or overflows.

*

*	Complexity Weight : 1

*

*	Inputs :

*

*		var1

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var1 <= 0x0000 7fff.

*

*		var2

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var2 <= 0x0000 7fff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		var_out

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var_out <= 0x0000 7fff.

*

************************************************************************/

Word16 shl(Word16 var1,Word16 var2)

{

Word16 var_out;

Word32 resultat;

if (var2 < 0)

{

var_out = shr( var1,(Word16)(-var2) );

}

else

{

resultat = (Word32) var1 * ((Word32) 1 << var2);

if ((var2 > 15 && var1 != 0) || (resultat != (Word32)((Word16) resultat)))

{

Overflow = 1;

var_out = (var1 > 0) ? MAX_16 : MIN_16;

}

else

{

var_out = extract_l(resultat);

}

}

return(var_out);

}

/************************************************************************

*

*	Function Name : shr

*

*	Purpose :

*

*		Arithmetically shift the 16 bit input var1 right var2 positions with

*		sign extension. If var2 is negative, arithmetically shift var1 left by

*		-var2 with sign extension. Saturate the result in case of underflows or

*		overflows.

*

*	Complexity Weight : 1

*

*	Inputs :

*

*		var1

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var1 <= 0x0000 7fff.

*

*		var2

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var2 <= 0x0000 7fff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		var_out

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var_out <= 0x0000 7fff.

*

************************************************************************/

Word16 shr(Word16 var1,Word16 var2)

{

Word16 var_out;

if (var2 < 0)

{

var_out = shl( var1,(Word16)(-var2) );

}

else

{

if (var2 >= 15)

{

var_out = (var1 < 0) ? -1 : 0;

}

else

{

if (var1 < 0)

{

var_out = ~(( ~var1) >> var2 );

}

else

{

var_out = var1 >> var2;

}

}

}

return(var_out);

}

/************************************************************************

*

*	Function Name : sub

*

*	Purpose :

*

*			Performs the subtraction (var1+var2) with overflow control and satu-

*			ration; the 16 bit result is set at +32767 when overflow occurs or at

*			-32768 when underflow occurs.

*

*	Complexity Weight : 1

*

*	Inputs :

*

*		var1

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var1 <= 0x0000 7fff.

*

*		var2

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var2 <= 0x0000 7fff.

*

*	Outputs :

*

*		none

*

*	Returned Value :

*

*		var_out

*			16 bit short signed integer (Word16) whose value falls in the

*			range : 0xffff 8000 <= var_out <= 0x0000 7fff.

*

************************************************************************/

Word16 sub(Word16 var1,Word16 var2)

{

Word16 var_out;

Word32 L_diff;

L_diff = (Word32) var1 - var2;

var_out = sature(L_diff);

return(var_out);

}