# C	C Code for Speech Channel Coding

## C.1	Main Program for Speech Channel Coding

/************************************************************************

*

*	FILENAME		:	ccoder.					c

*

*	DESCRIPTION		:	Main program for speech channel encoding

*

************************************************************************

*

*	USAGE			:	ccoder serial_file cenc_file

*					(Executable_File Input1 Output1)

*

*	INPUT FILE(S)		:

*

*		INPUT1	:	- Description : serial stream input file

*					- Format : binary file 16 bit-samples

*					  each 16 bit-sample represents one encoded bit

*					  138 (= 1 + 137) bits per frame

*

*	OUTPUT FILE(S)	:

*

*		OUTPUT1	:	- Description : channel encoded serial stream

*					- Format : binary file 16 bit-samples

*					  each 16 bit-sample represents one encoded bit

*

*	COMMENTS		:	- Values of channel encoded samples are

*					either -127 or +127

*					- 2 input frames are encoded together to produce

*					1 single output frame of 432 samples written in

*					the ouput1 file according to a specific format

*					compatible with the error insertion device

*

************************************************************************

*

*	INCLUDED FILES	:	channel.h

*					stdlib.h

*

************************************************************************/

/* LIBRARIES USED */

#include <stdlib.h>

#include "channel.h"

#ifndef TRUE

# define TRUE   (1==1)

# define FALSE  (1==0)

#endif

Word16  FS_Flag = 0;	/* Frame Stealing Flag :

0 = no stealing in the time-slot,

!0 = stealing of the first speech frame

in the time-slot

This flag is set/reset by external world */

int    main( int argc, char *argv[] )

{

FILE    *fin, *fout;

Word32  Loop_counter = 0;

short   first_pass = TRUE;

Word16  i;

Word16  Vocod_array[274];       /* Input Buffer : 2 vocoder frames */

Word16  Coded_array[432];

Word16  Interleaved_coded_array[432]; /* Output Buffer */

/* Parse arguments */

if ( argc != 3 )

{

puts( "usage : ccoder infile outfile" );

puts( "Channel coding time-slot per time-slot" );

puts( "format for input file : 138 (BFI + 137) bits frame" );

puts( "format for output :  $6B21...114 bits...$6B22...114..." );

puts( "        ...$6B26...114...$6B21");

exit( 1 );

}

if ( (fin = fopen( argv[1], "rb" )) == NULL )

{

puts("chanlcod: can't open input file" );

exit( 1 );

}

if ( (fout = fopen( argv[2], "wb" )) == NULL )

{

puts("chanlcod: can't open output file" );

exit( 1 );

}

while( 1 )

{

/* Skip over bfi bit */

if( fread( Vocod_array, sizeof(short), 1, fin ) != 1 ) {

puts( "chanlcod: reached end of file" );

break;

}

/* 1st speech frame */

if( fread( Vocod_array, sizeof(short), 137, fin ) != 137 ) {

puts( "chanlcod: reached end of file" );

break;

}

/* Skip over bfi bit */

if( fread( Vocod_array+137, sizeof(short), 1, fin ) != 1 ) {

puts( "chanlcod: reached end of file" );

break;

}

/* 2nd speech frame */

if( fread( Vocod_array+137, sizeof(short), 137, fin ) != 137 ) {

puts( "chanlcod: reached end of file" );

break;

}

/* Channel Encoding */

Channel_Encoding(first_pass,FS_Flag,Vocod_array,Coded_array);

first_pass = FALSE;

/* Interleaving */

if (!FS_Flag) 	Interleaving_Speech(Coded_array,

Interleaved_coded_array);

else	{

Interleaving_Signalling(	Coded_array + 216,

Interleaved_coded_array + 216);

for (i = 0; i < 216; i++)	Interleaved_coded_array[i] =

Coded_array[i];

}

/* Increment Loop counter */

Loop_counter++;

/* write Output_array (1 TETRA frame = 2 speech frames) to output file */

if (Write_Tetra_File (fout, Interleaved_coded_array) == -1)

{

puts ("chanlcod: cannot write to output file");

break;

}

}

printf("%ld Speech Frames processed\n",2*Loop_counter);

printf("ie %ld Channel Frames\n",Loop_counter);

/* closing files */

fclose( fin );

fclose( fout );

return (EXIT_SUCCESS);

}

/************************************************************************

*	FILENAME		:	ccod_tet.c

*

*	DESCRIPTION		:	Main routine for speech channel encoding

*

************************************************************************

*

*	SUB-ROUTINES	:	- Channel_Encoding()

*

************************************************************************

*

*	INCLUDED FILES	:	channel.h

*					const.tab

*

************************************************************************/

#include "channel.h"

#include "const.tab"

/**************************************************************************

*

*	ROUTINE			:	Channel_Encoding

*

*	DESCRIPTION			:	Main speech channel coding function

*

**************************************************************************

*

*	USAGE				:	Channel_Encoding(first,flag,buffer_in, buffer_out)

*						(Routine_Name(input1,input2,input3,output1))

*

*	INPUT ARGUMENT(S)		:

*

*			INPUT1		:	- Description :	- True only if first call to this routine

*						- Format : Short

*

*			INPUT2		:	- Description :	- (flag = 0) : standard mode

*								- (flag  0) : frame stealing activated

*						- Format : Word16

*

*		INPUT3		:	- Description : two concatened speech frames

*						- Format : 274 * 16 bit-samples

*							   In case of frame stealing, only the second

*							  	   frame is encoded

*

*	OUTPUT ARGUMENT(S)	:

*

*		OUTPUT1		:	- Description : encoded frame

*							- Format : 432 * 16 bit-samples

*								   In case of frame stealing , only the second

*							   half is filled

*

*	RETURNED VALUE		:	None

*

**************************************************************************/

void Channel_Encoding(short first_pass, Word16 Frame_Stealing, Word16 Input_Frame[], Word16 Output_Frame[])

{

Word16  Ordered_array[286];     /* 2 vocoder frames + 8 + 4 */

Word16  i, Nber_Info_Bits;

/* Init for channel encoder */

if (!Frame_Stealing)

Nber_Info_Bits = N0_2 + N1_2 + N2_2 + SIZE_CRC;

else

Nber_Info_Bits = N0 + N1 + N2 + Fs_SIZE_CRC;

if ( first_pass ) Init_Rcpc_Coding(Frame_Stealing,

Ordered_array);

else {  /* Zero the (K - 1) last bits of Ordered_array */

for (i = 0; i < K - 1; i++)

Ordered_array[Nber_Info_Bits++] = 0;

}

if (!Frame_Stealing)

/* Ordering of the two speech frames in three classes */

Build_Sensitivity_Classes(Frame_Stealing, Input_Frame,

Ordered_array);

else

/* Ordering of the second speech frame in three classes */

Build_Sensitivity_Classes(Frame_Stealing,

Input_Frame + 137, Ordered_array);

/* "Coding" for non-protected class (class 0) : 0 --> 127, 1 --> -127 */

Transform_Class_0(Frame_Stealing, Ordered_array);

/* CRC computation on the last class (most sensitive bits) */

Build_Crc(Frame_Stealing, Ordered_array);

if (!Frame_Stealing)

/* Channel encoding of the 2 frames */

Rcpc_Coding(Frame_Stealing, Ordered_array, Output_Frame);

else

/* Channel encoding of 2nd frame */

Rcpc_Coding(Frame_Stealing, Ordered_array,

Output_Frame + 216);

return;

}

## C.2	Library of Speech Channel Coding Subroutines

/************************************************************************

*	FILENAME		:	sub_cc.c

*

*	DESCRIPTION		:	Sub-routines for speech channel encoding

*

************************************************************************

*

*	SUBROUTINES		:	- Build_Crc()

*					- Build_Sensitivity_Classes()

*					- Combination()

*					- Init_Rcpc_Coding()

*					- Interleaving_Signalling()

*					- Interleaving_Speech()

*					- Rcpc_Coding()

*					- Transform_Class_0()

*					- Write_Tetra_File()

*

************************************************************************

*

*	INCLUDED FILES	:	arrays.tab

*					channel.h

*					const.tab

*					stdlib.h

*

************************************************************************/

#include <stdlib.h>

#include "channel.h"

#include "const.tab" /* contains constants for channel coding/decoding */

#include "arrays.tab" /* contains arrays for channel coding/decoding */

/* #define DEBUG */	   /*	This is a compilation directive eventually used

in the routine Rcpc_Coding to check that the

puncturing matrices A1 and A2 are correctly set

The results given by this software shall be the

same, independently of the definition of DEBUG

The complexity of the channel encoding

component has been evaluated with the

assumption that DEBUG is NOT defined */

/**************************************************************************

*

*	ROUTINE				:	Build_Crc

*

*	DESCRIPTION			:	Computation of a 8 bit CRC and concatenation

*							to the input frame

*

**************************************************************************

*

*	USAGE				:	Build_Crc(flag,buffer)

*							(Routine_Name(arg1,arg2))

*

*	ARGUMENT(S)			:

*

*		ARG1				:	- Description :	- (flag = 0) : standard mode

*								- (flag  0) : frame stealing activated

*						- Format : Word16

*

*		ARG2				:	- Description : input serial stream containing

*						parameters of 2 speech frames re-ordered in 3

*						sensitivity classes

*							- Format :

*

*	RETURNED VALUE		:	None

*

*	COMMENTS			:	8 bit CRC is computed on the third sensitivity class

*

**************************************************************************/

Word16     Build_Crc(Word16 FS_Flag, Word16 Input_Frame[])

{

/* Variables */

Word16             i, temp;

if (!FS_Flag) {

/* The class protected by the CRC (Class 2) starts at index N0_2 + N1_2 */

/* First Bit of the CRC : */

temp = 0;

for (i = 0; i < SIZE_TAB_CRC1; i++)

temp = add(temp,Input_Frame[N0_2 + N1_2 + TAB_CRC1[i] - 1]);

/* The CRC bit is the LSB of temp : */

temp = temp & 1;

/* Concatenation of the CRC bit to Input_Frame : */

Input_Frame[N0_2 + N1_2 + N2_2] = temp;

/* Second Bit of the CRC : */

temp = 0;

for (i = 0; i < SIZE_TAB_CRC2; i++)

temp = add(temp,Input_Frame[N0_2 + N1_2 + TAB_CRC2[i] - 1]);

temp = temp & 1;

Input_Frame[N0_2 + N1_2 + N2_2 + 1] = temp;

/* Third Bit of the CRC : */

temp = 0;

for (i = 0; i < SIZE_TAB_CRC3; i++)

temp = add(temp,Input_Frame[N0_2 + N1_2 + TAB_CRC3[i] - 1]);

temp = temp & 1;

Input_Frame[N0_2 + N1_2 + N2_2 + 2] = temp;

/* Fourth Bit of the CRC : */

temp = 0;

for (i = 0; i < SIZE_TAB_CRC4; i++)

temp = add(temp,Input_Frame[N0_2 + N1_2 + TAB_CRC4[i] - 1]);

temp = temp & 1;

Input_Frame[N0_2 + N1_2 + N2_2 + 3] = temp;

/* Fifth Bit of the CRC : */

temp = 0;

for (i = 0; i < SIZE_TAB_CRC5; i++)

temp = add(temp,Input_Frame[N0_2 + N1_2 + TAB_CRC5[i] - 1]);

temp = temp & 1;

Input_Frame[N0_2 + N1_2 + N2_2 + 4] = temp;

/* Sixth Bit of the CRC : */

temp = 0;

for (i = 0; i < SIZE_TAB_CRC6; i++)

temp = add(temp,Input_Frame[N0_2 + N1_2 + TAB_CRC6[i] - 1]);

temp = temp & 1;

Input_Frame[N0_2 + N1_2 + N2_2 + 5] = temp;

/* Seventh Bit of the CRC : */

temp = 0;

for (i = 0; i < SIZE_TAB_CRC7; i++)

temp = add(temp,Input_Frame[N0_2 + N1_2 + TAB_CRC7[i] - 1]);

temp = temp & 1;

Input_Frame[N0_2 + N1_2 + N2_2 + 6] = temp;

/* Eighth Bit of the CRC : */

temp = 0;

for (i = 0; i < SIZE_TAB_CRC8; i++)

temp = add(temp,Input_Frame[N0_2 + N1_2 + TAB_CRC8[i] - 1]);

temp = temp & 1;

Input_Frame[N0_2 + N1_2 + N2_2 + 7] = temp;

return(0);

} /* If !FS_Flag */

else {

/* The class protected by the CRC (Class 2) starts at index N0 + N1 */

/* First Bit of the CRC : */

temp = 0;

for (i = 0; i < Fs_SIZE_TAB_CRC1; i++)

temp = add(temp,Input_Frame[N0 + N1 + Fs_TAB_CRC1[i] - 1]);

/* The CRC bit is the LSB of temp : */

temp = temp & 1;

/* Concatenation of the CRC bit to Input_Frame : */

Input_Frame[N0 + N1 + N2] = temp;

/* Second Bit of the CRC : */

temp = 0;

for (i = 0; i < Fs_SIZE_TAB_CRC2; i++)

temp = add(temp,Input_Frame[N0 + N1 + Fs_TAB_CRC2[i] - 1]);

temp = temp & 1;

Input_Frame[N0 + N1 + N2 + 1] = temp;

/* Third Bit of the CRC : */

temp = 0;

for (i = 0; i < Fs_SIZE_TAB_CRC3; i++)

temp = add(temp,Input_Frame[N0 + N1 + Fs_TAB_CRC3[i] - 1]);

temp = temp & 1;

Input_Frame[N0 + N1 + N2 + 2] = temp;

/* Fourth Bit of the CRC : */

temp = 0;

for (i = 0; i < Fs_SIZE_TAB_CRC4; i++)

temp = add(temp,Input_Frame[N0 + N1 + Fs_TAB_CRC4[i] - 1]);

temp = temp & 1;

Input_Frame[N0 + N1 + N2 + 3] = temp;

return(0);

}

}
/**************************************************************************

*

*	ROUTINE				:	Build_Sensitivity_Classes

*

*	DESCRIPTION			:	Reorders two concatened speech frames into a single

*						frame ordered in three sensitivity classes

*						(from least, ie Class 0, to most sensitive bits,

*						ie Class 2)

*

**************************************************************************

*

*	USAGE				:	Build_Sensitivity_Classes(flag,buffer_in, buffer_out)

*							(Routine_Name(input1,input2,output1))

*

*	INPUT ARGUMENT(S)		:

*

*		INPUT1			:	- Description :	- (flag = 0) : standard mode

*								- (flag  0) : frame stealing activated

*							- Format : Word16

*

*		INPUT2			:	- Description : 2 concatenated speech frames

*							- Format : 274 * 16 bit-samples

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : 1 reordered frame

*							- Format : 274 * 16 bit-samples

*

*	RETURNED VALUE		:	None

*

**************************************************************************/

Word16     Build_Sensitivity_Classes(Word16 FS_Flag, Word16 Input_Frame[], Word16 Output_Frame[])

{

/* Variables */

Word16             i;

if (!FS_Flag) {

/* Class 0 : */

for (i = 0; i < N0; i++) {

Output_Frame[2*i] = Input_Frame[TAB0[i] - 1];

Output_Frame[2*i + 1] = Input_Frame[Length_vocoder_frame + TAB0[i] - 1];

}

/* Building and Concatenation of Class 1 : */

for (i = 0; i < N1; i++) {

Output_Frame[2*(N0 + i)] = Input_Frame[TAB1[i] - 1];

Output_Frame[2*(N0 + i) + 1] = Input_Frame[Length_vocoder_frame + TAB1[i] - 1];

}

/* Building and Concatenation of Class 2 : */

for (i = 0; i < N2; i++) {

Output_Frame[2*(N0 + N1 + i)] = Input_Frame[TAB2[i] - 1];

Output_Frame[2*(N0 + N1 + i) + 1] = Input_Frame[Length_vocoder_frame + TAB2[i] - 1];

}

return(0);

} /* If !FS_Flag */

else {

/* Class 0 : */

for (i = 0; i < N0; i++)

Output_Frame[i] = Input_Frame[TAB0[i] - 1];

/* Building and Concatenation of Class 1 : */

for (i = 0; i < N1; i++)

Output_Frame[N0 + i] = Input_Frame[TAB1[i] - 1];

/* Building and Concatenation of Class 2 : */

for (i = 0; i < N2; i++)

Output_Frame[N0 + N1 + i] = Input_Frame[TAB2[i] - 1];

return(0);

}

}

/**************************************************************************

*

*	ROUTINE				:	Combination

*

*	DESCRIPTION			:	Computes the (convolutional) coded bit for a given

*							polynomial generator and a given state of the encoder

*

**************************************************************************

*

*	USAGE				:	Combination(gene,state)

*							(Routine_Name(input1,input2))

*

*	INPUT ARGUMENT(S)		:

*

*		INPUT1			:	- Description : Polynomial generator

*							- Format : Word16

*

*		INPUT2			:	- Description : State of the encoder

*							- Format : Word16

*

*	RETURNED VALUE		:	1 coded bit

*

**************************************************************************/

Word16     Combination(Word16 A, Word16 B)

{

Word16             Comb;

Word16             i, temp1, temp2, temp3;

Comb = -1;

temp1 = A & B;

for (i = 0;i <= (K - 1);i++)

{

temp2 = shl( (Word16)1,i );

temp3 = temp1 & temp2;

if (temp3 != 0) Comb = negate(Comb);

}

return(Comb);

}

/**************************************************************************

*

*	ROUTINE				:	Init_Rcpc_Coding

*

*	DESCRIPTION			:	Initialization for channel encoding

*

**************************************************************************

*

*	USAGE				:	Init_Rcpc_Coding(flag,buffer)

*							(Routine_Name(arg1,arg2))

*

*	ARGUMENT(S)			:

*

*		ARG1				:	- Description :	- (flag = 0) : standard mode

*								- (flag  0) : frame stealing activated

*						- Format : Word16

*

*		ARG2				:	- Description : Frame to be encoded

*							- Format : 286 * 16 bit-samples

*

*	RETURNED VALUE		:	None

*

*	COMMENTS			:	4 zeroes are concatenated to the input buffer

*							to clear the encoder

*

**************************************************************************/

void	Init_Rcpc_Coding(Word16 FS_Flag, Word16 Input_Frame[])

{

/* Variables */

Word16             i, M;

Word16             Arrival_state;  /* index for Loop on the Lattice States */

Word16             Starting_state;

Word16             Msb;    /* Value used for computation of coded bit */

Word16             Lsb;    /* Value used for computation of coded bit */

Word16             Lsb_bits;

Word16             Msbs_starting_state;

Word16             Involved_bits;

if (!FS_Flag) {

/* Initialization of Nber_Info_Bits, number of bits to be encoded :

-       Class 1, N1_2 bits,

-       Class 2, N2_2 bits,

-       CRC bits

-       (K - 1) zeros to empty the encoder */

Nber_Info_Bits = N1_2 + N2_2 + SIZE_CRC;

/* Zero the (K - 1) last bits of the Input_Frame */

for (i = 0; i < K - 1; i++) {

Input_Frame[N0_2 + Nber_Info_Bits] = 0;

Nber_Info_Bits++;

}

/* Nber_Info_Bits = N1_2 + N2_2 + SIZE_CRC + (K - 1) */

} /* If FS_Flag */

else {

/* Initialization of Nber_Info_Bits, number of bits to be encoded :

-       Class 1, N1 bits,

-       Class 2, N2 bits,

-       Fs_CRC bits

-       (K - 1) zeros to empty the encoder */

Nber_Info_Bits = N1 + N2 + Fs_SIZE_CRC;

/* Zero the (K - 1) last bits of the Input_Frame */

for (i = 0; i < K - 1; i++) {

Input_Frame[N0 + Nber_Info_Bits] = 0;

Nber_Info_Bits++;

}

/* Nber_Info_Bits = N1 + N2 + Fs_SIZE_CRC + (K - 1) */

}

/* Number of states in the Viterbi Lattice : */

M = shl( (Word16)1,(Word16)(K - 1) );

/* Last State of the Viterbi Lattice */

M_1 = sub( M,(Word16)1 );

Msb_bit = shl( (Word16)1,(Word16)(K - 2) );

Lsb_bits = Msb_bit - 1;

/* Description of the Lattice : Loop on Arrival_State */

for (Arrival_state = 0; Arrival_state <= M_1; Arrival_state++) {

/* Computation of the MSB for the Arrival State */

Msb = Arrival_state & Msb_bit;

/* Computation of the (K - 1)MSBs for the Starting State */

Msbs_starting_state = Arrival_state & Lsb_bits;

/* Loop on Lsb, LSB of the Starting State */

for (Lsb = 0; Lsb <= 1; Lsb++) {

Starting_state = add( shl( Msbs_starting_state,(Word16)1 ),Lsb );

Previous[Arrival_state][Lsb] = Starting_state;

/*   TRANSITION BITS T1, T2, T3   */

Involved_bits = add(shl( Msb,(Word16)1 ),Starting_state);

T1[Arrival_state][Lsb] = Combination( Involved_bits,(Word16)G1 );

T2[Arrival_state][Lsb] = Combination( Involved_bits,(Word16)G2 );

T3[Arrival_state][Lsb] = Combination( Involved_bits,(Word16)G3 );

} /* End Loop on Lsb  */

} /* End Loop on Arrival_state */

}

/**************************************************************************

*

*	ROUTINE				:	Interleaving_Signalling

*

*	DESCRIPTION			:	Signalling channel-type interleaving

*							of a single frame (216 bits)

*

**************************************************************************

*

*	USAGE				:	Interleaving_Signalling(buffer_in,buffer_out)

*							(Routine_Name(input1,output1))

*

*	INPUT ARGUMENT(S)		:

*

*		INPUT1			:	- Description : Input buffer

*							- Format : 216 * 16 bit-samples

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : Interleaved buffer

*							- Format : 216 * 16 bit-samples

*

*	RETURNED VALUE		:	None

*

**************************************************************************/

Word16     Interleaving_Signalling(Word16 Input_Frame[],

Word16 Output_frame[])

{

static Word16   K3_const = 216;

static Word16   K_const = 216;

static Word16   a_const = 101;

Word16  i, k;

for (i = 0; i < K3_const; i++) {

k = (Word16)((Word32)((Word32)a_const * (Word32)(i+1)) % K_const);

Output_frame[k] = Input_Frame[i];

}

return(0);

}

/**************************************************************************

*

*	ROUTINE				:	Interleaving_Speech

*

*	DESCRIPTION			:	Matrix interleaving of a  432 bits frame

*

**************************************************************************

*

*	USAGE				:	Interleaving_Speech(buffer_in,buffer_out)

*							(Routine_Name(input1,output1))

*

*	INPUT ARGUMENT(S)		:

*

*		INPUT1			:	- Description : Input buffer

*							- Format : 432 * 16 bit-samples

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : Interleaved buffer

*							- Format : 432 * 16 bit-samples

*

*	RETURNED VALUE		:	None

*

**************************************************************************/

Word16     Interleaving_Speech(Word16 Input_Frame[],

Word16 Output_frame[])

{

Word16             index_lines, index_columns;

for (index_columns = 0; index_columns < COLUMNS; index_columns++) {

for (index_lines = 0; index_lines < LINES; index_lines++)

Output_frame[index_columns * LINES + index_lines] =

Input_Frame[index_lines * COLUMNS + index_columns];

} /* End Loop COLUMNS */

return(0);

}

/**************************************************************************

*

*	ROUTINE				:	Rcpc_Coding

*

*	DESCRIPTION			:	Routine for channel encoding

*

**************************************************************************

*

*	USAGE				:	Rcpc_Coding(flag,buffer_in,buffer_out)

*							(Routine_Name(input1,input2,output1))

*

*	INPUT ARGUMENT(S)		:

*

*		INPUT1			:	- Description :	- (flag = 0) : standard mode

*								- (flag  0) : frame stealing activated

*						- Format : Word16

*

*		INPUT2			:	- Description : Frame to be encoded

*							- Format : 286 * 16 bit-samples

*

*	OUTPUT ARGUMENT(S)		:

*

*		OUTPUT1			:	- Description : Encoded frame

*							- Format : 432 * 16 bit-samples

*

*	RETURNED VALUE		:	None

*

*	COMMENTS			:	- Buffer_in shall be ordered by

* 						sensitivity classes :

*							Class 0

*							Class 1

*							Class 2

*							8 CRC bits

*							4 zeroes

*						- Buffer_out :

*							0 --> +127

*							1 --> -127

*							Class 0 (not encoded)

*							Class 1 (8/12 encoded)

*							Class 2 (8/18 encoded)

*							8 CRC bits (8/18 encoded)

*							4 zeroes (8/18 encoded)

*

**************************************************************************/

void	Rcpc_Coding(Word16 FS_Flag, Word16 Input_Frame[],

Word16 Output_Frame[])

{

/* Variables */

Word16             i, temp, Msb_bit_div2;

Word16             Nber_coded_bits;

Word16             Index_puncturing;

Word16             Coder_state;

Word16             Lsb;

Word16             Size_Class0, Size_Class1, Size_Class2, Size_Error_Control;

if (!FS_Flag) {

Size_Class0 = N0_2;

Size_Class1 = N1_2;

Size_Class2 = N2_2;

Size_Error_Control = SIZE_CRC;

} /* If FS_Flag */

else {

Size_Class0 = N0;

Size_Class1 = N1;

Size_Class2 = N2;

Size_Error_Control = Fs_SIZE_CRC;

}

/* Recopy of Class 0 (unprotected) */

for (i = 0; i < Size_Class0; i++)

Output_Frame[i] = Input_Frame[i];

/* Init of the Starting State of the encoder at zero */

Coder_state = 0;

/* Init Number of Coded Bits */

Nber_coded_bits = 0;

Msb_bit_div2 = shr( Msb_bit,(Word16)1 );

/*-------------------------------------------------------------------------*/

/* Coding of Class 1 */

/* Init of Index for Puncturing: 0 <= Index_puncturing < Period_pct */

Index_puncturing = 0;

for (i = 0; i < Size_Class1; i++) {   /* Loop on class 1 */

/* Output Bit (LSB of the previous state), to define the transition number */

Lsb = Coder_state & 1;

/* Change of state : New state of the encoder */

temp = extract_l(L_mult(Msb_bit_div2,Input_Frame[Size_Class0 + i]));

Coder_state = add( temp,shr( Coder_state,(Word16)1 ) );

/* According to the Puncturing matrix, computation of a coded bit */

/* First coded bit, if any, per info bit */

#ifdef DEBUG

if ((A1[Index_puncturing]) != 0)

{

#endif

Output_Frame[Size_Class0 + Nber_coded_bits] = (T1[Coder_state][Lsb] > 0) ? -127 : 127;

Nber_coded_bits++;

#ifdef DEBUG

}

#endif

/* Second coded bit, if any, per info bit */

if ((A1[Index_puncturing + Period_pct]) != 0)

{

Output_Frame[Size_Class0 + Nber_coded_bits] = (T2[Coder_state][Lsb] > 0) ? -127 : 127;

Nber_coded_bits++;

}

#ifdef DEBUG

/* Third coded bit, if any, per info bit */

if ((A1[Index_puncturing + 2*Period_pct]) != 0)

{

Output_Frame[Size_Class0 + Nber_coded_bits] = (T3[Coder_state][Lsb] > 0) ? -127 : 127;

Nber_coded_bits++;

}

#endif

Index_puncturing++;

if (sub( Index_puncturing,(Word16)Period_pct ) == 0)

Index_puncturing = 0;

} /* End Loop on class 1 */

/* End coding class 1 */

/*-------------------------------------------------------------------------*/
/*-------------------------------------------------------------------------*/

/* Coding of Class 2 */

/* Init of Index for Puncturing: 0 <= Index_puncturing < Period_pct */

Index_puncturing = 0;

for (i = 0; i < Size_Class2 + Size_Error_Control + (K - 1); i++) {   /* Loop on class 2 */

/* Output Bit (LSB of the previous state), to define the transition number */

Lsb = Coder_state & 1;

/* Change of state : New state of the encoder */

temp = extract_l(L_mult(Msb_bit_div2,Input_Frame[Size_Class0 + Size_Class1 + i]));

Coder_state = add( temp,shr( Coder_state,(Word16)1 ) );

/* According to the Puncturing matrix, computation of a coded bit */

/* First coded bit, if any, per info bit */

#ifdef DEBUG

if ((A2[Index_puncturing]) != 0)

{

#endif

Output_Frame[Size_Class0 + Nber_coded_bits] = (T1[Coder_state][Lsb] > 0) ? -127 : 127;

Nber_coded_bits++;

#ifdef DEBUG

}

#endif

/* Second coded bit, if any, per info bit */

#ifdef DEBUG

if ((A2[Index_puncturing + Period_pct]) != 0)

{

#endif

Output_Frame[Size_Class0 + Nber_coded_bits] = (T2[Coder_state][Lsb] > 0) ? -127 : 127;

Nber_coded_bits++;

#ifdef DEBUG

}

#endif

if (!FS_Flag) {

if ((A2[Index_puncturing + 2*Period_pct]) != 0)

/* Third coded bit, if any, per info bit */

{

Output_Frame[Size_Class0 + Nber_coded_bits] = (T3[Coder_state][Lsb] > 0) ? -127 : 127;

Nber_coded_bits++;

}

} /* If FS_Flag */

else {

if ((Fs_A2[Index_puncturing + 2*Period_pct]) != 0)

/* Third coded bit, if any, per info bit */

{

Output_Frame[Size_Class0 + Nber_coded_bits] = (T3[Coder_state][Lsb] > 0) ? -127 : 127;

Nber_coded_bits++;

}

}

Index_puncturing++;

if (sub( Index_puncturing,(Word16)Period_pct ) == 0)

Index_puncturing = 0;

} /* End Loop on class 2 */

/* End coding class 2 */

/*-------------------------------------------------------------------------*/

}

/**************************************************************************

*

*	ROUTINE				:	Transform_Class_0

*

*	DESCRIPTION			:	Transformation ("encoding") of class 0 of the frame

*						0 --> +127

*						1 --> -127

*						the remaining of the frame is not modified

*

**************************************************************************

*

*	USAGE				:	Transform_Class_0(flag,buffer)

*							(Routine_Name(arg1,arg2))

*

*	ARGUMENT(S)			:

*

*		ARG1				:	- Description :	- (flag = 0) : standard mode

*								- (flag  0) : frame stealing activated

*						- Format : Word16

*

*		ARG2				:	- Description : Frame to be encoded

*							- Format : 286 * 16 bit-samples

*

*	RETURNED VALUE		:	None

*

*	COMMENTS			:	Class 0 corresponds to the N0_2 first bits

*						of the buffer (N0 in case of stealing)

*

**************************************************************************/

void	Transform_Class_0(Word16 FS_Flag, Word16 Input_Frame[])

{

/* Variables */

Word16          i;

Word16          Size_Class0;

if (!FS_Flag)

Size_Class0 = N0_2;

else

Size_Class0 = N0;

for (i = 0; i < Size_Class0; i++) {

if (Input_Frame[i] == 0)

Input_Frame[i] = 127;

else

Input_Frame[i] = -127;

} /* End Loop Size_Class0 */

}

/**************************************************************************

*

*	ROUTINE				:	Write_Tetra_File

*

*	DESCRIPTION			:	Writes a file in the TETRA hardware test

*							frame format

*

**************************************************************************

*

*	USAGE				:	Write_Tetra_File(file_pointer,buffer)

*							(Routine_Name(input1,input2))

*

*	INPUT ARGUMENT(S)		:

*

*		INPUT1			:	- Description : File pointer on output file

*							- Format : FILE

*

*		INPUT2			:	- Description : buffer containing the TETRA frame

*							- Format : 432 * 16 bit-samples

*

*	RETURNED VALUE		:	0 if process correct, -1 if EOF

*

*	COMMENTS			:	Data written in the output file are short integers

*

**************************************************************************/

short           Write_Tetra_File (FILE *fout, short *array)

{

static short    initial = 0;

static short    block[690];    /* 960 if FFFF filling is included */

short          *ptr_block;

short i;

/* Return value: -1  EOF

*                0  array TETRA frame written to 'fout'

*/

/* if first call to this routine, then set up output block */

if (initial == 0)

{

initial = 1;

ptr_block = block;

*ptr_block++ = 0x6b21;

for (i = 0; i < 114; i++)

*ptr_block++ = 0;

/* for (i = 0; i < 45; i++)

*ptr_block++ = 0xFFFF;  */

*ptr_block++ = 0x6b22;

for (i = 0; i < 114; i++)

*ptr_block++ = 0;

/* for (i = 0; i < 45; i++)

*ptr_block++ = 0xFFFF;  */

*ptr_block++ = 0x6b23;

for (i = 0; i < 114; i++)

*ptr_block++ = 0;

/* for (i = 0; i < 45; i++)

*ptr_block++ = 0xFFFF;  */

*ptr_block++ = 0x6b24;

for (i = 0; i < 114; i++)

*ptr_block++ = 0;

/* for (i = 0; i < 45; i++)

*ptr_block++ = 0xFFFF;  */

*ptr_block++ = 0x6b25;

for (i = 0; i < 114; i++)

*ptr_block++ = 0;

/* for (i = 0; i < 45; i++)

*ptr_block++ = 0xFFFF;  */

*ptr_block++ = 0x6b26;

for (i = 0; i < 114; i++)

*ptr_block++ = 0;

/* for (i = 0; i < 45; i++)

*ptr_block++ = 0xFFFF;  */

}

/* Fill first valid block */

ptr_block = block+1;

for (i = 0; i < 114; i++)

*ptr_block++ = *array++ & 0x00FF;

/* Fill second valid block */

ptr_block = block + 161 - 45;

for (i = 0; i < 114; i++)

*ptr_block++ = *array++ & 0x00FF;

/* Fill third valid block */

ptr_block = block + 321 - 45 - 45;

for (i = 0; i < 114; i++)

*ptr_block++ = *array++ & 0x00FF;

/* Fill fourth valid block */

ptr_block = block + 481 - 45 - 45 - 45;

for (i = 0; i < 90; i++)

*ptr_block++ = *array++ & 0x00FF;

/* Write out TETRA frame */

if (fwrite (block, sizeof (short), 690, fout) != 690)

return -1;

return 0;

}

## C.3	Tables and Constants for Speech Channel

/************************************************************************

*

*	FILENAME		:		channel.h

*

*	DESCRIPTION	:		DEFINITIONS

*

*						FUNCTION PROTOTYPES for the TETRA

*						speech channel coder and

*						speech channel decoder

*

*						OPERATOR PROTOTYPES for TETRA codec

*

************************************************************************/

/***********************************************************************

*

*      DEFINITIONS

*

************************************************************************/

#ifndef TYPEDEF_H

#define TYPEDEF_H

typedef short Word16;

typedef long  Word32;

typedef int   Flag;

#endif

/************************************************************************

*

*	DESCRIPTION		:	FUNCTION PROTOTYPES for the TETRA

*						speech channel coder

*

************************************************************************/

#ifndef SUB_CC_H

#define SUB_CC_H

#include <stdio.h>

Word16	Build_Crc(Word16 FS_Flag, Word16 Input_Frame[]);

Word16	Build_Sensitivity_Classes(Word16 FS_Flag, Word16 Input_Frame[], 			Word16 Output_Frame[]);

Word16	Combination(Word16 A, Word16 B);

void		Channel_Encoding(short first_pass, Word16 Frame_Stealing,

Word16 Input_Frame[], Word16 Output_Frame[]);

void		Init_Rcpc_Coding(Word16 FS_Flag, Word16 Input_Frame[]);

Word16	Interleaving_Signalling(Word16 Input_Frame[],

Word16 Output_frame[]);

Word16	Interleaving_Speech(Word16 Input_Frame[], Word16 Output_frame[]);

void		Rcpc_Coding(Word16 FS_Flag, Word16 Input_Frame[],

Word16 Output_Frame[]);

void		Transform_Class_0(Word16 FS_Flag, Word16 Input_Frame[]);

short		Write_Tetra_File (FILE *fout, short *array);

#endif

/************************************************************************

*

*	DESCRIPTION		:	FUNCTION PROTOTYPES for the TETRA

*						speech channel decoder

*

************************************************************************/

#ifndef SUB_CD_H

#define SUB_CD_H

Word16	Bfi(Word16 FS_Flag, Word16 Input_Frame[]);

Word16	Channel_Decoding(short first_pass, Word16 Frame_Stealing,

Word16 Input_Frame[], Word16 Output_Frame[]);

Word16	Combination(Word16 A, Word16 B);

Word16	Desinterleaving_Signalling(Word16 Input_Frame[],

Word16 Output_frame[]);

Word16	Desinterleaving_Speech(Word16 Input_Frame[],

Word16 Output_frame[]);

void		Init_Rcpc_Decoding(void);

void		Rcpc_Decoding(Word16 FS_Flag, Word16 Input_Frame[],

Word16 Output_Frame[]);

short		Read_Tetra_File (FILE *fin, short *array);

Word16	Unbuild_Sensitivity_Classes(Word16 FS_Flag, Word16 Input_Frame[],

Word16 Output_Frame[]);

Word16	Untransform_Class_0(Word16 FS_Flag, Word16 Input_Frame[]);

#endif

/************************************************************************

*

*	DESCRIPTION		:	OPERATOR PROTOTYPES for TETRA codec

*

************************************************************************/

#ifndef TETRA_OP_H

#define TETRA_OP_H

/*-----------------------*

* Constants and Globals *

*-----------------------*/

extern Flag Overflow;

extern Flag Carry;

#define MAX_32 (Word32)0x7fffffff

#define MIN_32 (Word32)0x80000000

#define MAX_16 (Word16)0x7fff

#define MIN_16 (Word16)0x8000

/*-----------------------*

* Operators prototypes  *

*-----------------------*/

Word16 abs_s(Word16 var1);                /* Short abs,           1 */

Word16 add(Word16 var1, Word16 var2);     /* Short add,           1 */

Word16 div_s(Word16 var1, Word16 var2);   /* Short division,     18 */

Word16 extract_h(Word32 L_var1);          /* Extract high,        1 */

Word16 extract_l(Word32 L_var1);          /* Extract low,         1 */

Word16 mult(Word16 var1, Word16 var2);    /* Short mult,          1 */

Word16 mult_r(Word16 var1, Word16 var2);  /* Mult with round,     2 */

Word16 negate(Word16 var1);               /* Short negate,        1 */

Word16 norm_l(Word32 L_var1);             /* Long norm,          30 */

Word16 norm_s(Word16 var1);               /* Short norm,         15 */

Word16 round(Word32 L_var1);              /* Round,               1 */

Word16 shl(Word16 var1, Word16 var2);     /* Short shift left,    1 */

Word16 shr(Word16 var1, Word16 var2);     /* Short shift right,   1 */

Word16 sub(Word16 var1, Word16 var2);     /* Short sub,           1 */

Word32 L_abs(Word32 L_var1);              /* Long abs,            3 */

Word32 L_add(Word32 L_var1, Word32 L_var2);  /* Long add,         2 */

Word32 L_deposit_h(Word16 var1);          /* 16 bit var1 -> MSB   2 */

Word32 L_deposit_l(Word16 var1);          /* 16 bit var1 -> LSB,  2 */

Word32 L_mac(Word32 L_var3, Word16 var1, Word16 var2);  /* Mac,   1 */

Word32 L_mac0(Word32 L_var3, Word16 var1, Word16 var2); /* no shi 1 */

Word32 L_msu(Word32 L_var3, Word16 var1, Word16 var2);  /* Msu,   1 */

Word32 L_msu0(Word32 L_var3, Word16 var1, Word16 var2); /* no shi 1 */

Word32 L_mult(Word16 var1, Word16 var2);  /* Long mult,           1 */

Word32 L_mult0(Word16 var1, Word16 var2); /* Long mult no shift,  1 */

Word32 L_negate(Word32 L_var1);           /* Long negate,         2 */

Word32 L_shl(Word32 L_var1, Word16 var2); /* Long shift left,     2 */

Word32 L_shr(Word32 L_var1, Word16 var2); /* Long shift right,    2 */

Word32 L_shr_r(Word32 L_var1, Word16 var2);  /* L_shr with round, 3 */

Word32 L_sub(Word32 L_var1, Word32 L_var2);  /* Long sub,         2 */

#endif

/**************************************************************************

*

*	FILENAME				:	arrays.tab

*

*	DESCRIPTION			:	Arrays for speech channel coding and decoding

*

**************************************************************************/

/* ARRAYS FOR INITIALIZATION OF THE CHANNEL CODING */

/* Definition of the sensitivity classes : 3 classes */

const Word16	TAB0[N0] =	{35,36,37,38,39,40,41,42,43,  47,48,

56,  61,62,63,64,65,66,67,68,69,70,

74,75,  83,  88,89,90,91,92,93,94,95,

96,97,  101,102,  110,115,116,117,118,

119,120,121,122,123,124,  128,129,

137};

/* Class 0

51 terms : Non protected class

Least sensitive bits */

const Word16	TAB1[N1] =	{58,85,112,  54,81,108,135,  50,77,

104,131,  45,72,99,126,  55,82,109,

136,  5,13,34,  8,16,17,  22,23,24,

25,26,  6,14,7,15,  60,87,114,  46,

73,100,127,  44,71,98,125,  33,49,

76,103,130,  59,86,113,  57,84,111};

/* Class 1

56 terms : Protected by Rate 8/12

Ranked with the most important bits at the boundaries,

and least important at the middle */

const Word16	TAB2[N2] =		{18,19,20,21,	/* LSF3 */

31,32,		/* Pitch0 */

53,80,107,134,	/* Energie 3 */

1,2,3,4,		/* LSF1 */

9,10,11,12,		/* LSF2 */

27,28,29,30,	/* Pitch0 */

52,79,106,133,	/* Energie 4 */

51,78,105,132};/* Energie 5 */

/* Class 2

30 terms : Protected by Rate 8/18

Ranked from least to most important bits */

/* PUNCTURING CHARACTERISTICS: */

/* Puncturing Tables for the two protected classes (Class 1 and 2) */

const Word16	A1[Period_pct*3]		=

{1, 1, 1, 1, 1, 1, 1, 1,

1, 0, 1, 0, 1, 0, 1, 0,

0, 0, 0, 0, 0, 0, 0, 0};

/* Puncturing Table for Class 1 : Rate 8/12 (2/3) */

const Word16	A2[Period_pct*3]		=

{1, 1, 1, 1, 1, 1, 1, 1,

1, 1, 1, 1, 1, 1, 1, 1,

1, 0, 0, 0, 1, 0, 0, 0};

/* Puncturing Table for Class 2 : Rate 8/18 */

const Word16	Fs_A2[Period_pct*3]	=

{1, 1, 1, 1, 1, 1, 1, 1,

1, 1, 1, 1, 1, 1, 1, 1,

1, 0, 0, 0, 0, 0, 0, 0};

/* In case of Frame Stealing */

/* Puncturing Table for Class 2 : Rate 8/17 */

/* BAD FRAME INDICATOR : CRC (Cyclic Redundant Check) */

/* Definition of the 8 CRC bits */

const Word16	TAB_CRC1[SIZE_TAB_CRC1] =

{1,  5,  8,9,  13,  15,16,

17,  19,  21,22,  24,25,  31,

32,  35,36,  38,  40,  43,44,

45,  48,49,50,51,  53,54,  56};

/* Ranks of the bits in Class2 protected by the First bit of the CRC */

const Word16	TAB_CRC2[SIZE_TAB_CRC2] =

{2,  6,  9,10,  14,

16,17,18,  20,  22,23,  25,26,

32,33,  36,37,  39,  41,  44,45,

46,  49,50,51,52,  54,55,  57};

/* Ranks of the bits in Class 2 protected by the Second bit of the CRC */

const Word16	TAB_CRC3[SIZE_TAB_CRC3] =

{3,  7,  10,11,  15,

17,18,19,  21,  23,24,  26,27,

33,34,  37,38,  40,  42,  45,

46,47,  50,51,52,53,  55,56,  58};

/* Ranks of the bits in Class 2 protected by the Third bit of the CRC */

const Word16	TAB_CRC4[SIZE_TAB_CRC4] =

{1,  4,5,  9,  11,12,13,  15,

17,18,  20,21,  27,28,  31,

32,  34,  36,  39,40,41,  44,45,

46,47,  49,50,  52,  57,  59};

/* Ranks of the bits in Class 2 protected by the Fourth bit of the CRC */

const Word16	TAB_CRC5[SIZE_TAB_CRC5] =

{2,  5,6,  10,  12,13,14,

16,  18,19,  21,22,  28,29,

32,33,  35,  37,  40,41,42,  45,

46,47,48,  50,51,  53,  58,  60};

/* Ranks of the bits in Class 2 protected by the Fifth bit of the CRC */

const Word16	TAB_CRC6[SIZE_TAB_CRC6] =

{3,  6,7,  11,  13,14,15,

17,  19,20,  22,23,  29,30,

33,34,  36,  38,  41,42,43,

46,47,48,49,  51,52,  54,  59};

/* Ranks of the bits in Class 2 protected by the Sixth bit of the CRC */

const Word16	TAB_CRC7[SIZE_TAB_CRC7] =

{4,  7,8,  12,  14,15,

16,  18,  20,21,  23,24,  30,

31,  34,35,  37,  39,  42,43,44,

47,48,49,50,  52,53,  55,  60};

/* Ranks of the bits in Class 2 protected by the Seventh bit of the CRC */

const Word16	TAB_CRC8[SIZE_TAB_CRC8] =

{1,2,3,4,  8,  13,14,  16,  19,

20,  22,23,  25,26,27,28,29,30,

32,33,34,  36,37,  40,41,42,  44,

48,  50,  53,  56,57,58,59,60};

/* Ranks of the bits in Class 2 protected by the Eighth bit of the CRC */

/* IN CASE OF FRAME STEALING :

BAD FRAME INDICATOR : CRC (Cyclic Redundant Check)

Definition of the 4 CRC bits */

#define	Fs_SIZE_TAB_CRC1	16

#define	Fs_SIZE_TAB_CRC2	16

#define	Fs_SIZE_TAB_CRC3	16

#define	Fs_SIZE_TAB_CRC4	16

const Word16	Fs_TAB_CRC1[Fs_SIZE_TAB_CRC1] =

{1,  4,5,  7,  9,10,11,12,

16,  19,20,  22,  24,25,26,27  };

/* Ranks of the bits in Class2 protected by the First bit of the CRC */

const Word16	Fs_TAB_CRC2[Fs_SIZE_TAB_CRC2] =

{1,2,  4,  6,7,8,9,  13,

16,17,  19,  21,22,23,24,  28  };

/* Ranks of the bits in Class 2 protected by the Second bit of the CRC */

const Word16	Fs_TAB_CRC3[Fs_SIZE_TAB_CRC3] =

{  2,3,  5,  7,8,9,10,  14,

17,18,  20,  22,23,24,25,  29  };

/* Ranks of the bits in Class 2 protected by the Third bit of the CRC */

const Word16	Fs_TAB_CRC4[Fs_SIZE_TAB_CRC4] =

{  3,4,  6,  8,9,10,11,  15,

18,19,  21,  23,24,25,26,  30};

/* Ranks of the bits in Class 2 protected by the Fourth bit of the CRC */

#define	N1_coded	84	/* Size of Class1 (one speech frame) after coding 8/12 */

#define	N2_coded	81	/* Size of Class2 (one speech frame) after coding 8/17 */

Word16	Previous[(1 << (K - 1))][2];

Word16	Best_previous[(1 << (K - 1))][Decoding_delay];

Word16	T1[(1 << (K - 1))][2], T2[(1 << (K - 1))][2], T3[(1 << (K - 1))][2];

Word16	Score[(1 << (K - 1))];

Word16	Ex_score[(1 << (K - 1))];

Word16	Received[3];

Word16	Initialization;

Word16	Nber_Info_Bits;

Word16	Msb_bit;

Word16	M_1;

Word16	Min_value_allowed;

Word16	Max_value_allowed;

/* END GLOBAL VARIABLES ***********************************/

/**************************************************************************

*

*	FILENAME				:	const.tab

*

*	DESCRIPTION			:	Constants for speech channel coding and decoding

*

**************************************************************************/

/* CONSTANTS FOR CONVOLUTIONAL CODING */

#define	K				5 /* Constraint Length */

#define	Nber_bits_quantif		8 /* Quantization */

#define	Decoding_delay		30 /* Decoding Delay */

/* Polynomial generators : these polynomials should be under the format

1+...+X**4 to match the decoding procedure : */

#define	G1				0x01F /* First Polynomial */

#define	G2				0x01B /* Second Polynomial */

#define	G3				0x015 /* Third Polynomial */

/* FRAME FORMAT : */

/* Size of the three sensitivity classes  */

/* these sizes correspond to the coding of two speech frames at a time */

/*                                         ---                         */

#define	N0		51 /* One Frame : Less sensitive class (class 0) */

#define	N1		56 /* One Frame : Moderate sensitive (class 1) */

#define	N2		30 /* One Frame : Most sensitive (class2) */

#define	N0_2		102 /* Two Frames : Less sensitive class (class 0) */

#define	N1_2		112 /* Two Frames : Moderate sensitive (class 1) */

#define	N2_2		60 /* Two Frames : Most sensitive (class 2) */

#define	N1_2_coded	168 /* Two Frames after coding : size of class 1 after coding */

#define	N2_2_coded	162 /* Two Frames after coding : size of class 2 after coding including CRC bits, (K-1) bits to empty the encoder */

/* N0_2 + N1_2_coded + N2_2_coded	=	size of a coded frame (60 ms  at 7200b/s)

=	432

=	Long_time_slot */

#define	Length_vocoder_frame	137 /* 30 ms speech frame at 4567 b/s */

#define	Length_2_frames		274 /* Two 30 ms speech frames */

#define	Length_time_slot		432 /* 60 ms frame at 7200 b/s */

/* PUNCTURING CHARACTERISTICS: */

#define	Period_pct			8 /* Puncturing Period */

#define	Ai_j_SIZE		Period_pct*6 /* Size of Puncturing Arrays */

/* BAD FRAME INDICATOR: CRC */

#define	SIZE_CRC			8 /* Number of bits of the CRC (Cyclic Redundant Check) */

#define	Fs_SIZE_CRC			4 /* Number of bits of the CRC (Frame Stealing Active) */

#define	SIZE_TAB_CRC1		29	/* Amount of bits taken into account */

#define	SIZE_TAB_CRC2		29	/* In every of the 8 CRC bits */

#define	SIZE_TAB_CRC3		29

#define	SIZE_TAB_CRC4		30

#define	SIZE_TAB_CRC5		30

#define	SIZE_TAB_CRC6		29

#define	SIZE_TAB_CRC7		29

#define	SIZE_TAB_CRC8		35

/* CONSTANTS FOR MATRIX INTERLEAVING */

#define	LINES				24

#define	COLUMNS			18

/* LINES * COLUMNS = Length_time_slot  = 432 */