Annex B:
Word format presentation of the NFV Information Model

# Introduction

This annex contains the Word format presentation of the NFV Information model generated from the NFV UML Information model using the tool Gendoc available at [i.4]. The format is provided to assist the reader that do not use UML or have no access to UML tools.

# NFVCoreModel

## Overview

Qualified Name: NfvInformationModel::NFVCoreModel

Description:

Defines the NFV core information model objects

## NFVCommonDomain

### Overview

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain

Description:

Group all the classes and types common between other domains

### CommonStateModule

#### Overview

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonStateModule

Description:

Defines common state elements used in the model

##### Diagrams

Figure B.1: Diagram CommonStates

##### Datatypes

###### InstantiationState enumeration

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonStateModule::InstantiationState

Description:

The instantiation state of the VNF or NS. Possible values: NOT_INSTANTIATED (VNF instance or NS instance is terminated or not instantiated,and the identifier of the VNF/NS instance exists), INSTANTIATED (VNF instance or NS instance is instantiated).

Applied Stereotypes:

Experimental

Contains Enumeration Literals:

NOT_INSTANTIATED

INSTANTIATED

###### OperationalState enumeration

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonStateModule::OperationalState

Description:

Operational state of the object. Possible values: ENABLED, DISABLED.

Applied Stereotypes:

Preliminary

Contains Enumeration Literals:

ENABLED

DISABLED

###### UsageState enumeration

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonStateModule::UsageState

Description:

Usage state of the object. Possible values: IN_USE, NOT_IN_USE.

Applied Stereotypes:

Preliminary

Contains Enumeration Literals:

IN_USE

NOT_IN_USE

### CommonTemplateModule

#### Overview

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonTemplateModule

Description:

Defines common template elements used in the model

##### Diagrams

Figure B.1: Diagram CommonTemplate

Figure B.1: Diagram CommonTemplateHierarchy

##### Classes

###### AffinityOrAntiAffinityGroup class

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonTemplateModule::AffinityOrAntiAffinityGroup

Description:

The AffinityOrAntiAffinityGroup class describes the affinity or anti-affinity relationship.

For NSs, those rules are applicable between the VNF instances created using different VnfProfiles, the Virtual Link instances created using different VlProfiles or the nested NS instances created using different NsProfiles.

For VNFs, those rules are applicable between the virtualization containers to be created based on different VDUs, or between internal VLs to be created based on different VnfVirtualLinkDesc(s).

Per VNF, the affinity / anti-affinity rules defined using this class, using the LocalAffinityOrAntiAffinityRule datatypet, and using the placement constraints in the GrantLifecycleOperation as defined in ETSI GS NFV IFA 007 should be conflict-free. In case of conflicts, the placement constraints in the GrantLifecycleOperation shall take precedence.

Note: the description of this class are different in ETSI GS NFV IFA 011 and ETSI GS NFV IFA 014. The present definition merges both.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class AffinityOrAntiAffinityGroup

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| groupId | Identifier | 1 | Identifier of this AffinityOrAntiAffinityGroup class. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| affinityOrAntiAffinity | AffinityOrAntiAffinity | 1 | Specifies the type of relationship that the members of the group have: "affinity" or "anti-affinity". | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| scope | AffinityOrAntiAffinityScope | 1 | Specifies the scope of the affinity or anti-affinity relationship e.g. a NFVI node, an NFVI PoP, etc. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### Cpd class

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonTemplateModule::Cpd

Description:

The Cpd class specifies the characteristics of connection points attached to NFs and NSs. This is an abstract class used as parent for the various Cpd classes.

Note 1: the description of this class are different in ETSI GS NFV IFA 011 and ETSI GS NFV IFA 014. The present definition uses the definition in ETSI GS NFV IFA 014 as it is the more general.

Note 2: the description of this class in  ETSI GS NFV IFA 011 includes the attribute addressData. As this attribute is not present in  ETSI GS NFV IFA 014 and the Cpd class is common to both, this attribute has been moved to the classes VduCpd and VnfExtCpd.

This class is abstract.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class Cpd

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| cpdId | Identifier | 1 | Identifier of this Cpd class. | OpenModelAttribute - isInvariant: true - support: MANDATORY |
| layerProtocol | LayerProtocol | 1..* | Identifies a protocol that the connection points corresponding to the CPD support for connectivity purposes (e.g. Ethernet, MPLS, ODU2, IPV4, IPV6, Pseudo-Wire, etc.). NOTE: This information determines, amongst other things, which type of address to assign to the connection point at instantiation time. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| cpdRole | String | 0..1 | Identifies the role of the connection points corresponding to the CPD in the context of the traffic flow patterns in the VNF, PNF or NS. For example an NS with a tree flow pattern within the NS will have legal cpRoles of ROOT and LEAF. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| description | String | 0..1 | Provides human-readable information on the purpose of the connection point (e.g. connection point for control plane traffic). | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| trunkMode | Boolean | 0..1 | Information about whether the Cp instantiated from this CPD is in Trunk mode (802.1Q or other). When operating in "trunk mode", the Cp is capable of carrying traffic for several VLANs. A cardinality of 0 implies that trunkMode is not configured for the Cp i.e.it is equivalent to Boolean value "false". | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### ExtCpd class

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonTemplateModule::ExtCpd

This class is abstract.

Parent class: Cpd

Applied Stereotypes:

Experimental

OpenModelClass

support:  MANDATORY

Table A.1: Attributes for Class ExtCpd

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| cpdId | Identifier | 1 | Identifier of this Cpd class. | Inherited from Class Cpd OpenModelAttribute - isInvariant: true - support: MANDATORY |
| layerProtocol | LayerProtocol | 1..* | Identifies a protocol that the connection points corresponding to the CPD support for connectivity purposes (e.g. Ethernet, MPLS, ODU2, IPV4, IPV6, Pseudo-Wire, etc.). NOTE: This information determines, amongst other things, which type of address to assign to the connection point at instantiation time. | Inherited from Class Cpd OpenModelAttribute - isInvariant: false - support: MANDATORY |
| cpdRole | String | 0..1 | Identifies the role of the connection points corresponding to the CPD in the context of the traffic flow patterns in the VNF, PNF or NS. For example an NS with a tree flow pattern within the NS will have legal cpRoles of ROOT and LEAF. | Inherited from Class Cpd OpenModelAttribute - isInvariant: false - support: MANDATORY |
| description | String | 0..1 | Provides human-readable information on the purpose of the connection point (e.g. connection point for control plane traffic). | Inherited from Class Cpd OpenModelAttribute - isInvariant: false - support: MANDATORY |
| trunkMode | Boolean | 0..1 | Information about whether the Cp instantiated from this CPD is in Trunk mode (802.1Q or other). When operating in "trunk mode", the Cp is capable of carrying traffic for several VLANs. A cardinality of 0 implies that trunkMode is not configured for the Cp i.e.it is equivalent to Boolean value "false". | Inherited from Class Cpd Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VirtualLinkDesc class

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonTemplateModule::VirtualLinkDesc

Description:

Common part of the various VLDs.

This class is abstract.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VirtualLinkDesc

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| virtualLinkDescId | Identifier | 1 | Uniquely identifies a VLD in the parent descriptor. For VnfVirtualLinkDesc, the parent descriptor is the VNFD. For NsVirtualLinkDesc, the parent descriptor is the NSD. Note: the description of this class are different in ETSI GS NFV IFA 011 and ETSI GS NFV IFA 014. The present definition merges the 2 definitions. | OpenModelAttribute - isInvariant: true - support: MANDATORY |
| connectivityType | ConnectivityType | 1 | Specifies the protocol exposed by a VL and the flow pattern supported by the VL. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| testAccess | String | 0..* | Specifies test access facilities expected on the VL (e.g. none, passive monitoring, or active (intrusive) loopbacks at endpoints. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| description | String | 0..1 | Provides human-readable information on the purpose of the VL (e.g. control plane traffic). | OpenModelAttribute - isInvariant: false - support: MANDATORY |

##### Datatypes

###### AddressData datatype

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonTemplateModule::AddressData

Description:

The AddressData datatype supports providing information about the addressing scheme and parameters applicable to a CP.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type AddressData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| addressType | AdressType | 1 | Describes the type of the address to be assigned to the CP instantiated from the parent CPD. Values: • MAC address • IP address • …. The content type shall be aligned with the address type supported by the layerProtocol attribute of the parent CPD. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| l2AddressData |  | 0..1 | Provides the information on the MAC addresses to be assigned to the CP(s) instantiated from the parent CPD. Shall be present when the addressType is MAC address. | OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: |
| l3AddressData | L3AddressData | 0..1 | Provides the information on the IP addresses to be assigned to the CP instantiated from the parent CPD. Shall be present when the addressType is IP address. | OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: |

###### AdressType enumeration

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonTemplateModule::AdressType

Description:

Describes the type of the address to be assigned to the CP instantiated from the parent CPD.

Applied Stereotypes:

Preliminary

###### AffinityOrAntiAffinityScope enumeration

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonTemplateModule::AffinityOrAntiAffinityScope

Description:

Specifies the scope of the affinity or anti-affinity relationship.

Applied Stereotypes:

Preliminary

Contains Enumeration Literals:

NFVI_NODE

RESOURCE_ZONE

ZONE_GROUP

NFVI-POP

###### ConnectivityType datatype

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonTemplateModule::ConnectivityType

Description:

The ConnectivityType datatype specifies the protocol exposed by a VL and the flow pattern supported by the VL.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type ConnectivityType

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| layerProtocol | LayerProtocol | 1 | Identifies the protocol that the VL supports/give access to (Ethernet, MPLS, ODU2, IPV4, IPV6, Pseudo-Wire, etc.). Note: the description of this class are different in ETSI GS NFV IFA 011 and ETSI GS NFV IFA 014. The present definition merges both. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| flowPattern | String | 0..1 | Identifies the flow pattern of the connectivity (Line, Tree, Mesh, etc.). | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### CpProtocolData datatype

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonTemplateModule::CpProtocolData

Description:

A CpProtocolData information element describes and associates the protocol layer that a CP uses together with other protocol and connection point information.

Applied Stereotypes:

Experimental

Table B.1: Attributes for Data Type CpProtocolData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| associatedLayerProtocol | LayerProtocol | 1 | One of the values of the attribute layerProtocol of the Cpd IE | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| addressData | AddressData | 1 | Provides information on the addresses to be assigned to the CP(s) instantiated from the CPD. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### IpAddressType enumeration

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonTemplateModule::IpAddressType

Description:

Define address type. Values:

• IPv4 address,

• IPv6 address,

….

Applied Stereotypes:

Preliminary

###### L3AddressData datatype

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonTemplateModule::L3AddressData

Description:

The L3AddressData datatype supports providing information about Layer 3 level addressing scheme and parameters applicable to a CP.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type L3AddressData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| iPAddressAssignment | Boolean | 1 | Specify if the address assignment is the responsibility of management and orchestration function or not. If it is set to True, it is the management and orchestration function responsibility. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| floatingIPActivated | Boolean | 1 | Specify if the floating IP scheme is activated on the CP or not. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| iPAddressType | IpAddressType | 0..1 | Define address type. Values: • IPv4 address • IPv6 address NOTE: The address type should be aligned with the address type supported by the layerProtocol attribute of the parent VnfExtCpd. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| numberOfIPaddress | Integer | 0..1 | Minimum number of IP addresses to be assigned based on this L3AddressData datatype. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### LayerProtocol enumeration

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonTemplateModule::LayerProtocol

Description:

Identifies the protocol this VL gives access to (Ethernet, MPLS, ODU2, IPV4, IPV6, Pseudo-Wire…)

Applied Stereotypes:

Preliminary

###### LinkBitrateRequirements datatype

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonTemplateModule::LinkBitrateRequirements

Description:

The LinkBitrateRequirements datatype describes the requirements in terms of bitrate for a Virtual Link.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type LinkBitrateRequirements

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| root | Number | 1 | Specifies the throughput requirement of the link (e.g. bitrate of E-Line, root bitrate of E-Tree, aggregate capacity of E-LAN). | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| leaf | Number | 0..1 | Specifies the throughput requirement of leaf connections to the link when applicable to the connectivity type (e.g. for E-Tree and E-LAN branches). NOTE: The present document does not specify the means to declare different bitrate requirements for leaf connections (e.g. E-LAN leaves). | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### LocalAffinityOrAntiAffinityRule datatype

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonTemplateModule::LocalAffinityOrAntiAffinityRule

Description:

When used in an NSD, the LocalAffinityOrAntiAffinityRule datatype specifies affinity or anti-affinity rules applicable to VNFs or VLs instantiated from the same VNFD or VLD.

When used in a VNFD, the LocalAffinityOrAntiAffinityRule describes the affinity or anti-affinity rule applicable between the virtualization containers to be created based on a particular VDU, or between internal VLs to be created based on a particular VnfVirtualLinkDesc.

Per VNF, the affinity/anti-affinity rules defined using this datatype, using the AffinityOrAntiAffinityGroup class, and using the placement constraints in the GrantLifecycleOperation as defined in ETSI GS NFV IFA 007 should be conflict-free. In case of conflicts, the placement constraints in the GrantLifecycleOperation shall take precedence.

Note: the descriptrion of this datatype are different in ETSI GS NFV IFA 011 and ETSI GS NFV IFA 014. The present definition merges both.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type LocalAffinityOrAntiAffinityRule

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| affinityOrAntiAffinity | AffinityOrAntiAffinity | 1 | Specifies the type of the rule: "affinity" or "anti-affinity". | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| scope | AffinityOrAntiAffinityScope | 1 | Specifies whether the scope of the rule is an NFVI-node, an NFVI-PoP, etc. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### MonitoringParameter datatype

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonTemplateModule::MonitoringParameter

Description:

This information element specifies the virtualised resource related performance metric to be tracked by the VNFM, e.g. for auto-scaling purposes. The VNFM collects the values of performance metrics identified by this information element from the VIM(s) using one or more locally initiated PM Jobs. These values can be used as inputs to auto-scaling rules

Applied Stereotypes:

Experimental

Table B.1: Attributes for Data Type MonitoringParameter

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| monitoringParameterId | Identifier | 1 | Unique identifier of this monitoring parameter datatype. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| name | String | 0..1 | Human readable name of the monitoring parameter. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| performanceMetric | String | 1 | Identifies the virtualised resource performance metric. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| collectionPeriod |  | 0..1 | An attribute that describes the recommended periodicity at which to collect the performance information. VNFM determines if this parameter is considered. The vendor may provide this information as a guidance for creating PmJobs if needed. NOTE: The MANO or NFVI may not support the recommended collectionPeriod based on their functionalities, and can reject the requests based on the recommended collectionPeriod in this case. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsScaleInfo datatype

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonTemplateModule::NsScaleInfo

Description:

The NsScaleInfo datatype represents a scale level for a particular scaling aspect.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type NsScaleInfo

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| nsScalingAspectId | NsScalingAspect | 1 | Identifier of the NS scaling aspect. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| nsScaleLevelId | NsLevel | 1 | Identifier of the NS scale level. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |

###### QoS datatype

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonTemplateModule::QoS

Description:

The QoS datatype specifies quality of service parameters applicable to a VL.

It is the asbstract parent of the NsQos and VnfQos datatypes, as the NsQoS (QoS in ETSI GS NFV IFA 014) includes a priority attribute, while VnfQoS (QoS in ETSI GS NFV IFA 011) does not.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type QoS

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| latency | Number | 1 | Specifies the maximum latency in ms. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| packetDelayVariation | Number | 1 | Specifies the maximum jitter in ms. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| packetLossRatio | Number | 0..1 | Specifies the maximum packet loss ratio. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### ScaleInfo datatype

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonTemplateModule::ScaleInfo

Description:

The ScaleInfo datatype represents a scale level for a particular scaling aspect.

NOTE: Vertical scaling (scale up, scale down) is not supported in the present document.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type ScaleInfo

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| aspectId | Identifier | 1 | Identifier of the scaling aspect. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| scaleLevel | Integer | 1 | The scale level, greater than or equal to 0. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### SecurityParameters datatype

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonTemplateModule::SecurityParameters

Description:

The SecurityParameters contains the signature of a NSD, VLD, PNFD or VNFFGD instance together with information required to validate the signature.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type SecurityParameters

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| signature | String | 1 | Provides the signature of the signed part of the descriptor. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| algorithm | String | 1 | Identifies the algorithm used to compute the signature. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| certificate |  | 0..1 | Provides a certificate or a reference to a certificate to validate the signature. NOTE: Cardinality of 0 corresponds to the case where the certificate is provided by means outside the NSD | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### ServiceAvailabilityLevel enumeration

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonTemplateModule::ServiceAvailabilityLevel

Description:

Specifies one of the three levels defined in ETSI GS NFV-REL 001:

• Level 1

• Level 2

• Level 3

Applied Stereotypes:

Preliminary

Contains Enumeration Literals:

LEVEL_1

LEVEL_2

LEVEL_3

###### SupportedOperations enumeration

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonTemplateModule::SupportedOperations

Description:

Indicates which operations are available for this deployment flavour via the VNF LCM interface.

Instantiate VNF, Query VNF and Terminate VNF are supported in all DFs and therefore need not be included in this list.

Applied Stereotypes:

Preliminary

###### VnfIndicatorSource enumeration

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonTemplateModule::VnfIndicatorSource

Description:

Describe the source of an indicator.

Applied Stereotypes:

Preliminary

Contains Enumeration Literals:

Vnf

Em

Both

### CommonTopologyModule

#### Overview

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonTopologyModule

Description:

Defines common topological elements used in the model

##### Diagrams

Figure B.1: Diagram CommonTopology

Figure B.1: Diagram CommonTopologyHierarchy

##### Classes

###### Cp class

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonTopologyModule::Cp

Description:

Parent for all connection point classes.

This class is abstract.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class Cp

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| cpId | Identifier | 1 | Identifier of this CP. | OpenModelAttribute - isInvariant: true - support: MANDATORY |

###### LinkPort class

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonTopologyModule::LinkPort

Description:

Represents the port of the link.

This class is abstract.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class LinkPort

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| cp | Cp | 0..1 | CP to be connected to this link port. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VirtualLink class

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonTopologyModule::VirtualLink

Description:

A set of connection points along with the connectivity relationship between them and any associated target performance metrics (e.g. bandwidth, latency, QoS). NOTE: The Virtual Link can interconnect two or more entities (VNF components, VNFs, or PNFs) and it is supported by a Virtual Network (VN) of the NFVI.

This class is abstract.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VirtualLink

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| linkPort | LinkPort | 0..* | Link ports of this VL. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

##### Datatypes

###### ConnectionPointType enumeration

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonTopologyModule::ConnectionPointType

Description:

Indicates whether the connectivity is provided at layer 1, 2 or 3.

Applied Stereotypes:

Preliminary

Contains Enumeration Literals:

LAYER_1

LAYER_2

LAYER_3

##### Associations

###### LinkPortTerminatesOnCp association

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonTopologyModule::LinkPortTerminatesOnCp

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association LinkPortTerminatesOnCp

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| cp | none | Navig. | 0..1 | Cp | CP to be connected to this link port. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| linkPort | none | Not navig. | 0..1 | LinkPort |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VirtualLinkHasLinkPort association

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonTopologyModule::VirtualLinkHasLinkPort

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VirtualLinkHasLinkPort

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| linkPort | composite | Navig. | 0..* | LinkPort | Link ports of this VL. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualLink | none | Not navig. | 1 | VirtualLink |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

### CommonTypesModule

#### Overview

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonTypesModule

Description:

Defines common types used in the model

##### Diagrams

Figure B.1: Diagram Common Types

##### Datatypes

###### AffinityOrAntiAffinity enumeration

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonTypesModule::AffinityOrAntiAffinity

Description:

The type of the constraint.

Applied Stereotypes:

Preliminary

Contains Enumeration Literals:

AFFINITY

ANTI_AFFINITY

###### Binary primitive type

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonTypesModule::Binary

Description:

This primitive type defines a binary object.

###### DateTime primitive type

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonTypesModule::DateTime

Description:

DateTime primitive type

###### Filter primitive type

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonTypesModule::Filter

Description:

Primitive Filter type.  Can be used whenever a filter attribute is needed

###### Identifier primitive type

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonTypesModule::Identifier

Description:

Identifier primitive type

###### IpAddress primitive type

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonTypesModule::IpAddress

Description:

This primitive type is used to describe an IP address.

###### IpVersion enumeration

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonTypesModule::IpVersion

Description:

Defines the IP Version

Applied Stereotypes:

Preliminary

Contains Enumeration Literals:

IPV4

IPV6

###### KeyValuePair datatype

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonTypesModule::KeyValuePair

Description:

Datatype for describing a key-value-pair.

Table B.1: Attributes for Data Type KeyValuePair

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| key | String | 1 |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| value | Value | 1 |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### Location primitive type

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonTypesModule::Location

Description:

This primitive type provides information about the geographic location (e.g., geographic, building or others) of an element.

###### MacAddress primitive type

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonTypesModule::MacAddress

Description:

This primitive type is used to describe a MAC address.

###### Number primitive type

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonTypesModule::Number

Description:

This primitive type is a superset of the standard UML numbering primitives, e.g. Integer and Real.

###### PositiveInteger primitive type

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonTypesModule::PositiveInteger

Description:

This primitive type defines a positive Integer value.

###### Rule primitive type

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonTypesModule::Rule

Description:

Primitive type describing a rule.

###### TimeDuration primitive type

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonTypesModule::TimeDuration

Description:

This primitive type specifies a time duration.

###### TimePeriod datatype

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonTypesModule::TimePeriod

Description:

This datatype specifies a time period.

Table B.1: Attributes for Data Type TimePeriod

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| startTime | DateTime | 1 | The start time of the time period | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| stopTime | DateTime | 1 | The stop time of the time period | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### Value primitive type

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonTypesModule::Value

Description:

This primitive type is used to describe generic Values.

###### Version primitive type

Qualified Name: NfvInformationModel::NFVCoreModel::NFVCommonDomain::CommonTypesModule::Version

Description:

This primitive type defines the version of an element.

## NsDomain

### Overview

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain

Description:

Group all the core classes and types related to Network Services (NSs).

### NsModule

#### Overview

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsModule

Description:

Group all the classes and types related to NSs.

The classes in this module have no direct representation in an IFA specification and are abstracted from the corresponding classes in the NFV Interface Information Model.

##### Diagrams

Figure B.1: Diagram NsLogicalModel

##### Classes

###### NetworkService class

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsModule::NetworkService

Description:

A composition of Network Functions and defined by its functional and behavioural specification.

NOTE: The Network Service (NS) contributes to the behaviour of the higher layer service, which is characterized by at least performance, dependability, and security specifications. The end-to-end network service behaviour is the result of the combination of the individual network function behaviours as well as the behaviours of the network infrastructure composition mechanism.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class NetworkService

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| nsInstanceId | Identifier | 1 | Identifier of this NetworkService, identifying the NS instance. | OpenModelAttribute - isInvariant: true - support: MANDATORY |
| nsName | String | 1 | Human readable name of the NS instance. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| description | String | 1 | Human readable description of the NS instance | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nsd | Nsd | 1 | Reference to the NSD used to instantiate this NS. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| nf | NetworkFunction | 0..* | Reference to consistituent VNFs and PNFs on this NS. NOTE: Cardinality of zero is only valid for a non-instantiated NS. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| nsVirtuaLlink | NsVirtualLink | 0..* | Information on the VLs of this NS. NOTE: Cardinality of zero is only valid for a non-instantiated NS. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnffg | Vnffg | 0..* | Information on the VNFFGs of this NS. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| sap | Sap | 0..* | Information on the SAPs of this NS. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nestedNs | NetworkService | 0..* | Reference to information on nested NSs of this NS. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |

###### NsLinkPort class

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsModule::NsLinkPort

Description:

Respresents the port of a NS VL.

Parent class: LinkPort

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table A.1: Attributes for Class NsLinkPort

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| cp | Cp | 0..1 | CP to be connected to this link port. | Inherited from Class LinkPort OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsVirtualLink class

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsModule::NsVirtualLink

Description:

This class provides run-time information about an NS VL instance.

Parent class: VirtualLink

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class NsVirtualLink

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| nsVirtualLinkDesc | NsVirtualLinkDesc | 1 | VLD in the NSD for this VL. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| virtualNetwork | VirtualNetwork | 0..* | Virtualised network resource(s) realizing this VL. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| linkPort | LinkPort | 0..* | Link ports of this VL. | Inherited from Class VirtualLink OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### Pnf class

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsModule::Pnf

Description:

An implementation of a NF via a tightly coupled software and hardware system.

Parent class: NetworkFunction

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class Pnf

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| pnfd | Pnfd | 1 | PNFD information related to this PNF. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| pnfExternalCp | PnfExtCp | 1..* | External CP of the PNF. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### PnfExtCp class

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsModule::PnfExtCp

Description:

External CP of the PNF.

Parent class: Cp

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class PnfExtCp

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| cpd | Cpd | 1 | Cpd used to deploy this PNF CP. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| cpId | Identifier | 1 | Identifier of this CP. | Inherited from Class Cp OpenModelAttribute - isInvariant: true - support: MANDATORY |

###### Sap class

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsModule::Sap

Description:

Provides information on a Service Access Point (SAP) of a NS

Parent class: Cp

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class Sap

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| sapd | Sapd | 1 | Reference to the SAPD for this SAP. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| cpId | Identifier | 1 | Identifier of this CP. | Inherited from Class Cp OpenModelAttribute - isInvariant: true - support: MANDATORY |

###### Vnffg class

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsModule::Vnffg

Description:

A graph of logical links connecting NF nodes,where at least one node is a VNF, for the purpose of describing traffic flow between these network functions

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class Vnffg

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vnffgId | Identifier | 1 | Identier of this Vnffg class. | OpenModelAttribute - isInvariant: true - support: MANDATORY |
| vnffgd | Vnffgd | 1 | VNFFGD used to deploy this VNFFG. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| nfId | NetworkFunction | 1..* | Identifier of the constituent NFs of the VNFFG | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| virtualLinkId | NsVirtualLink | 1..* | Identifier(s) of the constituent VL instance(s) of the VNFFG. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| cpId | Cp | 1..* | Identifiers of the CP instances attached to the constituent VNFs and PNFs or the sap instances of the VNFFG. NOTE: It indicates an exhaustive list of all the CP instances and SAP instances of the VNFFG. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| nfp | Nfp | 1..* | Information on the NFPs of this VNFFG. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

##### Associations

###### NfpContainsCp association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsModule::NfpContainsCp

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association NfpContainsCp

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| cp | shared | Navig. | 1..* | Cp | The identification of the Connection Point and its position in the NFP queried for. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nfp | none | Not navig. | 0..* | Nfp |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsExposesSap association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsModule::NsExposesSap

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association NsExposesSap

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| sap | composite | Navig. | 0..* | Sap | Information on the SAPs of this NS. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| ns | none | Not navig. | 1 | NetworkService |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsHasNestedNs association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsModule::NsHasNestedNs

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association NsHasNestedNs

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| nestedNs | shared | Navig. | 0..* | NetworkService | Reference to information on nested NSs of this NS. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| compositeNs | none | Not navig. | 0..* | NetworkService |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsIncludesNf association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsModule::NsIncludesNf

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association NsIncludesNf

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| nf | shared | Navig. | 0..* | NetworkFunction | Reference to consistituent VNFs and PNFs on this NS. NOTE: Cardinality of zero is only valid for a non-instantiated NS. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| ns | none | Not navig. | 0..* | NetworkService |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsUsesVirtualLink association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsModule::NsUsesVirtualLink

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association NsUsesVirtualLink

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| nsVirtuaLlink | composite | Navig. | 0..* | NsVirtualLink | Information on the VLs of this NS. NOTE: Cardinality of zero is only valid for a non-instantiated NS. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| networkService | none | Not navig. | 1 | NetworkService |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsUsesVnffg association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsModule::NsUsesVnffg

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association NsUsesVnffg

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| vnffg | composite | Navig. | 0..* | Vnffg | Information on the VNFFGs of this NS. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| ns | none | Not navig. | 1 | NetworkService |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsVirtualLinkRealizedAsVn association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsModule::NsVirtualLinkRealizedAsVn

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association NsVirtualLinkRealizedAsVn

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| virtualNetwork | none | Navig. | 0..* | VirtualNetwork | Virtualised network resource(s) realizing this VL. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nsVirtualLink | none | Not navig. | 0..1 | NsVirtualLink |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### PnfExposesExtCp association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsModule::PnfExposesExtCp

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association PnfExposesExtCp

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| pnfExternalCp | composite | Navig. | 1..* | PnfExtCp | External CP of the PNF. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| pnf | none | Not navig. | 1 | Pnf |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnffgHasConstituentCp association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsModule::VnffgHasConstituentCp

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnffgHasConstituentCp

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| cpId | none | Navig. | 1..* | Cp | Identifiers of the CP instances attached to the constituent VNFs and PNFs or the sap instances of the VNFFG. NOTE: It indicates an exhaustive list of all the CP instances and SAP instances of the VNFFG. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vnffg | none | Not navig. | 0..1 | Vnffg |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnffgHasConstituentNF association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsModule::VnffgHasConstituentNF

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnffgHasConstituentNF

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| nfId | none | Navig. | 1..* | NetworkFunction | Identifier of the constituent NFs of the VNFFG | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vnffg | none | Not navig. | 0..* | Vnffg |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnffgHasConstituentVirtualLink association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsModule::VnffgHasConstituentVirtualLink

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnffgHasConstituentVirtualLink

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| virtualLinkId | none | Navig. | 1..* | NsVirtualLink | Identifier(s) of the constituent VL instance(s) of the VNFFG. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vnffg | none | Not navig. | 0..* | Vnffg |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfgHasNfp association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsModule::VnfgHasNfp

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfgHasNfp

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| nfp | composite | Navig. | 1..* | Nfp | Information on the NFPs of this VNFFG. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnffg | none | Not navig. | 1 | Vnffg |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

### NsTemplateModule

#### Overview

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule

Description:

Group all the classes and types related to NS templates

##### Diagrams

Figure B.1: Diagram NsDeploymentView

Figure B.1: Diagram NsDetailedDeploymentView

Figure B.1: Diagram LinkingLogicalAndDeploymentViews_NS

##### Classes

###### CpProfile class

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::CpProfile

Description:

The CpProfile information element specifies a CPD or SAPD for a given VnfProfile, PnfProfile or NsProfile.

The usage of profiles enables differentiating the CPDs or SAPDs specified in the same VNFD/PNFD/NSD that are applicable to different VNF/PNF/NS instances derived from the same VNFD/PNFD/NSD.

Applied Stereotypes:

Experimental

OpenModelClass

support:  MANDATORY

Table B.1: Attributes for Class CpProfile

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| cpProfileId | Identifier | 1 | Identifier of this CpProfile information element. It uniquely identifies a CpProfile. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfProfileId | VnfProfile | 0..1 | Reference to the VnfProfile of a constituent VNF. NOTE 1: For the VNF external CPD, both vnfProfileId and vnfExtCpdId shall be present as a pair. NOTE 4: One pair of identifiers (VNF external CPD, PNF external CPD or SAPD) shall be present. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vnfExtCpdId | VnfExtCpd | 0..1 | Identifier of the VNF external CPD for a given VnfProfile identified by the "vnfProfileId" attribute. NOTE 1: For the VNF external CPD, both vnfProfileId and vnfExtCpdId shall be present as a pair. NOTE 4: One pair of identifiers (VNF external CPD, PNF external CPD or SAPD) shall be present. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| pnfProfileId | PnfProfile | 0..1 | Reference to the PnfProfile of a constituent PNF. NOTE 2: For the PNF external CPD, both pnfProfileId and pnfExtCpdId shall be present as a pair. NOTE 4: One pair of identifiers (VNF external CPD, PNF external CPD or SAPD) shall be present | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| pnfExtCpdId | PnfExtCpd | 0..1 | Identifier of the PNF external CPD for a given PnfProfile identified by the "pnfProfileId" attribute. NOTE 2: For the PNF external CPD, both pnfProfileId and pnfExtCpdId shall be present as a pair. NOTE 4: One pair of identifiers (VNF external CPD, PNF external CPD or SAPD) shall be present. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| nsProfileId | NsProfile | 0..1 | References the NsProfile of a nested NS. NOTE 3: For the SAPD, both nsProfileId and nsSapdId shall be present as a pair. NOTE 4: One pair of identifiers (VNF external CPD, PNF external CPD or SAPD) shall be present | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| nsSapdId | Sapd | 0..1 | Identifier of the NS SAPD for a given NsProfile identified by the "nsProfileId" attribute. NOTE 3: For the SAPD, both nsProfileId and nsSapdId shall be present as a pair. NOTE 4: One pair of identifiers (VNF external CPD, PNF external CPD or SAPD) shall be present. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |

###### CpdPool class

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::CpdPool

Description:

The CpdPool class specifies a pool of descriptors of connection points and service access points.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class CpdPool

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| cpdPoolId | Identifier | 1 | Identifier of this CpdPool class. It identifies a pool of descriptors of connection points and NS SAPs. | OpenModelAttribute - isInvariant: true - support: MANDATORY |
| cpdId | Cpd | 1..* | A reference to the descriptor of a connection point attached to one of the constituent VNFs and PNFs or to the descriptor of a NS SAP. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |

###### Dependencies class

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::Dependencies

Description:

The Dependencies class provides indications on the order in which VNFs associated to different VNF Profiles and/or nested NSs associated to different NS Profiles are to be instantiated.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class Dependencies

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| primaryId | NsOrVnfProfile | 1..* | References a VnfProfile or NsProfile. NOTE: NFV Management and Orchestration functions shall instantiate VNFs from the VnfProfile and/or nested NSs from the NsProfile referenced in the primary attribute before instantiating VNFs from the VnfProfile and/or nested NSs from the NsProfile referenced in the secondary attribute. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| secondaryId | NsOrVnfProfile | 1..* | References a VnfProfile or NsProfile. NOTE: NFV Management and Orchestration functions shall instantiate VNFs from the VnfProfile and/or nested NSs from the NsProfile referenced in the primary attribute before instantiating VNFs from the VnfProfile and/or nested NSs from the NsProfile referenced in the secondary attribute | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |

###### Nfpd class

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::Nfpd

Description:

The Nfpd class associates traffic flow criteria to a list of descriptors associated to the connection points and service access points to be visited by traffic flows matching these criteria.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class Nfpd

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| nfpdId | Identifier | 1 | Identifies this nfpd class within a VNFFGD. | OpenModelAttribute - isInvariant: true - support: MANDATORY |
| nfpRule | Rule | 0..1 | Provides an NFP classification and selection rule. The rule may be expressed as a criteria constructed out of atomic assertions linked by Boolean operators AND, OR and NOT. NOTE: Examples of atomic assertions are assertions on packet header fields' values, date and time ranges, etc. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| cpd | Cpd | 1..* | References the descriptor of a connection point to be traversed by the traffic flows matching the criteria. This shall be a connection point attached to one of the constituent VNFs and PNFs of the parent VNFFG, or a SAP of one of the constituent nested NSs of the parent VNFFG. NOTE: When multiple values are provided, the order is significant and specifies the sequence of connection points to be traversed. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| cpPoolManagement | CpPoolManagement | 0..* | Provides rules on how to route traffic flows when multiple CP or SAP instances are created from the same CPD or SAPD. NOTE: When no rules are provided in the Nfpd information element, the VIM and/or the NFVI are expected to apply NS-independent rules determined by means outside the scope of the present document. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| cpProfileId | CpProfile | 1..* | References the descriptor of a connection point to be traversed by the traffic flows matching the criteria. This shall be a connection point profile, which is either a CPD associated with the VnfProfile of a constituent VNF, or a CPD associated with the PnfProfile of a constituent PNF, or a SAPD associated with the NsProfile of a nested NS. The related VnfProfile, PnfProfile and NsProfile shall be included in the parent VNFFGD. NOTE: When multiple values are provided, the order is significant and specifies the sequence of connection points to be traversed. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |

###### NsDf class

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::NsDf

Description:

The NsDf class specifies the properties of a variant of an NS.

NOTE: Every VNF, VL and nested NS whose descriptor is referenced by the NS DF shall be involved in an NS instantiation level of the parent NS. If no instances of a given VNF/nested NS have to be deployed at NS instantiation time, the numberOfInstances attribute in the corresponding VnfToLevelMapping/NsToLevelMapping datatype shall be set to 0.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class NsDf

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| nsDfId | Identifier | 1 | Identifies this NsDf class. It identifies a NS DF within the NSD. | OpenModelAttribute - isInvariant: true - support: MANDATORY |
| flavourKey | String | 1 | Assurance parameter against which this flavour is being described. The key can be a combination of multiple assurance parameters with a logical relationship between them. The parameters should be present as a monitoredInfo attribute in the NSD. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfProfile | VnfProfile | 0..* | VNF Profile to be used for the NS flavour. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| pnfProfile | PnfProfile | 0..* | PNF profile to be used for the NS flavour. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualLinkProfile | VirtualLinkProfile | 0..* | VL profile to be used for the NS flavour. There shall be at most one profile per constituent VNF. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| scalingAspect | NsScalingAspect | 0..* | The scaling aspects supported by this DF of the NS. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| affinityOrAntiAffinityGroup | AffinityOrAntiAffinityGroup | 0..* | Specifies affinity or anti-affinity relationship applicable between the VNF instances created using different VNFDs, the Virtual Link instances created using different NsVirtualLinkDescs or the nested NS instances created using different NSDs in the same affinity or anti-affinity group. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nsInstantiationLevel | NsLevel | 1..* | Describes the details of an NS level. NOTE: Every VNF, VL and nested NS whose descriptor is referenced by the NS DF shall be involved in an NS instantiation level of the parent NS. If no instances of a given VNF/nested NS have to be deployed at NS instantiation time, the numberOfInstances attribute in the corresponding VnfToLevelMapping/NsToLevelMapping datatype shall be set to 0. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| defaultNsInstantiationLevelId | NsLevel | 0..1 | Identifies the NS level which represents the default NS instantiation level for this DF. It shall be present if there are multiple "nsInstantiationLevel" entries. | OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: It shall be present if there are multiple "nsIinstantiationLevel" entries. PassedByReference |
| nsProfile | NsProfile | 0..* | Specifies a NS Profile supported by this NS DF. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| dependencies | Dependencies | 0..* | Specifies the order in which instances of the VNFs and/or nested NSs have to be created. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsLevel class

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::NsLevel

Description:

The NsLevel class describes the details of an NS level. An NS level consists of a list of involved entities, i.e. VNFs, VLs and/or nested NSs.

For each involved VNF/nested NS, the number of instances required by the NS level is specified. For each involved VL, the bitrate requirements corresponding to the NS level are specified.

NS levels shall be used in the context of NS instantiation and in this case they are referred to as NS instantiation levels.

Every VNF, VL and nested NS whose descriptor is referenced by the NS DF shall be involved in an NS instantiation level of the parent NS. If no instances of a given VNF/nested NS have to be deployed at NS instantiation time, the numberOfInstances attribute in the corresponding VnfToLevelMapping/NsToLevelMapping datatype shall be set to 0.

NS levels may be used in the context of NS scaling and in this case they are referred to as NS scale levels.

Only a subset of the VNFs, VLs and nested NSs whose descriptor is referenced by the NS DF may be involved in an NS scale level of the parent NS.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class NsLevel

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| nsLevelId | Identifier | 1 | Identifier of this NsLevel class. It uniquely identifies an NS level within the DF. | OpenModelAttribute - isInvariant: true - support: MANDATORY |
| description | String | 1 | Human readable description of the NS level. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfToLevelMapping | VnfToLevelMapping | 0..* | Specifies the profile of the VNFs involved in this NS level and, for each of them, the required number of instances. | OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: At least one attribute between vnfToLevelMapping, vlirtualLinkToLevelMapping and nsToLevelMapping shall be present. |
| virtualLinkToLevelMapping | VirtualLinkToLevelMapping | 0..* | Specifies the profile of the VLs involved in this NS level and, for each of them, the needed bandwidth. | OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: At least one attribute between vnfToLevelMapping, vlirtualLinkToLevelMapping and nsToLevelMapping shall be present. |
| nsToLevelMapping | NsToLevelMapping | 0..* | Specifies the profile of the nested NSs involved in this NS level and, for each of them, the required number of instances. | OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: At least one attribute between vnfToLevelMapping, vlirtualLinkToLevelMapping and nsToLevelMapping shall be present. |

###### NsOrVnfProfile class

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::NsOrVnfProfile

Description:

This is an abstract proxy class, allowing to provide reference to either NsProfile or VnfProfile.

This class is abstract.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

###### NsProfile class

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::NsProfile

Description:

The NsProfile class specifies the profile to be used for a nested NS.

Parent class: NsOrVnfProfile

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class NsProfile

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| nsProfileId | Identifier | 1 | Identifies an NS profile. | OpenModelAttribute - isInvariant: true - support: MANDATORY |
| nsdId | Nsd | 1 | Identifies the NSD applicable to NS instantiated according to this profile. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| nsDfId | NsDf | 1 | Identifies the applicable NS DF within the scope of the NSD. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| nsInstantiationLevelId | NsLevel | 0..1 | Identifies the NS level within the referenced NS DF to be used in the context of the parent NS instantiation. If not present, the default NS instantiation level as declared in the referenced NSD shall be used. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| minNumberOfInstances | Integer | 1 | Minimum number of nested NS instances based on the referenced NSD that is permitted to exist for this NsProfile. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| maxNumberOfInstances | Integer | 1 | Maximum number of nested NS instances based on the referenced NSD that is permitted to exist for this NsProfile. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| affinityOrAntiaffinityGroupId | AffinityOrAntiAffinityGroup | 0..* | Identifies an affinity or anti-affinity group the NSs created according to this NsProfile belong to. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nsVirtualLinkConnectivity | NsVirtualLinkConnectivity | 0..* | Defines the connection information of the NS, it contains connection relationship between an NS SAP and a NS virtual Link of the composite NS. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsScalingAspect class

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::NsScalingAspect

Description:

The NsScalingAspect class describes the details of an NS scaling aspect. An NS scaling aspect is an abstraction representing a particular "dimension" or "property" along which a given NS can be scaled.

Defining NS levels, in this context also known as NS scale levels, within an NS scaling aspect allows to scale NS instances "by steps", i.e. to increase/decrease their capacity in a discrete manner moving from one NS scale level to another.

Scaling by a single step does not imply that exactly one instance of each entity involved in the NS scale level is created or removed.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class NsScalingAspect

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| nsScalingAspectId | Identifier | 1 | Identifier of this NsScalingAspect class. It Uniquely identifies the NS scaling aspect in an NSD. | OpenModelAttribute - isInvariant: true - support: MANDATORY |
| name | String | 1 | Provides a human readable name of the NS scaling aspect. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| description | String | 1 | Provides a human readable description of the NS scaling aspect. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nsScaleLevel | NsLevel | 1..* | Describes the details of an NS level. NOTE: Only a subset of the VNFs, VLs and nested NSs whose descriptor is referenced by the NS DF may be involved in an NS scale level of the parent NS. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsVirtualLinkDesc class

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::NsVirtualLinkDesc

Description:

The NsVirtualLinkDesc class provides general information enabling the instantiation of virtual links.

Parent class: VirtualLinkDesc

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class NsVirtualLinkDesc

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vitualLinkDescProvider | String | 0..1 | Defines the organization generating the VLD. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualLinkDescVersion | Version | 1 | Specifies the version of the VLD. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualLinkDf | VirtualLinkDf | 1..* | Specifies properties for instantiating a VL according to a specific flavour. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| security | SecurityParameters | 0..1 | Provides a signature to prevent tampering. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualLinkDescId | Identifier | 1 | Uniquely identifies a VLD in the parent descriptor. For VnfVirtualLinkDesc, the parent descriptor is the VNFD. For NsVirtualLinkDesc, the parent descriptor is the NSD. Note: the description of this class are different in ETSI GS NFV IFA 011 and ETSI GS NFV IFA 014. The present definition merges the 2 definitions. | Inherited from Class VirtualLinkDesc OpenModelAttribute - isInvariant: true - support: MANDATORY |
| connectivityType | ConnectivityType | 1 | Specifies the protocol exposed by a VL and the flow pattern supported by the VL. | Inherited from Class VirtualLinkDesc OpenModelAttribute - isInvariant: false - support: MANDATORY |
| testAccess | String | 0..* | Specifies test access facilities expected on the VL (e.g. none, passive monitoring, or active (intrusive) loopbacks at endpoints. | Inherited from Class VirtualLinkDesc OpenModelAttribute - isInvariant: false - support: MANDATORY |
| description | String | 0..1 | Provides human-readable information on the purpose of the VL (e.g. control plane traffic). | Inherited from Class VirtualLinkDesc OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### Nsd class

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::Nsd

Description:

The NSD class is a deployment template whose instances are used by the NFVO for the lifecycle management of NSs.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class Nsd

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| nsdIdentifier | Identifier | 1 | Identifier of this NSD class. It globally uniquely identifies an instance of the NSD | OpenModelAttribute - isInvariant: true - support: MANDATORY |
| designer | String | 1 | Identifies the designer of the NSD. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| version | Version | 1 | Identifies the version of the NSD. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nsdName | String | 1 | Provides the human readable name of the NSD. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nsdInvariantId | Identifier | 1 | Identifies an NSD in a version independent manner. This attribute is invariant across versions of NSD. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nestedNsdId | Nsd | 0..* | References the NSD of a constituent nested NS. | OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: At least one between nestedNsd and vnfdId attributes shall be present. PassedByReference |
| vnfdId | Vnfd | 0..* | References the VNFD of a constituent VNF. | OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: At least one between nestedNsd and vnfdId attributes shall be present. PassedByReference |
| pnfdId | Pnfd | 0..* | References the PNFD of a constituent PNF. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| sapd | Sapd | 0..* | Provides the descriptor of a SAP of the NS. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualLinkDesc | NsVirtualLinkDesc | 0..* | Provides the constituent VLDs. Cardinality of 0 means that the NS is a NF set with unspecified connectivity. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnffgd | Vnffgd | 0..* | Provides the descriptors of the applicable forwarding graphs. Cardinality of 0 means that the NS is a NF set with unspecified connectivity. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| monitoredInfo | MonitoredData | 0..* | Identifies either a virtualised resource-related performance metric or a VNF Indicator. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| autoScalingRule | Rule | 0..* | Specifies a rule to trigger a scaling action on a NS instantiated according to the NSD. NOTE 1: The rule is based on a combination of assertions on the values of virtualised resource-related performance metrics and VNF Indicators identified by the monitoredInfo attribute. NOTE 2: There may be multiple data sources (each identified as monitoredInfo) per rule | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| lifeCycleManagementScript | NsLifeCycleManagementScript | 0..* | Provides a life cycle management script written in a Domain Specific Language (DSL). | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nsDf | NsDf | 1..* | Identifies a DF within the scope of an NSD. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| security | SecurityParameters | 1 | Provides a signature to prevent tampering. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### PnfExtCpd  class

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::PnfExtCpd

Description:

A PnfExtCpd is a type of Cpd and describes the characteristics of an external interface, a.k.a external CP, where to connect the PNF to a VL.

A PnfExtCpd inherits from the Cpd Class.

Parent class: ExtCpd

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table A.1: Attributes for Class PnfExtCpd

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| cpdId | Identifier | 1 | Identifier of this Cpd class. | Inherited from Class Cpd OpenModelAttribute - isInvariant: true - support: MANDATORY |
| layerProtocol | LayerProtocol | 1..* | Identifies a protocol that the connection points corresponding to the CPD support for connectivity purposes (e.g. Ethernet, MPLS, ODU2, IPV4, IPV6, Pseudo-Wire, etc.). NOTE: This information determines, amongst other things, which type of address to assign to the connection point at instantiation time. | Inherited from Class Cpd OpenModelAttribute - isInvariant: false - support: MANDATORY |
| cpdRole | String | 0..1 | Identifies the role of the connection points corresponding to the CPD in the context of the traffic flow patterns in the VNF, PNF or NS. For example an NS with a tree flow pattern within the NS will have legal cpRoles of ROOT and LEAF. | Inherited from Class Cpd OpenModelAttribute - isInvariant: false - support: MANDATORY |
| description | String | 0..1 | Provides human-readable information on the purpose of the connection point (e.g. connection point for control plane traffic). | Inherited from Class Cpd OpenModelAttribute - isInvariant: false - support: MANDATORY |
| trunkMode | Boolean | 0..1 | Information about whether the Cp instantiated from this CPD is in Trunk mode (802.1Q or other). When operating in "trunk mode", the Cp is capable of carrying traffic for several VLANs. A cardinality of 0 implies that trunkMode is not configured for the Cp i.e.it is equivalent to Boolean value "false". | Inherited from Class Cpd Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### PnfProfile class

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::PnfProfile

Description:

The PnfProfile class describes additional data for a given PNF instance used in a DF.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class PnfProfile

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| pnfProfileId | Identifier | 1 | Identifier of this PnfProfile class. It uniquely identifies a PnfProfile. | OpenModelAttribute - isInvariant: true - support: MANDATORY |
| pnfdId | Pnfd | 1 | References a PNFD. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| pnfVirtualLinkConnectivity | NsVirtualLinkConnectivity | 1..* | Defines the connection information of the PNF, it contains connection relationship between a PNF connection point and a NS virtual Link. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### Pnfd class

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::Pnfd

Description:

The Pnfd class is a deployment template enabling on-boarding PNFs and referencing them from an NSD. It focuses on connectivity aspects only.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class Pnfd

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| pnfdId | Identifier | 1 | Identifier of this Pnfd class. It uniquely identifies the PNFD. | OpenModelAttribute - isInvariant: true - support: MANDATORY |
| functionDescription | String | 1 | Describes the PNF function. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| provider | String | 1 | Identifies the provider of the PNFD. NOTE: The provider of the PNFD might be different from the provider of the PNF. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| version | Version | 1 | Identifies the version of the PNFD | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| pnfdInvariantId | Identifier | 1 | Identifies a PNFD in a version independent manner. This attribute is invariant across versions of PNFD. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| name | String | 1 | Provides the human readable name of the PNFD. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| security | SecurityParameters | 0..1 | Provides a signature to prevent tampering. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| pnfExtCpd | PnfExtCpd | 1..* | Specifies the characteristics of one or more connection points where to connect the PNF to a VL. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| geographicalLocationInfo |  | 0..1 | It provides information about the geographical location (e.g. geographic coordinates or address of the building, etc.) of the PNF. The cardinality 0 is used when the location is unknown. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### Sapd class

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::Sapd

Description:

The Sapd class specifies the information used to instantiate the service access points of an NS.

A Sapd inherits from the corresponding Cpd class. All attributes of the Cpd are also attributes of the Sapd.

Parent class: ExtCpd

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class Sapd

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| sapAddressAssignment | Boolean | 1 | Specify whether the SAP address assignment is under the responsibility of management and orchestration functions or not. If it is set to True, management and orchestration functions are responsible for assigning addresses to the access points instantiated from this SAPD. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nsVirtualLinkDescId | NsVirtualLinkDesc | 0..1 | References the descriptor of the NS VL instance to which the SAP instantiated from this SAPD connects to. | OpenModelAttribute - isInvariant: false - support: CONDITIONAL_MANDATORY - condition: At most one of the two attributes associatedCpdId and nsVirtualLinkDescId shall be included. PassedByReference |
| associatedCpdId | ExtCpd | 0..1 | References the descriptor of VNF or PNF external connection points or the descriptor of the SAP of a nested NS of this N, which the SAPs instantiated from this SAPD are mapped to. | Experimental OpenModelAttribute - isInvariant: false - support: CONDITIONAL_MANDATORY - condition: At most one of the two attributes associatedCpdId and nsVirtualLinkDescId shall be included. PassedByReference |
| cpdId | Identifier | 1 | Identifier of this Cpd class. | Inherited from Class Cpd OpenModelAttribute - isInvariant: true - support: MANDATORY |
| layerProtocol | LayerProtocol | 1..* | Identifies a protocol that the connection points corresponding to the CPD support for connectivity purposes (e.g. Ethernet, MPLS, ODU2, IPV4, IPV6, Pseudo-Wire, etc.). NOTE: This information determines, amongst other things, which type of address to assign to the connection point at instantiation time. | Inherited from Class Cpd OpenModelAttribute - isInvariant: false - support: MANDATORY |
| cpdRole | String | 0..1 | Identifies the role of the connection points corresponding to the CPD in the context of the traffic flow patterns in the VNF, PNF or NS. For example an NS with a tree flow pattern within the NS will have legal cpRoles of ROOT and LEAF. | Inherited from Class Cpd OpenModelAttribute - isInvariant: false - support: MANDATORY |
| description | String | 0..1 | Provides human-readable information on the purpose of the connection point (e.g. connection point for control plane traffic). | Inherited from Class Cpd OpenModelAttribute - isInvariant: false - support: MANDATORY |
| trunkMode | Boolean | 0..1 | Information about whether the Cp instantiated from this CPD is in Trunk mode (802.1Q or other). When operating in "trunk mode", the Cp is capable of carrying traffic for several VLANs. A cardinality of 0 implies that trunkMode is not configured for the Cp i.e.it is equivalent to Boolean value "false". | Inherited from Class Cpd Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VirtualLinkProfile class

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::VirtualLinkProfile

Description:

The VirtualLinkProfile class specifies a profile for instantiating VLs of a particular NS DF according to a specific VLD and VL DF.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VirtualLinkProfile

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| virtualLinkProfileId | Identifier | 1 | Uniquely identifies this VirtualLinkProfile class. | OpenModelAttribute - isInvariant: true - support: MANDATORY |
| virtualLinkDescId | NsVirtualLinkDesc | 1 | Uniquely references a VLD. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| flavourId | VirtualLinkDf | 1 | Identifies a flavour within the VLD. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| localAffinityOrAntiAffinityRule | LocalAffinityOrAntiAffinityRule | 0..* | Specifies affinity and anti-affinity rules applicable between VLs instantiated from the referenced VLD. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| affinityOrAntiAffinityGroupId | AffinityOrAntiAffinityGroup | 0..* | Identifies an affinity or anti-affinity group the VLs instantiated according to the VlProfile belong to. NOTE : Each identifier references an affinity or anti-affinity group which expresses affinity or anti-affinity relationship between the VL(s) using this VirtualLinkProfile and the VL(s) using other VirtualLinkProfile(s) in the same group. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| maxBitrateRequirements | LinkBitrateRequirements | 1 | Specifies the maximum bitrate requirements for a VL instantiated according to this profile. NOTE: These attributes are used to control scaling boundaries. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| minBitrateRequirements | LinkBitrateRequirements | 1 | Specifies the minimum bitrate requirements for a VL instantiated according to this profile. NOTE: These attributes are used to control scaling boundaries. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfProfile class

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::VnfProfile

Description:

The VnfProfile class specifies a profile for instantiating VNFs of a particular NS DF according to a specific VNFD and VNF DF.

Parent class: NsOrVnfProfile

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VnfProfile

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vnfProfileId | Identifier | 1 | Identifier of this vnfProfile class. It uniquely identifies a VnfProfile. | OpenModelAttribute - isInvariant: true - support: MANDATORY |
| vnfdId | Vnfd | 1 | References a VNFD. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| flavourId | VnfDf | 1 | Identifies a flavour within the VNFD. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| instantiationLevel | Identifier | 0..1 | Identifier of the instantiation level of the VNF DF to be used for instantiation. If not present, the default instantiation level as declared in the VNFD shall be used. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| minNumberOfInstances | Integer | 1 | Minimum number of instances of the VNF based on this VNFD that is permitted to exist for this VnfProfile. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| maxNumberOfInstances | Integer | 1 | Maximum number of instances of the VNF based on this VNFD that is permitted to exist for this VnfProfile. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| localAffinityOrAntiAffinityRule | LocalAffinityOrAntiAffinityRule | 0..* | Specifies affinity and anti-affinity rules applicable between VNF instances created from this profile. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| affinityOrAntiAffinityGroupId | AffinityOrAntiAffinityGroup | 0..* | Identifier(s) of the affinity or anti-affinity group(s) the VnfProfile belongs to. NOTE: Each identifier references an affinity or anti-affinity group which expresses affinity or anti-affinity relationships between the VNF instance(s) created using this VnfProfile and the VNF instance(s) created using other VnfProfile(s) in the same group. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| nsVirtualLinkConnectivity | NsVirtualLinkConnectivity | 1..* | Defines the connection information of the VNF, it contains connection relationship between a VNF connection point and a NS Virtual Link. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### Vnffgd class

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::Vnffgd

Description:

The Vnffgd class specifies a topology of connectivity of a NS and optionally forwarding rules applicable to the traffic conveyed over this topology.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class Vnffgd

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vnffgdId | Identifier | 1 | Identifier of this Vnffgd class. It uniquely identifies a VNFFGD. | OpenModelAttribute - isInvariant: true - support: MANDATORY |
| vnfdId | Vnfd | 1..* | References the VNFD of a constituent VNF. NOTE 1: The list of constituent VNFs, PNFs, and nested NS SAPs of a VNFFG can be identical to the list of constituent VNFs, PNFs and nested NSs of the parent NSD, or can be a subset of it. NOTE 2: The presence of a VNFD or PNFD identifier in a VNFFGD does not imply that all connection points instantiated from all embedded CPDs are connected to the VNFFG instantiated using the VNFFGD. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| pnfdId | Pnfd | 0..* | References the PNFD of a constituent PNF. NOTE 1 : The list of constituent VNFs, PNFs, and nested NS SAPs of a VNFFG can be identical to the list of constituent VNFs, PNFs and nested NSs of the parent NSD, or can be a subset of it. NOTE 2: The presence of a VNFD or PNFD identifier in a VNFFGD does not imply that all connection points instantiated from all embedded CPDs are connected to the VNFFG instantiated using the VNFFGD. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| virtualLinkDescId | NsVirtualLinkDesc | 1..* | References the VLD of a constituent VL. NOTE: The presence of a VLD identifier in a VNFFGD does not imply that all connection points of the VLs instantiated using this VL are connected to the VNFFG instantiated using the VNFFGD. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| cpdPoolId | CpdPool | 1..* | A reference to a pool of descriptors of connection points attached to one of the constituent VNFs and PNFs and/or one of the SAPs of the parent NS or of a nested NS. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| nfpd | Nfpd | 0..* | The network forwarding path associated to the VNFFG. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

##### Datatypes

###### CpPoolManagement datatype

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::CpPoolManagement

Description:

The CpPoolManagement information element provides rules on how to route traffic flows when multiple CP or SAP instances are created from the same CPD or SAPD.

Applied Stereotypes:

Experimental

Table B.1: Attributes for Data Type CpPoolManagement

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| cpdId | Cpd | 1 | References the descriptor of a connection point to be traversed by the traffic flows matching the criteria. This shall be a connection point attached to one of the constituent VNFs and PNFs of the parent VNFFG, or a SAP of one of the constituent nested NSs of the parent VNFFG. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| forwardingBehaviour | ForwardingBehaviourType | 1 | Identifies a rule to apply to forward traffic to instances created from the referenced CPD. The minimum list of rules to be supported shall include: ALL = Traffic flows shall be forwarded simultaneously to all CP or SAP instances created from this CPD or SAPD. LB = Traffic flows shall be forwarded to one CP or SAP instance created from this CPD or SAPD selected based on a load-balancing algorithm. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| forwardingBehaviourInputParameters |  | 0..1 | Provides input parameters to configure the forwarding behaviour (e.g. identifies a load balancing algorithm). | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### ForwardingBehaviourType enumeration

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::ForwardingBehaviourType

Description:

Identifies a rule to apply to forward traffic to instances created from the referenced CPD.

The minimum list of rules to be supported shall include:

ALL = Traffic flows shall be forwarded simultaneously to all CP or SAP instances created from this CPD or SAPD.

LB = Traffic flows shall be forwarded to one CP or SAP instance created from this CPD or SAPD selected based on a load-balancing algorithm.

Applied Stereotypes:

Experimental

###### MonitoredData datatype

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::MonitoredData

Description:

The MonitoredData datatype identifies information to be monitored during the lifetime of a network service instance.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type MonitoredData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vnfIndicatorInfo | VnfIndicatorData | 0..1 | Uniquely identifies the VNF Indicator class. One and only one of the attributes shall be included (i.e. there is a XOR relationship between the attributes). | OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: |
| monitoringParameter | MonitoringParameter | 0..1 | Specifies the virtualised resource related performance metric to be monitored on an NS level or VNF level. One and only one of the attributes shall be included (i.e. there is a XOR relationship between the attributes). | OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: |

###### NsLifeCycleManagementScript datatype

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::NsLifeCycleManagementScript

Description:

The LifeCycleManagementScript information element specifies a script for the NS.

Applied Stereotypes:

Experimental

Table B.1: Attributes for Data Type NsLifeCycleManagementScript

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| event | String | 1..* | Describes NS lifecycle event(s) or an external stimulus detected on an NFVO reference point. NOTE 1: A minimum set of NS lifecycle events triggered internally by the NFVO includes: start instantiation, end instantiation, start scaling, end scaling, start healing, end healing, start termination, end termination, start update, end update. NOTE 2: A minimum set of external stimulus includes: the receipt of request message of instantiation, scaling, healing, termination, update of NS. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| script |  | 1 | Includes an NS LCM script (e.g., written in a DSL) triggered to react to one of the events listed in the event attribute. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsQoS datatype

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::NsQoS

Description:

The NsQoS datatype specifies quality of service parameters applicable to a NS VL.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type NsQoS

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| priority | Integer | 0..1 | Specifies the priority level in case of congestion on the underlying physical links. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| latency | Number | 1 | Specifies the maximum latency in ms. | Inherited from Data Type QoS OpenModelAttribute - isInvariant: false - support: MANDATORY |
| packetDelayVariation | Number | 1 | Specifies the maximum jitter in ms. | Inherited from Data Type QoS OpenModelAttribute - isInvariant: false - support: MANDATORY |
| packetLossRatio | Number | 0..1 | Specifies the maximum packet loss ratio. | Inherited from Data Type QoS OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsToLevelMapping datatype

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::NsToLevelMapping

Description:

The NsToLevelMapping datatype specifies the profile to be used for a nested NS involved in a given NS level and the required number of instances.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type NsToLevelMapping

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| nsProfileId | NsProfile | 1 | Identifies the profile to be used for a nested NS involved in the NS level. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| numberOfInstances | Integer | 1 | Specifies the number of nested NS instances required for the NS scale level. NOTE 1: It shall be in the range of minNumberOfInstances-maxNumberOfInstances, as specified in the referenced NsProfile. NOTE 2: When the corresponding NS level is used in the context of NS instantiation, the numberOfInstances attribute specifies the number of nested NS instances to be deployed. NOTE 3: When the corresponding NS level is used in the context of NS scaling, the numberOfInstances attribute does not specify the number of nested NS instances to be added/removed when reaching that NS scale level. The actual number of nested NS instances to be added/removed can be derived by subtracting the numberOfInstances declared in the source NS scale level from the numberOfInstances declared in the target NS level of the scaling request. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsVirtualLinkConnectivity datatype

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::NsVirtualLinkConnectivity

Description:

The NsVirtuallLinkConnectivity datatype describes connection information between a connection point and a NS virtual Link.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type NsVirtualLinkConnectivity

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| virtualLinkProfileId | VirtualLinkProfile | 1 | Reference an NS VL profile. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| cpdId | Cpd | 1..* | References the descriptor of a connection point on a VNF/PNF or a SAP which connects to virtual links instantiated from the profile identified in the virtualLinkProfileId attribute. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |

###### VirtualLinkDf datatype

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::VirtualLinkDf

Description:

The VirtualLinkDf datatype specifies properties for instantiating a VL according to a specific flavour.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type VirtualLinkDf

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| flavourId | Identifier | 1 | Identifies this VirtualLinkDF datatype within a VLD. | OpenModelAttribute - isInvariant: true - support: MANDATORY |
| qos | NsQoS | 0..1 | Specifies quality of service parameters applicable to a VL. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| serviceAvaibilityLevel | ServiceAvailabilityLevel | 0..1 | Specifies one of the three levels defined in ETSI GS NFV-REL 001: • Level 1 • Level 2 • Level 3 | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VirtualLinkToLevelMapping datatype

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::VirtualLinkToLevelMapping

Description:

The VirtualLinkToLevelMapping datatype specifies the profile to be used for a VL involved in a given NS level and bitrate requirements.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type VirtualLinkToLevelMapping

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| virtualLinkProfileId | VirtualLinkProfile | 1 | Identifies the profile to be used for a VL involved in an NS level. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| bitRateRequirements | LinkBitrateRequirements | 1 | Specifies the bit rate requirements for the NS level. NOTE: It shall be in the range of minBitrateRequirements - maxBitrateRequirements, as specified in the referenced VirtualLinkProfile. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfIndicatorData datatype

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::VnfIndicatorData

Description:

The VnfIndicatorData datatype identifies a VNF indicator in a VNFD.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type VnfIndicatorData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vnfdId | Vnfd | 1 | Identifies a VNFD. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vnfIndicator | VnfIndicator | 1 | Identifies a VNF indicator within the VNFD. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |

###### VnfToLevelMapping datatype

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::VnfToLevelMapping

Description:

The VnfToLevelMapping datatype specifies the profile to be used for a VNF involved in a given NS level and the required number of instances.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type VnfToLevelMapping

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vnfProfileId | VnfProfile | 1 | Identifies the profile to be used for a VNF involved in an NS level. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| numberOfInstances | Integer | 1 | Specifies the number of VNF instances required for an NS level. NOTE 1: It shall be in the range of minNumberOfInstances-maxNumberOfInstances, as specified in the referenced VnfProfile. NOTE 2: When the corresponding NS level is used in the context of NS instantiation, the numberOfInstances attribute specifies the number of VNF instances to be deployed. NOTE 3: When the corresponding NS level is used in the context of NS scaling, the numberOfInstances attribute does not specify the number of VNF instances to be added/removed when reaching that NS scale level. The actual number of VNF instances to be added/removed can be derived by subtracting the numberOfInstances declared in the source NS scale level from the numberOfInstances declared in the target NS level of the scaling request. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

##### Associations

###### CpProfileRefersToNsProfile association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::CpProfileRefersToNsProfile

Applied Stereotypes:

Experimental

Table B.1: Member ends for Association CpProfileRefersToNsProfile

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| nsProfileId | none | Navig. | 0..1 | NsProfile | References the NsProfile of a nested NS. NOTE 3: For the SAPD, both nsProfileId and nsSapdId shall be present as a pair. NOTE 4: One pair of identifiers (VNF external CPD, PNF external CPD or SAPD) shall be present | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| cpprofile | none | Not navig. | 0..* | CpProfile |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### CpProfileRefersToPnfExtCpd association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::CpProfileRefersToPnfExtCpd

Applied Stereotypes:

Experimental

Table B.1: Member ends for Association CpProfileRefersToPnfExtCpd

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| pnfExtCpdId | none | Navig. | 0..1 | PnfExtCpd | Identifier of the PNF external CPD for a given PnfProfile identified by the "pnfProfileId" attribute. NOTE 2: For the PNF external CPD, both pnfProfileId and pnfExtCpdId shall be present as a pair. NOTE 4: One pair of identifiers (VNF external CPD, PNF external CPD or SAPD) shall be present. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| cpprofile | none | Not navig. | 0..* | CpProfile |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### CpProfileRefersToPnfProfile association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::CpProfileRefersToPnfProfile

Applied Stereotypes:

Experimental

Table B.1: Member ends for Association CpProfileRefersToPnfProfile

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| pnfProfileId | none | Navig. | 0..1 | PnfProfile | Reference to the PnfProfile of a constituent PNF. NOTE 2: For the PNF external CPD, both pnfProfileId and pnfExtCpdId shall be present as a pair. NOTE 4: One pair of identifiers (VNF external CPD, PNF external CPD or SAPD) shall be present | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| cpprofile | none | Not navig. | 0..* | CpProfile |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### CpProfileRefersToSapd association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::CpProfileRefersToSapd

Applied Stereotypes:

Experimental

Table B.1: Member ends for Association CpProfileRefersToSapd

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| nsSapdId | none | Navig. | 0..1 | Sapd | Identifier of the NS SAPD for a given NsProfile identified by the "nsProfileId" attribute. NOTE 3: For the SAPD, both nsProfileId and nsSapdId shall be present as a pair. NOTE 4: One pair of identifiers (VNF external CPD, PNF external CPD or SAPD) shall be present. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| cpprofile | none | Not navig. | 0..* | CpProfile |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### CpProfileRefersToVnfExtCpd association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::CpProfileRefersToVnfExtCpd

Applied Stereotypes:

Experimental

Table B.1: Member ends for Association CpProfileRefersToVnfExtCpd

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| vnfExtCpdId | none | Navig. | 0..1 | VnfExtCpd | Identifier of the VNF external CPD for a given VnfProfile identified by the "vnfProfileId" attribute. NOTE 1: For the VNF external CPD, both vnfProfileId and vnfExtCpdId shall be present as a pair. NOTE 4: One pair of identifiers (VNF external CPD, PNF external CPD or SAPD) shall be present. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| cpprofile | none | Not navig. | 0..* | CpProfile |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### CpProfileRefersToVnfProfile association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::CpProfileRefersToVnfProfile

Applied Stereotypes:

Experimental

Table B.1: Member ends for Association CpProfileRefersToVnfProfile

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| vnfProfileId | none | Navig. | 0..1 | VnfProfile | Reference to the VnfProfile of a constituent VNF. NOTE 1: For the VNF external CPD, both vnfProfileId and vnfExtCpdId shall be present as a pair. NOTE 4: One pair of identifiers (VNF external CPD, PNF external CPD or SAPD) shall be present. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| cpprofile | none | Not navig. | 0..* | CpProfile |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### CpdPoolReferencesCpd association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::CpdPoolReferencesCpd

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association CpdPoolReferencesCpd

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| cpdId | shared | Navig. | 1..* | Cpd | A reference to the descriptor of a connection point attached to one of the constituent VNFs and PNFs or to the descriptor of a NS SAP. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| cpdpool | none | Not navig. | 0..* | CpdPool |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### DependenciesHasPrimaryProfileReference association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::DependenciesHasPrimaryProfileReference

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association DependenciesHasPrimaryProfileReference

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| primaryId | none | Navig. | 1..* | NsOrVnfProfile | References a VnfProfile or NsProfile. NOTE: NFV Management and Orchestration functions shall instantiate VNFs from the VnfProfile and/or nested NSs from the NsProfile referenced in the primary attribute before instantiating VNFs from the VnfProfile and/or nested NSs from the NsProfile referenced in the secondary attribute. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| dependencies | none | Not navig. | 0..* | Dependencies |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### DependenciesHasSecondaryProfileReference association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::DependenciesHasSecondaryProfileReference

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association DependenciesHasSecondaryProfileReference

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| secondaryId | none | Navig. | 1..* | NsOrVnfProfile | References a VnfProfile or NsProfile. NOTE: NFV Management and Orchestration functions shall instantiate VNFs from the VnfProfile and/or nested NSs from the NsProfile referenced in the primary attribute before instantiating VNFs from the VnfProfile and/or nested NSs from the NsProfile referenced in the secondary attribute | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| dependencies | none | Not navig. | 0..* | Dependencies |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NfpDeployedUsingNfpd association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::NfpDeployedUsingNfpd

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association NfpDeployedUsingNfpd

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| nfpd | none | Navig. | 1 | Nfpd |  | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| nfp | none | Not navig. | 0..* | Nfp |  | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |

###### NfpdReferencesCpProfile association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::NfpdReferencesCpProfile

Applied Stereotypes:

Experimental

Table B.1: Member ends for Association NfpdReferencesCpProfile

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| cpProfileId | shared | Navig. | 1..* | CpProfile | References the descriptor of a connection point to be traversed by the traffic flows matching the criteria. This shall be a connection point profile, which is either a CPD associated with the VnfProfile of a constituent VNF, or a CPD associated with the PnfProfile of a constituent PNF, or a SAPD associated with the NsProfile of a nested NS. The related VnfProfile, PnfProfile and NsProfile shall be included in the parent VNFFGD. NOTE: When multiple values are provided, the order is significant and specifies the sequence of connection points to be traversed. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| nfpd | none | Not navig. | 0..* | Nfpd |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NfpdReferencesCpd association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::NfpdReferencesCpd

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association NfpdReferencesCpd

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| cpd | none | Navig. | 1..* | Cpd | References the descriptor of a connection point to be traversed by the traffic flows matching the criteria. This shall be a connection point attached to one of the constituent VNFs and PNFs of the parent VNFFG, or a SAP of one of the constituent nested NSs of the parent VNFFG. NOTE: When multiple values are provided, the order is significant and specifies the sequence of connection points to be traversed. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| nfpd | none | Not navig. | 0..* | Nfpd |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsDeployedUsingNsd association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::NsDeployedUsingNsd

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association NsDeployedUsingNsd

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| nsd | none | Navig. | 1 | Nsd | Reference to the NSD used to instantiate this NS. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| networkService | none | Not navig. | 0..* | NetworkService |  | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |

###### NsDfHasAffinityOrAntiAffinityGroup association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::NsDfHasAffinityOrAntiAffinityGroup

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association NsDfHasAffinityOrAntiAffinityGroup

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| affinityOrAntiAffinityGroup | composite | Navig. | 0..* | AffinityOrAntiAffinityGroup | Specifies affinity or anti-affinity relationship applicable between the VNF instances created using different VNFDs, the Virtual Link instances created using different NsVirtualLinkDescs or the nested NS instances created using different NSDs in the same affinity or anti-affinity group. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nsDf | none | Not navig. | 1 | NsDf |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsDfHasDefaultLevel association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::NsDfHasDefaultLevel

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association NsDfHasDefaultLevel

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| defaultNsInstantiationLevelId | composite | Navig. | 0..1 | NsLevel | Identifies the NS level which represents the default NS instantiation level for this DF. It shall be present if there are multiple "nsInstantiationLevel" entries. | OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: It shall be present if there are multiple "nsIinstantiationLevel" entries. PassedByReference |
| nsDf | none | Not navig. | 1 | NsDf |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsDfHasNsLevel association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::NsDfHasNsLevel

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association NsDfHasNsLevel

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| nsInstantiationLevel | composite | Navig. | 1..* | NsLevel | Describes the details of an NS level. NOTE: Every VNF, VL and nested NS whose descriptor is referenced by the NS DF shall be involved in an NS instantiation level of the parent NS. If no instances of a given VNF/nested NS have to be deployed at NS instantiation time, the numberOfInstances attribute in the corresponding VnfToLevelMapping/NsToLevelMapping datatype shall be set to 0. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nsDf | none | Not navig. | 1 | NsDf |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsDfHasNsProfile association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::NsDfHasNsProfile

Description:

Defines the connection information of the NS, it contains connection relationship between an NS SAP and a NS virtual Link of the composite NS.

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association NsDfHasNsProfile

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| nsProfile | composite | Navig. | 0..* | NsProfile | Specifies a NS Profile supported by this NS DF. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nsDf | none | Not navig. | 1 | NsDf |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsDfHasPnfProfile association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::NsDfHasPnfProfile

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association NsDfHasPnfProfile

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| pnfProfile | composite | Navig. | 0..* | PnfProfile | PNF profile to be used for the NS flavour. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nsDf | none | Not navig. | 1 | NsDf |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsDfHasProfileDependencies association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::NsDfHasProfileDependencies

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association NsDfHasProfileDependencies

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| dependencies | none | Navig. | 0..* | Dependencies | Specifies the order in which instances of the VNFs and/or nested NSs have to be created. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nsDf | none | Not navig. | 1 | NsDf |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsDfHasScalingAspect association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::NsDfHasScalingAspect

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association NsDfHasScalingAspect

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| scalingAspect | composite | Navig. | 0..* | NsScalingAspect | The scaling aspects supported by this DF of the NS. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nsDf | none | Not navig. | 1 | NsDf |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsDfHasVirtualLinkProfile association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::NsDfHasVirtualLinkProfile

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association NsDfHasVirtualLinkProfile

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| virtualLinkProfile | composite | Navig. | 0..* | VirtualLinkProfile | VL profile to be used for the NS flavour. There shall be at most one profile per constituent VNF. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nsDf | none | Not navig. | 1 | NsDf |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsDfHasVnfProfile association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::NsDfHasVnfProfile

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association NsDfHasVnfProfile

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| vnfProfile | composite | Navig. | 0..* | VnfProfile | VNF Profile to be used for the NS flavour. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nsDf | none | Not navig. | 1 | NsDf |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsProfileHasApplicableNsLevel association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::NsProfileHasApplicableNsLevel

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association NsProfileHasApplicableNsLevel

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| nsInstantiationLevelId | none | Navig. | 0..1 | NsLevel | Identifies the NS level within the referenced NS DF to be used in the context of the parent NS instantiation. If not present, the default NS instantiation level as declared in the referenced NSD shall be used. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nsprofile | none | Not navig. | 0..* | NsProfile |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsProfileRefersToApplicableNsDf association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::NsProfileRefersToApplicableNsDf

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association NsProfileRefersToApplicableNsDf

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| nsDfId | none | Navig. | 1 | NsDf | Identifies the applicable NS DF within the scope of the NSD. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| nsProfile | none | Not navig. | 0..* | NsProfile |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsProfileRefersToNsd association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::NsProfileRefersToNsd

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association NsProfileRefersToNsd

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| nsdId | none | Navig. | 1 | Nsd | Identifies the NSD applicable to NS instantiated according to this profile. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| nsProfile | none | Not navig. | 0..* | NsProfile |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsProfileUsesAffinityOrAntiAffinityGroup association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::NsProfileUsesAffinityOrAntiAffinityGroup

Description:

Identifies an affinity or anti-affinity group the NSs created according to this NsProfile belong to.

NOTE:  Each identifier references an affinity or anti-affinity group which expresses affinity or anti-affinity relationships between the NS instance(s) created using this NsProfile and the NS instance(s) created using other NsProfile(s) in the same group.

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association NsProfileUsesAffinityOrAntiAffinityGroup

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| affinityOrAntiaffinityGroupId | none | Navig. | 0..* | AffinityOrAntiAffinityGroup | Identifies an affinity or anti-affinity group the NSs created according to this NsProfile belong to. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nsProfile | none | Not navig. | 0..* | NsProfile |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsScalingAcpectHasNsLevel association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::NsScalingAcpectHasNsLevel

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association NsScalingAcpectHasNsLevel

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| nsScaleLevel | none | Navig. | 1..* | NsLevel | Describes the details of an NS level. NOTE: Only a subset of the VNFs, VLs and nested NSs whose descriptor is referenced by the NS DF may be involved in an NS scale level of the parent NS. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nsScalingAspect | none | Not navig. | 0..* | NsScalingAspect |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsVirtualLinkDeployedUsingNsVirtualLinkDesc association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::NsVirtualLinkDeployedUsingNsVirtualLinkDesc

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association NsVirtualLinkDeployedUsingNsVirtualLinkDesc

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| nsVirtualLinkDesc | none | Navig. | 1 | NsVirtualLinkDesc | VLD in the NSD for this VL. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| nsVirtualLink | none | Not navig. | 0..* | NsVirtualLink |  | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |

###### NsdHasNestedNsd association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::NsdHasNestedNsd

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association NsdHasNestedNsd

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| nestedNsdId | shared | Navig. | 0..* | Nsd | References the NSD of a constituent nested NS. | OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: At least one between nestedNsd and vnfdId attributes shall be present. PassedByReference |
| compositeNsd | none | Not navig. | 1 | Nsd |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsdHasNsDf association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::NsdHasNsDf

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association NsdHasNsDf

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| nsDf | composite | Navig. | 1..* | NsDf | Identifies a DF within the scope of an NSD. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nsd | none | Not navig. | 1 | Nsd |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsdHasSapd association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::NsdHasSapd

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association NsdHasSapd

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| sapd | composite | Navig. | 0..* | Sapd | Provides the descriptor of a SAP of the NS. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nsd | none | Not navig. | 1 | Nsd |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsdHasVirtualLinkDesc association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::NsdHasVirtualLinkDesc

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association NsdHasVirtualLinkDesc

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| virtualLinkDesc | composite | Navig. | 0..* | NsVirtualLinkDesc | Provides the constituent VLDs. Cardinality of 0 means that the NS is a NF set with unspecified connectivity. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nsd | none | Not navig. | 1 | Nsd |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsdHasVnffgd association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::NsdHasVnffgd

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association NsdHasVnffgd

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| vnffgd | composite | Navig. | 0..* | Vnffgd | Provides the descriptors of the applicable forwarding graphs. Cardinality of 0 means that the NS is a NF set with unspecified connectivity. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nsd | none | Not navig. | 1 | Nsd |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsdReferencesPnfd association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::NsdReferencesPnfd

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association NsdReferencesPnfd

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| pnfdId | shared | Navig. | 0..* | Pnfd | References the PNFD of a constituent PNF. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| nsd | none | Not navig. | 0..* | Nsd |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsdReferencesVnfd association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::NsdReferencesVnfd

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association NsdReferencesVnfd

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| vnfdId | shared | Navig. | 0..* | Vnfd | References the VNFD of a constituent VNF. | OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: At least one between nestedNsd and vnfdId attributes shall be present. PassedByReference |
| nsd | none | Not navig. | 0..* | Nsd |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### PnfDeployedUsingPnfd association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::PnfDeployedUsingPnfd

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association PnfDeployedUsingPnfd

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| pnfd | none | Navig. | 1 | Pnfd | PNFD information related to this PNF. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| pnf | none | Not navig. | 0..* | Pnf |  | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |

###### PnfExtCpDeployedUsingCpd association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::PnfExtCpDeployedUsingCpd

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association PnfExtCpDeployedUsingCpd

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| cpd | none | Navig. | 1 | Cpd | Cpd used to deploy this PNF CP. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| pnfExtCp | none | Not navig. | 0..* | PnfExtCp |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### PnfProfileReferencesPnfd association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::PnfProfileReferencesPnfd

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association PnfProfileReferencesPnfd

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| pnfdId | none | Navig. | 1 | Pnfd | References a PNFD. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| pnfProfile | none | Not navig. | 0..* | PnfProfile |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### PnfdHasPnfExtCpd association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::PnfdHasPnfExtCpd

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association PnfdHasPnfExtCpd

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| pnfExtCpd | composite | Navig. | 1..* | PnfExtCpd | Specifies the characteristics of one or more connection points where to connect the PNF to a VL. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| pnfd | none | Not navig. | 1 | Pnfd |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### SapDeployedUsingSapd association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::SapDeployedUsingSapd

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association SapDeployedUsingSapd

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| sapd | none | Navig. | 1 | Sapd | Reference to the SAPD for this SAP. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| sap | none | Not navig. | 0..* | Sap |  | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |

###### SapdConnectsTo NsVirtualLinkDesc association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::SapdConnectsTo NsVirtualLinkDesc

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association SapdConnectsTo NsVirtualLinkDesc

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| nsVirtualLinkDescId | none | Navig. | 0..1 | NsVirtualLinkDesc | References the descriptor of the NS VL instance to which the SAP instantiated from this SAPD connects to. | OpenModelAttribute - isInvariant: false - support: CONDITIONAL_MANDATORY - condition: At most one of the two attributes associatedCpdId and nsVirtualLinkDescId shall be included. PassedByReference |
| sapd | none | Not navig. | 0..1 | Sapd |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### SapdHasAssociatedCpd association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::SapdHasAssociatedCpd

Applied Stereotypes:

Experimental

Table B.1: Member ends for Association SapdHasAssociatedCpd

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| associatedCpdId | none | Navig. | 0..1 | ExtCpd | References the descriptor of VNF or PNF external connection points or the descriptor of the SAP of a nested NS of this N, which the SAPs instantiated from this SAPD are mapped to. | Experimental OpenModelAttribute - isInvariant: false - support: CONDITIONAL_MANDATORY - condition: At most one of the two attributes associatedCpdId and nsVirtualLinkDescId shall be included. PassedByReference |
| sapd | none | Not navig. | 0..1 | Sapd |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VirtualLinkProfileReferencesVirtualLinkDesc association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::VirtualLinkProfileReferencesVirtualLinkDesc

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VirtualLinkProfileReferencesVirtualLinkDesc

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| virtualLinkDescId | none | Navig. | 1 | NsVirtualLinkDesc | Uniquely references a VLD. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vlProfile | none | Not navig. | 0..* | VirtualLinkProfile |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfFgdHasConstituentPnfds association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::VnfFgdHasConstituentPnfds

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfFgdHasConstituentPnfds

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| pnfdId | shared | Navig. | 0..* | Pnfd | References the PNFD of a constituent PNF. NOTE 1 : The list of constituent VNFs, PNFs, and nested NS SAPs of a VNFFG can be identical to the list of constituent VNFs, PNFs and nested NSs of the parent NSD, or can be a subset of it. NOTE 2: The presence of a VNFD or PNFD identifier in a VNFFGD does not imply that all connection points instantiated from all embedded CPDs are connected to the VNFFG instantiated using the VNFFGD. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vnffgd | none | Not navig. | 0..* | Vnffgd |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfProfileReferencesVnfDf association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::VnfProfileReferencesVnfDf

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfProfileReferencesVnfDf

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| flavourId | none | Navig. | 1 | VnfDf | Identifies a flavour within the VNFD. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vnfProfile | none | Not navig. | 0..* | VnfProfile |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfProfileReferencesVnfd association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::VnfProfileReferencesVnfd

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfProfileReferencesVnfd

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| vnfdId | none | Navig. | 1 | Vnfd | References a VNFD. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vnfProfile | none | Not navig. | 0..* | VnfProfile |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnffgDeployedUsingVnffgd association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::VnffgDeployedUsingVnffgd

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnffgDeployedUsingVnffgd

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| vnffgd | none | Navig. | 1 | Vnffgd | VNFFGD used to deploy this VNFFG. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vnffg | none | Not navig. | 0..* | Vnffg |  | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |

###### VnffgdHasConstituentVirtualLinkDesc association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::VnffgdHasConstituentVirtualLinkDesc

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnffgdHasConstituentVirtualLinkDesc

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| virtualLinkDescId | shared | Navig. | 1..* | NsVirtualLinkDesc | References the VLD of a constituent VL. NOTE: The presence of a VLD identifier in a VNFFGD does not imply that all connection points of the VLs instantiated using this VL are connected to the VNFFG instantiated using the VNFFGD. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vnffgd | none | Not navig. | 0..* | Vnffgd |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnffgdHasConstituentVnfds association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::VnffgdHasConstituentVnfds

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnffgdHasConstituentVnfds

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| vnfdId | shared | Navig. | 1..* | Vnfd | References the VNFD of a constituent VNF. NOTE 1: The list of constituent VNFs, PNFs, and nested NS SAPs of a VNFFG can be identical to the list of constituent VNFs, PNFs and nested NSs of the parent NSD, or can be a subset of it. NOTE 2: The presence of a VNFD or PNFD identifier in a VNFFGD does not imply that all connection points instantiated from all embedded CPDs are connected to the VNFFG instantiated using the VNFFGD. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vnffgd | none | Not navig. | 0..* | Vnffgd |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnffgdHasNfpd association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::VnffgdHasNfpd

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnffgdHasNfpd

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| nfpd | composite | Navig. | 0..* | Nfpd | The network forwarding path associated to the VNFFG. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnffgd | none | Not navig. | 1 | Vnffgd |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnffgdReferencesCpdPool association

Qualified Name: NfvInformationModel::NFVCoreModel::NsDomain::NsTemplateModule::VnffgdReferencesCpdPool

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnffgdReferencesCpdPool

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| cpdPoolId | shared | Navig. | 1..* | CpdPool | A reference to a pool of descriptors of connection points attached to one of the constituent VNFs and PNFs and/or one of the SAPs of the parent NS or of a nested NS. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vnffgd | none | Not navig. | 1 | Vnffgd |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

## VirtualNetworkFunctionDomain

### Overview

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain

Description:

Group all the core classes related to VNFs

### VnfModule

#### Overview

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfModule

Description:

Group all the core classes related to VNFs.

The classes in this module have no direct representation in an IFA specification and are abstracted from the corresponding classes in the NFV Interface Information Model.

##### Diagrams

Figure B.1: Diagram BasicVnfLogicalModel

Figure B.1: Diagram VNFLogicalModel

Figure B.1: Diagram BasicVNFApplicationModel

##### Classes

###### ManagedFunction class

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfModule::ManagedFunction

Description:

Describes the application part of the VNF. Outside of ETSI NFV scope.

Applied Stereotypes:

Obsolete

OpenModelClass

support:  MANDATORY

Table B.1: Attributes for Class ManagedFunction

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vnf | Vnf | 0..1 |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfc | Vnfc | 0..* |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NetworkFunction class

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfModule::NetworkFunction

Description:

A functional block within a network infrastructure that has well-defined external interfaces and well-defined functional behaviour. NOTE: In practical terms, a Network Function is today often a network node or physical appliance

This class is abstract.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

###### Vnf class

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfModule::Vnf

Description:

An implementation of an NF that can be deployed on a Network Function Virtualisation Infrastructure (NFVI).

Parent class: NetworkFunction

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class Vnf

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vnfInstanceId | Identifier | 1 | Identifier of the VNF instance | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfd | Vnfd | 1 | Identifier of the VNFD on which the VNF instance is based. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vnfc | Vnfc | 1..* | VNFCs of this VNF | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| VnfExtCp | VnfExtCp | 1..* | External CPs exposed by the VNF instance. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfVirtualLink | VnfVirtualLink | 0..* | VLs of this VNF instance | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualStorage | VirtualStorage | 0..* | Virtual storage for this VNF. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfExtCp class

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfModule::VnfExtCp

Description:

Describes an external CP exposed by a VNF.

Parent class: Cp

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VnfExtCp

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vnfExtCpd | VnfExtCpd | 1 | VnfExtCpd applicable to this VnfExtCp. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| cp | VnfcCp | 0..1 | Vnfc internal CP that this external CP maps to. | OpenModelAttribute - isInvariant: false - support: CONDITIONAL_MANDATORY - condition: Only one of cp or linkPort can be present. |
| linkPort | LinkPort | 0..1 | Link port that this external CP maps to. | OpenModelAttribute - isInvariant: false - support: CONDITIONAL_MANDATORY - condition: Only one of cp or linkPort can be present. |
| cpId | Identifier | 1 | Identifier of this CP. | Inherited from Class Cp OpenModelAttribute - isInvariant: true - support: MANDATORY |

###### VnfLinkPort class

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfModule::VnfLinkPort

Description:

Represents a port of a Vnf VL

Parent class: LinkPort

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table A.1: Attributes for Class VnfLinkPort

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| cp | Cp | 0..1 | CP to be connected to this link port. | Inherited from Class LinkPort OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfVirtualLink class

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfModule::VnfVirtualLink

Parent class: VirtualLink

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VnfVirtualLink

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vnfVirtualLinkDesc | VnfVirtualLinkDesc | 1 | VnfVirtualLinkDesc applicable to this VL. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| virtualNetwork | VirtualNetwork | 0..1 | The virtual network that realises the virtual link. NOTE: Cardinality 0..1 is to reflect the case that a VirtualLink may be realized by other means than a VirtualNetwork. Therefore, the model is not complete as these other realizations need to be defined. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| linkPort | LinkPort | 0..* | Link ports of this VL. | Inherited from Class VirtualLink OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### Vnfc class

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfModule::Vnfc

Description:

An internal component of a VNF providing a VNF Provider a defined sub-set of that VNF's functionality, with the main characteristic that a single instance of this component maps 1:1 against a single Virtualisation Container.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class Vnfc

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vnfcInstanceId | Identifier | 1 | Identifier of this VNFC instance. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vdu | Vdu | 1 | Applicable Vdu class in the VNFD. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| cp | VnfcCp | 1..* | CPs of this VNFC. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualCompute | VirtualCompute | 1 | VirtualCompute resource used by this VNFC. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualStorage | VirtualStorage | 0..* | VirtualStorage resource(s) used by this VNFC. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualisationContainer | VirtualisationContainer | 1 | Virtualisation container that this VNFC runs on. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfcCp class

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfModule::VnfcCp

Description:

Internal Cps of the VNFC

Parent class: Cp

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VnfcCp

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vduCpd | VduCpd | 1 | Cpd applicable to this CP. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| cpId | Identifier | 1 | Identifier of this CP. | Inherited from Class Cp OpenModelAttribute - isInvariant: true - support: MANDATORY |

##### Associations

###### ManagedFunctionRealizedAsVnf association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfModule::ManagedFunctionRealizedAsVnf

Applied Stereotypes:

Obsolete

Table B.1: Member ends for Association ManagedFunctionRealizedAsVnf

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| vnf | none | Navig. | 0..1 | Vnf |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| managedFunction | none | Not navig. | 0..* | ManagedFunction |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### ManagedFunctionRealizedAsVnfc association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfModule::ManagedFunctionRealizedAsVnfc

Applied Stereotypes:

Obsolete

Table B.1: Member ends for Association ManagedFunctionRealizedAsVnfc

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| vnfc | none | Navig. | 0..* | Vnfc |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| managedfunction | none | Not navig. | 0..* | ManagedFunction |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfExposesExtCp association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfModule::VnfExposesExtCp

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfExposesExtCp

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| VnfExtCp | composite | Navig. | 1..* | VnfExtCp | External CPs exposed by the VNF instance. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnf | none | Not navig. | 1 | Vnf |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfExtCpMapsToLinkPort association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfModule::VnfExtCpMapsToLinkPort

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfExtCpMapsToLinkPort

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| linkPort | none | Navig. | 0..1 | LinkPort | Link port that this external CP maps to. | OpenModelAttribute - isInvariant: false - support: CONDITIONAL_MANDATORY - condition: Only one of cp or linkPort can be present. |
| vnfExtCp | none | Not navig. | 0..1 | VnfExtCp |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfExtCpMapsToVnfcCp association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfModule::VnfExtCpMapsToVnfcCp

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfExtCpMapsToVnfcCp

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| cp | none | Navig. | 0..1 | VnfcCp | Vnfc internal CP that this external CP maps to. | OpenModelAttribute - isInvariant: false - support: CONDITIONAL_MANDATORY - condition: Only one of cp or linkPort can be present. |
| vnfExtCp | none | Not navig. | 0..1 | VnfExtCp |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfHasVirtualLink association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfModule::VnfHasVirtualLink

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfHasVirtualLink

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| vnfVirtualLink | shared | Navig. | 0..* | VnfVirtualLink | VLs of this VNF instance | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnf | none | Not navig. | 0..* | Vnf |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfHasVnfc association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfModule::VnfHasVnfc

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfHasVnfc

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| vnfc | composite | Navig. | 1..* | Vnfc | VNFCs of this VNF | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnf | none | Not navig. | 1 | Vnf |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfUsesVirtualStorage association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfModule::VnfUsesVirtualStorage

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfUsesVirtualStorage

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| virtualStorage | none | Navig. | 0..* | VirtualStorage | Virtual storage for this VNF. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnf | none | Not navig. | 0..1 | Vnf |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfVirtualLinkRealisedAsVn association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfModule::VnfVirtualLinkRealisedAsVn

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfVirtualLinkRealisedAsVn

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| virtualNetwork | none | Navig. | 0..1 | VirtualNetwork | The virtual network that realises the virtual link. NOTE: Cardinality 0..1 is to reflect the case that a VirtualLink may be realized by other means than a VirtualNetwork. Therefore, the model is not complete as these other realizations need to be defined. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualLink | none | Not navig. | 0..1 | VnfVirtualLink |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfcHasCp association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfModule::VnfcHasCp

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfcHasCp

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| cp | composite | Navig. | 1..* | VnfcCp | CPs of this VNFC. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfc | none | Not navig. | 1 | Vnfc |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfcRunsOnVc association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfModule::VnfcRunsOnVc

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfcRunsOnVc

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| virtualisationContainer | none | Navig. | 1 | VirtualisationContainer | Virtualisation container that this VNFC runs on. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfc | none | Not navig. | 1 | Vnfc |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfcUsesVirtualCompute association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfModule::VnfcUsesVirtualCompute

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfcUsesVirtualCompute

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| virtualCompute | none | Navig. | 1 | VirtualCompute | VirtualCompute resource used by this VNFC. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfc | none | Not navig. | 0..1 | Vnfc |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfcUsesVirtualStorage association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfModule::VnfcUsesVirtualStorage

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfcUsesVirtualStorage

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| virtualStorage | none | Navig. | 0..* | VirtualStorage | VirtualStorage resource(s) used by this VNFC. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfc | none | Not navig. | 0..1 | Vnfc |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

### VnfTemplateModule

#### Overview

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule

Description:

Group all the core classes related to VNF templates.

##### Diagrams

Figure B.1: Diagram BasicVNFDeploymentModel

Figure B.1: Diagram VNFDeploymentView

Figure B.1: Diagram VNFDetailedDeploymentView

Figure B.1: Diagram LinkingLogicalAndDeploymentViews_VNF

Figure B.1: Diagram VduDeploymentView

Figure B.1: Diagram VnfDeploymentFlavour

Figure B.1: Diagram VnfSimplifiedDeploymentView

Figure B.1: Diagram VduSimplifiedDeploymentView

##### Classes

###### InstantiationLevel class

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::InstantiationLevel

Description:

The InstantiationLevel class describes a given level of resources to be instantiated within a DF in term of the number of VNFC instances to be created from each VDU.

All the VDUs referenced in the level shall be part of the corresponding DF and their number shall be within the range (min/max) for this DF.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class InstantiationLevel

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| levelId | Identifier | 1 | Uniquely identifies a level with the DF. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| description | String | 1 | Human readable description of the level. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vduLevel | VduLevel | 1..* | Indicates the number of instance of this VDU to deploy for this level. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualLinkBitRateLevel | VirtualLinkBitRateLevel | 0..* | Specifies bitrate requirements applicable to virtual links created from particular virtual link descriptors for this level. If not present, it is assumed that the bitrate requirements can be derived from those specified in the VduCpd instances applicable to the internal VL. If present in both the InstantiationLevel and the VduCpd instances applicable to the internal VL, the highest value takes precedence. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| scaleInfo | ScaleInfo | 0..* | Represents for each aspect the scale level that corresponds to this instantiation level. scaleInfo shall be present if the VNF supports scaling. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### ScalingAspect class

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::ScalingAspect

Description:

The ScalingAspect class describes the details of an aspect used for horizontal scaling.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class ScalingAspect

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| id | Identifier | 1 | Unique identifier of this aspect in the VNFD. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| name | String | 1 | Human readable name of the aspect. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| description | String | 1 | Human readable description of the aspect. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| associatedGroup |  | 0..1 | Reference to the group of VNFD elements defining this aspect. If present, scaling for this aspect is limited to the elements of the group, but there is no assumption that all the elements of the groups will be increased at each step. NOTE 1: In the present release of the specification, support for modifying the internal VNF topology during the scaling of the internal VLs, is not required. NOTE 4: It is discouraged to provide both associatedGroup and aspectIncrementDetails, but if they are both present they shall not conflict. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| maxScaleLevel | PositiveInteger | 1 | The maximum scaleLevel for total number of scaling steps that can be applied w.r.t. this aspect. The value of this attribute corresponds to the number of scaling steps can be applied to this aspect when scaling it from the minimum scale level (i.e. 0) to the maximum scale level defined by this attribute. NOTE: A scaling step is the smallest increment by which a VNF can be scaled for a particular aspect. Scaling by a single step does not imply that only one VNFC instance is created or removed. It means that one or more VNFC instances are created from the same VDU or from different VDUs, or that a more complex setup occurs. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| aspectDeltaDetails | AspectDeltaDetails | 0..1 | A specification of the deltas in terms of number of instances of VNFCs and virtual link bit rates that correspond to the scaling steps of this aspect. A cardinality of zero indicates that this mapping has to be specified in a lifecycle management script or be otherwise known to the VNFM. The information in this attribute, if provided, shall be consistent with the information provided in the "InstantiationLevel" information element. If this attribute is provided, it shall be provided for all scaling aspects NOTE 3: The presence of this attribute does not preclude associating lifecycle management scripts to scaling-related events in the VNFD. NOTE 4: It is discouraged to provide both associatedGroup and aspectIncrementDetails, but if they are both present they shall not conflict. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### SwImage class

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::SwImage

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

###### SwImageDesc class

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::SwImageDesc

Description:

This information element describes the software image for a particular VDU or a virtual storage resource.

Applied Stereotypes:

Experimental

OpenModelClass

support:  MANDATORY

Table B.1: Attributes for Class SwImageDesc

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| id | Identifier | 1 | The identifier of this software image. | OpenModelAttribute - isInvariant: true - support: MANDATORY |
| name | String | 1 | The name of this software image. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| version | String | 1 | The version of this software image. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| checksum | String | 1 | The checksum of the software image file. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| containerFormat | String | 1 | The container format describes the container file format in which software image is provided. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| diskFormat | String | 1 | The disk format of a software image is the format of the underlying disk image | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| minDisk | Number | 1 | The minimal disk size requirement for this software image. The value of the "size of storage" attribute of the VirtualStorageDesc referencing this SwImageDesc shall not be smaller than the value of minDisk. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| minRam | Number | 0..1 | The minimal RAM requirement for this software image. The value of the "size" attribute of VirtualMemoryData of the Vdu referencing this SwImageDesc shall not be smaller than the value of minRam. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| size | Number | 1 | The size of this software image. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| swImage | SwImage | 1 | This is a reference to the actual software image. The reference can be relative to the root of the VNF Package or can be a URL | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| operatingSystem | String | 0..1 | Identifies the operating system used in the software image. This attribute may also identify if a 32bit or 64 bit software image is used. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| supportedVirtualisationEnvironment | String | 0..* | Identifies the virtualisation environments (e.g. hypervisor) compatible with this software image. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### Vdu class

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::Vdu

Description:

The Virtualisation Deployment Unit (VDU) is a construct supporting the description of the deployment and operational behaviour of a VNFC.

A VNFC instance created based on the VDU maps to a single virtualisation container (e.g.; a VM).

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class Vdu

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vduId | Identifier | 1 | Unique identifier of this VDU in VNFD. | OpenModelAttribute - isInvariant: true - support: MANDATORY |
| name | String | 1 | Human readable name of the Vdu. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| description | String | 1 | Human readable description of the Vdu. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| intCpd | VduCpd | 1..* | Describes network connectivity between a VNFC instance (based on this Vdu) and an internal Virtual Link (VL). | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualComputeDesc | VirtualComputeDesc | 1 | Describes CPU, Memory and acceleration requirements of the Virtualisation Container realizing this Vdu. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualStorageDesc | VirtualStorageDesc | 0..* | Describes storage requirements for a VirtualStorage instance attached to the virtualisation container created from virtualComputeDesc defined for this Vdu. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| bootOrder | KeyValuePair | 0..* | The key indicates the boot index (lowest index defines highest boot priority). The Value references a descriptor from which a valid boot device is created e.g. VirtualStorageDesc from which a VirtualStorage instance is created. NOTE: If no boot order is defined the default boot order defined in the VIM or NFVI shall be used. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| swImageDesc | SwImageDesc | 0..1 | Describes the software image which is directly loaded on the virtualisation container realizing this Vdu. NOTE: More software images can be attached to the virtualisation container using VirtualStorage resources. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| nfviConstraint | String | 0..* | Describes constraints on the NFVI for the VNFC instance(s) created from this Vdu. For example, aspects of a secure hosting environment for the VNFC instance that involve additional entities or processes. NOTE: These are constraints other than stipulating that a VNFC instance has access to a certain resource, as a prerequisite to instantiation. The attributes virtualComputeDesc and virtualStorageDesc define the resources required for instantiation of the VNFC instance. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| monitoringParameter | MonitoringParameter | 0..* | Specifies the virtualised resource related performance metrics on the VDU level to be tracked by the VNFM. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| configurableProperties | VnfcConfigurableProperties | 1 | Describes the configurable properties of all VNFC instances based on this VDU | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VduCpd class

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VduCpd

Description:

Describes network connectivity between a VNFC instance (based on this VDU) and an internal Virtual Link.

Parent class: Cpd

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VduCpd

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| intVirtualLinkDesc | VnfVirtualLinkDesc | 0..1 | Reference(s) of the Virtual Link Descriptor which this Connection Point Descriptor(s) connects to. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| bitrateRequirement | Number | 0..1 | Bitrate requirement on this CP. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualNetworkInterfaceRequirements | VirtualNetworkInterfaceRequirements | 0..* | Specifies requirements on a virtual network interface realising the CPs instantiated from this CPD. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| addressData | AddressData | 0..* | Provides information on the addresses to be assigned to the CP(s) instantiated from this CPD. | Obsolete OpenModelAttribute - isInvariant: false - support: MANDATORY |
| cpProtocol | CpProtocolData | 1..* | Identifies the protocol layering information the CP uses for connectivity purposes and associated information. There shall be one cpProtocol for each layer protocol as indicated by the attribute layerProtocol. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| cpdId | Identifier | 1 | Identifier of this Cpd class. | Inherited from Class Cpd OpenModelAttribute - isInvariant: true - support: MANDATORY |
| layerProtocol | LayerProtocol | 1..* | Identifies a protocol that the connection points corresponding to the CPD support for connectivity purposes (e.g. Ethernet, MPLS, ODU2, IPV4, IPV6, Pseudo-Wire, etc.). NOTE: This information determines, amongst other things, which type of address to assign to the connection point at instantiation time. | Inherited from Class Cpd OpenModelAttribute - isInvariant: false - support: MANDATORY |
| cpdRole | String | 0..1 | Identifies the role of the connection points corresponding to the CPD in the context of the traffic flow patterns in the VNF, PNF or NS. For example an NS with a tree flow pattern within the NS will have legal cpRoles of ROOT and LEAF. | Inherited from Class Cpd OpenModelAttribute - isInvariant: false - support: MANDATORY |
| description | String | 0..1 | Provides human-readable information on the purpose of the connection point (e.g. connection point for control plane traffic). | Inherited from Class Cpd OpenModelAttribute - isInvariant: false - support: MANDATORY |
| trunkMode | Boolean | 0..1 | Information about whether the Cp instantiated from this CPD is in Trunk mode (802.1Q or other). When operating in "trunk mode", the Cp is capable of carrying traffic for several VLANs. A cardinality of 0 implies that trunkMode is not configured for the Cp i.e.it is equivalent to Boolean value "false". | Inherited from Class Cpd Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VduLevel class

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VduLevel

Description:

The VduLevel class indicates for a given VDU in a given level the number of instances to deploy.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VduLevel

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vduId | Vdu | 1 | Uniquely identifies a VDU. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| numberOfInstances | Integer | 1 | Number of instances of VNFC based on this VDU to deploy for an instantiation level or for a scaling delta. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VduProfile class

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VduProfile

Description:

The VduProfile describes additional instantiation data for a given VDU used in a DF.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VduProfile

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vduId | Vdu | 1 | Uniquely identifies a VDU. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| minNumberOfInstances | Integer | 1 | Minimum number of instances of the VNFC based on this VDU that is permitted to exist for this flavour. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| maxNumberOfInstances | Integer | 1 | Maximum number of instances of the VNFC based on this VDU that is permitted to exist for this flavour. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| localAffinityOrAntiAffinityRule | LocalAffinityOrAntiAffinityRule | 0..* | Specifies affinity or anti-affinity rules applicable between the virtualisation containers (e.g. virtual machines) to be created based on this VDU. When the cardinality is greater than 1, both affinity rule(s) and anti-affinity rule(s) with different scopes (e.g. "Affinity with the scope resource zone and anti-affinity with the scope NFVI node") are applicable to the virtualisation containers (e.g. virtual machines) to be created based on this VDU. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| affinityOrAntiAffinityGroupId | Identifier | 0..* | Identifier(s) of the affinity or anti-affinity group(s) the VDU belongs to. NOTE: Each identifier references an affinity or anti-affinity group which expresses affinity or anti-affinity relationships between the virtualisation container(s) (e.g. virtual machine(s)) to be created using this VDU and the virtualisation container(s) (e.g. virtual machine(s)) to be created using other VDU(s) in the same group. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VirtualComputeDesc class

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VirtualComputeDesc

Description:

The VirtualComputeDesc class supports the specification of requirements related to virtual compute resources.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VirtualComputeDesc

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| virtualComputeDescId | Identifier | 1 | Unique identifier of this VirtualComputeDesc in the VNFD. | OpenModelAttribute - isInvariant: true - support: MANDATORY |
| logicalNode | LogicalNodeData | 1..* | The logical Node requirements | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| requestAdditionalCapabilities | RequestedAdditionalCapabilityData | 0..* | Specifies requirements for additional capabilities. These may be for a range of purposes. One example is acceleration related capabilities. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| computeRequirements |  | 0..* | Specifies compute requirements | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualMemory | VirtualMemoryData | 1 | The virtual memory of the virtualised compute. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualCpu | VirtualCpuData | 1 | The virtual CPU(s) of the virtualised compute. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VirtualLinkDescFlavour class

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VirtualLinkDescFlavour

Description:

The VirtualLinkDescFlavour describes additional instantiation data for a given internal VL used in a DF.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VirtualLinkDescFlavour

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| flavourId | Identifier | 1 | Identifies a flavour within a VnfVirtualLinkDesc | OpenModelAttribute - isInvariant: true - support: MANDATORY |
| bitrateRequirements | LinkBitrateRequirements | 0..1 | Bitrate requirements for a VL created from this VirtualLinkDescFlavour. NOTE: If not present, it is assumed that the bitrate requirements can be derived from those specified in the VduCpd instances applicable to the internal VL. If present in both the VirtualLinkDescFlavour and the VduCpd instances applicable to the internal VL, the highest value takes precedence. | Obsolete OpenModelAttribute - isInvariant: false - support: MANDATORY |
| qos | VnfQoS | 0..1 | QoS of the VL. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VirtualLinkProfile class

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VirtualLinkProfile

Description:

The VirtualLinkProfile describes additional instantiation data for a given VL used in a DF.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VirtualLinkProfile

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vnfVirtualLinkDescId | VnfVirtualLinkDesc | 1 | Uniquely identifies a Vnf VLD. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| flavourId | VirtualLinkDescFlavour | 1 | Identifies a flavour within the VnfVirtualLinkDesc. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| localAffinityOrAntiAffinityRule | LocalAffinityOrAntiAffinityRule | 0..* | Specifies affinity or anti-affinity rules applicable between the VLs based on this VnfVirtualLinkDesc. When the cardinality is greater than 1, both affinity rule(s) and anti-affinity rule(s) with different scopes are applicable to the VLs based on this VnfVirtualLinkDesc. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| affinityOrAntiAffinityGroupId | Identifier | 0..* | Identifier(s) of the affinity or anti-affinity group(s) the VnfVirtualLinkDesc belongs to. NOTE: Each identifier references an affinity or anti-affinity group which expresses affinity or anti-affinity relationship between the VL(s) using this VnfVirtualLinkDesc and the VL(s) using other VnfVirtualLinkDesc(s) in the same group. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| maxBitRateRequirements | LinkBitrateRequirements | 1 | Specifies the maximum bitrate requirements for a VL instantiated according to this profile. minBitRateRequirements and maxBitRateRequirementsare used to control scaling boundaries. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| minBitRateRequirements | LinkBitrateRequirements | 1 | Specifies the minimum bitrate requirements for a VL instantiated according to this profile. minBitRateRequirements and maxBitRateRequirementsare used to control scaling boundaries. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VirtualNetworkInterfaceRequirements class

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VirtualNetworkInterfaceRequirements

Description:

This class specifies requirements on a virtual network interface.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VirtualNetworkInterfaceRequirements

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| name | String | 0..1 | Provides a human readable name for the requirement. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| description | String | 0..1 | Provides a human readable description of the requirement. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| supportMandatory | Boolean | 1 | Indicates whether fulfilling the constraint is mandatory (TRUE) for successful operation or desirable (FALSE). | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| requirement |  | 1 | Specifies a requirement such as the support of SR-IOV, a particular data plane acceleration library, an API to be exposed by a NIC, etc. | Obsolete OpenModelAttribute - isInvariant: false - support: MANDATORY |
| networkInterfaceRequirements |  | 1 | The network interface requirements. An element from an array of key-value pairs that articulate the network interface deployment requirements. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nicIoRequirements | LogicalNodeData | 0..1 | This references (couples) the CPD with any logical node I/O requirements (for network devices) that may have been created. Linking these attributes is necessary so that so that I/O requirements that need to be articulated at the logical node level can be associated with the network interface requirements associated with the CPD. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |

###### VirtualStorageDesc class

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VirtualStorageDesc

Description:

The VirtualStorageDesc class supports the specifications of requirements related to virtual storage resources.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VirtualStorageDesc

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| id | Identifier | 1 | Unique identifier of this VirtualStorageDesc in the VNFD. | OpenModelAttribute - isInvariant: true - support: MANDATORY |
| typeOfStorage | String | 1 | Type of virtualised storage resource (e.g. volume, object). | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| sizeOfStorage | Number | 1 | Size of virtualised storage resource (e.g. size of volume, in GB). | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| rdmaEnabled | Boolean | 0..1 | Indicate if the storage support RDMA. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| swImageDesc | SwImageDesc | 0..1 | Software image to be loaded on the VirtualStorage resource created based on this VirtualStorageDesc. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |

###### VnfDf class

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VnfDf

Description:

A specific deployment version of a VNF with specific requirements for capacity and performance

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VnfDf

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| flavourId | Identifier | 1 | Identifier of this DF within the VNFD. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| description | String | 1 | Human readable description of the DF. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vduProfile | VduProfile | 1..* | Describes additional instantiation data for the VDUs used in this flavour. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualLinkProfile | VirtualLinkProfile | 0..* | Defines the internal VLD along with additional data which is used in this DF. NOTE 1: This allows for different VNF internal topologies between DFs. NOTE 2: virtualLinkProfile needs to be provided for all VLs that the CPs of the VDUs in the VDU profiles connect to. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| instantiationLevel | InstantiationLevel | 1..* | Describes the various levels of resources that can be used to instantiate the VNF using this flavour. Examples: Small, Medium, Large. If there is only one "instantiationLevel" entry, it shall be treated as the default instantiation level for this DF. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| defaultInstantiationLevel | Identifier | 0..1 | This attribute references the "instantiationLevel" entry which defines the default instantiation level for this DF. It shall be present if there are multiple "instantiationLevel" entries. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| supportedOperation | SupportedOperations | 0..* | Indicates which operations are available for this DF via the VNF LCM interface. Instantiate VNF, Query VNF and Terminate VNF are supported in all DFs and therefore need not be included in this list. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfLcmOperationsConfiguration | VnfLcmOperationsConfiguration | 1 | Configuration parameters for the VNF Lifecycle Management operations. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| affinityOrAntiAffinityGroup | AffinityOrAntiAffinityGroup | 0..* | Specifies affinity or anti-affinity relationship applicable between the virtualisation containers (e.g. virtual machines) to be created using different VDUs or internal VLs to be created using different VnfVirtualLinkDesc(s) in the same affinity or anti-affinity group. NOTE: In the present specification, including either VDU(s) or VnfVirtualLinkDesc(s) into the same affinity or anti-affinity group is supported. Extension to support including both VDU(s) and VnfVirtualLinkDesc(s) into the same affinity or anti-affinity group is left for future specification. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfIndicator | VnfIndicator | 0..* | Declares the VNF indicators that are supported by this VNF (specific to this DF). | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| monitoringParameter | MonitoringParameter | 0..* | Specifies the virtualised resource related performance metrics to be tracked by the VNFM. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| scalingAspect | ScalingAspect | 0..* | The scaling aspects supported by this DF of the VNF. scalingAspect shall be present if the VNF supports scaling. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| initialDelta | ScalingDelta | 0..1 | Represents the minimum size of the VNF (i.e. scale level zero for all scaling aspects). Shall be present if the "aspectDeltaDetails" attribute is present in the "ScalingAspect" information element. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfExtCpd class

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VnfExtCpd

Description:

A VnfExtCpd is a type of Cpd and describes an external interface, a.k.a external CP, exposed by this VNF enabling connection with a VL.

When the VnfExtCpd is mapped to a VduCpd, the values for the attributes type, subType and description shall be identical for both elements.

Parent class: ExtCpd

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VnfExtCpd

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| intVirtualLinkDesc | VnfVirtualLinkDesc | 0..1 | Reference to the internal Virtual Link Descriptor (VLD) to which CPs instantiated from this external CP Descriptor (CPD) connect. | OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: Either intVirtualLinkDesc or intCpd shall be present. PassedByReference |
| intCpd | VduCpd | 0..1 | Reference to the internal VDU CPD which is used to instantiate internal CPs. These internal CPs are, in turn, exposed as external CPs defined by this external CPD. | OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: Either intVirtualLinkDesc or intCpd shall be present. PassedByReference |
| virtualNetworkInterfaceRequirements | VirtualNetworkInterfaceRequirements | 0..* | Specifies requirements on a virtual network interface realising the CPs instantiated from this CPD. NOTE: In case of referencing an intCpd via its identifier, the virtualNetworkInterfaceRequirements attribute of the referenced intCpd applies. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| addressData | AddressData | 0..* | Provides information on the addresses to be assigned to the CP(s) instantiated from this CPD. | Obsolete OpenModelAttribute - isInvariant: false - support: MANDATORY |
| cpProtocol | CpProtocolData | 1..* | Identifies the protocol layering information the CP uses for connectivity purposes and associated information. There shall be one cpProtocol for each layer protocol as indicated by the attribute layerProtocol. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| cpdId | Identifier | 1 | Identifier of this Cpd class. | Inherited from Class Cpd OpenModelAttribute - isInvariant: true - support: MANDATORY |
| layerProtocol | LayerProtocol | 1..* | Identifies a protocol that the connection points corresponding to the CPD support for connectivity purposes (e.g. Ethernet, MPLS, ODU2, IPV4, IPV6, Pseudo-Wire, etc.). NOTE: This information determines, amongst other things, which type of address to assign to the connection point at instantiation time. | Inherited from Class Cpd OpenModelAttribute - isInvariant: false - support: MANDATORY |
| cpdRole | String | 0..1 | Identifies the role of the connection points corresponding to the CPD in the context of the traffic flow patterns in the VNF, PNF or NS. For example an NS with a tree flow pattern within the NS will have legal cpRoles of ROOT and LEAF. | Inherited from Class Cpd OpenModelAttribute - isInvariant: false - support: MANDATORY |
| description | String | 0..1 | Provides human-readable information on the purpose of the connection point (e.g. connection point for control plane traffic). | Inherited from Class Cpd OpenModelAttribute - isInvariant: false - support: MANDATORY |
| trunkMode | Boolean | 0..1 | Information about whether the Cp instantiated from this CPD is in Trunk mode (802.1Q or other). When operating in "trunk mode", the Cp is capable of carrying traffic for several VLANs. A cardinality of 0 implies that trunkMode is not configured for the Cp i.e.it is equivalent to Boolean value "false". | Inherited from Class Cpd Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfIndicator class

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VnfIndicator

Description:

The VnfIndicator class defines the indicator the VNF supports.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VnfIndicator

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| id | Identifier | 1 | Unique identifier. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| name | String | 0..1 | The human readable name of the VnfIndicator. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| indicatorValue | String | 1..* | Defines the allowed values or value ranges of this indicator. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| source | VnfIndicatorSource | 1 | Describe the source of the indicator. The possible values are: • VNF, • EM, • Both. This tells the consumer where to send the subscription request. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfVirtualLinkDesc class

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VnfVirtualLinkDesc

Description:

Represents the type of network connectivity mandated by the VNF vendor between two or more Connection Points which includes at least one Internal Connection Point.

Parent class: VirtualLinkDesc

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VnfVirtualLinkDesc

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| monitoringParameter | MonitoringParameter | 0..* | Specifies the virtualised resource related performance metrics on VLD level to be tracked by the VNFM. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualLinkDescFlavour | VirtualLinkDescFlavour | 1..* | Describes a specific flavour of the VL with specific bitrate requirements. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualLinkDescId | Identifier | 1 | Uniquely identifies a VLD in the parent descriptor. For VnfVirtualLinkDesc, the parent descriptor is the VNFD. For NsVirtualLinkDesc, the parent descriptor is the NSD. Note: the description of this class are different in ETSI GS NFV IFA 011 and ETSI GS NFV IFA 014. The present definition merges the 2 definitions. | Inherited from Class VirtualLinkDesc OpenModelAttribute - isInvariant: true - support: MANDATORY |
| connectivityType | ConnectivityType | 1 | Specifies the protocol exposed by a VL and the flow pattern supported by the VL. | Inherited from Class VirtualLinkDesc OpenModelAttribute - isInvariant: false - support: MANDATORY |
| testAccess | String | 0..* | Specifies test access facilities expected on the VL (e.g. none, passive monitoring, or active (intrusive) loopbacks at endpoints. | Inherited from Class VirtualLinkDesc OpenModelAttribute - isInvariant: false - support: MANDATORY |
| description | String | 0..1 | Provides human-readable information on the purpose of the VL (e.g. control plane traffic). | Inherited from Class VirtualLinkDesc OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### Vnfd class

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::Vnfd

Description:

A VNFD is a deployment template which describes a VNF in terms of deployment and operational behaviour requirements. It also contains connectivity, interface and virtualised resource requirements.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class Vnfd

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vnfdId | Identifier | 1 | Identifier of this VNFD class. This attribute shall be globally unique. The format will be defined in the data model specification phase. NOTE: The VNFD Identifier shall be used as the unique identifier of the VNF Package that contains this VNFD. Any modification of the content of the VNFD or the VNF Package shall result in a new VNFD Identifier. | OpenModelAttribute - isInvariant: true - support: MANDATORY |
| vnfProvider | String | 1 | Provider of the VNF and of the VNFD. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfdProductName | String | 1 | Name to identify the VNF Product. Invariant for the VNF Product lifetime. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfSoftwareVersion | Version | 1 | Software version of the VNF. This is changed when there is any change to the software that is included in the VNF Package. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfdVersion | Version | 1 | Identifies the version of the VNFD. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfProductInfoName | String | 0..1 | Human readable name for the VNF Product. Can change during the VNF Product lifetime. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfProductInfoDescription | String | 0..1 | Human readable description of the VNF Product. Can change during the VNF Product lifetime. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfmInfo | String | 1..* | Identifies VNFM(s) compatible with the VNF described in this version of the VNFD. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| localizationLanguage |  | 0..* | Information about localization languages of the VNF (includes e.g. strings in the VNFD). NOTE: This allows to provide one or more localization languages to support selecting a specific localization language at VNF instantiation time. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| defaultLocalizationLanguage |  | 0..1 | Default localization language that is instantiated if no information about selected localization language is available. | OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: Shall be present if "localizationLanguage" is present and shall be absent otherwise |
| vdu | Vdu | 1..* | Virtualisation Deployment Unit. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualComputeDesc | VirtualComputeDesc | 1..* | Defines descriptors of virtual compute resources to be used by the VNF. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualStorageDesc | VirtualStorageDesc | 0..* | Defines descriptors of virtual storage resources to be used by the VNF. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| swImageDesc | SwImageDesc | 0..* | Defines descriptors of software images to be used by the VNF. NOTE: This shall be used to describe both the software image loaded on the virtualization container used to realize a VDU and the software images to be stored on VirtualStorage resources (e.g., volumes) attached to a virtualization container. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| intVirtualLinkDesc | VnfVirtualLinkDesc | 0..* | Represents the type of network connectivity mandated by the VNF provider between two or more CPs which includes at least one internal CP. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfExtCpd | VnfExtCpd | 1..* | Describes an external interface exposed by this VNF enabling connection with a VL. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| deploymentFlavour | VnfDf | 1..* | Describes specific DF(s) of a VNF with specific requirements for capacity and performance. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| configurableProperties | VnfConfigurableProperties | 0..1 | Describes the configurable properties of the VNF (e.g. related to auto scaling and auto healing). | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| modifiableAttributes | VnfInfoModifiableAttributes | 1 | Describes the modifiable attributes of the VNF. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| lifeCycleManagementScript | VnfLifeCycleManagementScript | 0..* | Includes a list of events and corresponding management scripts performed for the VNF. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| elementGroup | VnfdElementGroup | 0..* | Describes the associated elements of a VNFD for a certain purpose during VNF lifecycle management. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfIndicator | VnfIndicator | 0..* | Declares the VNF indicators that are supported by this VNF. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| autoScale | Rule | 0..* | Rule that determines when a scaling action needs to be triggered on a VNF instance e.g. based on certain VNF indicator values or VNF indicator value changes or a combination of VNF indicator value(s) and monitoring parameter(s). NOTE 2: Monitoring parameters are specified as part of VNF flavour, VDU and VL descriptions. NOTE 3: The rule (conditions and actions) can be expressed as a script. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfdElementGroup class

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VnfdElementGroup

Description:

A VNFD Element Group is a mechanism for associating elements of a VNFD (Vdus and VnfVirtualLinkDesc(s)) for a certain purpose, for example, scaling aspects.

A given element can belong to multiple groups.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VnfdElementGroup

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vnfdElementGroupId | Identifier | 1 | Unique identifier of this group in the VNFD. | OpenModelAttribute - isInvariant: true - support: MANDATORY |
| description | String | 1 | Human readable description of the group. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vdu | Vdu | 0..* | References to Vdus that are part of this group. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| virtualLinkDesc | VnfVirtualLinkDesc | 0..* | References to VnfVirtualLinkDesc that are part of this group. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |

##### Datatypes

###### AspectDeltaDetails datatype

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::AspectDeltaDetails

Description:

The AspectDeltaDetails information element defines the increments in terms of number of instances of VNFCs and virtual link flavours that correspond to the scaling steps of a scaling aspect.

Applied Stereotypes:

Experimental

Table B.1: Attributes for Data Type AspectDeltaDetails

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| uniformDelta | ScalingDelta | 0..1 | Scaling delta to be applied for every scaling step of this aspect. This attribute allows to model uniform scaling steps. | OpenModelAttribute - isInvariant: false - support: CONDITIONAL_MANDATORY - condition: Either "uniformDelta" or "deltas" but not both shall be present. |
| deltas | ScalingDelta | 0..* | Scaling deltas to be applied for the different subsequent scaling steps of this aspect. The first entry in the array shall correspond to the first scaling step (between scale levels 0 to 1) and the last entry of the array shall correspond to the last scaling step (between maxScaleLevel-1 and maxScaleLevel). | OpenModelAttribute - isInvariant: false - support: CONDITIONAL_MANDATORY - condition: Either "uniformDelta" or "deltas" but not both shall be present. |

###### ChangeExtVnfConnectivityOpConfig  datatype

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::ChangeExtVnfConnectivityOpConfig

Description:

This information element defines attributes that affect the invocation of the ChangeExtVnfConnectivity operation.

Applied Stereotypes:

Experimental

Table B.1: Attributes for Data Type ChangeExtVnfConnectivityOpConfig

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| parameter |  | 0..* | Array of KVP requirements for VNF-specific parameters to be passed when invoking the ChangeExtVnfConnectivity operation. Note: It is assumed that the KVP requirements will be implicitly used to define the value type. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### ChangeVnfFlavourOpConfig datatype

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::ChangeVnfFlavourOpConfig

Description:

This information element defines attributes that affect the invocation of the ChangeVnfFlavour operation.

Applied Stereotypes:

Experimental

Table B.1: Attributes for Data Type ChangeVnfFlavourOpConfig

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| parameter |  | 0..* | Array of KVP requirements for VNF-specific parameters to be passed when invoking the ChangeVnfFlavour operation. Note: It is assumed that the KVP requirements will be implicitly used to define the value type | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### ExternalStimulusEventType enumeration

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::ExternalStimulusEventType

Description:

List of external stimulus events

Applied Stereotypes:

Experimental

Contains Enumeration Literals:

RECEIPT_OF_REQUEST_MESSAGE_OF_INSTANTIATION

RECEIPT_OF_REQUEST_MESSAGE_OF_SCALING

RECEIPT_OF_REQUEST_MESSAGE_OF_HEALING

RECEIPT_OF_REQUEST_MESSAGE_OF_TERMINATION

RECEIPT_OF_REQUEST_MESSAGE_OF_CHANGE _VNF_FLAVOUR

RECEIPT_OF_REQUEST_MESSAGE_OF_OPERATE_VNF

RECEIPT_OF_REQUEST_MESSAGE_OF_CHANGE _VNF_EXT_CONN

RECEIPT_OF_REQUEST_MESSAGE_OF_VNFINFO_MODIFICATION

RECEIPT_OF_VNF_INDICATOR_VALUE_CHANGE_NOTIFICATION

###### HealVnfOpConfig datatype

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::HealVnfOpConfig

Description:

This datatype defines attributes that affect the invocation of the HealVnf operation.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type HealVnfOpConfig

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| parameter |  | 0..* | Array of KVP requirements for VNF-specific parameters to be passed when invoking the HealVnf operation. Note: It is assumed that the key-value pairKVP requirements will be implicitly used to define the value type. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| cause | String | 0..* | Supported "cause" parameter values. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### InstantiateVnfOpConfig datatype

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::InstantiateVnfOpConfig

Description:

This datatype defines attributes that affect the invocation of the InstantiateVnf operation.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type InstantiateVnfOpConfig

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| parameter |  | 0..* | Array of KVP requirements for VNF-specific parameters to be passed when invoking the InstantiateVnf operation. Note: It is assumed that the KVP requirements will be implicitly used to define the value type. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### LcmScriptEventType enumeration

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::LcmScriptEventType

Description:

Common parent of VnfLcmEvent and externalStimulusEvent

Applied Stereotypes:

Experimental

###### LogicalNodeData datatype

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::LogicalNodeData

Description:

This information element describes compute, memory and I/O requirements that are to be associated with the logical node of infrastructure. The logical node requirements are a sub-component of the VDU level requirements. As an example for illustration purposes, a logical node correlates to the concept of a NUMA cell in libvirt terminology.

Applied Stereotypes:

Experimental

Table B.1: Attributes for Data Type LogicalNodeData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| logicalNodeRequirements |  | 0..* | The logical node-level compute, memory and I/O requirements. An array of key-value pairs that articulate the deployment requirements. This could include the number of CPU cores on this logical node, a memory configuration specific to a logical node (e.g. such as available in the Linux kernel via the libnuma library) or a requirement related to the association of an I/O device with the logical node. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### OperateVnfOpConfig datatype

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::OperateVnfOpConfig

Description:

This datatype defines attributes that affect the invocation of the OperateVnf operation.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type OperateVnfOpConfig

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| minGracefulStopTimeout | Number | 1 | Minimum timeout value for graceful stop of a VNF instance. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| maxRecommendedGracefulStopTimeout | Number | 0..1 | Maximum recommended timeout value that can be needed to gracefully stop a VNF instance of a particular type under certain conditions, such as maximum load condition. This is provided by VNF provider as information for the operator facilitating the selection of optimal timeout value. This value is not used as constraint. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| parameter |  | 0..* | Array of KVP requirements for VNF-specific parameters to be passed when invoking the OperateVnf operation. Note: It is assumed that the KVP requirements will be implicitly used to define the value type. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### RequestedAdditionalCapabilityData datatype

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::RequestedAdditionalCapabilityData

Description:

This datatype describes requested additional capability for a particular VDU. Such a capability may be for acceleration or specific tasks.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type RequestedAdditionalCapabilityData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| requestedAdditionalCapabilityName | String | 1 | Identifies a requested additional capability for the VDU. ETSI GS IFA 002 describes acceleration capabilities. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| supportMandatory | Boolean | 1 | Indicates whether the requested additional capability is mandatory for successful operation. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| minRequestedAdditionalCapabilityVersion | String | 0..1 | Identifies the minimum version of the requested additional capability required. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| preferredRequestedAdditionalCapabilityVersion | String | 0..1 | Identifies the preferred version of the requested additional capability. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| targetPerformanceParameters | KeyValuePair | 1..* | Identifies specific attributes, dependent on the requested additional capability type. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### ScaleVnfOpConfig datatype

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::ScaleVnfOpConfig

Description:

This datatype defines attributes that affect the invocation of the ScaleVnf operation.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type ScaleVnfOpConfig

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| parameter |  | 0..* | Array of KVP requirements for VNF-specific parameters to be passed when invoking the ScaleVnf operation. Note : It is assumed that the KVP requirements will be implicitly used to define the value type. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| scalingByMoreThanOneStepSupported | Boolean | 1 | Signals whether passing a value larger than one in the numberOfSteps parameter is supported by this VNF. Default is FALSE, i.e. "not supported". | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### ScaleVnfToLevelOpConfig datatype

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::ScaleVnfToLevelOpConfig

Description:

This datatype defines attributes that affect the invocation of the ScaleVnfToLevel operation.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type ScaleVnfToLevelOpConfig

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| parameter |  | 0..* | Array of KVP requirements for VNF-specific parameters to be passed when invoking the ScaleVnfToLevel operation. Note: It is assumed that the KVP requirements will be implicitly used to define the value type. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| arbitraryTargetLevelsSupported | Boolean | 1 | Signals whether scaling according to the parameter "scaleInfo" is supported by this VNF | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### ScalingDelta datatype

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::ScalingDelta

Description:

For a particular scaling aspect, the ScalingDelta information element defines the number of instances per VNFC and the bitrate delta per virtual link that corresponds to a single scaling step. When scaling out by one step, this delta is added to the resources of the VNF instance, whereas when scaling in, this delta is removed.

Applied Stereotypes:

Experimental

Table B.1: Attributes for Data Type ScalingDelta

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vduDelta | VduLevel | 0..* | The number of VNFC instances based on particular VDUs to be created or removed. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualLinkBitRateDelta | VirtualLinkBitRateLevel | 0..* | The bitrate to be added or removed to virtual links created from particular virtual link. descriptors. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### TerminateVnfOpConfig datatype

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::TerminateVnfOpConfig

Description:

This datatype defines attributes that affect the invocation of the TerminateVnf operation.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type TerminateVnfOpConfig

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| minGracefulTermination Timeout | Number | 1 | Minimum timeout value for graceful termination of a VNF instance. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| maxRecommendedGracefulTermination timeout | Number | 0..1 | Maximum recommended timeout value that can be needed to gracefully terminate a VNF instance of a particular type under certain conditions, such as maximum load condition. This is provided by VNF provider as information for the operator facilitating the selection of optimal timeout value. This value is not used as constraint. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| parameter |  | 0..* | Array of KVP requirements for VNF-specific parameters to be passed when invoking the TerminateVnf operation. Note: It is assumed that the KVP requirements will be implicitly used to define the value type. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VirtualCpuData datatype

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VirtualCpuData

Description:

The VirtualCpuData information element supports the specification of requirements related to virtual CPU(s) of a virtual compute resource.

Applied Stereotypes:

Experimental

Table B.1: Attributes for Data Type VirtualCpuData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| cpuArchitecture | String | 0..1 | CPU architecture type. Examples are x86, ARM. The cardinality can be 0 during the allocation request, if no particular CPU architecture type is requested | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| numVirtualCpu | Integer | 1 | Number of virtual CPUs | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualCpuClock | Number | 0..1 | Minimum CPU clock rate (e.g. in MHz) available for the virtualised CPU resources. The cardinality can be 0 during the allocation request, if no particular value is requested. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualCpuOversubscriptionPolicy |  | 0..1 | The CPU core oversubscription policy e.g. the relation of virtual CPU cores to physical CPU cores/threads. The cardinality can be 0 during the allocation request, if no particular value is requested. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vduCpuRequirements |  | 0..* | Array of key-value pair requirements on the Compute (CPU) for the VDU. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualCpuPinning | VirtualCpuPinningData | 0..1 | The virtual CPU pinning configuration for the virtualised compute resource | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VirtualCpuPinningData datatype

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VirtualCpuPinningData

Description:

The VirtualCpuPinningData information element supports the specification of requirements related to the virtual CPU pinning configuration of a virtual compute resource.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type VirtualCpuPinningData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| virtualCpuPinningPolicy | CpuPinningPolicy | 0..1 | The policy can take values of "static" or "dynamic". In case of "static", the virtual CPU cores are requested to be allocated to logical CPU cores according to the rules defined in virtualCpuPinningRules. In case of "dynamic", the allocation of virtual CPU cores to logical CPU cores is decided by the VIM, (e.g.: SMT (Simultaneous Multi-Threading) requirements). | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| cpuPinningMap |  | 0..1 | If cpuPinningPolicy is defined as "static", the cpuPinninMap provides the map of pinning virtual CPU cores to physical CPU cores/threads. Cardinality can be 0 if cpuPinningPolicy has a different value. | Obsolete OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualCpuPinningRules |  | 0..1 | A list of rules that should be considered during the allocation of the virtual CPUs to logical CPUs in case of "static" virtualCpuPinningPolicy. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VirtualLinkBitRateLevel datatype

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VirtualLinkBitRateLevel

Description:

The VirtualLinkBitRateLevel information element specifies bitrate requirements applicable to a virtual link instantiated from a particular VnfVirtualLinkDesc.

Applied Stereotypes:

Experimental

Table B.1: Attributes for Data Type VirtualLinkBitRateLevel

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vnfVirtualLinkDescId | VnfVirtualLinkDesc | 1 | Uniquely identifies a VnfVirtualLinkDesc | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| bitrateRequirements | LinkBitrateRequirements | 1 | Bitrate requirements for an instantiation level or bitrate delta for a scaling step. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VirtualMemoryData datatype

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VirtualMemoryData

Description:

The VirtualMemoryData information element supports the specification of requirements related to virtual memory of a virtual compute resource.

Applied Stereotypes:

Experimental

Table B.1: Attributes for Data Type VirtualMemoryData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| virtualMemSize | Number | 1 | Amount of virtual Memory (e.g., in MB) | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualMemOversubscriptionPolicy |  | 0..1 | The memory core oversubscription policy in terms of virtual memory to physical memory on the platform. The cardinality can be 0 during the allocation request, if no particular value is requested. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vduMemRequirements |  | 0..* | Array of key-value pair requirements on the memory for the VDU | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| numaEnabled | Boolean | 0..1 | It specifies the memory allocation to be cognisant of the relevant process/core allocation. The cardinality can be 0 during the allocation request, if no particular value is requested. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfConfigurableProperties datatype

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VnfConfigurableProperties

Description:

This datatype defines the configurable properties of a VNF (e.g. related to auto scaling and auto healing). For a VNF instance, the value of these properties can be modified by the VNFM.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type VnfConfigurableProperties

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| IsAutoscaleEnabled | Boolean | 0..1 | It permits to enable (TRUE) / disable (FALSE) the auto-scaling functionality. NOTE: A cardinality of "0" indicates that configuring this present VNF property is not supported. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| isAutoHealEnabled | Boolean | 0..1 | It permits to enable (TRUE) / disable (FALSE) the auto-healing functionality. NOTE: A cardinality of "0" indicates that configuring this present VNF property is not supported. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| additionalConfigurableProperty |  | 0..* | It provides VNF specific configurable properties that can be modified using the ModifyVnfInfo operation. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfInfoModifiableAttributes datatype

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VnfInfoModifiableAttributes

Description:

This datatype defines the VNF-specific extension and metadata attributes of the VnfInfo that are writeable via the ModifyVnfInfo operation.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type VnfInfoModifiableAttributes

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| extension |  | 0..* | "Extension" attributes of VnfInfo that are writeable. NOTE: The exact data structure describing the attribute is left for data model solution specification, but it should include: name, and any constraints on the values, such as ranges, predefined values, etc. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| metadata |  | 0..* | "Metadata" attributes of VnfInfo that are writeable. NOTE: The exact data structure describing the attribute is left for data model solution specification, but it should include: name, and any constraints on the values, such as ranges, predefined values, etc. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfLcmEventType enumeration

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VnfLcmEventType

Description:

Set of lifecycle events triggered internally by the VNFM.

Applied Stereotypes:

Experimental

Contains Enumeration Literals:

EVENT_START_INSTANTIATION

EVENT_END_INSTANTIATION

EVENT_START_SCALING_IN

EVENT_END_SCALING_IN

EVENT_START_SCALING_OUT

EVENT_END_SCALING_OUT

EVENT_START_HEALING

EVENT_END_HEALING

EVENT_START_TERMINATION

EVENT_END_TERMINATION

EVENT_START_VNF_FLAVOR_CHANGE

EVENT_END_VNF_FLAVOR_CHANGE

EVENT_START_VNF_OPERATION_CHANGE

EVENT_END_VNF_OPERATION_CHANGE

EVENT_START_VNF_EXT_CONN_CHANGE

EVENT_END_VNF_EXT_CONN_CHANGE

EVENT_START_VNFINFO_MODIFICATION

EVENT_END_VNFINFO_MODIFICATION

###### VnfLcmOperationsConfiguration datatype

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VnfLcmOperationsConfiguration

Description:

This datatype is a container for all attributes that affect the invocation of the VNF Lifecycle Management operations, structured by operation.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type VnfLcmOperationsConfiguration

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| instantiateVnfOpConfig | InstantiateVnfOpConfig | 0..1 | Configuration parameters for the InstantiateVnf operation. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| scaleVnfOpConfig | ScaleVnfOpConfig | 0..1 | Configuration parameters for the ScaleVnf operation. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| scaleVnfToLevelOpConfig | ScaleVnfToLevelOpConfig | 0..1 | Configuration parameters for the ScaleVnfToLevel operation. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| changeVnfFlavourOpConfig | ChangeVnfFlavourOpConfig | 0..1 | Configuration parameters for the ChangeVnfFlavour operation. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| healVnfOpConfig | HealVnfOpConfig | 0..1 | Configuration parameters for the HealVnf operation. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| terminateVnfOpConfig | TerminateVnfOpConfig | 0..1 | Configuration parameters for the TerminateVnf operation. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| operateVnfOpConfig | OperateVnfOpConfig | 0..1 | Configuration parameters for the OperateVnf operation. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| changeExtVnfConnectivityOpConfig | ChangeExtVnfConnectivityOpConfig | 0..1 | Configuration parameters for the ChangeExtVnfConnectivity operation. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfLifeCycleManagementScript datatype

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VnfLifeCycleManagementScript

Description:

Defines the information elements related to the lifecycle management script for the VNF.

Applied Stereotypes:

Experimental

Table B.1: Attributes for Data Type VnfLifeCycleManagementScript

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| event | LcmScriptEventType | 0..* | Describes VNF lifecycle event(s) or an external stimulus detected on a VNFM reference point. The set of lifecycle events triggered internally by the VNFM includes: EVENT_START_INSTANTIATION, EVENT_END_INSTANTIATION, EVENT_START_SCALING_IN, EVENT_END_SCALING_IN, EVENT_START_SCALING_OUT, EVENT_END_SCALING_OUT,EVENT_START_HEALING, EVENT_END_HEALING, EVENT_START_TERMINATION, EVENT_END_TERMINATION, EVENT_START_VNF_FLAVOR_CHANGE, EVENT_END_VNF_FLAVOR_CHANGE, EVENT_START_VNF_OPERATION_CHANGE, EVENT_END_VNF_OPERATION_CHANGE The set of external stimulus includes: the receipt of request message of instantiation, scaling, healing, termination, change of VNF flavour, change of the operation state of the VNF or the receipt of a notification regarding the change of a VNF indicator value | Experimental OpenModelAttribute - isInvariant: false - support: CONDITIONAL_MANDATORY - condition: At least one of the 2 attributes event or lcmTransitionEvent shall be included. |
| lcmTransitionEvent | String | 0..* | Describes the transition VNF lifecycle event(s) that cannot be mapped to any of the enumerated values defined for the event attribute. | Experimental OpenModelAttribute - isInvariant: false - support: CONDITIONAL_MANDATORY - condition: At least one of the 2 attributes event or lcmTransitionEvent shall be included. |
| script |  | 1 | Includes a VNF LCM script (e.g., written in a DSL as specified in requirement VNF_PACK.LCM.001) triggered to react to one of the events listed in the event attribute. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| scriptDsl | String | 1 | Defines the domain specific language (i.e. the type) of script that is provided. Types of scripts could include bash, python, etc. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| scriptInput |  | 0..* | Array of KVP requirements with the key as the parameter name and the value as the parameter that need to be passed as an input to the script. NOTE : The scriptInput values are passed to the scripts in addition to the parameters received in the operation invocation request or indicator value change. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfQoS datatype

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VnfQoS

Applied Stereotypes:

Preliminary

Table A.1: Attributes for Data Type VnfQoS

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| latency | Number | 1 | Specifies the maximum latency in ms. | Inherited from Data Type QoS OpenModelAttribute - isInvariant: false - support: MANDATORY |
| packetDelayVariation | Number | 1 | Specifies the maximum jitter in ms. | Inherited from Data Type QoS OpenModelAttribute - isInvariant: false - support: MANDATORY |
| packetLossRatio | Number | 0..1 | Specifies the maximum packet loss ratio. | Inherited from Data Type QoS OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfcConfigurableProperties datatype

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VnfcConfigurableProperties

Description:

This datatype defines the configurable properties of a VNFC. For a VNFC instance, the value of these properties can be modified through the VNFM.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type VnfcConfigurableProperties

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| additionalVnfcConfigurableProperty |  | 0..* | It provides VNFC configurable properties that can be modified using the ModifyVnfInfo operation. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

##### Associations

###### InstantiationLevelHasVduLevel association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::InstantiationLevelHasVduLevel

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association InstantiationLevelHasVduLevel

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| vduLevel | composite | Navig. | 1..* | VduLevel | Indicates the number of instance of this VDU to deploy for this level. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| instantiationlevel | none | Not navig. | 1 | InstantiationLevel |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### SwImageDescReferencesSwImage association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::SwImageDescReferencesSwImage

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association SwImageDescReferencesSwImage

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| swImage | shared | Navig. | 1 | SwImage | This is a reference to the actual software image. The reference can be relative to the root of the VNF Package or can be a URL | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| swImageDesc | none | Not navig. | 0..* | SwImageDesc |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VduCpdHasVirtualNetworkInterfaceRequirements association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VduCpdHasVirtualNetworkInterfaceRequirements

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VduCpdHasVirtualNetworkInterfaceRequirements

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| virtualNetworkInterfaceRequirements | composite | Navig. | 0..* | VirtualNetworkInterfaceRequirements | Specifies requirements on a virtual network interface realising the CPs instantiated from this CPD. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vduCpd | none | Not navig. | 1 | VduCpd |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VduCpdReferencesVirtualLinkDesc association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VduCpdReferencesVirtualLinkDesc

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VduCpdReferencesVirtualLinkDesc

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| intVirtualLinkDesc | none | Navig. | 0..1 | VnfVirtualLinkDesc | Reference(s) of the Virtual Link Descriptor which this Connection Point Descriptor(s) connects to. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| cpdId | none | Not navig. | 0..1 | VduCpd | Reference(s) of the Connection Point Descriptor(s) which this Virtual Link Descriptor connects to. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VduHasCpd association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VduHasCpd

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VduHasCpd

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| intCpd | shared | Navig. | 1..* | VduCpd | Describes network connectivity between a VNFC instance (based on this Vdu) and an internal Virtual Link (VL). | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vdu | none | Not navig. | 1..* | Vdu |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VduHasSwimageDesc association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VduHasSwimageDesc

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VduHasSwimageDesc

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| swImageDesc | shared | Navig. | 0..1 | SwImageDesc | Describes the software image which is directly loaded on the virtualisation container realizing this Vdu. NOTE: More software images can be attached to the virtualisation container using VirtualStorage resources. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vdu | none | Not navig. | 1..* | Vdu |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VduHasVirtualComputeDesc association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VduHasVirtualComputeDesc

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VduHasVirtualComputeDesc

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| virtualComputeDesc | shared | Navig. | 1 | VirtualComputeDesc | Describes CPU, Memory and acceleration requirements of the Virtualisation Container realizing this Vdu. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vdu | none | Not navig. | 1..* | Vdu |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VduHasVirtualStorageDesc association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VduHasVirtualStorageDesc

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VduHasVirtualStorageDesc

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| virtualStorageDesc | shared | Navig. | 0..* | VirtualStorageDesc | Describes storage requirements for a VirtualStorage instance attached to the virtualisation container created from virtualComputeDesc defined for this Vdu. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vdu | none | Not navig. | 0..* | Vdu |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VduLevelReferencesVdu association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VduLevelReferencesVdu

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VduLevelReferencesVdu

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| vduId | shared | Navig. | 1 | Vdu | Uniquely identifies a VDU. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vdulevel | none | Not navig. | 0..* | VduLevel |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VduProfileReferencesVdu association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VduProfileReferencesVdu

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VduProfileReferencesVdu

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| vduId | shared | Navig. | 1 | Vdu | Uniquely identifies a VDU. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vduprofile | none | Not navig. | 0..* | VduProfile |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VirtualComputeDescDescribesRequirementsOnVirtualComputeFlavour association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VirtualComputeDescDescribesRequirementsOnVirtualComputeFlavour

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VirtualComputeDescDescribesRequirementsOnVirtualComputeFlavour

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| virtualComputeFlavour | none | Not navig. | 0..* | VirtualComputeFlavour |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualComputeDesc | none | Navig. | 1 | VirtualComputeDesc |  | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |

###### VirtualLinkProfileReferencesVirtualLinkDescFlavour association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VirtualLinkProfileReferencesVirtualLinkDescFlavour

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VirtualLinkProfileReferencesVirtualLinkDescFlavour

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| flavourId | shared | Navig. | 1 | VirtualLinkDescFlavour | Identifies a flavour within the VnfVirtualLinkDesc. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vlprofile | none | Not navig. | 1 | VirtualLinkProfile |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VirtualLinkProfileReferencesVnfVirtualLinkDesc association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VirtualLinkProfileReferencesVnfVirtualLinkDesc

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VirtualLinkProfileReferencesVnfVirtualLinkDesc

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| vnfVirtualLinkDescId | shared | Navig. | 1 | VnfVirtualLinkDesc | Uniquely identifies a Vnf VLD. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vlprofile | none | Not navig. | 1 | VirtualLinkProfile |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VirtualStorageDescDescribesRequirementsOnVirtualStorageFlavour association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VirtualStorageDescDescribesRequirementsOnVirtualStorageFlavour

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VirtualStorageDescDescribesRequirementsOnVirtualStorageFlavour

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| virtualStorageFlavour | none | Not navig. | 0..* | VirtualStorageFlavour |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualStorageDesc | none | Navig. | 1 | VirtualStorageDesc |  | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |

###### VirtualStorageDescReferencesSwImageDesc association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VirtualStorageDescReferencesSwImageDesc

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VirtualStorageDescReferencesSwImageDesc

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| swImageDesc | shared | Navig. | 0..1 | SwImageDesc | Software image to be loaded on the VirtualStorage resource created based on this VirtualStorageDesc. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| virtualStorageDesc | none | Not navig. | 0..* | VirtualStorageDesc |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfDeployedUsingVnfd association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VnfDeployedUsingVnfd

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfDeployedUsingVnfd

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| vnfd | none | Navig. | 1 | Vnfd | Identifier of the VNFD on which the VNF instance is based. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vnf | none | Not navig. | 0..* | Vnf |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfDeploymentFlavorHasInstantiationLevel association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VnfDeploymentFlavorHasInstantiationLevel

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfDeploymentFlavorHasInstantiationLevel

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| instantiationLevel | shared | Navig. | 1..* | InstantiationLevel | Describes the various levels of resources that can be used to instantiate the VNF using this flavour. Examples: Small, Medium, Large. If there is only one "instantiationLevel" entry, it shall be treated as the default instantiation level for this DF. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfDeploymentFlavour | none | Not navig. | 1 | VnfDf |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfDeploymentFlavourHasAffinityOrAntiAffinityGroup association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VnfDeploymentFlavourHasAffinityOrAntiAffinityGroup

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfDeploymentFlavourHasAffinityOrAntiAffinityGroup

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| affinityOrAntiAffinityGroup | shared | Navig. | 0..* | AffinityOrAntiAffinityGroup | Specifies affinity or anti-affinity relationship applicable between the virtualisation containers (e.g. virtual machines) to be created using different VDUs or internal VLs to be created using different VnfVirtualLinkDesc(s) in the same affinity or anti-affinity group. NOTE: In the present specification, including either VDU(s) or VnfVirtualLinkDesc(s) into the same affinity or anti-affinity group is supported. Extension to support including both VDU(s) and VnfVirtualLinkDesc(s) into the same affinity or anti-affinity group is left for future specification. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfdeploymentflavour | none | Not navig. | 1..* | VnfDf |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfDeploymentFlavourHasScalingAspect association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VnfDeploymentFlavourHasScalingAspect

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfDeploymentFlavourHasScalingAspect

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| scalingAspect | shared | Navig. | 0..* | ScalingAspect | The scaling aspects supported by this DF of the VNF. scalingAspect shall be present if the VNF supports scaling. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfDeploymentFlavour | none | Not navig. | 1..* | VnfDf |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfDeploymentFlavourHasVduProfile association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VnfDeploymentFlavourHasVduProfile

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfDeploymentFlavourHasVduProfile

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| vduProfile | shared | Navig. | 1..* | VduProfile | Describes additional instantiation data for the VDUs used in this flavour. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfdeploymentflavour | none | Not navig. | 1..* | VnfDf |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfDeploymentFlavourHasVirtualLinkProfile association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VnfDeploymentFlavourHasVirtualLinkProfile

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfDeploymentFlavourHasVirtualLinkProfile

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| virtualLinkProfile | shared | Navig. | 0..* | VirtualLinkProfile | Defines the internal VLD along with additional data which is used in this DF. NOTE 1: This allows for different VNF internal topologies between DFs. NOTE 2: virtualLinkProfile needs to be provided for all VLs that the CPs of the VDUs in the VDU profiles connect to. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfdeploymentflavour | none | Not navig. | 1..* | VnfDf |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfExtCpDeployedUsingVnfExtCpd association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VnfExtCpDeployedUsingVnfExtCpd

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfExtCpDeployedUsingVnfExtCpd

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| vnfExtCpd | none | Navig. | 1 | VnfExtCpd | VnfExtCpd applicable to this VnfExtCp. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vnfExtCp | none | Not navig. | 0..* | VnfExtCp |  | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |

###### VnfExtCpdMapsToVduCpd association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VnfExtCpdMapsToVduCpd

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfExtCpdMapsToVduCpd

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| intCpd | none | Navig. | 0..1 | VduCpd | Reference to the internal VDU CPD which is used to instantiate internal CPs. These internal CPs are, in turn, exposed as external CPs defined by this external CPD. | OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: Either intVirtualLinkDesc or intCpd shall be present. PassedByReference |
| vnfExternalCpd | none | Not navig. | 0..1 | VnfExtCpd |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfExtCpdMapsToVirtualLinkDesc association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VnfExtCpdMapsToVirtualLinkDesc

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfExtCpdMapsToVirtualLinkDesc

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| intVirtualLinkDesc | none | Navig. | 0..1 | VnfVirtualLinkDesc | Reference to the internal Virtual Link Descriptor (VLD) to which CPs instantiated from this external CP Descriptor (CPD) connect. | OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: Either intVirtualLinkDesc or intCpd shall be present. PassedByReference |
| vnfExternalCpd | none | Not navig. | 0..1 | VnfExtCpd |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfVirtualLinkDeployedUsingVnfVirtualLinkDesc association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VnfVirtualLinkDeployedUsingVnfVirtualLinkDesc

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfVirtualLinkDeployedUsingVnfVirtualLinkDesc

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| vnfVirtualLinkDesc | none | Navig. | 1 | VnfVirtualLinkDesc | VnfVirtualLinkDesc applicable to this VL. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vnfVl | none | Not navig. | 0..* | VnfVirtualLink |  | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |

###### VnfcCpDeployedUsingVduCpd association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VnfcCpDeployedUsingVduCpd

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfcCpDeployedUsingVduCpd

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| vduCpd | none | Navig. | 1 | VduCpd | Cpd applicable to this CP. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vnfcCp | none | Not navig. | 0..* | VnfcCp |  | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |

###### VnfcDeployedUsingVdu association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VnfcDeployedUsingVdu

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfcDeployedUsingVdu

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| vdu | none | Navig. | 1 | Vdu | Applicable Vdu class in the VNFD. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vnfc | none | Not navig. | 0..* | Vnfc |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfdElementGroupReferencesVdu association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VnfdElementGroupReferencesVdu

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfdElementGroupReferencesVdu

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| vdu | shared | Navig. | 0..* | Vdu | References to Vdus that are part of this group. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vnfdelementgroup | none | Not navig. | 0..* | VnfdElementGroup |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfdElementGroupReferencesVnfVirtualLinkDesc association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VnfdElementGroupReferencesVnfVirtualLinkDesc

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfdElementGroupReferencesVnfVirtualLinkDesc

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| virtualLinkDesc | shared | Navig. | 0..* | VnfVirtualLinkDesc | References to VnfVirtualLinkDesc that are part of this group. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vnfdElementGroup | none | Not navig. | 0..* | VnfdElementGroup |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfdHasDeploymentFlavour association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VnfdHasDeploymentFlavour

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfdHasDeploymentFlavour

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| deploymentFlavour | composite | Navig. | 1..* | VnfDf | Describes specific DF(s) of a VNF with specific requirements for capacity and performance. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfd | none | Not navig. | 1 | Vnfd |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfdHasSwImageDesc association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VnfdHasSwImageDesc

Applied Stereotypes:

Experimental

Table B.1: Member ends for Association VnfdHasSwImageDesc

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| swImageDesc | composite | Navig. | 0..* | SwImageDesc | Defines descriptors of software images to be used by the VNF. NOTE: This shall be used to describe both the software image loaded on the virtualization container used to realize a VDU and the software images to be stored on VirtualStorage resources (e.g., volumes) attached to a virtualization container. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfd | none | Not navig. | 1 | Vnfd |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfdHasVdu association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VnfdHasVdu

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfdHasVdu

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| vdu | composite | Navig. | 1..* | Vdu | Virtualisation Deployment Unit. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfd | none | Not navig. | 1 | Vnfd |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfdHasVirtualComputeDesc association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VnfdHasVirtualComputeDesc

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfdHasVirtualComputeDesc

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| virtualComputeDesc | composite | Navig. | 1..* | VirtualComputeDesc | Defines descriptors of virtual compute resources to be used by the VNF. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfd | none | Not navig. | 1 | Vnfd |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfdHasVirtualLinkDesc association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VnfdHasVirtualLinkDesc

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfdHasVirtualLinkDesc

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| intVirtualLinkDesc | composite | Navig. | 0..* | VnfVirtualLinkDesc | Represents the type of network connectivity mandated by the VNF provider between two or more CPs which includes at least one internal CP. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfd | none | Not navig. | 1 | Vnfd |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfdHasVirtualStorageDesc association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VnfdHasVirtualStorageDesc

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfdHasVirtualStorageDesc

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| virtualStorageDesc | composite | Navig. | 0..* | VirtualStorageDesc | Defines descriptors of virtual storage resources to be used by the VNF. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfd | none | Not navig. | 1 | Vnfd |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfdHasVnfExtCpd association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VnfdHasVnfExtCpd

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfdHasVnfExtCpd

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| vnfExtCpd | composite | Navig. | 1..* | VnfExtCpd | Describes an external interface exposed by this VNF enabling connection with a VL. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfd | none | Not navig. | 1 | Vnfd |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfdHasVnfIndicator association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VnfdHasVnfIndicator

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfdHasVnfIndicator

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| vnfIndicator | shared | Navig. | 0..* | VnfIndicator | Declares the VNF indicators that are supported by this VNF. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfd | none | Not navig. | 1 | Vnfd |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfdHasVnfdElementGroup association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::VnfdHasVnfdElementGroup

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfdHasVnfdElementGroup

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| elementGroup | shared | Navig. | 0..* | VnfdElementGroup | Describes the associated elements of a VNFD for a certain purpose during VNF lifecycle management. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfd | none | Not navig. | 1 | Vnfd |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### vnfExtCpdHasVirtualNetworkInterfaceRequirements association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::vnfExtCpdHasVirtualNetworkInterfaceRequirements

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association vnfExtCpdHasVirtualNetworkInterfaceRequirements

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| virtualNetworkInterfaceRequirements | composite | Navig. | 0..* | VirtualNetworkInterfaceRequirements | Specifies requirements on a virtual network interface realising the CPs instantiated from this CPD. NOTE: In case of referencing an intCpd via its identifier, the virtualNetworkInterfaceRequirements attribute of the referenced intCpd applies. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfExtCpd | none | Not navig. | 1 | VnfExtCpd |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### vnfVirtualLinkDescHasVirtualLinkDescFlavour association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualNetworkFunctionDomain::VnfTemplateModule::vnfVirtualLinkDescHasVirtualLinkDescFlavour

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association vnfVirtualLinkDescHasVirtualLinkDescFlavour

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| virtualLinkDescFlavour | composite | Navig. | 1..* | VirtualLinkDescFlavour | Describes a specific flavour of the VL with specific bitrate requirements. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfVirtualLinkDesc | none | Not navig. | 1 | VnfVirtualLinkDesc |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

## VirtualisedResourceDomain

### Overview

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualisedResourceDomain

Description:

Group all the core classes related to Virtualised Resources

### NfpModule

#### Overview

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualisedResourceDomain::NfpModule

##### Diagrams

Figure B.1: Diagram NFP Resources

##### Classes

###### Nfp class

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualisedResourceDomain::NfpModule::Nfp

Description:

An ordered list of connection points forming a chain of NFs, along with policies associated to the list.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class Nfp

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| nfpId | Identifier | 1 | Identification of the NFP | OpenModelAttribute - isInvariant: true - support: MANDATORY |
| cp | Cp | 1..* | The identification of the Connection Point and its position in the NFP queried for. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| totalCp | Integer | 1 | Total number of Connection Points in this NFP queried for | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nfpRule | Rule | 0..1 | NFP classification and selection rule(s) | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nfpState | OperationalState | 1 | An indication of whether the NFP is enabled or disabled. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

### VirtualisedResourceModule

#### Overview

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualisedResourceDomain::VirtualisedResourceModule

##### Diagrams

Figure B.1: Diagram VirtualisedResourceCore

Figure B.1: Diagram VirtualCompute

Figure B.1: Diagram VirtualNetwork

Figure B.1: Diagram VirtualStorage

Figure B.1: Diagram VirtualResourceHierarchy

Figure B.1: Diagram VirtualisationContainer

##### Classes

###### NetworkQoS class

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualisedResourceDomain::VirtualisedResourceModule::NetworkQoS

Description:

This type gives QoS options to be supported on the virtualised network, e.g., latency, jitter, etc.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class NetworkQoS

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| qosName | String | 1 | Name given to the QoS parameter. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| qosValue | Value | 1 | Value of the QoS parameter. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NetworkSubnet class

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualisedResourceDomain::VirtualisedResourceModule::NetworkSubnet

Description:

The NetworkSubnet class encapsulates information of an instantiated virtualised sub-network.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class NetworkSubnet

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| resourceId | Identifier | 1 | Identifier of the virtualised sub-network. | OpenModelAttribute - isInvariant: true - support: MANDATORY |
| networkId | VirtualNetwork | 0..1 | The identifier of the virtualised network that the virtualised sub-network is attached to. The cardinality can be 0 to cover the case where this type is used to describe the L3 attributes of a network rather than a subnetwork. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| ipVersion | IpVersion | 1 | The IP version of the network/subnetwork. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| gatewayIp | IpAddress | 1 | The IP address of the network/subnetwork gateway. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| cidr |  | 1 | The CIDR of the network/subnetwork, i.e., network address and subnet mask. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| isDhcpEnabled | Boolean | 1 | True when DHCP is enabled for this network/subnetwork, or false otherwise. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| operationalState | OperationalState | 1 | The operational state of the virtualised sub-network. | Obsolete OpenModelAttribute - isInvariant: false - support: MANDATORY |
| addressPool |  | 0..* | Address pools for the network/subnetwork. The cardinality can be 0 when VIM is allowed to allocate all addresses in the CIDR except for the address of the network/subnetwork gateway | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| metadata | KeyValuePair | 0..* | List of metadata key-value pairs used by the consumer to associate meaningful metadata to the related virtualised resource. | OpenModelAttribute - isInvariant: false - support: OPTIONAL |

###### NfviPop class

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualisedResourceDomain::VirtualisedResourceModule::NfviPop

Description:

The NfviPop class contains basic data to identify an NFVI-PoP in a VIM. It provides geographic location information of the NFVI resources that the VIM manages, as well as other attributes which help consumer functional blocks build topological information relative to NFVI-PoP connectivity to other NFVI-PoP or N-PoP.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class NfviPop

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| nfviPopId | Identifier | 1 | Identification of the NFVI-PoP. | OpenModelAttribute - isInvariant: true - support: MANDATORY |
| vimId | Identifier | 1 | Identification of the VIM. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| geographicalLocationInfo | Location | 1 | It provides information about the geographic location (e.g., geographic coordinates or address of the building, etc) of the NFVI resources that the VIM manages. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| networkConnectivityEndpoint |  | 1 | Information about network connectivity endpoints to the NFVI-PoP that the VIM manages which helps build topology information relative to NFVI-PoP connectivity to other NFVI-PoP or N-PoP. These endpoints enable the entities instantiated in the NFVI-PoP to be reachable by networks outside of the NFVI-PoP. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### ResourceZone class

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualisedResourceDomain::VirtualisedResourceModule::ResourceZone

Description:

The Resource Zone class contains information about the Resource Zone.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class ResourceZone

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| zoneId | Identifier | 1 | The identifier of the Resource Zone. | OpenModelAttribute - isInvariant: true - support: MANDATORY |
| zoneName | String | 1 | The name of the Resource Zone. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| zoneState | String | 1 | Information about the current state of the Resource Zone, e.g., if the Resource Zone is available. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nfviPopId | NfviPop | 1..* | The identifier of the NFVI-PoP the Resource Zone belongs to. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| zoneProperty |  | 1..* | Set of properties that define the capabilities associated to the Resource Zone. Examples of capabilities may include: support of certain compute resource types (e.g., low performance, acceleration capabilities, etc., association to certain NFVI-PoP physical segregation (e.g., different power or network sub-systems, availability of redundancy power sub-systems), etc. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| metadata | KeyValuePair | 0..* | Other metadata associated to the Resource Zone. | OpenModelAttribute - isInvariant: false - support: OPTIONAL |

###### VirtualCompute class

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualisedResourceDomain::VirtualisedResourceModule::VirtualCompute

Description:

The VirtualCompute class encapsulates data of an instantiated virtualised compute resource.

Parent class: VirtualResource

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VirtualCompute

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| computeId | Identifier | 1 | Identifier of the virtualised compute resource | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| computeName | String | 0..1 | Name of the virtualised compute resource | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| flavourId | Identifier | 1 | Identifier of the given compute flavour used to instantiate this virtual compute | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| accelerationCapability |  | 0..* | Selected acceleration capabilities (e.g. crypto, GPU) from the set of capabilities offered by the compute node acceleration resources. The cardinality can be 0, if no particular acceleration capabilitiy is provided. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualCpu | VirtualCpu | 1 | The virtual CPU(s) of the virtualised compute | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualMemory | VirtualMemory | 1 | The virtual memory of the compute | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualNetworkInterface | VirtualNetworkInterface | 0..* | Element with information of the instantiated virtual network interfaces of the compute resource | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualDisks | VirtualStorage | 1..* | Element with information of the virtualised storage resources (volumes, ephemeral that are attached to the compute resource. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vcImageId | Identifier | 0..1 | Identifier of the virtualised container software image (e.g., virtual machine image). Cardinality can be 0 if an "empty" virtualisation container is allocated. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| zoneId | Identifier | 0..1 | If present, it identifies the Resource Zone where the virtual compute resources have been allocated. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| hostId | Identifier | 1 | Identifier of the host the virtualised compute resource is allocated on | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| operationalState | OperationalState | 1 | Operational state of the compute resource | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| metadata | KeyValuePair | 0..* | List of metadata key-value pairs used by the consumer to associate meaningful metadata to the related virtualised resource | OpenModelAttribute - isInvariant: false - support: OPTIONAL |

###### VirtualCpu class

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualisedResourceDomain::VirtualisedResourceModule::VirtualCpu

Description:

The virtual CPU(s) of the virtualised compute.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VirtualCpu

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| cpuArchitecture | String | 1 | CPU architecture type. Examples are x86, ARM. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| numVirtualCpu | Integer | 1 | Number of virtual CPUs | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| cpuClock | Number | 1 | Minimum CPU clock rate (e.g. in MHz) available for the virtualised CPU resources. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualCpuOversubscriptionPolicy |  | 0..1 | The CPU core oversubscription policy e.g. the relation of virtual CPU cores to physical CPU cores/threads. The cardinality can be 0 if no policy has been defined during the allocation request. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualCpuPinning | VirtualCpuPinning | 0..1 | The virtual CPU pinning configuration for the virtualised compute resource | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VirtualMachine class

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualisedResourceDomain::VirtualisedResourceModule::VirtualMachine

Description:

A virtualized computation environment that behaves very much like a physical computer/server. NOTE: A VM has all its ingredients (processor, memory/storage, interfaces/ports) of a physical computer/server and is generated by a Hypervisor, which partitions the underlying physical resources and allocates them to VMs. Virtual Machines are capable of hosting a VNF Component (VNFC).

Parent class: VirtualisationContainer

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VirtualMachine

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| virtualCompute | VirtualCompute | 1 | The Virtual Compute running on this VM. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VirtualMemory class

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualisedResourceDomain::VirtualisedResourceModule::VirtualMemory

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VirtualMemory

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| virtualMemSize | Number | 1 | Amount of virtual Memory (e.g., in MB) | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualMemOversubscriptionPolicy |  | 0..1 | The memory core oversubscription policy in terms of virtual memory to physical memory on the platform. The cardinality can be 0 if no policy has been defined during the allocation request. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| numaEnabled | Boolean | 1 | It specifies the memory allocation to be cognisant of the relevant process/core allocation. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VirtualNetwork class

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualisedResourceDomain::VirtualisedResourceModule::VirtualNetwork

Description:

The Virtual Network class encapsulates information of an instantiated virtualised network resource. In the NFVI, a virtual network transports information among the network interfaces of VM instances and physical network interfaces, providing the necessary connectivity.

Parent class: VirtualResource

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VirtualNetwork

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| networkResourceId | Identifier | 1 | Identifier of the virtualised network resource. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| networkResourceName | String | 0..1 | Name of the virtualised network resource. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| subnet | NetworkSubnet | 0..* | Only present if the network provides layer 3 connectivity. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| networkPort | VirtualNetworkPort | 0..* | Element providing information of an instantiated virtual network port. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| bandwidth | Number | 1 | Minimum network bandwidth (in Mbps). | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| networkType | String | 1 | The type of network that maps to the virtualised network. This list is extensible. Examples are: "local", "vlan", "vxlan", "gre", "l3-vpn", etc. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| segmentType | String | 0..1 | The isolated segment for the virtualised network. For instance, for a "vlan" networkType, it corresponds to the vlan identifier; and for a "gre" networkType, this corresponds to a gre key. The cardinality can be "0" for flat networks without any specific segmentation. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| networkQoS | NetworkQoS | 0..* | Element providing information about Quality of Service attributes that the network supports. Cardinality can be "0" for virtual network without any QoS requirements | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| isShared | Boolean | 1 | It defines whether the virtualised network is shared among consumers. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| sharingCriteria |  | 0..1 | Only present for shared networks. Indicate the sharing criteria for this network. This criteria might be a list of authorised consumers. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| zoneId | Identifier | 0..1 | If present, it identifies the Resource Zone where the virtual network resources have been allocated. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| operationalState | OperationalState | 1 | The operational state of the virtualised network. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| metadata | KeyValuePair | 0..* | List of metadata key-value pairs used by the consumer to associate meaningful metadata to the related virtualised resource. | OpenModelAttribute - isInvariant: false - support: OPTIONAL |

###### VirtualNetworkInterface class

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualisedResourceDomain::VirtualisedResourceModule::VirtualNetworkInterface

Description:

A virtual network interface resource is a communication endpoint under an instantiated compute resource.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VirtualNetworkInterface

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| resourceId | Identifier | 1 | Identifier of the virtual network interface. | OpenModelAttribute - isInvariant: true - support: MANDATORY |
| ownerId | Identifier | 1 | Identifier of the owner of the network interface (e.g., a virtualised compute resource). | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| networkId | VirtualNetwork | 0..1 | In the case when the virtual network interface is attached to the network, it identifies such a network. The cardinality can be 0 in the case that a network interface is created without being attached to any specific network. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| networkPortId | VirtualNetworkPort | 0..1 | If the virtual network interface is attached to a specific network port, it identifies such a network port. The cardinality can be 0 in the case that a network interface is created without any specific network port attachment. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| ipAddress | IpAddress | 0..* | The virtual network interface can be configured with specific IP address(es) associated to the network to be attached to. The cardinality can be 0 in the case that a network interface is created without being attached to any specific network, or when an IP address can be automatically configured, e.g., by DHCP. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| typeVirtualNic |  | 1 | Type of network interface. The type allows for defining how such interface is to be realized, e.g., normal virtual NIC, with direct PCI passthrough, etc. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| typeConfiguration |  | 0..* | Extra configuration that the virtual network interface supports based on the type of virtual network interface, including support for SR-IOV with configuration of virtual functions (VF). | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| macAddress | MacAddress | 1 | The MAC address of the virtual network interface. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| bandwidth | Number | 1 | The bandwidth of the virtual network interface (in Mbps). | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| accelerationCapabilty |  | 0..* | Shows the acceleration capabilities utilized by the virtual network interface. The cardinality can be 0, if no acceleration capability is utilized. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| operationalState | OperationalState | 1 | The operational state of the virtual network interface. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| metadata | KeyValuePair | 0..* | List of metadata key-value pairs used by the consumer to associate meaningful metadata to the related virtualised resource. | OpenModelAttribute - isInvariant: false - support: OPTIONAL |

###### VirtualNetworkPort class

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualisedResourceDomain::VirtualisedResourceModule::VirtualNetworkPort

Description:

The VirtualNetworkPort class encapsulates information of an instantiated virtual network port. A network port resource is a communication endpoint instantiated under a network resource.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VirtualNetworkPort

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| resourceId | Identifier | 1 | Identifier of the virtual network port. | OpenModelAttribute - isInvariant: true - support: MANDATORY |
| networkId | VirtualNetwork | 1 | Identifier of the network that the port belongs to. When creating a port, such port needs to be part of a network. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| portType | String | 1 | Type of network port. Examples of types are access ports (layer 2 or 3), or trunk ports (layer 1) that become transport for multiple layer 2 or layer 3 networks. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| segmentId | Identifier | 0..1 | The isolated segment the network port belongs to. For instance, for a "vlan", it corresponds to the vlan identifier; and for a "gre", this corresponds to a gre key. The cardinality can be "0" for flat networks without any specific segmentation. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| bandwidth | Number | 0..1 | The bandwidth of the virtual network port (in Mbps). Cardinality can be "0" to allow for virtual network ports without any specific allocated bandwidth. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| attachedResourceId | VirtualNetworkInterface | 0..1 | Identifier of the attached resource to the network port (e.g., a virtualised compute resource, or identifier of the virtual network interface). The cardinality can be "0" if there is no specific resource connected to the network port. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| operationalState | OperationalState | 1 | The operational state of the virtual network port. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| metadata | KeyValuePair | 0..* | List of metadata key-value pairs used by the consumer to associate meaningful metadata to the related virtualised resource. | OpenModelAttribute - isInvariant: false - support: OPTIONAL |

###### VirtualResource class

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualisedResourceDomain::VirtualisedResourceModule::VirtualResource

This class is abstract.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

###### VirtualStorage class

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualisedResourceDomain::VirtualisedResourceModule::VirtualStorage

Description:

The VirtualStorage class encapsulates information of an instantiated virtualised storage resource.

Parent class: VirtualResource

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VirtualStorage

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| storageId | Identifier | 1 | Identifier of the virtualised storage resource. | OpenModelAttribute - isInvariant: true - support: MANDATORY |
| storageName | String | 0..1 | Name of the virtualised storage resource. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| flavourId | Identifier | 1 | Identifier of the storage flavour used to instantiate this virtual storage | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| typeOfStorage | String | 1 | Type of virtualised storage resource (e.g. volume, object). | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| sizeOfStorage | Number | 1 | Size of virtualised storage resource (e.g. size of volume, in GB). | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| rdmaEnabled | Boolean | 1 | Indicate if the storage support RDMA | OpenModelAttribute - isInvariant: false - support: OPTIONAL |
| ownerId | Identifier | 0..1 | Identifier of the virtualised resource that owns and uses such a virtualised storage resource. The value can be NULL if the virtualised storage is not attached yet to any other resource (e.g., a virtual machine). | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| zoneId | Identifier | 0..1 | If present, it identifies the Resource Zone where the virtual storage resources have been allocated. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| hostId | Identifier | 0..1 | Identifier of the host where the virtualised storage resource is allocated. A cardinality of 0 refers to distributed storage solutions. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| operationalState | OperationalState | 1 | Operational state of the resource. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| metadata | KeyValuePair | 0..* | List of metadata key-value pairs used by the consumer to associate meaningful metadata to the related virtualised resource. | OpenModelAttribute - isInvariant: false - support: OPTIONAL |

###### VirtualisationContainer class

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualisedResourceDomain::VirtualisedResourceModule::VirtualisationContainer

Description:

A partition of a compute node that provides an isolated virtualized computation environment. NOTE: Examples of virtualization container includes virtual machine and OS container.

This class is abstract.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

##### Datatypes

###### CpuPinningPolicy enumeration

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualisedResourceDomain::VirtualisedResourceModule::CpuPinningPolicy

Applied Stereotypes:

Preliminary

Contains Enumeration Literals:

STATIC

DYNAMIC

###### VirtualCpuPinning datatype

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualisedResourceDomain::VirtualisedResourceModule::VirtualCpuPinning

Applied Stereotypes:

Experimental

Table B.1: Attributes for Data Type VirtualCpuPinning

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| cpuPinningPolicy | CpuPinningPolicy | 1 | The policy can take values of "static" or "dynamic". In case of "static", the virtual CPU cores have been allocated to physical CPU cores according to the rules defined in cpuPinningRules. In case of "dynamic", the allocation of virtual CPU cores to physical CPU cores is decided by the VIM. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| cpuPinningMap |  | 0..1 | If cpuPinningPolicy is defined as "static", the cpuPinninMap provides the map of pinning virtual CPU cores to physical CPU cores/threads. Cardinality is 0 if cpuPinningPolicy has a different value. | Obsolete OpenModelAttribute - isInvariant: false - support: MANDATORY |
| cpuPinningRules |  | 0..* | A list of rules that should be considered during the allocation of the virtual CPUs to physical CPUs in case of "static" cpuPinningPolicy | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| cpuMap |  | 1 | Shows the association of virtual CPU cores to physical CPU cores. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

##### Associations

###### NetworkHasPorts association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualisedResourceDomain::VirtualisedResourceModule::NetworkHasPorts

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association NetworkHasPorts

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| networkPort | composite | Navig. | 0..* | VirtualNetworkPort | Element providing information of an instantiated virtual network port. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| networkId | none | Navig. | 1 | VirtualNetwork | Identifier of the network that the port belongs to. When creating a port, such port needs to be part of a network. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |

###### NetworkHasQoS association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualisedResourceDomain::VirtualisedResourceModule::NetworkHasQoS

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association NetworkHasQoS

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| networkQoS | shared | Navig. | 0..* | NetworkQoS | Element providing information about Quality of Service attributes that the network supports. Cardinality can be "0" for virtual network without any QoS requirements | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualNetwork | none | Not navig. | 0..* | VirtualNetwork |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NetworkHasSubnet association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualisedResourceDomain::VirtualisedResourceModule::NetworkHasSubnet

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association NetworkHasSubnet

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| subnet | none | Navig. | 0..* | NetworkSubnet | Only present if the network provides layer 3 connectivity. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| networkId | none | Navig. | 0..1 | VirtualNetwork | The identifier of the virtualised network that the virtualised sub-network is attached to. The cardinality can be 0 to cover the case where this type is used to describe the L3 attributes of a network rather than a subnetwork. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |

###### NetworkInterfaceAttachedToNetwork association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualisedResourceDomain::VirtualisedResourceModule::NetworkInterfaceAttachedToNetwork

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association NetworkInterfaceAttachedToNetwork

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| networkId | none | Navig. | 0..1 | VirtualNetwork | In the case when the virtual network interface is attached to the network, it identifies such a network. The cardinality can be 0 in the case that a network interface is created without being attached to any specific network. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| virtualNetworkInterface | none | Not navig. | 0..* | VirtualNetworkInterface |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NetworkPortHasAttachedNetworkInterface association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualisedResourceDomain::VirtualisedResourceModule::NetworkPortHasAttachedNetworkInterface

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association NetworkPortHasAttachedNetworkInterface

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| attachedResourceId | none | Navig. | 0..1 | VirtualNetworkInterface | Identifier of the attached resource to the network port (e.g., a virtualised compute resource, or identifier of the virtual network interface). The cardinality can be "0" if there is no specific resource connected to the network port. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| networkPortId | none | Navig. | 0..1 | VirtualNetworkPort | If the virtual network interface is attached to a specific network port, it identifies such a network port. The cardinality can be 0 in the case that a network interface is created without any specific network port attachment. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |

###### NfviPopContainsResourceZones  association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualisedResourceDomain::VirtualisedResourceModule::NfviPopContainsResourceZones

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association NfviPopContainsResourceZones

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| resourceZone | none | Not navig. | 1 | ResourceZone |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nfviPopId | shared | Navig. | 1..* | NfviPop | The identifier of the NFVI-PoP the Resource Zone belongs to. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |

###### VMHasVirtualCompute association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualisedResourceDomain::VirtualisedResourceModule::VMHasVirtualCompute

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VMHasVirtualCompute

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| virtualCompute | composite | Navig. | 1 | VirtualCompute | The Virtual Compute running on this VM. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualMachine | none | Not navig. | 0..1 | VirtualMachine |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VirtualComputeHasVirtualCpu association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualisedResourceDomain::VirtualisedResourceModule::VirtualComputeHasVirtualCpu

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VirtualComputeHasVirtualCpu

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| virtualCpu | shared | Navig. | 1 | VirtualCpu | The virtual CPU(s) of the virtualised compute | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualCompute | none | Not navig. | 1 | VirtualCompute |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VirtualComputeHasVirtualMemory association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualisedResourceDomain::VirtualisedResourceModule::VirtualComputeHasVirtualMemory

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VirtualComputeHasVirtualMemory

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| virtualMemory | shared | Navig. | 1 | VirtualMemory | The virtual memory of the compute | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualCompute | none | Not navig. | 1 | VirtualCompute |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VirtualComputeHasVirtualNetworkInterface association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualisedResourceDomain::VirtualisedResourceModule::VirtualComputeHasVirtualNetworkInterface

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VirtualComputeHasVirtualNetworkInterface

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| virtualCompute | none | Not navig. | 1 | VirtualCompute |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualNetworkInterface | shared | Navig. | 0..* | VirtualNetworkInterface | Element with information of the instantiated virtual network interfaces of the compute resource | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VirtualComputeHasVirtualStorage association

Qualified Name: NfvInformationModel::NFVCoreModel::VirtualisedResourceDomain::VirtualisedResourceModule::VirtualComputeHasVirtualStorage

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VirtualComputeHasVirtualStorage

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| virtualDisks | shared | Navig. | 1..* | VirtualStorage | Element with information of the virtualised storage resources (volumes, ephemeral that are attached to the compute resource. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualCompute | none | Not navig. | 1 | VirtualCompute |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

# NFVInterfaceInformationModel

## Overview

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel

Description:

Extends the NFV Core Model with information elements specific to NFV interfaces

## NFVCommonDomain

### Overview

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NFVCommonDomain

Description:

Group all the classes and types common between other domains that are needed for NFV interfaces

### CommonElements

#### Overview

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NFVCommonDomain::CommonElements

Description:

Model elements used in more than one module.

##### Classes

###### AffinityOrAntiAffinityResourceList class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NFVCommonDomain::CommonElements::AffinityOrAntiAffinityResourceList

Description:

The AffinityOrAntiAffinityResourceList class defines an explicit list of resources to express affinity or anti-affinity between these resources and a current resource. The scope of the affinity or anti-affinity can also be defined.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class AffinityOrAntiAffinityResourceList

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| scope | AffinityOrAntiAffinityResourceScope | 0..1 | Qualifies the scope of the constraint, e.g. NFVI-PoP, NFVI Node. Defaults to NFVI Node if absent. | Obsolete OpenModelAttribute - isInvariant: false - support: MANDATORY |
| resource | Identifier | 1..* | List of identifiers of virtualised resources. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### CpProtocolInfo class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NFVCommonDomain::CommonElements::CpProtocolInfo

Description:

This information element describes and associates the protocol layer that a CP uses together with other protocol-related information, like addresses.

Applied Stereotypes:

Experimental

OpenModelClass

support:  MANDATORY

Table B.1: Attributes for Class CpProtocolInfo

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| layerProtocol | LayerProtocol | 1 | Identifies which protocol the CP uses for connectivity purposes. NOTE 1: The layerProtocol values shall be compatible with the ones defined in the CPD. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| address |  | 1..* | For a SAP or a PnfExtCp, address of this CP. For a VnfExtCp or a VnfcCp, list of network addresses that have been configured (statically or dynamically) on the CP. NOTE 2: The address information shall be compatible with the layerProtocol attribute. NOTE 3: For a SAP or a PnfExtCp, only a single address shall be provided. For a SAP, in some cases, the NFVO provides the address. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VimConnectionInfo class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NFVCommonDomain::CommonElements::VimConnectionInfo

Description:

This class provides information regarding a VIM.

It is assumed that during the protocol design stage, VimInfo will be specified such that it allows interfacing to different VIM types.

Applied Stereotypes:

Experimental

OpenModelClass

support:  MANDATORY

Table B.1: Attributes for Class VimConnectionInfo

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vimConnectionInfoId | Identifier | 1 | The identifier of this VimConnectionInfo instance, for the purpose of referencing it from other classes. This identifier is managed by the NFVO. | Experimental OpenModelAttribute - isInvariant: true - support: MANDATORY |
| vimId | Identifier | 0..1 | The identifier of the VIM. This identifier is managed by the NFVO. Shall be present to address additional information about the VIM if such information has been configured into the VNFM by means outside the scope of the present document, and should be absent otherwise. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| interfaceInfo |  | 0..* | Information about the interface(s) to the VIM, if available, including interface endpoint e.g. URL, VIM provider type, API version, and protocol type. Alternatively, such information may have been configured into the VNFM and bound to the VimId. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| accessInfo |  | 0..* | Authentication credentials for accessing the VIM. Examples can include those to support different authentication schemes, e.g., OAuth, Token, Username/password, etc. NOTE: If needed, this attribute also provides information about the resourceGroupIds that are accessible using a particular set of credentials. | OpenModelAttribute - isInvariant: false - support: MANDATORY Preliminary |
| interfaceEndpoint | String | 0..1 | Information about the interface endpoint, if applicable. An example is a URL. More complex interface endpoints can be modelled as part of interfaceInfo. | Obsolete OpenModelAttribute - isInvariant: false - support: MANDATORY |
| extra |  | 0..* | VIM type specific additional information, if applicable. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

##### Datatypes

###### ExtLinkPortData datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NFVCommonDomain::CommonElements::ExtLinkPortData

Description:

This information element represents an externally provided link port to be used to connect an external connection point to an external VL.

Applied Stereotypes:

Experimental

Table B.1: Attributes for Data Type ExtLinkPortData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| extLinkPortId | Identifier | 1 | Identifier of this link port as provided by the entity that has created the link port. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| resourceHandle | ResourceHandle | 1 | Resource handle of the virtualised resource that realizes the external link port. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### ExtManagedVirtualLinkData datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NFVCommonDomain::CommonElements::ExtManagedVirtualLinkData

Description:

This datatype provides the information of an externally-managed internal VL to be used as a parameter passed to multiple interfaces.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type ExtManagedVirtualLinkData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| extManagedVirtualLinkId | Identifier | 1 | Identifier of this externally-managed internal VL instance. The identifier is assigned by the NFV-MANO entity that manages this VL instance. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualLinkDescId | VnfVirtualLinkDesc | 1 | Identifier of the VLD in the VNFD for this VL. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vimConnectionId | VimConnectionInfo | 0..1 | Identifier of the VIM connection to manage this resource. NOTE: The VimConnectionInfo is a data structure known to the VNFM. The identifier of the VIM connection provides scope to the resourceId. | Experimental OpenModelAttribute - isInvariant: false - support: CONDITIONAL_MANDATORY - condition: This attribute shall be supported and present if VNF-related resource management in direct mode is applicable. PassedByReference |
| resourceProviderId | Identifier | 0..1 | Identifies the entity responsible for the management of the resource. | OpenModelAttribute - isInvariant: false - support: CONDITIONAL_MANDATORY - condition: This attribute shall be supported and present when VNF-related Resource Management in indirect mode is applicable. |
| resourceId | Identifier | 1 | Identifier of the resource in the scope of the VIM or the resource provider. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### ExtVirtualLinkData datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NFVCommonDomain::CommonElements::ExtVirtualLinkData

Description:

This datatype provides the information of an external VL to be used as a parameter passed to multiple interfaces.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type ExtVirtualLinkData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| extVirtualLinkId | Identifier | 1 | Identifier of this external VL instance. The identifier is assigned by the NFV-MANO entity that manages this VL instance. | Experimental OpenModelAttribute - isInvariant: true - support: MANDATORY |
| vimConnectionId | VimConnectionInfo | 0..1 | Identifier of the VIM connection to manage this resource. NOTE: The VimConnectionInfo is a data structure known to the VNFM. The identifier of the VIM connection provides scope to the resourceId. | Experimental OpenModelAttribute - isInvariant: false - support: CONDITIONAL_MANDATORY - condition: This attribute shall be supported and present if VNF-related resource management in direct mode is applicable PassedByReference |
| resourceProviderId | Identifier | 0..1 | Identifies the entity responsible for the management of the resource. | OpenModelAttribute - isInvariant: false - support: CONDITIONAL_MANDATORY - condition: This attribute shall be supported and present when VNF-related Resource Management in indirect mode is applicable. |
| resourceId | Identifier | 1 | Identifier of the resource in the scope of the VIM or the resource provider. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| extCp | VnfExtCpData | 1..* | External CPs of the VNF to be connected to this VL. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| extLinkPorts | ExtLinkPortData | 1 | Externally provided link ports to be used to connect external connection points to this external VL. If this attribute is not present, the VNFM shall create the link ports on the external VL. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### OnboardingState enumeration

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NFVCommonDomain::CommonElements::OnboardingState

Description:

On-bording state of the information object and its associated NSD/VNFD.

Applied Stereotypes:

Experimental

Contains Enumeration Literals:

CREATED

UPLOADING

PROCESSING

ONBOARDED

###### ResourceHandle datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NFVCommonDomain::CommonElements::ResourceHandle

Description:

This datatype provides information that allows addressing a resource that is used by a VNF instance or by an NS instance.

Information about the resource is available from the corresponding Virtualised Compute/Storage/Network Resource Management interfaces.

Applied Stereotypes:

Experimental

Preliminary

Table B.1: Attributes for Data Type ResourceHandle

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vimConnectionId | VimConnectionInfo | 0..1 | Reference to the identifier of the VimConnectionInfo information element defining the VIM Connection to manage this resource. It shall also be supported for resources that are part of an NS instance such as virtual link resources. NOTE: The VimConnectionInfo is a data structure known to the VNFM. The identifier of the VIM connection provides scope to the resourceId. | Experimental OpenModelAttribute - isInvariant: false - support: CONDITIONAL_MANDATORY - condition: This parameter shall be supported when VNF-related Resource Management in direct mode is applicable PassedByReference |
| resourceProviderId | Identifier | 0..1 | Identifies the entity responsible for the management of the virtualized resource. | OpenModelAttribute - isInvariant: false - support: CONDITIONAL_MANDATORY - condition: This parameter shall be supported when VNF-related Resource Management in indirect mode is applicable |
| resourceId | Identifier | 1 | Identifier of the resource in the scope of the VIM or the resource provider. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vimLevelResourceType |  | 1 | Type of the resource in the scope of the VIM or the resource provider. NOTE: The value set of the "vimLevelResourceType" attribute is within the scope of the VIM or the resource provider and can be used as information that complements the ResourceHandle. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfExtCpConfig datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NFVCommonDomain::CommonElements::VnfExtCpConfig

Description:

This information element represents an externally provided link port or network address information per instance of an external connection point.

In case a link port is provided, the VNFM shall use that link port when connecting the external CP to the external VL.

In case no link port is provided, the VNFM shall create a link port on the external VL, and use that link port to connect the external CP to the external VL.

NOTE:  The following conditions apply to the attributes "linkPortId" and " cpProtocolData":

1) The "linkPortId" and "cpProtocolData" attributes shall both be absent for the deletion of an existing external CP instance addressed by cpInstanceId.

2) At least one of these attributes shall be present for a to-be-created external CP instance or  an existing external CP instance.

3) If the "linkPortId" attribute is absent, the VNFM shall create a link port.

4) If the "cpProtocolData" attribute is absent, the "linkPortId" attribute shall be provided referencing a pre-created link port, and the VNFM can use means outside the scope of the present document to obtain the pre-configured address information for the connection point from the resource representing the link port.

5) If both "cpProtocolData" and "linkportId" are provided, the NFVO shall ensure that the cpProtocolData can be used with the pre-created link port referenced by "linkPortId".

Applied Stereotypes:

Experimental

Table B.1: Attributes for Data Type VnfExtCpConfig

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| cpInstanceId | Identifier | 0..1 | Identifier of the external CP instance to which this set of configuration parameters is requested to be applied. Shall be present if this instance has already been created. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| linkPortId | ExtLinkPortData | 0..1 | Identifier of a pre-configured link port to which the external CP will be associated. See note in the description of VnfExtCpConfig. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| cpProtocolData |  | 0..* | Parameters for configuring fixed and dynamic network addresses for the CP, including the information on applicable layer protocol(s). For dynamic addresses, it should be possible to define per parameter set the number of network addresses to be assigned dynamically. Other parameters could be, e.g. valid address ranges or subnet. See note in the description of VnfExtCpConfig. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfExtCpData datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NFVCommonDomain::CommonElements::VnfExtCpData

Description:

This datatype provides input information related to one or more external CP instances created based on the same CPD.

Applied Stereotypes:

Experimental

Table B.1: Attributes for Data Type VnfExtCpData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| cpdId | Identifier | 1 | Identifier of the CPD in the VNFD. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| fixedAddresses |  | 0..* | List of (fixed) network addresses that need to be configured on the CP. It shall be provided for configuring fixed addresses. | Obsolete Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| dynamicAddresses |  | 0..* | List of parameter sets for the assignment of dynamic addresses. It should be possible to define per parameter set the number of network addresses to be assigned dynamically. Other parameters could be, e.g., valid address ranges or subnets. It shall be provided if dynamic addresses need to be configured on the CP. | Obsolete OpenModelAttribute - isInvariant: false - support: MANDATORY |
| cpConfig | VnfExtCpConfig | 1..* | List of instance data that need to be configured on the CP instances created from the respective CPD. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

### CommonStateModule

#### Overview

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NFVCommonDomain::CommonStateModule

##### Datatypes

###### OperationStatus enumeration

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NFVCommonDomain::CommonStateModule::OperationStatus

Description:

Indicates the start of a lifecycle operation or the result of a lifecycle operation.

Applied Stereotypes:

Preliminary

Contains Enumeration Literals:

START_OF_OPERATION

RESULT_OF_OPERATION

### FaultModule

#### Overview

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NFVCommonDomain::FaultModule

##### Diagrams

Figure B.1: Diagram Fault Management Resources

##### Classes

###### Alarm class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NFVCommonDomain::FaultModule::Alarm

Description:

The Alarm class encapsulates information about an alarm.

The managed object depends on the reference point using this class.  When used on Or-Vi or Vi-Vnfm, it will be a virtualised resource. When used on Or-Vnfm or Ve-Vnfm reference point, it will be a VNF instance. When used on Os-Ma-nfvo reference point, it will be a Network Service instance.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class Alarm

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| alarmId | Identifier | 1 | Identifier of this Alarm class. | OpenModelAttribute - isInvariant: true - support: MANDATORY |
| managedObjectId | Identifier | 1 | Identifier of the affected managed object. The managed object depends on the reference point using this class. When used on Or-Vi or Vi-Vnfm, it will be a virtualised resource. When used on Or-Vnfm or Ve-Vnfm reference point, it will be a VNF instance. When used on Os-Ma-nfvo reference point, it will be a Network Service instance. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfcId | Identifier | 1..* | Identifier of the affected VNFC(s). | OpenModelAttribute - isInvariant: false - support: CONDITIONAL_MANDATORY - condition: Only present in IFA008 |
| rootCauseFaultyComponent | FaultyComponentInfo | 1 | Only for IFA013. The NS components (e.g. nested NS, NS virtual link and VNF) that are causing the NS fault. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| rootCauseFaultyResource | FaultyResourceInfo | 0..1 | The virtualised resources that are causing the VNF or NS fault. For IFA013, it shall be present when the faulty component is "NS Virtual Link" or "VNF" | Experimental OpenModelAttribute - isInvariant: false - support: CONDITIONAL_MANDATORY - condition: Only present in IFA007, IFA008 and IFA013 |
| alarmRaisedTime | DateTime | 1 | Timestamp indicating when the alarm was raised by the managed object. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| alarmChangedTime | DateTime | 0..1 | Timestamp indicating when the alarm was last changed. It shall be present if the alarm has been updated. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| alarmClearedTime | DateTime | 0..1 | Timestamp indicating when the alarm was cleared. It shall be present if the alarm has been cleared. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| ackState | AckState | 1 | State of the alarm, permitted values include: • Acknowledged • Unacknowledged | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| perceivedSeverity | PerceivedSeverity | 1 | Perceived severity of the managed object failure, legal values. e.g.: • Critical • Major • Minor • Warning • Indeterminate • Cleared | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| eventTime | DateTime | 1 | Timestamp indicating when the fault was observed. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| eventType | EventType | 1 | Type of the event. The allowed values for the eventType attribute use the event type defined in [x]: • Communication Alarm • Processing Alarm • Environment Alarm • QoS Alarm • Equipment Alarm | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| faultType | String | 0..1 | Additional information related to the type of the fault | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| probableCause | String | 1 | Information about the probable cause of the fault. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| isRootCause | Boolean | 1 | Parameter indicating if this fault is the root for other correlated alarms. If TRUE, then the alarms listed in the parameter CorrelatedAlarmId are caused by this fault. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| correlatedAlarmId | Alarm | 0..* | List of other alarms correlated to this fault. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| faultDetails |  | 0..* | Provides additional information about the fault. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### FaultyResourceInfo class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NFVCommonDomain::FaultModule::FaultyResourceInfo

Description:

The FaultyResourceInfo class encapsulates information about faulty resource that has a negative impact on a managed object.

Applied Stereotypes:

Experimental

OpenModelClass

condition:  Only present in IFA007 and IFA008

support:  CONDITIONAL_MANDATORY

Table B.1: Attributes for Class FaultyResourceInfo

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| faultyVnfcResource | VnfcResourceInfo | 0..* | Information on the faulty virtualised compute and storage resource(s). | Obsolete OpenModelAttribute - isInvariant: false - support: MANDATORY |
| faultyVirtualLinkResource | VnfVirtualLinkResourceInfo | 0..* | Information on the faulty virtualised network resource(s). | Obsolete OpenModelAttribute - isInvariant: false - support: MANDATORY |
| faultyVirtualStorageResource | VirtualStorageResourceInfo | 0..* | Information on the faulty virtualised storage resource(s). | Obsolete OpenModelAttribute - isInvariant: false - support: MANDATORY |
| faultyResource | ResourceHandle | 1 | Information that identifies the faulty resource instance and its managing entity. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| faultyResourceType | FaultyResourceType | 0..* | Type of the faulty resource. Values: - COMPUTE - STORAGE - NETWORK | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

##### Notifications

###### AlarmClearedNotification notification

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NFVCommonDomain::FaultModule::AlarmClearedNotification

Description:

This notification informs the receiver of the clearing of an alarm. The alarm's perceived severity shall be set to "cleared" since the corresponding fault has been solved.

Applied Stereotypes:

OpenModelNotification

triggerConditionList:

An alarm has been cleared.

support:    MANDATORY

Preliminary

Table B.1: Attributes for Signal AlarmClearedNotification

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| alarmId | Alarm | 1 | Alarm identifier. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| alarmClearedTime | DateTime | 1 | Timestamp indicating when the alarm was cleared. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### AlarmListRebuiltNotification notification

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NFVCommonDomain::FaultModule::AlarmListRebuiltNotification

Description:

This notification informs the receiver that the active alarm list has been rebuilt by the VNFM. Upon receipt of this notification, the receiver needs to use the "Get Alarm List" operation to synchronize its view on current active alarms with that of the VNFM.

Applied Stereotypes:

Experimental

OpenModelNotification

triggerConditionList:

Active alarm list has been rebuilt by the VNFM, e.g. if the VNFM detects its storage holding the alarm list is corrupted.

support:    MANDATORY

###### AlarmNotification notification

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NFVCommonDomain::FaultModule::AlarmNotification

Description:

This notification informs the receiver of alarms resulting from the faults related to the managed objects.

The managed object depends on the reference point using this notification.  When used on Or-Vi or Vi-Vnfm, it will be a virtualised resource. When used on Or-Vnfm or Ve-Vnfm reference point, it will be a VNF instance. When used on Os-Ma-nfvo reference point, it will be a Network Service instance.

Applied Stereotypes:

OpenModelNotification

triggerConditionList:

An alarm has been created.

An alarm has been updated, e.g. if the severity of the alarm has changed.

support:    MANDATORY

Preliminary

Table B.1: Attributes for Signal AlarmNotification

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| alarm | Alarm | 1 | Information about an alarm including alarmId, affected managedObjectId, and faultDetails. For notifications related to changes in the state of virtualised resources (indicated using the attribute faultType), the alarm shall indicate: • the cause for the state change of the virtualised resource using the attribute probableCause, with possible values such as: maintenance of NFVI component, evacuation of NFVI component, etc. • the identifier of the origin (VIM) responsible for the management of the virtualised resource with state change using the attribute faultDetails. Note: In case the alarm cause cannot be determined, the notification should identify the alarm as cause as being undetermined. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

##### Datatypes

###### AckState enumeration

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NFVCommonDomain::FaultModule::AckState

Description:

Defines the state of the alarm.

Applied Stereotypes:

Experimental

Contains Enumeration Literals:

ACKNOWLEDGED

UNACKNOWLEDGED

###### FaultyComponentInfo datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NFVCommonDomain::FaultModule::FaultyComponentInfo

Description:

The FaultyComponentInfo information element encapsulates information about faulty component that has a negative impact on an NS.

Applied Stereotypes:

Experimental

Table B.1: Attributes for Data Type FaultyComponentInfo

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| faultyNestedNsInstanceId | NsInfo | 0..1 | Identifier of the faulty nested NS instance. NOTE: At least one of the parameters shall be present. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| faultyNsVirtualLinkInstanceId | NsVirtualLinkInfo | 0..1 | Identifier of the faulty NS virtual link instance. NOTE: At least one of the parameters shall be present. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| faultyVnfInstanceId | VnfInfo | 0..1 | Identifier of the faulty VNF instance. NOTE: At least one of the parameters shall be present. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |

###### FaultyResourceType enumeration

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NFVCommonDomain::FaultModule::FaultyResourceType

Applied Stereotypes:

Experimental

Contains Enumeration Literals:

COMPUTE

STORAGE

NETWORK

###### PerceivedSeverity enumeration

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NFVCommonDomain::FaultModule::PerceivedSeverity

Description:

Perceived severity of an alarm.

Applied Stereotypes:

Preliminary

Contains Enumeration Literals:

CRITICAL

MAJOR

MINOR

WARNING

INDETERMINATE

CLEARED

##### Associations

###### AlarmHasCorrelatedAlarms association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NFVCommonDomain::FaultModule::AlarmHasCorrelatedAlarms

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association AlarmHasCorrelatedAlarms

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| correlatedAlarmId | none | Navig. | 0..* | Alarm | List of other alarms correlated to this fault. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| parentAlarm | none | Not navig. | 0..1 | Alarm |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

### PerformanceModule

#### Overview

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NFVCommonDomain::PerformanceModule

##### Diagrams

Figure B.1: Diagram Performance Management Resources

##### Classes

###### ObjectSelection class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NFVCommonDomain::PerformanceModule::ObjectSelection

Description:

This class allows to specify object instances on which performance information will be provided.

The ObjectSelection is a pattern to select object instances. The pattern is used in multiple interfaces.

The pattern proposes 2 exclusive options:

1)      Provide a list of object types and a filter to specify object properties

2)      Provide a list of object instances

The objects depend on the reference point using this class.

When used on Or-Vi or Vi-Vnfm, it will be virtualised resources.

When used on Or-Vnfm or Ve-Vnfm reference point, it will be VNF instances (represented by VNFD) and the filter will be based on some VNF properties.

When used on Os-Ma-nfvo reference point, it will be Network Service instances.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class ObjectSelection

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| objectType | String | 0..* | Provides the object type. One of the two attributes (objectType+ objectFilter or objectInstanceId) shall be present. Note: this atribute is MANDATORY with a cardinality of 0..* in ETSI GS NFV IFA 007, ETSI GS NFV IFA 008 and ETSI GS NFV IFA 013, while it is CM with a cardinality of 1..* in ETSI GS NFV IFA 005 and ETSI GS NFV IFA 006. The former definition is used. | OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: |
| objectFilter | Filter | 0..1 | The filter will apply on the object types to specify on which object instances the performance information is requested to be collected. One of the 2 attributes (objectType+ objectFilter or objectInstanceId) shall be present. Note: this atribute is MANDATORY with a cardinality of 0..1 in ETSI GS NFV IFA 007, ETSI GS NFV IFA 008 and ETSI GS NFV IFA 013, while it is CM with a cardinality of 1 in ETSI GS NFV IFA 005 and ETSI GS NFV IFA 006. The former definition is used. | OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: |
| objectInstanceId | Identifier | 0..* | Identifies the object instances for which performance information is requested to be collected. One of the 2 attributes (objectType+ objectFilter or objectInstanceId) shall be present. Note: this atribute is MANDATORY in ETSI GS NFV IFA 007, ETSI GS NFV IFA 008 and ETSI GS NFV IFA 013, while it is CM in ETSI GS NFV IFA 005 and ETSI GS NFV IFA 006. The former definition is used. | OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: |

###### PerformanceReport class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NFVCommonDomain::PerformanceModule::PerformanceReport

Description:

This class defines the format of a performance report provided by the producer to the consumer on a specified object instance or a set of them.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class PerformanceReport

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| performanceReportEntry | PerformanceReportEntry | 1..* | List of performance information entries. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### PerformanceReportEntry class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NFVCommonDomain::PerformanceModule::PerformanceReportEntry

Description:

This class defines a single performance report entry. This performance report entry is for a given metric of a given object instance, but can include multiple collected values.

The object instances depend on the reference point using this class.

When used on Or-Vi or Vi-Vnfm, it will be virtualised resources.

When used on Or-Vnfm or Ve-Vnfm reference point, it will be VNF instances.

When used on Os-Ma-nfvo reference point, it will be Network Service instances.

The object type depends on the reference point using this class.

When used on Or-Vi or Vi-Vnfm, it will be virtualised resource type.

When used on Or-Vnfm or Ve-Vnfm reference point, it will be VNF (represented by VNFD).

When used on Os-Ma-nfvo reference point, it will be Network Service (represented by NSD).

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class PerformanceReportEntry

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| objectType | String | 1 | Defines the object type. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| objectInstanceId | Identifier | 1 | The object instance for which the performance metric is reported. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| performanceMetric | String | 1 | Name of the metric collected. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| performanceValue | PerformanceValueEntry | 1..* | List of performance values with associated timestamp. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### PerformanceValueEntry class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NFVCommonDomain::PerformanceModule::PerformanceValueEntry

Description:

This class defines a single performance value with its associated time stamp.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class PerformanceValueEntry

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| timeStamp | DateTime | 1 | Timestamp indicating when the data was collected. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| performanceValue | Value | 1 | Value of the metric collected. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### PmJob class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NFVCommonDomain::PerformanceModule::PmJob

Description:

This class provides the details of the PM Job.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class PmJob

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| pmJobId | Identifier | 1 | Identifier of this PmJob class. | OpenModelAttribute - isInvariant: true - support: MANDATORY |
| objectSelector | ObjectSelection | 1 | Defines the object instances for which performance information is requested to be collected. The object instances depend on the reference point using this class. When used on Or-Vi or Vi-Vnfm, it will be virtualised resources. When used on Or-Vnfm or Ve-Vnfm reference point, it will be VNF instances. When used on Os-Ma-nfvo reference point, it will be Network Service instances. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| performanceMetric | String | 0..* | This defines the type of performance metric(s) for the specified object instances. At least one of the two (performance metric or group) shall be present. Note: this atribute is MANDATORY in ETSI GS NFV IFA 007, ETSI GS NFV IFA 008 and ETSI GS NFV IFA 013, while it is CM in ETSI GS NFV IFA 005 and ETSI GS NFV IFA 006. The former definition is used. | OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: |
| performanceMetricGroup | String | 0..* | Group of performance metrics. A metric group is a pre-defined list of metrics, known to the producer that it can decompose to individual metrics. At least one of the two (performance metric or group) shall be present. Note: this atribute is MANDATORY in ETSI GS NFV IFA 007, ETSI GS NFV IFA 008 and ETSI GS NFV IFA 013, while it is CM in ETSI GS NFV IFA 005 and ETSI GS NFV IFA 006. The former definition is used. | OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: |
| collectionPeriod | CollectionPeriod | 1 | Specifies the periodicity at which the producer will collect performance information. NOTE: at the end of each reportingPeriod, the producer will inform the consumer about availability of the performance data collected for each completed collection period during this reportingPeriod. While the exact definition of the types for collectionPeriod and reportingPeriod is left for further specification, it is recommended that the reportingPeriod be equal or a multiple of the collectionPeriod. In the latter case, the performance data for the collection periods within one reporting period would be reported together. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| reportingPeriod | ReportingPeriod | 1 | Specifies the periodicity at which the producer will report to the consumer about performance information. NOTE: at the end of each reportingPeriod, the producer will inform the consumer about availability of the performance data collected for each completed collection period during this reportingPeriod. While the exact definition of the types for collectionPeriod and reportingPeriod is left for further specification, it is recommended that the reportingPeriod be equal or a multiple of the collectionPeriod. In the latter case, the performance data for the collection periods within one reporting period would be reported together. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| reportingBoundary |  | 0..1 | Identifies a boundary after which the reporting will stop. The boundary shall allow a single reporting as well as periodic reporting up to the boundary. | OpenModelAttribute - isInvariant: false - support: OPTIONAL |

###### Threshold class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NFVCommonDomain::PerformanceModule::Threshold

Description:

This class provides the details of a threshold.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class Threshold

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| thresholdId | Identifier | 1 | Identifier of this Threshold class. | OpenModelAttribute - isInvariant: true - support: MANDATORY |
| objectSelector | ObjectSelection | 1 | Defines the object instances associated with the threshold. The object instances depend on the reference point using this class. When used on Or-Vi or Vi-Vnfm, it will be virtualised resources. When used on Or-Vnfm or Ve-Vnfm reference point, it will be VNF instances. When used on Os-Ma-nfvo reference point, it will be Network Service instances. Note: this atribute has a cardinality of 1 in ETSI GS NFV IFA 007, ETSI GS NFV IFA 008 and ETSI GS NFV IFA 013, while it has a cardinality of 1..* in ETSI GS NFV IFA 005 and ETSI GS NFV IFA 006. The former definition is used. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| performanceMetric | String | 1 | Defines the performance metric associated with the threshold. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| thresholdType | ThresholdType | 1 | Type of threshold. The list of possible values is left for later stage and might include: single/ multi valued threshold, static/dynamic threshold, template based threshold. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| thresholdDetails |  | 1 | Details of the threshold: value to be crossed, details on the notification to be generated... | OpenModelAttribute - isInvariant: false - support: MANDATORY |

##### Notifications

###### PerformanceInformationAvailableNotification notification

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NFVCommonDomain::PerformanceModule::PerformanceInformationAvailableNotification

Description:

This notification informs the receiver that performance information is available.  Delivery mechanism for the performance reports is left for later specification.

Applied Stereotypes:

OpenModelNotification

triggerConditionList:

New performance information is available

support:    MANDATORY

Preliminary

Table B.1: Attributes for Signal PerformanceInformationAvailableNotification

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| objectInstanced | Identifier | 1..* | Object instance(s) for which performance information is available. The object instances depend on the reference point using this notification. When used on Or-Vi or Vi-Vnfm, it will be virtualised resources. When used on Or-Vnfm or Ve-Vnfm reference point, it will be VNF instances. When used on Os-Ma-nfvo reference point, it will be Network Service instances. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### ThresholdCrossedNotification notification

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NFVCommonDomain::PerformanceModule::ThresholdCrossedNotification

Description:

This notification informs the receiver that a threshold value has been crossed.

Applied Stereotypes:

Experimental

OpenModelNotification

triggerConditionList:

A Threshold has been crossed. Depending on threshold type, there might be a single or multiple crossing values.

support:    MANDATORY

Table B.1: Attributes for Signal ThresholdCrossedNotification

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| thresholdId | Threshold | 1 | Threshold which has been crossed. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| crossingDirection | ThresholdCrossing | 1 | An indication of whether the threshold was crossed in upward or downward direction. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| objectInstanceId | Identifier | 1 | Object instance for which the threshold has been crossed. The object instances depend on the reference point using this notification. When used on Or-Vi or Vi-Vnfm, it will be virtualised resources. When used on Or-Vnfm or Ve-Vnfm reference point, it will be VNF instances. When used on Os-Ma-nfvo reference point, it will be Network Service instances. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| performanceMetric | String | 1 | Performance metric associated with the threshold. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| performanceValue | Value | 1 | Value of the metric that resulted in threshold crossing. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

##### Datatypes

###### CollectionPeriod enumeration

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NFVCommonDomain::PerformanceModule::CollectionPeriod

Description:

Specifies the periodicity at which the producer will collect performance information.

Note: at the end of each reportingPeriod, the producer will inform the consumer about availability of the performance data collected for each completed collection period during this reportingPeriod. While the exact definition of the types for collectionPeriod and reportingPeriod is left for further specification, it is recommended that the reportingPeriod be equal or a multiple of the collectionPeriod. In the latter case, the performance data for the collection periods within one reporting period would be reported together.

Applied Stereotypes:

Preliminary

###### ReportingPeriod enumeration

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NFVCommonDomain::PerformanceModule::ReportingPeriod

Description:

Specifies the periodicity at which the producer will report to the consumer about performance information.

Note: at the end of each reportingPeriod, the producer will inform the consumer about availability of the performance data collected for each completed collection period during this reportingPeriod. While the exact definition of the types for collectionPeriod and reportingPeriod is left for further specification, it is recommended that the reportingPeriod be equal or a multiple of the collectionPeriod. In the latter case, the performance data for the collection periods within one reporting period would be reported together.

Applied Stereotypes:

Preliminary

###### ThresholdCrossing enumeration

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NFVCommonDomain::PerformanceModule::ThresholdCrossing

Description:

Indicates the direction in which the threshold was crossed.

Applied Stereotypes:

Preliminary

Contains Enumeration Literals:

UP

DOWN

###### ThresholdType enumeration

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NFVCommonDomain::PerformanceModule::ThresholdType

Description:

Type of threshold. The list of possible values is left for later stage and might include: single/ multi valued threshold, static/dynamic threshold, template based threshold and whether the threshold is a raise or clear.

Applied Stereotypes:

Preliminary

##### Associations

###### PMJobUsesResourceSelection association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NFVCommonDomain::PerformanceModule::PMJobUsesResourceSelection

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association PMJobUsesResourceSelection

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| objectSelector | composite | Navig. | 1 | ObjectSelection | Defines the object instances for which performance information is requested to be collected. The object instances depend on the reference point using this class. When used on Or-Vi or Vi-Vnfm, it will be virtualised resources. When used on Or-Vnfm or Ve-Vnfm reference point, it will be VNF instances. When used on Os-Ma-nfvo reference point, it will be Network Service instances. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| pmjob | none | Not navig. | 1 | PmJob |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### PerfReportEntryIncludesPerfValueEntry association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NFVCommonDomain::PerformanceModule::PerfReportEntryIncludesPerfValueEntry

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association PerfReportEntryIncludesPerfValueEntry

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| performanceValue | composite | Navig. | 1..* | PerformanceValueEntry | List of performance values with associated timestamp. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| performancereportentry | none | Not navig. | 1 | PerformanceReportEntry |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### PerfReportIncludesPerfReportEntry association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NFVCommonDomain::PerformanceModule::PerfReportIncludesPerfReportEntry

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association PerfReportIncludesPerfReportEntry

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| performanceReportEntry | composite | Navig. | 1..* | PerformanceReportEntry | List of performance information entries. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| performancereport | none | Not navig. | 1 | PerformanceReport |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### ThresholdUsesResourceSelection association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NFVCommonDomain::PerformanceModule::ThresholdUsesResourceSelection

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association ThresholdUsesResourceSelection

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| objectSelector | composite | Navig. | 1 | ObjectSelection | Defines the object instances associated with the threshold. The object instances depend on the reference point using this class. When used on Or-Vi or Vi-Vnfm, it will be virtualised resources. When used on Or-Vnfm or Ve-Vnfm reference point, it will be VNF instances. When used on Os-Ma-nfvo reference point, it will be Network Service instances. Note: this atribute has a cardinality of 1 in ETSI GS NFV IFA 007, ETSI GS NFV IFA 008 and ETSI GS NFV IFA 013, while it has a cardinality of 1..* in ETSI GS NFV IFA 005 and ETSI GS NFV IFA 006. The former definition is used. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| threshold | none | Not navig. | 1 | Threshold |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

## NsDomain

### Overview

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain

Description:

Group all the classes and types related to NS that are needed for interfaces

### NsLcmDataModule

#### Overview

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmDataModule

Description:

Group all datatypes that are needed for Network Service Lifecycle Management operations

##### Diagrams

Figure B.1: Diagram NS LCM datatypes

##### Datatypes

###### AddVnffgData  datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmDataModule::AddVnffgData

Description:

This datatype specifies the parameters that are needed for the creation of a new VNFFG instance.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type AddVnffgData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vnffgdId | Vnffgd | 1 | Identifier of the VNFFGD which defines the VNFFG to be added. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vnfFgName | String | 1 | Human readable name for the VNFFG. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| description | String | 1 | Human readable description for the VNFFG. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### AffinityOrAntiAffinityRule datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmDataModule::AffinityOrAntiAffinityRule

Description:

The AffinityOrAntiAffinityRule datatype describes the additional affinity or anti-affinity rule applicable between the VNF instances to be instantiated in the NS instantiation operation request or between the VNF instances to be instantiated in the NS instantiation operation request and the existing VNF instances.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type AffinityOrAntiAffinityRule

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| descriptorId | Identifier | 1..* | Reference to a VNFD or vnfProfile defined in the NSD. At least one VnfProfile which is used to instantiate VNF for the NS to be instantiated as the subject of the affinity or anti-affinity rule shall be present. When the VNFD or the VnfProfile which is not used to instantiate VNF, it presents all VNF instances of this type as the subjects of the affinity or anti-affinity rule. The VNF instance which the VnfProfile or the VNFD presents is not necessary as a part of the NS to be instantiated. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfInstanceId | Identifier | 0..* | Reference to the existing VNF instance as the subject of the affinity or anti-affinity rule. The existing VNF instance is not necessary as a part of the NS to be instantiated. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| affinityOrAntiAffiinty | Boolean | 1 | Specifies whether the rule is an affinity rule (TRUE) or an anti-affinity rule (FALSE). | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| scope | AffinityOrAntiAffinityResourceScope | 1 | Specifies whether the scope of the rule is a node, a PoP, etc… | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### AssocNewNsdVersionData datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmDataModule::AssocNewNsdVersionData

Description:

The AssocNewNsdVersionData datatype specifies a new NSD version that is associated to the NS instance. After issuing the Update NS operation with updateType=AssocNewNsdVersion, the NFVO shall use the referred NSD as a basis for the given NS instance.

Different versions of the same NSD have same nsdInvariantId, but different nsdId attributes, therefore if the nsdInvariantId of the NSD version that is to be associated to this NS instance is different from the one used before, the NFVO shall reject the request. Only new versions of the same NSD can be associated to an existing NS instance

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type AssocNewNsdVersionData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| newNsdId | Nsd | 1 | Identifier of the new NSD version that is to be associated to the NS instance. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| synch | Boolean | 0..1 | Specify whether the NS instance should be automatically synchronised to the new NSD by the NFVO (in case of true value) or the NFVO should not do any action (in case of a false value) and wait for further guidance from OSS/BSS (i.e. waiting for OSS/BSS to issue NS lifecycle management operation to explicitly add/remove VNFs and modify information of VNF instances according to the new NSD). The synchronisation to the new NSD means e.g. instantiating/adding those VNFs whose VNFD is referenced by the new NSD version but not referenced by the old one, terminating/removing those VNFs whose VNFD is referenced by the old NSD version but not referenced by the new NSD version, modifying information of VNF instances to the new applicable VNFD provided in the new NSD version. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### ChangeExtVnfConnectivityData datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmDataModule::ChangeExtVnfConnectivityData

Description:

The ChangeExtVnfConnectivityData information element specifies the external connectivity to change for the VNF.  The types of changes that this operation supports are:

- Disconnect the external CPs that are connected to a particular external VL, and connect them to a different external VL.

- Change the connectivity parameters of the existing external CPs, including changing addresses.

NOTE:  Depending on the capabilities of the underlying VIM resources, certain changes (e.g. modifying the IP address assignment) might not be supported without deleting the resource and creating another one with the modified configuration.

Applied Stereotypes:

Experimental

Table B.1: Attributes for Data Type ChangeExtVnfConnectivityData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vnfInstanceId | Identifier | 1 | Identifier of the VNF instance. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| extVirtualLink | ExtVirtualLinkData | 1..* | Information about external VLs to change (e.g., connect the VNF to). | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| additionalParam | KeyValuePair | 0..* | Additional parameters passed by the OSS as input to the external connectivity change process, specific to the VNF being changed, as declared in the VNFD. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### ChangeNsFlavourData datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmDataModule::ChangeNsFlavourData

Description:

The ChangeNsFlavourData datatype specifies a new DF to be applied to the NS instance.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type ChangeNsFlavourData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| newFlavourId | Identifier | 1 | Identifier of the new NS DF to be applied to this NS instance. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nsInstantiationLevelId | Identifier | 1 | Identifier of the NS instantiation level of the DF to be used. If not present, the default NS instantiation level as declared in the NSD shall be used. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### ChangeVnfFlavourData datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmDataModule::ChangeVnfFlavourData

Description:

The ChangeVnfFlavourData datatype specifies existing VNF instance for which the DF needs to be changed. This specifies the new DF, the instantiationLevel of the new DF that may be used and the additional parameters as input for the flavour change.

The change of VNF DF depends on VNF capabilities and its support by the VNF is declared in the VNFD.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type ChangeVnfFlavourData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vnfInstanceId | Identifier | 1 | Identifier of the VNF instance to be modified. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| newFlavourId | VnfDf | 1 | Identifier of the new VNF deployment flavour to apply to this VNF instance. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| instantiationLevel | InstantiationLevel | 0..1 | Identifier of the instantiation level of the deployment flavour to be used. If not present, the default instantiation level as declared in the VNFD shall be used. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| extVirtualLink | ExtVirtualLinkData | 1 | Information about external VLs to connect the VNF to. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| extManagedVirtualLink | ExtManagedVirtualLinkData | 0..* | Information about internal VLs that are managed by other entities than the VNFM. NOTE: The indication of externally-managed internal VLs is needed in case networks have been pre-configured for use with certain VNFs, for instance to ensure that these networks have certain properties such as security or acceleration features, or to address particular network topologies.The present document assumes that externally-managed internal VLs are managed by the NFVO and created towards the VIM as supported by the virtualised network resource management interface specified in ETSI GS NFV-IFA 005. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| additionalParam | KeyValuePair | 0..* | Additional parameters passed by the OSS/BSS as input to the flavour change process, specific to the VNF being modified,as declared in the VNFD. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### HealNsData datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmDataModule::HealNsData

Description:

This datatype describes the information needed to heal an NS.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type HealNsData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| degreeHealing | NsDegreeHealing | 1 | Indicates the degree of healing. Allowed values include: "complete healing of the NS restoring the state of the NS before the failure occurred", "complete healing based on the newest QoS values", "complete healing resetting to the original instantiation state of the NS" and "partial healing". | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| actionsHealing | String | 0..* | Used to specify dedicated healing actions in a particular order (e.g., as a script). The actionsHealing can be used to provide a specific script whose content and actions might only be possible to be derived during runtime. | Experimental OpenModelAttribute - isInvariant: false - support: CONDITIONAL_MANDATORY - condition: Either the actionsHealing or healScript attribute shall be present, not both attributes. |
| healScript | VnfLifeCycleManagementScript | 0..1 | Reference to a script from the NSD that shall be used to execute dedicate healing actions in a particular order. The healScript, since it refers to a script in the NSD, can be used to execute healing actions which are defined during NS design time. | Experimental OpenModelAttribute - isInvariant: false - support: CONDITIONAL_MANDATORY - condition: Either the actionsHealing or healScript attribute shall be present, not both attributes. PassedByReference |
| additionalParamForNs | KeyValuePair | 0..* | Allows the OSS/BSS to provide additional parameter(s) to the healing process at the NS level. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### HealVnfData datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmDataModule::HealVnfData

Description:

This datatype describes the information needed to heal a VNF that is part of an NS. The NFVO shall then invoke the HealVNF operation towards the appropriate VNFM.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type HealVnfData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vnfInstanceId | Identifier | 1 | Identifies the VNF instance, part of the NS, requiring a healing action. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| cause | String | 0..1 | Indicates the reason why a healing procedure is required. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| additionalParam | KeyValuePair | 0..* | Additional parameters passed by the OSS/BSS as input to the healing process, specific to the VNF being healed,as declared in the VNFD. EXAMPLE: input parameters to VNF-specific healing procedures. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### InstantiateVnfData datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmDataModule::InstantiateVnfData

Description:

The InstantiateVnfData datatype specifies the parameters that are needed for VNF instantiation. This datatype is used for the bottom-up NS creation when the OSS explicitly requests VNF instantiation for a given NS. When the NFVO invokes the Instantiate VNF operation  a set of these parameters are then passed by the NFVO to the VNFM.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type InstantiateVnfData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vnfdId | Vnfd | 1 | Information sufficient to identify the VNF Descriptor which defines the VNF to be instantiated. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| flavourId | VnfDf | 1 | Identifier of the VNF deployment flavour to be instantiated. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| instantiationLevelId | InstantiationLevel | 0..1 | Identifier of the instantiation level of the deployment flavour to be instantiated. If not present, the default instantiation level as declared in the VNFD shall be instantiated. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vnfInstanceName | String | 0..1 | Human-readable name of the VNF instance to be created. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfInstanceDescription | String | 0..1 | Human-readable description of the VNF instance to be created. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| extVirtualLink | ExtVirtualLinkData | 0..* | Information about external VLs to connect the VNF to. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| extManagedVirtualLink | ExtManagedVirtualLinkData | 0..* | Information about internal VLs that are managed by other entities than the VNFM. NOTE: The indication of externally-managed internal VLs is needed in case networks have been pre-configured for use with certain VNFs, for instance to ensure that these networks have certain properties such as security or acceleration features, or to address particular network topologies. The present document assumes that externally-managed internal VLs are managed by the NFVO and created towards the VIM as supported by the virtualised network resource management interface specified in ETSI GS NFV-IFA 005. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| localizationLanguage |  | 0..1 | Localization language of the VNF to be instantiated. The localization languages supported by a VNF can be declared in the VNFD. If this parameter is not provided and the "defaultLocalizationLanguage" attribute is declared in the VNFD, the "defaultLocalizationLanguage" shall be used to determine the localization language VNF to be instantiated. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| additionalParam | KeyValuePair | 0..* | Additional parameters passed by the OSS/BSS as input to the instantiation process, specific to the VNF being instantiated,as declared in the VNFD. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| locationConstraint | VnfLocationConstraint | 0..1 | Defines the location constraints for the VNF to be instantiated as part of the NS instantiation. An example can be a constraint for the VNF to be in a specific geographic location. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### ModifyVnfConfigData datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmDataModule::ModifyVnfConfigData

Description:

The ModifyVnfConfigData datatype specifies VNF configuration parameters that can be changed during operational state of the VNF. The configuration parameters referred in this clause include those set at initial configuration and any other configurable parameter declared in the VNFD.

Applied Stereotypes:

Obsolete

Table B.1: Attributes for Data Type ModifyVnfConfigData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vnfInstanceId | Identifier | 1 | Identifier of the VNF instance. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfConfigurationData | KeyValuePair | 0..* | Configuration data for the VNF instance. Cardinality "0" applies to information elements that are not requested to be modified. Configuration data can include values for the properties that are declared as configurable in the VNFD (see VnfConfigurableProperties). | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| extVirtualLink | ExtVirtualLinkData | 0..* | Information about external virtual links to connect the VNF to. Cardinality "0" applies to information elements that are not requested to be modified. External links can only be modified by the same functional block (FB) that supplied them. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### ModifyVnfInfoData datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmDataModule::ModifyVnfInfoData

Description:

The ModifyVnfInfoData datatype specifies for a VNF instance the information that is requested to be modified. The information to be modified shall comply with the associated NSD.

EXAMPLE: When the vnfPkgInfoId attribute value of VnfInfo needs to be updated, the value would need to match the identifier's value of an VnfPkgInfo whose vnfdId is present in the associated VNF type and profile of the NSD.

Applied Stereotypes:

Experimental

Table B.1: Attributes for Data Type ModifyVnfInfoData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vnfInstanceId | Identifier | 1 | Identifier of the VNF instance for which the writable attributes of VnfInfo are requested to be modified. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| newValues | KeyValuePair | 1..* | Contains the set of attributes to update. The key in the KeyValuePair indicates the name of an attribute that is writable through the interface whose value is to be updated. The value in the KeyValuePair indicates the new attribute value. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### MoveVnfInstanceData datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmDataModule::MoveVnfInstanceData

Description:

The MoveVnfInstanceData datatype specifies existing VNF instances that needs to be moved from one NS instance (source) to another NS instance (destination). The NS instance defined in the Update NS operation (refer to nsInstanceId in Table 7.3.5-2) indicates the source NS instance and the destination NS instance is specified in the present datatype.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type MoveVnfInstanceData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| targetNsInstanceId | Identifier | 1 | Specify the target NS instance where the VNF instances are moved to. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfInstanceId | Identifier | 1..* | Specify the VNF instance that is moved. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NfpData datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmDataModule::NfpData

Description:

This datatype contains information needed to create or modify an NFP instance.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type NfpData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| nfpId | Identifier | 0..1 | Identifier of the NFP to be modified. It shall be present for modified NFPs and shall be absent for the new NFP. | OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: It shall be present for modified NFPs and shall be absent for the new NFP. |
| nfpName | String | 0..1 | Human readable name for the NFP. It shall be present for the new NFP, and it may be present otherwise. | OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: It shall be present for the new NFP, and it may be present otherwise. |
| description | String | 0..1 | Human readable description for the NFP. It shall be present for the new NFP, and it may be present otherwise. | OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: It shall be present for the new NFP, and it may be present otherwise. |
| cp | CpInfo | 0..* | Identifier(s) of the CPs and SAPs which the NFP passes by. Cardinality can be 0 if only updated or newly created NFP classification and selection rule which applied to an existing NFP is provided. When multiple identifiers are included, the position of the identifier in the information element value specifies the position of the CP or the SAP in the path. | OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: At least a CP or an nfpRule shall be present. PassedByReference |
| nfpRule | NfpRule | 0..1 | NFP classification and selection rule. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: At least a CP or an nfpRule shall be present. |

###### NsDegreeHealing enumeration

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmDataModule::NsDegreeHealing

Description:

Indicates the degree of healing for an NS.

Applied Stereotypes:

Preliminary

Contains Enumeration Literals:

COMPLETE_HEALING_OF_THE_NS_RESTORING_THE_STATE_OF_THE_NS_BEFORE_THE_FAILURE_OCCURRED

COMPLETE_HEALING_BASED_ON_THE_NEWEST_QOS_VALUES

COMPLETE_HEALING_RESETTING_TO_THE_ORIGINAL_INSTANTIATION_STATE_OF_THE_NS

PARTIAL_HEALING

###### NsScaleDirection enumeration

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmDataModule::NsScaleDirection

Description:

Specifies the scaling direction for an NS.

Applied Stereotypes:

Preliminary

Contains Enumeration Literals:

SCALE_IN

SCALE_OUT

###### OperateVnfData datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmDataModule::OperateVnfData

Description:

The OperateVnfData datatype specifies the VNF instance for which the operational state needs to be changed and the requested new state.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type OperateVnfData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vnfInstanceId | Identifier | 1 | Identifier of the VNF instance. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| changeStateTo | VnfState | 1 | The desired state to change the VNF to. Allowed values are: start, stop. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| stopType | VnfStopType | 0..1 | It signals whether forceful or graceful stop is requested. Allowed values are: forceful and graceful. In case of forceful stop, the VNF is stopped immediately. Note that if the VNF is still in service, this may adversely impact network service, and therefore, operator policies apply to determine if forceful stop is allowed in the particular situation. In case of graceful stop, the VNFM first arranges to take the VNF out of service (by means out of scope of the present specification, e.g. involving interaction with EM, if required). Once this is successful, or after a timeout, the VNFM stops the VNF. Only applicable when changing state to stop. | OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: Only applicable when changing state to stop. |
| gracefulStopTimeout | TimeDuration | 0..1 | The time interval to wait for the VNF to be taken out of service during graceful stop, before stopping the VNF. If not given, it is expected that the VNFM waits for the successful taking out of service of the VNF, no matter how long it takes, before stopping the VNF. See note. Minimum timeout or timeout range are specified by the VNF vendor (e.g. defined in the VNFD or communicated by other means). The parameter is not relevant in case of forceful stop | OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: The parameter is not relevant in case of forceful stop. |
| additionalParam | KeyValuePair | 0..* | Additional parameters passed by the OSS/BSS as input to the Operate VNF operation, specific to the VNF being operated, as declared in the VNFD | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### ParamsForVnf datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmDataModule::ParamsForVnf

Description:

The ParamsForVnf datatype specifies additional parameters for an NS instance on a per VNF instance basis.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type ParamsForVnf

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vnfProfileId | VnfProfile | 1 | Identifier of (reference to) a vnfProfile to which the additional parameters apply. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| additionalParam | KeyValuePair | 0..* | Additional parameters that are to be applied per VNF instance. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### SapData datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmDataModule::SapData

Description:

The SapData datatype defines information related to a SAP of an NS.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type SapData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| sapdId | Sapd | 1 | Reference to the SAP descriptor for this SAP. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| sapName | String | 1 | Human readable name for the SAP. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| description | String | 1 | Human readable description for the SAP. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| address |  | 0..1 | Address for this SAP,including the information on applicable layer protocol(s). In some cases, the NFVO provides the address (refer to attribute sapAddressAssignment of Sapd information element in ETSI GS NFV-IFA 014). NOTE: The address information shall be compatible with the layerProtocol values defined in the CPD. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### ScaleByStepData datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmDataModule::ScaleByStepData

Description:

This datatype describes the information needed to scale a VNF instance by aspects and steps.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type ScaleByStepData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| type | VnfScaleByStepType | 1 | Defines the type of the scale operation requested (scale out, scale in). The set of types actually supported depends on the capabilities of the VNF being managed, as declared in the VNFD. | Obsolete OpenModelAttribute - isInvariant: false - support: MANDATORY |
| aspectId | ScalingAspect | 1 | Identifier of (reference to) the aspect of the VNF that is requested to be scaled, as declared in the VNFD. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| numberOfSteps | Integer | 0..1 | Number of scaling steps. It shall be a positive number. Defaults to 1. The VNF provider defines in the VNFD whether or not a particular VNF supports performing more than one step at a time. Such a property in the VNFD applies for all instances of a particular VNF. A scaling step is the smallest unit by which a VNF instance can be scaled w.r.t a particular scaling aspect. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| additionalParam | KeyValuePair | 0..* | Additional parameters passed by the OSS/BSS as input to the scaling process, specific to the VNF instance being scaled, as declared in the VNFD. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### ScaleNsByStepsData datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmDataModule::ScaleNsByStepsData

Description:

The ScaleNsByStepsData datatype describes the information needed to scale an NS instance by one or more scaling steps, with respect to a particular NS scaling aspect. Performing a scaling step means increasing/decreasing the capacity of an NS instance in a discrete manner, i.e. moving from one NS scale level to another. The NS scaling aspects and their corresponding NS scale levels applicable to the NS instance are declared in the NSD.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type ScaleNsByStepsData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| scalingDirection | NsScaleDirection | 1 | Specifies the scaling direction. Possible values: SCALE_IN, SCALE_OUT. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| aspectId | NsScalingAspect | 1 | Provides the aspect of the NS that is requested to be scaled, as declared in the NSD. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| numberOfSteps | Integer | 0..1 | Specifies the number of scaling steps to be performed. Defaults to 1. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### ScaleNsData  datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmDataModule::ScaleNsData

Description:

The ScaleNsData datatype describes the information needed to scale an NS instance either by explicitly adding /removing existing VNF instances or by leveraging on the abstraction mechanism provided by the NS scaling aspects and NS levels information elements declared in the NSD.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type ScaleNsData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vnfInstanceToBeAdded | VnfInstanceData | 0..* | Specifies an existing VNF instance to be added to the NS instance as part of the scaling operation. If needed, the VNF Profile to be used for this VNF instance is also provided. NOTE: The DF of the VNF instance shall match the VNF DF present in the associated VNF Profile of the new NS flavour. NOTE: This functionality is the same as the one provided by the Update NS operation when the AddVnf update type is selected | OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: No more than two attributes between vnfInstanceToBeAdded, vnfInstanceToBeRemoved, scaleNsByStepsData and scaleNsToLevelData shall be present. In case of two, the attributes shall be vnfInstanceToBeAdded and vnfInstanceToBeRemoved. |
| vnfInstanceToBeRemoved | Identifier | 0..* | Specifies a VNF instance to be removed from the NS instance as part of the scaling operation. NOTE: This functionality is the same as the one provided by the Update NS operation when the RemoveVnf update type is selected. | OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: No more than two attributes between vnfInstanceToBeAdded, vnfInstanceToBeRemoved, scaleNsByStepsData and scaleNsToLevelData shall be present. In case of two, the attributes shall be vnfInstanceToBeAdded and vnfInstanceToBeRemoved |
| scaleNsByStepsData | ScaleNsByStepsData | 0..1 | Specifies the information needed to scale an NS instance by one or more scaling steps. | OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: No more than two attributes between vnfInstanceToBeAdded, vnfInstanceToBeRemoved, scaleNsByStepsData and scaleNsToLevelData shall be present. In case of two, the attributes shall be vnfInstanceToBeAdded and vnfInstanceToBeRemoved. |
| scaleNsToLevelData | ScaleNsToLevelData | 0..1 | Specifies the information needed to scale an NS instance to a target size. | OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: No more than two attributes between vnfInstanceToBeAdded, vnfInstanceToBeRemoved, scaleNsByStepsData and scaleNsToLevelData shall be present. In case of two, the attributes shall be vnfInstanceToBeAdded and vnfInstanceToBeRemoved. |
| additionalParamForNs | KeyValuePair | 0..* | Allows the OSS/BSS to provide additional parameter(s) at the NS level necessary for the NS scaling (as opposed to the VNF level, which is covered in additionalParamForVnf). | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| additionalParamForVnf | ParamsForVnf | 0..* | Allows the OSS/BSS to provide additional parameter(s) per VNF instance (as opposed to the NS level, which is covered in additionalParamforNs). This is for VNFs that are to be created by the NFVO as part of the NS scaling and not for existing VNF that are covered by the scaleVnfData. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| locationConstraints | VnfLocationConstraint | 0..* | Defines the location constraints for the VNF to be instantiated as part of the NS scaling. An example can be a constraint for the VNF to be in a specific geographic location. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### ScaleNsToLevelData datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmDataModule::ScaleNsToLevelData

Description:

The ScaleNsToLevelData datatype describes the information needed to scale an NS instance to a target size. The target size is either expressed as an NS instantiation level or as a list of NS scale levels, one per NS scaling aspect, of the current DF. The NS instantiation levels, the NS scaling aspects and their corresponding NS scale levels applicable to the NS instance are declared in the NSD.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type ScaleNsToLevelData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| nsInstantiationLevel | NsLevel | 0..1 | Identifier of the target NS instantiation level of the current DF to which the NS instance is requested to be scaled. | OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: Either nsInstantiationLevel or nsScaleInfo, but not both, shall be present. PassedByReference |
| nsScaleInfo | NsScaleInfo | 0..* | For each NS scaling aspect of the current DF, defines the target NS scale level to which the NS instance is to be scaled. | OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: Either nsInstantiationLevel or nsScaleInfo, but not both, shall be present. |

###### ScaleToLevelData datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmDataModule::ScaleToLevelData

Description:

The ScaleToLevelData datatype describes the information needed to scale a VNF instance to a target size. The target size is either expressed as an instantiation level of that DF as defined in the VNFD, or given as a list of scale levels, one per scaling aspect of that DF. Instantiation levels and scaling aspects are declared in the VNFD. The NFVO shall then invoke the ScaleVnfToLevel operation towards the appropriate VNFM.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type ScaleToLevelData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| instantiationLevelId | InstantiationLevel | 0..1 | Identifier of (reference to) the target instantiation level of the current deployment flavour to which the VNF instance is requested to be scaled. | OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: Either instantiationLevelId or scaleInfo but not both shall be present. PassedByReference |
| scaleInfo | ScaleInfo | 0..* | For each scaling aspect of the current deployment flavour, defines the target scale level to which the VNF instance is to be scaled. The VNF provider defines in the VNFD whether or not a particular VNF supports scaling according to this parameter. Such a property in the VNFD applies for all instances of a particular VNF. | OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: Either instantiationLevelId or scaleInfo but not both shall be present. |
| additionalParam | KeyValuePair | 0..* | Additional parameters passed by the OSS/BSS as input to the scaling process, specific to the VNF instance being scaled, as declared in the VNFD. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### ScaleVnfData datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmDataModule::ScaleVnfData

Description:

This datatype describes the information needed, either to scale a VNF instance to a given level, or to scale a VNF instance by steps.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type ScaleVnfData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vnfInstanceId | Identifier | 1 | Identifier of the VNF instance being scaled. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| type | VnfScaleType | 1 | Defines the type of the scale VNF operation requested (scale out, scale in, scale to instantiation level, scale to scale level(s)). The set of types actually supported depends on the capabilities of the VNF being managed. NOTE: ETSI GS NFV-IFA 010 specifies that the lifecycle management operations that expand or contract a VNF instance include scale in, scale out, scale up and scale down. Vertical scaling (scale up, scale down) is not supported in the present document. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| scaleToLevelData | ScaleToLevelData | 0..1 | Provides the information needed when scaling to a given level. NOTE: Either scaletoLevelData or scaleByStepData but not both shall be present. The scaleByStepData is used for scale out/in type of scaling, and the scaleToLevelData is used for scale to instantiation/scale level type of scaling. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: Either scaletoLevelData or scaleByStepData but not both shall be present |
| scaleByStepData | ScaleByStepData | 0..1 | Provides the information needed when scaling by steps. NOTE: Either scaletoLevelData or scaleByStepData but not both shall be present. The scaleByStepData is used for scale out/in type of scaling, and the scaleToLevelData is used for scale to instantiation/scale level type of scaling. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: Either scaletoLevelData or scaleByStepData but not both shall be present. |

###### UpdateVnffgData datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmDataModule::UpdateVnffgData

Description:

This datatype specifies the parameters needed for the update of an existing VNFFG instance.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type UpdateVnffgData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vnffgId | VnffgInfo | 1 | Identifier of an existing VNFFG to be updated for the Network Service Instance. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| nfp | NfpData | 0..* | Indicate the desired new NFP(s) for a given VNFFG after the operations of addition/removal of NS components (e.g. VNFs, VLs, etc.) have been completed, or indicate the updated or newly created NFP classification and selection rule which applied to an existing NFP. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nfpId | NfpInfo | 0..* | Identifier(s) of the NFP to be deleted from a given VNFFG. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |

###### VnfInstanceData datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmDataModule::VnfInstanceData

Description:

The VnfInstanceData datatype specifies an existing VNF instances to be used in the NS instance and if needed, the VNF Profile to use for this VNF instance.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type VnfInstanceData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vnfInstanceId | Identifier | 1 | Identifier of the existing VNF instance to be used in the NS. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfProfileId | VnfProfile | 0..1 | Identifier of (rReference to) a vnfProfile defined in the NSD which the existing VNF instance shall be matched with. If not presents, the NFVO will select the VnfProfile matching the information in the VNF instance. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |

###### VnfLocationConstraint datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmDataModule::VnfLocationConstraint

Description:

The VnfLocationConstraint datatype defines the location constraints for the VNF to be instantiated.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type VnfLocationConstraint

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vnfdId | Vnfd | 1 | Identifier (reference to) of a VnfProfile in the NSD used to manage the lifecycle of the VNF instance. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| locationConstraints |  | 1 | Defines the location constraints for the VNF instance to be created | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfScaleByStepType enumeration

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmDataModule::VnfScaleByStepType

Description:

Defines the type of the scale operation requested (scale out, scale in). The set of types actually supported depends on the capabilities of the VNF being managed, as declared in the VNFD.

Applied Stereotypes:

Preliminary

Contains Enumeration Literals:

SCALE_OUT

SCALE_IN

###### VnfScaleType enumeration

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmDataModule::VnfScaleType

Description:

Defines the type of the scale VNF operation requested (scale out, scale in, scale up, scale down, scale to instantiation level, scale to scale level(s)).

Applied Stereotypes:

Preliminary

Contains Enumeration Literals:

SCALE_OUT

SCALE_IN

SCALE_UP

SCALE_DOWN

SCALE_TO_INSTANTIATION_LEVEL

SCALE_TO_SCALE_LEVEL

###### VnfStopType enumeration

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmDataModule::VnfStopType

Description:

It signals whether forceful or graceful stop is requested. Allowed values are: forceful and graceful.

Applied Stereotypes:

Preliminary

Contains Enumeration Literals:

FORCEFUL

GRACEFUL

### NsLcmModule

#### Overview

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmModule

Description:

Group all model elements related to NsInfo

##### Diagrams

Figure B.1: Diagram NS Info

Figure B.1: Diagram Connection Point Hierarchy

Figure B.1: Diagram NS Classes Mapping

##### Classes

###### CpInfo class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmModule::CpInfo

This class is abstract.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

###### NfpInfo class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmModule::NfpInfo

Description:

The Nfp class defines the information related to the NFP.

Applied Stereotypes:

Experimental

OpenModelClass

support:  MANDATORY

Table B.1: Attributes for Class NfpInfo

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| nfpId | Identifier | 1 | Identifier of this Nfp class. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nfpdId | Nfpd | 0..1 | Identifier of the NFPD used to instantiate this NFP. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| nfpName | String | 0..1 | Human readable name for the NFP. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| description | String | 0..1 | Human readable description for the NFP. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| cpHandle | CpInfo | 1..* | Identifier(s) of the CPs and/or SAPs which the NFP passes by. When multiple identifiers are included, the position of the identifier in the information element value specifies the position of CP or SAP in the path. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| totalCp | Integer | 0..1 | Total number of CPs in this NFP. | OpenModelAttribute - isInvariant: false - support: OPTIONAL |
| nfpRule | NfpRule | 1 | NFP classification and selection rule. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nfpState | OperationalState | 1 | An indication of whether the NFP is enabled or disabled. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsInfo class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmModule::NsInfo

Description:

This class provides run time information about an NS instance.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class NsInfo

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| nsInstanceId | Identifier | 1 | Identifier of this NsInfo class, identifying the NS instance. | OpenModelAttribute - isInvariant: true - support: MANDATORY |
| nsName | String | 1 | Human readable name of the NS instance. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| description | String | 1 | Human readable description of the NS instance | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nsdId | Nsd | 1 | Reference to the NSD associated with this NS. This is the NSD used to instantiate this NS or an NSD explicitly associated after instantiation. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| nsdinfoId | NsdInfo | 1 | Reference to the NSD information object associated with the NS. This identifier was allocated by the NFVO. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| flavourId | NsDf | 0..1 | Reference to the flavour of the NSD used to instantiate this NS. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vnfInfo | VnfInfo | 0..* | Information on constituent VNFs of this NS. Cardinality of zero is only valid for a non-instantiated NS. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| pnfInfo | PnfInfo | 0..* | Information on the PNF(s) that are part of this NS. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualLinkInfo | NsVirtualLinkInfo | 0..* | Information on the VLs of this NS. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnffgInfo | VnffgInfo | 0..* | Information on the VNFFGs of this NS. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| sapInfo | SapInfo | 0..* | Information on the SAPs of this NS. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nestedNsId | NsInfo | 0..* | Reference to information on nested NSs of this NS. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| instantiationState | InstantiationState | 1 | The state of the NS. Possible values: NOT_INSTANTIATED (i.e. the NS instance is not instantiated or terminated), INSTANTIATED (i.e. the NS instance is instantiated). | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nsScaleStatus | NsScaleInfo | 0..* | Represents for each NS scaling aspect declared in the applicable DF, how "big" the NS instance has been scaled w.r.t. that aspect. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| additionalAffinityOrAntiAffinityRule | AffinityOrAntiAffinityRule | 0..* | Information on the additional affinity or anti-affinity rule from NS instantiation operation. Shall not conflict with rules already specified in the NSD. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsLinkPortInfo class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmModule::NsLinkPortInfo

Description:

This class provides information about a port of an NS VL.

When the NsVirtualLinkInfo, from which the present NsLinkPortInfo is part of, is provided as an ExtVirtualLinkInfo as input of a VNF LCM operation, the id of the ExtLinkPortInfo shall be identical to the one of the corresponding NsLinkPortInfo.

Applied Stereotypes:

Experimental

OpenModelClass

support:  MANDATORY

Table B.1: Attributes for Class NsLinkPortInfo

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| nsLinkPortId | Identifier | 1 | Identifier of this link port. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| resourceHandle | ResourceHandle | 1 | Identifier(s) of the virtualised network resource(s) realizing this link port. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| cpId | CpInfo | 0..1 | CP connected to this link port. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsVirtualLinkInfo class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmModule::NsVirtualLinkInfo

Description:

This class provides run-time information about an NS VL instance.

As an NS can include NFs deployed in NFVI PoPs under the control of several different VIMs, deploying an NS VL can involve several VIMs each allocating different virtualised network resources.

When this NsVirtualLinkInfo is provided as an ExtVirtualLinkInfo as input of a VNF LCM operation, the id of the ExtVirtualLinkInfo shall be identical to the one of the corresponding NsVirtualLinkInfo.

NOTE: The connectivity between virtualized network resources allocated in different VIMs and part of the same VL is not addressed in the present document.

Applied Stereotypes:

Experimental

OpenModelClass

support:  MANDATORY

Table B.1: Attributes for Class NsVirtualLinkInfo

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| nsVirtualLinkInstanceId | Identifier | 1 | Identifier of this NsVirtualLinkInfo information element, identifying the NS VL instance. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nsVirtualLinkDescId | NsVirtualLinkDesc | 1 | Identifier of the VLD in the Network Service descriptor for this Virtual Link. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| resourceHandle | ResourceHandle | 1..* | Identifier(s) of the virtualised network resource(s) realizing this VL. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nslinkport | NsLinkPortInfo | 0..* | Link ports of this VL. Cardinality of zero indicates that no port has yet been created for this VL. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### PnfExtCpInfo class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmModule::PnfExtCpInfo

Description:

This class provides information about the external CP of the PNF.

Parent class: CpInfo

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class PnfExtCpInfo

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| cpInstanceId | Identifier | 1 | Identifier of this external CP instance and of this PnfExtCpInfo information element | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| cpdId | Cpd | 1 | Identifier of (reference to) the Connection Point Descriptor (CPD) for this CP | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| address |  | 1 | Address for this CP NOTE: The address information shall be compatible with the type and sub-type attribute of the CPD. | Obsolete OpenModelAttribute - isInvariant: false - support: MANDATORY |
| cpProtocolInfo | CpProtocolInfo | 1..* | Protocol information for this CP. There shall be one cpProtocolInfo for each layer protocol supported. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### PnfInfo class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmModule::PnfInfo

Description:

This class provides information about a PNF that is part of an NS instance.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class PnfInfo

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| pnfId | Identifier | 1 | Identifier of the PNF. Assigned by OSS and provided to NFVO. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| pnfName | String | 1 | Name of the PNF | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| pnfdId | Pnfd | 1 | Identifier of the PNFD. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| pnfdInfoId | PnfdInfo | 1 | Identifier of (reference to) the PNFD related to this PNF | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| pnfProfileId | PnfProfile | 1 | Identifier of (reference to) the PNF Profile to be used for this PNF. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| cpInfo | PnfExtCpInfo | 1..* | Information on the external CP of the PNF | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### SapInfo class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmModule::SapInfo

Description:

This class provides information about an SAP of an NS instance.

Parent class: CpInfo

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class SapInfo

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| sapInstanceId | Identifier | 1 | Identifier of this SapInfo class, identifying the SAP instance. | OpenModelAttribute - isInvariant: true - support: MANDATORY |
| sapdId | Sapd | 1 | Reference to the SAP Descriptor for this SAP. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| sapName | String | 1 | Human readable name for the SAP. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| description | String | 1 | Human readable description for the SAP. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| address |  | 1 | Address for this SAP. In some cases, the NFVO provides the address. The address information shall be compatible with the type and sub-type attribute of the SAPD. | Obsolete OpenModelAttribute - isInvariant: false - support: MANDATORY |
| cpProtocolInfo | CpProtocolInfo | 1..* | Protocol information for this SAP. There shall be one cpProtocolInfo for each layer protocol supported. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnffgInfo class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmModule::VnffgInfo

Description:

This class contains information about a VNFFG instance.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VnffgInfo

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vnffgId | Identifier | 1 | Identifier of the Vnffg class. | OpenModelAttribute - isInvariant: true - support: MANDATORY |
| vnffgdId | Vnffgd | 1 | Identifier of the VNFFGD used to instantiate this VNFFG. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vnfId | VnfInfo | 1..* | Identifier(s) of the constituent VNF instance(s) of the VNFFG. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| pnfId | PnfInfo | 0..* | Identifier(s) of the constituent PNF instance(s) of the VNFFG. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| virtualLinkId | NsVirtualLinkInfo | 1..* | Identifier(s) of the constituent VL instance(s) of the VNFFG. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| cpId | CpInfo | 1..* | Identifiers of the CP instances attached to the constituent VNFs and PNFs or the sap instances of the VNFFG. It indicates an exhaustive list of all the CP instances and SAP instances of the VNFFG. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| nfp | NfpInfo | 1..* | Information on the NFPs of this VNFFG. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

##### Datatypes

###### AddPnfData datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmModule::AddPnfData

Description:

This information element provides input information about PNF which needs to be added into an NS instance.

Table B.1: Attributes for Data Type AddPnfData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| pnfId | Identifier | 1 | Identifier of the PNF. Assigned by OSS and provided to NFVO. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| pnfName | String | 1 | Human readable name of the PNF. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| pnfdId | Pnfd | 1 | Identifier of (reference to) the PNFD related to this PNF. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| pnfProfileId | PnfProfile | 1 | Identifier of (reference to) the PNF Profile to be used for this PNF. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| cpData |  | 0..* | Information on the external CP of the PNF. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### ModifyPnfData datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmModule::ModifyPnfData

Description:

This information element provides input information about PNFs which need to be modified in an NS instance.

Table B.1: Attributes for Data Type ModifyPnfData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| pnfId | Identifier | 1 | Identifier of the PNF. Assigned by OSS and provided to NFVO. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| pnfName | String | 0..1 | Human readable name of the PNF. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| cpData |  | 0..* | Information on the external CP of the PNF. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NfpRule datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmModule::NfpRule

Description:

The NfpRule information element is an expression of the conditions that must be met for the NFP to be applicable to the packet.

The condition acts as a flow classifier and it is met only if all the values expressed in the condition are matched by those in the packet.

Applied Stereotypes:

Experimental

Table B.1: Attributes for Data Type NfpRule

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| etherType | IpVersion | 0..1 | Indicates the protocol carried over the Ethernet layer. Possible values: IPv4, IPv6. | OpenModelAttribute - isInvariant: false - support: CONDITIONAL_MANDATORY - condition: Note: At least one attribute must be present. |
| protocol | String | 0..1 | Indicates the L4 protocol, e.g. "TCP", "UDP", etc. | OpenModelAttribute - isInvariant: false - support: CONDITIONAL_MANDATORY - condition: Note: At least one attribute must be present. |
| sourcePortRange | PortRange | 0..1 | Indicates a range of source ports. | OpenModelAttribute - isInvariant: false - support: CONDITIONAL_MANDATORY - condition: Note: At least one attribute must be present. |
| destinationPortRange | PortRange | 0..1 | Indicates a range of destination ports. | OpenModelAttribute - isInvariant: false - support: CONDITIONAL_MANDATORY - condition: Note: At least one attribute must be present. |
| sourceIPAddressPrefix | IpAddress | 0..1 | Indicates the source IP address or prefix. | OpenModelAttribute - isInvariant: false - support: CONDITIONAL_MANDATORY - condition: Note: At least one attribute must be present. |
| destinationIPAddressPrefix | IpAddress | 0..1 | Indicates the destination IP address or prefix. | OpenModelAttribute - isInvariant: false - support: CONDITIONAL_MANDATORY - condition: Note: At least one attribute must be present. |

###### NsScaleInfo datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmModule::NsScaleInfo

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type NsScaleInfo

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| nsScalingAspectId | NsScalingAspect | 1 | Identifier of the NS scaling aspect. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| nsScaleLevelId | NsLevel | 1 | Identifier of the NS scale level. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |

###### PnfExtCpData datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmModule::PnfExtCpData

Description:

This information element provides input information about the external CP of the PNF.

Applied Stereotypes:

Experimental

Table B.1: Attributes for Data Type PnfExtCpData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| cpInstanceId | Identifier | 0..1 | Identifier of this external CP instance. Shall be present for existing CP. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| cpdId | Cpd | 0..1 | Identifier of (reference to) the Connection Point Descriptor (CPD) for this CP. Shall be present for new CP | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| address |  | 1..* | Address for this CP, including the information on applicable layer protocol(s). NOTE: The address information shall be compatible with the layerProtocol values defined in the CPD. In case of an IP address, a port number may be included. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### PortRange datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmModule::PortRange

Description:

The PortRange information element provides the lower and upper bounds of a range of Internet ports.

Applied Stereotypes:

Experimental

Table B.1: Attributes for Data Type PortRange

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| lowerPort | Integer | 1 | Identifies the lower bound of the port range. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| upperPort | Integer | 1 | Identifies the upper bound of the port range. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

##### Associations

###### HsHasNestedNs association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmModule::HsHasNestedNs

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association HsHasNestedNs

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| nestedNsId | shared | Navig. | 0..* | NsInfo | Reference to information on nested NSs of this NS. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| compositeNs | none | Not navig. | 0..* | NsInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NSDeployedUsingNsd association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmModule::NSDeployedUsingNsd

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association NSDeployedUsingNsd

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| nsdId | none | Navig. | 1 | Nsd | Reference to the NSD associated with this NS. This is the NSD used to instantiate this NS or an NSD explicitly associated after instantiation. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| nsInfo | none | Not navig. | 0..* | NsInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NfpInfoChainsCps association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmModule::NfpInfoChainsCps

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association NfpInfoChainsCps

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| cpHandle | shared | Navig. | 1..* | CpInfo | Identifier(s) of the CPs and/or SAPs which the NFP passes by. When multiple identifiers are included, the position of the identifier in the information element value specifies the position of CP or SAP in the path. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| nfpInfo | none | Not navig. | 0..* | NfpInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NfpInfoCreatedUsingNfpd association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmModule::NfpInfoCreatedUsingNfpd

Applied Stereotypes:

Experimental

Table B.1: Member ends for Association NfpInfoCreatedUsingNfpd

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| nfpdId | none | Navig. | 0..1 | Nfpd | Identifier of the NFPD used to instantiate this NFP. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| nfpinfo | none | Not navig. | 0..* | NfpInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsHasAssociatedNsdInfo association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmModule::NsHasAssociatedNsdInfo

Applied Stereotypes:

Experimental

Table B.1: Member ends for Association NsHasAssociatedNsdInfo

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| nsdinfoId | none | Navig. | 1 | NsdInfo | Reference to the NSD information object associated with the NS. This identifier was allocated by the NFVO. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nsinfo | none | Not navig. | 0..* | NsInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsHasConstituentPnfs association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmModule::NsHasConstituentPnfs

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association NsHasConstituentPnfs

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| pnfInfo | composite | Navig. | 0..* | PnfInfo | Information on the PNF(s) that are part of this NS. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nsInfo | none | Not navig. | 1 | NsInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsHasConstituentVls association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmModule::NsHasConstituentVls

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association NsHasConstituentVls

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| virtualLinkInfo | composite | Navig. | 0..* | NsVirtualLinkInfo | Information on the VLs of this NS. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nsInfo | none | Not navig. | 1 | NsInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsHasConstituentVnfs association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmModule::NsHasConstituentVnfs

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association NsHasConstituentVnfs

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| vnfInfo | composite | Navig. | 0..* | VnfInfo | Information on constituent VNFs of this NS. Cardinality of zero is only valid for a non-instantiated NS. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nsInfo | none | Not navig. | 0..* | NsInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsHasSaps association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmModule::NsHasSaps

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association NsHasSaps

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| sapInfo | composite | Navig. | 0..* | SapInfo | Information on the SAPs of this NS. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nsInfo | none | Not navig. | 1 | NsInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsHasVnfFgs association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmModule::NsHasVnfFgs

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association NsHasVnfFgs

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| vnffgInfo | composite | Navig. | 0..* | VnffgInfo | Information on the VNFFGs of this NS. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nsInfo | none | Not navig. | 1 | NsInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsLinkPortTerminateOnCP association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmModule::NsLinkPortTerminateOnCP

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association NsLinkPortTerminateOnCP

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| cpId | none | Navig. | 0..1 | CpInfo | CP connected to this link port. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nslinkport | none | Not navig. | 0..1 | NsLinkPortInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsVLInfoDeployedUsingNsVld association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmModule::NsVLInfoDeployedUsingNsVld

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association NsVLInfoDeployedUsingNsVld

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| nsVirtualLinkDescId | none | Navig. | 1 | NsVirtualLinkDesc | Identifier of the VLD in the Network Service descriptor for this Virtual Link. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| nsVlInfo | none | Not navig. | 0..* | NsVirtualLinkInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsVlHasLinkPort association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmModule::NsVlHasLinkPort

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association NsVlHasLinkPort

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| nslinkport | none | Navig. | 0..* | NsLinkPortInfo | Link ports of this VL. Cardinality of zero indicates that no port has yet been created for this VL. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nsvirtuallinkinfo | none | Not navig. | 1 | NsVirtualLinkInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### PnfExtCpCreatedUsingCpd association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmModule::PnfExtCpCreatedUsingCpd

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association PnfExtCpCreatedUsingCpd

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| cpdId | none | Navig. | 1 | Cpd | Identifier of (reference to) the Connection Point Descriptor (CPD) for this CP | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| pnfExternalCpInfo | none | Not navig. | 0..* | PnfExtCpInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### PnfInfoCreatedUsingPnfProfile association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmModule::PnfInfoCreatedUsingPnfProfile

Applied Stereotypes:

Experimental

Table B.1: Member ends for Association PnfInfoCreatedUsingPnfProfile

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| pnfProfileId | none | Navig. | 1 | PnfProfile | Identifier of (reference to) the PNF Profile to be used for this PNF. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| pnfInfo | none | Not navig. | 0..* | PnfInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### PnfInfoCreatedUsingPnfd association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmModule::PnfInfoCreatedUsingPnfd

Applied Stereotypes:

Experimental

Table B.1: Member ends for Association PnfInfoCreatedUsingPnfd

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| pnfdId | none | Navig. | 1 | Pnfd | Identifier of the PNFD. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| pnfinfo | none | Not navig. | 0..* | PnfInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### PnfInfoCreatedUsingPnfdInfo association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmModule::PnfInfoCreatedUsingPnfdInfo

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association PnfInfoCreatedUsingPnfdInfo

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| pnfdInfoId | none | Navig. | 1 | PnfdInfo | Identifier of (reference to) the PNFD related to this PNF | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| pnfInfo | none | Not navig. | 0..* | PnfInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### PnfInfoHasExtCpInfo association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmModule::PnfInfoHasExtCpInfo

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association PnfInfoHasExtCpInfo

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| cpInfo | composite | Navig. | 1..* | PnfExtCpInfo | Information on the external CP of the PNF | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| pnfInfo | none | Not navig. | 1 | PnfInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### SapInfoCreatedUsingSapd association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmModule::SapInfoCreatedUsingSapd

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association SapInfoCreatedUsingSapd

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| sapdId | none | Navig. | 1 | Sapd | Reference to the SAP Descriptor for this SAP. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| sapInfo | none | Not navig. | 0..* | SapInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfFgHasConstituentCps association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmModule::VnfFgHasConstituentCps

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfFgHasConstituentCps

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| cpId | shared | Navig. | 1..* | CpInfo | Identifiers of the CP instances attached to the constituent VNFs and PNFs or the sap instances of the VNFFG. It indicates an exhaustive list of all the CP instances and SAP instances of the VNFFG. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vnffgInfo | none | Not navig. | 0..* | VnffgInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfFgInfoHasConsituentPnfs association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmModule::VnfFgInfoHasConsituentPnfs

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfFgInfoHasConsituentPnfs

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| pnfId | shared | Navig. | 0..* | PnfInfo | Identifier(s) of the constituent PNF instance(s) of the VNFFG. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vnffgInfo | none | Not navig. | 0..* | VnffgInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfFgInfoHasConstituentVls association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmModule::VnfFgInfoHasConstituentVls

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfFgInfoHasConstituentVls

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| virtualLinkId | shared | Navig. | 1..* | NsVirtualLinkInfo | Identifier(s) of the constituent VL instance(s) of the VNFFG. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vnffgInfo | none | Not navig. | 0..* | VnffgInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfFgInfoHasConstituentVnfs association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmModule::VnfFgInfoHasConstituentVnfs

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfFgInfoHasConstituentVnfs

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| vnfId | shared | Navig. | 1..* | VnfInfo | Identifier(s) of the constituent VNF instance(s) of the VNFFG. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vnffgInfo | none | Not navig. | 0..* | VnffgInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfFgInfoHasNfps association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmModule::VnfFgInfoHasNfps

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfFgInfoHasNfps

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| nfp | composite | Navig. | 1..* | NfpInfo | Information on the NFPs of this VNFFG. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnffgInfo | none | Not navig. | 1 | VnffgInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnffgCreatedUsingVnffgd association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLcmModule::VnffgCreatedUsingVnffgd

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnffgCreatedUsingVnffgd

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| vnffgdId | none | Navig. | 1 | Vnffgd | Identifier of the VNFFGD used to instantiate this VNFFG. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vnffgInfo | none | Not navig. | 0..* | VnffgInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

### NsLifecycleChangeNotificationModule

#### Overview

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLifecycleChangeNotificationModule

##### Diagrams

Figure B.1: Diagram NS Lifecycle Change notifications

##### Notifications

###### NsChangeNotification notification

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLifecycleChangeNotificationModule::NsChangeNotification

Description:

This notification informs the receiver of changes on the NS instance caused by the LCM operation occurrence, which directly or indirectly impacts its NS component and is triggered without any context of this NS instance.

This notification is different from the NsLcmOperationOccurenceNotification, which is triggered by the LCM operation occurrence on the NS instance itself.

If this is a notification about the start of an LCM operation occurrence impacting the NS component, the notification shall be provided as soon as the impact on the NS component is identified.

If this is a notification about the result of an LCM operation occurrence impacting the NS component, the notification shall be provided after the impact on the NS component has been executed.

Applied Stereotypes:

Experimental

OpenModelNotification

triggerConditionList:

LCM operation occurrence which directly or indirectly impacts the NS component (start and result)

support:    MANDATORY

Table B.1: Attributes for Signal NsChangeNotification

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| nsInstanceId | Identifier | 1 | Identifier of the NS instance affected. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nsComponentType |  | 1 | Indicates the affected NS component type. Permitted values include: VNF, nested NS, PNF. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nsComponentId | Identifier | 1 | Identifier of the affected NS component instance. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| lcmOpOccIdImpactingNsComponent | Identifier | 1 | Identifier of the lifecycle management operation occurrence impacting the NS component and associated to this notification. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| lcmOpOccNameImpactingNsComponent | String | 1 | Name of the lifecycle management operation occurrence impacting the NS component and associated to this notification. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| lcmOpOccStatusImpactingNsComponent |  | 1 | Status of the lifecycle management operation occurrence impacting the NS component and associated to this notification. Indicates whether this notification reports about the start of a lifecycle operation occurrence or the final result of a lifecycle operation occurrence, e.g. start, completed, failed, etc. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsIdentifierCreationNotification notification

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLifecycleChangeNotificationModule::NsIdentifierCreationNotification

Description:

This notification informs the receiver of the creation of a new NS instance identifier and of the associated instance of an NsInfo class, identified by that identifier.

Applied Stereotypes:

OpenModelNotification

triggerConditionList:

Creation of an NS instance identifier and of the associated instance of an NsInfo class

support:    MANDATORY

Preliminary

Table B.1: Attributes for Signal NsIdentifierCreationNotification

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| nsInstanceId | Identifier | 1 | The newly created NS instance identifier | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsIdentifierDeletionNotification notification

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLifecycleChangeNotificationModule::NsIdentifierDeletionNotification

Description:

This notification informs the receiver of the deletion of an NS instance identifier and of the associated instance of an NsInfo class identified by that identifier.

Applied Stereotypes:

OpenModelNotification

triggerConditionList:

Deletion of an NS instance identifier and of the associated instance of a NSInfo class

support:    MANDATORY

Preliminary

Table B.1: Attributes for Signal NsIdentifierDeletionNotification

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| nsInstanceId | Identifier | 1 | The NS instance identifier to be deleted. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsLcmOperationOccurrenceNotification notification

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLifecycleChangeNotificationModule::NsLcmOperationOccurrenceNotification

Description:

This notification informs the receiver of changes in the NS lifecycle caused by NS lifecycle management operation occurrences,which may be manually triggered by the OSS/BSS or automatically triggered by the NFVO.

The automatic trigger inside the NFVO includes auto-scaling, auto-healing and impact on the nested NS instances triggered by the NS lifecycle operation on its composite NS.

If this is a notification about the start of an LCM operation occurrence, the notification shall be sent before any action is taken, however, after acknowledging the LCM operation request to the consumer.

If this is a notification about the result of an LCM operation, the notification shall be sent after all other actions of the LCM operation have been executed.

Applied Stereotypes:

Experimental

OpenModelNotification

triggerConditionList:

Instantiation of the NS (start and result)

Scaling of the NS (start and result, including the auto-scaling)

Update of the NS (start and result)

Termination of the NS (start and result)

Healing of the NS (start and result, including the auto-healing)

Impact on the nested NS instances triggered by the NS lifecycle operation on its composite NS

support:    MANDATORY

Table B.1: Attributes for Signal NsLcmOperationOccurrenceNotification

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| nsInstanceId | Identifier | 1 | Identifier of the NS instance affected. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| lifecycleOperationOccurrenceId | Identifier | 1 | Identifier of the NS lifecycle operation occurrence associated to the notification. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| operation | String | 1 | The lifecycle operation. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| notificationType | OperationStatus | 1 | Indicates whether this notification reports about the start of a lifecycle operation or the result of a lifecycle operation. | Obsolete OpenModelAttribute - isInvariant: false - support: MANDATORY |
| status | OperationStatus | 1 | Indicates whether this notification reports about the start of a lifecycle operation occurrence or the result of a lifecycle management operation occurrence. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| isAutomaticInvocation | Boolean | 1 | Set to true if the NS lifecycle operation occurrence has been automatically triggered by the NFVO. The automatic trigger inside the NFVO includes auto-scaling, auto-healing and impact on the nested NS instances triggered by the NS lifecycle operation on its composite NS. Set to false otherwise. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| affectedVnf | AffectedVnf | 0..* | Information about the VNF instances that were affected during the lifecycle operation, if this notification represents the result of a lifecycle operation. | OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: If the notification respresents the succesful result of a lifecycle operation, at least an affectedVnf, or affectedPnf, or affectedVl, or affectedVnffg or affectedNs, or affectedSap shall be present. |
| affectedPnf | AffectedPnf | 0..* | Information about the PNF instances that were affected during the lifecycle operation, if this notification represents the result of a lifecycle operation. | OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: If the notification respresents the succesful result of a lifecycle operation, at least an affectedVnf, or affectedPnf, or affectedVl, or affectedVnffg or affectedNs, or affectedSap shall be present. |
| affectedVl | AffectedVirtualLink | 0..* | Information about the VL instances that were affected during the lifecycle operation, if this notification represents the result of a lifecycle operation. | OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: If the notification respresents the succesful result of a lifecycle operation, at least an affectedVnf, or affectedPnf, or affectedVl, or affectedVnffg or affectedNs, or affectedSap shall be present. |
| affectedVnffg | AffectedVnffg | 0..* | Information about the VNFFG instances that were affected during the lifecycle operation, if this notification represents the result of a lifecycle operation. | OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: If the notification respresents the succesful result of a lifecycle operation, at least an affectedVnf, or affectedPnf, or affectedVl, or affectedVnffg or affectedNs, or affectedSap shall be present. |
| affectedNs | AffectedNs | 0..* | Information about the nested NS instances that were affected during the lifecycle operation, if this notification represents the result of a lifecycle operation. | OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: If the notification respresents the succesful result of a lifecycle operation, at least an affectedVnf, or affectedPnf, or affectedVl, or affectedVnffg or affectedNs, or affectedSap shall be present. |
| affectedSap | AffectedSap | 0..* | Information about the SAP instances that were affected during the lifecycle operation, if this notification represents the result of a lifecycle operation. | OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: If the notification respresents the succesful result of a lifecycle operation, at least an affectedVnf, or affectedPnf, or affectedVl, or affectedVnffg or affectedNs, or affectedSap shall be present. |

##### Datatypes

###### AffectedNs datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLifecycleChangeNotificationModule::AffectedNs

Description:

This datatype provides information about affected nested NSs.

Applied Stereotypes:

Experimental

Table B.1: Attributes for Data Type AffectedNs

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| nsInstanceId | Identifier | 1 | Identifier of the nested NS instance. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nsdId | Nsd | 1 | Identifier of the NSD of the nested NS instance. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| changeType | NestedNsChangeType | 1 | Signals the type of lifecycle change (instantiated, scaled, updated, healed, terminated). | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| changeResult | ChangeResultType | 1 |  | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### AffectedPnf datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLifecycleChangeNotificationModule::AffectedPnf

Description:

This datatype provides information about affected PNFs from an NS.

Applied Stereotypes:

Experimental

Table B.1: Attributes for Data Type AffectedPnf

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| pnfId | Identifier | 1 | Identifier of the PNF. Assigned by OSS. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| pnfName | String | 1 | Human readable name of the PNF. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| pnfdId | Pnfd | 1 | Identifier of the PNFD | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| pnfProfileId | PnfProfile | 1 | Identifier of the PNF profile of the NSD. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| cpInstanceId | PnfExtCpInfo | 1..* | Identifier of the affected PNF external CP instance. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| changeType | PnfChangeType | 1 | Signals the type of lifecycle change. Permitted values include: added, modified and removed. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| changeResult | ChangeResultType | 1 | Signal the result of lifecycle change, e.g., completed, failed, etc. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### AffectedSap datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLifecycleChangeNotificationModule::AffectedSap

Description:

This datatype provides information about affected SAP of an NS.

Applied Stereotypes:

Experimental

Table B.1: Attributes for Data Type AffectedSap

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| sapInstanceId | Identifier | 1 | Identifier of this SapInfo class, identifying the SAP instance. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| sapdId | Sapd | 1 | Reference to the SAPD for this SAP. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| sapName | String | 1 | Human readable name for the SAP. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| changeType | SapChangeType | 1 | Signals the type of lifecycle change of a SAP. Permitted values include: added, removed, modified. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| changeResult | ChangeResultType | 1 | Signal the result of lifecycle change, e.g., completed, failed, etc. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### AffectedVirtualLink datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLifecycleChangeNotificationModule::AffectedVirtualLink

Description:

This datatype provides information about affected VLs of an NS.

Applied Stereotypes:

Experimental

Table B.1: Attributes for Data Type AffectedVirtualLink

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| nsVirtualLinkId | Identifier | 1 | Identifier of the VL instance. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nsVirtualLinkDescId | NsVirtualLinkDesc | 1 |  | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vlProfileId | VirtualLinkProfile | 1 | Identifier of the VL profile of the NSD. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| changeType | VirtualLinkChangeType | 1 | Signals the type of lifecycle change. Permitted values include: added, delete, modified, link port added, link port removed. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| changeResult | ChangeResultType | 1 | Signal the result of lifecycle change, e.g., completed, failed, etc. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### AffectedVnf datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLifecycleChangeNotificationModule::AffectedVnf

Description:

This datatype provides information about affected VNF instances.

Applied Stereotypes:

Experimental

Table B.1: Attributes for Data Type AffectedVnf

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vnfInstanceId | Identifier | 1 | Identifier of the VNF instance. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfdId | Vnfd | 1 | Identifier of the VNFD of the VNF instance | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vnfProfileId | VnfProfile | 1 | Identifier of the VNF profile of the NSD. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vnfName | String | 1 | Name of the VNF instance | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| changeType | VnfChangeType | 1 | Signals the type of lifecycle change (instantiated, terminated, scaled, flavour changed, healed, operated, information/configuration modified). | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| changeResult | ChangeResultType | 1 | Signal the result of lifecycle change, e.g., completed, failed, etc. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| changedInfo |  | 0..1 | Information about the changed VNF instance information, including VNF configurable properties, if applicable. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### AffectedVnffg datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLifecycleChangeNotificationModule::AffectedVnffg

Description:

This datatype provides information about affected VNFFG instances.

Applied Stereotypes:

Experimental

Table B.1: Attributes for Data Type AffectedVnffg

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vnffgId | Identifier | 1 | Identifier of the VNFFG instance. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnffgdId | Vnffgd | 1 | Identifier of the VNFFGD of the VNFFG instance. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| changeType | VnffgChangeType | 1 | Signals the type of lifecycle change (added, removed, modified). NOTE: CP or NFP information might be modified for the VNFFG. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| changeResult | ChangeResultType | 1 | Signal the result of lifecycle change, e.g., completed, failed, etc. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### ChangeResultType enumeration

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLifecycleChangeNotificationModule::ChangeResultType

Description:

Signal the result of lifecycle change, e.g., completed, failed, etc

Applied Stereotypes:

Experimental

###### NestedNsChangeType enumeration

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLifecycleChangeNotificationModule::NestedNsChangeType

Description:

Signals the type of lifecycle change of a nested NS.

Applied Stereotypes:

Experimental

Contains Enumeration Literals:

ADD

REMOVE

INSTANTIATE

TERMINATE

SCALE

HEAL

UPDATE

###### PnfChangeType enumeration

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLifecycleChangeNotificationModule::PnfChangeType

Description:

Signals the type of lifecycle change for a PNF.

Applied Stereotypes:

Experimental

Contains Enumeration Literals:

ADD

MODIFY

REMOVE

###### SapChangeType enumeration

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLifecycleChangeNotificationModule::SapChangeType

Description:

Signals the type of lifecycle change of a SAP.

Applied Stereotypes:

Experimental

Contains Enumeration Literals:

ADD

REMOVE

MODIFY

###### VirtualLinkChangeType enumeration

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLifecycleChangeNotificationModule::VirtualLinkChangeType

Description:

Signals the type of lifecycle change for a VL.

Applied Stereotypes:

Experimental

Contains Enumeration Literals:

ADD

DELETE

MODIFY

ADD_LINK_PORT

REMOVE_LINK_PORT

###### VnfChangeType enumeration

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLifecycleChangeNotificationModule::VnfChangeType

Description:

Signals the type of lifecycle change of a VNF.

Applied Stereotypes:

Experimental

Contains Enumeration Literals:

ADD

REMOVE

INSTANTIATE

TERMINATE

SCALE

HEAL

OPERATE

MODIFY_INFORMATION

CHANGE_FLAVOUR

CHANGE_EXT_VNF_CONNECTIVITY

###### VnffgChangeType enumeration

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsLifecycleChangeNotificationModule::VnffgChangeType

Description:

Signals the type of lifecycle change of a VNF FG.

Applied Stereotypes:

Experimental

Contains Enumeration Literals:

ADD

REMOVE

MODIFY

### NsdModule

#### Overview

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsdModule

##### Diagrams

Figure B.1: Diagram NSD Information

##### Classes

###### NsdInfo class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsdModule::NsdInfo

Description:

This class provides the details of an  NsdInfo information element.

Applied Stereotypes:

Experimental

OpenModelClass

support:  MANDATORY

Table B.1: Attributes for Class NsdInfo

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| nsdInfoId | Identifier | 1 | Identifier of the NSD information object. | Experimental OpenModelAttribute - isInvariant: true - support: MANDATORY |
| nsdId | Identifier | 0..1 | Identifier of the on-boarded NSD. NOTE: This attribute shall be present after the NSD is on-boarded. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| name | String | 0..1 | Name of the on-boarded NSD. NOTE: This attribute shall be present after the NSD is on-boarded. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| version | Version | 0..1 | Version of the on-boarded NSD. NOTE: This attribute shall be present after the NSD is on-boarded. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| designer | String | 0..1 | Designer of the on-boarded NSD. NOTE: This attribute shall be present after the NSD is on-boarded. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nsdInvariantId | Identifier | 0..1 | Identifies an NSD in a version independent manner. This attribute is invariant across versions of NSD. NOTE: This attribute is available after the NSD is on-boarded. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| nsd | Nsd | 0..1 | Reference to the on-boarded NSD, e.g. URL to the on-boarded NSD. NOTE: This attribute shall be present after the NSD is on-boarded. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vnfPackageInfoId | VnfPkgInfo | 0..* | Identifies the VnfPkgInfo objects for the VNFD referenced by the on-boarded NSD. NOTE: This attribute shall be present after the NSD is on-boarded. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| pnfdInfoId | PnfdInfo | 0..* | Identifies the PNFD information objects for the PNFD referenced by the on-boarded NSD. NOTE: This attribute shall be present after the NSD is on-boarded. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| nestedNsdInfoId | NsdInfo | 0..* | Identifies the NSD information objects for the nested NSD referenced by the on-boarded NSD. NOTE: This attribute shall be present after the NSD is on-boarded. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| previousNsdVersionId | NsdInfo | 0..1 | Reference to the previous version(s) if any of this NSD. This can be recursive if more than one previous version. | Obsolete OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| onboardingState | OnboardingState | 1 | On-boarding state of the NSD. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| operationalState | OperationalState | 1 | Operational state of the NSD. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| usageState | UsageState | 1 | Usage state of the NSD. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| deletionPending | Boolean | 1 | Indicates if deletion of this instance of the NSD has been requested but the NSD is still being used by instantiated NSs. This instance of the NSD will be deleted once all NSs instantiated from this descriptor are terminated. | Obsolete OpenModelAttribute - isInvariant: false - support: MANDATORY |
| userDefinedData | KeyValuePair | 1 | User defined data for the associated NSD. | Experimental OpenModelAttribute - isInvariant: false - support: OPTIONAL |

###### PnfdInfo class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsdModule::PnfdInfo

Description:

This class provides the details of a PNFD.

Applied Stereotypes:

Experimental

OpenModelClass

support:  MANDATORY

Table B.1: Attributes for Class PnfdInfo

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| pnfdInfoId | Identifier | 1 | Identifier of the on-boarded instance of the PNFD. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| pnfdId | Identifier | 0..1 | Identifier of the on-boarded PNFD. NOTE: This attribute shall be present after the PNFD is on-boarded. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| name | String | 0..1 | Name of the on-boarded PNFD. NOTE: This attribute shall be present after the PNFD is on-boarded. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| version | Version | 0..1 | Version of the on-boarded PNFD. NOTE: This attribute shall be present after the PNFD is on-boarded. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| provider | String | 0..1 | Provider of the on-boarded PNFD. NOTE: This attribute shall be present after the PNFD is on-boarded. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| pnfdInvariantId | Identifier | 1 | Identifies a PNFD in a version independent manner. This attribute is invariant across versions of PNFD. NOTE: This attribute shall be present after the PNFD is on-boarded. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| pnfd | Pnfd | 0..1 | Reference to the on-boarded PNFD, e.g. URL to the on-boarded PNFD. NOTE: This attribute shall be present after the PNFD is on-boarded. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| previousPnfdVersionId | PnfdInfo | 0..1 | Reference to the previous version) if any of this PNFD. This can be recursive if more than one previous version. | Obsolete OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| onboardingState | OnboardingState | 1 | On-boarding state of the PNFD. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| usageState | UsageState | 1 | Usage state of the PNFD. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| deletionPending | Boolean | 1 | Indicates if deletion of this instance of the PNFD has been requested but the PNFD is still being used by instantiated NSs. This instance of the PNFD will be deleted once all instantiated NSs using this descriptor are terminated, | Obsolete OpenModelAttribute - isInvariant: false - support: MANDATORY |
| userDefinedData | KeyValuePair | 0..* | User defined data for the PNFD. | OpenModelAttribute - isInvariant: false - support: OPTIONAL |

##### Notifications

###### NsdChangeNotification  notification

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsdModule::NsdChangeNotification

Description:

This notification indicates a change of state in an on-boarded NSD.

Only a change in operational state will be reported. A change in usage state is not reported.

Applied Stereotypes:

Experimental

OpenModelNotification

triggerConditionList:

Change of the operational state of an on-boarded NSD

support:    MANDATORY

Table B.1: Attributes for Signal NsdChangeNotification

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| nsdInfoId | NsdInfo | 1 | Identifier of the NSD information object. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| nsdId | Nsd | 1 | Identifies the on-boarded NSD. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| changeType | NsdInfoChangeType | 1 | It categorizes the type of change. Possible values can be "change of operational state of an on-boarded NSD", "NSD in deletion pending", and "deletion of an NSD". | Obsolete OpenModelAttribute - isInvariant: false - support: MANDATORY |
| operationalState | OperationalState | 0..1 | New operational state of the on-boarded NSD. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: Only present when changeType is "change of operational state". |
| deletionPending | Boolean | 0..1 | Indicates if the deletion instance of the NSD has been requested but the NSD still being used by instantiated NSs. Only present when changeType is "NSD in deletion pending". | Obsolete OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: Only present when changeType is "NSD in deletion pending" |

###### NsdDeletionNotification notification

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsdModule::NsdDeletionNotification

Description:

This notification indicates an on-boarded NSD is deleted.

Applied Stereotypes:

Experimental

OpenModelNotification

triggerConditionList:

An on-boarded NSD is deleted

support:    MANDATORY

Table B.1: Attributes for Signal NsdDeletionNotification

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| nsdInfoId | NsdInfo | 1 | Identifier of the deleted NSD information object. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| nsdId | Nsd | 1 | Identifies the deleted NSD. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |

###### NsdOnBoardingNotification notification

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsdModule::NsdOnBoardingNotification

Description:

This notification indicates that a new NSD is on-boarded, after all the on-boarding steps (e.g. uploading and processing) are done.

A change in on-boarding state before the NSD is on-boarded is not reported.

Applied Stereotypes:

Experimental

OpenModelNotification

triggerConditionList:

New NSD is on-boarded

support:    MANDATORY

Table B.1: Attributes for Signal NsdOnBoardingNotification

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| nsdInfoId | NsdInfo | 1 | Identifier of the NSD information object. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| nsdId | Nsd | 1 | Identifies the on-boarded NSD. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |

###### PnfdDeletionNotification notification

##### Datatypes

###### NsdInfoChangeType enumeration

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsdModule::PnfdDeletionNotification

Description:

This notification indicates an on-boarded PNFD is deleted.

Applied Stereotypes:

Experimental

OpenModelNotification

triggerConditionList:

An on-boarded PNFD is deleted

support:    MANDATORY

Table B.1: Attributes for Signal PnfdDeletionNotification

| Attribute Name | Type |  | Mult. |  | Description |  | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| pnfdInfoId | PnfdInfo |  |  | 1 |  |  |  |  |  | Identifier of the deleted PNFD information object. |  |

Experimental

| OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| --- |
| pnfdId | Pnfd |  | 1 |  | Identifies the deleted PNFD. |  | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |

###### PnfdOnBoardingNotification notification

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsdModule::PnfdOnBoardingNotification

Description:

This notification indicates that a new PNFD is on-boarded, after all the on-boarding steps (e.g. uploading and processing) are done.

A change in on-boarding state before the PNFD is on-boarded is not reported.

Applied Stereotypes:

Experimental

OpenModelNotification

triggerConditionList:

New PNFD is on-boarded

support:    MANDATORY

Table B.1: Attributes for Signal PnfdOnBoardingNotification

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| pnfdInfoId | PnfdInfo | 1 | Identifier of the PNFD information object. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| pnfdId | Pnfd | 1 | Identifies the on-boarded PNFD. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |

##### Datatypes

###### NsdInfoChangeType enumeration

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsdModule::NsdInfoChangeType

Description:

It categorizes the type of change.

Applied Stereotypes:

Preliminary

Contains Enumeration Literals:

CHANGE_OF_OPERATIONAL_STATE_OF_AN_ON-BOARDED_NSD

NSD_IN_DELETION_PENDING

DELETION_OF_AN_NSD

##### Associations

###### NsdInfoHasNestedNsdInfo association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsdModule::NsdInfoHasNestedNsdInfo

Applied Stereotypes:

Experimental

Table B.1: Member ends for Association NsdInfoHasNestedNsdInfo

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| nestedNsdInfoId | none | Navig. | 0..* | NsdInfo | Identifies the NSD information objects for the nested NSD referenced by the on-boarded NSD. NOTE: This attribute shall be present after the NSD is on-boarded. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| parentNsdInfoId | none | Not navig. | 0..* | NsdInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |

###### NsdInfoHasPreviousVersionNsdInfo association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsdModule::NsdInfoHasPreviousVersionNsdInfo

Applied Stereotypes:

Obsolete

Table B.1: Member ends for Association NsdInfoHasPreviousVersionNsdInfo

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| previousNsdVersionId | none | Navig. | 0..1 | NsdInfo | Reference to the previous version(s) if any of this NSD. This can be recursive if more than one previous version. | Obsolete OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| parentNsd | none | Not navig. | 1 | NsdInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsdInfoReferencesPndfInfo association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsdModule::NsdInfoReferencesPndfInfo

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association NsdInfoReferencesPndfInfo

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| pnfdInfoId | none | Navig. | 0..* | PnfdInfo | Identifies the PNFD information objects for the PNFD referenced by the on-boarded NSD. NOTE: This attribute shall be present after the NSD is on-boarded. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| nsdInfo | none | Not navig. | 0..* | NsdInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsdInfoReferencesVnfPkgInfo association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsdModule::NsdInfoReferencesVnfPkgInfo

Applied Stereotypes:

Experimental

Table B.1: Member ends for Association NsdInfoReferencesVnfPkgInfo

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| vnfPackageInfoId | none | Navig. | 0..* | VnfPkgInfo | Identifies the VnfPkgInfo objects for the VNFD referenced by the on-boarded NSD. NOTE: This attribute shall be present after the NSD is on-boarded. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| nsdInfo | none | Not navig. | 0..* | NsdInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### NsdInfoRefersToNsd association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsdModule::NsdInfoRefersToNsd

Applied Stereotypes:

Experimental

Preliminary

Table B.1: Member ends for Association NsdInfoRefersToNsd

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| nsd | composite | Navig. | 0..1 | Nsd | Reference to the on-boarded NSD, e.g. URL to the on-boarded NSD. NOTE: This attribute shall be present after the NSD is on-boarded. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| nsdInfo | none | Not navig. | 1 | NsdInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### PnfdInfoHasPreviousVersionPnfdInfo association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsdModule::PnfdInfoHasPreviousVersionPnfdInfo

Applied Stereotypes:

Obsolete

Table B.1: Member ends for Association PnfdInfoHasPreviousVersionPnfdInfo

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| previousPnfdVersionId | none | Navig. | 0..1 | PnfdInfo | Reference to the previous version) if any of this PNFD. This can be recursive if more than one previous version. | Obsolete OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| pnfdInfo | none | Not navig. | 1 | PnfdInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### to PnfdInfoRefersToPnfd association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsdModule::to PnfdInfoRefersToPnfd

Applied Stereotypes:

Experimental

Preliminary

Table B.1: Member ends for Association to PnfdInfoRefersToPnfd

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| pnfd | composite | Navig. | 0..1 | Pnfd | Reference to the on-boarded PNFD, e.g. URL to the on-boarded PNFD. NOTE: This attribute shall be present after the PNFD is on-boarded. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
|  |  |  |  | PnfdInfo |  | - support: MANDATORY |

###### PnfdInfoHasPreviousVersionPnfdInfo association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::NsDomain::NsdModule::PnfdInfoHasPreviousVersionPnfdInfo

Applied Stereotypes:

Obsolete

Table B.1: Member ends for Association PnfdInfoHasPreviousVersionPnfdInfo

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| previousPnfdVersionId | none | Navig. | 0..1 | PnfdInfo | Reference to the previous version) if any of this PNFD. This can be recursive if more than one previous version. | Obsolete OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| pnfdInfo | none | Not navig. | 1 | PnfdInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

## VirtualNetworkFunctionDomain

### Overview

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain

Description:

Group all the classes and types related to VNFs that are needed for interfaces

### IndirectResourceManagementModule

#### Overview

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::IndirectResourceManagementModule

Description:

Used for VNF-related Resource Management in indirect mode

#### IndirectConsumableVirtualisedResourceModule

##### Overview

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::IndirectResourceManagementModule::IndirectConsumableVirtualisedResourceModule

###### Diagrams

Figure B.1: Diagram ConsumableVirtualisedResources - Indirect Mode

###### Classes

VirtualComputeResourceWithRpInfo class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::IndirectResourceManagementModule::IndirectConsumableVirtualisedResourceModule::VirtualComputeResourceWithRpInfo

Description:

The VirtualComputeResourceWithRpInfo class defines the characteristics of a consumable virtualised compute resources in indirect mode.

Parent class: VirtualComputeResourceInformation

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VirtualComputeResourceWithRpInfo

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| resourceProviderId | Identifier | 1 | It is used by NFVO to determine the entity responsible for the consumable virtualized resource and is used by the VNFM to uniquely identify consumable compute type resources by means of the tuple [resourceProviderId, computeResourceTypeId]. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualMemory | VirtualMemoryResourceInformation | 0..1 | It defines the virtual memory characteristics of the consumable virtualised compute resource. Note: Cardinality can be "0" if the attribute refers to a characteristic that is not being reported on a specific query or notification, e.g., through a InformationChangeNotification | Inherited from Class VirtualComputeResourceInformation OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualCpu | VirtualCpuResourceInformation | 0..1 | It defines the virtual CPU(s) characteristics of the consumable virtualised compute resource. Cardinality can be "0" if the attribute refers to a characteristic that is not being reported on a specific query or notification, e.g., through a InformationChangeNotification | Inherited from Class VirtualComputeResourceInformation Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| accelerationCapability |  | 0..* | Acceleration capabilities (e.g., crypto, GPU) for the consumable virtualised compute resources from the set of capabilities offered by the compute node acceleration resources. The cardinality can be 0, if no particular acceleration capability is provided. Cardinality can be "0" if the attribute refers to a characteristic that is not being reported on a specific query or notification, e.g., through a InformationChangeNotification | Inherited from Class VirtualComputeResourceInformation OpenModelAttribute - isInvariant: false - support: MANDATORY |
| resourceTypeId | Identifier | 1 | Identifier of the consumable virtualised resource type. | Inherited from Class VirtualResourceTypeInformation OpenModelAttribute - isInvariant: false - support: MANDATORY |

VirtualNetworkResourceWithRpInfo class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::IndirectResourceManagementModule::IndirectConsumableVirtualisedResourceModule::VirtualNetworkResourceWithRpInfo

Description:

The VirtualNetworkResourceWithRpInfo class defines the characteristics of a consumable virtualised network resource in indirect mode.

Parent class: VirtualNetworkResourceInformation

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VirtualNetworkResourceWithRpInfo

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| resourceProviderId | Identifier | 1 | It is used by NFVO to determine the entity responsible for the consumable virtualised resource and is used by the VNFM to uniquely identify consumable network type resources by means of the tuple [resourceProviderId, networkResourceTypeId]. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| bandwidth | Number | 1 | Minimum network bandwidth (in Mbps). | Inherited from Class VirtualNetworkResourceInformation OpenModelAttribute - isInvariant: false - support: MANDATORY |
| networkType | String | 0..1 | The type of network that maps to the virtualised network. Examples are: "local", "vlan", "vxlan", "gre", etc. | Inherited from Class VirtualNetworkResourceInformation OpenModelAttribute - isInvariant: false - support: MANDATORY |
| networkQoS | NetworkQoS | 0..* | Element providing information about Quality of Service attributes that the network shall support. | Inherited from Class VirtualNetworkResourceInformation OpenModelAttribute - isInvariant: false - support: MANDATORY |
| resourceTypeId | Identifier | 1 | Identifier of the consumable virtualised resource type. | Inherited from Class VirtualResourceTypeInformation OpenModelAttribute - isInvariant: false - support: MANDATORY |

VirtualStorageResourceWithRpInfo class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::IndirectResourceManagementModule::IndirectConsumableVirtualisedResourceModule::VirtualStorageResourceWithRpInfo

Description:

The VirtualStorageResourceWithRpInfo class defines the characteristics of a consumable virtualised storage resource in indirect mode

Parent class: VirtualStorageResourceInformation

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VirtualStorageResourceWithRpInfo

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| resourceProviderId | Identifier | 1 | It is used by NFVO to determine the entity responsible for the consumable virtualised resource and is used by the VNFM to uniquely identify consumable storage type resources by means of the tuple [resourceProviderId, storageResourceTypeId]. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| typeOfStorage | String | 1 | Type of virtualised storage resource (e.g., volume, object) | Inherited from Class VirtualStorageResourceInformation OpenModelAttribute - isInvariant: false - support: MANDATORY |
| sizeOfStorage | Number | 0..1 | Size of virtualised storage resource (e.g., size of volume, in GB). Cardinality "1" covers the case where a specific configuration for the consumable resource is advertised | Inherited from Class VirtualStorageResourceInformation OpenModelAttribute - isInvariant: false - support: MANDATORY |
| rdmaSupported | Boolean | 0..1 | It indicates if the storage supports RDMA. | Inherited from Class VirtualStorageResourceInformation OpenModelAttribute - isInvariant: false - support: OPTIONAL |
| resourceTypeId | Identifier | 1 | Identifier of the consumable virtualised resource type. | Inherited from Class VirtualResourceTypeInformation OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### Notifications

InformationWithRpChangeNotification notification

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::IndirectResourceManagementModule::IndirectConsumableVirtualisedResourceModule::InformationWithRpChangeNotification

Description:

This notification informs the receiver that information related to consumable virtualised resources is changed and is applicable in the indirect mode.

Applied Stereotypes:

OpenModelNotification

triggerConditionList:

Addition of virtualised resources

Removal of virtualised resources

Update of virtualised resources

support:    MANDATORY

Preliminary

Table B.1: Attributes for Signal InformationWithRpChangeNotification

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| resourceProviderId | Identifier | 1 | It is used by NFVO to determine the entity responsible for the change notification and is used by the VNFM to uniquely identify the consumable resource by means of the tuple [resourceProviderId, resourceTypeId]. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| changeId | Identifier | 1 | Unique identifier of the change on the consumable virtualised resource type | Inherited from Signal InformationChangeNotification OpenModelAttribute - isInvariant: false - support: MANDATORY |
| resourceTypeId | VirtualResourceTypeInformation | 1 | Identifier of the consumable virtualised resource type. Reference to VirtualComputeResourceInformation, VirtualStorageResourceInformation or VirtualNetworkResourceInformation. | Inherited from Signal InformationChangeNotification OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vimId | Identifier | 1 | Identifier of the VIM reporting the change. | Inherited from Signal InformationChangeNotification OpenModelAttribute - isInvariant: false - support: MANDATORY |
| changeType | InformationChangeType | 1 | It categorizes the type of change. Permitted values are: ADDITION, REMOVAL and UPDATE. | Inherited from Signal InformationChangeNotification OpenModelAttribute - isInvariant: false - support: MANDATORY |
| changedResourceData |  | 0..1 | Details of the changes of consumable virtualised resource information. Its content can differ based on the values of the resourceTypeId and changeType. | Inherited from Signal InformationChangeNotification OpenModelAttribute - isInvariant: false - support: MANDATORY |

#### IndirectFaultManagementModule

##### Overview

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::IndirectResourceManagementModule::IndirectFaultManagementModule

###### Diagrams

Figure B.1: Diagram Fault Management - Indirect Mode

###### Classes

AlarmWithRpInfo class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::IndirectResourceManagementModule::IndirectFaultManagementModule::AlarmWithRpInfo

Description:

The AlarmWithRpInfo class encapsulates data of a virtualised resource alarm in indirect mode.

Parent class: Alarm

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class AlarmWithRpInfo

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| resourceProviderId | Identifier | 1 | It is used by NFVO to determine the entity responsible for issuing the alarm, and is used by the VNFM to uniquely identify resources by means of the tuple [resourceProviderId, managedObjectId]. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| alarmId | Identifier | 1 | Identifier of this Alarm class. | Inherited from Class Alarm OpenModelAttribute - isInvariant: true - support: MANDATORY |
| managedObjectId | Identifier | 1 | Identifier of the affected managed object. The managed object depends on the reference point using this class. When used on Or-Vi or Vi-Vnfm, it will be a virtualised resource. When used on Or-Vnfm or Ve-Vnfm reference point, it will be a VNF instance. When used on Os-Ma-nfvo reference point, it will be a Network Service instance. | Inherited from Class Alarm OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfcId | Identifier | 1..* | Identifier of the affected VNFC(s). | Inherited from Class Alarm OpenModelAttribute - isInvariant: false - support: CONDITIONAL_MANDATORY - condition: Only present in IFA008 |
| rootCauseFaultyComponent | FaultyComponentInfo | 1 | Only for IFA013. The NS components (e.g. nested NS, NS virtual link and VNF) that are causing the NS fault. | Inherited from Class Alarm Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| rootCauseFaultyResource | FaultyResourceInfo | 0..1 | The virtualised resources that are causing the VNF or NS fault. For IFA013, it shall be present when the faulty component is "NS Virtual Link" or "VNF" | Inherited from Class Alarm Experimental OpenModelAttribute - isInvariant: false - support: CONDITIONAL_MANDATORY - condition: Only present in IFA007, IFA008 and IFA013 |
| alarmRaisedTime | DateTime | 1 | Timestamp indicating when the alarm was raised by the managed object. | Inherited from Class Alarm Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| alarmChangedTime | DateTime | 0..1 | Timestamp indicating when the alarm was last changed. It shall be present if the alarm has been updated. | Inherited from Class Alarm Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| alarmClearedTime | DateTime | 0..1 | Timestamp indicating when the alarm was cleared. It shall be present if the alarm has been cleared. | Inherited from Class Alarm Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| ackState | AckState | 1 | State of the alarm, permitted values include: • Acknowledged • Unacknowledged | Inherited from Class Alarm Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| perceivedSeverity | PerceivedSeverity | 1 | Perceived severity of the managed object failure, legal values. e.g.: • Critical • Major • Minor • Warning • Indeterminate • Cleared | Inherited from Class Alarm OpenModelAttribute - isInvariant: false - support: MANDATORY |
| eventTime | DateTime | 1 | Timestamp indicating when the fault was observed. | Inherited from Class Alarm OpenModelAttribute - isInvariant: false - support: MANDATORY |
| eventType | EventType | 1 | Type of the event. The allowed values for the eventType attribute use the event type defined in [x]: • Communication Alarm • Processing Alarm • Environment Alarm • QoS Alarm • Equipment Alarm | Inherited from Class Alarm Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| faultType | String | 0..1 | Additional information related to the type of the fault | Inherited from Class Alarm Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| probableCause | String | 1 | Information about the probable cause of the fault. | Inherited from Class Alarm OpenModelAttribute - isInvariant: false - support: MANDATORY |
| isRootCause | Boolean | 1 | Parameter indicating if this fault is the root for other correlated alarms. If TRUE, then the alarms listed in the parameter CorrelatedAlarmId are caused by this fault. | Inherited from Class Alarm OpenModelAttribute - isInvariant: false - support: MANDATORY |
| correlatedAlarmId | Alarm | 0..* | List of other alarms correlated to this fault. | Inherited from Class Alarm OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| faultDetails |  | 0..* | Provides additional information about the fault. | Inherited from Class Alarm OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### Notifications

AlarmClearedWithRpNotification notification

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::IndirectResourceManagementModule::IndirectFaultManagementModule::AlarmClearedWithRpNotification

Description:

This notification encapsulates information on a cleared alarm and is applicable in the indirect mode of VNF-related resource reservation management.

Applied Stereotypes:

OpenModelNotification

triggerConditionList:

An alarm has been cleared.

support:    MANDATORY

Preliminary

Table B.1: Attributes for Signal AlarmClearedWithRpNotification

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| resourceProviderId | Identifier | 1 | It is used by NFVO to determine the entity responsible for the alarm and is used by the VNFM to uniquely identify the alarm by means of the tuple [resourceProviderId, alarmId]. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| alarmId | Alarm | 1 | Alarm identifier. | Inherited from Signal AlarmClearedNotification OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| alarmClearedTime | DateTime | 1 | Timestamp indicating when the alarm was cleared. | Inherited from Signal AlarmClearedNotification OpenModelAttribute - isInvariant: false - support: MANDATORY |

AlarmWithRpNotification notification

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::IndirectResourceManagementModule::IndirectFaultManagementModule::AlarmWithRpNotification

Description:

This notification encapsulates information on an alarm and is applicable in the indirect mode of VNF-related resource reservation management.

Applied Stereotypes:

OpenModelNotification

triggerConditionList:

An alarm has been created.

An alarm has been updated, e.g. if the severity of the alarm has changed.

support:    MANDATORY

Preliminary

Table B.1: Attributes for Signal AlarmWithRpNotification

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| resourceProviderId | Identifier | 1 | It is used by NFVO to determine the entity responsible for the alarm and is used by the VNFM to uniquely identify the alarm by means of the tuple [resourceProviderId, alarmId]. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| alarm | Alarm | 1 | Information about an alarm including alarmId, affected managedObjectId, and faultDetails. For notifications related to changes in the state of virtualised resources (indicated using the attribute faultType), the alarm shall indicate: • the cause for the state change of the virtualised resource using the attribute probableCause, with possible values such as: maintenance of NFVI component, evacuation of NFVI component, etc. • the identifier of the origin (VIM) responsible for the management of the virtualised resource with state change using the attribute faultDetails. Note: In case the alarm cause cannot be determined, the notification should identify the alarm as cause as being undetermined. | Inherited from Signal AlarmNotification OpenModelAttribute - isInvariant: false - support: MANDATORY |

#### IndirectPerformanceManagementModule

##### Overview

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::IndirectResourceManagementModule::IndirectPerformanceManagementModule

###### Diagrams

Figure B.1: Diagram Performance Management - Indirect Mode

###### Notifications

PerformanceInformationWithRpAvailableNotification notification

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::IndirectResourceManagementModule::IndirectPerformanceManagementModule::PerformanceInformationWithRpAvailableNotification

Description:

This notification informs the receiver that performance information is available and is applicable in the indirect mode of VNF-related resource reservation management.

Applied Stereotypes:

OpenModelNotification

triggerConditionList:

New performance information is available

support:    MANDATORY

Preliminary

Table B.1: Attributes for Signal PerformanceInformationWithRpAvailableNotification

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| resourceProviderId | Identifier | 1 | It is used by NFVO to determine the entity responsible for the resources and is used by the VNFM to uniquely identify the resources for which information is available by means of the tuple [resourceProviderId, objectInstanceId]. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| objectInstanced | Identifier | 1..* | Object instance(s) for which performance information is available. The object instances depend on the reference point using this notification. When used on Or-Vi or Vi-Vnfm, it will be virtualised resources. When used on Or-Vnfm or Ve-Vnfm reference point, it will be VNF instances. When used on Os-Ma-nfvo reference point, it will be Network Service instances. | Inherited from Signal PerformanceInformationAvailableNotification OpenModelAttribute - isInvariant: false - support: MANDATORY |

ThresholdCrossedWithRpNotification notification

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::IndirectResourceManagementModule::IndirectPerformanceManagementModule::ThresholdCrossedWithRpNotification

Description:

This notification informs the receiver that a threshold value has been crossed and is applicable in the indirect mode of VNF-related resource reservation management.

Applied Stereotypes:

OpenModelNotification

triggerConditionList:

A Threshold has been crossed. Crossing includes both the initial triggering and the clearance of the threshold. Depending on threshold type, there might be multiple crossing values and/or different values for triggering and clearing

support:    MANDATORY

Preliminary

Table B.1: Attributes for Signal ThresholdCrossedWithRpNotification

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| resourceProviderId | Identifier | 1 | It is used by NFVO to determine the entity responsible for the resources and is used by the VNFM to uniquely identify the resources for which the threshold is crossed by means of the tuple [resourceProviderId, objectInstanceId]. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| thresholdId | Threshold | 1 | Threshold which has been crossed. | Inherited from Signal ThresholdCrossedNotification OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| crossingDirection | ThresholdCrossing | 1 | An indication of whether the threshold was crossed in upward or downward direction. | Inherited from Signal ThresholdCrossedNotification OpenModelAttribute - isInvariant: false - support: MANDATORY |
| objectInstanceId | Identifier | 1 | Object instance for which the threshold has been crossed. The object instances depend on the reference point using this notification. When used on Or-Vi or Vi-Vnfm, it will be virtualised resources. When used on Or-Vnfm or Ve-Vnfm reference point, it will be VNF instances. When used on Os-Ma-nfvo reference point, it will be Network Service instances. | Inherited from Signal ThresholdCrossedNotification OpenModelAttribute - isInvariant: false - support: MANDATORY |
| performanceMetric | String | 1 | Performance metric associated with the threshold. | Inherited from Signal ThresholdCrossedNotification OpenModelAttribute - isInvariant: false - support: MANDATORY |
| performanceValue | Value | 1 | Value of the metric that resulted in threshold crossing. | Inherited from Signal ThresholdCrossedNotification OpenModelAttribute - isInvariant: false - support: MANDATORY |

#### IndirectQuotaModule

##### Overview

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::IndirectResourceManagementModule::IndirectQuotaModule

###### Diagrams

Figure B.1: Diagram Quota - Indirect Mode

###### Classes

VirtualComputeQuotaWithRpInfo class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::IndirectResourceManagementModule::IndirectQuotaModule::VirtualComputeQuotaWithRpInfo

Description:

The VirtualComputeQuotaWithRpInfo class encapsulates information about a quota for virtualised compute resources.

Parent class: VirtualComputeQuota

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VirtualComputeQuotaWithRpInfo

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| resourceProviderId | Identifier | 1 | It is used by the NFVO to determine the entity responsible for the management of the virtualised resources quota and is used by the VNFM to uniquely identify resources quota by means of the tuple [resourceProviderId, resourceGroupId]. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| resourceGroupId | Identifier | 1 | Unique identifier of the "infrastructure resource group", logical grouping of virtual resources assigned to a tenant within an Infrastructure Domain. | Inherited from Class VirtualComputeQuota OpenModelAttribute - isInvariant: false - support: MANDATORY |
| numVCPUs | Integer | 0..1 | Number of CPU cores that have been restricted by the quota. The cardinality can be 0 if no specific number of CPU cores has been requested to be restricted by the quota. | Inherited from Class VirtualComputeQuota OpenModelAttribute - isInvariant: false - support: MANDATORY |
| numVcInstances | Integer | 0..1 | Number of virtualisation container instances that have been restricted by the quota. The cardinality can be 0 if no specific number of CPU cores has been requested to be restricted by the quota. | Inherited from Class VirtualComputeQuota OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualMemSize | Number | 0..1 | Size of virtual memory that has been restricted by the quota. The cardinality can be 0 if no specific number of CPU cores has been requested to be restricted by the quota. | Inherited from Class VirtualComputeQuota OpenModelAttribute - isInvariant: false - support: MANDATORY |

VirtualNetworkQuotaWithRpInfo class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::IndirectResourceManagementModule::IndirectQuotaModule::VirtualNetworkQuotaWithRpInfo

Description:

The VirtualNetworkQuotaWithRpInfo class encapsulates information about a quota for virtualised network resources.

Parent class: VirtualNetworkQuota

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VirtualNetworkQuotaWithRpInfo

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| resourceProviderId | Identifier | 1 | It is used by the NFVO to determine the entity responsible for the management of the virtualised resources quota and is used by the VNFM to uniquely identify resources quota by means of the tuple [resourceProviderId, resourceGroupId]. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| resourceGroupId | Identifier | 1 | Unique identifier of the "infrastructure resource group", logical grouping of virtual resources assigned to a tenant within an Infrastructure Domain. | Inherited from Class VirtualNetworkQuota OpenModelAttribute - isInvariant: false - support: MANDATORY |
| numPublicIps | Integer | 0..1 | Number of public IP addresses that have been restricted by the quota. The cardinality can be 0 if no specific number of public IP addresses has been requested to be restricted by the quota. | Inherited from Class VirtualNetworkQuota OpenModelAttribute - isInvariant: false - support: MANDATORY |
| numPorts | Integer | 0..1 | Number of ports that have been restricted by the quota. The cardinality can be 0 if no specific number of ports has been requested to be restricted by the quota. | Inherited from Class VirtualNetworkQuota OpenModelAttribute - isInvariant: false - support: MANDATORY |
| numSubnets | Integer | 0..1 | Number of subnets that have been restricted by the quota. The cardinality can be 0 if no specific number of subnets has been requested to be restricted by the quota. | Inherited from Class VirtualNetworkQuota OpenModelAttribute - isInvariant: false - support: MANDATORY |

VirtualStorageQuotaWithRpInfo class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::IndirectResourceManagementModule::IndirectQuotaModule::VirtualStorageQuotaWithRpInfo

Description:

The VirtualStorageQuotaWithRpInfo class encapsulates information abouta quota for virtualised storage resources.

Parent class: VirtualStorageQuota

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VirtualStorageQuotaWithRpInfo

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| resourceProviderId | Identifier | 1 | It is used by the NFVO to determine the entity responsible for the management of the virtualised resources quota and is used by the VNFM to uniquely identify resources quota by means of the tuple [resourceProviderId, resourceGroupId]. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| resourceGroupId | Identifier | 1 | Unique identifier of the "infrastructure resource group", logical grouping of virtual resources assigned to a tenant within an Infrastructure Domain. | Inherited from Class VirtualStorageQuota OpenModelAttribute - isInvariant: false - support: MANDATORY |
| storageSize | Number | 0..1 | Size of virtualised storage resource that has been restricted by the quota. Cardinality can be 0 if no specific size of virtualised storage resource has been requested to be restricted by the quota. | Inherited from Class VirtualStorageQuota OpenModelAttribute - isInvariant: false - support: MANDATORY |
| numSnapshots | Integer | 0..1 | Number of snapshots that have been restricted by the quota. The cardinality can be 0 if no specific number of snapshots has been requested to be restricted by the quota. | Inherited from Class VirtualStorageQuota OpenModelAttribute - isInvariant: false - support: MANDATORY |
| numVolumes | Integer | 0..1 | Number of volumes that have been restricted by the quota. The cardinality can be 0 if no specific number of volumes has been requested to be restricted by the quota. | Inherited from Class VirtualStorageQuota OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### Notifications

VirtualisedResourceQuotaWithRpChangeNotification  notification

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::IndirectResourceManagementModule::IndirectQuotaModule::VirtualisedResourceQuotaWithRpChangeNotification

Description:

This notification indicates a change in a virtualised resource quota and is applicable in the indirect mode of resourwce quota management.

Applied Stereotypes:

OpenModelNotification

triggerConditionList:

A resource quota is being updated.

support:    MANDATORY

Preliminary

Table B.1: Attributes for Signal VirtualisedResourceQuotaWithRpChangeNotification

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| resourceProviderId | Identifier | 1 | It is used by NFVO to determine the entity responsible for the change notification and is used by the VNFM to uniquely identify the resource quota by means of the tuple [resourceProviderId, resourceTypeId]. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| changeId | Identifier | 1 | Unique identifier of the change on the virtualised resource Quota | Inherited from Signal VirtualisedResourceQuotaChangeNotification OpenModelAttribute - isInvariant: false - support: MANDATORY |
| resourceGroupId | Identifier | 1 | Unique identifier of the "infrastructure resource group", logical grouping of virtual resources assigned to a tenant within an Infrastructure Domain. | Inherited from Signal VirtualisedResourceQuotaChangeNotification OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vimId | Identifier | 1 | The VIM reporting the change. | Inherited from Signal VirtualisedResourceQuotaChangeNotification OpenModelAttribute - isInvariant: false - support: MANDATORY |
| changeType | String | 1 | It categorizes the type of change. Possible values can be related to an update of the Quota. | Inherited from Signal VirtualisedResourceQuotaChangeNotification OpenModelAttribute - isInvariant: false - support: MANDATORY |
| changedQuotaData |  | 0..1 | Details of the changes of the Quota | Inherited from Signal VirtualisedResourceQuotaChangeNotification OpenModelAttribute - isInvariant: false - support: MANDATORY |

#### IndirectReservationModule

##### Overview

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::IndirectResourceManagementModule::IndirectReservationModule

###### Diagrams

Figure B.1: Diagram Reservation - Indirect Mode

###### Classes

ReservedVirtualComputeWithRpInfo class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::IndirectResourceManagementModule::IndirectReservationModule::ReservedVirtualComputeWithRpInfo

Description:

The ReservedVirtualComputeWithRpInfo class encapsulates information about a reservation for virtualised compute resources.

Parent class: ReservedVirtualCompute

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class ReservedVirtualComputeWithRpInfo

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| resourceProviderId | Identifier | 1 | It is used by NFVO to determine the entity responsible for the management of the Virtualised resource and is used by the VNFM to uniquely identify resources reservation by means of the tuple [resourceProviderId, reservationId]. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| reservationId | Identifier | 1 | Identifier of the resource reservation. | Inherited from Class ReservedVirtualCompute OpenModelAttribute - isInvariant: false - support: MANDATORY |
| computePoolReserved | ReservedComputePool | 0..1 | Information about compute resources that have been reserved, e.g. {"cpu_cores": 90, "vm_instances": 10, "ram": 10000}. | Inherited from Class ReservedVirtualCompute OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualisationContainerReserved | ReservedVirtualisationContainer | 0..* | Information about the virtualisation container(s) that have been reserved. | Inherited from Class ReservedVirtualCompute OpenModelAttribute - isInvariant: false - support: MANDATORY |
| reservationStatus | ReservationStatus | 1 | Status of the compute resource reservation, e.g., to indicate if a reservation is being used. | Inherited from Class ReservedVirtualCompute OpenModelAttribute - isInvariant: false - support: MANDATORY |
| startTime | DateTime | 0..1 | Indication when the consumption of the resources starts. If the value is 0, resources are reserved for immediate use. | Inherited from Class ReservedVirtualCompute Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| endTime | DateTime | 0..1 | Indication when the reservation ends (when it is expected that the resources will no longer be needed) and used by the VIM to schedule the reservation. If not present, resources are reserved for unlimited usage time. | Inherited from Class ReservedVirtualCompute Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| expiryTime | DateTime | 0..1 | Indication when the VIM can release the reservation in case no allocation request against this reservation was made. | Inherited from Class ReservedVirtualCompute Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

ReservedVirtualNetworkWithRpInfo  class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::IndirectResourceManagementModule::IndirectReservationModule::ReservedVirtualNetworkWithRpInfo

Description:

The ReservedVirtualNetworkWithRpInfo class encapsulates information about a reservation for virtualised network resources.

Parent class: ReservedVirtualNetwork

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class ReservedVirtualNetworkWithRpInfo

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| resourceProviderId | Identifier | 1 | It is used by NFVO to determine the entity responsible for the management of the Virtualised resource and is used by the VNFM to uniquely identify resources reservation by means of the tuple [resourceProviderId, reservationId]. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| reservationId | Identifier | 1 | Identifier of the resource reservation. | Inherited from Class ReservedVirtualNetwork OpenModelAttribute - isInvariant: false - support: MANDATORY |
| publicIps | IpAddress | 1 | List of public IP addresses that have been reserved | Inherited from Class ReservedVirtualNetwork OpenModelAttribute - isInvariant: false - support: MANDATORY |
| networkAttributes | ReservedVirtualNetworkAttributes | 0..1 | Information specifying additional attributes of the network resource that has been reserved | Inherited from Class ReservedVirtualNetwork OpenModelAttribute - isInvariant: false - support: MANDATORY |
| networkPorts | ReservedVirtualNetworkPort | 0..* | List of metadata key-value pairs used by the consumer to associate meaningful metadata to the related virtualised resource. | Inherited from Class ReservedVirtualNetwork OpenModelAttribute - isInvariant: false - support: MANDATORY |
| reservationStatus | ReservationStatus | 1 | Status of the network resource reservation, e.g., to indicate if a reservation is being used. | Inherited from Class ReservedVirtualNetwork OpenModelAttribute - isInvariant: false - support: MANDATORY |
| startTime | DateTime | 0..1 | Indication when the consumption of the resources starts. If the value is 0, resources are reserved for immediate use. | Inherited from Class ReservedVirtualNetwork Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| endTime | DateTime | 0..1 | Indication when the reservation ends (when it is expected that the resources will no longer be needed) and used by the VIM to schedule the reservation. If not present, resources are reserved for unlimited usage time. | Inherited from Class ReservedVirtualNetwork Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| expiryTime | DateTime | 0..1 | Indication when the VIM can release the reservation in case no allocation request against this reservation was made. | Inherited from Class ReservedVirtualNetwork Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| zoneId | ResourceZone | 1 | References the resource zone where the virtual network resources have been reserved. Cardinality can be 0 to cover the case where reserved network resources are not bound to a specific resource zone. | Inherited from Class ReservedVirtualNetwork OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |

ReservedVirtualStorageWithRpInfo class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::IndirectResourceManagementModule::IndirectReservationModule::ReservedVirtualStorageWithRpInfo

Description:

The ReservedVirtualStorageWithRpInfo class encapsulates information about a reservation for virtualised storage resources.

Parent class: ReservedVirtualStorage

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class ReservedVirtualStorageWithRpInfo

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| resourceProviderId | Identifier | 1 | It is used by NFVO to determine the entity responsible for the management of the Virtualised resource and is used by the VNFM to uniquely identify resources reservation by means of the tuple [resourceProviderId, reservationId]. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| reservationId | Identifier | 1 | Identifier of the resource reservation. | Inherited from Class ReservedVirtualStorage OpenModelAttribute - isInvariant: false - support: MANDATORY |
| storagePoolReserved | ReservedStoragePool | 0..1 | Information about storage resources that have been reserved, e.g. {"gigabytes": 1000, "snapshots": 10, "volumes": 10}. | Inherited from Class ReservedVirtualStorage OpenModelAttribute - isInvariant: false - support: MANDATORY |
| reservationStatus | ReservationStatus | 1 | Status of the storage resource reservation, e.g., to indicate if a reservation is being used. | Inherited from Class ReservedVirtualStorage OpenModelAttribute - isInvariant: false - support: MANDATORY |
| startTime | DateTime | 0..1 | Indication when the consumption of the resources starts. If the value is 0, resources are reserved for immediate use. | Inherited from Class ReservedVirtualStorage Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| endTime | DateTime | 0..1 | Indication when the reservation ends (when it is expected that the resources will no longer be needed) and used by the VIM to schedule the reservation. If not present, resources are reserved for unlimited usage time. | Inherited from Class ReservedVirtualStorage Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| expiryTime | DateTime | 0..1 | Indication when the VIM can release the reservation in case no allocation request against this reservation was made. | Inherited from Class ReservedVirtualStorage Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### Notifications

VirtualisedResourceReservationWithRpChangeNotification notification

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::IndirectResourceManagementModule::IndirectReservationModule::VirtualisedResourceReservationWithRpChangeNotification

Description:

This notification indicates a change in a virtualised resource reservation and is applicable in the indirect mode of VNF-related resource reservation management.

Applied Stereotypes:

OpenModelNotification

triggerConditionList:

A resource reservation has been updated.

A resource reservation changed due to changes in underlying resources that are part of this reservation.

support:    MANDATORY

Preliminary

Table B.1: Attributes for Signal VirtualisedResourceReservationWithRpChangeNotification

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| resourceProviderId | Identifier | 1 | It is used by NFVO to determine the entity responsible for the change notification and is used by the VNFM to uniquely identify the resource reservation by means of the tuple [resourceProviderId, reservationId]. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| changeId | Identifier | 1 | Unique identifier of the change on the virtualised resource reservation. | Inherited from Signal VirtualisedResourceReservationChangeNotification OpenModelAttribute - isInvariant: false - support: MANDATORY |
| reservationId | Identifier | 1 | The reservation being changed. | Inherited from Signal VirtualisedResourceReservationChangeNotification OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vimId | Identifier | 1 | The VIM reporting the change. | Inherited from Signal VirtualisedResourceReservationChangeNotification OpenModelAttribute - isInvariant: false - support: MANDATORY |
| changeType | String | 1 | It categorizes the type of change. Possible values can be related to an update of the reservation or a change in the resources part of the reservation. | Inherited from Signal VirtualisedResourceReservationChangeNotification OpenModelAttribute - isInvariant: false - support: MANDATORY |
| changedReservationData |  | 0..1 | Details of the changes of the reservation. | Inherited from Signal VirtualisedResourceReservationChangeNotification OpenModelAttribute - isInvariant: false - support: MANDATORY |

#### IndirectVirtualisedResourcesModule

##### Overview

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::IndirectResourceManagementModule::IndirectVirtualisedResourcesModule

###### Diagrams

Figure B.1: Diagram Virtualised Resources Management Interfaces - Indirect Mode

###### Classes

ComputeResourceWithRpId class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::IndirectResourceManagementModule::IndirectVirtualisedResourcesModule::ComputeResourceWithRpId

Description:

This class defines the identity of a virtualised compute resource in indirect mode.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class ComputeResourceWithRpId

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| resourceProvider | Identifier | 1 | It is used by NFVO to determine the entity responsible for the management of the Virtualised resource and is used by the VNFM to uniquely identify resources by means of the tuple [resourceProviderId, computeId]. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| computeId | Identifier | 1 | Identifier of the compute resource within the VIM. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

ComputeResourceWithRpInfo class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::IndirectResourceManagementModule::IndirectVirtualisedResourcesModule::ComputeResourceWithRpInfo

Description:

The ComputeResourceWithRpInfo class encapsulates data of an instantiated virtualised compute resource in indirect mode.

Parent class: VirtualCompute

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class ComputeResourceWithRpInfo

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| resourceProviderId | Identifier | 1 | It is used by NFVO to determine the entity responsible for the management of the Virtualised resource and is used by the VNFM to uniquely identify resources by means of the tuple [resourceProviderId, computeId]. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| computeId | Identifier | 1 | Identifier of the virtualised compute resource | Inherited from Class VirtualCompute OpenModelAttribute - isInvariant: false - support: MANDATORY |
| computeName | String | 0..1 | Name of the virtualised compute resource | Inherited from Class VirtualCompute OpenModelAttribute - isInvariant: false - support: MANDATORY |
| flavourId | Identifier | 1 | Identifier of the given compute flavour used to instantiate this virtual compute | Inherited from Class VirtualCompute OpenModelAttribute - isInvariant: false - support: MANDATORY |
| accelerationCapability |  | 0..* | Selected acceleration capabilities (e.g. crypto, GPU) from the set of capabilities offered by the compute node acceleration resources. The cardinality can be 0, if no particular acceleration capabilitiy is provided. | Inherited from Class VirtualCompute OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualCpu | VirtualCpu | 1 | The virtual CPU(s) of the virtualised compute | Inherited from Class VirtualCompute OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualMemory | VirtualMemory | 1 | The virtual memory of the compute | Inherited from Class VirtualCompute OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualNetworkInterface | VirtualNetworkInterface | 0..* | Element with information of the instantiated virtual network interfaces of the compute resource | Inherited from Class VirtualCompute OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualDisks | VirtualStorage | 1..* | Element with information of the virtualised storage resources (volumes, ephemeral that are attached to the compute resource. | Inherited from Class VirtualCompute OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vcImageId | Identifier | 0..1 | Identifier of the virtualised container software image (e.g., virtual machine image). Cardinality can be 0 if an "empty" virtualisation container is allocated. | Inherited from Class VirtualCompute OpenModelAttribute - isInvariant: false - support: MANDATORY |
| zoneId | Identifier | 0..1 | If present, it identifies the Resource Zone where the virtual compute resources have been allocated. | Inherited from Class VirtualCompute OpenModelAttribute - isInvariant: false - support: MANDATORY |
| hostId | Identifier | 1 | Identifier of the host the virtualised compute resource is allocated on | Inherited from Class VirtualCompute OpenModelAttribute - isInvariant: false - support: MANDATORY |
| operationalState | OperationalState | 1 | Operational state of the compute resource | Inherited from Class VirtualCompute OpenModelAttribute - isInvariant: false - support: MANDATORY |
| metadata | KeyValuePair | 0..* | List of metadata key-value pairs used by the consumer to associate meaningful metadata to the related virtualised resource | Inherited from Class VirtualCompute OpenModelAttribute - isInvariant: false - support: OPTIONAL |

NetworkResourceWithRpId class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::IndirectResourceManagementModule::IndirectVirtualisedResourcesModule::NetworkResourceWithRpId

Description:

This class defines the identity of a virtualised network resource in indirect mode.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class NetworkResourceWithRpId

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| resourceProvider | Identifier | 1 | It is used by NFVO to determine the entity responsible for the management of the Virtualised resource and is used by the VNFM to uniquely identify resources by means of the tuple [resourceProviderId, networkResourceId]. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| networkResourceId | Identifier | 1 | Identifier of the network resource within the VIM. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

NetworkResourceWithRpInfo class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::IndirectResourceManagementModule::IndirectVirtualisedResourcesModule::NetworkResourceWithRpInfo

Description:

The NetworkResourceWithRpInfo class encapsulates data of an instantiated virtualised network resource in indirect mode.

Parent class: VirtualNetwork

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class NetworkResourceWithRpInfo

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| resourceProviderId | Identifier | 1 | It is used by NFVO to determine the entity responsible for the management of the Virtualised resource and is used by the VNFM to uniquely identify resources by means of the tuple [resourceProviderId, networkResourceId]. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| networkResourceId | Identifier | 1 | Identifier of the virtualised network resource. | Inherited from Class VirtualNetwork OpenModelAttribute - isInvariant: false - support: MANDATORY |
| networkResourceName | String | 0..1 | Name of the virtualised network resource. | Inherited from Class VirtualNetwork OpenModelAttribute - isInvariant: false - support: MANDATORY |
| subnet | NetworkSubnet | 0..* | Only present if the network provides layer 3 connectivity. | Inherited from Class VirtualNetwork OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| networkPort | VirtualNetworkPort | 0..* | Element providing information of an instantiated virtual network port. | Inherited from Class VirtualNetwork OpenModelAttribute - isInvariant: false - support: MANDATORY |
| bandwidth | Number | 1 | Minimum network bandwidth (in Mbps). | Inherited from Class VirtualNetwork OpenModelAttribute - isInvariant: false - support: MANDATORY |
| networkType | String | 1 | The type of network that maps to the virtualised network. This list is extensible. Examples are: "local", "vlan", "vxlan", "gre", "l3-vpn", etc. | Inherited from Class VirtualNetwork OpenModelAttribute - isInvariant: false - support: MANDATORY |
| segmentType | String | 0..1 | The isolated segment for the virtualised network. For instance, for a "vlan" networkType, it corresponds to the vlan identifier; and for a "gre" networkType, this corresponds to a gre key. The cardinality can be "0" for flat networks without any specific segmentation. | Inherited from Class VirtualNetwork OpenModelAttribute - isInvariant: false - support: MANDATORY |
| networkQoS | NetworkQoS | 0..* | Element providing information about Quality of Service attributes that the network supports. Cardinality can be "0" for virtual network without any QoS requirements | Inherited from Class VirtualNetwork OpenModelAttribute - isInvariant: false - support: MANDATORY |
| isShared | Boolean | 1 | It defines whether the virtualised network is shared among consumers. | Inherited from Class VirtualNetwork OpenModelAttribute - isInvariant: false - support: MANDATORY |
| sharingCriteria |  | 0..1 | Only present for shared networks. Indicate the sharing criteria for this network. This criteria might be a list of authorised consumers. | Inherited from Class VirtualNetwork OpenModelAttribute - isInvariant: false - support: MANDATORY |
| zoneId | Identifier | 0..1 | If present, it identifies the Resource Zone where the virtual network resources have been allocated. | Inherited from Class VirtualNetwork OpenModelAttribute - isInvariant: false - support: MANDATORY |
| operationalState | OperationalState | 1 | The operational state of the virtualised network. | Inherited from Class VirtualNetwork OpenModelAttribute - isInvariant: false - support: MANDATORY |
| metadata | KeyValuePair | 0..* | List of metadata key-value pairs used by the consumer to associate meaningful metadata to the related virtualised resource. | Inherited from Class VirtualNetwork OpenModelAttribute - isInvariant: false - support: OPTIONAL |

StorageResourceWithRpId class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::IndirectResourceManagementModule::IndirectVirtualisedResourcesModule::StorageResourceWithRpId

Description:

This class defines the identity of a virtualised storage resource in indirect mode.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class StorageResourceWithRpId

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| resourceProvider | Identifier | 1 | It is used by NFVO to determine the entity responsible for the management of the Virtualised resource and is used by the VNFM to uniquely identify resources by means of the tuple [resourceProviderId, storageId]. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| storageId | Identifier | 1 | Identifier of the storage resource within the VIM. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

StorageResourceWithRpInfo class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::IndirectResourceManagementModule::IndirectVirtualisedResourcesModule::StorageResourceWithRpInfo

Description:

The StorageResourceWithRpInfo class encapsulates data of an instantiated virtualised storage resource.

Parent class: VirtualStorage

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class StorageResourceWithRpInfo

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| resourceProviderId | Identifier | 1 | It is used by NFVO to determine the entity responsible for the management of the Virtualised resource and is used by the VNFM to uniquely identify resources by means of the tuple [resourceProviderId, storageId]. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| storageId | Identifier | 1 | Identifier of the virtualised storage resource. | Inherited from Class VirtualStorage OpenModelAttribute - isInvariant: true - support: MANDATORY |
| storageName | String | 0..1 | Name of the virtualised storage resource. | Inherited from Class VirtualStorage OpenModelAttribute - isInvariant: false - support: MANDATORY |
| flavourId | Identifier | 1 | Identifier of the storage flavour used to instantiate this virtual storage | Inherited from Class VirtualStorage OpenModelAttribute - isInvariant: false - support: MANDATORY |
| typeOfStorage | String | 1 | Type of virtualised storage resource (e.g. volume, object). | Inherited from Class VirtualStorage OpenModelAttribute - isInvariant: false - support: MANDATORY |
| sizeOfStorage | Number | 1 | Size of virtualised storage resource (e.g. size of volume, in GB). | Inherited from Class VirtualStorage OpenModelAttribute - isInvariant: false - support: MANDATORY |
| rdmaEnabled | Boolean | 1 | Indicate if the storage support RDMA | Inherited from Class VirtualStorage OpenModelAttribute - isInvariant: false - support: OPTIONAL |
| ownerId | Identifier | 0..1 | Identifier of the virtualised resource that owns and uses such a virtualised storage resource. The value can be NULL if the virtualised storage is not attached yet to any other resource (e.g., a virtual machine). | Inherited from Class VirtualStorage OpenModelAttribute - isInvariant: false - support: MANDATORY |
| zoneId | Identifier | 0..1 | If present, it identifies the Resource Zone where the virtual storage resources have been allocated. | Inherited from Class VirtualStorage OpenModelAttribute - isInvariant: false - support: MANDATORY |
| hostId | Identifier | 0..1 | Identifier of the host where the virtualised storage resource is allocated. A cardinality of 0 refers to distributed storage solutions. | Inherited from Class VirtualStorage OpenModelAttribute - isInvariant: false - support: MANDATORY |
| operationalState | OperationalState | 1 | Operational state of the resource. | Inherited from Class VirtualStorage OpenModelAttribute - isInvariant: false - support: MANDATORY |
| metadata | KeyValuePair | 0..* | List of metadata key-value pairs used by the consumer to associate meaningful metadata to the related virtualised resource. | Inherited from Class VirtualStorage OpenModelAttribute - isInvariant: false - support: OPTIONAL |

###### Notifications

VirtualisedResourceWithRpChangeNotification notification

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::IndirectResourceManagementModule::IndirectVirtualisedResourcesModule::VirtualisedResourceWithRpChangeNotification

Description:

This notification informs the receiver of changes in the virtualised resources that are allocated and is applicable in the indirect mode of VNF-related resource reservation management.

Applied Stereotypes:

OpenModelNotification

triggerConditionList:

Maintenance of NFVI components, e.g., physical maintenance/repair, hypervisor software updates, etc.

Evacuation of physical hosts.

Addition and removal of physical resources.

Operation and management of NFVI resources, e.g., to support energy efficiency or resource usage optimization.

support:    MANDATORY

Preliminary

Table B.1: Attributes for Signal VirtualisedResourceWithRpChangeNotification

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| resourceProviderId | Identifier | 1 | It is used by NFVO to determine the entity responsible for the change notification and is used by the VNFM to uniquely identify the resource by means of the tuple [resourceProviderId, resourceTypeId]. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| changeId | Identifier | 1 | Unique identifier of the change on the virtualised resource. | Inherited from Signal VirtualisedResourceChangeNotification OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualisedResourceId | Identifier | 1 | Identifier of the instantiated virtualised resource for which the change notification is issued. This identifier value shall be the same as the one returned when the allocation of this virtualised resource was acknowledged. | Inherited from Signal VirtualisedResourceChangeNotification OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vimId | Identifier | 1 | Identifier of the VIM reporting the change. | Inherited from Signal VirtualisedResourceChangeNotification OpenModelAttribute - isInvariant: false - support: MANDATORY |
| changeType | String | 1 | It categorizes the type of change. Possible values can be related to maintenance and operation of the NFVI, including e.g., normal, maintenance, evacuation, optimization, etc | Inherited from Signal VirtualisedResourceChangeNotification OpenModelAttribute - isInvariant: false - support: MANDATORY |
| changedResourceData |  | 0..1 | Details of the changes of the resource Its content can differ based on the different values of the attribute changeType. | Inherited from Signal VirtualisedResourceChangeNotification OpenModelAttribute - isInvariant: false - support: MANDATORY |

### QuotaModule

#### Overview

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::QuotaModule

##### Diagrams

Figure B.1: Diagram Quota Resources at VNFM level

##### Notifications

###### VirtualisedResourceQuotaAvailableNotification  notification

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::QuotaModule::VirtualisedResourceQuotaAvailableNotification

Description:

This notification indicates the availability of a quota applicable to the consumer.

Applied Stereotypes:

OpenModelNotification

triggerConditionList:

A virtualised resources quota applicable to the consumer has been set.

support:    CONDITIONAL_MANDATORY

condition:   Only present in IFA007.  Support of this notification is mandatory if the Virtualised Resources Quota Available Notification interface is supported.

Preliminary

Table B.1: Attributes for Signal VirtualisedResourceQuotaAvailableNotification

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| resourceGroupId | Identifier | 1 | Unique identifier of the "infrastructure resource group", logical grouping of virtual resources assigned to a tenant within an Infrastructure Domain. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vimConnectionInfo | VimConnectionInfo | 0..1 | Information about the VIM connection to manage the virtualised resources quota. | Experimental OpenModelAttribute - isInvariant: false - support: CONDITIONAL_MANDATORY - condition: This attribute shall be supported when VNF-related Resource Management in direct mode is applicable. |
| resourceProviderId | Identifier | 0..1 | Identifies the entity responsible for the management of the virtualised resources quota. | OpenModelAttribute - isInvariant: false - support: CONDITIONAL_MANDATORY - condition: This attribute shall be supported when VNF-related Resource Management in indirect mode is applicable. |

### VnfConfigurationModule

#### Overview

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfConfigurationModule

Obsolete

##### Diagrams

Figure B.1: Diagram Vnf Configuration Resources

##### Notifications

###### VnfConfigAttributeValueChangeNotification notification

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfConfigurationModule::VnfConfigAttributeValueChangeNotification

Description:

This notification informs the receiver of updates in the VNF configuration attribute values.

Applied Stereotypes:

Obsolete

OpenModelNotification

triggerConditionList:

Modification of VNF configuration parameters explicitly through Modify VNF configuration operation.

support:    MANDATORY

Table B.1: Attributes for Signal VnfConfigAttributeValueChangeNotification

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vnfInstanceId | Identifier | 1 | The identifier of the VNF instance related to the configuration update. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| changedConfig |  | 1 | Information about the changed VNF configuration. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

##### Datatypes

###### CpAddress datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfConfigurationModule::CpAddress

Description:

This data type provides the list of attributes associated to a CP instance address.

Applied Stereotypes:

Experimental

Table B.1: Attributes for Data Type CpAddress

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| layerProtocol | LayerProtocol | 0..1 | Identifies which protocol the CP uses for connectivity purposes. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| address |  | 0..1 | The address assigned to the CP instance (e.g. IP address, MAC address, etc.). It shall be provided for configuring a fixed address. NOTE: The address information shall be compatible with the layerProtocol attribute. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| useDynamicAddress | Boolean | 0..1 | It determines whether an address shall be assigned dynamically. It shall be provided if a dynamic address needs to be configured on the CP. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| port |  | 0..1 | The port assigned to the CP instance (e.g. IP port number, Ethernet port number, etc.). | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### CpConfiguration datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfConfigurationModule::CpConfiguration

Description:

This data type provides the list of attributes for the configuration of a CP instance.

Applied Stereotypes:

Experimental

Table B.1: Attributes for Data Type CpConfiguration

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| cpId | Identifier | 1 | Uniquely identifies a CP instance within the namespace of a specific VNF instance or VNFC instance. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| cpLabel |  | 1 | Uniquely identifies a type of CP instance within the namespace of a VNFD. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| address | CpAddress | 1..* | Address and Port assigned to the CP. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfConfiguration datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfConfigurationModule::VnfConfiguration

Description:

This data type provides the list of attributes for the configuration of a VNF instance.

Applied Stereotypes:

Experimental

Table B.1: Attributes for Data Type VnfConfiguration

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| cpConfiguration | CpConfiguration | 0..* | External CPs. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| dhcpServer |  | 0..1 | Identifies a DHCP server that the VNF can use to obtain IP addresses to be assigned to its external CPs. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfSpecificData | KeyValuePair | 0..* | Configuration object containing values of VNF configurable properties (see VnfConfigurableProperties in clause 7.1.12 of ETSI GS NFV-IFA 011) applicable to whole VNF instance. NOTE 1: Cardinality "0" applies to information elements that are not requested to be modified. NOTE 2: These data can be generated by LCM scripts available in the VNF package and executed by the VNFM, or be provided by the EM or NFVO.= | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfcConfiguration datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfConfigurationModule::VnfcConfiguration

Description:

This data type provides the list of attributes for the configuration of a VNFC instance.

Applied Stereotypes:

Experimental

Table B.1: Attributes for Data Type VnfcConfiguration

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vnfcId | Identifier | 1 | Uniquely identifies a VNFC instance within the namespace of a specific VNF instance. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| cp | CpConfiguration | 0..* | Internal CPs. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| dhcpServer |  | 0..1 | Identifies a DHCP server that the VNF can use to obtain IP addresses to be assigned to its CPs. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfcSpecificData | KeyValuePair | 0..* | Configuration object containing values of VNFC configurable properties (see VnfcConfigurableProperties in clause 7.1.6.7 of ETSI GS NFV-IFA 011 applicable to a specific VNFC instance. NOTE: These data can be generated by LCM scripts available in the VNF package and executed by the VNFM, or be provided by the EM or NFVO. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfcConfigurationKvp datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfConfigurationModule::VnfcConfigurationKvp

Description:

This information element provides the list of key value pairs for the configuration of a VNFC instance.

Applied Stereotypes:

Experimental

Table B.1: Attributes for Data Type VnfcConfigurationKvp

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vnfcId | Identifier | 1 | Uniquely identifies a VNFC instance within the namespace of a specific VNF instance. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfcConfigKvp | KeyValuePair | 1..* | Configuration data for the VNFC instance. Configuration data can include values for the properties that are declared as configurable in the VNFD (see VnfcConfigurableProperties in clause 7.1.6.7 of ETSI GS NFV-IFA 011). | OpenModelAttribute - isInvariant: false - support: MANDATORY |

### VnfGrantingModule

#### Overview

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfGrantingModule

##### Diagrams

Figure B.1: Diagram Granting Resources

##### Classes

###### GrantInfo class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfGrantingModule::GrantInfo

Description:

This class contains information about a Compute, storage or network resource whose addition/update/deletion was granted in a GrantVnfLifecycleOperationResponse.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class GrantInfo

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| resourceDefinitionId | ResourceDefinition | 1 | Identifier of the related ResourceDefinition iclass from the grant request. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| reservationId | Identifier | 0..1 | The reservation identifier applicable to the VNFC/VirtualLink/VirtualStorage. It shall be present for new resources when policy is GRANT_RESERVE_MULTI and an applicable reservation exists; shall not be present otherwise. | OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: |
| vimConnectionId | VimConnectionInfo | 0..1 | Reference to the identifier of the VimConnectionInfo information element defining the VIM connection to be used to manage this resource. Shall be present for new resources, and shall be absent for resources that have already been allocated. | Experimental OpenModelAttribute - isInvariant: false - support: CONDITIONAL_MANDATORY - condition: This parameter shall be supported when VNF-related Resource Management in direct mode is applicable. PassedByReference |
| resourceProviderId | Identifier | 0..1 | Identifies the entity responsible for the management of the virtualized resource. Shall be present for new resources, and may be absent for resources that have already been allocated | OpenModelAttribute - isInvariant: false - support: CONDITIONAL_MANDATORY - condition: This parameter shall be supported when VNF-related Resource Management in indirect mode is applicable. |
| zoneId | ZoneInfo | 0..1 | Reference to the identifier of the ZoneInfo iclass defining the resource zone into which this resource is to be placed. Shall be present for new resources, and may be absent for resources that have already been allocated. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| resourceGroupId | Identifier | 0..1 | Identifier of the "infrastructure resource group", logical grouping of virtual resources assigned to a tenant within an Infrastructure Domain, to be provided when allocating the resource. If the VIM connection referenced by "vimConnectionId" applies to multiple infrastructure resource groups, this attribute shall be present for new resources. If the VIM connection referenced by "vimConnectionId" applies to a single infrastructure resource group, this attribute may be present for new resources. This attribute shall be absent for resources that have already been allocated. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### ResourceDefinition class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfGrantingModule::ResourceDefinition

Description:

This class provides information of an existing or proposed resource used by the VNF.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class ResourceDefinition

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| resourceDefinitionId | Identifier | 1 | Identifier of this ResourceDefinition class, unique at least within the scope of the Grant request. | OpenModelAttribute - isInvariant: true - support: MANDATORY |
| type | ResourceDefinitionType | 1 | Type of the resource definition referenced (e.g. Compute, VL, Storage etc.) | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vdu | Vdu | 0..1 | Reference to the related Vdu applicable to this resource in the VNFD. Shall only be present if a VDU is applicable to this resource in the VNFD. | OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: PassedByReference |
| resourceTemplateId | Identifier | 0..1 | Reference to a resource template (VnfVirtualLinkDesc, VirtualComputeDesc, VnfExtCpd, VirtualStorageDesc) in the VNFD. Shall be present for the planned creation of new resources, including temporary resources, and for the modification of existing resources. Shall be absent otherwise. | OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: |
| resourceHandle | ResourceHandle | 1 | Resource information for an existing resource. Shall be present for resources that are planned to be deleted or modified. Shall be absent otherwise. | OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: |

###### ZoneGroupInfo class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfGrantingModule::ZoneGroupInfo

Description:

This class provides information regarding a resource zone group. A resource zone group is a group of one or more related resource zones which can be used in resource placement constraints. To fulfil such constraint, the NFVO may decide to place a resource into any zone that belongs to a particular group.

NOTE: A resource zone group can be used to support overflow from one resource zone into another, in case a particular deployment supports only non-elastic resource zones

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class ZoneGroupInfo

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| zoneId | ZoneInfo | 1..* | References of identifiers of ZoneInfo instances, each of which provides information about a resource zone that belongs to this group. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |

###### ZoneInfo class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfGrantingModule::ZoneInfo

Description:

This class provides information regarding a resource zone.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class ZoneInfo

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| zoneInfoId | Identifier | 1 | The identifier of this ZoneInfo instance, for the purpose of referencing it from other classes. | OpenModelAttribute - isInvariant: true - support: MANDATORY |
| zoneId | Identifier | 1 | The identifier of the resource zone, as managed by the resource management layer (typically, the VIM). | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vimConnectionId | VimConnectionInfo | 1 | The identifier of the connection to the VIM that manages the resource zone. | Experimental OpenModelAttribute - isInvariant: false - support: CONDITIONAL_MANDATORY - condition: This attribute shall be supported when VNF-related Resource Management in direct mode is applicable. PassedByReference |
| resourceProviderId | Identifier | 1 | Identifies the entity responsible for the management the resource zone. | OpenModelAttribute - isInvariant: false - support: CONDITIONAL_MANDATORY - condition: This parameter shall be supported when VNF-related Resource Management in indirect mode is applicable |

##### Datatypes

###### ConstraintResourceRef datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfGrantingModule::ConstraintResourceRef

Description:

This datatype references a resource either by its VIM-level identifier for existing resources, or by the identifier of a resourceDefinition class in the grant request for new resources.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type ConstraintResourceRef

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| idType | ConstraintResourceReferenceType | 1 | The type of the identifier: "ResMgmt" (Resource-management-level identifier; this identifier is managed by the VIM in direct mode and is managed by the NFVO in indirect mode) or "Grant" (reference to identifier in the ResourceDefinition in the grant request). | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| resourceId | Identifier | 1 | An actual resource-management-level identifier (idType=ResMgmt), or an identifier that references the ResourceDefinition in the related grant request (idType=Grant). | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vimConnectionId | VimConnectionInfo | 0..1 | Identifier of the VIM Connection. It shall only be present when idType = ResMgmt. | Experimental OpenModelAttribute - isInvariant: false - support: CONDITIONAL_MANDATORY - condition: It shall be supported when VNF-related resource management in direct mode is applicable. PassedByReference |
| resourceProviderId | Identifier | 0..1 | Identifier of the resource provider. It shall only be present when idType = ResMgmt. | OpenModelAttribute - isInvariant: false - support: CONDITIONAL_MANDATORY - condition: It shall be supported when VNF-related resource management in indirect mode is applicable. |

###### ConstraintResourceReferenceType enumeration

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfGrantingModule::ConstraintResourceReferenceType

Description:

Defines the type of the reference

Applied Stereotypes:

Preliminary

Contains Enumeration Literals:

RES_MGMT

VIM-level identifier

GRANT

Reference to identifier in the ResourceDefinition in the Grant request

###### PlacementConstraint datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfGrantingModule::PlacementConstraint

Description:

This datatype provides information regarding a resource placement constraint. A set of such constraints may be sent by the VNFM to the NFVO to influence the resource placement decisions made by the NFVO as part of the granting process.

A placement constraint defines a condition to the placement of a new and existing resources.

EXAMPLE: The following rules influence the placement of a set of resources such that they are placed in the same Network Function Virtualisation Infrastructure Point of Presence (NFVI-PoP) but in different resource zones:

{type="affinity"; scope="NFVI-PoP"; {resource1,resource2}}

{type="anti-affinity"; scope="Zone"; {resource1,resource2}}

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type PlacementConstraint

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| affinityOrAntiAffinity | AffinityOrAntiAffinity | 1 | The type of the constraint: "affinity" or "anti-affinity". | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| scope | AffinityOrAntiAffinityScope | 1 | The scope of the placement constraint indicating the category of the "place" where the constraint applies. Possible values are "NFVI-PoP", "Zone", "ZoneGroup", "NFVI-node". | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| resource | ConstraintResourceRef | 1..* | References to resources in the constraint rule. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### ResourceDefinitionType enumeration

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfGrantingModule::ResourceDefinitionType

Description:

Type of the resource definition referenced (e.g. Compute, VL, Storage etc.)

Applied Stereotypes:

Preliminary

###### VimAssets datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfGrantingModule::VimAssets

Description:

This datatype contains references to the asset which are defined in VNFD and managed in the VIM by the NFVO, such as compute resource flavours and/or software images.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type VimAssets

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| computeResourceFlavour | VimComputeResourceFlavour | 0..* | Mappings between virtual compute descriptors defined in the VNFD and compute resource flavours managed in the VIM. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| softwareImage | VimSoftwareImage | 0..* | Mappings between software images defined in the VNFD and software images managed in the VIM. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VimComputeResourceFlavour datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfGrantingModule::VimComputeResourceFlavour

Description:

If the VIM requires the use of virtual compute resource flavours during compute resource instantiation, it is assumed that such flavours are selected or created by the NFVO based on the information in the VirtualComputeDesc classes defined in the VNFD.

This datatype defines the mapping between a VirtualComputeDesc in the VNFD and the corresponding compute resource flavour managed by the NFVO in the VIM.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type VimComputeResourceFlavour

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vimConnectionId | VimConnectionInfo | 0..1 | Identifier of the VIM connection to access the assets listed in this information element. | Experimental OpenModelAttribute - isInvariant: false - support: CONDITIONAL_MANDATORY - condition: Shall be supported and present if VNF-related resource management in direct mode is applicable. PassedByReference |
| resourceProviderId | Identifier | 0..1 | Identifies the entity responsible for the management of the virtualized resource. | OpenModelAttribute - isInvariant: false - support: CONDITIONAL_MANDATORY - condition: Shall be supported and present if VNF-related resource management in indirect mode is applicable. |
| vnfdVirtualComputeDescId | VirtualComputeDesc | 1 | Identifier which references the VirtualComputeDesc in the VNFD that maps to this flavour. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vimFlavourId | Identifier | 1 | Identifier of the compute resource flavour in the resource management layer (i.e. VIM). | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VimConstraint datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfGrantingModule::VimConstraint

Description:

This datatype provides information regarding a VIM selection constraint. A set of such constraints may be sent by the VNFM to the NFVO to influence the VIM selection decisions made by the NFVO as part of the granting process.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type VimConstraint

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| sameResourceGroup | Boolean | 0..1 | If present and set to true, this signals that the constraint applies not only to the same VIM connection, but also to the same infrastructure resource group. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| resource | ConstraintResourceRef | 1..* | References to resources in the constraint rule. The NFVO shall ensure that all resources in this list are managed through by the same VIM connection. If "sameResourceGroup" is set to true, the NFVO shall further ensure that all resources in this list are part of the same infrastructure resource group in that VIM connection. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VimSoftwareImage datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfGrantingModule::VimSoftwareImage

Description:

This datatype contains a mapping between a software image definition the VNFD and the corresponding software image managed by the NFVO in the VIM which is needed during compute resource instantiation.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type VimSoftwareImage

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vimConnectionId | VimConnectionInfo | 0..1 | Identifier of the VIM connection to access the assets listed in this information element. | Experimental OpenModelAttribute - isInvariant: false - support: CONDITIONAL_MANDATORY - condition: Shall be supported and present if VNF-related resource management in direct mode is applicable. PassedByReference |
| resourceProviderId | Identifier | 0..1 | Identifier used by NFVO to determine the entity responsible for the management of the VIM asset. | OpenModelAttribute - isInvariant: false - support: CONDITIONAL_MANDATORY - condition: Shall be supported and present if VNF-related resource management in indirect mode is applicable. |
| vnfdSoftwareImageId | SwImageDesc | 1 | Identifier of the software image descriptor in the VNFD. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vimSoftwareImageId | Identifier | 1 | Identifier of the software image in the resource management layer (i.e. VIM). | OpenModelAttribute - isInvariant: false - support: MANDATORY |

##### Associations

###### GrantInfoAppliesOnResourceDefinition association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfGrantingModule::GrantInfoAppliesOnResourceDefinition

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association GrantInfoAppliesOnResourceDefinition

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| resourceDefinitionId | composite | Navig. | 1 | ResourceDefinition | Identifier of the related ResourceDefinition iclass from the grant request. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| grantinfo | none | Not navig. | 0..1 | GrantInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### GrantInfoHasApplicalbeVimInfo association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfGrantingModule::GrantInfoHasApplicalbeVimInfo

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association GrantInfoHasApplicalbeVimInfo

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| vimConnectionId | none | Navig. | 0..1 | VimConnectionInfo | Reference to the identifier of the VimConnectionInfo information element defining the VIM connection to be used to manage this resource. Shall be present for new resources, and shall be absent for resources that have already been allocated. | Experimental OpenModelAttribute - isInvariant: false - support: CONDITIONAL_MANDATORY - condition: This parameter shall be supported when VNF-related Resource Management in direct mode is applicable. PassedByReference |
| grantInfo | none | Not navig. | 0..1 | GrantInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### GrantinfoHasApplicableZoneInfo association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfGrantingModule::GrantinfoHasApplicableZoneInfo

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association GrantinfoHasApplicableZoneInfo

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| zoneId | none | Navig. | 0..1 | ZoneInfo | Reference to the identifier of the ZoneInfo iclass defining the resource zone into which this resource is to be placed. Shall be present for new resources, and may be absent for resources that have already been allocated. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| grantinfo | none | Not navig. | 0..1 | GrantInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### ResourceDefinitionHasApplicableVdu association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfGrantingModule::ResourceDefinitionHasApplicableVdu

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association ResourceDefinitionHasApplicableVdu

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| resourcedefinition | none | Not navig. | 0..1 | ResourceDefinition |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vdu | none | Navig. | 0..1 | Vdu | Reference to the related Vdu applicable to this resource in the VNFD. Shall only be present if a VDU is applicable to this resource in the VNFD. | OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: PassedByReference |

###### ZoneGroupInfoGroupsZoneInfo association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfGrantingModule::ZoneGroupInfoGroupsZoneInfo

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association ZoneGroupInfoGroupsZoneInfo

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| zoneId | shared | Navig. | 1..* | ZoneInfo | References of identifiers of ZoneInfo instances, each of which provides information about a resource zone that belongs to this group. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| zoneGroup | none | Not navig. | 0..* | ZoneGroupInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### ZoneManagedByVim association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfGrantingModule::ZoneManagedByVim

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association ZoneManagedByVim

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| vimConnectionId | none | Navig. | 1 | VimConnectionInfo | The identifier of the connection to the VIM that manages the resource zone. | Experimental OpenModelAttribute - isInvariant: false - support: CONDITIONAL_MANDATORY - condition: This attribute shall be supported when VNF-related Resource Management in direct mode is applicable. PassedByReference |
| zoneInfo | none | Not navig. | 0..* | ZoneInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

### VnfIndicatorModule

#### Overview

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfIndicatorModule

##### Diagrams

Figure B.1: Diagram Indicator resources

##### Classes

###### IndicatorInformation class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfIndicatorModule::IndicatorInformation

Description:

This class provides the indicator values of a VNF instance.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class IndicatorInformation

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vnfInstanceId | Identifier | 1 | Identifies the VNF instance which provides the indicator value(s). | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| indicator | VnfIndicator | 1 | Identifies the indicator. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| indicatorValue | Value | 1 | Provides the value of the indicator. The value format is defined in the VNFD. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| indicatorName | String | 0..1 | Human readable name of the indicator. Shall be present if defined in the VNFD. | OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: Shall be present if defined in the VNFD. |

##### Notifications

###### IndicatorValueChangeNotification notification

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfIndicatorModule::IndicatorValueChangeNotification

Description:

This notification informs the receiver of a value change of an indicator related to the VNF.

Applied Stereotypes:

OpenModelNotification

triggerConditionList:

The value of an indicator has changed.

support:    MANDATORY

Preliminary

Table B.1: Attributes for Signal IndicatorValueChangeNotification

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| indicatorInformation | IndicatorInformation | 1 | This is to provide the indicator, the value of the indicator and the VNF instance the indicator is related to. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

### VnfLcmModule

#### Overview

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule

Description:

The state of the VNFC instance.

##### Diagrams

Figure B.1: Diagram VNF LCM Resources

Figure B.1: Diagram VNF Classes Mapping

##### Classes

###### ExtLinkPortInfo class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::ExtLinkPortInfo

Description:

This information element provides information about a port of an external VL, i.e., a port providing connectivity for the VNF to an NS VL.

Parent class: LinkPort

Applied Stereotypes:

Experimental

OpenModelClass

support:  MANDATORY

Table B.1: Attributes for Class ExtLinkPortInfo

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| extLinkPortId | Identifier | 1 | Identifier of this link port as provided by the entity that has created the link port. | Experimental OpenModelAttribute - isInvariant: true - support: MANDATORY |
| resourceHandle | ResourceHandle | 1 | Reference to the virtualised resource realizing this link port. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| cpInstanceId | VnfExtCpInfo | 0..1 | External CP of the VNF connected to this link port. NOTE: There shall be at most one link port associated with any external connection point instance. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| cp | Cp | 0..1 | CP to be connected to this link port. | Inherited from Class LinkPort OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### ExtManagedVirtualLinkInfo  class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::ExtManagedVirtualLinkInfo

Description:

This class provides a reference to an externally-managed internal VL.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class ExtManagedVirtualLinkInfo

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| extManagedVirtualLinkId | Identifier | 1 | Identifier of this externally-managed internal VL. The identifier is assigned by the NFV-MANO entity that manages this VL instance. | Experimental OpenModelAttribute - isInvariant: true - support: MANDATORY |
| virtualLinkResourceInfoId | VnfVirtualLinkResourceInfo | 1 | Reference to the VirtualLinkResourceInfo class which represents this externally-managed internal VL as part of the VnfInfo information element.. | Obsolete OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vnfVirtualLinkDescId | VnfVirtualLinkDesc | 1 | Identifier of the VNF Virtual Link Descriptor (VLD) in the VNFD. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| networkResource | ResourceHandle | 1 | Reference to the VirtualNetwork resource. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfLinkPort | VnfLinkPortInfo | 0..* | Link ports of this VL. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### ExtVirtualLinkInfo class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::ExtVirtualLinkInfo

Description:

This class provides a reference to an external VL.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class ExtVirtualLinkInfo

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| extVirtualLinkId | Identifier | 1 | Identifier of this external VL. The identifier is assigned by the NFV-MANO entity that manages this VL instance. | Experimental OpenModelAttribute - isInvariant: true - support: MANDATORY |
| resourceHandle | ResourceHandle | 1 | Reference to the resource realizing this VL. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| extLinkPort | ExtLinkPortInfo | 0..* | Link ports of this VL. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### InstantiatedVnfInfo class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::InstantiatedVnfInfo

Description:

This class provides run-time information specific to an instantiated VNF instance.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class InstantiatedVnfInfo

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| flavourId | VnfDf | 1 | Identifier of the VNF DF applied to this VNF instance. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vnfState | VnfState | 1 | The state of the VNF instance. Permitted values include: STARTED, STOPPED. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| scaleStatus | ScaleInfo | 0..* | Scale status of the VNF, one entry per aspect. Shall be present if the VNF supports scaling. Represents for every scaling aspect how "big" the VNF has been scaled w.r.t. that aspect. NOTE: For every scaling aspect, together with the information provided by the "maxScaleLevel" attribute of the ScalingAspect class in the VNFD, this allows an external entity to derive how many scaling steps are possible for scaling in or scaling out a VNF instance. Per aspect, the number of steps possible to scale in corresponds to the "scaleLevel" attribute for that aspect in the "scaleStatus" information element, and the possible number of steps to scale out corresponds to the difference between "maxScaleLevel" for that aspect, and the "scaleLevel" attribute for that aspect in the "scaleStatus" information element. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| extCpInfo | VnfExtCpInfo | 1..* | External CPs exposed by the VNF instance. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| extVirtualLinkInfo | ExtVirtualLinkInfo | 0..* | External VLs the VNF instance is connected to. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| extManagedVirtualLinkInfo | ExtManagedVirtualLinkInfo | 0..* | Externally-managed internal VLs of the VNF instance. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| monitoringParameter |  | 0..* | Performance metrics tracked by VNFM (e.g. for auto-scaling purposes) and their current (as known to the VNFM) values. NOTE: The monitoring parameters to be tracked by VNFM are identified by VNF provider in the VNFD. The VNFM collects the values of identified performance metrics using one or more locally initiated PM Jobs. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| localizationLanguage |  | 0..1 | Information about localization language of the VNF (includes e.g. strings in the VNFD). The localization languages supported by a VNF can be declared in the VNFD, and localization language selection can take place at instantiation time. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vimInfo | VimConnectionInfo | 0..* | Information about VIM(s) managing resources for the VNF instance. | Obsolete OpenModelAttribute - isInvariant: false - support: CONDITIONAL_MANDATORY - condition: Shall be supported and present if VNF-related resource management in direct mode is applicable. |
| vnfcResourceInfo | VnfcResourceInfo | 0..* | Information on the virtualised compute and storage resource(s) used by the VNFCs of the VNF instance. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfVirtualLinkResourceInfo | VnfVirtualLinkResourceInfo | 0..* | Information on the virtualised network resource(s) used by the VLs of the VNF instance. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualStorageResourceInfo | VirtualStorageResourceInfo | 0..* | Information on the virtualised storage resource(s) used as storage for the VNF instance. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfcInfo | VnfcInfo | 0..* |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VirtualStorageResourceInfo class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::VirtualStorageResourceInfo

Description:

This class provides information on virtualised storage resources used by a storage instance in a VNF.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VirtualStorageResourceInfo

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| virtualStorageInstanceId | Identifier | 1 | Identifier of this virtual storage resource instance. | OpenModelAttribute - isInvariant: true - support: MANDATORY |
| virtualStorageDescId | VirtualStorageDesc | 1 | Identifier of the VirtualStorageDesc in the VNFD. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| storageResource | ResourceHandle | 1 | Reference to the VirtualStorage resource. Information about the resource is available from the Virtualised Storage Resource Management interface | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| reservationId | Identifier | 0..1 | The reservation identifier applicable to the resource. It shall be present when an applicable reservation exists. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| metadata | KeyValuePair | 0..* | Metadata about this resource. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfCpInfo class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::VnfCpInfo

This class is abstract.

Parent class: CpInfo

Applied Stereotypes:

Experimental

OpenModelClass

support:  MANDATORY

Table A.1: Attributes for Class VnfCpInfo

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |

###### VnfExtCpInfo class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::VnfExtCpInfo

Description:

This class provides information related to an external CP.

Parent class: VnfCpInfo

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VnfExtCpInfo

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| cpInstanceId | Identifier | 1 | Identifier of this external CP instance and of this VnfExtCpInfo class. | Experimental OpenModelAttribute - isInvariant: true - support: MANDATORY |
| cpdId | VnfExtCpd | 1 | Identifier of the external Connection Point Descriptor (CPD), VnfExtCpd, in the VNFD. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| address |  | 0..* | List of network addresses that have been configured (statically or dynamically) on the the link port that connects the CP to a VL. | Obsolete OpenModelAttribute - isInvariant: false - support: MANDATORY |
| extLinkPort | ExtLinkPortInfo | 0..1 | Identifier of the "ExtLinkPortInfo" information element in the "ExtVirtualLinkInfo" information element. Shall be present if the CP is associated to a link port. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| cpProtocolInfo | CpProtocolInfo | 0..* | Protocol information for this CP. There shall be one cpProtocolInfo for each layer protocol supported. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfInfo class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::VnfInfo

Description:

This class provides run-time information about a VNF instance.

Note: In ETSI GS NFV-MAN 001, the concept of the VNF record (VNFR) was introduced which is a model for the totality of information managed by the VNFM regarding a running VNF instance. VNFR is not used in the present document. The VnfInfo class defined below exposes a part of the VNF instance runtime information.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VnfInfo

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vnfInstanceId | Identifier | 1 | Identifier of the VNF instance that is represented by this VnfInfo class. | OpenModelAttribute - isInvariant: true - support: MANDATORY |
| vnfInstanceName | String | 0..1 | VNF instance name. NOTE: This attribute in the VnfInfo shall be writable through the Modify VNF information operation | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfInstanceDescription | String | 0..1 | Human-readable description of the VNF instance. NOTE: This attribute in the VnfInfo shall be writable through the Modify VNF information operation | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfdId | Vnfd | 1 | Identifier of the VNFD on which the VNF instance is based. NOTE 2: This identifier, which is managed by the VNF provider, identifies the VNF Package and the VNFD in a globally unique way. NOTE 3: This information is copied from the VNFD of the on-boarded VNF Package which was used to instantiate the VNF instance. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vnfProvider | String | 1 | Note: This information is copied from the VNFD of the on-boarded VNF Package which was used to instantiate the VNF instance | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfProductName | String | 1 | Note: This information is copied from the VNFD of the on-boarded VNF Package which was used to instantiate the VNF instance | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfSoftwareVersion | Version | 1 | Note: This information is copied from the VNFD of the on-boarded VNF Package which was used to instantiate the VNF instance. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfdVersion | Version | 1 | Note: This information is copied from the VNFD of the on-boarded VNF Package which was used to instantiate the VNF instance. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| onboardedVnfPkgInfoId | VnfPkgInfo | 1 | Identifier of information held by the NVFO about the specific VNF Package on which the VNF is based. This identifier was allocated by the NVFO. NOTE 1: This attribute in the VnfInfo shall be writable through the Modify VNF information operation NOTE 4: Modifying the value of this attribute can be performed when no conflicts exist between the previous and the newly referred VNF Package, e.g., when the new VNFD is not changed with respect to the previous VNFD apart from referencing to other VNF software image(s). In order to avoid misalignment of the VnfInfo with the current VNF's on-boarded VNF Package, the values copied from the VNFD of the on-boarded VNF Package (see note 3) need to be kept in sync. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vnfConfigurableProperty | KeyValuePair | 0..* | Current values of the configurable properties of the VNF instance. Configurable properties referred in this attribute are declared in the VNFD (see clause 7.1.12 in ETSI GS NFV IFA 011). NOTE 1: This attribute in the VnfInfo shall be writable through the Modify VNF information operation (refer to clause 7.2.12). NOTE 5: VNF configurable properties are sometimes also referred to as configuration parameters applicable to a VNF. Some of these are set prior to instantiation and cannot be modified if the VNF is instantiated, some are set prior to instantiation (are part of initial configuration) and can be modified later, and others can be set only after instantiation. The applicability of certain configuration may depend on the VNF and the required operation of the VNF at a certain point in time. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vimConnectionInfo | VimConnectionInfo | 0..* | Information about VIM connection(s) for managing resources for the VNF instance. NOTE: This attribute in the VnfInfo shall be writable through the Modify VNF information operation | Experimental OpenModelAttribute - isInvariant: false - support: CONDITIONAL_MANDATORY - condition: Shall be supported and present if VNF-related resource management in direct mode is applicable. |
| instantiationState | InstantiationState | 1 | The instantiation state of the VNF instance. Possible values: NOT_INSTANTIATED (VNF instance is terminated or not instantiated , and the identifier of the VNF instance exists), INSTANTIATED (VNF instance is instantiated). | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| instantiatedVnfInfo | InstantiatedVnfInfo | 0..1 | Information specific to an instantiated VNF instance. Shall be present if the VNF is in INSTANTIATED instantiation state. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| metadata | KeyValuePair | 0..* | Additional VNF-specific metadata describing the VNF instance. Metadata that are writeable are declared in the VNFD (see clause 7.1.14.2 in ETSI GS NFV-IFA 011). NOTE: This attribute in the VnfInfo shall be writable through the Modify VNF information operation (refer to clause 7.2.12). | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| extension | KeyValuePair | 0..* | VNF-specific attributes that affect the lifecycle management of this VNF instance by the VNFM, or the lifecycle management scripts. Extensions that are writeable are declared in the VNFD (see clause 7.1.14.2 in ETSI GS NFV-IFA 011). NOTE: This attribute in the VnfInfo shall be writable through the Modify VNF information operation (refer to clause 7.2.12). | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfLinkPortInfo class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::VnfLinkPortInfo

Description:

This class provides information about a port of a VNF VL.

Applied Stereotypes:

Experimental

OpenModelClass

support:  MANDATORY

Table B.1: Attributes for Class VnfLinkPortInfo

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vnfLinkPortId | Identifier | 1 | Identifier of this link port as provided by the entity that has created the link port. | Experimental OpenModelAttribute - isInvariant: true - support: MANDATORY |
| resourceHandle | ResourceHandle | 1 | Reference to the virtualised resource realizing this link port. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| cpInstanceId | VnfcCpInfo | 0..1 | External CP of the VNF connected to this link port. Shall be present when the link port is used for external connectivity by the VNF. May be present if used to reference a VNFC CP NOTE: There shall be at most one link port associated with any external connection point instance or internal connection point (i.e., VNFC CP) instance. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |

###### VnfVirtualLinkResourceInfo class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::VnfVirtualLinkResourceInfo

Description:

This class provides information on virtualised network resources used by a VL instance in a VNF.

Applied Stereotypes:

Experimental

OpenModelClass

support:  MANDATORY

Table B.1: Attributes for Class VnfVirtualLinkResourceInfo

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| virtualLinkInstanceId | Identifier | 1 | Identifier of this VL instance. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfVirtualLinkDescId | VnfVirtualLinkDesc | 1 | Identifier of the Virtual Link Descriptor (VLD) in the VNFD. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| networkResource | ResourceHandle | 1 | Reference to the VirtualNetwork resource. Information about the resource is available from the Virtualised Network Resource Management interface. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| reservationId | Identifier | 0..1 | The reservation identifier applicable to the resource. It shall be present when an applicable reservation exists. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfLinkPort | VnfLinkPortInfo | 0..* | Links ports of this VL. Shall be present when the linkPort is used for external connectivity by the VNF. May be present otherwise. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| metadata | KeyValuePair | 0..* | Metadata about this resource. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfcCpInfo class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::VnfcCpInfo

Description:

This information element provides information related to a CP of a VNFC.

Parent class: VnfCpInfo

Applied Stereotypes:

Experimental

OpenModelClass

support:  MANDATORY

Table B.1: Attributes for Class VnfcCpInfo

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| cpInstanceId | Identifier | 1 | Identifier of this VnfcCpInfo information element. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| cpdId | VduCpd | 1 | Identifier of the VDU CPD in the VNFD. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vnfExtCpId | VnfExtCpInfo | 0..1 | When the VNFC CP is exposed as external CP of the VNF, the identifier of this external VNF CP. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| address |  | 0..* | List of network addresses that have been configured (statically or dynamically) on the link port that connects the CP to a VL. | Obsolete OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfLinkPortId | VnfLinkPortInfo | 0..1 | Identifier of the "VnfLinkPortInfo" information element in the "VnfVirtualLinkResourceInfo" information element. Shall be present if the CP is associated to a link port. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| cpProtocolInfo | CpProtocolInfo | 0..* | Protocol information for this CP. There shall be one cpProtocolInfo for each layer protocol supported. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfcInfo class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::VnfcInfo

Applied Stereotypes:

Experimental

OpenModelClass

condition:  Only applicable for IFA008

support:  CONDITIONAL_MANDATORY

Table B.1: Attributes for Class VnfcInfo

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vnfcInstanceId |  | 1 | Identifier of the VNFC instance that is represented by this VnfcInfo information element. | Experimental OpenModelAttribute - isInvariant: true - support: MANDATORY |
| vdu | Vdu | 1 | Identifier of the VDU in the VNFD. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfcState | VnfcState | 1 | The state of the VNFC instance. Permitted values include: STARTED, STOPPED. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfcConfigurableProperty | KeyValuePair | 0..* | Current values of the configurable properties of the VNFC instance. Configurable properties referred in this attribute are declared in the VNFD (see clause 7.1.6.7 in ETSI GS NFV IFA 011). NOTE: This attribute in the VnfInfo shall be writable through the corresponding Modify VNF information operation. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfcResourceInfo class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::VnfcResourceInfo

Description:

This class provides information on virtualised compute and storage resources used by a VNFC in a VNF instance.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VnfcResourceInfo

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vnfcInstanceId | Identifier | 1 | Identifier of this VNFC instance. | OpenModelAttribute - isInvariant: true - support: MANDATORY |
| vdu | Vdu | 0..1 | Reference to the applicable Vdu class in the VNFD. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| computeResource | ResourceHandle | 1 | Reference to the VirtualCompute resource. Detailed information about the resource is available from the Virtualised Compute Resource Management interface. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| storageResourceId | VirtualStorageResourceInfo | 0..* | Reference(s) to the VirtualStorage resource(s). Information about the resource(s) is available from the Virtualised Storage Resource Management interface. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| reservationId | Identifier | 0..1 | The reservation identifier applicable to the resource. It shall be present when an applicable reservation exists. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfcCpInfo | VnfcCpInfo | 0..* | CP(s) of the VNFC instance. Shall be present when that particular CP of the VNFC instance is associated to an external CP of the VNF instance. May be present otherwise | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| metadata | KeyValuePair | 0..* | Metadata about this resource. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

##### Notifications

###### VnfIdentifierCreationNotification  notification

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::VnfIdentifierCreationNotification

Description:

This notification informs the receiver of the creation of a new VNF instance identifier and the associated instance of a VnfInfo class, identified by that identifier.

Applied Stereotypes:

OpenModelNotification

triggerConditionList:

Creation of a VNF instance identifier and the associated instance of a VnfInfo class

support:    MANDATORY

Preliminary

Table B.1: Attributes for Signal VnfIdentifierCreationNotification

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vnfInstanceId | Identifier | 1 | The newly created VNF instance identifier. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfIdentifierDeletionNotification  notification

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::VnfIdentifierDeletionNotification

Description:

This notification informs the receiver of the deletion of a VNF instance identifier and the associated instance of a VnfInfo class identified by that identifier

Applied Stereotypes:

OpenModelNotification

triggerConditionList:

Deletion of a VNF instance identifier and the associated instance of a VnfInfo class

support:    MANDATORY

Preliminary

Table B.1: Attributes for Signal VnfIdentifierDeletionNotification

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vnfInstanceId | Identifier | 1 | The VNF instance identifier that has been deleted. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfInfoAttributeValueChangeNotification notification

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::VnfInfoAttributeValueChangeNotification

Description:

This notification informs the receiver of updates in the VNF information attribute values.

This notification is produced when there is an update in the VNF information, including configurable properties.

Applied Stereotypes:

Obsolete

OpenModelNotification

triggerConditionList:

Modification of VNF instance information explicitly through Modify VNF information operation.

support:    MANDATORY

Table B.1: Attributes for Signal VnfInfoAttributeValueChangeNotification

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vnfInstanceId | Identifier | 1 | The identifier of the VNF instance related to the information update. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| changedInfo |  | 1 | Information about the changed VNF information, including configurable properties. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfLcmOperationOccurrenceNotification notification

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::VnfLcmOperationOccurrenceNotification

Description:

This notification informs the receiver of changes in the VNF lifecycle caused by VNF lifecycle management operation occurrences.

If this is a notification about the start of an LCM operation occurence, the notification shall be sent before any action (including sending the grant request) is taken, however, after acknowledging the LCM operation request to the consumer.

If this is a notification about the result of an LCM operation occurence, the notification shall be sent after all other actions of the LCM operation have been executed.

NOTE: If this notification represents the result of a lifecycle management operation occurence that was not successful, the notification shall contain appropriate error information.

Applied Stereotypes:

Experimental

OpenModelNotification

triggerConditionList:

Instantiation of the VNF

Scaling of the VNF instance (including auto-scaling)

Healing of the VNF instance (including auto-healing)

Change of the state of the VNF instance (i.e., Operate VNF)

Change of the DF of the VNF instance

Changing the external virtual links of the VNF instance

Termination of the VNF instance

Modification of VNF instance information and/or VNF/VNFC configurable properties explicitly through Modify VNF Information operation

support:    MANDATORY

Table B.1: Attributes for Signal VnfLcmOperationOccurrenceNotification

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| status | OperationStatus | 1 | Indicates whether this notification reports about the start of a lifecycle management operation occurrence or the result of a lifecycle management operation occurrence. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfInstanceId | Identifier | 1 | The identifier of the VNF instance affected. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| operation | String | 1 | The lifecycle management operation. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| isAutomaticInvocation | Boolean | 1 | Set to true if this VNF LCM operation occurrence has been triggered by an automated procedure inside the VNFM (i.e. ScaleVnf / ScaleVnfToLevel triggered by auto-scale, or HealVnf triggered by auto-heal). Set to false otherwise. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| lifecycleOperationOccurrenceId | Identifier | 1 | The identifier of the VNF lifecycle management operation occurrence associated to the notification. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| affectedVnfc | AffectedVnfc | 0..* | Information about VNFC instances that were affected during the execution of the lifecycle management operation, if this notification represents the result of a lifecycle management operation occurrence. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| affectedVirtualLink | AffectedVirtualLink | 0..* | Information about VL instances that were affected during the execution of the lifecycle management operation, if this notification represents the result of a lifecycle management operation occurrence. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| affectedVirtualStorage | AffectedVirtualStorage | 0..* | Information about virtualised storage instances that were affected during the execution of the lifecycle managementoperation, if this notification represents the result of a lifecycle management operation occurrence. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| changedExtConnectivity | ExtVirtualLinkInfo | 0..* | Information about changed external connectivity, if this notification represents the result of a lifecycle management operation occurrence. Only relevant for the "Change External VNF Connectivity" operation. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| changedInfo |  | 0..1 | Information about the changed VNF information, including changed VNF configurable properties, if this notification represents the result of a lifecycle management operation occurrence. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

##### Datatypes

###### AffectedVirtualLink datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::AffectedVirtualLink

Description:

This datatype provides information about added, deleted, modified and temporary VLs, as well as about link port changes.

Applied Stereotypes:

Experimental

Table B.1: Attributes for Data Type AffectedVirtualLink

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| virtualLinkInstanceId | VnfVirtualLinkResourceInfo | 1 | Identifier of the VL instance. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference Preliminary |
| virtualLinkDescId | VnfVirtualLinkDesc | 1 | Identifier of the VLD in the VNFD. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference Preliminary |
| changeType | VirtualLinkChangeType | 1 | Signals the type of change including, not limited to, changes made to the characteristics of the existing VL, new VL added, existing VL removed, temporary VL exists, link port added, link port removed. For a temporary resource, an AffectedVirtualLink IE exists as long as the temporary resource exists. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| networkResource | ResourceHandle | 1 | Reference to the VirtualNetwork resource. Detailed information is (for new and modified resources) or has been (for removed resources) available from the Virtualised Network Resource Management interface. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| metadata | KeyValuePair | 0..* | Metadata about this resource. The content of this attribute shall be a copy of the content of the "metadata" attribute of the VnfVirtualLinkResourceInfo information element. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### AffectedVirtualStorage datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::AffectedVirtualStorage

Description:

This datatype provides information about added, deleted, modified and temporary virtual storage resources.

Applied Stereotypes:

Experimental

Table B.1: Attributes for Data Type AffectedVirtualStorage

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| virtualStorageInstanceId | VirtualStorageResourceInfo | 1 | Identifier of the virtual storage instance. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference Preliminary |
| virtualStorageDescId | VirtualStorageDesc | 1 | Identifier of the VirtualStorageDesc in the VNFD. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference Preliminary |
| changeType | VirtualStorageChangeType | 1 | Signals the type of change (added, removed, modified, temporary). For a temporary resource, an AffectedVirtualStorage IE exists as long as the temporary resource exists. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| storageResource | ResourceHandle | 1 | Reference to the VirtualStorage resource. Detailed information is (for new and modified resources) or has been (for removed resources) available from the Virtualised Storage Resource Management interface. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| metadata | KeyValuePair | 0..* | Metadata about this resource. The content of this attribute shall be a copy of the content of the "metadata" attribute of the VnfVirtualStorageResourceInfo information element. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### AffectedVnfc datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::AffectedVnfc

Description:

This datatype provides information about added, deleted, modified and temporary VNFCs.

Applied Stereotypes:

Experimental

Table B.1: Attributes for Data Type AffectedVnfc

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vnfcInstanceId | VnfcResourceInfo | 1 | Identifier of the VNFC instance. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference Preliminary |
| vduId | Vdu | 1 | Identifier of the VDU in the VNFD. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference Preliminary |
| changeType | VnfcChangeType | 1 | Signals the type of change (added, removed, modified, temporary). For a temporary resource, an AffectedVnfc IE exists as long as the temporary resource exists. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| computeResource | ResourceHandle | 1 | Reference to the VirtualCompute resource. Detailed information is (for new and modified resources) or has been (for removed resources) available from the Virtualised Compute Resource Management interface. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| affectedVnfcCpInstances | VnfcCpInfo | 0..* | Identifiers of CP(s) of the VNFC instance that were affected by the change. Shall be present for those affected CPs of the VNFC instance that are associated to an external CP of the VNF instance. May be present for further affected CPs of the VNFC instance. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| metadata | KeyValuePair | 0..* | Metadata about this resource. The content of this attribute shall be a copy of the content of the "metadata" attribute of the VnfcResourceInfo information element. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| storageResource | ResourceHandle | 0..* | Reference(s) to the VirtualStorage resource(s). It shall be provided if a storage resource is attached to the VNFC.. Detailed information is (for new and modified resources) or has been (for removed resources) available from the Virtualised Storage Resource Management interface. | Obsolete OpenModelAttribute - isInvariant: false - support: MANDATORY |
| addedStorageResourceIds | Identifier | 0..* | Reference(s) to VirtualStorage resource(s) that were added. Each value refers to a VirtualStorageResourceInfo item in the VnfInstance that was added to the VNFC. It shall be provided if at least one storage resource was added to the VNFC. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| removedStorageResourceIds | Identifier | 0..* | Reference(s) to VirtualStorage resource(s) that were removed. The value contains the identifier of a VirtualStorageResourceInfo item that has been removed from the VNFC, and might no longer exist in the VnfInstance. It shall be provided if at least one storage resource was removed from the VNFC. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### ScaleInfo datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::ScaleInfo

Description:

This datatype provides information about the scale level of a VNF instance w.r.t. one scaling aspect.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type ScaleInfo

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| aspectId | ScalingAspect | 1 | Reference to the scaling aspect. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| scaleLevel | Integer | 1 | The scale level for that aspect Minimum value 0, maximum value maxScaleLevel as declared in the VNFD. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VirtualLinkChangeType enumeration

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::VirtualLinkChangeType

Description:

Signals the type of change including, not limited to, changes made to the characteristics of the existing VL, new VL added, existing VL removed, link port added, link port removed.

Applied Stereotypes:

Preliminary

###### VirtualStorageChangeType enumeration

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::VirtualStorageChangeType

Description:

Signals the type of change of the Virtual Storage (added, removed, modified).

Applied Stereotypes:

Experimental

Contains Enumeration Literals:

ADDED

REMOVED

MODIFIED

TEMPORARY

###### VnfState enumeration

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::VnfState

Description:

The state of the VNF instance.

Applied Stereotypes:

Preliminary

Contains Enumeration Literals:

STARTED

STOPPED

###### VnfcChangeType enumeration

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::VnfcChangeType

Description:

Signals the type of change for a VNFC

Applied Stereotypes:

Experimental

Contains Enumeration Literals:

ADDED

MODIFIED

REMOVED

TEMPORARY

###### VnfcState enumeration

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::VnfcState

Applied Stereotypes:

Experimental

Contains Enumeration Literals:

STARTED

STOPPED

##### Associations

###### ExtLinkPortTerminatesOnExtCp association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::ExtLinkPortTerminatesOnExtCp

Applied Stereotypes:

Experimental

Table B.1: Member ends for Association ExtLinkPortTerminatesOnExtCp

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| cpInstanceId | none | Navig. | 0..1 | VnfExtCpInfo | External CP of the VNF connected to this link port. NOTE: There shall be at most one link port associated with any external connection point instance. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| extLinkPort | none | Navig. | 0..1 | ExtLinkPortInfo | Identifier of the "ExtLinkPortInfo" information element in the "ExtVirtualLinkInfo" information element. Shall be present if the CP is associated to a link port. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### ExtManageVLDeployedUsingVnfVLDesc association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::ExtManageVLDeployedUsingVnfVLDesc

Applied Stereotypes:

Experimental

Table B.1: Member ends for Association ExtManageVLDeployedUsingVnfVLDesc

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| vnfVirtualLinkDescId | none | Navig. | 1 | VnfVirtualLinkDesc | Identifier of the VNF Virtual Link Descriptor (VLD) in the VNFD. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| ExtManagedVirtualLinkInfo | none | Not navig. | 0..* | ExtManagedVirtualLinkInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### ExtManagedVirtualLinkHasLinkPort association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::ExtManagedVirtualLinkHasLinkPort

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association ExtManagedVirtualLinkHasLinkPort

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| vnfLinkPort | none | Navig. | 0..* | VnfLinkPortInfo | Link ports of this VL. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| extManagedVirtualLinkInfo | none | Not navig. | 1 | ExtManagedVirtualLinkInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### ExtManagedVirtualLinkUsesVirtualLinkResource association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::ExtManagedVirtualLinkUsesVirtualLinkResource

Applied Stereotypes:

Obsolete

Table B.1: Member ends for Association ExtManagedVirtualLinkUsesVirtualLinkResource

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| virtualLinkResourceInfoId | none | Navig. | 1 | VnfVirtualLinkResourceInfo | Reference to the VirtualLinkResourceInfo class which represents this externally-managed internal VL as part of the VnfInfo information element.. | Obsolete OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| extmanagedvirtuallinkinfo | none | Not navig. | 1 | ExtManagedVirtualLinkInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### ExtVirtualLinkHasExtLinkPort association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::ExtVirtualLinkHasExtLinkPort

Applied Stereotypes:

Experimental

Preliminary

Table B.1: Member ends for Association ExtVirtualLinkHasExtLinkPort

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| extLinkPort | composite | Navig. | 0..* | ExtLinkPortInfo | Link ports of this VL. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| extVirtualLinkInfo | none | Not navig. | 1 | ExtVirtualLinkInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### InstantiatedVnfExposesExtVnfCp association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::InstantiatedVnfExposesExtVnfCp

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association InstantiatedVnfExposesExtVnfCp

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| extCpInfo | composite | Navig. | 1..* | VnfExtCpInfo | External CPs exposed by the VNF instance. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| instantiatedVnfInfo | none | Not navig. | 1 | InstantiatedVnfInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### InstantiatedVnfHasResourcesManagedByVim association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::InstantiatedVnfHasResourcesManagedByVim

Applied Stereotypes:

Obsolete

Table B.1: Member ends for Association InstantiatedVnfHasResourcesManagedByVim

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| vimInfo | none | Navig. | 0..* | VimConnectionInfo | Information about VIM(s) managing resources for the VNF instance. | Obsolete OpenModelAttribute - isInvariant: false - support: CONDITIONAL_MANDATORY - condition: Shall be supported and present if VNF-related resource management in direct mode is applicable. |
| instantiatedVnfInfo | none | Not navig. | 0..* | InstantiatedVnfInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### InstantiatedVnfInfoHasExtManagedVirtualLink association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::InstantiatedVnfInfoHasExtManagedVirtualLink

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association InstantiatedVnfInfoHasExtManagedVirtualLink

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| extManagedVirtualLinkInfo | none | Navig. | 0..* | ExtManagedVirtualLinkInfo | Externally-managed internal VLs of the VNF instance. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| instantiatedVnfInfo | none | Not navig. | 0..1 | InstantiatedVnfInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### InstantiatedVnfInfoIsConnectedTo ExtVirtualLink association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::InstantiatedVnfInfoIsConnectedTo ExtVirtualLink

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association InstantiatedVnfInfoIsConnectedTo ExtVirtualLink

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| extVirtualLinkInfo | none | Navig. | 0..* | ExtVirtualLinkInfo | External VLs the VNF instance is connected to. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| instantiatedVnfInfo | none | Not navig. | 0..* | InstantiatedVnfInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### InstantiatedVnfInfoUsesVnfDf association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::InstantiatedVnfInfoUsesVnfDf

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association InstantiatedVnfInfoUsesVnfDf

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| flavourId | none | Navig. | 1 | VnfDf | Identifier of the VNF DF applied to this VNF instance. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| instantiatedVnfInfo | none | Not navig. | 0..1 | InstantiatedVnfInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

Applied Stereotypes:

Table B.1: Member ends for Association

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |

###### VirtualLinkResourceDeployedUsingVnfVirtualLinkDesc association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::VirtualLinkResourceDeployedUsingVnfVirtualLinkDesc

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VirtualLinkResourceDeployedUsingVnfVirtualLinkDesc

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| vnfVirtualLinkDescId | none | Navig. | 1 | VnfVirtualLinkDesc | Identifier of the Virtual Link Descriptor (VLD) in the VNFD. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| virtualLinkResourceInfo | none | Not navig. | 0..* | VnfVirtualLinkResourceInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VirtualStorageResourceDeployedUsingVirtualStorageDesc association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::VirtualStorageResourceDeployedUsingVirtualStorageDesc

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VirtualStorageResourceDeployedUsingVirtualStorageDesc

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| virtualStorageDescId | none | Navig. | 1 | VirtualStorageDesc | Identifier of the VirtualStorageDesc in the VNFD. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| virtualStorageResourceInfo | none | Not navig. | 0..* | VirtualStorageResourceInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfDeployedUsingVnfPackage association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::VnfDeployedUsingVnfPackage

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfDeployedUsingVnfPackage

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| onboardedVnfPkgInfoId | none | Navig. | 1 | VnfPkgInfo | Identifier of information held by the NVFO about the specific VNF Package on which the VNF is based. This identifier was allocated by the NVFO. NOTE 1: This attribute in the VnfInfo shall be writable through the Modify VNF information operation NOTE 4: Modifying the value of this attribute can be performed when no conflicts exist between the previous and the newly referred VNF Package, e.g., when the new VNFD is not changed with respect to the previous VNFD apart from referencing to other VNF software image(s). In order to avoid misalignment of the VnfInfo with the current VNF's on-boarded VNF Package, the values copied from the VNFD of the on-boarded VNF Package (see note 3) need to be kept in sync. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vnfInfo | none | Not navig. | 0..* | VnfInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfDeployedUsingVnfd association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::VnfDeployedUsingVnfd

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfDeployedUsingVnfd

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| vnfdId | none | Navig. | 1 | Vnfd | Identifier of the VNFD on which the VNF instance is based. NOTE 2: This identifier, which is managed by the VNF provider, identifies the VNF Package and the VNFD in a globally unique way. NOTE 3: This information is copied from the VNFD of the on-boarded VNF Package which was used to instantiate the VNF instance. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vnfinfo | none | Not navig. | 0..* | VnfInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfExtCpDeployedUsingVnfExtCpd association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::VnfExtCpDeployedUsingVnfExtCpd

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfExtCpDeployedUsingVnfExtCpd

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| cpdId | none | Navig. | 1 | VnfExtCpd | Identifier of the external Connection Point Descriptor (CPD), VnfExtCpd, in the VNFD. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vnfexternalconnectionpoint | none | Not navig. | 0..* | VnfExtCpInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfHasVirtualLinkResource association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::VnfHasVirtualLinkResource

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfHasVirtualLinkResource

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| vnfVirtualLinkResourceInfo | composite | Navig. | 0..* | VnfVirtualLinkResourceInfo | Information on the virtualised network resource(s) used by the VLs of the VNF instance. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| instantiatedVnfInfo | none | Not navig. | 1 | InstantiatedVnfInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfHasVirtualStorageResource association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::VnfHasVirtualStorageResource

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfHasVirtualStorageResource

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| virtualStorageResourceInfo | composite | Navig. | 0..* | VirtualStorageResourceInfo | Information on the virtualised storage resource(s) used as storage for the VNF instance. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| instantiatedVnfInfo | none | Not navig. | 1 | InstantiatedVnfInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfHasVnfcInfo association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::VnfHasVnfcInfo

Applied Stereotypes:

Cond

Experimental

Table B.1: Member ends for Association VnfHasVnfcInfo

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| vnfcInfo | none | Navig. | 0..* | VnfcInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| instantiatedVnfInfo | none | Not navig. | 1 | InstantiatedVnfInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfHasVnfcResource association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::VnfHasVnfcResource

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfHasVnfcResource

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| vnfcResourceInfo | composite | Navig. | 0..* | VnfcResourceInfo | Information on the virtualised compute and storage resource(s) used by the VNFCs of the VNF instance. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| instantiatedVnfInfo | none | Not navig. | 1 | InstantiatedVnfInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfInfoHasInstantiatedVnfInfo association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::VnfInfoHasInstantiatedVnfInfo

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfInfoHasInstantiatedVnfInfo

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| instantiatedVnfInfo | composite | Navig. | 0..1 | InstantiatedVnfInfo | Information specific to an instantiated VNF instance. Shall be present if the VNF is in INSTANTIATED instantiation state. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfInfo | none | Not navig. | 1 | VnfInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfLinkPortTerminatesOnVnfcCp association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::VnfLinkPortTerminatesOnVnfcCp

Applied Stereotypes:

Experimental

Table B.1: Member ends for Association VnfLinkPortTerminatesOnVnfcCp

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| cpInstanceId | none | Navig. | 0..1 | VnfcCpInfo | External CP of the VNF connected to this link port. Shall be present when the link port is used for external connectivity by the VNF. May be present if used to reference a VNFC CP NOTE: There shall be at most one link port associated with any external connection point instance or internal connection point (i.e., VNFC CP) instance. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vnfLinkPortId | none | Navig. | 0..1 | VnfLinkPortInfo | Identifier of the "VnfLinkPortInfo" information element in the "VnfVirtualLinkResourceInfo" information element. Shall be present if the CP is associated to a link port. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfVLHas VnfLinkPort association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::VnfVLHas VnfLinkPort

Applied Stereotypes:

Experimental

Table B.1: Member ends for Association VnfVLHas VnfLinkPort

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| vnfLinkPort | none | Navig. | 0..* | VnfLinkPortInfo | Links ports of this VL. Shall be present when the linkPort is used for external connectivity by the VNF. May be present otherwise. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfvirtualLinkResourceInfo | none | Not navig. | 0..1 | VnfVirtualLinkResourceInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfcCpDeployedUsingVduCpd association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::VnfcCpDeployedUsingVduCpd

Applied Stereotypes:

Experimental

Table B.1: Member ends for Association VnfcCpDeployedUsingVduCpd

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| cpdId | none | Navig. | 1 | VduCpd | Identifier of the VDU CPD in the VNFD. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vnfcCpInfo | none | Not navig. | 0..1 | VnfcCpInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfcCpExposesAsVnfExtCp association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::VnfcCpExposesAsVnfExtCp

Applied Stereotypes:

Experimental

Table B.1: Member ends for Association VnfcCpExposesAsVnfExtCp

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| vnfExtCpId | none | Navig. | 0..1 | VnfExtCpInfo | When the VNFC CP is exposed as external CP of the VNF, the identifier of this external VNF CP. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vnfccpinfo | none | Not navig. | 0..1 | VnfcCpInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfcHasStorageResource association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::VnfcHasStorageResource

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfcHasStorageResource

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| storageResourceId | composite | Navig. | 0..* | VirtualStorageResourceInfo | Reference(s) to the VirtualStorage resource(s). Information about the resource(s) is available from the Virtualised Storage Resource Management interface. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vnfcResourceInfo | none | Not navig. | 1 | VnfcResourceInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfcInfoReferencesVdu association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::VnfcInfoReferencesVdu

Applied Stereotypes:

Cond

Experimental

Table B.1: Member ends for Association VnfcInfoReferencesVdu

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| vdu | none | Navig. | 1 | Vdu | Identifier of the VDU in the VNFD. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfcInfo | none | Not navig. | 0..1 | VnfcInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfcResourceInfoReferencesVdu association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfLcmModule::VnfcResourceInfoReferencesVdu

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfcResourceInfoReferencesVdu

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| vdu | none | Navig. | 0..1 | Vdu | Reference to the applicable Vdu class in the VNFD. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vnfresourceinfo | none | Not navig. | 0..* | VnfcResourceInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

### VnfPackageModule

#### Overview

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfPackageModule

##### Diagrams

Figure B.1: Diagram VNF Package Resources

##### Classes

###### VnfPackageArtifactInformation class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfPackageModule::VnfPackageArtifactInformation

Description:

This class represents an artifact (other than a Software Image) which is contained in the VNF package.

Applied Stereotypes:

Experimental

OpenModelClass

support:  MANDATORY

Table B.1: Attributes for Class VnfPackageArtifactInformation

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| selector |  | 1 | Information (such as a path) that identifies / addresses this artifact in the VNF Package. Definition of the format is left to protocol design. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| metadata |  | 1 | The metadata of the artifact that are available in the VNF Package, such as Content type, size, creation date, etc. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfPackageSoftwareImageInformation class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfPackageModule::VnfPackageSoftwareImageInformation

Description:

This class represents an artifact contained in a VNF package which represents a Software Image.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VnfPackageSoftwareImageInformation

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| softwareImageInformation | SoftwareImageInformation | 1 | Information on the software image(s). | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| accessInformation |  | 1 | Information (such as a URL, a path in the VNF package, or an identifier) that allows to access a copy of this software image artifact. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfPkgInfo class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfPackageModule::VnfPkgInfo

Description:

This class provides the details of an on-boarded VNF Package, which the NFVO creates and stores as part of the on-boarding and ongoing operational management process.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VnfPkgInfo

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| vnfPkgInfoId | Identifier | 1 | Identifier of VNF Package information object. This identifier was allocated by the NFVO. | Experimental OpenModelAttribute - isInvariant: true - support: MANDATORY |
| vnfdId | Identifier | 0..1 | Identifier of the on-boraded VNF Package. NOTE 1: This identifier, which is managed by the VNF provider, identifies the VNF Package and the VNFD in a globally unique way. NOTE 2: This information is copied from the VNFD of the on-boarded VNF Package. NOTE 3: This attribute shall be present after the VNF Package is on-boarded. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfProvider | String | 0..1 | Provider of the on-boarded VNF package. NOTE 2: This information is copied from the VNFD of the on-boarded VNF Package. NOTE 3: This attribute shall be present after the VNF Package is on-boarded. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfProductName | String | 0..1 | Product name of the on-boarded VNF package. NOTE 2: This information is copied from the VNFD of the on-boarded VNF Package. NOTE 3: This attribute shall be present after the VNF Package is on-boarded. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfSoftwareVersion | Version | 0..1 | Software version of the on-boarded VNF package. NOTE 2: This information is copied from the VNFD of the on-boarded VNF Package. NOTE 3: This attribute shall be present after the VNF Package is on-boarded. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfdVersion | Version | 0..1 | VNFD version of the on-boarded VNF package. NOTE 2: This information is copied from the VNFD of the on-boarded VNF Package. NOTE 3: This attribute shall be present after the VNF Package is on-boarded. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| checksum |  | 0..1 | Checksum of the on-boarded VNF Package. NOTE 2: This information is copied from the VNFD of the on-boarded VNF Package. NOTE 3: This attribute shall be present after the VNF Package is on-boarded. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfd | Vnfd | 0..1 | Reference to the VNFD contained in the on-boarded VNF Package, e.g. URL to the on-boarded VNFD. NOTE: This attribute shall be present after the VNF Package is on-boarded. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| softwareImage | VnfPackageSoftwareImageInformation | 0..* | Information about VNF package artifacts that are software images. NOTE: This attribute shall be present after the VNF Package is on-boarded. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| additionalArtifact | VnfPackageArtifactInformation | 0..* | Information about VNF package artifacts contained in the VNF package that are not software images. NOTE: It may be present after the VNF Package is on-boarded and shall be absent otherwise. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| onboardingState | OnboardingState | 1 | On-boarding state of the VNF Package. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| operationalState | OperationalState | 1 | Operational state of the VNF Package. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| usageState | UsageState | 1 | Usage state of the VNF Package. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| deletionPending | Boolean | 1 | Indicates if deletion of this instance of the VNF Package has been requested but the VNF Package is still being used by instantiated VNFs. This instance of the VNF Package will be deleted once all VNFs instantiated from this package are terminated. | Obsolete OpenModelAttribute - isInvariant: false - support: MANDATORY |
| userDefinedData | KeyValuePair | 0..* | User defined data for the VNF Package. | OpenModelAttribute - isInvariant: false - support: OPTIONAL |

##### Notifications

###### VnfPackageChangeNotification notification

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfPackageModule::VnfPackageChangeNotification

Description:

This notification indicates a change of status in a VNF Package. Only changes in operational state and the deletion of VNF package will be reported. Change in usage state is not reported.

Applied Stereotypes:

Experimental

OpenModelNotification

triggerConditionList:

Change of the operational state of an on-boarded VNF Package.

Deletion of a VNF Package

support:    MANDATORY

Table B.1: Attributes for Signal VnfPackageChangeNotification

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| onboardedVnfPkgInfoId | Identifier | 1 | Identifier of VNF Package information object. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfdId | Identifier | 1 | Identifier of the on-boarded VNF Package. Note: This identifier, which is managed by the VNF provider, identifies the VNF Package and the VNFD in a globally unique way. This information is copied from the VNFD of the on-boarded VNF Package. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| changeType | VnfPackageChangeType | 1 | It categorizes the type of change. Possible values can be change of operational state of an on-boarded VNF Package and deletion of a VNF Package. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| operationalState | OperationalState | 0..1 | New operational state of the VNF Package. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: |
| deletionPending | Boolean | 0..1 | Indicates if the deletion instance of the VNF Package has been requested but the VNF Package is still being used by instantiated VNFs. Only present when changeType is VNF Package in deletion pending. | Obsolete OpenModelAttribute - isInvariant: false - support: MANDATORY - condition: |

###### VnfPackageOnBoardingNotification  notification

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfPackageModule::VnfPackageOnBoardingNotification

Description:

This notification indicates a VNF Package is on-boarded, after all the on-boarding steps (e.g. uploading and processing) are done.

A change in on-boarding state before the VNF Package is on-boarded is not reported.

Applied Stereotypes:

Experimental

OpenModelNotification

triggerConditionList:

New VNF Package on-boarded.

support:    MANDATORY

Table B.1: Attributes for Signal VnfPackageOnBoardingNotification

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| onboardedVnfPkgInfoId | Identifier | 1 | Identifier of the VNF Package information object. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfdId | Identifier | 1 | Identifier of the on-boarded VNF Package. Note: This identifier, which is managed by the VNF provider, identifies the VNF Package and the VNFD in a globally unique way. This information is copied from the VNFD of the on-boarded VNF Package. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

##### Datatypes

###### VnfPackageArtifact datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfPackageModule::VnfPackageArtifact

Description:

This datatype represents an artifact contained in the VNF package.

Applied Stereotypes:

Obsolete

Table B.1: Attributes for Data Type VnfPackageArtifact

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| accessInformation |  | 1 | The access information that was passed to fetch the artifact. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| metadata |  | 1 | The metadata of the artifact such as Content type, size, creation date etc. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| artifact | Binary | 1 | The actual artifact. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfPackageChangeType enumeration

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfPackageModule::VnfPackageChangeType

Description:

It categorizes the type of change.

Applied Stereotypes:

Experimental

Contains Enumeration Literals:

CHANGE_OF_OPERATIONAL_STATE

DELETION_OF_A_VNF_PACKAGE

##### Associations

###### VnfPackageContainsSoftwareImageInformation association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfPackageModule::VnfPackageContainsSoftwareImageInformation

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfPackageContainsSoftwareImageInformation

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| softwareImageInformation | composite | Navig. | 1 | SoftwareImageInformation | Information on the software image(s). | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfpackagesoftwareimageinformation | none | Not navig. | 1 | VnfPackageSoftwareImageInformation |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfPackageContainsVnfPackageArtifactInformation association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfPackageModule::VnfPackageContainsVnfPackageArtifactInformation

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfPackageContainsVnfPackageArtifactInformation

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| additionalArtifact | composite | Navig. | 0..* | VnfPackageArtifactInformation | Information about VNF package artifacts contained in the VNF package that are not software images. NOTE: It may be present after the VNF Package is on-boarded and shall be absent otherwise. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfpackageinfo | none | Not navig. | 1 | VnfPkgInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfPackageContainsVnfPackageSoftwareImageInformation association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfPackageModule::VnfPackageContainsVnfPackageSoftwareImageInformation

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VnfPackageContainsVnfPackageSoftwareImageInformation

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| softwareImage | composite | Navig. | 0..* | VnfPackageSoftwareImageInformation | Information about VNF package artifacts that are software images. NOTE: This attribute shall be present after the VNF Package is on-boarded. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vnfPackageInfo | none | Not navig. | 1 | VnfPkgInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VnfPackageRefersToVnfd association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualNetworkFunctionDomain::VnfPackageModule::VnfPackageRefersToVnfd

Applied Stereotypes:

Experimental

Preliminary

Table B.1: Member ends for Association VnfPackageRefersToVnfd

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| vnfd | composite | Navig. | 0..1 | Vnfd | Reference to the VNFD contained in the on-boarded VNF Package, e.g. URL to the on-boarded VNFD. NOTE: This attribute shall be present after the VNF Package is on-boarded. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vnfpackageinfo | none | Not navig. | 1 | VnfPkgInfo |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

## VirtualisedResourceDomain

### Overview

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain

Description:

Group all the classes and types related to Virtualised Resources that are needed for interfaces

### CapacityInformationModule

#### Overview

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::CapacityInformationModule

##### Diagrams

Figure B.1: Diagram CapacityInformation

##### Classes

###### CapacityInformation class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::CapacityInformationModule::CapacityInformation

Description:

Multiple instances of this class (at least one for each resource type) are used to specify the available (i.e. consumable), reserved, allocated and the total capacity managed by the VIM or in a resource zone managed by the VIM.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class CapacityInformation

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| availableCapacity |  | 0..1 | The free capacity available for allocation and reservation. It can be specified in terms of current capacity or minimum and maximum capacity, average capacity, or other statistical measurement in the specified time interval. The set of measurements is to be defined during Stage 3 | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| reservedCapacity |  | 0..1 | The reserved capacity. It can be specified in terms of current capacity or minimum and maximum capacity, average capacity, or other statistical measurement in the specified time interval. The set of measurements is to be defined during Stage 3. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| totalCapacity |  | 0..1 | The total capacity is usually specified as a fixed capacity without variations in time. Note: VIM does not keep schedules for equipment build-out. The set of measurements is left to Stage 3. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| allocatedCapacity |  | 0..1 | The allocated capacity is usually specified as the current allocated capacity. Note: The allocated capacity is given without time variation since the VIM does not have a schedule of future allocations and de-allocations | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### CapacityThreshold class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::CapacityInformationModule::CapacityThreshold

Description:

This class defines thresholds for sending capacity change notifications.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class CapacityThreshold

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| thresholdType | ThresholdType | 1 | Defines the type of threshold. The list of possible values is left for later stage and might include: absolute value, percentage of total capacity, delta related to current value, single/ multi valued threshold, static/dynamic threshold, etc. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| threshold |  | 1 | Details of the threshold: value to be crossed and direction in which it is crossed and capacity information to which it applies (available, total, reserved, allocated). | OpenModelAttribute - isInvariant: false - support: MANDATORY |

##### Notifications

###### CapacityChangeNotification notification

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::CapacityInformationModule::CapacityChangeNotification

Description:

This notification informs the receiver of changes in the capacity of virtual resources managed by the VIM.

Applied Stereotypes:

OpenModelNotification

triggerConditionList:

Allocation/termination/updating  of virtual resources affecting the available and/or allocated capacity

Creation/termination/updating of reservations affecting the available and/or reserved capacity

Addition/removal/upgrading of physical infrastructure affecting the available and total capacity

Faults and repair of physical infrastructure affecting the available and total capacity

support:    MANDATORY

Preliminary

Table B.1: Attributes for Signal CapacityChangeNotification

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| zoneId | Identifier | 0..1 | The Resource Zone for which the capacity has changed. When omitted the total capacity managed by the VIM is reported | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| resourceDescriptor |  | 1 | The resource type for which the capacity is changed | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| capacityInformation | CapacityInformation | 1 | The available, total, reserved and/or allocated capacity of the Resource Zone, or the available, total, reserved and/or allocated capacity of the VIM in case the Resource Zone is omitted. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| changeId | Identifier | 1 | It identifies a change in the capacity | OpenModelAttribute - isInvariant: false - support: MANDATORY |

### ConsumableVirtualisedResourceModule

#### Overview

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::ConsumableVirtualisedResourceModule

Description:

Provides information abour consumable virtualised resources, including

- The total capacity of the compute resources managed by a VIM instance, globally or per resource zone,

- The consumable capacity available for new virtualised compute resources, globally or per resource zone

##### Diagrams

Figure B.1: Diagram ConsumableVirtualisedResources

##### Classes

###### VirtualComputeResourceInformation class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::ConsumableVirtualisedResourceModule::VirtualComputeResourceInformation

Description:

This class defines VirtualCompute resource.

Parent class: VirtualResourceTypeInformation

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VirtualComputeResourceInformation

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| virtualMemory | VirtualMemoryResourceInformation | 0..1 | It defines the virtual memory characteristics of the consumable virtualised compute resource. Note: Cardinality can be "0" if the attribute refers to a characteristic that is not being reported on a specific query or notification, e.g., through a InformationChangeNotification | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualCpu | VirtualCpuResourceInformation | 0..1 | It defines the virtual CPU(s) characteristics of the consumable virtualised compute resource. Cardinality can be "0" if the attribute refers to a characteristic that is not being reported on a specific query or notification, e.g., through a InformationChangeNotification | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| accelerationCapability |  | 0..* | Acceleration capabilities (e.g., crypto, GPU) for the consumable virtualised compute resources from the set of capabilities offered by the compute node acceleration resources. The cardinality can be 0, if no particular acceleration capability is provided. Cardinality can be "0" if the attribute refers to a characteristic that is not being reported on a specific query or notification, e.g., through a InformationChangeNotification | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| resourceTypeId | Identifier | 1 | Identifier of the consumable virtualised resource type. | Inherited from Class VirtualResourceTypeInformation OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VirtualNetworkResourceInformation class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::ConsumableVirtualisedResourceModule::VirtualNetworkResourceInformation

Description:

This class defines the characteristics of consumable virtual network resources.

Parent class: VirtualResourceTypeInformation

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VirtualNetworkResourceInformation

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| bandwidth | Number | 1 | Minimum network bandwidth (in Mbps). | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| networkType | String | 0..1 | The type of network that maps to the virtualised network. Examples are: "local", "vlan", "vxlan", "gre", etc. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| networkQoS | NetworkQoS | 0..* | Element providing information about Quality of Service attributes that the network shall support. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| resourceTypeId | Identifier | 1 | Identifier of the consumable virtualised resource type. | Inherited from Class VirtualResourceTypeInformation OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VirtualResourceTypeInformation class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::ConsumableVirtualisedResourceModule::VirtualResourceTypeInformation

Description:

The VirtualResourceTypeInformation class is the common parent of VirtualcomputeResourceTypeInformation, VirtualStorageResourceTypeInformation  and VirtualNetworkResourceTypeInformation

This class is abstract.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VirtualResourceTypeInformation

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| resourceTypeId | Identifier | 1 | Identifier of the consumable virtualised resource type. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VirtualStorageResourceInformation class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::ConsumableVirtualisedResourceModule::VirtualStorageResourceInformation

Description:

This class defines the characteristics of consumable virtual storage resources.

Parent class: VirtualResourceTypeInformation

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VirtualStorageResourceInformation

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| typeOfStorage | String | 1 | Type of virtualised storage resource (e.g., volume, object) | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| sizeOfStorage | Number | 0..1 | Size of virtualised storage resource (e.g., size of volume, in GB). Cardinality "1" covers the case where a specific configuration for the consumable resource is advertised | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| rdmaSupported | Boolean | 0..1 | It indicates if the storage supports RDMA. | OpenModelAttribute - isInvariant: false - support: OPTIONAL |
| resourceTypeId | Identifier | 1 | Identifier of the consumable virtualised resource type. | Inherited from Class VirtualResourceTypeInformation OpenModelAttribute - isInvariant: false - support: MANDATORY |

##### Notifications

###### InformationChangeNotification notification

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::ConsumableVirtualisedResourceModule::InformationChangeNotification

Description:

This notification informs the receiver that information related to consumable virtualised resources has changed.

Applied Stereotypes:

OpenModelNotification

triggerConditionList:

Addition of consumable virtualised resources

Removal of consumable virtualised resources

Update of consumable virtualised resources

support:    MANDATORY

Preliminary

Table B.1: Attributes for Signal InformationChangeNotification

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| changeId | Identifier | 1 | Unique identifier of the change on the consumable virtualised resource type | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| resourceTypeId | VirtualResourceTypeInformation | 1 | Identifier of the consumable virtualised resource type. Reference to VirtualComputeResourceInformation, VirtualStorageResourceInformation or VirtualNetworkResourceInformation. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |
| vimId | Identifier | 1 | Identifier of the VIM reporting the change. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| changeType | InformationChangeType | 1 | It categorizes the type of change. Permitted values are: ADDITION, REMOVAL and UPDATE. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| changedResourceData |  | 0..1 | Details of the changes of consumable virtualised resource information. Its content can differ based on the values of the resourceTypeId and changeType. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

##### Datatypes

###### InformationChangeType enumeration

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::ConsumableVirtualisedResourceModule::InformationChangeType

Description:

It categorizes the type of change related on information related to consumable virtualised resources

Applied Stereotypes:

Preliminary

Contains Enumeration Literals:

ADDITION

REMOVAL

UPDATE

###### VirtualCpuResourceInformation datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::ConsumableVirtualisedResourceModule::VirtualCpuResourceInformation

Description:

The VirtualCpuResourceInformation datatype defines the virtual CPU(s) characteristics of consumable virtualised compute resource.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type VirtualCpuResourceInformation

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| cpuArchitecture | String | 1 | CPU architecture type. Examples are x86, ARM. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| numVirtualCpu | Number | 0..1 | Number of virtual CPUs. Cardinality "1" covers the case where a specific configuration for the consumable resource is advertised. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| cpuClock | Number | 1 | Minimum CPU clock rate (e.g. in MHz) available for the virtualised CPU resources. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualCpuOversubscriptionPolicy |  | 0..1 | The CPU core oversubscription policy, e.g. the relation of virtual CPU cores to physical CPU cores/threads. The cardinality can be 0 if no concrete policy is defined. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualCpuPinningSupported | Boolean | 1 | It defines whether CPU pinning capability is available on the consumable virtualised compute resource. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VirtualMemoryResourceInformation datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::ConsumableVirtualisedResourceModule::VirtualMemoryResourceInformation

Description:

The VirtualMemoryResourceInformation datatype defines the virtual memory characteristics of consumable virtualised compute resource.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type VirtualMemoryResourceInformation

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| virtualMemSize | Number | 0..1 | Amount of virtual memory (e.g. in MB). Cardinality "1" covers the case where a specific configuration for the consumable resource is advertised. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualMemOversubscriptionPolicy |  | 0..1 | The memory core oversubscription policy in terms of virtual memory to physical memory on the platform. The cardinality can be 0 if no concrete policy is defined. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| numaSupported | Boolean | 1 | It specifies if the memory allocation can be cognisant of the relevant process/core allocation. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

### QuotaModule

#### Overview

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::QuotaModule

##### Diagrams

Figure B.1: Diagram Quota

##### Classes

###### VirtualComputeQuota class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::QuotaModule::VirtualComputeQuota

Description:

This class describes the attributes for the VirtualComputeQuota.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VirtualComputeQuota

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| resourceGroupId | Identifier | 1 | Unique identifier of the "infrastructure resource group", logical grouping of virtual resources assigned to a tenant within an Infrastructure Domain. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| numVCPUs | Integer | 0..1 | Number of CPU cores that have been restricted by the quota. The cardinality can be 0 if no specific number of CPU cores has been requested to be restricted by the quota. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| numVcInstances | Integer | 0..1 | Number of virtualisation container instances that have been restricted by the quota. The cardinality can be 0 if no specific number of CPU cores has been requested to be restricted by the quota. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualMemSize | Number | 0..1 | Size of virtual memory that has been restricted by the quota. The cardinality can be 0 if no specific number of CPU cores has been requested to be restricted by the quota. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VirtualComputeQuotaData class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::QuotaModule::VirtualComputeQuotaData

Description:

This class describes the attributes for the VirtualComputeQuotaData.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VirtualComputeQuotaData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| numVCPUs | Integer | 0..1 | Number of CPU cores to be restricted by the quota. The cardinality can be 0 if no specific number of CPU cores is to be restricted by the quota or the quota for CPU cores is not to be update. At least one of the three attributes shall be present. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| numVcInstances | Integer | 0..1 | Number of virtualisation container instances to be restricted by the quota. The cardinality can be 0 if no specific number of virtualisation container instances is to be restricted by the quota or the quota for virtualisation container instances is not to be update. At least one of the three attributes shall be present. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualMemSize | Number | 0..1 | Size of virtual memory to be restricted by the quota. The cardinality can be 0 if no specific size of virtual memory is to be restricted by the quota or the quota for virtual memory is not to be update. At least one of the three attributes shall be present. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VirtualNetworkQuota class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::QuotaModule::VirtualNetworkQuota

Description:

This class describes the attributes for the VirtualNetworkQuota.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VirtualNetworkQuota

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| resourceGroupId | Identifier | 1 | Unique identifier of the "infrastructure resource group", logical grouping of virtual resources assigned to a tenant within an Infrastructure Domain. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| numPublicIps | Integer | 0..1 | Number of public IP addresses that have been restricted by the quota. The cardinality can be 0 if no specific number of public IP addresses has been requested to be restricted by the quota. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| numPorts | Integer | 0..1 | Number of ports that have been restricted by the quota. The cardinality can be 0 if no specific number of ports has been requested to be restricted by the quota. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| numSubnets | Integer | 0..1 | Number of subnets that have been restricted by the quota. The cardinality can be 0 if no specific number of subnets has been requested to be restricted by the quota. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VirtualNetworkQuotaData class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::QuotaModule::VirtualNetworkQuotaData

Description:

This class describes the attributes for the VirtualNetworkQuotaData.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VirtualNetworkQuotaData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| numPublicIps | Integer | 0..1 | Number of public IP addresses to be restricted by the quota. The cardinality can be 0 if no specific number of public IP addresses is to be restricted by the quota or the quota for public IP addresses is not to be update. At least one of the three attributes shall be present. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| numPorts | Integer | 0..1 | Number of ports to be restricted by the quota. The cardinality can be 0 if no specific number of ports is to be restricted by the quota or the quota for ports is not to be update. At least one of the three attributes shall be present. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| numSubnets | Integer | 0..1 | Number of subnets to be restricted by the quota. The cardinality can be 0 if no specific number of subnets is to be restricted by the quota or the quota for subnets is not to be update. At least one of the three attributes shall be present. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VirtualStorageQuota class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::QuotaModule::VirtualStorageQuota

Description:

This class describes the attributes for the VirtualStorageQuota.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VirtualStorageQuota

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| resourceGroupId | Identifier | 1 | Unique identifier of the "infrastructure resource group", logical grouping of virtual resources assigned to a tenant within an Infrastructure Domain. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| storageSize | Number | 0..1 | Size of virtualised storage resource that has been restricted by the quota. Cardinality can be 0 if no specific size of virtualised storage resource has been requested to be restricted by the quota. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| numSnapshots | Integer | 0..1 | Number of snapshots that have been restricted by the quota. The cardinality can be 0 if no specific number of snapshots has been requested to be restricted by the quota. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| numVolumes | Integer | 0..1 | Number of volumes that have been restricted by the quota. The cardinality can be 0 if no specific number of volumes has been requested to be restricted by the quota. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VirtualStorageQuotaData class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::QuotaModule::VirtualStorageQuotaData

Description:

This class describes the attributes for the VirtualStorageQuotaData.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VirtualStorageQuotaData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| storageSize | Number | 0..1 | Size of virtualised storage resource (e.g. size of volume, in GB) to be restricted by the quota. Cardinality can be 0 if no specific size of virtualised storage resource is to be restricted by the quota or the quota for the size of virtualised storage resource is not to be update. At least one of the three attributes shall be present. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| numSnapshots | Integer | 0..1 | Number of snapshots to be restricted by the quota. Cardinality can be 0 if no specific number of snapshots is to be restricted by the quota or the quota for the snapshots is not to be update. At least one of the three attributes shall be present. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| numVolumes | Integer | 0..1 | Number of volumes to be restricted by the quota. Cardinality can be 0 if no specific number of volumes is to be restricted by the quota or the quota for the volumes is not to be update. At least one of the three attributes shall be present. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

##### Notifications

###### VirtualisedResourceQuotaChangeNotification notification

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::QuotaModule::VirtualisedResourceQuotaChangeNotification

Description:

This notification indicates a Quota change in a virtualised resource.

Applied Stereotypes:

OpenModelNotification

triggerConditionList:

A resource Quota is being updated.

support:    MANDATORY

Preliminary

Table B.1: Attributes for Signal VirtualisedResourceQuotaChangeNotification

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| changeId | Identifier | 1 | Unique identifier of the change on the virtualised resource Quota | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| resourceGroupId | Identifier | 1 | Unique identifier of the "infrastructure resource group", logical grouping of virtual resources assigned to a tenant within an Infrastructure Domain. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vimId | Identifier | 1 | The VIM reporting the change. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| changeType | String | 1 | It categorizes the type of change. Possible values can be related to an update of the Quota. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| changedQuotaData |  | 0..1 | Details of the changes of the Quota | OpenModelAttribute - isInvariant: false - support: MANDATORY |

### ReservationModule

#### Overview

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::ReservationModule

##### Diagrams

Figure B.1: Diagram Reservation

##### Classes

###### ComputePoolReservation class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::ReservationModule::ComputePoolReservation

Description:

The compute reservation class encapsulates information about virtual compute resource pool reservation. The class contains details about number of CPU cores, number of virtualisation container instances, size of virtual memory, as well as different attributes of the virtual compute resource pool

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class ComputePoolReservation

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| numCpuCores | Integer | 1 | Number of CPU cores to be reserved. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| numVcInstances | Integer | 1 | Number of virtualized container instances to be reserved. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualMemSize | Number | 1 | Size of virtual memory to be reserved. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| computeAttributes | VirtualComputeAttributesReservationData | 0..1 | Information specifying additional attributes of the compute resource to be reserved. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### ReservedComputePool class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::ReservationModule::ReservedComputePool

Description:

This class describes the attributes for the ReservedComputePool.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class ReservedComputePool

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| numCpuCores | Integer | 1 | Number of CPU cores that have been reserved. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| numVcInstances | Integer | 1 | Number of virtual container instances that have been reserved. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualMemSize | Number | 1 | Size of virtual memory that has been reserved. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| computeAttributes | ReservedVirtualComputeAttributes | 0..1 | Information specifying additional attributes of the virtual compute resource that have been reserved. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| zoneId | ResourceZone | 0..1 | References the resource zone where the virtual compute resources have been reserved. Cardinality can be 0 to cover the case where reserved compute resources are not bound to a specific resource zone. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |

###### ReservedStoragePool class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::ReservationModule::ReservedStoragePool

Description:

This class describes the attributes for the ReservedStoragePool.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class ReservedStoragePool

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| storageSize | Number | 1 | Size of virtualised storage resource that has been reserved. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| numSnapshots | Integer | 1 | Number of snapshots that have been reserved. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| numVolumes | Integer | 1 | Number of volumes that have been reserved. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| zoneId | ResourceZone | 0..1 | References the resource zone where the virtual storage resources have been reserved. Cardinality can be 0 to cover the case where reserved storage resources are not bound to a specific resource zone | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |

###### ReservedVirtualCompute class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::ReservationModule::ReservedVirtualCompute

Description:

This class describes the attributes for the ReservedVirtualCompute.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class ReservedVirtualCompute

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| reservationId | Identifier | 1 | Identifier of the resource reservation. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| computePoolReserved | ReservedComputePool | 0..1 | Information about compute resources that have been reserved, e.g. {"cpu_cores": 90, "vm_instances": 10, "ram": 10000}. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualisationContainerReserved | ReservedVirtualisationContainer | 0..* | Information about the virtualisation container(s) that have been reserved. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| reservationStatus | ReservationStatus | 1 | Status of the compute resource reservation, e.g., to indicate if a reservation is being used. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| startTime | DateTime | 0..1 | Indication when the consumption of the resources starts. If the value is 0, resources are reserved for immediate use. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| endTime | DateTime | 0..1 | Indication when the reservation ends (when it is expected that the resources will no longer be needed) and used by the VIM to schedule the reservation. If not present, resources are reserved for unlimited usage time. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| expiryTime | DateTime | 0..1 | Indication when the VIM can release the reservation in case no allocation request against this reservation was made. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### ReservedVirtualComputeAttributes class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::ReservationModule::ReservedVirtualComputeAttributes

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class ReservedVirtualComputeAttributes

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| accelerationCapability |  | 0..* | Selected acceleration capabilities (e.g. crypto, GPU) from the set of capabilities offered by the compute node acceleration resources. The cardinality can be 0, if no particular acceleration capability is provided. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| cpuArchitecture |  | 0..1 | CPU architecture type. Examples are "x86", "ARM". The cardinality can be 0, if no particular CPU architecture type is provided. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualCpuOversubscriptionPolicy |  | 0..1 | The CPU core oversubscription policy in terms of virtual CPU cores to physical CPU cores/threads on the platform. The cardinality can be 0, if no particular value is provided. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### ReservedVirtualNetwork class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::ReservationModule::ReservedVirtualNetwork

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class ReservedVirtualNetwork

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| reservationId | Identifier | 1 | Identifier of the resource reservation. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| publicIps | IpAddress | 1 | List of public IP addresses that have been reserved | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| networkAttributes | ReservedVirtualNetworkAttributes | 0..1 | Information specifying additional attributes of the network resource that has been reserved | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| networkPorts | ReservedVirtualNetworkPort | 0..* | List of metadata key-value pairs used by the consumer to associate meaningful metadata to the related virtualised resource. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| reservationStatus | ReservationStatus | 1 | Status of the network resource reservation, e.g., to indicate if a reservation is being used. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| startTime | DateTime | 0..1 | Indication when the consumption of the resources starts. If the value is 0, resources are reserved for immediate use. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| endTime | DateTime | 0..1 | Indication when the reservation ends (when it is expected that the resources will no longer be needed) and used by the VIM to schedule the reservation. If not present, resources are reserved for unlimited usage time. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| expiryTime | DateTime | 0..1 | Indication when the VIM can release the reservation in case no allocation request against this reservation was made. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| zoneId | ResourceZone | 1 | References the resource zone where the virtual network resources have been reserved. Cardinality can be 0 to cover the case where reserved network resources are not bound to a specific resource zone. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |

###### ReservedVirtualNetworkAttributes class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::ReservationModule::ReservedVirtualNetworkAttributes

Description:

Describes the attributes for the ReservedVirtualNetworkAttributes class.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class ReservedVirtualNetworkAttributes

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| bandwidth | Number | 1 | Minimum network bitrate (in Mbps). | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| networkType | String | 1 | The type of network that maps to the virtualised network that has been reserved. Examples are: "local", "vlan", "vxlan", "gre", etc. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| segmentType | String | 1 | The isolated segment for the virtualised network that has been reserved. For instance, for a "vlan" networkType, it corresponds to the vlan identifier; and for a "gre" networkType, this corresponds to a gre key. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| isShared | Boolean | 1 | It defines whether the virtualised network that has been reserved is shared among consumers. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| metadata | KeyValuePair | 0..* | List of metadata key-value pairs used by the consumer to associate meaningful metadata to the related virtualised resource. | OpenModelAttribute - isInvariant: false - support: OPTIONAL |

###### ReservedVirtualNetworkPort class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::ReservationModule::ReservedVirtualNetworkPort

Description:

Describes the attributes for the ReservedVirtualNetworkPort class.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class ReservedVirtualNetworkPort

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| portId | Identifier | 1 | Identifier of the network port that has been reserved. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| portType |  | 1 | Type of network port. Examples of types are access ports, or trunk ports (layer 1) that become transport for multiple layer 2 or layer 3 networks. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| segmentId |  | 0..1 | The isolated segment the network port belongs to. For instance, for a "vlan", it corresponds to the vlan identifier; and for a "gre", this corresponds to a gre key. The cardinality can be 0 to allow for flat networks without any specific segmentation. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| bandwidth | Number | 0..1 | The bitrate of the virtual network port (in Mbps). | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| metadata | KeyValuePair | 0..* | List of metadata key-value pairs used by the consumer to associate meaningful metadata to the related virtualised resource. | OpenModelAttribute - isInvariant: false - support: OPTIONAL |

###### ReservedVirtualStorage class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::ReservationModule::ReservedVirtualStorage

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class ReservedVirtualStorage

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| reservationId | Identifier | 1 | Identifier of the resource reservation. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| storagePoolReserved | ReservedStoragePool | 0..1 | Information about storage resources that have been reserved, e.g. {"gigabytes": 1000, "snapshots": 10, "volumes": 10}. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| reservationStatus | ReservationStatus | 1 | Status of the storage resource reservation, e.g., to indicate if a reservation is being used. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| startTime | DateTime | 0..1 | Indication when the consumption of the resources starts. If the value is 0, resources are reserved for immediate use. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| endTime | DateTime | 0..1 | Indication when the reservation ends (when it is expected that the resources will no longer be needed) and used by the VIM to schedule the reservation. If not present, resources are reserved for unlimited usage time. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| expiryTime | DateTime | 0..1 | Indication when the VIM can release the reservation in case no allocation request against this reservation was made. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### ReservedVirtualisationContainer class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::ReservationModule::ReservedVirtualisationContainer

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class ReservedVirtualisationContainer

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| containerId | Identifier | 1 | The identifier of the virtualization container that has been reserved. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| flavourId | Identifier | 1 | Identifier of the given compute flavour used to reserve the virtualisation container | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| accelerationCapability |  | 0..* | Selected acceleration capabilities (e.g. crypto, GPU) from the set of capabilities offered by the compute node acceleration resources. The cardinality can be 0, if no particular acceleration capability is provided. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualMemory | VirtualMemory | 1 | The virtual memory of the reserved virtualisation container | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualCpu | VirtualCpu | 1 | The virtual CPU(s) of the reserved virtualisation container | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualDisks | VirtualStorage | 1..* | Element with information of the virtualised storage resources attached to the reserved virtualisation container. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualNetworkInterface | VirtualNetworkInterface | 0..* | Element with information of the virtual network interfaces of the reserved virtualisation container | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| zoneId | ResourceZone | 0..1 | It defines in what particular Resource Zone the virtualization container has been reserved. | OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |

###### StoragePoolReservation class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::ReservationModule::StoragePoolReservation

Description:

This class describes the attributes for the StoragePoolReservation.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class StoragePoolReservation

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| storageSize | Number | 1 | Size of virtualised storage resource (e.g. size of volume, in GB) to be reserved. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| numSnapshots | Integer | 0..1 | Number of snapshots to be reserved. Cardinality can be 0 if no specific number of snapshots is to be reserved. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| numVolumes | Integer | 0..1 | Number of volumes to be reserved. Cardinality can be 0 if no specific number of volumes is to be reserved. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VirtualNetworkReservation class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::ReservationModule::VirtualNetworkReservation

Description:

This class describes the attributes for the VirtualNetworkReservation.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VirtualNetworkReservation

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| numPublicIps | Integer | 0..1 | Number of public IP addresses to be reserved. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| networkAttributes | VirtualNetworkAttributesReservationData | 0..1 | Information specifying additional attributes of the network resource to be reserved. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| networkPorts | VirtualNetworkPortReservation/Data | 0..* | List of specific network ports to be reserved. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VirtualisationContainerReservation class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::ReservationModule::VirtualisationContainerReservation

Description:

The VirtualisationContainerReservation class encapsulates information about virtualization container reservations, including (among others), virtual memory, CPUs, storage, and virtual network interfaces, as well as a zone ID.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VirtualisationContainerReservation

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| containerId | Identifier | 1 | The identifier of the virtualisation container to be reserved. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| containerFlavour | VirtualComputeFlavour | 1 | The containerFlavour encapsulates information of the virtualisation container to be reserved. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

##### Notifications

###### VirtualisedResourceReservationChangeNotification notification

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::ReservationModule::VirtualisedResourceReservationChangeNotification

Description:

This notification indicates a change in a virtualised resource reservation.

Applied Stereotypes:

OpenModelNotification

triggerConditionList:

A resource reservation has been updated.

A resource reservation changed due to changes in underlying resources that are part of this reservation.

support:    MANDATORY

Preliminary

Table B.1: Attributes for Signal VirtualisedResourceReservationChangeNotification

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| changeId | Identifier | 1 | Unique identifier of the change on the virtualised resource reservation. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| reservationId | Identifier | 1 | The reservation being changed. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vimId | Identifier | 1 | The VIM reporting the change. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| changeType | String | 1 | It categorizes the type of change. Possible values can be related to an update of the reservation or a change in the resources part of the reservation. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| changedReservationData |  | 0..1 | Details of the changes of the reservation. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

##### Datatypes

###### ReservationStatus enumeration

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::ReservationModule::ReservationStatus

Description:

Status of the resource reservation, e.g., to indicate if a reservation is being used.

Applied Stereotypes:

Preliminary

###### VirtualComputeAttributesReservationData datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::ReservationModule::VirtualComputeAttributesReservationData

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type VirtualComputeAttributesReservationData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| accelerationCapability |  | 0..* | Selected acceleration capabilities (e.g. crypto, GPU) from the set of capabilities offered by the compute node acceleration resources. The cardinality can be 0, if no particular acceleration capability is requested. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| cpuArchitecture |  | 0..1 | CPU architecture type. Examples are "x86", "ARM". The cardinality can be 0, if no particular CPU architecture type is requested. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualCpuOversubscriptionPolicy |  | 0..1 | The CPU core oversubscription policy in terms of virtual CPU cores to physical CPU cores/threads on the platform. The cardinality can be 0, if no particular value is requested. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VirtualNetworkAttributesReservationData datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::ReservationModule::VirtualNetworkAttributesReservationData

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type VirtualNetworkAttributesReservationData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| bandwidth | Number | 1 | Minimum network bitrate (in Mbps). | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| networkType | String | 0..1 | The type of network that maps to the virtualised network to be reserved. Examples are: "local", "vlan", "vxlan", "gre", etc. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| segmentType | String | 0..1 | The isolated segment for the virtualised network to be reserved. For instance, for a "vlan" NetworkType, it corresponds to the vlan identifier; and for a "gre" NetworkType, this corresponds to a gre key. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| isShared | Boolean | 0..1 | It defines whether the virtualised network to be reserved is shared among consumers. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| metadata | KeyValuePair | 0..* | List of metadata key-value pairs used by the consumer to associate meaningful metadata to the related virtualised resource. | OpenModelAttribute - isInvariant: false - support: OPTIONAL |

###### VirtualNetworkPortReservation/Data datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::ReservationModule::VirtualNetworkPortReservation/Data

Description:

A network port is a communication endpoint under a network.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type VirtualNetworkPortReservation/Data

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| portId | Identifier | 1 | Identifier of the network port to reserve. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| portType |  | 1 | Type of network port. Examples of types are access ports, or trunk ports (layer 1) that become transport for multiple layer 2 or layer 3 networks. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| segmentId |  | 0..1 | The isolated segment the network port belongs to. For instance, for a "vlan", it corresponds to the vlan identifier; and for a "gre", this corresponds to a gre key. The cardinality can be 0 to allow for flat networks without any specific segmentation. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| bandwidth | Number | 0..1 | The bitrate of the virtual network port (in Mbps). | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| metadata | KeyValuePair | 0..* | List of metadata key-value pairs used by the consumer to associate meaningful metadata to the related virtualised resource. | OpenModelAttribute - isInvariant: false - support: OPTIONAL |

##### Associations

###### ComputePoolReservationHasVirtualComputeAttributesReservation association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::ReservationModule::ComputePoolReservationHasVirtualComputeAttributesReservation

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association ComputePoolReservationHasVirtualComputeAttributesReservation

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| computePoolReservation | none | Not navig. | 1 | ComputePoolReservation |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| computeAttributes | shared | Navig. | 0..1 | VirtualComputeAttributesReservationData | Information specifying additional attributes of the compute resource to be reserved. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### ReservedComputePoolHasVirtualComputeAttributesReservation association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::ReservationModule::ReservedComputePoolHasVirtualComputeAttributesReservation

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association ReservedComputePoolHasVirtualComputeAttributesReservation

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| reservedComputePool | none | Not navig. | 1 | ReservedComputePool |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| computeAttributes | shared | Navig. | 0..1 | ReservedVirtualComputeAttributes | Information specifying additional attributes of the virtual compute resource that have been reserved. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### ReservedVirtualComputeResourceHasReservedComputePool association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::ReservationModule::ReservedVirtualComputeResourceHasReservedComputePool

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association ReservedVirtualComputeResourceHasReservedComputePool

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| reservedVirtualComputeResource | none | Not navig. | 1 | ReservedVirtualCompute |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| computePoolReserved | shared | Navig. | 0..1 | ReservedComputePool | Information about compute resources that have been reserved, e.g. {"cpu_cores": 90, "vm_instances": 10, "ram": 10000}. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### ReservedVirtualComputeResourceHasReservedVirtualizationContainer association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::ReservationModule::ReservedVirtualComputeResourceHasReservedVirtualizationContainer

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association ReservedVirtualComputeResourceHasReservedVirtualizationContainer

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| reservedVirtualComputeResource | none | Not navig. | 1 | ReservedVirtualCompute |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualisationContainerReserved | shared | Navig. | 0..* | ReservedVirtualisationContainer | Information about the virtualisation container(s) that have been reserved. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### ReservedVirtualNetworkHasNetworkPorts association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::ReservationModule::ReservedVirtualNetworkHasNetworkPorts

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association ReservedVirtualNetworkHasNetworkPorts

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| networkPorts | shared | Navig. | 0..* | ReservedVirtualNetworkPort | List of metadata key-value pairs used by the consumer to associate meaningful metadata to the related virtualised resource. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| reservedvirtualnetwork | none | Not navig. | 1 | ReservedVirtualNetwork |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### ReservedVirtualNetworkHasResservedNetworkAttributes association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::ReservationModule::ReservedVirtualNetworkHasResservedNetworkAttributes

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association ReservedVirtualNetworkHasResservedNetworkAttributes

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| networkAttributes | shared | Navig. | 0..1 | ReservedVirtualNetworkAttributes | Information specifying additional attributes of the network resource that has been reserved | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| reservedvirtualnetwork | none | Not navig. | 1 | ReservedVirtualNetwork |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### ReservedVirtualStorageHasReservedStoragePool association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::ReservationModule::ReservedVirtualStorageHasReservedStoragePool

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association ReservedVirtualStorageHasReservedStoragePool

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| reservedVirtualStorage | none | Not navig. | 1 | ReservedVirtualStorage |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| storagePoolReserved | shared | Navig. | 0..1 | ReservedStoragePool | Information about storage resources that have been reserved, e.g. {"gigabytes": 1000, "snapshots": 10, "volumes": 10}. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VirtualNetworkReservationHasVirtualNetworkAttributesReservation association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::ReservationModule::VirtualNetworkReservationHasVirtualNetworkAttributesReservation

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VirtualNetworkReservationHasVirtualNetworkAttributesReservation

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| VirtualNetworkReservation | none | Not navig. | 1 | VirtualNetworkReservation |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| networkAttributes | shared | Navig. | 0..1 | VirtualNetworkAttributesReservationData | Information specifying additional attributes of the network resource to be reserved. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VirtualNetworkReservationHasVirtualNetworkPortReservation association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::ReservationModule::VirtualNetworkReservationHasVirtualNetworkPortReservation

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association VirtualNetworkReservationHasVirtualNetworkPortReservation

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| virtualNetworkReservation | none | Not navig. | 1 | VirtualNetworkReservation |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| networkPorts | shared | Navig. | 0..* | VirtualNetworkPortReservation/Data | List of specific network ports to be reserved. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

### SoftwareImageModule

#### Overview

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::SoftwareImageModule

##### Diagrams

Figure B.1: Diagram SoftwareImage

##### Classes

###### SoftwareImageInformation class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::SoftwareImageModule::SoftwareImageInformation

Description:

This class represents Software Image Information.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class SoftwareImageInformation

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| softwareImageId | Identifier | 1 | The identifier of this software image | OpenModelAttribute - isInvariant: true - support: MANDATORY |
| name |  | 1 | The name of this software image | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| provider |  | 1 | The provider of this software image. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| version |  | 1 | The version of this software image. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| checksum |  | 1 | The checksum of the software image file. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| containerFormat |  | 1 | The container format indicates whether the software image is in a file format that also contains metadata about the actual software | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| diskFormat |  | 1 | The disk format of a software image is the format of the underlying disk image | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| createdAt |  | 1 | The created time of this software image | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| updatedAt |  | 1 | The updated time of this software image | Obsolete OpenModelAttribute - isInvariant: false - support: MANDATORY |
| minDisk |  | 1 | The minimal Disk for this software image | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| minRam |  | 1 | The minimal RAM for this software image | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| size |  | 1 | The size of this software image | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| status |  | 1 | The status of this software image | Obsolete OpenModelAttribute - isInvariant: false - support: MANDATORY |
| userMetadata | KeyValuePair | 0..* | User-defined metadata. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

### VirtualisedResourceModule

#### Overview

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::VirtualisedResourceModule

##### Diagrams

Figure B.1: Diagram VirtualisedResourceInterface

Figure B.1: Diagram AffinityOrAntiAffinityResources

##### Classes

###### AffinityOrAntiAffinityConstraint class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::VirtualisedResourceModule::AffinityOrAntiAffinityConstraint

Description:

The AffinityOrAntiAffinityConstraint class includes affinity or anti-affinity information of a given virtualised resource (i.e. a virtualised compute, storage or network resource).

Two ways of specifying affinity or anti-affinity can be distinguished: Explicit resource lists and named resource groups. In case of an explicit resource list, the consumer manages the list of resources the actual resource is requested to be affine or anti-affine with, and builds the list as more resources are created. In case of a named resource group, the consumer needs to create the group first by invoking the appropriate operation to create a Compute/Storage/Network resource affinity or anti-affinity constraints group. Subsequently, as part of resource creation, the consumer passes the name(s) or identifier(s) of the group(s) to the producer which manages and builds the group of resources.

The VIM shall support both explicit resource lists and named resource groups for affinity / anti-affinity. The NFVO shall support at least one of these options.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class AffinityOrAntiAffinityConstraint

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| type | AffinityOrAntiAffinityConstraintType | 1 | Indicates whether this is an affinity or anti-affinity constraint. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| scope | AffinityOrAntiAffinityResourceScope | 0..1 | Qualifies the scope of the constraint. In case of compute resource: e.g. "NFVI-PoP" or "NFVI Node". In case of ports: e.g. "virtual switch or router" or "physical NIC", or "physical network" or "NFVI Node". In case of networks: e.g. "physical NIC", "physical network" or "NFVI Node". In case of subnets: it should be ignored. Defaults to "NFVI Node" if absent. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| affinityAntiAffinityResourceList | AffinityOrAntiAffinityResourceList | 0..1 | Consumer-managed list of identifiers of virtualised resources with which the actual resource is requested to be affine or anti-affine. | OpenModelAttribute - isInvariant: false - support: CONDITIONAL_MANDATORY - condition: Either affinityAntiAffinityResourceList or affinityAntiAffinityResourceGroup but not both shall be present |
| affinityAntiAffinityResourceGroup | Identifier | 0..1 | Identifier of the producer-managed group of virtualised resources with which the actual resource is requested to be affine or anti-affine. NOTE: If explicit resource lists for affinity / anti-affinity are supported, the resourceList IE shall be supported. If named resource groups for affinity / anti-affinity are supported, the resourceGroup IE shall be supported. The mechanisms shall not be mixed in the scope of a resourceGroup (aka VIM tenant) | OpenModelAttribute - isInvariant: false - support: CONDITIONAL_MANDATORY - condition: Either affinityAntiAffinityResourceList or affinityAntiAffinityResourceGroup but not both shall be present. |

###### VirtualComputeFlavour class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::VirtualisedResourceModule::VirtualComputeFlavour

Description:

The VirtualComputeFlavour class encapsulates information for compute flavours. A compute flavour includes information about number of virtual CPUs, size of virtual memory, size of virtual storage, and virtual network interfaces.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VirtualComputeFlavour

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| flavourId | Identifier | 1 | Identifier given to the compute flavour. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| accelerationCapability |  | 0..* | Selected acceleration capabilities (e.g. crypto, GPU) from the set of capabilities offered by the compute node acceleration resources. The cardinality can be 0 if no particular acceleration capability is requested | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualMemory | VirtualMemoryData | 1 | The virtual memory of the virtualised compute. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualCpu | VirtualCpuData | 1 | The virtual CPU(s) of the virtualised compute. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| storageAttributes | VirtualStorageData | 0..* | Element containing information about the size of virtualised storage resources (e.g. size of volume, in GB), the type of storage (e.g., volume, object). | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualNetworkInterface | VirtualNetworkInterfaceData | 0..* | The virtual network interfaces of the virtualised compute. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VirtualStorageFlavour class

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::VirtualisedResourceModule::VirtualStorageFlavour

Description:

The VirtualStorageFlavour class encapsulates information for storage flavours. A storage flavour includes information about the size of the storage, and the type of storage.

Applied Stereotypes:

OpenModelClass

support:  MANDATORY

Preliminary

Table B.1: Attributes for Class VirtualStorageFlavour

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| flavourId | Identifier | 1 | Identifier of the storage flavour. | OpenModelAttribute - isInvariant: true - support: MANDATORY |
| storageAttributes | VirtualStorageData | 1 | Element containing information about the size of virtualised storage resource (e.g. size of volume, in GB), the type of storage (e.g., volume, object), and support for RDMA. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

##### Notifications

###### VirtualisedResourceChangeNotification notification

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::VirtualisedResourceModule::VirtualisedResourceChangeNotification

Description:

This notification informs the receiver of changes in the virtualised resources that are allocated. The support of the notification is mandatory.

This notification is produced when the virtualised resource will be impacted due to changes in underlying resources produced by maintenance and operation of the NFVI

Applied Stereotypes:

OpenModelNotification

triggerConditionList:

Maintenance of NFVI components, e.g., physical maintenance/repair, hypervisor software updates, etc.

Evacuation of physical hosts.

Addition and removal of physical resources.

Operation and management of NFVI resources, e.g., to support energy efficiency or resource usage optimization.

NOTE: The above operations could trigger further actions, e.g., migration of virtualised resources.

support:    MANDATORY

Preliminary

Table B.1: Attributes for Signal VirtualisedResourceChangeNotification

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| changeId | Identifier | 1 | Unique identifier of the change on the virtualised resource. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualisedResourceId | Identifier | 1 | Identifier of the instantiated virtualised resource for which the change notification is issued. This identifier value shall be the same as the one returned when the allocation of this virtualised resource was acknowledged. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vimId | Identifier | 1 | Identifier of the VIM reporting the change. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| changeType | String | 1 | It categorizes the type of change. Possible values can be related to maintenance and operation of the NFVI, including e.g., normal, maintenance, evacuation, optimization, etc | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| changedResourceData |  | 0..1 | Details of the changes of the resource Its content can differ based on the different values of the attribute changeType. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

##### Datatypes

###### AffinityOrAntiAffinityConstraintType enumeration

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::VirtualisedResourceModule::AffinityOrAntiAffinityConstraintType

Description:

Indicates whether this is an affinity or anti-affinity constraint.

Applied Stereotypes:

Preliminary

Contains Enumeration Literals:

AFFINITY_CONSTRAINT

ANTI_AFFINITY_CONSTRAINT

###### AffinityOrAntiAffinityResourceScope enumeration

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::VirtualisedResourceModule::AffinityOrAntiAffinityResourceScope

Description:

Qualifies the scope of the constraint, e.g. NFVI-PoP, NFVI Node.

Applied Stereotypes:

Preliminary

###### NetworkSubnetData datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::VirtualisedResourceModule::NetworkSubnetData

Description:

The NetworkSubnetData datatype encapsulates information to allocate or update virtualised sub-networks.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type NetworkSubnetData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| networkId | Identifier | 0..1 | The identifier of the virtualised network that the virtualised sub-network is attached to. The cardinality can be 0 to cover the case where this type is used to describe the L3 attributes of a network rather than a subnetwork or when NetworkSubnetData is part of Update Virtualised Network Resource. NOTE: In these cases, changing the parameter is such a fundamental change that a new virtualised network resource should be created instead of updating an existing network resource. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| ipVersion | IpVersion | 1 | The IP version of the network/subnetwork. Cardinality can be 0 when NetworkSubnetData is part of Update Virtualised Network Resource . NOTE: In these cases, changing the parameter is such a fundamental change that a new virtualised network resource should be created instead of updating an existing network resource. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| gatewayIp | IpAddress | 0..1 | Specifies the IP address of the network/subnetwork gateway when the gateway is selected by the requestor. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| cidr |  | 0..1 | The CIDR of the network/subnetwork, i.e., network address and subnet mask. Cardinality can be 0 when NetworkSubnetData is part of Update Virtualised Network Resource. NOTE: In these cases, changing the parameter is such a fundamental change that a new virtualised network resource should be created instead of updating an existing network resource | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| isDhcpEnabled | Boolean | 0..1 | True when DHCP is to be enabled for this network/subnetwork, or false otherwise. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| addressPool |  | 0..* | Address pools for the network/subnetwork. The cardinality can be 0 when VIM is allowed to allocate all addresses in the CIDR except for the address of the network/subnetwork gateway. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| metadata | KeyValuePair | 0..* | List of metadata key-value pairs used by the consumer to associate meaningful metadata to the related virtualised resource. | OpenModelAttribute - isInvariant: false - support: OPTIONAL |

###### UserData datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::VirtualisedResourceModule::UserData

Description:

Datatype describing user data to customize a virtualized compute resource at boot-time

Applied Stereotypes:

Experimental

Table B.1: Attributes for Data Type UserData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| content | String | 1 | String containing the user data to customize a virtualized compute resource at boot-time. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| method | UserDataTransportationMethod | 0..1 | Method used as transportation media to convey the content of the UserData to the virtualized compute resource. Possible values: CONFIG-DRIVE. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### UserDataTransportationMethod enumeration

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::VirtualisedResourceModule::UserDataTransportationMethod

Applied Stereotypes:

Experimental

Contains Enumeration Literals:

CONFIG-DRIVE

###### VirtualCpuData datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::VirtualisedResourceModule::VirtualCpuData

Description:

Information describing a virtual CPU.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type VirtualCpuData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| cpuArchitecture | String | 0..1 | CPU architecture type. Examples are x86, ARM. The cardinality can be 0 during the allocation request, if no particular CPU architecture type is requested | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| numVirtualCpu | Integer | 1 | Number of virtual CPUs | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| cpuClock | Number | 0..1 | Minimum CPU clock rate (e.g. in MHz) available for the virtualised CPU resources. The cardinality can be 0 during the allocation request, if no particular value is requested. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualCpuOversubscriptionPolicy |  | 0..1 | The CPU core oversubscription policy e.g. the relation of virtual CPU cores to physical CPU cores/threads. The cardinality can be 0 during the allocation request, if no particular value is requested. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vduCpuRequirements |  | 0..* | Array of key-value pair requirements on the Compute (CPU) for the VDU. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualCpuPinning | VirtualCpuPinningData | 0..1 | The virtual CPU pinning configuration for the virtualised compute resource | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VirtualCpuPinningData datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::VirtualisedResourceModule::VirtualCpuPinningData

Description:

Information describing CPU pinning policy and rules for virtual CPU to physical CPU mapping of the virtualised compute resource.

Applied Stereotypes:

Experimental

Table B.1: Attributes for Data Type VirtualCpuPinningData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| virtualCpuPinningPolicy | CpuPinningPolicy | 0..1 | The policy can take values of "static" or "dynamic". In case of "static", the virtual CPU cores are requested to be allocated to logical CPU cores according to the rules defined in virtualCpuPinningRules. In case of "dynamic", the allocation of virtual CPU cores to logical CPU cores is decided by the VIM, (e.g.: SMT (Simultaneous Multi-Threading) requirements). | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| cpuPinningMap |  | 0..1 | If cpuPinningPolicy is defined as "static", the cpuPinninMap provides the map of pinning virtual CPU cores to physical CPU cores/threads. Cardinality can be 0 if cpuPinningPolicy has a different value. | Obsolete OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualCpuPinningRules |  | 0..* | A list of rules that should be considered during the allocation of the virtual CPUs to logical CPUs in case of "static" virtualCpuPinningPolicy. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VirtualInterfaceData datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::VirtualisedResourceModule::VirtualInterfaceData

Description:

A virtual interface represents the data of a virtual network interface specific to a Virtual Compute Resource instance.

Applied Stereotypes:

Experimental

Table B.1: Attributes for Data Type VirtualInterfaceData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| ipAddress | IpAddress | 0..* | The virtual network interface can be configured with specific IP address(es) associated to the network to be attached to. The cardinality can be 0 in the case that a network interface is created without being attached to any specific network, or when an IP address can be automatically configured, e.g. by DHCP. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| macAddress | MacAddress | 0..1 | The MAC address desired for the virtual network interface. The cardinality can be 0 to allow for network interface without specific MAC address configuration. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VirtualMemoryData datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::VirtualisedResourceModule::VirtualMemoryData

Description:

Information describing virtual memory.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type VirtualMemoryData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| virtualMemSize | Number | 1 | Amount of virtual Memory (e.g., in MB) | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| virtualMemOversubscriptionPolicy |  | 0..1 | The memory core oversubscription policy in terms of virtual memory to physical memory on the platform. The cardinality can be 0 during the allocation request, if no particular value is requested. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vduMemRequirements |  | 0..* | Array of key-value pair requirements on the memory for the VDU | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| numaEnabled | Boolean | 0..1 | It specifies the memory allocation to be cognisant of the relevant process/core allocation. The cardinality can be 0 during the allocation request, if no particular value is requested. | OpenModelAttribute - isInvariant: false - support: MANDATORY |

###### VirtualNetworkData datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::VirtualisedResourceModule::VirtualNetworkData

Description:

The VirtualNetworkData idatatype encapsulates information to allocate or update virtualised network resources.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type VirtualNetworkData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| bandwidth | Number | 1 | Minimum network bandwidth (in Mbps). | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| networkType | String | 0..1 | The type of network that maps to the virtualised network. This list is extensible. Examples are: "local", "vlan", "vxlan", "gre", "l3-vpn", etc. The cardinality can be "0" to cover the case where this attribute is not required to create the virtualised network. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| segmentType | String | 0..1 | The isolated segment for the virtualised network. For instance, for a "vlan" networkType, it corresponds to the vlan identifier; and for a "gre" networkType, this corresponds to a gre key. The cardinality can be "0" to allow for flat networks without any specific segmentation. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| networkQoS | NetworkQoS | 0..* | Element providing information about Quality of Service attributes that the network shall support. The cardinality can be "0" to allow for networks without any specified QoS requirements. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| isShared | Boolean | 0..1 | It defines whether the virtualised network is shared among consumers. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| sharingCriteria |  | 0..1 | Only present for shared networks. Indicate the sharing criteria for this network. This criteria might be a list of authorised consumers. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| layer3Attributes | NetworkSubnetData | 0..* | The attribute allows setting up a network providing defined layer 3 connectivity. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| metadata | KeyValuePair | 0..* | List of metadata key-value pairs used by the consumer to associate meaningful metadata to the related virtualised resource. | OpenModelAttribute - isInvariant: false - support: OPTIONAL |

###### VirtualNetworkInterfaceData datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::VirtualisedResourceModule::VirtualNetworkInterfaceData

Description:

Information describing a virtual network interface

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type VirtualNetworkInterfaceData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| networkId | Identifier | 0..1 | In the case when the virtual network interface is attached to the network, it identifies such a network. The cardinality can be 0 in the case that a network interface is created without being attached to any specific network. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| networkPortId | Identifier | 0..1 | If the virtual network interface is attached to a specific network port, it identifies such a network port. The cardinality can be 0 in the case that a network interface is created without any specific network port attachment. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| ipAddress | IpAddress | 0..* | The virtual network interface can be configured with specific IP address(es) associated to the network to be attached to. The cardinality can be 0 in the case that a network interface is created without being attached to any specific network, or when an IP address can be automatically configured, e.g., by DHCP. | Obsolete OpenModelAttribute - isInvariant: false - support: MANDATORY |
| typeVirtualNic |  | 1 | Type of network interface. The type allows for defining how such interface is to be realized, e.g., normal virtual NIC, with direct PCI passthrough, etc. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| typeConfiguration |  | 0..* | Extra configuration that the virtual network interface supports based on the type of virtual network interface. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| macAddress | MacAddress | 0..1 | The MAC address desired for the virtual network interface. The cardinality can be 0 to allow for network interface without specific MAC address configuration. | Obsolete OpenModelAttribute - isInvariant: false - support: MANDATORY |
| bandwidth | Number | 0..1 | The bandwidth of the virtual network interface (in Mbps). | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| accelerationCapabilty |  | 0..* | It specifies if the virtual network interface requires certain acceleration capabilities (e.g., RDMA, packet dispatch, TCP Chimney). | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| metadata | KeyValuePair | 0..* | List of metadata key-value pairs used by the consumer to associate meaningful metadata to the related virtualised resource. | OpenModelAttribute - isInvariant: false - support: OPTIONAL |

###### VirtualNetworkPortData datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::VirtualisedResourceModule::VirtualNetworkPortData

Description:

The VirtualNetworkPortData datatype encapsulates information to allocate or update virtual network ports for network resources. A network port is a communication endpoint under a network.

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type VirtualNetworkPortData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| portType | String | 1 | Type of network port. Examples of types are normal ports, trunk ports or subports. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| networkId | Identifier | 0..1 | Identifier of the network that the port belongs to. When creating a port, such port needs to be part of a network. Cardinality can be 0 when VirtualNetworkPortData is part of Update Virtualised Network Resource. NOTE: In this case, changing the parameter is such a fundamental change that a new virtualised network resource should be created instead of updating an existing network resource. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| segmentId | Identifier | 0..1 | The isolated segment the network port belongs to. For instance, for a "vlan", it corresponds to the vlan identifier; and for a "gre", this corresponds to a gre key. The cardinality can be "0" to allow for flat networks without any specific segmentation. | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| bandwidth | Number | 0..1 | The bandwidth of the virtual network port (in Mbps). Cardinality can be "0" to allow for virtual network ports without any specified bandwidth requirements | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| metadata | KeyValuePair | 0..* | List of metadata key-value pairs used by the consumer to associate meaningful metadata to the related virtualised resource. | OpenModelAttribute - isInvariant: false - support: OPTIONAL |

###### VirtualStorageData datatype

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::VirtualisedResourceModule::VirtualStorageData

Description:

Information describing a virtual storage

Applied Stereotypes:

Preliminary

Table B.1: Attributes for Data Type VirtualStorageData

| Attribute Name | Type | Mult. | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- |
| typeOfStorage | String | 1 | Type of virtualised storage resource (e.g. volume, object). | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| sizeOfStorage | Number | 1 | Size of virtualised storage resource (e.g. size of volume, in GB). | OpenModelAttribute - isInvariant: false - support: MANDATORY |
| vduStorageRequirements |  | 0..* | An array of key-value pairs that articulate the storage deployment requirements. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY |
| rdmaEnabled | Boolean | 0..1 | Indicate if the storage support RDMA | Obsolete OpenModelAttribute - isInvariant: false - support: OPTIONAL |
| swImageDesc | SwImageDesc | 1 | Software image to be loaded on the VirtualStorage Resource created based on this VirtualStorageDesc. | Experimental OpenModelAttribute - isInvariant: false - support: MANDATORY PassedByReference |

##### Associations

###### AffinityContrainstUsesResourceList association

Qualified Name: NfvInformationModel::NFVInterfaceInformationModel::VirtualisedResourceDomain::VirtualisedResourceModule::AffinityContrainstUsesResourceList

Applied Stereotypes:

Preliminary

Table B.1: Member ends for Association AffinityContrainstUsesResourceList

| Attribute Name | Aggreg. | Navig. | Mult. | Type | Description | Applied Stereotypes |
| --- | --- | --- | --- | --- | --- | --- |
| affinityAntiAffinityResourceList | composite | Navig. | 0..1 | AffinityOrAntiAffinityResourceList | Consumer-managed list of identifiers of virtualised resources with which the actual resource is requested to be affine or anti-affine. | OpenModelAttribute - isInvariant: false - support: CONDITIONAL_MANDATORY - condition: Either affinityAntiAffinityResourceList or affinityAntiAffinityResourceGroup but not both shall be present |
| affinityOrAntiAffinityConstraint | none | Not navig. | 1 | AffinityOrAntiAffinityConstraint |  | OpenModelAttribute - isInvariant: false - support: MANDATORY |