# Annex J: Word format presentation of the test suite for the Interconnected Tests

## J.1 Overview

The present annex contains a tabular representation of the high-level test descriptions developed as part of ETSI GS NFV TST 010.

In order to maintain compatibility with previous versions of the deliverable, the numbering of clauses and subclauses is kept as defined in ETSI GS NFV-TST 010. Clauses J.2 to J.9 are therefore to be considered Void.

## J.11 Tabular test descriptions

### 11.3.1	SOL003 Interconnected Test Cases

#### 11.3.1.1	VNFInstanceCreationAndQuery

##### 11.3.1.1.1	Create a new VNF instance resource and query information about multiple and individual VNF instance

| Test Purpose |  |
| --- | --- |
| Test ID | 11.3.1.1.1 |
| Test title | Create a new VNF instance resource and query information about multiple and individual VNF instance |
| Test objective | The objective is to test the interconnected case of VNF instance resource creation and query multiple and individual resources |
| Pre-conditions | None |
| Reference | ETSI GS NFV-SOL 003 [1] v4.5.1 |
| Config ID | Config_prod_NFVO, Config_prod_VNFM |
| Applicability | None |
| Post-conditions | None |
| Test Case |  |
| GET Individual VNF Package POST Create a new vnfInstance GET information about multiple VNF Instances GET information about an individual VNF Instance |  |

NOTE: Robot code can be found at https://forge.etsi.org/rep/nfv/api-tests/raw/4.5.1/interconnectedTestCases/SOL003/VNFInstanceCreationAndQuery.robot

#### 11.3.1.2	VNFInstanceModification

##### 11.3.1.2.1	Modify an individual VNF instance resource

| Test Purpose |  |
| --- | --- |
| Test ID | 11.3.1.2.1 |
| Test title | Modify an individual VNF instance resource |
| Test objective | The objective is to test the interconnected case of VNF instance resource creation and modification |
| Pre-conditions | none |
| Reference | ETSI GS NFV-SOL 003 [1] v4.5.1 |
| Config ID | Config_prod_NFVO, Config_prod_VNFM |
| Applicability | none |
| Post-Conditions | none |
| Test Case |  |
| GET Individual VNF Package POST Create a new vnfInstance PATCH Individual VNFInstance |  |

NOTE: Robot code can be found at https://forge.etsi.org/rep/nfv/api-tests/raw/4.5.1/interconnectedTestCases/SOL003/VNFInstanceModification.robot

#### 11.3.1.3	VNFInstanceDeletion

##### 11.3.1.3.1	Delete an individual VNF instance

| Test Purpose |  |
| --- | --- |
| Test ID | 11.3.1.3.1 |
| Test title | Delete an individual VNF instance |
| Test objective | The objective is to test the interconnected case of VNF instance resource creation and deletion |
| Pre-conditions | none |
| Reference | ETSI GS NFV-SOL 003 [1] v4.5.1 |
| Config ID | Config_prod_NFVO, Config_prod_VNFM |
| Applicability | none |
| Post-Conditions | none |
| Test Case |  |
| GET Individual VNF Package POST Create a new vnfInstance DELETE Individual VNFInstance |  |

NOTE: Robot code can be found at https://forge.etsi.org/rep/nfv/api-tests/raw/4.5.1/interconnectedTestCases/SOL003/VNFInstanceDeletion.robot

#### 11.3.1.4	VNFInstanceInstantiation

##### 11.3.1.4.1	Instantiate a VNF instance

| Test Purpose |  |
| --- | --- |
| Test ID | 11.3.1.4.1 |
| Test title | Instantiate a VNF instance |
| Test objective | The objective is to test the interconnected case of VNF instance resource creation and instantiation |
| Pre-conditions | none |
| Reference | ETSI GS NFV-SOL 003 [1] v4.5.1 |
| Config ID | Config_prod_NFVO, Config_prod_VNFM |
| Applicability | none |
| Post-Conditions | none |
| Test Case |  |
| GET Individual VNF Package POST Create a new vnfInstance VNF Instantiation |  |

NOTE: Robot code can be found at https://forge.etsi.org/rep/nfv/api-tests/raw/4.5.1/interconnectedTestCases/SOL003/VNFInstanceInstantiation.robot

#### 11.3.1.5	VNFInstanceTermination

##### 11.3.1.5.1	Terminate a VNF instance

| Test Purpose |  |
| --- | --- |
| Test ID | 11.3.1.5.1 |
| Test title | Terminate a VNF instance |
| Test objective | The objective is to test the interconnected case of VNF instance resource creation, instantiation and termination |
| Pre-conditions | none |
| Reference | ETSI GS NFV-SOL 003 [1] v4.5.1 |
| Config ID | Config_prod_NFVO, Config_prod_VNFM |
| Applicability | none |
| Post-Conditions | none |
| Test Case |  |
| GET Individual VNF Package POST Create a new vnfInstance VNF Instantiation Terminate a VNF Instance |  |

NOTE: Robot code can be found at https://forge.etsi.org/rep/nfv/api-tests/raw/4.5.1/interconnectedTestCases/SOL003/VNFInstanceTermination.robot

#### 11.3.1.6	VNFInstanceOperationOccurrences

##### 11.3.1.6.1	Retrieve VNF instance Operation Occurrences

| Test Purpose |  |
| --- | --- |
| Test ID | 11.3.1.6.1 |
| Test title | Retrieve VNF instance Operation Occurrences |
| Test objective | The objective is to test the interconnected case of VNF instance resource creation, instantiation and retrieval of information about VNF lifecycle management operation occurrences |
| Pre-conditions | none |
| Reference | ETSI GS NFV-SOL 003 [1] v4.5.1 |
| Config ID | Config_prod_NFVO, Config_prod_VNFM |
| Applicability | none |
| Post-Conditions | none |
| Test Case |  |
| GET Individual VNF Package POST Create a new vnfInstance VNF Instantiation GET status information about multiple VNF LCM Operation OCC Get Individual VNF LCM Operation occurrence |  |

NOTE: Robot code can be found at https://forge.etsi.org/rep/nfv/api-tests/raw/4.5.1/interconnectedTestCases/SOL003/VNFInstanceOperationOccurrences.robot

#### 11.3.1.7	SubscriptionCreationAndQuery

##### 11.3.1.7.1	Create a new subscription and query information about multiple and individual subscription resources

| Test Purpose |  |
| --- | --- |
| Test ID | 11.3.1.7.1 |
| Test title | Create a new subscription and query information about multiple and individual subscription resources |
| Test objective | The objective is to test the interconnected case of subscription resource creation and query multiple and individual resources |
| Pre-conditions | None |
| Reference | ETSI GS NFV-SOL 003 [1] v4.5.1 |
| Config ID | Config_prod_NFVO, Config_prod_VNFM |
| Applicability | None |
| Post-conditions | None |
| Test Case |  |
| GET Individual VNF Package POST Create a new subscription GET Subscriptions GET Individual Subscription |  |

NOTE: Robot code can be found at https://forge.etsi.org/rep/nfv/api-tests/raw/4.5.1/interconnectedTestCases/SOL003/SubscriptionCreationAndQuery.robot

#### 11.3.1.8	SubscriptionDeletion

##### 11.3.1.8.1	Delete individual subscription

| Test Purpose |  |
| --- | --- |
| Test ID | 11.3.1.8.1 |
| Test title | Delete individual subscription |
| Test objective | The objective is to test the interconnected case of subscription resource creation and deletion |
| Pre-conditions | none |
| Reference | ETSI GS NFV-SOL 003 [1] v4.5.1 |
| Config ID | Config_prod_NFVO, Config_prod_VNFM |
| Applicability | none |
| Post-Conditions | Subscription deleted from VNFM |
| Test Case |  |
| GET Individual VNF Package POST Create a new subscription GET Subscriptions GET Individual Subscription DELETE an individual subscription |  |

NOTE: Robot code can be found at https://forge.etsi.org/rep/nfv/api-tests/raw/4.5.1/interconnectedTestCases/SOL003/SubscriptionDeletion.robot

#### 11.3.1.9	VNFInstanceScaleOut

##### 11.3.1.9.1	Scale Out a VNF instance

| Test Purpose |  |
| --- | --- |
| Test ID | 11.3.1.9.1 |
| Test title | Scale Out a VNF instance |
| Test objective | The objective is to test the interconnected case of VNF instance resource creation, instantiation and scale out |
| Pre-conditions | none |
| Reference | ETSI GS NFV-SOL 003 [1] v4.5.1 |
| Config ID | Config_prod_NFVO, Config_prod_VNFM |
| Applicability | none |
| Post-Conditions | none |
| Test Case |  |
| GET Individual VNF Package POST Create a new vnfInstance VNF Instantiation VNF Instance Scale Out |  |

NOTE: Robot code can be found at https://forge.etsi.org/rep/nfv/api-tests/raw/4.5.1/interconnectedTestCases/SOL003/VNFInstanceScaleOut.robot

#### 11.3.1.10	VNFInstanceScaleIn

##### 11.3.1.10.1	Scale In a VNF instance

| Test Purpose |  |
| --- | --- |
| Test ID | 11.3.1.10.1 |
| Test title | Scale In a VNF instance |
| Test objective | The objective is to test the interconnected case of VNF instance resource creation, instantiation and scale in |
| Pre-conditions | none |
| Reference | ETSI GS NFV-SOL 003 [1] v4.5.1 |
| Config ID | Config_prod_NFVO, Config_prod_VNFM |
| Applicability | none |
| Post-Conditions | none |
| Test Case |  |
| GET Individual VNF Package POST Create a new vnfInstance VNF Instantiation VNF Instance Scale In |  |

NOTE: Robot code can be found at https://forge.etsi.org/rep/nfv/api-tests/raw/4.5.1/interconnectedTestCases/SOL003/VNFInstanceScaleIn.robot